| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0152 WG11 Number: m26447 |
| --- | --- |

| Title: | Chroma sampling filter hint SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

- Abstract

Although most of video format standardizations specify the colour transform matrix and bit-depth, filter coefficient set for up-sampling and down-sampling of chroma format have not been specified until recently. In order to reduce conversion losses of chroma format, this contribution proposes a method of SEI message for chroma sampling.

Introduction

The first version of HEVC [1] has specified a profile for 4:2:0 chroma format and 8-bit depth pixel sample. In the next version, a specification for 4:2:2 and 4:4:4 chroma format and n-bit depth pixel sample is planned. Although most of video format standardization specifies the colour transform matrix and bit-depth, filter coefficient set for up-sampling and down-sampling of chroma format have not been specified until recently.  For example, it is known that there is a problem of chroma shift when two or more 4:2:0 codecs are concatenated because the codecs are usually connected via HD-SDI (a 4:2:2 interconnect) resulting in up-sampling and down-sampling of the chroma component. In order to avoid those problems, SMPTE RP-2050-1:2012 [2], which defines filter sets for 4:2:2 / 4:2:0 and 4:2:0 / 4:2:2 conversions, has been published recently. In this contribution, we propose a method of SEI message for chroma sampling which can solve those problems generally. In the next section, the detailed specification is described.

Chroma sampling filter hint SEI

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else if( payloadType = = XXX) |  |
| chroma_sampling_filter_hint( payloadSize) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( !byte_aligned( ) ) { |  |
| bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

- D.1.X	Chroma sampling filter hint SEI syntax

| chroma_sampling_filter_hint(payloadSize) { | Description |
| --- | --- |
| coded_format_idc | ue(v) |
| coded_data_bit_depth | ue(v) |
| target_format_idc | ue(v) |
| target_bit_depth | ue(v) |
| max_value | ue(v) |
| min_value | ue(v) |
| if( (coded_fromat_idc = = 1 && target_format_idc > 1) ¦ ¦ (coded_fromat_idc > 1 && target_format_idc = = 1) ) { |  |
| progressive_flag | u(1) |
| implicit_vertical_flag | u(1) |
| if( implicit_vertical_flag = = 1 ) |  |
| vertical_filter_idc | u(4) |
| else { |  |
| tap_length_vertical_minus1 | ue(v) |
| for( i = 0; i <= tap_length_vertical_minus1; i++) |  |
| ver_filter_coeff[i] | se(v) |
| } |  |
| if( !progressive_flag && coded_fromat_idc = = 1 && target_format_idc > 1 ) |  |
| if( implicit_vertical_flag = = 1 ) |  |
| second_vertical_filter_idc | u(4) |
| else { |  |
| tap_length_second_vertical_minus1 | ue(v) |
| for( i = 0; i <= tap_length_second_vertical_minus1; i++) |  |
| second_ver_filter_coeff[i] | se(v) |
| } |  |
| } |  |
| if( ((coded_format_idc = =1 ¦ ¦ coded_format_idc = = 2) && target_format_idc = = 3)) ¦ ¦ (coded_format_idc = = 3 && (target_format_idc = = 1¦ ¦ target_format_idc = = 2))) { |  |
| implicit_horizontal_flag | u(1) |
| if( implicit_horizontal_flag = = 1 ) |  |
| horizontal_filter_idc | u(4) |
| else { |  |
| tap_length_horizontal_minus1 | ue(v) |
| for( i = 0; i <= tap_length_horizontal_minus1; i++) |  |
| hor_filter_coeff[i] | se(v) |
| } |  |
| } |  |
| } |  |

- D.2.X	Chroma sampling filter hint SEI semantics

coded_format_idc specifies the chroma_format_idc of the chroma component of the coded video sequence. It is used to identify the chroma sampling filter hint SEI message that is intended for use with the coded video sequence. If chroma sampling filter hint SEI messages are present that have coded_format_idc that is not equal to chroma_format_idc, these refer to the hypothetical result of a transcoding operation performed to convert the coded video to the chroma_format_idc corresponding to the value of coded_format_idc. The value of coded_format_idc shall be in the range of 1 to 3, inclusive.

coded_data_bit_depth specifies the BitDepthC of the chroma component of the coded video sequence. It is used to identify the chroma sampling filter hint SEI message that is intended for use with the coded video sequence. If chroma sampling filter hint SEI messages are present that have coded_data_bit_depth that is not equal to BitDepthC, these refer to the hypothetical result of a transcoding operation performed to convert the coded video to the BitDepthC corresponding to the value of coded_data_bit_depth. The value of coded_data_bit_depth shall be in the range of 8 to 14, inclusive.

target_format_idc specifies the output of chroma sampling relative to the luma sampling as specified in subclause 6.2. The value of target_format_idc shall be in the range of 0 to 3, inclusive.

target_bit_depth specifies the bit depth of the output samples of the chroma array. target_bit_depth  shall be in the range of 1 to 16, inclusive.

max_value specifies the maximum value in the signalled target_bit_depth. All values in the coded data that are larger than or equal to max_value are mapped to this maximum value in the target_bit_depth representation.

min_value specifies the minimum value in the signalled target_bit_depth. All values in the coded data that are less than or equal to min_value are mapped to this minimum value in the target_bit_depth representation.

progressive_flag qual to 1 indicates that the scan type of the associated picture should be interpreted as progressive. progressive_flag equal to 0 indicates that the scan type of the associated picture should be interpreted as interlaced.

implicit_vertical_flag equal to 1 specifies that the filter coefficient of vertical direction as specified in Table D-X. implicit_vertical_flag equal to 0 specifies the vertical filter coefficient information explicitly.

vertical_filter_idc indicates the value and tap-length of the filter coefficient of vertical direction as specified in Table D-X. The value of vertical_filter_idc shall be in the range of 0 to 6, inclusive.

tap_length_vertical_minus1 + 1 specifies the number of tap length of vertical filter. The value of tap_length_vertical_minus1 shall be in the range of 0 to 15, inclusive.

ver_filter_coeff[i] specifies the filter coefficient value for vertical direction. The value of ver_filter_coeff[i] shall be in the range of −231 + 1 to 231 − 1, inclusive.

second_vertical_filter_idc indicates the value and tap-length of the filter coefficient of vertical direction as specified in Table D-X. The value of second_vertical_filter_idc shall be in the range of 0 to 6, inclusive.

tap_length_second_vertical_minus1 + 1 specifies the number of tap length of second vertical filter. The value of tap_length_vertical_minus1 shall be in the range of 0 to 15, inclusive.

second_ver_filter_coeff[i] specifies the filter coefficient value for vertical direction. The value of second_ver_filter_coeff[i] shall be in the range of −231 + 1 to 231 − 1, inclusive.

implicit_horizontal_flag equal to 1 specifies that the filter coefficient of horizontal direction as specified in Table D-X. implicit_horizontal_flag equal to 0 specifies the horizontal filter coefficient information explicitly.

horizontal_filter_idc indicates the value and tap-length the filter coefficient of horizontal direction as specified in Table D-X. The value of horizontal_filter_idc shall be in the range of 0 to 6, inclusive.

tap_length_horizontal_minus1 + 1 specifies the number of tap length of horizontal filter. The value of tap_length_horizontal_minus1 shall be in the range of 0 to 15, inclusive.

hor_filter_coeff[i] specifies the filter coefficient value for horizontal direction. The value of hor_filter_coeff[i] shall be in the range of −231 + 1 to 231 − 1, inclusive.

Table D-X	Implicit filter coefficients

| Value | Tap length | Filter coefficients | Divisor | Informative remark |
| --- | --- | --- | --- | --- |
| 0 | 8 | -3,-19,34,500,500,34,-19,-3 | 1024 | Down-sampling filter coefficients for 4:2:2 / 4:2:0 conversion for progressive source SMPTE RP2050-1:2012 |
| 1 | 4 | 19,103,1037,-135 | 1024 | Up-sampling filter coefficients for 4:2:0 / 4:2:2 conversion for Even Lines of progressive source (Reverse order coefficients for Odd Lines) SMPTE RP2050-1:2012 |
| 2 | 8 | -8,-26,115,586,409,-48,-4,0 | 1024 | Down-sampling filter coefficients for 4:2:2 / 4:2:0 conversion for the top field of interlace source (Reverse order coefficients for the bottom field) SMPTE RP2050-1:2012 |
| 3 | 4 | 24,-41,1169,-128 | 1024 | Up-sampling filter coefficients for 4:2:0 / 4:2:2 conversion for Even Lines of the top field of interlace source (Reverse order coefficients for Odd Lines of the bottom field) SMPTE RP2050-1:2012 |
| 4 | 4 | -76,783,330,-13 | 1024 | Up-sampling filter coefficients for 4:2:0 / 4:2:2 conversion for Odd Lines of the top field of interlace source (Reverse order coefficients for Even Lines of the bottom field) SMPTE RP2050-1:2012 |
| 5 | 7 | -29,0,88,138,88,0,-29 | 256 | Down-sampling filter coefficients MPEG93/N0400 TM5 |
| 6 | 4 | -3,35,35,-3 | 64 | Up-sampling filter coefficients MPEG93/N0400 TM5 |
| 7..15 | Reserved |  |  | For future use by ITU-T ¦ ISO/IEC |

NOTE1 – SMPTE EG2050-2:2012 provides details on a possible implementation of the filter defined in SMPTE RP2050-1:2012.

References

[1] 	B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan and T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 8,” Joint Collaborative Team on Video Coding, JCTVC-J1003r7, Stockholm, July 2012.

[2] 	“4:2:2 / 4:2:0 Format Conversion Minimizing Color Difference Signal Degradation in Concatenated Operations – Filtering,” SMPTE RP2050-1:2012, Jan. 2012.

[3] 	“Test Model 5,” ISO/IEC JCT1/SC29/WG11 MPEG93/N0400, ITU-T Q.2/15 AVC-491b, April 1993.

[4] 	“4:2:2 / 4:2:0 Format Conversion Minimizing Color Difference Signal Degradation in Concatenated Operations – Application,” SMPTE EG2050-2:2012, Jan. 2012.

Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).