| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Shanghai, CN, 10–19 Oct. 2012 | Document: JCTVC-K0192 |
| --- | --- |

| Title: | AHG7: Chroma coding structure/tools in HEVC fidelity range extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Tomonobu Yoshino Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes the consistent coding structure and tools for all planes. Since 4:4:4 chroma format has the same resolution for all planes, some limitations can be relaxed. Furthermore, different structure and prediction mode between each plane cause the visual artifact in RGB 4:4:4 format.

Introduction

This contribution proposes to utilize the same structures and tools for all planes in HEVC fidelity range extension (FrExt). For instance, RGB444 chroma format has the same resolute and the same visual significance for all planes. Therefore it is reasonable that all planes use the same coding structure, prediction mode, and tools.

Problem statement and proposed structure/tools

Problem statement

In 4:4:4 chroma format, non-main plane like Cb/Cr and B/R have the same resolution as main plane like Y and G. Some coding structure, prediction mode, and tools are however different between main and non-main plane. In the following, three categories are described.

In 4:2:0 chroma format, coding structure of non-main plane is different from that of main plane. For example, PART_NxN indicate that luma has four prediction modes but chroma has only one prediction mode. For other example, restriction of minimum TU size causes the difference of TU depth between luma and chroma.

Regarding quantization, chroma QP clipping function is introduced for YUV color space. For RGB color space, since all planes have generally equaled visual significance, QP clipping is not needed.

Chroma intra prediction can select different prediction mode from luma prediction mode. For RGB color space, different prediction sometimes causes a visual artifact due to the RGB display. Since signal in YUV color space are transformed to that in RGB space of display, such artifact is suppressed.

Although it is implementation issue, current HEVC test-model decides coding structure and prediction mode based on only luma component in intra frame.

Coding structure

We proposed to allow use of the PART_NxN for non-main planes. This modification enables all PUs to have the same intra prediction mode.

Coding mode

For RGB color space, we propose to always use the same prediction mode for all planes. Consequently, no chroma intra-mode coding is necessary.

Coding tools

We proposed to apply some existing intra-smoothing-tools to all planes. On the other hand, after signal processing, we propose to independently use an entropy coding tool like significance map.

Implementation

The proposed scheme is realized by change the following macro in TypeDef.h.

#define ECF__COMBINED_LUMA_CHROMA_INTRA_MODE_SEARCH 1

#define ECF__REDUCED_CHROMA_INTRA_MODE_SET 1

#define ECF__ADDITIONAL_TRIAL_ENCODE_CHROMA_INTRA_MODE_SEARCH 0

#define ECF__INTRA_NxN_CU_CHROMA_PU_SPLIT_MODE 1

#define ECF__ENCODER_FULL_RATE_DISTORTION_SEARCH_OVER_ALL_COMPONENTS 1

#define ECF__CHROMA_INTRA_REFERENCE_SAMPLE_FILTERING 1

#define ECF__SET_INTRA_CHROMA_EDGE_FILTER_422 1

#define ECF__SET_INTRA_CHROMA_DC_FILTER_422 1

#define ECF__SET_INTRA_CHROMA_EDGE_FILTER_444 1

#define ECF__SET_INTRA_CHROMA_DC_FILTER_444 1

#define ECF__USE_LUMA_FILTER_FOR_CHROMA_QUARTER_SAMPLE_INTERPOLATION 1

#define ECF__ENABLE_MDDT_FOR_444_CHROMA 1

#define ECF__USE_TRANSFORM_DEPTH_FOR_444_CHROMA_CBF_CONTEXT_SELECTION 0

#define ECF__SIGNIFICANCE_MAP_CONTEXT_CHANNEL_SEPARATION 2

#define ECF__LAST_POSITION_CONTEXT_CHANNEL_SEPARATION 2

#define ECF__C1_C2_CONTEXT_CHANNEL_SEPARATION 2

#define ECF__CBF_CONTEXT_CHANNEL_SEPARATION 2

#define ECF__EXTENDED_CHROMA_SIGNIFICANCE_MAP_CONTEXT 1

#define ECF__EXTENDED_CHROMA_LAST_POSITION_CONTEXT 1

#define ECF__EXTENDED_CHROMA_C1_C2_CONTEXT 1

#define ECF__ADDITIONAL_CHROMA_QP_MAPPING_TABLES 1

Experimental results

Common test condition for AHG7 is follows the BoG report of JCTVC-J0581.

Table 1 shows the summary of BD-rate for YUV444 sequences.

|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Kimono | -5.4% | -1.4% | 3.4% | -3.8% | 5.7% | 6.7% | -3.4% | 3.6% | 7.8% |
| Parkscene | -6.2% | -1.3% | 0.6% | -5.0% | 3.7% | -1.2% | -4.7% | 2.6% | -0.1% |
| BirdsInCage | -3.4% | -4.2% | 5.7% | -3.8% | 14.1% | 10.0% | -2.7% | 9.1% | 4.8% |
| DucksAndLegs | -6.1% | -1.1% | 1.5% | -10.7% | 3.1% | -4.6% | -8.4% | 1.1% | -4.4% |
| Traffic | -5.0% | -1.1% | 3.9% | -5.4% | 5.0% | 9.3% | -4.8% | 6.0% | 12.5% |
| CrowdRun | -5.0% | -1.5% | -0.1% | -3.9% | 2.6% | 4.3% | -2.8% | 3.1% | 6.2% |
| OldTownCross | -1.0% | -4.2% | 3.0% | -1.9% | 4.9% | 11.9% | 0.5% | 1.9% | 14.6% |
| Seeking | -4.7% | -2.6% | -1.3% | -4.2% | 4.0% | 6.1% | -3.1% | 2.9% | 6.2% |
| Overall | -4.6% | -2.2% | 2.1% | -4.8% | 5.4% | 5.3% | -3.7% | 3.8% | 5.9% |
| Enc Time[%] | 132% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 111% |  |  | 110% |  |  |

Table 1 Results of YUV444 chroma format.

Due to the disable of chroma QP clipping, chroma BD-rate is significantly decreased. Unfortunately, current CTC of AHG7 has no RGB sequences. Because chroma QP clipping is effective for YUV chroma format, viewing test is required to show the effectiveness.

Conclusion

This contribution proposed the consistent coding structure, the coding mode, and the tools for 444 chroma format. Experimental results showed that the proposed method achieved 4.6%/4.8%/3.7% BD-rate reduction for AI/RA/LDB-HE10 configuration for YUV 444 sequences. Chroma BD-rate degradation was caused by the disabled chroma QP clipping.

References

- D. Flynn, “BoG report: Extended chroma formats,” JCTVC-J0581, Stockholm, July 2012.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).