| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013 | Document: JCTVC-L0049 |
| --- | --- |

| Title: | SEI message: independently decodable regions based on tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yong He, Yuwen He 9710 Scranton Rd, Suite 250 San Diego, CA 92121 Xiaofeng Yang, Peiyu Yue, Yuanyuan Zhang Nanjing R&D Center, 101 Software Avenue, Yuhuatai District, Nanjing, P.R.China 200012 Michael Horowitz 1450-1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: Tel: Email: Tel: Email: | +1 858.210.4803 yan.ye@interdigital.com +86 25 56620730 yx.yangxiaofeng@huawei.com +1 778.331.3432 x323 michael@ebriskvideo.com |
| Source: | InterDigital Communications, LLC, Huawei Technologies Co., Ltd., and eBrisk Video |  |  |

_____________________________

# Abstract

This is a follow-up proposal to JCTVC-K0116 and JCTVC-K0248. At the 11th JCT-VC meeting, JCTVC-K0116 and JCTVC-K0248 proposed an SEI message to support independently decodable regions using tiles. In this contribution, simulation results are provided to show that, for typical use case scenarios, the restrictions on motion compensated prediction as required by the proposed SEI message incur RD performance penalty of 1.4% and 2.5% when loop filter across tile boundary is disabled and enabled, respectively. Subjective viewing of the reconstructed video did not reveal noticeable coding artefacts.

# Introduction

A number of applications can benefit from the capability to independently decode a portion of the video frames from a video sequence. For example, in frame compatible 3D video (Figure 1), the user may have 2D-only display. In this case, the decoder will need to only decode half of the frame, e.g., the left view. In multi-way video conference system (Figure 2), when a speaker is giving a presentation and/or sharing a PPT, it is often useful to be able to display only a portion of the video screen that contains the presentation and/or the speaker on user’s demand. Another example is the e-learning system. Recently, Stanford researchers applied independently decodable region concept to H.264/AVC to an online e-learning project called ClassX [2]. Figure 3 shows an example of ClassX Mobile with user zooming into the video to focus on the content on the blackboard. As the recent boom of mobile video applications continues, providing support for independently decodable regions in HEVC is particularly crucial, as it allows the decoder the option to significantly reduce decoding complexity by skipping decoding and displaying of unwanted regions in the video frame.

The HEVC Main profile supports tiles, which are independently decodable bitstream units that represent rectangular regions of a video frame. This makes tiles a natural choice to support independently decodable regions in HEVC. At the last meeting, JCTVC-K0116 and JCTVC-K0248 proposed tiles-based SEI message for this purpose.

Fig. 1 Frame compatible 3D video

Fig. 2 Multi-point video conference

Fig. 3 ClassX Mobile

# Tile sections

In general, assume the ROI has overlap with N tiles. Here the ROI refers to the region that is independently decodable from the remaining portion of the video frame. These N tiles together form a so called ROI tile section. For example, in Fig. 4, the two tiles on the left form an ROI tile section.

In current HEVC draft, in-loop filtering can be applied across tile boundaries if loop_filter_across_tiles_enabled_flag (in PPS) is set to 1. If the decoder chooses to decode only the ROI tile section, but not the remaining non-ROI tiles, then it will not have neighbouring samples from tiles outside the ROI tile section to perform proper in-loop filtering, including SAO and deblocking. To avoid temporal error propagation, the samples that are affected by non-conformant decoding behaviour should not be used by MCP to predict samples in future pictures. That is, MCP should not only be constrained to be within the ROI tile section, a narrow range of sample locations along the border of the ROI tile section should be excluded from MCP as well. As shown in Figure 4, a band, 4 samples wide along the right border of the ROI tile section (shaded area), should be excluded from MCP. The width of the 4-sample-wide band is determined by the length of the deblocking filter, which has larger neighbourhood dependency than SAO. Although decoding only samples in the ROI tile section causes samples within the narrow band (shaded area) to have values that deviate from their correctly decoded values, such deviation will not cause temporal error propagation due to the MCP restriction thus imposed.

In Figure 4, the ROI tile section only has one boundary (out of four boundaries) that is not a picture boundary. In general, an ROI tile section can have more than one boundary that is not a picture boundary but is a boundary with other non-ROI tiles. In the general case, any boundary of an ROI tile section that is not a picture boundary should have the narrow band of 4 samples wide (shaded area in Figure 4); samples within these narrows bands should not be used for MCP.

Fig. 4 Constraint on range of MCP to support tile-based independently decodable regions

When loop_filter_across_tiles_enabled_flag is set to 0, the size of the narrow band for which MCP restriction is applied is set to 0. This can be useful for applications such as frame compatible 3D video (Figure 1) and multi-point video conference (Figure 2), for which discontinuity typically already exists inside the video frame.

# Proposed SEI message

| ROI_tile_section ( payloadSize ) { | Descriptor |
| --- | --- |
| num_ROI_tile_sections | ue(v) |
| for( i = 0; i < num_ROI_tile_sections; i++ ) { |  |
| num_tiles_in_ROI_section_minus1[ i ] | u(v) |
| for( j = 0; j <= num_tiles_in_ROI_section_minus1[ i ]; j++ ) |  |
| tile_index[ i ][ j ] | u(v) |
| } |  |
| } |  |

num_ROI_tile_sections specifies the number of ROI tile sections in the picture. num_ROI_tile_sections shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1), inclusive. If num_ROI_tile_sections is not present, it is inferred to be 0. An ROI tile section defines an area within a video picture for which motion compensated prediction is constrained. Motion compensated prediction of any samples within an ROI tile section should not depend on any samples from outside the ROI tile section, or from no more than 4 samples away from any ROI tile section boundary, unless the ROI tile section boundary is also a picture boundary.

num_tiles_in_ROI_section_minus1[ i ] plus 1 specifies the number of tiles in the i-th ROI tile section. num_tiles_in_ROI_section_minus1[ i ]  shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive.

tile_index[ i ] [ j ] specifies the index of the j-th tile in the i-th ROI tile section in the picture. tile_index[ i ] [ j ] shall be in the range of 0 to (num_tile_columns_minus1 + 1) * (num_tile_rows_minus1 + 1) − 1, inclusive. The index values of the tiles in a picture are assigned by starting from 0 in the top left corner, and incrementing by 1 following the raster scan order.

NOTE – When num_ROI_tile_section is equal to 0, no ROI tile section is defined for the pictures in the video sequence. That is, there is no constraint on the range of motion compensated prediction for all samples in the picture. When num_ROI_tile_section is equal to or greater than 1, composition of each of the ROI tile sections is signaled using tile_index[ i ][ j ].

# Simulation results

At the 11th JCT-VC meeting in Shanghai, the concept of tiles-based independently decodable regions, as proposed in JCTVC-K0116 and JCTVC-K0248, received non-proponent support. The remaining concerns about the proposals were possible visual artefacts and potential coding efficiency loss due to the restriction placed on motion compensated prediction. In this contribution, the proposed MCP restriction was implementation on HM8.1. The video frame is partitioned into two horizontal, even tiles (--NumTileColumnsMinus1=1 --UniformSpacingIdc=1). MCP is constrained to only use samples within the region covered by each tile, respectively. If loop filter is enabled across tile boundary (--LFCrossTileBoundaryFlag=1), MCP is further constrained to exclude samples that are no more than 4 samples away from the tile boundary in the middle of the frame.

Only Class A and B sequences were used in the simulation, since the concept of independently decodable regions is less applicable to frame sizes smaller than 1080p. QP 22 was used. Other test conditions were kept the same as the common test conditions. Table 1 shows the performance impact by imposing the MCP restriction for LFCrossTileBoundaryFlag = 0 (0-sample band). The bit rate is increased by 1.4% on average, with minimal PSNR difference. Table 2 shows the performance impact by imposing the MCP restriction for LFCrossTileBoundary = 1 (4-sample band). The performance loss is slightly larger than Table 1 (which is expected as the MCP restriction for this case is slightly more severe); on average the bit rate is increased by 2.5%, and PSNR loss is negligible.

The reconstructed video was viewed at normal frame rate. No obvious coding artefacts were observed.

Additional tests with frame compatible 3D sequences will be added shortly.

Table 1. Performance impact with MCP restriction when LFCrossTileBoundaryFlag = 0 (0-sample band), average rate increase is 1.4%

|  |  | HM8.1 |  |  |  | HM8.1+MCP restriction |  |  |  | % rate increase | Y PSNR drop |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |  |  |
| Class A RA | Traffic | 13157.84 | 41.66 | 41.54 | 44.22 | 13328.33 | 41.65 | 41.54 | 44.21 | 1.3% | -0.01 |
|  | PeopleOnStreet | 32763.69 | 40.18 | 44.89 | 44.67 | 33036.22 | 40.17 | 44.87 | 44.66 | 0.8% | -0.01 |
|  | Nebuta | 216482.93 | 39.04 | 39.14 | 37.71 | 217066.48 | 39.04 | 39.13 | 37.70 | 0.3% | -0.01 |
|  | SteamLocomotive | 23610.19 | 41.37 | 46.22 | 45.89 | 23795.98 | 41.37 | 46.21 | 45.89 | 0.8% | -0.01 |
| Class B RA | Kimono | 4792.63 | 41.61 | 43.47 | 45.26 | 4855.91 | 41.59 | 43.46 | 45.25 | 1.3% | -0.01 |
|  | ParkScene | 7663.60 | 40.07 | 42.38 | 43.79 | 7769.78 | 40.06 | 42.38 | 43.78 | 1.4% | -0.01 |
|  | Cactus | 18278.54 | 38.48 | 40.04 | 43.60 | 18506.47 | 38.47 | 40.04 | 43.59 | 1.2% | -0.01 |
|  | BasketballDrive | 17362.12 | 39.16 | 43.77 | 44.99 | 17624.34 | 39.15 | 43.77 | 44.97 | 1.5% | -0.01 |
|  | BQTerrace | 39673.70 | 37.42 | 42.11 | 44.29 | 40139.15 | 37.41 | 42.11 | 44.29 | 1.2% | -0.01 |
| Class B LDB | Kimono | 5209.49 | 41.68 | 43.26 | 44.75 | 5324.40 | 41.66 | 43.26 | 44.75 | 2.2% | -0.02 |
|  | ParkScene | 7947.19 | 39.91 | 41.93 | 43.07 | 8112.59 | 39.89 | 41.92 | 43.06 | 2.1% | -0.01 |
|  | Cactus | 19903.27 | 38.69 | 40.11 | 43.36 | 20184.61 | 38.68 | 40.10 | 43.35 | 1.4% | -0.01 |
|  | BasketballDrive | 19815.99 | 39.42 | 43.64 | 44.84 | 20252.70 | 39.40 | 43.62 | 44.82 | 2.2% | -0.01 |
|  | BQTerrace | 52655.31 | 38.23 | 41.95 | 44.08 | 53390.23 | 38.23 | 41.93 | 44.07 | 1.4% | -0.01 |

Table 2. Performance impact with MCP restriction when LFCrossTileBoundaryFlag = 1 (4-sample band), average rate increase is 2.5%

|  |  | HM8.1 |  |  |  | HM8.1+MCP restriction |  |  |  | % rate increase | Y PSNR drop |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |  |  |
| Class A RA | Traffic | 13155.03 | 41.66 | 41.54 | 44.21 | 13513.16 | 41.64 | 41.53 | 44.20 | 2.7% | -0.02 |
|  | PeopleOnStreet | 32765.69 | 40.17 | 44.88 | 44.66 | 33267.28 | 40.17 | 44.85 | 44.64 | 1.5% | -0.01 |
|  | Nebuta | 216469.62 | 39.04 | 39.13 | 37.70 | 217172.29 | 39.04 | 39.13 | 37.69 | 0.3% | -0.01 |
|  | SteamLocomotive | 23626.29 | 41.37 | 46.21 | 45.89 | 23836.37 | 41.36 | 46.21 | 45.89 | 0.9% | -0.01 |
| Class B RA | Kimono | 4791.76 | 41.60 | 43.46 | 45.26 | 4900.66 | 41.59 | 43.45 | 45.24 | 2.3% | -0.02 |
|  | ParkScene | 7659.24 | 40.07 | 42.38 | 43.78 | 7881.50 | 40.04 | 42.36 | 43.77 | 2.9% | -0.03 |
|  | Cactus | 18274.74 | 38.48 | 40.03 | 43.59 | 19104.61 | 38.46 | 40.02 | 43.56 | 4.5% | -0.02 |
|  | BasketballDrive | 17364.11 | 39.16 | 43.77 | 44.98 | 17687.30 | 39.14 | 43.75 | 44.96 | 1.9% | -0.01 |
|  | BQTerrace | 39673.09 | 37.42 | 42.10 | 44.28 | 40411.13 | 37.41 | 42.10 | 44.28 | 1.9% | -0.01 |
| Class B LDB | Kimono | 5207.92 | 41.68 | 43.25 | 44.75 | 5374.80 | 41.66 | 43.25 | 44.74 | 3.2% | -0.02 |
|  | ParkScene | 7948.53 | 39.90 | 41.92 | 43.07 | 8222.70 | 39.89 | 41.91 | 43.06 | 3.4% | -0.02 |
|  | Cactus | 19918.55 | 38.69 | 40.09 | 43.35 | 20919.17 | 38.68 | 40.08 | 43.33 | 5.0% | -0.01 |
|  | BasketballDrive | 19815.24 | 39.41 | 43.63 | 44.83 | 20339.15 | 39.40 | 43.61 | 44.81 | 2.6% | -0.01 |
|  | BQTerrace | 52660.01 | 38.23 | 41.94 | 44.08 | 53641.76 | 38.22 | 41.93 | 44.06 | 1.9% | -0.01 |

# Conclusion

This contribution proposes an SEI message that allows HEVC to support independently decodable regions using tiles. Simulation results were presented to show that the MCP restriction required by the proposed SEI message does not cause significant performance penalty or visible coding artefacts. It is therefore suggested to adopt this SEI message into HEVC WD.

# Patent rights declaration(s)

InterDigital Communications, LLC may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

eBrisk Video may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, High efficiency video coding (HEVC) text specification draft 9, Document no. JCTVC-K1003, October 2012.

A. Mavlankar, B. Girod, “Spatial-Random-Access-Enabled Video Coding for Interactive Virtual Pan/Tilt/Zoom Functionality,” IEEE Transactions on Circuits and Systems for Video Technology. vol. 21, no. 5, pp. 577-588, May 2011.