| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0259 |
| --- | --- |

| Title: | Encoder Optimization for SHVC: Enhancement Layer Lambda Refinement in RA Configurations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li Jianle Chen Marta Karczewicz | Tel: Email: | lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, an enhancement layer encoder optimization method is proposed for SHVC RA configurations. Lagrange multiplier in rate-distortion module of enhancement layer is refined based on temporal level, resolution ratio and QP gap between base and enhancement layer. It is reported that on average 0.8%, 1.1% and 1.2% BD-rate reduction was obtained for Y, U, and V components in both IBL and RefIdx frameworks.

# Introduction

Lagrange multiplier λ is a key parameter in rate-distortion module of video encoder such as HM and JM software. In the two reference software, lambda is determined by quantization step Q of the block, namely

|  | λ=c ⋅Q2 |  |
| --- | --- | --- |

In the reference software JSVM for H.264/AVC SVC extension, λ of enhancement layer can be determined by not only quantization step Q of the block, but also the spatial ratio β=(w1⋅h1)/(w0⋅h0) and QP gap ΔQP=QP0-QP1 between the two layers [1][2], i.e.,

|  | λ=c ⋅Q2⋅β⋅2ΔQP/6β⋅2ΔQP6+1 |  |
| --- | --- | --- |

where w0, h0, QP0, w1, h1, and QP1represent width, height, and quantization parameter of base and enhancement layer, respectively.

# Proposed method

When applying (2) to SHM-1.0, the bitrate of enhancement layer will be increased a lot due to a smaller λ. As a trade-off, a temporal level dependent λ refinement method is proposed for RA configurations (with GOP size no less than 8) as follows, where λL' and λC' represent the luma and chroma Lagrange multiplier after the refinement, λL denotes the luma Lagrange multiplier before the refinement, w is the weighting factor for chroma Lagrange multiplier derivation in SHM-1.0.

- For enhancement inter pictures at temporal level 0

|  | λL'=λL⋅1.1 |  |
| --- | --- | --- |
|  | λC'=λ'L/(1.15⋅w) |  |

- For enhancement pictures at highest temporal level (e.g. temporal level 3 when GOP size is 8)

|  | λL'=λL⋅β⋅2ΔQP/6β⋅2ΔQP6+1 |  |
| --- | --- | --- |
|  | λC'=λ'L/w |  |

- For other enhancement pictures,  λ derivation is unchanged.

As previously mentioned, when applying (5) and (6) to the highest temporal level, the bitrate of these frames are increased. To partially compensate the increase of bitrate, (3) and (4) are applied to temporal level 0 to suppress the bitrate a bit.

It is asserted that the proposed method is quite simple. The patch over SHM-1.0 is less than 80 lines.

# Simulation results

The proposed method is experimentally verified with RA configurations under the SHVC common test conditions defined in [3]. As shown in Table 1, compared to SHM-1.0 IBL anchor, 0.6%, 0.8%, and 1.1% luma BD-rate reduction was for RA-2x, RA-1.5x, and RA-SNR, respectively.

Table 2 shows that compared quite similar gain is obtained over SHM-1.0 RefIdx anchor.

Table 1 Compared to SHM-1.0 IBL anchor

Table 2 Compared to SHM-1.0 RefIdx anchor

# Conclusions

An enhancement layer encoder optimization method is proposed for SHM-1.0 RA configurations. The proposed method is simple yet efficient. It is proposed to adopt it into SHM software and enable it in SHVC common test conditions.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.

Reference

- X. Li, P. Amon, A. Hutter, A. Kaup, "Lagrange Multiplier Selection for Rate-Distortion Optimization in SVC", Joint Video Team of ISO/IEC MPEG & ITU-T VCEG, Doc. JVT-AD021, Geneva, Switzerland, Jan. 29 - Feb. 3, 2009.

- X. Li, P. Amon, A. Hutter, A. Kaup, "Lagrange Multiplier Selection for Rate-Distortion Optimization in SVC", Picture Coding Symposium (PCS 2009), Chicago, US, May 2009.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-L1009, Geneva, Switzerland, Jan. 2013.