| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O1121 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Rajan Joshi Karl Sharman | Email: | rajanj@qti.qualcomm.com karl.sharman@eu.sony.com |
| Source: | CE co-ordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 1 on high bit rate coding at high bit depths.

# Technical Description

This document defines HEVC Range Extensions Core Experiment 1 (RCE1) on high bit rate coding at high bit depths to be performed for the upcoming January 2014 JCT-VC meeting..

# List of Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| R. Joshi J. Sole | rajanj@qti.qualcomm.com joels@qti.qualcomm.com | Qualcomm | X | X |
| K. Sharman N. Saunders | Karl.sharman@eu.sony.com Nick.saunders@eu.sony.com | Sony Europe | X | X |
| C. Rosewarne | chris.rosewarne@cisra.canon.com.au | Canon |  |  |
| A. Saxena Guoxin Jin Felix Fernandes | asaxena@sta.samsung.com guoxin.jin@sta.samsung.com ffernandes@sta.samsung.com | Samsung Telecom America |  |  |
| T. Nguyen | tung.nguyen@hhi.fraunhofer.de | Fraunhofer HHI |  |  |
| Seung-Hwan Kim | kimse@sharplabs.com | Sharp |  |  |
| Shan Liu | Shan.liu@mediatek.com | Mediatek |  |  |
| Sunil Lee Chanyul Park | sunil.lee@samsung.com cy1205.park@samsung.com | Samsung |  |  |
| Akira Minezawa | Minezawa.Akira@ds.MitsubishiElectric.co.jp | Mitsubishi Electric Corporation |  |  |

# Tools to be tested

The CE is set up to explore combinations of alignment mechanisms with alignment conditions and change of the point in the entropy coder where the bypass bins are used.

Two alignment mechanisms for bypass bins will be tested:

Method 1 [M0178, N0190, O0046, O0207]: Alignment to 256. This can be expressed as:

ivlCurrRange = 256

Following this stage all subsequent EP bins can be interpreted as raw binary data.

Method 2 [M0178, N0190, O0046]: Alignment to 384. This can be expressed as:

if (ivlCurrRange >= 384)

if (ivlOffset < 256)	nextEPBitIsZero = true

else				ivlOffset -= 128

ivlCurrRange = 256

Following this stage all subsequent EP bins can be interpreted as raw binary data.

## Bypass of context coded bins for coefficient coding (subtest A)

In this subtest, various positions for switching to bypass in transform coefficient coding are explored.

Subtest A.1 [O0208]: Bypass after the significance map depending on the initial Rice parameter value for the current 4×4 subblock or the Rice parameter value at the end of the previous 4×4 subblock. Using the initial Rice parameter value as in RCE1 for the decision to bypass the flags will also be tested.

Subtest A.2 [O0209]: Bypass all the context-coded flags for the current 4×4 subblock (coded subblock flags, significance flags, greater than 1 flags and greater than 2 flags) depending on the initial Rice parameter value for the current 4×4 subblock or the Rice parameter value at the end of the previous 4×4 subblock. Using the initial Rice parameter value as in RCE1 for the decision to bypass the flags will also be tested.

Subtest A.3 [O0209 (ext.)]: Bypass all the context-coded flags in a transform block depending on the initial Rice parameter value at the beginning of the current transform block or the Rice parameter value at the end of the previous transform block.

Subtest A.4 [meeting discussion]:  All the bins (for coefficients as well as other syntax elements) are coded as raw. Residual data is modified according to subtest B.3. Additionally, the Golomb-Rice parameterization is changed.

## Combinations (subtest B)

Subtest B.1 [M0178]: Always apply method 1 before any offset bins in a 4×4 subblock.

Subtest B.2 [M0178]: Always apply method 2 before any offset bins in a 4×4 subblock.

Subtest B.3.a [O0046]: Method 1 after a greater than 2 flag has been coded for a 4×4 subblock.

Subtest B.3.b [O0207]: Method 1 after a greater than 2 flag has been coded for a 4×4 subblock.

Subtest B.5: Method 1 + A.1

Subtest B.6: Method 1 + A.2

Subtest B.7: Method 1 + A.3

# Crosschecks

# Test Conditions

RExt5.0 will be used for all experiments.

## AHG5 CTC

The AHG5 CTC will be used. These will be used to evaluate the loss for standard picture-quality operating points, which might be used for 8K at 240P.

[Should we also run some anchor half-width & half-height scaled simulations to estimate equivalent bit-rates for 8K at 240P?]

It is expected that AHG5 CTC will utilise the full set of RExt tools, rather than the restricted set used for the 4:2:2 and 4:2:0 profiles.

## AHG8 with lossless

The AHG8 lossless test conditions will be used to evaluate any potential issues at lossless operating points.

[Should we also run some anchor half-width & half-height scaled simulations to estimate equivalent bit-rates for 8K at 240P?]

## AHG18 test conditions on high bit depths

High bit-depth test conditions are defined as follows:

| Sequence name | Number of frames | Input bit depth | Additional MSBs |
| --- | --- | --- | --- |
| CrowdRun_1920x1080_50_16bit_444.rgb | 50 | 16 | 0 |
| ParkJoy_1920x1080_50_16bit_444.rgb | 50 | 16 | 0 |
| DucksTakeOff_1920x1080_50_16bit_444.rgb | 50 | 16 | 0 |
| InToTree_1920x1080_50_16bit_444.rgb | 50 | 16 | 0 |
| OldTownCross_1920x1080_50_16bit_444.rgb | 50 | 16 | 0 |
| Traffic_2560x1600_30_12bit_444_crop.rgb | 150 | 12 | 0,2,4 |
| EBUHorse_1920x1080_50_10bit_422.yuv | 100 | 10 | 2,4,6 |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 100 | 10 | 2,4,6 |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 100 | 10 | 2,4,6 |
| Kimono1_1920x1080_24_10bit_444.yuv | 240 | 10 | 2,4,6 |
| Others… |  |  |  |

Additional MSBs are used to simulate HDR-style material; an option will be provided in the RExt software to achieve this.

For each sequence, a number of internal bit-depths and QPs are to be tested:

| Internal bit depth | QPs – Medium Tier | QPs – High Tier |
| --- | --- | --- |
| 16 | -16, -19, -22, -25 | -28, -31, -34, -37 |
| 14 | -4, -7, -10, -13 | -16, -19, -22, -25 |
| 12 | 8, 5, 2, -1 | -4, -7, -10, -13 |

Extended precision processing will be enabled, and the encoder’s high precision forward transform will be used. The software settings required will be:

•	#define RExt__HIGH_BIT_DEPTH_SUPPORT  1

•	Apply '--ExtendedPrecision=1'

•	#define RExt__HIGH_PRECISION_FORWARD_TRANSFORM  1

RDOQ/RDOQTS will be disabled.

Results will show BD-rate changes over the 4 QPs and the compression ratio

Other settings as per the ‘encoder_intra_main_rextdev.cfg’ configuration file (i.e. ‘all-intra’ only).

# Evaluation of CE results

Results of the CE will be evaluated on the basis of BD-rate results, throughput and complexity. The complexity assessment will be performed on the basis of hardware and encoding/decoding complexity.

## Throughput

To measure the throughput the following bin counts will be provided:

- Total number of coded bins

- The number of context coded bins

- The number of bypass coded bins with CABAC

- The number of bypass coded bins without CABAC (raw bits)

For each TU size, the actual worst case bin counts with respect to items 2, 3, and 4 will also be provided. An analysis of the worst case complexity should be provided for each method for a coefficient and a 4×4 subblock.

## Complexity

To measure software run times, range extension software with the provided configurations and the proposals implemented on range extension software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- November 22, 2013: CE description finalized and uploaded.

- November x, 2013: Test sequences and test conditions are finalized.

- November 15, 2013 (tentative): Range extension reference software available.

- December 6, 2013: Cross-check begins. Proponents for subtest A provide software, draft specification text and results to CE participants.

- December 13, 2013: Proponents for subtest B provide software, draft specification text and results to CE participants.

- January 3, 2014: Input documents and summary report uploaded.

# References

[1] x