| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0283 |
| --- | --- |

| Title: | Adaptive MV precision for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this proposal, an adaptive MV precision method is proposed for screen content coding. A CU level flag is employed to indicate the MV precision of the CU. It is reported that 2%-4% luma BD rate reduction is achieved on average for screen content sequences. Coding gain is also observed for natural content sequences.

# Introduction

It is observed that sub-pixel MV precision is not always efficient for screen content and some range extension sequences (natural content). Therefore, adaptive MV precision is proposed in this contribution.

# Proposed method

It is proposed to signal a flag at CU level to indicate whether all PUs in the CU have integer-precision MVs. When the flag is on, MV predictors of PUs are rounded to integer precision and MV differrences are signaled in integer precision. When the flag is off, sub-pixel motion compensation is allowed.

The related syntax and semantics changes are as follows.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ){ |  |
| cu_imv_flag[ x0 ][ y0 ] | ae(v) |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| } |  |
| else { |  |
| … |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| cu_imv_flag[ x0 ][ y0 ] | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |

cu_imv_flag[ x0 ][ y0 ] equal to 1 specifies that motion vectors of all prediction units belonging to the current coding unit are in integer-pixel precision. cu_imv_flag[ x0 ][ y0 ] equal to 0 specifies that motion vectors of all prediction units belonging to the current coding unit are in quarter-pixel precision

When cu_imv_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

# Simulation results and discussions

The proposed method was implemented on HM12.1RExt5.1. The test sequences and conditions described in N14090 are used. The coding performance is summarized as follows.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 SC (Text and Graphics) | -2.4% | -2.4% | -2.4% | -3.1% | -3.1% | -3.1% |
| RGB 4:4:4 SC (Mixed Contents) | -1.6% | -1.9% | -1.9% | -2.1% | -2.0% | -2.0% |
| Animation RGB 4:4:4 | -1.3% | -1.7% | -1.9% | -2.1% | -2.5% | -2.4% |
| YUV 4:4:4 SC (Text and Graphics) | -2.2% | -2.3% | -2.2% | -2.5% | -2.5% | -2.4% |
| YUV 4:4:4 SC (Mixed Contents) | -1.1% | -1.6% | -1.5% | -1.4% | -1.9% | -1.9% |
| Animation YUV 4:4:4 | -1.0% | -1.8% | -1.8% | -1.3% | -2.0% | -2.0% |
| RangeExt | -0.7% | -0.5% | -0.8% | -0.3% | -0.2% | -0.6% |
| Enc Time[%] | 182% |  |  | 180% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 SC (Text and Graphics) | -3.4% | -3.3% | -3.4% | -4.2% | -4.2% | -4.1% |
| RGB 4:4:4 SC (Mixed Contents) | -2.3% | -2.4% | -2.4% | -2.8% | -2.6% | -2.6% |
| Animation RGB 4:4:4 | -2.5% | -2.9% | -3.0% | -3.9% | -3.9% | -3.8% |
| YUV 4:4:4 SC (Text and Graphics) | -2.8% | -2.9% | -2.9% | -3.2% | -3.2% | -3.2% |
| YUV 4:4:4 SC (Mixed Contents) | -1.1% | -1.8% | -1.7% | -1.4% | -2.0% | -1.9% |
| Animation YUV 4:4:4 | -0.7% | -2.4% | -1.9% | -1.3% | -2.7% | -2.3% |
| RangeExt | -0.5% | -0.4% | -0.5% | -0.3% | -0.2% | -0.6% |
| Enc Time[%] | 184% |  |  | 182% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

It can be noticed that significant luma BD reduction is achieved for screen content sequences and non-negligible gains for natural content.

# Conclusions

In this contribution, adaptive MV precision method is proposed. Simulations show that significant coding gain is achieved for screen content sequences. It is proposed to establish a CE to further study the proposed method.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).