INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11 m37543

February 2016, San Diego, USA

| Source | ARRIS Group Inc. |
| --- | --- |
| Purpose | Information and Discussion |
| Title | Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7) |
| Authors | Ajay Luthra, David Baylon, Koohyar Minoo, Yue Yu, Zhouye Gu |

# Abstract

This document summarizes the some of the observations made by us on the visual quality of the video when the Hybrid Log-Gamma (HLG) Transfer Function is used. The focus here is on the backward compatibility to SDR systems.

# 1   Introduction

The exploratory experiment 7 (CE7) was established at the 113th MPEG meeting (N15800). One of the purposes was to the do informal evaluation of the visual quality of the decoded HDR and SDR video.

Hybrid Log-Gamma (HLG) Transfer Function (TF) is designed to possibly provide backward compatibility with the “legacy” SDR systems. At this stage, in MPEG’s work, the “legacy” systems are taken to be the SDR systems not needing color gamut conversion (i.e., for example, not needing BT.2020 to BT.709 color gamut conversion) but needing only the dynamic range conversion. If the legacy SDR TV cannot process BT.2020 container then it is assumed the BT.2020 container is removed externally and the video is provided with appropriate format. Figure 1, taken from N15800 [1], shows at a high level the end-to-end chain associated with HLG based distribution.

There are two categories of experiments described in N15800 [1]. In the first one (category CE7.1) a fixed system gamma function is used, and in the second one (category CE7.2) the system gamma is varied. In this document we describe the observations associated with the visual quality of the video in CE7.1.

Figure 1. Generation of HLG TF based bit stream and video.

# 2   CE7.1 Backward Compatibility (BC) using fixed system gamma for the MPEG test sequences

Several bit streams were generated by BBC for Categories 7.1a and 7.1b (gamma = 1.6).  Test materials and coding conditions are described in N15800 [1]. We were able to match the md5sums provided by BBC for those bit streams. We were also able to generate video corresponding to the ShowGirl sequence. The decoded bitstreams were played back on a legacy SDR TV.   The legacy SDR TV used here does not understand a BT.2020 container format. Therefore, conversion from BT.2020 container to BT.709 was first performed externally on the decoded video before displaying it on the SDR TV via HDMI.  In the conversion, the decoded BT.2020 video was first converted to linear RGB assuming a source gamma of 2.2 (BT.1886), and then converted to BT.709. For reference, the original EXR sequences were also viewed on a SIM2 HDR display.

In the SDR backward compatibility experiment we provided the video to an SDR TV display as well as to a SIM2  HDR TV . We are finding that, in general, the video on the SDR TV looks washed out and there is significant change in the color hue in comparison to that in the original video. This is true even when the video is not compressed or lightly compressed. So, this does not seem to be associated with compression. Figures 2 - 3 show these distortions in two MPEG test sequences.  The pictures shown here were captured by a camera pointing to the legacy SDR TV that does not understand HDR and BT.2020. The corresponding original video is shown on the SIM2 HDR display in Figures 2(a) and 3(a), while Figures 2(b) and 3(b) show side-by-side portions of the 7.1a and 7.1b streams on the legacy SDR TV.

Although the shift in hue is noticed in a very wide range of colors, the most noticeable shift in hue in these sequences was observed in the colors that are in the region along the Red-Green curve of the CIE diagram. For example, in the Hurdles Test Sequence (Figure 2) the color of the ground looks quite different than the original orange color. In the GarageExit Test Sequence the orange in the conduit in the background looks significantly different than that in the original sequence (Figure 3). Also, the pictures on the SDR TV look “washed out”. The pictures generated by 7.1b looked significantly more “washed out” than those in 7.1a.

The amount and the direction of the shift in hue seems to be also a function of the brightness of the object. The part that is even more noticeable is that if an object moves from a less bright area to brighter area, the hue changes quite noticeably. We have an ARRIS internal video consisting of racing cars. The original color of the cars is orange where the RGB values are approximately in the ratio of 2:1:0.3. Two different original HDR frames of this sequence are shown in Figure 4a and 5a on a SIM2 HDR display. In Figure 4 the car in the front has bright light shining on it. In Figure 5 that car does not have as bright a light as in Figure 4. As the cars move from brighter (Figure 4) to less bright areas (Figure 5), the color shifts from yellow-orange (Figure 4b) to orange (Figure 5b) when the (uncompressed BT.709) HLG is viewed on the SDR TV. As this shift is observed within the same video sequence, it does not seem to be associated with any internal processing a TV might be doing.

(a)

(b)

Figure 2.  Hurdles frame 0 - (a) original HDR, (b) QP = 23, 7.1a (left) and 7.1b (right)

(a)

(b)

Figure 3.  GarageExit frame 209 - (a) original HDR, (b) QP = 24, 7.1a (left) and 7.1b (right)

(a)

(b)

Figure 4.  A scene in RaceCar with brighter light on the car - (a) original HDR,                     (b) original HLG

(a)

(b)

Figure 5.   A scene in RaceCar with less bright light on the car - (a) original HDR,                (b) original HLG

- References

- A. Luthra, E. Francois, L. van de Kerkhof, “Core Experiment 7 on  investigating how to generate compressed HDR/SDR video using HLG and HEVC”, ISO/IEC JTC1/SC29/WG11 N15800, October 2015, Geneva, Switzerland.