| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 31st Meeting: San Diego, US, 13–20 Apr. 2018 | Document: JCTVC-AE0027-v1 |
| --- | --- |

| Title: | Object tracking SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Palanivel Guruva reddiar | Email: | jill.boyce@intel.com palanivel.guruvareddiar@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

This contribution proposes to define an object tracking SEI message that carries parameters to indicate the position of tracked objects in a coded picture in the form of rectangular bounding boxes.  Attributes about the tracked objects can also be optionally signaled, including a label for the object and the confidence level associated with the tracking and detection.

The SEI message is also proposed for inclusion in AVC.

# Motivation

Object detection and tracking is common workload for end-to-end video systems, such as depicted in Figure 1, which includes smart cameras, video gateway, and datacenter/cloud.

For an intermediate processing stage, the following steps are typical:

- Decode compressed video

- Perform analytics on the video to identify object locations using a rectangular bounding box for each picture

- Composite video with a graphical outline of the bounding box(es)

- Re-encode the video containing bounding box(es)

Smart Cameras                     Video Gateway                   Datacenter / Cloud

Figure 1. End-to-End intelligent video solution

Figure 2. Video composited with graphical bounding box outline

Compositing the graphical bounding box outline onto the video has clear disadvantages. The unmodified video is no longer available. Refinement of the bounding box in later processing stages is not supported. Significant computation complexity is involved in the re-encoding process.

Using an SEI message to send the bounding box parameters as metadata addresses those disadvantages.

# Proposal

The proposed SEI message carries parameters to describe the bounding box of tracked objects within the compressed video bitstream. Optionally, object labels and confidence levels of detected and tracked objects may be provided. Object detection and tracking is computationally expensive, so it is advantageous in some systems to allow separation of objection detection/tracking functionality from video decoding.

The syntax uses persistence of parameters to avoid the need to re-signal information already available in previous SEI messages within the same persistence scope, e.g. within the same coded video sequence. For example, if one tracked object remains stationary for the current picture relative to a previously coded picture while another tracked object moves during that interval, the bounding box parameters are signaled only for the moving object.

A syntax flag is included to indicate if a coded video sequence is not intended for user viewing, but rather is intended for machine learning applications. For example, this flag could be used when the tracked objects are represented at high quality but areas outside of the tracked objects are represented at very low video quality.

Another syntax flag is included to indicate if the motion information (motion vectors, modes) were selected in order to accurately track objection motion, rather than to optimize coding efficiency, i.e. with RDO.

The number of bits of granularity of the confidence level is explicitly signaled, up to 16 bits.

## Object tracking SEI message syntax

| ###### object_tracking( payloadSize ) { | Descriptor |
| --- | --- |
| ###### object_tracking_seq_parameter_set_id | ue(v) |
| ###### object_tracking_not_intended_for_viewing_flag | u(1) |
| ###### object_tracking_true_motion_flag | u(1) |
| ###### object_tracking_item_label_present_flag | u(1) |
| ###### object_tracking_item_detection_confidence_info_present_flag | u(1) |
| ###### if (object_tracking_item_detection_confidence_info_present_flag ) |  |
| ###### object_tracking_item_detection_confidence_precision_num_bits | u(4) |
| ###### if ( object_tracking_item_label_ present_flag ) { |  |
| ###### object_tracking_label_language_present_flag | u(1) |
| ###### if ( object_tracking_label_language_ present_flag ) { |  |
| ###### while( !byte_aligned( ) ) |  |
| ###### object_tracking_zero_bit /* equal to 0 */ | f(1) |
| ###### object_tracking_label_language | st(v) |
| ###### } |  |
| ###### object_tracking_num_new_labels | ue(v) |
| ###### for( i = 0; i < object_tracking_num_new_labels; i++ ) |  |
| ###### object_tracking_label_idx[[ i ] | ue(v) |
| ###### for( i = 0; i < object_tracking_num_new_labels; i++ ) { |  |
| ###### while( !byte_aligned( ) ) |  |
| ###### object_tracking_zero_bit /* equal to 0 */ | f(1) |
| ###### object_tracking_label[[ object_tracking_label_idx[[ i ] ] | st(v) |
| ###### } |  |
| ###### } |  |
| ###### object_tracking_num_items_minus1 | ue(v) |
| ###### for( i = 0; i <= object_tracking_num_items_minus1; i++ ) { |  |
| ###### object_tracking_item_idx[[ i ] | ue(v) |
| ###### object_tracking_new_item_flag[[ object_tracking_item_idx[ i ] ] | u(1) |
| ###### if ( !object_tracking_new_item_flag[ object_tracking_item_idx[ i ] ] ) |  |
| ###### object_tracking_item_bounding_box_update_flag[ object_tracking_item_idx[ i ] ] | u(1) |
| ###### if( object_tracking_new_item_flag[ object_tracking_item_idx[ i ] && ###### object_tracking_item_label_ present_flag ) { |  |
| ###### object_tracking_item_label_idc[ object_tracking_item_idx[ i ] ] | ue(v) |
| ###### if( object_tracking_item_bounding_box_update_flag[ object_tracking_item_idx[ i ] ] ¦ ¦ ###### object_tracking_new_item_flag[ object_tracking_item_idx[ i ] ]) { |  |
| ###### object_tracking_item_top[ object_tracking_item_idx[ i ] ] | u(16) |
| ###### object_tracking_item_left[ object_tracking_item_idx[ i ] ] | u(16) |
| ###### object_tracking_item_width[ object_tracking_item_idx[ i ] ] | u(16) |
| ###### object_tracking_item_height[ object_tracking_item_idx[ i ] ] | u(16) |
| ###### if ( object_tracking_item_detection_confidence_info_present_flag ) |  |
| ###### object_tracking_item_detection_confidence[ object_tracking_item_idx[ i ] ] | u(v) |
| ###### } |  |
| ###### } |  |
| ###### } |  |

## Object tracking SEI message semantics

The object tracking SEI message specifies rectangular bounding boxes of one or more tracked objects which are visible within a decoded picture, and optional item labels and object detection confidence levels.

object_tracking_seq_parameter_set_id indicates and shall be equal to the sps_seq_parameter_set_id value of the active SPS. The value of object_tracking_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

object_tracking_not_intended_for_viewing_flag equal to 1 indicates that decoded picture is not intended for user viewing, but for other purposes. object_tracking_not_intended_for_viewing_flag equal to 0 indicates that the decoded picture is intended for user viewing.

object_tracking_true_motion_flag equal to 1 indicates that the motion information in the coded picture was selected with a goal of accurately representing object motion for tracked objects. object_tracking_true_motion_flag equal to 0 makes no indication about motion vector accuracy of tracked objects.

object_tracking_item_label_present_flag equal to 1 indicates that the label information corresponding to the bounding boxes is present in the coded bit stream. object_tracking_item_label_present_flag equal to 0 indicates that the label information corresponding to the bounding boxes is not present in the coded bit stream.

object_tracking_item_detection_confidence_info_present_flag equal to 1 indicates that object_tracking_item_detection_confidence[ ] is present in the bitstream. object_tracking_item_detection_confidence_info_present_flag equal to 0 indicates that object_tracking_item_detection_confidence[ ] is not present in the bitstream.

object_tracking_item_detection_confidence_precision_num_bits indicates the number of bits used to represent object_tracking_item_class_detection_confidence[ ].

object_tracking_label_language_present_flag equal to 1 indicates  that the object_tracking_label_language is present in the bit stream. object_tracking_label_language_present_flag equal to 0 indicates that the object_tracking_label_language is not present and that the language of the label is unspecified.

object_tracking_zero_bit shall be equal to zero.

object_tracking_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the object_tracking_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

object_tracking_num_new_labels indicates the total number of new labels associated with the bounding boxes that will be signaled.

object_tracking_label_idx[ i ] indicates the index to the label that shall be associated with the corresponding bounding box. The value of object_tracking_label_idx[ i ] shall be in the range of 0 to object_tracking_num_labels_minus1, inclusive.

object_tracking_label[ object_tracking_label_idx[ i ] ] contains a the label tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the object_tracking_label syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

object_tracking_num_items_minus1 plus 1 indicates the total number of objects being tracked which are visible in the current decoded picture.

object_tracking_item_idx[ i ]  specifies the index of the item present in the list of objects present in the current coded picture.

object_tracking_new_item_flag[ object_tracking_item_idx[ i ] ] equal to 1 indicates that the corresponding object was not represented in earlier object tracking SEI messages within the persistance scope. object_tracking_new_item_flag[ object_tracking_item_idx[ i ] ] equal to 0 indicates that the corresponding object was represented in earlier object tracking SEI messages within the persistance scope.

object_tracking_item_bounding_box_update_flag[ object_tracking_item_idx[ i ] ] equal to 1 indicates that the bounding box of the corresponding object has been changed from the values represented in earlier object tracking SEI messages within the persistance scope.  object_tracking_item_bounding_box_update[ object_tracking_item_idx[ i ] ]_flag equal to 0 indicates that the bounding box of the corresponding object persists from earlier object tracking SEI messages within the persistance scope.

object_tracking_item_label_idc[ object_tracking_item_idx[ i ] ] specifies the index of the label corresponding to the object. The value of object_tracking_item_label_idc[ object_tracking_item_idx[ i] ] persist from earlier object tracking SEI messages within the persistance scope. If object_tracking_item_label_idc[ object_tracking_item_idx[ i] ] was not present in earlier object tracking SEI messages within the persistance scope, their values are undefined.

object_tracking_item_top[ object_tracking_item_idx[ i ] ] and object_tracking_item_left[ object_tracking_item_idx[ i] ] specify, as luma samples, the top and left location, respectively, of the object_tracking_item_idx[ i ]–th object in the decoded picture. The values of object_tracking_item_left[ object_tracking_item_idx[ i ] ] shall be in the range of 0 to pic_width_in_luma_samples inclusive and the value of object_tracking_item_top[ object_tracking_item_idx[ i] ] will be in the range of 0 to pic_height_in_luma_samples inclusive. The values of object_tracking_item_top[ object_tracking_item_idx[ i ] ] and object_tracking_item_left[ object_tracking_item_idx[ i] ] persist from earlier object tracking SEI messages within the persistance scope. If object_tracking_item_top[ object_tracking_item_idx[ i ] ] or object_tracking_item_left[ object_tracking_item_idx[ i] ] were not present in earlier object tracking SEI messages within the persistance scope, their values are undefined.

object_tracking_item_width [ object_tracking_item_idx[ i ] ] and object_tracking_item_height[ object_tracking_item_idx[ i ] ] specify as luma samples the width and height, respectively, of the object_tracking_item_idx[ i ]–th object in the decoded picture. The value of  object_tracking_item_left[ object_tracking_item_idx[ i ] ]+object_tracking_item_width[ object_tracking_item_idx[ i ] ] shall be in the range of 0 to pic_width_in_luma_samples inclusive and the value of  object_tracking_item_top[ object_tracking_item_idx[ i ] ]+object_tracking_item_height [ object_tracking_item_idx[ i ] ] shall be in the range of 0 to pic_height_in_luma_samples inclusive.   The values of object_tracking_item_width[ object_tracking_item_idx[ i ] ] and object_tracking_item_height[ object_tracking_item_idx[ i ] ] persist from earlier object tracking SEI messages within the persistance scope. If object_tracking_item_width[ object_tracking_item_idx[ i ] ] or object_tracking_item_height[ object_tracking_item_idx[ i ] ] was not present in earlier object tracking SEI messages within the persistance scope, their values are undefined.

object_tracking_item_class_detection_confidence[ object_tracking_item_idx[ i ] ] specifies the confidence associated with the object_tracking_item_idx[ i ]-th item, in units of                                                 2-object_tracking_item_detection_confidence_precision_num_bits. The length of the object_tracking_item_class_detection_confidence[ object_tracking_item_idx[ i ] ] syntax element is object_tracking_item_detection_confidence_precision_num_bits bits. The value of object_tracking_item_class_detection_confidence[ object_tracking_item_idx[ i ] ] persists from earlier object tracking SEI messages within the persistance scope. If object_tracking_item_class_detection_confidence[ object_tracking_item_idx[ i ] ] was not present in earlier object tracking SEI messages within the persistance scope, its value is undefined.

# Example Scenarios

## Picture 0

At picture 0, two objects are present in the image, a car and a person.

Pic 0 key syntax:

| ###### object_tracking_item_label_present_flag | 1 |
| --- | --- |
| ###### object_tracking_num_new_labels | 2 |
| ###### object_tracking_label_idx[ 0 ] | 0 |
| ###### object_tracking_label_idx[ 1 ] | 1 |
| ###### object_tracking_label[ 0 ] | car |
| ###### object_tracking_label[ 1 ] | person |
| ###### object_tracking_num_items_minus1 | 1 |
| ###### object_tracking_item_idx[ 0 ] | 0 |
| ###### object_tracking_new_item_flag[ 0 ] | 1 |
| ###### object_tracking_item_idx[ 1 ] | 1 |
| ###### object_tracking_new_item_flag[ 1 ] | 1 |
| ###### object_tracking_item_label_idc[ 0 ] | 0 |
| ###### object_tracking_item_label_idc[ 1 ] | 1 |
| ###### object_tracking_item_top, left, width, height[ 0 ] | BB_A |
| ###### object_tracking_item_top, left, width, height[ 1 ] | BB_B |

## Picture 1

At picture 1, the car (item 0) stayed in the same position and the person (item 1) moved to a new position

Pic 1 key syntax:

| ###### object_tracking_item_label_present_flag | 1 |
| --- | --- |
| ###### object_tracking_num_new_labels | 0 |
| ###### object_tracking_num_items_minus1 | 1 |
| ###### object_tracking_item_idx[ 0 ] | 0 |
| ###### object_tracking_new_item_flag[ 0 ] | 0 |
| ###### object_tracking_item_bounding_box_update_flag[ 0 ] | 0 |
| ###### object_tracking_item_idx[ 1 ] | 1 |
| ###### object_tracking_new_item_flag[ 1 ] | 0 |
| ###### object_tracking_item_bounding_box_update_flag[ 1 ] | 1 |
| ###### object_tracking_item_top, left, width, height[ 1 ] | BB_C |

The position of the car (item 0) persists from Pic 0, as BB_A.

## Picture 2

At picture 2, the first car (item 0) is no longer in the picture, the person (item 1) moved within the picture, a different car (item 2) entered the picture, and a dog (item 3) entered the picture.

Pic 2 key syntax:

| ###### object_tracking_item_label_present_flag | 1 |
| --- | --- |
| ###### object_tracking_num_new_labels | 1 |
| ###### object_tracking_label_idx[ 0 ] | 2 |
| ###### object_tracking_label[ 2 ] | dog |
| ###### object_tracking_num_items_minus1 | 2 |
| ###### object_tracking_item_idx[ 0 ] | 1 |
| ###### object_tracking_new_item_flag[ 1 ] | 0 |
| ###### object_tracking_item_bounding_box_update_flag[ 1 ] | 1 |
| ###### object_tracking_item_idx[ 1 ] | 2 |
| ###### object_tracking_new_item_flag[ 2 ] | 1 |
| ###### object_tracking_item_label_idc[ 2 ] | 0 |
| ###### object_tracking_item_idx[ 2 ] | 3 |
| ###### object_tracking_new_item_flag[ 3 ] | 1 |
| ###### object_tracking_item_label_idc[ 3 ] | 2 |
| ###### object_tracking_item_top, left, width, height[ 1 ] | BB_D |
| ###### object_tracking_item_top, left, width, height[ 2 ] | BB_E |
| ###### object_tracking_item_top, left, width, height[ 3 ] | BB_F |

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).