| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A032 |
| --- | --- |

| Title: | Novel entropy coding concept |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Detlev Marpe, Heiko Schwarz, Thomas Wiegand |  |  |
| Contact: | Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49-30-31002-617 wiegand@hhi.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution describes a novel approach to entropy coding. The basic idea of the presented entropy coding concept is the usage of multiple parallel binary encoders/decoders that operate at fixed probabilities. The entropy coding concept supports a high degree of decoder parallel processing and can be configured in a way that it operates at a complexity level of variable-length coding without any loss in coding efficiency relative to the use of arithmetic codes.

# Overview

We present a novel approach for entropy coding, which consists of a binarization, an adaptive context modeling, a probability quantization, and multiple binary entropy coders with fixed probabilities. The concepts for binarization and context modeling are basically the same as in the CABAC framework of H.264/AVC. However, the actual coding of the bins is based on a novel concept that has been introduced in order to support parallelized implementations of entropy encoding and decoding as well as for decreasing the computational complexity of the entropy decoding.

## Novel entropy coding concept

Fig. 1:  Illustration of the novel entropy coding concept

In Fig. 1, the basic entropy coding concept is illustrated. If a syntax element does not already represent a binary syntax element, it is first binarizied, i.e., it is mapped onto a sequence of binary decisions (bins). For each bin a context is selected. A context represents a (binary) probability model for a class of bins; it is characterized by the probability and the value of the less significant bin (LPB). In our implementation, the LPB probability is represented by one out of 64 states (as in the CABAC framework of H.264/AVC). At the beginning of the encoding/decoding of a slice, the probability models are initialized using fixed values (as in CABAC of H.264/AVC). And after encoding/decoding a bin with a particular model, the model (i.e., the corresponding probability and value of the LPB) is updated. The probability model update, i.e., the probability estimation process is the same as in CABAC of H.264/AVC. The association of a bin with a context model is also similar as in CABAC of H.264/AVC. It depends on the syntax element, the bin number, and, for some bins, the values of neighboring syntax elements.

The above described binarization and association of the bins with context models is basically the same as in CABAC of H.264/AVC. The main difference is the next step. Instead of arithmetically coding the bins with the associated probability models using a single arithmetic coding engine as in CABAC of H.264/AVC, the estimated LPB probabilities are quantized, i.e., they are mapped onto a small number of LPB probability intervals and for each of these probability intervals a separate bin encoder/decoder is operated. In our implementation, we used 12 probability intervals and thus 12 different bin encoders/decoders. Each bin encoder/decoder operates at a fixed LPB probability, which can be considered as the representative probability for an LPB interval. The selection of a bin encoder/decoder is implemented via a look-up table that associates a bin encoder with each of the 64 state indices for the LPB probability. Hence, the 64 states that are used for estimating the LPB probability of a context model are mapped onto 12 probability classes, for each of which a separate bin encoder/decoder is operated. The partial bitstreams that are generated by the bin encoders can be written to different partitions of a NAL unit (with the partitioning of the NAL unit being indicated in the slice header) or they can be interleaved into a single bitstream.

For the bin encoders and decoders two alternatives have been implemented. In a first version, the bin encoders and decoders represent arithmetic encoding and decoder engines, respectively, which are similar to the arithmetic coding engines used in CABAC of H.264/AVC. In a second version, the bin encoders and decoders represent simple variable length codes, by which a variable number of bins is mapped onto variable length codewords and vice versa. Both versions have a very similar coding efficiency.

### Bin encoders using arithmetic coding

As mentioned above, arithmetic coding engines (similar to the coding engines that are used for CABAC in H.264/AVC) can be used for the separate bin encoder. The corresponding 12 arithmetic codewords are written to different partitions of the slice data NAL units; the corresponding partitioning information is transmitted in the slice header. An obvious advantage of this approach is that the arithmetic decoding can be parallelized. All 12 arithmetic decoding engines can be operated in parallel and write the sequences of decoded bins into particular bin buffers. The remaining entropy decoding process can then simply read the bins from the corresponding 12 bin buffers without the need to wait until a bin is arithmetically decoded before proceeding with the next bin. This is in particular a very useful feature for slice data NAL units that contain a large number of bits.

For small slice data NAL units, a parallelized decoding is usually not required. And since the partitioning information can represent a significant amount of the overall number of bits of a NAL unit, we adaptively switch between the presented coding with 12 arithmetic coding engines (and codewords) and the coding with a single arithmetic coding engines with variable LPB probabilities as it is found in CABAC of H.264/AVC. It is signaled in the slice header which of these variants (single-codeword arithmetic coding or multi-codeword arithmetic coding) is used.

### Bin encoders using variable length codes

Instead of using arithmetic coding for the bin encoders (cp. Fig. 1), it is also possible to employ simple variable length codes. It should be again noted that each bin encoder operates at a fixed LPB probability. And for a fixed LPB probability, we can simple construct a code that maps a variable number of bins onto a variable length codeword and vice versa. As a particular example, the following table shows an example for such a mapping. In the example the bins "0" represent the more probable bin values and the bins "1" represent the less probable bin values. The example code has been designed for a LPB probability of 0.15 and it yields a redundancy of only 0.25 % relative to the entropy limit for this probability. Generally, it is possible to get closer to the entropy limit when the table size is increased.

Table 1: Example for a mapping between a variable number of bins and variable length codewords

| sequence of bins (bin order is from left to right) | codewords (bits order is from left to right) |
| --- | --- |
| 0000 | 1 |
| 01 | 001 |
| 10 | 010 |
| 001 | 011 |
| 0001 00 | 0001 |
| 11 | 0000 1 |
| 0001 1 | 0000 00 |
| 0001 01 | 0000 01 |

At the end of a slice, it is possible that a bin sequence does not represent any bin sequence that is associated with a codeword. In order to terminate the codeword, the shortest codeword for a bin sequence that contains the current bin sequence as prefix is written. At the decoder side, the additionally decoded bins are discarded. As an example, we could use the mapping in the above table and have a bin sequence "00" at the end of a slice. Then we would write the codeword "1" (corresponding to the bin sequence "0000"). At the decoder side, the codeword "1" would be translated into the bin sequence "0000", but the decoder would discard (ignore) the last two decoded bins.

Similarly to the above presented concept with arithmetic coding engines, the partial bitstreams can be written to different partitions of the slice data NAL unit (with transmitting the partitioning information in the slice header). It is, however, also possible to interleave the codewords into one single bitstream with any overhead. This is in particular beneficial for small slice data NAL units, for which parallelized bin decoding is not required. In order to realize such an interleaving a codeword buffer for variable-length codewords can be introduced at the encoder side. When a particular bin encoder starts the encoding of a new bin sequence (i.e., the bin encoder receives the first bin after the previous codeword has been written), it reserves the next variable-length codeword entry in the codeword buffer. And when the bin sequence is finished (i.e., when the received bins represent a bin sequence that corresponds to a codeword), the codeword is written to the previously reserved codeword entry. Examples for the codeword buffer status are illustrated in Fig. 2. At the decoder side, the codewords can be directly read from the bitstream in decoding order.

Fig. 2:  Examples for the codeword buffer status

It is also possible to combine the codeword interleaving with a low-delay buffer control. That means, we can suitably restrict the buffer delay that is introduced by the codeword interleaving. With such a low-delay control, a particular codeword is terminated when the maximum number of bits (this maximum number of bits represents the maximum codeword length for a particular bin encoder and is given by the corresponding mapping table) that are associated with the reserved or completed buffer entries following the buffer entry for the particular codeword is greater than a particular threshold. At the decoder side, the extra decoded bins must be discarded; this can be done by using the same measure of the maximum number of bits as at the encoder side.

# Theoretical analysis of the entropy coding concept

In the following section, a theoretical analysis of the entropy coding concept is present.

## Event Space Projection

Consider the sequence of coding symbols s0,…,sN−1. Each symbol is drawn from an alphabet si∈Ai. The alphabets Ai=a0i,a1i,… contain two or more letters each being associated with a probability estimate ps(ami). The probability estimates ps(ami) are known to encoder and decoder and may be fixed or variable. It is assumed that variable probabilities are simultaneously estimated at encoder and decoder. The alphabets Ai may either be identical for the sequence of symbols or different symbol types are associated with different alphabets. In the latter case, it is assumed that the decoder knows the alphabet of each symbol in the sequence. This assumption is justified as practical source codec descriptions contain a syntax that stipulates the order of symbols and their alphabets.

The sequence of symbols s0,…,sN−1 is converted into a sequence of binary symbols, which are also referred to as bins. For each symbol si, the binarization

bi=b0i…=γbi(si)

represents a bijective mapping of the alphabet letters ami onto ordered sets of bins bmi. The binarization mapping γbi can be different for different symbols si or symbol categories. Each bin sequence bi for a particular symbol si consists of one or more bins bki. At the decoder side, a symbol si can be reconstructed by the inverse mapping si=(γbi)−1(bi) given the sequence of bins bi. As a result of the binarization, a sequence of bins b0,…,bB−1 is obtained that represents the sequence of source symbols s0,…,sN−1.

All bins bj are associated with the same binary alphabet B=0,1, but the corresponding binary pmfs p0j,p1j, with p1j=p0j, are usually different. A binary pmf p0j,p1j can be described by the less probable bin (LPB) value b LPBj and its probability p LPBj (with p LPBj≤0.5). This binary probability description b LPBj,p LPBj can be directly derived from the probability estimates ps(ami) for the symbol alphabets given the binarization mappings γbi. It is also possible (and often preferable) to directly estimate b LPBj,p LPBj simultaneously at encoder and decoder side. Therefore, the bins can be associated with a probability model (which is also referred to as context) based on the syntax and previously coded symbols or bins. And for each probability model, the probability description b LPBj,p LPBj can be estimated based on the values of the bins that are coded with the probability model. An example for such an binary probability modeling is probability modeling of CABAC in H.264/AVC.

Since the binary entropy function

H(p)=−p log2(p)−(1−p)log2(1−p)

is symmetric around p=0.5, the same binary coder can be used for coding all bins that are associated with the same LPB probability p LPBj, independent of the value of b LPBj. Therefore, the sequence of bins b0,…,bB−1 is converted into a sequence of coding bins b0c,…,bB−1c. For each bin bj, the corresponding bijective mapping γcj is specified by

bjc=γcj(bj)=bj⊕b LPBj

where ⊕ denotes the exclusive or operator. At the decoder side, the bins bj can be reconstructed given the coding bins bjc and the corresponding LPB value b LPBj by the inverse mapping bj=(γcj)−1(bjc)=bjc⊕b LPBj. A coding bin bjc=0 specifies that the value of corresponding bin bj is equal to the LPB value b LPBj and a coding bin bjc=1 specifies that the value of the corresponding bin bj is equal to the more probable bin (MPB) value 1−b LPBj.

The sequence of coding bins b0c,…,bB−1c does uniquely represent the sequence of source symbols s0,…,sN−1 and the corresponding probability estimates, which can be employed for entropy coding, are completely described by the LPB probabilities p LPBj (with p LPBj≤0.5). Hence, only probabilities in the half-open interval (0,0.5] need to be considered for designing the binary entropy coder for the coding bins bic.

For the actual binary entropy coding, the sequence of coding bins b0c,…,bB−1c is projected onto a small number of probability intervals Ik. The LPB probability interval (0,0.5] is partitioned into K intervals Ik=(pk,pk+1]

K−1Ik=(0,0.5] and IkIj=∅fork≠j

The set of K intervals is characterized by K−1 intervals borders pk with k=1,…,K−1. Without loss of generality we assume pk<pk+1 for k=0,…,K. The outer interval borders are fixed and given by p0=0 and pK=0.5. A simple non-adaptive binary entropy coder is designed for each interval Ik. All coding bin bjc with associated LPB probabilities p LPBj∈Ik are assigned to the interval Ik and are coded with the corresponding fixed entropy coder.

In the following description, all bins represent coding bins bjc and all probabilities p are LPB probabilities p LPBj.

## Probability Interval Discretization

For investigating the impact of the probability interval discretization on the coding efficiency, we assume that we can design an optimal entropy coder for a fixed probability that achieves the entropy bound. Each probability interval Ik=(pk,pk+1] is associated with a representative probability pIk∈Ik and the corresponding optimal entropy coder shall achieve the entropy limit for this representative probability. Under these assumption, the rate for coding a bin with probability p using the optimal entropy coder for the interval representative pIk is given by

R(p,pIk)= −p log2(pIk)−(1−p)log2(1−pIk) H(pIk)+(p−pIk) H'(pIk)

where H(p) represents the binary entropy function  and

H'(p)=log21−pp

is its first derivative. We further assume that the distribution of the probabilities in the interval (0,0.5] is given by f(p), with 0.5f(p) dp=1. Then, the expected rate, in bits per bin, for a given set of K intervals Ik with corresponding representative probabilities pIk can be written as

pk+1R(p,pIk) f(p) dp

The first partial derivative with respect to any representative probability pIk, with k=0,…,K−1, is given by

pk+1p f(p) dppIk (1−pIk) ln2

The equation ∂∂pIkR=0 has a single solution

pk+1f(p) dp

for the representative probability pIk inside the domain of definition Ik. The second partial derivative for this solution

pk+1f(p) dppIk (1−pIk) ln2

is always greater than zero if

pk+1f(p) dp>0

Hence, if condition  is fulfilled, the value pIk given in eq.  is the representative probability for an interval Ik that minimizes the expected overall rate R given the interval boundaries pk and pk+1. Otherwise, no bin is projected to the interval Ik and the representative probability pIk∈Ik can be arbitrarily chosen without any impact on the overall rate R; but such a configuration should be avoided, since the interval Ik would not be employed for entropy coding.

For finding a condition for optimal interval borders, we investigate the first derivatives of the expected overall rate R with respect to the interval borders pk with k=1,…,K−1. If f(p)>0 for all p∈[pIk−1,pIk), the equation ∂∂pkR=0 has a single solution

pk=H(pIk)−pIkH'(pIk)−H(pIk−1)+pIk−1H'(pIk−1)H'(pIk−1)−H'(pIk)

for the interval border pk inside the domain of definition [pIk−1,pIk) and the second partial derivative for this solution

∂2∂pk2R(pk)=f(pk)H'(pIk−1)−H'(pIk)

is always greater than zero, so that pk is the interval border pk∈[pIk−1,pIk) that minimizes the expected overall rate R given the interval representatives pIk−1 and pIk. If there exist probabilities p∈[pIk−1,pIk) with f(p)=0, the equation ∂∂pkR=0 has multiple solutions, but pk as given in eq.  is still optimal even though further optimal solutions may exist.

Given the number of intervals K and the probability distribution f(p), the interval borders pk, with k=1,…,K−1, and the interval representatives pIk, with k=0,…,K−1, that minimize the expected overall rate R can be obtained by solving the equation system given by eqs.  and  subject to the conditions  for k=0,…,K−1. This can be achieved with the following iterative algorithm.

Algorithm 1:

- Partition the interval (0,0.5] into K arbitrary intervals Ik=(pk,pk+1] with p0=0, pK=0.5, and pk<pk+1 for all k=0,…,K−1 in a way that the conditions  are obeyed for all k=0,…,K−1.

- Update the representatives pIk with k=0,…,K−1 according to eq.

- Update the interval borders pk with k=1,…,K−1 according to eq.

- Repeat the previous two steps until convergence

Fig. 3 shows an example for the optimal interval discretization using the described algorithm. For this example, we assumed a uniform probability distribution f(p)=2 for 0<p≤0.5 and partitioned the probability interval (0,0.5] into K=4 intervals. It can be seen that the probability interval discretization leads to a piecewise linear approximation A(p) of the binary entropy function H(p) with A(p)≥H(p) for all p∈(0,0.5].

Fig. 3:  Optimal probability interval discretization into K=4 intervals
 assuming a uniform probability distribution in (0,0.5]

As measure for the impact of the interval discretization on the coding efficiency the expected overall rate increase relative to the entropy limit

0.5H(p) f(p) dp−1

can be used. For the particular example of Fig. 3, the expectation value of the entropy 0.5H(p) f(p) dp is equal to 1/(2ln2) bit per bin and the rate overhead ρ is equal to 1.01 %.  Table 2 lists rate overheads ρ uni and ρ lin for the uniform probability distribution and a linear increasing probability distribution f(p)=8p with p∈(0,0.5], respectively, for selected numbers of intervals K.

Table 2: Rate overhead vs. the number of probability intervals for the uniform and a linear increasing probability distribution

| K | 1 | 2 | 4 | 8 | 12 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| ρ uni [%] | 12.47 | 3.67 | 1.01 | 0.27 | 0.12 | 0.07 |
| ρ lin [%] | 5.68 | 1.77 | 0.50 | 0.14 | 0.06 | 0.04 |

The investigations in this section showed that the discretization of the LPB probability interval (0,0.5] into a small number of intervals with a fixed probability (e.g., 8 to 10 intervals) has a very small impact on the coding efficiency.

## Entropy Coding for Probability Intervals

In the following, we first show how an simple code can be designed for fixed probabilities. Given these results, we develop an algorithms that jointly optimizes the code design and the partitioning of the LPB probability interval (0,0.5].

### Entropy coding with fixed probabilities

The entropy coding for fixed probabilities p=pIk can be done using arithmetic coding or variable length coding. For the latter case, the following approach appears to be simple and very efficient.

Fig. 4:  Tree of binary events for an LPB probability of p=0.38 and an associated variable length code obtained by the Huffman algorithm

We consider a binary entropy coding scheme by which a variable number of bins is mapped onto variable length codewords. For unique decodability, the inverse mapping of a codeword to a bin sequence must be unique. And since we want to design a code that approaches the entropy limit as close as possible, we constrain our considerations to bijective mappings. Such a bijective mapping can be represented by a binary tree where all leaf nodes are associated with codewords, as depicted in Fig. 4. The tree edges represent binary events. In the example of Fig. 4, the lower edges represent the LPB bin value and the upper edges represent the MPB bin value. The binary tree represents a prefix code for the bins if it is a full binary tree, i.e., if every node is either a leaf or has two descendants. Each leaf node is associated with a probability based on the given LPB probability p. The root node has the probability proot=1. The probability for all other nodes is obtained by multiplying the probability of the corresponding ancestor with p for the LPB descendants and q=1−p for the MPB descendants. Each leaf node Ll is characterized by the number of LPB edges al and the number MPB edges bl from the root node to the leaf node. For a particular LPB probability p, the probability pl for a leaf node Ll=al,bl is equal to

pl=pal (1−p)bl

The binary tree T is fully characterized by the number of leaf nodes L and the associated pairs al,bl with l=0,…,L−1.

Given a full binary tree T and a LPB probability p, the optimal assignment of codewords to the leaf nodes can be obtained by the Huffmann algorithm. The resulting variable number of bits to variable length codewords (VNB2VLC) mapping C is characterized by the number of codewords L, which is identical to the number of leaf nodes, and the tuples al,bl,ll for l=0,…,L−1, where ll represents the codeword length that is associated with the corresponding leaf node Ll=al,bl. It should be noted that there are multiple possibilities for the codeword assignment given the codeword lengths ll and the actual codeword assignment is not important as long as the codewords represent a uniquely decodable prefix code. The expected rate R(p,C) in bits per bin for a given code C and an LPB probability p is the ratio of the expected codeword length and the expected number of bins per codeword

L−1pal(1−p)bl(al+bl)

The code design is often limited by factors as the maximum number of codewords L, the maximum number of bins per codeword, or the maximum codeword length, or it is restricted to codes of particular structures (e.g., for allowing optimized parsing). If we assume that the set SC of usuable codes for a particular application is given, the optimum code C∈SC for a particular LPB probability p can be found by minimizing the expected rate R(p,C)

C(p)=argmin∀C∈SCR(p,C)

As faster alternative, the minimization can also proceed over a given set of binary trees ST and for each tree only one VNB2VLC code C that is obtained by the Huffmann algorithm is considered. As an example, we designed VNB2VLC codes for various LPB probabilities p by considering all binary trees T for which the number of leaf nodes L is less than or equal to a given maximum Lm. In Fig. 5, the relative rate increase ρ(p,C(p))=R(p,C(p))H(p) is plotted over the LPB probability p for selected maximum table sizes Lm. The rate increase ρ(p) can usually be reduced by allowing larger table sizes. For larger LPB probabilities, a small table size L of 8 to 16 codewords is usually sufficient for keeping the rate increase ρ(p) reasonably small, but for smaller LPB probabilities (e.g., p<0.1), larger table sizes L are required.

Fig. 5:  Relative bit rate increase ρ(p,C) for optimal codes C given a maximum number of table entries Lm

### Combined code design and interval partitioning

In the previous sections, we considered the optimal probability discretization assuming optimal codes and the code design for fixed LPB probabilities. But since, in general, we cannot achieve the entropy limit with real VNB2VLC codes of limited table sizes, the code design and the partitioning of the LPB probability interval (0,0.5] must be jointly considered for obtaining an optimized entropy coding design.

For a given interval Ik=(pk,pk+1], a code Ck of a given set SC in an optimal code Ck if it minimizes the expected rate pk+1R(p,Ck) f(p) dp for the given interval.

pk+1R(p,Ck) f(p) dp

For practical designs, the minimization of the integral in eq.  can be simplified, with a minor impact on the coding efficiency, by first determining an optimal representative probability pIk for the interval Ik according to eq.  and then choosing the optimal code Ck of the given set SC for the representive probability pIk according to eq. .

Optimal interval borders pk, with k=1,…,K−1, given the set of codes Ck, with k=0,…,K−1, can be derived by minimizing the expected overall rate

pk+1R(p,Ck) f(p) dp

Setting the first derivatives with respect to the interval borders equal to zero, ∂∂pkR=0, for k=1,…,K−1, yields

pk=pk with R(pk,Ck−1)=R(pk,Ck)

Similarly as for eq. , it can be shown that pk is always an optimal solution, but depending on the probability distribution f(p) further optimal solutions might exist. Hence, an optimal interval border pk between two intervals Ik−1 and Ik with given associated codes Ck−1 and Ck, respectively, is the intersection point of the functions R(p,Ck−1) and R(p,Ck).

Consequently, the following iterative algorithm can be used for jointly deriving the probability interval partitioning and the associated codes given the number K of probability intervals, the set of possible codes SC, and the probability distribution f(p), with p∈(0,0.5].

Algorithm 2:

- Derive initial probability interval boundaries pk, with k=0,…,K, using algorithm 1 specified in sec. 2.2

- Derive representatives pIk for the probability intervals Ik, with k=0,…,K−1, according to eq.

- Derive codes Ck∈SC for the interval representatives pIk, with k=0,…,K−1, according to eq.

- Update the interval borders pk, with k=1,…,K−1, according to eq.

- Repeat the previous three steps until convergence

The steps 2 and 3 in algorithm 2 could also be replaced by a direct derivation of the codes Ck∈SC, with k=0,…,K−1, based on the interval borders pk, with k=0,…,K, according to eq. . And, as mentioned in sec. 2.3.1, the minimization in step 3 can also proceed over a given set of binary trees ST where for each binary tree T only one VNB2VLC code Ck obtained by the Huffmann algorithm is considered.

As an example, we jointly derived the partitioning into K=12 probability intervals and corresponding VNB2VLC codes using algorithm 2. At this, the minimization in step 3 of the algorithm was replaced with an equivalent minimization over a given set of binary trees ST where the evaluated code C for each tree T was obtained by the Huffmann algorithm. We considered trees T with a maximum number of Lm=65 leaf nodes and hence codes C with up to 65 table entries. All binary trees T with up to 16 leaf nodes have been evaluated in the minimization; for trees with more than 16 leaf nodes, we employed a suboptimal search given the best results for trees with a smaller number of leaf nodes.

Fig. 6:  Rate increase for the theoretically optimal probability interval partitioning into K=12 intervals 
(cp. sec. 2.2) and a real design with VNB2VLC codes with a maximum number of Lm=65 table entries

In Fig. 6, the expected rate increase relative to the entropy limit ΔR(p)=R(p)−H(p) for the code design example is plotted over the LPB probability p. As comparison, we also plotted the expected rate increase ΔR for the theoretically optimal probability interval discretization (as developed in sec. 2.2) and the theoretically optimal probability discretization with the additional constraint pIK−1=0.5 inside the diagram. It can be seen that the joint probability interval discretization and VNB2VLC code design leads to a shifting of the interval borders (the interval borders pk, with k=1,…,K−1, are given by the local maxima of the ΔR(p) curves). The relative expected overall rate increase relative to the entropy limit for the design example with real VNB2VLC codes is ρ=0.24%, when assuming a uniform probability distribution f(p). The corresponding relative rate increases for the theoretically optimal probability interval discretization and the theoretically optimal probability discretization with the additional constraint pIK−1=0.5 are ρ=0.12% and ρ=0.13%, respectively.

### Codeword termination

When coding a finite sequence of symbols s0,…,sN−1, each of the K binary encoders processes a finite sequence of coding bins bkc=b0c,…,bBk−1ck, with k=0,…,K−1. And it has been to be ensured that, for each of the K binary encoders, all coding bins of the sequence bkc=b0c,…,bBk−1ck can be reconstructed given the codeword or sequence of codewords ck(kc).

When employing arithmetic coding, the arithmetic codeword for the sequence of coding bins has to be terminated in a way, that all coding bins can be decoded given the codeword. For the VNB2VLC codes described above, the bins at the end of the sequence bkc may not represent a bin sequence that is associated with a codeword. In such a case, any codeword that contains the remaining bin sequence as prefix can be written. The overhead can be minimized, if the corresponding codeword that has the minimum length (or one of these codewords) is chosen. At the decoder side, the additionally read bins at the end of the bin sequence, which can be identified given the bitstream syntax and binarization schemes, are discarded.

### Simple code design example

For illustration purposes, we consider the simple example of a source s with three letters and fixed associated probabilities of ps(a0)=0.7, ps(a1)=0.18, and ps(a2)=0.12. The corresponding ternary choice tree can be converted into a full binary tree as shown in Fig. 7.

Fig. 7:  Example for conversion of a ternary choice tree into a full binary choice tree

A binarization for the full binary tree in Fig. 7 is given in Table 2. The ternary symbol pmf ps is converted into two binary pmfs pb0=(0.7,0.3) and pb1=(0.6,0.4). For each symbol s in the bit stream, the bin b0 is present. When b0 is equal to 0, also b1 is present. Note that the binarization given in Table 2 is identical to an optimal single-letter Huffman code for the source s.

Table 3: Binarization of a three letter source. The LPB probabilities p LPB are 0.3 for the first bin
 and 0.4 for the second bin

| Symbol ai | Probability p(ai) | Bin b0 | Bin b1 |
| --- | --- | --- | --- |
| a0 | 0.7 | 1 |  |
| a1 | 0.18 | 0 | 1 |
| a2 | 0.12 | 0 | 0 |
| LPB Prob. | p LPB=p(bj=0) | 0.3 | 0.4 |

The entropy for the source s is

H(0.7,0.18,0.12)=H(0.7,0.3)+0.3 H(0.6,0.4)=1.1726  bit/symbol

The average code word length of the single-letter Huffman code is given as

M−1piℓiHC=1.3 bitsymbol

corresponding to a redundancy of ρHC=0.1274 bit/symbol or 10.87% expected rate overhead.

For the particular binarization example with fixed pmfs, the bins b0 and b1 already represent coding bins, since for both bins the LPB value b LPBj is equal to 0. The distribution f(s) of the LPB probabilities is discrete, with f(p)=0 except for p=0.3 and p=0.4. Consequently, the optimal probability discretization leads to K=2 intervals with the representatives pI0=0.3 and pI1=0.4. The interval border p1 between these intervals can be arbitrarily chosen in [0.3,0.4).

For encoding the source, he sequence of source symbols is binarized into a sequence of bins. The bin b0 is transmitted for every source symbol. The bin b1 is only transmitted when b0=0. The bins b0 and b1 are coded separately with constant LPB probabilities of pI0=0.3 and pI1=0.4, respectively.

An efficient coding of a binary alphabet with fixed probability can be achieved by a simple VNB2VLC mapping. Examples for VNB2VLC mappings with small coding tables for the LPB probabilities p LPB=0.3 and p LPB=0.4 are given in Table 4 and Table 5, respectively. The VNB2VLC mapping for p LPB=0.3 yields a redundancy of 0.0069 bitbin or 0.788%. For the LPB probability of p LPB=0.4, the redundancy is 0.0053 bitbin or 0.548%.

Table 4:  Bin tree and codes for an LPB probability of p LPB=0.3. The redundancy of this code is 0.788%

| Bin Tree | Probability | Codes |
| --- | --- | --- |
| '1 {1' | 0.72=0.49 | '1' |
| '0 {1' | 0.7⋅0.3=0.21 | '0 {1' |
| '0' | 0.3 | '0 {0' |

Table 5:  Bin tree and codes for an LPB probability of p LPB=0.4. The redundancy of this code is 0.548%

| Bin Tree | Probability | Code Tree |
| --- | --- | --- |
| '11 {1' | 0.63=0.216 | '1 {1' |
| '11 {0' | 0.62⋅0.4=0.144 | '00 {1' |
| '1 {0' | 0.6⋅0.4=0.24 | '1 {1' |
| '0 {1' | 0.4⋅0.6=0.24 | '0 {1' |
| '0 {0' | 0.42=0.16 | '00 {0' |

The overall expected rate incurred by the new coding method is

ℓNC=ℓb0+0.3⋅ℓb1=1.181 bitsymbol

The overall redundancy is 0.73% relative to the entropy limit, which represents a significant improvement in comparison to the single-letter Huffman code.

It could be argued that a similar coding efficiency improvement could be obtained by creating a run-length code. For the above example, we could construct a run-length code for the most probable symbol by considering runs of up to two symbols. Each of the events a0a0,a0a1,a0a2,a1,a2 would be associated with a separate codeword. Such a code yields a redundancy of 1.34% relative to the entropy limit. Actually, the VNB2VLC codes can be considered as a generalization of run-length codes for binary symbols (the VNB2VLC code in Table 4 does effectively represent a run-length code). For a single symbol alphabet with fixed probabilities, a similar coding efficiency as for the presented approach can also be achieved by creating a code that maps a variable number of source symbols to variable length codewords. The main advantage of the presented approach is its flexibility in mapping arbitrary source symbol sequences with fixed or adapative probability estimates to a small number of simple binary coders that are operated with fixed LPB probabilities.

## Unique Decodability

With the presented entropy coding scheme, the coding of a sequence of source symbols s=s0,…,sN−1 consists of the following three basic steps:

- symbol binarization b=b0,…,bB−1=γb(s) yielding a sequence of bins b=b0,…,bB−1

- conversion of the sequence of bins b into a sequence of coding bins bc=b0c,…,bB−1c=γc(b)

- binary entropy coding of the sequence of coding bins bc=b0c,…,bB−1c using probability interval discretization and K fixed binary coders

The symbol sequence s=s0,…,sN−1 is uniquely decodable, if the sequence of coding bins bc=b0c,…,bB−1c is uniquely decodable and the mappings γb and γc are invertible.

Let γe notify the encoder mapping of a sequence of one or more coding bins bc=b0c,… onto a sequence of one or more codewords c(bc)=c0,…

c(bc)=γe(bc)

For unique decodability of a sequence of coding bins bc given the sequence of codewords c(bc), the encoder mapping γe must have the property that a unique codeword c(bc) is assigned to each possible sequence of coding bins bc:

∀bic,bjc bic≠bjc ⇒ c(bic)≠c(bjc)

This property is always fulfilled when arithmetic codes or prefix codes are used. It is particularly fulfilled for the VNB2VLC codes described in sec. 2.3.1 (including the codeword termination described in sec. 2.3.3), since the VNB2VLC codes represent prefix codes for variable numbers of bins.

However, in the presented entropy coding approach, the sequence of coding bins bc is partitioned into K sub-sequences bkc, with k=0,…,K−1,

b0c,…,bK−1c=γp(bc)

and to each of the sub-sequences bkc, a sequence of codewords ck(bkc) is assigned using a particular encoder mapping γek. Consequently, the condition on unique decodability has to be extended. A sequence of coding bins bc is uniquely decodable given K sequences of codewords ck(bkc), with k=0,…,K−1, if each sub-sequence of coding bins bkc is uniquely decodable given the corresponding codeword ck(bkc) and the partitioning rule γp is known to the decoder. The partitioning rule γp is given by the LPB probability interval discretization Ik and the LPB probabilities p LPBj that are associated with the coding bins bjc, with j=0,…,B−1. Hence, the LPB probability interval discretization Ik has to be know at the decoder side and the LPB probability p LPBj for each coding bin bjc, with j=0,…,B−1, has to derived in the same way at encoder and decoder side.

For the mapping γc of a sequence of bins b onto a sequence of coding bins bc, each single bj, with j=0,…,B−1, is converted by the binary mapping bjc=γcj(bj)=bj⊕b LPBj. At the decoder side, the bin sequence b can be derived by the binary mappings

bj=(γcj)−1(bjc)=bjc⊕b LPBj

with j=0,…,B−1. If the LPB value b LPBj for each bin bj is derived in the same way at encoder and decoder side, these mappings (γcj)−1 represent the inverses of the corresponding encoder mappings γcj, since

bjc⊕b LPBj=bj⊕b LPBj⊕b LPBj=bj⊕0=bj

and hence, the conversion γb of a sequence of bins b into a sequence of coding bins bc is invertible.

Finally, we investigate the invertibility of the binarization b=γb(s) by which each symbol si, with i=0,…,N−1, is mapped onto a bin sequence bi=γbi(si). A symbol si can be uniquely decoded given the corresponding bin sequence bi if the binarization mapping γbi assigns a different bin sequence bmj to each letter ami of the alphabet Ai for the symbol si. However, this condition is not sufficient, since the partitioning of the bin sequence b=b0,…,bB−1 into bin sequences bi that correspond to the symbols si, with i=0,…,N−1, is not known to the decoder. A sufficient condition is given, when for each symbol si, the bin sequences bmj that are associated with the letters ami of the corresponding alphabet Ai form a prefix code and the binarization mappings γbi for each symbol si, with i=0,…,N−1, are known at the decoder side.

The conditions of unique decodability for the presented entropy coding approach can be summarized as follows:

- the binarization mappings γbi represent prefix codes and are known to the decoder (in symbol coding order)

- the probability models (b LPBj,p LPBj) for all bins bj are derived in the same way at encoder and decoder side

- the partitioning of the LPB probability interval (0,0.5] into K intervals Ik, with k=0,…,K−1, is known to the decoder

- the mapping γek for each probability interval Ik, with k=0,…,K−1, represents a uniquely decodable code

## Overall encoder and decoder design

Fig. 8:  Block diagram for an example encoder (left part) and decoder (right part)

In the following, we describe examples for the overall encoder and decoder design in more detail. We concentrate on coding schemes, in which the probability models b LPB,p LPB for the bins are directly estimated at encoder and decoder side and the K binary coders use VNB2VLC mappings described in sec. 2.3.1. Each source symbol s shall be associated with a symbol category cs, which determines the type of the symbol including its range of values. The order of symbols and associated symbol categories shall be given by the syntax, which is presumed to be known at encoder and decoder side.

### Overview

The block diagram for an example encoder and decoder design is illustrated in Fig. 8. At the encoder side, the symbols s with associated symbol categories cs are fed into the binarizer, which converts each symbol s into a sequence of bins bs=γbcs(s). The used binarization scheme γbcs is determined based on the symbol category cs. In addition, the binarizer associates each bin b of a bin sequence bs with a probability model indication cb, which specifies the probability model that is used for coding the bin b. The probability model indication cb can be derived based on the symbol category cs, the bin number of the current bin inside the bin sequence bs, and/or the values of already coded bins and symbols.

The probability estimator and assigner maintains multiple probability models, which are characterized by pairs of values b LPB,p LPB. It received bins b and associated probability model indications cb from the binarizer, and forwards the LPB value b LPB and the LPB probability p LPB of the indicated probability model to the coding bin deriver and the probability quantizer, respectively. Thereafter, the corresponding probability model b LPB,p LPB is updated using the value of the received bin b.

The coding bin deriver receives bins b and associated LPB values b LPB from the binarizer and the probability estimator and assigner, respectively, and sends coding bins bc, which are derived by bc=b⊕b LPB, to the probability quantizer. The probability quantizer forwards each coding bin bc to one of the K binary encoders. It contains information about the LPB probability interval quantization Ik. The LPB probability p LPB, which is associated with a coding bin bc and received from the probability estimator and assigner, is compared to the interval borders pk and probability interval index k, for which p LPB∈Ik, is derived. Then, the coding bin bc is forwarded to the associated binary encoder.

Each of the K binary encoders consists of a bin buffer and a bin encoder. The bin buffer receives coding bins bc from the probability quantizer and stores them in coding order. The bin encoder implements a particular VNB2VLC mapping and compares the bin sequence in the bin buffer with the bin sequences that are associated with codewords. If the bin sequence in the bin buffer is equal to one of those bin sequences, the bin encoder removes the bin sequence bc from the bin buffer and writes the associated codeword c(bc) to the corresponding codeword stream. At the end of the encoding process for a symbol sequence, for all binary encoders for which the bin buffers are not empty, a terminating codeword is written as described in sec. 2.3.3.

The K resulting codeword streams can be separately transmitted, packetized, or stored, or they can be interleaved for the purpose of transmission or storage.

At the decoder side, each of the K binary decoders consisting of a bin decoder and a bin buffer receives one codeword stream. The bin decoder reads codewords (bc) from the codeword stream and inserts the associated bin sequence bc, in coding order, into the bin buffer.

The decoding of the symbol sequence is driven by the underlying syntax. Requests for a symbol s are sent together with the symbol category cs to the binarizer. The binarizer converts these symbol requests into request for bins. A request for a bin is associated with a probability model indication cb, which is derived in the same way as in the encoder, and sent to the probability estimator and assigner. The probability estimator and assigner is operated similar to its counterpart at the encoder side. Based on the probability model indication cb, it identifies a probability model and forwards its LPB value b LPB and LPB probability p LPB to the bin deriver and the probability quantizer, respectively.

The probability quantizer determines one of the K binary decoders based on the LPB probability p LPB, in the same way as the binary encoder is determined at the encoder side, removes the first coding bin bc, in coding order, from the corresponding bin buffer, and forwards it to the bin deriver. The bin deriver receives coding bins bc and associated LPB values b LPB from the probability quantizer and probability estimator and assigner, respectively, and determines the bin values b=bc⊕b LPB. As final response to a bin request sent by the binarizer, the bin deriver send the decoded bin value b to the binarizer and the probability estimator and assigner.

In the probability estimator and assigner, the value of the decoded bin b is used to update the probability model b LPB,p LPB, which was chosen by the associated value cb, in the same way as at the encoder side. Finally, the binarizer adds the received bin b to the bin sequence bs which has been already received for a symbol request and compares this bin sequence bs with the bin sequences that are associated with symbol values by the binarization scheme γbcs. If the bin sequence bs matches one of those bin sequences, the corresponding decoded symbol s is output as final response to the symbol request. Otherwise, the binarizer sends further bin requests until the symbol s is decoded.

The decoding of a symbol sequence is terminated if no further symbol requests, which are driven by the syntax, are received. The coding bins bc that may be contained in the bin buffers at the end of the entropy decoding process (as a result of termination codewords) are discarded.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).