| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A101 |
| --- | --- |

| Title: | Description of video coding technology proposal by Texas Instruments Inc. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Vivienne Sze Massachusetts Institute of Technology, USA Mehmet Umut Demircin Texas Instruments Inc., USA Salih Dikbas Texas Instruments Inc., USA Minhua Zhou Texas Instruments Inc., USA Anantha P. Chandrakasan Massachusetts Institute of Technology, USA | Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1-214-480-1861 madhukar@ti.com +1-617-258-6405 sze@mit.edu +1-214-567-4005 demircin@ti.com +1-214-567-6650 salih@ti.com +1-214-480-3816 zhou@ti.com +1-617-258-7619 anantha@mit.edu |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution describes three tools that were submitted as a part of video coding technology proposal in response to CfP. The three tools are: orthogonal mode dependent directional transform (OMDDT) which is a simplification of mode dependent directional transform in KTA2.6r1, massively parallel CABAC for CABAC throughput improvement, and compressed reference frame buffer for memory bandwidth and memory size reduction. The video coding technology proposal consisted of these three tools plus the following tools of KTA2.6r1: extended macroblock size, adaptive loop filter, motion vector competition, and adaptive interpolation filter. OMDDT is found to save half the memory required to store transform matrix coefficients when compared to mode dependent directional transform (MDDT) of KTA2.6r1. On some hardwired architectures, OMDDT is expected to use about half the area of the original MDDT. Massively parallel CABAC is found to achieve an effective throughput improvement between 2.78x to 4.49x for Constraint Set 1 (CS1) conditions and 1.68x to 4.81x for Constraint Set 2 (CS2) on Class A, B and E video sequences. Compressed reference frame buffer tool is found to achieve 50% reduction in reference frame memory access bandwidth and memory size on Class A, B, and E video sequences. These gains in complexity reduction, throughput increase, and memory bandwidth and memory size reduction are achieved at a cost of average bit-rate increase of 0.48% for CS1 and 0.58% for CS2 when compared to KTA2.6r1 under similar coding conditions.

# Contents

Abstract	1

Contents	2

1.	Introduction	4

2.	Algorithm description	4

2.1	Motion representation	4

2.2	Intra-frame prediction	4

2.3	Spatial transforms	4

2.4	Quantization	5

2.5	In-loop filtering	5

2.6	Entropy coding	5

2.6.1	Syntax element partitions (SEP)	5

2.6.2	Interleaved Entropy Slices (IES)	6

2.6.3	Bin-level parallelism	7

2.7	Compressed Reference Frame Buffer (CRFB)	8

2.7.1	CRFB Transformation Algorithm	9

2.7.2	CRFB Quantization Algorithm	9

2.7.3	CRFB DC Prediction Algorithm	10

2.7.4	CRFB Entropy Coding Algorithm	10

3.	Compression performance discussion	10

3.1	Objective versus subjective compression performance	10

3.2	Constraint set 1 configuration relative to Alpha anchor	10

3.2.1	Class A	10

3.2.2	Class B	11

3.2.3	Class C	11

3.2.4	Class D	12

3.2.5	Overall	12

3.3	Constraint set 2 configuration relative to Beta and Gamma anchors	13

3.3.1	Class B	13

3.3.2	Class C	13

3.3.3	Class D	14

3.3.4	Class E	15

3.3.5	Overall	15

3.4	Constraint set 1 configuration relative to KTA2.6r1 Hierarchical-B coding	16

3.4.1	Class A	16

3.4.2	Class B	16

3.4.3	Class C	17

3.4.4	Class D	17

3.4.5	Overall	18

3.5	Constraint set 2 configuration relative to KTA2.6r1 IPPP coding	18

3.5.1	Class B	18

3.5.2	Class C	19

3.5.3	Class D	19

3.5.4	Class E	20

3.5.5	Overall	20

4.	Complexity analysis	21

4.1	Encoding time and measurement methodology	21

4.1.1	Constraint set 1 bitstream encoding times and used encoding platform	21

4.1.2	Constraint set 2 bitstream encoding times and used encoding platform	24

4.2	Decoding time and measurement methodology	26

4.2.1	Constraint set 1 bitstream decoding times and comparison with anchors	26

4.2.2	Constraint set 2 bitstream decoding times and comparison with anchors	29

4.3	Expected memory usage of encoder	31

4.4	Expected memory usage of decoder	32

4.5	Complexity characteristics of encoder motion estimation and motion segmentation selection	32

4.6	Complexity characteristics of decoder motion compensation	32

4.7	Complexity characteristics of encoder intra-frame prediction type selection	32

4.8	Complexity characteristics of decoder intra-frame prediction operation	32

4.9	Complexity characteristics of encoder transforms and transform type selection	32

4.10	Complexity characteristics of decoder inverse transform operation	33

4.11	Complexity characteristics of encoder quantization and quantization type selection	33

4.12	Complexity characteristics of decoder inverse quantization	33

4.13	Complexity characteristics of encoder in-loop filtering type selection	33

4.14	Complexity characteristics of decoder in-loop filtering operation	33

4.15	Complexity characteristics of encoder entropy coding type selection	33

4.16	Complexity characteristics of decoder entropy decoding operation	34

4.17	Degree of capability for encoder parallel processing	34

4.17.1	OMDDT	34

4.17.2	Massively Parallel CABAC	34

4.17.3	Compressed Reference Frame Buffer (CRFB)	34

4.18	Degree of capability for decoder parallel processing	34

4.18.1	OMDDT	34

4.18.2	Massively Parallel CABAC	35

4.18.3	Compressed Reference Frame Buffer (CRFB)	35

4.19	Complexity characteristics of compressed reference frame buffer (CRFB) algorithm	35

5.	Algorithmic characteristics	35

5.1	Random access characteristics	35

5.2	Delay characteristics	35

6.	Software implementation description	36

7.	Highlighted aspects discussion	36

8.	Closing remarks	36

9.	IPR notice(s)	36

References	37

Appendix A: OMDDT transform matrices	38

4x4 OMDDT transform matrices	38

8x8 OMDDT transform matrices	39

16x16 OMDDT transform matrices	42

# Introduction

Memory access bandwidth reduction, enhanced parallelism support, memory size reduction are important requirements for next-generation video standard [17][18][1] . This contribution describes three tools that were submitted as a part of video coding technology proposal in response to CfP. The three tools are: orthogonal mode dependent directional transform (OMDDT) which is a simplification of mode dependent directional transform in KTA2.6r1, massively parallel CABAC for CABAC throughput improvement, and compressed reference frame buffer for memory bandwidth and memory size reduction. The video coding technology proposal consisted of these three tools plus the following tools of KTA2.6r1: extended macroblock size, adaptive loop filter, motion vector competition, and adaptive interpolation filter. OMDDT is found to save half the memory required to store transform matrix coefficients when compared to mode dependent directional transform (MDDT) of KTA2.6r1. On some hardwired architectures, OMDDT is expected to use about half the area of the original MDDT. Massively parallel CABAC is found to achieve an effective throughput improvement between 2.78x to 4.49x for Constraint Set 1 (CS1) conditions and 1.68x to 4.81x for Constraint Set 2 (CS2) on Class A, B and E video sequences. Compressed reference frame buffer tool is found to achieve 50% reduction in reference frame memory access bandwidth and memory size on Class A, B, and E video sequences. These gains in complexity reduction, throughput increase, and memory bandwidth and memory size reduction are achieved at a cost of average bit-rate increase of 0.48% for CS1 and 0.58% for CS2 when compared to KTA2.6r1 under similar coding conditions.

# Algorithm description

Compressed reference frame buffer, orthogonal mode dependent directional transforms, and massively parallel CABAC tools are added on top of KTA2.6r1. KTA2.6r1 with KTA common conditions configuration given in [8] are used in the simulations. KTA tools enabled were as follows:

UseAdaptiveFilter       = 5  # Enhanced AIF

MVCompetition           = 1  # Enabled with default parameters

UseIntraMDDT            = 1  # Use MDDT for intra blocks

UseHPFilter             = 1  # Use High Precision H.264 filter

UseAdaptiveLoopFilter   = 1  # Use adaptive loop filtering

UseExtMB                = 2  # Use extended block size (64x64)

## Motion representation

Same as in KTA2.6r1. No modifications were done.

## Intra-frame prediction

Same as in KTA2.6r1. No modifications were done.

## Spatial transforms

No modifications were done to transforms used for Inter coding. They are the same as in KTA2.6r1. Mode dependent directional transform (MDDT) used for Intra coding [9] is replaced with the orthogonal version that had been presented in [10]. MDDT was introduced in [9] to improve efficiency of coding Intra macroblocks. In MDDT a set of transform matrices (Bi, Ai) are introduced, one for each of the Intra prediction directions i = 0, .., 8 for block sizes 4x4 and 8x8, and i = 0,..,3 for block size 16x16. Let X be the input block that is coded with Intra prediction direction of i. In MDDT, the transformed version of X is obtained as: Y = BiTXAi. Proposed orthogonal version of MDDT (OMDDT) calculates the transformed version of X as: Y = SiTXSi. The OMDDT requires half the memory to store transform matrix coefficients – only one transform matrix (Si) per Intra prediction direction needs to be stored instead of two (Bi, Ai) for MDDT. Also on some hardwired architectures, OMDDT uses about half the area of the original MDDT since hardwired matrix multiplication logic of the first matrix multiplication can be reused for the second matrix multiplication in the transform. In [10], only the 4x4 version of OMDDT had been presented by the authors. Since then OMDDT is extended to 8x8 and 16x16 block sizes. In this video encoding technology proposal, OMDDT is used for all block sizes – 4x4, 8x8, and 16x16. See Appendix A for actual 4x4, 8x8, and 16x16 OMDDT transform matrices.

## Quantization

Same as in KTA2.6r1. No modifications were done. To match the rate points of CfP closely, at most one QP setting change was allowed during the encoding.

## In-loop filtering

Same as is in KTA2.6r1. No modifications were done.

## Entropy coding

Low power and high frame rate/resolution requirements for future video coding applications make the need for parallelism in the video ever more important [1].  The CABAC entropy coding engine has been identified as a key bottleneck in the H.264/AVC video decoder [2]. However, parallelism is difficult to achieve with the existing H.264/AVC CABAC due to its inherent serial nature.

This contribution uses a modified version of the massively parallel CABAC (MP-CABAC), proposed in contribution VCEG-AL21[3] and demonstrated in KTA2.7 software [4], which has an improved trade-off between coding efficiency and throughput relative to H.264/AVC CABAC. MP-CABAC leverages a combination of three forms of parallelism at different data partition levels to enable improved coding efficiency vs. throughput trade-off and area cost vs. throughput trade-off.  First, it uses syntax element parallelism by processing different syntax element partitions simultaneously, allowing the context training to be performed across all instances of the elements, thus retaining good coding efficiency with low area cost. Second, macroblock/slice parallelism is achieved by processing interleaved entropy slices simultaneously with simple synchronization and minimal impact on coding efficiency. Finally, both this techniques can be combined with bin parallelism, which uses conditional context selection as proposed in [19] for additional throughput increase. Bin-level parallelism was not included in this contribution because of time constraints.

### Syntax element partitions (SEP)

Recall that the statistics of the syntax elements are unknown at the beginning of encoding/decoding and vary for different video sequences. Consequently, one of the properties that give CABAC its high coding efficiency is that the contexts are adaptive. Specifically, while encoding/decoding, the contexts undergo training to achieve an accurate estimate of the syntax element probability distribution. A better estimate of the probability results in better coding efficiency. One of the drawbacks of breaking up a picture into several slices is that there are fewer macroblocks, and consequently fewer syntax elements, per slice. Since the entropy coding engine is reset every slice, this means that the context undergoes less training and can results in a poorer estimate of the probability distribution.

To avoid reducing the training, rather than encoding/decoding various macroblocks/slices in parallel, this contribution proposes processing syntax elements in parallel. In other words, rather than grouping binary symbols (bins) by macroblocks and placing them in different slices, bins are grouped based on syntax elements and placed in different partitions which are then processed in parallel. As a result, each partition contains all the bins pertaining to the assigned syntax elements, and the context can then undergo the maximum amount of training (e.g. across all occurrences of the element in the frame) to eliminate the coding efficiency penalty from reduced training. Details of the syntax element to partition mapping can be found in contribution VCEG-AL21 and KTA2.7. The mapping should be done to enable optimal workload balance.

Figure 1 illustrates how concurrency is achieved with SEP. A start code prefix for demarcation is required at the beginning of each partition. The workload (i.e. processing cycles) for each partition is much less than if the bins were left in a single slice.

MB0

macroblock

H.264/AVC Slice

MBINFO

Slice header

Start code

different

syntax element

groups

LEGEND

MB1

MB2

SIGMAP

COEFF

Workload

Syntax Element Partitions

PRED

CBP

Figure 1 - Concurrency with syntax element partitions.

It should be noted that while throughput is correlated with degree of parallelism, it is not equal to it. It depends strongly on the workload balance between the parallel engines. If the workload is not equally distributed, some engines will be idle, and the effective throughput is reduced (i.e. Nx parallel hardware blocks will not result in an Nx throughput increase). Accordingly, to further improve the throughput, the entropy coding algorithm in this contribution features a new scheme which adaptively allocates syntax elements to partitions based on the quantization parameter (QP). This enables the workload (i.e. number of bins) per partition to be better balanced resulting in higher throughput, while reducing the overhead due to partition header bits (i.e. start code and terminate/byte alignment bits). The five groups of syntax elements described in VCEG-AL21 (i.e. MBINFO, PRED, CBP, SIGMAP, COEFF) are assigned to three partitions. There are two different modes used in this scheme: High QP and Low QP. In High QP, CBP, SIGMAP and COEFF are assigned to the same partition, while MBINFO and PRED are each assigned to their own partition. In Low QP, MBINFO, PRED and CBP are assigned to the same partition, while SIGMAP and COEFF are each assigned to their own partition. A threshold value can be sent in the PPS and slice QP is then compared to this threshold to determine whether it is encoded in the high or low QP mode.

### Interleaved Entropy Slices (IES)

To achieve additional throughput improvement, the previous approach (as well as bin level parallelism [19]) can be combined with interleaved entropy slices. As presented in [5], entropy slices can undergo independent entropy (de)coding. However, to achieve better coding efficiency than fully independent slices (i.e. H.264/AVC slices), there remains dependencies between the entropy slices for spatial and motion vector prediction.

The macroblocks are allocated to interleaved entropy slices as shown in Figure 2. Within each slice, the raster scan order processing is retained. Benefits of interleaved entropy slices [6] include

- cross slice context selection

- simple synchronization

- reduction in memory bandwidth

- low latency

- improved workload balance

 Slice 0

 Slice 1

Figure 2 - Macroblock allocation for interleaved entropy slices.

Entropy

Decoding 0

Entropy

Decoding 1

Backend

Decoding 0

Backend

Decoding 1

Encoded entropy slice 1

Encoded entropy slice 0

Macroblocks of row 2N

Macroblocks of row 2N+1

Figure 3 - Interleaved Entropy Slices architecture example for 2x parallel decoding. Note that the full decode path is parallelized.

With interleaved entropy slices, as long as the slice 0 decoding process is one macroblock ahead of slice 1 decoding process, the top-macroblock dependency is retained, which enables cross slice context selection during parallel processing. At the decoder, synchronization between entropy slices can easily be implemented through the use of FIFOs between the slices (Figure 3). Furthermore, both the front-end entropy processing and the back-end prediction processing can be done in this order (i.e. the entire decoder path is parallelized), which allows the decoded syntax elements to be immediately processed by the back-end. Consequently, no buffering is required to store the decoded syntax elements, which reduces memory costs. This can have benefits in terms of reducing system power and possibly improving performance (by avoiding read conflicts). Retaining raster scan order processing within each entropy slice also provides a favorable memory access pattern which enables caching techniques for further bandwidth reduction. Another benefit of interleaved entropy slices is that its latency D (as defined in [6]) is always equal to 2 regardless of the number of slices per frame. Thus interleaved entropy slices are suitable for low latency applications (e.g. video conferencing). Finally, in interleaved entropy slices the number of bins per slice tends to be more equally balanced; consequently, a higher effective throughput can be achieved for the same amount of parallelism.

### Bin-level parallelism

The previous two forms of parallelism can be used in conjunction with the bin level parallelism proposed in [19] for further increase in throughput.  Bin-level parallelism was found to have negligible impact on coding efficiency.  For a 2-bin/cycle implementation, the average coding efficiency penalty (Bjontegaard ∆Bitrate) was 0.76% relative to H.264/AVC., with a throughput improvement of 1.79 to 1.98x. Bin-level parallelism was not included in this submission because of time constraints.

## Compressed Reference Frame Buffer (CRFB)

The purpose of this algorithm is to reduce external memory access bandwidth and the reference frame buffer memory size in video coding. The reference frames are compressed before being stored in memory and they are decompressed after being read from the memory. Figure 4 illustrates where the compressed reference frame buffer (CRFB) algorithm is executed within the simplified video encoding block diagram. Decoder implementation is similar. An initial version of this algorithm was proposed in [12]. Two key steps in the proposed technique are: (1) Using fixed-length compression (FLC) to compress reference frames in order to maintain random access for predefined blocks of pixels in memory. (2) Carrying out reference frame compression in the core video coding loop so that quantization errors encountered during FLC show up in the residual after motion compensation thereby preventing drift between the encoder and the decoder. Proposed algorithm operates on the raw YUV format (YUV420 and 8-bit assumed in implementation) frames.

Figure 4- Video coding using compressed reference frame buffers (CRFB).

Reference frame compression algorithm is designed based on the following goals:

- Random memory access to predefined pixel blocks.

- Low-complexity implementation.

- Minimal quality degradation.

To provide random memory access capability compression ratio is fixed for a predefined block of YUV pixel data, which will be referred as memory access unit (MAU) here on, corresponding to a row of macroblocks (MBs). This video coding technology proposal uses 64x64 pixels MB size and CRFB compression ratio of 2.

Compressed reference frame buffer algorithm consists of basic image compression tools as depicted in Figure 5. Transformation, quantization, DC prediction and entropy coding are designed to minimize the hardware and software implementation complexity.

Figure 5 – Compressed reference frame buffer (CRFB) algorithm block diagram.

### CRFB Transformation Algorithm

2-D S transform, which is an integer transform based on lifting scheme, is selected as the transform [13], [14]. Transform operates on 8x8 Y, 4x4 U, and 4x4 V pixel blocks, which will be referred as CRFB block here on. 2-D S transform is separable and applied in horizontal and vertical directions consecutively. The order is not important, however, the forward and inverse transforms should have reverse orders of each other.

S transform has several advantages: 1) enables direct fixed-point implementation, 2) enables lossless coding, and 3) enables low-complexity implementation. S transform is reversible and implementable with only addition, subtraction and shift operations.

Illustration of forward one lifting step is shown in Figure 6, where P denotes prediction and U denotes the update stages. Inverse transform is obtained by reversing the steps of the forward transform and flipping the signs.

Figure 6 - Illustration of one step forward lifting

Forward and inverse S transformation equations for one lifting step are as below:

Forward:y[2n+1]=x[2n+1]−x[2n]

Inverse:x[2n]=y[2n]−y[2n+1]2

where x, y[2n], and y[2n+1] are input, low-pass subband, and high-pass subband signal, respectively

Three lifting steps are employed. To achieve that, illustration in Figure 6 is cascaded for approximation terms for 3 times. Inverse lifting step is also obtained similarly

### CRFB Quantization Algorithm

A midtread quantizer is used for transform coefficients other than the lowest frequency coefficients of each CRFB block. The below equation defines the quantization process where x is the transform coefficient, k is and integer representing the quantization scale, sgn() is the signature function, and ë×û represents rounding towards zero.

sgn(x)x2k+12

### CRFB DC Prediction Algorithm

Proposed technique involves predicting lowest frequency transform coefficients (DC coefficient) of a CRFB block using its left, top and, top-left neighbor blocks. The prediction method is same as the MPEG-4 DC prediction [20].

The residual DC coefficient is computed by subtracting the prediction from the actual DC coefficient value. The decoder performs exact same prediction procedure and reconstructs the luma and chroma DC coefficients by adding the residual to the prediction.

### CRFB Entropy Coding Algorithm

Exponential-Golomb (EG)[15] and Unary [16] variable length coding (VLC) techniques are employed. EG0 and EG3 codes are used to compress DC prediction residuals and quantized transform coefficients. Quantization indices are signaled after unary coding.

#### CRFB Encoder Rate Control

CRFB algorithm guarantees fixed compression ratio for a MAU to support random memory access in motion estimation and motion compensation. The MAU is selected as a row of MBs (MB size is 64x64) to minimize the distortion caused by CRFB. The objective of the CRFB encoder is to guarantee number of bits spent to compress a MAU is less than half the size of the uncompressed pixels. A simple leaky-bucket like rate control technique is used.

# Compression performance discussion

## Objective versus subjective compression performance

Informal subjective tests confirmed that proposed video coding technology had subjective quality improvement over anchors commensurate with the PSNR improvements that were observed in Section 3.2 and 3.3.

## Constraint set 1 configuration relative to Alpha anchor

### Class A

Table 1 – Class A, constraint set 1 rate-distortion comparison to Alpha anchor

| Sequence name | Sequence number and rate point | Alpha_anchor |  | Proposal |  | Proposal - Alpha_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| Traffic | S01R1 | 34.29 | 2432.58 | 36.03 | 2473.8 | 1.33 | -30.46 | BD-Low |
|  | S01R2 | 35.81 | 3430.24 | 37.34 | 3494.55 | 1.08 | -28.06 | BD-High |
|  | S01R3 | 37.19 | 4863.48 | 38.55 | 4995.91 |  |  |  |
|  | S01R4 | 38.96 | 7891.16 | 39.98 | 7940.66 |  |  |  |
|  | S01R5 | 40.7 | 13780.54 | 41.46 | 13905.87 |  |  |  |
| PeopleOnStreet | S02R1 | 26.44 | 2457.6 | 27.91 | 2484.09 | 1.19 | -20.73 | BD-Low |
|  | S02R2 | 28.22 | 3482.94 | 29.56 | 3497.79 | 1.02 | -18.27 | BD-High |
|  | S02R3 | 30.11 | 4938.25 | 31.29 | 4982.56 |  |  |  |
|  | S02R4 | 32.55 | 7844.08 | 33.57 | 7959.72 |  |  |  |
|  | S02R5 | 35.32 | 13745.59 | 36.24 | 13987.77 |  |  |  |
| Note: |  |  |  |  | Average | 1.16 | -24.38 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 1.33 | -18.27 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | 1.02 | -30.46 |  |

### Class B

Table 2 - Class B, constraint set 1 rate-distortion comparison to Alpha anchor

| Sequence name | Sequence number and rate point | Alpha_anchor |  | Proposal |  | Proposal - Alpha_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| Kimono | S03R1 | 34.68 | 997.34 | 36.54 | 999.59 | 1.43 | -33.38 | BD-Low |
|  | S03R2 | 36.64 | 1588.5 | 38.22 | 1599.82 | 1.15 | -31.20 | BD-High |
|  | S03R3 | 38.34 | 2493.93 | 39.65 | 2487.57 |  |  |  |
|  | S03R4 | 39.99 | 3999.06 | 40.92 | 3978.12 |  |  |  |
|  | S03R5 | 41 | 5980.18 | 41.76 | 5962.16 |  |  |  |
| ParkScene | S04R1 | 31.76 | 989.23 | 33.01 | 999.27 | 0.94 | -22.91 | BD-Low |
|  | S04R2 | 33.53 | 1589.84 | 34.57 | 1594.45 | 0.77 | -19.57 | BD-High |
|  | S04R3 | 35.21 | 2480.19 | 36.1 | 2499.03 |  |  |  |
|  | S04R4 | 37.01 | 3987.78 | 37.66 | 3989.19 |  |  |  |
|  | S04R5 | 38.36 | 5986.46 | 38.93 | 5993.07 |  |  |  |
| Cactus | S05R1 | 32.18 | 1997.66 | 33.54 | 1991.09 | 1.05 | -29.12 | BD-Low |
|  | S05R2 | 33.72 | 2997.26 | 34.87 | 2997.64 | 0.85 | -28.49 | BD-High |
|  | S05R3 | 35.06 | 4494.98 | 36.01 | 4469.46 |  |  |  |
|  | S05R4 | 36.29 | 6979.8 | 37 | 6974.09 |  |  |  |
|  | S05R5 | 37.04 | 9977.41 | 37.57 | 9927.38 |  |  |  |
| BasketballDrive | S06R1 | 32.2 | 1993.55 | 33.86 | 1988.00 | 1.15 | -30.44 | BD-Low |
|  | S06R2 | 33.86 | 2980.56 | 35.14 | 2970.96 | 0.90 | -28.38 | BD-High |
|  | S06R3 | 35.3 | 4490.39 | 36.3 | 4483.43 |  |  |  |
|  | S06R4 | 36.59 | 6967.81 | 37.32 | 6949.92 |  |  |  |
|  | S06R5 | 37.43 | 9966.3 | 38.01 | 9940.46 |  |  |  |
| BQTerrace | S07R1 | 32.03 | 1979.59 | 33.33 | 1988.32 | 0.85 | -39.58 | BD-Low |
|  | S07R2 | 33.1 | 2972.94 | 34.05 | 2976.52 | 0.66 | -40.28 | BD-High |
|  | S07R3 | 33.89 | 4452.56 | 34.61 | 4455.55 |  |  |  |
|  | S07R4 | 34.52 | 6940.69 | 35.07 | 6949.83 |  |  |  |
|  | S07R5 | 34.92 | 9943.07 | 35.39 | 9949.46 |  |  |  |
| Note: |  |  |  |  | Average | 0.97 | -30.33 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 1.43 | -19.57 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | 0.66 | -40.28 |  |

### Class C

Table 3 - Class C, constraint set 1 rate-distortion comparison to Alpha anchor

| Sequence name | Sequence number and rate point | Alpha_anchor |  | Proposal |  | Proposal - Alpha_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballDrill | S08R1 | 29.16 | 382.94 | 30.8 | 382.6 | 1.42 | -30.11 | BD-Low |
|  | S08R2 | 30.41 | 510.98 | 31.92 | 510.05 | 1.32 | -28.64 | BD-High |
|  | S08R3 | 32.1 | 766.84 | 33.45 | 765.16 |  |  |  |
|  | S08R4 | 33.85 | 1196.35 | 35.12 | 1194.89 |  |  |  |
|  | S08R5 | 35.92 | 1999.13 | 37.14 | 1990.25 |  |  |  |
| BQMall | S09R1 | 28.54 | 383.47 | 30.67 | 382.89 | 1.56 | -28.26 | BD-Low |
|  | S09R2 | 30.17 | 511.8 | 31.84 | 509.65 | 1.35 | -26.28 | BD-High |
|  | S09R3 | 32.16 | 766.05 | 33.59 | 764.75 |  |  |  |
|  | S09R4 | 34.23 | 1197.24 | 35.51 | 1193.46 |  |  |  |
|  | S09R5 | 36.47 | 1996.41 | 37.5 | 1987.21 |  |  |  |
| PartyScene | S10R1 | 25.11 | 382.86 | 26.33 | 382.37 | 1.27 | -30.27 | BD-Low |
|  | S10R2 | 25.93 | 510.31 | 27.3 | 508.97 | 1.25 | -28.75 | BD-High |
|  | S10R3 | 27.44 | 767.5 | 28.65 | 764.79 |  |  |  |
|  | S10R4 | 29.05 | 1199.76 | 30.28 | 1196.08 |  |  |  |
|  | S10R5 | 31.03 | 1997.68 | 32.28 | 1993.91 |  |  |  |
| RaceHorses | S11R1 | 27.59 | 383.3 | 29.08 | 383.34 | 1.27 | -27.57 | BD-Low |
|  | S11R2 | 28.72 | 509.69 | 30.1 | 510.03 | 1.13 | -24.97 | BD-High |
|  | S11R3 | 30.41 | 765.84 | 31.61 | 764.62 |  |  |  |
|  | S11R4 | 32.18 | 1198.06 | 33.28 | 1192.78 |  |  |  |
|  | S11R5 | 34.42 | 1996.27 | 35.22 | 1995.78 |  |  |  |
| Note: |  |  |  |  | Average | 1.32 | -28.10 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 1.56 | -24.97 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | 1.13 | -30.27 |  |

### Class D

Table 4 - Class D, constraint set 1 rate-distortion comparison to Alpha anchor

| Sequence name | Sequence number and rate point | Alpha_anchor |  | Proposal |  | Proposal - Alpha_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballPass | S12R1 | 30.11 | 255.86 | 31.17 | 255.38 | 1.04 | -20.22 | BD-Low |
|  | S12R2 | 31.84 | 383.32 | 32.89 | 383.14 | 1.03 | -18.39 | BD-High |
|  | S12R3 | 33.16 | 511.73 | 34.19 | 511.54 |  |  |  |
|  | S12R4 | 35.65 | 849.4 | 36.67 | 846.77 |  |  |  |
|  | S12R5 | 38.76 | 1496.82 | 39.72 | 1497.19 |  |  |  |
| BQSquare | S13R1 | 29.16 | 255.52 | 30.67 | 254.99 | 1.52 | -35.95 | BD-Low |
|  | S13R2 | 30.59 | 383.42 | 32.11 | 383.1 | 1.42 | -33.79 | BD-High |
|  | S13R3 | 31.56 | 511.51 | 33.1 | 510.34 |  |  |  |
|  | S13R4 | 33.33 | 848.09 | 34.74 | 846.2 |  |  |  |
|  | S13R5 | 35.51 | 1498.19 | 36.66 | 1493.7 |  |  |  |
| BlowingBubbles | S14R1 | 28.97 | 255 | 29.87 | 253.37 | 0.95 | -21.22 | BD-Low |
|  | S14R2 | 30.49 | 382.49 | 31.41 | 380.91 | 1.02 | -21.91 | BD-High |
|  | S14R3 | 31.64 | 511.69 | 32.53 | 507.51 |  |  |  |
|  | S14R4 | 33.64 | 847.6 | 34.7 | 844.05 |  |  |  |
|  | S14R5 | 36.11 | 1498.08 | 37.12 | 1489.18 |  |  |  |
| RaceHorsesD | S15R1 | 30.25 | 255.82 | 31.19 | 255.88 | 0.91 | -17.46 | BD-Low |
|  | S15R2 | 32.01 | 383.16 | 32.94 | 383.29 | 0.87 | -15.61 | BD-High |
|  | S15R3 | 33.36 | 509.68 | 34.27 | 510.03 |  |  |  |
|  | S15R4 | 35.94 | 845.7 | 36.82 | 845.97 |  |  |  |
|  | S15R5 | 39.07 | 1493.18 | 39.84 | 1494.74 |  |  |  |
| Note: |  |  |  |  | Average | 1.10 | -23.07 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 1.52 | -15.61 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | 0.87 | -35.95 |  |

### Overall

Table 5 - Constraint set 1 overall rate-distortion comparison to Alpha anchor

|  | Proposal - Alpha_anchor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-PSNR |  |  | BD-Rate |  |  |
|  | Avg | Max | Min | Avg | Max | Min |
| Class A | 1.16 | 1.33 | 1.02 | -24.38 | -18.27 | -30.46 |
| Class B | 0.97 | 1.43 | 0.66 | -30.33 | -19.57 | -40.28 |
| Class C | 1.32 | 1.56 | 1.13 | -28.10 | -24.97 | -30.27 |
| Class D | 1.10 | 1.52 | 0.87 | -23.07 | -15.61 | -35.95 |
| Overall | 1.12 | 1.56 | 0.66 | -27.01 | -15.61 | -40.28 |

## Constraint set 2 configuration relative to Beta and Gamma anchors

For constraint set 2 configuration, IPPP coding was used instead of Hierarchical-P coding because of better delay profile of IPPP when compared to Hierarchical-P. However it should be noted that, under similar coding conditions, Hierarchical-P can provide better coding performance than IPPP coding especially for sequences with low and moderate motion. The coding performance of proposal v/s Beta anchor is affected by this since Beta anchor uses Hierarchical-P and  proposal uses IPPP. The drop in coding performance is noticeable for BQSquare and BlowingBubbles. Note that KTA2.6r1 IPPP also shows identical drop in coding performance.

### Class B

Table 6 - Class B, constraint set 2 rate-distortion comparison to Beta and Gamma anchor

| Sequence name | Sequence number and rate point | Beta_anchor |  | Gamma_anchor |  | Proposal |  | Proposal - Beta_anchor |  | Proposal - Gamma_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate |  |
| Kimono | S03R1 | 33.95 | 995.88 | 33.01 | 999.36 | 35.8 | 996.47 | 1.57 | -34.63 | 2.55 | -49.66 | BD-Low |
|  | S03R2 | 35.94 | 1598.18 | 34.93 | 1599 | 37.59 | 1598.16 | 1.35 | -33.36 | 2.32 | -49.79 | BD-High |
|  | S03R3 | 37.66 | 2499.09 | 36.69 | 2499.2 | 39.18 | 2499.15 |  |  |  |  |  |
|  | S03R4 | 39.41 | 3997.71 | 38.42 | 3996.58 | 40.59 | 3997.19 |  |  |  |  |  |
|  | S03R5 | 40.63 | 5998.53 | 39.79 | 5998.08 | 41.6 | 5995.4 |  |  |  |  |  |
| ParkScene | S04R1 | 31.38 | 998.44 | 30.22 | 996.81 | 32.16 | 997.49 | 0.42 | -11.02 | 1.48 | -33.92 | BD-Low |
|  | S04R2 | 33.11 | 1598.96 | 32.02 | 1599.88 | 33.6 | 1599.51 | 0.28 | -7.68 | 1.25 | -29.43 | BD-High |
|  | S04R3 | 34.73 | 2499.44 | 33.69 | 2497.77 | 35.03 | 2497.3 |  |  |  |  |  |
|  | S04R4 | 36.4 | 3987.26 | 35.48 | 3997.85 | 36.61 | 3993.03 |  |  |  |  |  |
|  | S04R5 | 37.74 | 5999.1 | 36.97 | 5998.31 | 37.95 | 5992.37 |  |  |  |  |  |
| Cactus | S05R1 | 31.67 | 1998.87 | 30.76 | 1999.7 | 32.59 | 1994.8 | 0.62 | -17.80 | 1.62 | -40.12 | BD-Low |
|  | S05R2 | 33.23 | 2999.54 | 32.12 | 2998.4 | 33.87 | 2993.37 | 0.50 | -16.47 | 1.41 | -38.04 | BD-High |
|  | S05R3 | 34.51 | 4498.71 | 33.53 | 4498.7 | 35.05 | 4488.38 |  |  |  |  |  |
|  | S05R4 | 35.8 | 6998.63 | 34.93 | 6999.17 | 36.23 | 6975.17 |  |  |  |  |  |
|  | S05R5 | 36.68 | 9999.55 | 36.08 | 9997.46 | 37.04 | 9993.41 |  |  |  |  |  |
| BasketballDrive | S06R1 | 31.37 | 1998.18 | 30.16 | 1998.53 | 33.11 | 1978.91 | 1.26 | -31.61 | 2.21 | -46.88 | BD-Low |
|  | S06R2 | 33.09 | 2997.26 | 32.07 | 2999.03 | 34.48 | 2975.04 | 1.01 | -29.33 | 1.82 | -44.95 | BD-High |
|  | S06R3 | 34.59 | 4498.5 | 33.72 | 4499.19 | 35.68 | 4472.6 |  |  |  |  |  |
|  | S06R4 | 35.97 | 6997.97 | 35.25 | 6998.37 | 36.82 | 6978.18 |  |  |  |  |  |
|  | S06R5 | 36.94 | 9999.82 | 36.3 | 9999.84 | 37.68 | 9982.9 |  |  |  |  |  |
| BQTerrace | S07R1 | 31.35 | 1997.94 | 29.58 | 1999.98 | 32.34 | 1965.11 | 0.64 | -26.89 | 1.90 | -55.44 | BD-Low |
|  | S07R2 | 32.42 | 2998.66 | 31.01 | 2997.93 | 33.12 | 2972.16 | 0.51 | -26.03 | 1.47 | -51.23 | BD-High |
|  | S07R3 | 33.35 | 4498.87 | 32.22 | 4499.1 | 33.84 | 4461.86 |  |  |  |  |  |
|  | S07R4 | 34.08 | 6998.4 | 33.31 | 6995.12 | 34.52 | 6960.08 |  |  |  |  |  |
|  | S07R5 | 34.53 | 9997.31 | 34.09 | 9997.93 | 35.02 | 9972.18 |  |  |  |  |  |
| Note: |  |  |  |  |  |  | Average | 0.82 | -23.48 | 1.80 | -43.95 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  |  |  | Max | 1.57 | -7.68 | 2.55 | -29.43 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  |  |  | Min | 0.28 | -34.63 | 1.25 | -55.44 |  |

### Class C

Table 7 - Class C, constraint set 2 rate-distortion comparison to Beta and Gamma anchor

| Sequence name | Sequence number and rate point | Beta_anchor |  | Gamma_anchor |  | Proposal |  | Proposal - Beta_anchor |  | Proposal - Gamma_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate |  |
| BasketballDrill | S08R1 | 28.92 | 382.02 | 27.82 | 383.44 | 29.81 | 383.5 | 0.75 | -17.73 | 1.92 | -39.42 | BD-Low |
|  | S08R2 | 30.13 | 509.38 | 28.97 | 511.98 | 30.95 | 511.74 | 0.68 | -16.83 | 1.86 | -39.51 | BD-High |
|  | S08R3 | 31.74 | 764.57 | 30.58 | 767.82 | 32.47 | 766.86 |  |  |  |  |  |
|  | S08R4 | 33.41 | 1194.36 | 32.26 | 1199.32 | 34.12 | 1198.99 |  |  |  |  |  |
|  | S08R5 | 35.39 | 1991.56 | 34.17 | 1999.59 | 35.87 | 1999.45 |  |  |  |  |  |
| BQMall | S09R1 | 27.78 | 381.58 | 26.58 | 383.75 | 29.33 | 383.81 | 1.07 | -20.64 | 2.22 | -38.16 | BD-Low |
|  | S09R2 | 29.35 | 509.35 | 28.2 | 511.68 | 30.58 | 508.82 | 0.85 | -17.19 | 1.94 | -34.93 | BD-High |
|  | S09R3 | 31.29 | 762.98 | 30.19 | 767.72 | 32.29 | 767.18 |  |  |  |  |  |
|  | S09R4 | 33.42 | 1193.48 | 32.37 | 1198.85 | 34.19 | 1199.8 |  |  |  |  |  |
|  | S09R5 | 35.67 | 1991.04 | 34.65 | 1999.31 | 36.23 | 1998.19 |  |  |  |  |  |
| PartyScene | S10R1 | 24.26 | 382.71 | 23.22 | 383.8 | 24.7 | 383.45 | 0.13 | -3.39 | 1.30 | -33.84 | BD-Low |
|  | S10R2 | 25.16 | 511.34 | 24.15 | 511.58 | 25.4 | 510.46 | 0.01 | -0.19 | 1.30 | -31.62 | BD-High |
|  | S10R3 | 26.61 | 766.94 | 25.37 | 767.92 | 26.65 | 766.61 |  |  |  |  |  |
|  | S10R4 | 28.2 | 1194.82 | 26.87 | 1199.12 | 28.19 | 1197.96 |  |  |  |  |  |
|  | S10R5 | 30.29 | 1997.48 | 28.76 | 1999.38 | 30.08 | 1997.95 |  |  |  |  |  |
| RaceHorses | S11R1 | 27.86 | 382.27 | 27.5 | 383.57 | 28.82 | 382.55 | 0.77 | -18.45 | 1.25 | -28.66 | BD-Low |
|  | S11R2 | 28.93 | 511.64 | 28.47 | 511.94 | 29.76 | 510.56 | 0.72 | -16.89 | 1.20 | -26.69 | BD-High |
|  | S11R3 | 30.46 | 761.92 | 29.98 | 767.55 | 31.2 | 767.75 |  |  |  |  |  |
|  | S11R4 | 32.19 | 1190.98 | 31.73 | 1199.79 | 32.91 | 1197.32 |  |  |  |  |  |
|  | S11R5 | 34.22 | 1989.74 | 33.78 | 1999 | 34.91 | 1994.08 |  |  |  |  |  |
| Note: |  |  |  |  |  |  | Average | 0.62 | -13.91 | 1.63 | -34.10 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  |  |  | Max | 1.07 | -0.19 | 2.22 | -26.69 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  |  |  | Min | 0.01 | -20.64 | 1.20 | -39.51 |  |

### Class D

Table 8 - Class D, constraint set 2 rate-distortion comparison to Beta and Gamma anchor

| Sequence name | Sequence number and rate point | Beta_anchor |  | Gamma_anchor |  | Proposal |  | Proposal - Beta_anchor |  | Proposal - Gamma_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate |  |
| BasketballPass | S12R1 | 29.78 | 255.1 | 29.17 | 255.59 | 30.48 | 255.72 | 0.71 | -14.85 | 1.38 | -27.39 | BD-Low |
|  | S12R2 | 31.45 | 383.06 | 30.82 | 383.97 | 32.17 | 383.51 | 0.68 | -12.86 | 1.44 | -25.99 | BD-High |
|  | S12R3 | 32.68 | 510.56 | 32.02 | 512 | 33.41 | 511.73 |  |  |  |  |  |
|  | S12R4 | 35.11 | 847.23 | 34.32 | 849.85 | 35.77 | 846.3 |  |  |  |  |  |
|  | S12R5 | 38.15 | 1494.99 | 37.32 | 1499.93 | 38.8 | 1498.74 |  |  |  |  |  |
| BQSquare | S13R1 | 28.52 | 255.29 | 26.76 | 255.36 | 28.53 | 254.96 | -0.32 | 10.30 | 1.67 | -44.01 | BD-Low |
|  | S13R2 | 29.91 | 383.26 | 27.99 | 383.87 | 29.68 | 383.62 | -0.47 | 15.70 | 1.63 | -39.54 | BD-High |
|  | S13R3 | 30.85 | 510.78 | 28.82 | 511.87 | 30.46 | 510.41 |  |  |  |  |  |
|  | S13R4 | 32.53 | 847.7 | 30.38 | 849.52 | 31.98 | 848.96 |  |  |  |  |  |
|  | S13R5 | 34.49 | 1494.97 | 32.43 | 1499.18 | 34.04 | 1492.02 |  |  |  |  |  |
| BlowingBubbles | S14R1 | 28.31 | 254.86 | 27.11 | 255.9 | 28.1 | 255.54 | -0.28 | 7.77 | 1.04 | -24.11 | BD-Low |
|  | S14R2 | 29.71 | 383.55 | 28.5 | 383.95 | 29.52 | 382.55 | -0.31 | 7.92 | 1.08 | -23.45 | BD-High |
|  | S14R3 | 30.94 | 510.65 | 29.58 | 511.64 | 30.61 | 511.36 |  |  |  |  |  |
|  | S14R4 | 32.99 | 848.74 | 31.53 | 849.55 | 32.63 | 848.29 |  |  |  |  |  |
|  | S14R5 | 35.34 | 1496.67 | 34.04 | 1499.71 | 35.16 | 1498.75 |  |  |  |  |  |
| RaceHorses | S15R1 | 30.15 | 254.34 | 29.83 | 255.87 | 30.73 | 255.92 | 0.51 | -10.37 | 0.98 | -19.19 | BD-Low |
|  | S15R2 | 31.88 | 383.48 | 31.38 | 383.97 | 32.36 | 382.79 | 0.51 | -9.56 | 0.97 | -17.08 | BD-High |
|  | S15R3 | 33.17 | 510.52 | 32.68 | 511.69 | 33.68 | 511.89 |  |  |  |  |  |
|  | S15R4 | 35.7 | 843.48 | 35.27 | 849.7 | 36.23 | 846.33 |  |  |  |  |  |
|  | S15R5 | 38.78 | 1498.13 | 38.47 | 1499.83 | 39.35 | 1498.09 |  |  |  |  |  |
| Note: |  |  |  |  |  |  | Average | 0.13 | -0.74 | 1.27 | -27.59 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  |  |  | Max | 0.71 | 15.70 | 1.67 | -17.08 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  |  |  | Min | -0.47 | -14.85 | 0.97 | -44.01 |  |

### Class E

Table 9 - Class E, constraint set 2 rate-distortion comparison to Beta and Gamma anchor

| Sequence name | Sequence number and rate point | Beta_anchor |  | Gamma_anchor |  | Proposal |  | Proposal - Beta_anchor |  | Proposal - Gamma_anchor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate | BD-PSNR | BD-Rate |  |
| vidyo1 | S16R1 | 33.67 | 233.72 | 32.56 | 255.96 | 36.03 | 255.72 | 1.52 | -30.03 | 2.71 | -46.44 | BD-Low |
|  | S16R2 | 35.84 | 365.92 | 34.81 | 383.91 | 37.74 | 383.53 | 1.17 | -28.60 | 2.22 | -46.58 | BD-High |
|  | S16R3 | 37 | 460.85 | 36.3 | 511.83 | 38.86 | 511.33 |  |  |  |  |  |
|  | S16R4 | 39.09 | 776.35 | 38.37 | 849.11 | 40.47 | 849.4 |  |  |  |  |  |
|  | S16R5 | 41.13 | 1430.89 | 40.42 | 1499.95 | 42.01 | 1498.68 |  |  |  |  |  |
| vidyo3 | S17R1 | 32.8 | 228.93 | 31.85 | 255.56 | 34.56 | 255.07 | 1.07 | -21.93 | 2.38 | -41.83 | BD-Low |
|  | S17R2 | 34.66 | 335.07 | 33.95 | 383.79 | 36.46 | 382.6 | 0.82 | -20.36 | 1.97 | -40.22 | BD-High |
|  | S17R3 | 36.56 | 503.12 | 35.3 | 511.84 | 37.66 | 510.85 |  |  |  |  |  |
|  | S17R4 | 38.33 | 765.88 | 37.6 | 849.98 | 39.44 | 847.08 |  |  |  |  |  |
|  | S17R5 | 40.45 | 1472.8 | 39.71 | 1498.56 | 41.07 | 1498.75 |  |  |  |  |  |
| vidyo4 | S18R1 | 34.03 | 249.94 | 32.55 | 255.95 | 35.99 | 255.57 | 1.33 | -28.24 | 2.59 | -47.12 | BD-Low |
|  | S18R2 | 35.72 | 346.25 | 34.84 | 383.46 | 37.59 | 383.03 | 0.91 | -25.02 | 2.08 | -47.19 | BD-High |
|  | S18R3 | 37.28 | 494.1 | 36.21 | 511.77 | 38.63 | 511.09 |  |  |  |  |  |
|  | S18R4 | 38.9 | 747.2 | 38.14 | 849.39 | 40.14 | 848.02 |  |  |  |  |  |
|  | S18R5 | 40.89 | 1470.11 | 40.07 | 1499.68 | 41.41 | 1495.26 |  |  |  |  |  |
| Note: |  |  |  |  |  |  | Average | 1.14 | -25.70 | 2.32 | -44.90 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  |  |  | Max | 1.52 | -20.36 | 2.71 | -40.22 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  |  |  | Min | 0.82 | -30.03 | 1.97 | -47.19 |  |

### Overall

Table 10 - Constraint set 2 overall rate-distortion comparison to Beta anchor

|  | Proposal - Beta_anchor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-PSNR |  |  | BD-Rate |  |  |
|  | Avg | Max | Min | Avg | Max | Min |
| Class B | 0.82 | 1.57 | 0.28 | -23.48 | -7.68 | -34.63 |
| Class C | 0.62 | 1.07 | 0.01 | -13.91 | -0.19 | -20.64 |
| Class D | 0.13 | 0.71 | -0.47 | -0.74 | 15.70 | -14.85 |
| Class E | 1.14 | 1.52 | 0.82 | -25.70 | -20.36 | -30.03 |
| Overall | 0.66 | 1.57 | -0.47 | -15.82 | 15.70 | -34.63 |

Table 11 - Constraint set 2 overall rate-distortion comparison to Gamma anchor

|  | Proposal - Gamma_anchor |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-PSNR |  |  | BD-Rate |  |  |
|  | Avg | Max | Min | Avg | Max | Min |
| Class B | 1.80 | 2.55 | 1.25 | -43.95 | -29.43 | -55.44 |
| Class C | 1.63 | 2.22 | 1.20 | -34.10 | -26.69 | -39.51 |
| Class D | 1.27 | 1.67 | 0.97 | -27.59 | -17.08 | -44.01 |
| Class E | 2.32 | 2.71 | 1.97 | -44.90 | -40.22 | -47.19 |
| Overall | 1.72 | 2.71 | 0.97 | -37.58 | -17.08 | -55.44 |

## Constraint set 1 configuration relative to KTA2.6r1 Hierarchical-B coding

KTA common conditions configuration given in [8] were used in this comparison. KTA tools enabled were as follows:

UseAdaptiveFilter       = 5  # Enhanced AIF

MVCompetition           = 1  # Enabled with default parameters

UseIntraMDDT            = 1  # Use MDDT for intra blocks

UseHPFilter             = 1  # Use High Precision H.264 filter

UseAdaptiveLoopFilter   = 1  # Use adaptive loop filtering

UseExtMB                = 2  # Use extended block size (64x64)

### Class A

Table 12 - Class A, constraint set 1 rate-distortion comparison to KTA2.6r1 Hierarchical-B

| Sequence name | Sequence number and rate point | KTA_HierB |  | Proposal_HierB |  | Proposal_HierB - KTA_HierB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| Traffic | S01R1 | 36.04 | 2457.31 | 36.03 | 2473.8 | -0.02 | 0.67 | BD-Low |
|  | S01R2 | 37.34 | 3469.63 | 37.34 | 3494.55 | -0.02 | 0.54 | BD-High |
|  | S01R3 | 38.55 | 4966.61 | 38.55 | 4995.91 |  |  |  |
|  | S01R4 | 39.98 | 7900.2 | 39.98 | 7940.66 |  |  |  |
|  | S01R5 | 41.46 | 13861.1 | 41.46 | 13905.87 |  |  |  |
| PeopleOnStreet | S02R1 | 27.91 | 2472.92 | 27.91 | 2484.09 | -0.02 | 0.48 | BD-Low |
|  | S02R2 | 29.57 | 3480.48 | 29.56 | 3497.79 | -0.01 | 0.31 | BD-High |
|  | S02R3 | 31.29 | 4960.67 | 31.29 | 4982.56 |  |  |  |
|  | S02R4 | 33.56 | 7925.97 | 33.57 | 7959.72 |  |  |  |
|  | S02R5 | 36.23 | 13945.19 | 36.24 | 13987.77 |  |  |  |
| Note: |  |  |  |  | Average | -0.02 | 0.50 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | -0.01 | 0.67 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.02 | 0.31 |  |

### Class B

Table 13 - Class B, constraint set 1 rate-distortion comparison to KTA2.6r1 Hierarchical-B

| Sequence name | Sequence number and rate point | KTA_HierB |  | Proposal_HierB |  | Proposal_HierB - KTA_HierB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| Kimono | S03R1 | 36.55 | 991.09 | 36.54 | 999.59 | -0.03 | 0.89 | BD-Low |
|  | S03R2 | 38.23 | 1588.18 | 38.22 | 1599.82 | -0.02 | 0.66 | BD-High |
|  | S03R3 | 39.66 | 2477.68 | 39.65 | 2487.57 |  |  |  |
|  | S03R4 | 40.92 | 3960.95 | 40.92 | 3978.12 |  |  |  |
|  | S03R5 | 41.76 | 5941.33 | 41.76 | 5962.16 |  |  |  |
| ParkScene | S04R1 | 33.02 | 987.68 | 33.01 | 999.27 | -0.03 | 0.81 | BD-Low |
|  | S04R2 | 34.57 | 1582.16 | 34.57 | 1594.45 | -0.02 | 0.67 | BD-High |
|  | S04R3 | 36.1 | 2483.41 | 36.1 | 2499.03 |  |  |  |
|  | S04R4 | 37.67 | 3969.65 | 37.66 | 3989.19 |  |  |  |
|  | S04R5 | 38.93 | 5972.27 | 38.93 | 5993.07 |  |  |  |
| Cactus | S05R1 | 33.55 | 1967.17 | 33.54 | 1991.09 | -0.03 | 1.03 | BD-Low |
|  | S05R2 | 34.88 | 2974.75 | 34.87 | 2997.64 | -0.02 | 0.80 | BD-High |
|  | S05R3 | 36.01 | 4438.69 | 36.01 | 4469.46 |  |  |  |
|  | S05R4 | 37.01 | 6938.59 | 37 | 6974.09 |  |  |  |
|  | S05R5 | 37.57 | 9889.46 | 37.57 | 9927.38 |  |  |  |
| BasketballDrive | S06R1 | 33.88 | 1964.69 | 33.86 | 1988 | -0.03 | 1.03 | BD-Low |
|  | S06R2 | 35.15 | 2961.14 | 35.14 | 2970.96 | -0.02 | 0.96 | BD-High |
|  | S06R3 | 36.31 | 4439.53 | 36.3 | 4483.43 |  |  |  |
|  | S06R4 | 37.32 | 6910.61 | 37.32 | 6949.92 |  |  |  |
|  | S06R5 | 38.01 | 9911.86 | 38.01 | 9940.46 |  |  |  |
| BQTerrace | S07R1 | 33.34 | 1975.42 | 33.33 | 1988.32 | -0.02 | 1.41 | BD-Low |
|  | S07R2 | 34.06 | 2964.63 | 34.05 | 2976.52 | -0.02 | 2.06 | BD-High |
|  | S07R3 | 34.62 | 4435.57 | 34.61 | 4455.55 |  |  |  |
|  | S07R4 | 35.1 | 6931.73 | 35.07 | 6949.83 |  |  |  |
|  | S07R5 | 35.42 | 9884.79 | 35.39 | 9949.46 |  |  |  |
| Note: |  |  |  |  | Average | -0.02 | 1.03 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | -0.02 | 2.06 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.03 | 0.66 |  |

### Class C

Table 14 - Class C, constraint set 1 rate-distortion comparison to KTA2.6r1 Hierarchical-B

| Sequence name | Sequence number and rate point | KTA_HierB |  | Proposal_HierB |  | Proposal_HierB - KTA_HierB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballDrill | S08R1 | 30.8 | 382.15 | 30.8 | 382.6 | 0.00 | -0.04 | BD-Low |
|  | S08R2 | 31.9 | 509.15 | 31.92 | 510.05 | 0.00 | 0.03 | BD-High |
|  | S08R3 | 33.45 | 764.74 | 33.45 | 765.16 |  |  |  |
|  | S08R4 | 35.12 | 1193.73 | 35.12 | 1194.89 |  |  |  |
|  | S08R5 | 37.14 | 1988.53 | 37.14 | 1990.25 |  |  |  |
| BQMall | S09R1 | 30.67 | 382.46 | 30.67 | 382.89 | -0.02 | 0.41 | BD-Low |
|  | S09R2 | 31.85 | 508.32 | 31.84 | 509.65 | -0.02 | 0.44 | BD-High |
|  | S09R3 | 33.6 | 763.51 | 33.59 | 764.75 |  |  |  |
|  | S09R4 | 35.53 | 1192.87 | 35.51 | 1193.46 |  |  |  |
|  | S09R5 | 37.51 | 1988.82 | 37.5 | 1987.21 |  |  |  |
| PartyScene | S10R1 | 26.34 | 382.45 | 26.33 | 382.37 | -0.01 | 0.17 | BD-Low |
|  | S10R2 | 27.3 | 507.7 | 27.3 | 508.97 | 0.00 | 0.13 | BD-High |
|  | S10R3 | 28.65 | 763.89 | 28.65 | 764.79 |  |  |  |
|  | S10R4 | 30.29 | 1196.9 | 30.28 | 1196.08 |  |  |  |
|  | S10R5 | 32.27 | 1992.64 | 32.28 | 1993.91 |  |  |  |
| RaceHorses | S11R1 | 29.08 | 382.83 | 29.08 | 383.34 | -0.01 | 0.18 | BD-Low |
|  | S11R2 | 30.11 | 509.78 | 30.1 | 510.03 | -0.01 | 0.13 | BD-High |
|  | S11R3 | 31.61 | 763.32 | 31.61 | 764.62 |  |  |  |
|  | S11R4 | 33.28 | 1192.6 | 33.28 | 1192.78 |  |  |  |
|  | S11R5 | 35.23 | 1995.28 | 35.22 | 1995.78 |  |  |  |
| Note: |  |  |  |  | Average | -0.01 | 0.18 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 0.00 | 0.44 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.02 | -0.04 |  |

### Class D

Table 15 - Class D, constraint set 1 rate-distortion comparison to KTA2.6r1 Hierarchical-B

| Sequence name | Sequence number and rate point | KTA_HierB |  | Proposal_HierB |  | Proposal_HierB - KTA_HierB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballPass | S12R1 | 31.18 | 255.36 | 31.17 | 255.38 | 0.00 | 0.10 | BD-Low |
|  | S12R2 | 32.88 | 382.98 | 32.89 | 383.14 | -0.01 | 0.16 | BD-High |
|  | S12R3 | 34.19 | 510.96 | 34.19 | 511.54 |  |  |  |
|  | S12R4 | 36.68 | 846.83 | 36.67 | 846.77 |  |  |  |
|  | S12R5 | 39.73 | 1494.25 | 39.72 | 1497.19 |  |  |  |
| BQSquare | S13R1 | 30.65 | 254.54 | 30.67 | 254.99 | 0.00 | 0.03 | BD-Low |
|  | S13R2 | 32.11 | 382.09 | 32.11 | 383.1 | 0.00 | 0.07 | BD-High |
|  | S13R3 | 33.1 | 510.48 | 33.1 | 510.34 |  |  |  |
|  | S13R4 | 34.74 | 845.43 | 34.74 | 846.2 |  |  |  |
|  | S13R5 | 36.66 | 1493.74 | 36.66 | 1493.7 |  |  |  |
| BlowingBubbles | S14R1 | 29.87 | 253.61 | 29.87 | 253.37 | 0.00 | 0.00 | BD-Low |
|  | S14R2 | 31.41 | 381.33 | 31.41 | 380.91 | 0.00 | 0.06 | BD-High |
|  | S14R3 | 32.54 | 508.67 | 32.53 | 507.51 |  |  |  |
|  | S14R4 | 34.7 | 843.01 | 34.7 | 844.05 |  |  |  |
|  | S14R5 | 37.12 | 1488.77 | 37.12 | 1489.18 |  |  |  |
| RaceHorses | S15R1 | 31.2 | 256.06 | 31.19 | 255.88 | -0.01 | 0.12 | BD-Low |
|  | S15R2 | 32.93 | 382.2 | 32.94 | 383.29 | 0.00 | 0.03 | BD-High |
|  | S15R3 | 34.27 | 509.25 | 34.27 | 510.03 |  |  |  |
|  | S15R4 | 36.82 | 846.07 | 36.82 | 845.97 |  |  |  |
|  | S15R5 | 39.84 | 1494.1 | 39.84 | 1494.74 |  |  |  |
| Note: |  |  |  |  | Average | 0.00 | 0.07 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 0.00 | 0.16 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.01 | 0.00 |  |

### Overall

Table 16 - Constraint set 1 overall rate-distortion comparison to KTA2.6r1 Hierarchical-B

|  | Proposal_HierB - KTA_HierB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-PSNR |  |  | BD-Rate |  |  |
|  | Avg | Max | Min | Avg | Max | Min |
| Class A | -0.02 | -0.01 | -0.02 | 0.50 | 0.67 | 0.31 |
| Class B | -0.02 | -0.02 | -0.03 | 1.03 | 2.06 | 0.66 |
| Class C | -0.01 | 0.00 | -0.02 | 0.18 | 0.44 | -0.04 |
| Class D | 0.00 | 0.00 | -0.01 | 0.07 | 0.16 | 0.00 |
| Overall | -0.01 | 0.00 | -0.03 | 0.48 | 2.06 | -0.04 |

## Constraint set 2 configuration relative to KTA2.6r1 IPPP coding

KTA common conditions configuration given in [8] were used in this comparison. KTA tools enabled were as follows:

UseAdaptiveFilter       = 5  # Enhanced AIF

MVCompetition           = 1  # Enabled with default parameters

UseIntraMDDT            = 1  # Use MDDT for intra blocks

UseHPFilter             = 1  # Use High Precision H.264 filter

UseAdaptiveLoopFilter   = 1  # Use adaptive loop filtering

UseExtMB                = 2  # Use extended block size (64x64)

### Class B

Table 17 - Class B, constraint set 2 rate-distortion comparison to KTA2.6r1 IPPP

| Sequence name | Sequence number and rate point | KTA_IPPP |  | Proposal_IPPP |  | Proposal_IPPP - KTA_IPPP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| Kimono | S03R1 | 35.81 | 988.91 | 35.8 | 996.47 | -0.02 | 0.53 | BD-Low |
|  | S03R2 | 37.59 | 1589.39 | 37.59 | 1598.16 | -0.01 | 0.36 | BD-High |
|  | S03R3 | 39.18 | 2489.51 | 39.18 | 2499.15 |  |  |  |
|  | S03R4 | 40.59 | 3986.12 | 40.59 | 3997.19 |  |  |  |
|  | S03R5 | 41.6 | 5985.71 | 41.6 | 5995.4 |  |  |  |
| ParkScene | S04R1 | 32.15 | 986.88 | 32.16 | 997.49 | -0.02 | 0.51 | BD-Low |
|  | S04R2 | 33.6 | 1590.35 | 33.6 | 1599.51 | -0.01 | 0.37 | BD-High |
|  | S04R3 | 35.03 | 2485.99 | 35.03 | 2497.3 |  |  |  |
|  | S04R4 | 36.61 | 3982.68 | 36.61 | 3993.03 |  |  |  |
|  | S04R5 | 37.95 | 5977.64 | 37.95 | 5992.37 |  |  |  |
| Cactus | S05R1 | 32.59 | 1972.36 | 32.59 | 1994.8 | -0.03 | 1.01 | BD-Low |
|  | S05R2 | 33.88 | 2965.11 | 33.87 | 2993.37 | -0.02 | 0.85 | BD-High |
|  | S05R3 | 35.05 | 4456.43 | 35.05 | 4488.38 |  |  |  |
|  | S05R4 | 36.24 | 6945.43 | 36.23 | 6975.17 |  |  |  |
|  | S05R5 | 37.05 | 9951.2 | 37.04 | 9993.41 |  |  |  |
| BasketballDrive | S06R1 | 33.12 | 1958.84 | 33.11 | 1978.91 | -0.03 | 1.00 | BD-Low |
|  | S06R2 | 34.49 | 2952.69 | 34.48 | 2975.04 | -0.02 | 0.83 | BD-High |
|  | S06R3 | 35.69 | 4449 | 35.68 | 4472.6 |  |  |  |
|  | S06R4 | 36.83 | 6960.13 | 36.82 | 6978.18 |  |  |  |
|  | S06R5 | 37.69 | 9955.49 | 37.68 | 9982.9 |  |  |  |
| BQTerrace | S07R1 | 32.35 | 1949.2 | 32.34 | 1965.11 | -0.02 | 1.21 | BD-Low |
|  | S07R2 | 33.13 | 2952.5 | 33.12 | 2972.16 | -0.02 | 1.19 | BD-High |
|  | S07R3 | 33.85 | 4438.74 | 33.84 | 4461.86 |  |  |  |
|  | S07R4 | 34.53 | 6923.03 | 34.52 | 6960.08 |  |  |  |
|  | S07R5 | 35.03 | 9916.79 | 35.02 | 9972.18 |  |  |  |
| Note: |  |  |  |  | Average | -0.02 | 0.79 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | -0.01 | 1.21 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.03 | 0.36 |  |

### Class C

Table 18 - Class C, constraint set 2 rate-distortion comparison to KTA2.6r1 IPPP

| Sequence name | Sequence number and rate point | KTA_IPPP |  | Proposal_IPPP |  | Proposal_IPPP - KTA_IPPP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballDrill | S08R1 | 29.81 | 383.71 | 29.81 | 383.5 | 0.00 | -0.13 | BD-Low |
|  | S08R2 | 30.94 | 510.78 | 30.95 | 511.74 | 0.01 | -0.19 | BD-High |
|  | S08R3 | 32.47 | 767.59 | 32.47 | 766.86 |  |  |  |
|  | S08R4 | 34.11 | 1200.78 | 34.12 | 1198.99 |  |  |  |
|  | S08R5 | 35.88 | 2000.45 | 35.87 | 1999.45 |  |  |  |
| BQMall | S09R1 | 29.33 | 384.1 | 29.33 | 383.81 | -0.01 | 0.14 | BD-Low |
|  | S09R2 | 30.58 | 508.63 | 30.58 | 508.82 | 0.00 | 0.11 | BD-High |
|  | S09R3 | 32.3 | 766.95 | 32.29 | 767.18 |  |  |  |
|  | S09R4 | 34.19 | 1198.87 | 34.19 | 1199.8 |  |  |  |
|  | S09R5 | 36.23 | 2000.14 | 36.23 | 1998.19 |  |  |  |
| PartyScene | S10R1 | 24.7 | 383.12 | 24.7 | 383.45 | 0.00 | 0.04 | BD-Low |
|  | S10R2 | 25.4 | 510.23 | 25.4 | 510.46 | 0.00 | 0.12 | BD-High |
|  | S10R3 | 26.65 | 766.7 | 26.65 | 766.61 |  |  |  |
|  | S10R4 | 28.2 | 1198.91 | 28.19 | 1197.96 |  |  |  |
|  | S10R5 | 30.08 | 1995.34 | 30.08 | 1997.95 |  |  |  |
| RaceHorses | S11R1 | 28.81 | 382.82 | 28.82 | 382.55 | 0.00 | -0.01 | BD-Low |
|  | S11R2 | 29.76 | 510.54 | 29.76 | 510.56 | 0.00 | 0.03 | BD-High |
|  | S11R3 | 31.2 | 767.4 | 31.2 | 767.75 |  |  |  |
|  | S11R4 | 32.91 | 1196.96 | 32.91 | 1197.32 |  |  |  |
|  | S11R5 | 34.91 | 1993.85 | 34.91 | 1994.08 |  |  |  |
| Note: |  |  |  |  | Average | 0.00 | 0.01 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 0.01 | 0.14 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.01 | -0.19 |  |

### Class D

Table 19 - Class D, constraint set 2 rate-distortion comparison to KTA2.6r1 IPPP

| Sequence name | Sequence number and rate point | KTA_IPPP |  | Proposal_IPPP |  | Proposal_IPPP - KTA_IPPP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| BasketballPass | S12R1 | 30.47 | 255.25 | 30.48 | 255.72 | 0.00 | 0.06 | BD-Low |
|  | S12R2 | 32.19 | 384.2 | 32.17 | 383.51 | 0.00 | 0.01 | BD-High |
|  | S12R3 | 33.4 | 510.27 | 33.41 | 511.73 |  |  |  |
|  | S12R4 | 35.76 | 845.32 | 35.77 | 846.3 |  |  |  |
|  | S12R5 | 38.81 | 1499.92 | 38.8 | 1498.74 |  |  |  |
| BQSquare | S13R1 | 28.53 | 254.58 | 28.53 | 254.96 | -0.01 | 0.41 | BD-Low |
|  | S13R2 | 29.69 | 383.58 | 29.68 | 383.62 | -0.01 | 0.32 | BD-High |
|  | S13R3 | 30.47 | 509.52 | 30.46 | 510.41 |  |  |  |
|  | S13R4 | 31.99 | 848.89 | 31.98 | 848.96 |  |  |  |
|  | S13R5 | 34.05 | 1493.45 | 34.04 | 1492.02 |  |  |  |
| BlowingBubbles | S14R1 | 28.09 | 255.58 | 28.1 | 255.54 | 0.00 | 0.07 | BD-Low |
|  | S14R2 | 29.52 | 381.93 | 29.52 | 382.55 | 0.00 | 0.02 | BD-High |
|  | S14R3 | 30.62 | 511.83 | 30.61 | 511.36 |  |  |  |
|  | S14R4 | 32.63 | 848.33 | 32.63 | 848.29 |  |  |  |
|  | S14R5 | 35.15 | 1497.79 | 35.16 | 1498.75 |  |  |  |
| RaceHorses | S15R1 | 30.74 | 256.17 | 30.73 | 255.92 | 0.01 | -0.11 | BD-Low |
|  | S15R2 | 32.35 | 382.75 | 32.36 | 382.79 | 0.00 | -0.08 | BD-High |
|  | S15R3 | 33.67 | 511.72 | 33.68 | 511.89 |  |  |  |
|  | S15R4 | 36.22 | 845.15 | 36.23 | 846.33 |  |  |  |
|  | S15R5 | 39.36 | 1500.53 | 39.35 | 1498.09 |  |  |  |
| Note: |  |  |  |  | Average | 0.00 | 0.09 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | 0.01 | 0.41 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.01 | -0.11 |  |

### Class E

Table 20 - Class E, constraint set 2 rate-distortion comparison to KTA2.6r1 IPPP

| Sequence name | Sequence number and rate point | KTA_IPPP |  | Proposal_IPPP |  | Proposal_IPPP - KTA_IPPP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PSNR | Bitrate | PSNR | Bitrate | BD-PSNR | BD-Rate |  |
| vidyo1 | S16R1 | 36.02 | 249.13 | 36.03 | 255.72 | -0.07 | 2.06 | BD-Low |
|  | S16R2 | 37.76 | 376.01 | 37.74 | 383.53 | -0.05 | 1.60 | BD-High |
|  | S16R3 | 38.86 | 503.95 | 38.86 | 511.33 |  |  |  |
|  | S16R4 | 40.5 | 842.7 | 40.47 | 849.4 |  |  |  |
|  | S16R5 | 42.01 | 1493.11 | 42.01 | 1498.68 |  |  |  |
| vidyo3 | S17R1 | 34.52 | 248.15 | 34.56 | 255.07 | -0.06 | 1.60 | BD-Low |
|  | S17R2 | 36.47 | 375.73 | 36.46 | 382.6 | -0.04 | 1.15 | BD-High |
|  | S17R3 | 37.64 | 502.77 | 37.66 | 510.85 |  |  |  |
|  | S17R4 | 39.45 | 840.37 | 39.44 | 847.08 |  |  |  |
|  | S17R5 | 41.08 | 1493.2 | 41.07 | 1498.75 |  |  |  |
| vidyo4 | S18R1 | 36 | 249.56 | 35.99 | 255.57 | -0.06 | 1.92 | BD-Low |
|  | S18R2 | 37.6 | 376.17 | 37.59 | 383.03 | -0.04 | 1.57 | BD-High |
|  | S18R3 | 38.64 | 504.89 | 38.63 | 511.09 |  |  |  |
|  | S18R4 | 40.16 | 841.22 | 40.14 | 848.02 |  |  |  |
|  | S18R5 | 41.42 | 1485.42 | 41.41 | 1495.26 |  |  |  |
| Note: |  |  |  |  | Average | -0.05 | 1.65 |  |
| BD-Low: BD metrics for rate points R1, R2, R3, R4 |  |  |  |  | Max | -0.04 | 2.06 |  |
| BD-High: BD metrics for rate points R2, R3, R4, R5 |  |  |  |  | Min | -0.07 | 1.15 |  |

### Overall

Table 21 - Constraint set 2 overall rate-distortion comparison to KTA2.6r1 IPPP

|  | Proposal_IPPP - KTA_IPPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-PSNR |  |  | BD-Rate |  |  |
|  | Avg | Max | Min | Avg | Max | Min |
| Class B | -0.02 | -0.01 | -0.03 | 0.79 | 1.21 | 0.36 |
| Class C | 0.00 | 0.01 | -0.01 | 0.01 | 0.14 | -0.19 |
| Class D | 0.00 | 0.01 | -0.01 | 0.09 | 0.41 | -0.11 |
| Class E | -0.05 | -0.04 | -0.07 | 1.65 | 2.06 | 1.15 |
| Overall | -0.02 | 0.01 | -0.07 | 0.58 | 2.06 | -0.19 |

# Complexity analysis

## Encoding time and measurement methodology

Table 22, Table 23, Table 24, and Table 25 list the encoding time of the software implementation for each constraint set 1 CfP bitstream. Table 26, Table 27, Table 28, and Table 29 list the encoding time of the software implementation for each constraint set 2 CfP bitstream. Encoder is compiled and executed on 64-bit Linux operating system. Above mentioned tables also list the CPU model, frequency, and cache size of the execution platform. Encoding time is calculated using the encoder trace, which is same as the KTA 2.6 output trace. KTA software calculates the encoding times using the time and ftime functions of the C programming language UNIX platform implementation [11]. YUV file output is disabled in encoder executions.

### Constraint set 1 bitstream encoding times and used encoding platform

Table 22 – Class A, constraint set 1 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| A | Traffic | 2500 | 206133 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3500 | 200586 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 5000 | 202058 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 8000 | 207160 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 14000 | 240712 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  | People on Street | 2500 | 186532 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3500 | 187320 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 5000 | 190803 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 8000 | 228401 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 14000 | 240853 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |

Table 23 – Class B, constraint set 1 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | Kimono | 1000 | 164233 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 1600 | 182700 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 2500 | 179785 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 4000 | 193156 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 6000 | 202864 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  | ParkScene | 1000 | 202964 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1600 | 212391 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 2500 | 180487 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 4000 | 190435 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 6000 | 177987 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
| B2 | Cactus | 2000 | 368062 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 3000 | 383229 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 4500 | 377229 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 7000 | 413979 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 10000 | 478983 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  | BasketballDrive | 2000 | 421338 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 3000 | 394649 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 4500 | 403470 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 7000 | 361667 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 10000 | 396275 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | BQTerrace | 2000 | 426571 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3000 | 415793 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 4500 | 458105 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 7000 | 463176 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 10000 | 460674 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |

Table 24 – Class C, constraint set 1 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| C | BasketballDrill | 384 | 92942 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 512 | 89457 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 768 | 89358 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1200 | 89244 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 2000 | 101560 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  | BQMall | 384 | 116876 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 512 | 126962 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 768 | 98934 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 1200 | 91584 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 2000 | 121730 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  | PartyScene | 384 | 93832 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 512 | 78266 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 768 | 90770 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1200 | 71066 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 2000 | 78358 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | RaceHorses | 384 | 46183 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 56061 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 768 | 58243 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1200 | 58212 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 2000 | 63703 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |

Table 25 – Class D, constraint set 1 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| D | BasketballPass | 256 | 27282 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 384 | 25564 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 512 | 21526 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 850 | 26478 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1500 | 29337 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  | BQSquare | 256 | 43448 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 384 | 31761 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 512 | 34645 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 850 | 38349 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1500 | 40016 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  | BlowingBubbles | 256 | 24425 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 384 | 25962 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 512 | 25649 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 850 | 26354 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1500 | 32441 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  | RaceHorses | 256 | 16236 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 384 | 16356 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 512 | 16351 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 850 | 19199 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1500 | 20775 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |

### Constraint set 2 bitstream encoding times and used encoding platform

Table 26 – Class B, constraint set 2 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | Kimono | 1000 | 81556 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 1600 | 79520 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 2500 | 93800 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 4000 | 92199 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 6000 | 92963 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | ParkScene | 1000 | 91783 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1600 | 94663 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 2500 | 96528 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 4000 | 85462 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 6000 | 102206 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
| B2 | Cactus | 2000 | 214196 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3000 | 190704 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 4500 | 193009 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 7000 | 203288 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 10000 | 192995 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | BasketballDrive | 2000 | 172536 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3000 | 165224 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 4500 | 161858 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 7000 | 178533 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 10000 | 178887 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | BQTerrace | 2000 | 179008 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 3000 | 181399 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 4500 | 253112 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 7000 | 193557 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 10000 | 221572 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |

Table 27 – Class C, constraint set 2 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| C | BasketballDrill | 384 | 40856 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 50893 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 768 | 50215 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1200 | 51209 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 2000 | 52289 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  | BQMall | 384 | 52335 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 63009 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 768 | 60791 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1200 | 62525 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 2000 | 55827 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  | PartyScene | 384 | 56074 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 512 | 50688 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 768 | 47783 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1200 | 38070 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 2000 | 39064 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | RaceHorses | 384 | 27808 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 512 | 32351 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 768 | 36288 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1200 | 36806 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 2000 | 37650 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |

Table 28 – Class D, constraint set 2 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| D | BasketballPass | 256 | 13743 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 384 | 13757 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 512 | 15431 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 850 | 16095 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1500 | 16594 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | BQSquare | 256 | 10182 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 384 | 10514 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 14323 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 850 | 11642 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 1500 | 11906 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | BlowingBubbles | 256 | 11691 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 384 | 12236 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 13366 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 850 | 14037 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 1500 | 15931 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  | RaceHorses | 256 | 10217 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 384 | 8394 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 512 | 9574 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 850 | 11768 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1500 | 9529 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |

Table 29 – Class E, constraint set 2 encoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Encoding time (sec) | Cpu Model | Cpu Freq. (GHz) | Cache Size (KB) |
| --- | --- | --- | --- | --- | --- | --- |
| E | Vidyo1 | 256 | 80992 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 384 | 63347 | Intel(R) Xeon(R) CPU X5260 | 3.33 | 6144 |
|  |  | 512 | 93750 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 850 | 104007 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 1500 | 77658 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | Vidyo3 | 256 | 87156 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 384 | 70206 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 512 | 78902 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 850 | 93011 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1500 | 71677 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  | Vidyo4 | 256 | 76491 | Intel(R) Xeon(R) CPU X5570 | 2.93 | 8192 |
|  |  | 384 | 102067 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 512 | 103517 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |
|  |  | 850 | 102311 | Intel(R) Xeon(R) CPU X5470 | 3.33 | 6144 |
|  |  | 1500 | 95100 | Intel(R) Xeon(R) CPU X5460 | 3.16 | 6144 |

## Decoding time and measurement methodology

Table 30, Table 31, Table 32, and Table 33 list the decoding time of the software implementation for each constraint set 1 CfP bitstream. Table 34, Table 35, Table 36, and Table 37 list the decoding time of the software implementation for each constraint set 2 CfP bitstream. Decoder is compiled and executed on 32-bit Windows XP SP2 operating system. Above mentioned tables also list the decoding times of the anchor bitstreams using JM16.2 decoder. Decoding time is calculated using the decoder trace, which is same as the KTA 2.6/JM16.2 output trace. KTA/JM16.2 software calculates the decoding times using the time and _ftime functions of the C programming language [11]. YUV file output is enabled in decoder executions.

Decoding platform CPU specifications are as follows:

- CPU Model: Intel(R) Core(R) 2 CPU 6600

- CPU Frequency: 2.4 GHz

- Cache: 4 MB

The decoder run-time after optimization is expected to be significantly less.

### Constraint set 1 bitstream decoding times and comparison with anchors

Table 30 – Class A, constraint set 1 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Alpha Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- |
| A | Traffic | 2500 | 39.16 | 506.47 |
|  |  | 3500 | 39.47 | 497.24 |
|  |  | 5000 | 40.50 | 486.77 |
|  |  | 8000 | 42.32 | 466.19 |
|  |  | 14000 | 46.19 | 471.14 |
|  | People on Street | 2500 | 39.19 | 416.66 |
|  |  | 3500 | 39.88 | 430.19 |
|  |  | 5000 | 40.79 | 427.27 |
|  |  | 8000 | 42.74 | 432.74 |
|  |  | 14000 | 46.51 | 443.80 |

Table 31 – Class B, constraint set 1 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Alpha Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- |
| B1 | Kimono | 1000 | 33.19 | 350.99 |
|  |  | 1600 | 35.25 | 366.43 |
|  |  | 2500 | 36.84 | 383.47 |
|  |  | 4000 | 38.77 | 396.96 |
|  |  | 6000 | 41.46 | 409.29 |
|  | ParkScene | 1000 | 32.60 | 344.37 |
|  |  | 1600 | 34.87 | 363.68 |
|  |  | 2500 | 36.69 | 375.91 |
|  |  | 4000 | 38.30 | 391.35 |
|  |  | 6000 | 40.66 | 403.46 |
| B2 | Cactus | 2000 | 61.16 | 601.90 |
|  |  | 3000 | 63.16 | 627.24 |
|  |  | 4500 | 65.04 | 649.74 |
|  |  | 7000 | 67.33 | 680.08 |
|  |  | 10000 | 70.94 | 699.88 |
|  | BasketballDrive | 2000 | 68.48 | 725.26 |
|  |  | 3000 | 71.15 | 745.60 |
|  |  | 4500 | 73.61 | 778.60 |
|  |  | 7000 | 77.52 | 796.83 |
|  |  | 10000 | 82.02 | 824.51 |
|  | BQTerrace | 2000 | 84.62 | 953.24 |
|  |  | 3000 | 87.00 | 944.40 |
|  |  | 4500 | 89.02 | 953.13 |
|  |  | 7000 | 89.57 | 976.52 |
|  |  | 10000 | 93.00 | 1002.77 |

Table 32 – Class C, constraint set 1 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Alpha Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- |
| C | BasketballDrill | 384 | 10.79 | 57.38 |
|  |  | 512 | 11.37 | 61.75 |
|  |  | 768 | 11.70 | 66.33 |
|  |  | 1200 | 12.10 | 74.51 |
|  |  | 2000 | 12.63 | 84.52 |
|  | BQMall | 384 | 13.22 | 77.34 |
|  |  | 512 | 14.09 | 80.22 |
|  |  | 768 | 15.22 | 86.83 |
|  |  | 1200 | 15.07 | 94.78 |
|  |  | 2000 | 15.87 | 101.92 |
|  | PartyScene | 384 | 11.21 | 73.45 |
|  |  | 512 | 11.71 | 79.55 |
|  |  | 768 | 12.25 | 90.61 |
|  |  | 1200 | 13.06 | 98.00 |
|  |  | 2000 | 13.89 | 103.84 |
|  | RaceHorses | 384 | 7.68 | 42.92 |
|  |  | 512 | 7.83 | 45.83 |
|  |  | 768 | 8.32 | 50.39 |
|  |  | 1200 | 8.77 | 56.92 |
|  |  | 2000 | 9.51 | 62.55 |

Table 33 – Class D, constraint set 1 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Alpha Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- |
| D | BasketballPass | 256 | 3.08 | 15.42 |
|  |  | 384 | 2.99 | 16.88 |
|  |  | 512 | 3.31 | 18.05 |
|  |  | 850 | 3.71 | 19.56 |
|  |  | 1500 | 4.22 | 21.09 |
|  | BQSquare | 256 | 3.63 | 29.22 |
|  |  | 384 | 3.77 | 30.83 |
|  |  | 512 | 4.08 | 31.47 |
|  |  | 850 | 4.45 | 32.69 |
|  |  | 1500 | 4.99 | 33.94 |
|  | BlowingBubbles | 256 | 2.90 | 16.27 |
|  |  | 384 | 3.11 | 17.84 |
|  |  | 512 | 3.28 | 19.17 |
|  |  | 850 | 3.52 | 21.11 |
|  |  | 1500 | 4.14 | 22.73 |
|  | RaceHorses | 256 | 2.07 | 11.56 |
|  |  | 384 | 2.27 | 12.44 |
|  |  | 512 | 2.37 | 13.36 |
|  |  | 850 | 2.67 | 14.74 |
|  |  | 1500 | 3.17 | 15.50 |

### Constraint set 2 bitstream decoding times and comparison with anchors

Table 34 – Class B, constraint set 2 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Beta Anchor Decoding time (sec) | Gamma Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- | --- |
| B1 | Kimono | 1000 | 24.64 | 23.83 | 370.77 |
|  |  | 1600 | 26.82 | 25.43 | 389.52 |
|  |  | 2500 | 28.71 | 26.97 | 415.49 |
|  |  | 4000 | 31.33 | 29.26 | 429.46 |
|  |  | 6000 | 34.03 | 32.20 | 439.02 |
|  | ParkScene | 1000 | 23.52 | 22.13 | 337.77 |
|  |  | 1600 | 25.78 | 24.06 | 358.66 |
|  |  | 2500 | 27.99 | 25.93 | 367.82 |
|  |  | 4000 | 30.19 | 28.38 | 378.69 |
|  |  | 6000 | 32.71 | 31.18 | 395.86 |
| B2 | Cactus | 2000 | 45.96 | 43.51 | 606.56 |
|  |  | 3000 | 49.07 | 45.74 | 648.09 |
|  |  | 4500 | 51.60 | 48.70 | 673.91 |
|  |  | 7000 | 54.67 | 51.79 | 702.90 |
|  |  | 10000 | 58.57 | 55.85 | 724.68 |
|  | BasketballDrive | 2000 | 49.97 | 47.07 | 711.29 |
|  |  | 3000 | 54.03 | 50.23 | 761.52 |
|  |  | 4500 | 58.85 | 54.15 | 794.51 |
|  |  | 7000 | 62.50 | 57.79 | 826.99 |
|  |  | 10000 | 66.66 | 62.03 | 851.83 |
|  | BQTerrace | 2000 | 59.77 | 56.57 | 844.96 |
|  |  | 3000 | 62.56 | 58.94 | 869.29 |
|  |  | 4500 | 65.47 | 61.27 | 894.29 |
|  |  | 7000 | 69.42 | 64.95 | 939.89 |
|  |  | 10000 | 74.07 | 68.93 | 966.01 |

Table 35 – Class C, constraint set 2 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Beta Anchor Decoding time (sec) | Gamma Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- | --- |
| C | BasketballDrill | 384 | 9.21 | 8.53 | 57.62 |
|  |  | 512 | 9.13 | 8.79 | 61.08 |
|  |  | 768 | 9.19 | 8.84 | 68.03 |
|  |  | 1200 | 9.85 | 9.34 | 77.72 |
|  |  | 2000 | 10.70 | 9.92 | 87.56 |
|  | BQMall | 384 | 10.76 | 10.45 | 74.56 |
|  |  | 512 | 11.05 | 10.73 | 77.97 |
|  |  | 768 | 11.21 | 10.87 | 83.83 |
|  |  | 1200 | 11.99 | 11.66 | 91.12 |
|  |  | 2000 | 13.26 | 12.11 | 100.81 |
|  | PartyScene | 384 | 8.98 | 8.46 | 63.48 |
|  |  | 512 | 9.11 | 9.02 | 66.33 |
|  |  | 768 | 9.79 | 9.53 | 74.22 |
|  |  | 1200 | 10.56 | 10.28 | 85.44 |
|  |  | 2000 | 11.70 | 11.10 | 96.25 |
|  | RaceHorses | 384 | 5.99 | 5.88 | 42.81 |
|  |  | 512 | 6.32 | 5.87 | 45.92 |
|  |  | 768 | 6.81 | 6.38 | 53.11 |
|  |  | 1200 | 7.75 | 6.97 | 60.58 |
|  |  | 2000 | 8.54 | 7.80 | 67.44 |

Table 36 – Class D, constraint set 2 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Beta Anchor Decoding time (sec) | Gamma Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- | --- |
| D | BasketballPass | 256 | 2.47 | 2.17 | 15.44 |
|  |  | 384 | 2.57 | 2.44 | 16.50 |
|  |  | 512 | 2.65 | 2.69 | 17.17 |
|  |  | 850 | 3.21 | 3.03 | 19.19 |
|  |  | 1500 | 3.84 | 3.48 | 21.89 |
|  | BQSquare | 256 | 2.78 | 2.79 | 22.58 |
|  |  | 384 | 3.05 | 2.81 | 24.63 |
|  |  | 512 | 3.30 | 3.10 | 25.83 |
|  |  | 850 | 3.77 | 3.41 | 27.69 |
|  |  | 1500 | 4.44 | 3.98 | 28.30 |
|  | BlowingBubbles | 256 | 2.36 | 2.25 | 14.42 |
|  |  | 384 | 2.44 | 2.31 | 15.98 |
|  |  | 512 | 2.80 | 2.56 | 17.63 |
|  |  | 850 | 3.37 | 3.00 | 20.45 |
|  |  | 1500 | 4.03 | 3.37 | 22.13 |
|  | RaceHorses | 256 | 1.73 | 1.57 | 11.86 |
|  |  | 384 | 1.94 | 1.79 | 13.12 |
|  |  | 512 | 2.09 | 1.95 | 14.03 |
|  |  | 850 | 2.50 | 2.31 | 15.63 |
|  |  | 1500 | 3.03 | 2.66 | 17.06 |

Table 37 – Class E, constraint set 2 decoding times

| CLASS | Sequence Name | Bitrate (kbits/s) | Beta Anchor Decoding time (sec) | Gamma Anchor Decoding time (sec) | Proposal Decoding time (sec) |
| --- | --- | --- | --- | --- | --- |
| E | Vidyo1 | 256 | 20.17 | 19.82 | 258.22 |
|  |  | 384 | 20.80 | 19.86 | 265.81 |
|  |  | 512 | 21.05 | 20.36 | 273.31 |
|  |  | 850 | 21.59 | 20.42 | 283.95 |
|  |  | 1500 | 22.45 | 21.94 | 302.06 |
|  | Vidyo3 | 256 | 20.20 | 19.80 | 260.79 |
|  |  | 384 | 20.20 | 20.18 | 270.26 |
|  |  | 512 | 20.79 | 20.38 | 279.42 |
|  |  | 850 | 21.32 | 20.82 | 291.06 |
|  |  | 1500 | 22.56 | 21.83 | 302.98 |
|  | Vidyo4 | 256 | 20.25 | 19.99 | 264.79 |
|  |  | 384 | 20.92 | 20.27 | 276.68 |
|  |  | 512 | 20.84 | 20.45 | 282.98 |
|  |  | 850 | 21.63 | 20.92 | 298.54 |
|  |  | 1500 | 22.56 | 21.70 | 311.11 |

## Expected memory usage of encoder

The approximate maximum memory space used by the encoder software implementation (physical memory and swap) is measured. Table 38 and Table 39 list the expected memory usage for constraint set 1 and constraint set 2, respectively. Encoder is compiled and executed on 64-bit Linux operating system. The memory usage listed in Table 38 and Table 39 is what has been reported by the computer running the encoder. The actual memory usage in an optimized system is expected to be significantly less.

Table 38 – Constraint set 1 approximate encoder memory usage

| Class | Expected Memory Usage Range (MB) |
| --- | --- |
| A | ~ 6600 - 6700 |
| B | ~ 3600 - 3700 |
| C | ~ 590 – 600 |
| D | ~ 220 – 240 |

Table 39 – Constraint set 2 approximate encoder memory usage

| Class | Expected Memory Usage Range (MB) |
| --- | --- |
| B | ~ 2500 - 3100 |
| C | ~ 530 - 530 |
| D | ~ 200 - 230 |
| E | ~ 980 - 1110 |

## Expected memory usage of decoder

The approximate maximum memory space used by the decoder software implementation (physical memory and swap) is measured. Table 40 and Table 41 list the expected memory usage for constraint set 1 and constraint set 2, respectively. Decoder is compiled and executed on 64-bit Linux operating system. The memory usage listed in Table 40 and Table 41 is what has been reported by the computer running the decoder. The actual memory usage in an optimized system is expected to be significantly less.

Table 40 - Constraint set 1 approximate decoder memory usage

| Class | Expected Memory Usage Range (MB) |
| --- | --- |
| A | ~ 970 - 980 |
| B | ~ 780 - 810 |
| C | ~ 10 - 20 |
| D | ~ 10 |

Table 41 - Constraint set 2 approximate decoder memory usage

| Class | Expected Memory Usage Range (MB) |
| --- | --- |
| B | ~ 710 - 730 |
| C | ~ 10 |
| D | ~ 10 |
| E | ~ 10 |

## Complexity characteristics of encoder motion estimation and motion segmentation selection

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of decoder motion compensation

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of encoder intra-frame prediction type selection

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of decoder intra-frame prediction operation

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of encoder transforms and transform type selection

Transform type selection is same as in KTA2.6r1 for both Inter and Intra modes. The main difference is OMDDT [10] is used instead of MDDT [9] for Intra coding. OMDDT used half the number of transform matrix coefficients when compared to MDDT. The table below summarizes the memory requirement.

Table 42 - Memory requirements for MDDT and OMDDT transform coefficients

|  | Total storage for transform matrices (bytes) |  |
| --- | --- | --- |
|  | MDDT | OMDDT |
| 16x16 | 2*4*16*16 = 2048 | 4*16*16 = 1024 |
| 8x8 | 2*9*8*8 = 1152 | 9*8*8 = 576 |
| 4x4 | 2*9*4*4 = 288 | 9*4*4 = 144 |
| Total | 3488 | 1744 |

The computational complexity of OMDDT is the same as MDDT. However on some hardwired architectures, OMDDT uses about half the area of the original MDDT since hardwired matrix multiplication logic of the first matrix multiplication can be reused for the second matrix multiplication in the transform.

## Complexity characteristics of decoder inverse transform operation

Transform used for Inter is same as in KTA2.6r1. For inverse transform in the case of Intra coding, OMDDT [10] is used instead of MDDT [9] for Intra coding. OMDDT used half the number of transform matrix coefficients when compared to MDDT as shown in Table 42 .

The computational complexity of OMDDT inverse transform is the same as MDDT inverse transform. However on some hardwired architectures, OMDDT uses about half the area of the original MDDT since hardwired matrix multiplication logic of the first matrix multiplication can be reused for the second matrix multiplication in the inverse transform.

## Complexity characteristics of encoder quantization and quantization type selection

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of decoder inverse quantization

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of encoder in-loop filtering type selection

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of decoder in-loop filtering operation

Same as in KTA2.6r1. No modifications were done.

## Complexity characteristics of encoder entropy coding type selection

Syntax Element Partitions (SEP)

Implementations for parallel slice processing require that the entire CABAC be replicated which can lead to significant hardware area cost. An important benefit to syntax element parallelism is that the area cost is quite low since the finite state machine used for context selection, and the context memory, do not need to be replicated. The arithmetic coding engine needs to be replicated, which accounts for only a small percentage of the total hardware area (~15% of the CABAC area).

Furthermore, much of the existing H.264/AVC CABAC architecture can be reused for this approach. As a result, this approach can be easily integrated into multi-standard solutions that support H.264/AVC.

Adaptive allocation based on QP will require the encoder to set the QP threshold. This can be done manually in the configuration file, which was the method used in this contribution. Smart encoder algorithms can be developed to determine the threshold that would provide balanced partitions which will result in higher throughput.

Interleaved Entropy Slices (IES)

The hardware area of the entire CABAC (including the context memory) must be replicated for IES. Thus the area increases linearly with parallelism. However, IES tend to have more balanced workloads as compared to H.264/AVC slices, resulting in higher throughput; thus to achieve the same throughput, IES requires less area increase. Additional area savings is achieved when IES is combined with SEP. Please see contribution VCEG-AL21 for throughput-area tradeoff.

Retaining raster scan order processing within each entropy slice also provides a favorable memory access pattern which enables caching techniques for further bandwidth reduction.

It should be noted that another benefit of this approach is that the access to a large last line buffer is reduced [6]. Out of N slices, N-1 will access a small FIFO for the last line (top) data, and only one will access the large memory (full last line). The small FIFOs can be stored on-chip, and thus the off-chip memory bandwidth for last-line access is reduced by 1/N.

Bin-level parallelism

In bin-level parallelism [19], the context memory which typically dominates the area does not need to be replicated; only the decision tree needs to be added.  The arithmetic coding engine also needs to be extended from 9-bit to 14-bits.  A true multiplier can be used to avoid a large look up table.

## Complexity characteristics of decoder entropy decoding operation

The complexity characteristics of the decoder are similar to those described for the encoder (see above), with the exception that it does not need to choose the QP threshold. Furthermore, to address dependencies at the decoder between partitions and interleaved entropy slices, FIFOs can be used for synchronization. The simplicity of this approach allows the whole processing path to be parallelized. Consequently, no intermediate buffering is required to store the syntax elements between the entropy coding and the rest of the codec. This reduces memory cost and can have benefits in terms of reducing system power and possibly improving performance (by avoiding read conflicts).

## Degree of capability for encoder parallel processing

### OMDDT

The transforms can be implemented using matrix multiplication which is highly parallel operation.

### Massively Parallel CABAC

Syntax element partitions along with the adaptive allocation based on QP delivers up to 3x parallelism (i.e. the three partitions can be processed in parallel). SEP was enabled for all HD sequences (1280x720 and above). The number of interleaved entropy slices per frame is configurable. For 1920x1080 resolution sequences, the encoder was configured to use two interleaved entropy slices per frame. The impact syntax element partitioning and interleaved entropy slices are multiplicative, thus providing up to 6x parallelism.

### Compressed Reference Frame Buffer (CRFB)

CRFB encoder is suitable for parallel processing. Only dependency among the encoding process of CRFB blocks (8x8 Y, 4x4 U and 4x4 V pixels) and memory access units (MAU) are created by the DC prediction technique. MAUs can be encoded in parallel if DC prediction is restricted in MAU boundaries.

## Degree of capability for decoder parallel processing

### OMDDT

The inverse transform can be implemented using matrix multiplication which is highly parallel operation.

### Massively Parallel CABAC

The degree of parallelism at the decoder is similar to the encoder (see above).

### Compressed Reference Frame Buffer (CRFB)

CRFB decoder is suitable for parallel processing similar to the encoder. MAUs can be decoded in parallel if DC prediction is restricted in MAU boundaries.

Decoder implementation may choose to store reconstructed CRFB pixels instead of compressed. This approach will make the decoder memory access architecture same as KTA, however the memory bandwidth saving will not be achieved.

## Complexity characteristics of compressed reference frame buffer (CRFB) algorithm

CRFB algorithms can be implemented using basic addition and shift operations.

Approximate number of arithmetic operations performed by the CRFB encoder are listed in Table 43.

Table 43 – CRFB encoder complexity

| Algorithm | Number of Arithmetic Operations per Pixel |
| --- | --- |
| Transform | - (1+1/4+1/16) *(2 additions and 1 shift) per Luma pixel - (1+1/4)*(2 additions and 1 shift) per Chroma pixel |
| Quantization | - 1 addition and 1 shift per pixel |
| Entropy coding | - 1 EG coding per pixel (additions and shifts depend on image statistics) |
| Rate Control | - log(12)*2 EG coding length computation to select Qidx(12 candidates) and k=0,3 (2 candidates) per pixel. - log(12)*2 additions per pixel to accumulate the bit size of a CRFB block. |

Approximate number of arithmetic operations performed by the CRFB decoder are listed in Table 44.

Table 44 – CRFB decoder complexity

| Algorithm | Number of Arithmetic Operations per Pixel |
| --- | --- |
| Inverse Transform | - (1+1/4+1/16) *(2 additions and 1 shift) per Luma pixel - (1+1/4)*(2 additions and 1 shift) per Chroma pixel |
| Inverse Quantization | - 1 shift per pixel |
| Entropy decoding | - 1 EG decoding per pixel (additions and shifts depend on image statistics) |

CRFB algorithm cuts the memory access bandwidth demanded by reference frame store and read operations by half.

CRFB algorithm requires a line memory to store DC coefficients to be used in DC prediction. The size of the line memory is 3*W/8 bytes, where W is the picture width.

# Algorithmic characteristics

## Random access characteristics

Same as in KTA2.6r1 with KTA common conditions configuration given in [8]. For constraint set 1, Hierarchical-B coding is used with 4 levels (structural delay of processing units not larger than 8-picture “group of pictures”) with Intra period less than 1.1 sec. For constraint set 2, IPPP coding is used with only one I frame in the beginning of the sequence.

## Delay characteristics

Same as in KTA2.6r1 with KTA common conditions configuration given in [8]. For constraint set 1, Hierarchical-B coding is used with 4 levels (structural delay of processing units not larger than 8-picture “group of pictures”) with Intra period less than 1.1 sec. For constraint set 2, IPPP coding is used with only one I frame in the beginning of the sequence.

# Software implementation description

OMDDT, massively parallel CABAC, and compressed reference frame buffer were all implemented on top of KTA2.6r1. No optimization of the software was done.

# Highlighted aspects discussion

This video coding technology proposal focused on complexity reduction, throughput increase, and memory bandwidth and memory size reduction with minimal impact to coding efficiency. With the improvements presented in this proposal, we expect to reduce power consumption and cost of video codecs for coding high resolution video.

# Closing remarks

Memory access bandwidth and video throughput are key limiting factors in video codec chip design. Therefore, in addition to coding efficiency improvements, memory access bandwidth reduction and enhanced parallelism to increase throughput are important for the new video coding standard.

This contribution describes three tools that were submitted as a part of video coding technology proposal in response to CfP. The three tools are: orthogonal mode dependent directional transform (OMDDT) which is a simplification of mode dependent directional transform in KTA2.6r1, massively parallel CABAC for CABAC throughput improvement, and compressed reference frame buffer for memory bandwidth and memory size reduction. OMDDT is found to save half the memory required to store transform matrix coefficients when compared to mode dependent directional transform (MDDT) of KTA2.6r1. On some hardwired architectures, OMDDT is expected to use about half the area of the original MDDT. Massively parallel CABAC is found to achieve an effective throughput improvement between 2.78x to 4.49x for Constraint Set 1 (CS1) conditions and 1.68x to 4.81x for Constraint Set 2 (CS2) on Class A, B and E video sequences. Compressed reference frame buffer tool is found to achieve 50% reduction in reference frame memory access bandwidth and memory size on Class A, B, and E video sequences.  These gains in complexity reduction, throughput increase, and memory bandwidth and memory size reduction are achieved at a cost of average bit-rate increase of 0.48% for CS1 and 0.58% for CS2 when compared to KTA2.6r1 under similar coding conditions.

We request that core experiments be launched on MDDT simplification, parallel CABAC, and reference frame buffer compression for subsequent inclusion into the test model.

# IPR notice(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Massachusetts Institute of Technology may have IPR relating to the technology described in whole or in part by this contribution and is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Texas Instruments, Nokia, Polycom, Samsung AIT, Tandberg, “Desired features in future video coding standards”, Document T05-SG16-C-0215, June 2007.

- V. Sze, M. U. Demircin, and M. Budagavi, “CABAC throughput requirements for real-time decoding,” ITU-T SG 16/Q.6 Doc. VCEG-AJ31, San Diego, USA, October, 2008.

- V. Sze, M. Budagavi, and A. P. Chandrakasan, “Massively Parallel CABAC,” ITU-T SGI 6/Q.6 Doc. VCEG-AL21, Geneva, CH, July, 2009.

- http://iphome.hhi.de/suehring/tml/download/KTA/

- A. Segall and J. Zhao, “Entropy slices for parallel entropy decoding,” ITU-T SGI 6/Q.6 Doc. COM16-C405, Geneva, CH, April 2008.

- D. F. Finchelstein, V. Sze, A. P. Chandrakasan, "Multi-Core Processing and Efficient On-Chip Caching for H.264 and Future Video Decoders," IEEE Transactions on Circuits and Systems for Video Technology (TCSVT), Vol. 19, No. 11, pp. 1704-1713, November 2009.

- X. Guo, Y.-W. Huang, and S. Lei “Ordered Entropy Slices for Parallel CABAC,” ITU-T Q.6/SG16 VCEG, VCEG-AK25, Yokohama, Japan, 15-18 April, 2009.

- TK Tan, G. Sullivan, and T. Wedi, “Recommended Simulation Common Conditions for Coding Efficiency Experiments Revision 1,” ITU-T Q.6/SG16 VCEG, VCEG-AJ10, San Diego, USA, July 2008.

- M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG16 VCEG, VCEG-AF15, San Jose, USA, April 2007.

- M. Budagavi and M. Zhou, “Orthogonal MDDT and Mode Dependent DCT,” ITU-T Q.6/SG16 VCEG, VCEG-AM20, Kyoto, Japan, January 2010.

- Open Group/IEEE. The Open Group Base Specifications Issue 6, IEEE Std 1003.1, 2004 Edition. URL: http://www.opengroup.org/onlinepubs/009695399/basedefs/sys/timeb.h.html

- M. Budagavi and M. Zhou, “Video coding using compressed reference frames”, ITU-T SG16/Q6, 31st VCEG Meeting, Marrakech, Morocco, January 2007, Doc. VCEG-AE19.

- W. Sweldens, “The lifting scheme: A custom-design construction of biorthogonal wavelets,” Appl. Comput. Harmon. Anal., vol. 3, pp. 186–200, Apr. 1996

- R. Calderbank, I. Daubechies, W. Sweldens, and B. Yeo, “Wavelet transforms that map integers to integers,” Appl. Comput. Harmon. Anal., vol. 5, pp. 332–369, July 1998

- J. Teuhola, "A Compression Method for Clustered Bit-Vectors", Information Processing Letters, Vol. 7, pp. 308-311, Oct. 1978.

- Sayood, K. “Introduction to Data Compression, Third Edition, (Morgan Kaufmann Series in Multimedia Information and Systems),” Morgan Kaufmann Publishers Inc, 2005.

- ITU-T SG 16/Q.6, “Draft Requirements for next-generation video coding project”, ITU-T SG 16/Q.6 Doc. VCEG-AL96, London, UK / Geneva, CH, 1-8 July, 2009.

- MPEG Video and Requirements Subgroups, “Vision, Applications and Requirements for High-Performance Video Coding (HVC),” ISO/IEC JTC1/SC29/WG11/N11096, Kyoto, JP, January 2010.

- V. Sze and M. Budagavi, “Parallel CABAC,” ITU-T SG 16/Q.6 Doc. COM16-C334, Geneva, Switzerland, April 2008.

- ISO/IEC 14496-2, Information technology – coding of audio-visual objects – Part 2: Visual, 2001.

# Appendix A: OMDDT transform matrices

### 4x4 OMDDT transform matrices

const int KLTCol[9][4][4]=

{

{

{    38,    58,    75,    78},

{   -80,   -71,    25,    66},

{    78,   -53,   -61,    61},

{    50,   -72,    80,   -47},

},

{

{    34,    54,    73,    84},

{    64,    76,     7,   -80},

{   -80,     8,    87,   -49},

{    69,   -88,    59,   -22},

},

{

{    32,    59,    76,    78},

{   -71,   -77,    15,    72},

{    84,   -35,   -68,    59},

{    58,   -76,    75,   -40},

},

{

{    54,    67,    69,    65},

{   -94,   -35,    48,    64},

{   -58,    76,    42,   -74},

{    36,   -70,    87,   -51},

},

{

{    30,    58,    79,    77},

{   -65,   -80,    10,    75},

{    85,   -23,   -72,    59},

{    64,   -78,    70,   -37},

},

{

{    30,    54,    77,    82},

{   -58,   -84,     0,    77},

{   -88,    16,    77,   -50},

{    65,   -78,    68,   -37},

},

{

{    31,    57,    77,    79},

{    60,    81,    -5,   -78},

{   -82,    12,    80,   -55},

{    71,   -80,    63,   -32},

},

{

{    45,    66,    73,    68},

{    94,    46,   -44,   -60},

{    64,   -74,   -40,    72},

{    39,   -66,    87,   -54},

},

{

{    29,    57,    78,    79},

{    65,    80,    -6,   -76},

{   -83,    18,    76,   -58},

{    66,   -80,    67,   -32},

},

};

const int KLTRow[9][4][4]=

{

{

{    38,   -80,    78,    50},

{    58,   -71,   -53,   -72},

{    75,    25,   -61,    80},

{    78,    66,    61,   -47},

},

{

{    34,    64,   -80,    69},

{    54,    76,     8,   -88},

{    73,     7,    87,    59},

{    84,   -80,   -49,   -22},

},

{

{    32,   -71,    84,    58},

{    59,   -77,   -35,   -76},

{    76,    15,   -68,    75},

{    78,    72,    59,   -40},

},

{

{    54,   -94,   -58,    36},

{    67,   -35,    76,   -70},

{    69,    48,    42,    87},

{    65,    64,   -74,   -51},

},

{

{    30,   -65,    85,    64},

{    58,   -80,   -23,   -78},

{    79,    10,   -72,    70},

{    77,    75,    59,   -37},

},

{

{    30,   -58,   -88,    65},

{    54,   -84,    16,   -78},

{    77,     0,    77,    68},

{    82,    77,   -50,   -37},

},

{

{    31,    60,   -82,    71},

{    57,    81,    12,   -80},

{    77,    -5,    80,    63},

{    79,   -78,   -55,   -32},

},

{

{    45,    94,    64,    39},

{    66,    46,   -74,   -66},

{    73,   -44,   -40,    87},

{    68,   -60,    72,   -54},

},

{

{    29,    65,   -83,    66},

{    57,    80,    18,   -80},

{    78,    -6,    76,    67},

{    79,   -76,   -58,   -32},

},

};

### 8x8 OMDDT transform matrices

const int KLTRow8[9][8][8] =

{

{

{    14,    41,   -50,    56,    50,    57,   -47,    30},

{    25,    55,   -54,    31,     0,   -41,    65,   -54},

{    34,    57,   -20,   -37,   -60,   -39,   -35,    62},

{    43,    46,    29,   -59,    -5,    67,   -10,   -60},

{    51,    20,    60,    -4,    67,   -18,    47,    51},

{    56,   -13,    47,    62,   -14,   -39,   -60,   -40},

{    59,   -45,   -11,    32,   -60,    54,    49,    28},

{    58,   -61,   -62,   -51,    43,   -24,   -21,   -11},

},

{

{    24,   -58,    60,    55,    43,    45,   -37,    23},

{    32,   -64,    40,     4,   -31,   -57,    61,   -42},

{    38,   -51,   -17,   -65,   -53,     0,   -50,    51},

{    45,   -30,   -56,   -35,    47,    56,    21,   -57},

{    50,     0,   -58,    42,    43,   -57,    17,    58},

{    54,    29,   -21,    61,   -58,     3,   -49,   -51},

{    55,    48,    32,    -4,   -30,    55,    65,    43},

{    52,    49,    54,   -50,    48,   -42,   -37,   -21},

},

{

{    27,   -57,    61,    58,    50,    40,   -32,   -17},

{    34,   -62,    36,     3,   -38,   -59,    61,    37},

{    41,   -50,   -14,   -61,   -50,     7,   -56,   -49},

{    47,   -28,   -54,   -37,    48,    54,    29,    55},

{    52,     5,   -58,    37,    42,   -58,    14,   -60},

{    54,    31,   -22,    59,   -50,     2,   -52,    58},

{    52,    48,    30,     4,   -36,    57,    62,   -44},

{    49,    52,    58,   -55,    46,   -40,   -29,    20},

},

{

{    12,    31,   -42,   -64,   -60,   -55,   -46,   -27},

{    24,    56,   -59,   -39,     8,    47,    59,    43},

{    35,    62,   -24,    44,    60,    11,   -47,   -53},

{    44,    49,    33,    54,   -28,   -60,    13,    60},

{    52,    17,    63,   -13,   -51,    47,    30,   -60},

{    57,   -18,    41,   -58,    41,    17,   -57,    50},

{    58,   -46,   -19,   -15,    44,   -61,    59,   -34},

{    56,   -57,   -59,    48,   -46,    35,   -27,    14},

},

{

{     9,    22,   -47,   -55,   -57,   -57,   -55,   -32},

{    21,    46,   -66,   -45,    -3,    33,    63,    49},

{    34,    61,   -33,    32,    61,    32,   -37,   -57},

{    43,    56,    22,    58,   -12,   -63,    -4,    59},

{    53,    26,    57,    -7,   -62,    31,    41,   -55},

{    59,   -12,    44,   -62,    29,    29,   -57,    45},

{    59,   -47,   -11,   -18,    51,   -64,    50,   -31},

{    56,   -63,   -53,    53,   -44,    34,   -22,    14},

},

{

{    11,    29,   -43,   -57,   -62,   -55,   -48,   -34},

{    21,    50,   -58,   -43,    -3,    35,    62,    57},

{    31,    61,   -32,    25,    62,    35,   -33,   -62},

{    42,    52,    17,    62,     0,   -65,   -14,    58},

{    53,    26,    58,     9,   -63,    25,    50,   -49},

{    59,   -11,    50,   -59,    18,    35,   -61,    39},

{    61,   -46,   -11,   -28,    54,   -61,    48,   -27},

{    56,   -61,   -62,    50,   -40,    32,   -22,    10},

},

{

{    14,   -31,    47,    55,    57,    57,   -50,    31},

{    25,   -52,    62,    37,     1,   -41,    62,   -49},

{    35,   -57,    28,   -37,   -66,   -26,   -38,    57},

{    45,   -49,   -29,   -59,    10,    65,     1,   -58},

{    53,   -22,   -62,     4,    61,   -39,    35,    54},

{    57,    16,   -42,    64,   -33,   -23,   -57,   -45},

{    58,    50,    15,    19,   -44,    59,    58,    34},

{    53,    62,    55,   -52,    44,   -33,   -28,   -15},

},

{

{    10,    31,   -45,   -61,   -52,   -59,    53,    24},

{    21,    54,   -59,   -39,     0,    38,   -69,   -43},

{    33,    61,   -29,    36,    60,    32,    46,    53},

{    43,    51,    26,    60,   -10,   -65,    -3,   -58},

{    51,    25,    61,    -6,   -62,    31,   -33,    58},

{    57,   -13,    46,   -61,    32,    29,    51,   -52},

{    61,   -46,   -14,   -18,    53,   -58,   -50,    38},

{    58,   -58,   -58,    46,   -48,    32,    23,   -16},

},

{

{    35,   -64,    66,    55,    44,    34,   -22,   -11},

{    43,   -61,    24,   -21,   -53,   -64,    48,    25},

{    45,   -40,   -36,   -66,   -25,    36,   -59,   -41},

{    46,   -14,   -62,   -10,    65,    28,    47,    54},

{    47,    14,   -50,    57,    16,   -63,    -8,   -63},

{    48,    35,   -10,    52,   -62,    25,   -41,    62},

{    48,    50,    32,    -8,   -23,    51,    69,   -51},

{    47,    54,    52,   -50,    47,   -44,   -38,    23},

},

};

const int KLTCol8[9][8][8] =

{

{

{    14,    25,    34,    43,    51,    56,    59,    58},

{    41,    55,    57,    46,    20,   -13,   -45,   -61},

{   -50,   -54,   -20,    29,    60,    47,   -11,   -62},

{    56,    31,   -37,   -59,    -4,    62,    32,   -51},

{    50,     0,   -60,    -5,    67,   -14,   -60,    43},

{    57,   -41,   -39,    67,   -18,   -39,    54,   -24},

{   -47,    65,   -35,   -10,    47,   -60,    49,   -21},

{    30,   -54,    62,   -60,    51,   -40,    28,   -11},

},

{

{    24,    32,    38,    45,    50,    54,    55,    52},

{   -58,   -64,   -51,   -30,     0,    29,    48,    49},

{    60,    40,   -17,   -56,   -58,   -21,    32,    54},

{    55,     4,   -65,   -35,    42,    61,    -4,   -50},

{    43,   -31,   -53,    47,    43,   -58,   -30,    48},

{    45,   -57,     0,    56,   -57,     3,    55,   -42},

{   -37,    61,   -50,    21,    17,   -49,    65,   -37},

{    23,   -42,    51,   -57,    58,   -51,    43,   -21},

},

{

{    27,    34,    41,    47,    52,    54,    52,    49},

{   -57,   -62,   -50,   -28,     5,    31,    48,    52},

{    61,    36,   -14,   -54,   -58,   -22,    30,    58},

{    58,     3,   -61,   -37,    37,    59,     4,   -55},

{    50,   -38,   -50,    48,    42,   -50,   -36,    46},

{    40,   -59,     7,    54,   -58,     2,    57,   -40},

{   -32,    61,   -56,    29,    14,   -52,    62,   -29},

{   -17,    37,   -49,    55,   -60,    58,   -44,    20},

},

{

{    12,    24,    35,    44,    52,    57,    58,    56},

{    31,    56,    62,    49,    17,   -18,   -46,   -57},

{   -42,   -59,   -24,    33,    63,    41,   -19,   -59},

{   -64,   -39,    44,    54,   -13,   -58,   -15,    48},

{   -60,     8,    60,   -28,   -51,    41,    44,   -46},

{   -55,    47,    11,   -60,    47,    17,   -61,    35},

{   -46,    59,   -47,    13,    30,   -57,    59,   -27},

{   -27,    43,   -53,    60,   -60,    50,   -34,    14},

},

{

{     9,    21,    34,    43,    53,    59,    59,    56},

{    22,    46,    61,    56,    26,   -12,   -47,   -63},

{   -47,   -66,   -33,    22,    57,    44,   -11,   -53},

{   -55,   -45,    32,    58,    -7,   -62,   -18,    53},

{   -57,    -3,    61,   -12,   -62,    29,    51,   -44},

{   -57,    33,    32,   -63,    31,    29,   -64,    34},

{   -55,    63,   -37,    -4,    41,   -57,    50,   -22},

{   -32,    49,   -57,    59,   -55,    45,   -31,    14},

},

{

{    11,    21,    31,    42,    53,    59,    61,    56},

{    29,    50,    61,    52,    26,   -11,   -46,   -61},

{   -43,   -58,   -32,    17,    58,    50,   -11,   -62},

{   -57,   -43,    25,    62,     9,   -59,   -28,    50},

{   -62,    -3,    62,     0,   -63,    18,    54,   -40},

{   -55,    35,    35,   -65,    25,    35,   -61,    32},

{   -48,    62,   -33,   -14,    50,   -61,    48,   -22},

{   -34,    57,   -62,    58,   -49,    39,   -27,    10},

},

{

{    14,    25,    35,    45,    53,    57,    58,    53},

{   -31,   -52,   -57,   -49,   -22,    16,    50,    62},

{    47,    62,    28,   -29,   -62,   -42,    15,    55},

{    55,    37,   -37,   -59,     4,    64,    19,   -52},

{    57,     1,   -66,    10,    61,   -33,   -44,    44},

{    57,   -41,   -26,    65,   -39,   -23,    59,   -33},

{   -50,    62,   -38,     1,    35,   -57,    58,   -28},

{    31,   -49,    57,   -58,    54,   -45,    34,   -15},

},

{

{    10,    21,    33,    43,    51,    57,    61,    58},

{    31,    54,    61,    51,    25,   -13,   -46,   -58},

{   -45,   -59,   -29,    26,    61,    46,   -14,   -58},

{   -61,   -39,    36,    60,    -6,   -61,   -18,    46},

{   -52,     0,    60,   -10,   -62,    32,    53,   -48},

{   -59,    38,    32,   -65,    31,    29,   -58,    32},

{    53,   -69,    46,    -3,   -33,    51,   -50,    23},

{    24,   -43,    53,   -58,    58,   -52,    38,   -16},

},

{

{    35,    43,    45,    46,    47,    48,    48,    47},

{   -64,   -61,   -40,   -14,    14,    35,    50,    54},

{    66,    24,   -36,   -62,   -50,   -10,    32,    52},

{    55,   -21,   -66,   -10,    57,    52,    -8,   -50},

{    44,   -53,   -25,    65,    16,   -62,   -23,    47},

{    34,   -64,    36,    28,   -63,    25,    51,   -44},

{   -22,    48,   -59,    47,    -8,   -41,    69,   -38},

{   -11,    25,   -41,    54,   -63,    62,   -51,    23},

},

};

### 16x16 OMDDT transform matrices

const int KLTCol16[4][16][16] =

{

{

{     5,     9,    12,    15,    20,    24,    27,    30,    34,    37,    40,    41,    43,    44,    44,    43},

{   -10,   -18,   -24,   -30,   -36,   -40,   -40,   -38,   -34,   -24,    -9,     8,    24,    37,    47,    50},

{   -18,   -31,   -39,   -43,   -38,   -25,   -10,     7,    23,    38,    46,    40,    21,    -5,   -33,   -48},

{    17,    32,    37,    35,    17,    -6,   -25,   -40,   -39,   -19,    14,    41,    49,    26,   -19,   -53},

{    27,    42,    40,    15,   -22,   -46,   -44,   -18,    20,    46,    35,    -2,   -34,   -33,    -3,    33},

{    26,    33,    18,   -14,   -38,   -31,     2,    33,    41,     6,   -43,   -42,    15,    55,    16,   -44},

{    29,    39,    13,   -31,   -46,    -4,    39,    37,   -14,   -50,    -7,    44,    19,   -39,   -22,    29},

{    28,    30,   -10,   -39,   -14,    31,    32,   -18,   -43,     6,    51,    -9,   -53,    21,    43,   -32},

{    43,    31,   -33,   -47,    19,    46,   -13,   -44,    12,    35,   -26,   -19,    37,     0,   -39,    25},

{    35,    14,   -40,   -12,    47,    -3,   -52,    20,    40,   -37,   -10,    42,   -23,   -19,    42,   -23},

{    40,     0,   -47,    21,    29,   -40,    -1,    43,   -28,   -16,    41,   -32,    -3,    37,   -47,    22},

{    50,   -19,   -42,    49,   -13,   -31,    46,   -20,   -17,    38,   -31,    10,    16,   -35,    33,   -14},

{    28,   -25,    -8,    35,   -40,    22,    13,   -44,    51,   -32,     0,    29,   -44,    44,   -33,    13},

{   -53,    58,   -26,    -9,    34,   -47,    42,   -24,     6,    12,   -27,    33,   -30,    23,   -16,     6},

{   -33,    45,   -41,    32,   -21,     8,     8,   -25,    37,   -40,    44,   -44,    37,   -28,    18,    -6},

{    25,   -37,    40,   -43,    45,   -45,    44,   -40,    33,   -29,    25,   -20,    16,   -11,     6,    -2},

},

{

{    22,    27,    30,    31,    31,    34,    33,    32,    33,    34,    34,    35,    34,    34,    33,    31},

{   -48,   -51,   -47,   -40,   -30,   -22,   -11,    -1,    11,    19,    24,    29,    32,    34,    34,    32},

{    46,    43,    23,     0,   -24,   -42,   -49,   -41,   -30,   -21,    -9,     6,    22,    32,    36,    33},

{    39,    25,    -1,   -26,   -41,   -37,   -16,    12,    33,    46,    45,    32,     0,   -22,   -37,   -43},

{    52,    18,   -35,   -55,   -31,     9,    46,    41,    14,   -10,   -27,   -33,   -24,     1,    24,    32},

{    29,     4,   -36,   -31,    17,    40,    22,   -16,   -42,   -33,     0,    36,    53,    27,   -22,   -49},

{    34,    -5,   -46,   -13,    51,    33,   -33,   -47,    -5,    29,    31,     6,   -32,   -37,     3,    40},

{    22,   -10,   -34,    12,    42,   -16,   -41,     7,    50,    24,   -38,   -46,    17,    46,     7,   -40},

{    35,   -34,   -30,    42,    20,   -47,    -7,    49,     2,   -42,   -13,    38,    20,   -34,   -21,    30},

{    49,   -63,    -5,    52,   -36,    -4,    36,   -26,   -24,    15,    26,   -14,   -27,    23,    23,   -25},

{    20,   -32,    22,     4,   -42,    40,    -1,   -40,    40,    16,   -47,    -2,    48,   -19,   -40,    33},

{     7,   -16,    14,    -6,   -13,    35,   -31,    -2,    47,   -42,   -21,    57,   -39,   -11,    55,   -35},

{   -20,    39,   -39,    26,     3,   -35,    54,   -46,    10,    28,   -46,    31,     6,   -30,    30,   -14},

{   -17,    38,   -45,    40,   -28,    13,     3,   -21,    41,   -45,    27,     1,   -28,    45,   -44,    20},

{   -11,    27,   -40,    43,   -46,    44,   -39,    34,   -25,    11,    10,   -26,    35,   -39,    33,   -14},

{     2,    -4,     9,   -12,    14,   -18,    22,   -30,    44,   -52,    54,   -47,    44,   -40,    29,   -12},

},

{

{    12,    16,    20,    24,    28,    31,    33,    36,    38,    39,    39,    39,    38,    36,    34,    31},

{   -38,   -43,   -45,   -44,   -40,   -34,   -25,   -15,    -3,     8,    17,    25,    31,    35,    36,    34},

{    43,    44,    35,    20,     1,   -18,   -33,   -42,   -41,   -33,   -19,    -2,    16,    31,    40,    42},

{    43,    36,    14,   -13,   -35,   -44,   -35,   -12,    16,    37,    45,    36,    15,   -12,   -34,   -43},

{    44,    26,   -11,   -39,   -44,   -19,    19,    43,    41,    12,   -23,   -43,   -36,    -8,    24,    39},

{    44,    10,   -33,   -43,   -13,    31,    43,    15,   -26,   -44,   -21,    22,    45,    29,   -12,   -40},

{    41,    -3,   -44,   -24,    23,    43,     5,   -41,   -32,    19,    45,    15,   -34,   -42,     1,    40},

{    35,   -16,   -43,     4,    45,    12,   -44,   -24,    36,    35,   -23,   -42,     4,    44,    16,   -38},

{    35,   -28,   -36,    34,    33,   -38,   -28,    39,    22,   -37,   -20,    34,    22,   -35,   -26,    35},

{    29,   -35,   -17,    48,   -10,   -44,    33,    24,   -41,    -4,    39,    -7,   -40,    23,    37,   -34},

{    24,   -37,     3,    41,   -40,    -3,    44,   -38,    -9,    46,   -26,   -27,    45,    -4,   -39,    27},

{    24,   -46,    28,    15,   -44,    41,   -10,   -29,    46,   -23,   -22,    46,   -23,   -20,    41,   -22},

{    23,   -45,    38,   -13,   -16,    35,   -39,    29,    -3,   -28,    47,   -35,    -5,    39,   -46,    22},

{    18,   -39,    45,   -39,    24,    -3,   -18,    35,   -44,    39,   -19,    -9,    34,   -46,    39,   -16},

{    13,   -29,    37,   -42,    43,   -39,    33,   -23,     6,    12,   -28,    39,   -44,    41,   -30,    13},

{     5,   -13,    17,   -23,    28,   -33,    37,   -41,    44,   -47,    46,   -42,    35,   -28,    20,    -8},

},

{

{    10,    14,    18,    22,    27,    31,    34,    37,    39,    40,    40,    39,    38,    36,    33,    31},

{   -26,   -33,   -38,   -40,   -41,   -39,   -32,   -22,   -10,     2,    14,    25,    33,    39,    41,    40},

{    36,    42,    42,    34,    18,    -3,   -21,   -36,   -42,   -39,   -27,   -10,    11,    27,    39,    42},

{    47,    43,    25,    -2,   -28,   -42,   -41,   -24,     7,    33,    42,    37,    17,    -8,   -29,   -38},

{    44,    33,     1,   -34,   -47,   -26,    11,    43,    41,    11,   -20,   -41,   -38,    -8,    24,    38},

{    44,    16,   -26,   -42,   -19,    23,    42,    22,   -22,   -45,   -26,    20,    47,    31,   -12,   -40},

{    38,     6,   -35,   -34,    11,    44,    19,   -34,   -40,     9,    47,    20,   -36,   -42,     4,    39},

{    41,    -8,   -51,    -7,    47,    21,   -38,   -32,    31,    40,   -21,   -43,     4,    37,    10,   -25},

{    30,   -15,   -37,    19,    38,   -26,   -35,    32,    30,   -37,   -25,    44,    20,   -46,   -22,    37},

{    32,   -28,   -29,    43,    13,   -49,    13,    41,   -40,   -14,    47,   -12,   -36,    28,    22,   -27},

{   -21,    23,    12,   -36,    21,    23,   -43,    14,    31,   -51,    20,    34,   -50,    13,    43,   -36},

{   -38,    60,   -15,   -37,    48,   -21,   -17,    38,   -33,     5,    26,   -36,    21,    11,   -35,    21},

{   -23,    41,   -28,     1,    25,   -40,    35,   -12,   -15,    37,   -45,    33,    -4,   -34,    55,   -30},

{    21,   -43,    46,   -34,    13,    14,   -35,    44,   -41,    28,    -8,   -13,    30,   -41,    39,   -18},

{   -14,    32,   -40,    43,   -40,    31,   -20,     5,     9,   -22,    33,   -39,    44,   -44,    37,   -17},

{     9,   -19,    25,   -32,    37,   -42,    44,   -41,    40,   -38,    36,   -35,    31,   -25,    18,    -7},

},

};

const int KLTRow16[4][16][16] =

{

{

{     5,   -10,   -18,    17,    27,    26,    29,    28,    43,    35,    40,    50,    28,   -53,   -33,    25},

{     9,   -18,   -31,    32,    42,    33,    39,    30,    31,    14,     0,   -19,   -25,    58,    45,   -37},

{    12,   -24,   -39,    37,    40,    18,    13,   -10,   -33,   -40,   -47,   -42,    -8,   -26,   -41,    40},

{    15,   -30,   -43,    35,    15,   -14,   -31,   -39,   -47,   -12,    21,    49,    35,    -9,    32,   -43},

{    20,   -36,   -38,    17,   -22,   -38,   -46,   -14,    19,    47,    29,   -13,   -40,    34,   -21,    45},

{    24,   -40,   -25,    -6,   -46,   -31,    -4,    31,    46,    -3,   -40,   -31,    22,   -47,     8,   -45},

{    27,   -40,   -10,   -25,   -44,     2,    39,    32,   -13,   -52,    -1,    46,    13,    42,     8,    44},

{    30,   -38,     7,   -40,   -18,    33,    37,   -18,   -44,    20,    43,   -20,   -44,   -24,   -25,   -40},

{    34,   -34,    23,   -39,    20,    41,   -14,   -43,    12,    40,   -28,   -17,    51,     6,    37,    33},

{    37,   -24,    38,   -19,    46,     6,   -50,     6,    35,   -37,   -16,    38,   -32,    12,   -40,   -29},

{    40,    -9,    46,    14,    35,   -43,    -7,    51,   -26,   -10,    41,   -31,     0,   -27,    44,    25},

{    41,     8,    40,    41,    -2,   -42,    44,    -9,   -19,    42,   -32,    10,    29,    33,   -44,   -20},

{    43,    24,    21,    49,   -34,    15,    19,   -53,    37,   -23,    -3,    16,   -44,   -30,    37,    16},

{    44,    37,    -5,    26,   -33,    55,   -39,    21,     0,   -19,    37,   -35,    44,    23,   -28,   -11},

{    44,    47,   -33,   -19,    -3,    16,   -22,    43,   -39,    42,   -47,    33,   -33,   -16,    18,     6},

{    43,    50,   -48,   -53,    33,   -44,    29,   -32,    25,   -23,    22,   -14,    13,     6,    -6,    -2},

},

{

{    22,   -48,    46,    39,    52,    29,    34,    22,    35,    49,    20,     7,   -20,   -17,   -11,     2},

{    27,   -51,    43,    25,    18,     4,    -5,   -10,   -34,   -63,   -32,   -16,    39,    38,    27,    -4},

{    30,   -47,    23,    -1,   -35,   -36,   -46,   -34,   -30,    -5,    22,    14,   -39,   -45,   -40,     9},

{    31,   -40,     0,   -26,   -55,   -31,   -13,    12,    42,    52,     4,    -6,    26,    40,    43,   -12},

{    31,   -30,   -24,   -41,   -31,    17,    51,    42,    20,   -36,   -42,   -13,     3,   -28,   -46,    14},

{    34,   -22,   -42,   -37,     9,    40,    33,   -16,   -47,    -4,    40,    35,   -35,    13,    44,   -18},

{    33,   -11,   -49,   -16,    46,    22,   -33,   -41,    -7,    36,    -1,   -31,    54,     3,   -39,    22},

{    32,    -1,   -41,    12,    41,   -16,   -47,     7,    49,   -26,   -40,    -2,   -46,   -21,    34,   -30},

{    33,    11,   -30,    33,    14,   -42,    -5,    50,     2,   -24,    40,    47,    10,    41,   -25,    44},

{    34,    19,   -21,    46,   -10,   -33,    29,    24,   -42,    15,    16,   -42,    28,   -45,    11,   -52},

{    34,    24,    -9,    45,   -27,     0,    31,   -38,   -13,    26,   -47,   -21,   -46,    27,    10,    54},

{    35,    29,     6,    32,   -33,    36,     6,   -46,    38,   -14,    -2,    57,    31,     1,   -26,   -47},

{    34,    32,    22,     0,   -24,    53,   -32,    17,    20,   -27,    48,   -39,     6,   -28,    35,    44},

{    34,    34,    32,   -22,     1,    27,   -37,    46,   -34,    23,   -19,   -11,   -30,    45,   -39,   -40},

{    33,    34,    36,   -37,    24,   -22,     3,     7,   -21,    23,   -40,    55,    30,   -44,    33,    29},

{    31,    32,    33,   -43,    32,   -49,    40,   -40,    30,   -25,    33,   -35,   -14,    20,   -14,   -12},

},

{

{    12,   -38,    43,    43,    44,    44,    41,    35,    35,    29,    24,    24,    23,    18,    13,     5},

{    16,   -43,    44,    36,    26,    10,    -3,   -16,   -28,   -35,   -37,   -46,   -45,   -39,   -29,   -13},

{    20,   -45,    35,    14,   -11,   -33,   -44,   -43,   -36,   -17,     3,    28,    38,    45,    37,    17},

{    24,   -44,    20,   -13,   -39,   -43,   -24,     4,    34,    48,    41,    15,   -13,   -39,   -42,   -23},

{    28,   -40,     1,   -35,   -44,   -13,    23,    45,    33,   -10,   -40,   -44,   -16,    24,    43,    28},

{    31,   -34,   -18,   -44,   -19,    31,    43,    12,   -38,   -44,    -3,    41,    35,    -3,   -39,   -33},

{    33,   -25,   -33,   -35,    19,    43,     5,   -44,   -28,    33,    44,   -10,   -39,   -18,    33,    37},

{    36,   -15,   -42,   -12,    43,    15,   -41,   -24,    39,    24,   -38,   -29,    29,    35,   -23,   -41},

{    38,    -3,   -41,    16,    41,   -26,   -32,    36,    22,   -41,    -9,    46,    -3,   -44,     6,    44},

{    39,     8,   -33,    37,    12,   -44,    19,    35,   -37,    -4,    46,   -23,   -28,    39,    12,   -47},

{    39,    17,   -19,    45,   -23,   -21,    45,   -23,   -20,    39,   -26,   -22,    47,   -19,   -28,    46},

{    39,    25,    -2,    36,   -43,    22,    15,   -42,    34,    -7,   -27,    46,   -35,    -9,    39,   -42},

{    38,    31,    16,    15,   -36,    45,   -34,     4,    22,   -40,    45,   -23,    -5,    34,   -44,    35},

{    36,    35,    31,   -12,    -8,    29,   -42,    44,   -35,    23,    -4,   -20,    39,   -46,    41,   -28},

{    34,    36,    40,   -34,    24,   -12,     1,    16,   -26,    37,   -39,    41,   -46,    39,   -30,    20},

{    31,    34,    42,   -43,    39,   -40,    40,   -38,    35,   -34,    27,   -22,    22,   -16,    13,    -8},

},

{

{    10,   -26,    36,    47,    44,    44,    38,    41,    30,    32,   -21,   -38,   -23,    21,   -14,     9},

{    14,   -33,    42,    43,    33,    16,     6,    -8,   -15,   -28,    23,    60,    41,   -43,    32,   -19},

{    18,   -38,    42,    25,     1,   -26,   -35,   -51,   -37,   -29,    12,   -15,   -28,    46,   -40,    25},

{    22,   -40,    34,    -2,   -34,   -42,   -34,    -7,    19,    43,   -36,   -37,     1,   -34,    43,   -32},

{    27,   -41,    18,   -28,   -47,   -19,    11,    47,    38,    13,    21,    48,    25,    13,   -40,    37},

{    31,   -39,    -3,   -42,   -26,    23,    44,    21,   -26,   -49,    23,   -21,   -40,    14,    31,   -42},

{    34,   -32,   -21,   -41,    11,    42,    19,   -38,   -35,    13,   -43,   -17,    35,   -35,   -20,    44},

{    37,   -22,   -36,   -24,    43,    22,   -34,   -32,    32,    41,    14,    38,   -12,    44,     5,   -41},

{    39,   -10,   -42,     7,    41,   -22,   -40,    31,    30,   -40,    31,   -33,   -15,   -41,     9,    40},

{    40,     2,   -39,    33,    11,   -45,     9,    40,   -37,   -14,   -51,     5,    37,    28,   -22,   -38},

{    40,    14,   -27,    42,   -20,   -26,    47,   -21,   -25,    47,    20,    26,   -45,    -8,    33,    36},

{    39,    25,   -10,    37,   -41,    20,    20,   -43,    44,   -12,    34,   -36,    33,   -13,   -39,   -35},

{    38,    33,    11,    17,   -38,    47,   -36,     4,    20,   -36,   -50,    21,    -4,    30,    44,    31},

{    36,    39,    27,    -8,    -8,    31,   -42,    37,   -46,    28,    13,    11,   -34,   -41,   -44,   -25},

{    33,    41,    39,   -29,    24,   -12,     4,    10,   -22,    22,    43,   -35,    55,    39,    37,    18},

{    31,    40,    42,   -38,    38,   -40,    39,   -25,    37,   -27,   -36,    21,   -30,   -18,   -17,    -7},

},

};