| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A106 |
| --- | --- |

| Title: | Description of video coding technology proposal: self derivation of motion estimation and adaptive (Wiener) loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171917 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution contains mainly two techniques, Self Derivation of Motion Estimation (SDME) and Adaptive Loop (Wiener) Filter (ALF), to be considered as the video coding tools to improve the coding efficiency for the incoming new generation of video compression standard. With the motion vector information is self derived at video decoder side from SDME, the transmission of motion vector from video encoder side to video decoder side is skipped and thus better coding efficiency is achieved. Compared to the anchor bitstreams for the test scenario of Constraints Set 1, an average 13.91% BD Bitrate reduction and 0.47dB BD PSNR improvement is achieved for the SDME technology and an average 18.53% BD Bitrate reduction and 0.77dB BD PSNR improvement is achieved for the combined case of SDME + ALF on top of the baseline average 7.18% BD Bitrate reduction and 0.30dB BD PSNR improvement observed in the baseline KTA software version 2.6r1. Compared to the anchor bitstreams for the test scenario of Constraints Set 2, an average 5.96% BD Bitrate reduction and 0.22dB BD PSNR improvement is achieved for the ALF technology on top of the baseline average 0.49% BD Bitrate reduction and 0.01dB BD PSNR improvement observed in the baseline KTA software version 2.6r1.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	Motion representation	4

2.2	Intra-frame prediction	4

2.3	Spatial transforms	4

2.4	Quantization	4

2.5	In-loop filtering	4

2.5.1	Introduction	4

2.5.2	Algorithm description	4

2.6	Entropy coding	7

2.7	Self derivation of motion estimation	7

2.7.1	Introduction	7

2.7.2	Algorithm description	7

3	Compression performance discussion	14

3.1	Objective versus subjective compression performance	14

3.2	Constraint set 1 configuration relative to Alpha anchor	15

3.2.1	Class A	15

3.2.2	Class B	15

3.2.3	Class C	15

3.2.4	Class D	15

3.2.5	Overall	16

3.3	Constraint set 2 configuration relative to Beta and Gamma anchors	19

3.3.1	Class B	19

3.3.2	Class C	19

3.3.3	Class D	19

3.3.4	Class E	19

3.3.5	Overall	19

4	Complexity analysis	21

4.1	Encoding time and measurement methodology	21

4.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0	23

4.3	Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2	24

4.4	Expected memory usage of encoder	25

4.5	Expected memory usage of decoder	25

4.6	Complexity characteristics of encoder motion estimation and motion segmentation selection	25

4.7	Complexity characteristics of decoder motion compensation	25

4.8	Complexity characteristics of encoder intra-frame prediction type selection	25

4.9	Complexity characteristics of decoder intra-frame prediction operation	25

4.10	Complexity characteristics of encoder transforms and transform type selection	26

4.11	Complexity characteristics of decoder inverse transform operation	26

4.12	Complexity characteristics of encoder quantization and quantization type selection	26

4.13	Complexity characteristics of decoder inverse quantization	26

4.14	Complexity characteristics of encoder in-loop filtering type selection	26

4.15	Complexity characteristics of decoder in-loop filtering operation	26

4.16	Complexity characteristics of encoder entropy coding type selection	26

4.17	Complexity characteristics of decoder entropy decoding operation	26

4.18	Degree of capability for encoder parallel processing	26

4.19	Degree of capability for decoder parallel processing	26

5	Algorithmic characteristics	26

5.1	Random access characteristics	26

5.2	Delay characteristics	27

6	Software implementation description	27

7	Highlighted aspects discussion	27

8	Closing remarks	27

9	Patent rights declaration(s)	28

# Introduction

This contribution is to respond to the Joint Call for Proposals on Video Compression Technology issued by ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 Q.6 (VCEG) on January 2010 [1]. Mainly two techniques are presented in this contribution to be considered as the video coding tools to improve the coding efficiency: Self Derivation of Motion Estimation (SDME), and Adaptive Loop (Wiener) Filter (ALF).

# Algorithm description

## Motion representation

This section does not apply. A related topic of motion estimation tool about self derivation of motion estimation can be found in section 2.7.

## Intra-frame prediction

This section does not apply.

## Spatial transforms

This section does not apply.

## Quantization

This section does not apply.

## In-loop filtering

### Introduction

Wiener filter is the well-known optimal linear filter to cope with the pictures degraded by the Gaussian noise, blurring and distortion. In [2], the utilization of the Wiener filter has been proposed to look for interpolation filter coefficients at the half/quarter pixel level used for the modules of ME/MC to better predict the picture in order to improve the coding efficiency on top of H.264/MPEG-4 Part 10 AVC system. In [3], the Wiener filter was used as the post filter hints to be applied out-of-loop of the core coding loop to improve the picture quality. In [4,5], two proposals to utilize the in-loop adaptive (Wiener) filter on top of the de-blocked picture to produce the improved picture is used as the reference pictures for the later incoming pictures. A quadtree-based technique to encode the chosen blocks were presented in [6] to achieve improved coding efficiency, and the further improvement to explore the adaptive quadtree-based structure, time-delayed filter coefficient were presented in [7]. In the next sections, the key concept of ALF is introduced.

### Algorithm description

Figure 1 illustrates the addition of the module of the adaptive filter in between the modules of the deblocking filter and of motion compensation/estimation to improve the quality of the deblocked picture as well as to be served as the improved reference picture for the phase of the motion estimation of the next encoding pictures.

Figure 1: The flow of the adaptive loop filter for the typical H.264/MPEG-4 Part 10 AVC based video encoder and video decoder (top: video encoder, bottom: video decoder).

Filter Coefficient derivation

Consider the input pixel xk and the output of the Wiener filter zk consisting of the reconstructed mapped pixel yi in the filter support {S}, sized as L+1, with the weight ci. The adaptive filter formula is

zk=i∈Syi⋅ci		2-1

The Wiener filter is optimal by minimizing the residual signal errork= zk−xkwith the filter taps {ci}

ci=argminE[errork2]		2-2

Where E[] is the expectation of the square of the residual signal for the pixels inside a picture.

The autocorrelation function of {y} is denoted as the below Equation 2-3 and the cross-correlation function among {y} and {x} is denoted as below Equation 2-4.

ryy(i)=E[yyk+i]		2-3

rxy(i)=E[xyk+i]		2-4

The equation to minimize the residual error can be written in Equation 2-5 in the matrix form

ryy(0) ryy(1) ⋯ ryy(L) ryy(1) ryy(0) ⋯ ryy(L−1) ⋮ ⋮ ⋱ ⋮	2-5

The Wiener filter tap set {C} can be derived in the matrix format

Ryy⋅ C=Rxy => C=Ryy−1⋅Rxy		2-6

Where Ryy-1 is the inverse of the auto-correlation matrix consisting of the auto-correlation element in Equation 2-3.

The Equations 2-3 and 2-4 can be expressively indexed in the 2D format in the below Equations 2-7 and 2-8 for a non-separable filter with the size L+1=(2l+1)*(2l+1)

ryy(m,n)=j=0j=height−1i=0i=width−1yi,j⋅yi+m,j+n≈j=lj=height−l−1i=li=width−l−1yi,j⋅yi+m,j+n	2-7

rxy(m,n)=j=0j=height−1i=0i=width−1xi,j⋅yi+m,j+n≈j=lj=height−l−1i=li=width−l−1xi,j⋅yi+m,j+n	2-8

Where m, n are in the range of (-l, l).

Filter side information

The filter side information includes the filter type (symmetric), filter size (5x5, 7x7, 9x9, or else) and filter coefficient quantization bit. The symmetric filter could benefit the RD performance for low resolution video sequences such as QCIF and CIF. Similarly, using the adaptive filter size and the adaptive filter coefficient quantization bit can balance the distortion andRcoef for the content of different spatial resolutions. The decoder requires these filter side information for the proper operation of decoding. The filter side information can be set in sequence level and transmitted through sequence parameter set (SPS).

Filter coefficient prediction coding

The predictive coding can be used to reduce the bit counts in the transfer of Wiener filter coefficients. The prediction modes include temporal prediction and spatial prediction. The direct mode and the temporal mode could be used when the symmetric filter is selected on the filter side.

Filter coefficient entropy coding

The Exp-Golomb coder could be applied in coding the prediction error of the Wiener filter coefficient for the case of high filter coefficient quantization bit (8 bit by default).

Quadtree coding of the filter blocks

In addition to apply the ALF operation pixel to pixel, ALF can be applied in the unit of the block to achieve better coding efficiency. The block size could be 8x8. The RD criterion can be applied to determine if a block to be filtered or not. The pattern of the blocks to be filtered can be efficiently represented by the quadtree order block-based structure in [6]

The BD Bitrate and BD PSNR experiment result of ALF here is demonstrated based on the QALF implementation in KTA software version 2.6r1.

## Entropy coding

This section does not apply.

## Self derivation of motion estimation

### Introduction

In the video coding standards such as H.264/MPEG-4 Part 10 AVC, an MB can be partitioned into smaller blocks for encoding, e.g., 4x4, and the motion vector can be assigned to each sub-partitioned block. These smaller block sizes lead to much bandwidth to transmit motion vector information from a video encoder to a video decoder. To save the bandwidth for motion vector information, techniques to derive motion vectors at the video decoder side was presented in [8] about a technique by replacing the original B-Skip mode with a new mirror-based motion search operation to derive a motion vector. This motion vector is derived among the previously decoded pictures and can be fully operating at video decoder side.

In this proposal, the work of self derivation of motion estimation is further explored to design a new SDME coding mode with the merit of extending block size to increase the prediction accuracy, of including the neighboring reconstructed pixels in the current picture into cost metric to reduce the prediction error and of an adaptive motion search range technique to achieve better coding efficiency with the increase computational complexity of both video encoder and video decoder. The detail of the algorithm description follows as below.

### Algorithm description

System Flow

Figure 2 illustrates a typical H.264/MPEG-4 Part 10 AVC video encoder flow with the module of the self derivation of motion estimation (SDME). Figure 3 illustrates the corresponding flow in the video decoder. The identical module of the SDME locates at both video encoder and video decoder where the video encoder needs to synchronize the SDME operation same as the SDME operation at the video decoder side to ensure there is no error propagation at the video decoder side for the long video sequence.

Figure 2: The flow of a typical H.264/MPEG-4 Part 10 AVC video encoder system with the module of self derivation of motion estimation.

Figure 3: The flow of a typical H.264/MPEG-4 Part 10 AVC video decoder system with the module of self derivation of motion estimation.

B-Picture Motion Estimation

Motion estimation aims to improve video coding efficiency by removing or reducing the temporal redundancy among the video frames. The motion estimation can be forward, backward or bi-directional, where backward and bi-directional motion estimation are only used in B picture coding while forward motion estimation can be used for both P and B pictures. Figure 4 shows the ME for B frame where MV0 is forward motion vector obtained by forward motion search within the search window in forward reference picture, and MV1 is the backward motion vector obtained by backward motion search within the search window in backward reference picture. In forward motion estimation, the reference block pointed by MV0 is used as the prediction of current block from forward reference picture. In backward motion estimation, the reference block pointed by MV1 is used as the prediction of current block from backward reference picture. And in bi-directional motion estimation, the average of the two reference blocks pointed by MV0 and MV1 is used as the prediction of current block from both reference pictures.

Figure 4: Motion estimation for B picture

The motion of an object or the background in the consecutive pictures is usually modeled as a smooth trajectory, i.e., the motion in consecutive pictures have a very strong temporal correlations. Capitalized on this temporal correlation, a motion vector for the target block could be self derived at the decode-side on the reconstructed reference pictures instead of performing the traditional encoder side motion estimation. With the motion vector is derived at video decoder side locally, the bits for encoding the motion vectors can be saved to improve the coding efficiency.

Mirror Motion Estimation

Mirror motion estimation has been explored on SDME to predict the motion vector among forward and backward reference pictures. Figure 5 illustrates how mirror motion estimation is performed for the scenario of two B pictures between forward and backward reference picture. Consider B0 as the current encoding picture. When encoding a target block in B0, the SDME can be described as follows.

- Specify a search window in the forward reference picture.

- Specify a search pattern in the forward search window in B0. Full search or simplified search patterns can be the options to select and the same search pattern will apply on both video encoder side and video decoder side.

- For the MV0 in the forward search window, the mirror motion vector MV1 in the backward search window is derived in Equation 2-9 based on the temporal picture distance where d0 is the distance between the current picture and the forward reference picture and d1 is the distance among the current picture and the forward reference picture.

MV1=−d1d0MV0									2-9

- Calculate the cost metric of motion search SAD between the reference block (pointed by MV0) in the forward reference picture and the reference block (pointed by MV1) in the backward reference picture.

- The SDME motion vector is selected as the MV0 candidate with the minimum SAD value in spiral order of all candidates in the search pattern.

Figure 5: Mirror ME on decoder side

Mirror ME compensation

With the mirror ME, a pair of motion vectors MV0 and MV1 is derived. The forward prediction pixel in forward reference pictureR0, denoted asR0(MV0), can be found in the collocated current block location displaced by MV0 in the forward reference picture. The backward prediction pixel in backward reference pictureR1, denoted asR1(MV1), can be found in the collocated current block location displaced by MV1 in the backward reference picture. The bi-directional prediction of SDME could be

- the average ofR0(MV0)andR1(MV1)

- Or, the weighted average [R0(MV0)* d1 +R1(MV1)* d0 + (d0+d1)/2] / (d0 + d1).

SDME Block Partition

Multiple block partitions can be available for SDME. The encoder and decoder should adopt the same partition pattern through the coding syntax agreed at both sides. The block partitions of 16x16, 16x8, 8x16and 8x8 have been applied to the bi-prediction coding mode, and the 8x8 is in use only in the direct_8x8 coding mode.

Currently, the SDME technique is applied to the following traditional coding modes with a flag control bit in use to signal if SDME or the traditional H.264/MPEG-4 Part 10 AVC method is applied to derive the motion vector):

B_Skip, B_Direct_16x16, B_Bi_16x16

B_L0_Bi_16x8, B_L0_Bi_8x16, B_Bi_L0_16x8, B_Bi_L0_8x16,

B_L1_Bi_16x8, B_L1_Bi_8x16, B_Bi_L1_16x8, B_Bi_L1_8x16,

B_Bi_Bi_16x8, B_Bi_Bi_8x16.

B_Direct_8x8 (Use SDME directly for Direct_8x8. No flag bit is needed)

Extended block Size in the reference blocks

The cost of motion search metric is the sum of absolute difference (SAD) between the two mirror blocks in the two reference pictures. Consider the block size of M×Nand the motion vectorMV0=(mv,mv) in the forward search window and the motion vectorMV1=(mv,mv) in the backward reference window. The cost of motion search J is as followed.

J=J0=j=0N−1i=0M−1|R0(x+mv+i,y+mv+j)−R1(x+mv+i,y+mv+j)|	2-10

The cost of motion search metric SAD can be improved by extending the block size for the scenario of SDME. Consider the block size of M×Nand the block is extended on the four borders with the sizes ofW0,W1,H0andH1, the pictorial illustration of the block is depicted in Figure 6.

Figure 6: Reference block and the extension area

The cost of motion search J1 is defined as followed.

J1=j=−H0N+H1−1i=−W0M+W1−1|R0(x+mv+i,y+mv+j)−R1(x+mv+i,y+mv+j)|−J0 2-11

The cost of motion searchJwith the extended block size in the reference block is defined as below.

J=J0+α1J1											2-12

α1is a constant (=1 by default). W0,W1, H0 and H1could be constants. (=8 by default)

Extended block Size to include the reconstructed neighbors in the current picture

The cost of motion search metric SAD can be further improved for the scenario of SDME by extending the block size to include the neighbor pixels which has been reconstructed according to the decoding order in raster scan order. Consider the block size of M×Nwith the neighboring area extended with the sizes of WR,WLandHT.

Figure 7: The current block with the neighboring blocks

There are four possible available spatial neighboring blocks reconstructed in the raster scan order: leftA0, topA1, top-leftA2top-rightA3. The availability of the neighboring blocks is conditioned on the location of the target block to be on the picture border. The availability flags for the neighboring blocks are denoted asγ0,γ1,γ2andγ3. An area is available if the flag is equal to 1; otherwise the area is not available (equal to 0). An available area is denoted as Aavailfor the definition as followed.

Aavail=γ0A0+γ1A1+γ2A2+γ3A3								2-13

The cost of motion search J2 is defined as followed.

J2=(x,y)∈Aavail|C(x,y)−(ω0R0(x+mv,y+mv)+ω1R1(x+mv,y+mv))|	2-14

WhereC(x,y)are the pixels in the reconstructed neighboring blocks of the current picture,ω0 andω1are two weighting factor to be set according to the temporal picture distance or to be simply set to be 0.5

The cost of motion searchJwith the extended block size in the reference block is defined as below.

J=J0+α1J1+α2J2										2-15

α1andα2are constants (=1 by default). WR,WLandHTcould be constants (=8 by default).

Motion Search Pattern

Motion search pattern is identical on both the encoder and decoder sides to ensure the synchronization of the operation at both sides. Two integer pixel motion searches are first performed, followed by a fractional pixel motion search. In the integer pixel motion search, a spiral order full search is performed around two search centers: (0, 0) of current target block location and the predicted motion vector (PMV) location obtained based on H.264/MPEG-4 Part 10 AVC protocol. An adaptive search range or a fixed search range can be applied to the search process around the search center to achieve better coding efficiency. A best integer motion vector candidate is obtained with the motion vector candidate of the minimal cost metric among many search candidates of these two integer motion searches. Then, a spiral order full search is performed in quarter pixel precision around the best integer motion vector candidate to work out the final quarter-based motion vector.

Adaptive Search Range

The motion search range of SDME can be an agreed area concurred by both video encoder and video decoder, and there is no requirement to set the search ranges in horizontal and vertical direction to be identical. The size of the motion search range is usually a tradeoff of computational complexity and coding efficiency. An improvement is observed to use the average of the size of all of the motion vector of previous pictures collected from the video decoder at picture level to achieve better coding efficiency with the adaptive search range which may vary picture to picture.

New coding mode Syntax

SDME can be defined as a new MB coding mode to work with the existing coding modes. A control flag is needed to signal this SDME mode in the bitstream to synchronize the operation of video encoder and video decoder. The below Tables 1-3 illustrate the syntax which SDME is realized in the framework of H.264/MPEG-4 Part 10 AVC.

Table 1 - Slice data syntax

| slice_data( ) { | C | Descriptor |
| --- | --- | --- |
| if( entropy_coding_mode_flag ) |  |  |
| while( !byte_aligned( ) ) |  |  |
| cabac_alignment_one_bit | 2 | f(1) |
| CurrMbAddr = first_mb_in_slice * ( 1 + MbaffFrameFlag ) |  |  |
| moreDataFlag = 1 |  |  |
| prevMbSkipped = 0 |  |  |
| do { |  |  |
| if( slice_type != I && slice_type != SI ) |  |  |
| If( !entropy_coding_mode_flag ) { |  |  |
| mb_skip_run | 2 | ue(v) |
| prevMbSkipped = ( mb_skip_run > 0 ) |  |  |
| for( i=0; i<mb_skip_run; i++ ) |  |  |
| CurrMbAddr = NextMbAddress( CurrMbAddr ) |  |  |
| moreDataFlag = more_rbsp_data( ) |  |  |
| } else { |  |  |
| mb_skip_flag | 2 | ae(v) |
| if( mb_skip_flag && slice_type == B) |  |  |
| bskip_sdme_flag | 2 | ae(v) |
| moreDataFlag = !mb_skip_flag |  |  |
| } |  |  |
| … |  |  |
| } while( moreDataFlag ) |  |  |
| } |  |  |

Table 2 - Macroblock layer syntax

| macroblock_layer( ) { | C | Descriptor |
| --- | --- | --- |
| mb_type | 2 | ue(v) ¦ ae(v) |
| if( mb_type = = B_Direct_16x16 ¦¦ mb_type = = B_Bi_16x16 ¦¦ mb_type = = B_L_Bi_16x8 ¦¦ mb_type = = B_L_Bi_8x16 ¦¦ mb_type = = B_Bi_L_16x8 ¦¦ mb_type = = B_Bi_L_8x16 ¦¦ mb_type = = B_Bi_Bi_16x8 ¦¦ mb_type = = B_Bi_Bi_8x16) |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| If(MbPartPredMode ( mb_type, mbPartIdx ) == BiPred) |  |  |
| mb_part_sdme_flag[mbPartIdx] | 2 | ae(v) |
| if( mb_type = = I_PCM ) { |  |  |
| … |  |  |
| } else { |  |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_4x4 && MbPartPredMode( mb_type, 0 ) != Intra_16x16 && NumMbPart( mb_type ) = = 4 ) |  |  |
| sub_mb_pred( mb_type ) | 2 |  |
| Else |  |  |
| mb_pred( mb_type ) | 2 |  |
| if( MbPartPredMode( mb_type, 0 ) != Intra_16x16 ) |  |  |
| coded_block_pattern | 2 | me(v) ¦ ae(v) |
| if( CodedBlockPatternLuma > 0 ¦ ¦ CodedBlockPatternChroma > 0 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| mb_qp_delta | 2 | se(v) ¦ ae(v) |
| residual( ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 3 - Macroblock prediction syntax

| mb_pred( mb_type ) { | C | Descriptor |
| --- | --- | --- |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ¦ ¦ MbPartPredMode( mb_type, 0 ) = = Intra_16x16 ) { |  |  |
| if( MbPartPredMode( mb_type, 0 ) = = Intra_4x4 ) |  |  |
| for( luma4x4BlkIdx=0; luma4x4BlkIdx<16; luma4x4BlkIdx++ ) { |  |  |
| prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] | 2 | u(1) ¦ ae(v) |
| if( !prev_intra4x4_pred_mode_flag[ luma4x4BlkIdx ] ) |  |  |
| rem_intra4x4_pred_mode[ luma4x4BlkIdx ] | 2 | u(3) ¦ ae(v) |
| } |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } else if( MbPartPredMode( mb_type, 0 ) != Direct ) { |  |  |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l0_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L1 && !mb_part_sdme_flag[mbPartIdx] ) |  |  |
| ref_idx_l0[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( ( num_ref_idx_l1_active_minus1 > 0 ¦ ¦ mb_field_decoding_flag ) && MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && !mb_part_sdme_flag[mbPartIdx]) |  |  |
| ref_idx_l1[ mbPartIdx ] | 2 | te(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| if( MbPartPredMode ( mb_type, mbPartIdx ) != Pred_L1 && !mb_part_sdme_flag[mbPartIdx] ) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l0[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| for( mbPartIdx = 0; mbPartIdx < NumMbPart( mb_type ); mbPartIdx++) |  |  |
| If( MbPartPredMode( mb_type, mbPartIdx ) != Pred_L0 && !mb_part_sdme_flag[mbPartIdx]) |  |  |
| for( compIdx = 0; compIdx < 2; compIdx++ ) |  |  |
| mvd_l1[ mbPartIdx ][ 0 ][ compIdx ] | 2 | se(v) ¦ ae(v) |
| } |  |  |
| } |  |  |

bskip_sdme_flag specifies if the current MB applies SDME to derive the MVs for the B_Skip mode.

- If bskip_sdme_flag is equal to 0, the conventional B_Skip MV derivation method is in use.

- Otherwise, the SDME is performed to derive the MVs for the B_Skip mode.

mb_part_sdme_flag [mbPartIdx] specifies if the partition mbPartIdx of the current MB applies SDME to derive the MVs for the Bi-prediction mode. When mb_part_sdme_flag [mbPartIdx] is not present, the value of mb_part_sdme_flag [mbPartIdx] shall be inferred to be equal to 0.

- If mb_part_sdme_flag is equal to 0, the information of motion vector and reference index for the Bi-prediction will be obtained from the bitstream.

- Otherwise, the information of motion vector and reference index for the Bi-prediction will be derived from SDME.

# Compression performance discussion

## Objective versus subjective compression performance

## Constraint set 1 configuration relative to Alpha anchor

The configuration of the test condition is same as the configuration files provided by the Alpha anchor for all Classes A - D. The description of test condition is provided in [1]. The detailed information about per-frame bit rate and PSNR is provided in the accompanied spread sheet. The SDME and ALF are implemented on top of KTA 2.6r1, and the assessment metrics of BD Bitrate and BD PSNR can be referred in [9, 10].

### Class A

For the tests in the Class A, Table 4 lists that the baseline average 3.74% BD Bitrate reduction and 0.16dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the anchor bitstream for the test scenario of Constraint Set 1 in [1]. Table 5 lists an average 14.32% BD Bitrate reduction and 0.65dB BD PSNR improvement is achieved for the SDME technique in KTA 2.6r1. Table 6 lists an average 9.51% BD Bitrate reduction and 0.44dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 7 lists an average 19.63% BD Bitrate reduction and 0.93dB BD PSNR improvement is achieved for the combined case of SDME + ALF in KTA 2.6r1. Table 8 lists an average 20.03% BD Bitrate reduction and 0.95dB BD PSNR improvement is achieved for the combined case of SDME + ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class B

For the tests in the Class B, Table 4 lists that the baseline average 3.21% BD Bitrate reduction and 0.10dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the anchor bitstream for the test scenario of Constraint Set 1 in [1]. Table 5 lists an average 11.33% BD Bitrate reduction and 0.36dB BD PSNR improvement is achieved for the SDME technique in KTA 2.6r1. Table 6 lists an average 9.45% BD Bitrate reduction and 0.27dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 7 lists an average 17.26% BD Bitrate reduction and 0.54dB BD PSNR improvement is achieved for the combined case of SDME + ALF in KTA 2.6r1. Table 8 lists an average 18.32% BD Bitrate reduction and 0.57dB BD PSNR improvement is achieved for the combined case of SDME + ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class C

For the tests in the Class C, Table 4 lists that the baseline average 10.14% BD Bitrate reduction and 0.44dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the anchor bitstream for the test scenario of Constraint Set 1 in [1]. Table 5 lists an average 16.88% BD Bitrate reduction and 0.75dB BD PSNR improvement is achieved for the SDME technique in KTA 2.6r1. Table 6 lists an average 13.24% BD Bitrate reduction and 0.58dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 7 lists an average 20.04% BD Bitrate reduction and 0.91dB BD PSNR improvement is achieved for the combined case of SDME + ALF in KTA 2.6r1. Table 8 lists an average 20.52% BD Bitrate reduction and 0.93dB BD PSNR improvement is achieved for the combined case of SDME + ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class D

For the tests in the Class D, Table 4 lists that the baseline average 10.91% BD Bitrate reduction and 0.50dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the anchor bitstream for the test scenario of Constraint Set 1 in [1]. Table 5 lists an average 13.94% BD Bitrate reduction and 0.64dB BD PSNR improvement is achieved for the SDME technique in KTA 2.6r1. Table 6 lists an average 14.83% BD Bitrate reduction and 0.68dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 7 lists an average 18.08% BD Bitrate reduction and 0.84dB BD PSNR improvement is achieved for the combined case of SDME + ALF in KTA 2.6r1. Table 8 lists an average 18.46% BD Bitrate reduction and 0.86dB BD PSNR improvement is achieved for the combined case of SDME + ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Overall

Table 4 lists that the baseline average 7.18% BD Bitrate reduction and 0.30dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the anchor bitstream for the test scenario of Constraint set 1 in [1]. Table 5 lists an average 13.91% BD Bitrate reduction and 0.47dB BD PSNR improvement is achieved for the SDME technique in KTA 2.6r1. Table 6 lists an average 11.90% BD Bitrate reduction and 0.49dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 7 lists an average 18.53% BD Bitrate reduction and 0.77dB BD PSNR improvement is achieved for the combined case of SDME + ALF in KTA 2.6r1. Table 8 lists an average 19.17% BD Bitrate reduction and 0.80dB BD PSNR improvement is achieved for the combined case of SDME + ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

Table 4 - Coding gains of default KTA2.6r1 compared to alpha anchor bitstreams for constraint set 1

|  | Sequence | KTA2.6r1 Original |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR(Full) | BD_Bitrate(Full) | BD_PSNR(High) | BD_Bitrate(High) | BD_PSNR(Low) | BD_Bitrate(Low) |
| Class A | Traffic | 0.1719 | -4.4891 | 0.1748 | -5.0052 | 0.1718 | -3.9512 |
|  | PeopleOnStreet | 0.1566 | -2.9893 | 0.1377 | -2.6904 | 0.1802 | -3.3935 |
| Class B | Kimono1 | 0.1535 | -4.3257 | 0.1165 | -3.7698 | 0.1866 | -4.5951 |
|  | ParkScene | 0.076 | -2.0744 | 0.0631 | -1.7367 | 0.0883 | -2.3142 |
|  | Cactus | 0.1551 | -5.1667 | 0.1176 | -4.8498 | 0.1921 | -5.4043 |
|  | BasketballDrive | 0.0634 | -2.0686 | 0.0393 | -1.5347 | 0.0857 | -2.3086 |
|  | BQTerrace | 0.0411 | -2.3958 | 0.0418 | -3.409 | 0.0405 | -1.8283 |
| Class C | BasketballDrill | 0.3839 | -8.9794 | 0.3882 | -9.2601 | 0.3768 | -8.4388 |
|  | BQMall | 0.4463 | -9.1431 | 0.4571 | -9.8336 | 0.4266 | -8.0455 |
|  | PartyScene | 0.4261 | -11.1459 | 0.402 | -10.025 | 0.4575 | -13.0783 |
|  | RaceHorses | 0.4886 | -11.3027 | 0.4955 | -11.3387 | 0.4738 | -10.932 |
| Class D | BasketballPass | 0.5159 | -9.9616 | 0.5062 | -9.2365 | 0.5333 | -11.3205 |
|  | BQSquare | 0.4968 | -12.9274 | 0.5368 | -13.6754 | 0.4326 | -11.33 |
|  | BlowingBubbles | 0.4957 | -11.4414 | 0.5419 | -12.0928 | 0.4085 | -10.0581 |
|  | RaceHorses | 0.4876 | -9.3059 | 0.4753 | -8.6297 | 0.508 | -10.5245 |
| Average of Class A |  | 0.16425 | -3.7392 | 0.15625 | -3.8478 | 0.176 | -3.67235 |
| Average of Class B |  | 0.0978 | -3.2062 | 0.0757 | -3.0600 | 0.1186 | -3.2901 |
| Average of Class C |  | 0.4362 | -10.1428 | 0.4357 | -10.1144 | 0.4337 | -10.1237 |
| Average of Class D |  | 0.4990 | -10.9091 | 0.5151 | -10.9086 | 0.4706 | -10.8083 |
| Overall Average |  | 0.3039 | -7.1811 | 0.2996 | -7.1392 | 0.3042 | -7.1682 |

Table 5: Coding gains of SDME compared to alpha anchor bitstreams for constraint set 1

|  | Sequence | Original KTA+SDME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class A | Traffic | 0.4177 | -11.3203 | 0.3044 | -8.8136 | 0.5821 | -13.9747 |
|  | PeopleOnStreet | 0.888 | -17.3157 | 0.621 | -12.681 | 1.341 | -23.8056 |
| Class B | Kimono1 | 0.4887 | -13.3946 | 0.2862 | -9.2733 | 0.6816 | -16.3816 |
|  | ParkScene | 0.3661 | -9.5914 | 0.2183 | -5.9801 | 0.5086 | -13.0406 |
|  | Cactus | 0.4275 | -13.8728 | 0.2703 | -11.0105 | 0.5835 | -15.9671 |
|  | BasketballDrive | 0.3447 | -10.6504 | 0.1797 | -6.75 | 0.505 | -13.3564 |
|  | BQTerrace | 0.1621 | -9.1164 | 0.1062 | -8.4632 | 0.2188 | -9.673 |
| Class C | BasketballDrill | 0.5591 | -12.9818 | 0.4983 | -11.8754 | 0.6415 | -14.3079 |
|  | BQMall | 0.987 | -19.3371 | 0.824 | -17.4464 | 1.2104 | -22.1295 |
|  | PartyScene | 0.7364 | -18.7045 | 0.6664 | -16.4565 | 0.8215 | -22.4713 |
|  | RaceHorses | 0.7265 | -16.5147 | 0.636 | -14.484 | 0.8471 | -19.4258 |
| Class D | BasketballPass | 0.6943 | -13.2709 | 0.6293 | -11.4354 | 0.7936 | -16.761 |
|  | BQSquare | 0.6414 | -16.5879 | 0.6353 | -16.0683 | 0.6522 | -17.1643 |
|  | BlowingBubbles | 0.6229 | -14.3041 | 0.6431 | -14.3247 | 0.5795 | -14.0991 |
|  | RaceHorses | 0.6119 | -11.612 | 0.5514 | -9.9827 | 0.7061 | -14.6532 |
| Average of Class A |  | 0.65285 | -14.318 | 0.4627 | -10.7473 | 0.96155 | -18.89015 |
| Average of Class B |  | 0.3578 | -11.3251 | 0.2121 | -8.2954 | 0.4995 | -13.6837 |
| Average of Class C |  | 0.7523 | -16.8845 | 0.6562 | -15.0656 | 0.8801 | -19.5836 |
| Average of Class D |  | 0.6426 | -13.9437 | 0.6148 | -12.9528 | 0.6829 | -15.6694 |
| Overall Average |  | 0.5783 | -13.9050 | 0.4713 | -11.6697 | 0.7115 | -16.4807 |

Table 6 - Coding gains of ALF to alpha anchor bitstreams for constraint set 1

|  | Sequence | Original KTA+ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class A | Traffic | 0.3894 | -9.8459 | 0.3844 | -10.7444 | 0.3993 | -8.8749 |
|  | PeopleOnStreet | 0.5004 | -9.1793 | 0.5074 | -9.4897 | 0.4804 | -8.7161 |
| Class B | Kimono1 | 0.3956 | -10.7638 | 0.3671 | -11.7106 | 0.4197 | -9.9636 |
|  | ParkScene | 0.1977 | -5.2688 | 0.1877 | -5.1217 | 0.2072 | -5.347 |
|  | Cactus | 0.2756 | -8.9823 | 0.2485 | -10.0215 | 0.3019 | -8.3126 |
|  | BasketballDrive | 0.213 | -6.5623 | 0.1877 | -7.0445 | 0.2363 | -6.1648 |
|  | BQTerrace | 0.2893 | -15.6561 | 0.259 | -18.8975 | 0.3216 | -14.0437 |
| Class C | BasketballDrill | 0.5692 | -13.0065 | 0.5869 | -13.6782 | 0.5415 | -11.7751 |
|  | BQMall | 0.5823 | -11.7239 | 0.6069 | -12.8284 | 0.5415 | -10.0666 |
|  | PartyScene | 0.582 | -14.5956 | 0.6016 | -14.3948 | 0.5544 | -15.3479 |
|  | RaceHorses | 0.5971 | -13.6222 | 0.6083 | -13.7346 | 0.5757 | -13.0891 |
| Class D | BasketballPass | 0.6162 | -11.7592 | 0.623 | -11.2119 | 0.6031 | -12.6647 |
|  | BQSquare | 0.9034 | -22.4431 | 0.9584 | -23.3167 | 0.8203 | -20.0951 |
|  | BlowingBubbles | 0.6549 | -14.6329 | 0.7426 | -16.0618 | 0.5022 | -12.0678 |
|  | RaceHorses | 0.5536 | -10.4744 | 0.5521 | -9.9396 | 0.5563 | -11.4401 |
| Average of Class A |  | 0.4449 | -9.5126 | 0.4459 | -10.11705 | 0.43985 | -8.7955 |
| Average of Class B |  | 0.2742 | -9.4467 | 0.2500 | -10.5592 | 0.2973 | -8.7663 |
| Average of Class C |  | 0.5827 | -13.2371 | 0.6009 | -13.6590 | 0.5533 | -12.5697 |
| Average of Class D |  | 0.6820 | -14.8274 | 0.7190 | -15.1325 | 0.6205 | -14.0669 |
| Overall Average |  | 0.4880 | -11.9011 | 0.4948 | -12.5464 | 0.4708 | -11.1979 |

Table 7 - Coding gains of SDME+ALF to alpha anchor bitstreams for constraint set 1

|  | Sequence | Original KTA +SDME+ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class A | Traffic | 0.6444 | -16.6905 | 0.5258 | -14.6552 | 0.8152 | -18.898 |
|  | PeopleOnStreet | 1.2198 | -22.5648 | 0.9759 | -19.038 | 1.6287 | -27.6414 |
| Class B | Kimono1 | 0.7378 | -19.6881 | 0.5525 | -17.5189 | 0.9149 | -21.3475 |
|  | ParkScene | 0.4855 | -12.5878 | 0.3435 | -9.3201 | 0.6227 | -15.7607 |
|  | Cactus | 0.5522 | -17.5892 | 0.401 | -15.9719 | 0.7019 | -18.8758 |
|  | BasketballDrive | 0.4923 | -14.8318 | 0.3288 | -12.1056 | 0.6507 | -16.8394 |
|  | BQTerrace | 0.4074 | -21.5905 | 0.3236 | -23.2844 | 0.4922 | -21.0586 |
| Class C | BasketballDrill | 0.7371 | -16.7473 | 0.691 | -16.0642 | 0.7983 | -17.3634 |
|  | BQMall | 1.1394 | -22.0042 | 0.992 | -20.6214 | 1.3403 | -24.1586 |
|  | PartyScene | 0.934 | -22.7223 | 0.9083 | -21.4464 | 0.9573 | -25.1524 |
|  | RaceHorses | 0.8298 | -18.6792 | 0.7568 | -17.0327 | 0.9247 | -20.8486 |
| Class D | BasketballPass | 0.7909 | -14.9639 | 0.7432 | -13.3268 | 0.863 | -17.9887 |
|  | BQSquare | 1.0961 | -26.951 | 1.0797 | -26.0891 | 1.1216 | -27.3891 |
|  | BlowingBubbles | 0.7966 | -17.7313 | 0.8528 | -18.411 | 0.6945 | -16.4758 |
|  | RaceHorses | 0.6729 | -12.6599 | 0.6224 | -11.1764 | 0.7545 | -15.537 |
| Average of Class A |  | 0.9321 | -19.62765 | 0.75085 | -16.8466 | 1.22195 | -23.2697 |
| Average of Class B |  | 0.5350 | -17.2575 | 0.3899 | -15.6402 | 0.6765 | -18.7764 |
| Average of Class C |  | 0.9101 | -20.0383 | 0.8370 | -18.7912 | 1.0052 | -21.8808 |
| Average of Class D |  | 0.8391 | -18.0765 | 0.8245 | -17.2508 | 0.8584 | -19.3477 |
| Overall Average |  | 0.7691 | -18.5335 | 0.6732 | -17.0708 | 0.8854 | -20.3557 |

Table 8 - Coding gains of SDME+ALF+CodingTools to alpha anchor bitstreams for constraint set 1

|  | Sequence | Original KTA+KTATools+ALF+SDME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class A | Traffic | 0.6785 | -17.3837 | 0.5691 | -15.7194 | 0.8351 | -19.2316 |
|  | PeopleOnStreet | 1.2287 | -22.6805 | 0.9885 | -19.2414 | 1.6305 | -27.636 |
| Class B | Kimono1 | 0.7741 | -20.5587 | 0.5942 | -18.7145 | 0.9468 | -21.9848 |
|  | ParkScene | 0.5141 | -13.2735 | 0.3778 | -10.1795 | 0.6454 | -16.2786 |
|  | Cactus | 0.5731 | -18.1964 | 0.4216 | -16.7214 | 0.7238 | -19.3876 |
|  | BasketballDrive | 0.5394 | -16.1852 | 0.3762 | -13.747 | 0.6976 | -17.9592 |
|  | BQTerrace | 0.4464 | -23.3645 | 0.3564 | -25.3386 | 0.537 | -22.6157 |
| Class C | BasketballDrill | 0.7546 | -17.0915 | 0.722 | -16.7338 | 0.7975 | -17.2659 |
|  | BQMall | 1.1707 | -22.5261 | 1.0285 | -21.2593 | 1.3651 | -24.5099 |
|  | PartyScene | 0.9509 | -23.1103 | 0.9268 | -21.8189 | 0.9724 | -25.5543 |
|  | RaceHorses | 0.8631 | -19.3436 | 0.7945 | -17.8063 | 0.9511 | -21.4118 |
| Class D | BasketballPass | 0.8059 | -15.2013 | 0.7647 | -13.6787 | 0.8726 | -18.1336 |
|  | BQSquare | 1.1251 | -27.5481 | 1.1097 | -26.7708 | 1.1465 | -27.9255 |
|  | BlowingBubbles | 0.8176 | -18.1341 | 0.8776 | -18.8646 | 0.7101 | -16.7891 |
|  | RaceHorses | 0.6916 | -12.9866 | 0.6446 | -11.5303 | 0.7656 | -15.7389 |
| Average of Class A |  | 0.9536 | -20.0321 | 0.7788 | -17.4804 | 1.2328 | -23.4338 |
| Average of Class B |  | 0.5694 | -18.3157 | 0.4252 | -16.9402 | 0.7101 | -19.6452 |
| Average of Class C |  | 0.9348 | -20.5179 | 0.8680 | -19.4046 | 1.0215 | -22.1855 |
| Average of Class D |  | 0.8601 | -18.4675 | 0.8492 | -17.7111 | 0.8737 | -19.6468 |
| Overall Average |  | 0.7956 | -19.1723 | 0.7035 | -17.8750 | 0.9065 | -20.8282 |

## Constraint set 2 configuration relative to Beta and Gamma anchors

The configuration of the test condition is same as the configuration files provided by the Beta anchor for all Classes B - E. The description of test condition is provided in [1]. The detailed information about per-frame bit rate and PSNR is provided in the accompanied spread sheet. The ALF is implemented on top of KTA 2.6r1, and the assessment metrics of BD Bitrate and BD PSNR can be referred in [9, 10].

### Class B

For the tests in Class B, Table 9 lists that the baseline average 2.71% BD Bitrate reduction and 0.10dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the beta bitstreams for the test scenario of constraint Set 2 in [1]. Table 10 lists an average 9.37% BD Bitrate reduction and 0.30dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 11 lists an average 10.71% BD Bitrate reduction and 0.34dB BD PSNR improvement is achieved for the combined case of ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class C

For the tests in Class C, Table 9 lists that the baseline average 0.15% BD Bitrate reduction and 0.01dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the beta bitstreams for the test scenario of constraint Set 2 in [1]. Table 10 lists an average 4.86% BD Bitrate reduction and 0.20dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 11 lists an average 5.42% BD Bitrate reduction and 0.23dB BD PSNR improvement is achieved for the combined case of ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class D

For the tests in Class D, Table 9 lists that the baseline average 1.44% BD Bitrate increase and 0.07dB BD PSNR decrement is observed in the baseline KTA software version 2.6r1 compared to the beta bitstreams for the test scenario of constraint Set 2 in [1]. Table 10 lists an average 0.89% BD Bitrate reduction and 0.03dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 11 lists an average 2.32% BD Bitrate reduction and 0.09dB BD PSNR improvement is achieved for the combined case of ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Class E

For the tests in Class E, Table 9 lists that the baseline average 0.15% BD Bitrate increase and 0.01dB BD PSNR decrement is observed in the baseline KTA software version 2.6r1 compared to the beta bitstreams for the test scenario of constraint Set 2 in [1]. Table 10 lists an average 8.50% BD Bitrate reduction and 0.35dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 11 lists an average 9.47% BD Bitrate reduction and 0.39dB BD PSNR improvement is achieved for the combined case of ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

### Overall

Table 9 lists that the baseline average 0.50% BD Bitrate reduction and 0.01dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1 compared to the beta bitstreams for the test scenario of constraint Set 2 in [1]. Table 10 lists an average 5.96% BD Bitrate reduction and 0.22dB BD PSNR improvement is achieved for the ALF technique in KTA 2.6r1. Table 11 lists an average 7.06% BD Bitrate reduction and 0.26dB BD PSNR improvement is achieved for the combined case of ALF + available KTA tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] in KTA 2.6r1.

Table 9 - Coding gains of default KTA 2.6r1 compared to beta anchor bitstreams for constraint set 2

|  | Sequence | KTA2.6r1 Original |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class B | Kimono1 | 0.1859 | -5.1628 | 0.1471 | -4.3821 | 0.2183 | -5.364 |
|  | ParkScene | 0.1415 | -3.9121 | 0.0932 | -2.5888 | 0.1864 | -5.0696 |
|  | Cactus | 0.0341 | -1.0145 | 0.026 | -0.9692 | 0.0429 | -1.2301 |
|  | BasketballDrive | 0.0971 | -2.5973 | 0.0517 | -1.6645 | 0.1393 | -3.3959 |
|  | BQTerrace | 0.0193 | -0.8613 | 0.0283 | -1.8393 | 0.0098 | -0.363 |
| Class C | BasketballDrill | -0.0087 | 0.2023 | -0.0745 | 1.9287 | 0.0706 | -1.7942 |
|  | BQMall | -0.0196 | 0.502 | -0.0624 | 1.3664 | 0.0349 | -0.6147 |
|  | PartyScene | 0.1091 | -2.5474 | 0.0387 | -0.8212 | 0.2155 | -6.0404 |
|  | RaceHorses | -0.0473 | 1.2403 | -0.113 | 2.9729 | 0.0439 | -1.1955 |
| Class D | BasketballPass | -0.0754 | 1.6795 | -0.1253 | 2.5325 | 0.0115 | -0.2882 |
|  | BQSquare | 0.0866 | -2.5293 | 0.0878 | -2.5033 | 0.0417 | -1.3144 |
|  | BlowingBubbles | -0.0706 | 1.7559 | -0.1406 | 3.4485 | -0.008 | 0.1918 |
|  | RaceHorses | -0.2139 | 4.8569 | -0.2025 | 4.0562 | -0.1646 | 3.8316 |
| Class E | Vidyo1 | -0.0181 | 0.667 | -0.0666 | 1.8586 | 0.0384 | -0.7652 |
|  | Vidyo3 | -0.0973 | 2.2768 | -0.1446 | 3.9161 | 0.0057 | -0.1242 |
|  | Vidyo4 | 0.0818 | -2.4816 | 0.0669 | -2.3219 | 0.1507 | -3.1208 |
| Average of Class B |  | 0.0956 | -2.7096 | 0.0693 | -2.2888 | 0.1193 | -3.0845 |
| Average of Class C |  | 0.0084 | -0.1507 | -0.0528 | 1.3617 | 0.0912 | -2.4112 |
| Average of Class D |  | -0.0683 | 1.4408 | -0.0952 | 1.8835 | -0.0299 | 0.6052 |
| Average of Class E |  | -0.0112 | 0.1541 | -0.0481 | 1.1509 | 0.0649 | -1.3367 |
| Overall Average |  | 0.0128 | -0.4954 | -0.0244 | 0.3119 | 0.0648 | -1.6661 |

Table 10 - Coding gains of ALF to beta anchor bitstreams for constraint set 2

|  | Sequence | Original+ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class B | Kimono1 | 0.4457 | -11.432 | 0.4315 | -12.032 | 0.4546 | -10.7437 |
|  | ParkScene | 0.2639 | -7.1562 | 0.2094 | -5.7574 | 0.315 | -8.429 |
|  | Cactus | 0.1347 | -4.1742 | 0.1281 | -4.6526 | 0.1421 | -3.952 |
|  | BasketballDrive | 0.3055 | -8.3649 | 0.2468 | -8.035 | 0.3606 | -8.6289 |
|  | BQTerrace | 0.3411 | -15.7396 | 0.3032 | -18.4457 | 0.3787 | -14.5502 |
| Class C | BasketballDrill | 0.2696 | -6.7156 | 0.2043 | -5.3522 | 0.3452 | -8.395 |
|  | BQMall | 0.1661 | -3.3899 | 0.1297 | -2.8357 | 0.2138 | -4.1299 |
|  | PartyScene | 0.2306 | -5.7343 | 0.161 | -3.9736 | 0.3376 | -9.3707 |
|  | RaceHorses | 0.1413 | -3.6098 | 0.1318 | -3.3048 | 0.1522 | -3.9883 |
| Class D | BasketballPass | 0.0639 | -1.2337 | 0.0374 | -0.7162 | 0.1178 | -2.7845 |
|  | BQSquare | 0.1855 | -5.3585 | 0.1695 | -4.865 | 0.1717 | -5.1743 |
|  | BlowingBubbles | -0.0167 | 0.4081 | -0.085 | 2.0869 | 0.0462 | -1.1762 |
|  | RaceHorses | -0.1102 | 2.6195 | -0.0737 | 1.4549 | -0.0907 | 1.98 |
| Class E | Vidyo1 | 0.3703 | -8.3258 | 0.29 | -7.8868 | 0.4358 | -8.7854 |
|  | Vidyo3 | 0.34 | -8.2852 | 0.2658 | -7.321 | 0.4841 | -9.7958 |
|  | Vidyo4 | 0.3373 | -8.8866 | 0.3086 | -9.433 | 0.4331 | -8.8202 |
| Average of Class B |  | 0.2982 | -9.3734 | 0.2638 | -9.7845 | 0.3302 | -9.2608 |
| Average of Class C |  | 0.2019 | -4.8624 | 0.1567 | -3.8666 | 0.2622 | -6.4710 |
| Average of Class D |  | 0.0306 | -0.8912 | 0.0121 | -0.5099 | 0.0613 | -1.7888 |
| Average of Class E |  | 0.3492 | -8.4992 | 0.2881 | -8.2136 | 0.4510 | -9.1338 |
| Overall Average |  | 0.2168 | -5.9612 | 0.1787 | -5.6918 | 0.2686 | -6.6715 |

Table 11 - Coding gains of ALF+CodingTools to beta anchor bitstreams for constraint set 2

|  | Sequence | Original+KTATools+ALF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (Full) | BD_Bitrate (Full) | BD_PSNR (High) | BD_Bitrate (High) | BD_PSNR (Low) | BD_Bitrate (Low) |
| Class B | Kimono1 | 0.4872 | -12.3459 | 0.4842 | -13.3383 | 0.4854 | -11.3542 |
|  | ParkScene | 0.2963 | -8.0121 | 0.2424 | -6.6543 | 0.3464 | -9.2214 |
|  | Cactus | 0.1682 | -5.212 | 0.1629 | -5.8772 | 0.1742 | -4.8376 |
|  | BasketballDrive | 0.3648 | -9.8805 | 0.3145 | -10.1469 | 0.4106 | -9.7017 |
|  | BQTerrace | 0.3966 | -18.0887 | 0.3504 | -21.1355 | 0.4429 | -16.74 |
| Class C | BasketballDrill | 0.311 | -7.7577 | 0.2589 | -6.7684 | 0.3702 | -8.9576 |
|  | BQMall | 0.211 | -4.3193 | 0.1885 | -4.1184 | 0.2386 | -4.6027 |
|  | PartyScene | 0.2259 | -5.5438 | 0.1587 | -3.9066 | 0.3292 | -9.1167 |
|  | RaceHorses | 0.1594 | -4.049 | 0.16 | -3.9912 | 0.1564 | -4.0764 |
| Class D | BasketballPass | 0.0786 | -1.5702 | 0.0557 | -1.0762 | 0.1236 | -2.9173 |
|  | BQSquare | 0.3478 | -9.5153 | 0.3797 | -10.5009 | 0.2437 | -7.0714 |
|  | BlowingBubbles | 0.0198 | -0.5113 | -0.0295 | 0.7389 | 0.0513 | -1.3276 |
|  | RaceHorses | -0.096 | 2.3004 | -0.0588 | 1.16 | -0.0838 | 1.8368 |
| Class E | Vidyo1 | 0.4166 | -9.3551 | 0.3409 | -9.1813 | 0.476 | -9.5289 |
|  | Vidyo3 | 0.374 | -8.9751 | 0.3046 | -8.2728 | 0.4993 | -10.1356 |
|  | Vidyo4 | 0.3881 | -10.0792 | 0.3691 | -11.0442 | 0.4712 | -9.5011 |
| Average of Class B |  | 0.3426 | -10.7078 | 0.3109 | -11.4304 | 0.3719 | -10.3710 |
| Average of Class C |  | 0.2268 | -5.4175 | 0.1915 | -4.6962 | 0.2736 | -6.6884 |
| Average of Class D |  | 0.0876 | -2.3241 | 0.0868 | -2.4196 | 0.0837 | -2.3699 |
| Average of Class E |  | 0.3929 | -9.4698 | 0.3382 | -9.4994 | 0.4822 | -9.7219 |
| Overall Average |  | 0.2593 | -7.0572 | 0.2301 | -7.1321 | 0.2960 | -7.3283 |

# Complexity analysis

## Encoding time and measurement methodology

To measure the encoding time increase caused by ALF and SDME, the simulation to encode the bitstreams were going through the test cases in [1] on the modified KTA2.6r1 encoder. The portion of the encoding time spent on ALF and SDME were extracted from the total encoding time through the ftime() function in the C/C++ program.

- ALF: Table 12 reports an average 6.692% increase due to ALF in video encoding time compared to the KTA baseline with KTA Tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] enabled under constraint set 1. Table 13 reports an average 11.539% increase due to ALF in video encoding time compared to the KTA baseline with KTA Tools enabled under constraint set 2.

- SDME: Table 14 reports an average 118.508% increase in video encoding time due to SDME compared to the KTA baseline with KTA Tools enabled under constraint set 1. Fast SDME algorithms are under investigation to reduce the motion estimation complexity.

Table 12 - Encoding time increase by ALF for constraint set 1

|  | Encoding Time Increase of ALF (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 6.288 | 6.098 | 5.930 | 5.413 | 4.805 | 5.707 |
| ParkScene | 6.234 | 6.141 | 5.837 | 5.538 | 5.350 | 5.820 |
| Cactus | 7.227 | 6.990 | 6.429 | 5.936 | 5.588 | 6.434 |
| BasketballDrive | 6.025 | 5.967 | 5.846 | 5.562 | 5.144 | 5.709 |
| BQTerrace | 6.795 | 6.725 | 6.269 | 6.006 | 5.643 | 6.288 |
| BasketballDrill | 8.067 | 7.881 | 7.322 | 6.182 | 6.143 | 7.119 |
| BQMall | 7.218 | 6.679 | 6.176 | 5.520 | 5.296 | 6.178 |
| PartyScene | 7.513 | 7.102 | 6.303 | 5.881 | 5.596 | 6.479 |
| RaceHorses | 6.776 | 6.498 | 6.151 | 5.541 | 5.403 | 6.074 |
| BasketballPass | 8.134 | 8.083 | 7.574 | 7.050 | 6.337 | 7.436 |
| BQSquare | 9.521 | 9.007 | 8.112 | 7.415 | 7.115 | 8.234 |
| BlowingBubbles | 9.069 | 8.984 | 8.339 | 7.756 | 7.221 | 8.274 |
| RaceHorses | 7.140 | 7.630 | 7.645 | 7.208 | 6.590 | 7.243 |
| Average | 7.385 | 7.214 | 6.764 | 6.231 | 5.864 | 6.692 |

Table 13 - Encoding time increase by ALF for constraint set 2

|  | Encoding Time Increase of ALF (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 12.846 | 12.288 | 11.985 | 11.262 | 11.151 | 11.906 |
| ParkScene | 11.428 | 11.038 | 10.688 | 10.249 | 10.528 | 10.786 |
| Cactus | 12.576 | 11.000 | 11.584 | 11.063 | 10.819 | 11.408 |
| BasketballDrive | 11.864 | 11.640 | 11.083 | 10.815 | 10.535 | 11.187 |
| BQTerrace | 13.214 | 12.999 | 12.475 | 11.932 | 11.454 | 12.415 |
| BasketballDrill | 13.167 | 12.462 | 11.762 | 10.717 | 10.308 | 11.683 |
| BQMall | 10.633 | 10.119 | 9.851 | 9.819 | 9.629 | 10.010 |
| PartyScene | 10.941 | 10.256 | 9.654 | 9.606 | 9.690 | 10.029 |
| RaceHorses | 10.037 | 9.725 | 9.743 | 9.373 | 9.569 | 9.689 |
| BasketballPass | 12.536 | 12.079 | 11.507 | 11.282 | 11.069 | 11.695 |
| BQSquare | 14.987 | 15.396 | 15.370 | 14.234 | 13.522 | 14.702 |
| BlowingBubbles | 10.974 | 11.205 | 11.196 | 10.857 | 10.915 | 11.029 |
| RaceHorses | 11.657 | 11.828 | 11.165 | 11.353 | 10.979 | 11.396 |
| Vidyo1 | 14.633 | 13.179 | 11.865 | 10.985 | 10.520 | 12.236 |
| Vidyo3 | 15.452 | 13.992 | 11.922 | 11.292 | 10.406 | 12.613 |
| Vidyo4 | 14.477 | 12.790 | 11.003 | 10.763 | 10.150 | 11.837 |
| Average | 12.589 | 12.000 | 11.428 | 10.975 | 10.703 | 11.539 |

Table 14 - Encoding time increase by SDME for constraint set 1

|  | Encoding Time Increase of ALF & SDME (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 134.627 | 133.628 | 131.114 | 127.534 | 122.666 | 129.914 |
| ParkScene | 98.636 | 100.350 | 99.113 | 96.061 | 95.991 | 98.030 |
| Cactus | 97.422 | 94.764 | 86.836 | 80.257 | 77.552 | 87.366 |
| BasketballDrive | 189.043 | 188.999 | 186.901 | 177.570 | 165.766 | 181.656 |
| BQTerrace | 125.795 | 125.271 | 123.457 | 119.676 | 116.331 | 122.106 |
| BasketballDrill | 124.120 | 121.877 | 115.236 | 104.326 | 100.987 | 113.309 |
| BQMall | 129.644 | 123.158 | 117.131 | 112.960 | 110.620 | 118.703 |
| PartyScene | 89.920 | 88.438 | 85.390 | 85.236 | 86.082 | 87.013 |
| RaceHorses | 182.956 | 180.838 | 177.986 | 169.720 | 171.804 | 176.661 |
| BasketballPass | 126.838 | 130.893 | 130.500 | 126.817 | 119.937 | 126.997 |
| BQSquare | 85.128 | 84.359 | 83.481 | 82.039 | 80.514 | 83.104 |
| BlowingBubbles | 85.044 | 90.814 | 87.877 | 85.943 | 83.522 | 86.640 |
| RaceHorses | 118.805 | 129.137 | 135.087 | 132.835 | 129.669 | 129.107 |
| Average | 122.152 | 122.502 | 120.008 | 115.460 | 112.419 | 118.508 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0

To measure the decoding time increase caused by ALF and SDME, instead of directly comparing the amount of the time spending on decoding the anchor bitstreams on JM17.0, the simulation to decode the bitstreams were going through the test cases in [1] on the modified KTA2.6r1 software decoder. The portion of the decoding time spent on ALF and SDME were extracted from the total decoding time through the ftime() function in the program.

- ALF: Table 15 reports an average 14.999% increase due to ALF in video decoding time compared to the KTA baseline with KTA Tools [Adaptive Interpolation Filter (AIF) and High Precision Filter (HPF)] enabled under constraint set 1. Table 16 reports an average 13.446% increase due to ALF in video decoding time compared to the KTA baseline with KTA Tools enabled under constraint set 2.

- SDME: Table 17 reports an average 550.55% increase in video decoding time due to SDME compared to the KTA baseline with KTA Tools enabled under constraint set 1. Fast SDME algorithms are under investigation to reduce the motion estimation complexity.

Table 15 - Decoding time increase by ALF for constraint set 1

|  | Decoding Time Increase of ALF (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 21.407 | 21.421 | 21.937 | 23.632 | 24.435 | 22.566 |
| ParkScene | 16.962 | 19.674 | 20.257 | 20.024 | 19.645 | 19.312 |
| Cactus | 13.490 | 13.133 | 11.102 | 11.345 | 9.855 | 11.785 |
| BasketballDrive | 17.790 | 19.831 | 20.511 | 20.163 | 18.201 | 19.299 |
| BQTerrace | 19.116 | 20.528 | 19.316 | 19.557 | 18.737 | 19.451 |
| BasketballDrill | 8.986 | 13.435 | 11.070 | 8.189 | 6.338 | 9.604 |
| BQMall | 14.802 | 18.356 | 15.516 | 11.077 | 8.848 | 13.720 |
| PartyScene | 14.668 | 19.071 | 20.082 | 18.208 | 14.801 | 17.366 |
| RaceHorses | 12.888 | 12.841 | 9.876 | 9.598 | 8.822 | 10.805 |
| BasketballPass | 8.816 | 14.642 | 13.179 | 7.512 | 7.972 | 10.424 |
| BQSquare | 15.610 | 17.777 | 17.325 | 19.640 | 21.512 | 18.373 |
| BlowingBubbles | 12.968 | 16.492 | 16.434 | 9.159 | 8.387 | 12.688 |
| RaceHorses | 9.138 | 12.311 | 8.825 | 10.338 | 7.381 | 9.599 |
| Average | 14.357 | 16.886 | 15.802 | 14.496 | 13.456 | 14.999 |

Table 16 Decoding time increase by ALF under constraint set 2

|  | Decoding Time Increase of ALF (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 17.331 | 18.113 | 18.777 | 23.511 | 27.786 | 21.104 |
| ParkScene | 16.242 | 18.044 | 18.532 | 18.382 | 19.647 | 18.169 |
| Cactus | 13.480 | 12.483 | 11.797 | 11.750 | 11.092 | 12.120 |
| BasketballDrive | 14.617 | 15.344 | 17.002 | 18.450 | 19.303 | 16.943 |
| BQTerrace | 16.011 | 16.683 | 14.950 | 14.390 | 14.409 | 15.289 |
| BasketballDrill | 11.275 | 14.266 | 16.023 | 16.285 | 13.113 | 14.192 |
| BQMall | 11.712 | 14.638 | 16.595 | 16.433 | 11.779 | 14.231 |
| PartyScene | 6.611 | 7.262 | 7.245 | 7.248 | 7.140 | 7.101 |
| RaceHorses | 15.017 | 15.651 | 18.128 | 19.969 | 20.722 | 17.897 |
| BasketballPass | 10.218 | 12.807 | 16.160 | 17.004 | 15.908 | 14.419 |
| BQSquare | 2.592 | 3.275 | 4.566 | 4.275 | 8.840 | 4.710 |
| BlowingBubbles | 3.174 | 5.055 | 5.412 | 3.325 | 2.937 | 3.981 |
| RaceHorses | 12.647 | 17.321 | 17.722 | 20.501 | 22.687 | 18.176 |
| Vidyo1 | 16.745 | 14.505 | 11.227 | 6.539 | 4.193 | 10.642 |
| Vidyo3 | 13.650 | 14.382 | 14.016 | 12.770 | 10.793 | 13.122 |
| Vidyo4 | 15.164 | 15.836 | 14.832 | 12.723 | 6.679 | 13.047 |
| Average | 12.280 | 13.479 | 13.937 | 13.972 | 13.564 | 13.446 |

Table 17 - Decoding time increase by SDME under constraint set 1

|  | Decoding Time Increase of SDME (%) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bitrate_1 | bitrate_2 | bitrate_3 | bitrate_4 | bitrate_5 | Average |
| Kimono1 | 861.107 | 993.747 | 1009.068 | 998.923 | 851.656 | 942.900 |
| ParkScene | 485.018 | 578.245 | 614.093 | 563.729 | 504.774 | 549.172 |
| Cactus | 887.181 | 979.819 | 862.023 | 834.693 | 758.395 | 864.422 |
| BasketballDrive | 965.594 | 1059.686 | 1137.162 | 1202.798 | 1075.621 | 1088.172 |
| BQTerrace | 416.505 | 438.689 | 390.506 | 357.596 | 333.903 | 387.440 |
| BasketballDrill | 280.315 | 356.945 | 264.600 | 185.600 | 138.021 | 245.096 |
| BQMall | 465.514 | 461.843 | 397.861 | 321.709 | 262.157 | 381.817 |
| PartyScene | 339.439 | 389.798 | 358.516 | 291.692 | 247.379 | 325.365 |
| RaceHorses | 664.277 | 845.683 | 826.089 | 785.391 | 693.576 | 763.003 |
| BasketballPass | 326.802 | 430.411 | 450.741 | 441.258 | 405.878 | 411.018 |
| BQSquare | 242.413 | 262.204 | 236.884 | 227.173 | 232.162 | 240.167 |
| BlowingBubbles | 240.236 | 286.697 | 259.138 | 223.595 | 201.206 | 242.174 |
| RaceHorses | 543.906 | 730.292 | 743.775 | 815.125 | 748.947 | 716.409 |
| Average | 516.793 | 601.081 | 580.804 | 557.637 | 496.437 | 550.550 |

## Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

The configurations of the software computing platform used to determine the encoding and decoding time for the test scenario of constraint set 1 and constraint set 2 are listed in Table 18.

Table 18 - Computing platform configurations for encoding/decoding time determination

|  | Constraint set 1 | Constraint set 2 |
| --- | --- | --- |
| CPU | 2 Intel Xeon CPU@ 3.16GHz (8 cores) | 2 Intel Core CPU@ 3.19GHz (8 cores) |
| Memory | 16GB RAM | 8GB RAM |
| OS | WinXP Professional x64 Edition SP2 | WinXP Professional x64 Edition SP2 |
| Encoding | Run 5 tasks simultaneously | Run 5 tasks simultaneously |
| Decoding | Run 5 tasks simultaneously | Run 5 tasks simultaneously |

## Expected memory usage of encoder

- ALF: There is no expected extra picture level data storage required for video encoder. There could be potentially one extra pass of memory access for the reconstructed pixels of current picture and the input pixels of current picture to derive the filter coefficients of ALF based on the prediction of current picture. This extra pass of memory access can be neglected for the scenario of deriving the filter coefficients of ALF based on the prediction of previously decoded pictures.

- SDME: There is no expected extra picture level data storage required for video encoder. There will be one pass of memory access for the reconstructed pixels in the reference picture to derive the motion vector at the video decoder loop of video encoder side.

## Expected memory usage of decoder

- ALF: There is no expected extra picture level data storage required for video encoder. Depending on the implementation architecture, there could be potentially one extra pass of memory access for the reconstructed pixels to work out the filtered pixels.

- SDME: There is no expected extra picture level data storage required for video encoder. There will be one pass of memory access for the reconstructed pixels in the reference picture to derive the motion vector at the video decoder side. A note to mark that in some software implementation there could exist extra memory buffer to store the tentative interpolated reference pictures.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

- SDME: The complexity of encoder motion estimation is expected to increase with one more round of motion estimation executed at the video decoder loop at video encoder side. The complexity of this extra motion estimation could be reduced with the agreed protocol among video encoder and video decoder. The complexity measurement of SDME encoding presented in this proposal is based on encoding time of CPU software implementation for the reference purpose. A note to mark that the actual complexity of SDME realization will weight differently, most likely dependent of the implementation platform such as hardware fixed function or GPU specific accelerator and the execution unit.

## Complexity characteristics of decoder motion compensation

- SDME: The complexity of encoder motion estimation is expected to increase with one more round of motion estimation executed at the video decoder side. The complexity of this extra motion estimation could be reduced with the agreed protocol among video encoder and video decoder. The complexity measurement of SDME decoding presented in this proposal is based on decoding time of CPU software implementation for the reference purpose. A note to mark that the actual complexity of SDME realization will weight differently, most likely dependent of the implementation platform such as hardware fixed function or GPU specific accelerator and the execution unit.

## Complexity characteristics of encoder intra-frame prediction type selection

This section does not apply.

## Complexity characteristics of decoder intra-frame prediction operation

This section does not apply.

## Complexity characteristics of encoder transforms and transform type selection

This section does not apply.

## Complexity characteristics of decoder inverse transform operation

This section does not apply.

## Complexity characteristics of encoder quantization and quantization type selection

This section does not apply.

## Complexity characteristics of decoder inverse quantization

This section does not apply.

## Complexity characteristics of encoder in-loop filtering type selection

The complexity of video encoder will increase due to the derivation of filter coefficients required among the reconstructed pixels and the current pixels.

## Complexity characteristics of decoder in-loop filtering operation

The complexity of video decoder will increase due to the filter operation executed among the reconstructed pixels to work out the final target pixel.

## Complexity characteristics of encoder entropy coding type selection

This section does not apply.

## Complexity characteristics of decoder entropy decoding operation

This section does not apply.

## Degree of capability for encoder parallel processing

ALF: the collection of the auto-correlation and cross-correlation function could be accumulated based on per block calculation given there is no data dependency.

SDME: the current result is based on the adaptive motion search, and this doesn’t prevent the parallel-friendly motion estimation scheme to be applied in this merit.

## Degree of capability for decoder parallel processing

SDME: the current result is based on the adaptive motion search, and this doesn’t prevent the parallel-friendly motion estimation scheme to be applied in this merit.

# Algorithmic characteristics

## Random access characteristics

- ALF: It has no inherent constraint to use intra prediction type of picture (I-picture) coding.

- SDME: It can apply inter prediction type of picture (both P-picture and B-picture) coding. Therefore, there is no specific requirement to modify GOP structure, and the simulation is based on the Anchor compliant to Alpha and Beta test configurations in the call for proposal.

## Delay characteristics

- ALF: there will be an additional pass on the video encoder side to optimize the coding efficiency by deriving the filter coefficients based on the result of current reconstructed picture. On the other hand, this additional pass can be removed by utilizing on the filter coefficients predicted based on the previously decoded pictures. There is no extra pass required on the video decoder side.

- SDME: There is no extra pass required on both video encoder side and video decoder side.

# Software implementation description

The SW implementation is written in C/C++, based on the ITU-T VCEG KTA software version 2.6r1.

The additional coding options in KTA have been enabled:

- Adaptive Interpolation Filter (UseAdaptiveFilter = 1) (This AIF mode is disabled for B frames when the SDME mode is in use)

- High Precision Filter (UseHPFilter = 1)

- Adaptive Loop Filter (UseAdaptiveLoopFilter = 1)

# Highlighted aspects discussion

The section does not apply.

# Closing remarks

The techniques of self derivation of motion estimation (SDME) and adaptive (Wiener) loop filter (ALF) to improve video coding efficiency are presented in this proposal. Compared to the anchor bitstreams for the test scenario of Constraints Set 1, an average 13.91% BD Bitrate reduction and 0.47dB BD PSNR improvement is achieved for the SDME technology and an average 18.53% BD Bitrate reduction and 0.77dB BD PSNR improvement is achieved for the combined case of SDME + ALF on top of the baseline average 7.18% BD Bitrate reduction and 0.30dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1. Compared to the anchor bitstreams for the test scenario of Constraints Set 2, an average 5.96% BD Bitrate reduction and 0.22dB BD PSNR improvement is achieved for the ALF technology on top of the baseline average 0.49% BD Bitrate reduction and 0.01dB BD PSNR improvement is observed in the baseline KTA software version 2.6r1. With the evidence of coding efficiency improvement demonstrated from common test scenario, this proposal recommends JCT-VC committee to support the core experiment groups on SDME and ALF to cross study the techniques for the consideration to include them as the core video coding tools for the incoming new generation of video compression standard.

Reference

[1]	Joint Call for Proposals on Video Compression Technology, ITU-T Q6/16 & ISO/IEC JTC1/SC29WG11, January 2010.

[2]	Y. Vatis, B. Edler, I. Wassermann, D. T. Nguyen and J. Ostermann, “Coding of Coefficients of two-dimensional non-separable Adaptive Interpolation Filter”, Visual Communications and Image Processing (VCIP), Beijing, China, July 2005.

[3]	S. Wittmann and T. Wedi, “SEI message on post-filter hints”, JVT document JVT-U035, Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 & ITU-T SG16 Q.6, Hangzhou, China, October, 2006.

[4]	Y. Chiu, L. Xu, “Adaptive (Wiener) Filter for Video Compression”, ITU-T Q.6/SG16 Doc., VCEG-C437, Geneva, 22 April - 2 May 2008.

[5]	T. Chujoh, A. Tanizawa, T. Yamakage, “Adaptive Loop Filter for Improving Coding Efficiency”, ITU-T Q.6/SG16, VCEG-C402, Geneva, 22 April - 2 May 2008.

[6]	T. Chujoh, N. Wada, and G. Yasuda, “Quadtree-based adaptive loop filter”, ITU-T SG16 Document, C181, Geneva, Jan. 2009.

[7]	Y. Huang, C. Fu, X Guo, S. Lei, “Improved Quadtree-based Adaptive Loop Filter”, ITU-T SG16/Q6 , London, UK, July 2009, VCEG-AL24.

[8]	T. Murakami, and S. Saito, "Advanced B Skip Mode with Decoder-side Motion Estimation", ITU-T Q.6/SG16 VCEG, VCEG-AK12, Yokohama, Japan, April 2009.

[9]	Gisle Bjontegaard, "Calculation of Average PSNR Differences between RD curves", ITU-T SG16/Q6, Austin, Texas, USA, April, 2001, VCEG-M33.

[10]	Gisle Bjontegaard, "Improvements of the BD-PSNR model", ITU-T SG16/Q6, Berlin, Germany, July, 2008, VCEG-AI11.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).