| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A108 |
| --- | --- |

| Title: | Description of video coding technology proposal by JVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Satoru Sakazume JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Motoharu Ueda JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Hiroya Namamura JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Kazumi Arakage JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan Toru Kumakura JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: Tel: Fax: Email: | +81-46-836-6579 +81-46-836-9430 sakazume.satoru@jk-holdings.com +81-46-836-9945 +81-46-836-9430 ueda.motoharu@jk-holdings.com +81-46-836-6597 +81-46-836-9430 fukushima.shigeru@jk-holdings.com +81-46-836-6459 +81-46-836-9430 nakamura.hiroya@jk-holdings.com +81-46-836-5749 +81-46-836-9430 someya.kazumi@jk-holdings.com +81-46-836-5798 +81-46-836-9430 kumakura.toru@jk-holdings.com |
| Source: | Victor Company of Japan, Limited. |  |  |

_____________________________

# Abstract

In order to improve the performance for the enhancement (resolution and quality) of the conventional applications, the video coding such as MPEG-4 AVC/H.264 (AVC) is advantageous to develop new applications. Therefore this contribution presents the tools which achieve the improvement intra-frame prediction and motion representation as the extension of AVC, and it is based on JM16.2 software.

In intra-frame prediction, “AC prediction using DC and intra prediction mode” is proposed. In motion representation, “Geometric Transform Prediction (GTP)”, “Decoder-side Block Boundary Decision Motion Compensation (DBBD)” and “Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)” are proposed. In addition to the above tools, QALF tool is applied into the proposed software.

This proposal achieves the bitrate reduction of average 9.27% and up to 24.09% for constraint set 1 to alpha anchor, and achieves the bitrate reduction of average 3.18% and up to 22.71% for constraint set 2 to beta anchor, and achieves the bitrate reduction of average 26.49% and up to 53.66% for constraint set 2 to gamma anchor.

# Contents

Abstract	1

Contents	3

1	Introduction	5

2	Algorithm description	6

2.1	Motion representation	6

2.1.1	Geometric Transform Prediction (GTP)	7

2.1.2	Decoder-side Block Boundary Decision Motion Compensation (DBBD)	10

2.1.3	Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)	13

2.2	Intra-frame prediction	16

2.2.1	AC prediction using DC and intra prediction mode	16

2.3	Spatial transforms	18

2.4	Quantization	18

2.5	In-loop filtering	18

2.6	Entropy coding	19

2.7	Encoder Settings	19

3	Compression performance discussion	20

3.1	Objective versus subjective compression performance	20

3.2	Constraint set 1 configuration relative to Alpha anchor	20

3.2.1	Class A	20

3.2.2	Class B	21

3.2.3	Class C	24

3.2.4	Class D	26

3.2.5	Overall	27

3.3	Constraint set 2 configuration relative to Beta and Gamma anchors	28

3.3.1	Class B	28

3.3.2	Class C	30

3.3.3	Class D	32

3.3.4	Class E	34

3.3.5	Overall	35

4	Complexity analysis	36

4.1	Encoding time and measurement methodology	36

4.1.1	Constraint set 1 (C1)	36

4.1.2	Constraint set 2 (C2)	37

4.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0	38

4.2.1	Constraint set 1 (C1)	38

4.2.2	Constraint set 2 (C2)	40

4.3	Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2	42

4.4	Expected memory usage of encoder	42

4.5	Expected memory usage of decoder	42

4.6	Complexity characteristics of encoder motion estimation and motion segmentation selection	43

4.7	Complexity characteristics of decoder motion compensation	44

4.8	Complexity characteristics of encoder intra-frame prediction type selection	44

4.9	Complexity characteristics of decoder intra-frame prediction operation	45

4.10	Complexity characteristics of encoder transforms and transform type selection	45

4.11	Complexity characteristics of decoder inverse transform operation	45

4.12	Complexity characteristics of encoder quantization and quantization type selection	45

4.13	Complexity characteristics of decoder inverse quantization	45

4.14	Complexity characteristics of encoder in-loop filtering type selection	46

4.15	Complexity characteristics of decoder in-loop filtering operation	46

4.16	Complexity characteristics of encoder entropy coding type selection	46

4.17	Complexity characteristics of decoder entropy decoding operation	46

4.18	Degree of capability for encoder parallel processing	46

4.19	Degree of capability for decoder parallel processing	46

5	Algorithmic characteristics	47

5.1	Random access characteristics	47

5.2	Delay characteristics	47

6	Software implementation description	47

7	Highlighted aspects discussion	48

8	Closing remarks	48

9	Patent rights declaration(s)	48

References	49

# Introduction

This document describes our contribution to the Call for Proposals (CfP) on High Performance Video Coding Technologies [1].

It is necessary to improve the coding performance for the enhancement (resolution and quality) of the conventional applications, referring “Vision, Applications and Requirements for High-Performance Video Coding” [2]. And the block-based video coding such as MPEG-4 AVC/H.264 (AVC) is advantageous to achieve or develop new applications because conventional AV systems are based on these block-based video coding.

Thus this proposal, which is formed as the extension of AVC, includes tools which achieve the improvement of intra-frame prediction and motion representation.

In intra-frame prediction, “AC prediction using DC and intra prediction mode” is proposed. The tool can predict the luminance shift along the direction derived by the intra prediction.

In motion representation, “Geometric Transform Prediction (GTP)”, “Decoder-side Block Boundary Decision Motion Compensation (DBBD)” and “Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)” are proposed. “GTP” can reduce geometrical distortion. “DBBD” can improve the quality of predictive picture by applying flexible partitioning. “RMC” can improve the efficiency of uni-predictive by using decoder-side motion estimation between reference pictures.

In addition to the above tools, proposed software includes QALF tool [3] that is adopted into the KTA software [4], and these tools are integrated into JM16.2-based software [5].

# Algorithm description

The conceptual diagram of JVC proposal is shown in Fig. 1. The basic structure is not difference from the　AVC, but the parts of intra-frame prediction and motion representation. In the part of intra-frame prediction, the tool “AC prediction using DC and intra prediction mode” is added into the proposed model. Both of this tool and the conventional AVC intra prediction tools are implemented and one of these is selected as best mode decided by the block-based mode decision in the encoder. In the part of motion representation, three new tools “GTP”, “DBBD”, “RMC” are added into the proposed software. These all tools and the conventional AVC motion representation tools are implemented and one of these is selected as best mode decided by the block-based mode decision in the encoder. And QALF tool that is ported from KTA software is implemented into the proposed software.

Fig. 1 Conceptual Diagram of JVC proposal (decoder)

## Motion representation

We introduce three proposed techniques for motion representation. The first one is “Geometric Transform Prediction (GTP)”, which can reduce geometrical distortion. The second one is “Decoder-side Block Boundary Decision Motion Compensation (DBBD)”, which can improve the quality of predictive picture by applying flexible partitioning. The last one is “Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)”, which can improve the efficiency of uni-predictive by using decoder-side motion estimation between reference pictures.

### Geometric Transform Prediction (GTP)

#### Motivation

An inter prediction is the one of the main features of conventional coding of moving pictures (such as MPEG-4 AVC/H.264, MPEG-2 Video/H.262, MPEG-4 Visual, etc.). The inter prediction performs motion compensation in a translation. Fig. 2 shows an example of the case of conventional inter prediction using a motion vector per macroblock.

Fig. 2 Conventional inter prediction: 1 motion vector

In addition, AVC can divide a macroblock into 16 blocks in the most and can perform inter prediction by each 4x4 block as shown in Fig. 3.

Fig. 3 Conventional inter prediction: 16 motion vectors

However, it is difficult to represent a rotation only in a translation. Similarly, it is difficult to represent a transformation (such as a scaling, a torsion, a twist or a warp) only in a translation.

Consequently, we integrated the algorithm of “Geometric Transform Prediction (GTP)” into the proposed software.

#### Details of Geometric Transform Prediction (GTP)

Geometric Transform Prediction (GTP) is an inter prediction using geometric transform.

GTP supports a transformation including a scaling, a rotation, etc. in addition to a translation. In conventional inter prediction, a block is uniformly predicted using motion vector corresponding to each of the block as shown in Fig. 2 or Fig. 3. On the other hand, in GTP, a pixel in a block is predicted using a motion vector corresponding to each of the pixel. However, all motion vectors are not coded, and most motion vectors are calculated as described below.

There are three modes in the GTP as follows.

- Geometric Transform Mode A:

Each motion vector corresponding to four representative pixels is coded/decoded.

Each motion vector corresponding to all the remaining pixels is interpolated in horizontal and vertical direction.

Fig. 4 Geometric Transform Mode A

- Geometric Transform Mode B:

Each motion vector corresponding to two representative pixels is coded/decoded.

Each motion vector corresponding to all the remaining pixels is interpolated in vertical direction, and is set to the same value in horizontal direction.

Fig. 5 Geometric Transform Mode B

- Geometric Transform Mode C:

Each motion vector corresponding to two representative pixels is coded/decoded.

Each motion vector corresponding to all the remaining pixels is interpolated in horizontal direction, and are set to the same value in vertical direction.

Fig. 6 Geometric Transform Mode C

The decoding process for a GTP block is as follows.

- The motion vectors for two to four representative pixels per macroblock are decoded.

- The motion vectors for all the remaining pixels are calculated from the motion vector of the representative pixels using a linear interpolation.

However, the motion vectors are set to the same value in vertical direction in the case of Geometric Transform Mode B. Similarly, the motion vectors are set to the same value in horizontal direction in the case of Geometric Transform Mode C.

The precision of the calculated motion vectors is 1/256 sample, though the precision of the coding motion vectors is one-quarter luma sample.

- Each pixel is predicted according to calculated motion vectors.

A linear interpolation is used for the pixel interpolation.

Fig. 7 Decoding process for a Geometric Transform Prediction (GTP)

### Decoder-side Block Boundary Decision Motion Compensation (DBBD)

#### Motivation

Fixed block patterns such as 16x16/16x8/8x16/8x8 can be used for motion compensation and a coding mode which indicates the block pattern is signaled as ‘mb_type’ in AVC. It can obtain high coding performance in the case that two or more different motions are included in a macroblock. However it is difficult to obtain high coding performance in the case of the prediction with a mismatch between a block boundary and the actual motion boundary as shown in Fig. 8. In order to minimize the influence of this problem, some techniques which increase the number of fixed block patterns compared to AVC have been already proposed. But it is not always effective to increase the number of fixed block patterns because the additional coding bits are required for indicating the coding mode. For this reason, it is desirable to support various block patterns for motion compensation without additional coding bits. Thus we propose the technique named “Decoder-side Block Boundary Decision Motion Compensation (DBBD)”.

Fig. 8  The problem of fixed block patterns

#### Details of DBBD

DBBD can perform motion compensation with various block patterns without additional coding bits because the block boundary is decided on the decoder side. The conceptual diagram of DBBD is shown in Fig. 9. DBBD achieves high coding performance to exploit the correlation between an actual motion boundary and an object boundary on predicted pictures.

Fig. 9  Conceptual diagram of DBBD

The points of DBBD implemented into our proposed software are as follows.

- 16x8 and 8x16 modes in AVC are replaced with 16xA and Ax16, respectively.

- ‘A’ is constrained to multiples of 4.

- It can be applied to bi-prediction in B-slice to perform the process of DBBD after generating bi-predictive picture of each block.

- Same syntax is used as AVC at macroblock level. (No more coding bits are required)

- Motion vector prediction of 16xA and Ax16 is the same as 16x8 and 8x16 in AVC, respectively.

- Each activity on candidate boundary is used to decide a real boundary.

- The order of motion vectors indicates the corresponding region of motion compensation.

The flowchart of DBBD is shown in Fig. 10.

Fig. 10  The flowchart of DBBD

The decoding process of DBBD is as follows.

- Generate two predicted pictures based on two motion vectors by the size of macroblock.

- Decide a real boundary to evaluate each candidate boundary on two predicted pictures.

- Get a final motion compensated picture using the real boundary.

On the other hand, the encoding process of DBBD is as follows.

- Decide a provisional boundary for motion estimation.

- Perform a motion estimation using the block divided by the provisional boundary.

- Simulate the decoding process of DBBD to decide a real boundary and evaluate the final motion compensated picture.

DBBD is very useful for the next generation video coding that is requested to support various block patterns to obtain high coding performance.

### Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)

#### Motivation

Motion Compensation can improve coding efficiency using temporal correlation between a target picture and a decoded reference picture, but it is more susceptible to coding distortion of a reference picture. Coding distortion of a reference picture provides unneeded component of the distortion and the loss of texture with the predicted signal, so it degrades the prediction efficiency.

This proposed technique “Refinement Motion Compensation using Decoder-side Motion Estimation (RMC)” aims to improve the quality of reference block of motion compensation using decoder-side motion estimation between reference pictures.

#### Details of RMC

Fig. 11 shows the conceptual diagram of RMC.

RMC extracts the first MC block using reference picture 1 at first (same as uni-predictive prediction). And it calculates inter-reference motion vector between the first MC block and reference picture 2. Finally, it generates the new MC block (RMC block) by the synthesis of the first MC block and the second MC block extracted from reference picture 2 using inter-reference motion vector.

As a result, it can smooth the distortion of the reference picture and supplement the partially missing texture component to the reference picture like bi-predictive prediction. RMC needs only one motion vector for transmitting, so it can reduce additional information compared with bi-predictive prediction.

Fig. 11 Conceptual diagram of RMC

Fig. 12 RMC in the case of P Slice

RMC is also effective in the case of small spatial or temporal correlation in motion, because it follows non-temporal correlation by Decoder-side Motion Estimation (not Direct-Mode).

RMC is applicable to not only B slice but also P Slice.

In the case of B Slice, reference MC block can leads other past and future reference pictures as shown Fig. 11. In the case of P Slice, past reference MC block leads other past reference pictures as shown Fig. 12.

In this proposal, “Two Step Motion Compensation” method is used for improving precision of RMC block without enormous calculation on the encoder side. “Two Step Motion Compensation” algorithm on the decoder is shown by Fig. 13.

Fig. 13 Two Step Motion Compensation in RMC

On the decoder side, at first it extracts the first MC block using motion vector “MVa_down” (1 pixel accuracy) which is generated by rounding off the accuracy of received motion vector “MVa”, and it generates “base RMC block” using inter-reference motion estimation between the first MC block and reference picture 2 (Step 1).

Next it calculates the phase shift vector “α” (1/4 pixel accuracy) from “MVa” and “MVa_down” and extracts “RMC block” by the shift “α” for “base RMC block”. And “RMC block” is used for prediction block (Step2).

On the encoder side, at first it gets 1 pixel accuracy base vector “MVa_down”, and generates “base RMC block” using inter-reference motion estimation (Step 1).

Next it evaluates the prediction error between target block and “base RMC block” inner half pixel area (it can degrade to same 1 pixel accuracy motion vector) by shifting the phase, and it estimates the best phase “α” (1/4 pixel accuracy) of “base RMC block” as “RMC block”. And “RMC block” is used for prediction block (Step2).

The encoder sends motion vector “MVa” as summation of “MVa_down” and “α” in the same method of uni-predictive prediction (difference from PMV), and the decoder can divide “MVa_down” and “α” from “MVa”.

For using “Two Step Motion Compensation”, the decoder can use non-filtered block as the first MC block for decoder-side motion estimation. And the encoder can get high precision RMC block by using only 1 pixel accuracy base vectors (not use many 1/4 pixel accuracy vectors as base vectors).

In the proposed program, decoding flow for one macroblock motion compensation using RMC is shown as Fig. 14.

Uni-predictive mode is replaced by RMC mode per slice or macroblock. The flag of selection method of uni-predictive mode and RMC mode is sent in Slice Header. It decides that uni-predictive mode is replaced by RMC mode always or that uni-predictive mode is replaced by RMC mode per macroblock adaptive.

In the case of macroblock adaptive, selection flag is transmitted after reference list index.

Fig. 14 Decoding flow for one macroblock motion compensation using RMC

On this proposal, reference picture 2 in the case of RMC mode is decided as follows. (see Fig. 15)

P Slice's case        reference picture 1 (base)    reference picture 2

Idx0                 -> Idx1

Other                -> Idx0

B Slice's case        reference picture 1 (base)    reference picture 2

LIST0                -> LIST1 Idx0

LIST1                -> LIST0 Idx0

Fig. 15 Relation of first reference and second reference on RMC

## Intra-frame prediction

We introduce a proposed technique for intra-frame prediction in this section. The technique is “AC prediction using DC and intra prediction mode”. The tool achieves predicting the luminance change of the pixels to the direction derived by the intra prediction.

### AC prediction using DC and intra prediction mode

#### Motivation

AVC-based intra prediction is a good technique, which can predict high frequencies with a few bits. However, the AVC-based intra predicted signal consists of a direction only and it has same value in the direction. So it is impossible to create predicted signal which has a value shift in the direction of prediction.

When the value of the signal to be coded has the monotonic shift in the direction of prediction, low AC coefficients appear with a DC coefficient, even though the proper intra prediction is selected. It prevents the improvement of coding efficiency because the levels of the low AC coefficients are not small. From this point of view, we propose the technique which predicts the AC coefficients by using the DC coefficient and intra prediction mode to reduce the residual AC coefficients.

#### Details of the AC prediction using DC and intra prediction mode

The point of this technique is to create an AC-predicted signal with a residual DC coefficient and an intra prediction mode. The AC-predicted signal, which we implemented, consists of a linear function. The DC coefficient is used to calculate the slope of the linear function, and intra prediction mode is used to calculate the direction of the prediction. The line of the function passes through following two points: one is average value, which is calculated from the DC coefficient, at the center of the current block; the other is zero at a reference position of the current block. Thus the AC-predicted signal includes the DC coefficient.

The DC coefficient and the intra prediction mode, which are the information to create the AC-predicted signal, are the sent information in AVC commonly, thus no additional information is needed to create the AC-predicted signal.

In this technique, “intra_ac_pred_enable_flag” is added to each block, except the case where intra prediction is DC mode or the DC coefficient is zero (The prediction for these blocks is disabled implicitly). It indicates whether the AC prediction is enabled or not for each block. When the prediction is disabled implicitly, the AC prediction is not applied implicitly in the current block without the flag sending/receiving. We implemented this technique into intra 4x4 modes and intra 8x8 modes.

Fig. 16 illustrates the conceptual diagram in the decoder. At first, residual coefficients, an intra prediction mode and an intra predicted signal are created by conventional intra prediction. Residual coefficients are separated into residual AC coefficients and a DC coefficient. The AC residual signal is created by dequantizing and inverse transforming the residual AC coefficients. An AC-predicted signal is created by referring the DC coefficient and the intra prediction mode. A Decoded signal consists of the sum of the intra predicted signal, the AC-predicted signal and the AC residual signal.

Fig. 16  Conceptual diagram of AC prediction using DC and intra prediction mode (decoder)

Fig. 17 shows a flowchart of block-decoding process of this technique.

- Parse an intra prediction mode and residual coefficients and create an AVC-based intra predicted signal.

- If the DC coefficient level is equal to zero or the intra prediction mode is a DC prediction mode (disabled implicitly), perform dequantization and inverse transform residual coefficients, and a decoded signal is the sum of intra prediction signal and decoded residual (conventional operation), and go to next block.

- Otherwise, parse intra_ac_pred_enable_flag.

If intra_ac_pred_enable_flag is zero, create a decoded signal with typical operation, and go to next block.

- Otherwise, create an AC residual signal by dequantization and inverse transform residual AC coefficients, and AC predicted signal by referring dequantized DC coefficient and the intra prediction mode, and a decoded signal consists of the sum of intra prediction signal, AC predicted signal and AC residual signal, and go to next block.

Fig. 17  flowchart of decoding process

## Spatial transforms

New conceptual tools of spatial transforms are not applied into our proposal codec. The conventional spatial transforms based on AVC are used to our proposal codec.

## Quantization

New conceptual tools of quantization are not applied into our proposal codec. The conventional quantization tools based on AVC are used to our proposal codec.

## In-loop filtering

Adaptive loop filter is applied to reconstructed signals after deblocking filter process. In this proposal, Quad-tree based Adaptive Loop Filter (QALF) is used as adaptive loop filter. Implementation of this tool is based on KTA tools.

## Entropy coding

New conceptual tools of entropy coding are not applied to our proposal. The conventional entropy coding tools based on AVC are used to our proposal codec.

## Encoder Settings

We have performed our encoder based on the settings shown in Table 1 for C1 and C2 commonly. And individual settings for C1 and C2 are shown in Table 2.

Table 1 Encoder common settings

| Parameter Name | Value |
| --- | --- |
| ProfileIDC | 100 |
| LevelIDC | 51 |
| HierarchicalCoding | 3 |
| NumberReferenceFrames | 4 |
| SymbolMode | 1 |
| Transform8x8Mode | 1 |
| ScalingMatrixPresentFlag | 0 |
| RDOptimization | 1 |
| UseRDOQuant | 1 |
| RDOQ_Fast | 1 |
| RDOQ_QP_Num | 1 |
| AdaptiveRounding | 0 |
| WeightedPrediction | 1 |
| WeightedBiprediction | 1 |
| SearchMode | 3 |
| SearchRange | 128 |
| RDPictureDecision | 0 |

Table 2 Encoder individual settings

| Parameter Name | C1 Value | C2 Value |
| --- | --- | --- |
| EnableOpenGOP | 1 | 0 |
| LowDelay | 0 | 1 |
| IntraPeriod | 24-64 | 0 |
| ReferenceReorder | 1 | 2 |
| MemoryManagement | 1 | 2 |
| PReplaceBSlice | 0 | 1 |
| NumberBFrames | 7 | 3 |

# Compression performance discussion

## Objective versus subjective compression performance

Subjective performance of our proposal will be better depending on objective performance since our new techniques improve the predictive efficiency.

## Constraint set 1 configuration relative to Alpha anchor

Simulation results for constraint set 1 are shown below as the graph of PSNR. More detailed results are put in the attached Excel file.

### Class A

### Class B

### Class C

### Class D

### Overall

Table 3  BD-PSNR and BD-bitrate: C1 - alpha anchor

| Class | BD-PSNR (R1-R4, R2-R5) |  | BD-bitrate (R1-R4, R2-R5) |  |
| --- | --- | --- | --- | --- |
| Class A | 0.32 | 0.35 | -6.52 | -7.57 |
| Class B | 0.19 | 0.20 | -6.18 | -7.60 |
| Class C | 0.38 | 0.45 | -8.47 | -10.28 |
| Class D | 0.57 | 0.64 | -12.65 | -13.90 |
| Total average | 0.38 |  | -9.27 |  |

In summary, our proposal achieves the bitrate reduction of average 9.27% and up to 24.09% for constraint set 1 to alpha anchor. (Each value is calculated by BD-Bitrate[6] using the results R1 - R4 and R2 - R5.) Our techniques seem to be effective to all sequences.

## Constraint set 2 configuration relative to Beta and Gamma anchors

Simulation results for constraint set 2 are shown below as the graph of PSNR. More detailed results are put in the attached Excel file.

### Class B

### Class C

### Class D

### Class E

### Overall

Table 4  BD-PSNR and BD-bitrate: C2 - beta anchor

| Class | BD-PSNR (R1-R4, R2-R5) |  | BD-bitrate (R1-R4, R2-R5) |  |
| --- | --- | --- | --- | --- |
| Class B | 0.18 | 0.19 | -6.08 | -7.19 |
| Class C | 0.02 | 0.04 | -0.55 | -1.13 |
| Class D | -0.01 | -0.02 | -0.47 | -0.23 |
| Class E | 0.16 | 0.20 | -3.38 | -5.26 |
| Total average | 0.09 |  | -3.18 |  |

Table 5  BD-PSNR and BD-bitrate: C2 - gamma anchor

| Class | BD-PSNR (R1-R4, R2-R5) |  | BD-bitrate (R1-R4, R2-R5) |  |
| --- | --- | --- | --- | --- |
| Class B | 1.23 | 1.10 | -30.44 | -30.10 |
| Class C | 1.01 | 1.05 | -22.19 | -23.11 |
| Class D | 1.11 | 1.16 | -24.63 | -24.50 |
| Class E | 1.45 | 1.33 | -26.66 | -29.09 |
| Total average | 1.17 |  | -26.49 |  |

In summary, our proposal achieves the bitrate reduction of average 3.18% and up to 22.71% for constraint set 2 to beta anchor. And our proposal achieves the bitrate reduction of average 26.49% and up to 53.66% for constraint set 2 to gamma anchor. (Each value of bitrate reduction is calculated by BD-Bitrate using the results of R1 - R4 and R2 - R5.) Some sequences in class C and D seem to be worse than beta anchor. However we think our techniques are effective to all sequences according to our original experiment which compares our proposal with JM16.2 in the same settings which are described in section 2.7.

# Complexity analysis

## Encoding time and measurement methodology

The results of encoding time for both constraint sets and all classes are shown in the following tables.

The unit of the value in the tables is a second.

Encoding time is measured with system time (time function) in this proposed encoder.

The platform used for the execution time measurement is described in section 4.3.

### Constraint set 1 (C1)

Table 6  Encoding Time: C1 - Class A

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Traffic(S01) | 63,284 | 64,138 | 65,895 | 66,380 | 66,698 |
| PeopleOnStreet(S02) | 67,756 | 67,756 | 75,346 | 83,675 | 85,822 |

Table 7  Encoding Time: C1 - Class B

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 53,506 | 55,129 | 56,561 | 57,652 | 58,509 |
| ParkScene(S04) | 54,341 | 55,949 | 56,955 | 57,580 | 57,890 |
| Cactus(S05) | 112,318 | 114,226 | 116,151 | 118,251 | 120,062 |
| BasketballDrive(S06) | 117,960 | 121,166 | 123,746 | 126,151 | 128,147 |
| BQTerrace(S07) | 143,417 | 144,935 | 146,314 | 148,538 | 151,046 |

Table 8　Encoding Time: C1 - Class C

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 21,434 | 21,777 | 22,172 | 22,553 | 22,907 |
| BQMall(S09) | 27,263 | 27,718 | 28,231 | 28,562 | 28,727 |
| PartyScene(S10) | 23,679 | 24,225 | 24,925 | 25,496 | 25,841 |
| RaceHorses(S11) | 14,058 | 14,400 | 14,834 | 15,203 | 15,567 |

Table 9　Encoding Time: C1 - Class D

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 5,945 | 6,112 | 6,216 | 6,360 | 6,474 |
| BQSquare(S13) | 7,650 | 7,742 | 7,870 | 8,021 | 8,108 |
| BlowingBubbles(S14) | 6,177 | 6,298 | 6,366 | 6,498 | 6,574 |
| RaceHorses(S15) | 3,900 | 4,005 | 4,062 | 4,132 | 4,207 |

### Constraint set 2 (C2)

Table 10　Encoding Time: C2 - Class B

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 36,653 | 37,170 | 37,665 | 37,850 | 38,792 |
| ParkScene(S04) | 37,973 | 38,598 | 38,116 | 38,304 | 38,532 |
| Cactus(S05) | 73,474 | 73,878 | 75,520 | 75,341 | 75,495 |
| BasketballDrive(S06) | 77,601 | 79,698 | 81,053 | 80,885 | 82,727 |
| BQTerrace(S07) | 96,611 | 94,551 | 96,588 | 97,682 | 101,954 |

Table 11  Encoding Time: C2 - Class C

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 14,106 | 14,360 | 14,583 | 14,724 | 14,883 |
| BQMall(S09) | 17,898 | 18,068 | 18,202 | 18,234 | 18,158 |
| PartyScene(S10) | 15,723 | 15,977 | 16,258 | 16,568 | 16,772 |
| RaceHorses(S11) | 9,320 | 9,478 | 9,651 | 9,784 | 9,880 |

Table 12  Encoding Time: C2 - Class D

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 3,913 | 3,977 | 4,030 | 4,118 | 4,145 |
| BQSquare(S13) | 5,033 | 5,093 | 5,134 | 5,159 | 5,110 |
| BlowingBubbles(S14) | 4,173 | 4,223 | 4,248 | 4,298 | 4,305 |
| RaceHorses(S15) | 2,556 | 2,620 | 2,650 | 2,668 | 2,686 |

Table 13  Encoding Time: C2 - Class E

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Vidyo1(S16) | 34,307 | 34,386 | 34,559 | 34,885 | 35,260 |
| Vidyo3(S17) | 34,570 | 34,723 | 34,730 | 34,844 | 35,079 |
| Vidyo4(S18) | 34,302 | 34,457 | 34,638 | 35,074 | 35,505 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0

The results of decoding time for both constraint sets and all classes are shown in the following tables.

The unit of the value in the tables is a second.

Decoding time is measured with system time (time function) in this proposed decoder (same as JM17.0).

The predictive structure for constraint set 2 is the same as beta anchor.

The platform used for the execution time measurement is described in section 4.3.

### Constraint set 1 (C1)

- C1 - Class A

Table 14  Decoding Time of Proposed Decoder: C1 - Class A

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Traffic(S01) | 128.44 | 136.58 | 146.77 | 153.42 | 149.33 |
| PeopleOnStreet(S02) | 134.92 | 143.88 | 157.85 | 169.34 | 172.37 |

Table 15  Decoding Time of JM17.0 Decoder: C1 - Class A - Alpha anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Traffic(S01) | 23.33 | 23.87 | 25.15 | 27.01 | 30.85 |
| PeopleOnStreet(S02) | 23.84 | 24.73 | 25.79 | 28.20 | 31.75 |

- C1 - Class B

Table 16  Decoding Time of Proposed Decoder: C1 - Class B

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 110.64 | 119.75 | 132.58 | 144.33 | 150.54 |
| ParkScene(S04) | 91.14 | 106.64 | 117.08 | 121.42 | 120.77 |
| Cactus(S05) | 143.25 | 164.45 | 177.99 | 190.34 | 208.91 |
| BasketballDrive(S06) | 194.41 | 215.40 | 231.95 | 241.33 | 251.23 |
| BQTerrace(S07) | 233.00 | 248.96 | 259.62 | 273.03 | 280.63 |

Table 17  Decoding Time of JM17.0 Decoder: C1 - Class B - Alpha anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 26.61 | 28.53 | 30.44 | 32.57 | 35.17 |
| ParkScene(S04) | 25.77 | 27.50 | 29.52 | 31.20 | 33.36 |
| Cactus(S05) | 48.62 | 49.68 | 51.54 | 54.93 | 59.00 |
| BasketballDrive(S06) | 55.44 | 58.26 | 61.06 | 64.52 | 68.35 |
| BQTerrace(S07) | 68.03 | 69.74 | 71.15 | 73.34 | 76.96 |

- C1 - Class C

Table 18　 Decoding Time of Proposed Decoder: C1 - Class C

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 21.34 | 25.28 | 29.31 | 35.06 | 37.95 |
| BQMall(S09) | 30.87 | 35.39 | 42.98 | 49.08 | 53.45 |
| PartyScene(S10) | 27.25 | 32.19 | 39.24 | 44.71 | 47.44 |
| RaceHorses(S11) | 20.03 | 22.07 | 24.64 | 28.36 | 31.83 |

Table 19　 Decoding Time of JM17.0 Decoder: C1 - Class C - Alpha anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 6.88 | 7.32 | 7.75 | 8.41 | 9.46 |
| BQMall(S09) | 8.92 | 9.55 | 10.01 | 10.70 | 11.62 |
| PartyScene(S10) | 7.61 | 7.89 | 8.66 | 9.43 | 10.27 |
| RaceHorses(S11) | 5.33 | 5.68 | 6.14 | 6.57 | 7.52 |

- C1 - Class D

Table 20　 Decoding Time of Proposed Decoder: C1 - Class D

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 7.94 | 9.34 | 10.44 | 11.19 | 11.33 |
| BQSquare(S13) | 13.58 | 13.61 | 13.53 | 13.59 | 13.80 |
| BlowingBubbles(S14) | 7.74 | 9.66 | 10.89 | 11.43 | 11.00 |
| RaceHorses(S15) | 7.14 | 8.13 | 8.28 | 8.71 | 8.78 |

Table 21　 Decoding Time of JM17.0 Decoder: C1 - Class D - Alpha anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 1.88 | 2.12 | 2.34 | 2.72 | 3.22 |
| BQSquare(S13) | 2.34 | 2.60 | 2.77 | 3.26 | 3.81 |
| BlowingBubbles(S14) | 1.96 | 2.09 | 2.24 | 2.60 | 3.16 |
| RaceHorses(S15) | 1.49 | 1.60 | 1.76 | 2.12 | 2.65 |

### Constraint set 2 (C2)

- C2 - Class B

Table 22　 Decoding Time of Proposed Decoder: C2 - Class B

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 101.01 | 116.72 | 125.63 | 134.50 | 142.65 |
| ParkScene(S04) | 78.28 | 89.02 | 102.09 | 113.51 | 126.33 |
| Cactus(S05) | 102.57 | 112.99 | 126.75 | 145.61 | 170.83 |
| BasketballDrive(S06) | 149.59 | 172.94 | 192.50 | 210.15 | 220.64 |
| BQTerrace(S07) | 260.70 | 293.49 | 321.41 | 364.38 | 417.11 |

Table 23　 Decoding Time of JM17.0 Decoder: C2 - Class B - Beta anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 21.02 | 23.25 | 24.93 | 27.80 | 30.20 |
| ParkScene(S04) | 19.84 | 22.41 | 24.26 | 26.68 | 29.41 |
| Cactus(S05) | 38.83 | 41.94 | 45.00 | 48.42 | 52.32 |
| BasketballDrive(S06) | 43.09 | 46.45 | 51.45 | 55.81 | 59.34 |
| BQTerrace(S07) | 51.49 | 54.36 | 56.91 | 61.22 | 65.66 |

Table 24　 Decoding Time of JM17.0 Decoder: C2 - Class B - Gamma anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Kimono(S03) | 20.36 | 21.94 | 23.76 | 26.12 | 28.91 |
| ParkScene(S04) | 19.24 | 21.11 | 23.23 | 25.61 | 28.23 |
| Cactus(S05) | 37.73 | 39.98 | 42.32 | 45.96 | 49.76 |
| BasketballDrive(S06) | 41.24 | 44.49 | 47.96 | 51.74 | 56.15 |
| BQTerrace(S07) | 49.55 | 51.72 | 54.54 | 58.53 | 62.60 |

- C2 - Class C

Table 25  Decoding Time of Proposed Decoder: C2 - Class C

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 16.57 | 19.91 | 23.09 | 27.45 | 32.77 |
| BQMall(S09) | 26.49 | 31.19 | 36.64 | 41.01 | 46.35 |
| PartyScene(S10) | 23.00 | 27.87 | 36.97 | 47.47 | 59.94 |
| RaceHorses(S11) | 16.20 | 18.84 | 21.86 | 25.68 | 30.75 |

Table 26　 Decoding Time of JM17.0 Decoder: C2 - Class C - Beta anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 5.16 | 5.59 | 6.15 | 7.19 | 8.68 |
| BQMall(S09) | 6.70 | 7.06 | 7.90 | 8.83 | 9.98 |
| PartyScene(S10) | 5.45 | 6.03 | 7.02 | 8.02 | 9.31 |
| RaceHorses(S11) | 4.31 | 4.76 | 5.26 | 6.03 | 7.05 |

Table 27　 Decoding Time of JM17.0 Decoder: C2 - Class C - Gamma anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballDrill(S08) | 4.88 | 5.19 | 5.77 | 6.49 | 7.58 |
| BQMall(S09) | 6.38 | 6.80 | 7.11 | 8.10 | 9.24 |
| PartyScene(S10) | 5.11 | 5.72 | 6.56 | 7.46 | 8.67 |
| RaceHorses(S11) | 4.00 | 4.36 | 4.83 | 5.50 | 6.42 |

- C2 - Class D

Table 28  Decoding Time of Proposed Decoder: C2 - Class D

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 5.85 | 6.54 | 7.31 | 8.15 | 9.24 |
| BQSquare(S13) | 19.24 | 20.38 | 20.79 | 20.57 | 19.57 |
| BlowingBubbles(S14) | 8.23 | 10.07 | 11.08 | 12.21 | 13.55 |
| RaceHorses(S15) | 5.57 | 6.46 | 6.87 | 7.69 | 8.83 |

Table 29　 Decoding Time of JM17.0 Decoder: C2 - Class D - Beta anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 1.63 | 1.86 | 2.10 | 2.53 | 3.15 |
| BQSquare(S13) | 1.89 | 2.16 | 2.35 | 2.90 | 3.58 |
| BlowingBubbles(S14) | 1.61 | 1.83 | 2.16 | 2.53 | 3.31 |
| RaceHorses(S15) | 1.30 | 1.52 | 1.67 | 2.09 | 2.62 |

Table 30　 Decoding Time of JM17.0 Decoder: C2 - Class D - Gamma anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| BasketballPass(S12) | 1.51 | 1.66 | 1.91 | 2.24 | 2.75 |
| BQSquare(S13) | 1.84 | 2.04 | 2.19 | 2.56 | 3.07 |
| BlowingBubbles(S14) | 1.46 | 1.65 | 1.82 | 2.23 | 2.83 |
| RaceHorses(S15) | 1.17 | 1.29 | 1.50 | 1.86 | 2.31 |

- C2 - Class E

Table 31  Decoding Time of Proposed Decoder: C2 - Class E

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Vidyo1(S16) | 32.04 | 39.00 | 45.01 | 54.75 | 65.93 |
| Vidyo3(S17) | 37.86 | 42.58 | 48.30 | 57.62 | 69.87 |
| Vidyo4(S18) | 33.70 | 42.94 | 47.36 | 56.62 | 70.08 |

Table 32　 Decoding Time of JM17.0 Decoder: C2 - Class E - Beta anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Vidyo1(S16) | 11.08 | 12.01 | 12.29 | 13.40 | 15.01 |
| Vidyo3(S17) | 11.11 | 11.58 | 12.04 | 13.10 | 14.87 |
| Vidyo4(S18) | 11.47 | 12.16 | 12.64 | 13.32 | 15.04 |

Table 33　 Decoding Time of JM17.0 Decoder: C2 - Class E - Gamma anchor

| Test Sequence | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| Vidyo1(S16) | 10.90 | 11.70 | 12.19 | 13.23 | 14.57 |
| Vidyo3(S17) | 11.09 | 11.44 | 11.83 | 12.83 | 14.12 |
| Vidyo4(S18) | 11.35 | 11.82 | 12.18 | 13.16 | 14.52 |

## Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

The Experiment environment used for the execution time measurement of the encoder and the decoder is shown in Table 34.

Table 34 Experiment environment

| Item | Specification |
| --- | --- |
| CPU | Intel Xeon 5160 (2 CPU) 3.00GHz |
| Memory | 16GB (DDR2 SDRAM FB-DIMM) |
| Hard Disk | SATA300 1.5TB HDD 7200rpm (ST31500341AS) |
| OS | Windows 7 Professional (64bit) |
| Parallel Processing | Not Used |
| Executable | Win32b |

## Expected memory usage of encoder

There is no additional picture memory except some working memory compared with AVC.

The additional working memory is due to the implementation method, and it is expected that maximum working memory is about 1 picture memory for QALF tool.

## Expected memory usage of decoder

There is no additional picture memory except some working memory compared with AVC.

The additional working memory is due to the implementation method, and it is expected that maximum working memory is about 30x30 pixel memory for Decoder-side Motion Estimation on RMC tool.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

At motion representation part, three new techniques (GTP, DBBD, RMC) are added to AVC algorithm.

The additional complexity of motion estimation and motion segmentation of each tool in the encoder is as follows.

- Encoder Complexity of GTP

In order to obtain the motion vectors for the GTP, the motion estimation are applied on the encoder side. The method is the block matching and the complexity is equal to the one of earlier methods.

Three modes are added as candidates of the mode selection.

For mode selection, GTP needs the interpolation of the motion vector for each pixel.

- Encoder Complexity of DBBD

To apply DBBD, motion estimation is requested to decide a motion vector of each block. Although any algorithm of motion estimation can be applied into DBBD, we used the EPZS algorithm implemented in JM to equalize the condition to the anchors. In addition, simulation of the decoding process is requested to evaluate the final motion compensated picture.

- Encoder Complexity of RMC

To apply RMC, additional Motion Estimation is needed on both encoder and decoder for Decoder-side Motion Estimation.

On encoder side, it simulates Decoder-side Motion Estimation at 9 positions around uni-predictive ME’s results (output of JM16.2 EPZS method).

Specifications of Decoder-side Motion Estimation in RMC at one position on this proposal is as follows

RMC Block Size:

16x16 uni-predictive only

Decoder-side Motion Estimation Search Range:

Search Center Position:

Position shown by motion to 1 pixel accuracy motion vector (MVa_down) in ratio of the time between picture for target and reference pictures.

Search Range:

±4.5 pixel (hierarchical block matching: 1 pixel accuracy and 1/4 pixel accuracy)

Sub-pixel / Quad-pixel Filtering:

6-tap / 2-tap filter (same as AVC)

Matching Method:

SAD (luminance only)

Decoder-side Motion Estimation Block Size:

16x16 only

On encoder, Bi-Predictive Estimation based selection of phase shift vector runs after every Decoder-side Motion Estimation,

Search Range ;

-1/2 <= phase < 1/2 by 1/4 pixel accuracy on vertical and horizontal

Matching Method:

SATD (luminance only)

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder motion compensation

At motion representation part, three new techniques (GTP, DBBD, RMC) are added to AVC algorithm.

The additional complexity of motion compensation of new tools in the decoder is as follows.

- Decoder Complexity of GTP

When the GTP mode is selected, the interpolation of the motion vector is needed for each pixel in addition to the process of the conventional inter prediction.

- Decoder Complexity of DBBD

To apply DBBD, calculation of an activity on each candidate boundary is required to decide a real boundary which is used for getting a final motion compensated picture. The amount of computational cost depends on the number of candidates. In this proposal, we set the number of candidates to 3.

- Decoder Complexity of RMC

To apply RMC, additional Motion Estimation is needed on both encoder and decoder for Decoder-side Motion Estimation.

Specifications of Decoder-side Motion Estimation in RMC on this proposal is as follows

RMC Block Size:

16x16 uni-predictive only

Decoder-side Motion Estimation Search Range:

Search Center Position:

Position shown by motion to 1 pixel accuracy motion vector (MVa_down) in ratio of the time between picture for target and reference images.

Search Range:

±4.5 pixel (hierarchical block matching: 1 pixel accuracy and 1/4 pixel accuracy)

Sub-pixel / Quad-pixel Filtering:

6-tap / 2-tap filter (same as AVC)

Matching Method:

SAD (luminance only)

Decoder-side Motion Estimation Block Size:

16x16 only

- Decoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of encoder intra-frame prediction type selection

At intra–frame prediction part, a new technique (AC prediction using DC and intra prediction mode) is added to AVC algorithm.

The additional complexity of type selection of intra-frame prediction of new tool in the encoder is as follows.

- Encoder Complexity of AC prediction using DC and intra prediction mode

All enabled modes and all disabled modes are tried to all available intra prediction modes with RD optimization. Thus it needs almost twice deciding mode time of JM16.2 reference software.

The spatial complexity characteristic is the same as JM16.2 reference software.

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder intra-frame prediction operation

At intra–frame prediction part, a new technique (AC prediction using DC and intra prediction mode) is added to AVC algorithm.

The additional complexity of intra-frame prediction of new tool in the decoder is as follows.

- Decoder Complexity of AC prediction using DC and intra prediction mode

The process to create AC-predicted signal is added, the time to that is same as the time to create intra predicted signal. So it needs at most twice time to decode intra macroblocks of JM16.2 reference software.

The spatial complexity characteristic is the same as JM16.2 reference software.

- Decoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of encoder transforms and transform type selection

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder inverse transform operation

- Decoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of encoder quantization and quantization type selection

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder inverse quantization

- Decoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of encoder in-loop filtering type selection

At in-loop filtering part, QALF method is added to AVC algorithm.

The additional complexity of type selection of in-loop filtering of QALF for the encoder is as follows.

- Encoder Complexity of QALF

In QALF, it needs a selection of loop filter type as follows.

Filter Type:	9x9, 7x7, 5x5 symmetrical 2D filters

Selection Method:	Block/Quad-tree based On/Off selection and filter tap size selection (3types)
    			based on Rate-Distortion Optimization Method.

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder in-loop filtering operation

- Decoder Complexity of QALF

At in-loop filtering part, QALF method is added to AVC algorithm.

The additional complexity of in-loop filtering operation of QALF for the decoder is as follows.

- Decoder Complexity of others

At worst case, 9x9 symmetrical 2D filter is applied for all reconstructed signals.

## Complexity characteristics of encoder entropy coding type selection

- Encoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Complexity characteristics of decoder entropy decoding operation

- Decoder Complexity of others

The other complexity characteristics in this section are the same as JM16.2 reference software that is the implementation of AVC.

## Degree of capability for encoder parallel processing

The degree of capability is the same as JM16.2 reference software that is the implementation of AVC.

## Degree of capability for decoder parallel processing

The degree of capability is the same as JM16.2 reference software that is the implementation of AVC.

# Algorithmic characteristics

The new tools on this proposal don’t influence to “Random access characteristics” and “Delay characteristics” of based algorithm (AVC).

## Random access characteristics

The random access characteristics are the same as JM16.2 reference software that is the implementation of AVC. In this proposal, the random access characteristics for constraint Set 1 is the same as alpha anchor setting.

## Delay characteristics

The delay characteristics are the same as JM16.2 reference software that is the implementation of AVC. In this proposal, the delay characteristics for constraint Set 2 is the same as beta anchor setting.

# Software implementation description

The proposal software is based on JM16.2.

The development environment and settings are shown in Table 35.

Table 35 Development environment and settings

| Item | Specification |
| --- | --- |
| Language | C language |
| IDE | Microsoft Visual C++ 2008 Express Edition with sp1 |
| Compile Option | same as environment of JM16.2 |
| Linker Option | enable the option of “support over 2GB address” on the encoder only |
| Additional Compiler | No |
| Additional Library | No |
| Executable | Win32b |

# Highlighted aspects discussion

We think that four techniques which we present in this proposal are effective for a new coding standard.

But the implementation of new tools in this proposal is a part of realization of the concept of these technologies.

Thus, we will investigate and confirm the performance of these technologies during the work of standardization.

Some of the future work is as follows.

- GTP

For GTP, we will implement and investigate the coding efficiency as following.

- adding more Geometric Prediction Type

- improving motion search

- applying to the block of other sizes

- DBBD

We have introduced DBBD only to 16xA and Ax16 and constrained ‘A’ to multiples of 4 since AVC has the transform which sizes are 8x8 and 4x4. So we are planning to try to perform DBBD with more flexible shapes such as 16x6 and 12x4.

- RMC

For RMC, we will implement and investigate the coding efficiency as following.

- Adaptation to variable block size MC (including large block size)

- Synthesizing method (not only averaging)

- Relation of the first MC block size and size of decoder-side motion estimation.

- AC prediction using DC and intra prediction mode

We will try to improve coding efficiency in intra prediction, and by applying to inter prediction.

# Closing remarks

In this proposal, we introduce four new coding tools for improvement of intra-frame prediction and motion representation. These tools are integrated into our proposed software with one KTA tool on JM16.2 based codec, and our proposed software has the advance of coding efficiency for current standard (AVC).

We think that these tools are effective for a new coding standard, and can contribute to the sufficiently higher compression capability which is needed for the future high-quality applications.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] "Joint Call for Proposals on Video Compression Technology",

ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, ISO/IEC JTC1/SC29/WG11/N11113, Kyoto, January 2010.

[2] "Vision, Applications and Requirements for High-Performance Video Coding(HVC)",

ISO/IEC JTC1/SC29/WG11/N11096, Kyoto, January 2010.

[3] T.Chujoh, N.Wada, T.Watanabe, G.Yasuda and T.Yamakage,

"Specification and experimental results of Quadtree-based Adaptive Loop Filter,"

ITU-T SG16 Q.6 Document, VCEG-AK22, Yokohama, April 2009.

[4] KTA Reference Software, http://iphome.hhi.de/suehring/tml/download/KTA/.

[5] H.264/AVC Reference Software, http://iphome.hhi.de/suehring/tml/download/.

[6] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.