| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010 | Document: JCTVC-A119 |
| --- | --- |

| Title: | Description of video coding technology proposal by Tandberg, Nokia, Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur Nokia Research Center Visiokatu 1 33720 Tampere, Finland Kenneth R. Andersson, LM Ericsson 164 80 Stockholm, Sweden Arild Fuldseth Tandberg Telecom AS Philip Pedersens vei 22, Norway | Tel: Email: Tel: Email: Tel: Email: | +358 50 4860857 kemal.ugur@nokia.com +46 10 7141538 kenneth.r.andersson@ericsson.com +47 92414082 arild.fuldseth@tandberg.com |
| Source: | Tandberg, Nokia, Ericsson |  |  |

_____________________________

# Abstract

This document presents the Tandberg-Ericsson-Nokia Test Model (TENTM), which is designed to fulfill the requirements of the mobile, video-conferencing and broadcast industries. For more detailed description of the TENTM codec, please refer to the attached decoder description document including the syntax and semantics of the TENTM bitstreams.

TENTM is a codec providing both high performance and low decoding complexity enabling high resolution video services everywhere. It is argued that TENTM provides significant visual improvement over H.264/AVC (both High Profile and Baseline Profile) with decoder complexity lower than H.264/AVC Baseline Profile.  These improvements can be achieved with significantly lower encoding complexity than for H.264/AVC (both High Profile and Baseline Profile) which is valuable for real-time communication and services on mobile devices. In order to have a clean design from the start, TENTM is designed with a back-to-basics approach in mind. Several possible extensions could be added during the standardization process to improve the coding gain significantly, e.g. CABAC, additional reference frames, etc.

Encoder time measurements show that the complete CS1 test set (all sequences, 5 ratepoints) can be simulated in 5 ½ hours while the CS2 test set can be simulated in less than 4 hours. In the same computing platform, TENTM encoding is around 25 times faster than JM17.0 for coding alpha and beta anchors and around 10 times faster for coding gamma anchor. TENTM encoding is significantly faster than JM as the software is written from scratch in a clean fashion, and many brute-force techniques such as frame-level multipass encoding; large number of reference frames etc. are avoided.

Decoder simulations show that the TENTM decoder runs more than twice as fast as the JM17.0 High Profile decoder on average. This is because TENTM decoder has less algorithmic complexity than H.264/AVC and it is implemented in a clean fashion from scratch.

# Contents

Abstract	1

Contents	2

1	Introduction	4

2	Algorithm description	4

2.1	Motion representation	4

2.1.1	Supported reference frame structures	5

2.1.2	Skip MB	5

2.1.3	Bi-prediction	5

2.1.4	DIRECT Mode	5

2.2	Intra-frame prediction	5

2.2.1	Planar prediction for Intra 16x16	6

2.2.2	Angular prediction for Intra 8x8	6

2.3	Spatial Transforms	7

2.4	Quantization	9

2.5	In-loop filtering	9

2.6	Entropy coding	9

2.7	Macroblock scan order and syntax to support large macroblocks	12

2.8	Non-normative encoder features	13

2.9	Future extensions and additional features	16

3	Compression performance discussion	17

3.1	Objective versus subjective compression performance	17

3.2	Constraint set 1 configuration relative to Alpha anchor	17

3.2.1	Class A	17

3.2.2	Class B	17

3.2.3	Class C	17

3.2.4	Class D	18

3.2.5	Overall	18

3.3	Constraint set 2 configuration relative to Beta anchor	18

3.3.1	Class B	18

3.3.2	Class C	18

3.3.3	Class D	18

3.3.4	Class E	18

3.3.5	Overall	18

3.4	Constraint set 2 configuration relative to Gamma anchor	19

3.4.1	Class B	19

3.4.2	Class C	19

3.4.3	Class D	19

3.4.4	Class E	19

3.4.5	Overall	19

4	Complexity analysis	19

4.1	Encoding time and measurement methodology	19

4.2	Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0	23

4.3	Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2	28

4.4	Expected memory usage of encoder	28

4.5	Expected memory usage of decoder	29

4.6	Complexity characteristics of encoder motion estimation and motion segmentation selection	29

4.7	Complexity characteristics of decoder motion compensation	29

4.8	Complexity characteristics of encoder intra-frame prediction type selection	29

4.9	Complexity characteristics of decoder intra-frame prediction operation	29

4.10	Complexity characteristics of encoder transforms and transform type selection	30

4.11	Complexity characteristics of decoder inverse transform operation	30

4.12	Complexity characteristics of encoder quantization and quantization type selection	30

4.13	Complexity characteristics of decoder inverse quantization	30

4.14	Complexity characteristics of encoder in-loop filtering type selection	30

4.15	Complexity characteristics of decoder in-loop filtering operation	30

4.16	Complexity characteristics of encoder entropy coding type selection	31

4.17	Complexity characteristics of decoder entropy decoding operation	31

4.18	Degree of capability for encoder parallel processing	31

4.19	Degree of capability for decoder parallel processing	31

4.20	Non-normative encoder complexity issues	31

5	Algorithmic characteristics	31

5.1	Random access characteristics	31

5.2	Delay characteristics	31

6	Software implementation description	32

7	Highlighted aspects discussion	32

8	Closing remarks	32

9	Patent rights declaration(s)	32

10	References	33

# Introduction

The TENTM algorithm consists of the following normative tools that provide a coding gain over H.264/AVC:

- Improved interpolation filters.

- Larger macroblock sizes.

- Improved definition of skip mode, including the possibility of two MV skip candidates.

- Improved bi-directional prediction.

- Improved Intra prediction.

- Larger transform sizes (16x4, 16x16, 32x32, and 64x64).

- Special intra mode (planar mode) for improved subjective quality.

- Improved adaptive (non-arithmetic) entropy coding compared to CAVLC.

- Improved in-loop filtering.

This document provides an overview of the TENTM algorithm. More details could be found on the TENTM decoder description document attached to the contribution, which has significantly more detail and includes the syntax and semantics of the bitstreams used by TENTM.

The following experts have contributed to the development of TENTM:

Nokia: 	Kemal Ugur, Jani Lainema, Antti Hallapuro, Justin Ridge, Cixun Zhang, Dmytro Rusanovskyy

Ericsson:	Kenneth Andersson, Per Fröjdh, Andrey Norkin, Clinton Priddle, Thomas Rusert,  Jonatan Samuelsson, Rickard Sjöberg, Jacob Ström, Per Wennersten, Zhuangfei Wu

Tandberg:	Gisle Bjøntegaard, Arild Fuldseth, Lars Petter Endresen, Robert Folland, Greg Håkonsen, Mark Blake

# Algorithm description

## Motion representation

TENTM utilizes a translational motion model with motion vectors having quarter pixel accuracy. Motion vectors can be signalled for macroblocks of size 64x64, 32x32, and 16x16. In addition, motion vectors can be signalled for the following sub-partitions of 16x16 macroblocks: 16x8, 8x16, and 8x8.

Motion vectors are encoded separately in the x and y dimension, as the difference between the motion vector and a motion vector predictor. The motion vector predictor is derived from neighbour macroblocks but using a slightly different method from H.264/AVC. For more details, see the attached decoder description document.

For the 9 innermost quarter-pixel positions, one additional bit is signalled to choose between two different sets of interpolation filters.

The first set of interpolation filter is referred to as the Directional Interpolation Filter (DIF) and is used for all 15 quarter-pixel positions. For each of the three horizontal positions and the three vertical positions which are aligned with full pixel positions, a single 6-tap filter is used. For the 9 innermost quarter-pixel positions two 6-tap filters at +45 degree and -45 degree angles are used. The exception is the center position where a 12-tap non-separable filter is used.

The second set of interpolation filters consists of Separable Filters (SF) where interpolated samples are calculated by first applying a 6-tap filter horizontally and then vertically.

The filter coefficients for FDIF and SF are

[3 -15 111 37 -10 2]/128 for ¼ displacement (and mirrored for ¾ displacement)

[3 -17 78 78 -17 3]/128 for ½ displacement

For the center position of FDIF, the filter coefficients are:

[0   5    5    0

5   22  22   5

5   22  22   5

0    5    5    0 ] / 128

All interpolated sample values can be calculated using 16 bit arithmetic. The resulting interpolated values are scaled, clipped and stored using 8 bit/sample.

### Supported reference frame structures

Inter prediction can be performed using one or two reference frames. Both forward and backward prediction, as well as bi-prediction is supported. Hierarchical B frames similar to H.264/AVC is supported. For IPPP coding structure, a simple signalling scheme for long term reference frames is supported. This provides coding gains similar to “Hierarchical P frames” in the beta anchor.

### Skip MB

Skip MBs are encoded as runs similar to H.264/AVC. Depending on motion vectors of neighbour macroblocks, a skip MB can have one or two candidate motion vectors. If there are two candidate motion vectors and the magnitudes of these motion vectors are different, an extra bit is signalled to choose between the two skip MV candidates. For B pictures, the Skip MBs are always bi-predicted using skip motion vectors rounded to integer.

### Bi-prediction

For B pictures, the reference index indicates whether the macroblock or large-macroblock uses forward, backward or bi-directional prediction, see Figure 1. The reference index is predictively coded from the reference indices of neighbouring macroblocks.

I

P

b

B

B

display

order

0

1

2

3

4

5

6

7

8

b

b

B

b

0

1

2

x : reference index

frame to be predicted

hierarchical

level

Figure 1 Illustration of prediction possibilities of a frame with hierarchical 7 B frame coding

### DIRECT Mode

TENTM supports DIRECT mode, similar to spatial DIRECT mode in H.264/AVC. In addition, the motion vectors used for DIRECT mode are rounded to integer.

## Intra-frame prediction

Intra-prediction is performed within intra macroblocks for blocks of size 16x16, 8x8, and 4x4. The block sizes used for intra prediction conforms to the transform size. Intra prediction is performed using reconstructed pixels of previously coded blocks.

For 16x16 blocks, four prediction modes are used: DC, vertical, horizontal and planar.

For 8x8 blocks, DC prediction and freely selected angular prediction are used.

For 4x4 blocks, three prediction modes are used: DC, vertical, and horizontal.

The vertical, horizontal and angular intra prediction modes use filtered samples that are calculated using a three-tap filter similar to H.264/AVC. The DC prediction is also calculated similarly to H.264/AVC utilizing the reconstructed border pixels of the neighboring block. The 16x16 planar prediction and 8x8 angular prediction are further described in the sections below.

### Planar prediction for Intra 16x16

The planar prediction is designed to be able to reconstruct smooth image segments in a visually pleasing way. It provides maximal continuity of the image plane at the macroblock borders and is able to follow gradual changes of the pixel values by signalling a planar gradient for each macroblock coded in this mode.  When a macroblock is coded in planar mode its bottom-right sample is signalled in the bitstream, the rightmost and bottom samples of the macroblock are linearly interpolated, and the middle samples are bilinearly interpolated from the border samples. When planar mode is signalled, the same algorithm is applied to luminance and both chrominance components separately with individual signalling of the bottom-right samples (16x16 based operation for luminance and 8x8 based for chrominance). The process is illlustrated in the picture below.

Figure 2 Planar prediction of an 8x8 (chrominance) block. Bottom-right sample is signalled in the bitstream, rightmost and bottom samples are interpolated linearly, and the middle samples are interpolated bi-linearly.

### Angular prediction for Intra 8x8

In order to be able to accurately represent directional structures the Intra 8x8 coding mode provides a possibility to predict the blocks at any direction shown below.

Figure 3 Available prediction directions in the angular mode of Intra 8x8 macroblocks.

In the angular mode the prediction direction is given by the displacement of the bottom row of the block and the reference row above the block in the case of vertical prediction or displacement of the rightmost column of the block and reference column left from the block in the case of the horizontal prediction. The displacement is signalled at 1 pixel accuracy. When projection of the predicted pixel falls inbetween reference samples, the predicted value for the pixel is linearly interpolated from the reference samples (at 1/8th pel accuracy). The process is illustrated in the picture below for the sixth row of the block when building vertical prediction at +1 pixel displacement.

Figure 4 An example of angular prediction when operating the sixth row of the block with +1 pixel displacement. Projections of the pixels now fall at the 6/8 sub-pixel location (circles) in the reference row of pixels.

## Spatial Transforms

Spatial transforms of size 64x64, 32x32, 16x16, 8x8, and 4x4 are used. The 64x64 and 32x32 transforms are used for 64x64 and 32x32 macroblocks respectively. The 16x16, 8x8, and 4x4 transforms are used for 16x16 macroblocks and their use is restricted to fall within the motion vector partitions of sizes 16x16, 16x8, 8x16, and 8x8. The one dimensional basis vectors of the 4 pixel and 8 pixel transforms are shown in Figure 5 (16, 32 and 64 pixel transform details could be found in the attached decoder description document).

Note that except for the 4 pixel transform, the basis vectors are not exactly orthogonal and do not have exactly the same norm.  However, the deviations represent no bigger inaccuracy than is introduced by necessary rounding/truncations in the inverse transformations.

13  13  13  13

17   7  -7 -17

13 -13 -13  13

7 -17  17  -7

13  13  13  13  13  13  13  13

17  16  11   3  -3 -11 -16 -17

17   7  -7 -17 -17  -7   7  17

16  -3 -17 -11  11  17   3 -16

13 -13 -13  13  13 -13 -13  13

11 -17   3  16 -16  -3  17 -11

7 -17  17  -7  -7  17 -17   7

3 -11  16 -17  17 -16  11  -3

Figure 5 4x4 and 8x8 transforms

For the 64x64, 32x32, and 16x16 transforms, the basis vectors corresponding to the highest frequency components are set to zero so that only the 8 low frequency basis vectors are used. This implies that quantization operates on block sizes no larger than 8x8.

After horizontal and vertical transforms (forward and inverse), appropriate scaling is applied to ensure that all intermediate results can be stored with 16 bit.

All transforms are defined so that the same norm can be obtained for all transform coefficients by appropriate shifts, regardless of the transform size. This means that all coefficients are scaled equally during quantization, and that only one quantization function is needed for all transform sizes.

In addition, spatially varying transforms (SVT) are used. In SVT, only one transform block is coded inside one macroblock. The transform block is either 8x8, 16x4 or 4x16 and its position inside the macroblock can be varied. This is shown in Figure 6 and Figure 7. When 8x8 SVT is used as shown in Figure 6, the position of the transform block is denoted by (Δx, Δy) where it is selected from the set Φ8×8={(Δx, Δy), Δx=0,…,8, Δy=0; Δx=0,…,8, Δy=8; Δx=0, Δy=1,…,7; Δx=8, Δy=1,…,7}. When 16x4 SVT or 4x16 SVT is used as shown in Figure 7, the position of the transform block is denoted by Δy or Δx and it can be selected form the set Φ16×4={Δy, Δy=0,…,12} or set Φ4×16={Δx, Δx=0,…,12}. The edges between the SVT transform block and the remaining of the macroblock are filtered.

Figure 6 Illustration of SVT, type 0

Figure 7 Illustration of SVT, type 1

If an SVT block straddles over at least two different motion compensation partitions and the absolute difference of x and y components of the motion vector of these two different motion compensation partitions is larger than or equal to 1 pixel, then this SVT position is marked as not available. The selected SVT position and size is coded using an index value using variable length code.

Whether a full set of SVT indexes or only a partial set of SVT indexes is used is specified by one bit in the picture header. The partial set of SVT indexes is a combination of Φ’8×8={(Δx, Δy), Δx=0, 8, Δy=0; Δx=0, 8, Δy=8}, Φ’16×4={Δy, Δy=0, 4, 8, 12} and Φ’4×16={Δx, Δx=0, 4, 8, 12}.

## Quantization

Scalar quantization using a quantization parameter similar to H.264/AVC is used. On the encoder side, zero-memory Rate Distortion Optimized Quantization (RDOQ) is used (no trellis quantization).

## In-loop filtering

An in-loop deblocking filter is applied on block boundaries between transform blocks of size 8x8 or larger. Both a weak and a strong filter is used to be able to remove blocking artifacts efficiently while avoiding removal of desirable structures. The deblocking filter has lower computational complexity than the H.264/AVC deblocking filter since only half the number of block edges are filtered, and also due to less complex logic for enabling/disabling the filter on each 8x8 block edge. The filtering process can be performed in parallel first for each of the vertical block edges and in the second step for each of the horizontal block edges.

## Entropy coding

General

All syntax elements are encoded using either FLC or “UVLC-like” VLC tables. Each syntax element use one out of 10 pre-determined VLC tables that are designed for different probability distributions. All VLC tables are organized in the conventional way with the shortest binary codeword at the beginning.

Encoding of a parameter/event using a VLC table is typically done in three steps:

- Convert the parameter/event value to a table index by using some enumeration scheme.

- Use the table index to generate a code number through lookup in a sorting table. The purpose of the sorting table is to assign code numbers according to increasing probability so that parameters/events with the high probability are assigned a code number with a low value.

- Use the code number to generate a binary codeword by lookup in the pre-determined VLC table.

The VLC encoding process is illustrated in Figure 8, while the VLC decoding process is illustrated in Error: Reference source not found.

event

table_index

code number

binary codeword

e.g. inter/

16x16/

ref=0

e.g. 12

e.g. 7

e.g. 010

enumeration

sorting table

VLC table

Figure 8 VLC encoding process

code number

table_index

event

e.g. 010

e.g. 7

e.g. 12

e.g. inter/

16x16/

ref=0

inverse VLC table

inverse sorting table

’inverse enumeration’

Figure 9 VLC decoding process

Adaptivity

Two different forms of adaptivity are applied to VLC encoding:

- For some parameters/events, the particular VLC table to be used depends on the value of previously encoded parameters/events. A context variable is maintained that indicates expected probability distribution of the events. This variable is updated as the events are encoded and it is used to look-up VLC table number.

- For some parameters/events, the sorting table is made adaptive. This process is most easily described in terms of decoder operations as follows: For each binary codeword that is decoded into a code number, determine the corresponding table index by lookup into an inverse sorting table. Next, swap that entry in the inverse sorting table with the entry immediately above. This mechanism ensures that a table index value that occurs frequently eventually propagates towards the top of the inverse sorting table, corresponding to the most likely value and the shortest binary codeword. On the encoder side, both a sorting table and the corresponding inverse sorting table need to be updated simultaneously.  Updating the inverse sorting table on the decoder side is illustrated in Figure 9.

code number = 7

before sorting:

after sorting:

12

3

9

3

12

9

0

6

7

0

6

7

table_index = 12

Figure 9 Update of inverse sorting table - example

Transform coefficients

Entropy coding of transform coefficients is performed by an improved version of CAVLC as follows:

  - Organize the quantized transform coefficients, each represented by a level and sign in a 1D array using the conventional zig-zag scan.

  - Coefficients are encoded backwards along the 1D array from the most high frequency coefficient towards the DC coefficients.

  - Each event (to be described below) is encoded using an “UVLC-like” table. Different VLC tables are used for different events.

  - First, the last significant coefficient (last_coeff) is coded in two steps:

    - Code the position (in backward scan order) of last_coeff (last_pos) and a flag (levelFlag = level>1) as a combined event.

    - If level>1 code the value of level as a single event

    - Code the sign as one bit

  - If level==1, continue encoding non-zero levels in run-mode.

  - If level>1, switch to level-mode if any of the following is true:

    - Cumulative sum of all level of coefficients coded so far >1 (excluding the level of last_coeff), is bigger than a threshold

    - The position of the current level is bigger than the threshold position

  - In run-mode, encode each non-zero coefficient using the preceding run and levelFlag as a combined event. Then encode the sign and level (if levelFlag==1).

  - In level-mode, encode each coefficient (including zero coefficients) as level and sign.

For the combined last_coeff/levelFlag event, an adaptive sorting table is used.

Side information

The following parameters are coded as a combined event for each macroblock:

- macroblock mode

- motion vector partition

- transform size

- macroblock level CBP information

- reference frame index for P frames

- dqp flag (indicating whether or not dqp is signalled for that MB)

For this combined event, an adaptive sorting table is used.

The following side information parameters are encoded separately as single events:

- block level CBP

- motion vector differences

- reference frame index for B frames

- intra prediction modes

Block level CBP is encoded using an adaptive sorting table.

## Macroblock scan order and syntax to support large macroblocks

Macroblocks of size 32x32 and 64x64 are supported by the MB scan order illustrated in Figure 10. Macroblocks of size 16x16, 32x32, and 64x64, can be combined in various ways as illustrated in Figure 11. For simplicity, both figures assume a picture width of 8 MBs (or 128 pixels).

Figure 10 Scan order to support large macroblocks.

LMB64

LMB32

LMB32

LMB32

LMB32

LMB32

LMB32

Figure 11 Possible combinations of various macroblock sizes.

Signalling to support various macroblock sizes can be described as follows:

  - Runs of skip MB is always signalled on the 16x16 MB level.

  - If a skip run terminates at the upper left corner of a 64x64 MB, one bit is used to signal 16x16 MB size or 32x32/64x64 MB size. If 32x32/64x64 is signalled, one additional bit is used to signal 32x32 MB or 64x64 MB.

  - If a skip run terminates on the upper left corner of a 32x32 MB that is not the upper left corner of a 64x64 MB, one bit is used to signal 16x16 or 32x32 MB.

## Non-normative encoder features

Mode decision

RDO-based mode decision is used to determine the following parameters:

- MB size (16x16, 32x32, or 64x64)

- MB mode (Skip, Direct, Intra, or Inter)

- MV sub partitions (16x16, 16x8, 8x16, 8x8)

- Transform sizes (16x16, 8x8, or 4x4)

- Reference frame index

The cost function is based on either Sum of Squared Differences (SSD) or Sum of Absolute Differences (SAD). Using SAD instead of SSD reduces the implementation cost at a marginal cost in objective performance.  It might however improve the subjective performance somewhat.

Planar intra coding mode is checked only if one of the following conditions holds:

- The intra mode decision results to Intra 16x16 and all the quantized AC coefficients are zero.

- High frequency energy of the original block is below a threshold or there is an edge on the boundary pixels.

If the reconstruction of planar mode is similar to Intra 16x16, then Intra 16x16 mode is chosen.

The corner sample indicating the gradient of the surface is selected by averaging the 2x2 samples in the original image at the lower right corner of the macroblock.

Spatially Varying Transform

The best SVT index is selected from the set of available SVT indexes using RDO to achieve the minimum RD cost of the whole macroblock. The total number of SVT positions and candidates is 58, but only 10 of those are checked in RDO. The 10 candidate positions/sizes are calculated using the SAD of the prediction error for each one of the respective positions and sizes.

Whether full set or partial set of SVT indexes is used for the current picture is determined based on the coded bits of the previous coded picture.

Interpolation

To reduce complexity, only DIF interpolation filters are used for high resolution sequences (resolutions larger or equal to 720p) at high QP’s (larger than 20).

High Quality Reference frames

In order to provide coding gains similar to “Hierarchical P frames” the concept of High Quality Reference (HQR) frames is used. When using the HQR frame scheme, one short-term and a long-term frame are used for prediction. The long-term frames are inserted periodically, and have higher quality (typically QP = QPref – 5) than the other frames. Inter prediction is performed by referring to either the most recent frame (short term) or the most recent HQR (long-term) frame. In the submitted bitstreams we use a period of 4 and 8 for 24/30 fps and 50/60 fps respectively. The reference frame usage with HQR frames using a period of 4 is illustrated in Figure 12.

I

P

P

P

HQR- P

P

P

P

HQR Period = 4

HQR- P

Figure 12 Prediction structure with High Quality Reference (HQR) Frames. The arrows indicate that the choice between inter prediction from the most recent frame (short arrow) and from a HQR frame (long arrow) is possible on MB level. For HQR  and I pictures, SSD based cost function is used in mode decision and for P pictures SAD based cost function is used.

QP values

Similar to the anchor bitstreams, different QP values are used for different frame types. The QP values for various frame types can be expressed relative to a ‘nominal’ QP value as follows:

CS1 bitstreams:

- I frames:		QP-2

- P frames:		QP

- B frames – level 1:	QP+4

- B frames – level 2:	QP+5

- B frames – level 3:	QP+6

CS2 bitstreams:

- I frames:		QP-2

- P frames, HQR:	QP

- P frames, non-HQR:	QP+5

The nominal QP value is kept unchanged for the entire sequence except that it is increased by one once to hit the target bitrates. The frame numbers where the nominal QP value is increased is shown in Table 1

Table 1. Frame number where the nominal QP value increases by one.

| Sequence | Number | Rate | CS1 | CS2 |
| --- | --- | --- | --- | --- |
| Traffic_2560x1600_30_crop | S01 | R5 | 8 | NA |
| Traffic_2560x1600_30_crop | S01 | R4 | 95 | NA |
| Traffic_2560x1600_30_crop | S01 | R3 | 20 | NA |
| Traffic_2560x1600_30_crop | S01 | R2 | 90 | NA |
| Traffic_2560x1600_30_crop | S01 | R1 | 135 | NA |
| PeopleOnStreet_2560x1600_30_crop | S02 | R5 | 35 | NA |
| PeopleOnStreet_2560x1600_30_crop | S02 | R4 | 50 | NA |
| PeopleOnStreet_2560x1600_30_crop | S02 | R3 | 80 | NA |
| PeopleOnStreet_2560x1600_30_crop | S02 | R2 | 80 | NA |
| PeopleOnStreet_2560x1600_30_crop | S02 | R1 | 15 | NA |
| Kimono_1920x1080_24 | S03 | R5 | 180 | 30 |
| Kimono_1920x1080_24 | S03 | R4 | 239 | 140 |
| Kimono_1920x1080_24 | S03 | R3 | 100 | 170 |
| Kimono_1920x1080_24 | S03 | R2 | 40 | 130 |
| Kimono_1920x1080_24 | S03 | R1 | 100 | 225 |
| Parkscene_1920x1080_24 | S04 | R5 | 202 | 12 |
| Parkscene_1920x1080_24 | S04 | R4 | 50 | 150 |
| Parkscene_1920x1080_24 | S04 | R3 | 197 | 175 |
| Parkscene_1920x1080_24 | S04 | R2 | 5 | 50 |
| Parkscene_1920x1080_24 | S04 | R1 | 180 | 60 |
| Cactus_1920x1080_50 | S05 | R5 | 155 | 360 |
| Cactus_1920x1080_50 | S05 | R4 | 100 | 380 |
| Cactus_1920x1080_50 | S05 | R3 | 36 | 385 |
| Cactus_1920x1080_50 | S05 | R2 | 100 | 70 |
| Cactus_1920x1080_50 | S05 | R1 | 76 | 32 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 0 | 25 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 233 | 291 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 216 | 297 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 91 | 207 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 126 | 209 |
| BQTerrace_1920x1080_60 | S07 | R5 | 0 | 250 |
| BQTerrace_1920x1080_60 | S07 | R4 | 160 | 0 |
| BQTerrace_1920x1080_60 | S07 | R3 | 227 | 60 |
| BQTerrace_1920x1080_60 | S07 | R2 | 246 | 326 |
| BQTerrace_1920x1080_60 | S07 | R1 | 465 | 260 |
| BasketballDrill_832x480_50 | S08 | R5 | 445 | 173 |
| BasketballDrill_832x480_50 | S08 | R4 | 73 | 310 |
| BasketballDrill_832x480_50 | S08 | R3 | 305 | 60 |
| BasketballDrill_832x480_50 | S08 | R2 | 387 | 138 |
| BasketballDrill_832x480_50 | S08 | R1 | 224 | 430 |
| BQMall_832x480_60 | S09 | R5 | 484 | 5 |
| BQMall_832x480_60 | S09 | R4 | 50 | 270 |
| BQMall_832x480_60 | S09 | R3 | 300 | 50 |
| BQMall_832x480_60 | S09 | R2 | 550 | 230 |
| BQMall_832x480_60 | S09 | R1 | 400 | 70 |
| PartyScene_832x480_50 | S10 | R5 | 30 | 320 |
| PartyScene_832x480_50 | S10 | R4 | 216 | 230 |
| PartyScene_832x480_50 | S10 | R3 | 15 | 60 |
| PartyScene_832x480_50 | S10 | R2 | 229 | 500 |
| PartyScene_832x480_50 | S10 | R1 | 465 | 50 |
| Racehorses_832x480_30 | S11 | R5 | 139 | 8 |
| Racehorses_832x480_30 | S11 | R4 | 30 | 260 |
| Racehorses_832x480_30 | S11 | R3 | 270 | 285 |
| Racehorses_832x480_30 | S11 | R2 | 130 | 150 |
| Racehorses_832x480_30 | S11 | R1 | 146 | 200 |
| BasketballPass_416x240_50 | S12 | R5 | 25 | 418 |
| BasketballPass_416x240_50 | S12 | R4 | 266 | 381 |
| BasketballPass_416x240_50 | S12 | R3 | 456 | 150 |
| BasketballPass_416x240_50 | S12 | R2 | 348 | 100 |
| BasketballPass_416x240_50 | S12 | R1 | 425 | 130 |
| BQSquare_416x240_60 | S13 | R5 | 224 | 440 |
| BQSquare_416x240_60 | S13 | R4 | 300 | 270 |
| BQSquare_416x240_60 | S13 | R3 | 340 | 591 |
| BQSquare_416x240_60 | S13 | R2 | 10 | 370 |
| BQSquare_416x240_60 | S13 | R1 | 565 | 180 |
| BlowingBubbles_416x240_50 | S14 | R5 | 491 | 312 |
| BlowingBubbles_416x240_50 | S14 | R4 | 16 | 233 |
| BlowingBubbles_416x240_50 | S14 | R3 | 326 | 208 |
| BlowingBubbles_416x240_50 | S14 | R2 | 464 | 490 |
| BlowingBubbles_416x240_50 | S14 | R1 | 108 | 236 |
| Racehorses_416x240_30 | S15 | R5 | 264 | 250 |
| Racehorses_416x240_30 | S15 | R4 | 133 | 0 |
| Racehorses_416x240_30 | S15 | R3 | 150 | 80 |
| Racehorses_416x240_30 | S15 | R2 | 203 | 155 |
| Racehorses_416x240_30 | S15 | R1 | 212 | 175 |
| Vidyo1_1280x720_60 | S16 | R5 | NA | 400 |
| Vidyo1_1280x720_60 | S16 | R4 | NA | 302 |
| Vidyo1_1280x720_60 | S16 | R3 | NA | 10 |
| Vidyo1_1280x720_60 | S16 | R2 | NA | 556 |
| Vidyo1_1280x720_60 | S16 | R1 | NA | 300 |
| Vidyo3_1280x720_60 | S17 | R5 | NA | 300 |
| Vidyo3_1280x720_60 | S17 | R4 | NA | 10 |
| Vidyo3_1280x720_60 | S17 | R3 | NA | 5 |
| Vidyo3_1280x720_60 | S17 | R2 | NA | 419 |
| Vidyo3_1280x720_60 | S17 | R1 | NA | 400 |
| Vidyo4_1280x720_60 | S18 | R5 | NA | 250 |
| Vidyo4_1280x720_60 | S18 | R4 | NA | 350 |
| Vidyo4_1280x720_60 | S18 | R3 | NA | 200 |
| Vidyo4_1280x720_60 | S18 | R2 | NA | 300 |
| Vidyo4_1280x720_60 | S18 | R1 | NA | 90 |

## Future extensions and additional features

Several well-know tools that are known to provide additional coding gain have not been implemented in the current algorithm, and could be added during the course of standardization:

- CABAC:	
The work with TENTM was started with a low complexity perspective.  Hence VLC based entropy coding was chosen.  It represents both higher coding efficiency and lower implementation complexity compared to the CAVLC in H.264/AVC.  But we see no particular reason why CABAC can’t be implemented in TENTM and we expect it to provide a significant gain

- Additional reference frames:
For complexity reasons the number of reference frames in TENTM is limited to 2.  However, we see it as a natural extension to increase this number.  We have done some work on this in combination with different coding structures.  Preliminary results indicate 3-4% additional BDR gain by increasing the average number of reference frames from 2 to about 2.5-3.

- Sub LMB partitions (e.g. 64x32):
Preliminary results indicate 1-2% BDR gain from this.

- Improved MV coding:
Preliminary results indicate that 2% additional BDR gain can be achieved by allowing for more prediction possibilities in the encoding of MVs for constrained set 1 configuration and 1% for constrained set 2.

- Frame-level multi-pass techniques
For complexity and delay reasons, frame-level multi-pass techniques such as those used for AIF and WP in the JM&KTA sw have not been implemented in TENTM. However, such tools can easily be added for high complexity/high delay operating points.

- In-loop Wiener post-filter
We consider this a high complexity tool that can easily be added to TENTM.

Also, some features that provide additional functionality can easily be added:

- High level syntax
High level syntax elements such as slices, SPS, PPS, etc. could be added.

- Scalability
We foresee that some scalability tools could be added.

# Compression performance discussion

## Objective versus subjective compression performance

The main focus has been on producing good subjective quality - especially at high resolutions.  Therefore we also believe that the subjective gain over the anchor bitstreams is significantly higher than the objective gain, especially at resolutions WVGA and above.

## Constraint set 1 configuration relative to Alpha anchor

BDPSNR results are most adequate to use when both anchor and test have similar bitrates but the PSNR differs, see [3]. Average BDrate numbers are also shown but they have less reliability in this case than the BDPSNR numbers but give at least a good hint of the performance at fixed PSNR. The average BDPSNR for luma is computed by integration between the highest and lowest rate point using polynomials fitted to go through the PSNR values similar as in [1]. The average BDrate for luma is also computed similar as in [1]. Since the 5 pair of data points (PSNR,bitrate) is used a fourth order polynomial is used here. The computation of the low and high BDPSNR for luma is in accordance with [2].

### Class A

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Traffic_2560x1600_30_crop | -13,86 | 0,52 | 0,75 | 0,33 |
| PeopleOnStreet_2560x1600_30_crop | -4,00 | 0,22 | 0,55 | -0,04 |

### Class B

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Kimono1_1920x1080_24 | -20,59 | 0,76 | 1,13 | 0,36 |
| ParkScene_1920x1080_24 | -10,62 | 0,41 | 0,71 | 0,11 |
| Cactus_1920x1080_50 | -11,12 | 0,34 | 0,63 | 0,06 |
| BQTerrace_1920x1080_60 | -12,45 | 0,23 | 0,43 | 0,10 |
| BasketballDrive_1920x1080_50 | -15,76 | 0,53 | 0,89 | 0,21 |

### Class C

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQMall_832x480_60 | -16,24 | 0,83 | 1,47 | 0,44 |
| PartyScene_832x480_50 | -16,71 | 0,66 | 0,88 | 0,48 |
| RaceHorses_832x480_30 | -14,68 | 0,65 | 1,03 | 0,34 |
| BasketballDrill_832x480_50 | -17,75 | 0,78 | 0,99 | 0,66 |

### Class D

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQSquare_416x240_60 | -8,92 | 0,33 | 0,58 | 0,07 |
| RaceHorses_416x240_30 | -1,07 | 0,06 | 0,38 | -0,19 |
| BasketballPass_416x240_50 | -8,61 | 0,44 | 0,70 | 0,20 |
| BlowingBubbles_416x240_50 | -9,02 | 0,38 | 0,48 | 0,30 |

### Overall

| Test Class | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Class A | -8,93 | 0,37 | 0,65 | 0,15 |
| Class B | -14,11 | 0,45 | 0,76 | 0,17 |
| Class C | -16,34 | 0,73 | 1,09 | 0,48 |
| Class D | -6,90 | 0,30 | 0,53 | 0,09 |

## Constraint set 2 configuration relative to Beta anchor

### Class B

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Kimono1_1920x1080_24 | -26,76 | 1,08 | 1,39 | 0,72 |
| ParkScene_1920x1080_24 | -15,54 | 0,59 | 0,86 | 0,36 |
| Cactus_1920x1080_50 | -18,86 | 0,60 | 0,90 | 0,29 |
| BQTerrace_1920x1080_60 | -23,77 | 0,50 | 0,88 | 0,17 |
| BasketballDrive_1920x1080_50 | -22,36 | 0,81 | 1,20 | 0,46 |

### Class C

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQMall_832x480_60 | -15,29 | 0,77 | 1,33 | 0,38 |
| PartyScene_832x480_50 | -21,80 | 0,87 | 1,15 | 0,62 |
| RaceHorses_832x480_30 | -5,42 | 0,23 | 0,55 | 0,03 |
| BasketballDrill_832x480_50 | -12,59 | 0,52 | 0,64 | 0,49 |

### Class D

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQSquare_416x240_60 | -7,97 | 0,28 | 0,68 | -0,07 |
| RaceHorses_416x240_30 | 3,14 | -0,15 | 0,06 | -0,30 |
| BasketballPass_416x240_50 | -7,66 | 0,38 | 0,53 | 0,22 |
| BlowingBubbles_416x240_50 | -4,55 | 0,19 | 0,52 | -0,07 |

### Class E

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Vidyo1_1280x720_60 | -32,90 | 1,50 | 2,03 | 0,98 |
| Vidyo3_1280x720_60 | -26,66 | 1,16 | 1,57 | 0,77 |
| Vidyo4_1280x720_60 | -31,44 | 1,29 | 1,95 | 0,80 |

### Overall

| Test Class | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Class E | -30,33 | 1,31 | 1,80 | 0,87 |
| Class B | -21,46 | 0,72 | 1,05 | 0,40 |
| Class C | -13,78 | 0,60 | 0,92 | 0,38 |
| Class D | -4,26 | 0,18 | 0,45 | -0,06 |

## Constraint set 2 configuration relative to Gamma anchor

### Class B

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Kimono1_1920x1080_24 | -44,39 | 2,07 | 2,39 | 1,65 |
| ParkScene_1920x1080_24 | -36,28 | 1,60 | 1,98 | 1,21 |
| Cactus_1920x1080_50 | -40,88 | 1,56 | 1,94 | 1,04 |
| BQTerrace_1920x1080_60 | -52,74 | 1,61 | 2,47 | 0,78 |
| BasketballDrive_1920x1080_50 | -39,64 | 1,70 | 2,31 | 1,14 |

### Class C

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQMall_832x480_60 | -34,15 | 1,88 | 2,53 | 1,44 |
| PartyScene_832x480_50 | -47,02 | 2,10 | 2,16 | 2,05 |
| RaceHorses_832x480_30 | -16,97 | 0,71 | 0,96 | 0,50 |
| BasketballDrill_832x480_50 | -35,90 | 1,70 | 1,79 | 1,68 |

### Class D

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| BQSquare_416x240_60 | -54,09 | 2,31 | 2,52 | 2,05 |
| RaceHorses_416x240_30 | -6,25 | 0,31 | 0,50 | 0,10 |
| BasketballPass_416x240_50 | -21,80 | 1,12 | 1,16 | 1,06 |
| BlowingBubbles_416x240_50 | -33,07 | 1,53 | 1,72 | 1,34 |

### Class E

| Sequence | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Vidyo1_1280x720_60 | -49,40 | 2,61 | 3,41 | 1,91 |
| Vidyo3_1280x720_60 | -45,17 | 2,35 | 2,97 | 1,73 |
| Vidyo4_1280x720_60 | -51,06 | 2,54 | 3,39 | 1,85 |

### Overall

| Test Class | BDrate Average | BDPSNR Average | BDPSNR Low | BDPSNR High |
| --- | --- | --- | --- | --- |
| Class E | -48,54 | 1,31 | 1,80 | 0,87 |
| Class B | -42,79 | 0,72 | 1,05 | 0,40 |
| Class C | -33,51 | 0,60 | 0,92 | 0,38 |
| Class D | -28,80 | 0,18 | 0,45 | -0,06 |

# Complexity analysis

## Encoding time and measurement methodology

The encoder timing results for CS1 and CS2 are shown in Table 2 and Table 3 respectively. To report encoder times, internal timing within the C source code was used (clock function).

Table 2 Encoder times (in seconds) for CS1

| Sequence | Number | Rate | Seconds |
| --- | --- | --- | --- |
| Traffic_2560x1600_30_crop | S01 | R5 | 615,287 |
| Traffic_2560x1600_30_crop | S01 | R4 | 433,284 |
| Traffic_2560x1600_30_crop | S01 | R3 | 319,811 |
| Traffic_2560x1600_30_crop | S01 | R2 | 252,264 |
| Traffic_2560x1600_30_crop | S01 | R1 | 207,945 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R5 | 559,393 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R4 | 425,641 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R3 | 346,908 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R2 | 303,899 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R1 | 268,878 |
| Kimono_1920x1080_24 | S03 | R5 | 570,094 |
| Kimono_1920x1080_24 | S03 | R4 | 426,326 |
| Kimono_1920x1080_24 | S03 | R3 | 333,617 |
| Kimono_1920x1080_24 | S03 | R2 | 263,324 |
| Kimono_1920x1080_24 | S03 | R1 | 211,221 |
| Parkscene_1920x1080_24 | S04 | R5 | 494,622 |
| Parkscene_1920x1080_24 | S04 | R4 | 356,907 |
| Parkscene_1920x1080_24 | S04 | R3 | 262,123 |
| Parkscene_1920x1080_24 | S04 | R2 | 208,210 |
| Parkscene_1920x1080_24 | S04 | R1 | 162,409 |
| Cactus_1920x1080_50 | S05 | R5 | 887,004 |
| Cactus_1920x1080_50 | S05 | R4 | 693,754 |
| Cactus_1920x1080_50 | S05 | R3 | 564,806 |
| Cactus_1920x1080_50 | S05 | R2 | 466,481 |
| Cactus_1920x1080_50 | S05 | R1 | 402,178 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 983,146 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 825,666 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 676,937 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 551,952 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 454,858 |
| BQTerrace_1920x1080_60 | S07 | R5 | 1091,206 |
| BQTerrace_1920x1080_60 | S07 | R4 | 866,413 |
| BQTerrace_1920x1080_60 | S07 | R3 | 704,518 |
| BQTerrace_1920x1080_60 | S07 | R2 | 581,700 |
| BQTerrace_1920x1080_60 | S07 | R1 | 574,852 |
| BasketballDrill_832x480_50 | S08 | R5 | 180,505 |
| BasketballDrill_832x480_50 | S08 | R4 | 129,805 |
| BasketballDrill_832x480_50 | S08 | R3 | 107,014 |
| BasketballDrill_832x480_50 | S08 | R2 | 89,512 |
| BasketballDrill_832x480_50 | S08 | R1 | 80,385 |
| BQMall_832x480_60 | S09 | R5 | 177,525 |
| BQMall_832x480_60 | S09 | R4 | 128,542 |
| BQMall_832x480_60 | S09 | R3 | 106,000 |
| BQMall_832x480_60 | S09 | R2 | 89,854 |
| BQMall_832x480_60 | S09 | R1 | 81,057 |
| PartyScene_832x480_50 | S10 | R5 | 142,082 |
| PartyScene_832x480_50 | S10 | R4 | 106,484 |
| PartyScene_832x480_50 | S10 | R3 | 88,138 |
| PartyScene_832x480_50 | S10 | R2 | 74,535 |
| PartyScene_832x480_50 | S10 | R1 | 66,548 |
| Racehorses_832x480_30 | S11 | R5 | 157,121 |
| Racehorses_832x480_30 | S11 | R4 | 112,583 |
| Racehorses_832x480_30 | S11 | R3 | 93,099 |
| Racehorses_832x480_30 | S11 | R2 | 80,962 |
| Racehorses_832x480_30 | S11 | R1 | 73,428 |
| BasketballPass_416x240_50 | S12 | R5 | 63,859 |
| BasketballPass_416x240_50 | S12 | R4 | 47,890 |
| BasketballPass_416x240_50 | S12 | R3 | 38,500 |
| BasketballPass_416x240_50 | S12 | R2 | 34,573 |
| BasketballPass_416x240_50 | S12 | R1 | 29,579 |
| BQSquare_416x240_60 | S13 | R5 | 77,985 |
| BQSquare_416x240_60 | S13 | R4 | 52,529 |
| BQSquare_416x240_60 | S13 | R3 | 39,654 |
| BQSquare_416x240_60 | S13 | R2 | 33,211 |
| BQSquare_416x240_60 | S13 | R1 | 24,819 |
| BlowingBubbles_416x240_50 | S14 | R5 | 56,767 |
| BlowingBubbles_416x240_50 | S14 | R4 | 39,265 |
| BlowingBubbles_416x240_50 | S14 | R3 | 31,933 |
| BlowingBubbles_416x240_50 | S14 | R2 | 28,547 |
| BlowingBubbles_416x240_50 | S14 | R1 | 23,945 |
| Racehorses_416x240_30 | S15 | R5 | 63,366 |
| Racehorses_416x240_30 | S15 | R4 | 42,384 |
| Racehorses_416x240_30 | S15 | R3 | 33,882 |
| Racehorses_416x240_30 | S15 | R2 | 29,936 |
| Racehorses_416x240_30 | S15 | R1 | 25,287 |
| Class A average |  |  | 373,331 |
| Class B average |  |  | 544,573 |
| Class C average |  |  | 108,259 |
| Class D average |  |  | 40,896 |
| Average |  |  | 271,076 |

Table 3 Encoder times (in seconds) for CS2

| Sequence | Number | Rate | Seconds |
| --- | --- | --- | --- |
| Kimono_1920x1080_24 | S03 | R5 | 390,665 |
| Kimono_1920x1080_24 | S03 | R4 | 324,413 |
| Kimono_1920x1080_24 | S03 | R3 | 258,239 |
| Kimono_1920x1080_24 | S03 | R2 | 210,192 |
| Kimono_1920x1080_24 | S03 | R1 | 168,088 |
| Parkscene_1920x1080_24 | S04 | R5 | 361,650 |
| Parkscene_1920x1080_24 | S04 | R4 | 299,157 |
| Parkscene_1920x1080_24 | S04 | R3 | 255,057 |
| Parkscene_1920x1080_24 | S04 | R2 | 209,396 |
| Parkscene_1920x1080_24 | S04 | R1 | 173,922 |
| Cactus_1920x1080_50 | S05 | R5 | 650,917 |
| Cactus_1920x1080_50 | S05 | R4 | 537,584 |
| Cactus_1920x1080_50 | S05 | R3 | 462,322 |
| Cactus_1920x1080_50 | S05 | R2 | 384,566 |
| Cactus_1920x1080_50 | S05 | R1 | 326,145 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 677,327 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 555,165 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 457,713 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 375,190 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 317,752 |
| BQTerrace_1920x1080_60 | S07 | R5 | 758,165 |
| BQTerrace_1920x1080_60 | S07 | R4 | 641,666 |
| BQTerrace_1920x1080_60 | S07 | R3 | 553,886 |
| BQTerrace_1920x1080_60 | S07 | R2 | 456,496 |
| BQTerrace_1920x1080_60 | S07 | R1 | 378,607 |
| BasketballDrill_832x480_50 | S08 | R5 | 118,792 |
| BasketballDrill_832x480_50 | S08 | R4 | 96,219 |
| BasketballDrill_832x480_50 | S08 | R3 | 80,214 |
| BasketballDrill_832x480_50 | S08 | R2 | 67,422 |
| BasketballDrill_832x480_50 | S08 | R1 | 59,342 |
| BQMall_832x480_60 | S09 | R5 | 130,383 |
| BQMall_832x480_60 | S09 | R4 | 106,796 |
| BQMall_832x480_60 | S09 | R3 | 89,449 |
| BQMall_832x480_60 | S09 | R2 | 76,719 |
| BQMall_832x480_60 | S09 | R1 | 67,266 |
| PartyScene_832x480_50 | S10 | R5 | 116,436 |
| PartyScene_832x480_50 | S10 | R4 | 95,267 |
| PartyScene_832x480_50 | S10 | R3 | 85,081 |
| PartyScene_832x480_50 | S10 | R2 | 74,005 |
| PartyScene_832x480_50 | S10 | R1 | 64,505 |
| Racehorses_832x480_30 | S11 | R5 | 95,470 |
| Racehorses_832x480_30 | S11 | R4 | 77,749 |
| Racehorses_832x480_30 | S11 | R3 | 65,535 |
| Racehorses_832x480_30 | S11 | R2 | 54,833 |
| Racehorses_832x480_30 | S11 | R1 | 48,218 |
| BasketballPass_416x240_50 | S12 | R5 | 43,868 |
| BasketballPass_416x240_50 | S12 | R4 | 35,362 |
| BasketballPass_416x240_50 | S12 | R3 | 30,059 |
| BasketballPass_416x240_50 | S12 | R2 | 27,034 |
| BasketballPass_416x240_50 | S12 | R1 | 23,527 |
| BQSquare_416x240_60 | S13 | R5 | 49,017 |
| BQSquare_416x240_60 | S13 | R4 | 39,943 |
| BQSquare_416x240_60 | S13 | R3 | 34,289 |
| BQSquare_416x240_60 | S13 | R2 | 28,494 |
| BQSquare_416x240_60 | S13 | R1 | 22,619 |
| BlowingBubbles_416x240_50 | S14 | R5 | 46,674 |
| BlowingBubbles_416x240_50 | S14 | R4 | 37,423 |
| BlowingBubbles_416x240_50 | S14 | R3 | 32,213 |
| BlowingBubbles_416x240_50 | S14 | R2 | 28,844 |
| BlowingBubbles_416x240_50 | S14 | R1 | 24,913 |
| Racehorses_416x240_30 | S15 | R5 | 34,881 |
| Racehorses_416x240_30 | S15 | R4 | 28,875 |
| Racehorses_416x240_30 | S15 | R3 | 24,538 |
| Racehorses_416x240_30 | S15 | R2 | 22,354 |
| Racehorses_416x240_30 | S15 | R1 | 19,156 |
| Vidyo1_1280x720_60 | S16 | R5 | 184,296 |
| Vidyo1_1280x720_60 | S16 | R4 | 141,646 |
| Vidyo1_1280x720_60 | S16 | R3 | 114,284 |
| Vidyo1_1280x720_60 | S16 | R2 | 101,632 |
| Vidyo1_1280x720_60 | S16 | R1 | 87,156 |
| Vidyo3_1280x720_60 | S17 | R5 | 165,373 |
| Vidyo3_1280x720_60 | S17 | R4 | 131,335 |
| Vidyo3_1280x720_60 | S17 | R3 | 106,578 |
| Vidyo3_1280x720_60 | S17 | R2 | 94,831 |
| Vidyo3_1280x720_60 | S17 | R1 | 82,944 |
| Vidyo4_1280x720_60 | S18 | R5 | 177,229 |
| Vidyo4_1280x720_60 | S18 | R4 | 139,572 |
| Vidyo4_1280x720_60 | S18 | R3 | 114,128 |
| Vidyo4_1280x720_60 | S18 | R2 | 100,353 |
| Vidyo4_1280x720_60 | S18 | R1 | 87,702 |
| Class B average |  |  | 407,371 |
| Class C average |  |  | 83,485 |
| Class D average |  |  | 31,704 |
| Class E average |  |  | 121,937 |
| Average |  |  | 178,964 |

## Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by JM 17.0

The decoder times for alpha anchor vs. TENTM-CS1, beta anchor vs. TENTM-CS2, and gamma anchor vs. TENTM-CS2 and are shown in Table 4, Table 5, and Table 6 respectively. To report decoder times for the JM17.0 decoder and the TENTM decoder, the time command was used running cygwin under Windows.

Table 4 Decoding times (in seconds) for JM 17.0 (alpha anchor bitstreams) and TENTM-CS1

|  |  |  | YUV output enabled |  |  | YUV output disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Number | Rate | JM17.0 | TENTM | Ratio | JM17.0 | TENTM | Ratio |
| Traffic_2560x1600_30_crop | S01 | R1 | 14,89 | 5,88 | 0,395 | 14,72 | 4,71 | 0,320 |
| Traffic_2560x1600_30_crop | S01 | R2 | 15,06 | 5,58 | 0,371 | 14,83 | 4,97 | 0,335 |
| Traffic_2560x1600_30_crop | S01 | R3 | 16,19 | 6,08 | 0,376 | 16,08 | 5,53 | 0,344 |
| Traffic_2560x1600_30_crop | S01 | R4 | 17,51 | 6,89 | 0,393 | 17,32 | 6,66 | 0,385 |
| Traffic_2560x1600_30_crop | S01 | R5 | 19,89 | 9,03 | 0,454 | 19,69 | 8,71 | 0,442 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R1 | 14,85 | 6,08 | 0,409 | 14,92 | 5,64 | 0,378 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R2 | 15,56 | 6,66 | 0,428 | 15,19 | 6,21 | 0,409 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R3 | 16,59 | 7,09 | 0,427 | 16,65 | 6,97 | 0,419 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R4 | 18,29 | 7,76 | 0,424 | 17,98 | 8,33 | 0,463 |
| PeopleOnStreet_2560x1600_30_crop | S02 | R5 | 20,40 | 9,28 | 0,455 | 20,25 | 10,10 | 0,499 |
| Kimono_1920x1080_24 | S03 | R1 | 18,23 | 5,61 | 0,308 | 18,56 | 5,11 | 0,275 |
| Kimono_1920x1080_24 | S03 | R2 | 16,89 | 6,42 | 0,380 | 16,70 | 5,89 | 0,353 |
| Kimono_1920x1080_24 | S03 | R3 | 19,42 | 7,03 | 0,362 | 19,80 | 6,92 | 0,349 |
| Kimono_1920x1080_24 | S03 | R4 | 20,77 | 8,44 | 0,406 | 20,76 | 8,12 | 0,391 |
| Kimono_1920x1080_24 | S03 | R5 | 22,63 | 9,23 | 0,408 | 22,84 | 9,59 | 0,420 |
| Parkscene_1920x1080_24 | S04 | R1 | 16,16 | 4,85 | 0,300 | 15,75 | 4,04 | 0,257 |
| Parkscene_1920x1080_24 | S04 | R2 | 17,67 | 4,94 | 0,280 | 17,30 | 4,50 | 0,260 |
| Parkscene_1920x1080_24 | S04 | R3 | 19,09 | 5,57 | 0,292 | 18,76 | 5,17 | 0,276 |
| Parkscene_1920x1080_24 | S04 | R4 | 20,32 | 6,05 | 0,298 | 19,90 | 6,07 | 0,305 |
| Parkscene_1920x1080_24 | S04 | R5 | 21,51 | 7,31 | 0,340 | 21,21 | 7,32 | 0,345 |
| Cactus_1920x1080_50 | S05 | R1 | 27,53 | 10,90 | 0,396 | 26,52 | 9,07 | 0,342 |
| Cactus_1920x1080_50 | S05 | R2 | 28,47 | 12,02 | 0,422 | 27,64 | 9,85 | 0,356 |
| Cactus_1920x1080_50 | S05 | R3 | 29,99 | 11,82 | 0,394 | 29,01 | 10,83 | 0,373 |
| Cactus_1920x1080_50 | S05 | R4 | 31,96 | 12,63 | 0,395 | 31,12 | 12,17 | 0,391 |
| Cactus_1920x1080_50 | S05 | R5 | 34,97 | 14,08 | 0,403 | 33,52 | 13,45 | 0,401 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 34,03 | 12,05 | 0,354 | 33,19 | 10,47 | 0,315 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 36,86 | 12,68 | 0,344 | 35,69 | 11,76 | 0,330 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 38,98 | 13,80 | 0,354 | 37,73 | 13,28 | 0,352 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 41,02 | 15,49 | 0,378 | 40,07 | 15,26 | 0,381 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 43,07 | 16,78 | 0,390 | 42,18 | 17,17 | 0,407 |
| BQTerrace_1920x1080_60 | S07 | R1 | 43,13 | 14,11 | 0,327 | 41,91 | 8,78 | 0,209 |
| BQTerrace_1920x1080_60 | S07 | R2 | 44,00 | 13,44 | 0,305 | 42,97 | 9,65 | 0,225 |
| BQTerrace_1920x1080_60 | S07 | R3 | 45,17 | 14,58 | 0,323 | 43,78 | 10,84 | 0,248 |
| BQTerrace_1920x1080_60 | S07 | R4 | 46,36 | 14,77 | 0,319 | 44,92 | 11,87 | 0,264 |
| BQTerrace_1920x1080_60 | S07 | R5 | 48,34 | 15,30 | 0,317 | 47,00 | 13,43 | 0,286 |
| BasketballDrill_832x480_50 | S08 | R1 | 4,08 | 2,09 | 0,512 | 3,80 | 1,46 | 0,384 |
| BasketballDrill_832x480_50 | S08 | R2 | 4,22 | 2,10 | 0,498 | 4,07 | 1,57 | 0,386 |
| BasketballDrill_832x480_50 | S08 | R3 | 4,53 | 2,30 | 0,508 | 4,38 | 1,84 | 0,420 |
| BasketballDrill_832x480_50 | S08 | R4 | 4,93 | 2,59 | 0,525 | 4,72 | 2,16 | 0,458 |
| BasketballDrill_832x480_50 | S08 | R5 | 5,56 | 3,07 | 0,552 | 5,36 | 2,69 | 0,502 |
| BQMall_832x480_60 | S09 | R1 | 5,94 | 1,95 | 0,328 | 5,33 | 1,52 | 0,285 |
| BQMall_832x480_60 | S09 | R2 | 5,95 | 2,04 | 0,343 | 5,77 | 1,48 | 0,256 |
| BQMall_832x480_60 | S09 | R3 | 6,39 | 2,21 | 0,346 | 6,14 | 1,66 | 0,270 |
| BQMall_832x480_60 | S09 | R4 | 6,80 | 2,52 | 0,371 | 6,59 | 1,99 | 0,302 |
| BQMall_832x480_60 | S09 | R5 | 7,39 | 2,91 | 0,394 | 7,11 | 2,32 | 0,326 |
| PartyScene_832x480_50 | S10 | R1 | 4,71 | 1,84 | 0,391 | 4,47 | 1,18 | 0,264 |
| PartyScene_832x480_50 | S10 | R2 | 5,10 | 1,80 | 0,353 | 4,88 | 1,29 | 0,264 |
| PartyScene_832x480_50 | S10 | R3 | 5,50 | 1,87 | 0,340 | 5,25 | 1,49 | 0,284 |
| PartyScene_832x480_50 | S10 | R4 | 5,95 | 2,15 | 0,361 | 5,80 | 1,66 | 0,286 |
| PartyScene_832x480_50 | S10 | R5 | 6,53 | 2,41 | 0,369 | 6,36 | 2,01 | 0,316 |
| Racehorses_832x480_30 | S11 | R1 | 3,41 | 1,29 | 0,378 | 3,26 | 1,04 | 0,319 |
| Racehorses_832x480_30 | S11 | R2 | 3,76 | 1,48 | 0,394 | 3,57 | 1,15 | 0,322 |
| Racehorses_832x480_30 | S11 | R3 | 3,99 | 1,52 | 0,381 | 3,83 | 1,31 | 0,342 |
| Racehorses_832x480_30 | S11 | R4 | 4,29 | 1,77 | 0,413 | 4,21 | 1,51 | 0,359 |
| Racehorses_832x480_30 | S11 | R5 | 4,82 | 2,10 | 0,436 | 4,68 | 1,77 | 0,378 |
| BasketballPass_416x240_50 | S12 | R1 | 1,31 | 0,62 | 0,473 | 1,26 | 0,59 | 0,468 |
| BasketballPass_416x240_50 | S12 | R2 | 1,45 | 0,68 | 0,469 | 1,38 | 0,68 | 0,493 |
| BasketballPass_416x240_50 | S12 | R3 | 1,57 | 0,74 | 0,471 | 1,51 | 0,74 | 0,490 |
| BasketballPass_416x240_50 | S12 | R4 | 1,76 | 0,82 | 0,466 | 1,68 | 0,88 | 0,524 |
| BasketballPass_416x240_50 | S12 | R5 | 2,09 | 0,96 | 0,459 | 2,04 | 1,06 | 0,520 |
| BQSquare_416x240_60 | S13 | R1 | 1,71 | 0,56 | 0,327 | 1,65 | 0,51 | 0,309 |
| BQSquare_416x240_60 | S13 | R2 | 1,82 | 0,67 | 0,368 | 1,80 | 0,59 | 0,328 |
| BQSquare_416x240_60 | S13 | R3 | 1,93 | 0,71 | 0,368 | 1,95 | 0,65 | 0,333 |
| BQSquare_416x240_60 | S13 | R4 | 2,15 | 0,82 | 0,381 | 2,12 | 0,81 | 0,382 |
| BQSquare_416x240_60 | S13 | R5 | 2,57 | 0,98 | 0,381 | 2,46 | 1,04 | 0,423 |
| BlowingBubbles_416x240_50 | S14 | R1 | 1,23 | 0,57 | 0,463 | 1,18 | 0,51 | 0,432 |
| BlowingBubbles_416x240_50 | S14 | R2 | 1,40 | 0,60 | 0,429 | 1,31 | 0,56 | 0,427 |
| BlowingBubbles_416x240_50 | S14 | R3 | 1,46 | 0,64 | 0,438 | 1,37 | 0,62 | 0,453 |
| BlowingBubbles_416x240_50 | S14 | R4 | 1,65 | 0,73 | 0,442 | 1,62 | 0,74 | 0,457 |
| BlowingBubbles_416x240_50 | S14 | R5 | 2,04 | 0,96 | 0,471 | 2,01 | 0,93 | 0,463 |
| Racehorses_416x240_30 | S15 | R1 | 1,06 | 0,51 | 0,481 | 0,98 | 0,46 | 0,469 |
| Racehorses_416x240_30 | S15 | R2 | 1,12 | 0,49 | 0,438 | 1,12 | 0,54 | 0,482 |
| Racehorses_416x240_30 | S15 | R3 | 1,29 | 0,59 | 0,457 | 1,23 | 0,59 | 0,480 |
| Racehorses_416x240_30 | S15 | R4 | 1,46 | 0,60 | 0,411 | 1,37 | 0,68 | 0,496 |
| Racehorses_416x240_30 | S15 | R5 | 1,74 | 0,76 | 0,437 | 1,68 | 0,82 | 0,488 |
| Class A average |  |  | 16,923 | 7,033 | 0,416 | 16,763 | 6,783 | 0,405 |
| Class B average |  |  | 30,663 | 10,796 | 0,352 | 29,953 | 9,624 | 0,321 |
| Class C average |  |  | 5,193 | 2,101 | 0,405 | 4,979 | 1,655 | 0,332 |
| Class D average |  |  | 1,641 | 0,701 | 0,427 | 1,586 | 0,700 | 0,441 |
| Average |  |  | 14,299 | 5,283 | 0,369 | 13,970 | 4,741 | 0,339 |
| Total (minutes) |  |  | 17,874 | 6,604 |  | 17,463 | 5,926 |  |

Table 5 Decoding times (in seconds) for JM 17.0 (beta anchor bitstreams) and TENTM-CS2

|  |  |  | YUV output enabled |  |  | YUV output disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Number | Rate | JM17.0 | TENTM | Ratio | JM17.0 | TENTM | Ratio |
| Kimono_1920x1080_24 | S03 | R1 | 13,49 | 6,53 | 0,484 | 12,65 | 5,60 | 0,443 |
| Kimono_1920x1080_24 | S03 | R2 | 14,66 | 6,67 | 0,455 | 14,18 | 6,40 | 0,451 |
| Kimono_1920x1080_24 | S03 | R3 | 15,83 | 7,55 | 0,477 | 15,28 | 7,13 | 0,467 |
| Kimono_1920x1080_24 | S03 | R4 | 17,26 | 8,22 | 0,476 | 16,67 | 8,05 | 0,483 |
| Kimono_1920x1080_24 | S03 | R5 | 18,64 | 8,59 | 0,461 | 18,17 | 8,96 | 0,493 |
| Parkscene_1920x1080_24 | S04 | R1 | 12,24 | 4,85 | 0,396 | 11,90 | 4,02 | 0,338 |
| Parkscene_1920x1080_24 | S04 | R2 | 13,94 | 4,94 | 0,354 | 13,33 | 4,53 | 0,340 |
| Parkscene_1920x1080_24 | S04 | R3 | 15,38 | 5,47 | 0,356 | 14,75 | 5,19 | 0,352 |
| Parkscene_1920x1080_24 | S04 | R4 | 16,72 | 6,00 | 0,359 | 16,13 | 6,13 | 0,380 |
| Parkscene_1920x1080_24 | S04 | R5 | 18,34 | 6,75 | 0,368 | 17,86 | 7,09 | 0,397 |
| Cactus_1920x1080_50 | S05 | R1 | 22,13 | 11,26 | 0,509 | 21,02 | 10,07 | 0,479 |
| Cactus_1920x1080_50 | S05 | R2 | 24,46 | 12,10 | 0,495 | 23,49 | 11,06 | 0,471 |
| Cactus_1920x1080_50 | S05 | R3 | 26,20 | 12,80 | 0,489 | 25,14 | 12,06 | 0,480 |
| Cactus_1920x1080_50 | S05 | R4 | 28,01 | 13,60 | 0,486 | 27,06 | 13,15 | 0,486 |
| Cactus_1920x1080_50 | S05 | R5 | 30,52 | 14,22 | 0,466 | 29,43 | 14,35 | 0,488 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 25,42 | 13,15 | 0,517 | 24,46 | 11,33 | 0,463 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 28,11 | 13,97 | 0,497 | 27,15 | 12,92 | 0,476 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 31,55 | 15,22 | 0,482 | 30,62 | 14,73 | 0,481 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 34,35 | 16,72 | 0,487 | 33,11 | 16,62 | 0,502 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 36,41 | 17,83 | 0,490 | 35,33 | 18,07 | 0,511 |
| BQTerrace_1920x1080_60 | S07 | R1 | 31,91 | 13,05 | 0,409 | 30,24 | 8,85 | 0,293 |
| BQTerrace_1920x1080_60 | S07 | R2 | 33,44 | 13,99 | 0,418 | 32,12 | 10,05 | 0,313 |
| BQTerrace_1920x1080_60 | S07 | R3 | 35,22 | 13,96 | 0,396 | 33,88 | 10,49 | 0,310 |
| BQTerrace_1920x1080_60 | S07 | R4 | 37,84 | 14,29 | 0,378 | 36,53 | 11,71 | 0,321 |
| BQTerrace_1920x1080_60 | S07 | R5 | 40,71 | 14,46 | 0,355 | 39,32 | 12,80 | 0,326 |
| BasketballDrill_832x480_50 | S08 | R1 | 3,43 | 2,24 | 0,653 | 2,93 | 1,65 | 0,563 |
| BasketballDrill_832x480_50 | S08 | R2 | 3,30 | 2,21 | 0,670 | 3,16 | 1,91 | 0,604 |
| BasketballDrill_832x480_50 | S08 | R3 | 3,74 | 2,66 | 0,711 | 3,52 | 2,23 | 0,634 |
| BasketballDrill_832x480_50 | S08 | R4 | 4,29 | 3,07 | 0,716 | 4,11 | 2,58 | 0,628 |
| BasketballDrill_832x480_50 | S08 | R5 | 5,08 | 3,36 | 0,661 | 4,93 | 2,98 | 0,604 |
| BQMall_832x480_60 | S09 | R1 | 4,58 | 1,95 | 0,426 | 3,96 | 1,46 | 0,369 |
| BQMall_832x480_60 | S09 | R2 | 4,43 | 2,05 | 0,463 | 4,29 | 1,59 | 0,371 |
| BQMall_832x480_60 | S09 | R3 | 4,97 | 2,35 | 0,473 | 4,74 | 1,82 | 0,384 |
| BQMall_832x480_60 | S09 | R4 | 5,47 | 2,46 | 0,450 | 5,33 | 2,02 | 0,379 |
| BQMall_832x480_60 | S09 | R5 | 6,31 | 2,83 | 0,448 | 6,06 | 2,35 | 0,388 |
| PartyScene_832x480_50 | S10 | R1 | 3,43 | 1,88 | 0,548 | 3,27 | 1,38 | 0,422 |
| PartyScene_832x480_50 | S10 | R2 | 3,91 | 2,10 | 0,537 | 3,68 | 1,46 | 0,397 |
| PartyScene_832x480_50 | S10 | R3 | 4,46 | 2,07 | 0,464 | 4,35 | 1,65 | 0,379 |
| PartyScene_832x480_50 | S10 | R4 | 5,13 | 2,18 | 0,425 | 4,97 | 1,82 | 0,366 |
| PartyScene_832x480_50 | S10 | R5 | 6,06 | 2,41 | 0,398 | 5,83 | 2,10 | 0,360 |
| Racehorses_832x480_30 | S11 | R1 | 2,82 | 1,42 | 0,504 | 2,65 | 1,17 | 0,442 |
| Racehorses_832x480_30 | S11 | R2 | 2,96 | 1,52 | 0,514 | 2,93 | 1,34 | 0,457 |
| Racehorses_832x480_30 | S11 | R3 | 3,36 | 1,65 | 0,491 | 3,21 | 1,49 | 0,464 |
| Racehorses_832x480_30 | S11 | R4 | 3,94 | 1,79 | 0,454 | 3,72 | 1,70 | 0,457 |
| Racehorses_832x480_30 | S11 | R5 | 4,57 | 1,99 | 0,435 | 4,35 | 1,85 | 0,425 |
| BasketballPass_416x240_50 | S12 | R1 | 1,13 | 0,65 | 0,575 | 1,09 | 0,62 | 0,569 |
| BasketballPass_416x240_50 | S12 | R2 | 1,29 | 0,68 | 0,527 | 1,26 | 0,70 | 0,556 |
| BasketballPass_416x240_50 | S12 | R3 | 1,40 | 0,73 | 0,521 | 1,37 | 0,74 | 0,540 |
| BasketballPass_416x240_50 | S12 | R4 | 1,70 | 0,79 | 0,465 | 1,57 | 0,87 | 0,554 |
| BasketballPass_416x240_50 | S12 | R5 | 2,01 | 0,92 | 0,458 | 1,98 | 0,99 | 0,500 |
| BQSquare_416x240_60 | S13 | R1 | 1,40 | 0,57 | 0,407 | 1,32 | 0,51 | 0,386 |
| BQSquare_416x240_60 | S13 | R2 | 1,56 | 0,60 | 0,385 | 1,48 | 0,60 | 0,405 |
| BQSquare_416x240_60 | S13 | R3 | 1,66 | 0,62 | 0,373 | 1,56 | 0,67 | 0,429 |
| BQSquare_416x240_60 | S13 | R4 | 1,96 | 0,70 | 0,357 | 1,87 | 0,73 | 0,390 |
| BQSquare_416x240_60 | S13 | R5 | 2,37 | 0,82 | 0,346 | 2,30 | 0,88 | 0,383 |
| BlowingBubbles_416x240_50 | S14 | R1 | 1,18 | 0,56 | 0,475 | 1,09 | 0,54 | 0,495 |
| BlowingBubbles_416x240_50 | S14 | R2 | 1,29 | 0,62 | 0,481 | 1,26 | 0,59 | 0,468 |
| BlowingBubbles_416x240_50 | S14 | R3 | 1,42 | 0,67 | 0,472 | 1,35 | 0,67 | 0,496 |
| BlowingBubbles_416x240_50 | S14 | R4 | 1,76 | 0,71 | 0,403 | 1,62 | 0,76 | 0,469 |
| BlowingBubbles_416x240_50 | S14 | R5 | 2,15 | 0,82 | 0,381 | 2,07 | 0,92 | 0,444 |
| Racehorses_416x240_30 | S15 | R1 | 0,93 | 0,48 | 0,516 | 0,93 | 0,48 | 0,516 |
| Racehorses_416x240_30 | S15 | R2 | 1,04 | 0,49 | 0,471 | 1,04 | 0,53 | 0,510 |
| Racehorses_416x240_30 | S15 | R3 | 1,15 | 0,53 | 0,461 | 1,17 | 0,62 | 0,530 |
| Racehorses_416x240_30 | S15 | R4 | 1,40 | 0,59 | 0,421 | 1,37 | 0,67 | 0,489 |
| Racehorses_416x240_30 | S15 | R5 | 1,68 | 0,68 | 0,405 | 1,70 | 0,78 | 0,459 |
| Vidyo1_1280x720_60 | S16 | R1 | 6,39 | 3,68 | 0,576 | 5,88 | 2,38 | 0,405 |
| Vidyo1_1280x720_60 | S16 | R2 | 7,14 | 3,63 | 0,508 | 6,47 | 2,46 | 0,380 |
| Vidyo1_1280x720_60 | S16 | R3 | 7,28 | 4,44 | 0,610 | 6,75 | 2,65 | 0,393 |
| Vidyo1_1280x720_60 | S16 | R4 | 7,89 | 4,27 | 0,541 | 7,42 | 3,02 | 0,407 |
| Vidyo1_1280x720_60 | S16 | R5 | 8,90 | 4,74 | 0,533 | 8,42 | 3,41 | 0,405 |
| Vidyo3_1280x720_60 | S17 | R1 | 6,44 | 3,58 | 0,556 | 5,86 | 2,13 | 0,363 |
| Vidyo3_1280x720_60 | S17 | R2 | 6,84 | 3,54 | 0,518 | 6,24 | 2,29 | 0,367 |
| Vidyo3_1280x720_60 | S17 | R3 | 7,69 | 3,77 | 0,490 | 6,69 | 2,49 | 0,372 |
| Vidyo3_1280x720_60 | S17 | R4 | 7,89 | 4,32 | 0,548 | 7,22 | 2,82 | 0,391 |
| Vidyo3_1280x720_60 | S17 | R5 | 9,01 | 4,47 | 0,496 | 8,37 | 3,24 | 0,387 |
| Vidyo4_1280x720_60 | S18 | R1 | 6,78 | 3,74 | 0,552 | 6,17 | 2,30 | 0,373 |
| Vidyo4_1280x720_60 | S18 | R2 | 7,09 | 3,79 | 0,535 | 6,61 | 2,57 | 0,389 |
| Vidyo4_1280x720_60 | S18 | R3 | 7,50 | 4,10 | 0,547 | 6,91 | 2,71 | 0,392 |
| Vidyo4_1280x720_60 | S18 | R4 | 8,03 | 4,33 | 0,539 | 7,64 | 2,98 | 0,390 |
| Vidyo4_1280x720_60 | S18 | R5 | 9,01 | 4,66 | 0,517 | 8,51 | 3,37 | 0,396 |
| Class B average |  |  | 24,054 | 11,048 | 0,459 | 23,163 | 10,054 | 0,434 |
| Class C average |  |  | 4,097 | 2,210 | 0,539 | 3,913 | 1,828 | 0,467 |
| Class D average |  |  | 1,524 | 0,662 | 0,434 | 1,470 | 0,694 | 0,472 |
| Class E average |  |  | 7,592 | 4,071 | 0,536 | 7,011 | 2,721 | 0,388 |
| Average |  |  | 10,667 | 4,933 | 0,462 | 10,205 | 4,283 | 0,420 |
| Total (minutes) |  |  | 14,223 | 6,578 |  | 13,606 | 5,710 |  |

Table 6 Decoding times (in seconds) for JM 17.0 (gamma anchor bitstreams) and TENTM-CS2

|  |  |  | YUV output enabled |  |  | YUV output disabled |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | Number | Rate | JM17.0 | TENTM | Ratio | JM17.0 | TENTM | Ratio |
| Kimono_1920x1080_24 | S03 | R1 | 13,32 | 6,53 | 0,490 | 12,35 | 5,60 | 0,453 |
| Kimono_1920x1080_24 | S03 | R2 | 14,08 | 6,67 | 0,474 | 13,44 | 6,40 | 0,476 |
| Kimono_1920x1080_24 | S03 | R3 | 15,16 | 7,55 | 0,498 | 14,50 | 7,13 | 0,492 |
| Kimono_1920x1080_24 | S03 | R4 | 16,59 | 8,22 | 0,495 | 15,97 | 8,05 | 0,504 |
| Kimono_1920x1080_24 | S03 | R5 | 18,03 | 8,59 | 0,476 | 18,12 | 8,96 | 0,494 |
| Parkscene_1920x1080_24 | S04 | R1 | 11,77 | 4,85 | 0,412 | 11,31 | 4,02 | 0,355 |
| Parkscene_1920x1080_24 | S04 | R2 | 13,18 | 4,94 | 0,375 | 12,65 | 4,53 | 0,358 |
| Parkscene_1920x1080_24 | S04 | R3 | 14,41 | 5,47 | 0,380 | 13,94 | 5,19 | 0,372 |
| Parkscene_1920x1080_24 | S04 | R4 | 15,89 | 6,00 | 0,378 | 15,53 | 6,13 | 0,395 |
| Parkscene_1920x1080_24 | S04 | R5 | 17,69 | 6,75 | 0,382 | 17,19 | 7,09 | 0,412 |
| Cactus_1920x1080_50 | S05 | R1 | 21,52 | 11,26 | 0,523 | 20,60 | 10,07 | 0,489 |
| Cactus_1920x1080_50 | S05 | R2 | 23,22 | 12,10 | 0,521 | 22,10 | 11,06 | 0,500 |
| Cactus_1920x1080_50 | S05 | R3 | 24,75 | 12,80 | 0,517 | 23,85 | 12,06 | 0,506 |
| Cactus_1920x1080_50 | S05 | R4 | 27,08 | 13,60 | 0,502 | 25,80 | 13,15 | 0,510 |
| Cactus_1920x1080_50 | S05 | R5 | 28,81 | 14,22 | 0,494 | 27,90 | 14,35 | 0,514 |
| BasketballDrive_1920x1080_50 | S06 | R1 | 24,33 | 13,15 | 0,540 | 23,35 | 11,33 | 0,485 |
| BasketballDrive_1920x1080_50 | S06 | R2 | 27,01 | 13,97 | 0,517 | 25,91 | 12,92 | 0,499 |
| BasketballDrive_1920x1080_50 | S06 | R3 | 29,23 | 15,22 | 0,521 | 28,08 | 14,73 | 0,525 |
| BasketballDrive_1920x1080_50 | S06 | R4 | 31,66 | 16,72 | 0,528 | 30,62 | 16,62 | 0,543 |
| BasketballDrive_1920x1080_50 | S06 | R5 | 33,97 | 17,83 | 0,525 | 32,94 | 18,07 | 0,549 |
| BQTerrace_1920x1080_60 | S07 | R1 | 30,04 | 13,05 | 0,434 | 28,84 | 8,85 | 0,307 |
| BQTerrace_1920x1080_60 | S07 | R2 | 31,77 | 13,99 | 0,440 | 30,28 | 10,05 | 0,332 |
| BQTerrace_1920x1080_60 | S07 | R3 | 33,43 | 13,96 | 0,418 | 32,01 | 10,49 | 0,328 |
| BQTerrace_1920x1080_60 | S07 | R4 | 35,75 | 14,29 | 0,400 | 34,19 | 11,71 | 0,342 |
| BQTerrace_1920x1080_60 | S07 | R5 | 38,22 | 14,46 | 0,378 | 36,55 | 12,80 | 0,350 |
| BasketballDrill_832x480_50 | S08 | R1 | 3,07 | 2,24 | 0,730 | 2,74 | 1,65 | 0,602 |
| BasketballDrill_832x480_50 | S08 | R2 | 3,24 | 2,21 | 0,682 | 2,99 | 1,91 | 0,639 |
| BasketballDrill_832x480_50 | S08 | R3 | 3,46 | 2,66 | 0,769 | 3,24 | 2,23 | 0,688 |
| BasketballDrill_832x480_50 | S08 | R4 | 3,91 | 3,07 | 0,785 | 3,71 | 2,58 | 0,695 |
| BasketballDrill_832x480_50 | S08 | R5 | 4,57 | 3,36 | 0,735 | 4,35 | 2,98 | 0,685 |
| BQMall_832x480_60 | S09 | R1 | 4,04 | 1,95 | 0,483 | 3,82 | 1,46 | 0,382 |
| BQMall_832x480_60 | S09 | R2 | 4,27 | 2,05 | 0,480 | 4,08 | 1,59 | 0,390 |
| BQMall_832x480_60 | S09 | R3 | 4,75 | 2,35 | 0,495 | 4,39 | 1,82 | 0,415 |
| BQMall_832x480_60 | S09 | R4 | 5,07 | 2,46 | 0,485 | 4,78 | 2,02 | 0,423 |
| BQMall_832x480_60 | S09 | R5 | 5,71 | 2,83 | 0,496 | 5,49 | 2,35 | 0,428 |
| PartyScene_832x480_50 | S10 | R1 | 3,13 | 1,88 | 0,601 | 2,93 | 1,38 | 0,471 |
| PartyScene_832x480_50 | S10 | R2 | 3,63 | 2,10 | 0,579 | 3,43 | 1,46 | 0,426 |
| PartyScene_832x480_50 | S10 | R3 | 4,36 | 2,07 | 0,475 | 4,04 | 1,65 | 0,408 |
| PartyScene_832x480_50 | S10 | R4 | 5,21 | 2,18 | 0,418 | 4,61 | 1,82 | 0,395 |
| PartyScene_832x480_50 | S10 | R5 | 5,49 | 2,41 | 0,439 | 5,35 | 2,10 | 0,393 |
| Racehorses_832x480_30 | S11 | R1 | 2,60 | 1,42 | 0,546 | 2,41 | 1,17 | 0,485 |
| Racehorses_832x480_30 | S11 | R2 | 2,77 | 1,52 | 0,549 | 2,73 | 1,34 | 0,491 |
| Racehorses_832x480_30 | S11 | R3 | 3,18 | 1,65 | 0,519 | 3,05 | 1,49 | 0,489 |
| Racehorses_832x480_30 | S11 | R4 | 3,51 | 1,79 | 0,510 | 3,40 | 1,70 | 0,500 |
| Racehorses_832x480_30 | S11 | R5 | 4,02 | 1,99 | 0,495 | 3,91 | 1,85 | 0,473 |
| BasketballPass_416x240_50 | S12 | R1 | 1,20 | 0,65 | 0,542 | 1,03 | 0,62 | 0,602 |
| BasketballPass_416x240_50 | S12 | R2 | 1,17 | 0,68 | 0,581 | 1,12 | 0,70 | 0,625 |
| BasketballPass_416x240_50 | S12 | R3 | 1,24 | 0,73 | 0,589 | 1,17 | 0,74 | 0,632 |
| BasketballPass_416x240_50 | S12 | R4 | 1,48 | 0,79 | 0,534 | 1,40 | 0,87 | 0,621 |
| BasketballPass_416x240_50 | S12 | R5 | 1,76 | 0,92 | 0,523 | 1,68 | 0,99 | 0,589 |
| BQSquare_416x240_60 | S13 | R1 | 1,35 | 0,57 | 0,422 | 1,27 | 0,51 | 0,402 |
| BQSquare_416x240_60 | S13 | R2 | 1,45 | 0,60 | 0,414 | 1,37 | 0,60 | 0,438 |
| BQSquare_416x240_60 | S13 | R3 | 1,57 | 0,62 | 0,395 | 1,48 | 0,67 | 0,453 |
| BQSquare_416x240_60 | S13 | R4 | 1,74 | 0,70 | 0,402 | 1,71 | 0,73 | 0,427 |
| BQSquare_416x240_60 | S13 | R5 | 2,10 | 0,82 | 0,390 | 2,01 | 0,88 | 0,438 |
| BlowingBubbles_416x240_50 | S14 | R1 | 1,03 | 0,56 | 0,544 | 0,99 | 0,54 | 0,545 |
| BlowingBubbles_416x240_50 | S14 | R2 | 1,10 | 0,62 | 0,564 | 1,10 | 0,59 | 0,536 |
| BlowingBubbles_416x240_50 | S14 | R3 | 1,29 | 0,67 | 0,519 | 1,21 | 0,67 | 0,554 |
| BlowingBubbles_416x240_50 | S14 | R4 | 1,48 | 0,71 | 0,480 | 1,40 | 0,76 | 0,543 |
| BlowingBubbles_416x240_50 | S14 | R5 | 1,77 | 0,82 | 0,463 | 1,76 | 0,92 | 0,523 |
| Racehorses_416x240_30 | S15 | R1 | 0,87 | 0,48 | 0,552 | 0,84 | 0,48 | 0,571 |
| Racehorses_416x240_30 | S15 | R2 | 0,95 | 0,49 | 0,516 | 0,96 | 0,53 | 0,552 |
| Racehorses_416x240_30 | S15 | R3 | 1,10 | 0,53 | 0,482 | 0,98 | 0,62 | 0,633 |
| Racehorses_416x240_30 | S15 | R4 | 1,28 | 0,59 | 0,461 | 1,24 | 0,67 | 0,540 |
| Racehorses_416x240_30 | S15 | R5 | 1,52 | 0,68 | 0,447 | 1,51 | 0,78 | 0,517 |
| Vidyo1_1280x720_60 | S16 | R1 | 6,39 | 3,68 | 0,576 | 5,94 | 2,38 | 0,401 |
| Vidyo1_1280x720_60 | S16 | R2 | 7,00 | 3,63 | 0,519 | 6,42 | 2,46 | 0,383 |
| Vidyo1_1280x720_60 | S16 | R3 | 7,30 | 4,44 | 0,608 | 6,74 | 2,65 | 0,393 |
| Vidyo1_1280x720_60 | S16 | R4 | 7,91 | 4,27 | 0,540 | 7,37 | 3,02 | 0,410 |
| Vidyo1_1280x720_60 | S16 | R5 | 8,98 | 4,74 | 0,528 | 8,33 | 3,41 | 0,409 |
| Vidyo3_1280x720_60 | S17 | R1 | 6,55 | 3,58 | 0,547 | 5,92 | 2,13 | 0,360 |
| Vidyo3_1280x720_60 | S17 | R2 | 6,84 | 3,54 | 0,518 | 6,28 | 2,29 | 0,365 |
| Vidyo3_1280x720_60 | S17 | R3 | 7,16 | 3,77 | 0,527 | 6,50 | 2,49 | 0,383 |
| Vidyo3_1280x720_60 | S17 | R4 | 7,80 | 4,32 | 0,554 | 7,11 | 2,82 | 0,397 |
| Vidyo3_1280x720_60 | S17 | R5 | 8,51 | 4,47 | 0,525 | 8,00 | 3,24 | 0,405 |
| Vidyo4_1280x720_60 | S18 | R1 | 6,67 | 3,74 | 0,561 | 6,10 | 2,30 | 0,377 |
| Vidyo4_1280x720_60 | S18 | R2 | 7,02 | 3,79 | 0,540 | 6,50 | 2,57 | 0,395 |
| Vidyo4_1280x720_60 | S18 | R3 | 7,31 | 4,10 | 0,561 | 6,78 | 2,71 | 0,400 |
| Vidyo4_1280x720_60 | S18 | R4 | 8,33 | 4,33 | 0,520 | 7,37 | 2,98 | 0,404 |
| Vidyo4_1280x720_60 | S18 | R5 | 8,76 | 4,66 | 0,532 | 8,20 | 3,37 | 0,411 |
| Class B average |  |  | 22,793 | 11,048 | 0,485 | 21,918 | 10,054 | 0,459 |
| Class C average |  |  | 3,803 | 2,210 | 0,581 | 3,594 | 1,828 | 0,509 |
| Class D average |  |  | 1,373 | 0,662 | 0,482 | 1,312 | 0,694 | 0,529 |
| Class E average |  |  | 7,502 | 4,071 | 0,543 | 6,904 | 2,721 | 0,394 |
| Average |  |  | 10,136 | 4,933 | 0,487 | 9,666 | 4,283 | 0,443 |
| Total (minutes) |  |  | 13,515 | 6,578 |  | 12,888 | 5,710 |  |

## Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

Encoding/decoding time for each sequence/rate point is measured using a single core on the following platform:

- OS: 			Windows 7, 64 bit

- Processor:		Intel Core i5 670 @ 3,47 GHz

- Cache:		L1 Data: 	2x32 KBytes

L1 Inst.: 	2x32 KBytes

L2:		2x256 Kbytes

L3:		4 Mbytes

- RAM:		DDR3, 6MB

- Compiler:		Intel(R) C++ Compiler Professional Edition 11.1

## Expected memory usage of encoder

The minimum requirement is mainly given by the number of reference frames (2) plus the reconstructed frame. Depending on the preferred CPU/memory/compression tradeoffs one might choose to pre-compute the result of quarter pixel interpolation and/or use estimated interpolation during ME.

## Expected memory usage of decoder

The minimum requirement is mainly given by the number of reference frames (2) plus the reconstructed frame. We expect most implementations to do quarter pixel interpolations on-the-fly implying no significant contribution to memory requirements from the quarter pixel interpolation.

## Complexity characteristics of encoder motion estimation and motion segmentation selection

For each MB, motion estimation is done independently for each:

- reference frame (0 and 1)

- MV partition (16x16, 16x8, 8x16, 8x8)

For each combination of the above, ME is performed as follows:

- Full integer search (16x4) on reference frame 0 for 16x16 MV partition. For CS1, this search window scales with the prediction distance for P frames.

- Candidate integer search among 4 MV candidates

- Diamond search with adaptive number of layers. Less than 3 layers is used in most cases.

- Half-pixel search among 8 candidates

- Quarter-pixel search among 9 candidates

- For the 9 innermost quarter pixel positions, the half-pixel and quarter-pixel search is repeated for the separable interpolation filter.

Two early exit techniques for good complexity/compression trade-offs are used:

- Early skip exit based on residual of skip MV residual.

- If small MV partitions provide identical MVs, don’t do ME for larger MV partitions.

## Complexity characteristics of decoder motion compensation

The worst case complexity of TENTM interpolation is the same as H.264/AVC interpolation (due to the 6x6 separable filters), whereas average interpolation complexity of TENTM is less than H.264/AVC (due to DIF). 6x6 separable filters are disabled for high resolution sequences at low bitrates, making the worst case interpolation complexity of TENTM lower than that of H.264/AVC.

The memory bandwidth due to motion compensation is less in TENTM than in H.264/AVC. This is because motion partitions smaller than 8x8 are not used.

The interpolation complexity for B pictures is less in TENTM than in H.264/AVC as the SKIP and DIRECT modes use motion vectors rounded to integer. In addition, since temporal DIRECT mode is not utilizied, the memory requirement for TENTM is somewhat less than in H.264/AVC.

## Complexity characteristics of encoder intra-frame prediction type selection

4x4 transform:	Less prediction modes (6) than H.264/AVC.

8x8 transform:	Higher than H.264/AVC due to the additional prediction directions. However, TENTM encoder uses SAD based decision for P&B pictures instead of RDO and same coding efficiency is observed, making the additional complexity negligible.

16x16 transform:	Similar to H.264/AVC.

Intra prediction mode selection is done as follows:

- RDO-based for I pictures

- SAD-based for P&B pictures

## Complexity characteristics of decoder intra-frame prediction operation

Similar to H.264/AVC.

## Complexity characteristics of encoder transforms and transform type selection

The transform size selection is RDO-based. Only macroblocks of size 16x16 have more than one transform size (16x16, 8x8, and 4x4).

The complexity of the larger transform sizes (>= 16x16) is significantly reduced by using only the 8 basis vectors in each dimension. This means that the number of multiply/adds per pixel (horizontal plus vertical) for the various transform sizes (without exploiting symmetry properties) can be expressed as follows:

4x4 transform:	(4*4*4 + 4*4*4)/(4x4) = 8

8x8 transform:	(8*8*8  + 8*8*8)/(8x8) = 16

16x16 transform:	(8*16*16 + 8*8*16)/(16*16) = 12

32x32 transform:	(8*32*32 + 8*8*32)/(32*32) = 10

64x64 transform:	(8*64*64 + 8*8*64)/(64x64) = 9

This implies that for the larger transform sizes (>8x8), the number of operations per pixel decreases with the transform size.

The transforms (and inverse transforms) have been designed with appropriate shift between the first and second pass. Together with appropriate scaling in the inverse quantization, this ensures that all intermediate and output values can be represented with 16 bit resolution.

## Complexity characteristics of decoder inverse transform operation

Same as for the forward transform.

## Complexity characteristics of encoder quantization and quantization type selection

RDOQ is used for quantization. The choice of representation level is done on a sample-by-sample basis without any delayed decision trellis-like decisions. This implies that the only additional complexity as compared to traditional quantization is the determination of number of bits, and evaluation of the Lagrangian cost function.

## Complexity characteristics of decoder inverse quantization

This is a simple multiplication with a QP-dependent scale factor for each non-zero coefficient.

## Complexity characteristics of encoder in-loop filtering type selection

There is no frame- or macroblock-level decision between different in-loop filter types.

## Complexity characteristics of decoder in-loop filtering operation

In-loop filtering is similar to the H.264/AVC deblocking filter in structure, but with significantly lower computational complexity. There are two reasons for this:

  - No filtering is done on 4x4 block edges inside an 8x8 block.

  - The decision on whether to apply filtering or not (similar to computation of the filterSamplesFlag in H.264/AVC) is performed once for each 8x8 block edge rather than once for each row/column.

## Complexity characteristics of encoder entropy coding type selection

There is no selection between different entropy coding types.

## Complexity characteristics of decoder entropy decoding operation

Entropy decoding of transform coefficients is lower than CAVLC in H.264/AVC. This is because TENTM algorithm requires less branching (if-elseif-else) than CAVLC, and uses a higher degree of repetitive operations when going through the 1D scan of transform coefficients.

## Degree of capability for encoder parallel processing

This is similar to H.264/AVC JM encoder used to generate gamma anchor.

## Degree of capability for decoder parallel processing

This is similar or slightly better than H.264/AVC Baseline Profile. Frame-level parallelism can be achieved by breaking dependencies between macroblocks.

For inter macroblocks, MB-level parallelism can be achieve by processing transform blocks of size 4x4 and 8x8 in parallel.

The in-loop deblocking filter is easier to parallelize as the deblocking filter does not process 4x4 block edges (no pixel values across the edges overlaps).

Since arithmetic coding is not utilized, TENTM does not suffer from the parallelism issues associated with CABAC.

## Non-normative encoder complexity issues

In terms of reference frames and multi- vs. single pass, the encoder complexity is closer to the gamma anchor than to the beta anchor, since only single-pass and 2 reference frames is used.

# Algorithmic characteristics

## Random access characteristics

CS1 bitstreams: Random access is supported by intra frames that are inserted with the same intervals as in the alpha anchor.

CS2 bitstreams. There is no random access. Decoding from the first frame is necessary.

## Delay characteristics

Frame reordering is used within the limitations of the CfP for CS1 and CS2 respectively.

Since frame-level multipass is not used, the encoding delay for CS2 is limited by the size of the largest macroblock size. This means that the encoder can start transmitting bits after having received 64 pixel rows from the input device.

For use of HQR frames (see Section 2.l), there is some additional delay due to bitrate fluctuations. This can be reduced significantly applying HQR at different frames for the top and bottom half of each frame (see Figure 13). In the CfP bitstream submission we choose not to use this technique due to the static QP requirement of the CfP.

top half of frame:

bottom half of frame:

Figure 13 Reduced delay HQR

# Software implementation description

TENTM is implemented using clean ANSI-C programming language, without any low level optimizations.

# Highlighted aspects discussion

# Closing remarks

Several contributions have been made earlier arguing a need for i) significant improvement in coding efficiency, ii) a low complexity operating point (with complexity not higher than H.264/AVC Baseline Profile) with high coding efficiency and iii) faster standardization for the low complexity operating point[4][5][6][7][8][9]. TENTM was designed to fulfill these requirements.

We believe TENTM is a good candidate for standardization of both a low complexity operating point and a high coding efficiency operating point, due to its clean and low-complexity design.

# Patent rights declaration(s)

Tandberg Telecom may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LM Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Bjøntegaard, “Calculation of average PSNR differences between RD-Curves,” ITU-T SG16 Q.6 Document, VCEG-M33, Austin, US, April 2001

- G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document, VCEG-AI11, Berlin, Germany, July 2008

- K. Andersson, R Sjöberg, A. Norkin, ”Reliability measure for BD measurements” , ITU-T SG16 Q.6 Document, VCEG-AL22, Geneva, Switzerland, July 2009

- Nokia, Tandberg, “Toward a H.265 low complexity profile”, COM 16 – C 145 – E, January 2009

- Nokia, “On requirements for next generation video coding standard”, COM 16 – C 110 – E January 2009.

- Nokia, “Requirements for high-performance video standards”, M16207, January 2009, Lausanne, Switzerland.

- LM Ericsson, Tandberg Telecom, Nokia, “Recommended course of action for EPVC”, COM 16 – C 228 – E, October 2009.

- LM Ericsson, Nokia, Tandberg Telecom, "Comments on HVC development", M16939 October 2009, Xian China

- Tandberg, “Proposal for early standardization of an NGVC Baseline Profile”, COM 16 – C 355 – E, October 2009