| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B047 |
| --- | --- |

| Title: | TE1: Fast techniques to improve self derivation of motion estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution presents the techniques to improve the derivation of motion vector at decoder (DMVD) side with the aim to increase coding efficiency of B pictures as well as to realize parallel friendly implementation of a video decoder. With the motion vector (MV) information self derived at video decoder side, the transmission of these MVs from video encoder side to video decoder side is skipped and thus better coding efficiency can be achieved. The proposed self derivation of motion estimation (SDME) techniques are friendly to the parallel implementation by utilizing on the temporal correlation among the available pixels in the previously-decoded reference pictures, instead of operating on the previously reconstructed pixels of the neighboring area of the current picture, which poses an inherent decoding ordering causal relationship among decoding blocks of current picture. Experiments have demonstrated that the BD bitrate improvement on top of ITU-T/VCEG Key Technology Area (KTA) Reference Software platform with an overall about 7.56% improvement on the hierarchical IbBbBbBbP coding structure under the test conditions with the block size 16x16 from the Tool Experiment group at JCT-VC. A fast version of the candidate based scheme has demonstrated an overall saving of the SDME computation time by about 93.9% with an overall 7.52% BD bitrate improvement. In addition, a parallel-friendly version to further remove the MV prediction neighbor dependency has shown an overall 6.78% improvement in BD bitrate reduction.

# Introduction

Various decoder-side motion vector derivation (DMVD) techniques [1-8] have been proposed to improve the coding efficiency of hybrid based video encoder. Template match (TM) techniques were first proposed in [1] by utilizing the spatial correlation of block motions. It defines an L-shape template area around current target coding block and performs L-shape template based ME on reference pictures. The multi-reference based TM [2], multi-hypothesis based TM [3], and 1/8 pixel accuracy based TM [5] were further proposed to improve the RD performance. To reduce the ME complexity of TM, a fast TM version was presented in [4] by performing candidate based predictive search. TM based DMVD techniques have been able to greatly improve the coding, but it suffers the strong neighbor dependency in decoding process, i.e., decoding operation of a DMVD coded block can only be started when all of the neighboring blocks which contain the template pixels have been completely decoded. This poses a challenging problem in the implementation of DMVD-based technique and is unfriendly to a parallel implementation of a video decoder. In [6], mirror-based ME was proposed to derive motion vectors for B_SKIP mode by employing the temporal correlation of block motions, in which full search ME is performed within a 128x128 search window between the two quarter pixel accuracy planes of the nearest forward and backward reference pictures. The size of the matching block is 64x64 for a 16x16 MB, and the computational complexity of the ME process is extremely high. In [7], a refinement MC scheme is proposed, in which a new MC block is performing decoder side ME between two references, and then the average of this new MC block and the traditional MC block obtained using the transmitted MV is used as the final MC block.

A typical TM based DMVD technique inherits the strong neighbor dependency which poses a technical challenge in the parallel design of the decoding process.

- Latency in the initialization of TM process: TM process is conditional on the reconstructed spatial neighboring pixels in the current decoding picture. The pixels in the L-shape template needed to be fully decoded prior to the decoding of the current block, that is, the decoding operation of a DMVD coded block can only be started upon when all of the neighboring blocks consisting of the template pixels have been completely decoded.

- Unbalanced complexity in MV prediction parsing: DMVD coding mode introduces a sudden increase of complexity in the decoding process of MV prediction. Unlike the typical simple parsing process of MV decoding, the decoding of MV prediction process for a neighbor block of the DMVD coding mode requires heavy computation to execute a TM process to search candidate area.

In our response to the HEVC CFP [8], a mirror-based self derivation of motion estimation (SDME) technique was presented to improve the coding efficiency of the bi-prediction modes of B pictures. We attack the above-mentioned issue #1 by removing the spatial neighbor dependency of SDME in [9] and pursue a study for the above-mentioned issue #2 by introducing a new prediction process in this proposal. Section 2 provides an introduction of the proposed mirror-based SDME, and Section 3 describes the proposed fast techniques to improve the SDME. The performance data about the coding efficiency and coding complexity of the proposed scheme is contained in Section 4. Section 5 concludes this proposal.

# Overview of SDME

The proposed SDME is a mirror-based ME technique to improve the coding efficiency of B picture for the next generation of video coding standard. Figure 1 shows the block diagram of a typical hybrid-based video encoder with SDME module being added. The SDME module performs motion search only on reference pictures, and the produced motion vector could be applied to current target coding block for motion compensation when the best RDO performance is achieved with SDME mode during the phase of coding mode selection. On the video decoder side, the decoder performs the identical SDME process to produce the motion compensated predictions for the target coding block when the said target coding block is encoded in SDME mode.

Figure 1 - Block diagram of hybrid video encoding with SDME being supported

## SDME process

The pictorial illustration of B-picture based Mirror-based ME is depicted in Figure 2. The application of mirror-based ME is based on two assumptions.

- The inter picture block motions can be treated as a linear trajectory in a short time interval.

- The current coding block and its spatial neighboring blocks in a stable area have a very similar motion.

Figure 2 - SDME for B pictures

FW Ref and BW Ref with display time denoted as tfand tb are the closest forward and backward reference pictures to the current B picture at display time tcin Figure 2. PMV0 and PMV1, illustrated as the dotted line arrows, are the two specified motion vectors pointing from current block to the search centers in the forward and backward reference pictures respectively. The midpoint of the line occurs at the display time （tf+tb)2. The mirror-based ME is performed within a pre-defined search window starting from the search center. Shown in the bottom part of Figure 2, the mirror-based ME is symmetrical by the center point: if the search path in forward search window is from left to right and from top to bottom, the search path in backward search window is correspondingly from right to left and from bottom to top. A 2-stage small full search scheme is employed for integer pixel motion search process followed by fractional pixel refinement motion search around the best integer pixel location obtained from integer based ME. The sum of absolute difference (SAD) between the two reference blocks in forward and backward search windows is used as the measurement metric of mirror-based ME. The motion vector pair,  MV0 and MV1shown in Figure 2, with the minimum SAD is selected as the output of mirror-based ME. If multiple motion vector pairs achieve the same minimum SAD values, the one with the shortest distance to the search center will be selected.

With the assumption that the current target coding block and its spatial neighboring blocks have a very similar motion in a stable area, the output motion vectorsMV0 and MV1can be shifted to cross the current block position to obtain the final forward and backward motion vectors, FMV0 and FMV1, for coding the current target coding block as in (1). Round() is a rounding operator. All of the motion vectors MV0, MV1, FMV0 and FMV1 are in the degree of quarter pixel precision.

FMV0=Round((MV0−MV1)∗(tc−tf)(tb−tf))	(1)

With FMV0 and FMV1, two prediction blocks can be obtained from the forward and backward reference pictures and the weighted average of the two prediction blocks are used as the final bi-directional predictions of current target coding block. The weighting factor can be set according to the temporal picture distances between current picture and the reference pictures (=1/2 in this GOP structure IbBbBbBbP).

In mirror-based ME, multiple search centers can be specified in one reference picture, i.e., the PMV0can have multiple values. If multiple search centers are specified, mirror-based ME is performed for each search center to get one best motion vector pair candidate. Among the multiple motion vector pair candidates, the one with the minimum SAD will be selected to calculate the FMV0 and FMV1as in (1).

## Extended ME block

In the regular process of ME, the ME block size is the same size as the current block size. In mirror-based ME, the ME blocks, as shown in Figure 2, are in reference pictures. The extended ME block, as shown in Figure 3, can be used to improve the accuracy of the produced MVs for current block where the reference block is in the same size with current coding block, and the extended ME block is produced by extending the reference block with W pixels in both left and right borders and with H pixels in both top and bottom borders.

Figure 3 - Extended ME block

In our simulation, the mirror-based ME is only applied to the block partitions whose sizes are not smaller than 8x8. For each available block partition, the corresponding extended ME block size is shown in Table 1. If current block is bigger than 16x16, the ME block is in the same size of current block.

Table 1 - Extended block size for different block partitions

| Current block size | 16x16 | 16x8 | 8x16 | 8x8 |
| --- | --- | --- | --- | --- |
| Extended ME block size | 32x32 | 32x16 | 16x32 | 16x16 |

## SDME based coding modes

The SDME based motion vector derivation method can be used as a new candidate in addition to the regular ME based one. The encoder will use the one with the smaller RD cost as the final motion vector derivation method for current target coding block. In bitstream syntax, a SDME flag is added for each block to signal the motion vector derivation method if the mirror-based ME is available for this block coding mode. And, the bits for encoding this flag should be counted in RD cost. Currently, the SDME is only applied to the bi-prediction coding modes whose block sizes are not smaller than 8x8. For other coding modes, the regular ME method is applied and no SDME flag is needed.

# Fast techniques to improve SDME

## Candidates based SMDE

Full search based SDME is used in [8] as a typical usage. To reduce the complexity of SDME, candidate based mirror-based ME can be employed, in which the SDME module first check the selected candidate motion vector pairs, and then perform a small range fractional pixel refinement ME around the best candidate motion vector pair.  With a few candidate locations instead of full search locations, the ME complexity can be greatly reduced as a cost of the slightly reduced coding efficiency. There are 9 candidates are used in our implementation: The zero MV and the other eight candidates produced from the spatial neighbor MVs and temporal neighbor MVs as described in Table 2 where the description of the forward MV is described and the associated mirror backward MV can be derived accordingly.

Table 2 - Description of the candidates

| Base MV | 0 | the zero MV |
| --- | --- | --- |
| Predicted MVs | 1 | the FW predicted MV pred_fmv |
|  | 2 | the mirrored MV of the BW predicted MV pred_bmv |
|  | 3 | (pred_fmv - pred_bmv) / 2 |
| Spatial neighbor MVs Scaled by POC distances | 4 | MV of left neighbor block: a. If it is forward predicted, use its FW MV mv_fw b. If it is backward predicted, use the mirrored MV of its BW MV mv_bw c. If it is bi-predicted, use (mv_fw - mv_bw) / 2 d. If it is intra coded, this candidate will be unavailable |
|  | 5 | MV of top neighbor block (as described in 4) |
|  | 6 | MV of top-left neighbor block (as described in 4) |
| Temporal Neighbor MVs Scaled by POC distances | 7 | MV of FW collocated neighbor block a. If it has BW MV, scale this BW MV and then mirror it b. Otherwise, this candidate will be unavailable |
|  | 8 | MV of BW collocated neighbor block a. If it has FW MV, scale this FW MV b. Otherwise, this candidate will be unavailable |

## MV prediction dependency on DMVD blocks

The unbalanced complexity in MV prediction parsing poses a technical challenge in the realization of a parallel friendly decoder. If the neighboring MBs were signaled as a DMVD coding mode, the decoding of MV predictor has to wait until the task of the ME process of the spatial neighbor blocks to be complete. To eliminate the MV dependency on the DMVD block, we propose

- If a neighboring block is coded in DMVD model, the MV predictor based on the median operator of MPEG-4 Part 10 AVC/H.264 will be in use, instead of the DMVD-based MV, to predict the MVs of current block.

# Performance

## Compression performance

The proposed SDME techniques were implemented onto KTA2.6r1 and evaluated under the test conditions defined by the DMVD TE group. The proposed SDME is for improving the coding efficiency of B picture and the testing points defined under constraint set 1 (CS1) are considered. Some key testing configurations are listed as follows.

- Hierarchical coding structure of IbBbBbBbP

- QPI=27, 30, 34, and 38; QPP=QPI+1; QPB= QPI+2

- Intra MDDT On, High Precision Filter On, Adaptive Loop Filter On

- RDO_Q Off

Table 3 shows the full search based SDME performance compared with the TE1 anchors. The BD measurements [10] in PSNR increase and bitrate reduction were employed. In the case of “UseExtMB = 0”, the bitrate reduction is about 7.56% on average and up to 15.81% for Kimono1 sequence. With the larger MB size of 32x32 is enabled (“UseExtMB = 0”), the average BD bitrate reduction is about 3.74%.

Table 3 - Full search based SDME performance compared with TE1 anchors

|  | Sequence | UseExtMB = 1 |  | UseExtMB = 0 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (dB) | BD_Bitrate (%) | BD_PSNR (dB) | BD_Bitrate (%) |
| Class A | Traffic | 0.198 | -5.044 | 0.391 | -9.351 |
|  | PeopleOnStreet | 0.236 | -5.195 | 0.362 | -7.809 |
| Class B | Kimono1 | 0.300 | -9.075 | 0.588 | -15.808 |
|  | ParkScene | 0.157 | -4.631 | 0.373 | -9.994 |
|  | Cactus | 0.175 | -5.686 | 0.324 | -9.730 |
|  | BasketballDrive | 0.074 | -2.601 | 0.265 | -8.309 |
|  | BQTerrace | 0.048 | -2.169 | 0.177 | -7.246 |
| Class C | BasketballDrill | 0.037 | -0.964 | 0.109 | -2.801 |
|  | BQMall | 0.162 | -3.770 | 0.237 | -5.339 |
|  | PartyScene | 0.057 | -1.384 | 0.156 | -3.672 |
|  | RaceHorses | 0.023 | -0.618 | 0.121 | -3.048 |
| Average of Class A |  | 0.217 | -5.120 | 0.376 | -8.580 |
| Average of Class B |  | 0.151 | -4.832 | 0.345 | -10.217 |
| Average of Class C |  | 0.070 | -1.684 | 0.156 | -3.715 |
| Overall Average |  | 0.133 | -3.740 | 0.282 | -7.555 |

Table 4 shows the candidates based fast SDME performance compared with TE1 anchors. Compared to the performance of full search based scheme, the BD bitrate performance of candidate based fast SDME remains similar. (The average difference is only about 0.04% in the case of “UseExtMB = 0” and is about 0.2% in the case of “UseExtMB = 1”).

Table 4 - Candidates based fast SDME performance compared with TE1 anchors

|  | Sequence | UseExtMB = 1 |  | UseExtMB = 0 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (dB) | BD_Bitrate (%) | BD_PSNR (dB) | BD_Bitrate (%) |
| Class A | Traffic | 0.158 | -4.023 | 0.350 | -8.382 |
|  | PeopleOnStreet | 0.200 | -4.363 | 0.321 | -6.955 |
| Class B | Kimono1 | 0.314 | -9.483 | 0.609 | -16.390 |
|  | ParkScene | 0.169 | -4.961 | 0.391 | -10.480 |
|  | Cactus | 0.175 | -5.686 | 0.334 | -10.019 |
|  | BasketballDrive | 0.087 | -3.049 | 0.285 | -8.940 |
|  | BQTerrace | 0.045 | -2.060 | 0.178 | -7.261 |
| Class C | BasketballDrill | 0.003 | -0.088 | 0.082 | -2.112 |
|  | BQMall | 0.145 | -3.348 | 0.238 | -5.362 |
|  | PartyScene | 0.066 | -1.593 | 0.166 | -3.903 |
|  | RaceHorses | 0.010 | -0.289 | 0.117 | -2.932 |
| Average of Class A |  | 0.179 | -4.193 | 0.335 | -7.668 |
| Average of Class B |  | 0.158 | -5.048 | 0.359 | -10.618 |
| Average of Class C |  | 0.056 | -1.330 | 0.151 | -3.577 |
| Overall Average |  | 0.125 | -3.540 | 0.279 | -7.521 |

With the scheme of eliminating the MV prediction dependency on the DMVD MVs on top of the candidates based fast SDME framework, the coding performance is shown in Table 5. Compared to the TE1 anchors, the neighbor-DMVD-dependency-free scheme preserves the coding efficiency BD bitrate saving about 6.78% for the case of UseExtMB = 0 and about 2.76% for the case of UseExtMB=1.

Table 5 - Candidates based fast SDME performance compared with TE1 anchors (No MV prediction dependency on self derived MVs)

|  | Sequence | UseExtMB = 1 |  | UseExtMB = 0 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BD_PSNR (dB) | BD_Bitrate (%) | BD_PSNR (dB) | BD_Bitrate (%) |
| Class A | Traffic | 0.133 | -3.469 | 0.339 | -8.334 |
|  | PeopleOnStreet | 0.161 | -3.523 | 0.279 | -6.127 |
| Class B | Kimono1 | 0.302 | -9.176 | 0.621 | -16.855 |
|  | ParkScene | 0.138 | -4.122 | 0.376 | -10.177 |
|  | Cactus | 0.151 | -4.913 | 0.321 | -9.699 |
|  | BasketballDrive | 0.049 | -1.738 | 0.226 | -7.170 |
|  | BQTerrace | 0.016 | -0.745 | 0.131 | -5.450 |
| Class C | BasketballDrill | -0.023 | 0.586 | 0.049 | -1.265 |
|  | BQMall | 0.123 | -2.863 | 0.205 | -4.666 |
|  | PartyScene | 0.044 | -1.090 | 0.151 | -3.584 |
|  | RaceHorses | -0.026 | 0.651 | 0.046 | -1.223 |
| Average of Class A |  | 0.147 | -3.496 | 0.309 | -7.230 |
| Average of Class B |  | 0.131 | -4.139 | 0.335 | -9.870 |
| Average of Class C |  | 0.030 | -0.679 | 0.113 | -2.684 |
| Overall Average |  | 0.097 | -2.764 | 0.249 | -6.777 |

## Encoding and decoding complexities

The detailed encoding and decoding time of the testing points are listed in the spread sheet included in this proposal. In our testing, the encoding jobs are running on the machines of difference performances in a cluster. Therefore the absolute total encoding time is not a reliable metric for complexity comparison. Instead, the comparison among the total coding time compared to the portion of time spent in SDME module is used as the metric to estimate complexity. The data estimates that the full search based SDME increases the encoding complexity by about 100.67% on average and the candidates based SDME increases the encoding complexity by only about 5.58% on average. Comparing with the full search scheme, the candidates based ME saves the portion of SDME time by a huge amount.

All of our decoding tests are performed on the same workstation. With this, the absolute decoding time is used as the metric for complexity comparison. We compared the decoding time of candidates based fast DMVD decoder with the decoding time of the anchor decoder. The testing results estimate that the decoding complexity is increased by about 71.1% on average.

# Conclusions

This contribution presents the techniques to improve the derivation of motion vector at decoder side with the aim to increase coding efficiency of B pictures as well as to realize parallel friendly implementation of a video decoder. The proposed self derivation of motion estimation techniques are friendly to the parallel implementation by utilizing on the temporal correlation among the available pixels in the previously-decoded reference pictures, instead of operating on the previously reconstructed pixels of the neighboring area of the current picture, which poses an inherent decoding ordering causal relationship among decoding blocks of current picture. Experiments have demonstrated that the BD bitrate improvement on top of ITU-T/VCEG Key Technology Area (KTA) Reference Software platform with an overall about 7.56% improvement on the hierarchical IbBbBbBbP coding structure under the test conditions with the block size 16x16 from the Tool Experiment group at JCT-VC. A fast version of the candidate based scheme has demonstrated an overall saving of the SDME computation time by about 93.9% with an overall 7.52% BD bitrate improvement. In addition, a parallel-friendly version to further remove the MV prediction neighbor dependency has shown an overall 6.78% improvement in BD bitrate reduction.

# References

- S. Kamp, M. Evertz, M. Wien, “Decoder side motion vector derivation,” ITU-T Q.6/SG16 33rd Meeting Document VCEG-AG16, October 2007.

- S. Kamp, M. Evertz, M. Wien, “Decoder side motion vector derivation with multiple reference pictures,” ITU-T Q.6/SG16 34th Meeting Document VCEG-AH15, January 2008.

- S. Kamp, M. Wien, “Multi-hypothesis prediction with decoder side motion vector derivation,” JVT 27th Meeting Document JVT-AA040, April 2008.

- S. Kamp, B. Bross, M. Evertz, M. Wien, “Fast decoder side motion vector derivation,” ITU-T Q.6/SG16 36th Meeting Document VCEG-AJ18, October 2008.

- Y. Huang, C. Fu, Y. Tsai, and et al, “A technical description of MediaTek’s proposal to the JCT-VC CfP,” JCT-VC 1st Meeting Document JCTVC-A109, April 2010.

- T. Murakami, and S. Saito, “Advanced B skip mode with decoder-side motion estimation,” ITU-T Q.6/SG16 37th Meeting Document VCEG-AK12, April 2009.

- S. Sakazume, M. Ueda, S. Fukushima, and etc., “Description of video coding technology proposed by JVC”, meeting Document JCTVC-A108, April 2010

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “Description of video coding technology proposal: self derivation of motion estimation and adaptive (Wiener) loop filtering”, Meeting Document JCTVC-A106, April 2010.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “Self derivation of motion estimation techniques to improve video coding efficiency” SPIE conference, San Diego, August, 2010. To appear.

- Gisle Bjøntegaard, “Calculation of average PSNR differences between RD curves,” ITU-T Q.6/SG16 13th Meeting Document VCEG-M33, April 2001.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).