| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B049 |
| --- | --- |

| Title: | Geometry Motion Partition |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peisong Chen, Wei-jung Chien, Rahul Panchal, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Results of geometry motion partition are presented in this contribution. The average BR rate reduction is 4.26% for IPPP configuration and 3.93% for hierarchical-B configuration.

# Introduction

In the first JCTVC meeting in Dresden, April 2010, Tool Experiment 3 was setup to improve temporal prediction and geometric block partition [2]. Geometry motion partition was part of the submission JCT-VC A121 [3] to CfP [1]. This tool has been selected to be incorporated into TMuC software. In this contribution, separate results of geometry motion partition are provided. The average BR rate reduction is 4.26% for IPPP configuration and 3.93% for hierarchical-B configuration.

# Geometry motion partition

In geometry motion partition, a square block is divided into 2 regions. One motion vector is sent for each region. The boundary separating the 2 regions is defined by a straight line. Assuming the origin is at the center of the block, each geometry partition is defined by a line passing through the origin that is perpendicular to the line defining the partition boundary. This is shown in Fig. 1. The geometry partition is defined by the angle subtended by the perpendicular line with the X axis θ and the distance of the partition line from the originρ. The equation of the line defining the partition boundary can be specified as

y=−1tanθx+ρsinθ=mx+c

We use two lookup tables, one to store the slope, , and the other to store the Y-intercept, . The region to which each pixel belongs is calculated on the fly.

Geometry motion partition is applied to 3 different block sizes: 64×64, 32×32 and 16×16. At each block size, 32 different values of θare permitted (from 0 to 3600  in steps of 11.250). The number of values ρ can take depends on the block size. For block size of 16×16,  ρcan take 8 possible values (from 0 to 7 in steps of 1). For block sizes of 32×32 and 64×64, ρ can take 16 and 32 possible values, respectively. Thus for block sizes of 16×16, 32×32, and 64×64, there are 256, 512, and 1024 possible geometry partitions respectively.

Figure 1: Parameters defining a geometry motion partition

# Encoder complexity reduction

To reduce the encoder complexity of geometry motion partition, we designed fast motion estimation and fast geometry mode decision. During the motion estimation of each partition, we use the motion vectors of the overlapped square or rectangular blocks as the starting point and then do refinement rather than doing full EPZS. To find the best geometry mode, we first select the best 16 modes based on SAD and then do full RD analysis for each one of the best modes.

# Simulation Results

Geometry motion partition was implemented in Qualcomm test model [3]. In the simulation, we tested on class B, C, and D test sequences and for constraint set 1 and 2 as defined in the CfP document [1]. The configuration parameters are as follows: MDDT=1; MVC=1; HPF=4(SIFO case); QALF=1;MVResolution=1; EXT_MB = 2 (64x64). The results are presented in Tables 1.

Table1 :BD rate reduction by using geometry motion partition

# References

[1] ISO/IEC JTC1/SC29/WG11, “Joint Call for Proposals on Video Compression Technology”, MPEG Document N11113, January 2010

[2] Andreas Kruts, et. al, “Tool Experiment 3: Inter Prediction in HEVC”, JCTVC-A303, Dresden, Germany, April 2010.

[3] Marta Karczewicz, Peisong Chen, Rajan Joshi, Xianglin Wang, Wei-Jung Chien, Rahul Panchal,  “Video coding technology proposal by Qualcomm Inc” JCTVC Contribution JCTVC-A121, Dresden, Germany, April 2010.

[4] O. Divorra, P. Yin and C. Gomila, “Geometry-Adaptive Block Partitioning”, ITU-TQ.6/SG16 VCEG, VCEG-AF10, April 2007.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).