| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B121 |
| --- | --- |

| Title: | Notes from break-out session on TMuC and AVC high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Rickard Sjoberg | Tel: Email: | rickard.sjoberg@ericsson.com |
| Source: | Break-out group |  |  |

_____________________________

# Introduction

A break-out session was held on July 24 from 11.30pm to 2pm in room A to discuss what high-level elements to carry over from AVC to TMuC as a starting point.

The following companies were represented at the meeting:

Ericsson, Nokia, HHI, Samsung, Tandberg, Siemens, Polycom, ETRI, NEC, Qualcomm, LG, Sony, Panasonic, Canon, MediaTek and NTT DOCOMO.

# Notes

## Assumptions

Use sequence and picture parameter sets

Use the NAL/AU structure

No introduction of interlaced tools from AVC

## Break-out group conclusions

### Recommendations

- Align bit-positions of header syntax elements between TMuC and AVC when possible, e.g. level_idc in the sequence parameter set

- Add all three POC types to the TMuC

Some discussion on POC type 1

- Add HRD description

The need of it is questionable for some media transport specifications

Painful to re-design

- Add bumping process description, part of HRD

Painful to redesign

- Use the List0 and List1 concept

- MMCO

part 1: dec_ref_pic_marking() // MMCO

part 2: ref_pic_list_modification() // slice L0/L1 modifications

Heavyweight for initial implementations

Painful to redesign

Could be done in a more easy way

Functionality seems to be needed

The break-out concluded to copy both to TMuC

- SVC and MVC

- Not include at this time but enable later extension, e.g. by reserved code words.

- Use temporal_id (decide later where to put it)

- Use output_flag (decide later where to put it)

- Use Annex B Byte stream encapsulation

- Use slice trailing (byte alignment 1-8 bits)

- AVC NAL unit types (currently not in TMuC)

- 2 Data partitioning A – Keep reserved

- 3 Data partitioning B - Keep reserved

- 4 Data partitioning C - Keep reserved

- 6 SEI – Keep mechanism

- 9 AU delimiter – Add to TMuC

- 10 End of sequence - Keep reserved

- 11 End of stream - Keep reserved

- 12 Filler data – Add to TMuC

- 13 Sequence parameter set extension - Keep reserved

- 14 Prefix NAL unit  - Keep reserved

- 15 Subset sequence parameter set  - Keep reserved

- 19 Coded slice of an auxiliary coded picture without partitioning  - Keep reserved

- 20 Coded slice extension  - Keep reserved

- VUI – As appropriate, the break out group did not look at in detail but the group concluded that there are some valuable VUI elements

### For later considerations

- Slice groups

Slice group type 2 is interesting for parallelization purposes and SCP.

Align with entropy slices?

- Arbitrary slice order.

- AVC type weighted prediction

### Do not move to the TMuC

- Redundant pictures

- SP, SI pictures