Notes break-out

Mon 7/26 3-4pm Room C

Participants: majority of JCT-VC participants

Purpose of break-out: Identify differences between software configurations discussed so far in 2nd JCT-VC meeting and those proposed in B122.

Reviewed appendix 6 of B122, including tool table and software encoder configuration file.

Several questions were raised about the current state of intra prediction (number of modes, etc). Configuration proposed in B122 is not supported by TMuC software (e.g., 9 AVC modes for 4x4 (defined in A205) and 8x8 (not defined in A205)).

Identified proposed changes to software configurations are:

- Reduce the maximum transform size from 64x64 to 32x32 (intra and inter). Set QuadtreeTULog2MaxSize : 5.

- Use 12-tap DCT-based interpolation filter for high-efficiency and low-complexity configurations instead of SIFO (high-efficiency) and directional filter (low-complexity). Set InterpFilterType : 0.

- Use PIPE entropy coder for low-complexity configurations. Set SymbolMode : 2. Nevermind.

- Enable IBDI (4 bits) for low-complexity configurations. Set BitIncrement : 4. Nevermind.

- Disable “merge mode” tool. Set MRG : 0. Nevermind.

- Disable “combined intra prediction” tool. Set CIP : 0.

- Disable “rotational transform” tool. Set ROT : 0.

- Disable “adaptive intra smoothing” tool. Set AIS : 0.

The following configuration flag modifications identified in B122 do not require change:

- Setting NRF : 0. The purpose of the flag was misunderstood and should be kept to 1.

- Setting IMP : 0. The flag is already set to 0 in configuration files shared on 7/25 on the JCT-VC reflector.

The spirit of B122 may also require changes to compile-time configuration (e.g., macros defined in TypeDef.h). Time was insufficient to identify such possible changes.

Note: some TMuC proponents felt current discussion seems mostly restricted to those tools configurable in the configuration file.