| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B312 |
| --- | --- |

| Title: | Tool Experiment 12: Evaluation of TMuC Tools |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | Tool Experiment Description |  |  |
| Author(s) or Contact(s): | Ken McCann Samsung/ZetaCast | Email: | ken@zetacast.com |
| Source: | TE Coordinator |  |  |

_____________________________

# Introduction

This tool experiment aims to explore the performance of individual tools in the Test Model under Consideration (TMuC)[1].

It is anticipated that the results of this work will be used to guide improvements in the default operating points of the TMuC.  In addition, it should provide insight on how the various tools contribute to the overall performance of the TMuC and hence should help inform the process of forming the first Test Model. However, it should be noted that the various tools in TMuC interact in a complex non-linear manner, so that careful interpretation of the results will be required.

# Participants List

| Participant | Organisation | Email address |
| --- | --- | --- |
| Thomas Wiegand | Fraunhofer HHI | wiegand@hhi.de |
| Tomoo Yamakage | Toshiba | tomoo.yamakage@toshiba.co.jp |
| Woo-Jin Han | Samsung Electronics | wjhan.han@samsung.com |
| Andrew Segall | Sharp | asegall@sharplabs.com |
| Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |
| Arild Fuldseth | Cisco Systems Norway | arilfuld@cisco.com |
| Kemal Ugur | Nokia | kemal.ugur@nokia.com |
| Gaëlle Martin-Cocher | RIM | gmartincocher@rim.com |
| Matthias Narroschke | Panasonic | Matthias.Narroschke@eu.panasonic.com |
| Peisong Chen | Qualcomm | peisongc@qualcomm.com |
| Teruhiko Suzuki | Sony | teruhikos@jp.sony.com |
| Keiichi Chono | NEC | chono@ct.jp.nec.com |
| Joel Jung | Orange Labs | joelb.jung@orange-ftgroup.com |
| Kazuo Sugimoto | Mitsubishi | Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Rickard Sjöberg | Ericsson | rickard.sjoberg@ericsson.com |
| Haoping Yu | Huawei | haopingyu@huawei.com |
| David Flynn | BBC | davidf@rd.bbc.co.uk |
| Faisal Ishtiaq | Motorola Inc | faisal@motorola.com |
| Yi-Jen Chiu | Intel | Yi-jen.chiu@intel.com |
| Minhua Zhou | TI | zhou@ti.com |
| Seung-Wook Park | LG Electronics | seungwook.park@lge.com |
| Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| Purvin Pandit | Harmonic Inc | Purvin.Pandit@harmonicinc.com |
| Xiaolin Shen | Zhejiang University | babylion@zju.edu.cn |
| Hui Yong Kim | ETRI | hykim5@etri.re.kr |
| TK Tan | NTT DOCOMO | tktan.p@rd.nttdocomo.co.jp |
| Edouard Francois | Technicolor | edouard.francois@technicolor.com |
| Laurent Guillo | INRIA | laurent.guillo@inria.fr |
| Ji-Zheng Xu | Microsoft | jzxu@microsoft.com |
| Oscar Au | Hong Kong University of Science and Technology | eeau@ust.hk |
| Patrice Onno | Canon Research France | Patrice.Onno@crf.canon.fr |
| Shohei Matsuo | NTT Corporation | matsuo.shohei@lab.ntt.co.jp |

# Tool Experiments

## Experiments on Individual Tools

The incremental contribution of each individual tool in TMuC is highly dependent on the settings of the other tools in use; in effect, it is a multi-dimensional non-linear system.   The most useful starting points for the tests are therefore the default configurations of the TMuC, since these represent the current best estimate of realistic use cases.

Each tool in the TMuC will be tested individually as follows.

- Some tools are on/off in nature (e.g. ROT).  Such a tool will be tested by being turned on (if off in the default configuration) or off (if on in the default configuration) unless it has a direct alternative (see below).

- Some tools have direct alternatives either within the TMuC itself or else added to the software for comparison purposes (e.g. PIPE and the VLC entropy coding within the TMuC, with CABAC for comparison).  Such a tool will be tested by being replaced by each direct alternative in turn (e.g. PIPE will be tested against both the VLC and CABAC).

- Some tools can have minimum or maximum limits defined (e.g. Large Transform).  Such a tool will be tested by having either the minimum or maximum limit made stricter as appropriate (e.g. maximum transform size being reduced from 64x64 to 32x32).

In each case, only one tool at a time will take an alternative setting from that of the default configuration; combinations of tools will not be exercised in this phase of testing.  Tools for which there is no alternative in the TMuC (e.g. inter transforms) will not be tested.

## Analysis of Combinations of Tools

Some preliminary testing and analysis of combinations of tools will be performed, in order to improve understanding of the interaction between tools (e.g. between ROT and MDDT).

# Tools under Test

## Tools Available in TMuC 0.7

Simulation results will be generated to exercise individual switchable tools in the four inter-coding configurations specified for TMuC 0.7 in JCTVC-B300[2] and (for relevant tools) the two intra-coding configurations.

The following tools will be tested in all six configurations.  For tools where the low-complexity setting differs from the intra high-efficiency setting, the low complexity setting is designated “LoCo” and is shown in brackets in the table below:

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Coding unit | Min size = 8x8 Max size = 64x64 | Min size = 8x8 Max size = 32x32 | MaxCUWidth : 32 MaxCUHeight : 32 MaxPartitionDepth : 3 |
| Transform unit | Quadtree | 2-level | #define HHI_RQT 0 #define HHI_RQT_CHROMA_CDF_MOD 0 #define HHI_RQT_INTRA 0 |
| Large transform | Max size = 64 | Max size = 32 | QuadTreeTULog2MaxSize : 5 |
| Rotational transform (ROT) | Enabled | Disabled | ROT : 0 |
| Mode depend. directional transform (MDDT) | Enabled | Disabled | #define QC_MDDT 0 |
| Combined ROT and MDDT test | Both enabled | Both disabled | ROT : 0 #define QC_MDDT 0 |
| Adaptive Intra Smoothing (AIS) | Enabled | Disabled | AIS : 0 |
| Combined Intra Prediction (CIP) | Enabled | Disabled | CIP : 0 |
| Planar prediction | Enabled | Disabled | #define PLANAR_INTRA 0 |
| Edge-based prediction | Enabled | Disabled | EdgePredictionEnable : 0 |
| Deblocking filter | Enabled | Disabled | LoopFilterDisable : 1 |
| Adaptive loop filter (ALF) | Diamond (LoCo: disabled) | Disabled (LoCo: diamond) | ALF : 0 (LoCo: ALF : 1) |
| Entropy coders: tested against each other | PIPE (LoCo: LCEC) | LCEC (LoCo: PIPE) | SymbolMode : 0, MultiCodewordThreshold : 0 (LoCo: SymbolMode : 2, MultiCodewordThreshold : 96000) |
| Entropy coders: tested against CABAC | PIPE (LoCo: LCEC) | CABAC | SymbolMode : 1 MultiCodewordThreshold : 0 |
| Transform coefficient coding | HHI | Samsung | #define HHI_TRANSFORM_CODING 0 |
| Internal Bit-Depth Increase (IBDI) | 4 bits (LoCo: off) | Off (LoCo: 4bits) | BitIncrement : 0 (LoCo: BitIncrement : 4) |
| Rate Distortion Optimized Quantization (RDOQ) | On | Off | RDOQ : 0 |
| CABAC-based RDO | On | Off | SBACRD : 0 |

The following additional tests will be performed for the four inter-coding configurations only.

| Tool name | Default setting | Alternative setting | Parameters for Alternative setting |
| --- | --- | --- | --- |
| Asymmetric Motion Prediction Unit (AMP) | Enabled | Disabled | AMP: 0 |
| Block Merging (MRG) | Enabled | Disabled | MRG: 0 |
| Motion vector prediction (AMVP/IMVP) | AMVP method | IMVP method | IMP : 1 #define HHI_AMVP_OFF 1 |
| Interpolation filter: test defaults against each other | SIFO (LoCo: DIF) | DIF (LoCo: SIFO) | InterpFilterType: 3 (LoCo: InterpFilterType : 4) |
| Interpolation filter: test defaults against DCT-IF | SIFO (LoCo: DIF) | DCT-IF | InterpFilterType: 0 |
| Interpolation filter: test defaults against MOMS | SIFO (LoCo: DIF) | MOMS | InterpFilterType : 2 |
| Adaptive motion vector resolution (AMVRES) | Enabled | Disabled | AMVRES : 0 |

## Further Tools to be Available and Switchable by 3rd JCT-VC Meeting

Tools that are integrated in TMuC versions subsequent to 0.7, but prior to the 3rd JCT-VC meeting, will be tested as a second priority.  It is understood that the following tools are planned to be integrated into the TMuC in later software releases, prior to the 3rd JCT-VC meeting:

- Transform skip flag (part of TU)

- Event-based VLC coding

- Improved context adaptivity for VLC

- Rate distortion optimizations for VLC

- Motion vector prediction: scaling for prediction of MVs

- Adaptive-loop filter using prediction and residual data

- Block-based illumination compensation

- Signaling of adaptive loop filter parameters in slice header

- Non-rectangular partitions for PU  + Motion vector prediction for non-rectangular partitions

- Block merging at PU level

The default and alternative settings for these tools will be defined after they are integrated into the TMuC.

# Test Plan for Tools

Each test point in the plan consists of a combination of the tool to be tested and the configuration used as the basis for the test.  All inputs to the TE shall provide simulation results for all sequences at all QP values specified in JCTVC-B300[2].

The test plan for individual tools is as indicated overleaf, where each test point is intended to be tested by at least two separate organizations.

The following additional proposed test has been proposed, but is still awaiting sign-up from a second tester:

- Adaptive intra smoothing slow/fast (#define AIS_TEST_BEST 0 or 1) (proposed by HHI)

|  | Intra, high-efficiency | Intra, low-complexity | Rand. acc, high-efficiency | Rand. acc, low-complexity | Low delay, high-efficiency | Low delay, low-complexity |
| --- | --- | --- | --- | --- | --- | --- |
| Unit Definitions |  |  |  |  |  |  |
| Max CU size 64x64/32x32 |  | Harmonic |  | Harmonic |  | Harmonic |
|  | Motorola | Motorola | Motorola | Motorola | Motorola | Motorola |
| Transform unit quadtree/2-level | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Max TU size 64x64/32x32 | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon | Huawei/HiSilicon |
|  | TI | TI | TI | TI | TI | TI |
| Asymmetric PU (AMP) on/off |  |  | Technicolor/INRIA | Technicolor/INRIA | Technicolor/INRIA | Technicolor/INRIA |
|  |  |  | NHK, Huawei/HiSilicon | NHK, Huawei/HiSilicon | NHK, Huawei/HiSilicon | NHK, Huawei/HiSilicon |
| Block Merging (MRG) on/off |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  |  |  | Mitsubishi, HHI | Mitsubishi, HHI | Mitsubishi, HHI | Mitsubishi, HHI |
| Motion Representation |  |  |  |  |  |  |
| MV prediction AMVP / IMVP |  |  | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi |
|  |  |  | Sony | Sony | Sony | Sony |
| Interpolation filter: SIFO / DIF |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  |  |  | NTT | NTT | NTT | NTT |
| Interpolation filter: SIFO (DIF) / DCT-IF |  |  | Toshiba | Toshiba | Toshiba | Toshiba |
|  |  |  | Sony, Samsung | Sony, Samsung | Sony, Samsung | Sony, Samsung |
| Interpolation filter: SIFO (DIF) / MOMS |  |  | HHI | HHI | HHI | HHI |
|  |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Adaptive MV resolution on/off |  |  | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  |  |  | Sharp | Sharp | Sharp | Sharp |
| Intra-frame Prediction |  |  |  |  |  |  |
| Adapt. intra smooth (AIS) on/off | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi | Mitsubishi |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Combined Intra pred. (CIP) on/off | NEC | NEC | NEC | NEC | NEC | NEC |
|  | Sharp | Sharp | Sharp | Sharp | Sharp | Sharp |
| Planar prediction on/off | BBC | BBC | BBC | BBC | BBC | BBC |
|  | Nokia | Nokia | Nokia | Nokia | Nokia | Nokia |
| Edge-based prediction on/off | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic | Panasonic |
|  | Sony | Sony | Sony | Sony | Sony | Sony |
| Transforms |  |  |  |  |  |  |
| Rotational transform (ROT) on/off | Sharp | Sharp | Sharp | Sharp | Sharp | Sharp |
|  | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
| Mode dep. directional trans. (MDDT) on/off | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm | HKUST, Qualcomm |
| Combined ROT+MDDT on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
| Transform coding HHI/Samsung | Samsung | Samsung | Samsung | Samsung | Samsung | Samsung |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Filtering |  |  |  |  |  |  |
| Deblocking filter on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | MediaTek | MediaTek | MediaTek | MediaTek | MediaTek | MediaTek |
| Adaptive loop filter (ALF) on/off | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm | Qualcomm |
|  | Intel | Intel | Intel | Intel | Intel | Intel |
| Entropy Coding |  |  |  |  |  |  |
| PIPE tested against LCEC | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway | Cisco Norway |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| PIPE (and LCEC) test against CABAC | TI | TI | TI | TI | TI | TI |
|  | HHI | HHI | HHI | HHI | HHI | HHI |
| Other |  |  |  |  |  |  |
| Internal Bit Depth Increase (IBDI) on/off | Toshiba | Toshiba | Toshiba | Toshiba | Toshiba | Toshiba |
|  | TI | TI | TI | TI | TI | TI |
| Rate Distortion Opti. Quant (RDOQ) on/off | RIM |  | RIM |  | RIM |  |
| CABAC-based RDO on/off |  |  |  |  |  |  |

# Definition of Performance Measurement Criteria

## Coding Performance Measurements

The performance measurements will be evaluated by objective rate-distortion measurements using PSNR, BD-PSNR and BD-Rate [3][4].

The subjective visual quality will be also be checked by informal expert viewing.   Any discrepancies between the actual visual quality and the expected visual quality as implied by the objective measurements will be noted.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (relative to the tool in opposite state)

- Decoding time (relative to the tool in opposite state)

# Time-line

2010-August-9:		Deadline for signing up to Tool Experiment 12

2010-August-13:	Final version of Tool Experiment Description for tools included in TMuC 0.7

2010- September-17:	Testing completed by first tester

2010-September-24: 	Cross-checking completed by second and additional testers

2010-October-01:	Input Document uploaded

# References

- JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.