| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C053 WG11 Number: m18074 |
| --- | --- |

| Title: | TE12.3: Adaptive intra smoothing (AIS) test (slow vs. fast) by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Tool experiment result report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of testing slow and fast encoder estimations for AIS within the scope of tool experiment 12 evaluating TMuC Tools [1]. For the test, AIS fast was enabled and tested against the AIS slow default. For an intra coding only constraint set, average bit rate increases between 0.9% and 1.2% are shown for the fast implementation. Encoder runtimes for this constraint set range from 76% to 72% of the default slow implementation. For the inter-translative coding constraint sets, the average bit rate increases between 0.1% and 0.5% are smaller. Encoder and decoder runtimes are similar for these cases and deviations cannot be clearly distinguished from measurement inaccuracies.

- Simulation environment

## Software

The software used to run the simulations is TMuC 0.7 [2]. The binaries were compiled on a 64 bit Ubuntu 9.04 Linux using the included make files and gcc 4.3.3. The coding conditions used for the simulations are specified in [3]. These default settings, both for configuration file and compile-time, are used in this test as anchor. Changes from this anchor configuration are listed in the next sections.

### Alternative configuration file settings

No changes of configuration file settings are needed.

### Alternative compile-time settings

The following macro has to be disabled in TypeDef.h:

#define AIS_TEST_BEST 0

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in  Table 1-1.

Table 1-1 Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 24 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 9.04, 64 bit |
| 2 | 8 | Intel(R) Xeon(R) CPU X5460 @ 3.16GHz | 32 GB | Ubuntu 9.04, 64 bit |
| 3 | 8 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 9.04, 64 bit |

# Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C053.xls.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented in three tables: Table 2-2, Table 2-3 and Table 2-4. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-2 Average BD-rates and relative encoding/decoding runtimes for Intra constraint set.

|  | Intra HE |  |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.4 | 0.4 |  | 0.5 | 0.4 | 0.4 |
| Class B | 0.7 | 0.6 | 0.7 |  | 0.7 | 0.6 | 0.6 |
| Class C | 1.7 | 1.5 | 1.5 |  | 1.1 | 0.9 | 0.9 |
| Class D | 1.4 | 1.1 | 1.2 |  | 1.3 | 1.0 | 1.0 |
| Class E | 1.5 | 1.3 | 1.4 |  | 0.9 | 0.8 | 0.8 |
| All | 1.2 | 1.0 | 1.1 |  | 0.9 | 0.8 | 0.8 |
| Enc Time[%] | 76% |  |  |  | 72% |  |  |
| Dec Time[%] | 108% |  |  |  | 105% |  |  |

Table 2-3 Average BD-rates and relative encoding/decoding runtimes for random access constraint set.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.1 | 0.3 | 0.1 | 0.0 |
| Class B | 0.3 | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 |
| Class C | 0.6 | 0.5 | 0.5 | 0.5 | 0.4 | 0.3 |
| Class D | 0.6 | 0.7 | 0.5 | 0.4 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.4 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |

Table 2-4 Average BD-rates and relative encoding/decoding runtimes for low delay constraint set.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.2 | 0.1 | 0.4 | 0.1 | 0.1 | 0.2 |
| Class D | 0.1 | 0.2 | 0.5 | 0.1 | 0.2 | 0.0 |
| Class E | 0.3 | 0.3 | 0.6 | 0.2 | -0.3 | 0.8 |
| All | 0.2 | 0.2 | 0.4 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 108% |  |  |

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Test Model under Consideration Software 0.7”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.7

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.