| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C070 |
| --- | --- |

| Title: | Cross-check for I2R proposal on mode-dependent fast separable KLT for block-based intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingguo Zhu Lu Yu Zheda Road No.38 Hangzhou, China, 310027 | Tel: Email: | +86-571-87953107 {05xxgcsgx, yul}@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

# Abstract

This contribution presents the cross check results of I2R proposal on mode-dependent fast separable KLT for block-based intra coding [1]. The cross check has two aspects: data check and codec check. Because of the limitation of time, not all data but some randomly selected points have been checked. The sequences checked have the same PSNR and Bitrate with the corresponding date given by proponent. And source code of the codec is conformance with JCTVC-B024.

- More information

## Data cross check

We have checked the following sequences (showed in Table 1) of Class C and Class D. The output PSNR(Y/U/V) and Bitrate are the same with the corresponding date given by I2R. The encoder and the decoder are matched.

Table 1		Sequences which were checked

|  | Sequence Name | cfg (QP) |
| --- | --- | --- |
| Class C | RaceHorses | Intra(27) & Randm_access(22) |
| Class D | BasketballPass | Intra(32) |
|  | BQSquare | Randm_access(22) |
|  | BlowingBubbles | Randm_access(37) |
|  | RaceHorses | Intra(27) |

The encoding and decoding time ratio in our test can be seen in Table 2.

Here, time ratio = (encoding or decoding time when using fast KLT [1]) / (encoding or decoding time of anchor)

Table 2		Encoding and decoding time ratio

|  | I2R |  | Our test |  |
| --- | --- | --- | --- | --- |
|  | encoder | decoder | encoder | decoder |
| Intra | 100% | 100% | 98% | 95% |
| Randm_access | 100% | 107% | 99% | 94% |

Table 2 shows that, in our test the encoding or decoding time when using fast KLT is less than the corresponding anchor.

## Codec check

We checked the source code (both encoder and decoder), and it is conformance with that specified in JCTVC-B024. There is fast algorithm for 4×4 KLT, both forward and inverse transform. However, there is no fast algorithm for 8×8 KLT yet.

References:

- Chuohao Yeo, Yih Han Tan, Zhengguo Li, Susanto Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding ”, JTC1/SC29/WG11, JCTVC-B024, Geneva, July, 2010