| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C084 WG11 Number: m18107 |
| --- | --- |

| Title: | Coding efficiency report of modification by TMuC draft005 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage Takeshi Chujoh Takashi Watanabe 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2152 tomoo.yamakage@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution shows experimental results of the modification by TMuC draft005 that signals ALF filtering control map in slice header instead of signaling in CU header. Experimental results show that the coding efficiency loss is 0.04% on average while interleaving process of the ALF filtering control map at encoder side is removed.

# Introduction

At the Dresden meeting, there was an agreement to move ALF filtering control map (ALF_flag) to slice header. The text of TMuC draft005 incorporated the modification, and the software has been submitted to the SVN server as a branch of TMuC v0.8.

This contribution shows experimental results of the modification by TMuC draft005. Experimental results show that the coding efficiency loss is 0.04% on average while encoder implementation is simplified.

# Explanation of the modification by TMuC draft005

In the previous version of TMuC text and software, ALF_flag is signaled in slice header. Since CU partitioning for ALF and ALF_flag can be determined only when a slice is encoded completely, it is necessary for encoder to store the coded data for the slice and interleave ALF_flag into the coded data after its determination. Since the coded data is entropy coded, the position of ALF_flag for each CU is not simply found.

The modification points of TMuC draft005 is as follows:

- Signal ALF_flag in slice header

- Gathering all ALF_flags and signal them continuously

- Signal the number of ALF_flags (add some overhead)

By signaling all ALF_flags continuously, the interleaving ALF_flag into the coded data becomes unnecessary.

# Experimental results

As mentioned in the modification item 3 in section 2, there is some overhead to signal the number of ALF_flags in this modification. In order to confirm the effect by this modification, a simulation is conducted based on the test conditions in JCTVC-B300 [6] and JCTVC-B310_r3 [7]. However, TMuC v0.8 is used as anchor and the modification is implemented on TMuC v0.8.

Table 1 indicates results of BD-rate for above experiments. Values in the parenthesis are BD-rate of the anchor (QC_ALF on) of TMuC v0.7 against QC_ALF_off. Although this is not TMuC v0.7, this will show the coding gain when using QC_ALF. The coding efficiency loss is 0.04% on average, which corresponds to less than 1.5% (negligible) loss of overall coding efficiency gain of QC_ALF.

Table 1  BD-rate (%) for Random Access case (positive value indicates loss)

| High Efficiency |  |  | Random Access | Low Delay |
| --- | --- | --- | --- | --- |
| A | S01 | Traffic | -0.01 (-3.0) | N/A |
|  | S02 | PeopleOnStreet | 0.03 (-6.3) | N/A |
| B | S03 | Kimono | 0.04 (-2.7) | 0.03 (-6.5) |
|  | S04 | ParkScene | 0.01 (-1.8) | 0.14 (-3.7) |
|  | S05 | Cactus | 0.04 (-4.3) | 0.20 (-4.0) |
|  | S06 | BasketballDrive | 0.02 (-13.3) | 0.01 (-5.8) |
|  | S07 | BQTerrace | 0.02 (-2.5) | -0.03 (-7.9) |
| C | S08 | BasketballDrill | 0.07 (-2.5) | -0.14 (-4.7) |
|  | S09 | BQMall | 0.02 (-2.1) | 0.02 (-4.2) |
|  | S10 | PartyScene | 0.00 (-2.3) | -0.01 (-2.0) |
|  | S11 | RaceHorses | 0.06 (-3.3) | -0.01 (-4.8) |
| D | S12 | BasketballPass | 0.00 (-0.6) | 0.06 (-2.7) |
|  | S13 | BQSquare | -0.03 (-3.8) | -0.03 (-4.6) |
|  | S14 | BlowingBubbles | -0.05 (-0.7) | 0.03 (-0.9) |
|  | S15 | RaceHorses | 0.01 (-1.6) | 0.07 (-2.4) |
| E | S16 | Vidyo1 | N/A | 0.15 (-5.0) |
|  | S17 | Vidyo3 | N/A | 0.38 (-12.2) |
|  | S18 | Vidyo4 | N/A | 0.24 (-6.0) |
| Class A |  |  | 0.01 (-4.7) | N/A |
| Class B |  |  | 0.03 (-5.3) | 0.07 (-5.6) |
| Class C |  |  | 0.04 (-2.5) | -0.04 (-3.9) |
| Class D |  |  | -0.02 (-1.7) | 0.03 (2.6) |
| Class E |  |  | N/A | 0.26 (-7.7) |
| Total |  |  | 0.02 (-3.5) | 0.07 (-4.8) |
| Encoding time |  |  | 100 | 100 |
| Decoding time |  |  | 100 | 100 |

# Conclusion

In this contribution, experimental results of the modification by TMuC draft005 are reported. Experimental results show that the coding efficiency loss is 0.04% on average while encoder implementation is simplified.

# Reference

[1]	I. S. Chong, W.J. Chien, N. Malayath, and M. Karczewicz, “Encoder complexity analysis and performance report on adaptive loop filter”, JCTVC-B095, July 2010.

[2]	M. Karczewicz, P. Chen, R. Joshi, X. Wang, W. Chien, R. Panchal, “Video coding technology proposal by Qualcomm Inc.”, JCTVC-A121, April 2010.

[3]	T. Chujoh, N. Wada, T. Watanabe, G. Yasuda and T. Yamakage, “Specification and experimental results of Quadtree-based Adaptive Loop Filter”, ITU-T Q.6/SG16 Doc., VCEG-AK22, Yokohama, April 2009.

[4]	http://iphome.hhi.de/suehring/tml/download/KTA/jm11.0kta2.7.zip

[5]	T. Yamakage, T. Chujoh and T. Watanabe, “TE10: Reduction of number of encoding passes for quadtree-based adaptive loop filter (QALF)”, JCTVC-C082, October 2010.

[6]	“Common test conditions and software reference configurations,” JCTVC-B300, Geneva, July 2010.

[7]	T. Yamakage, T. Chujoh, S. Lei, K. Chono, M. Karczewicz, A. Segall, T. Yamamoto, B. Jeon, J. Lim, J. Xu, “Tool Experiment 10: In-loop filtering,” JCTVC-B310_r3, Geneva, July 2010.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).