| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, China, 07-15 October, 2010 | Document: JCTVC-C100 |
| --- | --- |

| Title: | TE5: Huawei & HiSilicon report on verification test results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Lingzhi Liu, Hisilicon, Jiantong Zhou, Huawei, Haoping Yu, Huawei | Tel: Email: | +1-408-966-2148 lzliu@huawei.com zhoujiantong@huawei.com haopingyu@huawei.com |
| Source: | Huawei Technologies and Hisilicon Technologies |  |  |

_____________________________

# Introduction

This document summarizes the Tool Experiment TE5 activities related to the evaluation of Simplification of Unified Intra Prediction, [1].

This evaluation was performed by Huawei/Hisilicon.

# Tests conditions

The following platforms and compilers have been used:

| company | platform | Compiler |
| --- | --- | --- |
| Huawei/Hisilicon | Windows XP 64bits | VS2008 |

The testers have run WVGA and WQVGA test cases with different definitions of “SIMPLIFIED_ANGULAR_PREDICTION” and “TI_INTRA_PRED_SIMPLIFICATION” using similar computers, in order to obtain coherent Encoding and Decoding times for both configurations.

# Cross-checking results

The testers have observed mismatches of bitrate compared with other company’s results using Linux platforms. However the differences remain minor, as depicted in the following tables, and our test results can be considered as consistent with those reported by the proponents.

The following tables show the sequences and qp values and the corresponding mismatching bitrates.

| Case: | Intra |  |  |  |
| --- | --- | --- | --- | --- |
| Condition: | SIMPLIFIED_ANGULAR_PREDICTION 1; TI_INTRA_PRED_SIMPLIFICATION 0 |  |  |  |
| Sequence | QP | bitrate-Linux (Docomo) | bitrate-Windows (Huawei) | Δbitrate |
| BQMall_832 | 37 | 4794.4016 | 4794.2784 | -0.1232 |
| PartyScene_832 | 22 | 45711.3288 | 45711.3896 | 0.0608 |
| RaceHorse_832 | 22 | 15484.3056 | 15484.2856 | -0.02 |

| Case: | Intra loco |  |  |  |
| --- | --- | --- | --- | --- |
| Condition: | SIMPLIFIED_ANGULAR_PREDICTION 1; TI_INTRA_PRED_SIMPLIFICATION 0 |  |  |  |
| Sequence | QP | bitrate-Linux (Docomo) | bitrate-Windows (Huawei) | Δbitrate |
| PartyScene_832 | 37 | 9583.28 | 9583.2968 | 0.0168 |
| RaceHorse_832 | 37 | 2802.0208 | 2802.0872 | 0.0664 |

| Case: | Intra |  |  |  |
| --- | --- | --- | --- | --- |
| Condition: | SIMPLIFIED_ANGULAR_PREDICTION 0; TI_INTRA_PRED_SIMPLIFICATION 1 |  |  |  |
| Sequence | QP | bitrate-Linux (TI) | bitrate-Windows (Huawei) | Δbitrate |
| PartyScene_832 | 22 | 45710.0976 | 45710.3048 | 0.2072 |
| RaceHorse_832 | 22 | 15478.7464 | 15478.8904 | 0.144 |
| RaceHorse_832 | 37 | 2696.5008 | 2696.5656 | 0.0648 |

| Case: | Intra loco |  |  |  |
| --- | --- | --- | --- | --- |
| Condition: | SIMPLIFIED_ANGULAR_PREDICTION 0; TI_INTRA_PRED_SIMPLIFICATION 1 |  |  |  |
| Sequence | QP | bitrate-Linux (TI) | bitrate-Windows (Huawei) | Δbitrate |
| BQMall_832 | 37 | 5296.7936 | 5296.848 | 0.0544 |
| PartyScene_832 | 37 | 9581.7632 | 9581.796 | 0.0328 |
| RaceHorse_832 | 37 | 2801.748 | 2801.7784 | 0.0304 |

# Performance

The simulation results of WVGA and WQVA sequences can be found in files TE5_results _Huawei_DOCOMO.xls and TE5_results _Huawei_TI.xls.

The following tables summarize the performance evaluation obtained by the testing:

| Condition: |  | SIMPLIFIED_ANGULAR_PREDICTION 1; TI_INTRA_PRED_SIMPLIFICATION 0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  | Intra LoCo |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class C | 0.1 |  | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 |  |
| Class D | 0.0 |  | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |  |
| Dec Time[%] | 94% |  |  |  | 99% |  |  |  |

| Condition: |  | SIMPLIFIED_ANGULAR_PREDICTION 0; TI_INTRA_PRED_SIMPLIFICATION 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  | Intra LoCo |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class C | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Class D | 0.0 |  | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Dec Time[%] | 73% |  |  |  | 95% |  |  |  |

# References

- TK Tan, Madhukar Budagavi and Jani Lainema “Tool Experiment 5: Simplification of Unified Intra Prediction”, Doc. JCTVC-B305r1, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and Hisilicon Technologies Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).