| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, China, 07-15 October, 2010 | Document: JCTVC-C105 |
| --- | --- |

| Title: | TE12: Huawei report on max transform unit size evaluation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Tool Experiment Report |  |  |
| Author(s) or Contact(s): | Jiantong Zhou Dong Wang Haoping Yu | Tel: Email: | +86-755-28567191 zhoujiantong@huawei.com haopingyu@huawei.com wangdong1210@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Introduction

This document summarizes the Tool Experiment TE12 activities related to the evaluation of Max TU Size [1].

This evaluation was performed by Huawei and TI.

# Tests conditions

The following platforms and compiler have been used:

| Test cases | Hardware | OS | Compiler |
| --- | --- | --- | --- |
| Encoding Intra and Low Delay | Intel Xeon E5520@2.27GHz, 16G Ram | Windows 2003 Server R2 X64 platform | Visual Studio 2005 |
| Encoding Random Access Class A and B | Intel Xeon Dual E5520@2.27GHz, 48G Ram |  |  |
| Encoding Random Access Class C and D | Intel Xeon CPU Dual X7350@2.27GHz, 48G Ram |  |  |
| Decoding all | Intel Xeon CPU E5310@2.2GHz, 16G Ram |  |  |

The testers have run each test case with ‘transforms from 32x32 to 4x4’, ‘transforms from 32x32 to 8x8’, ‘transforms from 16x16 to 4x4’,  ‘transforms from 16x16 to 8x8’ and ‘All transforms up to 64x64’ using  computers with the same configuration, in order to obtain comparable Encoding and Decoding times for both configurations.

When running the tests, the following two problems occurred:

1. Huawei found that both TMuC0.7.1 and TMuC0.7.2 compiled by VS 2005 crashed when encoding some of test cases with min transform size 8x8, while TMuC0.7.3 run well.

2. TI found that TMuC0.7.1 had issues with LCEC. However, TMuC0.7.3 with HHI_RQT_ROOT enabled run well.

So different versions of the TMuC software were used in the evaluation as following:

| Test cases | TMuC Version |
| --- | --- |
| TU_Max5_Min2_HE TU_Max4_Min2_HE TU_Max6_Min2_HE | TMuC 0.7.1 |
| TU_Max5_Min3_HE TU_Max4_Min3_HE | TMuC 0.7.3 |
| All transforms tests with the LC configuration. | TMuC 0.7.3 with HHI_RQT_ROOT enabled |

# Cross-checking results

The testers have observed mismatches when using Linux and Windows platforms. However the differences remain tiny and occur only after the 3rd digit of the bit rate values in some of the tests, while the PSNR values are all same.

# Max TU Size performance

The full evaluation results can be found in files TE12-TUSize _maxTU16x16_minTU4x4-Huawei.xls, TE12-TUSize_maxTU16x16_minTU8x8-Huawei.xls, TE12-TUSize_maxTU32x32_minTU4x4-Huawei.xls and TE12-TUSize_maxTU32x32_minTU8x8-Huawei.xls.

The following tables summarize the Max TU Size performance evaluation obtained by Huawei:

Performance with max TU size 16x16 and min TU size 4x4

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 1.4 | 1.6 | 0.6 | 1.0 | 1.5 |
| Class B | 2.1 | 3.9 | 4.3 | 3.1 | 3.2 | 3.8 |
| Class C | 0.4 | 1.0 | 0.8 | 0.4 | 0.5 | 0.6 |
| Class D | 0.4 | 0.8 | 0.7 | 0.2 | 0.4 | 0.4 |
| Class E | 1.8 | 8.0 | 6.8 | 3.1 | 7.2 | 6.3 |
| All | 1.1 | 3.0 | 2.8 | 1.5 | 2.4 | 2.5 |
| Enc Time[%] | 91% |  |  | 78% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 3.0 | 3.2 | 0.7 | 1.3 | 1.6 |
| Class B | 3.3 | 7.0 | 6.9 | 4.2 | 5.2 | 7.2 |
| Class C | 1.5 | 2.8 | 2.6 | 1.5 | 1.3 | 1.3 |
| Class D | 0.7 | 2.0 | 1.7 | 0.4 | 1.8 | 0.9 |
| Class E |  |  |  |  |  |  |
| All | 1.8 | 4.0 | 3.9 | 2.0 | 2.7 | 3.2 |
| Enc Time[%] | 95% |  |  | 97% |  |  |
| Dec Time[%] | 88% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.9 | 15.8 | 20.5 | 6.4 | 13.7 | 19.5 |
| Class C | 1.4 | 6.7 | 7.8 | 2.4 | 3.8 | 4.3 |
| Class D | 0.5 | 6.2 | 6.5 | 1.0 | 4.8 | 3.3 |
| Class E | 8.0 | 35.8 | 26.7 | 6.4 | 23.2 | 17.0 |
| All | 2.9 | 14.9 | 15.0 | 4.0 | 10.8 | 11.2 |
| Enc Time[%] | 96% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

Performance with max TU size 16x16 and min TU size 8x8

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 4.0 | 6.5 | 6.0 | 1.0 | 4.1 | 3.0 |
| Class B | 5.4 | 8.9 | 9.2 | 4.0 | 7.4 | 7.0 |
| Class C | 9.2 | 10.1 | 10.3 | 6.8 | 10.8 | 10.0 |
| Class D | 10.8 | 13.2 | 14.3 | 8.7 | 12.6 | 13.8 |
| Class E | 6.0 | 9.9 | 9.9 | 3.4 | -1.7 | 1.1 |
| All | 7.4 | 10.0 | 10.3 | 5.2 | 7.4 | 7.8 |
| Enc Time[%] | 42% |  |  | 35% |  |  |
| Dec Time[%] | 85% |  |  | 84% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 6.3 | 12.6 | 11.6 | 3.6 | 10.7 | 10.6 |
| Class B | 7.7 | 14.7 | 14.5 | 5.9 | 12.3 | 12.9 |
| Class C | 11.3 | 16.3 | 17.0 | 10.3 | 17.4 | 17.2 |
| Class D | 13.1 | 19.9 | 20.7 | 12.0 | 20.0 | 21.6 |
| Class E |  |  |  |  |  |  |
| All | 9.9 | 16.2 | 16.4 | 8.4 | 15.5 | 16.1 |
| Enc Time[%] | 52% |  |  | 66% |  |  |
| Dec Time[%] | 85% |  |  | 106% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |
| Class B | 7.3 | 22.9 | 28.1 | 7.9 | 18.7 | 22.1 |
| Class C | 10.6 | 18.4 | 18.9 | 11.3 | 15.8 | 15.6 |
| Class D | 11.4 | 20.5 | 21.5 | 12.5 | 17.2 | 16.9 |
| Class E | 12.2 | 38.3 | 31.1 | 7.5 | 19.3 | 17.2 |
| All | 10.1 | 24.1 | 24.7 | 9.8 | 17.7 | 18.3 |
| Enc Time[%] | 56% |  |  | 53% |  |  |
| Dec Time[%] | 88% |  |  | 89% |  |  |

Performance with max TU size 32x32 and min TU size 4x4

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.1 | 0.3 | 0.3 | 0.0 | 0.1 | 0.1 |
| Class B | 0.4 | 1.0 | 1.0 | 0.1 | 0.2 | 0.3 |
| Class C | 0.1 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.3 | 1.8 | 1.3 | 0.4 | 0.8 | 0.7 |
| All | 0.2 | 0.7 | 0.6 | 0.1 | 0.2 | 0.2 |
| Enc Time[%] | 96% |  |  | 83% |  |  |
| Dec Time[%] | 92% |  |  | 91% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 0.2 | 0.5 | 0.4 | 0.1 | 0.2 | 0.2 |
| Class B | 0.6 | 1.4 | 1.4 | 0.4 | 1.0 | 1.6 |
| Class C | 0.3 | 0.3 | 0.4 | 0.2 | 0.2 | 0.1 |
| Class D | 0.1 | 0.2 | 0.2 | 0.0 | 0.4 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.7 | 0.7 | 0.2 | 0.5 | 0.6 |
| Enc Time[%] | 96% |  |  | 99% |  |  |
| Dec Time[%] | 86% |  |  | 106% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 3.9 | 4.8 | 0.8 | 2.1 | 3.1 |
| Class C | 0.2 | 1.2 | 1.3 | 0.3 | 0.4 | 0.4 |
| Class D | 0.1 | 0.8 | 0.9 | 0.1 | 0.7 | 0.4 |
| Class E | 1.7 | 9.6 | 7.3 | 1.3 | 4.3 | 3.0 |
| All | 0.5 | 3.5 | 3.4 | 0.6 | 1.7 | 1.7 |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 94% |  |  |

Performance with max TU size 32x32 and min TU size 8x8

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 3.5 | 5.4 | 4.6 | 0.6 | 3.4 | 1.8 |
| Class B | 3.6 | 5.8 | 5.7 | 1.2 | 4.8 | 4.0 |
| Class C | 8.8 | 9.2 | 9.5 | 6.5 | 10.4 | 9.5 |
| Class D | 10.5 | 12.3 | 13.6 | 8.6 | 12.3 | 13.5 |
| Class E | 4.4 | 3.7 | 4.0 | 1.0 | -7.4 | -3.8 |
| All | 6.4 | 7.6 | 7.9 | 3.9 | 5.5 | 5.8 |
| Enc Time[%] | 49% |  |  | 38% |  |  |
| Dec Time[%] | 90% |  |  | 89% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | 5.3 | 10.1 | 8.6 | 2.9 | 9.7 | 9.3 |
| Class B | 4.8 | 8.9 | 8.7 | 2.3 | 8.4 | 8.3 |
| Class C | 9.9 | 13.6 | 14.4 | 9.1 | 16.3 | 16.0 |
| Class D | 12.4 | 17.8 | 19.3 | 11.6 | 19.2 | 20.9 |
| Class E |  |  |  |  |  |  |
| All | 8.3 | 12.7 | 13.0 | 6.7 | 13.6 | 13.9 |
| Enc Time[%] | 58% |  |  | 61% |  |  |
| Dec Time[%] | 85% |  |  | 106% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A |  |  |  |  |  |  |
| Class B | 4.7 | 9.4 | 11.5 | 2.6 | 6.1 | 6.5 |
| Class C | 9.2 | 11.6 | 11.7 | 9.3 | 11.8 | 11.2 |
| Class D | 10.8 | 14.3 | 15.4 | 11.7 | 12.5 | 13.5 |
| Class E | 5.9 | 11.9 | 11.9 | 3.0 | -0.3 | 2.8 |
| All | 7.6 | 11.7 | 12.6 | 6.6 | 7.9 | 8.7 |
| Enc Time[%] | 59% |  |  | 61% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

It is observed that:

- TU size 4x4 impacts the RD performance greatly when compared with TU size 32x32. In the case for the luma picture Y, average 6% BD rate saving is achieved when TU 4x4 is turned on, while TU size 32x32 has only got an average of 2% BD rate saving.

- Difference CU sizes do not have strong impact to the decoder time. But if TU size 4x4 is turned off, around 50% encoder time can be saved.

# References

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Doc. JCTVC-A312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, July 2010.