| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C173 WG11 Number: m18202 |
| --- | --- |

| Title: | TE10 subtest 2: Parallel adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai, Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution is one of the TE10 subset 2 [1] (Wiener-based in-loop filters) proposals. In this contribution, the novel Parallel adaptive loop filter technique [2], which was implemented into TMuC0.7[3], was evaluated according to the TE10 common condition.

The proposed technique uses both pre-DF (De-blocking filter) signal and post-DF signal as inputs, where both inputs’ weights are optimized with Wiener-filter technique. So the whole system can be seen as one in-loop filter where De-blocking filter and Wiener-based filter are combined.  This combination provides coding efficiency improvement as well as the functionality to process the two filters in parallel.

The experimental results show the proposed technique provides 0.7 % / 0.4 % / 0.4 % bitrate reduction (IntraOnly / RandamAccess / LowDelay) compared to the anchor where Sum-modified Laplacian based ALF is used. The complexity of the proposed method was also evaluated and the results show the decoding time is 93 % to 96 % of the anchor and the encoding time is almost the same as the anchor (100 %). The proposal was cross-checked by MediaTek [4].

# Parallel adaptive loop filter

In the proposed technique, Wiener-based in-loop filter is applied using both pre-DF signal and post-DF signal as inputs. (cf. The traditional ALF uses only post-DF signal.) The ALF in the proposed system is defined by the following formula.

Figure 1: Parallel adaptive loop filter system

Where, sout is ALF output, si is pre-DF signal, and spost is post-DF signal. And wiand w are ALF coefficients and c is dc-offset. (wi, w and c are signaled in the bitstream.)  It is noted that the ALF is parallel-processing friendly due to the following feature, although the proposed system utilizes both pre-DF and post-DF signals.

- pre-DF signal is mainly used (spatial filtered)

- post-DF signal is merely weighted.

Because DF and ALF-spatial filter are independent, the two filters can be processed in parallel.

In TMuC 0.7 implementation, the region used in the ALF is diamond shape as the same as the anchor ALF, which is a change from the original KTA base proposal where rectangle shape was used.

# Performance experiments

## TE10 condition test

The coding efficiency was evaluated in IntraOnly, RandamAccess and LowDelay conditions defined in TE10. The proposed ALF was compared to Sum-modified Laplacian based ALF which was used in the anchor.

The experimental results are shown in Table 1 (IntraOnly), Table 2 (RandamAccess) and Table 3 (LowDelay). Average bitrate reduction is shown in Figure 1. The detailed result of BD-rate Y is shown in Table 4.

Compared to the anchor ALF, the proposed technique shows 0.7 %, 0.4 %, and 0.4 % bitrate reduction on IntraOnly, RandamAccess and LowDelay respectively. In many cases, coding efficiency improves. The largest reduction is 1.7% for ParkScene at IntraOnly, 2.1% for Traffic at RandamAccess, and 3.0% for BQSquare at LowDelay. In some cases there are small degradations. The worst case is -1.0% for BQTerrace at RandamAccess.

Table 1: Experimental results in IntraOnly

|  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Class A | -0.9 | -0.2 | 0.3 |
| Class B | -0.9 | -0.8 | -0.5 |
| Class C | -0.5 | -1.2 | -1.0 |
| Class D | -0.8 | -1.1 | -1.0 |
| Class E | -0.5 | 0.1 | -0.3 |
| Average | -0.7 | -0.7 | -0.6 |

Table 2: Experimental results in RandamAccess

|  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Class A | -0.7 | -0.1 | -0.1 |
| Class B | 0.0 | -1.1 | -1.0 |
| Class C | -0.5 | -1.2 | -0.9 |
| Class D | -0.8 | -1.1 | -0.7 |
| Class E | - | - | - |
| Average | -0.4 | -1.0 | -0.8 |

Table 3: Experimental results in LowDelay

|  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- |
| Class A | -0.1 | -0.9 | -0.8 |
| Class B | -0.3 | -0.9 | -0.6 |
| Class C | -1.0 | -0.8 | -0.7 |
| Class D | -0.2 | -0.1 | 0.1 |
| Class E | - | - | - |
| Average | -0.4 | -0.7 | -0.5 |

Figure 1: Average bitrate reduction

Table 4: Detailed results of BD-rate Y

| # | Sequences | IntraOnly | RandamAccess | LowDelay |
| --- | --- | --- | --- | --- |
| S01 | Traffic | -1.0 | -2.1 | 0 |
| S02 | PeopleOnStreet | -0.7 | 0.6 | 0 |
| S03 | Kimono | -0.2 | 0.2 | 0.6 |
| S04 | ParkScene | -1.7 | -1.3 | -1.0 |
| S05 | Cactus | -0.9 | 0 | 0 |
| S06 | BasketballDrive | -0.7 | 0.2 | 0.7 |
| S07 | BQTerrace | -0.8 | 1.0 | -0.7 |
| S08 | BasketballDrill | -0.7 | -1.0 | -0.6 |
| S09 | BQMall | -0.7 | -0.6 | -0.5 |
| S10 | PartyScene | -0.4 | -1.1 | -0.7 |
| S11 | RaceHorses | -0.3 | 0.5 | 0.4 |
| S12 | BasketballPass | -0.9 | -0.4 | -0.1 |
| S13 | BQSquare | -0.4 | -1.7 | -3.0 |
| S14 | BlowingBubbles | -0.8 | -0.7 | -0.3 |
| S15 | RaceHorses | -1.0 | -0.3 | -0.6 |
| S16 | Vidyo1 | 0 | 0 | 0.4 |
| S17 | Vidyo3 | -0.9 | 0 | 0.1 |
| S18 | Vidyo4 | -0.5 | 0 | -1.1 |
| Average | Class A | -0.9 | -0.7 | 0 |
|  | Class B | -0.9 | 0 | -0.1 |
|  | Class C | -0.5 | -0.5 | -0.3 |
|  | Class D | -0.8 | -0.8 | -1 |
|  | Class E | -0.5 | 0 | -0.2 |
|  | All | -0.7 | -0.4 | -0.4 |

# Complexity

Complexity is measured by means of decoding time and encoding time. The results are shown in Table 5 and Table 6. Decoding time is smaller than the anchor (Time ratio are 93 % to 96 %). Encoding time is almost the same as the anchor (Time ratio is 100 %).

The proposed technique only adds one additional pixel to the 5x5, 7x7, 9x9 diamond pixels for the filtering. The impact on the processing complexity must be negligible. From the experiment, it can be said that the proposed technique is less complex than the anchor which uses multi-filter technique.

This good characteristics to improve coding efficiency is not exclusive of other techniques, such as multi-filter technique, but can be easily used with those ones.

Table 5: Experimental condition

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Windows Vista(32bit) | Linux(64bit) |
| CPU | Intel Core 2 Quad CPU Q9650@3.00GHz | Intel(R) Core(TM)2 Quad CPU Q9550 @ 2.83GHz Intel(R) Xeon(R) CPU X5667 @ 3.07GHz Intel(R) Core(TM)2 Quad CPU @ 2.66GHz |
| Compiler | Visual Studio 2005 | gcc 4.3.3 |

Table 6: Experimental results of time ratio

|  | Decoding Time ratio | Encoding Time ratio |
| --- | --- | --- |
| IntraOnly | 93 % | 100 % |
| RandamAccess | 96% | 100 % |
| LowDelay | 96 % | 100 % |

Table 7: Encoding pass

|  |  | Proposal | Anchor | Note |
| --- | --- | --- | --- | --- |
| Process | First filtering | 1 | 1 | 5x5 |
|  | Region selection | 12 | 12 | 12 = 4depths x 3 times number of each depth’s pass is ALF_NUM_OF_REDESIGN |
|  | Tap decision | 3 | 3 | 5x5, 7x7, 9x9 |
| Total |  | 16 | 16 |  |

# Conclusion

In this contribution, the parallel adaptive loop filter was evaluated as the part of TE10 (subset 2). The proposed approach combines De-blocking filter and Winner-based filter, where both pre-DF signal and post-DF signal are used as inputs. It provides coding efficiency improvement and parallel processing capability. The experiments show that the proposed approach provides 0.7 %, 0.4 %, and 0.4 % bitrate reduction on IntraOnly, RandamAccess, and LowDelay respectively. As for complexity, the decoding time is 93 % to 96 % of the anchor and the encoding time is almost the same as the anchor (100 %).

The proposal was crosschecked by MediaTek [4].  We think in the viewpoint of coding efficiency and complexity, the proposed technique has good characteristics which are suitable to a Test Model technique.

# Reference

- Tomoo Yamakage, Takeshi Chujoh, Shawmin Lei, Keiichi Chono, Marta Karczewicz, Andrew Segall, Tomoyuki Yamamoto, Byeungwoo Jeon, Jungyoup Yang, Jeongyeon Lim, Jizheng Xu, “Tool Experiment 10: In-loop filtering”, JCTVC-B310, 2nd JCT-VC Meeting, Geneva, July 2010.

- T. Ikai, T. Yamamoto, Y. Kitaura, “A parallel adaptive loop filter”, JCTVC-B064, 2nd JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Test Model under Consideration”, JCTVC-A205.

- C. Tsai, Y. Huang , S. Lei, “TE10 Subtest2: Crosscheck on SHARP’s Proposal of Adaptive Loop Filter by MediaTek”, JCTVC-C145, 3rd JCT-VC Meeting, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).