| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C186 WG11 Number: m18217 |
| --- | --- |

| Title: | TE5: Results for simplified intra prediction tests by Nokia |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema, Kemal Ugur Nokia Research Center Visiokatu 1 Tampere, Finland | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports Nokia results for the TE5 tests and verifies the correctness of the results submitted by other TE participants.

- Introduction

This TE evaluated performance of two approaches simplifying the reconstruction process for directional intra prediction. Both approaches (“simplified angular prediction” proposed in JCTVC-B093 and “lookup table approach” proposed in JCTVC-B118) resulted in negligible effect on coding efficiency.

- TE5 Results

The tables below summarize the Nokia results on this experiment. The full results are included in the accompanying Excel documents (JCTVC-C186_B093_Results.xls for JCTVC-B093 and JCTVC-C186_B118_Results.xls for B118). It should be noted that the results were generated on a cluster with nodes having different capabilities, and thus the reported encoder and decoder times are not comparable.

Intra configurations:

Results of simplified angular prediction (JCTVC-B093):

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 | 0.2 |
| Class D | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 102 % |  |  | 81 % |  |  |
| Dec Time[%] | 79 % |  |  | 91 % |  |  |

Results of using lookup table and additions (JCTVC-B118):

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100 % |  |  | 98 % |  |  |
| Dec Time[%] | 95 % |  |  | 86 % |  |  |

Random Access configurations:

Results of simplified angular prediction (JCTVC-B093):

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 89 % |  |  | 102 % |  |  |
| Dec Time[%] | 88 % |  |  | 87 % |  |  |

Results of using lookup table and additions (JCTVC-B118):

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 104 % |  |  | 100 % |  |  |
| Dec Time[%] | 104 % |  |  | 97 % |  |  |

Low Delay configurations:

Results of simplified angular prediction (JCTVC-B093):

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | -0.1 | 0.3 | 0.1 | 0.0 | 0.0 |
| Class E | 0.0 | 0.8 | 0.0 | 0.1 | -0.3 | 0.7 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Enc Time[%] | 101 % |  |  | 91 % |  |  |
| Dec Time[%] | 111 % |  |  | 89 % |  |  |

Results of using lookup table and additions (JCTVC-B118):

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | -0.1 |
| Class C | -0.1 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.0 | -0.1 | 0.2 | 0.0 | 0.1 | 0.0 |
| Class E | 0.0 | 0.5 | -0.4 | 0.1 | -0.1 | 0.7 |
| All | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 99 % |  |  | 98 % |  |  |
| Dec Time[%] | 111 % |  |  | 87 % |  |  |

# Patent rights declaration(s)

Nokia Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).