| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 Nov 2011 | Document: JCTVC-G_Notes_d0 |
| --- | --- |

| Title: | Meeting report of the seventh meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 21-30 Nov 2011 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11 held its seventh meeting during 21-30 Nov 2011 at the ITU-T premises in Geneva, CH. During the first two days of the meeting, rooms at the WMO headquarters were also used. The JCT-VC meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found in section 1.13 of this document.

The JCT-VC meeting sessions began at approximately 1100 hours on Monday 21 Nov 2011. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 30 Nov. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project now referred to as High Efficiency Video Coding (HEVC).

The primary goals of the meeting were to review the work that was performed in the interim period since the sixth JCT-VC meeting in implementing the 4th HEVC Test Model (HM4) and editing the 4th HEVC specification Working Draft (WD4), review the results from interim Core Experiments (CE), review technical input documents, further develop Working Draft and HEVC Test Model (HM), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-VC produced three particularly important output documents from the meeting: the HEVC Test Model 5 (HM5), the HEVC specification Working Draft 5 (WD5), and a document specifying common conditions and software reference configurations for HEVC coding experiments. Moreover, XX documents describing the planning of future CEs were drafted.

For the organization and planning of its future work, the JCT-VC established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 1–10 February 2012 under WG 11 auspices in San José, USA, 2–9 May 2012 under ITU-T auspices in Geneva, CH, 11–20 July 2012 under WG 11 auspices in Stockholm, SE, and XX-19 Oct 2012 under WG 11 auspices in Suzhou, CN.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 21-30 Nov 2011 at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 1100 hours on Monday 21 Nov 2011. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wedesday 30 Nox. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project now referred to as High Efficiency Video Coding (HEVC).

Some statistics for historical reference purposes:

- 1st meeting (Dresden):		188 people, 40 input documents

- 2nd meeting (Geneva):		221 people, 120 input documents

- 3rd meeting (Guangzhou):	244 people, 300 input documents

- 4th meeting (Daegu):		248 people, 400 input documents

- 5th meeting (Geneva):		226 people, 500 input documents

- 6th meeting (Torino):		254 people, 700 input documents

- 7th meeting (Geneva)		XXX people, XXX input docunents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2011_11_G_Geneva/JCTVC-G_Logistics.doc.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the sixth JCT-VC meeting in producing the 4th HEVC Test Model (HM) software and editing the 4th HEVC specification Working Draft (WD4), review the results from interim Core Experiments (CEs), review technical input documents, and establish fifth versions of the Working Draft (WD5) and HEVC Test Model (HM5).

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 8 Nov 2011.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday Nov 9 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

The following documents that did not arrive until the last two days of the meeting were not presented or discussed due to time constraints: JCTVC-Gxxx, … .

All contribution documents with registration numbers JCTVC-G867 to JCTVC-Gxxx were registered after the "officially late" deadline (and therefore were also uploaded late). Some documents in this range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

The following other technical proposal contributions were registered in time but were uploaded late:

- JCTVC-Gxxx (a technical proposal) [uploaded xx-xx]

- …

The following other document not proposing normative technical content was registered in time but uploaded late:

- JCTVC-Gxxx (a study on screen content test sequences)

The following cross-verification reports were uploaded late: JCTVC-Gxxx, … .

The following document registrations were later cancelled or otherwise never discussed or provided: JCTVC-Gxxx, … .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline:

- JCTVC-Gxxx (a cross-verification report, corrected 2011-xx-xx)

- …

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

### Measures to facilitate the consideration of contributions

It was agreed that, due to the increasingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report JCTVC-F800, the HEVC Test Model (HM) JCTVC-F802, and the Working Draft (WD) JCTVC-F803, were approved. The HM reference software produced by the AHG on software development and HM software technical evaluation was also approved.

Versions of the WD, the HM document, the HM software and the CE descriptions had been made available in a reasonably timely fashion.

The chair asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

Some such issues had been brought up on the reflector for group clarification on how to proceed.

It was remarked that in some cases (none specifically mentioned) the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

The activities in some CEs may have diverged from the original plans by bringing in somewhat different technology that may not have been fully understood even by the cross-checking participants.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

CE9 was suggested as a CE where there has been a need for greater discipline and where the situation became confusing.

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- AIS: Adaptive intra smoothing.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVR: Adaptive motion vector resolution.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- BUDI: Bidirectional UDI.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CE: Core experiment – a coordinated experiment conducted after the 3rd or 4th meeting.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIF: Directional interpolation filter.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ET: Encoding time.

- GPB: Generalized P/B – a not-particularly-well-chosen name for B pictures in which the two reference picture lists are identical.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- JM: Joint model – the primary software codebase developed for the AVC standard.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B frames.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LCEC: Low-complexity entropy coding.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- MC: Motion compensation.

- MDDT: Mode-dependent directional transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-VC).

- MRG: block merging mode for CUs.

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PIPE: Probability interval partitioning entropy coding (roughly synonymous with V2V for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QP: Quantization parameter.

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RPLM: Reference picture list modification.

- ROT: Rotation operation for low-frequency transform coefficients.

- RQT: Residual quadtree.

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SEI: Supplemental enhancement information (as in AVC).

- SPS: Sequence parameter set (as in AVC).

- TE: Tool Experiment – a coordinated experiment conducted after the 1st or 2nd JCT-VC meeting.

- TM: Test Model – a video coding design containing selected coding tools; as contrasted with the TMuC, see HM.

- TMuC: Test Model under Consideration – a video coding design containing selected proposed coding tools that are under study by the JCT-VC for potential inclusion in the HEVC standard.

- TPE: Transform precision extension.

- UDI: Unified directional intra.

- Unit types:

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- V2V: variable-length to variable-length prefix coding (roughly synonymous with PIPE for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

No particular non-routine opening remarks were recorded.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Jens-Rainer Ohm, and discussions on topic categorized as "Track B" were primarily chaired by Gary Sullivan.

Note: Counts may not be 100% precise

- AHG reports (22) Track P (section 2)

- Project development, status, and guidance (0) Track P (section 3)

- CE summary reports (13) – Reviewed with individual CE-related contributions

- CE1: Entropy coding investigation (26) Track P (section 4.1)

- CE2: Motion partitioning and OBMC (6) Track A (section 4.2)

- CE3: Motion compensation (33) Track A (section 4.3)

- CE4: Quantization (45) Track B (section 4.4)

- CE5: CAVLC entropy coding improvement (13) Track B (section 4.5)

- CE6: Intra prediction improvement (48) Track B (section 4.6)

- CE7: Additional transforms (14) Track B (section 4.7)

- CE8: Non-deblocking loop filtering (36) Track A (section 4.8)

- CE9: MV coding and skip/merge operation (30) Track A (section 4.9)

- CE10: Core transform design (8) Track B (section 4.10)

- CE11: Coefficient scanning and coding (15) Track B (section 4.11)

- CE12: Deblocking filter (38) Track A (section 4.12)

- CE13: Motion data parsing robustness and throughput (17) Track A (section 4.13)

- Clarifications and bug fix issues (1) Track P (section 5.1)

- HM settings and common test conditions (8) Track P (section )

- Source video test material (1) Track P (section )

- Functionalities (10) Track A (section )

- Loop filtering (85) Track A (section )

- Block structures and partitioning (22) Track A (section )

- Motion compensation operation and interpolation filters (31) Track A (section )

- Motion and mode coding (68) Track A (section )

- High-level syntax and slice structure (54) Track B (section 5.9)

- Quantization (35) Track B (section )

- Alternative coding modes (22) Track A (section )

- Entropy coding (38) Track B (section )

- Transform coefficient coding (60) Track B (section )

- Intra prediction and mode coding (72) Track B (section )

- Transforms (31) Track B (section 5.14.3)

- IBDI and memory compression (2) Track A (section )

- Parsing throughput, robustness and error resilience (13) Track A (section )

- Complexity assessment (3) Track A (section )

- Encoder optimization (6) Track A (section )

- Category not clear (1 to be resolved) (section Error: Reference source not found)

Overall Track P: 59; Track A: 428; Track B: 439

# AHG reports

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCTVC-G001 JCT-VC AHG Report: Project Management (AHG 1) [G. J. Sullivan, J.-R. Ohm (AHG chairs)] [miss]

AHG2 DOC

AHG3 DOC

JCTVC-G004 JCT-VC AHG report: Picture Partitioning and LCU scan order (AHG4) [R. Sjöberg (AHG chair), Y. Chen, F. Henry, M. Horowitz, K. Kazui, A. Segall (vice chairs)] [miss]

JCTVC-G005 JCT-VC AHG Report: Spatial Transforms (AHG 5) [P. Topiwala (AHG Chair), M. Budagavi, R. Cohen, R. Joshi (vice chairs)] [miss]

JCTVC-G006 JCT-VC AHG report: In-loop and post-processing filtering (AHG 6) [T. Yamakage, K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke] [miss]

AHG7 DOC

JCTVC-G008 JCT-VC AHG report: Reference pictures memory compression (AHG 8) [K. Chono (chair), T. Chujoh, D. Hoang, C. S. Lim, A. Tabatabai, M. Zhou (vice-chairs)]

AHG9 DOC

JCTVC-G010 JCT-VC AHG report: Quantization (AHG 10) [M. Budagavi, M. Karczewicz, G. Martin-Cocher, K. Sato] [miss]

AHG11 DOC

JCTVC-G012 JCT-VC AHG report: Complexity assessment (AHG 12) [D. Alfonso (AHG chair), J. Ridge, X. Wen (vice-chairs)]

JCTVC-G013 JCT-VC AHG report: Screen Content Coding (AHG 13) [O. Au, J. Xu, H. Yu (AHG chairs)] [miss]

AHG14 DOC

JCTVC-G015 JCT-VC AHG report: High-level syntax (AHG 15) [Y. -K. Wang (chair), J. Boyce, Y. Chen, M. M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan (vice chairs)] [miss]

AHG16 DOC

JCTVC-G017 JCT-VC AHG Report: Scalable coding investigation(AHG 17) [J. Boyce, J. Kang, K. Minoo, W. Wan, Y.-K. Wang]

JCTVC-G018 JCT-VC AHG report: Resolution adaption (AHG 18) [T. Davies (AHG chair), P. Topiwala, P. Wu (Vice-chairs)]

JCTVC-G019 JCT-VC AHG Report: Transform Skipping (AHG19) [M. Mrak (AHG chair), J. Sole, I.-K. Kim, J. Xu, H. Yu (vice chairs)] [late] [miss]

AHG20 DOC

JCTVC-G021 JCT-VC AHG report: Reference picture buffering and list construction (AHG21) [D. Flynn, R. Sjöberg (AHG chairs), Y. Chen, T.K. Tan, W. Wan, Y.-K. Wang (vice chairs)] [miss]

AHG22 DOC

# Project development, status, and guidance

See the section discussing functionalities for relevant contributions.

# Core experiments

## CE1: Entropy coding investigation

### Summary

JCTVC-G031 CE1: Summary report of core experiment on entropy coding [R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema (CE coordinators)] [miss]

### Contributions

Subset A

JCTVC-G200 CE1.A.3: Reducing line buffers for CABAC [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G298 CE1: Cross-check report for CE1 Subset A MediaTek's proposal on Reducing Line Buffers for CABAC [H. Sasai, T. Nishi (Panasonic)]

JCTVC-G349 CE1: SubsetA: Parallel context processing for coefficient coding using block-based context updates [H. Sasai, T. Nishi (Panasonic)]

JCTVC-G471 CE1.A.2: Crosscheck for Panasonic's parallel context processing for coefficient coding using block-based context updates in JCTVC-G349 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G576 CE1: Delayed state update for CABAC [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G822 CE1: Cross-check for delayed probability up-date from Qualcomm by Samsung [E. Alshina, J.H. Park] [late]

JCTVC-G769 CE1 subtest A: Line buffer removal for CU split flag context model [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G472 CE1.A.4: Crosscheck for Samsung's line buffer removal for CU split flag context model in JCTVC-G769 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G763 CE1: Table-based bit estimation for CABAC [F. Bossen (DOCOMO Innovations)]

Subset B

JCTVC-G764 CE1 (subset B): Multi-parameter probability up-date for CABAC [Alexander Alshin, Elena Alshina, JeongHoon Park (Samsung)]

JCTVC-G553 CE1: Cross-check of Samsung’s Multi-parameter probability model up-date for CABAC (JCTVC-F254) [J. Stegemann (HHI)]

Subset C

JCTVC-G210 CE1: Subtest 12, Entropy coding comparisons with simplified RDO [T. Davies (Cisco)]

JCTVC-G233 CE1: Subtest 13, Disabling CABAC PMU [T. Davies (Cisco)]

JCTVC-G765 CE1: Crosscheck of Subtest C - Tests 12 and 13 (JCTVC-G210 and JCTVC-G233) [R. Joshi (Qualcomm)]

JCTVC-G458 CE1c: results of PIPE/V2F [K. Sugimoto, R. Hattori, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G435 CE1 subtest C: Cross-checking reports of PIPE/V2F for low complexity entropy coding (JCTVC-F176) [A. Tanizawa, T. Shiodera, T. Yamakage (Toshiba)]

JCTVC-G459 CE1c: crosscheck of PIPE/V2V, CABAC [K. Sugimoto, R. Hattori, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G633 CE1: Report of test results related to PIPE-based Unified Entropy Coding [Heiner Kirchhoffer, Benjamin Bross, Anastasia Henkel, Detlev Marpe, Tung Nguyen, Matthias Preiß, Mischa Siekmann, Jan Stegemann, Thomas Wiegand (Fraunhofer HHI)]

JCTVC-G753 CE1: Crosscheck of CE1 subtest C.6 (JCTVC-G633) proposed by HHI [X. Zheng (HiSilicon)] [late] [miss]

JCTVC-G932 CE1: Cross-check for bug-fix version of test 12 in subset C from HHI (JCTVC-G633) by Samsung [E. Alshina, J.H. Park (Samsung)] [late] [miss]

JCTVC-G072 CE1: Cross-check of Subtest C (Tests 1, 2, 4, 6, 17) [C. Yeo (I2R)]

JCTVC-G300 CE1: Cross-check report for CE1 Subset C Test 7 [H. Sasai, T. Nishi (Panasonic)]

JCTVC-G612 CE1: Cross-check of test 9: Mitsubishi’s study on PIPE/V2F with LowDelay buffer control [M. Siekmann (Fraunhofer HHI)]

JCTVC-G641 CE1: Cross-check of Subtest C (test 1 and test 11) and Subtest B - multi-parameter probability update for CABAC (test 11) [Jinwen Zan, Dake He] [late] [miss]

JCTVC-G766 CE1: Crosscheck of Subtest C - Test 5 (JCTVC-G633) [R. Joshi (Qualcomm)]

JCTVC-G771 CE1 (subset C, test 11): Multi-parameter probability up-date for PIPE [A. Alshin, E. Alshina, J.H. Park (Samsung), H. Kirchhoffer (HHI)]

### Discussion and Conclusions

## CE2: Motion partitioning and OBMC

### Summary

JCTVC-G032 CE2: Summary report of core experiment on Motion Partitioning and OBMC [X. Zheng, I. S Chong, I.-K Kim] [miss]

### Contributions

JCTVC-G517 CE2 subtest C.1: Harmonization of unified scan and NSQT [X. Zheng (HiSilicon), Y. Yuan, Y. He (Tsinghua)]

JCTVC-G518 CE2 subtest C.1: Non-square quadtree (NSQT) with 2x8 and 8x2 transform [Y. Yuan (Tsinghua), X. Zheng (HiSilicon), Y. He (Tsinghua)]

JCTVC-G571 CE2: Cross-check of Non-Square Quadtree Transform (NSQT) JCTVC-G517 & JCTVC-G518 [S. Oudin, B. Bross(Fraunhofer HHI)]

JCTVC-G749 CE2: Overlapped Block Motion Compensation [L. Guo, I.S. Chong, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G432 CE2: Cross verification of Qualcomm's Overlapped Block Motion Compensation (JCTVC-G749) [S. Lee, S. Cho, N. Eum (ETRI)]

JCTVC-G939 Non-CE2: Crosscheck for Qualcomm's overlapped block motion compensation in JCTVC-G749 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late] [miss]

### Discussion and Conclusions

## CE3: Motion compensation

### Summary

JCTVC-G033 CE3: Summary report of Core Experiment on Motion Compensation [T.Chujoh, E.Alshina (CE coordinators)] [miss]

### Contributions

JCTVC-G057 CE3: An Adaptive Interpolation Filtering Technique [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz (eBrisk Video Inc.)]

JCTVC-G395 CE3: Cross check for eBrisk's proposal JCTVC-G057 (tool 6) [K. Kondo, T. Suzuki (Sony)]

JCTVC-G058 CE3: Interpolation using different-length horizontal and vertical filters [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz (eBrisk Video Inc.)]

JCTVC-G063 CE3: Adaptive interpolation using different-length horizontal and vertical filters [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz (eBrisk Video Inc.)]

JCTVC-G821 CE3: Cross-check for eBrisk proposals on interpolation MC by Samsung. [E. Alshina, J.H. Park] [late]

JCTVC-G258 CE3: LCU-based adaptive interpolation filter [S. Matsuo, S. Takamura, H. Jozawa (NTT)]

JCTVC-G379 CE3:Cross-check report of LCU-based adaptive interpolation filter (JCTVC-G258) [T. Yoshino, S. Naito (KDDI)]

JCTVC-G277 CE3: Progressive Motion Vector Resolution [J. An, X. Li, X. Guo, S. Lei (MediaTek)]

JCTVC-G178 CE3: Cross-check report for Tool 10 on Progressive MV Resolution [S. Park, B. Jeon (LGE)]

JCTVC-G189 CE3: Cross-verification result of Tool 10 [K.Kazui (Fujitsu)]

JCTVC-G534 CE3: Cross-check report of Tool 10 (Progressive MV resolution) by Panasonic [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G390 CE3: MC boundary filter (tool 7) [K. Kondo, T. Suzuki (Sony)]

JCTVC-G428 CE3: Cross-check report for Sony's tool 7 (JCTVC-G390) [T.Chujoh (Toshiba)]

JCTVC-G391 CE3: Tap length reduction for small block (tool 8) [K. Kondo, T. Suzuki (Sony), K. Ugur (Nokia)]

JCTVC-G250 CE3: Cross-check of JCTVC-G391 on MC memory band-width reduction (tool8) from Sony [E. François (Canon)] [late]

JCTVC-G427 CE3: Non-uniform tap length filtering (tool 2) [T.Chujoh, T.Yamakage (Toshiba)]

JCTVC-G393 CE3: Cross check for Toshiba's proposal JCTVC-G427 (tool 2) [K. Kondo, T. Suzuki (Sony)]

JCTVC-G535 CE3: Experimental Result on Tool 9 (Picture Adaptive 1/8-pel Motion Compensation Method) [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G137 CE3 : Cross-check of Panasonic’s Picture Adaptive 1/8-pel Motion Compensation : Test2b (F472) [S. Park (ETRI)]

JCTVC-G672 CE3: Crosscheck for Panasonic's Picture Adaptive 1/8 MC in JCTVC-G535 [J. An, X. Guo (MediaTek)]

JCTVC-G696 CE3: Fixed interpolation filter tests by Motorola Mobility [J. Lou, K. Minoo, D. Baylon, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-G697 CE3: Joint sub-pixel interpolation filter tests for bi-predicted motion compensation by Motorola Mobility [J. Lou, K. Minoo (Motorola Mobility)]

JCTVC-G820 CE3: Cross-check for Motorola Mobility proposals on interpolation MC by Samsung. [E. Alshina] [late]

JCTVC-G727 CE3: Adaptive resolution on motion vector difference [W.-J. Chien, X. Wang, M. Karczewicz]

JCTVC-G671 CE3: Crosscheck for Qualcomm's Adaptive MVD Resolution in JCTVC-G727 [J. An, X. Guo (MediaTek)]

JCTVC-G342 CE3: Cross-verification of Qualcomm’s adaptive motion vector resolution (JCTVC-G727) by Intel [Y. Chiu, W. Zhang, L. Xu, Y. Han (Intel)]

JCTVC-G775 CE3: 7Q6H taps interpolation filters test by Samsung [E. Alshina, A. Alshin, J.H. Park (Samsumg)]

JCTVC-G394 CE3: Cross check for Samsung's proposal (tool 1) [K. Kondo, T. Suzuki (Sony)]

JCTVC-G695 CE3: Cross-check report for Samsung’s interpolation filter 6H+7Q (JCTVC-G775) [J. Lou, K. Minoo, L. Wang (Motorola Mobility)]

JCTVC-G778 CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility [E. Alshina, A. Alshin, J.-H. Park, (Samsung), J. Lou, K. Minoo, (Motorola Mobility)]

JCTVC-G059 CE3: Cross-Verification of Samsung/Motorola’s interpolation filter variant B (JCTVC-Gxxx) [F. Kossentini, N. Mahdi (eBrisk Video Inc.)]

JCTVC-G636 CE3: Cross-Verification of Samsung/Motorola’s interpolation filter variant A [K. Ugur, O. Bici (Nokia)] [late] [miss]

JCTVC-G780 CE3: The worst case memory band-width reduction by 2D->1D interpolation replacement (from Samsung). [E. Alshina, A. Alshin, J. Chen, J.H. Park (Samsung)]

### Discussion and Conclusions

## CE4: Quantization

### Summary

JCTVC-G034 CE4: Summary report of Core Experiment on Quantization [K. Sato, M. Budagavi, M. Coban, H. Aoki, X. Li (CE coordinators)] [miss]

### Contributions

Subtest 1 (delta QP)

JCTVC-G721 CE4 Subtest 1.1.a: QP adaptation at sub-CU level [Xue Fang, Jae Hoon Kim, Krit Panusopone, Limin Wang (Motorola Mobility)]

JCTVC-G071 CE4 Subset 1.1.a: Cross-verification of delta QP coding at sub_CU level from Motorola (JCTVC-F577) [R. Sjoberg, J. Sun (Ericsson)]

JCTVC-G113 CE4 Subtest 1.1.a: Cross-verification of Motorola’s QP adaptation at sub_CU level (JCTVC-F577) [K. Chono, H. Aoki (NEC)]

JCTVC-G462 CE4 subset 1.2.a: results of Max/Min QP signaling [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G054 CE4 Subtest1.2.a: Study and cross-verification of Mitsubishi's signaling of Max and Min QP in slice (JCTVC-F174) [K. Chono, H. Aoki (NEC)]

JCTVC-G070 CE4 Subset 1.2.b: Ericsson's table-based delta QP coding method [R. Sjoberg, J. Sun (Ericsson)]

JCTVC-G055 CE4 Subtest 1.2.b: Cross-verification of Ericsson's table-based delta QP coding method (JCTVC-G70) [K. Chono, H. Aoki (NEC)]

JCTVC-G773 CE4 Subtest 1.2.c: Higher granularity of quantization parameter scaling [T. Lee, J. Chen, J. H. Park (Samsung), K. Chono (NEC)]

JCTVC-G508 CE4: cross-check result of subtest 1.2.c [K. Sato (Sony)]

JCTVC-G363 CE4: Improvement of delta-QP coding (1.3.a) [J. Xu, K. Kondo, K. Sato, A. Tabatabai (Sony)]

JCTVC-G755 CE4: Crosscheck of Sony's improvement of delta-QP coding [X. Zheng (HiSilicon)] [late] [miss]

JCTVC-G066 CE4 Subtest 1: Spatial QP prediction based on intra prediction (test 1.3.b) [H. Aoki, K. Chono (NEC), M. Kobayashi, M. Shima (Canon)]

JCTVC-G460 CE4 subset 1.3.b: crosscheck of QP prediction based on intra prediction [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G067 CE4 Subtest1: test 1.3.c [H. Aoki, K. Chono (NEC), M. Kobayashi, M. Shima (Canon), K. Sato (Sony)]

JCTVC-G313 CE4 Subtest 1.3.c: cross-verification of JCTVC-G067 [Y. Yasugi (Sharp)]

JCTVC-G731 CE4 Subtest 1.3.c: cross-verification of JCTVC-G067 [M. Coban (Qualcomm)] [late] [miss]

JCTVC-G728 CE4 Subtest 1: Spatial QP prediction (test 1.3.e): combination of test 1.3.b and test 1.3.d [M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G726 CE4 Subtest 1.3.d: Cross-check report for Qualcomm's JCTVC-G728 by Motorola Mobility [K. Panusopone (Motorola Mobility)]

JCTVC-G073 CE4: Cross-check of Subtest 1.3.e - Spatial QP prediction [C. Yeo (I2R)]

JCTVC-G068 CE4 Subtest1: QP prediction based on intra/inter prediction (test 1.4) [H. Aoki, K. Chono (NEC), M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G461 CE4 subset 1.4: crosscheck of QP prediction based on intra/inter prediction with QP buffer compression [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

Subtest 2 (de-quantization offset)

JCTVC-G278 CE4 Subtest 2: Adaptive-Dequantization Offset [X. Li, X. Guo, S. Lei (MediaTek)]

JCTVC-G901 CE4 Subtest 2.1.b: Cross-verification of JCTVC-G278 [M. Yang (Huawei)] [late]

JCTVC-G074 CE4: Cross-check of Subtest 2 on de-quantization offset (2.1.a, 2.3.d) [C. Yeo (I2R)]

JCTVC-G140 CE4: Subtest 2.1.c Cross Check Report for Mediatek's AQO CaseC by HKUST [F. Zou, O.C. Au (HKUST)]

JCTVC-G660 Cross-check of MediaTek's CE4 Subtest2.1 Adaptive Quantization Offset [X. Yu, D. He (RIM)]

JCTVC-G382 CE4 Subtest-2 Adaptive Reconstruction Levels [X. Yu, J. Wang, D. He, G. M.Cocher, E. Yang (RIM)]

JCTVC-G141 CE4: Subtest 2.2.d Cross Check Report for RIM's ARL Case D by HKUST [F. Zou, O.C. Au (HKUST)]

JCTVC-G403 CE4-Subtest-2 Cross-check of Adaptive Reconstruction Levels for Quantization Design (JCTVC-G382) [B. Li (USTC), J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-G902 CE4 Subtest 2.1.b: Cross-verification of JCTVC-G382 [M. Yang(Huawei)] [late]

JCTVC-G557 CE4.2.A: Cross-check of RIM's ARL [X. Li, X. Guo (MediaTek)] [late]

JCTVC-G823 CE4: Cross-check for ARL from RIM by Samsung [E. Alshina, J.H. Park] [late]

JCTVC-G843 CE4: Fine granularity QP offset [X. Wang, R. Joshi, G. Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G560 CE4.3.A: Cross-check of Qualcomm's fine-QP [X. Li, X. Guo (MediaTek)] [late]

JCTVC-G662 Cross-check of QualCom's CE4 Subtest2.3 Fine QP [X. Yu, J. Wang, D. He (RIM)]

JCTVC-G400 CE4: Equal expected magnitude (EEM) encoder-only adaptive quantization rule for HEVC [B. Li (USTC), J. Xu, G. J. Sullivan (Microsoft)]

Subtest 3 (quant matrices)

JCTVC-G083 CE4: Test results on compact representation of quantization matrices [M. Zhou, V. Sze (TI)]

JCTVC-G306 CE-4 Subset-3: Cross-check for TI’s Proposal (G083) on Quantization Matrix [Z. Zhou, X. Zhang (MediaTek)]

JCTVC-G503 CE4 Subtest 3.2: Crosscheck report of compact representation of quantization matrices proposed by TI (JCTVC-G083) [M. Shima (Canon)]

JCTVC-G528 CE4: Cross-Check report for CE4 subset3 of TI compact representation of quantization matrices (JCTVC-G083) [J. Zheng (HiSilicon)]

JCTVC-G434 CE4 subtest 3: Quantization matrix for HEVC based on JCTVC-F362 and F475 [Y. Morigami, J. Tanaka, T. Suzuki (Sony)]

JCTVC-G303 CE-4 Subset-3: Cross-check for Sony’s Proposal (G434) on Quantization Matrix [Z. Zhou, X. Zhang (MediaTek)]

JCTVC-G502 CE4 Subtest 3.1: Crosscheck report of AVC based quantization matrix support (JCTVC-G434) [M. Shima (Canon)]

JCTVC-G505 CE4 Subtest 3.3: Crosscheck report of Enhancement of quantization matrix coding of HEVC proposed by Sony (JCTVC-G434) [M. Shima (Canon)]

JCTVC-G527 CE4: Cross-Check report for CE4 subset3 of Sony proposal on Quantization matrix for HEVC (JCTVC-G434) [J. Zheng (HiSilicon)]

### Discussion and Conclusions

## CE5: CAVLC entropy coding improvement

### Summary

JCTVC-G035 CE5: Summary report on CAVLC entropy coding improvements [X. Wang, P. Wu, C. Kim (CE Coordinators)] [miss]

### Contributions

JCTVC-G310 CE5: CAVLC Adaptation using Difference Counter [T. Yamamoto (Sharp)]

JCTVC-G402 CE5: Cross-check of CAVLC Adaptation using Difference Counter (JCTVC-G310) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G360 CE5: Redundancy removal for Run-mode in CAVLC (JCTVC-F286) [J. Xu, A. Tabatabai (Sony)]

JCTVC-G389 CE5: CAVLC coding table modification [S. Kim, J. Lee, S. Lee (Yonsei Univ.), C. Kim, Y. Park, J. Park (Samsung)]

JCTVC-G851 CE5: Crosscheck of Yongsei and Samsung's contribution (JCTVC-G389) on CAVLC coding table modification [X. Wang (Qualcomm)] [late]

JCTVC-G532 CE5 2.1 : Improvement of CAVLC run- coding by prediction mode [C. Kim, Y. Park, K.P.Choi (Samsung)]

JCTVC-G367 CE5: cross-check for Samsung’s CAVLC (JCTVC-G532) [J. Xu, M. Haque (Sony)]

JCTVC-G563 CE5 2.2 : Handling for exception cases longer than 32bit code-word in CAVLC [C. Kim, Y.Park, K.P. Choi(Samsung), M. Karczewicz, X. Wang, W.-J. Chien, L. Guo(Qualcomm)]

JCTVC-G841 CE5.2.2: Crosscheck for Samsung and Qualcomm's handling for exception cases longer than 32bit code-word in CAVLC in JCTVC-G563 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-G674 CE5: Sub-block coding of transform coefficients with CAVLC [M. Karczewicz, L. Guo, Y. Zheng, X. Wang (Qualcomm)]

JCTVC-G340 CE5: Cross-check of CAVLC coefficients coding (JCTVC-G674) [T. Yamamoto (Sharp)]

JCTVC-G677 CE5: Limitation on VLC codeword length [M. Karczewicz, X. Wang, W.J. Chien, L. Guo (Qualcomm)]

JCTVC-G924 CE5: Cross-verification of Qualcomm’s proposal (JCTVC-G677) [C. Kim (Samsung)] [late]

### Discussion and Conclusions

## CE6: Intra prediction improvements

### Summary

JCTVC-G036 CE6: Summary Report on Intra Coding Improvements [A. Tabatabai, E. Francois, K. Chono, H. Yu, R. Joshi, J. Lainema] [miss]

### Contributions

Experiment A

JCTVC-G129 CE6.a: Sub-sampling portion of neighboring pixels in calculation of LM parameters [M. Budagavi (TI), K. Sato (Sony)]

JCTVC-G297 CE6: Cross-check report for CE6a TI&Sony's proposal on Sub-sampling portion of neighboring pixels in calculation of LM parameters [H. Sasai, T. Matsunobu, T. Nishi (Panasonic)]

JCTVC-G172 CE6a: Modified down-sampling filter for LM mode of intra chroma prediction [Y. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel)]

JCTVC-G469 CE6a: Cross-check on LM mode of intra chroma prediction(JCTVC-G172) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-G455 CE6: results of filtering for LM_mode [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-G168 CE6: Cross-check report for Subtest CE6a on Intra Chroma Prediction [S. Cho, S. Lee (ETRI)] [late]

JCTVC-G169 CE6: Cross-check report for Subtest CE6a on Intra Chroma Prediction [S. Cho, S. Lee (ETRI)] [late]

JCTVC-G510 CE6.a: Result of Subtest 4.1.2.2 [K. Sato (Sony)]

JCTVC-G294 CE6: Cross-check report for CE6a Sony's proposal on Improvement to chroma intra prediction from luma [H. Sasai, T. Matsunobu, T. Nishi (Panasonic)]

JCTVC-G512 CE6: combination of subtest 4.1.2.1 & 4.1.2.2 [K. Sato (Sony)] [late]

JCTVC-G847 CE6: Cross-check results for combination of subtest 4.1.2.1 & 4.1.2.2 (JCTVC-G512) [S. Cho, S. Lee, N. Eum] [late]

Experiment B

JCTVC-G192 CE6b: Intra remaining mode coding with mode ranking [J. Park, B. Jeon (LG)]

JCTVC-G203 CE6b: Intra prediction mode coding [T.-D. Chuang, C.-Y. Chen, M. Guo, X. Guo, Y.-W. Huang, S. Lei (MediaTek), W.-J. Chien, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G186 CE6: Cross-verification of MediaTek and Qualcomm's proposal (JCTVC-G203) by JVC KENWOOD [T. Kumakura, S. Fukushima (JVC Kenwood)]

JCTVC-G252 CE6b: Cross-check of JCTVC-G203 on intra mode coding from Qualcomm/MediaTek [E. François (Canon)] [late]

JCTVC-G242 CE6b: Mode ranking for remaining mode coding with 2 or 3 MPMs [E. François, S. Pautet, C. Gisquet (Canon)]

JCTVC-G243 CE6b: Intra mode coding with 4 MPMs and mode ranking [E. François, S. Pautet (Canon), Joonyoung Park, Byeongmoon Jeon (LG), Tzu-Der Chuang, Ching-Yeh Chen, Mei Guo, Xun Guo, Yu-Wen Huang, Shawmin Lei (MediaTek), Wei-Jung Chien (Qualcomm), Ehsan Maani, Ali Tabatabai (Sony)]

JCTVC-G311 CE6b: Cross-check of Intra Mode Coding (JCTVC-G242 and JCTVC-G243) [T. Yamamoto (Sharp)]

JCTVC-G869 CE6: Combinations of MPM derivation and remaining mode coding [Ehsan Maani, Ali Tabatabai] [late]

JCTVC-G080 CE6: Cross-check report for Subtest CE6b on Intra Mode Coding [H. L. Tan, C. Yeo, Y. H. Tan (I2R)]

JCTVC-G167 CE6: Cross-check report for Subtest CE6b on Intra Mode Coding [S. Cho, S. Lee, N. Eum (ETRI)] [late]

JCTVC-G276 CE6: Cross-Check report for CE6b subtests of Intra Mode Coding (F062, F459, F091, F106, F269)) [G. Li, N. Ling (Santa Clara Univ.), L. Liu, J. Zheng, P. Zhang (Huawei)]

JCTVC-G868 CE6: Test results of DCIM [Ehsan Maani, Ali Tabatabai, Tomoyuki Yamamoto] [late]

JCTVC-G789 CE6.b Crosscheck for DCIM [C. Lai] [late]

Experiment C

JCTVC-G142 CE6.c: LM mode harmonization on SDIP [J. Lim, B. Jeon (LG)]

JCTVC-G476 CE6.c: Crosscheck for LG's LM mode harmonization on SDIP in JCTVC-G142 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G143 CE6.c: VLC improvement for intra partitioning on SDIP [J. Lim, B. Jeon (LG)]

JCTVC-G478 CE6.c: Crosscheck for LG's VLC improvement for intra partitioning on SDIP in JCTVC-G143 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G800 CE6.c Crosscheck report for LG's JCTVC-G142 and JCTVC-G143 [C. Lai] [late]

JCTVC-G267 CE6.c Report on SDIP chroma extension scheme [J. Song, C. Lai, H. Yang, H. Yu (Huawei)]

JCTVC-G836 CE6.c: Cross-verification of Huawei’s SDIP chroma extension scheme (JCTVC-G267) [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G322 CE6.c: Harmonization of HE residual coding with non-square block transforms [J. Sole, R. Joshi, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G797 CE6.c Crosscheck report for JCTVC-G322 [C. Lai (HiSilicon)] [late]

JCTVC-G890 CE 6: Cross-check of G322 on harmonization of non-square residual coding [Marta Mrak, Andrea Gabriellini (BBC) [late] [miss]

JCTVC-G556 CE6.c Report on Simplification of Short Distance Intra Prediction (SDIP) Method [X. Cao, Y. He (Tsinghua), X. Peng (USTC), C. Lai, L. Liu, J. Zheng (HiSilicon), J. Xu (Microsoft), H. Yang, H. Yu (Huawei)]

JCTVC-G558 CE6.c Report on Combination of SDIP and Its Improvements [X. Cao, Y. He(Tsinghua), X. Peng(USTC), C. Lai, L. Liu, J. Zheng(HiSilicon), J. Xu (Microsoft), H. Yang, J. Song, H. Yu(Huawei), J. Lim, B. Jeon(LGE), J. Sole, R. Joshi, X. Wang, M. Karczewicz(Qualcomm), J. Xu, E. Maani, A. Tabatabai(Sony)]

JCTVC-G895	CE6: Cross-verification of Combination of SDIP and Its Improvements (JCTVC-G558) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-G260 CE6.c: cross-check of case 1, 2 and 3 (Huawei JCTVC-Gxxx) J. Jung (Orange Labs) [late] [miss]

JCTVC-G369 CE6.c: cross-check for SDIP (JCTVC-F532) [J. Xu, A. Tabatabai (Sony)]

Experiment D

JCTVC-G280 CE6 Subtest d: Intra Prediction with Secondary Boundary [M. Guo, X. Guo, X. Zhao, S. Lei (MediaTek), J. Lainema, K. Ugur (Nokia), K. Sugimoto, S. Sekiguchi, A. Minezawa (Mitsubishi), J. Lee, S.-C. Lim, H.Y. Kim, J.S. Choi (ETRI)]

JCTVC-G420 CE6.d: Results of experiment 4.4.3 [J. Lee, S.-C. Lim, H. Y. Kim (ETRI)]

JCTVC-G565 CE6.d: Nokia report on intra prediction with secondary boundary [J. Lainema, K. Ugur (Nokia)]

JCTVC-G081 CE6: Cross-check report for Subtest CE6d on Intra prediction with secondary boundary (Test 7) [H. L. Tan, Y. H. Tan, C. Yeo (I2R)]

JCTVC-G293 CE6.d: Cross Check of Category 4 on Tool Combinations for Intra Prediction with Secondary Boundary [G. Van der Auwera (Qualcomm)]

JCTVC-G361 CE6.d: Crosscheck of JCTVC-F122 on gradient based intra prediction for vertical and horizontal set secondary boundary intra prediction from MediaTek [C. Auyeung (Sony)]

JCTVC-G436 CE6.d category 2: Cross-checking reports of intra prediction with secondary boundary (JCTVC-F172 and JCTVC-F456) [A. Tanizawa, T. Shiodera (Toshiba)]

JCTVC-G561 CE6d: Cross-check report on Intra prediction with secondary boundary (Test 7) (JCTVC-F456) [T Guionnet, L Guillo]

JCTVC-G790 CE6.d crosscheck [C. Lai] [late]

### Discussion and Conclusions

## CE7: Additional transforms

### Summary

JCTVC-G037 CE7: Summary report of Core Experiment on additional transforms [R. Cohen, C. Yeo, R. Joshi, F. Fernandes] (CE coordinators) [miss]

### Contributions

JCTVC-G108 CE 7 : On secondary transforms for intra prediction residual [A. Saxena (Samsung), Y.Shibahara (Panasonic), F. Fernandes (Samsung), T.Nishi (Panasonic)]

JCTVC-G075 CE7: Cross-check of Unified Tools 1 & 2 - Mode-Dependent Secondary Transform [C. Yeo, Y. H. Tan, Z. Li (I2R)]

JCTVC-G425 CE7: Cross-check of the unified design on mode-dependent secondary transform (JCTVC-G108) by Huawei [H. Yang, H. Yu (Huawei)]

JCTVC-G581 CE7: Crosscheck of combination of tool 1 and tool 2 for mode dependent secondary transform sizes 3x3 and 4x4 (JCTVC-G108) [R. Joshi (Qualcomm)]

JCTVC-G814 CE7: Cross verification of JCTVC-G108, On secondary transforms for intra prediction residual [R. Cohen (MERL)]

JCTVC-G888 CE 7: Crosscheck of G108 on secondary transforms [Marta Mrak, Andrea Gabriellini (BBC)] [late]

JCTVC-G930 CE7: Cross check report of “On secondary transforms for intra prediction residual (G108)” [A. Ichigaya, (NHK)] [late]

JCTVC-G304 CE 7: Experimental Results for the ROT [Z. Ma, F. Fernandes, E. Alshina, A. Alshin (Samsung)]

JCTVC-G375 CE7: Cross Check Report for CE7 Tool 3, Rotational Only Transform [Y. Shibahara, T. Nishi (Panasonic)]

JCTVC-G580 CE7: Crosscheck of rotational transform (JCTVC-G304) [R. Joshi (Qualcomm)]

JCTVC-G734 CE 7: Crosscheck of G304 ROT Transform [P. Topiwala (FastVDO)]

JCTVC-G107 CE 7: Mode-Dependent DCT/DST for 4x4 Chroma Blocks [A. Saxena, F. Fernandes, E. Alshina, J. Chen (Samsung)]

JCTVC-G076 CE7: Cross-check of Tool 4 - DST for Intra Chroma Residual Coding [C. Yeo, Y. H. Tan, Z. Li (I2R)]

JCTVC-G376 Cross-check report of CE7 Tool 4, Mode-Dependent DCT/DST for 4x4 Chroma Blocks (G107) [Y. Shibahara, T. Nishi (Panasonic)]

### Discussion and Conclusions

## CE8: Non-deblocking loop filtering

### Summary

JCTVC-G038 CE8: Summary report of Core Experiment on non-deblocking loop filtering [T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang] [miss]

### Contributions

Subtest A

JCTVC-G316 CE8.a.1: 2-D mergeable syntax [T. Ikai (Sharp), I. S. Chong, M. Karczewicz (Qualcomm), T. Yamakage, T. Watanabe, T. Chujoh (Toshiba), C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G649 CE8 Subtest a: Cross-check of Tool 1 (JCTVC-G316) 2-D mergeable syntax [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-G609 CE8.a.2: Directional feature calculation on subset of pixels [T. Yamakage, T. Watanabe, T. Chujoh (Toshiba), C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), M. Karczewicz, I. S. Chong (Qualcomm), T. Ikai, A. Segall, T. Yamamoto (Sharp)]

JCTVC-G464 CE8 subtest a tool 2: crosscheck of Directional feature calculation on subset of pixels [K. Sugimoto, K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G650 CE8 Subtest a: Cross-check of Tool 2 (JCTVC-G609) Directional feature calculation on subset of pixels [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-G647 CE8 Subtest a, Tool 3: Block-based filter adaptation with up to 8 filters (HV8) [P. Lai, F. C. A. Fernandes, I.-K. Kim (Samsung)]

JCTVC-G611 CE8.a.3: Cross check of Samsung's directional feature calculation on subset of pixels [I. S. Chong, M. Karczewicz (Qualcomm)]

Subtest B

JCTVC-G498 CE8: ALF with low latency and reduced complexity [A. Fuldseth, G. Bjøntegaard (Cisco)]

JCTVC-G371 CE8 Subset b: cross-check for Cisco’s ALF (JCTVC-G498) [J. Xu, A. Tabatabai (Sony)]

JCTVC-G864 CE8 Subset b.1: Cross check of Cisco's ALF with low latency and reduced complexity [I. S. Chong, M. Karczewicz]

Subtest C

JCTVC-G564 CE8 subtest c tool 1: Line memory reduction for ALF and SAO decoding [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G051 CE8 Subset 3: Cross-check of Panasonic’s line memory reduction for in-loop filtering (JCTVC-F272) [S. Park, S. Lee, N. Eum (ETRI)]

JCTVC-G479 CE8.c.1: Crosscheck for Panasonic's line memory reduction for ALF and SAO decoding in JCTVC-G564 [C.-Y. Chen, Y.-W. Huang (MediaTek)]

JCTVC-G204 CE8.c.2: Single-source SAO and ALF virtual boundary processing [C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G559 CE8 Subtest c: Cross-Check report for JCTVC-G204 [S. Esenlik, A. Kotra, M. Narroschke(Panasonic)]

JCTVC-G614 CE8 Subset c.2: Cross-Verification of MediaTek’s Line memory reduction in ALF and SAO [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G205 CE8.c.3: Multi-source SAO and ALF virtual boundary processing [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G234 CE8.c.3: Cross-verification of MediaTek's and Panasonic's proposal on multi-source SAO and ALF virtual boundary processing (JCTVC-G205) [M. Matsumura, S. Takamura, H. Jozawa (NTT)]

JCTVC-G206 CE8.c.4: SAO and ALF virtual boundary processing with cross9x9 [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G433 CE8.c.4: Cross-check of MediaTek/Panasonic's SAO and ALF virtual boundary processing with cross9x9 JCTVC-G206 [T. Yamakage, T. Watanabe (Toshiba)]

JCTVC-G207 CE8.c.5: Non-cross-slices SAO [C.-M. Fu, C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek), M. Budagavi (TI)]

JCTVC-G414 CE8.c.5: Cross-check of MediaTek/TI's non-cross-slices SAO JCTVC-G207 [T. Yamakage, T. Watanabe (Toshiba)]

Subtest D

JCTVC-G208 CE8.d.1: Snowflake5x5 and cross9x9 for luma and chroma ALF shapes [C.-Y. Tsai, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), T. Yamakage, T. Watanabe, T. Chujoh (Toshiba)]

JCTVC-G133 CE8: Crosscheck of JCTVC-G208 - Snowflake5x5 and cross9x9 for luma and chroma ALF shapes [M. Budagavi (TI)] [late]

JCTVC-G648 CE8 Subtest d, Tool 2: ALF filters with 9 coefficients and up to vertical-size 7 [P. Lai, F. C. A. Fernandes (Samsung), H. Guermazi, F. Kossentini, M.Horowitz (eBrisk)]

JCTVC-G613 CE8 Subset d.2: Cross-Verification of Samsung’s Filter Shapes and Coefficients constraints [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G130 CE8 Subtest d - Chroma ALF with reduced vertical filter size [M. Budagavi, V. Sze, M. Zhou (TI)]

JCTVC-G652 CE8 Subtest d: Cross-check of Tool 3 (JCTVC-G130): Chroma ALF with reduced vertical filter size, proposed by TI [P. Lai, F. C. A. Fernandes (Samsung)]

Subtest E

JCTVC-G665 CE8.e.3: ALF coefficient prediction [K. Andersson (Ericsson)]

JCTVC-G928 CE8.e.3: Cross-check of Ericsson's ALF coefficient prediction JCTVC-G665 [T. Yamakage, T. Watanabe (Toshiba)] [late] [miss]

Subtest F

JCTVC-G060 CE8 Subtest F: Verification results of Panasonic's Proposal [F. Kossentini, H. Guermazi (eBrisk Video Inc.)]

Subtest G

JCTVC-G056 CE8 Subtest G: Adaptive Loop Filtering of Chrominance Samples Using Luma Map [F. Kossentini, H. Guermazi, N. Mahdi, M. Horowitz (eBrisk Video Inc.)]

JCTVC-G480 CE8.g.1: Crosscheck for eBrisk's adaptive loop filtering of chrominance samples using luma map in JCTVC-G056 [C.-Y. Tsai, Y.-W. Huang (MediaTek)]

Subtest H

JCTVC-G235 CE8.h: CU-based ALF with non-local means filter [M. Matsumura, S. Takamura, H. Jozawa (NTT)]

JCTVC-G299 CE8 Subtest h: Cross verification of NTT’s CU-based ALF with NLM filter (JCTVC-G235) by Intel [Y. Chiu, L. Xu (Intel)]

JCTVC-G482 CE8.h.1: Crosscheck for NTT's CU-based ALF with non-local means filter in JCTVC-G235 [C.-Y. Tsai, Y.-W. Huang] (MediaTek)

### Discussion and Conclusions

## CE9: MV coding and skip/merge operation

### Summary

JCTVC-G039 CE9: Summary Report of Core Experiment on MV Coding and Skip/Merge Operations [B. Bross, J. Jung, W.-J. Chien, I.-K. Kim, M. Zhou (CE coordinators)] [miss]

### Contributions

JCTVC-G052 CE9: Results of MRG_MVD series [S. Fukushima, M. Ueda, K. Arakage, S. Sakazume (JVC Kenwood)]

JCTVC-G179 CE9: Cross-check report of MRG_PAR03 and 04 on Parallel ME [S. Park, B. Jeon (LGE)]

JCTVC-G190 CE9: Cross-verification result of MRG_MVD01 [K.Kazui (Fujitsu)]

JCTVC-G536 CE9: Cross-check report of MRG_MVD02 by Panasonic [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G688 CE09: Crosscheck report of JCTVC-G52 test MRG_MVD03 [Y. Zheng, X. Wang (Qualcomm)]

JCTVC-G053 CE9: Results of AMVP_SEL01 and AMVP _SEL02 [H. Nakamura, S. Fukushima, M. Nishitani, H. Takehara (JVC Kenwood)]

JCTVC-G084 CE9: Test results on SP01, SP02, SP03 and SP04 [M. Zhou (TI)]

JCTVC-G261 CE9: cross-check of experiment SP01 – Remove TMVP center position (JCTVC-G084) [J. Jung (Orange Labs)]

JCTVC-G706 CE9: Cross-check report for TI's JCTVC-F083 by Motorola Mobility [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G689 CE09: Crosscheck report of JCTVC-G084 test SP04 [Y. Zheng, X. Wang (Qualcomm)]

JCTVC-G085 CE9: Test results on parallelized merge/skip mode [M. Zhou (TI)]

JCTVC-G404 CE9: Cross-check of parallelized merge/skip mode (JCTVC-G085) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G439 CE9 subtest 2.6: Results of MRG_ID01 in Merge/Skip operations for identical motion [A. Tanizawa, T. Shiodera, T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-G422 CE9: Cross-check for MRG_ID01 (JCTVC-G439) [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee (ETRI)]

JCTVC-G421 CE9: Results of Experiments MRG_ID02, MRG_ID03, and MRG_ID04 [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee (ETRI)]

JCTVC-G745 CE9: Cross-check report on MRG_ID02 A and B (JCTVC-G421) [I.-K. Kim (Samsung)]

JCTVC-G161 CE9: cross-verification of ETRI proposal on merge/skip operations for identical motion (MRG_ID03 A, MRG_ID03 B) [Y. Jeon, B. Jeon (LG)]

JCTVC-G437 CE9 subtest 2.6: Cross-checking reports of MRG_ID04 (ETRI/KHU’s proposal) [A. Tanizawa, T. Shiodera (Toshiba)]

JCTVC-G702 CE9: Simplification of MVP Design for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G513 CE9: Cross-check result of Sel01&02 [K. Sato (Sony)]

JCTVC-G101 CE9 subset 2.3: cross-verification of MMI’s proposal on AMVP simplification Sel03 & Sel04 [M. Zhou (TI)]

### Discussion and Conclusions

## CE10: Core transforms

### Summary

JCTVC-G040 CE10: Summary Report of Core Experiment on Core Spatial Transforms [P. Topiwala, M. Budagavi, R. Joshi, E. Alshina (CE coordinators)]

### Contributions

JCTVC-G266 CE10: Lossless Core Transforms for HEVC [W. Dai, M. Krishnan, J. Topiwala, P. Topiwala (FastVDO), E. Alshina (Samsung)]

JCTVC-G694 CE10: Cross-check report for FastVDO/Samsung's core Transform by Motorola Mobility (JCTVC-G266) [J. Lou, L. Wang (Motorola Mobility)]

JCTVC-G737 CE10: Full Factorization Core Transforms for HEVC [E. Alshina, A. Alshin, W. Lee, J. Park, K. Pachauri (Samsung), P. Topiwala (FastVDO)]

JCTVC-G863 CE10: Crosscheck of FastVideo/Samsung core transforms for high and low QP range [Rajan Joshi] [late] [miss]

JCTVC-G495 CE10: Core transform design for HEVC [A. Fuldseth, G. Bjøntegaard (Cisco), M. Budagavi (TI)]

JCTVC-G579 CE10: Scaled integer transforms supporting recursive factorization structure [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G953 CE10: Cross-check of JCTVC- G579 core transform - low and high QP range [M. Budagavi (TI)] [late] [miss]

JCTVC-G819 CE10: Cross check for core transform proposed by Qualcomm by Samsung [E. Alshina, J.H. Park] [late]

JCTVC-G887 CE 10: hardware test of inverse transform proposals [Sumit Johar, Daniele Alfonso (STM)] [late]

### Discussion and Conclusions

## CE11: Coefficient scanning and coding

### Summary

JCTVC-G041 CE11: Summary report of Core Experiment on coefficient scanning and coding [V. Sze, J. Chen, T. Nguyen, K. Panusopone, J. Sole (CE coordinators)] [miss]

### Contributions

JCTVC-G121 CE11: Reduction in contexts used for coefficient level [V. Sze (TI)]

JCTVC-G327 CE11: Cross-check of TI’s reduction in contexts used for coefficient level (JCTVC-G121) [J. Sole (Qualcomm)]

JCTVC-G269 CE11 Report on Prediction Unit Dependent Coefficient Scanning For Inter Frame [J. Song, X. Zheng, H. Yang, H. Yu (Huawei)]

JCTVC-G708 CE11:Cross-check report for Huawei's JCTVC-F501 by Motorola Mobility [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G792 Cross-check for Huawei’s proposal on PU dependent coefficient scan for inter block (JCTVC-G269) [Y. Piao, J. Chen, J. Min, J.H. Park (Samsung)]

JCTVC-G284 CE11: Extended Mode Dependent Coefficient Scanning [X. Zhao, X. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU)]

JCTVC-G364 CE11: Crosscheck of JCTVC-F124 on Extended Mode-Dependent Coefficient Scanning from MediaTek [C. Auyeung (Sony)]

JCTVC-G320 CE11: Scanning of Residual Data in HE [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-G412 CE11: Cross-check result of Significance Map interleaving part of Qualcomm proposal on Parallel Processing of Residual data in HE (G320) [C. Rosewarne, M. Maeda (Canon)]

JCTVC-G321 CE11: Removal of the parsing dependency of residual coding on intra mode [J. Sole, Y. Zheng, W.-J. Chien, R. Joshi, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G124 CE11: Cross-check of proposal on Removal of the parsing dependency of residual coding on intra mode G321 (F550) [V. Sze (TI)]

JCTVC-G302 CE11: Cross-check report for CE11 B2 Qualcomm's proposal on Removal of the parsing dependency of residual coding on intra mode [H. Sasai, T. Nishi (Panasonic)]

JCTVC-G679 CE11: Extending horizontal and vertical scan to big block for CAVLC [M. Karczewicz, Y. Zheng, L. Guo, X. Wang (Qualcomm)]

JCTVC-G703 CE11: Adaptive Scan for Large Blocks for HEVC [Y. Yu, K. Panusopone, J. Lou, L. Wang (Motorola Mobility)]

JCTVC-G077 CE11: Cross-check of CE.B1 - Scans for large blocks in CAVLC [C. Yeo (I2R)]

### Discussion and Conclusions

## CE12: Deblocking filter

### Summary

JCTVC-G042 CE12: Summary report of Core Experiment on deblocking filtering [A. Norkin, X. Guo, B. Jeon, M. Narroschke (CE coordinators)] [miss]

### Contributions

Subtest 1

JCTVC-G286 CE12 Subtest 1: Chroma Deblocking Filter [Q. Huang, J. An, X. Guo, S. Lei (MediaTek), A. Norkin, K. Andersson, R. Sjöberg (Ericsson)]

JCTVC-G410 CE12, Subset 1: Cross-verification report for MediaTek and Ericsson's proposal (JCTVC-G286) [Z. Shi (USTC), J. Xu (Microsoft)]

JCTVC-G287 CE12 Subtest 1: Simplified Chroma Deblocking Filter in JCTVC-G286 [Q. Huang, J. An, X. Guo, S. Lei (MediaTek), A. Norkin, K. Andersson, R. Sjöberg (Ericsson)]

JCTVC-G411 CE12, Subset 1: Cross-verification report for MediaTek and Ericsson's proposal (JCTVC-G287) [Z. Shi (USTC), J. Xu (Microsoft)]

JCTVC-G383 CE12 Subtest1: Deblocking of New Non-Square Blocks: Edge Shift for AMP [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G485 CE12.1.5: Crosscheck for Qualcomm's AMP deblocking with edge shift in JCTVC-G383 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G409 CE12, Subset 1: Report of Deblocking for Large Size Blocks [Z. Shi (USTC), X. Sun, J. Xu (Microsoft)]

JCTVC-G673 CE12 Subtest 1: Crosscheck for Microsoft's Deblocking for Larger Blocks in JCTVC-G409 [Q. Huang, X. Guo (MediaTek)]

JCTVC-G590 CE12 Subtest 1: Results for modified decisions for deblocking [Matthias Narroschke, Semih Esenlik, Thomas Wedi (Panasonic)]

JCTVC-G238 CE12 Subtest1: Cross-verification of Panasonic's proposal JCTVC-G590 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-G621 CE12: Cross-verification of F191 from Panasonic [Andrey Norkin (Ericsson)] [late] [miss]

Subtest 2

JCTVC-G255 CE12 Subtest2: Parallel deblocking improvement based on JCTVC-F214 tool1 [M. Ikeda, J. Tanaka, T. Suzuki (Sony)]

JCTVC-G585 CE12 Subtest 2: Cross-check results of the parallel deblocking tool 1 of Sony (JCTVC-G255) [Matthias Narroschke, Semih Esenlik (Panasonic)]

JCTVC-G256 CE12 Subtest2: Parallel decision improvement and reduction based on JCTVC-F214 tool2 [M. Ikeda, J. Tanaka, T. Suzuki (Sony)]

JCTVC-G587 CE12 Subtest 2: Cross-check results of the parallel deblocking tool 2 of Sony (JCTVC-G256) [Matthias Narroschke, Semih Esenlik (Panasonic)]

JCTVC-G103 CE12 subset 2: cross-verification of SONY’s parallel de-blocking filtering proposals JCTVC-G255 and JCTVC-G256 [M. Zhou, V. Sze (TI)]

JCTVC-G622 CE12: Cross-verification of parallel deblocking from Sony F14 tool 2 [Andrey Norkin (Ericsson)] [late] [miss]

Subtest 3

JCTVC-G228 CE12.3.2: Reducing pixel line buffers by modifying DF for horizontal LCU edges [C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G253 CE12 Subtest3: Cross-verification of MediaTek's proposal JCTVC-G228 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-G229 CE12.3.3: Reducing motion data line buffers [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G292 CE12 Subtest3: Cross Check of Mediatek’s Motion Data Line Buffer Reduction Proposal JCTVC-G229 [G. Van der Auwera (Qualcomm)]

JCTVC-G257 CE12 Subtest3: Deblocking vertical tap reduction for line buffer based on JCTVC-F215 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-G486 CE12.3.1: Crosscheck for Sony's deblocking vertical tap reduction for line buffer in JCTVC-G257 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late]

JCTVC-G180 CE12: Cross-check report of subtest 3 on Deblock filter line memory reduction [S. Park, B. Jeon (LGE)]

Subtest 4

JCTVC-G087 CE12 subset 4.10: Test results on unification of luma and chroma filtering [M. Zhou, O. Sezer, V. Sze (TI)]

JCTVC-G249 CE12 Subtest1: Cross-verification of TI's proposal JCTVC-G087 [M. Ikeda, T. Suzuki (Sony)]

Subtest 5

JCTVC-G088 CE12 subset 5.6: Test results and architectural study on de-blocking filter without parallel on/off filter decision [M. Zhou, O. Sezer, V. Sze (TI)]

JCTVC-G588 CE12 Subtest 2: Cross-check results of the deblocking proposal of Texas Instruments (JCTVC-G088) [Matthias Narroschke, Semih Esenlik (Panasonic)]

JCTVC-G746 CE12 subset 5.6: Cross-check report on parallel deblocking decision (JCTVC-G088) [I.-K. Kim, J. Chen (Samsung)]

JCTVC-G574 CE12 Subtest 5: Deblocking filter using adaptive weighting factors [Matthias Narroschke, Ann-Kathrin Seifert (Panasonic)]

JCTVC-G237 CE12 Subtest5: Cross-verification of Panasonic’s proposal relating to JCTVC-F405 [S. Lu, M. Ikeda, T. Suzuki (Sony)]

Subtest 6(?)

JCTVC-G174 CE12: Deblocking filter parameter adjustment in slice level [T. Yamakage, S. Asaka, T. Chujoh (Toshiba), M. Karczewicz, I.S. Chong (Qualcomm)]

JCTVC-G465 CE12: crosscheck of deblocking filter parameter adjustment in slice level [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G466 CE12: results on signaling of boundary filtering strength [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G388 CE12: Cross-check of Mitsubishi's deblocking filter JCTVC-F175/Gxxx [T. Yamakage, S. Asaka (Toshiba)]

Subtest 7

JCTVC-G086 CE12 subset 7.4: Test results on decreasing worst case complexity of deblocking filter [M. Zhou, O. Sezer, V. Sze (TI)]

JCTVC-G251 CE12 Subtest4: Cross-verification of TI's proposal JCTVC-G086 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-G623 CE12 : Cross-verification of deblocking simplification from TI F484 [Andrey Norkin (Ericsson)] [late] [miss]

### Discussion and Conclusions

## CE13: Motion data parsing robustness and throughput

### Summary

CE13 DOC

### Contributions

JCTVC-G091 CE13: Test results on maxNumMergeCand signaling and simplification of merge MVP list pruning process [M. Zhou (TI)]

JCTVC-G231 CE13: Results of section 3.1 tests 1, 3d, and 3e on replacing redundant MVPs and its combination with adaptive MVP list size [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G236 CE13: Cross-check of Mediatek results section 3.1 [G. Laroche, P. Onno (Canon)] [late]

JCTVC-G686 CE13: Crosscheck report of JCTVC-G231 Test 1 [Y. Zheng, X. Wang (Qualcomm)]

JCTVC-G845 CE13 Section 3.1: Cross-verification of MediaTek’s test 3d (JCTVC-G231) [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G232 CE13: Results of section 3.1 tests 2d and 2e on adaptive MVP list size [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G240 CE13: Experiment regarding section 3.5 [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-G424 CE13: Cross-check report of JCTVC-G240 (section 3.5 test 1 and test 4) [S.-C. Lim, J. Lee, H. Y. Kim (ETRI)]

JCTVC-G489 CE13: Crosscheck for Canon's section 3.5 tests 5 and 6 on additional MVP candidates in JCTVC-G240 [J.-L. Lin, Y.-W. Huang (MediaTek)]

JCTVC-G842 CE13 Section 3.5: Cross-verification of Canon’s test 3 (JCTVC-G240) [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G776 CE13: Merge candidates list construction [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G487 CE13: Crosscheck for Samsung's section 3.4 test 5' on adaptive list size and additional MVP candidates in JCTVC-G776 [J.-L. Lin, Y.-W. Huang (MediaTek)]

JCTVC-G191 CE13: Cross-verification result of JCTVC-F402 Test3 [K. Kazui (Fujitsu)]

JCTVC-G275 CE13: cross-check of experiments T1 (TI - JCTVC-Gxxx), S1 (Samsung – JCTVC-Gxxx) and C2 (Canon - JCTVC-Gxxx) [J. Jung (Orange Labs)] [late]

JCTVC-G514 CE13: Cross-Check Result of Test3b&4b [K. Sato (Sony)]

JCTVC-G539 CE13: Cross-check report of subset 3.3 (test2a, 3a, 4a) by Panasonic [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G540 CE13: Cross-check report of subset 3.2 series by Panasonic [T. Sugio, T. Nishi(Panasonic)]

### Discussion and Conclusions

# Non-CE Technical Contributions

## Clarification and Bug Fix Issues

JCTVC-G116 On PCM memory usage reduction in HM software [K. Chono, H. Aoki (NEC)]

## HM settings and common test conditions

JCTVC-G111 Common test conditions to specify 8-bit internal bit depth for all 8-bit source material [T. Hellman, Y. Yu, W. Wan (Broadcom)]

JCTVC-G136 Suggestion on picture quality hierarchy for Low Delay configurations [S. Liu, X. Zhang, S. Lei (MediaTek)]

JCTVC-G634 Cross check of MediaTek's G136: Suggestion on picture quality hierarchy for Low Delay configurations [E. S. Ryu, Y.Ye (InterDigital)] [late]

JCTVC-G150 Proposed Error Pattern Files for JCT-VC [S. Wenger (Vidyo)]

JCTVC-G399 Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

JCTVC-G678 A tool for rate-constrained performance test [S. Campbell, J. Wang, X. Yu (RIM)]

JCTVC-G855 Performance evaluation of full search mode decision for Intra of HM4.0 [C. Lai, Y. Lin, L. Liu, J. Zheng (HiSilicon)] [late]

JCTVC-G910 Non-CE6: Cross-check of HiSilicon’s performance evaluation of full search mode decision for intra of HM4.0 (JCTVC-G855) by Panasonic [S. M. T. Naing, V. Wahadaniah, C. S. Lim (Panasonic)] [late]

## Source video test material

JCTVC-G732 Study on test materials in common test condition [T. Suzuki (Sony)] [late]

## Functionalities

### Scalable coding

JCTVC-G078 Information for HEVC scalability extension [J. Boyce, D. Hong, W. Jang, A. Abbas (Vidyo)]

JCTVC-G248 Low Complexity Scalable Extension of HEVC intra pictures [S. Lasserre, F. Le Léannec, E. Nassor (Canon)]

JCTVC-G948 Draft Requirements and Discussion on the scalable enhancement of HEVC [Ajay Luthra] [late] [miss]

JCTVC-G949 Draft requirements for the scalable enhancement of HEVC [Ajay Luthra] [late] [miss]

JCTVC-G950 Draft use cases for the scalable enhancement of HEVC [Ajay Luthra] [late] [miss]

JCTVC-G951 Draft Call for Proposals on the Scalable Video Coding Extensions of HEVC [A. Luthra (Motorola)] [late] [miss]

### Stereo/Multi-view

JCTVC-G582 Multiview HEVC – experimental results [Marek Domanski, Tomasz Grajek, Damian Karwowski, Krzysztof Klimaszewski, Jacek Konieczny, Maciej Kurz, Adam Luczak, Robert Ratajczak, Jakub Siast, Olgierd Stankiewicz, Jakub Stankowski, Krzysztof Wegner]

### Interlace

JCTVC-G170 On issues for interlaced format support in HEVC standard [K. Chono, H. Aoki (NEC)]

JCTVC-G196 Modification of derivation process of motion vector information for interlace format [J. Koyama, A. Yamori, K. Kazui, S. Shimada, A. Nakagawa (Fujitsu)]

JCTVC-G296 Picture-adaptive Field/Frame Coding: support for legacy video [O. Bar-Nir (Harmonic)]

JCTVC-G450 High level syntax to support interlace format [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G667 HEVC field coded sequences vs. deinterlaced progressive coding [Chad Fogg] [late] [miss]

JCTVC-G877 Interlaced and 4:2:2 color format support in HEVC standard [Jerome Vieron] [late]

JCTVC-G912 On interlaced format [K. Chono, H. Aoki (NEC)] [late] [miss]

## Loop Filtering

### Deblocking filter

JCTVC-G089 Non-CE12: Enable low-complexity and fully parallelized de-blocking filter with minor changes [M. Zhou, O. Sezer, V. Sze (TI)]

JCTVC-G624 Non-CE12: Cross-verification of parallel deblocking proposal JCTVC-G089 from TI Andrey Norkin [late] [miss]

JCTVC-G090 Non-CE12: Testing results on using HM3.0 delta calculation for luma weak filter [M. Zhou, O.Sezer, V. Sze (TI)]

JCTVC-G162 Non-CE12: cross-verification of JCTVC-G090 on testing results on using HM3.0 delta calculation for luma weak filter by TI [Y. Jeon, S. Park, B. Jeon (LG)]

JCTVC-G138 Deblocking of IPCM Blocks Containing Reconstructed Samples [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G883 Cross-check report of Qualcomm’s Deblocking of IPCM Blocks Containing Reconstructed Samples (JCTVC-G138) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-G793 Non-CE12: Cross check of deblocking of IPCM blocks containing reconstructed samples (JCTVC-G138) [S. M. T. Naing, V. Wahadaniah, C. S. Lim (Panasonic)] [late]

JCTVC-G171 Parallel deblocking filter [J. Yu, S. Yang, J. Byun, Y. Kim, J. Kim (Yonsei Univ.)]

JCTVC-G175 BS decision tree simplification [S. Park, N. Park, B. Jeon (LGE), X. Guo, J. An, C. Hsu, Y. Huang, S. Lei (MediaTek)]

JCTVC-G176 Simplified BS calculation process [S. Park, B. Jeon (LGE)]

JCTVC-G675 Non-CE12: Crosscheck for LGE's Simplified Bs Calculation in JCTVC-G176 [J. An, X. Guo (MediaTek)]

JCTVC-G230 Non-CE12.3: Reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU edges [C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek), M. Ikeda, T. Suzuki (Sony), S. Park, B. Jeon (LG)]

JCTVC-G844 Cross-verification of MediaTek and Sony's reducing pixel line buffers by modifying DF to R3W2 for horizontal LCU edges (JCTVC-G230) [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G290 Deblocking Filter Simplifications [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G929 Cross check of Qualcomm‘s deblocking filter simplifications (JCTVC-G290 [A. Kotra, M. Narroschke, T. Wedi (Panasonic)] [late]

JCTVC-G291 Transform Dependent Deblocking Filter Parameter Adjustment in Slice Level [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G616 On parallel deblocking [Andrey Norkin (Ericsson)]

JCTVC-G619 Non-CE12: deblocking parameters signalling in slice header [A. Norkin, R. Sjöberg, K. Andersson, J. Enhorn (Ericsson)]

JCTVC-G620 Clean-up of deblocking filter description [Andrey Norkin (Ericsson)] [late]

JCTVC-G638 Deblocking boundary strength and filtering process simplifications [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G639 Deblocking simplification and rounding optimization [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G385 Cross check of Panasonic's JCTVC-G638 and JCTVC-G639 on Deblocking [G. Van der Auwera (Qualcomm)] [late]

JCTVC-G640 Deblocking bug fix for CU-Varying QP’s and IPCM blocks [A. Kotra, M. Narroschke, T. Wedi (Panasonic)]

### Adaptive loop filter

JCTVC-G146 Non-CE8: Information on ALF coefficient re-designing process [J. Lim, S. Park, B. Jeon (LG)]

JCTVC-G339 Crosscheck of LG's ALF (JCTVC-G146) [T. Ikai (Sharp)] [late]

JCTVC-G919 Non-CE8: Cross-check of JCTVC-G146 [P. Lai, F. C. A. Fernandes (Samsung)] [late]

JCTVC-G214 Non-CE8: Constrained ALF coefficients [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G853 Crosscheck of JCTVC-G214 on constrained ALF coefficients from MediaTek [S. Park, B. Jeon (LG)] [late]

JCTVC-G215 Non-CE8: Limited number of filters per picture for ALF [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G147 Non-CE8: Cross-check of MediaTek’s limited number of ALF filter by LG (JCTVC-G215) [J. Lim, S. Park, B. Jeon (LG)]

JCTVC-G216 Non-CE8: Removing the 15th merge flag for BA mode in ALF [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G218 Non-CE8: One-stage non-deblocking loop filtering [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh (Toshiba)]

JCTVC-G846 Cross-Check for JCTVC-G218 [C. Kim, Y. Park(Samsung)] [late]

JCTVC-G351 Modification of ALF classification [E. Maani, M. Ikeda (Sony)]

JCTVC-G803 Non-CE8: Cross-check for Simplification of ALF Classification [T. Yamamoto, T. Ikai (Sharp)] [late]

JCTVC-G380 Non-CE8: Coding tree level signaling of alf_cu_flag [T. Nishi, K. Uchibayashi (Panasonic)]

JCTVC-G938 Cross-check of JCTVC-G380, "Non-CE8: Coding tree level signaling of alf_cu_flag" [A. Fuldseth (Cisco)] [late] [miss]

JCTVC-G384 Varying QP Deblocking [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G445 Removing DC component of ALF filter coefficients [A. Minezawa, K. Sugimoto, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G918 Non-CE8: Cross-check of JCTVC-G445 on removing DC component of ALF filter coefficients [P. Lai, F. C. A. Fernandes (Samsung)] [late]

JCTVC-G615 Non-CE8: Cross check of Mitsubishi's removing DC component of ALF filter coefficients [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G446 Reduction of the number of pixels used in Adaptive Loop Filter [K. Miyazawa, K. Sugimoto, A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

JCTVC-G920 Cross-check of Mitsubishi's reduction of the number of pixels used in Adaptive Loop Filter JCTVC-G446 [T. Yamakage, T. Itoh (Toshiba)] [late]

JCTVC-G463 Block-based filter adaptation with intra prediction mode and CU depth information [S. Wang, S. Ma, J. Jia (LG)]

JCTVC-G835 Cross verification of LG’s Block-based filter adaptation with intra prediction mode and CU depth information (JCTVC-G463) [Y. Chiu, L. Xu (Intel)] [late]

JCTVC-G499 Improved ALF with low latency and reduced complexity [A. Fuldseth, G. Bjøntegaard (Cisco)]

JCTVC-G651 Crosscheck of JCTVC-G499 - Improved ALF with low latency and reduced complexity [M. Budagavi (TI)] [late] [miss]

JCTVC-G610 Improvements to ALF [J. Zhao, A. Segall (Sharp)]

JCTVC-G952 Non-CE8: Cross-check of JCTVC-G610 on improvements to ALF [??] [late] [miss]

JCTVC-G656 Non-CE8 Subtest a: Harmonization of CE8a Tool 2 Shape-dependent BA (SDBA) and Tool 3 Block-based filter adaptation with up to 8 filters (HV8) [P. Lai, F. C. A. Fernandes, I.-K. Kim (Samsung)]

JCTVC-G338 Non-CE8.a: Crosscheck of Non-CE8 Subtest a: Harmonization of CE8a Tool 2(SDBA) and Tool 3(HV8) (JCTVC-G656) [T. Ikai (Sharp)]

JCTVC-G666 1D- DCT based frequency domain adaptive loop filter (FD-ALF) [Jeongyoen Lim, Ju Ock Lee, Hae-Kwang Kim, Joo-Hee Moon]

JCTVC-G691 Non-CE8: Combination of CE8.a.1 and CE8.a.2 [T. Yamakage, T. Watanabe, T. Chujoh (Toshiba), C. Y. Chen, C. M. Fu, C. Y. Tsai, Y. W. Huang, S. Lei (MediaTek), M. Karczewicz, I. S. Chong (Qualcomm), T. Ikai, A. Segall, T. Yamamoto (Sharp)]

JCTVC-G653 Non-CE8: Cross-check of JCTVC-G691 Combination of CE8.a Tool 1 and Tool 2 [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-G774 Non-CE8: Modified ALF DC coefficients [T. Yamakage, T. Watanabe, T. Chujoh (Toshiba), C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), M. Karczewicz, I. S. Chong (Qualcomm)]

JCTVC-G892 Non-CE8: Cross-check of JCTVC-G774 on modified ALF DC coefficients P. Lai, F. C. A. Fernandes (Samsung) [late]

JCTVC-G813 ALF with single filter type [M. Budagavi (TI)]

JCTVC-G923 ALF decoding time reduction by adopting a simple SIMD code (Informative) [T. Yamakage, T. Itoh (Toshiba)] [late] [miss]

JCTVC-G954 Non-CE8: Cross check of ALF decoding time reduction by adopting a simple SIMD code (JCTVC-G923) [I. S. Chong, M. Karczewicz] [late] [miss]

### Sample adaptive offset

JCTVC-G222 Non-CE8: Offset coding in SAO [C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G818 Cross-check for Offset coding in SAO from MediaTek and Qualcomm by Samsung [E. Alshina, J.H. Park] [late]

JCTVC-G246 On additional SAO Band Offset classifications [G. Laroche(Canon), T. Poirier, P. Onno (Canon)]

JCTVC-G828 Non-CE8: Crosscheck for Canon's additional SAO band offset classifications in JCTVC-G246 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-G309 Modifications of offset classifications in SAO [Y. Yasugi, T. Ikai (Sharp)]

JCTVC-G483 Non-CE8: Crosscheck for SHARP's modifications of offset classifications in SAO in JCTVC-G309 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-G490 Modified SAO edge offsets [K. Andersson, P. Wennersten, R. Sjöberg (Ericsson)]

JCTVC-G827 Non-CE8: Crosscheck for Ericsson's modified SAO edge offsets in JCTVC-G490 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-G680 Non-CE8: Method of visual coding artifact removal for SAO [W.-S. Kim, D.-K. Kwon (TI)]

JCTVC-G808 Cross-check for TI's proposal (JCTVC-G680) [T.Matsunobu, T.Sugio, T.Nishi (Panasonic)] [late]

JCTVC-G682 Non-CE8: Reduced number of band offsets in SAO [W.-S. Kim, D.-K. Kwon (TI)]

JCTVC-G748 Non-CE8: Crosscheck of TI's Reduced number of band offsets in SAO [T. Ikai (Sharp)] [late]

JCTVC-G831 Non-CE8: Sample Adaptive Offset with LCU-based Syntax [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), M. Karczewicz, I. S. Chong (Qualcomm)]

JCTVC-G935 Cross-check for LCU based SAO from MediaTek and Qualcomm (JCTVC-G831) by Samsung [E.Alshina, J.H.Park (Samsung)] [late]

JCTVC-G915 Coding and selection of SAO parameters [D. Baylon, K. Minoo (Motorola Mobility)] [late]

### Combined approaches

JCTVC-G122 VLC for high level syntax (ALF and SAO parameters) [V. Sze (TI)]

JCTVC-G606 Crosscheck - VLC for high level syntax (ALF and SAO parameters) (G122) [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-G211 Non-CE8.c.6: Multi-source SAO and ALF virtual boundary processing with cross9x9 [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), S. Esenlik, M. Narroschke, T. Wedi (Panasonic), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G477 Non-CE8.c.6: Cross-check of MediaTek/Panasonic/Qualcomm's multi-source SAO and ALF virtual boundary processing with cross9x9 JCTVC-G211 [T. Yamakage, T. Watanabe (Toshiba)]

JCTVC-G212 Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9 [C.-Y. Chen, C.-M. Fu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), S. Esenlik, M. Narroschke, T. Wedi (Panasonic), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G337 Non-CE8.c.7: Crosscheck of Single-source SAO and ALF virtual boundary processing (JCTVC-G212) [T. Ikai (Sharp)]

JCTVC-G654 Non-CE8: Cross-check of CE8.c Tool 7-1 (JCTVC-G212) on single-source SAO and ALF virtual boundary processing with cross9x9 [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-G220 Non-CE8: Pure VLC for SAO and ALF [C.-Y. Tsai, C.-M. Fu, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G617 Non-CE8: Cross check of MediaTek's pure VLC for SAO and ALF [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G566 Syntax refinements for SAO and ALF [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G608 Unifed Deblocking and SAO [A. Segall, J. Zhao (Sharp)]

JCTVC-G684 Subjective Tests on ALF and SAO Using HM-4.0 [W.-S. Kim, O. G. Sezer, M. Budagavi (TI)]

## Block structures and partitioning

### NSQT (ref CE2)

JCTVC-G202 Non-CE2: Modified NSQT coefficient scan for CAVLC [C.-W. Hsu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G747 Non-CE2: Crosscheck of MediaTek's proposal on NSQT coefficient scan for CAVLC (JCTVC-G202) [X. Zheng (HiSilicon)] [late] [miss]

JCTVC-G519 Non-CE2: Harmonization of implicit TU, AMP and NSQT [X. Zheng (HiSilicon), Y. Yuan, Y. He (Tsinghua)]

JCTVC-G852 Cross-check report on Harmonization of implicit TU, AMP and NSQT (JCTVC-G519) [I.-K. Kim (Samsung)] [late]

JCTVC-G521 Non-CE2: Non-square Hadamard transform for motion estimation and merge estimation [X. Zheng, L. Liu (HiSilicon), Y. Yuan, Y. He (Tsinghua)]

JCTVC-G599 Non-CE2: Cross-Check of Non-square Hadamard transform for motion estimation and merge estimation [S. Oudin, B. Bross(Fraunhofer HHI)] [late]

JCTVC-G750 Non-CE2: Harmonization of HE residual coding and NSQT [J. Sole, X. Wang, M. Karczewicz (Qualcomm), J. Kim, B. Jeon (LGE)]

JCTVC-G312 Non-Square Partition Mode Grouping for CAVLC [T. Yamamoto (Sharp)]

JCTVC-G743 Cross-check report on Non-Square Partition Mode Grouping for CAVLC (JCTVC-G312) [I.-K. Kim (Samsung)]

### Partition mode/size coding

JCTVC-G151 Prediction and partition mode binarization for Low Delay P [X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-G655 Cross-check of JCTVC-G151 on prediction and partition mode binarization for Low Delay P [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-G156 CU Depth Pruning for Fast Coding Tree Block Decision [H. L. Tan, C. Yeo, Y. H. Tan (I2R)]

JCTVC-G283 Residue Quad Tree Depth for Chroma in Intra Coding [X. Zhao, X. Guo, X. Li, S. Lei (MediaTek), S. Ma, W. Gao (PKU)]

JCTVC-G798 Cross verification of MediaTek’s proposed residue quad-tree depth for intra chroma coding (JCTVC-G283) [Y. Chiu, L. Xu (Intel)] [late]

JCTVC-G442 Improvement to chroma TU specification [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi), A. Ichigaya, S. Sakaida (NHK)]

JCTVC-G378 Cross Check Report for Improvement to chroma TU specification (G442) [Y. Shibahara, T. Nishi (Panasonic)]

JCTVC-G605 Cross-check of Improvement to chroma TU specification (JCTVC-G442) [P. Helle, B. Bross (Fraunhofer HHI)] [late] [miss]

JCTVC-G416 CU-based Merge Candidate List Construction [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

JCTVC-G899 Cross-check of CU-based Merge Candidate List Construction [S. Oudin, B. Bross(Fraunhofer HHI)] [late]

JCTVC-G417 Improved coding of CU-level information in CAVLC S. Y. Yi (KAU), H. Choi (Hanbat Univ.), S.-C. Lim (ETRI), J.-G. Kim (KAU), [H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

JCTVC-G881 Cross-check for Improved coding of CU-level information in CAVLC (JCTVC-G417) [T. Yamamoto (Sharp)] [late]

JCTVC-G709 On partition size information coding using CABAC [T. Yamamoto, K. Misra, A. Segall (Sharp)]

## Motion compensation operation and interpolation filters

### Interpolation filters and MV precision (ref CE3)

JCTVC-G062 non-CE3: 7-tap quarter-pel luma interpolation filter with accurate phase shift [Hongbo Zhu]

JCTVC-G131 PU-size dependent motion compensation filtering order [M. Budagavi, R. R. Srinivasan (TI)]

JCTVC-G884 Verification of JCTVC-G131 on interpolation filter order [F. Bossen] [late]

JCTVC-G259 Modifications of LCU-based adaptive interpolation filter [S. Matsuo, S. Takamura, H. Jozawa (NTT)]

JCTVC-G347 Cross-check report of NTT's interpolation filter approach (JCTVC-G259) [T. Yoshino, S. Naito (KDDI)]

JCTVC-G392 Non-CE3: Report on a restriction for small block [K. Kondo, T. Suzuki (Sony)]

JCTVC-G429 Non-CE3: Cross-check report of Sony proposal on restriction for small block (JCTVC-G392) [T.Chujoh (Toshiba)]

JCTVC-G600 Non-CE3: Adaptive Motion Vector Resolution based on the PU Size [J. Jung, J. Heo, S. Yea (LG)]

JCTVC-G698 On Chroma interpolation filter [J. Lou, K. Minoo, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-G699 Motion vector scaling for non-uniform interpolation offset [J. Lou, K. Minoo, L. Wang (Motorola Mobility)]

JCTVC-G825 Cross-check for Motorola Mobility proposals on MV scaling by Samsung [E. Alshina, J.H. Park] [late]

JCTVC-G736 On luma/chroma interpolation precision [M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G693 Cross-check of Qualcomm's proposal on interpolation precision (G736) [Y. H. Tan, C. Yeo (I2R)] [late] [miss]

JCTVC-G770 Non-CE3: A restriction of motion vector for small block size [T.Chujoh, T.Yamakage (Toshiba)]

JCTVC-G898 Non-CE3: Cross check for Toshiba's proposal (JCTVC-G770)	[K. Kondo, T. Suzuki(Sony)] [late]

JCTVC-G931 Non CE3: Cross-check for memory band-width reduction from Toshiba (G-770) by Samsung [E. Alshina, J.H. Park (Samsung)] [late]

JCTVC-G806 Non CE3: On the phase offset selection for motion compensation interpolation filters [Koohyar Minoo, David Baylon, Jian Lou]

JCTVC-G807 Non CE3: On design of interpolation filters for Uni and Bi Predictive Motion Compensation [Koohyar Minoo, David Baylon, Jian Lou] [late]

JCTVC-G824 Non CE3: Cross-check for Motorola Mobility proposals on Chroma interpolation filter by Samsung [E. Alshina, J.H. Park] [late]

### Prediction methods

JCTVC-G065 Improved Weighted Prediction [Y. Ye, E. S. Ryu (InterDigital)]

JCTVC-G307 Bi-prediction restriction in small PU [T. Ikai (Sharp)]

JCTVC-G289 Cross-verification of Sharp’s proposal JCTVC-G307 on Bi-prediction restriction in small PUs [M. Zhou (TI)]

JCTVC-G415 MC Complexity Reduction for Bi-prediction [H. Y. Kim (ETRI), K. Y. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, J. S. Choi (ETRI)]

JCTVC-G874 Cross-check MC Complexity Reduction for Bi-prediction (JCTVC-G415) from ETRI [Patrice Onno (Canon)] [late]

JCTVC-G438 On complexity reduction of bi-prediction for identical motion [A. Tanizawa, T. Shiodera, T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-G441 Redundancy removal of explicit weighted prediction syntax [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-G525 Cross-check of JCTVC-G441: Redundancy removal of explicit weighted prediction syntax [P.Bordes, P.Salmon (Technicolor)] [late] [miss]

JCTVC-G524 On weighted prediction [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)] [late]

JCTVC-G549 Syntax rearrangement for list combination [Y. Park, S. Jeong, C. Kim (Samsung)]

JCTVC-G858 Crosscheck of JCTVC-G549 proposed by Samsung [X. Zheng] [late] [miss]

## Motion and mode coding

### AMVP/skip/merge (related to CE9)

JCTVC-G082 non-CE9: Modified H positions for memory bandwidth reduction in TMVP derivation [M. Zhou (TI)]

JCTVC-G274 Non-CE9: cross-check of contribution JCTVC-G082 (TI) on modified H TMVP position [J. Jung (Orange Labs)]

JCTVC-G134 Motion Vector Predictor Candidate Clipping Removal [M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G344 Cross-verification report for motion vector predictor candidate clipping removal (JCTVC-G134) [H. Aoki, K. Chono (NEC)]

JCTVC-G163 Non-CE9: simplification of merge/skip TMVP ref_idx derivation [Y. Jeon, S. Park, J. Park, B. Jeon (LG)]

JCTVC-G099 Non-CE9: cross-verification of LGE’s proposal JCTVC-G163 on simplification of merge/skip ref_idx derivation [M. Zhou (TI)]

JCTVC-G164 Non-CE9: improvement on parallelized merge/skip mode [Y. Jeon, S. Park, B. Jeon (LG)]

JCTVC-G100 Non-CE9: cross-verification of LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode [M. Zhou (TI)]

JCTVC-G922 Non-CE9: cross-check of LGE’s proposal JCTVC-G164 on improvement on parallel merge/skip mode [Y. Park, H. Yang (Samsung)] [late] [miss]

JCTVC-G165 Non-CE9/Non-CE13: new MVP positions for merge/skip modes and its combination with replacing redundant MVPs [Y. Jeon, S. Park, B. Jeon (LG), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G860 Non-CE9/Non-CE13: Cross-check of JCTVC-G165 on new MVP positions for merge/skip modes [V. Seregin (Qualcomm)] [late]

JCTVC-G104 Non-CE9: cross-verification of LGE and MediaTek’s proposal JCTVC-G165 on new MVP positions for merge/skip modes and its combination with replacing redundant MVPs [M. Zhou (TI)]

JCTVC-G181 Non-CE9: Merging candidate reordering [H. Takehara, S. Fukushima (JVC Kenwood)]

JCTVC-G468 Non-CE9: cross-verification of JVC KENWOOD's proposal on Merge candidate reordering (JCTVC-G181) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-G182 Non-CE9: AMVP syntax for bi-prediction [H. Takehara, S. Fukushima (JVC Kenwood)]

JCTVC-G431 Non-CE9: Cross-verification of JVC Kenwood proposal on AMVP syntax for bi-prediction (JCTVC-G182) [T.Chujoh (Toshiba)]

JCTVC-G195 Non-CE9/13: Averaged merge candidate [S. Shimada, K. Kazui, J. Koyama, A. Nakagawa (Fujitsu)]

JCTVC-G467 Non-CE9/13: cross-verification of Fujitsu's proposal on Merge (JCTVC-G195) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-G217 Non-CE9: Derivation process of reference indices for temporal merge candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G742 Cross-check report on derivation process of reference indices for temporal merge candidates (JCTVC-G217) [I.-K. Kim (Samsung)]

JCTVC-G219 Non-CE9: Construction of MVP list without using scaling [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G105 Non-CE9: cross-verification of JVC’s proposal JCTVC-G219 on construction of MVP list without using scaling operation [M. Zhou (TI)]

JCTVC-G223 Non-CE9: Division-free MV scaling [T.-D. Chuang, Y.-W. Chen, J.-L. Lin, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G523 Non-CE9: Cross-check of division-free MV scaling [P. Onno (Canon)] [late] [miss]

JCTVC-G225 Non-CE9: Simplified AMVP derivation for Inter mode [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G341 Crosscheck of MediaTek's AMVP simplification (JCTVC-G225) [J. Park, S. Park, B. Jeon (LG)]

JCTVC-G241 Non-CE9: On parallel derivation of the temporal predictor for Merge/Skip modes [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-G098 Non-CE9: cross-verification of Canon’s proposal JCTVC-G241 on parallel derivation of the temporal predictor for Merge/Skip modes [M. Zhou (TI)]

JCTVC-G305 Non-CE9: Bi-prediction for low delay coding [Y. Suzuki, A. Fujibayashi (NTT Docomo)]

JCTVC-G603 Non-CE9: Cross-check of Bi-prediction for low delay coding (JCTVC-G305) [P. Helle, B. Bross (Fraunhofer HHI)] [late] [miss]

JCTVC-G343 Non-CE9: Improvement in temporal candidate of merge mode and AMVP [N. Zhang, X. Fan, S. Ma, D. Zhao (Harbin Inst. Tech.)]

JCTVC-G406 Non-CE9: Improvement in temporal candidate of merge mode and AMVP (JCTVC-G343) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G387 Non-CE9 Parallel Merge/skip Mode for HEVC [X. Wen, O. Au, W. Dai, C. Pang, J. Dai, F. Zou, X. Zhan (HKUST)]

JCTVC-G407 Non-CE9: Cross-check of parallelized merge/skip mode for HEVC (JCTVC-G387) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G396 Non-CE9: swapping of merge candidate [C. Kim, Y. Jeon, B. Jeon (LG)]

JCTVC-G484 Non-CE9: Crosscheck for LG's swapping of merge candidate in JCTVC-G396 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-G927 Non-CE9: Cross-check of JCTVC-G396 on reordering of merge candidate [V. Seregin (Qualcomm)] [late] [miss]

JCTVC-G944 Cross-check report on G396 [I.-K. Kim] [late] [miss]

JCTVC-G397 Non-CE9/Non-CE13: Simplification of adding new merge candidates [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

JCTVC-G426 Cross-check of Microsoft’s proposal on simplification of adding new merge candidates (JCTVC-G397) by Huawei [H. Yang, H. Yu (Huawei)]

JCTVC-G516 On Spatial MV Prediction [K. Sato (Sony)] [late] [miss]

JCTVC-G799 Non-CE9: Crosscheck for Sony's proposals (JCTVC-G516) by JVC KENWOOD [Hiroya Nakamura (JVC Kenwood)] [late] [miss]

JCTVC-G541 Non-CE9: Simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G102 Non-CE9: cross-verification of Panasonic’s proposal JCTVC-G541 simplified scaling calculation method for temporal/spatial MVP of AMVP/Merge [M. Zhou (TI)]

JCTVC-G542 Non-CE9/Non-CE13: Simplification on AMVP/Merge [T. Sugio, T. Nishi(Panasonic)]

JCTVC-G891 Non-CE9: cross-verification of Panasonic's simplification on AMVP/Merge (JCTVC-G542) [M. Zhou (TI)] [late]

JCTVC-G878 Non-CE9: Crosscheck for Panasonic's proposals (JCTVC-G542) by JVC-KENWOOD [Hiroya Nakamura, Shigeru Fukushima (JVC Kenwood)] [late]

JCTVC-G551 Restriction on motion vector scaling for Merge and AMVP [I.-K. Kim, Y. Park, N. Shlyakhov, J. H. Park (Samsung)]

JCTVC-G859 Crosscheck of JCTVC-G551 proposed by Samsung [X. Zheng] [late] [miss]

JCTVC-G552 Simplification of temporal motion vector (TMVP) candidate derivation for Merge and AMVP [I.-K. Kim, N. Shlyakhov, J. H. Park(Samsung)]

JCTVC-G126 Cross-check of Panasonic’s proposal on context reduction for coefficient levels (G552) [V. Sze (TI)] [late] [miss]

JCTVC-G227 Non-CE9: Cross-check for Samsung's proposals (JCTVC-G552) by JVC KENWOOD [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G515 Cross-check of Samsung's TMVP simplification by Sony [K. Sato (Sony)] [late]

JCTVC-G592 Non-CE9: Removal of reference index derivation for TMVP in merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G681 Non-CE9: Simplified Merge candidate derivation [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G830 Non-CE9: Crosscheck for Qualcomm's simplified merge candidate derivation in JCTVC-G681 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-G690 Non-CE9: Some possible motion vector coding related simplifications [Y. H. Tan, C. Yeo, Z. Li (I2R)]

JCTVC-G752 Non-CE9: Crosscheck of I2R's proposal on motion vector coding related simplifications (JCTVC-G690) [X. Zheng (HiSilicon)] [late] [miss]

JCTVC-G733 cross check for I2R AMVP simplification [Yue Yu, Krit Panusopone, Limin Wang] [late] [miss]

JCTVC-G710 Non-CE9: The Parallel Friendly MVP Candidate Calculation for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G712 Non-CE9: The Simplification of MVP for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-G740 Cross-check of Motorola's proposals on motion vector coding (G710, G712) [Y. H. Tan, C. Yeo (I2R)] [late] [miss]

### MV and mode coding

JCTVC-G209 Modified method for coding mvd in the CABAC mode [S.-T. Hsiang, S. Lei (MediaTek)]

JCTVC-G705 CABAC simplification for explicit signaling mode of AMVP [K. Misra, A. Segall (Sharp)]

JCTVC-G785 Unified Pred_type coding in CABAC [Y. Piao, J. Min, J.H. Park (Samsung)]

JCTVC-G906 Cross check report of Samsung's Unified Pred_type coding in CABAC (JCTVC-785) [W. Zhang (ZTE)] [late]

JCTVC-G829 Context modeling of split flag for CABAC [W. -J. Chien, M. Karczewicz]

JCTVC-G849 Non-CE1: Crosscheck for Qualcomm's context modeling of split flag for CABAC in JCTVC-G829 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

### Reference picture list management (subsumed under HL syntax)

## High-level syntax and slice structure

### High-level syntax and systems usage of bitstreams

JCTVC-G110 Implication of parallelized bitstreams on single core decoder architectures [W. Wan, T. Hellman (Broadcom)]

JCTVC-G149 Options for High-Level Syntax for Multistandard Scalability [S. Wenger (Vidyo)]

JCTVC-G159 Random Access Detection and Notification [Hendry, S. Park, Y. Jeon, B. Jeon (LG)]

JCTVC-G607 High-Level Syntax for Bitstream Extraction [Rickard Sjöberg, Thomas Rusert (Ericsson)]

### Supplemental information

JCTVC-G079 SEI message for display orientation information [J. Boyce, D. Hong, S. Wenger (Vidyo)]

JCTVC-G092 AHG22: High-level signaling of lossless coding mode in HEVC [M. Zhou (TI)]

JCTVC-G583 Reducing output delay for "bumping" process [Jonatan Samuelsson, Rickard Sjöberg (Ericsson)]

### Parameter sets and slice header

JCTVC-G325 AHG15: Picture size signaling [Y. Chen, Y. -K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G330 AHG15: Syntax elements in adaptation parameter set [Y. Chen, Y. -K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G332 Multiple Adaptation Parameter Sets Referring [M. Li, P. Wu (ZTE)]

JCTVC-G334 AHG15: On sequence parameter set and picture parameter set [Y. -K. Wang, Y. Chen, Y. Zheng, W. -J. Chien (Qualcomm)]

### Tiles

JCTVC-G183 Test result of low delay tile [K. Kazui, S. Shimada, J. Koyama, A.Nakagawa (Fujitsu)]

JCTVC-G194 AHG4: Non-cross-tiles loop filtering for independent tiles [C.-Y. Tsai, C.-W. Hsu, C.-Y. Chen, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), A. Fuldseth (Cisco)]

JCTVC-G197 AHG4: Low latency CABAC initialization for dependent tiles [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G315 AHG4: Unification of picture partitioning schemes [M. Coban, Y. -K. Wang, M. Karczewicz, Y. Chen, I. S. Chong (Qualcomm)]

JCTVC-G317 AHG4: Dependency and loop filtering control over tile boundaries [Y. -K. Wang, Y. Chen, I. S. Chong, M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G318 AHG4: Tile groups [Y. -K. Wang, Y. Chen, M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-G453 New definition for Tile/slice boundary [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G454 Parallel processing of ALF and SAO for tiles [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G618 Line buffers problem in HEVC [Andrey Norkin, Rickard Sjöberg (Ericsson)]

JCTVC-G802 Cross-check of “AHG4: Non-cross-tiles loop filtering for independent tiles” (JCT-VC-G194) [M. Horowitz, S. Xu (eBrisk)]

### Wavefront parallel processing

JCTVC-G199 AHG4: Wavefront tile parallel processing [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G627 Cross-check of JCTVC-G199, "AHG4: Wavefront tile parallel processing" [A. Fuldseth (Cisco)] [late]

JCTVC-G722 Harmonization of entry points for tiles and wavefront processing [A. Segall, K. Misra (Sharp)] [late]

JCTVC-G815 Category-prefixed data batching for HEVC wavefront and PIPE/V2V/V2F coding [G. J. Sullivan (Microsoft)]

### NAL unit header

JCTVC-G331 AHG15: On NAL unit types and slice types [Y. -K. Wang, Y. Chen (Qualcomm)]

JCTVC-G336 AHG 17: Unified NAL unit header design for HEVC and its extensions [Y. Chen, Y. -K. Wang, M. Karczewicz (Qualcomm)]

### Random access

JCTVC-G158 Undiscardable Leading Picture for CRA [Hendry, S. Park, B. Jeon (LG)]

JCTVC-G159 Random Access Detection and Notification [Hendry, S. Park, Y. Jeon, B. Jeon (LG)]

JCTVC-G319 AHG15: Conforming bitstreams starting with CRA pictures [Y. Chen, Y. -K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G533 On syntax for clean random access (CRA) pictures [Y. Park, H. Yang, C. Kim (Samsung)]

JCTVC-G584 Temporal layer access pictures and CRA [Jonatan Samuelsson, Rickard Sjöberg (Ericsson)]

JCTVC-G834 Cross-check result of Samsung’s proposal JCTVC-G533 by LG Electronics [Hendry, S. Park, B. Jeon (LG)] [late]

### Decoded picture buffering and reference picture signaling

JCTVC-G157 Reference List Construction for Random Access Settings [Hendry, S. Park, B. Jeon (LG)]

JCTVC-G500 Cross-verification of LG's Reference List Construction for Random Access Settings (JCTVC-G157) by Panasonic [C. S. Lim, S. M. T. Naing, V. Wahadaniah (Panasonic)]

JCTVC-G166 AHG21: Explicit Reference Pictures Signaling with Output Latency Count Scheme [Hendry, S. Park, B. Jeon (LG)]

JCTVC-G832 Cross-check of "AHG21: Explicit Reference Pictures Signaling with Output Latency Count Scheme"(JCTVC-G166) [Y. Park, I.-K. Kim, C. Kim (Samsung)] [late]

JCTVC-G198 AHG21: Inter reference picture set prediction syntax and semantics. [T.K. Tan, C.S. Boon (NTT Docomo)]

JCTVC-G314 AHG21: On DPB management [Y. -K. Wang, Y. Chen (Qualcomm)]

JCTVC-G398 High-level Syntax: Marking process for non-TMVP pictures [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

JCTVC-G526 AHG21: Combined signaling for reference picture set [Y. Park, I.-K. Kim, C. Kim (Samsung)]

JCTVC-G546 On high-level syntax for maximum DPB size and frame latency [Y. Park, K. P. Choi, C. Kim (Samsung)]

JCTVC-G548 AHG21: Construction and modification of predefined reference picture sets and reference picture lists [V. Wahadaniah, C. S. Lim, S. M. T. Naing (Panasonic)]

JCTVC-G635 Coding with a unified reference picture list [M. Naccari (BBC), G. Van Wallendael (Ghent University), M. Mrak (BBC), D. Flynn (BBC)]

JCTVC-G637 AHG21: Long-term pictures and pruning of reference picture sets [Rickard Sjöberg, Jonatan Samuelsson (Ericsson)]

JCTVC-G643 AHG21: On reference picture list construction and reference picture marking [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

JCTVC-G668 POC type 1 [M. M. Hannuksela (Nokia)]

JCTVC-G713 AHG21: Long term picture referencing using wrapped POC [K. Misra, S. Deshpande, A. Segall (Sharp)]

JCTVC-G714 AHG21: On picture referencing [K. Misra, S. Deshpande, A. Segall (Sharp)]

JCTVC-G717 The Reference Picture Construction of Combined List for HEVC [Y. Yu, K. Panusopone, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-G779 Proposed constraint on reordering latency (for further consideration of JCTVC-F541) [G. J. Sullivan (Microsoft)]

JCTVC-G788 AHG21: Proposal on Decoded Picture Buffer Description Syntax Relating to JCTVC-F493 [G. J. Sullivan, Y. Wu, J. Xu (Microsoft), B. Li (USTC)]

### HRD issues

JCTVC-G188 Enhancement on operation of coded picture buffer [K. Kazui, S. Shimada, J. Koyama, A. Nakagawa (Fujitsu)]

## Quantization

### QP prediction / delta QP coding

JCTVC-G213 Non-CE4: Performance comparison between temporal QP prediction and temporal MV prediction [H. Aoki, K. Chono (NEC)]

JCTVC-G936 cross verification of NEC's proposal JCTVC-G213 on TMVP [Y. Lin, J. Zheng(HiSilicon)] [late] [miss]

JCTVC-G353 Non-CE4: QP prediction using spatial QP [H. Nakamura, M. Nishitani, S. Fukushima, T. Kumakura (JVC Kenwood)]

JCTVC-G772 Cross-check result of JCTVC-G353 [K Sato (Sony)] [late] [miss]

JCTVC-G357 Non-CE4: Improvement of Intra prediction based QP prediction [J. Xu, A. Tabatabai, K. Sato (Sony)]

JCTVC-G739 Non-CE4: Cross-verification report for improvement of intra prediction based QP prediction (JCTVC-G357) [H. Aoki, K. Chono (NEC)] [late]

JCTVC-G362 Non-CE4: Efficient binarization of delta_QP in CABAC with signaling of Max and Min QP [J. Xu, K. Kondo, K. Sato, A. Tabatabai (Sony)]

JCTVC-G850 Non CE4: Fine granularity QP offset [X. Li, X. Guo, S. Lei (MediaTek), X. Wang, R. Joshi, G. Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G451 Qp offset for intra block [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G509 Support of ChromaQPOffset in HEVC [S. Liu (MediaTek), K. Sato (Sony)]

JCTVC-G907 Crosscheck of JCTVC-G509 - Support of ChromaQPOffset in HEVC [E. S. Ryu (InterDigital)] [late] [miss]

JCTVC-G759 Cross-check report for ChromaQPOffset [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-G550 TU_size based dQP [C. Kim, Y. Park, K.P. Choi, J.H. Park(Samsung)]

JCTVC-G925 Cross-check of JCTVC-G550 on TU_size based dQP [V. Seregin (Qualcomm)] [late] [miss]

JCTVC-G893 Unified QP signaling X. Fang [late] [miss]

### Quantization matrices

JCTVC-G094 non-CE4: Carriage of large block size quantization matrices with up-sampling [M. Zhou, V. Sze (TI)]

JCTVC-G506 Non-CE4: Crosscheck report of carriage of large block size quantization matrices with up-sampling proposed by TI (JCTVC-G094) [M. Shima (Canon)]

JCTVC-G152 Method and syntax for quantization matrices representation [X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-G529 Non-CE4: Cross-Check for MediaTek quantization matrices representation (JCTVC-G152) [J. Zheng (HiSilicon)] [late] [miss]

JCTVC-G711 Non-CE4: Cross-verification of MediaTek's proposal JCTVC-G152 on method and syntax for quantization matrices representation [M. Zhou (TI)] [late]

JCTVC-G295 Non-CE4 Subtest3 : Extension of Adaptation Parameter Sets syntax for Quantization matrix [J. Tanaka, Y. Morigami, T. Suzuki (Sony)]

JCTVC-G352 Parameterization of Default Quantization Matrices [E. Maani, M. Haque, A. Tabatabai (Sony)]

JCTVC-G530 Non-CE4: Layered quantization matrices compression [Y. Wang (Tsinghua), J. Zheng, X. Zheng (HiSilicon), Y. He (Tsinghua)]

JCTVC-G730 Non-CE4: Cross check of layered quantization matrices compression in JCTVC-G530 [T. Suzuki (Sony)] [late] [miss]

JCTVC-G578 Non-CE4: Quantization matrix compression and signaling [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G826 Non-CE4: Cross check of quantization matrix coding in JCTVC-G578 [T. Suzuki (Sony)] [late] [miss]

JCTVC-G658 Quantization matrices in fragmented APS [Y. Chen, Y.-K. Wang, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-G880 HVS Model based Default Quantization Matrices [Munsi Haque, Ali Tabatabai, Yoshitaka Morigami (Sony)] [late]

### Dequantization

JCTVC-G279 Non-CE4: Improved Adaptive-Quantization Offset [X. Li, X. Guo, S. Lei (MediaTek), X. Wang, R. Joshi (Qualcomm)] [late] [miss]

JCTVC-G154 De-quantization without Rounding Offset [X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-G816 Cross verification of JCTVC-G154, De-quantization without Rounding Offset [R. Cohen (MERL)]

JCTVC-G555 One-addition dequantization [N. Shlyakhov, I.-K. Kim, J. H. Park (Samsung)]

JCTVC-G719 Structured Level Limits [L. Kerofsky, A. Segall, K. Misra (Sharp), E. Alshina, A. Alshin (Samsung)]

JCTVC-G720 Dequantization with symmetric reconstruction points [L. Kerofsky, A. Segall, K. Misra (Sharp)]

JCTVC-G723 Support for finer QP granularity in HEVC [Krit Panusopone, Ajay Luthra, Xue Fang, Jae Hoon Kim, Limin Wang (Motorola Mobility)]

## Alternative coding modes

### Lossless coding and I_PCM

JCTVC-G093 AHG22: Sample-based angular prediction (SAP) for HEVC lossless coding [M. Zhou (TI)]

JCTVC-G115 AHG22: Cross-verification of TI’s Sample-based angular prediction (SAP) for HEVC lossless coding (JCTVC-G93) [K. Chono, H. Aoki (NEC)]

JCTVC-G118 Potential enhancement of signaling of I_PCM blocks [K. Chono, H. Aoki (NEC)]

JCTVC-G268 AHG22: Lossless Transforms for Lossless Coding [W. Dai, M. Krishnan, J. Topiwala, P.Topiwala (FastVDO)]

JCTVC-G664 AHG22: A lossless coding solution for HEVC [W. Gao, M. Jiang, Y. He, J. Song, H. Yu (Huawei Technologies)]

JCTVC-G885 Cross-check of JCTVC-G664 [B. Li (USTC), J. Xu (Microsoft)] [late] [miss]

### Variable resolution coding

JCTVC-G264 AHG18: Adaptive Resolution Coding (ARC) [T. Davies (Cisco), P. Topiwala (FastVDO)]

JCTVC-G597 AHG 18: Cross-check of Adaptive Resolution Coding (ARC) (JCTVC-G264) [Glenn Van Wallendael, Jan De Cock, Rik Van de Walle (Ghent Univ.), David Flynn (BBC)] [late]

JCTVC-G329 AHG18: Comments on the Implementations of Resolution Adaption on HEVC [M. Li, P. Wu (ZTE)]

JCTVC-G715 AHG18/21: Absolute signaling for resolution switching [K. Misra, S. Deshpande, L. Kerofsky, A. Segall (Sharp)] [late]

JCTVC-G862 Advanced Resampling Filters For HEVC Applications [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

### Transform skip

JCTVC-G575 Transform skip mode [Marta Mrak, Andrea Gabriellini, David Flynn (BBC)]

JCTVC-G894 Cross check for G575 K. Panusopone [late] [miss]

JCTVC-G908 Cross-check of BBC’s transform skip mode (JCTVC-G575) [J. Sole (Qualcomm)] [late] [miss]

JCTVC-G933 Cross-check for skipped transform from BBC(JCTVC-G575) by Samsung [I.-K. Kim, E. Alshina, J.H. Park (Samsung)] [late] [miss]

JCTVC-G577 Transform skipping dependant on block parameters [Glenn Van Wallendael, Jan De Cock, Rik Van de Walle (Ghent Univ.), Marta Mrak (BBC)]

JCTVC-G663 AHG19: Modification to HE transform coefficient coding for transform skip mode [R. Joshi, J. Sole, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G941 AHG19: Cross-check of JCTVC-G663, modifications to HE transform coefficient coding for transform skip mode [R. Cohen (MERL)] [late]

JCTVC-G945 Cross-check of JCTVC-G663, modifications to HE transform coefficient coding for transform skip mode [Andrea Gabriellini, Marta Mrak (BBC)] [late] [miss]

### Other

JCTVC-G669 Delayed Duplicate I-Frame for Video Conferencing [R.Srinivasan, C. Ghone, M. Mody, M. Zhou (TI)]

JCTVC-G795 Crosscheck of TI’s JCTVC-G669 on delay dependent intra frame for video conferencing [J. Min, Y. Piao, J.H. Park (Samsung)] [late]

## Entropy coding

### CAVLC

JCTVC-G247 Non-CE5: CAVLC counters normalization per LCU [E. François, S. Pautet, C. Gisquet (Canon)]

JCTVC-G405 Non-CE5: Cross-check of CAVLC counters normalization per LCU (JCTVC-G247) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G355 Non-CE5: joint coding of splitting flag and inter modes [W. Zhang, P. Wu (ZTE)]

JCTVC-G741 Cross-check report on joint coding of splitting flag and inter modes (JCTVC-G355) [I.-K. Kim (Samsung)] [late]

JCTVC-G905 Crosscheck report of ZTE's joint coding of splitting flag and inter modes (JCTVC-G355) [X. Wang (Qualcomm)] [late]

JCTVC-G365 Non-CE5: Redefined contexts for last nonzero coefficient coding of 4x4 TU in CAVLC [J. Xu, A. Tabatabai (Sony)]

JCTVC-G626 Cross-check of JCTVC-G365, "Nonzero coefficient coding in CAVLC" [A. Fuldseth (Cisco)] [late]

JCTVC-G692 Intra Table for CAVLC [S.-H. Kim, A. Segall] [late]

### CABAC

JCTVC-G155 Non-CE1: On CABAC context initialization [C. Yeo, Y. H. Tan, Z. Li (I2R)]

JCTVC-G867 Non-CE1: Crosscheck of I2R's CABAC Context Initialization (JCTVC-G155) by Qualcomm [L. Guo (Qualcomm)] [late]

JCTVC-G326 Non-CE1.b: On the exponential memory decay probability update [J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G440 Non-CE1: Modified probability model update for complexity reduction [A. Tanizawa, T. Shiodera, T. Yamakage (Toshiba)]

JCTVC-G767 Non-CE1: Bypass coding of Intra prediction modes in CABAC [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G833 Non-CE1: Adaptive initialization for CABAC with fixed probability contexts [L. Guo, J. Sole, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G909 Non-CE1: Cross-check of JCTVC-G833 on adaptive initialization for CABAC [C. Yeo (I2R)] [late]

JCTVC-G718 Context reduction for CABAC [W. -J. Chien, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G848 Non-CE1: Crosscheck for Qualcomm's context reduction for CABAC in JCTVC-G718 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-G324 Modified LPS range and state transition tables for BAC J. Sole, M. Karczewicz (Qualcomm)

JCTVC-G413 Modified probability update and table removal for multi-parameter CABAC update (F254) [C. Rosewarne, M. Maeda (Canon)]

JCTVC-G805 Cross-check of Canon’s modified probability update and table removal for multi-parameter CABAC update (JCTVC-G413) [J. Sole (Qualcomm)] [late] [miss]

JCTVC-G492 Maximum VLC Limits in CABAC Escape Coding [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G493 CABAC Stream Termination [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G494 CABAC Packet-based Stream [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G501 CABAC with Arithmetic Context Variables [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G504 CABAC with Combined Context Variables [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G934 Non CE1: cross-check for CABAC probability up-date modification from Sony (G501, 504) by Samsung [A. Alshin, E. Alshina, J.H. Park (Samsung)] [late] [miss]

JCTVC-G570 Byte alignment overhead reduction [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-G716 On CABAC Init IDC [K. Misra, A. Segall (Sharp)] [late]

JCTVC-G837 Non-CE1: 8-bit Initialization for CABAC [L. Guo, R. Joshi, J. Sole, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G913 Cross-check for Qualcomm’s proposal on 8-bit initialization for CABAC (JCTVC-G837) [Y. Piao, J. Min, J.H. Park (Samsung)] [late]

JCTVC-G917 On significance map coding for CABAC [??] [late] [miss]

### Other

JCTVC-G064 Results of the PA-Coder [H. Zhu]

JCTVC-G452 SABAC - Scalable-complexity Adaptive Binary Arithmetic Coding [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G547 Counter-based probability model update with adapted arithmetic coding engine [J. Stegemann, H. Kirchhoffer, D. Marpe, T. Wiegand (Fraunhofer HHI)]

JCTVC-G470 Non-CE1: Cross-check on HHI's proposal (JCTVC-G547) [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-G568 HM 4.0 entropy coding complexity study and software improvements [M. Viitanen, J. Vanne, T. D. Hämäläinen (TUT), J. Lainema, K. Ugur (Nokia)]

JCTVC-G569 Single entropy coder for HEVC with a high throughput binarization mode [J. Lainema, K. Ugur, A. Hallapuro (Nokia)]

JCTVC-G659 Non CE1: Study of Entropy Coding Methods Complexity [M. Karczewicz, I.S. Chong, X. Wang, R. Joshi (Qualcomm)]

## Transform coefficient coding

JCTVC-G112 Changing luma/chroma coefficient interleaving from CU to TU level [T. Hellman, Y. Yu (Broadcom)]

JCTVC-G645 Crosscheck of JCTVC-G112 - Changing Luma/Chroma Coefficient Interleaving from CU to TU level [M. Budagavi (TI)] [late]

JCTVC-G201 Non-CE1: Codeword reordering for last_significant_coeff_x and last_significant_coeff_y [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G761 Cross-check report for MediaTek's proposal (JCTVC-G201) [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-G123 Non-CE11: Simplified Coefficient Scans for NSQT [V. Sze (TI)]

JCTVC-G758 Non-CE11: Cross-check report for TI's proposal (JCTVC-G123) on Simplified Coefficient Scans for NSQT [H. Sasai, T. Nishi (Panasonic)] [late] [miss]

JCTVC-G226 Non-CE11: Extending MDCS to 16x16 and 32x32 TUs [C.-W. Hsu, X. Zhao, X. Guo, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G187 Non-CE11: Cross-verification of MediaTek's proposal (JCTVC-G226) by JVC KENWOOD [S. Fukushima (JVC Kenwood)]

JCTVC-G239 Non-CE11: Modified method for coding the positions of last significant coefficients in the CABAC mode [S.-T. Hsiang, S. Lei (MediaTek)]

JCTVC-G947 Crosscheck for Modified method for coding the positions of last significant coefficients in the CABAC mode (JCTVC-G239) [G. Clare, F. Henry (Orange FT)] [late] [miss]

JCTVC-G271 Sign Data Hiding [G. Clare (Orange Labs), F. Henry (Orange Labs)]

JCTVC-G889 Crosscheck of G271 on Sign Data Hiding [Andrea Gabriellini, Marta Mrak (BBC)] [late]

JCTVC-G285 Non-CE11: Methods for Solving the Parsing Issue of MDCS [X. Zhao, X. Guo, C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G531 Cross verification of MediaTek’s proposed methods for solving the parsing issue of MDCS (JCTVC-G285) [Y. Chiu, L. Xu (Intel)] [late]

JCTVC-G301 Test Results On Context simplification for coefficients entropy coding [X. Che, W. Ding, Y. Shi (Beijing Univ. Tech.)]

JCTVC-G128 Cross-check of proposal on context simplification for coefficients entropy coding G301 (F148) [V. Sze (TI)]

JCTVC-G308 Complexity reduction of significant map coding [T. Ikai (Sharp)]

JCTVC-G751 Cross-check report for Sharp's proposal (JCTVC-G308) on Complexity reduction of significant map coding [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-G323 Non-CE11: Diagonal sub-block scan for HE residual coding [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-G810 Crosscheck of JCTVC-G323 on sub-block scan for residual coding from Qualcomm [C. Auyeung (Sony)] [late] [miss]

JCTVC-G366 Non-CE11: Context reduction of significance map coding with CABAC [C. Auyeung, J. Xu (Sony)]

JCTVC-G127 Cross-check of Sony’s proposal on context reduction for significance map (G366) [V. Sze (TI)]

JCTVC-G370 Binarisation of last position for higher throughput [C. Auyeung (Sony)]

JCTVC-G900 Cross-check of binarisation of last position for higher throughput (G370) [V. Drugeon, M. Narroschke (Panasonic)] [late] [miss]

JCTVC-G372 Coding order of sign and level minus 3 with CABAC [C. Auyeung, T. Suzuki (Sony)]

JCTVC-G744 Cross-check report on Coding order of sign and level minus 3 with CABAC (JCTVC-G372) [I.-K. Kim (Samsung)]

JCTVC-G381 Nearest placement of Y/Cb/Cr transform coefficients locating at same spatial position [Y. Shibahara, K. Uchibayashi, T. Nishi (Panasonic)]

JCTVC-G904 Cross Check of G-381 [Ankur Saxena, Felix Fernandes (Samsung)] [late] [miss]

JCTVC-G444 Proposed fix on cbf flag signaling [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-G760 Cross-check report for Mitsubishi's proposal (JCTVC-G444) on Proposed fix on cbf flag signaling [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-G448 Non-CE11: Context reduction for coding transform coefficients [S.-T. Hsiang, S. Lei (MediaTek)]

JCTVC-G491 Hybrid Horizontal/Vertical with Diagonal Mode Dependent Coefficient Scan [K. Sharman, J. Gamei, N. Saunders, P. Silcock (Sony)]

JCTVC-G943 Non-CE7: Crosscheck for Sony's MDCS in Proposal JCTVC-G491 [X. Zhao, X. Guo (MediaTek)] [late] [miss]

JCTVC-G520 Non-CE11: Modified Context Derivation for last coefficient flag [H. Sasai, T. Nishi (Panasonic)]

JCTVC-G942 Non-CE11: Cross-check of Modified Context Derivation for last coefficient flag (JCTVC-G520) [T. Tsukuba, T. Ikai (Sharp)] [late]

JCTVC-G522 Non-CE11:Context reduction for coefficient level [K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-G537 Table reduction and Improvement of last position coding in CAVLC [C. Kim, Y. Park, J.H. Park(Samsung)]

JCTVC-G838 Cross-check of CAVLC last position coding (JCTVC-G537) [T. Yamamoto (Sharp)] [late]

JCTVC-G545 Improvement of level coding in CAVLC [C. Kim, Y. Park, J.H. Park(Samsung)]

JCTVC-G801 Cross-check for Samsung's proposal (JCTVC-G545) [J. Xu (Sony)] [late] [miss]

JCTVC-G554 Grouping of bypass bins for last position coding of transform coefficients [I.-K. Kim, V. Seregin, J. H. Park(Samsung)]

JCTVC-G586 Parallelizable context for significance coding of large transform blocks [J. Kang, J. Lainema, A. Hallapuro, K. Ugur (Nokia)]

JCTVC-G644 Multi-level Significant Maps for Large Transform Units [N. Nguyen, T. Ji, D. He, G. Martin-Cocher, L. Song (RIM)]

JCTVC-G657 Encoding and decoding significant coefficient flags for small Transform Units using partition sets [Gergely Korodi, Jinwen Zan, Dake He (RIM)]

JCTVC-G796 Cross-check for RIM’s proposal on small TU significance map coding (JCTVC-G657) [Y. Piao, E. Alshina, J. Min, J.H. Park (Samsung)]

JCTVC-G685 Selective Run-Level Coding for CAVLC [S.-H. Kim, A. Segall (Sharp)]

JCTVC-G700 On coeff_abs_level_minus3 coding [J. Lou, L. Wang (Motorola Mobility)]

JCTVC-G854 Crosscheck - On coeff_abs_level_minus3 coding (G700) [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-G701 On last significant coefficient position coding [J. Lou, J. Kim, L. Wang (Motorola Mobility)] [late] [miss]

JCTVC-G704 Last position coding for CABAC [W.-J. Chien, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-G873 Cross-check of JCTVC-G704 on last position coding for CABAC from Qualcomm [Edouard François (Canon)] [late]

JCTVC-G724 Non-CE11: Entropy coding for non-square TU blocks [Vivian Kung, Krit Panusopone (Motorola Mobility)]

JCTVC-G875 Cross-check of Motorola's proposal (JCTVC-G724) - entropy coding for non-square blocks [H. Yang, H. Yu (Huawei)] [late] [miss]

JCTVC-G725 Non-CE11: Simplification of significant coding for CABAC [Vivian Kung, Krit Panusopone (Motorola Mobility)]

JCTVC-G876 Cross-check of Motorola's proposal (JCTVC-G725) - Simplification of significant map coding for CABAC [H. Yang, H. Yu (Huawei)] [late] [miss]

JCTVC-G768 Reduced contexts for significance map coding of large transform in CABAC [Y. Piao, J. Min, J.H. Park (Samsung)]

JCTVC-G642 Cross-check of Samsung context reduction in significance map coding [Jinwen Zan, Dake He] [late] [miss]

JCTVC-G781 Reduced chroma contexts for significance map coding in CABAC [Y. Piao, J. Min, E. Alshina, J.H. Park (Samsung)]

JCTVC-G783 Context number reduction for level coding in CABAC [Y. Piao, J. Min, J.H. Park (Samsung)]

JCTVC-G125 Cross-check of proposal on context reduction for coefficient levels (Gxxx) [V. Sze (TI)] [late] [miss]

JCTVC-G903 On last position coding for CABAC [??] [late] [miss]

## Intra prediction and mode coding (ref CE6)

### Intra prediction

JCTVC-G135 Non-CE6: Rectangular (2NxN and Nx2N) Intra Prediction [S. Liu, X. Zhang, Z. Zhou, S. Lei (MediaTek)]

JCTVC-G809 Crosscheck of JCTVC-G135 on rectangular (2NxN and Nx2N) intra prediction from MediaTek [C. Auyeung (Sony)] [late] [miss]

JCTVC-G139 Non-CE6.d: Intra Prediction With Selective Secondary Boundary [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-G145 Non-CE6: Reducing Line Buffers for intra mode [J. Lim, Y. Jeon, S. Park, B. Jeon (LG)]

JCTVC-G474 Non-CE6b: Crosscheck for LG's intra mode line buffer reduction in JCTVC-G145 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G173 Cross-channel intra chroma residual prediction [Y. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel)]

JCTVC-G676 Non-CE6: Crosscheck for Intel's Intra Chroma Prediction in JCTVC-G173 [M. Guo, X. Guo (MediaTek)] [late] [miss]

JCTVC-G244 Luma-based chroma prediction - Model correction [C. Gisquet, E. François (Canon)]

JCTVC-G646 Crosscheck of JCTVC-G244 - Luma-based chroma prediction - Model correction [M. Budagavi (TI)] [late]

JCTVC-G270 Non-CE6a: Cross-checking of JCTVC-G244 - Luma-based chroma prediction - Model correction [P.Bordes, P.Salmon (Technicolor)] [late]

JCTVC-G346 Chroma intra prediction based on residual luma samples [K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI)]

JCTVC-G069 Cross-verification of KDDI's proposal on intra coding (JCTVC-G346) [Masaaki Matsumura, Shohei Matsuo, Seishi Takamura, Hirohisa Jozawa (NTT)]

JCTVC-G911 Cross verification of KDDI’s Chroma intra prediction based on residual luma samples (JCTVC-G346) [Y. Chiu, W. Zhang, L. Xu, Y. Han (Intel)] [late]

JCTVC-G350 Modification of angular intra prediction [Shohei Matsuo, Seishi Takamura, Hirohisa Jozawa (NTT)]

JCTVC-G348 Cross-check report of NTT's intra prediction approach (JCTVC-G350) [T. Yoshino, S. Naito (KDDI)]

JCTVC-G354 Non-CE6: Improvements for SDIP [J. Xu, E. Maani, A. Tabatabai (Sony)]

JCTVC-G804 Crosscheck for Sony's JCTVC-G354 on Improvements for SDIP [C. Lai, L. Liu, J. Zheng(HiSilicon)] [late]

JCTVC-G358 New modes for chroma intra prediction [X. Zhang, O. C. Au, J. Dai, F. Zou, C. Pang, X. Wen (HKUST)]

JCTVC-G273 Crosscheck for JCTVC-G358 new modes for chroma intra prediction [J. Dong (InterDigital)] [late] [miss]

JCTVC-G373 Non-CE6: Performance of secondary boundary DC intra prediction [C. Auyeung (Sony)]

JCTVC-G861 Crosscheck results of JCTVC-G373 on the performance of secondary boundary DC intra prediction from Sony [X. Zhang, S. Liu (MediaTek)] [late]

JCTVC-G374 Improving the Intra Prediction Based on a Uniform Probability Model [L. Liu (HiSilicon and Huawei)]

JCTVC-G481 Cross verification of Huawei’s intra prediction improvement based on a uniform probability model (JCTVC-G374) by Intel [Y. Chiu, L. Xu (Intel)] [late]

JCTVC-G762 Cross-check of JCTVC-G374 on improving Intra prediction [H. L. Tan, C. Yeo (I2R)]

JCTVC-G443 Improved directional intra prediction smoothing [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-G896 Cross verification of improved directional intra prediction smoothing (JCTVC-G443) [J. Lainema, K. Ugur (Nokia)] [late] [miss]

JCTVC-G447 Reduced number of intra 64x64 prediction mode [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G193 Cross-verification result of JCTVC-G447 on reduced number of intra 64x64 prediction mode [K.Kazui (Fujitsu)]

JCTVC-G449 Reduced number of intra chroma prediction mode [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-G840 Cross-verification of Mitsubishi’s reduced number of intra chroma prediction mode (JCTVC-G449) [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G511 Performance Evaluation of Luma-based Chroma Intra Prediction [K. Sato (Sony)]

JCTVC-G567 Simplified DC prediction [J. Lainema, K. Ugur (Nokia)]

JCTVC-G594 Improved Extrapolation for Intra Prediction Coding [J. Kim, S. Bae, H. Sabirin, M. Kim, J. Kim, B. Jeon]

JCTVC-G595 Linear model based chroma intra prediction coding for no access to LCU boundary reference pixels [J. Kim, M. Kim, B. Kim, S. Hahm, K. Lee]

JCTVC-G598 Non-CE6: Intra prediction based on weighted template matching predictors (WTM) [T. Guionnet, L. Guillo (INRIA)]

JCTVC-G604 Non-CE6: Cross-checking of JCTVC-G598: Intra prediction based on weighted template matching predictors (WTM) [P Bordes, P Salmon (Technicolor)] [late]

JCTVC-G738 On angular intra prediction main array extension [M. Coban (Qualcomm)]

JCTVC-G882 Cross-verification report for 8-bit precision slope computation on angular prediction array extension (JCTVC-G738) [H. Aoki, K. Chono (NEC)] [late]

JCTVC-G791 Non-CE6: Simplified reference samples padding for intra prediction [T. Lee, J. Chen, J. H. Park (Samsung)]

JCTVC-G120 Cross-verification of Samsung's proposal on padding process (JCTVC-G791) [Y. Lin, C. Lai (HiSilicon)] [late]

### Intra mode coding

JCTVC-G109 On Intra Mode Mapping [X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-G148 Non-CE6: Cross-check of MediaTek’s MPM mapping by LG (JCTVC-G109) [J. Lim, S. Park, B. Jeon (LG)]

JCTVC-G119 Modifications to Intra-frame coding [Y. Lin, H. Yang, L. Liu, J. Zheng, H. Yu (Huawei)]

JCTVC-G117 Non-CE6: Cross-verification of Huawei's Modifications to intra frame coding (JCTVC-G119) [K. Chono, H. Aoki (NEC)]

JCTVC-G144 Non-CE6: on MPM mapping [J. Lim, S. Park, B. Jeon (LG)]

JCTVC-G473 CE6b: Crosscheck for LG's MPM mapping modification in JCTVC-G144 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G153 Non-CE6: On intra prediction mode coding [C. Yeo, H. L. Tan, Y. H. Tan, Z. Li (I2R)]

JCTVC-G106 Cross-Check for G-153: On Intra Mode Coding [A. Saxena, F. Fernandes (Samsung)]

JCTVC-G254 Non-CE6b: Cross-check of JCTVC-G153 on MPM index bypass coding from I2R [E. François (Canon)]

JCTVC-G184 Non-CE6: Unified neighboring positions for intra mode coding [S. Fukushima, H. Nakamura (JVC Kenwood)]

JCTVC-G475 Non-CE6b: Crosscheck for JVC KENWOOD's unified neighboring positions for intra mode coding in JCTVC-G184 [T.-D. Chuang, Y.-W. Huang (MediaTek)]

JCTVC-G185 Non-CE6: On intra chroma mode coding [T. Kumakura, S. Fukushima (JVC Kenwood)]

JCTVC-G784 Crosscheck on JCTVC-G185 Non-CE6: On intra chroma mode coding [W.-J. Chien]

JCTVC-G245 Non-CE6a: Use of chroma phase in LM mode [E. François, C. Gisquet, S. Pautet (Canon)]

JCTVC-G488 Non-CE6a : Cross check on use of chroma phase in LM mode (JCTVC-G245) [T. Guionnet, L. Guillo (INRIA)]

JCTVC-G756 Non-CE6: Crosscheck of Canon's improvement on LM mode (JCTVC-G245) [X. Zheng] [late] [miss]

JCTVC-G359 Non-CE6: Coding of luma intra prediction modes that are not in the MPM set [R. Cohen, X. Xu, A. Vetro, H. Sun (MERL)]

JCTVC-G456 Non-CE6: Cross check report of MERL’s intra prediction mode coding (G359) [Atsuro Ichigaya (NHK)]

JCTVC-G377 Non-CE6: Cross-check report of proposal on coding of luma intra prediction modes that are not in the MPM set (JCTVC-G359) [Y. Shibahara, T. Nishi (Panasonic)]

JCTVC-G418 Simplification of intra prediction mode mapping method [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

JCTVC-G870 Cross-check report of JCTVC-G418 [K Sato (Sony)] [late]

JCTVC-G419 Inconsistency of intra LM mode between HM and WD [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

JCTVC-G886 Cross-verification of ETRI's Inconsistency of intra LM mode between HM and WD (JCTVC-G419) [T. Lee, J. Chen, J. H. Park] [late]

JCTVC-G423 Non-CE6: Remove potential duplicate modes from the candidate mode list for chroma intra prediction [H. Yang, J. Zhou, H. Yu (Huawei)]

JCTVC-G408 Non-CE6: Cross-check of Remove potential duplicate modes from the candidate mode list for chroma intra prediction (JCTVC-G423) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-G707 Using CABAC bypass mode for coding intra prediction mode [K. Misra, A. Segall (Sharp)]

JCTVC-G754 Non-CE6: Line buffer reduction for CABAC context of SDIP syntax [L. Guo, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-G871 Non-CE6: Crosscheck of Qualcomm's buffer reduction for CABAC context of SDIP syntax (JCTVC-G754) by HKUST [Xing Wen, Oscar Au, Xingyu Zhang] [late]

### Padding

JCTVC-G572 AHG16: Reference sample padding harmonization for intra DC mode [V. Wahadaniah, C. S. Lim (Panasonic)]

JCTVC-G897 AHG16: Cross-verification and additional results of reference sample padding harmonization for intra DC mode (JCTVC-G572) [K. Chono, H. Aoki (NEC)] [late] [miss]

JCTVC-G812 AHG16: Padding process simplification [X. Wang, W.J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G916 AHG16: Crosscheck of JCTVC-G812 on Padding process simplification [X. Li, X. Guo (MediaTek)] [late] [miss]

## Transforms

### Core transform implementation (ref CE10)

JCTVC-G272 Non-CE10: Core Transform Design for HEVC [J. Dong, Y. Ye (InterDigital)]

JCTVC-G386 Non-CE10: Cross Check Report for JCTVC-G272 Core Transform Design for HEVC. [X. Zhang, O. C. Au, X. Wen (HKUST)] [late] [miss]

JCTVC-G333 Accuracy improvement of HM's transform bases [Y. Sugito, A. Ichigaya, S. Sakaida (NHK)]

JCTVC-G625 Cross-check of JCTVC-G333, "Accuracy improvement of HM's transform bases" [A. Fuldseth (Cisco)] [late]

JCTVC-G496 Core transform design for HEVC with 7 bit coefficients [A. Fuldseth, G. Bjøntegaard (Cisco), M. Budagavi, V. Sze (TI)]

JCTVC-G335 Cross-check on 7 bit transform matrices (JCTVC-G496) [Y. Sugito, A. Ichigaya (NHK)]

JCTVC-G628 Comparison of core transform proposals [A. Fuldseth, G. Bjøntegaard (Cisco), M. Budagavi (TI)]

JCTVC-G782 AhG7: Overflow Prevention in HEVC inverse transform [E. Alshina, A. Alshin, J.H. Park (Samsung)]

JCTVC-G856 AHG7: IDCT output range after T+Q+IQ+IT with valid residual inputs [M. Zhou (TI)]

JCTVC-G497 SIMD optimization of proposed HEVC core transforms [A. Fuldseth, L. P. Endresen, S. Selnes (Cisco), V. Arbatov, F. Franchetti (SpiralGen), M. Puschel (ETH Zurich)]

JCTVC-G811 Crosscheck of JCTVC-G497 on SIMD optimization of core transforms [C. Auyeung (Sony)] [late] [miss]

JCTVC-G132 Hardware analysis of transform and quantization [M. Budagavi (TI), V. Sze (TI), M. Sadafale]

JCTVC-G263 Hardware complexity analysis of spatial transform [X. Tian (Altera)]

JCTVC-G265 Core Transform Property for Practical Throughput Hardware Design [M. Tikekar, C.-T. Huang, C. Juvekar, A. Chandrakasan (MIT)]

JCTVC-G857 SIMD Analysis of Some Core Spatial Transforms [S. Riabtsev (CSR)] [late] [miss]

JCTVC-G601 Cross verification of SIMD-optimized partial butterfly HM transforms [?? (BBC)] [late] [miss]

JCTVC-G865 New Results for Guaranteeing 16-bit Transform Dynamic Range [Kiran Misra, Louie Kerofsky, Andrew Segall (Sharp)] [late]

JCTVC-G866 On transforms and higher bitdepth coding [Kiran Misra, Louie Kerofsky, Andrew Segall (Sharp)] [late] [miss]

### Alternative transforms

JCTVC-G281 Non-CE7: Boundary Dependent Transform for Inter Predicted Residue [J. An, X. Zhao, X. Guo, S. Lei (MediaTek)]

JCTVC-G631 Cross-Check of G-281: Boundary Dependent Transform for Inter Predicted Residue [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-G282 Non-CE7: Mode Dependent DCT/DST for Chroma [X. Zhao, X. Guo, M. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU)]

JCTVC-G879 Cross-Check of G-282 [A. Saxena, F. Fernandes]	 [late]

JCTVC-G345 Non-CE7: Restricted mode-dependent 8x8 DST for Intra prediction [Atsuro Ichigaya, Yasuko Sugito, Shin-ichi Sakaida, (NHK)]

JCTVC-G630 Cross Check of G-345: Restricted Mode-Dependent 8x8 DST for Intra prediction [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-G817 Non-CE7: Cross verification of JCTVC-G345, Restricted mode-dependent 8x8 DST for Intra prediction [R. Cohen (MERL)]

JCTVC-G591 Non-CE 7: Supplementary Results for the Rotational Transform [Zhan Ma, Felix Fernandes, Elena Alshina, Alexander Alshin (Samsung)]

JCTVC-G735 non-CE 7: Crosscheck of G597 Supplemental ROT Results [P. Topiwala (FastVDO)] [late] [miss]

JCTVC-G602 Non-CE7: Directional transform for inter blocks [D. Lee, J. Heo, J. Kim, S. Yea (LG)]

JCTVC-G629 Non CE 7: Harmonization of SDIP and Mode-Dependent Secondary Transforms [A. Saxena (Samsung), Y. Shibahara (Panasonic), F. Fernandes (Samsung), T Nishi (Panasonic)]

JCTVC-G937 Cross-check of JCTVC-G629 : Harmonization of SDIP and Mode-Dependent Secondary Transforms [H. Yang (Huawei), C. Lai (Hisilicon)] [late] [miss]

JCTVC-G632 Non CE 7: On secondary transforms for inter prediction residual [Ankur Saxena, F. Fernandes (Samsung)]

## IBDI and memory compression

JCTVC-G328 AHG8: Bit depth of output pictures [Y. Chen, Y. -K. Wang, X. Wang, I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-G430 An experimental comparison of memory bandwidth between 8-bit and 10-bit coding [T.Chujoh (Toshiba), K.Chono (NEC)]

## Parsing throughput, robustness and error resilience (ref CE13)

JCTVC-G221 Non-CE13: The maximum number of merging candidates in P-slice [H. Nakamura, S. Fukushima, H. Takehara, M. Ueda (JVC Kenwood)]

JCTVC-G224 Non-CE13: Multiple-scaled merging candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-G544 Non-CE13: Cross check report of JCTVC-G224 by panasonic [T. Sugio, T. Nishi(Panasonic)] [late]

JCTVC-G288 Non-CE13: Complexity Reduction of MVP List Contruction [Y. Yu, T. Hellman (Broadcom)]

JCTVC-G097 Non-CE13: cross-verification of Broadcom’s proposal JCTVC-G288 complexity reduction of MVP list construction [M. Zhou (TI)]

JCTVC-G593 Non-CE13: Simplification of merge mode [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-G940 Cross-check for JCTVC-593: Non-CE13: Simplification of merge mode [Semih Esenlik (Panasonic)] [late] [miss]

JCTVC-G683 Non-CE13: Simplification and improvement of additional merge candidate [Y. Zheng, X. Wang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-G872 Non-CE13: crosscheck of Qualcomm’s proposal G683 on simplification and improvement of additional merge candidate [Y. Jeon, B. Jeon (LG)] [late]

JCTVC-G787 Non-CE13: Additional merge candidates with MV dependent offsets [T. Lee, J. Chen, J. H. Park (Samsung), G. Laroche, P. Onno (Canon), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-G926 Non-CE13: Cross-check of JCTVC-G787 on MV dependent offset [V. Seregin (Qualcomm)] [late] [miss]

JCTVC-G839 Non-CE13: cross-verification of Samsung, Canon, MediaTek proposal JCTVC-G787 on additional merge candidates with MV dependent offsets [Y. Jeon, B. Jeon (LG)] [late]

JCTVC-G946 Non-CE13: Cross-check report of JCTVC-G787 (MvDepOffset+Fast) [S.-C. Lim, J. Lee, H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU)] [late] [miss]

## Complexity assessment

JCTVC-G095 Virtual motion compensation memory bandwidth verifier (VMBV) [M. Zhou (TI)]

JCTVC-G262 HM decoder complexity assessment on ARM [T. Anselmo, D. Alfonso (STM)]

JCTVC-G757 On software complexity [F. Bossen (DOCOMO Innovations)]

## Encoder optimization

JCTVC-G401 RDO with weighted distortion in HEVC [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

JCTVC-G543 Early skip detection for HEVC [Jungyoup Yang, Jaehwan Kim, Kwanghyun Won, Hoyoung Lee, Byeungwoo Jeon (SKKU)]

JCTVC-G573 Cross-check of Early skip detection for HEVC with Early CU Termination (JCTVC-G543, JCTVC-F092) [S. Lee, S. Cho, S. Park, N. Eum (ETRI)] [late]

JCTVC-G794 Cross-check of Early skip detection for HEVC (JCTVC-G543) [Kiho Choi, Sang-hyo Park, Euee S. Jang] [late]

JCTVC-G589 Encoder optimization: Reduced number of reference pictures for RA [Per Wennersten, Rickard Sjöberg, Jonatan Samuelsson (Ericsson)]

JCTVC-G687 Fast fractional motion search with adaptive searching point reduction [Wan-Chi Siu (HKPU), Yan-Ho Kam (HKPU), Wai-Lam Hui (HKPU), Yui-Lam Chan (HKPU), Yu Liu (ASTRI), Jenny Yan Huo (ASTRI)]

## Withdrawn

JCTVC-G114 [withdrawn]

JCTVC-G160 [withdrawn]

JCTVC-G177 [withdrawn]

JCTVC-G356 [withdrawn]

JCTVC-G368 [withdrawn]

JCTVC-G538 [withdrawn]

JCTVC-G562 [withdrawn]

JCTVC-G670 [withdrawn]

JCTVC-G921 [withdrawn]

## Category not clear

JCTVC-G596 crosscheck [J. Kim, M. Kim] [late] [miss]

## Not presented due to late arrival or lack of availability

# Plenary Discussions and BoG Reports

## Project development

JCTVC-G061 HEVC issues (comments from USNB of WG11) [A. G. Tescher for USNB of WG11]

JCTVC-G096 Items to be clarified in HEVC design [M. Zhou (TI), W. Wan (Broadcom), T. Suzuki (Sony), A. Tabatabai (Sony)]

JCTVC-G507 Syntax Issues [K. Sato (Sony)]

JCTVC-G661 Parallel partition Profile & Level limits [Chad Fogg, Aaron Wells]

JCTVC-G729 Proposal to start the discussion on HEVC profile/level [T. Suzuki (Sony)]

JCTVC-G914 UKNB Comment on HEVC Extensions [UKNB of WG11] [late]

## BoGs

# Project planning

## WD drafting and software

JCTVC-G777 The Art of Writing Standards: Some Shalls and Shoulds for Better Quality Interop Specs [G. J. Sullivan (Microsoft)]

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be 8 Nov. 2011.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs

Because a draft design and HEVC test model (referred to as the HM) have now been established, group coordinated experiments are now referred to as "core experiments" rather than "tool experiments".

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-F900.

A deadline of four weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within three weeks after the release of the HM 4.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision). [clean up notes]

The CE description for each planned CE is described in an associated output document JCTVC-F9xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at the preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-F900.

## Software development

The software coordinator had already started integrating bug fixes on top of HM 3.3 software, and had strongly recommended for proponents of adopted proposals to re-implement them in HM 3.3 and test in this environment before integrating them into HM 4.x. All tools were planned to again be thoroughly tested after integration in HM 4.x.

HM 4.0 should be available within 4 weeks after the meeting, and will be used for CEs. HM 4.1 is planned to be available 3 weeks later.

## Subjective test plan for design

Plan a subjective test enabling results at the Feb meeting.

Plans for subjective testing: Preparation of visual comparison for the February meeting will need to be discussed in the next meeting. Initial idea: Use a set of rates e.g. as from CfP, and run HM and JM on same conditions. It may not be necessary to run all different constraint cases, RA may be sufficient.

Need to prepare JM & HM encodings, roughly as was done for the CfP.

# Establishment of ad hoc groups (tbd)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts - Report on project status to JCT-VC reflector - Provide report to next meeting on project coordination status | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-F802 HEVC Test Model 4 (HM 4) Encoder Description - Produce and finalize JCTVC-F803 HEVC text specification Working Draft 4 - Gather and address comments for refinement of these documents - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text | B. Bross, K. McCann (co-chairs), W.-J. Han, J.-R. Ohm, S. Sekiguchi, G. J. Sullivan, T. Wiegand (vice chairs) | N |
| Software development and HM software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCTVC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 4.0 software version and the reference configuration encodings according to JCTVC-F900 based on common conditions suitable for use in most core experiments (expected within four weeks after the meeting). - Prepare and deliver HM 4.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 4.0 version (expected within three weeks after the 4.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text | F. Bossen (chair), D. Flynn, K. Sühring (vice chairs) | N |
| Picture Partitioning and LCU scan order (AHG4) (jct-vc@lists.rwth-aachen.de) - Study technical proposals related to picture partitioning and alternative LCU scan processing including slices, tiles and wavefront - Identify and work to resolve issues relating to the draft text description of picture partitioning and LCU scan order, and the associated reference software functionality - Study interactions and combinations of picture partitioning and LCU scan order related technical proposals - Study the coding efficiency and loss resilience impact of picture partitioning and LCU scan order - Study the use of picture partitioning and LCU scan order for high-level parallelism - Study the use of picture partitioning and LCU scan order for ultra low delay - Identify and discuss additional issues relating picture partitioning and LCU scan order | R. Sjöberg (chair), Y. Chen, F. Henry, M. Horowitz, K. Kazui, A. Segall (vice chairs) | N |
| Spatial transforms (AHG5) (jct-vc@lists.rwth-aachen.de) - Study the ("core" and additional) transforms in the HM design, including compression performance, computational complexity, dynamic range, clipping, storage requirements, etc. - Discuss transform-related Core Experiments, and identify potential synergies or incompatibilities related to the tools being tested in the CEs. - Report the results and conclusions of these studies, discussions and experiments to the JCT-VC. | P. Topiwala (chair), M. Budagavi, R. Cohen, R. Joshi (vice chairs) | N |
| In-loop and post-processing filtering (AHG6) (jct-vc@lists.rwth-aachen.de) - Study enhancement schemes of in-loop filtering, including de-blocking/de-banding/de-noising filters, and adaptive Wiener-based filters including variants with various inputs, and combination of filters - Study trade-offs and characteristics of filter designs including complexity and subjective and objective performance - Discuss relationships and evaluation procedures for the filtering techniques - Identify possibilities for harmonization of enhanced in-loop filtering technologies - Study the relationship between in-loop and post-processing filtering | T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke (vice chairs) | N |
| Transform dynamic range (AHG7) (jct-vc@lists.rwth-aachen.de) - Study methods for managing transform dynamic range - Find solutions guaranteeing 16 bit buffers in inverse transform without overflow and with minimal clipping operations - Study hardware and software aspects of inverse transform implementations | E. Alshina, A. Segall (co-chairs) | N |
| Reference pictures memory compression (AHG8) (jct-vc@lists.rwth-aachen.de) - Study motion compensation memory access bandwidth of HM design and proposed reference picture memory compression schemes - Study reference picture memory compression schemes and other motion compensation memory access reduction schemes - Study data format alignment between reference picture memory compression and display processing - Study the visual quality impact of reference picture memory compression - Report on conclusions reached | K. Chono (chair), T. Chujoh, D. Hoang, C. S. Lim, A. Tabatabai, M. Zhou (vice chairs) | N |
| Entropy coding architecture (AHG9) (jct-vc@lists.rwth-aachen.de) - Study the entropy coding complexity and compression characteristics of single entropy coder architectures, scalable entropy coder architectures and switchable entropy coder architectures - Consider the potential for introducing scalability in the entropy coding options in the current HM and other potential means ofharmonization of HE and LC entropy coding designs - Characterize entropy coding throughput, memory, silicon area, power requirements, etc. - Study interdependencies between entropy coding and other processes and the consequences of these interdependencies - Define practical use cases and corresponding HM configurations to enable testing of different entropy coding architectures in relevant environments - Study and develop approaches for hardware and software evaluation of entropy coding methods - Identify and discuss additional issues on entropy coding | K. McCann (primary), A. Fuldseth, J. Lainema, D. Marpe, A. Segall, K. Sugimoto, V. Sze, W. Wan, X. Wang (vice chairs) | N |
| Quantization (AHG10) (jct-vc@lists.rwth-aachen.de) - Study quantization issues in the HM design, including step size control, RDOQ, etc. - Study trade offs and characteristics of quantization design, including coding efficiency and complexity - Study the impact of quantization on subjective quality - Study proposed quantization schemes such as adaptive quantization level (AQL), quantization matrix support, and adaptive reconstruction offsets, and their effects - Study adequacy of current mapping of QP to quantizer step-size for rate control at different coding levels (LCU, slice, frame, etc.) | M. Budagavi (chair), M. Karczewicz, G. Martin-Cocher, K. Sato (vice-chairs) | N |
| Video test material selection (AHG11) (jct-vc@lists.rwth-aachen.de) - Identify, collect, and make available a variety of video sequence test material - Study the coding performance and characteristics of test materials - Identify and recommend appropriate test materials and corresponding test conditions for use in HEVC development | T. Suzuki (chair) | N |
| Complexity assessment (AHG12) (jct-vc@lists.rwth-aachen.de) - Summarize and evaluate the various complexity assessment methods with regard to: - computational complexity, - parallelism, - memory bandwidth, - memory capacity, - dynamic range requirements, - any other aspects of complexity identified as being of interest. - Develop and propose a set of general measurement metrics. - Identify criteria to determine the hardware implementability of key hardware modules. - Identify bottlenecks in the current design with regard to implementation complexity. | D. Alfonso (chair), J. Ridge, X. Wen (vice chairs) | N |
| Screen content coding (AHG13) (jct-vc@lists.rwth-aachen.de) - To coordinate the submission, evaluation and selection of "screen content" video test material - To study characteristics of screen content video - Analyze the effects of existing and proposed coding technology on screen content video - To study and establish evaluation methods, test conditions, and metrics for coding of screen content video - Study technology that may be particularly well suited to the coding of screen content video - Study use cases in which screen content video is prevalent and identify potential associated technical implications | O. Au (chair), J. Xu, H. Yu (vice chairs) | N |
| Loss robustness (AHG14) (jct-vc@lists.rwth-aachen.de) - Identify techniques and conditions for testing the loss robustness of the design. - Study the degree of loss robustness of the HM design and identify deficiencies - Identify and study the interdependencies in the HM design in relation to loss robustness, and the potential consequences of these interdependencies - Identify techniques and conditions for testing the loss robustness of the design. - Investigate solutions to improve loss robustness - Investigate the trade-off between coding efficiency and loss robustness - Discuss related Core Experiments, and identify potential synergies or incompatibilities related to the tools being tested in the CEs | S. Wenger (chair), M. Coban, Y. W. Huang, P. Onno, Y. K. Wang, J. Xu (vice chairs) | N |
| High-level syntax (AHG15) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, proposed additional types of parameter sets, and slice header syntax designs - Study and identify needs for SEI messages and VUI - Study possible improvements to the reference picture list construction processes - Study possible simplifications and improvements to reference picture marking process (e.g., the need of the processes for generating and handling of "non-existing" pictures) - Study the hypothetical reference decoder behaviour - Assist in software development and text drafting for the high-level syntax in the HEVC design | Y. K. Wang (chair), J. Boyce, Y. Chen, M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan W. Wan (vice chairs) | N |
| Padding process (AHG16) (jct-vc@lists.rwth-aachen.de) - Study schemes of padding for unavailable neighbouring samples needed by intra prediction schemes of HEVC HM4.x/WD4, including the trade-offs between complexity and coding efficiency performance. - Identify possibilities for harmonization between the neighbouring sample padding process with the processing and usage of neighboring samples for intra prediction (such as MDIS, intra DC prediction mode, etc.). - Report the results and conclusions to the JCT-VC. | V. Wahadaniah (chair), K. Chono, Y. Lin (vice chairs) | N |
| Scalable coding investigation (AHG17) (jct-vc@lists.rwth-aachen.de) - Investigate hooks that would be needed for support of bitstream scalability in HEVC syntax - Study the applicability and effectiveness (e.g., relative to simulcast and single-layer coding) of scalability tools - Study potential experimental conditions for evaluation of scalable video coding technologies | J. Boyce (chair), J. Kang, K. Minoo, W. Wan, Y.-K. Wang (vice chairs) | N |
| Resolution adaption (AHG18) (jct-vc@lists.rwth-aachen.de) - Assess the complexity and efficiency of resolution adaption (as in JCTVC-F158) against alternative approaches e.g. increased QP, and pre-filtering. - Study loss robustness properties and compare with techniques such as adaptive reference frame selection. - Investigate suitable resolution switching filters. - Consider the implications in terms of signalling, HRD, DPB management, and random access. | T. Davies (chair), P. Topiwala, P. Wu (vice chair) | N |
| Transform skipping (AHG19) (jct-vc@lists.rwth-aachen.de) - Study the adequacy of different entropy coding tools (scans, contexts) with different transform skip modes - Study transform skip mode flags signalling in the context of the RQT design - Identify and discuss additional issues relating to transform skip mode - Study throughput and complexity issues in the context of transform skipping | M. Mrak (chair), J. Sole, I.-K. Kim, J. Xu, H. Yu (vice chairs) | N |
| Chroma format support (AHG20) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and software that need modification to support non-4:2:0 chroma formats. - Assist and advise in the work of removing implicit assumptions of 4:2:0 formatting from the WD and software (where feasible, without introducing technical design changes). | D. Flynn (chair), D. Hoang, K. McCann (vice chairs) | N |
| Reference picture buffering and list construction (AHG21) (jct-vc@lists.rwth-aachen.de) - Study possible improvements to the reference picture list construction processes - Examine the behaviour of unified lists with the JCTVC-F493 proposed default list construction processes and identify any robustness issues. - Examine the merits of the list 0/1, combined and unified lists - Discuss and identify solutions for support of weighted prediction methods - Produce a single candidate WD text specification for picture buffer management based on JCTVC-F493 for review at the next meeting - Produce software supporting picture marking including the following features: - Flexible support for cyclic picture structures and temporal layering - Decoder support to decode a subset of temporal layers - Rudimentary error concealment for lost reference picture slices (frame copy of closest picture based on POC, set all MVs of lost reference picture to zero) | D. Flynn, R. Sjöberg (co-chairs), Y. Chen, T. K. Tan, W. Wan, Y. K. Wang (vice chairs) | N |
| Lossless coding (AHG22) (jct-vc@lists.rwth-aachen.de) - Study and investigate lossless coding techniques. - Consider complexity and efficiency tradeoffs. - Identify appropriate intra and inter predictive techniques for lossless coding. - Consider bit depth effects. | W. Gao (chair), K. Chono, J. Xu, M. Zhou (vice chairs) | N |

# Output documents (tbd)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-F634 HEVC Reference Software Manual [F. Bossen, D. Flynn, K. Sühring (AHG chairs)]

JCTVC-F688 Revised HEVC Software Guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

JCTVC-F800 Meeting Report of 6th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-F802 High Efficiency Video Coding (HEVC) Test Model 4 (HM 4) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, S. Sekiguchi, G. J. Sullivan] (WG 11 N 12185)

JCTVC-F803 High Efficiency Video Coding (HEVC) text specification Working Draft 4 [B. Bross (primary), W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand] (WG 11 N 12186)

JCTVC-F900 Common HM test conditions and software reference configurations [F. Bossen]

Software HM5.0 availability XX weeks after the meeting

Document deadline of February 2012 meeting is likely to be Jan XXth.

JCTVC-F901 Core Experiment 1: Entropy Coding Investigation [R. Joshi (primary), E. Alshina, H. Kirchhoffer, J. Lainema, H. Sasai]

JCTVC-F902 Core Experiment 2: Motion Partitioning and OBMC [X. Zheng (primary), I. S. Chong, I.-K. Kim]

JCTVC-F903 Core Experiment 3: Motion Compensation [T. Chujoh, E. Alshina]

JCTVC-F904 Core Experiment 4: Quantization [K. Sato (primary), H. Aoki, M. Budagavi, M. Coban, X. Li]

JCTVC-F905 Core Experiment 5: CAVLC Entropy Coding Improvement [X. Wang (primary), P. Wu, C. Y. Kim]

JCTVC-F906 Core Experiment 6: Intra coding improvements [A. Tabatabai (primary), E. François, K. Chono, R. Joshi, J. Lainema, H. Yu]

JCTVC-F907 Core Experiment 7: Additional transforms [R. Cohen (primary), F. Fernandes, R. Joshi, C. Yeo]

JCTVC-F908 Core Experiment 8: Non-deblocking loop filtering [T. Yamakage (primary), I. S. Chong, M. Narroschke, Y.-W. Huang]

JCTVC-F909 Core Experiment 9: MV Coding and Skip/Merge Operation [B. Bross (primary), W. J. Chien, J. Jung, I. K. Kim, M. Zhou]

JCTVC-F910 Core Experiment 10: Core Transforms [P. Topiwala (primary), M. Budagavi, A. Fuldseth, R. Joshi, E. Alshina]

JCTVC-F911 Core Experiment 11: Coefficient Scanning and Coding [V. Sze (primary), J. Chen, T. Nguyen, K. Panusopone, J. Sole]

JCTVC-F912 Core Experiment 12: Deblocking Filter [A. Norkin (primary), X. Guo, B. Jeon, M. Narroschke]

JCTVC-F913 Core Experiment 13: Motion data parsing robustness and throughput [J. Jung (primary), B. Bross, J. Chen, P. Onno, M. Zhou]

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 1–10 February 2012 under WG 11 auspices in San José, USA.

- 2–9 May 2012 under ITU-T auspices in Geneva, CH.

- 11–20 July 2012 under WG 11 auspices in Stockholm, SE.

- …

ITU was thanked for its excellent hosting of the 7th meeting of the JCT-VC. XXX were thanked for providing the viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately XXXX hours on Wednesday 30 Nov 2011.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixth meeting of the JCT-VC, according to a sign-in sheet passed around during the meeting (approximately 254 in total), were as follows:

- …