| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G1036 |
| --- | --- |

| Title: | Common conditions for reference picture marking and list construction proposals |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Miska M. Hannuksela TK Tan (editors) | Email: | yekuiw@qualcomm.com miska.hannuksela@nokia.com tktan.p@rd.nttdocomo.co.jp |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This document describes common test conditions for the following types of input contributions:

- Signaling of picture identification in slice header (similar to picture order count)

- Signaling of reference picture set

- Signaling of reference picture lists (including reference picture list modification commands, if present), including lists 0, 1, and the combined list.

- Signaling of decoded reference picture marking, if present

Note that the common conditions may also be applied for other high-level syntax proposals, such as coding of syntax elements in sequence parameter set (including VUI), picture parameter set, adaptation parameter set, and slice header (including syntax elements other than picture identification).

A few open issues are listed below, in the places that suit the contexts, highlighted in yellow, or included in Word comments. These open issues are to be resolved over the JCT-VC reflector after this JCT-VC meeting, preferably within the two-week editing period.

# Procedure for using the common conditions

This document specifies the common test conditions for the following types of input contributions:

- Signaling of picture identification in slice header (similar to picture order count)

- Signaling of reference picture set

- Signaling of reference picture lists (including reference picture list modification commands, if present), including lists 0, 1, and the combined list.

- Signaling of decoded reference picture marking, if present

The input contributions using these common conditions shall report the bit count used for all related syntax elements and structures in sequence parameter set, picture parameter set, adaptation parameter set, and slice header.

The subsequent sections specify the prediction structures for which the bit counts shall be reported. Many of the prediction structures are described below in a table form. One row in the table corresponds to one frame. The rows are in decoding order.

In addition, the input contributions using these common conditions

- should verify that the proposed technology also operates on temporal subsets of the bitstreams corresponding to the prediction structures in the following sections

- should analyze the error robustness, for example assuming a maximum of two consecutive entirely lost pictures having the same temporal_id [Open issue: this should only apply for low-delay prediction structures]

# High-delay prediction structures

The reference picture lists and pictures marked as "used for reference" for the high-delay prediction structures differ from the first “random access cycle” of a coded video sequence (from the beginning of the sequence starting with an IDR access unit to the next IDR/CRA access unit, exclusive) and for each subsequent random access cycle (from a CRA access unit, inclusive, to the next IDR/CRA access unit, exclusive) in the same coded video sequence. The bit count overhead shall be reported separately for the first random access cycle and a subsequent random access cycle.

For high-delay prediction structures, results of all the following slicing methods should be applied:

- 1 slice per picture

- 2 slices per picture (evenly partitioned)

- 4 slices per picture (evenly partitioned)

[Open issue: For a particular test sequence, the lowest QP as required to be tested per the common conditions as specified in JCTVC-Gxxxx shall be used.]

## Random access common conditions

[It would be nice to write down the RA common conditions in a table like the ones below.]

## Dyadic and nested temporal scalability with 1 reference frame per list

GOP size 8 is used. Random access points (RAPs) are coded in roughly every one second. For example, if the frame rate is 30 fps, then a CRA picture is coded for every 4 GOPs, and if the frame rate is 60 fps, a CRA picture is coded for every 8 GOPs.

This structure may be used by encoders which are unable to estimate motion from more than one reference frame per list, when temporal scalability is desired, and temporal layer switching is desired at any access unit.

Results for both structures as shown in Table 1 and Table 2 shall be provided.

Table 1

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 8 | B | 1 | 0 | 0 | 0 | - |
| 4 | B | 1 | 1 | 0 | 8 | - |
| 2 | B | 1 | 2 | 0 | 4 | 8 |
| 1 | B | 0 | 3 | 0 | 2 | 4, 8 |
| 3 | B | 0 | 3 | 2 | 4 | 8 |
| 6 | B | 1 | 2 | 4 | 8 | - |
| 5 | B | 0 | 3 | 4 | 6 | 8 |
| 7 | B | 0 | 3 | 6 | 8 | - |
| 16 | CRA | 1 | 0 | - | - | 8 |
| 12 | B | 1 | 1 | 8 | 16 | - |
| 10 | B | 1 | 2 | 8 | 12 | 16 |
| 9 | B | 0 | 3 | 8 | 10 | 12, 16 |
| 11 | B | 0 | 3 | 10 | 12 | 16 |
| 14 | B | 1 | 2 | 12 | 16 | - |
| 13 | B | 0 | 3 | 12 | 14 | 16 |
| 15 | B | 0 | 3 | 14 | 16 | - |
| 24 | B | 1 | 0 | 16 | 16 | - |
| … |  |  |  |  |  |  |

Table 2 (Note: the difference here compared to in Table 1 is that herein all pictures with the same temporal_id are continuous in decoding order)

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 8 | B | 1 | 0 | 0 | 0 | - |
| 4 | B | 1 | 1 | 0 | 8 | - |
| 2 | B | 1 | 2 | 0 | 4 | 8 |
| 6 | B | 1 | 2 | 4 | 8 | - |
| 1 | B | 0 | 3 | 0 | 2 | 4, 8 |
| 3 | B | 0 | 3 | 2 | 4 | 8 |
| 5 | B | 0 | 3 | 4 | 6 | 8 |
| 7 | B | 0 | 3 | 6 | 8 | - |
| 16 | CRA | 1 | 0 | - | - | 8 |
| 12 | B | 1 | 1 | 8 | 16 | - |
| 10 | B | 1 | 2 | 8 | 12 | 16 |
| 14 | B | 1 | 2 | 12 | 16 | - |
| 9 | B | 0 | 3 | 8 | 10 | 12, 16 |
| 11 | B | 0 | 3 | 10 | 12 | 16 |
| 13 | B | 0 | 3 | 12 | 14 | 16 |
| 15 | B | 0 | 3 | 14 | 16 | - |
| 24 | B | 1 | 0 | 16 | 16 | - |
| … |  |  |  |  |  |  |

## Dyadic and nested temporal scalability with 2 reference frames per list

GOP size 8 is used. RAPs are coded in roughly every one second. For example, if the frame rate is 30 fps, then a CRA picture is coded for every 4 GOPs, and if the frame rate is 60 fps, a CRA picture is coded for every 8 GOPs.

This structure may be used by encoders which are unable to estimate motion from more than two reference frames per list, when temporal scalability is desired, and temporal layer switching is desired at any access unit.

[Open issue: It might be good to allow for a reference picture with the same temporal_id to be included in either or both of RefPicList0 and RefPicList1. For example, for picture with POC 14, the reference picture with POC 10 may be included.]

Results for both structures as shown in Table 3 and Table 4 shall be provided.

Table 3

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 8 | B | 1 | 0 | 0 | 0 | - |
| 4 | B | 1 | 1 | 0, 8 | 8, 0 | - |
| 2 | B | 1 | 2 | 0, 4 | 4, 8 | - |
| 1 | B | 0 | 3 | 0, 2 | 2, 4 | 8 |
| 3 | B | 0 | 3 | 2, 0 | 4, 8 | - |
| 6 | B | 1 | 2 | 4, 8 | 8, 4 | - |
| 5 | B | 0 | 3 | 4, 6 | 6, 8 | - |
| 7 | B | 0 | 3 | 6, 8 | 8, 6 | - |
| 16 | CRA | 1 | 0 | - | - | 8 |
| 12 | B | 1 | 1 | 8, 16 | 16, 8 | - |
| 10 | B | 1 | 2 | 8, 12 | 12, 16 | - |
| 9 | B | 0 | 3 | 8, 10 | 10, 12 | 16 |
| 11 | B | 0 | 3 | 10, 8 | 12, 16 | - |
| 14 | B | 1 | 2 | 12, 16 | 16, 12 | - |
| 13 | B | 0 | 3 | 12, 14 | 14, 16 | - |
| 15 | B | 0 | 3 | 14, 16 | 16, 14 | - |
| 24 | B | 1 | 0 | 16 | 16 | - |
| … |  |  |  |  |  |  |

Table 4 (Note: the difference here compared to in Table 3 is that herein all pictures with the same temporal_id are continuous in decoding order)

| POC | pic type | nal_ref_flg | t_id | Reference picture list 0 | Reference picture list 1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 8 | B | 1 | 0 | 0 | 0 | - |
| 4 | B | 1 | 1 | 0, 8 | 8, 0 | - |
| 2 | B | 1 | 2 | 0, 4 | 4, 8 | - |
| 6 | B | 1 | 2 | 4, 8 | 8, 4 | - |
| 1 | B | 0 | 3 | 0, 2 | 2, 4 | 8 |
| 3 | B | 0 | 3 | 2, 0 | 4, 8 | - |
| 5 | B | 0 | 3 | 4, 6 | 6, 8 | - |
| 7 | B | 0 | 3 | 6, 8 | 8, 6 | - |
| 16 | CRA | 1 | 0 | - | - | 8 |
| 12 | B | 1 | 1 | 8, 16 | 16, 8 | - |
| 10 | B | 1 | 2 | 8, 12 | 12, 16 | - |
| 14 | B | 1 | 2 | 12, 16 | 16, 12 | - |
| 9 | B | 0 | 3 | 8, 10 | 10, 12 | 16 |
| 11 | B | 0 | 3 | 10, 8 | 12, 16 | - |
| 13 | B | 0 | 3 | 12, 14 | 14, 16 | - |
| 15 | B | 0 | 3 | 14, 16 | 16, 14 | - |
| 24 | B | 1 | 0 | 16 | 16 | - |
| … |  |  |  |  |  |  |

## Dyadic and nested temporal scalability with 2 reference frames per list and higher quality references (semi-long term)

GOP size 8 is used. A CRA picture is coded for every 8 GOPs. Higher quality reference are kept in the buffer for a longer time and used for predicting the key pictures to reduce the rate of decay in the quality of the pictures.

Table 5

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 8 | B | 1 | 0 | 0 | 0 | - |
| 4 | B | 1 | 1 | 0, 8 | 8, 0 | - |
| 2 | B | 1 | 2 | 0, 4 | 4, 8 | - |
| 1 | B | 0 | 3 | 0, 2 | 2, 4 | 8 |
| 3 | B | 0 | 3 | 2, 0 | 4, 8 | - |
| 6 | B | 1 | 2 | 4, 0 | 8, 4 | - |
| 5 | B | 0 | 3 | 4, 0 | 6, 8 | - |
| 7 | B | 0 | 3 | 6, 4 | 8, 6 | - |
| …. |  |  |  |  |  |  |
| 48 | B | 1 | 0 | 40, 32, 24, 16 | 40, 32, 24, 16 | - |
| 44 | B | 1 | 1 | 40, 32 | 48, 40 | 24 |
| 42 | B | 1 | 2 | 40, 32 | 44, 48 | 24 |
| 41 | B | 0 | 3 | 40, 32 | 42, 44 | 24, 48 |
| 43 | B | 0 | 3 | 42, 40 | 44, 48 | 24, 32 |
| 46 | B | 1 | 2 | 44, 40 | 48, 44 | 24, 32 |
| 45 | B | 0 | 3 | 44, 40 | 46, 48 | 24, 32 |
| 47 | B | 0 | 3 | 46, 44 | 48, 46 | 24, 32, 40 |
| … | B | 1 | 0 |  |  |  |
| 64 | CRA | 1 | 0 | - | - | 56, 48, 40 |

## Dyadic and nested temporal scalability for high frame rate

Modern digital still and video cameras can capture at high frame rates, such as 240 Hz, thus enabling impressive slow motion capturing. It is advantageous if such high-frame-rate sequences are coded in a temporally scalable manner in order to enable sub-bitstream extraction and playback at different speeds and with different decoder capabilities.

Dyadic and nested temporal scalability and reference picture lists are as in Section 2.2, but GOP size is 64 and temporal_id values from 0 to 7, inclusive, are allocated. GOP size 64 at frame rate 240 Hz provides that same interval for consecutive pictures having temporal_id equal to 0 than GOP size 8 at frame rate 30 Hz.

## Long-term referenece picture use in high-delay prediction structures

This test simulates interviewing scenarios, wherein two or more sides are talking, the video to be shown switches between the sides, from a few tens of seconds to a few minutes.

Results of the following two set of tests in this category should be reported.

In the first test set, the following configurations apply:

- The first scene (the interviewer) lasts 20 seconds, followed by the second scene (the interviewee) that lasts 60 seconds, in output order, and the pattern repeats in the same manner.

- When encoding the first picture of one scene after the period of another scene, in output order, the last picture, in output order, of the same scene with the same or lower temporal_id is used for reference, e.g., as a long-term reference picture. However, it is also allowed to use only short-term reference pictures, at the discretion of the proponents.

In the second test set, the same configuration as in the first test set, but with the first scene period being 10 seconds and the second scene being 30 seconds.

# Low-delay prediction structures

For low-delay prediction structures, the 1500 bytes per slice encoding configuration shall be applied.

For a particular test sequence, the highest QP as required to be tested per the common conditions as specified in JCTVC-Gxxxx shall be used.

## Low-delay common conditions

[To be described.]

## Low-delay temporal scalability

This prediction structure enables low-delay and use of system-layer error robustness methods unequally per temporal_id.

Table 6

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | IDR | 1 | 0 | - | - | - |
| 1 | P | 0 | 2 | 0 | - | - |
| 2 | P | 1 | 1 | 0 | - | - |
| 3 | P | 0 | 2 | 2 | - | 0 |
| 4 | P | 1 | 0 | 0 | - | - |
| 5 | P | 0 | 2 | 4 | - | - |
| 6 | P | 1 | 1 | 4 | - | - |
| 7 | P | 0 | 3 | 6 | - | 4 |
| 8 | P | 1 | 0 | 4 | - | - |
| … |  |  |  |  |  |  |

## Low-delay with feedback

In this prediction structure it is assumed that the encoder gets feedback from the far-end decoder on the success of decoding. Based on the feedback, the encoder maintains two “key” frames, which are marked as “used for reference”. In addition, two most recent frames are used as reference.

The use case is simulated with the following prediction structure, where Ni = {[include a pseudo-randomly generated increasing series of POCs here, e.g. 6 items]} and Mi = {[ Ni - Ri where Ri is a pseudo-random value]}.

Table 7

| POC | pic type | nal_ref_flg | t_id | RefPicList0 | RefPicList1 | Other pictures marked as “used for reference” |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| Ni | P | 1 | 0 | Ni-1, Ni-2, Mi, Mi-1 |  |  |
| Ni+1 | P | 1 | 0 | Ni, Ni-1, Mi, Mi-1 |  |  |
| Ni+2 | P | 1 | 0 | Ni+1, Ni, Mi, Mi-1 |  |  |
| … |  |  |  |  |  |  |

The delay between the time when a particular picture is captured at the encoder side and when the encoder side receives the feedback is assumed to be equal to the round trip time (RTT) plus one frame during (this is the minimum, depending on other factors, the value can be higher, but using the minimum should be reasonable). Results of two RTT values, 200 ms (representing a pretty good connection) and 1000ms (representing a not-so-good connection), should be reported. It is at the discretion of the proponents on whether it is desirable to use long-term reference pictures or not.

When the RTT is set to a very high value, this configuration can also be used to simulate the use case wherein a picture of a background of a telepresence conference room, which is marked as a long-term reference picture at some point (e.g., after the distance between this picture and the current picture is beyond the maximum POC distance for short-term reference pictures), and which stays a very long time in the DPB, for efficient coding of uncovered background regions, and for error recovery. Due to that the length of the sequences is relatively short (less than 10 minutes), for this use case, it is assumed that the “background” long-term reference picture, once available, is kept throughout the entire test sequence.

# Additional test sets

The following additional test sets are to be finalized, and then included in either or both of Sections 2 and 3.

[Open issue: Add a test set for explicit adaptive reference picture selection in the slice header. Possibly use random seeds. A sub open issue here is to decide whether this is for high-delay or low-delay prediction structures, or both.]

[Open issue: Add a test set to enable using of reference pictures with the same temporal_id, e.g., by simply using the current config file for RA and just add the temporal_id values, as shown below:

#        Type POC QPoffset QPfactor temporal_id ref_buf_size ref_pic #ref_pics reference pictures

Frame1:  B    8   1        0.442    0           4            1       4         -8 -10 -12 -14

Frame2:  B    4   2        0.3536   1           2            1       3         -4 -6 4

Frame3:  B    2   3        0.3536   2           2            1       4         -2 -4 2 6

Frame4:  B    6   3        0.3536   2           2            1       5         -2 -4 -6 -8 2

Frame5:  B    1   4        0.68     3           2            0       6         -1 -3 1 3 5 7

Frame6:  B    3   4        0.68     3           2            0       5         -1 -3 1 3 5

Frame7:  B    5   4        0.68     3           2            0       4         -1 -3 1 3

Frame8:  B    7   4        0.68     3           2            0       4         -1 -3 -5 1

which gives this:

POC	type	ref_idc	t_id	L0	L1

0	IDR	1	0	-	-

8	B	1	0	0	0

4	B	1	1	0 8	8 0

2	B	1	2	0 4	4 8

6	B	1	2	4 2	8 4

1	b	0	3	0 2	2 4

3	b	0	3	2 0	4 6

5	b	0	3	4 2	6 8

7	b	0	3	6 4	8 6

16	B	1	0	8	8

12	B	1	1	8 16	16 8

10	B	1	2	8 6	12 16

14	B	1	2	12 10	16 12

9	b	0	3	8 6	10 12

11	b	0	3	10 8	12 14

13	b	0	3	12 10	14 16

15	b	0	3	14 12	16 14

24	B	1	0	16	16

]