| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San Jose, US, 01-10 Feb 2012 | Document: JCTVC-H_Notes_d3 |
| --- | --- |

| Title: | Meeting report of the eighth meeting of the Joint Collaborative Team on Video Coding (JCT-VC), San Jose, US, 01-10 Feb. 2012 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11 held its eighth meeting during 01-10 Feb 2012 at the XXXX in San Jose, US. The JCT-VC meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found in section 1.13 of this document.

The JCT-VC meeting sessions began at approximately XXXX hours on Wednesday 01 Feb 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 10 Feb. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project now referred to as High Efficiency Video Coding (HEVC).

The primary goals of the meeting were to review the work that was performed in the interim period since the sixth JCT-VC meeting in implementing the 5th HEVC Test Model (HM5) and editing the 5th HEVC specification Working Draft (WD5), review the results from interim Core Experiments (CE), review technical input documents, further develop Working Draft and HEVC Test Model (HM), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-VC produced three particularly important output documents from the meeting: the HEVC Test Model 6 (HM6), the HEVC specification Commitee Draft (CD), and a document specifying common conditions and software reference configurations for HEVC coding experiments. Moreover, XX documents describing the planning of future CEs were drafted.

For the organization and planning of its future work, the JCT-VC established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 30 April – 8 May 2012 under ITU-T auspices in Geneva, CH, 11–20 July 2012 under WG 11 auspices in Stockholm, SE, and 10–19 Oct 2012 under WG 11 auspices in Suzhou, CN. Jan 2013

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics (update)

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventh meeting during 21–30 Nov 2011 at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr. Gary Sullivan (Microsoft/USA) and Dr. Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately XXXX hours on Wednesday 01 Feb 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 10 Feb. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project now referred to as High Efficiency Video Coding (HEVC).

Some statistics for historical reference purposes:

- 1st meeting (Dresden):		188 people, 40 input documents

- 2nd meeting (Geneva):		221 people, 120 input documents

- 3rd meeting (Guangzhou):		244 people, 300 input documents

- 4th meeting (Daegu):		248 people, 400 input documents

- 5th meeting (Geneva):		226 people, 500 input documents

- 6th meeting (Torino):		254 people, 700 input documents

- 7th meeting (Geneva)		284 people, 1000 input documents

- 8th meeting (San Jose)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2011_11_G_Geneva/JCTVC-G_Logistics.doc.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the seventh JCT-VC meeting in producing the 5th HEVC Test Model (HM) software and editing the 5th HEVC specification Working Draft (WD4), review the results from interim Core Experiments (CEs), review technical input documents, and establish the Commitee Draft (CD) and the sixth version of HEVC Test Model (HM6).

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:". Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 20 Jan 2012.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Saturday Jan 21 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

The following documents that did not arrive until the last two days of the meeting were not presented or discussed due to time constraints: JCTVC-Hxxx, … .

All contribution documents with registration numbers JCTVC-H572 to JCTVC-Hxxx were registered after the "officially late" deadline (and therefore were also uploaded late), except JCTVC-H666...H668, which were occasionally registered out of the regular numbering sequence. Some documents in the “572+” range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

The following other technical proposal contributions were registered in time but were uploaded late:

- JCTVC-Hxxx (a technical proposal) [uploaded xx-xx]

- …

The following other document not proposing normative technical content was registered in time but uploaded late:

- JCTVC-Hxxx (a study on screen content test sequences)

The following cross-verification reports were uploaded late: JCTVC-Gxxx, … .

The following document registrations were later cancelled or otherwise never discussed or provided: JCTVC-Hxxx, … .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

Remark: Also a problem are documents that are substantially revised after the initial upload, which becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion.

Remark: This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" and were not corrected until after the upload deadline:

- JCTVC-Hxxx (a cross-verification report, corrected 2011-xx-xx)

- …

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

### Measures to facilitate the consideration of contributions

It was agreed that, due to the increasingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report JCTVC-G1100, the HEVC Test Model (HM) JCTVC-G1102, and the Working Draft (WD) JCTVC-G1103, were approved. The HM reference software produced by the AHG on software development and HM software technical evaluation was also approved.

qq The group was asked to review the prior meeting report for finalization.

Versions of the WD, the HM document, the HM software and the CE descriptions had been made available in a reasonably timely fashion.

The chair asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

Remark: In regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

Remark: The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it has previously been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that in the case of CE11, there was one design change that was to be tested that was only vaguely described in concept in the CE plan document, and there was some discussion on the reflector to resolve the situation.

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered as result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- AIS: Adaptive intra smoothing.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- APS: Adapation parameter set.

- AMVR: Adaptive motion vector resolution.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- BUDI: Bidirectional UDI.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CE: Core experiment – a coordinated experiment conducted after the 3rd or 4th meeting.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIF: Directional interpolation filter.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ET: Encoding time.

- GPB: Generalized P/B – a not-particularly-well-chosen name for B pictures in which the two reference picture lists are identical.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- JM: Joint model – the primary software codebase developed for the AVC standard.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B frames.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LCEC: Low-complexity entropy coding.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- MC: Motion compensation.

- MDDT: Mode-dependent directional transform.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-VC).

- MRG: block merging mode for CUs.

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PIPE: Probability interval partitioning entropy coding (roughly synonymous with V2V for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QP: Quantization parameter.

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RPLM: Reference picture list modification.

- ROT: Rotation operation for low-frequency transform coefficients.

- RQT: Residual quadtree.

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SPS: Sequence parameter set (as in AVC).

- TE: Tool Experiment – a coordinated experiment conducted after the 1st or 2nd JCT-VC meeting.

- TM: Test Model – a video coding design containing selected coding tools; as contrasted with the TMuC, see HM.

- TMuC: Test Model under Consideration – a video coding design containing selected proposed coding tools that are under study by the JCT-VC for potential inclusion in the HEVC standard.

- TPE: Transform precision extension.

- UDI: Unified directional intra.

- Unit types:

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- V2V: variable-length to variable-length prefix coding (roughly synonymous with PIPE for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

No particular non-routine opening remarks were recorded.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm.

Note: Counts may not be 100% precise

- AHG reports (22) Track P (section 2)

- Project development, status, and guidance (0) Track P (section 3)

- CE1: Entropy coding investigation (19) Track B (section 4.1)

- CE2: Chroma RQT depth (9) Track B (section 4.2)

- CE3: Chroma interpolation filter (4) Track A (section 4.3) [done]

- CE4: Quantization (27) Track B (section 4.4)

- CE5: Transform skipping (15) Track B (section 4.5)

- CE6: Intra prediction improvement (24) Track B (section 4.6)

- CE7: Additional transforms (10) Track B (section 4.7)

- CE8: Non-deblocking loop filtering (33) Track A (section 4.8) [Subj. Viewing]

- CE9: MV coding and skip/merge operation (9) Track A (section 4.9) [done]

- CE10: Deblocking filter (31) Track A (section 4.10) [Subj. Viewing]

- CE11: Coefficient scanning and coding (27) Track B (section 4.11)

- Clarifications and bug fix issues (1) Track P (section 5.1)

- HM settings and common test conditions (0) Track P (section 5.2)

- HM coding performance (3) Track P (section 5.3)

- Profile/level definitions (5) Track P (section 5.4)

- Source video test material (2) Track P (section 5.5)

- Functionalities (8) Track A (section 5.6)

- Deblocking filter (10) Track A (section 5.7)

- Non-deblocking loop filters (32) Track A (section 5.8)

- NSQT/SDIP harmonization (8) Track B (section 5.9)

- Block structures and partitioning (11) Track A (section 5.10)

- Motion compensation operation and interpolation filters (8) Track A (section 5.11)

- Motion and mode coding (53) Track A (section 5.12) [BoG B. Bross – done]

- High-level syntax and slice structure (66) Track A (section 5.13) [Current in Track A]

- Quantization (25) Track B (section 5.14)

- Entropy coding (18) Track B (section 5.15)

- Transform coefficient coding (49) Track B (section 5.16)

- Intra prediction and mode coding (46) Track B (section 5.17)

- Transforms (4) Track B (section 5.18)

- Memory bandwidth reduction (11) Track A (section 5.19)

- Alternative coding modes (27) Track B (section 5.20)

- Encoder optimization (2) Track A (section 5.21)

- Category not clear (13 to be resolved) (section 5.23)

Overall Track P: 33; Track A: 287; Track B: 299

# AHG reports

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

JCTVC-H0001 JCT-VC AHG Report: JCT-VC Project Management (AHG 1) [G. J. Sullivan, J.-R. Ohm (AHG chairs)] [miss]

Discussed verbally prior to upload. All documents delivered. No critical issues.

JCTVC-H0002 JCT-VC AHG report: HEVC Draft and Test Model editing (AHG 2) [B. Bross, K. McCann, W.-J. Han, J.-R. Ohm, S. Sekiguchi, G. J. Sullivan, T. Wiegand]

The fifth High Efficiency Video Coding (HEVC) test model (HM5) was developed from the fourth HEVC test model (HM4), following the decisions taken at the 7th JCT-VC meeting in Geneva (21-30 November, 2011).

Two editorial teams were formed to work on the two documents that were to be produced:

JCTVC-G1102 HEVC Test Model 5 (HM 5) Encoder Description

- Il-Koo Kim

- Shun-ichi Sekiguchi

- Benjamin Bross

- Woo-Jin Han

- Ken McCann

JCTVC-G1103 WD5: Working Draft 5 of High Efficiency Video Coding [2]

- Benjamin Bross

- Woo-Jin Han

- Jens-Rainer Ohm

- Gary J. Sullivan

- Thomas Wiegand

Editing JCTVC-G1103 was assigned a higher priority than editing JCTVC-G1102.

An issue tracker (http://hevc.kw.bbc.co.uk/trac) was created in order to facilitate the reporting issues on the text of both documents.

One draft of JCTVC-G1102 and nine drafts of JCTVC-G1103 were published by the Editing AHG between the 7th JCT-VC Meeting in Geneva (21-30 November, 2011) and the 8th JCT-VC Meeting in San José (1-10 February, 2012).  JCTVC-G1102 still needs further improvement, whilst the final draft of JCTVC-G1103 is reasonably complete.

The recommendations of the HEVC Draft and Test Model Editing AHG were to:

- Approve the edited JCTVC-G1102 and JCTVC-G1103 documents as JCT-VC outputs.

- Continue to edit both documents to ensure that all agreed elements of HEVC are fully described.

- Encourage the use of the issue tracker (http://hevc.kw.bbc.co.uk/trac) to facilitate the reporting of issues with the text of either document.

- Compare the HEVC documents with the HEVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the HEVC WD, to allow it to proceed to CD ballot shortly after the San José meeting.

- Ensure that, when considering the addition of new tools to HEVC, properly drafted text for addition to both the HEVC Draft and the HM Test Model (if appropriate) is made available in a timely manner.

It was suggested that WG11 issue an abbreviated (2 month) CD ballot at this meeting.

Discussion about timeline: Suggestion to target short CD ballot with option to issue DIS in May. This would allow a longer editing period for DIS, and also have the advantage that the DIS ballot would arrive in advance to the January 2013 meeting. This requires a short editing period for CD (weekend after the Feb 2012 meeting).

Regarding HRD, although the specification may not be complete in the current text, in concept it is intended to be essentially the same as in AVC at this point. The SEI message status, in principle, has been previously documented. Byte stream formatting is already documented, except perhaps for mentioning APS start codes, which should be handled the same as PPS and SPS start codes. This was agreed. Editing work during the meeting to improve the text was encouraged.

JCTVC-H0003 JCT-VC AHG report: Software development and HM software technical evaluation (AHG 3) [F. Bossen, D. Flynn, K. Suehring] [miss]

This was discussed verbally prior to upload.

More items needed to be integrated than ever before, nevertheless HM5.0 was issued before Christmas, and HM5.1 two weeks ago. Everything integrated currently. Comparison against previous HM4 performance in case of LC highly influenced by removal of CAVLC; for HE losses were expected due to removal of 10 bit precision in default condition; in test cases where 10 bit were retained, still gain (roughly 1%) was observed. Post-integration testing of single tools was done, but not strictly evaluated.

Regarding retesting of individual tools, this was not always done to the extent that it was intended.

In one case there was actually an accidental benefit in coding efficiency that occurred.

However, the overall result seemed to be about what was expected.

One issue to be observed is that even if every tool implementation is OK under common test conditions, crashes and other issues may occur when tested under non-common conditions (e.g. slice implementation). Investigation has been started in more detail. It is important to avoid problems in cases outside the common conditions, such as when slices are used.

The use of so-called Hungarian notation for variable names is now discouraged.

Difficult areas of the software were suggested to include aspects related to the high-level basic structuring of information (i.e. slices, entropy slices, tiles, and wavefronts), and ALF (which was the subject of work on AHG6, resulting in a software attachment to that report). General cleanup of unnecessary macro-dependent software was requested (e.g. remnants of CAVLC).

It was requested for software modifications to be supplied as patch/difference files rather than as copies of the whole codebase, to make it easier to find and inspect the changes.

JCTVC-H0004 JCT-VC AHG report: Picture Partitioning and LCU scan order (AHG4) [R. Sjöberg (AHG chair), Y. Chen, F. Henry, M. Horowitz, K. Kazui, A. Segall, W. Wan (vice chairs)]

Slice implementation was broken in HM5.0, but has been fixed in 5.1.

WD tickets #263 and #264 reported a number of issues with tiles and wavefronts. Those were discussed on the reflector, in particular restrictions on the entropy_coding_synchro syntax element. The conclusion was to discuss this further at the meeting, input documents JCTVC-H0349 and JCTVC-H0517 are related.

HM software slice support was broken in HM-5.0 which caused problems for some JCT-VC sub-groups, e.g. CE8 and AHG21. Tickets #225, #256, #257, #258 and #286 describe the problems. These tickets were all addressed and slices now works fine in HM-5.1.

The coding efficiency of 1500 byte slices was tested for HM-5.0-dev-bugfix. The overhead of 1500 byte slices was reported in the AHG report as being similar to earlier HM versions.

A substantial number of contributions related to this AHG were noted.

JCTVC-H0005 JCT-VC AHG Report: Spatial Transforms (AHG 5) [P. Topiwala (AHG Chair), M. Budagavi, R. Cohen, R. Joshi (vice chairs)]

The purpose of AhG is to investigate the design and application of transforms in HEVC.  This includes the architectural improvements of the conventional block integer DCT-like transforms, as well as alternative transforms proposed for use in intra coding. This AhG addresses the design of transforms that achieve high performance, but with reasonable complexity.  Complexity is measured not only in terms of operations count, but memory access impact and software and hardware implementation convenience.

The following tasks were performed in this AhG:

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency vs. complexity of proposed designs according to test conditions;

- Analyze complexity in terms of operations, memory access, and hardware implementability ease;

- Report the results and conclusions of these experiments.

The activity in this AhG was mainly focused on the CE7 work on additional primary and secondary transforms. Only one document, not yet uploaded, was on the topic of core transforms. The additional transform work is covered in detail in the CE7 Summary Report. Several discussions took place via email among CE7 participants regarding how these transforms worked, along with updates and corrections to documents being submitted for this JCT-VC meeting. An overview of transform-related documents for CE7 is provided in the tables below.

Relevant contributions were noted.

[Tables in report]

JCTVC-H0006 JCT-VC AHG report: In-loop filtering (AHG 6) [T. Yamakage (AHG chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin (vice chairs)]

[Add additional notes]

Code cleanup work was a significant focus.

It was remarked that the decoder software still seems to need a lot of work, whereas this AHG was focused primarily on the encoder software.

There was an investigation on one-pass coding – however, this resulted in a substantial loss (1.6%) compared to 14-pass.

JCTVC-H0007 JCT-VC AHG report: Memory bandwidth restrictions in motion compensation (AHG 7) [T. Suzuki (Sony)] [miss]

The proposed constraints to ensure MC memory bandwidth can be classified as follows.

- Defining a VMV (Virtual Bandwidth Verifier) and requiring the encoder to ensure not to exceed VMV. The constraint is applied at picture level. (JCTVC-H0089)

- Specific constraints on prediction mode, 2D interpolation, the number of MVs

  - Restricting bi-pred for 4x8, 8x4 and/or 8x8 depending on PU/MinCUSize (JCTVC-H0096)

  - Restricting to uni-pred only for 4x8 and 8x4 rather than restricting on the number of MVs (JCTVC-H0441)

  - Replacing bi-pred of merge candidate with uni-pred for 4x4, 4x8, 8x4 and 8x8 (JCTV-H0221)

  - One MV for L0/L1 being restricted to integer pel for 4x8 and 8x4 (JCTVC-H0181)

  - Restricting the number of MVs per LCU rather than block size based mode restrictions (JCTVC-H0267)

  - Restricting both the number of MVs and uni/bi-pred (JCTVC-H0600)

The impact on coding efficiency and complexity should be considered as well as reduction level of MC memory bandwidth. Good trade off should be identified.

Related contributions were noted. Some proposals would specify and encoder-side constraint, while others would specify modifications of the decoder operation.

JCTVC-H0008 JCT-VC AHG report: Profile and level definitions (AHG 8) [M. Horowitz, K. McCann (co-chairs), T. Suzuki, T. K. Tan, W. Wan, Y.-K. Wang (vice chairs)]

Relevant input contributions were reviewed.

Specification of a "Main" profile was discussed in the report.

Contribution JCTVC-H0168 suggested the following constraints for the Main Profile:

- LCU size: Max 64x64, Min 16x16

- Slice constraints: Min slice granularity of LCU size and each slice starts from LCU boundary

- Max number of reference frames: max value of 4 (maybe 5, if necessary)

- Number of PS: Max SPS of 8, Max PPS of 16, Max APS of 8

- Parallel processing: Constraints on tiles

An alternative approach would be to define two profiles to cover mainstream requirements, such as the Baseline Profile and High Profile proposed in JCTVC-H0353.

JCTVC-H0116 was noted to be rich in analysis, particularly focused on RDOQ, ALF, SAO, LMChroma, AMP, and NSQT.

It was noted that we may not want what seemed to be envisioned in the original requirements that was talking about one “high efficiency” and one “low complexity” operational point.

JCTVC-H0009 JCT-VC AHG report: Entropy Coding Architecture (AHG 9) [K. McCann (chair), J. Lainema, D. Marpe, A. Segall, K. Sugimoto, V. Sze, W. Wan, X. Wang (vice chairs)]

There were 4 email messages relating to the work of AHG9 exchanged on the reflector.

The difficulty of measuring throughput was identified. One option is to count the coded bins per pixel, but it was recognised that this does not take into account differences in the binarization schemes.

No better measurement was identified, so expert judgement probably remains the most important criterion for characterizing throughput.

It was suggested that this AHG may not need to be re-established for the next meeting cycle.

JCTVC-H0010 JCT-VC AHG report: Parallel merge/skip (AHG 10) [M. Zhou (chair), H. Y. Kim, P. Onno, X. Wen (vice chairs)] [miss]

The general understanding of underlying motion estimation throughput issues is as follows:

- There is motion estimation throughput issue on the encoder side due to dependency of merge/skip MVP list (MCL) derivation on the regular motion estimation, if an encoder chooses to leverage full quality potential of HEVC merge/skip mode.

- Excessive amount of MCL derivation required for motion estimation worsens the motion estimation throughput issue.

General design goals of parallel merge/skip are:

- Decouple the merge/skip MVP list derivation process and merge motion estimation from the regular motion estimation, so that both threads can run fully parallel

- Reduce the number of merge/skip MVP list derivation

- Remove remaining dependency at CU and SCU level

- Configurability to enable flexible coding efficiency and throughput trade-offs on the encoder side.

The relevant issues were explained in the report and the relevant contributions were noted.

JCTVC-H0011 JCT-VC AHG Report: Video test material selection (AHG11) [T. Suzuki] [miss]

A problem of the current class E test sequences was reported in JCTVC-G732 in Geneva. ”De-interlace” artifact was observed in the current class E. The effort to generate new class E test materials is continued. When the materials are ready, it should be checked during JCT-VC San Jose meeting and should be study the common test conditions. It was remarked that JCTVC-H0389 is relevant.

There are two contributions on test material (see section 5.5).

It was also remarked that JCTVC-H0294 is relevant (describing 4:4:4 video for screen content coding).

JCTVC-H0012 JCT-VC AHG report: Objective quality metric and alternative methods for measuring coding efficiency (AHG12) [K. Minoo, G. Sullivan (co-chairs)] [miss]

For the first two mandates, on objective quality metrics there were some discussions on the reflector. These discussions mainly considered the suitability of different color spaces for the purpose of objective quality assessment and how a mean squared error metric should be modified by effective sub-sampling in different color components.

On the last two mandates (i.e. error/quality pooling for the purpose of reporting a single coding efficiency value) there was an email message sent to the reflector by the AHG12 chairs. The message (quoted below) described computing a multi-component (weighted or unweighted) average MSE in the YUV domain and converting this to the PSNR domain, rather than using our historical practice of computing signle-component PSNR values and then averaging these together in the PSNR domain.

In response, various issues were discussed on the e-mail thread. These included the following:

- Adjustments for 4:2:0 versus 4:4:4 or 4:2:2 sampling

- Potentially measuring error in the RGB domain rather than the YUV domain (and the extent to which these may be equivalent, and clipping effects).

- How to select MSE weighting values for individual components (e.g., weighting G more in an RGB measurement, picking weights based on perceptual importance, or picking weights based on slopes of rate-distortion curves)

One relevant input document was noted, JCTVC-H0063.

During the discussion of the AHG report, one suggestion was to pipe the output of the decoder to a separate tool that computes one or more quality metrics.

The idea of adding an additional column of combined YUV metric information to the usual reported statistics had some support in the discussion.

JCTVC-H0013 JCT-VC AHG report: Interlace indication (AHG13) [K. Chono (chair), C. Fogg, K. Sato, S. Sekiguchi, W. Wan (vice chairs)]

A SEI message based on H.263 Annex W was defined according to the last meeting decisions. The starting text specifying the VUI flag and SEI message was distributed to the JCTVC main reflector on December 22. The text was uploaded in a revision of the AHG report.

The following opinions were expressed on the starting text (or interlace indication):

- Move the VUI flag to SPS in order to simplify the signaling of the type of a coded sequence;

- Reserve more bits of the field of the SEI message to indicate other ways of interpreting the decoded arrays in addition to interlace;

- Perhaps we need to define some stronger means than an SEI message to convey this kind of information to avoid "external hacks". The file format, for example, needs to specify the presence of the SEI message;

- AVC style indication (conditional signaling of bottom_field_flag in slice headers) can be such a stronger means but it uses normative bits.

In discussion of the draft, one question was why a value was identified as "unspecified" rather than "reserved". This seemed to be an error.

The AHG report also described some of the remarks made in email on the JCT-VC reflector discussing suggestions beyond interlace indication:

- Define an unified sequence level SEI message that contains information at a sequence level about characteristics of the sequence (e.g., color space, progressive/interlace, etc). In the meeting discussion, it seemed that VUI is equivalent to this.

- Define an unified picture level SEI message that contains information at a picture level about characteristics of the picture (e.g., top/bottom indication for interlaced video, stereoscopic side-by-side/mono indication for frame-compatible stereo video, etc).

- Define a picture-level signalling means other than SEI message to indicate dynamically changing source material properties.

The AHG report discussed the relevant input contributions.

JCTVC-H0014 JCT-VC AHG Report: Loss robustness (AHG14) [S. Wenger (chair), M. Coban, Y. W. Huang, P. Onno, Y.-K. Wang (vice chairs)]

There was reportedly no directly-related AHG activity. Some relative input contributions were listed. The most directly-related identified contributions were listed as:

- JCTVC-H0072 (late) provides a loss simulation software.

- JCTVC-H0132 includes simulation results for a lost APS, and proposes an optimization method JCTVC-to deal with such losses.

- JCTVC-H0520 appears to propose an FMO-like picture segmentation mechanism on the tile level, facilitating error concealment.

- JCTVC-H0570 proposes a bit in the slice header to signal temporal MV prediction, which is claimed to be beneficial for error resilience.

Other related contributions were listed as:

- JCTVC-H0168 advocates, among many other things, that slices need to start at LCU boundaries (no FG slices) (which may create overhead when matching MTU size), and that tiles be independent (which may be advantageous from an error resilience viewpoint).

- JCTVC-H0345 advocates, among other things, independent tiles (only) through profiling which may be good for error resilience.

- JCTVC-H0348 proposes slices local to tiles only, which may make MTU size matching harder for some slices.

- JCTVC-H0463 advocates, through profiling, restrictions to tiles, especially number of columns (max 2), slices local to tiles, independent tiles only, and others.

- JCTVC-H0472 appears to advocate the use of tiles with limited column width (level dependent) to reduce line buffer requirements.

- JCTVC-H0449 and JCTVC-H0501 relate to error resilient derivation of POC values

- JCTVC-H0668 advocates, among other things, “no more than 4 coded slices” per picture with non-empty slice headers, which may make sending a sufficient number of redundant copies impossible.

- Numerous documents on (partial) APS updates, including JCTVC-H0069, JCTVC-H0070, JCTVC-H0255, JCTVC-H0381, JCTVC-H0505, JCTVC-H0512,

- Scalability stuff, including JCTVC-H0386, JCTVC-H0388, JCTVC-H0410, JCTVC-H0423)

JCTVC-H0015 JCT-VC AHG report: High-level syntax (AHG15) [Y. K. Wang (chair), J. Boyce, Y. Chen, M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan, P. Wu (vice chairs)]

One recommendation this AHG is making is on signaling of picture size, as concluded through mailing list discussions. The recommendation was to leave the signaling of decoded picture width and height as they are in the SPS in the latest HEVC WD, in units of luma samples, but to add a cropping process before a decoded picture is stored into the DPB, to make sure that the decoded picture size is exactly the same as signalled in the SPS. Review of relevant input contributions was desired before making a decision on this by the JCT-VC.

On the LCU concept, there was no clear conclusion. Among those who were involved in the discussion, more were inclined to add some clarification to the draft HEVC specification, such that an LCU can be incomplete, in terms that some pixels in the LCU may be outside the boundary of the decoded picture. This way, even when the picture width or height is not an integer number of LCU sizes, tiles, which are defined as consisting of an integer number of LCUs, may still be applied.

Related contributions and relevant work in other AHGs were listed.

JCTVC-H0485 advocates for the internal decoding process to operate with the granularity of the minimum CU size – and then for a cropping window to be applied outside the coding loop for output only (e.g. as in AVC). Decision: Agreed.

In discussion it was suggested to constrain the syntax-expressed width and height parameters (which express the width and height prior to application of the cropping window) to be an integer multiple of the minimum CU size. Decision: Agreed. (Note that this is a constraint, not a description of the units in which the width and height are expressed.)

In discussion it was suggested for the profile/level constraints to apply to the full DPB-stored width & height. Decision: Agreed.

In discussion the cropping was suggested to have the same type of flexibility of the cropping window as in AVC. Decision: Agreed.

In discussion it was suggested for the motion compensation padding to apply to the minimum CU size granularity boundary (e.g. as AVC). Decision: Agreed.

In principle, there may be LCUs at the right and bottom boundaries that are not fully occupied by samples in the decoded picture that is stored in the DPB. Decision: Agreed.

These agreements, at least to some extent, express what was already intended and is already reflected in the software operation.

JCTVC-H0016 JCT-VC AHG report: Unification of NSQT and SDIP (AHG 16) [X. Zheng (chair), J. Xu, X. Wang, A. Tabatabai, J. Lim (vice chairs)]

(Presented in Track B – NSQT/SDIP harmonization – section 5.9)

JCTVC-H0017 JCT-VC AHG Report: Hooks for scalable coding (AHG 17) [J. Boyce (chair), J. Kang, J. Samuelsson, W. Wan, Y.-K. Wang (vice chairs)] [miss]

(Track A – Functionalities)

JCTVC-H0018 JCT-VC AHG report: Resolution adaption (AHG 18) [T. Davies (chair), P. Topiwala, P. Wu (Vice-chairs)]

(Track A – Functionalities)

A kick-off email was sent on 4 January 2012. A brief discussion on the relationship between resolution adaption and reduced resolution update took place on the reflector. Related contributions were listed in the report.

JCTVC-H0019 JCT-VC AHG Report: Lossless coding (AHG19) [W. Gao (chair), K. Chono, J. Xu, M. Zhou, P. Topiwala (vice chairs)] [miss]

(Track B – Alternative coding modes)

JCTVC-H0020 JCT-VC AHG report: Chroma format support (AHG 20) [D. Flynn (chair), D. Hoang, K. McCann, P. Topiwala (vice chairs)] [miss]

(Track A – Functionalities)

Related topics and contributions included the following topics

- Hybrid coding (JCTVC-H0065, JCTVC-H0073)

- Sub-sampling filters (JCTVC-H0394)

- Current issues affecting 4:2:0 (JCTVC-H0177)

- Syntax for monochrome (JCTVC-H0457)

- Modifications for 4:2:2 (JCTVC-H0650)

JCTVC-H0021 JCT-VC AHG report: Reference picture buffering and list construction (AHG21) [R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, W. Wan, Y.-K. Wang (vice chairs)]

(Track A – High-level syntax) – to be reviewed in J. Boyce BoG.

JCTVC-H0022 JCT-VC AHG report: HM subjective quality investigation (AHG22) [G. Sullivan, J.-R. Ohm (co-chairs), F. Bossen, T. Wiegand (vice chairs)]

(Plenary – First day.)

The goal of this AHG work was to quantify, to the extent feasible, the rate savings for which similar subjective quality is achieved for the current HM5 compared to a similarly-configured JM encoder/decoder.

It was assessed to be important to conduct the subjective tests with non-expert viewers. Due to the limited resources that were available for this purpose, tests were restricted to the RA-HE settings of the HM (ref JCTVC-G1200) and to classes B and C. A modified version of the JM encoder was used in the test, which includes some non-normative improvements as described previously in JCTVC-G399 and updated in JCTVC-H0360. The release of a new version of the JM is planned to support these non-normative modifications.

By negotiation with the test coordinator, Vittorio Baroncini, the maximum number of cases that could be tested was set to 72. With 9 sequences in classes B and C and two codecs, it was possible to test four rate points for each. Instead of testing pre-defined rate points, it was considered to try finding points of similar subjective quality, and run both codecs with fixed QP. In terms of subjective quality that would provide reasonable results when assessed by non-expert viewers, QP settings 27, 30, 33 and 36 were selected for the JM codec, with confirmation by the test coordinator. Several experts involved in the preparation performed pre-assessment in comparing the JM versus the HM codec, and the overall opinion unveiled that with same QP settings, the HM would provide substantially better visual quality than the JM, which in fact can also be confirmed by PSNR numbers. Finally, it was decided to use QPHM= QPJM+4 settings for the HM encoded cases. Though PSNR numbers are slightly worse for this case, the experts involved in pre-viewing came to the opinion that this gave a quality which is approximately equal to the JM quality in most cases. It was therefore decided to run the tests with QP settings of 31, 34, 37 and 40 for the HM (and QP settings of 27, 30, 33 and 36 for the JM).

Subjective tests were planned using a Double Stimulus Impairment Scale (DSIS) method equivalent to the approach described in [ref JCTVC-A204]. As it is not likely that the results of the subjective tests will unveil exactly same quality for the different test cases, they are planned to be evaluated through appropriate short-range interpolation of RD graphs (MOS over rate). Such an approach appears appropriate since the QP differences between the various test points are relatively close, such that a fairly precise estimate of the rate difference between the codecs at same quality can be achieved. It is planned to compute these values of rate savings for each test case, on average per sequence, per class and globally.

# Project development, status, and guidance

# Core experiments

## CE1: Entropy coding investigation (Track A)

### Summary

JCTVC-H0031 CE1: Summary report of core experiment on entropy coding [R. Joshi]

The purpose of Core Experiment 1 (CE1) is to evaluate changes to the existing entropy coding. The core experiment was divided into 4 subtests. In subtest A, proposal for modification of the existing arithmetic coding engine was investigated. In subtest B, different methods for 8-bit initialization of the CABAC context models were evaluated. In subtest C, alternatives to the arithmetic coding probability model update were explored. In subtest D, methods for increasing the throughput of CABAC were studied. This report provides a summary of activities and results for this core experiment.

### Contributions

Subtest A

JCTVC-H0444 CE1: Modified LPS range and state transition tables for BAC [J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included):

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm JCTVC-H0444 | 64x4 range rescaling table | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.3% | -0.1% | 0.1% |
|  | 32x4 range rescaling table | -0.2% | -0.6% | -0.6% | -0.2% | -0.4% | -0.5% | -0.3% | -0.1% | -0.3% |

Reduces range table but increases state transition table.

Number of bits in context memory increases

Subtest A: No clear advantage in terms of complexity/efficiency tradeoff

JCTVC-H0210 CE1: Crosscheck of Test A Modified LPS range and state transition tables for BAC (G324, Qualcomm) by Cisco [T. Davies (Cisco)]

Subtest B

B1/B3

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
|  | Formula 2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for low QPs = {17, 22, 27, 32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
|  | Formula 2 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for high QPs = {27, 32, 37, 42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Qualcomm, I2R JCTVC-H0535 | Formula 1 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% |

B2

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included).

For low QPs = {17, 22, 27, 32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,1% | 0,1% | 0,0% | 0,2% | 0,1% | 0,0% | -0,1% | 0,0% |
|  | Test3: ‘disabled’ mapping | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,1% | 0,0% | 0,0% | 0,0% |

For common QPs = {22, 27, 32, 37}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,1% | 0,1% | 0,0% | 0,2% | 0,1% | 0,0% | -0,1% | 0,1% |
|  | Test3: ‘disabled’ mapping | 0,0% | 0,0% | 0,0% | 0,0% | 0,2% | 0,1% | 0,0% | 0,0% | 0,1% |

For high QPs = {27,32,37,42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVC-H0266 | Test1: ‘linear’ mapping | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,3% |
|  | Test3: ‘disabled’ mapping | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% |

B4

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for low QPs = {17,22,27,32}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for high QPs = {27,32,37,42}.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Sharp JCTVC-H0540 | CABAC init. |  |  |  |  |  |  |  |  |  |

JCTVC-H0266 CE1: Report of test results related to subtests B2 (8-bit-init) and C3 (Alt. PMU) [H. Kirchhoffer, B. Bross, P. Helle, D. Marpe, T. Nguyen, M. Siekmann, J. Stegemann, T. Wiegand (Fraunhofer HHI)]

JCTVC-H0076 CE1: Cross-check of Subtest B2 – Test 3 (H0266) [C. Yeo, Y. H. Tan, Z. G. Li (I2R)]

JCTVC-H0161 CE1: verification of C3 sub-test from HHI (JCTVC-H0266) by Samsung and Qualcomm [E. Alshina, A. Alshin, J. H. Park (Samsung), R. Joshi, J. Sole (Qualcomm)]

JCTVC-H0535 CE1 B3: 8-bit Linear Initialization for CABAC [L. Guo, J. Sole, R. Joshi, M. Karczewicz (Qualcomm), C. Yeo, Y.H. Tan, Z. Li (I2R)]

Decision: Adopt 8-bit initialization, formula 2; revisit: cross-checker to confirm that this is the better choice, is simpler implementation wise.

JCTVC-H0395 CE1: Cross-check report for Subtest B3 [P. Helle, H. Kirchhoffer (Fraunhofer HHI)]

JCTVC-H0581 Crosscheck of context modeling for parition mode (JCTVC-H0535) [T. Yamamoto (Sharp)] [late]

JCTVC-H0680 Cross-check of Modified LPS range and state transition tables combined with 8-bit Linear Initialization for CABAC [?? (??)] [late]

Remark from plenary: Was this combination of H0444 and H0535 planned in the CE? If yes, which is the document reporting the combination? A: Is included in H0444.

JCTVC-H0540 CE1: Subtest B4 - On cabac_init_idc [Kiran Misra, Andrew Segall (Sharp)]

Two elements: Implementation of cabac_init_idc which is specified in text but not in software

Currently, up to 7 tables (i+3P+3B) could be supported by text, but the alternative tables for P and B are not specified. 3 tables used in software, suggested to align text and software reduce to maximum 3 (each one for I,B,P) and to allow usage of B table for P and vice versa. Gives small gain (0.2%) with 2-pass coding, but no gain with current test set in one-pass coding

Clarify with SW coordinator what is currently used in HM5

Other contributions related (646, 113, 561) - revisit

JCTVC-H0167 CE1: Cross-check report for Sharp's proposal (Subtest B4) [H. Sasai, T. Nishi (Panasonic)] [late]

Subtest C

(C1 was withdrawn)

C2

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Canon JCTVC-H0140 | Probability update mod | -0.7% | -0.5% | -0.4% | -0.7% | -0.7% | -0.8% | -0.8% | -0.1% | -0.1% |

C3

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition. The size of rangeTabLPS is 16×8 in all cases.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| HHI JCTVH0266 | Test 4 | -0.2% | -0.4% | -0.2% | -0.3% | -0.5% | -0.4% | -0.4% | -0.1% | 0.1% |
|  | new init (test 8) | -0.3% | -0.4% | -0.4% | -0.3% | -0.5% | -0.5% | -0.4% | 0.0% | 0.1% |
|  | Window size 32 (test 9) | -0.3% | -0.1% | 0.0% | -0.3% | -0.1% | -0.1% | -0.3% | 0.2% | 0.2% |
|  | Window size 32 new init (test 10) | -0.3% | -0.2% | -0.1% | -0.3% | -0.1% | -0.2% | -0.3% | 0.2% | 0.1% |

C4

Test results summary (BD-rate difference is averaged across AI, RA, LD(B) for HE, LC and HE-10 respectively, class F is not included) for QPs = {22, 27, 32, 37} in common condition.

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Samsung JCTVC-H0112 | Single parameter | -0.3% | 0.1% | 0.1% | -0.3% | 0.1% | -0.1% | -0.3% | 0.8% | 0.7% |
|  | Multi- parameter | -0.7% | -0.4% | -0.3% | -0.7% | -0.7% | -0.8% | -0.7% | -0.1% | 0.0% |

JCTVC-H0140 CE1: Modified probability update and table removal for multi-parameter CABAC update [C. Rosewarne, M. Maeda (Canon)]

JCTVC-H0165 CE1: verification of C2 sub-test from Canon (JCTVC-H0140) by Samsung [E. Alshina, A. Alshin, J. H. Park (Samsung)]

JCTVC-H0112 CE1, subtest C4: Multi-parameter probability estimation for CABAC [E. Alshina, A. Alshin, J. H. Park (Samsung)]

JCTVC-H0135 CE1: Cross check of Multi-parameter probability up-date for CABAC by Samsung: Sub-test C2 (H0112) [C. Rosewarne, M. Maeda (Canon)]

Subtest C: No clear advantage in terms of complexity/efficiency tradeoff on any of the proposals. Multi-parameter approaches significantly higher complexity.

Subtest D

| Proponent | Description | BD-rate for HE |  |  | BD-rate for LC |  |  | BD-rate for HE-10 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| Nokia JCTVC-H0232 | D1: normal QPs | 4.4% | 2.8% | 3.1% | 3.8% | 5.5% | 5.6% | 6.2% | 2.0% | 3.7% |
|  | D1: low QPs (1,5,9,13) | 7.7% | 7.9% | 8.2% | 5.7% | 5.4% | 5.2% | 9.0% | 10.5% | 10.1% |
| Qualcomm JCTVC-H0536 | D2 | 1.3% | 0.6% | 0.8% | 1.5% | 0.3% | 0.5% | 1.7% | 1.9% | 2.2% |

JCTVC-H0232 CE1.D1: Nokia report on high throughput binarization [J. Lainema, K. Ugur, A. Hallapuro (Nokia)]

Several experts express support to the idea of improving the throughput in CABAC, however this specific approach would in principle mean implementing elements of a second entropy coder for the binarization which may not be the most desirable approach. Several other contributions (non CE) are also addressing the issue in a potentially better feasible way.

JCTVC-H0212 CE1: Subtest D1 - cross-verification of Nokia’s Single entropy coder for HEVC with a high throughput binarization mode (JCTVC-G569) by Cisco [D. Sychov, T. Davies (Cisco)]

JCTVC-H0378 CE1: Cross-check of test D1: Coding of coefficient bins in bypass mode [M. Siekmann (Fraunhofer HHI)] [late]

JCTVC-H0536 CE1 D2: Adaptive initialization for CABAC with fixed probability contexts [L. Guo, J. Sole, X. Wang, M. Karczewicz (Qualcomm)]

Loss 1.3-1.7% (see table above)

JCTVC-H0652 CE1: Cross-verification of Qualcomm's Adaptive initialization for CABAC with fixed probability contexts (sub-test D2) [Gergely Korodi, Dake He RIM)] [late]

Cross-checker confirms that this increases capability for parallel processing. It is also pointed out that the current measurement of throughput (number of bypass bins) is not optimum.

None of the suggested solutions is ideal in the way that lower complexity or higher amount of parallelism must be paid by loss of compression efficiency. Might only make sense if we would define different profiles for different levels of complexity (which may not be what we want according to AHG8 discussion).

There are other contributions that may be interesting targeting the same which are potentially more interesting 233,458,510,554

### Discussion and Conclusions

## CE2: Chroma RQT depth

### Summary

JCTVC-H0032 CE2: Summary report of Core Experiment on Chroma RQT depth [K. Sugimoto, X. Guo, X. Wang (CE Coordinators)]

The following tools are tested:

| test | Proposal Document | Proponent | Tool description | Cross-checker & Document # |
| --- | --- | --- | --- | --- |
| 4.1 JCTVC-G283 | JCTVC-H0317 | MediaTek PKU | - Signaling Max chroma RQT depth for intra - Chroma TU split for inter same as done in luma with max depth=1 - no 32x32 transform for chroma | Qualcomm JCTVC-H0543 |
| 4.2 JCTVC-G442 | JCTVC-H0235 | Mitsubishi NHK | - Implicit chroma RQT depth depending on intra and inter PU mode - allow 32x32 transform for chroma | MediaTek JCTVC- H0319 |
| 4.3 JCTVC-G980 | JCTVC- H0538 | Qualcomm | - Signaling Max chroma RQT depth for intra - Chroma TU split for inter same as done in luma with max depth=1 - allow 32x32 transform for chroma | Mitsubishi JCTVC- H0236 |
| 4.4.0 | JCTVC- H0308 | Harmonization | - Signaling Max chroma RQT depth for intra and inter - no 32x32 transform for chroma | HiSilicon JCTVC- H0258 |
| 4.4.1 | JCTVC- H0308 | Harmonization | - Signaling Max chroma RQT depth for intra and inter - allow 32x32 transform for chroma | Panasonic JCTVC-H0122 |

harmonized solution H308 in principle includes elements from all 3 original proposals (G283, G442, G980), it was decided decided by reflector discussion to also test G442 separately. Therefore, only H308 and H235 (the previous G442) need to be considered. Among the CE participants, it is suggested to adopt H308 without 32x32 transform (configuration 4.4.0)

The approach is to add signaling (2 flags for intra and inter) at SPS for limitation of chroma RQT depth (similar approach as currently for luma and chroma combined). The maximum RCT depth for chroma would still be the same as currently, therefore it is not a decoder complexity reduction. What was tested in 4.4.0 was using intra chroma RQT depth =1 and inter chroma RQT depth =2 (not sequence adaptive), luma RQT depth =3 as in common test conditions. With fixed restricted chroma RQT depth (not sequence adaptive), slight gain on chroma (1-2%) is reported.

One comment: An encoder could not decide how to make it adaptive per sequence when it is signaled at SPS.

Several experts express concern why is it necessary to signal the chroma depth depth at all? Why not make it fixed relative to luma? Restriction of chroma RQT depth (without signaling) seems to be useful and if made non-adaptive could even mean a decoder complexity reduction. It appears that either luma depth minus one (as from H0317) or minus two may make sense, but the latter case was not studied. Further it is not known how the performance would be with luma RQT depth other than 3. CE participants will investigate this -  revisit.

Average BD-rate gain for each test is shown below.

| test | High Efficiency |  |  |  |  |  | Low Complexity |  |  |  |  |  | High 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Intra |  | Random Access |  | Low Delay |  | Random Access |  |
|  | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV | Y | UV |
| 4.1 | 0.02 | -2.47 | -0.05 | -1.32 | -0.15 | -0.02 | 0.02 | -1.92 | -0.01 | -1.39 | -0.02 | -0.93 | 0.02 | -2.59 |
| 4.2 | -0.05 | -2.46 | -0.06 | -1.43 | -0.07 | -1.14 | 0.02 | -2.24 | -0.01 | -1.59 | -0.03 | -2.16 | -0.02 | -4.45 |
| 4.3 | -0.05 | -2.46 | -0.09 | -1.35 | -0.10 | -0.53 | 0.02 | -2.24 | -0.01 | -1.72 | -0.05 | -2.13 | -0.03 | -4.08 |
| 4.4.0 | 0.02 | -2.47 | -0.04 | -1.61 | -0.02 | -0.92 | 0.02 | -1.92 | 0.00 | -1.40 | -0.01 | -1.11 | -0.01 | -2.93 |
| 4.4.1 | -0.05 | -2.46 | -0.05 | -1.67 | -0.04 | -1.21 | 0.02 | -2.24 | 0.00 | -1.61 | -0.02 | -1.38 | -0.01 | -4.29 |

Average run-time for encoding and decoding compared with the anchor are shown below.

| test | High Efficiency |  |  |  |  |  | Low Complexity |  |  |  |  |  | High 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  | Random Access |  | Low Delay |  | Intra |  | Random Access |  | Low Delay |  | Random Access |  |
|  | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC | ENC | DEC |
| 4.1 | 100% | 99% | 97% | 100% | 98% | 100% | 101% | 99% | 98% | 100% | 99% | 100% | 98% | 100% |
| 4.2 | 99% | 99% | 96% | 100% | 97% | 100% | 100% | 99% | 97% | 100% | 98% | 100% | 96% | 100% |
| 4.3 | 99% | 100% | 97% | 100% | 97% | 100% | 98% | 100% | 98% | 100% | 98% | 100% | 97% | 99% |
| 4.4.0 | 99% | 100% | 98% | 100% | 98% | 100% | 99% | 100% | 98% | 100% | 99% | 100% | 97% | 99% |
| 4.4.1 | 98% | 100% | 99% | 100% | 99% | 99% | 98% | 100% | 99% | 100% | 99% | 99% | 98% | 100% |

### Contributions

Test 1

JCTVC-H0317 On Chroma Cbf Signaling [L. Zhao, X. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU), D. Zhao (HIT)] [late]

JCTVC-H0543 CE2: Cross verification of subtest 4.1 [L. Guo (Qualcomm)] [late]

Test 2

JCTVC-H0235 CE2 4.2: report on fixed TU depth for chroma [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi), A. Ichigaya, S. Sakaida (NHK)]

JCTVC-H0319 CE2 subtest 4.2: Crosscheck Report [X. Zhao, X. Guo (MediaTek)] [late]

Test 3

JCTVC-H0538 CE2 4.3: Report on Limiting Chroma Transform Depth in RQT [L. Guo, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0236 CE2 4.3: cross-verification of signaling max chroma TU depth [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

Test 4

JCTVC-H0308 CE2: Subset 4.4: Test Results on Harmonization of JCTVC-G283, JCTVC-G442 and JCTVC-G980 [X. Zhao, J. An, X. Guo, S. Lei (MediaTek), L. Guo, X. Wang, M. Karczewicz (Qualcomm), A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi), A. Ichigaya, S. Sakaida (NHK), S. Ma, W. Gao (PKU)]

JCTVC-H0122 CE2: Cross Check of Test 4.4.1 [Y. Shibahara, T. Nishi (Panasonic)] [late]

JCTVC-H0258 CE2: Cross-verification of CE2 subtest4 [X. Zheng (HiSilicon)] [late]

### Discussion and conclusions

## CE3: Chroma interpolation filter

### Summary

JCTVC-H0033 CE3: Summary of Core Experiment on Chroma Interpolation Filter [T.Chujoh, E.Alshina]

In this CE report test results of interpolation filter for Chroma are reported. One proposal (JCTVC-H0494) has been registered in CE3 and two half-pel interpolation filters have been evaluated on the common condition with cross-verification after all. There was

Related non-CE contributions were noted to include JCTVC-H0180, JCTVC-H0610, JCTVC-H0569, JCTVC-H0207, and JCTVC-H0304.

Current half-pel Chroma filter of HM5.0 is {-4, 36, 36, -4}.  CE3 tests the following two filters compared to HM5.0.

- Variant A: {-7, 39, 39, -7}

- Variant B: {-5, 37, 37, -5}

Both variant A and B have wider pass-band than the current half-pel Chroma filter.

Variant A has wider pass-band than variant B.

The following comments have been concluded at this moment.

- Variant A of 0.1 % and variant B of 0.2% gain for Chroma on average have been achieved.

- Both variants provide no performance change in terms of Luma BD-rate.

- In several sequences on LC-LP when SAO is off, variant A causes loss of coding efficiency; however they are improved if SAO is on.

- Regarding subjective quality assessment, the differences between proposals and anchor are negligible or very small.

When SAO is on and ALF is off, there was more gain. However, it was noted that there were also some cases with loss when SAO is off – e.g. a BQSquare case.

No action taken.

### Contributions

JCTVC-H0494 CE3: Chroma interpolation filter with half pixel phase offset [J. Lou, K. Minoo, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-H0608 CE3: Cross-check for half-pel Chroma interpolation filter modification (JCTVC-H0494) by Samsung [E. Alshina, A. Alshin, J. H. Park (Samsung)] [late]

JCTVC-H0182 CE3: Cross-check report for Motorola Mobility chroma interpolation filter (JCTVC-H0494) [T.Chujoh (Toshiba)]

## CE4: Quantization

### Summary

JCTVC-H0034 CE4: Summary report of Core Experiment on Quantization [K Sato (Sony), M Budagavi (TI), M Coban (Qualcomm), H Aoki (NEC), X Li (MediaTek)] [miss]

Subtest 1: Delta QP coding

1.1 is the current WD5 (differential encoding relative to previous QP) wherever it came from

1.2 takes reference to left QP from same LCU (if available) otherwise last QP from same LCU – saves 0.1% for intra, 0.0% for inter

1.3 also only takes reference to QPs from same LCU, but makes it dependent on intra pred mode, or in inter case average of left & top depending on availability – saves 0.5% for intra, 0.3/0.2% for RA/LD

1.4 is weighted average depending on equality of neighbored QP values – saves approx. 0.2% on average

1.3 and 1.4 are too complex compared to the benefit they give

1.2 is slightly more complex than 1.1 and gives very little gain

Conclusion: No change to WD

Subtest 2: Quant matrix coding

11 proposals, 6 are lossy or configurable as lossy

Subjective tests to identify whether lossy compression makes a problem

Flat matrix as well? Only if it is at same bit rate

Check whether the different proposals have same bit rate, but likely to be in same range

Perform tests at QP 32 and 37

Select 3 lossy compression proposals and one lossless

Breakout (K. Sato and T. Suzuki) to suggest a test setup, and assess the complexity of the different lossy and lossless proposals (including memory size, e.g. using same matrix values for groups of coefficients)

Concern expressed whether current set of matrices (6 in total) is sufficient

Revisit after subjective tests: Identify whether lossy compression causes problems

Subtest 3: QP signaling granularity:

JCTVC-H0506/508 TU/PU instead CU level (minimum 8x8)

Objective results: Loss in BD rate/PSNR, loss for TU based method (506) 0.5-1%, loss for PU based method (508) lower (0.1/0.2%)

Some results are shown that it improves quality (still frames) – not fully obvious

Argument that it is needed for rate control – argued that it is beneficial to have QP switching where motion vectors are switched (i.e. switching at PU level would be needed here)

Instead of results from QP maps that are different from the current CE conditions (and increase rate due to finer granularity), it would be better to compare with same maps a CU-only versus a CU/PU based method (i.e. varying the smallest CU size).

Unclear from the results whether the additional PU level signalling costs overhead or gives benefit (compared to same QP changes encoded CU only). Same granularity could be achieved by using 8x8 CUs with current syntax, if needed. It would be anyway only for some specific applications.

No support except by proponents – no action.

JCTVC-H0260 reports PSNR gains by using different QP values for 8x8 and 4x4 TUs. As this is influencing the decision of RDO (actual bitrate varies by 2% typically).

Does this impact visual quality?

May have some impact on rate control decisions, may hurt perceptual quantization

No action.

Subtest 4 – see below (no action)

### Contributions

Subtest 1

JCTVC-H0053 CE4 Subtest 1: Rate control friendly spatial QP prediction based on intra prediction (test 1.3) [H. Aoki, K. Chono (NEC), M. Kobayashi, M. Shima (Canon), M. Coban, M. Karczewicz (Qualcomm), K. Sato (Sony)]

JCTVC-H0054 CE4 Subtest 1: HM 4 QP prediction with reset to preceding QP in scan order at LCU boundary (test 1.2) [H. Aoki, K. Chono (NEC)]

JCTVC-H0186 CE4: Result of subtest 1.1&1.2 [K Sato (Sony)]

JCTVC-H0205 CE4: Cross-check of subtest1.3 by JVC Kenwood (JCTVC-H0053) [H. Nakamura, M. Nishitani, S. Fukushima (JVC Kenwood)]

JCTVC-H0197 CE4: Results of subtest 1.4 [H. Nakamura, M. Nishitani, S. Fukushima (JVC Kenwood]

Subtest 2

JCTVC-H0230 CE4 subtest 2.1: Enhancements of HEVC quantization matrix [Y. Morigami, J. Tanaka, T. Suzuki (Sony), V. Sze, M. Zhou (TI)]

JCTVC-H0150 CE4 Subtest 2.1: Crosscheck report of Enhancements of AVC quantization matrix in HEVC (JCTVC-H0230) [M. Shima (Canon)]

JCTVC-H0303 CE4: Cross-check report of subtest 2.1 for Sony/TI on Enhancements of AVC quantization matrix in HEVC [J. Zheng(HiSilicon)]

JCTVC-H0384 CE4 Subset2.2: Results for up-sampling large size quantization matrices based on JCTVC-G152 [S. Liu, X. Zhang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-H0151 CE4 Subtest 2.2: Crosscheck report of up-sampling large size quantization matrices based on JCTVC-G152 (JCTVC-H0384) [M. Shima (Canon)]

JCTVC-H0306 CE4: Cross-check report of subtest 2.2 for MediaTek on quantization matrices representation Method and syntax [J. Zheng(HiSilicon)]

JCTVC-H0314 CE4: Subtest 2.4 Layered quantization matrices representation and compression [Y. Wang(Tsinghua), J. Zheng(HiSilicon), Y. He(Tsinghua)]

JCTVC-H0152 CE4 Subtest 2.4: Crosscheck report of Layered quantization matrices compression (JCTVC-H0314) [M. Shima (Canon)]

JCTVC-H0231 CE4: Cross check of CE4 Subset2.4 on JCTVC-H0314 [Y. Morigami, J. Tanaka, T. Suzuki (Sony)]

JCTVC-H0451 CE4: Subtest 2.5: Coding of quantization matrices based on modified prediction and mapping to unsigned values [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-H0153 CE4 Subtest 2.5: Crosscheck report of Coding of quantization matrices based on modified prediction and mapping to unsigned values (JCTVC-H0451) [M. Shima (Canon)]

JCTVC-H0307 CE4: Cross-check report of subtest 2.5 for Qualcomm on Quantization matrix compression and signaling [J. Zheng(HiSilicon)]

Subtest 3

JCTVC-H0506 CE4 subtest 3.1.a: QP adaptation at sub_CU level [J. H. Kim, X. Fang, K. Panusopone, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-H0184 CE4 Subtest 3: Cross-verification report for test 3.1.a (JCTVC-H0506) [H. Aoki, K. Chono (NEC)]

JCTVC-H0508 CE4 subtest 3.1.b: Additional constraint for QP adaptation at sub_CU level [J. H. Kim, X. Fang, K. Panusopone, L. Wang, A. Luthra (Motorola Mobility)]

JCTVC-H0191 CE4 Subtest 3: Cross-verification report for test 3.1.b (JCTVC-H0508) [H. Aoki, K. Chono (NEC)] [late]

JCTVC-H0260 CE4:Subset 3.2 Implicit signalling of QP offset based on TU size  [C. Kim, Y.Park, K.Choi, J.H.Kim, J.H. Park (Samsung)]

JCTVC-H0518 CE4: cross check report for subtest 3.2 (JCTVC-H0260) [K. Panusopone (Motorola)] [late]

Subtest 4

JCTVC-H0336 CE4 Subtest 4.1: Higher granularity of quantization parameter scaling [T. Lee, J. Park (Samsung), K. Keiichi (NEC)]

The argument is that finer QP scaling is needed in HEVC because rate control may need to be aligned with CU boundaries which may be larger than MBs in AVC. However, no rate control algorithm is currently available to give proven evidence about this.

The results unveil that it may be useful with constant QP but this is not relevant in practice

No support by other experts unless a real proof would be given.

JCTVC-H0318 CE4.4: Crosscheck for Samsung's Higher Granularity of Quantization Parameter Scaling [X. Li, X. Guo (MediaTek)] [late]

JCTVC-H0254 CE4: Cross-verification of JVC KENWOOD's proposal [X. Zheng (HiSilicon)] [late]

### Discussion and Conclusions

## CE5: Transform skipping

### Summary

JCTVC-H0035 CE5: Summary Report of Core Experiment on Transform Skipping [M. Mrak, J. Sole, J. Xu, A. Saxena (CE coordinators)]

Two types of transform skipping:

- Transform skipping on intra coded blocks, namely Residual Scalar Quantization (RSQ), presented in JCTVC-E145, and

- Transform skipping on inter coded blocks, namely Transform Skip Mode (TSM), presented in JCTVC-G1003.

Test a: Quantization (adapting the scaling factors). Without this, slight losses by transform skipping. When invoked, gain is 0.4% for RA and 0.7% in LD (excluding class F); higher in class F (1.8/4%).

When using quant matrices (and compared to the case of using default matrices without TS), the gain is 0.7/0.8% for RA and LD.

Encoder runtime increase by TS is approx. 30-35%.

Test b: If restricted to 4x4 blocks, encoder runtime increase is 8%, and the gain is 0.2/0.2% for RA and LD cases.

Test c: Mode dependent scans: no gain

Test d: Coding last coefficient in bypass mode does not affect coding efficiency for default conditions, and decreases rate by approx. 0.2% for class F. Note: For screen content (class F), a large amount of blocks (up to 70%) uses transform skipping.

Question: Are the horizontal and vertical modes useful? Reported to be used in majority of cases for camera content (approx. 50% no skip, 20% vert/hor, 10% full skip). For screen content, approx. 30% no skip, and percentage full skip increases.

Test e: withdrawn

Test f: Intra coding (RSQ): Only choice of 2D or no transform, usual prediction modes and mode-dependent scans are used. Deblocking filter is disabled when one of the blocks is fully skipped. No gain for common test conditions, 8% gain for class F in AI-HE mode. If modified contexts are used for the skip mode, 0.2% gain in common conditions, 10% in class F. If deblocking filter is used as conventional, the gain in class F all-intra is 6.5%.

Test g: Combination of tests a-d. Compared to HM, 0.4/0.6% gain for common test conditions, 2.1/4.7% for class F.

Transform skipping does not give benefit under common testing conditions, would be a specific tool for screen content (class F) – note: Class F sequences may be less challenging for inter case.

A thorough investigation of the complexity impact is missing. Proponents argue that complexity is decreased but in fact it is increasing the decoder complexity (additional logic for switching the transform and the scaling for de-quantization).

Some concern is expressed for hardware implementations particularly about the horizontal/vertical options. Frequent switching the de-quantization is also of concern.

For inter coding, the version which restricts transform skipping to 4x4 blocks (only changing transform and scaling) seems to be most beneficial, but this still requires hor/vert switching and change of scaling/dequant.

Related non-CE contributions: H0061 uses only full skip for inter, H0141 is reducing complexity for scaling/dequant.

It is said that if the transform is fully skipped, the change in dequantization can be implemented by a shift operation (further clarification whether this resolves the concern above?)

Relation with lossless coding? In that case, 2D transform would be skipped, but no change in de-quantization – this would be more acceptable

Request is also made to investigate subjective quality

No support by other companies except proponents, but some concern expressed about the complexity impact and that the benefit of this tool is highly sequence dependent.

Conclusion: The additional complexity implication of encoder and decoder would not justify the benefit it gives for specific class F sequences.

### Contributions

JCTVC-H0208 CE5.a: Quantization for transform skipping [M. Naccari, A. Gabriellini, M. Mrak (BBC)]

JCTVC-H0330 CE5.a: Crosscheck for BBC's quantization for transform skipping in JCTVC-H0208 [S.-T. Hsiang, T.-D. Chuang (MediaTek)] [late]

JCTVC-H0169 CE5.b: Transform skipping choices based on block parameters [G. Van Wallendael, S. Van Leuven, J. De Cock, R. Van de Walle (Ghent Univ.)]

JCTVC-H0560 CE 5: Cross-Check of H0169 on transform skipping choices based on block size [Ankur Saxena, Felix Fernandes (Samsung)] [late]

JCTVC-H0246 CE5: Cross-verification of Ghent University's proposal on transform skipping [X. Zheng (HiSilicon)] [late]

JCTVC-H0064 CE5.c – Mode-dependent significance scans for transform skip [M. Mrak, A. Gabriellini (BBC)]

JCTVC-H0584 CE5: Cross-verification of subtest CE5.c of BBC (JCTVC-H0064) by Nokia [J. Kang, A. Hallapuro, J. Lainema (Nokia)] [late]

JCTVC-H0454 CE5: Subtest c: Crosscheck report [R. Joshi (Qualcomm)] [late]

JCTVC-H0452 CE5: Subtest d: Modifications to last significant coefficient position coding for transform skip mode [R. Joshi, J. Sole, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0142 CE5.d: Cross-verification for coefficients coding for transform skipping [E.S. Ryu, J. Dong (InterDigital)] [late]

JCTVC-H0361 CE5.f: Residual Scalar Quantization for HEVC [C. Lan (Xidian Univ.), J. Xu, F. Wu (Microsoft)]

JCTVC-H0373 CE5.f: Cross-check of Microsoft's JCTVC-H0361 Residual Scalar Quantization for HEVC [X. Wen, O. Au (HKUST)] [late]

JCTVC-H0211 CE5.g: Integration of tools CE5.a to CE5.e [M. Mrak, A. Gabriellini (BBC), G. Van Wallendael (Ghent university), R. Joshi (Qualcomm)]

JCTVC-H0479 Cross-check of CE5.g: Integration of tools CE5.a to CE5.e [J. Wang, X. Yu, D. He (RIM)] [late][miss]

JCTVC-H0320 CE5: Cross-check report of CE5.g combined tool by Cisco [T. Davies (Cisco)] [late]

### Discussion and Conclusions

## CE6: Intra prediction improvements

### Summary

JCTVC-H0036 CE6: Summary Report of Core Experiments on Intra Coding Improvements [Ali Tabatabai, Keiichi Chono, Edouard Francois (CE Coordinators)]

The experiments are divided into 3 subsets, as indicated below:

- Subset CE6a:	Intra Chroma Prediction

- Subset CE6b:	Improved Intra Prediction

- Subset CE6c:	Intra mode coding cleanup and simplification

### Contributions

Subtest a

Every proposal adds some complexity:

173 is using reconstructed Cb residual as a refinement for Cr prediction

244 is checking irregularities to find the most suitable value for alpha in LM mode

346 uses reconstructed luma residual for chroma prediction additionally to LM mode

358 adds two new variants of LM mode (LML and LMA)

Note: Current HM5 has 6 chroma prediction modes, 244, 346 and 358 introduce more modes.

None of the proposals adds a big advantage in compression performance.

Some concern is expressed that the intra prediction is already fairly complex (particularly in LM mode). 173 seems to be the least complex, but introduces a new dependency in all modes. Check the impact on hardware implementation (Madhukar, Wade) - revisit

JCTVC-H0117 CE6.a: Chroma intra prediction based on residual luma samples [K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI)]

JCTVC-H0118 CE6.a: Cross-check results of intra chroma prediction [K. Kawamura, S. Naito (KDDI)]

JCTVC-H0129 CE6.a: Cross-check results of intra chroma prediction [S. Matsuo, M. Matsumura, S. Takamura, H. Jozawa (NTT)]

JCTVC-H0171 CE6: Results on experiments related to section a on Intra Chroma Prediction [C.Gisquet, E. François (Canon), Y.-J. Chiu, Y. Han (Intel), X. Zhang, O. Au (HKUST), K. Kawamura, T. Yoshino (KDDI)]

JCTVC-H0172 CE6: Cross verification of results related to section CE6.A [E. François, C.Gisquet (Canon)]

JCTVC-H0295 CE6.a: Cross-channel prediction refinement to improve intra chroma prediction [Y.J. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang(Intel)]

JCTVC-H0573 Cross Check for Intel's intra chroma prediction (JCTVC-H0295) [K. Kawamura (KDDI)][late]

JCTVC-H0296 CE6.a: Cross-check report for CE6a on intra chroma prediction [Y.J. Chiu, Y. Han, L. Xu, W. Zhang(Intel)]

JCTVC-H0323 CE6.a: Cross-check of three combined cases of the four intra chroma prediction tools by Huawei [H. Yang (Huawei)]

JCTVC-H0539 CE6.a: Cross-check report for intra chroma prediction [X. Zhang, O. Au (HKUST)]

JCTVC-H0544 CE6.a: New modes (LML and LMA) for chroma intra prediction [X. Zhang, O. Au, J. Dai, X. Wen, C. Pang, F. Zou (HKUST)]

Subtest b

JCTVC-G135/H0437 proposes to add two prediction unit (PU) types (i.e. 2NxN and Nx2N) to Intra coding units (CU) in addition to the existing PU types (i.e. 2Nx2N and NxN) in current HM. It also reports NSQT harmonization results with and without 8x2/2x8 transforms.

JCTVC-G119/H0059 proposes to make the total number of remaining intra modes a power of 2 and accordingly use FLC for binarization.

Relation of 437 and 455 (the latter in AHG16 contribution)? They should give same results, but are different. 437 is subsumed in 455 and can be discussed in context of AHG16.

JCTVC-H0437 CE6.b: Rectangular (2NxN and Nx2N) PU for Intra Prediction  [X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-H0297 CE6.b: Cross verification of MediaTek’s rectangular (2NxN and Nx2N) intra prediction (JCTVC-H0437) [Y.J. Chiu, Y. Han, L. Xu, W. Zhang(Intel)]

JCTVC-H0346 CE6.b crosscheck for MediaTek's proposal [Y. Lin, C. Lai, L. Liu, J. Zheng (HiSilicon)]

JCTVC-H0268 CE6: Cross-check for Subtest CE6b of Rectangular (2NxN and Nx2N) Intra Prediction [S. Oudin (Fraunhofer HHI)]

JCTVC-H0403 CE6.b: Cross-check report for Rectangular (2NxN and Nx2N) PU for Intra Prediction [J. Xu, A. Tabatabai (Sony)]

JCTVC-H0187 CE6: Crosscheck of JCTVC-G119 [K Sato (Sony)]

JCTVC-H0057 CE6 subset 5.2.2 and 6.2.2: Intra coding improvements [Y. Lin, L. Liu, J. Zheng (HiSilicon), H. Yang, H. Yu (Huawei)]

Relating to subtest b: Removing Hor+8 and Ver+8 modes, add a new mode which is average of both. In terms of compression, no benefit (at most 0.1% BR reduction). Is this a simplification? It reduces one mode, but needs to add in the averaging process. Therefore it is not a real simplification.

Relating to subtest c: Remove Hor+8 entirely. No loss in compression.

JCTVC-H0192 CE6: Cross-check report for Subtest CE6b and CE6c on modifications to Intra-frame coding [H. L. Tan, C. Yeo, Y. H. Tan (I2R)] [late]

Subtest c

JCTVC-H0075 CE6: Subtest 6c (6.2.1) – Intra mode coding simplification [C. Yeo, Y. H. Tan, H. L. Tan, Z. G. Li (I2R)]

This contribution proposes a modification to the intra prediction mode coding in HM based on the proposal JCTVC-G153. The remaining mode coding uses a binary+FLC binarization for remaining mode coding. When the binary flag is coded in bypass mode, there is reportedly no loss in coding performance for AI-HE and AI-LC configurations. When the binary flag is coded using a context, there is reportedly an average BD-Rate gain of 0.2% for both AI-HE and AI-LC configurations.

General opinion: This is a straightforward approach of simplification.

Conceptually similar to 3 MPM approach

Various similar methods targeting simplification (see CE summary) - revisit both 0057 and 0075, estblish breakout group on simplification of intra mode coding (K. Chono).

JCTVC-H0058 Cross-verification of CE6 subset 6.2.1: On intra prediction mode coding (JCTVC-H0075) [Y. Lin, L. Liu (HiSilicon)]

JCTVC-H0440 CE6.c: Cross-check of JCTVC-H0075: Intra mode coding simplification [X. Zhang, S. Liu (MediaTek)] [late]

JCTVC-H0478 CE6.c: Cross check of Huawei/HiSilicon’s proposal JCTVC-H0057 [J Kim, C Kim, B Jeon (LG)]

### Discussion and conclusions

## CE7: Additional transforms

### Summary

JCTVC-H0037  CE7: Summary Report of Core Experiment on Additional Transforms [R. Cohen, C. Yeo, R. Joshi, F. Fernandes (CE coordinators)]

Three types of additional transforms for luma TUs have been evaluated:

- Tool 1 is a mode-dependent secondary transform for 8x8, 16x16, and 32x32 Intra TUs, applied after the HM core transform.

- Tool 2 applies a boundary-dependent primary transform for 4x4, 8x8, and 16x16 Inter TUs. Size 4 transforms use the HM DCT or HM DST Type-VII (currently used for Intra TUs) extended to the Inter case. At size 8 and larger, HM DCT and DST-Type IV are used. The DST-Type IV is generated from the DCT-Type IV by flipping basis vectors.

- Tool 3 applies the boundary-dependent primary transform from Tool 2 for 4x4 Inter TUs and a boundary-dependent secondary transform for 8x8, 16x16, and 32x32 Inter TUs. The secondary transform matrices in Tools 1 and 3 are the same. The performance of this tool is also evaluated with Tool 1.

Performance summaries for All-Intra configurations (N/A: Not Applicable)

| Tool | AI-HE |  |  |  |  |  | AI-LC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg Y BD-Rate (%) | Min,Max Y BD-Rate (%) |  | Enc Time (%) |  | Dec Time (%) | Avg Y BD-Rate (%) |  | Min,Max Y BD-Rate (%) |  | Enc Time (%) |  | Dec Time (%) |
| 1: JCTVC-H0125 Mode-dependent secondary (Intra) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4x4 | -0.5 | -0.2,-0.8% |  | 101 |  | 99 | -0.6 |  | -0.3,-1.1% |  | 102 |  | 102 |
| 8x8 | -0.7 | -0.3,-1.2% |  | 103 |  | 102 | -0.8 |  | -0.4,-1.5% |  | 105 |  | 102 |
| 2: JCTVC-H0309 Boundary-dependent primary (Inter) | N/A |  |  |  |  |  | N/A |  |  |  |  |  |  |
| 3: JCTVC-H0126 Boundary-dependent secondary | N/A |  |  |  |  |  | N/A |  |  |  |  |  |  |

Performance summaries for Random Access 8-bit configurations

| Tool | RA-HE |  |  |  | RA-LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg Y BD-Rate (%) | Min,Max Y BD-Rate (%) | Enc Time (%) | Dec Time (%) | Avg Y BD-Rate (%) | Min,Max Y BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| 1: JCTVC-H0125 Mode-dependent secondary (Intra) |  |  |  |  |  |  |  |  |
| 4x4 | -0.2 | -0.1,-0.3 | 101 | 101 | -0.3 | -0.2,-0.6 | 100 | 102 |
| 8x8 | -0.3 | -0.2,-0.5 | 100 | 100 | -0.4 | -0.2,-0.7 | 100 | 101 |
| 2: JCTVC-H0309 Boundary-dependent primary (Inter) | -0.3 | -0.2,-0.5 | 101 | 101 | -0.3 | -0.3,-0.6 | 101 | 100 |
| 3: JCTVC-H0126 Boundary-dependent secondary |  |  |  |  |  |  |  |  |
| 4x4 Inter | -0.3 | -0.2,-0.4 | 102 | 100 | -0.3 | -0.3,-0.5 | 101 | 102 |
| 8x8 Inter | -0.3 | -0.2,-0.4 | 102 | 100 | -0.4 | -0.3,-0.6 | 102 | 101 |
| 4x4 Inter + Tool 1 4x4 Intra | -0.5 | -0.4,-0.7 | 102 | 102 | -0.7 | -0.5,-1.1 | 101 | 101 |
| 8x8 Inter + Tool 1 8x8 Intra | -0.6 | -0.4,-0.9 | 102 | 100 | -0.8 | -0.5,-1.3 | 102 | 102 |

Performance summaries for Low Delay configurations

| Tool | LB-HE |  |  |  | LB-LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg Y BD-Rate (%) | Min,Max Y BD-Rate (%) | Enc Time (%) | Dec Time (%) | Avg Y BD-Rate (%) | Min,Max Y BD-Rate (%) | Enc Time (%) | Dec Time (%) |
| 1: JCTVC-H0125 Mode-dependent secondary (Intra) |  |  |  |  |  |  |  |  |
| 4x4 | -0.1 | +0.1,-0.2 | 100 | 100 | -0.1 | +0.1,-0.2 | 100 | 100 |
| 8x8 | -0.1 | +0.2,-0.2 | 100 | 101 | -0.1 | +0.2,-0.2 | 101 | 102 |
| 2: JCTVC-H0309 Boundary-dependent primary (Inter) | -0.5 | -0.4,-0.6 | 101 | 101 | -0.6 | -0.6,-0.7 | 102 | 100 |
| 3: JCTVC-H0126 Boundary-dependent secondary |  |  |  |  |  |  |  |  |
| 4x4 Inter | -0.3 | -0.2,-0.4 | 101 | 99 | -0.4 | -0.3,-0.5 | 101 | 100 |
| 8x8 Inter | -0.4 | -0.2,-0.4 | 101 | 99 | -0.5 | -0.4,-0.6 | 101 | 100 |
| 4x4 Inter + Tool 1 4x4 Intra | -0.4 | -0.1,-0.5 | 101 | 101 | -0.5 | -0.2,-0.7 | 101 | 101 |
| 8x8 Inter + Tool 1 8x8 Intra | -0.5 | -0.1,-0.6 | 102 | 101 | -0.6 | -0.2,-0.8 | 102 | 100 |

- Note: Last two rows in “tool 3 performance” are combination of tool 1 and tool 3.

Most attractive solution w.r.t. performance vs. complexity: Tool3 with 4x4 secondary for inter, Tool1 with 4x4 for intra

Concern expressed about increased latency due to the additional transform stage, and increased number of building blocks, decision elements, buffers etc. in general (same concerns were expressed in last meeting)

Latency problem is more severe for intra due to the higher amount of dependencies

Latency could be assessed to be 17 instead of 16 cycles for 8x8 blocks with 4x4 secondary transform, for larger blocks additional latency could be zero with appropriate implementation. Implication of boundary-dependent decision on complexity/latency not fully clear

Cross-checker supports proposal emphasizing that the gain is remarkable for high resolution sequences

No consensus reached, no action.

### Contributions

JCTVC-H0125 CE 7 : On secondary transforms for intra prediction residual [A. Saxena, Y. Shibahara, F. Fernandes, T. Nishi (Samsung)]

JCTVC-H0126 CE 7: On secondary transforms for inter prediction residual[A. Saxena, Y. Shibahara, F. Fernandes, T. Nishi (Samsung)]

JCTVC-H0589 CE7: Cross-check for Tool 1 and 3 (H0125 and H0126) by BBC [M. Mrak, D.Flynn (BBC)] [late]

JCTVC-H0124 CE7: Cross check report of Tool1, Mode dependent secondary transform [A. Ichigaya, Y. Sugito (NHK)]

JCTVC-H0393 CE7: Cross verification of JCTVC-H0126, on secondary transforms for inter prediction residual [R. Cohen (MERL)]

JCTVC-H0309 CE7: Boundary-Dependent Transform for Inter-Predicted Residue [J. An, X. Zhao, X. Guo, S. Lei (MediaTek)]

JCTVC-H0080 CE7: Cross-check of Tool 2, Boundary-dependent transform for inter [Y. Sugito, A. Ichigaya (NHK)]

JCTVC-H0127 CE 7: Cross-Check for H-0309 [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-H0324 CE7: Cross verification of JCTVC-H0309, Boundary Dependent Transform for Inter Predicted Residue [R. Cohen (MERL)]

### Discussion and Conclusions

## CE8: Non-deblocking loop filtering

### Summary

JCTVC-H0038 CE8: Summary report of Core Experiment on non-deblocking loop filtering [T. Yamakage, I. S. Chong, M. Narroschke, Y.-W. Huang]

A summary of core experiment 8 (CE8) on non-deblocking loop filtering was reported. There were six Subtests in CE8:

- Subtest a: Implementation friendly improvement of non-deblocking in-loop filtering (four proposals),

- Subtest b: ALF simplification (two proposals),

- Subtest c: Necessity of sign bits for SAO offsets (one proposal),

- Subtest d: SAO simplification and line buffer reduction (two proposals),

- Subtest e: SAO improvement (five proposals),

- Subtest f: Subjective picture quality assessment of ALF.

These were evaluated based on the common test conditions in JCTVC-G1200 and additional conditions in JCTVC-G1208. All mandatory results are verified by cross-checkers.

Some formal subjective viewing on CE8.f was conducted prior to the official San Jose meeting on Jan 31.

An informal subjective viewing will be conducted at “Silicon Valley” on the first day of the official meeting on Feb 1. The detailed information will be announced during the meeting.

H0714 is a report of subjective test results for subtest f. The other subjective testing is reported in a powerpoint deck for H0038. The results were made available and were discussed

on Fri 02-03.

H0382 reports overall gain from ALF.

H0277 reports gains of ALF and SAO. The gain of ALF (with SAO on in both cases) was reported as , 1.6/3.3/2.8/4.2% in AI/RA/LB/LP HE (more for Class A), more in LP, with the benefit asserted to be visible in some sequences.

The gain is greater for HD and higher resolution (and higher for non-CTC sequences).

H0714

ALF on vs. off visual quality improvement was tested at equal bit rate.

For post filtering, the bit rates were somewhat higher (2% on average). Somewhat higher complexity was also noted to occur, as generally there is more filtering applied in the post-filtering case (reported in H0382 as 10-17% in non-intra-only use cases – although there may be some implementation issues involved in that analysis, it).

H0714 did not (in preliminary analysis) find a statistically significant visual difference in quality between ALF on, ALF off, and ALF as a post-filter.

It was noted that it is generally difficult to find statistically significant differences in subjective tests when gains are relatively small.

It was also noted that the method of determining, in an encoder, the filtering parameters that will be sent to the decoder, is unknown and outside the scope of the standard (and may not be MSE-based). However, it was acknowledged that this is a general problem for the whole codec design.

It was suggested that generally any quality difference was difficult to perceive, and most likely to be found at the highest QP values.

The test methodology, i.e. reference, A, B rather than just A, B and selection of which is better, was somewhat questioned.

A participant commented that visual effects at very high QP values (perhaps beyond the range of what was tested in this test) may provide additional insight.

Regarding subtest a,

It was remarked that the LCU-based signalling effectiveness is affected by LCU size. These tests were done with 64x64 LCUs.

It was remarked that LCU-based operation for ALF doesn't really help unless the rest of the codec can operate in an LCU-based fashion as well.

### Contributions

Subtest A

JCTVC-H0272 CE8.a.1: One-stage SAO and ALF with LCU-based syntax [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh (Toshiba)]

JCTVC-H0102 CE8.a.1: Crosscheck of MQT's One-stage SAO and ALF with LCU-based syntax (JCTVC-H0272) [T. Ikai (Sharp)][late]

JCTVC-H0066 CE8.a.2: ALF with LCU-based syntax [A. Fuldseth, G. Bjøntegaard (Cisco)]

JCTVC-H0586 Crosscheck of Cisco proposal JTCVC-H0066 [Gordon Clare, Felix Henry (Orange)] [late]

JCTVC-H0170 CE8.a.2: Cross-check of Cisco's proposal "ALF with LCU-based syntax (JCTVC-H066) [T.Yamakage (Toshiba)] [late]

JCTVC-H0273 CE8.a.3: SAO with LCU-based syntax [C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung)]

JCTVC-H0103 CE8.a.3: Crosscheck of MQS's SAO with LCU-based syntax (JCTVC-H0273) [T. Ikai (Sharp)] [late]

JCTVC-H0274 CE8.a.4: One-stage/Two-stage SAO and ALF with LCU-based syntax [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm), T. Yamakage, T. Itoh, T. Watanabe, T. Chujoh (Toshiba)]

JCTVC-H0408 CE8.a.4 Cross-check [A. Fuldseth (Cisco)] [late]

JCTVC-H0422 CE8: Cross-check CE8.a.4. ALF_ONLY [P. Lai, F. C. A. Fernandes (Samsung)] [late]

JCTVC-H0371 CE8 subtest A: Cross-check report for JCTVC-H0274 [S. Esenlik (Panasonic)]

Subtest B

JCTVC-H0413 CE8.b.1: ALF BA feature computation at upper-left 4 pixels in a 4×4 block [P. Lai, F. C. A. Fernandes (Samsung), E. Maani, A. Tabatabai, M. Ikeda (Sony)]

JCTVC-H0385 CE8.b.1: Crosscheck of Modification of ALF BA classification (JCTVC-H0413) by Qualcomm [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-H0068 CE8.b: ALF with single filter type [M. Budagavi (TI)]

JCTVC-H0416 CE8.b.2 Crosscheck TI's single shape ALF [P. Lai, F. C. A. Fernandes (Samsung)] [late]

Subtest C

JCTVC-H0434 CE8 Subset c: Necessity of Sign Bits for SAO Offsets [W.-S. Kim, D.-K. Kwon (TI)]

JCTVC-H0331 CE8.c.1: Crosscheck for TI's necessity of sign bits for SAO offsets in JCTVC-H0434 [E. Alshina, A. Alshin (Samsung), C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

Subtest D

JCTVC-H0436 CE8 Subset d.1: Reduced Number of Band Offsets in SAO [W.-S. Kim, D.-K. Kwon (TI)]

JCTVC-H0332 CE8.d.1: Crosscheck for TI's reduced number of band offsets in SAO in JCTVC-H0436 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0375 CE8.d.2: SAO Line Buffer Reduction [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-H0334 CE8.d.2: Crosscheck for Panasonic's SAO line buffer reduction in JCTVC-H0375 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

Subtest E

JCTVC-H0276 CE8.e.1: Offset coding in SAO [C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-H0173 CE8: Cross verification of Mediatek results regarding section e.1 [G. Laroche, T. Poirier, P. Onno (Canon)] [late]

JCTVC-H0174 CE8: Results related to section e.2  [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-H0229 CE8.e.2: Cross check of CE8.e.2 (JCTVC-H0174) [Kenneth Andersson(Ericsson)]

JCTVC-H0223 CE8.e.3: Modified SAO edge offsets [Kenneth Andersson, Per Wennersten, Rickard Sjöberg (Ericsson), Semih Esenlik, Matthias Narroschke, Thomas Wedi(Panasonic)]

JCTVC-H0390 CE8.e.3: Crosscheck of Modified SAO edge offsets (JCTVC-H0223) by Qualcomm [I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-H0557 CE8.e.4 Selection and Coding of SAO Parameters [Koohyar Minoo, David Baylon (Motorola)]

JCTVC-H0335 CE8.e.4: Crosscheck for Motorola Mobility's selection and coding of SAO parameters in JCTVC-H0557 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0225 CE8.e.5: Combination of CE8.e.2 and CE8.e.3 [Kenneth Andersson, Per Wennersten, Rickard Sjöberg (Ericsson), Guillaume Laroche, Patrice Onno, Tangi Poirier (Canon), Semih Esenlik, Matthias Narroschke, Thomas Wedi (Panasonic)]

JCTVC-H0337 CE8.e.5: Crosscheck for combination of CE8.e.2 and CE8.e.3 in JCTVC-H0225 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

Subtest F

JCTVC-H0622 CE8.f : Crosscheck report on objective evaluation of visual testing [O. Nakagami, M. Ikeda, T. Suzuki (Sony)] [late]

### Discussion and Conclusions

## CE9: MV coding and skip/merge operation

### Summary

JCTVC-H0039 CE9: Summary report of Core Experiment on MV Coding and Skip/Merge Operations [I.-K Kim, J. Jung, S. Oudin (CE coordinators)]

In this CE report, test results of two different categories, modification of bi-prediction (BP) and simplification of the first merge pruning process (SP), are reported. Eight experiments for BP and one experiment for SP were conducted.

For modification of bi-prediction, 8 experiments were conducted to verify the efficiency of bi-prediction modification, which was originally presented in JCTVC-G305. For all these tests, the only reference used is HM-5.0. In BP01, as the original form of the proposal, the motion vector difference (MVD) for List1 is not signaled and sets to (0, 0) for GPB pictures (reference picture list 1 is identical to reference picture list 0). A variation, which alters L0 MVD instead of L1 MVD, is tested in BP02. In BP03, the same approach as BP01 is applied to all inter pictures. Also, in order to investigate the effects of syntax change, two experiments are done with encoder only changes in BP04 and BP05. The effect of combination with 2nd motion estimation for bi-prediction in non-GPB pictures is investigated by BP06. In BP07 and BP08, the same modifications to BP01 and BP02 are tested with the addition of the HM ticket #175 fix. The HM ticket #175 is applied since this fix is related to the bi-predictive motion estimation (ME) enhancement. The impact on visual quality by the modification of ME part is also investigated.

Summary of BP-series tests:

- LD (LD-HE, LD-LC)

  - Gain of BP08 (including a syntax addition and bi-predictive ME fix) is 1.1% in average without encoding and decoding time increase for classes A-E. For class F it is 1.9% in average. The cross-checker concludes that BP08 does not show any unusual or undesired effect on reconstruction quality.

  - Gain of BP05 (gain from encoder only 2nd motion estimation for bi-predictive mode) is 0.7 % in average with 31% encoding time increase for classes A-E. For class F is 0.7% in this case. It is worth noting that BP05 does not include bi-predictive ME fix. (With bi-predictive ME fix, the gain will increase to 0.7% for classes A-E case and 1.7% for class F case, respectively. These results are reported in JCTVC-H0111).

  - Gain from adding a syntax element is average 0.7%, which is derived by comparing results between BP01 and BP04.

- RA (RA-HE, RA-LC, RA-HE-10 bits)

  - Gain of BP08 (including a syntax addition and bi-predictive ME fix) is 0.3% in average without encoding and decoding time increase for classes A-E. For class F is 0.7% in average. The cross-checker concludes that BP08 does not show any unusual or undesired effect on reconstruction quality.

  - Gain of BP05 (gain from encoder only 2nd motion estimation for bi-predictive mode) is 0.6 % in average with 21% encoding time increase for classes A-E. For class F is 0.4% in this case. It is worth noting that BP05 does not include bi-predictive ME fix. (With bi-predictive ME fix, the gain will increase to 0.7% for classes A-E case and 1.1% for class F case, respectively. These results are reported in JCTVC-H0111).

  - Gain of BP06 (gain from BP01 + encoder only 2nd motion estimation for bi-predictive mode) is 0.7 % in average with 18% encoding time increase for classes A-E. For class F is 0% in this case. It is worth noting that BP06 does not include bi-predictive ME fix. (With bi-predictive ME fix, the gain will increase to 0.7% for classes A-E case and 1.1% for class F case, respectively. These results are reported in JCTVC-H0111).

  - Gain from adding a syntax element is average 0.2 %, which is derived by comparing results between BP01 and BP04.

Summary of SP test:

- SP is reported 0.1% loss in average. But it is claimed to have several benefits: reduction of complexity of merge list generation, decoding time reduction by early exit and improved error resilience.

Recommendations:

- Fix the HM ticket #175 (encoder-only modification to remove a clipping) – Decision (SW): Agreed.

- Recommend BP08 as the best method among 8 test cases and BP08 will be discussed as a candidate of adoption at the meeting with non-CE contributions.

It was remarked that this is a syntax change essentially to optimize for a particular encoder design, and that a modification that does not add syntax could achieve similar gain without the syntax change.

It was suggested to review JCTVC-H0431 in that regard.

Deferred for review of non-CE input – later closed by BoG (see notes for H0701).

In the simplified pruning (SP) test, the first pruning process in HM5.0 is removed and instead, a smaller number of parallelizable motion information comparisons are performed only between the spatial candidates. The motion information of a spatial candidate is inserted into the merge list depending of a specific condition on this spatial candidate.

There are two aspects of the proposal. For the first aspect, another contribution (JCTVC-H0133) proposes the same thing. After some side discussion, the proponent of JCTVC-H0133 also considers the other aspect to be beneficial.

For the second aspect, the spatial candidate is not compared against the TMVP candidate. This is asserted to simplify the checking, improve loss resilience, and improve parallelism. This has a loss of 0.1% for RA and 0.2% for LB.

Decision: Adopted (both aspects, text to be checked during meeting).

### Contributions

JCTVC-H0111 CE9: Modification of bi-prediction [Y. Suzuki (NTT DOCOMO)]

JCTVC-H0344 CE9: Cross-check report for tests BP01 and BP04 (JCTVC-H0111) [P. Helle (Fraunhofer HHI)]

JCTVC-H0583 CE9: cross-check of experiment BP02 and BP06 (JCTVC-H0111) [J. Jung (Orange Labs)] [late]

JCTVC-H0074 CE9: Cross-verification of BP03 (JCTVC-H0111) [S. Fukushima, M. Ueda (JVC Kenwood)]

JCTVC-H0217 CE9: Cross-check report of BP05 by Panasonic [T. Sugio, T. Nishi (Panasonic)]

JCTVC-H0625 CE9: Cross-check report on experiment BP07 and BP08 of JCTVC-H0111 [I.-K Kim (Samsung)] [late]

JCTVC-H0252 CE9: Results of SP experiments on simplification of merge process [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-H0143 CE9: Cross verification of Nokia’s proposal JCTVC-G593 [S. Lu, K. Sato (Sony)]

JCTVC-H0655 CE9: Motion information derivation of deformable block [Na Zhang, Xiaopeng Fan, Siwei Ma, Debin Zhao (Harbin Inst. Tech.)] [late]

## CE10: Deblocking filter

### Summary

JCTVC-H0040 CE10: Summary report of Core Experiment on deblocking filtering [A. Norkin, X. Guo, B. Jeon, M. Narroschke (CE coordinators)] [miss]

### Contributions

Subtest 1

JCTVC-H0084 CE10 Subtest 1: Test results on unification of luma and chroma filtering [M. Zhou (TI)]

JCTVC-H0147 CE10.1: Symmetric rounding in the deblocking filter [M. Narroschke, A. Kotra, T. Wedi (Panasonic)]

JCTVC-H0262 CE10 Subtest1: Cross-verification for Panasonic's proposal JCTVC-H0147 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0263 CE10 Subtest1: Cross-verification for TI's proposal JCTVC-H0084 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0298 CE10 Subtest1: Cross verification of MediaTek’s chroma deblocking filter (JCTVC-H0310) [Y.J. Chiu, L. Xu, Y. Han, W. Zhang(Intel)]

JCTVC-H0299 CE10 Subtest1: Cross verification of MediaTek’s simplified chroma deblocking (JCTVC-H0311) [Y.J. Chiu, L. Xu, Y. Han, W. Zhang(Intel)]

JCTVC-H0310 CE10 Subtest1: Chroma Deblocking Filter [Q. Huang, J. An, X. Guo, S. Lei (MediaTek)]

JCTVC-H0311 CE10 Subtest1: Simplified Chroma Deblocking Filter in JCTVC-H0310 [Q. Huang, J. An, X. Guo, S. Lei (MediaTek)]

Subtest 2

JCTVC-H0149 CE10.2: Cross check results for the deblocking filter line buffer reduction of Sony and MediaTek [A. Kotra, S. Esenlik, M. Narroschke (Panasonic)]

JCTVC-H0264 CE10 Subtest2: Cross-verification for Ericsson's proposal [M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0280 CE10.2.1: Reducing one pixel line buffer by modified deblocking filter for horizontal LCU boundaries [C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek), M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0472 CE10.2: Level restrictions on tiles for line buffer reduction [Andrey Norkin, Rickard Sjöberg (Ericsson)]

JCTVC-H0575 CE10.2: cross-verification of Sony's and Mediatec's proposal on deblocking line memory reduction [A. Norkin (Ericsson)] [late]

Subtest 3

JCTVC-H0108 CE10 Subtest3: Simplified BS calculation process in deblocking filter [S. Park, B. Jeon (LG)]

JCTVC-H0146 CE10.3: Simplified delta calculation in the deblocking filter [M. Narroschke, A. Kotra, T. Wedi (Panasonic)]

JCTVC-H0155 CE10.3: Simplification of First Delta Calculation in Deblocking by SKT/SKKU [J. Yang, J. Kim, K. Won, B. Jeon (SKKU), J. Lim (SKT)]

JCTVC-H0156 CE10.3: Simplification of Second Delta Calculation in Deblocking by SKT/SKKU [J. Yang, J. Kim, K. Won, B. Jeon (SKKU), J. Lim (SKT)]

JCTVC-H0157 CE10.3: Cross verification of SKT/SKKU's Simplification of Second Delta Calculation in Deblocking (JCTVC-H0156) [S. Lee, N. Eum (ETRI)] [late]

JCTVC-H0261 CE10 Subtest3: Cross-verification for Panasonic's proposal JCTVC-H0146 [M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0396 CE10 Subtest 3: Deblocking Filter Simplifications [G. Van der Auwera, X. Wang, M. Karczewicz (Qualcomm)]

This contribution contains two aspects, part 1 of which is within the scope of CE10 and part 2 of which was new. Part 2 should be considered to be outside of CE10.

JCTVC-H0209 CE10.3: Cross-verification Results of Qualcomm's Simplification of Deblocking (JCTVC-Hxxx) by SKT/SKKU [J. Yang, J. Kim, K. Won, B. Jeon (SKKU), J. Lim (SKT)] [late]

JCTVC-H0459 CE10.3: Cross Check of SKT/SKKU Proposal JCTVC-H0155 [G. Van der Auwera (Qualcomm)]

JCTVC-H0473 CE10.3: Deblocking filter simplifications: Bs computation and strong filtering decision [Andrey Norkin (Ericsson)]

JCTVC-H0618 Cross-verification of Ericsson's proposal on deblocking filter simplifications (JCTVC-H0473) [S. Park, B. Jeon (LG)] [late]

JCTVC-H0576 CE10.3: Cross-verifiaction of proposal on deblocking Bs simplification from LG [A. Norkin (Ericsson)] [late]

Subtest 4

JCTVC-H0148 CE10.4: Deblocking filter using adaptive weighting factors [A.-K. Seifert, T. Wedi, M. Narroschke (Panasonic)]

JCTVC-H0350 CE10 Subtest4: Cross-verification for Panasonic's proposal JCTVC-H0148 [M. Ikeda, T. Suzuki (Sony)] [late]

JCTVC-H0398 CE10 Subtest 4: Transform Dependent Deblocking Filter Parameter Adjustment [G. Van der Auwera, R. Joshi, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0656 CE10 Subtest 4: Cross-check of Qualcomm's Transform Dependent Deblocking Filter Parameter Adjustment JCTVC-H0398 [T. Yamakage, S. Asaka (Toshiba)] [late]

JCTVC-H0474 CE10.4: Cross check of Ericsson's proposal [G. Van der Auwera (Qualcomm)] [late]

JCTVC-H0574 CE10.4: deblocking parameters signaling [A. Norkin, K. Andersson, J. Enhorn, R. Sjöberg (Ericsson)] [late]

### Discussion and Conclusions

## CE11: Coefficient scanning and coding

### Summary

JCTVC-H0041 CE11: Summary report of Core Experiment on coefficient scanning and coding [V. Sze, T. Nguyen, K. Panusopone, Y. Piao, J. Sole (CE coordinators)]

Various methods have been proposed for coding the transform coefficients of the residual signal to reduce complexity and/or improve the coding efficiency. This core experiment evaluates the coding efficiency and complexity impact of:

- context modeling/selection for syntax elements related to transform coefficients

- signaling method (e.g. data hiding)

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient coding on the coding efficiency and complexity

- Coding efficiency and complexity measurement of transform coefficient scanning order methods

Complexity measures are described in Section 6.2.

| Doc# | Summary of Changes | AI HE | RAHE | LBHE | AI-LC | RALC | LBLC | RA-HE10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H0095 | Reduce number of neighbors (to 2) used in newly defined high frequency region | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | zero neighbors for high freq. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| H0282 | Group contexts of high frequency for 4x4 and 8x8 TU; sub-sample for 8x8 TU | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 |
| H0509 | Share luma and chroma for significant_coeffgroup_flag; for 16x16/32x32 TU, significant_coeff_flag calculated with max (ctxIdx, 3), ctxIdx based on 5 neighbors | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| H0352 | Remove all neighbors for 3 bin parallel processing. | 0.6 | 0.4 | 0.4 | 0.5 | 0.3 | 0.2 | 0.6 |
|  | Adapted method removing recently processed neighbors from context selection for 3bin parallel processing* | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.1 |
|  | 4bin parallel processing* | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| H0160 | Reduce number of contexts for ctxSet 1, 2, 4 and 5, 43 for coeff_abs_level_greater1_flag and 32 coeff_abs_level_greater2_flag, respectively. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| H0130 | Reduce number of ctxSets for luma level coding from 6 to 4. | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| H0224 | Hide sign of first non-zero coeff into parity of sum of non-zero coeff. | -0.6 | -0.5 | -0.6 |  |  |  | -0.4 |
| H0283 | Use vertical scan for 16x4 and 32x8 and horizontal scan for 4x16 and 8x32. Use position based context of 4x4 TU for 4x16/16x4, and use position based context of 8x8 TU for 8x32/32x8. |  | 0.1 | 0.2 |  | 0.0 | 0.0 | 0.1 |
| H0511 | Use diagonal scan for 4x16/16x4; Share luma and chroma for significant_coeffgroup_flag; significant_coeff_flag calculated with max (ctxIdx, 3) based on 5 neighbors | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 |  |
| H0443 | Use position based context of 8x8 TU for 4x16/16x4 |  | 0.1 | 0.1 |  |  |  | 0.0 |

*Not in original document G917 from Geneva Meeting, tests were planned and designed for harmonization with HM5.0 and were confirmed as part of CE11 on e-mail reflector on 01/12/12

Various non-CE proposals with similar targets

Major goal should be to make the design as clean and clear as possible, implementation friendly, without losing compression performance

H0352 (and other non-CE proposals) mainly target parallelism

BoG on significance map coding (G. Martin-Cocher) and level coding (V. Sze)

### Contributions

Subtest 1 Significance map coding

JCTVC-H0095 CE11.1 Simplified significant_coeff_flag context derivation at high frequency area [T. Ikai (Sharp)]

JCTVC-H0645 CE11.1 Cross-check report for Sharp's method 2.1 of simplified significant_coeff_flag context derivation at high frequency area (JCTVC-H0095) [T.Chujoh (Toshiba)] [late]

JCTVC-H0354 CE11: Cross-check results for subtest 3.1.1 [V. Sze (TI)] [late]

JCTVC-H0282 CE11.1.2: Simplified method for coding 4x4 & 8x8 significance maps [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-H0101 CE11: Crosscheck of MediaTek's significant_coeff_flag modification (JCTVC-H0282) [T. Ikai (Sharp)] [late]

JCTVC-H0616 CE11: Cross-check of subtest 3.1.2 (JCTVC-H0282) [Y. Piao, J. H. Park (Samsung)] [late]

JCTVC-H0377 CE11: Cross-check of Mediatek's proposal H0282 by BBC [M. Mrak, D. Flynn (BBC)] [late]

JCTVC-H0509 CE11 subtest 3.1.3: Simplification of significant map coding for CABAC [V. Kung, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-H0705 CE11.3.1.3: Crosscheck for Motorola Mobility's simplification of significant map coding for CABAC in JCTVC-H0509 [C.-W. Hsu, T.-D. Chuang, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-H0352 CE11: Test results for Subtest 3.1.4. [V. Sze (TI)]

JCTVC-H0519 CE11: cross check report for subtest 3.1.4 (JCTVC-H0352) [K. Panusopone (Motorola)] [late][miss]

JCTVC-H0162 CE11: Cross-check report for TI's proposal (Subtest 3.1.4 fixed context) [H. Sasai, K. Terada, T. Nishi (Panasonic)] [late]

Subtest 2 Coefficient level coding

JCTVC-H0134 Cross-check of CE11 subtest2: On context simplification for coefficients entropy coding(JCTVC-H060) [Y. Piao, J. H. Park (Samsung)] [late]

JCTVC-H0484 CE11: Crosscheck of JCTVC-G301 on context simplification for coefficients entropy coding  [C. Auyeung (Sony)] [late]

JCTVC-H0130 CE11 Subtest 2: Level luma context reduction [Y. Piao, J. Min, J. H. Park (Samsung)]

JCTVC-H0355 CE11: Cross-check results for subtest 3.2.2 [V. Sze (TI)] [late]

JCTVC-H0446 CE11: Cross-check of Samsung’s context number reduction for level coding in CABAC [J. Sole (Qualcomm)] [late]

JCTVC-H0160 CE11:Context reduction for coefficient level [K. Terada, H. Sasai, T. Nishi (Panasonic)]

Subtest 3 Sign coding

JCTVC-H0224 CE11: Sign Data Hiding [Gordon Clare, Felix Henry, Joel Jung, Stephane Pateux (Orange)]

This contribution presents CE11 subtest “Sign Data Hiding”, a method to embed sign information into the residue. More specifically, the sign of the first non-nul coefficient of a residual block is not always explicitly written in the bitstream. Instead, at decoding time, the sign is directly inferred from the parity of the sum of quantized coefficients, using a predefined convention. At encoding time, if the sign of the first non-nul coefficient and the parity do not match, one of the quantized coefficients is changed in order to obtain the desired parity. The quantized coefficient changed is the one providing the smallest Rate-Distortion (RD) penalty. To determine it, RD costs computed during the RDOQ stage are used. This alternate way of transmitting the sign of the first non-nul coefficient is dependent on a decoder-side decision function. The proposal yields an average BD-rate compression performance of -0.6% (I), -0.5% (RA), -0.6% (LDB) in High Efficiency configurations with an average encoder runtime of 103% (I), 103% (RA) and 103% (LDB) and decoder runtime of 99% (I), 99% (RA) and 100% (LDB) compared to HM5.0 anchors.

Non-RDOQ version is reported in H0227 with similar results

Impact on additional decoder operation? Required to check the parity of residues, depends on number of zero/nonzero coefficients in block. One expert mentions that he has checked this since the last meeting from the viewpoint of hardware implementation and it is likely not to have a major impact on decoder complexity. It would not affect throughput, and the additional logic would be minimum.

Visual distortion in RDOQ off case? not observed (Madhukar Budagavi to check)

Several experts express opinion that this is providing interesting gain vs. moderate increase of complexity

H0481 is also related – see further conclusion there.

JCTVC-H0651 CE11: cross verification of sign data hiding from Orange Labs (H0224) [Zhan Ma, Ankur Saxena (Samsung)] [late]

JCTVC-H0480 Cross-check of CE11 Subtest3: Orange Labs’ Sign Data Hiding [J. Wang, X. Yu, D. He (RIM)] [late]

Subtest 4 NSQT/SQT remapping

All three proposals suggest defining specific scan methods depending on the non-square TU size. Questionable whether this is a better solution than the current HM method where sub-block scanning as borrowed from 16x16 is applied for all. Would be desirable to even go further and define a unique way of scanning and context definition across all square and non-square block sizes. Some of the non-CE proposals in significance scanning may point in that direction – further discuss in the BoG.

JCTVC-H0511 CE11 subtest 3.4.4: Entropy coding for non square blocks [V. Kung, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-H0245 CE11: cross-verification of Motorola's proposal [X. Zheng (HiSilicon)] [late]

JCTVC-H0283 CE11.4.2: Modified NSQT coefficient scan [C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0356 CE11: Cross-check results for subtest 3.4.2 [V. Sze (TI)] [late]

JCTVC-H0443 CE11: Harmonization of residual coding and NSQT [J. Kim, B. Jeon (LGE), J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-H0402 CE11 subset 4: Cross-check report of JCTVC-H0443 [J. Xu, A. Tabatabai (Sony)]

### Discussion and Conclusions

# Non-CE Technical Contributions

## Clarification and Bug Fix Issues

JCTVC-H0486 Suggested figures for the HEVC specification [C. Fogg (Harmonic)]

## HM settings and common test conditions

## HM coding performance

JCTVC-H0063 Coding efficiency comparison between HM5.0 and JM16.2 based on PQI, PSNR and SSIM [Yin Zhao (Zheijang Univ.)]

JCTVC-H0360 Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

JCTVC-H0562 Informal Subjective Quality Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz, S. Xu (eBrisk Video Inc.), B. Li (University of Science and Technology of China), G. J. Sullivan, J. Xu (Microsoft Corp)]

## Profile/level definitions

JCTVC-H0116 [AHG8] Objective and subjective evaluation of HM5.0 [TK Tan, A Fujibayashi, J Takiue (NTT DOCOMO)]

JCTVC-H0168 Proposal of HEVC profile/level definitions [T. Suzuki (Sony), H. Sasai, T. Nishi (Panasonic)]

JCTVC-H0353 AHG8: Comments on HEVC version 1 Profile and Level definition [M. Zhou, V. Sze, H. Tamama (TI)]

JCTVC-H0668 Profiles and Level considerations [Chad Fogg (Harmonic)]

JCTVC-H0637 Profiles and Levels for HEVC [Ajay Luthra (Motorola)] [late]

## Source video test material

JCTVC-H0294 YUV4:4:4 test sequences for screen content coding [Jiaomeng Cao, Kailun Zhou, Shuhui Wang, Tao Lin (Tongji Univ.)]

JCTVC-H0389 New Class E videoconferencing source content [W. Jang, J. Boyce, D. Hong (Vidyo)] [late]

## Functionalities

### Low delay

JCTVC-H0487 Importance of very high bit rate and ultra-low latency applications [Alberto Duenas, Francisco Javier Roncero (Cavium)]

This contribution asserts that there is a rising importance of some applications that require video compression standards that allow for very high bit rates to be supported and ultra-low latency to be achieved.

Application characteristics were asserted to include up to 100 Mbps or higher, in some cases less than one frame of latency, and 4:4:4. Entropy coding throughput and entropy coding latency were asserted to be important issues. Operation of the CPB with sub-frame decoding units was proposed.

Discussed applications included local wireless display and wireless cameras.

JCTVC-H0471 Proposed Mechanism for Improving Ultra Low Delay Coding Efficiency [O. Bar-Nir (Harmonic)] [late]

This contribution proposes an approach for ultra low delay coding in HEVC standard. Intra coded pictures in HEVC are significantly larger than inter coded pictures, hence they require significantly longer transmission delay. The extra delay increases with the ratio between the sizes of intra and inter pictures. Ultra low delay coding requires a more uniform bit distribution between pictures, and preferably also within the picture. A way to achieve more uniform distribution of bits between pictures is to replace the I-pictures with a progressive refresh mechanism, using rows or columns of Intra CUs.

It was asserted that row intra refresh produces an uneven distribution of bits over the top-to-bottom representation of the picture.

The main hurdle in implementing ultra low delay in HEVC was asserted to be the various in-loop filters that "contaminate" refreshed pixels that are filtered with non-refreshed pixels.

Past ultra low delay related proposals such as JCTVC-G183 and others proposed similar approach to low delay coding, but were suggested to have been rejected due to added burden on the decoder (JCTVC-E600). This proposal suggests an alternative approach that is asserted to have a minimal impact on the decoder.

It was proposed to provide a means for the encoder to inform the location of the refresh line to the decoder, in the PPS or in the slice header, or by using a dedicated SEI message and a VUI flag. (In discussion, the APS was suggested as being better.) The decoder would then use this information to disable deblocking, SAO and ALF across the refresh line.

It was proposed to disable in-loop filtering across the refresh border.

Tiles were suggested to be a way to do this. It was commented that the current tile design might need some modification to do this better and achieve low latency.

It was commented that such a scheme may produce visual artifacts.

It was commented that column refresh is not necessarily the only or best approach, such that it may not be desirable to customize the design and decoding process for that specific scheme.

It was commented that with non-normative control on the encoder side and recovery point SEI messages, and perhaps with disabling of loop filtering across slice boundaries (or disabling it altogether), there are various ways to achieve the described "gradual decoding refresh" capability.

For further study.

### Scalable coding

related to HL syntax category

JCTVC-H0388 High level syntax hooks for future extensions [J. Boyce, S. Wenger, W. Jang, D. Hong (Vidyo), Y.-K. Wang, Y. Chen (Qualcomm)]

Hooks to the high-level syntax of the HEVC design are proposed to enable future scalable and 3DV extensions in a backwards-compatible manner, maintaining the existing temporal scalability capability.

The NAL unit header design is proposed to be revised and a Video Parameter Set (VPS) is proposed.

The proposed VPS provides a common set of parameters that describes the CVS for all layers (including, e.g. dependency structure information). Some syntax of the current SPS is moved to the CVS.

Temporal layers are called sub-layers rather than layers.

A Group Parameter Set (GPS) is also proposed, with referencing of multiple different types of parameter sets in the slice header with only one parameter set ID. The GPS identifies the VPS, SPS, PPS and APS. The slice header carries a GPS ID (and no other PS IDs).

A 5-bit layer_id field is proposed to be added to the NAL unit header, while maintaining a 2-byte NAL unit header length, fixed for both VCL (currently 2 bytes) and non-VCL NAL units (currently 1 byte), in a manner intended to enable allowing Media Aware Network Elements (MANEs) to perform bitstream adaptation based on either or both of layer_id and temporal_id. To make room for the 5 bit layer_id, the output_flag and 4 reserved bits are removed. It is proposed for the output_flag to be moved to the PPS or slice header. It was remarked that moving the output_flag out of the NAL unit header is desirable regardless of the rest of the proposal.

For the first version of the standard, the layer_id would be require to be zero, so it would not really make any difference whether those bits are called layer_id or reserved_5zero_bits in the first version of the standard.

So the immediate request in the proposal, as far as the effect on the NAL unit header is concerned, is to make the NAL unit header one byte longer for non-VCL NAL units and to move the output_flag elsewhere.

The proposed layer_id is somewhat similar to the prior priority ID.

An example design that support both scalability and 3DV is provided for information in JCTVC-H0386, to illustrate how the proposed high-level syntax hooks enable future scalable and 3DV extensions in a way that is backwards-compatible with the base HEVC specification.

Revisit.

JCTVC-H0386 Information for scalable extension high layer syntax [J. Boyce, S. Wenger, W. Jang, D. Hong (Vidyo), Y.-K. Wang, Y. Chen (Qualcomm)]

Provides an explanation of the type of use envisioned in a next phase of work for the hooks proposed in JCTVC-H0388.

JCTVC-H0410 Simple NAL Unit Header for HEVC [J. W. Kang, H. Lee, J. S. Choi, T. C. Thang (ETRI)]

NAL unit header is crucial for a network node to process packet by packet. This contribution proposes some options for the NAL unit header that can support extensions of HEVC. The main objective of the proposed options is to have self-contained packet header so as to facilitate simple adaptation at the packet level.

The contribution proposes two variants for the NAL unit header syntax.

In both variants, the NAL unit header is variable length and has a scalability information flag that determines whether some scalability information is present in the header or not.

The first option has flags to indicate the types of other scalability-related fields that are present in the NAL unit header and those flags have a fixed length when present.

The second option uses higher-level information (e.g. SPS level) to determine how to parse the NAL unit header.

In the discussion of the proposal, it was commented that the variable length nature and dependency structure of the NAL unit header size and parsing may be a burden on MANEs and, to some extent, on decoders.

JCTVC-H0701 Hooks for scalable extensions [M. M. Hannuksela (Nokia)] [late]

The proposal can be summarized to contain the following aspects:

- A component picture is defined, which in the current HEVC WD is the coded picture of an access unit, and in the future scalable extensions would be for example a view component, a depth map, or a layer representation.

- A component picture delimiter NAL unit, which may carry some of the slice header syntax elements and, in the future, scalable extensions could carry scalability properties of component pictures. (In discussion, it was remarked that this is similar in concept to a picture header or APS, with a predictive updating scheme.)

- A 2-byte NAL unit header including a component picture delimiter ID (cpd_id).

- A prediction mechanism of slice header parameters between component picture delimiter NAL units of the same access unit to reduce the slice header byte count overhead in the future scalable extensions.

- Sub-bitstream extraction based on cpd_id.

Open.

JCTVC-H0566 AHG15: Temporal layer access pictures [J. Samuelsson, R. Sjöberg (Ericsson)]

This contribution presents a proposal for the signalling of Clean Random Access (CRA) pictures and Temporal Layer Switching Points in what is referred to as Temporal Layer Access (TLA) pictures. It is proposed to replace the CRA Network Abstraction Layer (NAL) unit type with a TLA NAL unit type. The TLA NAL unit type imposes constraints on the bitstream but does not in this contribution have any impact on the decoding process.

It is stated in the contribution that both random access information and temporal layer switching information is of high value to a network node and thus should be available in NAL unit header, independent of data outside that NAL unit header, specifically the Sequence Parameter Set (SPS) and Picture Parameter Set (PPS). It is further stated that a unified syntax and definition of TLA pictures makes the standard text more readable and comprehensive.

The proposal is essentially a repetition of JCTVC-G584.

It was noted that there are some switching that can be expressed with the current syntax that cannot be expressed with the proposal.

Some support.

It was commented that the structuring of the TLA and CRA definitions might be better as independent definitions.

Revisit.

JCTVC-H0568 AHG15: Specification of bitstream subsets [J. Samuelsson, R. Sjöberg (Ericsson)]

This document analyses some high level aspects of the HEVC Working Draft text and proposes to include text regarding bitstream conformance and specification of bitstream subsets. More specifically the following is proposed:

- Copy AVC/H.264 bitstream conformance section into the HEVC WD and split it into two conformance points

  - Output timing conforming bitstreams (copied from H.264/AVC) – in principle, we already have this.

  - Output order conforming bitstreams

- Copy SVC specification of bitstream subsets for temporal_id with the following changes

  - Separate output timing conformance from output order conformance and make output timing conformance optional

  - Add a sub-bitstream extraction process for removal of individual access units

- Make all information needed for sub-bitstream extraction available in fixed length without dependency on PPS or SPS

  - Temporal Layer Access pictures for defining temporal layer switching points (JCTVC-H0566)

  - Add a fixed length copy of picture order count into the access unit delimiters

The concept of making output timing conformance optional from the bitstream perspective would be a departure from past practice. It was commented that it would become difficult to determine whether a bitstream could be decoded at full speed or not or with any limit on the required buffering capacity if there is no assurance of bitstream timing conformance. This could enable creation of "evil bitstreams" from the decoder perspective.

The proposed temporal sub-bitstream extraction process enables the extraction process to vary the layer that it is extracting as the bitstream is processed from beginning to end – i.e., to operate using "adaptive bitstream thinning". It was remarked that we believe we may have already achieved this – at least in VBR operation – such that there may not be a need to describe such an operation explicitly in the standard.

It was agreed that we should have at least the AVC style of extraction and conformance supported for temporal layers in HEVC as it is in AVC. This is already our implicit understanding, if not fully expressed in the draft text.

It was commented that the interaction with the system level is conceptually difficult to understand and specify, and trying to change our understanding about that high level behaviour substantially while trying to finalize the basic coding design may be overly ambitious.

The contribution includes a proposal to send POC in the AUD as well as sending the POC LSBs in the slice header. (Without any normative effect on the decoder.) This idea was mentioned at the preceding meeting.

It was discussed whether, if we do this, we should send the entire POC value (including the inferred MSBs) or just the LSBs as sent in the slice header. In previous work, we have generally considered it unnecessary to send the MSBs of the POC, as only relative order is necessary to understand from the bitstream.

It was discussed whether primary_pic_type is actually useful. (Potentially removing this was mentioned in the JCTVC-H0386, which is an information document.) It was remarked that it at least lets the decoder detect intra pictures (including those that are not CRA pictures).

Left open.

JCTVC-H0670 Downsampling filter for anchor generation for scalable extensions of HEVC [Jie Dong, Yong He, Yan Ye (InterDigital)] [late]

Outside the current scope of work (misregistered).

JCTVC-H0051 Sharp proposals for HEVC scalability Extension [H. Kumai, T. Yamamoto, A. Segall, N. Ito (Sharp)] [late]

Outside the current scope of work.

### Adaptive resolution coding

JCTVC-H0234 AHG18: Design considerations for Adaptive Resolution Coding (ARC) [T. Davies (Cisco)]

This contribution considers various ARC features from a complexity perspective. It proposes level constraints on the number of reference resampling operations per frame period and the frequency with which resolution can be changed. A variety of short-length reference resampling filters were evaluated on low delay configurations, and their effects were reported to be transient, especially for up-sampling. For reference up-sampling, both simple linear and DCTIF upsampling filters had no loss on average against an anchor employing common filtering for reference and input/output pictures. For reference down-sampling, a 3-lobe Lanczos-windowed filter was observed to perform best of those tested against the anchor with mean luma BD-Rate losses of 0.45%.

Particularly for high QP, coding gains were reported previously in some cases in JCTVC-G264.

Constraints were proposed for how often the resolution could change (in output order) and how many reference pictures could be resampled when it happens. It was commented that it may be beneficial to constrain the resolution changes in output order as well as in decoding order.

It is proposed for collocated motion vectors to be marked as not available when resolution differs.

It was remarked that unless the higher resolution uses a coded picture size that is a multiple of two times the SCU size, there would be different alignment effects at the border. In the proposal software, padding was proposed.

Various short filters were tested for the normative reference resampling filters and the non-normative encoder-only filtering. Only 2:1 resolution changes were tested. It was reported that for the upsampling filters, it basically didn't matter what the filters were, and a co-sited linear interpolation filter was suggested. For downsampling, it mattered a bit more, and a 3-lobe Lanczos filter (5 non-zero taps [-1 0 9 16 9 0 -1]) was suggested.

A participant suggested alternatively considering the resampling filters that had been proposed in JCTVC-G264.

It was commented that such a feature, if adopted, might need to be left out of some profiles (e.g. not used for broadcast).

It was commented that the relationship with SVC should be studied. The proponent asserted that DRC is a simpler approach than SVC, and may have better coding efficiency.

JCTVC-H0185 Cross-check of Adaptive Resolution Coding (ARC) [G. Van Wallendael, S. Van Leuven, J. De Cock, R. Van de Walle (Ghent Univ.)] [late]

JCTVC-H0321 AHG18: Resolution Adaptation Coding (ARC) using single resolution in DPB [P. Bordes, P. Andrivon (Technicolor)]

This contribution presents a method for implementing Adaptive Resolution Coding (ARC) in which the DPB holds one coded size for each picture, which is its decoded size. The resampling and motion compensation filtering are combined in the decoding process (which lengthens the filter). It is reported the scheme has encoding gains, relative to a different approach for dynamic resolution change, of between 0.1% to 1.8% in Luma and up to 4.9% in Chroma on average depending on configuration cases and scenarios (Low-to-High or High-to-Low). The DPB memory size is reportedly reduced at the decoder side, compared to previous ARC proposals, by avoiding storage of more than one resolution for each picture in the DPB. At the encoder side, both resolutions are still necessary to compute motion estimation only. The proponent indicated an interest in investigating the use of shorter filters. Further study would be needed to determine the potential value for such an approach.

JCTVC-H0351 AHG18: Cross-check of JCTVC-H0321 by Cisco [T. Davies (Cisco)] [late]

JCTVC-H0447 Reduced Resolution Update Mode for enhanced compression [Alexis Tourapis, Lowell Winger (Magnum)]

(Informative contribution.)

This contribution proposes the consideration of Reduced Resolution Update (RRU) techniques (e.g. as in H.263 Annex Q) as potential methods for enhancing encoding performance for video compression. RRU techniques allow encoding a video signal by, commonly predicting that signal at full resolution while encoding any residual information at a lower resolution. Such information needs to be up-sampled before used for the reconstruction of the final image. It has reportedly been shown in the past that RRU methods can provide considerable coding advantages, both subjectively and objectively, especially at lower rates, versus full resolution as well as reduced resolution encoding. The proponent indicated that, unlike what was reported in previous contributions on the same topic, additional coding advantages or flexibility may be achieved by considering applying RRU concepts separately at different color components or color component groups, as well as extending RRU methods at the slice or region, and even at the coding unit level. Support also of down-sampling of not only the residual but also of the prediction signal was suggested to also be considered. This was asserted to essentially make the RRU process a superset that already potentially accounts for the techniques already presented in Adhoc Group 18. It was suggested by the submitter that such methods should be carefully evaluated.

It was noted that there is some overlap in concept between RRU and the use of large block sizes (for MC and residual) and possibly selective dropping of high-frequency coefficients.

### Color component sampling

JCTVC-H0394 Coding in 4:4:4: Preferred Conversions in Color and Sampling [P. Topiwala (FastVDO)]

It was asserted that, for the most part, cameras capture, and displays render, RGB 4:4:4, whereas the HEVC draft design works in YUV420. Two key components are needed to make the connection between what is captured/rendered, and the codec.  These are color transforms, and resampling filters. The contribution proposed integer color transforms and resampling filters. The resampling filters are targeted for dynamic resolution conversion, and correspond to the prior proposal G264.

It was asked whether the proposed filters had been tested for screen content.

JCTVC-H0065 Mixed Chroma Sampling-rate coding: combining the merits of 4:4:4 and 4:2:0 and increasing the value of past 4:2:0 investment [Tao Lin, Shuhui Wang (Tongji Univ.)]

Presenter not available Thu 02-02.

JCTVC-H0073 4:4:4 screen content coding using Macroblock-Adaptive Mixed Chroma-Sampling-Rate [Shuhui Wang, Tao Lin (Tongji Univ.)]

Presenter not available Thu 02-02.

JCTVC-H0457 Removal of Monochrome Chroma Format Support [T. Hellman, W. Wan (Broadcom)]

This submission proposes a claimed simplification to the standard by the removal of monochrome chroma format.  It contends that the gain from adding specific monochrome support versus coding monochrome material in 4:2:0 is likely quite small, and in any case only applies to a small fraction of video source material.  The proposal recommends removing support for the format, or performing further study to measure the actual gain for a future decision.

It was remarked that alpha has been supported using auxiliary coded pictures in AVC, and the separate colour plane scheme in AVC is also built on the monochrome concept.

Monochrome could be left out of a profile if that is desirable. If it is left out of a profile, there should be some (e.g. VUI) indicator that the decoded video is actually monochrome. Consideration of true monochrome may be best left to some later phase of work when the same parts of the design are being modified to also support 4:4:4. Decision: We will have some profile that does not support monochrome, and an indicator flag in VUI. If feasible, we would like the flag to indicate that the chroma will be exactly 128 if decoded.

JCTVC-H0650 AHG20: 4:2:2 support in HEVC [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

This contribution proposes support of 4:2:2 format in the HEVC standard, which could expand opportunities of its market adoption especially in digital broadcasting area. The main point of this proposal is to introduce modifications as small as possible in order to accommodate 4:2:2 format. Modifications of the current specification to support 4:2:2 format were described (at the concept level, without draft text). The proposal requested no change of syntax other than sending additional transform coefficients.

It was commented that 4:4:4 is conceptually simpler to support – basically applying luma-like processing for all components.

For the residual coding there are basically two possible approaches, which would be to use more blocks or to use vertically larger blocks. This proposal is to use larger blocks. Both approaches seem desirable to investigate.

To be studied in AHG.

### Interlace

JCTVC-H0056 AHG13: On interlace indication [K. Chono, H. Aoki (NEC)]

Summarized in AHG13 report as:

- Move the VUI flag in the AHG13 starting text to SPS and modify its definition to specify the type of the associated coded sequence in addition to the presence of the SEI message in each of access units.

- One-bit interlace field indication in SEI message to indicate the parity of each field.

- Modified access unit delimiter RBSP to indicate dynamically changing source material properties.

The first element proposes two aspects: 1) moving the location of the SPS-level flag, and 2) defining the flag as an indication that the coded video sequence is interlaced fields.

The second element proposes to reduce the length of the field in the SEI message to use only 1 bit rather than 4 bits.

The last element was provided as information, rather than for immediate adoption.

JCTVC-H0588 AHG13: Proposed interlace format indication for HEVC [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

Summarized in AHG13 report as:

- Define a constraint that a coded sequence shall contain coded pictures consisting of frames only (progressive or interlace) or fields only.

- One-bit interlace format indication in SPS or VUI

- One-bit interlace field indication in SEI message to indicate the parity of each field.

Very similar to JCTVC-H0056.

It was remarked that the question of whether the content is coded as fields is different than the question of whether the input sampling structure was progressive or interlaced.

It was suggested to avoid the use of interlaced sampling as a concept in the specification – rather, simply using the geometric concept of top and bottom fields (although a note with some acknowledgement of a relationship may be appropriate).

It was suggested that it may be desirable, when frames are coded, to enable an indication that the frames are interlaced and which field is temporally first.

JCTVC-H0666 Interlace Indicators [Chad Fogg (Harmonic)][late]

Summarized in AHG13 report as:

- A flag to indicate the scanning method of the original source of a coded sequence.

- A flag to indicate whether a coded sequence contains frames only or fields only.

- A flag to indicate the parity of each field.

- A flag to indicate whether the coded frame or field is a "dummy frame" resulting from a repetition process, such as 3:2 pulldown.

One thing not in the proposal was the aforementioned indication which field is temporally first when a picture is a frame. A suggestion was to provide a flag to indicate a conceptual pairing of fields – i.e. which field is first in a pair.

The 3 remaining bits of the byte would be reserved.

We still want a flag to indicate whether all pictures are frames vs. all pictures being fields.

When that flag indicates fields, the picture-level SEI must be present on all pictures. When the flag indicates frames, the presence of the picture-level SEI is optional (and may be on some pictures and not others). The name of the flag and the semantics of the flag will not refer to "interlace".

Decision: Adopt as modified by the above.

BoG (K. Chono & C. Fogg) to prepare text. Revisit to review draft text that reflects this.

Probably re-establish the AHG.

JCTVC-H0222 Introduction of AVC's derivation process of chroma motion vector for interlace format support [K. Kazui, J. Koyama, S. Shimada, A. Nakagawa (Fujitsu)]

Summarized in AHG13 report as:

- SPS flag to specify the scanning method of an input source.

- One-bit interlace field indication in slice header when SPS flag indicates that the input source is interlaced scan.

- AVC-style chroma MV phase adjustment.

Proposes part of the prior JCTVC-G196, field motion compensation for chroma as in AVC.

Claimed improvement: 0.4 dB average BD PSNR benefit (up to 1 dB) for chroma components (using self-selected interlaced content). No cross-verification was provided.

It was asked whether the improvement is visible. It was remarked by a non-proponent that there previously has been a conclusion that there is some visual benefit in AVC chroma artifact reduction as a result of this technique.

If adopted, we might need to have a profile that excludes it. It was commented that if we are willing to customize the decoding process for field coding, we should probably not just to this but should go further – however, we really don't plan that. No action taken.

### Robustness against transmission loss

JCTVC-H0072 NAL Unit loss software [Stephan Wenger (Vidyo)] [late]

Attached is a NAL unit loss simulator (loss.c) that can be used to apply error pattern files to a bitstream according to Annex B bitstream format.  Further included is a converter program (convert.c) that converts the error pattern files as generated by the JCTVC-G1050 software to a more compact interim format—the same format used previously in JCT-VC, as well as converted error pattern files.  Both software and patterns could benefit from testing.

This software seems useful for loss testing purposes, and its use is encouraged. We will put it, along with the example error patterns, into the reference software tools package for distribution as part of the reference software. Agreed.

## Deblocking filter

JCTVC-H0190 Non-CE10: Difference-QP-based Deblocking [K Sato (Sony)]

JCTVC-H0644 Non-CE10: Cross-verification report for difference-QP-based deblocking (JCTVC-H0190) [H. Aoki, K. Chono (NEC)] [late] [miss]

JCTVC-H0275 Non-CE10: Introduction of strong filter clipping in deblocking filter [M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0587 Non-CE10: Cross check of the introduction of strong filter clipping in deblocking filter of Sony (JCTVC-H0275) [Matthias Narroschke, Anand Kotra, Semih Esenlik (Panasonic)] [late]

JCTVC-H0281 Non-CE10: Removing QP line buffer [Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0639 Cross-check report on Removing QP line buffer (JCTVC-H0281) [I.-K Kim (Samsung)] [late]

JCTVC-H0424 Non-CE10 Subtest 4: On Deblocking Parameter Signalling [G. Van der Auwera, Y.-K. Wang, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0577 Cross-verification of Qualcomm's proposal JCTVC-H0424 on deblocking filtering [A. Norkin (Ericsson)] [late]

JCTVC-H0448 AHG6: Deblocking of IPCM Blocks Containing Reconstructed Samples [G. Van der Auwera, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0578 AHG6: Inspection report on deblocking of IPCM Blocks containing reconstructed samples (JCTVC-H0448) [K. Chono, H. Aoki (NEC)] [late]

## Non-deblocking loop filters

### Adaptive loop filter

JCTVC-H0094 Non-CE8: Modification of directional classification for block-based adaptation on ALF [M. Matsumura, S. Takamura, A. Shimizu, H. Jozawa (NTT)]

JCTVC-H0158 Non-CE8: Cross-check of modified directional classification for block-based adaptation on ALF (JCTVC-H0094) [T. Yoshino, K. Kawamura, S. Naito (KDDI)]

JCTVC-H0136 Non-CE8: Report on visual artifacts by ALF virtual boundary processing and proposal on the filtering process modification [O. Nakagami, M. Ikeda, T. Suzuki (Sony)]

JCTVC-H0163 Cross-check of JCTVC-H136 [A. Ichigaya, S. Iwamura (NHK)] [late]

JCTVC-H0372 Non-CE8: Line memory reduction fix at independent slice and tile boundaries [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-H0661 Non-CE8: Crosscheck for Panasonic's line memory reduction fix at independent slice and tile boundaries in JCTVC-H0372 [C.-Y. Chen, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0415 Non-CE8: ALF BA feature computation only accessing 3x3 area [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-H0683 Non-CE8: Cross check of Modification of ALF BA classification [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-H0417 Non-CE8: Supplementary analysis of ALF with single shape [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-H0418 Non-CE8: Unification of k-index mapping for filter coefficients of the two ALF shapes [P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-H0419 Non-CE8: Techniques in LCU-based ALF in CE8.a.2 and CE8.a.4 [P. Lai, F. C. A. Fernandes (Samsung)] [late]

JCTVC-H0420 Non-CE8: Variable codebook size for LCU-based ALF [Zhan Ma, P. Lai, F. C. A. Fernandes (Samsung)]

JCTVC-H0097 Subjective quality assessment of ALF on still images [Y. Yasugi (Sharp)]

JCTVC-H0198 Comments on Slice-independent Loop Filtering [M. Li, P. Wu (ZTE)]

JCTVC-H0382 Inclusion of adaptive loop filtering in the HEVC standard [T. Yamakage (Toshiba), I. S. Chong (Qualcomm), Y.-W. Huang (MediaTek)]

JCTVC-H0409 Harmonization of number of ALF classes between RA and BA modes [Ehsan Maani, A. Tabatabai (Sony)]

JCTVC-H0674 Non-CE8: Cross-check of H0409 on Harmonization of number of ALF classes between RA and BA modes [?? (??)] [late] [miss]

JCTVC-H0483 Harmonization of ALF Luma and Chroma Center Coefficient Prediction [J. Zhao, A. Segall (Sharp)]

JCTVC-H0571 ALF with multi shapes versus ALF with single shape [Faouzi Kossentini, Hsan Guermazi (eBrisk Video Inc.), Wang Lai, Felix Fernandes(Samsung)]

### Sample adaptive offset

JCTVC-H0067 Non-CE8.a.2: SAO with LCU-based syntax [A. Fuldseth, G. Bøntegaard (Cisco)]

JCTVC-H0711 Cross-verification of Non-CE8.a.2: SAO with LCU-based syntax (JCTVC-H0067) [W.-S. Kim, M. Zhou (TI)] [late]

JCTVC-H0088 Non-CE8.a.2: Cross-check of SAO with LCU-based syntax (JCTVC-H0067) [T. Nishi, K. Uchibayashi (Panasonic)] [late]

JCTVC-H0114 Non-CE8: Sample adaptive offset with constrained ranges of edge offset [E. Alshina, A. Alshin, J. H. Park (Samsung), I. S. Chong, M. Karczewicz (Qualcomm), C.-M. Fu, C.-I. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0183 Non-CE8: Cross-check report for sample adaptive offset with constrained ranges of edge offset (JCTVC-H0114) [T.Chujoh (Toshiba)] [late]

JCTVC-H0115 Sample adaptive offset in Low Complexity test [E. Alshina, A. Alshin, J. H. Park (Samsung)]

JCTVC-H0579 Cross-verification of sample adaptive offset in low complexity test (JCTVC-H0115) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-H0201 Non-CE8: Method of removing boundary artifacts for SAO [T.Matsunobu, H.Sasai, T.Nishi(Panasonic)]

JCTVC-H0439 Cross-verification of Non-CE8: Method of removing boundary artifacts for SAO (JCTVC-H0201) [W.-S. Kim, D.-K. Kwon (TI)] [late]

JCTVC-H0438 Non-CE8: New Chroma Band Offset (BO) Design in SAO [W.-S. Kim, D.-K. Kwon (TI), Y. Yasugi, T. Ikai (Sharp)]

JCTVC-H0338 Non-CE8: Crosscheck for Sharp and TI's BO in JCTVC-Hxxx [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0552 Non-CE8: Modified EO offset operation and offset quantization [K. Minoo, D. Baylon (Motorola Mobility)]

JCTVC-H0553 Non-CE8: Harmonization of LCU-based SAO [K. Minoo, Y. Yu, D. Baylon, L. Wang (Motorola Mobility)]

JCTVC-H0708 Non-CE8: Crosscheck for Motorola Mobility's harmonization of LCU-based SAO in JCTVC-H0553 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-H0406 Flexible Band Offset Mode in SAO [Ehsan Maani, O. Nakagami (Sony)]

JCTVC-H0617 Cross-verification of Flexible Band Offset Mode in SAO (JCTVC-H0406) [W.-S. Kim, M. Zhou (TI)] [late]

### Combined approaches

JCTVC-H0277 Non-CE8: Additional simulation results of SAO and ALF [T.-D. Chuang, Y.-W. Chen, J.-L. Lin, Y.-W. Huang, S. Lei (MediaTek)]

## NSQT/SDIP harmonization

Note AHG report JCTVC-H0016.

JCTVC-H0347 AHG16: SDIP unified with NSQT [X. Cao, Y. He (Tsinghua), X. Peng (USTC), C. Lai, X.Zheng, L. Liu, J. Zheng (HiSilicon), J. Xu (Microsoft), H. Yang, J. Song, H. Yu (Huawei), J. Lim, B. Jeon (LGE), J. Sole, L.Guo, X. Wang, M. Karczewicz (Qualcomm), J. Xu, E. Maani, A. Tabatabai (Sony), X. Zhang, S. Liu, S. Lei (MediaTek)]

This contribution reports the results of the unification of SDIP and NSQT. SDIP with 2NxN/Nx2N and 2NxhN/hNx2N PU were tested with the same non square TUs which are the same with NSQT. Results with or without 8x2/2x8 TU are also reported. For all intra HE case, when 2x8/8x2 TU are used, 1.8% and 1.7% gain can be achieved with 128% and 130% encoding time increase for SDIP with 2NxN/Nx2N PU and SDIP with 2NxhN/hNx2N PU respectively. Without 2x8/8x2 TU, for SDIP with 2NxN/Nx2N PU and SDIP with 2NxhN/hNx2N PU, the gain is 1.0% and 1.0%, with 117% and 119% encoding time increase. No obvious decoding time increase was observed.

Presentation to be uploaded.

“hN”=”Half N”

Some reservations expressed against 2x8 and 8x2 modes. These are also  not in harmony with current non-square inter coding

How is de-blocking done? At 8x8 boundaries, as in NSQT.

For intra-only coding without 8x2, no remarkable difference between Nx2N and hNx2N

2 WD texts included, one for Nx2N, one for hNx2N. None of them includes 2x8/8x2.

For RA coding, BR gain is 0.9% with the 2x8/8x2 modes, and 0.4% without.(Nx2N/2NxN), and a little bit more for hNx2N.

More information is requested to understand what are the detailed changes

The encoder implementation uses an early skip decision for non-square blocks.

A short review of the WD text version for Nx2N was done which unveiled that various changes are necessary to the decoding process of intra (PU, MPM, mode derivation, transform, deblocking filter)

A new version was also shown which is not uploaded yet

Concerns expressed about the maturity of the text, amount of changes and implications on decoder, also about complexity of encoder.

No action.

JCTVC-H0106 AHG16: Crosscheck of SDIP and NSQT harmonization (JCTVC-H0347) [T. Yamamoto (Sharp)] [late]

JCTVC-H0300 AHG16: Crosscheck of SDIP and NSQT unification (JCTVC-H0347) [Y.J. Chiu, L. Xu, W. Zhang, Y. Han (Intel)]

JCTVC-H0455 AHG16: Harmonization of 2NxN/Nx2N Intra PU with SDIP and NSQT [X. Zhang, S. Liu, C.-W. Hsu, T.-D. Chuang, S. Lei (MediaTek)]

This contribution proposes a set of methods for harmonization of 2NxN/Nx2N PU with SDIP and NSQT. The coding efficiency and run-time of the proposed methods built on top of AHG 16 software are reported. Experimental results report average 1.8% BD-rate reduction for All Intra HE with encoding runtime increased by 32%1 and decoding runtime increased by 3% with 8x2/2x8 transform enabled. Experimental results report average 1.4% BD-rate reduction for All Intra HE with encoding runtime increased by 31% and decoding runtime increased by 2% when 8x2/2x8 transform disabled. With further encoder complexity reduction, experimental results report average 1.2% BD-rate reduction for All Intra HE with encoding runtime increased by 23%2 and decoding runtime increased by 2%. By applying the proposed PU type signaling method on 2Nx0.5N/0.5Nx2N intra prediction unit, experimental results report average 0.0% BD-rate difference for All Intra HE with negligible encoding and decoding runtime change compared to AHG16 software. Two context models are removed.

No WD text available, but it is said that it would be identical to the text of JCTVC-H347, version with Nx2N.

JCTVC-H0302 AHG16: Crosscheck of MediaTek’s harmonization of 2NxN/Nx2N Intra PU with SDIP and NSQT (JCTVC-H0455) [Y.J. Chiu, L. Xu, W. Zhang, Y. Han (Intel)] [late]

JCTVC-H0558 Cross-Check of AhG16: SDIP [Ankur Saxena, Felix Fernandes (Samsung)] [late]

JCTVC-H0492 Clean-up of NSQT implementation [Y. Yuan (Tsinghua), X. Zheng (HiSilicon), H. Yu (Huawei), Y. He (Tsinghua)]

(include abstract)

Removes approx. 1000 lines of code by unifying square and non-square transform. Slight deviation of results due to bug fix in BS calculation.

Confirmed by cross-checker

Recommendation: Adopt (software only).

JCTVC-H0605 Cross-check of JCTVC-H0492 on Clean-up of NSQT implementation [Edouard François (Canon)] [late]

## Block structures and partitioning

### RQT

JCTVC-H0315 Non-CE2: Separate RQT structure for Y, U and V components [J. An, X. Zhao, X. Guo, S. Lei (MediaTek)]

JCTVC-H0542 Non-CE2: Cross-verification of Separate RQT for Y,U and V from MediaTek's (H-0315) [L. Guo (Qualcomm)] [late]

JCTVC-H0123 RQT: Merge transform_tree and transform_coeff [Y. Shibahara, T. Nishi (Panasonic)]

JCTVC-H0128 Cross-Check of H0123 on merge transform tree and transform coefficient [A. Saxena, E. Alshina, F. Fernandes (Samsung)] [late]

### Coding of LCU/TU/PU

Some relate to HL syntax category

JCTVC-H0154 Side information signaling improvement for TU [B. Lee, M. Kim (KAIST)][late]

JCTVC-H0664 Cross check of KAIST’s proposal JCTVC-H0154 [J. Kim, C. Kim (LGE)] [late]

JCTVC-H0202 PU structure for intra NxN [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-H0638 Cross-check report on PU structure for intra NxN (JCTVC-H0202) [I.-K Kim (Samsung)] [late]

JCTVC-H0269 AHG15 : Independent control of Max CU and LCU size for small LCU [J.B.Choi, J.H. Kim, K.H. Lee, J.H. Park, C. Kim (Samsung)]

JCTVC-H0623 AHG15: Crosscheck of JCTVC-H0269 on independent control of Max CU and LCU size for small LCU [Y. Jeon, B. Jeon (LG)] [late]

  - Motion compensation operation and interpolation filters

### Interpolation filters and MV precision (ref CE3)

JCTVC-H0180 Non-CE3: Phase adjustment of interpolation filter [T.Chujoh, T.Yamakage (Toshiba)]

JCTVC-H0610 Non-CE3: Cross-check for phase adjustment of interpolation filter (JCTVC-H0180) by Samsung [E. Alshina, A. Alshin, J. H. Park (Samsung)] [late]

JCTVC-H0569 Non-CE3: An Adaptive Interpolation Filtering Technique for Improved Subjective Quality in the Low Complexity Configuration [F. Kossentini, N. Mahdi, H. Guermazi, M. Horowitz, S. Xu (eBrisk Video Inc.)]

JCTVC-H0207 Non-CE3: Crosscheck of JCTVC-Hxxx : an AIF for low complexity configuration [J. Park, Hendry, B. Jeon (LG)]

JCTVC-H0304 6H7Q luma interpolation filter [Hongbo Zhu] [late][miss]

### Prediction methods

JCTVC-H0401 Differentiated weighting parameter sets for weighted prediction [J. Xu (Sony), A. Tourapis (Magnum), A. Tabatabai (Sony)]

JCTVC-H0546 On weighted prediction precision [M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-H0643 Cross-check of weighted prediction precision reduction (JCTVC-H0546) [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)] [late]

## Motion and mode coding

H709 BoG coordinated by B. Bross

[Add notes from BoG report on actions taken and further study recommendations.]

Regarding H0082 and H0090, it was agreed to adopt H0082, but reluctance was generally expressed regarding the potential for H0090 due to the desire to keep the decoder impact to a minimum for enabling this encoder parallelism.

There was some Track A discussion of H0111 and H0431, although the group ultimately decided to agree with the BoG recommendation in that regard. Remarks of that discussion included the following:

- H0111 is also a customization for a particular type of encoding, which is the type of encoding done by the HM in the context of uni-directional biprediction. More extensive searching in the HM context could give 0.7% improvement of GPB (with 30% increase in runtime), and encoder-only modification without more extensive searching could give 0.2%, and the proposed modification give 1% – all in the LB context. To some extent the question may be how important we consider the LB usage case to be.

- Another proposal H0431 suggested to separate the context models for the zero value of the motion vector differences into three cases: uniprediction, List 0 in bipred, and List 1 in bipred. It seems to have about the same gain when used the same way, but appears to be a more general approach rather than a customization to one specific encoding method which always sets the list 1 motion vector to 0 (by measuring statistics rather than forcing one value all the time). The text for this proposal had some problems.

It was indicated that text was nearly fully developed for H0709 and would be uploaded as a revision of the report.

Although further study of some items was encouraged, there seemed to be no need for a CE on this topic for the next meeting cycle.

### Parallel skip/merge

JCTVC-H0082 AHG10: Configurable and CU-group level parallel merge/skip [M. Zhou (TI)]

JCTVC-H0580 AHG10: Crosscheck of TI's Configurable and CU-group level parallel merge/skip (JCTVC-H0082 combination case) [T. Ikai (Sharp)] [late]

JCTVC-H0220 AHG10: Cross-check report of TI's configurable and CU-group level parallel merge/skip (JCTVC-H0082) by Panasonic [T. Sugio, T. Nishi (Panasonic)] [late]

JCTVC-H0369 AHG10: Cross-check of configurable and CU-group level parallel merge/skip (JCTVC-H0082) [J. Xu (Microsoft)] [late]

JCTVC-H0681 AHG10: Cross-check of TI's configurable and CU-group level parallel merge/skip (JCTVC-H0082) by Sony [J. Xu, A. Tabatabai (Sony)] [late]

JCTVC-H0090 AHG10: Unified design on parallel merge/skip [Y. Jeon, B. Jeon (LG), M. Zhou (TI), X. Wen, O. C. Au (HKUST), H. Y. Kim (ETRI), K. Y. Kim (KHU)] [late]

JCTVC-H0591 AhG10: Cross-verification of JCTVC-H0090 on unified design on parallel merge/skip [V. Seregin, X. Wang, J. Chen (Qualcomm)] [late]

JCTVC-H0092 Non-CE9: Removing PU dependency in TMVP reference index derivation [Y. Jeon, B. Jeon (LG), V. Seregin, X Wang, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0087 Non-CE9: Cross-verification of removing PU dependency in TMVP reference index derivation (JCTVC-H0092 from LGE and Qualcomm) [M. Zhou (TI)]

JCTVC-H0199 Non-CE9: Derivation process of reference indices for temporal merging candidates [H. Nakamura, S. Fukushima (JVC Kenwood)]

JCTVC-H0218 Non-CE9: Cross-check report of JVC KENWOOD’s modified derivation process of the reference index for temporal merging candidate (JCTVC-H0199) by Panasonic [T. Sugio, T. Nishi (Panasonic)] [late]

JCTVC-H0214 Non-CE9: Modification of derivation process for the reference index for temporal merging candidate [T. Sugio, T. Nishi (Panasonic)]

JCTVC-H0093 Non-CE9: Crosscheck of Panasonic's proposal on TMVP refIdx derivation in merge/skip (JCTVC-H0214) [Y. Jeon, B. Jeon (LG)]

JCTVC-H0247 AHG10: Unified design on parallel merge/skip [H. Y. Kim (ETRI), K. Y. Kim (KHU), Y. Jeon, B. Jeon (LG), M. Zhou (TI), X. Wen, O. C. Au (HKUST)] [late]

JCTVC-H0635 Cross verification of AHG10: Unified design on parallel merge/skip with reduced candidates [C. Kim (Samsung)] [late] [miss]

JCTVC-H0278 Non-CE9: CU-based parallel merge mode [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0592 Non-CE9: Cross-verification of JCTVC-H0278 on CU-based parallel merge mode [V. Seregin, X. Wang, J. Chen (Qualcomm)] [late]

JCTVC-H0649 AHG10: Cross Check for JCTVC-H0010 PME A [X. Wen, O. Au (HKUST)] [late] [miss]

JCTVC-H0676 Cross verification of crosscheck of AHG10 (JCTVC-H0010) HM5 PME B [Jongbum Choi (Samsung)] [late] [miss]

### MV and mode coding

JCTVC-H0077 Non-CE9: Merge candidates pruning without reference indices [S. Fukushima, K. Arakage (JVC Kenwood)]

JCTVC-H0365 Non-CE9: Cross-verification of merge candidates pruning without reference indices (JCTVC-H0077) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-H0078 Non-CE9: Simplification of AMVP spatial candidate derivation [S. Fukushima, H. Nakamura, H. Takehara (JVC Kenwood)]

JCTVC-H0340 Non-CE9: Crosscheck for JVC Kenwood's simplification of AMVP spatial candidate derivation in JCTVC-H0078 [J.-L. Lin, Y.-W. Huang (MediaTek)]  [late]

JCTVC-H0091 Non-CE9: Parallel Merge Candidate Derivation for Inter_NxN partition type [Y. Jeon, B. Jeon (LG), V. Seregin, X Wang, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0219 Non-CE9: Cross-check report of parallel merge candidate derivation for Inter_NxN partition type (JCTVC-H0091) by Panasonic [T. Sugio, T. Nishi (Panasonic)] [late]

JCTVC-H0133 Non-CE9: Merge candidates pruning without TMVP [H. Takehara, K. Arakage, S. Fukushima (JVC Kenwood)]

JCTVC-H0139 Non-CE9: Cross-verification of Merge candidates pruning without TMVP (JCTVC-H0133 from JVC Kenwood) [Y. Suzuki (NTT DOCOMO)]

JCTVC-H0164 Non CE9: Removing non-scaled bi-predictive merging candidates [Y. H. Tan, C. Yeo, H. L. Tan, Z. Li (I2R)]

JCTVC-H0216 Non-CE9: Modification of derivation process for MVP candidate in AMVP/Merge [T. Sugio, T. Nishi (Panasonic)]

JCTVC-H0086 Non-CE9: Cross-verification of modification of derivation process for MVP candidate in AMVP/Merge (JCTVC-H0216 from Panasonic) [M.Zhou (TI)]

JCTVC-H0239 Non-CE9: Simplification on AMVP list construction [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

JCTVC-H0634 Non-CE9: Cross-check report of ETRI’s simplification on AMVP list construction (JCTVC-H0239) by Panasonic [T. Sugio, T. Nishi (Panasonic)] [late]

JCTVC-H0240 Non-CE9: Throughput improvement for merge/skip mode [H. Y. Kim (ETRI), K. Y. Kim, S. M. Kim, G. H. Park (KHU), S.-C. Lim, J. Lee, S. Cho, J. S. Choi (ETRI)]

JCTVC-H0629 Non-CE9: cross-check of H0240 on throughput improvement for merge/skip mode [P. Onno (Canon)] [late]

JCTVC-H0250 Non-CE9: Removal of non-scaled merge candidate [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-H0368 Non-CE9: Cross-check of removal of non-scaled merge candidate (JCTVC-H0250) [J. Xu (Microsoft)]

JCTVC-H0279 Non-CE9: CU-based parallel inter mode [J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0585 Non-CE9: cross-check of JCTVC-H0279 on CU based parallel Inter [J. Jung (Orange Labs)] [late]

JCTVC-H0316 Non-CE9: Simplification of AMVP [L. Zhao, X. Guo, S. Lei (MediaTek), S. Ma, W. Gao (PKU), D. Zhao (HIT)]

JCTVC-H0606 Non-CE 9: Cross-Check of H0316 on simplification of AMVP [Ankur Saxena, Felix Fernandes (Samsung)] [late]

JCTVC-H0429 Non-CE9: Moving fifth spatial MV candidate after TMVP for merge mode [V. Seregin, X. Wang, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0341 Non-CE9: Crosscheck for Qualcomm's merge list reordering in JCTVC-H0429 [Y.-W. Chen, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0430 Non-CE9: Single TMVP for merge mode [V. Seregin, X. Wang, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0582 Non-CE9: Crosscheck of JCTVC-H0430 on single TMVP for merge mode [Y. Jeon, B. Jeon (LG)] [late]

JCTVC-H0431 Non-CE9: Splitting contexts for MVD coding [V. Seregin, J. Chen, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-H0640 Cross-check report on Splitting contexts for MVD coding [I.-K Kim (Samsung)] [late]

JCTVC-H0442 Limiting Collocated Temporal Reference to One Per Picture [T. Hellman, W. Wan (Broadcom)]

JCTVC-H0570 AHG14/AHG15/non-CE9: Loss robustness issue in TMVP disabling [V. Wahadaniah, C. S. Lim (Panasonic)]

JCTVC-H0327 Simplification of MVP index coding and AMP mode coding [H. Li, B. Li (USTC), H. Yang, H. Yu (Huawei)]

JCTVC-H0121 Cross Check for Huawei's CABAC simplification (JCTVC-H0327) [Y. Shibahara, T. Nishi (Panasonic)]

JCTVC-H0243 Simplification of spatial motion vector scaling process [I.-K Kim, J. H. Park (Samsung)]

JCTVC-H0251Context reduction for merge index coding [O. Bici, J. Lainema, K. Ugur (Nokia)]

JCTVC-H0367 Cross-check of context reduction for merge index coding (JCTVC-H0251) [J. Xu (Microsoft)]

JCTVC-H0362 Motion prediction at entropy slice boundary [B. Li (USTC), J. Xu (Microsoft), H. Li (USTC)]

JCTVC-H0462 Spatial Motion Vector Predictor Mismatch and Possible Unintended Behavior [B. Heng, P. Chen, W. Wan (Broadcom)]

JCTVC-H0555 Range restriction of motion vector predictor [J. Chen, M. Coban, M. Karczewicz (Qualcomm)]

## High-level syntax and slice structure

### High-level syntax and systems usage of bitstreams

JCTVC-H0288 Syntax on picture size signalling [Y. Park, C. Kim, J. H. Kim, J. H. Park (Samsung)]

This contribution proposes a design for picture size signalling in the SPS. The proposal is to send, as ue(v), an indication of the number of bits used to code the width and height. This is motivated by a desire to save bits on the picture size expression syntax. No action taken on this.

See AHG15 report discussion notes.

JCTVC-H0485 Picture size signalling and cropping operation [K. Suehring, B. Bross (Fraunhofer HHI)]

This contribution proposes a modified picture size coding scheme based on coding units and a cropping operation. Bit rate savings are reported for typically used picture sizes. The contribution also analyzes memory requirements for cropping operations inside and outside the coding loop.

Proposes to perform the cropping outside the coding loop (which is agreed as documented above). No further action taken.

JCTVC-H0423 Indication of the temporal structure of coded video sequences [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

The contribution discusses the topic of describing the temporal structure of coded video sequences, which was proposed earlier in documents JCTVC-D200 and JCTVC-E279. A primary use case for knowing the temporal structure a priori is to assist transraters and bitstream extractors in pruning the coded video sequence.

By referring to the reference picture set index, the proposal is asserted to provide a more accurate description of the temporal structure than what was proposed in JCTVC-D200 and JCTVC-E279. Furthermore, by specifying the temporal structure in the sequence parameter set, it is reportedly possible to omit certain syntax elements from the slice header and hence reduce the bit count of the slice header.

The proposal can be considered to consist of the following ingredients:

- The short term reference picture set syntax structure is moved from PPS to SPS.

- A SOP description syntax structure is added in the SPS. The syntax structure describes a structure of pictures (SOP) in decoding order.

- A coded video sequence can be described as part of the SPS with reference to described SOPs.

- When the SOP description is used, the slice header includes a SOP index referring to a particular SOP description and a picture index within the SOP structure. Certain syntax elements of the slice header are excluded provided that the SOP description is used.

It is asserted that up to about 3% bitrate savings may be possible at a selected low bit rate and high frame rate relating to the test case “dyadic and nested temporal scalability for high frame rate” in the common test conditions specified in JCTVC-G1036, while typical bit rate savings are expected to be an order of magnitude smaller.

It was noted that, when used, having such a structure established in the SPS would prohibit adaptive structuring decisions within the CVS. If it is sent at the picture level (e.g. in SEI), an encoder could make dynamic changes.

A participant commented that a possible approach could be to have the syntax in the SPS which establishes a default behaviour that can be overridden at the PPS or slice level.

A suggestion was to put this syntax in the PPS instead of in the SPS.

The proposal would remove from the slice header the slice_type, pic_order_cnt_lsb, and all reference picture set syntax. Long-term reference pictures would not be supported when the SOP syntax is used.

The proposal seemed interesting. Open.

JCTVC-H0496 On bitstreams starting with CRA pictures [Y. -K. Wang, Y. Chen, M. Karczewicz (Qualcomm)]

This document reproposes the proposal in JCTVC-G319.

In the current HEVC design, CRA (Clean Random Access) pictures are identified by a new NAL unit type. It is very common that a device with a conforming decoder performs random access at a CRA picture. However, a bitstream starting at a CRA picture is considered non-conforming, thus a conforming decoder may not be able to properly handle such bitstreams.

In this proposal, it is proposed that a bitstream starting from a CRA picture can be conforming. Such a conforming bitstream may or may not contain leading pictures associated with the CRA picture. A leading picture associated with a CRA picture is a coded picture that follows the CRA picture in decoding order but precedes the CRA picture in output order. The proposed normative changes include: 1) skipping the decoding and output of the leading pictures associated with the starting CRA picture, when present; 2) HRD modifications to guarantee that all bitstream conforming conditions are fulfilled by a conforming bitstream starting with a CRA picture, regardless of whether the leading pictures associated with the CRA picture are present.

It was commented that bitstream conformance may be a bit tricky to specify. A suggestion was to specify for a reference decoding process to be applied as in the prior recovery point SEI message description, with enforcing conformance of the bitstream when that operates, and then discarding the final decoded picture without output of it.

A modification of the buffering period SEI message was proposed to assist with the HRD CPB flow modification that is introduced if the "leading pictures" are removed from the bitstream.

It was asserted that there should be no problem with the case where the "leading pictures" are not removed from the bitstream.

In spirit, we would like to adopt this. Revisit to check text.

JCTVC-H0567 AHG15: Syntax controlled output process [J. Samuelsson, R. Sjöberg (Ericsson)]

This document proposes to add two syntax elements to the slice header to control the output process. A new output process is proposed to replace the "bumping" process and it is claimed that the use of the proposed output process controlled by syntax elements in the slice header reduces unnecessary picture output delay. Further, it is claimed that the syntax controlled output process is better suited for temporal scalability since the encoder does not need separate decoder picture buffer (DPB) models for each temporal layer and since network nodes and decoders can rely on that temporal layer switching information is correct. The document describes how the flexible and efficient DPB usage provided by the "bumping" process is maintained by the proposed output process. A restriction is proposed to prohibit those cases that make the DPB status inconsistent among different temporal layers and that violate the concept of temporal layer switching points. It is also proposed that max_dec_frame_buffering and max_num_ref_frames can have different values for different temporal layers. The document also contains a proposed sequence parameter set (SPS) flag that can be used to indicate the most common case of output process usage, when both output delay and DPB size is minimized. With the proposed SPS flag the total bitrate increase for the common test conditions is one bit per sequence.

Proposes having different max_dec_frame_buffering and max_num_ref_frames for different temporal layers. It was commented that num_reorder_frames should be treated this way too.

It was commented that the bumping process should use num_reorder_frames and max_latency_increase, not just max_dec_frame_buffering. Decision: Agreed (depending on potential later decisions on related issues).

It was commented that we may no longer need max_num_ref_frames at all (since we no longer have a sliding window reference picture marking process). Decision: Remove it.

Proposes to define a variable OutputDistance for each picture and introduce a flag that can set that variable to 0.

It was asked whether the new syntax could be pushed up to the APS or PPS level, rather than residing in the slice header, and this seemed feasible.

Open, but likely to be for further study.

JCTVC-H0654 High level syntax [J. Lou, Y. Yu, L. Wang (Motorola Mobility)] [late]

This document proposes some modifications on sequence parameter set RBSP syntax, picture parameter set RBSP syntax and slice header syntax.

Proposes to remove some syntax elements when max_num_ref_frames is equal to 0.

It was noted that the PPS modifications introduce a parsing dependency on the SPS content.

Also there is a parsing problem with the first modification proposed in the slice level. The last modification proposed in the slice level would be a no-op. The remaining modification would save 1 bit in the case of max_num_ref_frames = 0.

It was acknowledged that we have a bit of recognizable redundancy in some of these syntax areas.

Open, but not much enthusiasm.

JCTVC-H0380 AHG15: SPS and PPS syntax modifications [V. Drugeon, T. Wedi, M. Narroschke, V. Wahadaniah (Panasonic)]

This contribution proposes several modifications of the SPS and PPS syntax. In particular, modifications are proposed for:

- block size signalling (MinTr <= MinCU <= MaxCU, MinTr <= MaxTr <= MaxCU)

- PCM related syntax (MinCU <= MinPCMCU <= MaxPCMCU <= MaxCU)

- slice granularity (currently 2 bits in PPS, proposed to move this SPS and change its coding)

It was commented that it seems desirable to keep the ability to change slice granularity flexibility at the PPS level (e.g. to use bigger granularity for higher temporal layers).

Open, but low importance – such low-level cleanups can be deferred.

### Supplemental information

### Bit depth

JCTVC-H0500 Bit depth of output pictures [Y. Chen, Y. -K. Wang, X. Wang, I. S. Chong, M. Karczewicz (Qualcomm)]

This document reproposes the proposal in JCTVC-G328.

In the current HEVC design, IBDI (Internal Bit Depth Increase) is supported in a way that the decoded picture and the output picture always have the bit depth as signalled in the bitstream (e.g., 10 bits), regardless of the bit depth of the input video (e.g., 8 bits). In this document, it is proposed that: 1) a conforming decoder outputs a decoded video sequence with a target output bit depth (signalled in the bitstream); and 2) if the output bit depth is lower than the bit depth of the decoded pictures, it is asserted that better memory consumption can be achieved in a way that if a picture is never or no longer used for reference, it is converted to lower bit depth immediately.

It was intended for the proposal to affect the DPB capacity for picture storage, to make it dependent on the bit depth of the stored pictures. This part of the text was missing from the proposal. The practicality of an actual decoder taking advantage of this was questioned.

The decoder would be required to perform bit depth downconversion by truncation. It was remarked that prohibiting more sophisticated downconversion may be undesirable.

For further study.

### Parameter sets and slice header

JCTVC-H0069 APS partial update through referencing [Stephan Wenger, Jill Boyce, Ming Li, Ping Wu (Vidyo)]

JCTVC-H0070 APS partial update through conditional replacement [Stephan Wenger, Jill Boyce (Vidyo)]

JCTVC-H0071 On the need for an Adaptation Parameter Set [Stephan Wenger, Jill Boyce, Won Kap Jang (Vidyo)]

JCTVC-H0132 On Adaptation Parameter Signalling [Y. Ye, E. S. Ryu (InterDigital)]

JCTVC-H0255 On partial updating of APS parameters [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-H0376 Non-CE8: Low-delay support for APS [S. Esenlik, M. Narroschke, V. Drugeon, T. Wedi (Panasonic)]

JCTVC-H0381 AHG15: Partial APS update [V. Drugeon, S. Esenlik, M. Narroschke, T. Wedi, V. Wahadaniah (Panasonic)]

JCTVC-H0505 On APS partial update  [Y. -K. Wang, Y. Chen, G. Van der Auwera (Qualcomm)]

JCTVC-H0507 Referencing different APS’s in the same picture [S. Wenger (Vidyo), Y. -K. Wang, I. S. Chong (Qualcomm)]

JCTVC-H0512 APS fragmentation [Y. Chen, Y. -K. Wang, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-H0119 Non-CE4 Subtest2: Extension of Adaptation Parameter Sets syntax for scaling list matrix [J. Tanaka, Y. Morigami, T. Suzuki (Sony)]

JCTVC-H0313 AHG15: On Signalling of Several Syntax Elements in SPS [X. Li, J. An, X. Guo, S. Lei (MediaTek)]

JCTVC-H0380 AHG15: SPS and PPS syntax modifications [V. Drugeon, T. Wedi, M. Narroschke, V. Wahadaniah (Panasonic)]

JCTVC-H0391 AHG15: Slice-Level Control of In-Loop Filter [R. Srinivasan, C. Ghone (TI)]

JCTVC-H0392 AHG15: Enable picture-level CU/TU size signaling in HEVC [N. Srinivasamurthy, K. Prasad, Y. Dutt, M. Zhou (TI)]

JCTVC-H0343 Redundancy removal of syntax [T. Lee, J. Park (Samsung)]

### Tiles

A BoG (coordinated by M. Horowitz) was requested to review contributions in the tiles and wavefronts categories).

JCTVC-H0179 Storage reduction for dependent tiles [E. François (Canon), S. Pautet]

JCTVC-H0658 AHG4: Crosscheck for Canon's storage reduction for dependent tiles in JCTVC-H0179 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0203 AHG4: Further latency reduction of CABAC initialization for dependent Tiles [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

JCTVC-H0414 Cross-check of “AHG4: Further latency reduction of CABAC initialization for dependent Tiles” (JCTVC-H203) [M. Horowitz, S. Xu (eBrisk)]

JCTVC-H0329 AHG4: Crosscheck for LG's further latency reduction of CABAC initialization for dependent tiles in JCTVC-H0203 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0270 AHG4: Avoiding lower left LCUs at dependent tile column boundaries [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0345 Constrained Tile for parallel decoding [H. Sasai, K. Uchibayashi, T. Nishi (Panasonic)]

JCTVC-H0348 Simplification on tiles and slices [T. Lee, B. Choi, J. Park (Samsung)]

JCTVC-H0593 Cross-verification of JCTVC-H0348 on simplification on tiles and slices [V. Seregin, M. Coban (Qualcomm)] [late]

JCTVC-H0463 Comments on specifying tile support in HEVC Profiles and Levels [W. Wan, T. Hellman (Broadcom)]

JCTVC-H0520 Tile groups [Y. -K. Wang, A. K. Ramasubramonian, Y. Chen, M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-H0521 Flexible tile dependency and loop filter control [Y. -K. Wang, J. Chon, Y. Chen, I. S. Chong, M. Coban, M. Karczewicz (Qualcomm)]

JCTVC-H0556 AHG4: Harmonized method for signalling entry points of tiles and WPP substreams [Hendry, Sang Oh Jeong, Seung Wook Park, Byeong Moon Jeon (LG), Kiran Misra, Andrew Segall (Sharp)]

JCTVC-H0370 Crosscheck of LGE/Sharp proposal for byte-aligned substreams and harmonized entry points (JTCVC-H0556) [Felix Henry, Gordon Clare (Orange)]

JCTVC-H0293 Syntax on parallel processing information signaling [S. S. Jeong, Y. Park, C. Kim, J. H. Kim, J. H. Park (Samsung)]

### Wavefront parallel processing

JCTVC-H0226 On derivation of luma QP predictor for Wavefront Parallel Processing [S. Shimada, K. Kazui, J. Koyama, A. Nakagawa (Fujitsu)]

JCTVC-H0349 Simplification on WPP [T. Lee, J. Choi, J. Park (Samsung)]

JCTVC-H0328 AHG4: Crosscheck for Samsung's simplification on WPP in JCTVC-H0349 [C.-Y. Tsai, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0513 On parallel processing schemes [Y. -K. Wang, M. Coban (Qualcomm)]

JCTVC-H0515 On entropy slices [M. Coban, Y. -K. Wang, J. Chon, M. Karczewicz (Qualcomm)]

JCTVC-H0642 Cross-verification of JCTVC-H0515 on Entropy Slices [T. Lee, J. Park (Samsung)] [late]

JCTVC-H0517 On wavefront parallel processing [Y. -K. Wang, M. Coban (Qualcomm)]

### NAL unit header

### Random access

JCTVC-H0215 Market needs and practicality of sub-picture based CPB operation [K. Kazui, S. Shimada, J. Koyama, A. Nakagawa (Fujitsu), A.Duenas (CAVIUM)]

### Decoded picture buffering and reference picture signalling

A BoG (coordinated by J. Boyce) was requested to review this category of contributions and the AHG21 report.

JCTVC-H0137 AHG21: Reference picture lists combination syntax [Y. Ye, Y. He (InterDigital)]

JCTVC-H0653 AHG21: Cross-verification of reference picture lists combination syntax from InterDigital (JCTVC-H0137) [L. Guo (Qualcomm)] [late]

JCTVC-H0138 AHG21: Unification of reference picture list modification processes [Y. He, Y. Ye (InterDigital)]

JCTVC-H0679 AHG21: Cross-check of H0138 on Unification of reference picture list modification processes [E. Francois (Canon)] [late]

JCTVC-H0196 AHG21: Comments on signalling of short-term reference pictures [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

JCTVC-H0200 AHG21: Comments on Signaling of Long-term Reference Pictures [Hendry, S. Jeong, S. Park, B. Jeon (LG)]

JCTVC-H0412 AHG21: Reference picture list restrictions [R. Sjöberg, J. Samuelsson (Ericsson)]

JCTVC-H0426 AHG21: Removal of reference picture list modification [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

JCTVC-H0433 AHG21: Reference Picture Set Signaling Method [Sachin Deshpande, Kiran Misra, Andrew Segall]

JCTVC-H0449 AHG21: A constraint to simplify and improve the robustness of POC computation [G. J. Sullivan (Microsoft)]

JCTVC-H0466 AHG21: Unified reference list for single-directional prediction [V. Wahadaniah, C. S. Lim (Panasonic)]

JCTVC-H0467 AHG21: Signalling periodic reference picture lists [V. Wahadaniah, S. M. T. Naing, C. S. Lim (Panasonic)]

JCTVC-H0468 AHG21: Flexible signalling of long term reference pictures [V. Wahadaniah, C. S. Lim (Panasonic)]

JCTVC-H0531 AHG21: Comments on Signaling of Reference Pictures [Sachin Deshpande, Andrew Segall (Sharp)] [late]

JCTVC-H0565 AHG21: Short-term reference picture range [J. Samuelsson, R. Sjöberg (Ericsson)]

JCTVC-H0257 Cyclic POC [J. Koyama, K. Kazui, S. Shimada, A. Nakagawa (Fujitsu)]

JCTVC-H0292 Syntax on List Combination [Y. Park, T. Lee, J. H. Kim, J. H. Park (Samsung)]

JCTVC-H0482 Improvement to Intra RPS Prediction [J. Zhao, S. Deshpande, A. Segall (Sharp)]

JCTVC-H0501 Efficient POC signaling for temporal scalability [Y. Chen, Y. -K. Wang (Qualcomm)]

JCTVC-H0425 POC granularity for reference picture sets and slice header [M. M. Hannuksela, S. M. Gopalakrishna (Nokia)]

JCTVC-H0502 Signaling of long-term reference pictures in the PPS [Y. -K. Wang, Y. Chen (Qualcomm)]

JCTVC-H0504 Comments on Reference Picture Lists Combination Syntax [Hendry, Y. J. Jeon, S. W. Park, S. O. Jeong, B. M. Jeon (LG), Y. Chen, Y. -K. Wang, W. -J. Chien (Qualcomm)]

JCTVC-H0548 Signaling of collocated picture for HEVC [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-H0695 Cross check report of Motorola Mobility proposal JCTVC-H0548 [?? (??)] [late] [miss]

JCTVC-H0551 Parameterized RPS Models [J. Zhao, A. Segall (Sharp)]

JCTVC-H0694 Long-term picture coding [K. Suehring, H. Schwarz, T. Wiegand (Fraunhofer HHI)] [late]

### HRD issues

void

## Quantization

### QP prediction / delta QP coding

JCTVC-H0189 Non-CE4: low-complexity dQP coding [K Sato (Sony)] [late]

QP prediction has been investigated in CE4: subtest 1. The method proposed in JCTVC-G1024 [3] is the one that is best spatial QP prediction in terms of coding efficiency among the proposals in the previous CE4, and it is also rate control friendly.

At the last meeting some expert commented that he rather preferred simpler method like just averaging.

This document provides result with simple QP prediction method of just averaging of leftQP and topQP.

Compared to CE4 subtest 1.3: Same left/top averaging method used for inter and intra; does not use mode dependency any more, gain is 0.4% for intra, and 0.3/0.2% for RA/LD.

JCTVC-H0675 Non-CE4: Cross verification of low complexity dQP coding (JCTVC-H0189) [M. Coban (Qualcomm)] [late] [miss]

JCTVC-H0204 Non-CE4: Compatible QP prediction with RC and AQ [H. Nakamura, M. Nishitani, S. Fukushima (JVC Kenwood)]

This contribution proposes spatial QP prediction method under restricting QP reference across LCU boundary. The proposed method is based on the CE4 Subtest 1.3 and modified to use QP of the preceding QGCU (Quantization Group of Coding Unit) more frequently for further rate control friendliness. In the combination with the QP prediction based on intra prediction direction in CE4 Subtest1.3, it can be expected to achieve higher efficiency improvement. The simulation results in the case of MaxCUDepthDQP=3(8x8) is reported as follows. The proposed method with the QP prediction based on intra prediction direction provides 0.5%, 0.4%, 0.3%, 0.3%, 0.2%, 0.3% and 0.3% luminance BD-rate gains for AIHE, AILC, RAHE, RALC, RAHE10, LDHE, and LDLC configurations, respectively. The proposed method without the QP prediction based on intra prediction direction provides 0.3%, 0.3%, 0.3%, 0.3%, 0.2%, 0.3% and 0.3% luminance BD-rate gains  for AIHE, AILC, RAHE, RALC, RAHE10, LDHE, and LDLC configurations, respectively.

Difference compared to H0189: A non-available value (left or top) is replaced by the previous QP, and averaging is performed on whatever is remaining (i.e. if both are not available, the previous QP is averaged with itself). Slightly less rate reduction than H0189 (0.3% instead of 0.4%) for AI case, otherwise similar performance, but may be a bit more regular data flow.

Decision: Adopt

JCTVC-H0194 Non-CE4: Cross-verification report for modified spatial QP prediction based on intra prediction direction by JVC KENWOOD (method 1 of JCTVC-H0204) [H. Aoki, K. Chono (NEC)]

JCTVC-H0601 Non-CE4: Crosscheck report of Compatible QP prediction with RC and AQ (JCTVC-H0204) [M. Shima (Canon)] [late]

JCTVC-H0289 Non-CE4: Spatial QP prediction based on motion vector predictors (AMVP) [P. Andrivon, P. Bordes (Technicolor)]

This contribution presents results of spatial QP prediction based on:

- Test 1: motion vector predictor cues (in Inter),

- Test 2: a combination of test 1 and an intra direction based method (JCTVC-G1204 Subtest 1.3),

- Test 3: a non mode dependant QP prediction (above and left QGCU averaging if within LCU otherwise last coded QP).

Simulations results show on average 0.2% to 0.5% BD-rate gains for test 1 and test 2 in RA, LB and AI cases respectively under the test condition defined in CE4 Subtest 1 (AVC like).

Compared to the non mode dependant QP prediction (test 3), gains are equivalent.

Test 3 is same as H0189.

JCTVC-H0525 Non-CE4: Cross-verification report for spatial QP prediction based on motion vector predictors (JCTVC-H0289) [H. Aoki, K. Chono (NEC)] [late]

JCTVC-H0188 On the syntax of ChromaQpOffset [K Sato (Sony)]

At the 7th JCT-VC meeting ChromaQpOffset, same as the one in AVC, has been adopted. This document proposes two modifications:

- Moving ChromaQpOffset from PPS to APS

- Change the names as “Cb_QP_Offset” and “Cr_QP_Offset”

JCTVC-H0400 Chroma QP extension [J. Xu (Sony), A. Tourapis (Magnum), E. Maani, K. Sato (Sony)]

In HM5.0, Chroma QP can only take values in the range of [0, 39]. This proposal extends Chroma QP to the range of [0, 51]. Such extension enhances the functionality, flexibility and friendliness of HEVC for various potential applications, such as rate control at low bit-rate including the flexibility to obtain the desired performance balance point between luma/chroma, and for handling video sources with different color formats (e.g. RGB).

Proposal: QPCx = Clip(0, 51, QPY+Cx_QP_offset)

Decision: Adopt chroma QP mapping from H0400, change the names as suggested in H0188, keep the offset values in PPS (seems not to need frequent switching)

### Quantization matrices

JCTVC-H0237 Non-CE4 Subtest 2: Improvement on quantization matrix signaling [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

This contribution presents an improvement on quantization matrix signaling. The proposed method restricts quantization matrix signaling by minimum and maximum TU sizes and sends a flag that indicates whether to use default matrix or proprietary matrix for each SizeID and MatrixID.

Not to signal unnecessary quant matrices is something reasonable to do. This should be discussed in the BoG on quant matrices as it is related to compression (e.g. if a method would be used where a larger matrix is derived from a smaller one this would not be an issue)

Partially related to HL syntax issues (other proposals on partial update of APS, e.g. H0255); derivation based on max TU size might only be desirable if that parameter itself is part of APS.

JCTVC-H0387 Non-CE4: Handling large size quantization matrices [S. Liu, X. Zhang, S. Lei (MediaTek)]

JCTVC-H0641 Cross-check report for MediaTek's quantization matrix compression (JCTVC-H0387)) [J. Lou, L. Wang (Motorola Mobility)] [late]

JCTVC-H0453 Non-CE4 subtest 2: Signaling a subset of quantizer matrix coefficients for 32x32 quantization matrices [R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-H0460 High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding [Munsi Haque, Ehsan Maani, Ali Tabatabai]

JCTVC-H0602 Non-CE4: Crosscheck report of High-level Syntaxes for the Scaling List Matrices Parameters and Parametric coding (JCTVC-H0460) [M. Shima (Canon) [late]

JCTVC-H0461 On HVS only Default Scaling List Matrices [M. Haque, A. Tabatabai, J. Xu, C. Auyeung (Sony)]

JCTVC-H0696 Cross-check of JCTVC-H0461 on HVS only Default Scaling List Matrices Edouard Francois (Canon) [late] [miss]

JCTVC-H0477 On Just Noticeable Distortion Quantization in the HEVC Codec [Matteo Naccari, Marta Mrak (BBC)]

This contribution presents the application of a quantisation tool that exploits masking phenomena related to the Human Visual System (HVS) in the High Efficiency Video Coding (HEVC). In particular, the pixel intensity masking phenomenon is considered in this contribution which leads the HVS to be less sensitive in image areas where the average pixel intensity is either low or higher. The pixel intensity masking contributes to the HVS Just Noticeable Distortion (JND) which corresponds to the maximum distortion level that can be introduced in an image area without being noticeable by human observers. In this contribution, a mapping between the average block pixel intensity and the JND level is considered and used to modulate the step in the HEVC quantizer for each Transform Unit (TU). In this way for each image block being coded, the quantization level is perceptually adapted to its content. The JND varying quantization step needs to be communicated to the decoder to properly decode the received bitstream. It should be noted that communicating the varying quantization step for each TU would increase the coded rate. To avoid this communication, this proposal presents a method to estimate the JND level at the decoder. Furthermore, also the mapping between the average TU pixel intensity and the JND levels must be communicated to the decoder. To this end, this proposal presents a method to parameterise the JND profile and transmitting it to the decoder. The methods presented in this contribution have been integrated in the HM-5.0 codec and the reconstructed videos have been viewed to assess their subjective quality. It is reported that for high bitrates, the contents coded with JND quantization present the same subjective quality of their respective anchors with bitrate reductions up to 25%.

Request to signal a JND profile and how to derive the local QP parameter from the local intensity.

Results at higher QPs would be more interesting (QP22 is already close to visual lossless). Typically rate difference at same quality is less at high QP

Results for all intra?

Some experts express interest to receive further information

JCTVC-H0682 Cross-check of BBC’s proposal on “On Just Noticeable Distortion Quantization in the HEVC Codec” (JCTVC-H0477) [J. Wang, X. Yu, D. He (RIM)] [late] [miss]

JCTVC-H0495 Quantization matrices for 4x4 and 8x8 TUs matching HEVC integer transforms [J. Lou, J. Kim, L. Wang (Motorola Mobility)]

JCTVC-H0522 Weighted predictive coding for Quantization matrices [J. Zheng(HiSilicon), J. Chen(UCLA)]

JCTVC-H0700 Non-CE4: Crosscheck report of Weighted predictive coding for Quantization matrices (JCTVC-H0522) [?? (??)] [late] [miss]

JCTVC-H0523 On partial updating the set of quantization matrices [J. Zheng(HiSilicon), J. Chen(UCLA)]

### Dequantization

JCTVC-H0312 Non-CE4: Clipping of Transformed Coefficients before De-Quantization [X. Li, X. Guo, S. Lei (MediaTek)]

Presented together with H0541.

JCTVC-H0366 Non-CE4: Cross-check of clipping of transformed coefficients before de-quantization (JCTVC-H0312) [B. Li (USTC), J. Xu (Microsoft)]

JCTVC-H0532 Quantization reconstruction offset [J. Wang, X. Yu, D. He, G. Martin-Cocher (RIM)]

The document proposes to include a quantization reconstruction offset on the picture level as an extra functionality for supporting grain noise coding and texture preservation.

Could this be achieved by encoder optimization? Not fully clear

No action

JCTVC-H0541 Non CE4: Limiting dynamic range when using a quantization weighting matrix [Louie Kerofsky, Andrew Segall (Sharp)]

This contribution examines the dynamic range issues related to quantization weighting matrices identified in JVTVC-G1044.  This contribution reports that under extremely low weight and QP values, levels exceeding 16-bits may occur and that under extremely high weight and QP values intermediate values exceeding 32-bits may occur.  This contribution proposes limiting level values prior to dequantization to 15-bits for 4x4 block size and limiting levels to 16-bits for all other block sizes.  The proposed method reportedly allows a dequantization implementation with weighting matrix requiring 16-bit multiplies and intermediate values within 32-bits.

Presentation not uploaded. The presentation says that this is a joint contribution with H0312.

Proposed text available in H0312

Required computations: Levellimit used for clipping depends on QP, TU size and bit depth

Problem only appears in context of quant matrices

Comments:

- could be achieved by encoder restriction (similar discussion in previous meetings)

- no doubt that there is a problem to be solved

Decision: Adopt (text in H0312) as a placeholder to solve the problem, further study whether this is the ultimate solution in quantization AHG.

## Entropy coding

### CABAC improvement

JCTVC-H0113 Non-CE1: Accurate initialization and conditional probability for CABAC performance improvement [E. Alshina, A. Alshin, J. H. Park (Samsung)]

JCTVC-H0265 Non-CE1: Dimension reduced rangeTabLPS table for the interval subdivision of the arithmetic coding engine [J. Stegemann, H. Kirchhoffer, D. Marpe, T. Wiegand (Fraunhofer HHI)]

JCTVC-H0590 Non-CE1: Cross-check of proposal H0265 on rLPSTable [V. Sze (TI)] [late] [miss]

JCTVC-H0099 On CABAC context for partition mode [T. Yamamoto (Sharp)]

JCTVC-H0687 Cross-check of JCTVC-H0099 on CABAC context for partition mode [W.-J. Chien (Qualcomm)] [late]

JCTVC-H0248 Clean-up of redundant CABAC memorization process [Y. Chiu, W. Zhang, L. Xu, Y. Han (Intel)]

JCTVC-H0271 AHG9: CABAC with constrained outstanding bits [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0648 AHG9: Cross-check result of MediaTek's CABAC with constrained outstanding bits (JCTVC-H0271) [K. Kazui (Fujitsu)] [late]

JCTVC-H0397 Context reduction of split_transform_flag in CABAC [J. Xu, A. Tabatabai(Sony)]

JCTVC-H0626 Cross-check of Sony’s proposal on context reduction of split_transform_flag in CABAC (JCTVC-H0397) [H. Yang, H. Yu (Huawei)] [late]

JCTVC-H0545 Context modeling for asymmetric partitioning on partition mode [W. -J. Chien, X. Wang, M. Karczewicz (Qualcomm)] [late]

JCTVC-H0561 On CABAC Initialization [L. Guo, X. Wang, C. Muhammd, M. Karczewicz (Qualcomm)]

JCTVC-H0671 Cross-check for Qualcomm's CABAC Initialization in JCTVC-H0561 [X. Guo (MediaTek)] [late]

JCTVC-H0646 Reduction of initialization tables for CABAC contexts [K. Sugimoto, A. Minezawa, S. Sekiguchi, K. Asai, T. Murakami (Mitsubishi)] [late]

### Throughput improvement

JCTVC-H0554 Non-CE1: throughput improvement on CABAC coefficients level coding [J. Chen, W.-J. Chien, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)] [late]

JCTVC-H0660 Non-CE1: Crosscheck for Qualcomm's throughput improvement on CABAC coefficients level coding in JCTVC-H0554 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-H0233 Non-CE1: CABAC bypass for coefficient data [J. Lainema, K. Ugur, A. Hallapuro (Nokia)]

JCTVC-H0458 Non-CE1: High Throughput Coding Scheme with Rice Binarization [T. Nguyen, D. Marpe, T. Wiegand (Fraunhofer HHI)]

JCTVC-H0703 Cross-verification for HHI's (JCTVC-H0458) by Samsung [E. Alshina (Samsung)] [late] [miss]

JCTVC-H0510 Non-CE1: High Throughput Binarization (HTB) method with modified level coding [S.H. Kim, K. Misra, L. Kerofsky, A. Segall (Sharp)]

JCTVC-H0704 Cross-check of JCTVC-H0510: High Throughput Binarization (HTB) method with modified level coding [J. Lainema, A. Hallapuro, K. Ugur (Nokia)] [late] [miss]

JCTVC-H0450 Limiting the Bin to Bit Expansion Ratio [Y. Yu, T. Hellman, W. Wan (Broadcom)]

JCTVC-H0489  Complexity analysis of high throughput CABAC Entropy codecs [Alberto Duenas, Prashant Arora, Oscar Patino, Francisco Javier Roncero (Cavium)]

JCTVC-H0533 On CABAC bin parsing throughput [W. Zhang, P. Wu (ZTE)][late]

## Transform coefficient coding

### Significance map coding

JCTVC-H0100 Non-CE11.1: Context reduction of significance map coding [T. Tsukuba (Sharp)]

JCTVC-H0620 Non-CE11: Cross-check report [H. Sasai, K. Terada, T. Nishi (Panasonic)] [late]

JCTVC-H0131 Non-CE11: Simplified multi-level significance map coding [J. Min, Y. Piao, J. H. Park (Samsung)]

JCTVC-H0488 Cross-check of Samsung's H0131: Simplified multi-level significance map coding [N.Nguyen, T.Ji, D.He, G.Martin-Cocher (RIM)][late]

JCTVC-H0253 Improvement on parallelism for context derivation process of significant_coeff_flag [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-H0599 Cross-verification of Mitsubishi's improvement on parallelism for context derivation process of significant_coeff_flag (JCTVC-H0253) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-H0285 Non-CE11: Block-based significance map context selection [T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0619 Non-CE11: Cross-check report for JCTVC-H0285 on Block-based significance map context selection [H. Sasai, K. Terada, T. Nishi (Panasonic)] [late]

JCTVC-H0286 Non-CE11: Parallel significance map context assignment [T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-H0688 Cross-check of JCTVC-H0286 on Parallel significance map context assignment [W. -J. Chien (Qualcomm)] [late] [miss]

JCTVC-H0287 Non-CE11: Modified method for two-level coding of significance maps [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-H0110 Non-CE11: Cross-verification of MediaTek's proposal (JCTVC-H0287) [S. Fukushima, T. Kumakura (JVC Kenwood)] [late]

JCTVC-H0290 Non-CE11:Simplification of context selection for significant_coeff_flag [K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-H0357 Non-CE11: Cross-check of proposal H0290 on significant_coeff_flag context selection [V. Sze (TI)] [late] [miss]

JCTVC-H0379 Context reduction of 8x8 significance map coding by hybrid scan with mask [C. Auyeung, J. Xu, A. Tabatabai (Sony)]

JCTVC-H0621 Cross-check report for JCTVC-H0379 on Context reduction of 8x8 significance map coding by hybrid scan with mask [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-H0383 Complexity reduction of significance map coding [C. Auyeung (Sony)]

JCTVC-H0609 Crosscheck of context reduction of significe map coding (JCTVC-H0383) [T. Tsukuba T. Yamamoto (Sharp)] [late]

JCTVC-H0411 Significant Coefficient Flag Context Models for Modular Processing [N.Nguyen, T.Ji, D.He, G.Martin-Cocher (RIM)]

JCTVC-H0702 Cross-verification for RIM's (JCTVC-H0411) by Samsung [E. Alshina (Samsung)] [late] [miss]

JCTVC-H0594 Cross-check of RIM’s significant coefficient flag context models for modular processing (JCTVC-H0411) [J. Sole (Qualcomm)] [late]

JCTVC-H0427 Non-CE11: Support modification for parallel calculation of significant map contexts [V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-H0663 Non-CE11: Cross check of Qualcomm’s proposal JCTVC-H0427 [J. Kim, C. Kim (LGE)] [late]

JCTVC-H0432 Sub-Block Based Significance Map Region Classification [T.Ji, N.Nguyen, D.He, G.Martin-Cocher (RIM)]

JCTVC-H0359 Non-CE11: Cross-check of proposal H0432 on significant_coeff_flag context selection [V. Sze (TI)] [late] [miss]

JCTVC-H0445 Non-CE11: Low complexity parallel significance map coding [J.Sole, V. Seregin, M. Karczewicz (Qualcomm), T. Ikai, T. Tsukuba (Sharp)]

JCTVC-H0630 Non-CE11: Cross-check of Low complexity parallel significance map coding by Qualcomm and Sharp (H0445) [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-H0476 Adjusted significant coefficient map context for Non-Square blocks [J. Kim, B. Jeon (LG)]

JCTVC-H0697 Crosscheck of LG's adjusted significant coefficient map context for non-square blocks (JCTVC-H0476) [J. Kim, M. Kim (KAIST)] [late] [miss]

JCTVC-H0526 Multi-level Significance Maps for Small Transform Units [N.Nguyen, T.Ji, D.He, G.Martin-Cocher (RIM)]

JCTVC-H0615 Cross-check of JCTVC-H0526 on multi-level significance maps for small TU [Y. Piao, E. Alshina, J. H. Park (Samsung)] [late]

JCTVC-H0547 Significance map contexts reduction for small TU [V. Kung, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-H0628 Cross-check of Motorola’s proposal on significance map contexts reduction for small TU (JCTVC-H0547) [H. Yang, H. Yu (Huawei)] [late] [miss]

JCTVC-H0678 Non-CE11: Signaling of significant_coeffgroup_flag for significance map [V. Sze, A. Osamoto (TI)] [late]

### Last coefficient flags (x and y)

JCTVC-H0514 Chroma Context reduction for Last Coefficient Position Coding in HEVC [X. Fang, K. Panusopone, L. Wang (Motorola Mobility)]

This document describes the implementation details and reports the results of Motorola Mobility’s chroma context reduction for last coefficient position coding. Two context index assignments are proposed. The first version uses same contexts for 4x4 and 8x8 chroma TUs and the second version uses same contexts for all chroma TUs.

One comment: The current design is quite regular and common for luma and chroma coding. No reason to change.

JCTVC-H0627 Cross-check of Motorola proposal on chroma context reduction for last coefficient position coding (JCTVC-H0514) [H. Yang, H. Yu (Huawei)] [late] [miss]

JCTVC-H0537 CABAC Contexts Reduction for Last Position Coding [L. Guo, W.-J. Chien, X. Wang, M. Karczewicz (Qualcomm)]

In this proposal, a method to reduce the contexts for last position coding is proposed. First, a single context is used for the last bins (with context) from different block sizes; Second, chroma 8x8 and 16x16 blocks share the same set of contexts for last position coding. It is reported a total context saving of 16 (from 60 to 44) is achieved. Experimental results reportedly show there is 0.0% Y-BD rate change under the common test conditions.

One comment: Even though the number of contexts is lower, regularity of the table is reduced (for luma) which may be a problem with implementations (e.g. optimized code of ref software)

Other experts express the opinion that the reduction of number of contexts is beneficial.

Revisit: Tentatively adopt combination of H0537 luma and H514 chroma (version with 3 contexts) – report back with results and CD text.

JCTVC-H0612 Crosscheck of Qualcomm proposal (JCTVC-H0537) on CABAC Contexts Reduction for Last Position Coding [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

### Coefficient level coding

JCTVC-H0062 Context Simplification on coeff_abs_level_greater1_flag [Wenpeng Ding, Yunhui Shi, Baocai Yin (BJUT)]

JCTVC-H0690 Cross-check of JCTVC-H0062 on context simplification of coeff_abs_level_greater1_flag [H. Yang (Huawei)] [late]

JCTVC-H0363 Cross-check of context simplification on coeff_abs_level_greater1_flag (JCTVC-H0062) [J. Xu (Microsoft)] [late][miss]

JCTVC-H0404 Context Set Selection for Coefficient Level Coding [N.Nguyen, T.Ji, D.He, G.Martin-Cocher (RIM)]

JCTVC-H0358 Non-CE11: Cross-check of proposal H0404 on coefficient level context selection [V. Sze (TI)] [late] [miss]

JCTVC-H0497 Improving coeff_abs_level_minus3 coding performance under high bitrate [J. Lou, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-H0607 Cross-check of JCTVC-H0497: Improving coeff_abs_level_minus3 coding performance under high bitrate [X. Zhang, S. Liu (Mediatek)] [late]

JCTVC-H0498 Simplified coeff_abs_level_minus3 coding [J. Lou, L. Wang (Motorola Mobility)]

JCTVC-H0633 Cross-verification of Simplified coeff_abs_level_minus3 coding from Motorola Mobility (H0498) [L. Guo (Qualcomm)] [late] [miss]

JCTVC-H0550 Non-CE11: context assignment for parallel coefficient level coding [W. -J. Chien, J. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-H0659 Non-CE11: Crosscheck for Qualcomm's context assignment for parallel coefficient level coding in JCTVC-H0550 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

### Scans

JCTVC-H0284 Non-CE11: Unification of transform coefficient coding [C.-W. Hsu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

Proposal to use 4x4 sub-block scanning instead of diagonal also for 8x8 TU. No noticeable change in BD rate.

Horizontal/vertical scan are not change

H145, H524; H399 propose the same.

Decision: Adopt the replacement of 8x8 diagonal scan by 4x4 diagonal sub-scans.

JCTVC-H0662 Non-CE11: Cross check of Mediatek’s proposal JCTVC-H0284 [J. Kim, C. Kim (LGE)] [late]

JCTVC-H0145 Non-CE11: Harmonisation of 8x8 TU residual scan [C. Rosewarne, M. Maeda (Canon)]

Beyond H0284, this also replaces horizontal/vertical scans of 8x8 blocks by hor/ver in 4x4 sub-blocks. This loses 0.3% in AI, 0.1% in RA&LD cases.

One opinion is expressed that the horizontal/vertical scans are simple to implement (unlike the diagonal), therefore the harmonization may not be too important for these.

JCTVC-H0595 Cross-check of Canon harmonisation of 8x8 TU residual scan (JCTVC-H0145) [J. Sole (Qualcomm)] [late]

Same as H0284.

JCTVC-H0524 Extending 4×4 sub-block disagonal scan to 8×8 TU [J. Kim, C. Kim, J Chen, J. Sole, W. Chien, M. Karczewicz, B. Jeon (Qualcomm)]

JCTVC-H0698 Crosscheck of JCTVC-H0524 on extending 4×4 sub-block disagonal scan to 8×8 TU (JCTVC-H0524) [J. Kim, M. Kim (KAIST)] [late] [miss]

### Sign coding

JCTVC-H0227 Non-CE11: Sign Data Hiding without RDOQ [Gordon Clare, Felix Henry, Joel Jung, Stephane Pateux (Orange)]

JCTVC-H0481 Multiple Sign Bits Hiding [X. Yu, J. Wang, D. He, G. Martin-Cocher, S. Campbell (RIM)]

Main difference compared to H0224 is that this is based on groups of coefficients, typically one coefficient per group is changed with probability of 0.5. Better results in low-complexity configuration. The number of coefficients that is changed could effectively be higher than in H0224.

It affects the parsing process, as the existence of the sign bit can only be determined after the significance map is decoded.

One condition is checked per 4x4 block here, compared to one per TU in case of 0224. In worst case, number of condition checking would be similar

Proponents of 0224 would also support this proposal as a consistent extension of theirs

Revisit: Madhukar Budagavi will investigate the impact on implementation complexity and potential impact on visual quality and report back.

JCTVC-H0503 Cross-check of RIM’s multiple sign bits hiding (JCTVC-H0481) [J. Sole (Qualcomm)] 	[late]

### CBF

### General/other

JCTVC-H0228 Non-CE11: Proposed Cleanup for Transform Coefficient Coding [T. Nguyen, D. Marpe, T. Wiegand (Fraunhofer HHI)]

This document proposes a cleanup for transform coefficient coding of the current HEVC design. In particular, this document describes a generic scheme for the parameter selection related to the coding of transform coefficient levels with CABAC. To be more specific, the proposed cleanup relies on the same simple subset of rules for the context model selection and the Rice parameter derivation results in a simplification and harmonization. Further, all transform block sizes and shapes apply the proposed context model selection, Rice parameter derivation method, and use the same set of context models for the respective bin index resulting in a unified scheme. By applying the generic parameter selection, the proposed scheme allows the usage of a meaningful number of context models. On the top of that, the cleanup scheme has advantage over the existing coding scheme in terms of coding performance.

Presentation not uploaded

Significance coding and level coding are substantially changed

Last coeff. coding unchanged

- Same subset of rules for context model selection and Rice parameter derivation

- Usage of the same subset of rules for all transform block sizes and shapes

- Usage of the same subset of context models for all transform block sizes and shapes

- Enabling the pre-calculation of context models selected for next scan positions with respective Rice parameter

BR reduction (HE) 0.7% AI, 0.5% RA, 0.2% LD

Q: Anything changed in the scan? No, same as in current design (no cleanup)

It is mentioned that various things are changed in the software which are not documented in the contribution. As one example, init value for ALF was changed.Some concerns expressed about the possibility of reduced throughput and parallelism.due to larger amount of dependencies. For example, necessary to completely decode certain coefficients before the next ones can be decoded.

Memory requirement likely to be higher.

Further study (CE). Investigate benefit of the different elements and impact on throughput.

JCTVC-H0699 Non-CE11: Cross-check of proposal JCTVC-H0228 on transform coefficient coding [?? (??)] [late] [miss]

Cross checker mentions that this proposal introduces some additional dependencies and may provide lower throughput.

JCTVC-H0399 Harmonization of coefficient coding [J. Xu, A. Tabatabai (Sony)]

This proposal harmonizes coefficient coding for TUs with diagonal scan. The proposed solution has neat design and coding efficiency improvement compared to HM5.0. Experimental results show that coding efficiency gain is: 0.00% for AI_HE, -0.13% for AI_LC, -0.12% for RA_HE, -0.24% for RA_LC, -0.13% for LD_HE, -0.30% for LD_LC and 0.08% for RA_HE10.

4x4 sub-scan for 8x8 blocks as in 284

For diagonal scan: Multi-level significance scan and context models from 16x16 also applied in 4x4 and 8x8

Main reason for gain? Neighbor-based context model most likely.

Conclusion: This approach is supported by several experts as a desirable harmonization, BoG on significance map coding should review other proposals under this aspect. Revisit.

JCTVC-H0684 Cross-verification of JCTVC-H0399 on harmonization of coefficient coding [V. Seregin, J. Sole (Qualcomm)] [late] [miss]

Cross-verifier supports this as a desirable harmonization. Comments that it would be even better if the coding in horizontal/vertical scans would be further unified with the diagonal case.

JCTVC-H0249 Non-CE11: Coefficient sub-sampling for large transform [Y. Chiu, W. Zhang, L. Xu, Y. Han (Intel)] [late]

This contribution presents a sub-sampling process to the transform coefficients with the aim to improve the hardware design bottleneck of the worst case of the maximum number of transform coefficient to be decoded. The sub-sampling operation is applied on the transform coefficients of high frequency band, and it results in the reduction of the number of transform coefficient. Compared to the test anchor of HM5.0, one configuration of the proposed sub-sampling technique reports the worst case of the maximum number of transform coefficient to be reduced from 1024 to 768 for 32x32 transform, and the worst case of the maximum number of transform coefficient to be reduced from 256 to 224 for 16x16 transform with an overall 0.1% BD bitrate change along with slight reduction in both encoding and decoding CPU run time. Subjective tests showed that no obvious visual quality degradation was introduced by the proposed coefficient sub-sampling.

Comment: This can be done non-normatively by any encoder.

No support by other experts. No action.

JCTVC-H0677 Non-CE11: Crosscheck for Intel Coefficient Sub-sampling for Large Transform in JCTVC-H0249 [Q. Huang, X. Guo (MediaTek)] [late]

## Intra prediction and intra mode coding

### Intra prediction

#### Chroma intra prediction

JCTVC-H0177 Non-CE6a: Use of chroma phase in LM mode [E. François, S. Pautet, C. Gisquet (Canon)]

JCTVC-H0475 Signalling chroma intra prediction mode [J. Kim, B. Jeon (LG)]

JCTVC-H0339 Non-CE6: Crosscheck for LG's intra chroma mode codeword design in JCTVC-H0475 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late][

JCTVC-H0691 non-CE6.a: A joint contribution of cross-channel prediction method for chroma [K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI), Y. Chiu, Y. Han, L. Xu, W. Zhang, H. Jiang (Intel)] [late]

#### SDIP-related

see under NSQT

#### Intra prediction complexity reduction

JCTVC-H0176 Border adaptive decimation for LM mode [C.Gisquet, E. François (Canon)]

JCTVC-H0206 Crosscheck of JCTVC-H0176 [K Sato (Sony)] [late]

JCTVC-H0238 Non-CE6: Simplification of intra vertical/horizontal prediction [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

JCTVC-H0614 Crosscheck of ETRI's proposal(JCTVC-H0238) on simplification of intra vertical/horizontal prediction [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-H0244 Non-CE6: Intra-mode bypass parallelism (IMBP) [C. Rosewarne, M. Maeda (Canon)]

JCTVC-H0686 Crosscheck of JCTVC-H0244 on Intra-mode bypass parallelism [W.-J. Chien (Qualcomm)] [late]

JCTVC-H0301 Non-CE6.a: Crosscheck of Huawei’s removal of large multiplier for LM mode calculation (JCTVC-H0491) [Y.J. Chiu, L. Xu, Y. Han, W. Zhang(Intel)]

JCTVC-H0464 Non-CE6a: Using averaged down-sampling reference pixels in LM parameter generation [Lingzhi Liu, Jianhua Zheng, Philipp Zhang (HiSilicon), Guichun Li, Nam Ling, Li Song (SCU)]

JCTVC-H0490 Non-CE6a: Reduce the look-up table entries for LM mode calculation [Lingzhi Liu (HiSilicon), Guichun Li (SCU)]

JCTVC-H0624 Cross-check of JCTVC-H0464 and JCTVC-H0490 [D. Hoang (Zenverge)] [late]

JCTVC-H0491 Non-CE6a: Remove the large multiplier for LM mode calculation [Lingzhi Liu, Yongbing Lin (HiSilicon), Guichun Li (SCU)]

JCTVC-H0549 Enabling Sub-LCU level parallel decoding [S. Kumar, H. -C. Chuang, S. Xiao, X. Wang, W. -J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-H0604 Cross-check of JCTVC-H0549 [T. Davies (Cisco)] [late]

#### Intra prediction filtering

JCTVC-H0120 Non-CE6: Improvement of Intra DC prediction for cases without neighbouring samples [Alberto Duenas, Francisco Javier Roncero (Cavium)]

JCTVC-H0238 Non-CE6: Simplification of intra vertical/horizontal prediction [J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi (ETRI)]

JCTVC-H0614 Crosscheck of ETRI's proposal(JCTVC-H0238) on simplification of intra vertical/horizontal prediction [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-H0465 Simplification of mode dependent intra smoothing [Guichun Li, Nam Ling, Li Song (SCU), Lingzhi Liu, Nam Ling, Jianhua Zheng, Philipp Zhang (HiSilicon)]

JCTVC-H0374 Cross-check of Hisilicon's JCTVC-H0465 Simplification of mode dependent intra smoothing [X. Wen, O. Au (HKUST)] [late]

#### Angular intra prediction

JCTVC-H0109 Non-CE6: Modification/simplification of intra angular prediction [S. Matsuo, M. Takagi, S. Takamura, A. Shimizu, H. Jozawa (NTT)]

JCTVC-H0159 Non-CE6: Cross-check of modified/simplified intra angular prediction (JCTVC-H0109) [T. Yoshino, K. Kawamura, S. Naito (KDDI)]

### Intra mode coding

JCTVC-H0079 Non-CE6.c: Unified neighboring positions for intra mode coding [S. Fukushima, H. Nakamura (JVC Kenwood)]

JCTVC-H0081 Non-CE6.c: Fixed length binarization of remaining intra prediction mode [T. Kumakura, S. Fukushima, M. Ueda (JVC Kenwood)]

JCTVC-H0085 Non-CE6.c: Cross-verification of unified neighboring positions for intra mode coding (JCTVC-H0079 from JVC) [M. Zhou(TI)]

JCTVC-H0098 Non-CE6: Modified definitions of intra mode and most probable mode [T. Yamamoto (Sharp)]

JCTVC-H0166 Non CE6: Unification of the number of intra prediction modes [H. L. Tan, C. Yeo, Y. H. Tan, Z. Li (I2R)]

JCTVC-H0322 Cross-check of I2R’s proposal on intra luma mode coding (JCTVC-H0166) [H. Yang (Huawei)]

JCTVC-H0342 Non CE6: set intra mode number to be 35 for intra 4x4 PU [C. Lai, L. Liu, J. Zheng (HiSilicon)]

JCTVC-H0364 Non CE6: Cross-check of 35 intra prediction modes for intra 4x4 PU (JCTVC-H0342) [J. Xu (Microsoft)]

JCTVC-H0175 Non-CE6c: Adaptations of intra mode coding  [E. François, S. Pautet, C.Gisquet (Canon)]

JCTVC-H0597 Non-CE6: Cross-verification of Canon's adaptations of Intra mode coding (JCTVC-H0175) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-H0193 Non-CE6: Cross-check report for JCTVC-H0175 on Intra mode coding [H. L. Tan, C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-H0326 Non-CE6: Simplification of intra chroma mode coding [H. Li, B. Li, L. Li, J. Zhang (USTC), H. Yang, H. Yu (Huawei)]

JCTVC-H0195 Non-CE6: Cross-check report for HXXX Huawei’s on Chroma Intra coding [H. L. Tan, C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-H0428 Non-CE6: Postpone MPM sorting [V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-H0632 Non-CE6: Cross-check of Postpone MPM sorting by Qualcomm (H0428); [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-H0435 Intra mode coding with fixed length binarization [X. Zhang, S. Liu, M. Guo, X. Guo, S. Lei (MediaTek)]

JCTVC-H0493 Cross-verification of JCTVC-H0435: Intra mode coding with fixed length binarization [E. S. Ryu, Y. Ye (InterDigital)] [late]

JCTVC-H0534 Non-CE6: Modifications of intra mode coding [W. -J. Chien, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0631 Non-CE6: Cross-check of Modifications of intra mode coding by Qualcomm (H0534) [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-H0563 Non-CE6: Intra mode coding simplification [W.-J. Chien, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-H0596 Non-CE6: Cross-verification of Qualcomm's intra mode coding simplification (JCTVC-H0563) [S. Fukushima, T. Kumakura (JVC Kenwood)] [late]

JCTVC-H0059 Cross-verification of JVC's proposal on Intra mode reduction (JCTVC-H0081) [Y. Lin (HiSilicon)]

JCTVC-H0242 DC mode as a default intra mode [I.-K Kim, T. Lee, J. H. Park (Samsung)]

JCTVC-H0613 Non-CE6: Cross-verification of Samsung’s DC mode as a default mode (JCTVC-H0242) [S. Fukushima, H. Nakamura (JVC Kenwood)] [late]

JCTVC-H0405 Bypass coding in chroma Intra modes [Ehsan Maani, Ali Tabatabai (Sony)]

JCTVC-H0598 Cross-verification of Sony's Bypass coding in chroma intra modes (JCTVC-H0405) [K. Chono, H. Aoki (NEC)] [late]

JCTVC-H0407 Intra mode coding using logical mode numbering [Ehsan Maani, A. Tabatabai (Sony)]

JCTVC-H0107 Non-CE6: Crosscheck of intra mode coding (JCTVC-H0407) [T. Yamamoto (Sharp)] [late]

JCTVC-H0516 On MPM determination and Planar mode signaling [Y. Yu, K. Panusopone, L. Wang (Motorola Mobility)]

JCTVC-H0665 Cross-check of Motorola Mobility's proposal on intra mode coding (H0516) [C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-H0527 Cross-check for Intra mode renaming by Sharp [Ehsan Maani (Sony)] [late]

## Transforms

### Core transform implementation

JCTVC-H0305 The core spatial transform with fast algorithm [Hongbo Zhu] [late] [miss]

### Alternative transforms

JCTVC-H0456 Non CE 7: Experimental Results for the ROT [Z. Ma, F. Fernandes, E. Alshina, A. Alshin (Samsung)]

JCTVC-H0559 Non-CE 7 : Recent results for secondary transforms for intra/inter prediction residual [Ankur Saxena, Youji Shibahara, Elena Alshina, Felix Fernandes, T Nishi (Samsung)]

JCTVC-H0692 Non-CE7: Cross-check for Samsung's Results for Secondary transforms in JCTVC-H0559 Andrea Gabriellini, Marta Mrak (BBC) [late]

JCTVC-H0689 Cross-check of JCTVC-H0559 on the combination of secondary transform and SDIP [H. Yang (Huawei)] [late]

JCTVC-H0672 Non-CE7: Cross-check for Samsung's Results for Secondary transforms in JCTVC-H0559 [J. An, X. Guo (MediaTek)] [late]

JCTVC-H0603 Crosscheck of Samsung additional results JCTVC-H0559 for proposals JCTVC-H0125 and JCTVC-H0126 [Felix Henry (Orange)][late]

JCTVC-H0685 Cross-verification of JCTVC-H0559 on boundary dependent transform [V. Seregin (Qualcomm)] [late]

## Memory bandwidth reduction

JCTVC-H0052 Coding with Group of LCUs (GOC) [L. Liang, M. Li, J. Lin, N. Wang, W. Zuo (ZTE)]

JCTVC-H0089 AHG7: Feasibility study results on virtual motion compensation memory bandwidth verifier (VMBV) [H. Kim, M. Zhou (TI)]

JCTVC-H0096 AHG7: Controllable memory bandwidth reduction with bi-pred to uni-pred conversion [T. Ikai (Sharp)]

JCTVC-H0421 AHG7: Cross-verification of controllable memory bandwidth reduction with bi-pred to uni-pred conversion (JCTVC-H0096 from Sharp) [M. Zhou (TI)] [late]

JCTVC-H0181 AHG7: A restriction of motion vector for small PU size [T.Chujoh, T.Yamakage (Toshiba)]

JCTVC-H0104 AHG7: Crosscheck of Toshiba's memory bandwidth reduction proposal (JCTVC-H0181) [T. Ikai (Sharp)] [late]

JCTVC-H0221 AHG7: Modification of merge candidate derivation to reduce MC memory bandwidth [K. Kondo, T. Suzuki (Sony)]

JCTVC-H0105 AHG7: Crosscheck of Sony's memory bandwidth reduction proposal (JCTVC-H0221) [T. Ikai (Sharp)] [late]

JCTVC-H0267 AHG15: Constraint the number of motion vector for memory bandwidth reduction [C.S.Park, T. Kosuge, J.H. Kim, K.H. Lee, J.H.Park, C. Kim (Samsung)]

JCTVC-H0706 Cross-check report for Samsung's constraint the number of motion vector for memory bandwidth reduction (128-MV restriction in JCTVC-H0267) [S.-C. Lim, H. Y. Kim, J. Lee (ETRI)] [late]

JCTVC-H0441 Motion Compensation Restrictions to Alleviate Memory Bandwidth Concerns for High Resolution Video [T. Hellman, W. Wan (Broadcom)]

JCTVC-H0600 AHG7: Level definition to limit memory bandwidth of MC [K. Kondo, Y. Morigami, T. Suzuki (Sony)] [late]

## Alternative coding modes

### I_PCM

JCTVC-H0051 Burst transmission of I_PCM blocks [K. Chono, H. Aoki (NEC)]

JCTVC-H0611 Crosscheck of NEC's proposal (JCTVC-H0051) on Burst transmission of I_PCM blocks [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-H0469 Cross-check of NEC's proposal H0051 on burst transmission of I_PCM blocks by Panasonic [S. M. T. Naing, V. Wahadaniah, C. S. Lim (Panasonic)] [late]

### Transform skipping

JCTVC-H0141 Non-CE5: Improved transform skipping mode [J. Dong, Y. Ye (InterDigital)]

JCTVC-H0657 Non-CE5: A crosscheck report on complexity reduction for transform skipping mode (JCTVC-H0141) [R. Joshi (Qualcomm)] [late]

JCTVC-H0061 Non Transform Mode for Inter Coding [Weijia Zhu, Wenpeng Ding, Yunhui Shi, Baocai Yin (BJUT)]

JCTVC-H0325 Cross-check of BJUT’s proposal on transform skip mode (JCTVC-H0061) [H. Yang (Huawei)] [late]

JCTVC-H0259 Efficient signaling of transform skipping mode [J. Kim, M. Kim (KAIST)] [late] [miss]

### Lossless compression

JCTVC-H0083 AHG19: Method of frame-based lossless coding mode for HEVC [M. Zhou (TI)]

JCTVC-H0055 AHG19: Cross-verification of a method of frame-based lossless coding mode for HEVC [K. Chono, H. Aoki (NEC)] [late][miss]

JCTVC-H0499 Entropy coding for Lossless compression [S.H. Kim, A. Segall (Sharp)] [late]

JCTVC-H0528 AHG19: A QP-based enabling method for lossless coding in HEVC [W. Gao, M. Jiang, H. Yu (Huawei)]

JCTVC-H0530 AHG19: A lossless coding solution for HEVC [W. Gao, M. Jiang, H. Yu (Huawei), M. Zhou (TI)]

JCTVC-H0673 AHG19: Cross-check of H528 and H530 on lossless coding [Y. H. Tan, C. Yeo, Z. Li (I2R)] [late]

### Encoder optimization

JCTVC-H0178 Encoder speed-up for the MV cost estimation [G. Laroche, T. Poirier, P. Onno (Canon)]

This contribution proposes a non-normative (encoder-only) change regarding the introduction of an early termination rule. This rule deals with the computation of the rate-distortion cost of the motion vector predictors at the encoder side. More precisely, a termination rule is proposed to avoid evaluating all the rate-distortion costs of the merge candidates. It is reported that with this early termination rule, an average decrease of 5% of the encoding time is observed without any impact on the BDR measure (average BDR gain of 0%).

The modification is indicated to be 10 lines of modification in the HM code.

It was suggested to expose this as a config file option rather than a macro-enabled feature (and as a general rule to have such features become switchable in this fashion).

Decision (SW): Adopt (and enable it in common conditions – but keep in mind that experiments affecting AMVP should perhaps operate with it disabled).

JCTVC-H0241 Cross-check report for Canon's encoder speed-up for the MV cost estimation (JCTVC-H0178) [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)] [late]

JCTVC-H0213 Rate control based on unified RQ model for HEVC [H. Choi, J. Nam, J. Yoo, D. Sim (KWU), I.V. Bajić (SFU)]

## Withdrawn

JCTVC-H0060 CE11:SubTest 2 Context Simplification of Coefficient Entropy Coding – withdrawn [Wenpeng ding, Xiaoyin Che, Yunhui Shi, Baocai Yin (BJUT)]

JCTVC-H0256 Improvement on LCU-based ALF/SAO - Withdrawn [A. Minezawa, K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-H0291 Information for video editing at CRA pictures - withdrawn [C. Kim, Y. Park, J. H. Kim, J. H. Park (Samsung)]

JCTVC-H0333 Two additions dequantization – withdrawn [N. Shlyakhov, J. H. Park (Samsung)]

JCTVC-H0470 AHG4: Cross-check of LG and Sharp's proposal H0556 on harmonized method for signalling entry points of tiles and WPP substreams by Panasonic – withdrawn [S. M. T. Naing, V. Wahadaniah, C. S. Lim (Panasonic)]

JCTVC-H0529 Withdrawn/duplicate

JCTVC-H0636 Withdrawn/duplicate

JCTVC-H0647 Withdrawn

JCTVC-H0693 Withdrawn

## Category not clear

JCTVC-H0564 cross-check	(??) [late][miss]

JCTVC-H0572 AHG10: Crosscheck [?? (??)] [late] [miss]

JCTVC-H0667 Implementation issues [?? (Harmonic, Ambarella)] [late][miss]

## Not presented due to late arrival or lack of availability

# Plenary Discussions and BoG Reports

## Project development

## BoGs

JCTVC-H0709 BoG Report on MV Coding [B. Bross]

JCTVC-H0710 BoG Report on Quantization Matrices [T Suzuki, K.Sato]

Tests should be conducted immediately with the main purpose to identify whether lossy coding is affecting the quality. If proposals originally planned for being subjectively viewed are not able to provide material by 02-03 noon, they will participate in the test. The test should at minimum be run with lossless and one lossy method (one of the more low-rate versions), and in addition comparison against flat matrix.

# Project planning

## WD drafting and software

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be 20 Jan. 2012.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs

Because a draft design and HEVC test model (referred to as the HM) have now been established, group coordinated experiments are now referred to as "core experiments" rather than "tool experiments".

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-F900.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the HM 5.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision). [clean up notes]

The CE description for each planned CE is described in an associated output document JCTVC-F9xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at the preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-F900.

## Software development

The software coordinator had already started integrating bug fixes on top of HM 3.3 software, and had strongly recommended for proponents of adopted proposals to re-implement them in HM 3.3 and test in this environment before integrating them into HM 4.x. All tools were planned to again be thoroughly tested after integration in HM 4.x.

HM 4.0 should be available within 4 weeks after the meeting, and will be used for CEs. HM 4.1 is planned to be available 3 weeks later.

## Subjective test plan for design

Plan a subjective test enabling results at the Feb meeting.

Plans for subjective testing: Preparation of visual comparison for the February meeting will need to be discussed in the next meeting. Initial idea: Use a set of rates e.g. as from CfP, and run HM and JM on same conditions. It may not be necessary to run all different constraint cases, RA may be sufficient.

Need to prepare JM & HM encodings, roughly as was done for the CfP.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts - Report on project status to JCT-VC reflector - Provide report to next meeting on project coordination status | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-G1102 HEVC Test Model 5 (HM 5) Encoder Description - Produce and finalize JCTVC-G1103 HEVC text specification Working Draft 5 - Gather and address comments for refinement of these documents - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text | B. Bross, K. McCann (co-chairs), W.-J. Han, J.-R. Ohm, S. Sekiguchi, G. J. Sullivan, T. Wiegand (vice chairs) | N |
| Software development and HM software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCT-VC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 5.0 software version and the reference configuration encodings according to JCTVC-G1200 based on common conditions suitable for use in most core experiments (expected within 2.5 weeks after the meeting). - Prepare and deliver HM 5.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 5.0 version (expected within three weeks after the 5.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text | F. Bossen (chair), D. Flynn, K. Sühring (vice chairs) | N |
| Picture Partitioning and LCU scan order (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the interactions and combinations of picture partitioning and LCU scan order - Identify and work to resolve issues relating to the draft text description of picture partitioning and LCU scan order, and the associated HM functionality - Identify and discuss additional issues relating to picture partitioning and LCU scan order | R. Sjöberg (chair), Y. Chen, F. Henry, M. Horowitz, K. Kazui, A. Segall, W. Wan (vice chairs) | N |
| Spatial transforms (AHG5) (jct-vc@lists.rwth-aachen.de) - Study and discuss the "core" transforms. - Discuss the transform-related Core Experiment (CE7) along with additional transform-related proposals, and identify potential synergies or incompatibilities related to the core transform tools being investigated. - Report the results and conclusions of these studies, discussions and experiments to the JCT-VC. | P. Topiwala (chair), M. Budagavi, R. Cohen, R. Joshi (vice chairs) | N |
| In-loop filtering (AHG6) (jct-vc@lists.rwth-aachen.de) - Study simplification and harmonization of in-loop filtering technologies - Study the relationship between IPCM and deblocking filtering behavior - Clean up and stabilize the HM software, the WD text and the HM encoder description on non-deblocking in-loop filtering | T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin (vice chairs) | N |
| Memory bandwidth restrictions in motion compensation (AHG7) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth in motion compensation. - Study restrictions to reduce the average and worst case of memory bandwidth. - Study level specific restrictions, for example restrict usage of small PUs, restrict usage of 2D interpolation, restrict subpel accuracy etc, to reduce memory bandwidth in motion compensation. - Evaluate the impact of such restrictions on coding efficiency. | T. Suzuki (chair) | N |
| Profile and level definitions (AHG8) (jct-vc@lists.rwth-aachen.de) - Study potential architectures of profiles that could be used to define operation points with trade-off between complexity and compression efficiency, facilitating maximum interoperability. - Study a potential set of levels to span the target range of applications with a reasonable degree of granularity - Investigate the feasibility of meeting the vast majority of industry requirements for complexity / compression efficiency trade-off in the target 8-bit 4:2:0 video applications with a single profile. - Identify potential clustering of coding tools and constraints in terms of coding efficiency vs. complexity trade-off for the majority of applications. - Evaluate the coding tools identified in the previous mandate to help assist profiling decisions by JCT-VC. | M. Horowitz, K. McCann (co-chairs), T. Suzuki, T. K. Tan, W. Wan, Y.-K. Wang (vice chairs) | N |
| Entropy coding architecture (AHG9) (jct-vc@lists.rwth-aachen.de) - Characterize entropy coding throughput, memory, silicon area, power requirements, etc. - Study interdependencies between entropy coding and other processes and the consequences of these interdependencies - Identify and discuss additional issues on entropy coding | K. McCann (primary), J. Lainema, D. Marpe, A. Segall, K. Sugimoto, V. Sze, W. Wan, X. Wang (vice chairs) | N |
| Parallel merge/skip (AHG10) (jct-vc@lists.rwth-aachen.de) - Develop common understanding of underlying motion estimation throughput issues due to sequential merge mode - Evaluate parallel merge/skip solutions including configurable and CU-based approaches - Work on alternative solutions that would narrow the performance gap between parallel and non-parallel merge - Report the results of these studies, discussions and experiments to the JCT-VC - Recommend a unified solution if available | M. Zhou (chair), H. Y. Kim, P. Onno, X. Wen (vice chairs) | N |
| Video test material selection (AHG11) (jct-vc@lists.rwth-aachen.de) - Identify, collect, and make available a variety of video sequence test material - Study the coding performance and characteristics of test materials - Identify and recommend appropriate test materials and corresponding test conditions for use in HEVC development | T. Suzuki (chair) | N |
| Objective quality metric and alternative methods for measuring coding efficiency (AHG12) (jct-vc@lists.rwth-aachen.de) - Study and report suggested methods for objective quality assessment of reconstructed video sequences (for purposes of HEVC coding experiment evaluations). - Identify problems and possible solutions for extending any proposed objective quality metrics to color spaces other than 4:2:0 YCbCr. (e.g. 4:4:4 RGB) - Study methods to compare and report the overall coding efficiency of coding tools as a single number which captures overall rate and overall quality (over all color components) of a sequence. - Identify associated changes for HM software and Excel template (used for reporting simulation results) to support the suggested methods. | K. Minoo, G. Sullivan (co-chairs) | N |
| Interlace indication (K. Chono) (AHG13) (jct-vc@lists.rwth-aachen.de) - Develop candidate text for an interlaced format indicator, starting from a VUI flag signaling the presence of an SEI message. | K. Chono (chair), C. Fogg, K. Sato, S. Sekiguchi, W. Wan (vice chairs) | N |
| Loss robustness (AHG14) (jct-vc@lists.rwth-aachen.de) - Create and/or maintain tools to test loss robustness including error patterns and a loss simulator - Identify techniques and conditions for testing the loss robustness of the design. - Study the degree of loss robustness of the HM design and identify deficiencies - Identify and study the interdependencies in the HM design in relation to loss robustness, and the potential consequences of these interdependencies - Investigate solutions to improve loss robustness - Investigate the trade-off between coding efficiency and loss robustness - Discuss related Core Experiments, and identify potential synergies or incompatibilities related to the tools being tested in the CEs | S. Wenger (chair), M. Coban, Y. W. Huang, P. Onno, Y.-K. Wang (vice chairs) | N |
| High-level syntax (AHG15) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and slice header syntax designs - Study the partial update issue associated with adaptation parameter set - Study and identify needs for SEI messages and VUI - Study the hypothetical reference decoder (HRD) behaviour, including the need and feasibility of sub-picture based HRD operations - Assist in software development and text drafting for the high-level syntax in the HEVC design | Y. K. Wang (chair), J. Boyce, Y. Chen, M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan W. Wan, P. Wu (vice chairs) | N |
| Unification of NSQT and SDIP (AHG16) (jct-vc@lists.rwth-aachen.de) - Implement and harmonize the non-square partitioning techniques in SDIP (JCTVC-G558 and JCTVC-G754) with the NSQT in HM5. - Study and investigate combinations of PU and TU sizes that lead to the best coding efficiency and complexity tradeoff - Provide the WD text that is entirely consistent with the unified software (HM5 + JCTVC-G558 + JCTVC-G754). | X. Zheng (chair), J. Xu, X. Wang, A. Tabatabai, J. Lim (vice chairs) | N |
| Hooks for scalable coding (AHG17) (jct-vc@lists.rwth-aachen.de) - Investigate hooks that would be needed for support of bitstream scalability in HEVC syntax | J. Boyce (chair), J. Kang, J. Samuelsson, W. Wan, Y.-K. Wang (vice chairs) | N |
| Resolution adaption (AHG18) (jct-vc@lists.rwth-aachen.de) - Study methods of resolutions adaptation and their impact on complexity and compression performance - Investigate reasonable methods to assess the benefit in measurable compression performance - Investigate the impact of different filters (simple/more complex) for normative down- and up-sampling - Investigate subjective quality of resolution change e.g. limiting frequent switching - Determine the necessity of keeping both resolutions in buffer and suitable limitations on DPB for managing complexity - Investigate the relationship with alternatives such as pre- and post-filtering | T. Davies (chair), P. Topiwala, P. Wu (vice chair) | N |
| Lossless coding (AHG19) (jct-vc@lists.rwth-aachen.de) - Study and investigate techniques which enable lossless coding with minimum changes to the current design - Develop software for support of lossless coding modes - Produce a single candidate WD text specification with lossless coding capability for review at the next meeting - Examine the potential impact to the common test cases from the lossless coding modes | W. Gao (chair), K. Chono, J. Xu, M. Zhou, P. Topiwala(vice chairs) | N |
| Chroma format support (AHG20) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and software that need modification to support non-4:2:0 chroma formats. - Assist and advise in the work of removing implicit assumptions of 4:2:0 formatting from the WD and software (where feasible, without introducing technical design changes). | D. Flynn (chair), D. Hoang, K. McCann, P. Topiwala (vice chairs) | N |
| Reference picture buffering and list construction (AHG21) (jct-vc@lists.rwth-aachen.de) - Finalize the test specification document for reference picture buffering and list construction proposals for the 8th JCT-VC meeting in a timely manner (JCTVC-G1036) - Provide source code that enables HM encoding of all test cases described in JCTVC-G1036 and produce anchor data - Identify and work to resolve issues relating to the draft text description of reference picture handling and list construction, and the associated HM software functionality - Study possible improvements related to reference picture buffering and list construction - Study the loss resilience properties of reference picture handling and its support in the HM software (in coordination with loss resilience AHG) | R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, W. Wan, Y.-K. Wang (vice chairs) | N |
| HM subjective quality investigation (AHG22) (jct-vc@lists.rwth-aachen.de) - Establish a testing scenario for comparison of HM versus JM together with the test coordinator, Vittorio Baroncini - Perform encoding of HM and JM test cases. - Prepare the evaluation of test results as input to the February 2012 meeting. | G. Sullivan, J.-R. Ohm (co-chairs), F. Bossen, T. Wiegand (vice chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-H0707 HEVC software guidelines [K. Suehring, D. Flynn, F. Bossen, (software coordinators)] [late]

JCTVC-G1100 Meeting Report of 7th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-G1102 High Efficiency Video Coding (HEVC) Test Model 5 (HM 5) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, S. Sekiguchi, G. J. Sullivan] (WG 11 N 12345)

JCTVC-G1103 High Efficiency Video Coding (HEVC) text specification Working Draft 5 [B. Bross (primary), W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand] (WG 11 N 12346)

JCTVC-G1200 Common HM test conditions and software reference configurations [F. Bossen]

Based on update of JCTVC-G1000 as discussed in Wednesday closing plenary.

Software HM5.0 availability Dec. 19

Any adopted proposals where software is not delivered by the scheduled by the scheduled date will be rejected.

If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description, otherwise it cannot be claimed to be part of the CE

Document deadline of February 2012 meeting is likely to be Jan. 20th.

[Fix links below]

JCTVC-G1201 Core Experiment 1: Entropy Coding Investigation [R. Joshi (primary), E. Alshina, H. Kirchhoffer, J. Lainema, H. Sasai]

JCTVC-G1202 Core Experiment 2: Chroma RQT depth [K. Sugimoto]

JCTVC-G1203 Core Experiment 3: Chroma Interpolation Filter [T. Chujoh, E. Alshina]

Only JCTVC-G698, including subjective investigation.

Remaining issues to be investigated are related to memory bandwidth AHG

JCTVC-G1204 Core Experiment 4: Quantization [K. Sato (primary), H. Aoki, M. Budagavi, M. Coban, X. Li]

JCTVC-G1205 Core Experiment 5: Transform skipping [M.Mrak, J. Sole, J. Xu, A. Saxena]

JCTVC-G1206 Core Experiment 6: Intra coding improvements [A. Tabatabai (primary), E. François, K. Chono, R. Joshi, J. Lainema, H. Yu]

Includes all intra coding aspects (also chroma prediction, intra mode coding simplification, SDIP improvements over "reference" of AHG).

JCTVC-G1207 Core Experiment 7: Additional transforms [R. Cohen (primary), F. Fernandes, R. Joshi, C. Yeo]

JCTVC-G1208 Core Experiment 8: Non-deblocking loop filtering [T. Yamakage (primary), I. S. Chong, M. Narroschke, Y.-W. Huang]

JCTVC-G1209 Core Experiment 9: MV coding and skip/merge operation [I. K. Kim (primary), W. J. Chien, J. Jung, I. K. Kim, M. Zhou]

Combined with former CE13

- JCTVC-G1210 Core Experiment 10: Deblocking filter [A. Norkin (primary), X. Guo, B. Jeon, M. Narroschke]

- JCTVC-G1211 Core Experiment 11: Coefficient scanning and coding [V. Sze (primary), J. Chen, T. Nguyen, K. Panusopone, J. Sole]

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 30 April – 8 May 2012 under ITU-T auspices in Geneva, CH.

- 11–20 July 2012 under WG 11 auspices in Stockholm, SE.

- 10–19 October 2012 under WG 11 auspices in Shanghai, CN.

- 16–23 January 2013 under ITU-T auspices in Geneva, CH.

- …

ITU was thanked for its excellent hosting of the 7th meeting of the JCT-VC and for providing the viewing equipment used at the meeting. David Flynn and Kenneth Andersson were thanked for assisting with the setup and operation of the equipment.

The JCT-VC meeting was closed at approximately 1335 hours on Wednesday 30 Nov 2011.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixth meeting of the JCT-VC, according to a sign-in sheet passed around during the meeting (approximately XXX in total), were as follows: