| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21–30 November, 2011 | Document: JCTVC-G1103_d9 |
| --- | --- |

| Title: | WD5: Working Draft 5 of High-Efficiency Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | Working Draft of HEVC |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Woo-Jin Han Gachon University Jens-Rainer Ohm RWTH Aachen Gary J. Sullivan Microsoft Thomas Wiegand Fraunhofer HHI / TU Berlin | Email: Email: Email: Email: Email: | benjamin.bross@hhi.fraunhofer.de hurumi@gmail.com ohm@ient.rwth-aachen.de garysull@microsoft.com thomas.wiegand@hhi.fraunhofer.de |
| Source: | Editor |  |  |

_____________________________

# Abstract

Changes to relevant text in Working Draft 5 of High-Efficiency Video Coding to incoporate JCTVC-H0531.

### Slice header syntax

| slice_header( ) { |  | Descriptor |  |
| --- | --- | --- | --- |
| first_slice_in_pic_flag |  | u(1) |  |
| if( first_slice_in_pic_flag = = 0 ) |  |  |  |
| slice_address |  | u(v) |  |
| slice_type |  | ue(v) |  |
| entropy_slice_flag |  | u(1) |  |
| if( !entropy_slice_flag ) { |  |  |  |
| pic_parameter_set_id |  | ue(v) |  |
| if( IdrPicFlag ) { |  |  |  |
| idr_pic_id |  | ue(v) |  |
| no_output_of_prior_pics_flag |  | u(1) |  |
| } |  |  |  |
| else { |  |  |  |
| pic_order_cnt_lsb |  | u(v) |  |
| short_term_ref_pic_set_pps_flag |  | u(1) |  |
| if( !short_term_ref_pic_set_pps_flag ) |  |  |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |  |  |
| Else |  |  |  |
| short_term_ref_pic_set_idx |  | u(v) |  |
| if( long_term_ref_pics_present_flag ) { |  |  |  |
| num_long_term_pics |  | ue(v) |  |
| for( i = 0; i < num_long_term_pics; i++ ) { |  |  |  |
| delta_poc_lsb_lt [ i ] |  | ue(v) |  |
| delta_poc_msb_present_flag[ i ] |  | u(1) |  |
| if( delta_poc_msb_present_flag[ i ] = = 1 ) |  |  |  |
| delta_poc_msb_cycle_lt_minus1[ i ] |  | ue(v) |  |
| used_by_curr_pic_lt_flag[ i ] |  | u(1) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| if( scaling_list_enable_flag ¦ ¦ deblocking_filter_in_APS_enabled_flag ¦ ¦ sample_adaptive_offset_enabled_flag ¦ ¦ adaptive_loop_filter_enabled_flag ) { |  |  |  |
| if( sample_adaptive_offset_enabled_flag ) |  |  |  |
| Slice_sample_adaptive_offset_flag |  | u(1) |  |
| if( adaptive_loop_filter_enabled_flag) |  |  |  |
| Slice_adaptive_loop_filter_flag |  | u(1) |  |
| aps_id |  | ue(v) |  |
| } |  |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |  |
| Num_ref_idx_active_override_flag |  | u(1) |  |
| if( num_ref_idx_active_override_flag ) { |  |  |  |
| Num_ref_idx_l0_active_minus1 |  | ue(v) |  |
| if( slice_type = = B ) |  |  |  |
| num_ref_idx_l1_active_minus1 |  | ue(v) |  |
| } |  |  |  |
| } |  |  |  |
| ref_pic_list_modification( ) |  |  |  |
| ref_pic_list_combination( ) |  |  |  |
| } |  |  |  |
| if( slice_type != I ) |  |  |  |
| cabac_init_idc |  | ue(v) |  |
| if( !entropy_slice_flag ) { |  |  |  |
| slice_qp_delta |  | se(v) |  |
| inherit_dbl_params_from_APS_flag |  | u(1) |  |
| if ( !inherit_dbl_params_from_APS_flag ) { |  |  |  |
| Disable_deblocking_filter_flag |  | u(1) |  |
| if ( !disable_deblocking_filter_flag ) { |  |  |  |
| beta_offset_div2 |  | se(v) |  |
| tc_offset_div2 |  | se(v) |  |
| } |  |  |  |
| } |  |  |  |
| if( slice_type = = B ) |  |  |  |
| collocated_from_l0_flag |  | u(1) |  |
| if( ( weighted_pred_flag && slice_type = = P) ¦ ¦ ( weighted_bipred_idc = = 1 && slice_type = = B ) ) |  |  |  |
| pred_weight_table( ) |  |  |  |
| } |  |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) |  |  |  |
| 5_minus_max_num_merge_cand |  | ue(v) |  |
| if( adaptive_loop_filter_enabled_flag && aps_adaptive_loop_filter_flag ) |  |  |  |
| alf_cu_control_param( ) |  |  |  |
| for( i = 0; i < num_substreams_minus1 + 1; i++ ){ |  |  |  |
| Substream_length_mode |  | u(2) |  |
| Substream_length[i] |  | u(v) |  |
| } |  |  |  |
| } |  |  |  |

### Slice header semantics

When present, the value of the slice header syntax elements pic_parameter_set_id, idr_pic_id, no_output_of_prior_pics_flag, pic_order_cnt_lsb, short_term_ref_pic_set_pps_flag, short_term_ref_pic_set_idx and num_long_term_pics shall be the same in all slice headers of a coded picture. When present, the value of the slice header syntax elements delta_poc_lsb_lt_minus1[ i ] and used_by_curr_pic_lt_flag[ i ] shall be the same in all slice headers of a coded picture for each i in the range of 0 to num_long_term_pics, inclusive.

first_slice_in_pic_flag indicates whether the slice is the first slice of the picture. If first_slice_in_pic_flag is equal to 1, the variables SliceAddress and LCUAddress are both set to 0 and the decoding starts with the first LCU in the picture.

slice_address specifies the address in slice granularity resolution in which the slice starts and shall be represented by ( Ceil( Log2( NumLCUsInPicture ) ) + SliceGranularity ) bits in the bitstream where NumLCUsInPicture is the number of LCUs in a picture.

The variable LCUAddress is set to ( slice_address >> SliceGranularity ) and represents the LCU part of the slice address in raster scan order. The variable GranularityAddress is set to ( slice_address - ( LCUAddress << SliceGranularity ) ) and represents the sub-LCU part of the slice address expressed in z-scan order.

[Ed. (BB): “z-scan order” is not defined anywhere and should be defined, or the phrase herein is changed not to use the term.]

The variable SliceAddress is then set to ( LCUAddress << ( log2_diff_max_min_coding_block_size << 1 ) ) + ( GranularityAddress << ( ( log2_diff_max_min_coding_block_size << 1 ) – SliceGranularity ) and the slice decoding starts with the largest coding unit possible at the slice starting coordinate.

slice_type specifies the coding type of the slice according to Table 7-3.

Table 07-13 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | P (P slice) |
| 1 | B (B slice) |
| 2 | I (I slice) |

When nal_unit_type is equal to 5 (IDR picture), slice_type shall be equal to 2.

When max_num_ref_frames is equal to 0, slice_type shall be equal to 2.

entropy_slice_flag equal to 1 specifies that the value of slice header syntax elements not present shall be inferred to be equal to the value of slice header syntax elements in a proceeding slice, where a proceeding slice is defined as the slice containing treeblock with location (LCUAddress – 1). entropy_slice_flag shall be equal to 0 when LCUAddress equal to 0.

pic_parameter_set_id specifies the picture parameter set in use. The value of pic_parameter_set_id shall be in the range of 0 to 255, inclusive.

idr_pic_id identifies an IDR picture. The values of idr_pic_id in all the slices of an IDR picture shall remain unchanged. When two consecutive access units in decoding order are both IDR access units, the value of idr_pic_id in the slices of the first such IDR access unit shall differ from the idr_pic_id in the second such IDR access unit. The value of idr_pic_id shall be in the range of 0 to 65535, inclusive.

no_output_of_prior_pics_flag specifies how the previously-decoded pictures in the decoded picture buffer are treated after decoding of an IDR picture. See Annex C. When the IDR picture is the first IDR picture in the bitstream, the value of no_output_of_prior_pics_flag has no effect on the decoding process. When the IDR picture is not the first IDR picture in the bitstream and the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_frame_buffering derived from the active sequence parameter set is different from the value of pic_width_in_luma_samples or pic_height_in_luma_samples or max_dec_frame_buffering derived from the sequence parameter set active for the preceding picture, no_output_of_prior_pics_flag equal to 1 may (but should not) be inferred by the decoder, regardless of the actual value of no_output_of_prior_pics_flag.

pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When pic_order_cnt_lsb is not present, pic_order_cnt_lsb shall be inferred to be equal to 0.

short_term_ref_pic_set_pps_flag equal to 1 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the active picture parameter set. short_term_ref_pic_set_pps_flag equal to 0 specifies that the short-term reference picture set of the current picture shall be created using syntax elements in the short_term_ref_pic_set( ) syntax structure in the slice header.

short_term_ref_pic_set_idx specifies the index to the list of the short-term reference picture sets specified in the active picture parameter set that shall be used for creation of the reference picture set of the current picture. The syntax element short_term_ref_pic_set_idx shall be represented by ceil( log2( num_short_term_ref_pic_sets ) ) bits. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets – 1, inclusive, where num_short_term_ref_pic_sets is the syntax element from the active picture parameter set.

The variable StRpsIdx is derived as follows.

if( short_term_ref_pic_set_pps_flag )
	StRpsIdx = short_term_ref_pic_set_idx	(7-9)
else
	 StRpsIdx = num_short_term_ref_pic_sets

num_long_term_pics specifies the number of the long-term reference pictures that are to be included in the long-term reference picture set of the current picture. The value of num_long_term_pics shall be in the range of 0 to max_num_ref_frames – NumNegativePics[ StRpsIdx ] – NumPositivePics[ StRpsIdx ] , inclusive. When not present, the value of num_long_term_pics shall be inferred to be equal to 0.

delta_poc_lsb_lt [ i ] is used to determine the value of the least significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture. delta_poc_lsb_lt_minus1[ i ] shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

The variable DeltaPocLt[ i ] is derived as follows.

if (i  = =  0) 
	DeltaPocLt[ i ] = delta_poc_lsb_lt[ i ]	(7-9)
else
	DeltaPocLt[ i ] = delta_poc_lsb_lt[ i ] + DeltaPocLt[ i – 1 ]

The value of DeltaPocLt[ i ] shall be in the range of 0 to MaxPicOrderCntLsb, inclusive.

delta_poc_msb_present_flag[i] equal to 1 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is present.  delta_poc_msb_present_flag[ i ] equal to 0 specifies that delta_poc_msb_cycle_lt_minus1[ i ] is not present.  delta_poc_msb_present_flag[ i ] shall be equal to 1 when DeltaPocLt[ i ] is equal to DeltaPocLt[ j ] for values of j in the range of 0 to num_long_term_pics – 1, inclusive, and being unequal to i.

delta_poc_msb_cycle_lt_minus1[ i ] is used to determine the value of the most significant bits of the picture order count value of the i-th long-term reference picture that is included in the long-term reference picture set of the current picture.

The variable DeltaPocMSBCycleLt[ i ] is derived as follows.

if ( i  = =  0  | |  DeltaPocLt[ i – 1 ]  ! =  DeltaPocLt[ i ] ) 
	  DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt_minus1[ i ] + 1				
	else
	  DeltaPocMSBCycleLt[ i ] = delta_poc_msb_cycle_lt_minus1[ i ] + 1 +

DeltaPocMSBCycleLt [ i – 1 ]

used_by_curr_pic_lt_flag[ i ] equal to 0 specifies that the i-th long-term reference picture included in the long-term reference picture set of the current picture is not used for reference by the current picture.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_l0_active_minus1 is present for P and B slices and that the syntax element num_ref_idx_l1_active_minus1 is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 are not present.

When the current slice is a P or B slice and field_pic_flag is equal to 0 and the value of num_ref_idx_l0_default_active_minus1 in the picture parameter set exceeds 15, num_ref_idx_active_override_flag shall be equal to 1.

When the current slice is a B slice and field_pic_flag is equal to 0 and the value of num_ref_idx_l1_default_active_minus1 in the picture parameter set exceeds 15, num_ref_idx_active_override_flag shall be equal to 1.

num_ref_idx_l0_active_minus1 specifies the maximum reference index for reference picture list 0 that shall be used to decode the slice.

When the current slice is a P or B slice and num_ref_idx_l0_active_minus1 is not present, num_ref_idx_l0_active_minus1 shall be inferred to be equal to num_ref_idx_l0_default_active_minus1.

The range of num_ref_idx_l0_active_minus1 is specified as follows.p

–	If field_pic_flag is equal to 0, num_ref_idx_l0_active_minus1 shall be in the range of 0 to 15, inclusive. When MbaffFrameFlag is equal to 1, num_ref_idx_l0_active_minus1 is the maximum index value for the decoding of frame macroblocks and 2 * num_ref_idx_l0_active_minus1 + 1 is the maximum index value for the decoding of field macroblocks.

–	Otherwise (field_pic_flag is equal to 1), num_ref_idx_l0_active_minus1 shall be in the range of 0 to 31, inclusive.

num_ref_idx_l1_active_minus1 specifies the maximum reference index for reference picture list 1 that shall be used to decode the slice.

When the current slice is a B slice and num_ref_idx_l1_active_minus1 is not present, num_ref_idx_l1_active_minus1 shall be inferred to be equal to num_ref_idx_l1_default_active_minus1.

The range of num_ref_idx_l1_active_minus1 is constrained as specified in the semantics for num_ref_idx_l0_active_minus1 with l0 and list 0 replaced by l1 and list 1, respectively.

cabac_init_idc specifies the index for determining the initialisation table used in the initialisation process for context variables. The value of cabac_init_idc shall be in the range of 0 to 2, inclusive.

slice_qp_delta specifies the initial value of QPY to be used for all the macroblocks in the slice until modified by the value of cu_qp_delta in the coding unit layer. The initial QPY quantisation parameter for the slice is computed as

SliceQPY = 26 + pic_init_qp_minus26 + slice_qp_delta	(7-9)

The value of slice_qp_delta shall be limited such that SliceQPY is in the range of −QpBdOffsetY to +51, inclusive.

disable_deblocking_filter_flag equal to 1 specifies that the operation of the deblocking filter shall be disabled and disable_deblocking_filter_flag equal to 0 specifies that the operation of the deblocking filter shall be enabled.

collocated_from_l0_flag equal to 1 specifies the picture that contains the co-located partition shall be derived from list 0, otherwise the picture shall be derived from list 1.

5_minus_max_num_merge_cand specifies the maximum number of merging MVP candidates supported in the slice subtracted from 5. The maximum number of merging MVP candidates, MaxNumMergeCand is computed as

MaxNumMergeCand = 5 − 5_minus_max_num_merge_cand	(7-9)

The value of 5_minus_max_num_merge_cand shall be limited such that MaxNumMergeCand is in the range of 0 to 5, inclusive.

substream_length_mode specifies the number of bits to be used to decode syntax element substream_length[ ], as defined in Table 7-4.

Table 0-2 – Number of bits used to decode substream_length

| substream_length_mode | Number of bits |
| --- | --- |
| 0 | 8 |
| 1 | 16 |
| 2 | 24 |
| 3 | 32 |

substream_length[i] specifies the number of bits to be used during the initialization process for the current bitstream pointer i.

### Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 0.1.4. The process may result in marking one or more reference pictures as "unused for reference".

NOTE 1 – The reference picture set is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The reference picture set signaling is explicit in the sense that all reference pictures included in the reference picture set are listed explicitly and there is no default reference picture set construction process in the decoder that depends on the status of the decoded picture buffer.

Short-term reference pictures are identified by their PicOrderCntVal values. Long-term reference pictures are identified by their pic_order_cnt_lsb values.

Five lists of picture order count values are constructed to derive the reference picture set; PocStCurr0, PocStCurr1, PocStFoll, PocLtCurr, and PocLtFoll with NumPocStCurr0, NumPocStCurr1, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll number of elements, respectively.

If the current picture is an IDR picture, PocStCurr0, PocStCurr1, PocStFoll, PocLtCurr, and PocLtFoll are all set to empty, and NumPocStCurr0, NumPocStCurr1, NumPocStFoll, NumPocLtCurr, and NumPocLtFoll are all set to 0.

Otherwise, the following applies for derivation of the five lists of picture order count values and the numbers of entries.

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] ; i++ ) {
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		PocStCurr0[ j++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS0[ StRpsIdx ][ i ]
}
NumPocStCurr0 = j

for( i = 0, j = 0; i < NumPositivePics[ StRpsIdx ]; i++ ) {
	if( UsedByCurrPicS1[ StRpsIdx ][ i ] )
		PocStCurr1[ j++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
	else
		PocStFoll[ k++ ] = PicOrderCntVal + DeltaPocS1[ StRpsIdx ][ i ]
}
NumPocStCurr1 = j
NumPocStFoll = k		(8-9)

for( i = 0, j = 0, k = 0; i < num_long_term_pics; i++ ) {

if( delta_poc_msb_present_flag[ i ]  = =  1 ) {

if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] )* MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb ) – ( DeltaPocMSBCycleLt[ i ] ) * MaxPicOrderCntLsb
	}

else{
		if( used_by_curr_pic_lt_flag[ i ] )
			PocLtCurr[ j++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) % 
				MaxPicOrderCntLsb
		else
			PocLtFoll[ k++ ] = ( PicOrderCntVal − DeltaPocLt[ i ] + MaxPicOrderCntLsb ) %
				MaxPicOrderCntLsb

}
   }

NumPocLtCurr = j
NumPocLtFoll = k

where PicOrderCntVal is the picture order count of the current picture as specified in subclause 8.2.1.

NOTE 2 – A value of StRpsIdx in the range from 0 to num_short_term_ref_pic_sets – 1, inclusive, indicates that a short-term reference picture set from the active parameter set is being used, where StRpsIdx is the index of the short-term reference picture set to the list of short-term reference picture sets in the order in which they are signalled in the picture parameter set. StRpsIdx equal to num_short_term_ref_pic_sets indicates that a short-term reference picture set explicitly signalled in the slice header is being used.

The reference picture set consists of five lists of reference pictures; RefPicSetStCurr0, RefPicSetStCurr1, RefPicSetStFoll, RefPicSetLtCurr and RefPicSetLtFoll.

NOTE 3 –RefPicSetStCurr0, RefPicSetStCurr1 and RefPicSetLtCurr contains all reference pictures that may be used in inter prediction of the current picture and that may be used in inter prediction of one or more of the pictures following the current picture in decoding order. RefPicSetStFoll and RefPicSetLtFoll consists of all reference pictures that are not used in inter prediction of the current picture but may be used in inter prediction of one or more of the pictures following the current picture in decoding order.

The marking of a reference picture can be "unused for reference", "used for short-term reference", or "used for long-term reference", but only one among these three. When a reference picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both). A reference picture that is marked as "used for short-term reference" is referred to as a short-term reference picture. A reference picture that is marked as "used for long-term reference" is referred to as a long-term reference picture.

Derivation process for the reference picture set and picture marking shall be performed according to the following ordered steps, where DPB refers to the decoded picture buffer as described in Annex C:

- The following applies:

for( i = 0; i < NumPocLtCurr; i++ ) {

if(delta_poc_msb_present_flag[i] ==0){
	if( there is a long-term reference picture picX in the DPB
			           with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
		RefPicSetLtCurr[ i ] = picX
	else if( there is a short-term reference picture picY in the DPB
			                  with pic_order_cnt_lsb equal to PocLtCurr[ i ] )
		RefPicSetLtCurr[ i ] = picY
	else 
		RefPicSetLtCurr[ i ] = "no reference picture"

}

else{

if( there is a long-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocLtCurr[ i ] )
		RefPicSetLtCurr[ i ] = picX
	else if( there is a short-term reference picture picY in the DPB
			                  with PicOrderCntVal equal to PocLtCurr[ i ] )
			RefPicSetLtCurr[ i ] = picY
	else 
			RefPicSetLtCurr[ i ] = "no reference picture"	
}

}

for( i = 0; i < NumPocLtFoll; i++ ) {

if(delta_poc_msb_present_flag[i] ==0){
	if( there is a long-term reference picture picX in the DPB
			           with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
			RefPicSetLtFoll[ i ] = picX
	else if( there is a  short-term reference picture picY in the DPB
			                  with pic_order_cnt_lsb equal to PocLtFoll[ i ] )
		RefPicSetLtFoll[ i ] = picY
	else 
		RefPicSetLtFoll[ i ] = "no reference picture"

}

else{

if( there is a long-term reference picture picX in the DPB
			           with PicOrderCntVal to PocLtFoll[ i ] )
			RefPicSetLtFoll[ i ] = picX
	else if( there is a  short-term reference picture picY in the DPB
			                  with PicOrderCntVal equal to PocLtFoll[ i ] )
		RefPicSetLtFoll[ i ] = picY
	else 
		RefPicSetLtFoll[ i ] = "no reference picture"
}

}

- All reference pictures included in RefPicSetLtCurr and RefPicSetLtFoll are marked as "used for long-term reference"

- The following applies:

for( i = 0; i < NumPocStCurr0; i++ ) {
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurr0[ i ])
			RefPicSetStCurr0[ i ] = picX
	else
		RefPicSetStCurr0[ i ] = "no reference picture"
}

for( i = 0; i < NumPocStCurr1; i++ ) {
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStCurr1[ i ])
			RefPicSetStCurr1[ i ] = picX
	else
		RefPicSetStCurr1[ i ] = "no reference picture"
} 				(8-9)

for( i = 0; i < NumPocStFoll; i++ ) {
	if( there is a short-term reference picture picX in the DPB
			           with PicOrderCntVal equal to PocStFoll[ i ])
			RefPicSetStFoll[ i ] = picX
	else
		RefPicSetStFoll[ i ] = "no reference picture"
}

- All reference pictures included in RefPicSetStCurr0, RefPicSetStCurr1 and RefPicSetStFoll are marked as "used for short-term reference".

- All reference pictures in the decoded picture buffer that are not included in RefPicSetLtCurr, RefPicSetLtFoll, RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetStFoll are marked as "unused for reference".

NOTE 4 – There may be one or more reference pictures that are included in the reference picture set but that are not present in the decoded picture buffer. Entries in RefPicSetStFoll or RefPicSetLtFoll equal to "no reference picture" should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSetStCurr0, RefPicSetStCurr1 and RefPicSetLtCurr equal to "no reference picture".

The reference picture set is restricted as follows:

- There shall be no reference picture with temporal_id greater than that of the current picture included in RefPicSetStCurr0, RefPicSetStCurr1 and RefPicSetLtCurr.

- There shall be no reference picture included in the reference picture set which precedes, in output order, any CRA picture that precedes the current picture both in decoding order and output order.

- There shall be no entry in RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetLtCurr equal to "no reference picture".

NOTE 5 – A reference picture can not be included in more than one of the five reference picture set lists.

### Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P, SP, or B slice.

Decoded reference pictures are marked as "used for short-term reference" or "used for long-term reference" as specified by the bitstream and specified in subclause 8.2.2.4. Short-term reference pictures are identified by the value of frame_num. Long-term reference pictures are assigned a long-term picture index as specified by the bitstream and specified in subclause 8.2.2.4.

Subclause 8.2.2.1 is invoked to specify

–	the assignment of variables FrameNum, FrameNumWrap, and PicNum to each of the short-term reference pictures, and

–	the assignment of variable LongTermPicNum to each of the long-term reference pictures.

Reference pictures are addressed through reference indices as specified in subclause Error! Reference source not found.. A reference index is an index into a reference picture list. When decoding a P or SP slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, reference picture list RefPicList0, and for B slices RefPicList1, are derived as specified by the following ordered steps:

- An initial reference picture list RefPicList0 and for B slices RefPicList1 are derived as specified in subclause 8.2.2.2.

- When ref_pic_list_modification_flag_l0 is equal to 1 or, when decoding a B slice, ref_pic_list_modification_flag_l1 is equal to 1, the initial reference picture list RefPicList0 and, for B slices, RefPicList1 are modified as specified in subclause 8.2.2.3.

NOTE – The modification process for reference picture lists specified in subclause 8.2.2.3 allows the contents of RefPicList0 and for B slices RefPicList1 to be modified in a flexible fashion. In particular, it is possible for a picture that is currently marked "used for reference" to be inserted into RefPicList0 and for B slices RefPicList1 even when the picture is not in the initial reference picture list derived as specified in subclause 8.2.2.2.

The number of entries in the modified reference picture list RefPicList0 is num_ref_idx_l0_active_minus1 + 1, and for B slices the number of entries in the modified reference picture list RefPicList1 is num_ref_idx_l1_active_minus1 + 1. A reference picture may appear at more than one index in the modified reference picture lists RefPicList0 or RefPicList1.

#### Initialisation process for reference picture lists

This initialisation process is invoked when decoding a P or B slice header.

When decoding a P or B slice, there shall be at least one reference picture in RefPicSetStCurr0, RefPicSetStCurr1 or RefPicSetLtCurr.

The following procedure is conducted to construct the initial RefPicList0:

cIdx = 0
while( cIdx <= num_ref_idx_l0_active_minus1 )
{
	for( i=0; i < NumPocStCurr0 && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicList0[ cIdx ] = RefPicSetStCurr0[ i ]
	for( i=0;  i < NumPocStCurr1 && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )	(8-9)
		RefPicList0[ cIdx ] = RefPicSetStCurr1[ i ]
	for( i=0; i < NumPocLtCurr && cIdx <= num_ref_idx_l0_active_minus1; cIdx++, i++ )
		RefPicList0[ cIdx ] = RefPicSetLtCurr[ i ]
}

The following procedure is conducted to construct the initial RefPicList1:

cIdx = 0
while( cIdx <= num_ref_idx_l1_active_minus1 ) {
	for( i=0; i < NumPocStCurr1 && cIdx <= num_ref_idx_l1_active_minus1; cIdx++, i++ )
		RefPicList1[ cIdx ] = RefPicSetStCurr1[ i ] 
	for( i=0; i < NumPocStCurr0 && cIdx <= num_ref_idx_l1_active_minus1; cIdx++, i++ ) 	(8-9)
		RefPicList1[ cIdx ] = RefPicSetStCurr0[ i ]
	for( i=0; i < NumPocLtCurr && cIdx <= num_ref_idx_l1_active_minus1; cIdx++, i++ )
		RefPicList1[ cIdx ] = RefPicSetLtCurr[ i ]
}

#### Modification process for reference picture lists

After the invocation of this process, there shall be no reference pictures with greater temporal_id than the current slice included in the output RefPicList0 or RefPicList1.

When ref_pic_list_modification_flag_l0 is equal to 1, the following applies:

- Let refIdxL0 be an index into the reference picture list RefPicList0. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause 8.2.2.3.1 is invoked with refIdxL0 as input, and the output is assigned to refIdxL0.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause 8.2.2.3.2 is invoked with refIdxL0 as input, and the output is assigned to refIdxL0.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList0 is finished.

When the current slice is a B slice and ref_pic_list_modification_flag_l1 is equal to 1, the following applies:

- Let refIdxL1 be an index into the reference picture list RefPicList1. It is initially set equal to 0.

- The corresponding syntax elements modification_of_pic_nums_idc are processed in the order they occur in the bitstream. For each of these syntax elements, the following applies.

–	If modification_of_pic_nums_idc is equal to 0 or equal to 1, the process specified in subclause 8.2.2.3.1 is invoked with refIdxL1 as input, and the output is assigned to refIdxL1.

–	Otherwise, if modification_of_pic_nums_idc is equal to 2, the process specified in subclause 8.2.2.3.2 is invoked with refIdxL1 as input, and the output is assigned to refIdxL1.

–	Otherwise (modification_of_pic_nums_idc is equal to 3), the modification process for reference picture list RefPicList1 is finished.

##### Modification process of reference picture lists for short-term reference pictures

Input to this process is an index refIdxLX (with X being 0 or 1).

Output of this process is an incremented index refIdxLX.

The variable picNumLXNoWrap is derived as follows.

If ref_pic_list_modification_idc is equal to 0, the following applies.

–	If the current reference picture list is RefPicList0, curRefPicSet is set to RefPicSetStCurr0.
–	Otherwise (the current reference picture list is RefPicList1), curRefPicSet is set to RefPicSetStCurr1.

–	Otherwise, if ref_pic_list_modification_idc is equal to 1, the following applies.

–	If the current reference picture list is RefPicList0, curRefPicSet is set to RefPicSetStCurr1.
–	Otherwise (the current reference picture list is RefPicList1), curRefPicSet is set to RefPicSetStCurr0.

–	Otherwise, if ref_pic_list_modification_idc is equal to 2, curRefPicSet is set to RefPicSetLtCurr.

The variable pocLX is derived as follows.

pocLX = curRefPicSet[ ref_pic_set_idx ] 		(8-9)

The following procedure is conducted to place the picture picR with PicOrderCnt( picR ) equal to pocLX into the index position refIdxLX, shift the position of any other remaining pictures to later in the list, and increment the value of refIdxLX.

for( cIdx = num_ref_idx_lX_active_minus1 + 1; cIdx > refIdxLX; cIdx− − )
	RefPicListX[ cIdx ] = RefPicListX[ cIdx − 1]
RefPicListX[ refIdxLX++ ] =  pocLX
nIdx = refIdxLX		(8-9)
for( cIdx = refIdxLX; cIdx <= num_ref_idx_lX_active_minus1 + 1; cIdx++ ) 
	if( PicOrderCnt( RefPicListX[ cIdx ] ) != pocLX )
		RefPicListX[ nIdx++ ] = RefPicListX[ cIdx ]

NOTE 2 – Within this pseudo-code procedure, the length of the list RefPicListX is temporarily made one element longer than the length needed for the final list. After the execution of this procedure, only elements 0 through num_ref_idx_lX_active_minus1 of the list need to be retained.

#### Mapping process for reference picture lists combination in B slices

[Ed.: (WJ) needs to be checked once again. Try to find better way to represent]

This initialisation process is invoked when decoding a B slice header.

Input to this process are the reference picture list RefPicListX and num_ref_idx_lX_active_minus1 with X being 0 or 1.

Outputs of this process are arrays PredLCToPredLx and RefIdxLCToRefIdxLx.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 0, the following ordered steps apply:

      - Let refIdxL0 and refIdxL1 be indices into the reference picture lists RefPicListL0 and RefPicListL1. They are initially set equal to 0.

      - Let refIdxLC be an index into PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

      - The following process is repeated until refIdxL0 and refIdxL1 are both greater than num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1, respectively:

–	If refIdxL0 is less than or equal to num_ref_idx_l0_active_minus1,

–	If the entry RefPicListL0[ refIdxL0 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-9)
			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL0.

–	refIdxL0++.

–	If refIdxL1 is less than or equal to num_ref_idx_l1_active_minus1 and ref_pic_list_combination_flag equal to 1,

–	If the entry RefPicListL1[ refIdxL1 ] is the first occurance of the reference picture,

PredLCToPredLx[ refIdxLC ] = Pred_L1, 	(8-10)			RefIdxLCToRefIdxLx[ refIdxLC++ ] = refIdxL1.

–	refIdxL1++.

When the current slice is a B slice and ref_pic_list_modification_flag_lc is equal to 1, the following ordered steps apply:

- Let refIdxLC be an index into the reference picture list PredLCToPredLx and RefIdxLCToRefIdxLx. It is initially set equal to 0.

- The corresponding syntax elements pic_from_list_0_flag and ref_idx_list_curr are processed in the order they occur in the bitstream. For each of these syntax elements pairs, the following applies.

–	If pic_from_list_0_flag is equal to 1,

PredLCToPredLx[ refIdxLC ] = Pred_L0, 	(8-11)

–	Otherwise,

PredLCToPredLx[ refIdxLC ] = Pred_L1	(8-12)

–	RefIdxLCToRefIdxLx[ refIdxLC++ ] = ref_idx_list_curr

When refIdxLC is greater than num_com_ref_list_active_minus1+ 1, the extra entries past position num_com_ref_list_active_minus1 are discarded from PredLCToPredLx and RefIdxLCToRefIdxLx.

When refIdxLC is less than num_com_ref_list_active_minus1 + 1, the remaining entries in PredLCToPredLx and RefIdxLCToRefIdxLx are set equal to Pred_L0 and 0, respectively.

### Marking of reference pictures before decoding

This process is invoked once per picture, after decoding of a slice header and the decoding process for reference picture set as specified in 0.1.3 but prior to the decoding of any coding unit and prior to the decoding process for reference picture list construction of the slice as specified in subclause 0.1.4.

When enable_temporal_mvp_flag of the current picture is equal to 0 and temporal_id of the current picture is equal to 0, all the reference pictures in the DPB are marked as “unused for temporal motion vector prediction”.

### Marking of the current picture after decoding

This process is invoked after all slices of the current picture have been decoded.

–	If nal_ref_idc of the current picture equals 0, the current picture is marked as "unused for reference".

- Otherwise, the current picture is marked as "used for short-term reference".

- ---

