| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 9th Meeting: Geneva, CH, 27 April – 7 May 2012 | Document: JCTVC-I0607r2 |
| --- | --- |

| Title: | On allocation of NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8345 yekuiw@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes an allocation scheme of NAL unit types, as well as restrictions on decoding order of NAL unit types with an access unit.

# Current allocation of NAL unit types

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-IDR, non-CRA and non-TLA picture slice_layer_rbsp( ) | VCL |
| 2 | Reserved | n/a |
| 3 | Coded slice of a TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 6 | Supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 7 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 8 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 9 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 10-11 | Reserved | n/a |
| 12 | Filler data filler_data_rbsp( ) | non-VCL |
| 13 | Reserved | n/a |
| 14 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 15-23 | Reserved | n/a |
| 24..63 | Unspecified | non-VCL |

# Summary of current NAL unit type allocation and restrictions

- Unspecified: 0, 24..63

  - shall not precede the first VCL NAL unit of the coded picture

- Reserved: 2, 10, 11, 13, 15..23

  - 15..18: may start a new access unit

  - 20..23: shall not precede the first VCL NAL unit of the coded picture

  - 2, 10, 11, 13: no order restriction specified

- Other (specified and not reserved) NAL unit types: 1, 3..9, 12, 14

  - 1, 4..9, 14: may start a new access unit

    - 6 (SEI): shall precede the first VCL NAL unit of the coded picture

    - 9 (AUD): shall be the first NAL unit in an access unit

  - 12 (FD): shall not precede the first VCL NAL unit of the coded picture

# Proposed allocation of NAL unit types

Table 3-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- |
| 0 | Unspecified | non-VCL |
| 1 | Coded slice of a non-RAP, non-TLA and non-TFD picture slice_layer_rbsp( ) | VCL |
| 2 | Coded slice of a TFD picture slice_layer_rbsp( ) | VCL |
| 3 | Coded slice of a non-TFD TLA picture slice_layer_rbsp( ) | VCL |
| 4 | Coded slice of a CRA picture with TFD pictures slice_layer_rbsp( ) | VCL |
| 5 | Coded slice of a CRA picture without TFD pictures slice_layer_rbsp( ) | VCL |
| 6 | Coded slice of BLC picture with TFD pictures slice_layer_rbsp( ) | VCL |
| 7 | Coded slice of BLC picture without TFD pictures slice_layer_rbsp( ) | VCL |
| 8 | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9..24 | Reserved | n/a |
| 25 | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 26 | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 27 | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 28 | Adaptation parameter set aps_rbsp( ) | non-VCL |
| 29 | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 30 | Filler data filler_data_rbsp( ) | non-VCL |
| 31 | AU-level supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32 | Sub-AU-level supplemental enhancement information (SEI) sei_rbsp( ) | non-VCL |
| 32..47 | Reserved | n/a |
| 48..63 | Unspecified | non-VCL |

# Summary of proposed NAL unit type allocation and restrictions

- Unspecified: 0, 48..63

  - shall not precede the first VCL NAL unit of the access unit

    - The 16 NAL unit types 48..63 are "Unspecified" and cannot be used future extensions, as they can be used in external specifications, such as HEVC RTP payload format and HEVC file format

- Reserved: 9..24

  - (These 16 NAL unit types are "Reserved" potentially for use as VCL NAL units in future extensions, but we don't need to say this now)

  - shall not precede the first VCL NAL unit of the access unit

- Reserved: 33..47

  - (These 15 NAL unit types are "Reserved" potentially for use as non-VCL NAL units in future extensions, but we don't need to say this now)

  - 33..43 may start a new access unit

  - 44..47 shall not precede the first VCL NAL unit of the coded picture

    - These 16 NAL unit types are "Reserved" for use as VCL NAL units in future extensions

- Other (specified and not reserved) NAL unit types: 1..9

  - 1..8: VCL NAL units

  - 25..31 (VPS, SPS, PPS, APS, AUD, FD, AU-level SEI)

    - As currently specified for VPS, SPS, PPS, APS, AUD, FD and AU-level SEI

  - 32 (Sub-AU-level SEI): shall precede the last VCL NAL unit of the access unit

    - This is being proposed to be different than in the HEVC WD 6 - this way it is better for both future scalability and 3DV extensions as well as transmission systems (e.g., in SVC RTP payload format, SEI NAL units can be transported in different sessions in multi-session transport, due to the restriction in SVC, a special reordering process has to be specified in the payload format, RFC 6190).

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).