| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the tenth meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Stockholm, SE, 11-20 July 2012 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its tenth meeting during 11–20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin, in Stockholm, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

A meeting of AHG 9 (high level syntax) was held on Tuesday 10 July 2012. Discussions and recommendations of this AHG are in included in section 5.13, where decisions on these issues were made by the JCT plenary.

The JCT-VC meeting sessions began at approximately 900 hours on Wednesday 11 July 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1300 hours on Friday 20 July. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC).

The primary goals of the meeting were to review the work that was performed in the interim period since the ninth JCT-VC meeting in producing the 7th HEVC Test Model (HM7) software and text and editing the 7th HEVC specification Draft (which was issued as an ISO/IEC Study of Committee Draft document), review the results from an interim Core Experiment (CE), review technical input documents, establish the 8th draft of the HEVC specification (to be issued as an ISO/IEC Draft International Standard – DIS) and the eighth version of the HEVC Test Model (HM8), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-VC produced three particularly important output documents from the meeting: the HEVC Test Model 8 (HM8), the HEVC specification draft 8 a.k.a. Draft International Standard (DIS), and a document specifying common conditions and software reference configurations for HEVC coding experiments. Moreover, plans were established to conduct X future CEs in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-VC established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 10–19 Oct 2012 under WG 11 auspices in Shanghai, CN, 14–23 January 2013 under ITU-T auspices in Geneva, CH, 17-26 April 2013 under WG 11 auspices in Incheon, KR, and XX July – 02 Aug 2013 under WG 11 auspices in Vienna, AT.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its tenth meeting during 11-20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin in Stockholm, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

A meeting of AHG 9 (high level syntax) was held on Tuesday 10 July 2012, the day before the JCT-VC meeting started, at the same meeting site.

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Wednesday 11 July 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1300 hours on Friday 20 July. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC).

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		XXX people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2012_04_I_Geneva/.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the ninth JCT-VC meeting in producing the 7th HEVC Test Model (HM) software and text and editing the 7th HEVC specification Working Draft (WD7), review the results from interim Core Experiments (CEs), review technical input documents, establish the 8th draft of the HEVC specification and the 8th version of the HEVC Test Model (HM8), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 2 July 2012.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 3 July 2012 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-J0371 to JCTVC-J0XXX were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "J0370+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical proposal contributions were registered in time but were uploaded late:

- JCTVC-J0XXX (a technical proposal) [uploaded XX-XX]

- ...

The following other documents not proposing normative technical content were registered in time but uploaded late:

- JCTVC-J0XXX (a contribution ...)

- ...

The following cross-verification reports were uploaded late: JCTVC-J0XXX, ... .

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCTVC-J0XXX, ... .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-J0XXX (a contribution of ... , corrected ...)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report JCTVC-I1000, the HEVC Test Model (HM) JCTVC-I1002, the Draft Specification JCTVC-I1003, and the Draft Disposition of Comments JCTVC-I1004 were approved. The HM reference software produced by the AHG on software development and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chair asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it has previously been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- AIS: Adaptive intra smoothing.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- AMVR: Adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution coding.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ET: Encoding time.

- GPB: Generalized P/B – a not-particularly-well-chosen name for B pictures in which the two reference picture lists are identical.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PIPE: Probability interval partitioning entropy coding (roughly synonymous with V2V for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SPS: Sequence parameter set (as in AVC).

- TE: Tool Experiment – a coordinated experiment conducted between the 1st and 2nd or 2nd and 3rd JCT-VC meeting.

- TM: Test Model – a video coding design containing selected coding tools; as contrasted with the TMuC, see HM.

- TMuC: Test Model under Consideration – a video coding design containing selected proposed coding tools that are under study by the JCT-VC for potential inclusion in the HEVC standard.

- Unit types:

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- V2V: variable-length to variable-length prefix coding (roughly synonymous with PIPE for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

DIS ballot timing issue.

## Scheduling of discussions

Scheduling: Generally 0800 – 2200

Wednesday: 0900 start; Plenary to 1pm; Split tracks 1430 onwards; Track A ended 2000.

Mon morning not meet for MPEG plenary.

Wed morning not meet for MPEG plenary.

Fri 20 end by lunchtime.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm.

- AHG reports (14) Track P (section 2)

- Project development, status, and guidance (3) Track P (section 3 and section 5.1)

- CE1: Intra transform mode dependency simplifications (8) Track B (section 4.1) Thu 3.

- Clarifications and bug fix issues (4) Track P (section 5.1)

- HM settings and common test conditions (0) Track P (section 5.4)

- HM coding performance (2) Track P (section 5.5)

- Profile/level definitions (16) Track P (section 5.6)

- Source video test material (1) Track P (section 5.7)

- Functionalities (11) Track A (section 5.8)

- Deblocking filter (16) Track B (section 5.9) Thu. 1.

- Non-deblocking loop filters (68) Track B (section 5.10) (BOG ALF:Yamakage; SAO: Elena): Review Thu 2.

- Block structures and partitioning (8) Track B (section 5.11) Wed 3.

- Motion and mode coding (36) Track B (section 5.12) Thu 4.

- High-level syntax and tile/slice structures (108) Track A (section 5.13)

- Quantization (14) Track B (section 5.14) Wed 1.

- Entropy coding (4) Track B (section 5.15) Thu 5.

- Transform coefficient coding (47) Track B (section 5.16) Thu 6.

- Intra prediction and mode coding (7) Track B (section 5.17) Wed 2.

- Transforms (3) Track B (section 5.18) With CE1

- Memory bandwidth reduction (6) Track A (section 5.19)

- Alternative coding modes (41) Track A (section 5.20)

- Non-normative: Encoder optimization, post filtering (5) Track A (section 5.21)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: 40; Track A: 171; Track B: 211.

Regarding CE, decide about subjective viewing later.

# AHG reports

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

AHG1 [miss]

given verbally (business as usual, no issues of concern)

JCTVC-J0002 JCT-VC AHG report: HEVC Draft and Test Model editing (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs)]

The seventh High Efficiency Video Coding (HEVC) test model (HM7) was developed from the sixth HEVC test model (HM6), following the decisions taken at the 9th JCT-VC meeting in Geneva (27 April to 7 May 2012).

Two editorial teams were formed to work on the two documents that were to be produced:

JCTVC-I1002 HEVC Test Model 7 (HM 7) Encoder Description

- Il-Koo Kim

- Ken McCann

- Kazuo Sugimoto

- Benjamin Bross

- Woo-Jin Han

JCTVC-I1003 High Efficiency Video Coding (HEVC) text specification draft 7 [2]

- Benjamin Bross

- Woo-Jin Han

- Jens-Rainer Ohm

- Gary J. Sullivan

- Thomas Wiegand

Editing JCTVC-I1003 was assigned a higher priority than editing JCTVC-I1002.

An issue tracker (http://hevc.kw.bbc.co.uk/trac) was used in order to facilitate the reporting of issues on the text of both documents.

One version of JCTVC-I1002 and eight successive versions of JCTVC-I1003 were published by the Editing AHG following the 9th JCT-VC meeting in Geneva.

The main changes in JCTVC-I1002 and JCTVC-I1003, relative to the previous JCTVC- H1002, were listed in the report.

The recommendations of the HEVC Draft and Test Model Editing AHG were to:

- Approve the edited JCTVC-I1002 and JCTVC-I1003 documents as JCT-VC outputs

- Continue to edit both documents to ensure that all agreed elements of HEVC are fully described

- Encourage the use of the issue tracker (http://hevc.kw.bbc.co.uk/trac) to facilitate the reporting of issues with the text of either document

- Compare the HEVC documents with the HEVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG

- Continue to improve the overall editorial quality of the HEVC draft text specification, to allow it to proceed to DIS ballot

- Ensure that properly drafted candidate text for both the HEVC draft text specification and the HM Test Model (if appropriate) is available prior to making any decision to change the HEVC specification

The last item above was particularly noted, as a "ratcheting up" of the need for stability and focus on getting things complete, coherent and finalized.

The AHG recommends that text should be provided and approved before making a decision for inclusion – in the previous period some text was arriving quite late in some cases. Confirmed by group consensus.

JCTVC-J0003 JCT-VC AHG report: Software development and HM software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring]

A brief summary of activities related to each mandate is given below.

1. Development of the software was coordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN servers set up at HHI and the BBC, as announced on the JCT-VC email reflector.

2. Version 7.0 of the software was delivered to schedule and reference configuration encodings were provided according to the common test conditions through an ftp site at the BBC.

ftp://ftp.kw.bbc.co.uk/hevc/hm-7.0-anchors/

3. Version 7.1 of the software was delivered ahead of the 10th JCT-VC meeting.

4. Some high-level adoptions were still outstanding at the time of writing.

Multiple versions of the HM software had been produced and announced on the JCT-VC email reflector. The changes made for each version were summarized in the report. A detailed history of all changes made to the software can be viewed at http://hevc.kw.bbc.co.uk/trac/timeline.

Released versions of the software are available on the SVN server at the following URL:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-7.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-7.0-dev).

Version 7.0 of the software was released on 23rd May 2012. It includes all the changes adopted at the 9th JCT-VC meeting that affect the common test conditions. This release was announced on the email reflector.

The performance change since HM-6.0 was tabulated in the report.

Some gain was achieved from adding AMP, but a number of small losses tended to offset that. Class F AI HE10 showed substantial gain, primarily due to adding the transform skip feature.

Version 7.1 of the software was released on 30th June 2012. It contains a number of bug fixes and the majority of adoptions form the 9th JCT-VC meeting that do not affect the common conditions. A number of integrations were still outstanding at the point of writing. There is virtually no performance change between HM-7.0 and HM-7.1 under the common conditions.

Version 7.2 was planned for release during the current meeting.

In addition to the regular HM development process, one branch was created to expose tools to a wider audience:

- HM-7.1-dev-ahg13, which contains contains modifications to the reference picture buffers and list construction.

Recommendations of the AHG were as follows:

- Continue to develop reference software based on HM version 7.1 and improve its quality.

- Remove macros introduced inHM previous HMversions before startingintegration towards HM8.0 such as to make the software more readable

- Continue to identify bugs and discrepancies with text, and address them

- Test reference software more extensively outside of common test conditions

Several simplifications (restriction of motion comp in small PUs, SAO, etc.) added up in losses of up to 1%; AMP compensates for that such that the performance of HM7 is only very slightly inferior than HM6. The approx. 1% loss appears in HE10, where AMP was already switched on before (except class F, where transform skip which was adopted provides additional gain (4% rate reduction of HM7 vs. HM6).

JCTVC-J0004 JCT-VC AHG report: High-level parallelism (AHG4) [M. Horowitz (eBrisk), M. Coban (Qualcomm), F. Henry (Orange Labs), K. Kazui (Fujitsu), A. Segall (Sharp Labs), W. Wan (Broadcom), S. Wenger (Vidyo), M. Zhou (TI)]

There was no significant email activity for this AHG.

The chairs of AHG4 were unaware of any high-level parallel processing related open issues at this time.

The relevant changes made to the HM were reviewed.

The 22 related input documents to the Stockholm meeting were listed and categorized into five categories:

- General (not specific to tiles or WPP)

- Tiles

- WPP (wavefront parallel processing)

- Entry points

- P&L (profile and level)

JCTVC-J0005 JCT-VC AHG report: Entropy Coding Improvements (AHG 5) [A. Segall (chair), C. Auyeung, K. Chono, G. Martin-Cocher, T. Nguyen, J. Sole, V. Sze, W. Wan]

There were approximately 28 e-mail messages exchanged on the reflector.  Messages were primarily related toward discussing and recommending (i) test conditions, (ii) software and (iii) reporting methods.  Five of these messages included both AHG5 and AHG6 activities.

Recommendations made by the AHG were reviewed in the report, and included

- Test conditions related to cu_qp_delta

- Reporting methods for contributions considering the case of context bin reduction

- The use of a software patch and Excel spreadsheet for collecting the necessary data for the above.

- Recommending that only the worst case number of context and bypass coded bins be mandatory for reporting in related contributions.

The 38 input contributions related to the AhG activities were listed in the report. They were categorized into the following subjects:

- Delta QP (4)

- Reference Index (3)

- SAO (9)

- Transform level coding (4)

- Other (4)

- Cross checks (14)

Note that JCTVC-J0194 is categorized in the “Other” sub-category but also related to Delta QP, Reference Index and SAO sub-categories.

Overview of characteristics of delta QP contributions:

|  | HM7.0 | JCTVC-J0060 | JCTVC-J0226 | JCTVC-J0089 |  | JCTVC-J0298 |
| --- | --- | --- | --- | --- | --- | --- |
| Binarization | TU | EG0 | TU+EG0 |  | TU+EG0 | TU+FLC |
| Max_bin | 27 | 11 | 15 |  | 15 | 16 |
| Max_ctx_bin | 26 | 1 | Method1: 5 Method2: 3 Method3: 2 Method4: 1 |  | 5 | 4 |
| Max_bypass_bin | 1 | 10 | Method1: 10 Method2: 12 Method3: 13 Method4: 14 |  | 10 | 12 |
| Interleaving of context bins and bypass bins | Yes | No | No |  | No | No |
| Max Average BD rate changes (%) (Y,U,V) Class F is excluded |  | 0.20/0.15/0.55 (Y/U/V) | Method 1 0.03/0.10/0.34 (Y/U/V) Method 2 0.06/0.11/0.09 (Y/U/V) Method 3 0.06/0.20/0.26 (Y/U/V) Method 4 0.09/0.20/0.33 (Y/U/V) |  | 0.03/0.09/0.14 (Y/U/V) | 0.02/0.11/0.24 (Y/U/V) |

Overview of characteristics of reference index related contributions:

|  | HM7.0 | JCTVC-J0098 |  | JCTVC-J0176 | JCTVC-J0297 |
| --- | --- | --- | --- | --- | --- |
| Binarization | TU | Method1 TU | Method2 TU+EG0 | TU | TU+FLC |
| Max_bin | 15 | 15 | 11 | 15 | 8 |
| Max_ctx_bin | 15 | 2 | 2 | 4 | 4 |
| Max_bypass_bin | 0 | 13 | 9 | 11 | 4 |
| Max Average BD rate changes (Y,U,V) Class F is excluded |  | 0.01%(Y) 0.13%(U) 0.13%(V) |  | 0.01%(Y) 0.13%(U) 0.13%(V) | 0.00%(Y) 0.00%(U) 0.00%(V) |

JCTVC-J0006 JCT-VC AHG report: In-loop filtering (AHG6) [T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin, P. Onno (vice-chairs)]

There were about 40 email exchanges for ALF on the JCT-VC main reflector. Most of the emails were for decoding time discussion for ALF. About 10 emails for SAO were exchanged related to merge_up_flag and frame-based SAO parameter optimization.  In response to the discussion for merge_up_flag, there are several contributions for this topic.

In addition, HM/WD tickets are reported and most of the tickets have been solved.

The relevant ticket issues and input contributions were listed in the report.

The AHG recommended to study all input contributions and to create a BoG for DF, SAO and ALF.

For ALF, the AHG recommended to conduct an informal subjective picture quality viewing.

BoG to be run

JCTVC-J0007 JCTVC AHG Report: Memory bandwidth restrictions in motion compensation (AHG7) [T. Suzuki (chair), W. Wan, M. Zhou (vice-chairs)]

Six contributions were noted to be relevant. The relevant technical proposes were noted as follows:

- On bi-predictive motion vectors for inter PUs of 8x4 and 4x8, JCTVC-J0086 and JCTVC-0312 propose to disable to encode bi-pred MV for 8x4 and 4x8 by changing CABAC. In the current spec, it is allowed to send such MV, but decoder discards it.

- On constraints on high resolution and high frame-rate application, JCTVC-J0175 proposes to constrain bi-pred 8x8 for large picture only (e.g. 4K). Since such constraint is not necessary for HDTV, this contribution proposes to change the constraint depending on the level (picture size).

- On bi-pred merge candidate derivation, JCTVC-J0218 proposes to restrict the merge MV during merge candidate derivation.

JCTVC-J0008 JCTVC AHG Report: Loss robustness (AHG8) [Arturo Rodriguez (Chair)]

No activity was reported

JCTVC-J0009 JCT-VC AHG report: High-level syntax (AHG9) [G. J. Sullivan (AHG meeting co-chair), Y.-K. Wang (AHG chair and AHG meeting co-chair), J. Boyce, Y. Chen, M. M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan, P. Wu (AHG vice chairs)] [miss]

This AHG report was reviewed in Track A after lunch on Wed.

There were some email discussions relating to this AHG on the following topics:

- Constraint on number of bits per coding tree block

- Usefulness of restricted_ref_pic_lists_flag and related

- Slice granularity and end_of_slice_flag

There are input documents to this meeting addressing the issues related to the first topics.

Related contributions were listed in the AHG report.

The AHG held a face-to-face meeting from 0900‒1800 on Tuesday 10 July 2012 at the Conference Center venue of 10th JCT-VC meeting that was to begin on the following day. The AHG meeting was chaired by Ye-Kui Wang and Gary Sullivan. Meeting minutes and AHG recommendations made during the face-to-face meeting are also included in the report.

In the time available in the AHG meeting, the AHG reviewed 12 contributions in the following three topic areas:

- NAL unit header (4 contributions)

- Picture order count (3 contributions)

- Slices (5 contributions)

The notes reported by the AHG on these contributions were used as the starting basis of the notes recorded in this report.

During the review of this AHG report, a participant remarked that we should consider imposing some restriction on changes of the APS within a picture.

JCTVC-J0010 JCT-VC AHG report: Hooks for scalable coding (AHG10) [J. Boyce, J. Kang, J. Samuelsson, W. Wan, Y. K. Wang]

No particular email reflector discussion was reported. There were 17 contributions noted as relevant, categorized into the following areas.

- NUH

- VPS

- SPS

- MV coding

- VUI

- SEI

- Layer switching

JCTVC-J0011 JCT-VC AHG report: Lossless Coding (AHG11) [Wen Gao (chair), Keiichi Chono, Felix Henry, Jizheng Xu, Minhua Zhou, Pankaj Topiwala (vice chairs)]

During the interim period between 9th and 10th JCT-VC meeting, the adopted lossless coding related contributions had been integrated into HM7.1 software and HEVC specification text (JCTVC-I1003). During the integration process, An HM Ticket, #580, reported an encoder/decoder mismatch for lossless coding under LB configurations. The bug was fixed in HM7.0-dev-r2461.

During the AHG11 discussions on JCT-VC reflector, it was suggested to use HM7.0-dev-r2461 as the reference software since it was not clear whether HM7.1 would be released in time for lossless coding related simulations. It was also noted that HM7.0-dev-r2461 only has code on frame-level lossless coding.

Furthermore, the following test conditions and test scenarios are discussed and agreed on JCT-VC reflector, listed as follows:

Test Conditions: AI-main, LB-main, and RA-main, with following setting on lossless coding

- LosslessCuEnabled set to 0

- TransquantBypassEnableFlag set to 1

- CUTransquantBypassFlagValue set to 1

Test Scenarios:

- Frame level Lossless coding: All frames are lossless coded.

  - QP setting: QP = 0

  - Class A-F sequences

- Region based lossless coding: In each frame, three regions with the following top-left and bottom right coordinates are lossless coded.

  - Region 1: (272, 32) , (1119, 143)

  - Region 2: (208, 352), (495, 623)

  - Region 3: (1152,  192+16*floor(POC/100)),  (1231, 223+16*floor(POC/100))

- Two F-class 720p sequences to be tested: SlideShow and SlideEditing.

- QP setting: QP=22, 27, 32, 37

There were 10 contributions identified as related to AHG11, together with two cross verification reports.

AHG12

Ten relevant documents were noted to have been contributed, covering the following areas:

- Non-8-bit coding (1 contribution)

- Non-4:2:0 coding by extension of current design (4 contributions)

- Non-4:2:0 coding using new methods (3 contributions)

- Deriving 4:2:0 from 4:4:4 (1 contribution)

- Test sequences (1 contribution)

It was recommended to present the identified documents and seek to define a timeline for range extensions.

In discussion of the AHG report, the primary timeline expectation seemed to be 1 year beyond version 1.

- Timeline for range extensions to be specified

- How many profiles would be useful?

JCTVC-J0013 JCT-VC AHG report: Reference picture buffering and list construction (AHG13) [R. Sjöberg, Y. Chen, Hendry, T.K. Tan, Y.-K. Wang]

The test recommendation for reference picture buffering and list construction proposals was discussed on the reflector. It was decided to include reference picture duplication into test case 2.8 and remove all references to the combined list as that was taken out from the draft standard at the previous JCT-VC meeting. The document was sent out for review on June 18 on the main reflector. No comments were received and that document was uploaded as JCTVC-I0608.

It was noted that JCTVC-I0608 was not uploaded until after the previous meeting had ended, although the document was uploaded as an input to that meeting because of the way the web site was functioning at the time it was uploaded. It was suggested for that to also be provided as a new input to the current meeting so that it will be more appropriately categorized as being input to the current meeting.

The RPS bit cost measurements were reported to show the average percentage of related syntax bits that are spent on RPS related syntax. This was in the range of 0.0–0.7%.

Twelve relevant proposal contributions were identified in the report.

JCTVC-J0014 JCTVC AHG Report: Study on HEVC conformance requirements (AHG14) [T. Suzuki, W. Wan]

There was no discussion on the reflector, and no relevant contributions were submitted.

The AHG chairs raise the following questions on the reflector to initiate discussions.

- Whether to define HEVC conformance similar to the past standards

- Test methodology: The followings are defined for AVC:

  - dynamic test: to confirm decoder can decode in real time

  - static test: to check the decoded picture is perfectly matched with HM output

- What kind of bitstreams should be generated.

- The need for a plan to develop the conformance spec.

It was noted that the development of bitstreams eventually used for the prior conformance test set used for AVC was begun with bitstream exchange activity.

Design and exchange of bitstreams should be started after this meeting.

# Project development, status, and guidance

## Conformance test set development

JCTVC-J0291 Instructive (and sometimes evil) conformance bitstreams [C. Fogg (Harmonic), A. Wells (Ambarella)]

## Draft text specification improvements [the right place for this?]

# Core experiments

## CE1: Intra transform mode dependency simplifications

### Summary

JCTVC-J0021 CE1: Summary report of Core Experiment on intra transform mode dependency simplifications [K. Ugur, A. Saxena (CE coordinators)]

Three non-CE contributions were also noted to be relevant. These are listed in section 5.18.

In this core experiment, two simplifications were tested. Simplification 1 uses 2D DST for all intra prediction modes of 4x4 luma TUs rather than using mixed transform types (4 difference cases). Simplification 2 uses DST for all intra prediction modes of 4x4 luma TUs except that the DC mode is coded with DCT. Both simplifications show coding efficiency loss ranging between 0.0–0.1% on average excluding class F. In class F there was some more loss – between 0.2‒0.8% on average.

Visual testing was planned to be done for simplification 1 only.

It is intended to do visual testing

### Contributions

JCTVC-J0030 CE1: Cross-verification of Intra transform mode dependency simplifications (JCTVC-J0021) [R. Cohen (MERL)]

JCTVC-J0034 CE1: Cross-check of Intra transform mode dependency simplifications  [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

JCTVC-J0035 CE1: Nokia’s results on intra transform mode dependency simplifications [K. Ugur, O. Bici (Nokia)]

This contribution presents the CE1 results by Nokia on intra transform mode dependency simplifications. In this core experiment, two simplifications were tested. Simplification 1 uses 2D DST for all intra prediction modes of 4x4 luma TU’s. Simplification 2 uses 2D DST for all intra prediction modes of 4x4 luma TU’s except the DC mode is coded with 2D DCT. Both simplifications show coding efficiency loss ranging between 0.0%-0.1%.

BR increase is 0.5% in class F (0.8% for low QP) for simplification 1, 0.3% for simplification 2.

JCTVC-J0276 CE1: Crosscheck of Nokia’s results on intra transform mode dependency simplifications (JCTVC-J0035) for low QPs [R. Joshi (Qualcomm)]

JCTVC-J0129 CE1: Cross-check of mode-dependent transform simplifications [C. Yeo, Y. H. Tan (I2R)]

An additional variant is tested where DST is used for all 4x4 TUs for both luma and chroma and for both inter and intra. This was not tested visually, and was not advocated for adoption. It was remarked that the 4x4 transform is a subset of the larger transforms anyway.

JCTVC-J0388 Cross-check of simplification 3 of JCTVC-J0129 [K. Ugur (Nokia)] [late]

JCTVC-J0243 CE1: Cross-check of intra transform mode dependency simplifications [J. Xu (Microsoft)]

# Non-CE Technical Contributions

## HEVC Standard Development

### Technical suggestions

## JCTVC-J0292 Suggested figures for HEVC specification [C. Fogg (Harmonic)]

JCTVC-J0293 Lumpy Intra frames in HEVC [C. Fogg (Harmonic)]

## Clarification and Bug Fix Issues

JCTVC-J0335 Explicit Specification of Bounds on Syntax Elements [B. Heng, T. Hellman, W. Wan (Broadcom)]

JCTVC-J0133 raises the same/similar issue

JCTVC-J0336 Clarification of the semantics of no_residual_data_flag [Z. Yang, P. Chen, W. Wan (Broadcom)]

  - HM settings and common test conditions

No contributions were noted to be specific to this subject area.

## HM coding performance

JCTVC-J0128 On software complexity: decoding 720p content on a tablet [F. Bossen (Docomo Innovations)]

JCTVC-J0236 Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile [J. Xu (Microsoft)]

## Profile/level definitions (for version 1 of HEVC)

### NB comments

JCTVC-J0437 UKNB Comment on HEVC Profiles	[UK National Body] [late]

JCTVC-J0477 JNB comments on HEVC extensions to support non-4:2:0, n-bit video [Japan National Body]

JCTVC-J0478 JNB comments on UHDTV support in HEVC [Japan National Body]

### Main Profile

JCTVC-J0125 On profiling [Y.-K. Wang, H. Reddy (Qualcomm), A. Luthra (Motorola), L. Winger (Magnum)]

JCTVC-J0151 Study on HEVC profiles and levels [T. Suzuki, M. Haque, A. Tabatabai (Sony)]

JCTVC-J0264 Dependent slices support in HEVC main profile [T. Schierl, V. George, A. Henkel, D. Marpe (HHI)]

JCTVC-J0289 On parallel partitions in profiles [A. Wells (Ambarella), C. Fogg (Harmonic)]

JCTVC-J0307 Inclusion of adaptive loop filtering (ALF) in Main profile of the HEVC standard [T. Yamakage (Toshiba), Y.-W. Huang (MediaTek), I. S. Chong (Qualcomm)]

JCTVC-J0330 Inclusion of ALF in Main profile and additional test results [T. Ikai, Y. Yasugi, T. Yamamoto, T. Tsukuba, T. Aono (Sharp

JCTVC-J0338 Specification of Parallelization Tools in HEVC Version 1 [W. Wan, T. Hellman (Broadcom)]

JCTVC-J0282 AHG4: On mandatory parallelism [S. Worrall, A. Wise (Aspex), J. Viéron (ATEME), P. Andrivon, P. Bordes (Technicolor)]

JCTVC-J0284 Tiles and Wavefront Parallel Processing Restrictions For The Main Profile [S. Worrall, A. Wise]

JCTVC-J0147 Subjective evaluation on ALF [J. Takiue, T.K. Tan, A. Fujibayashi, Y. Suzuki (NTT Docomo)]

### Levels

JCTVC-J0056 Additional level for coding high resolution pictures [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0154 Proposal for definition of levels in HEVC [Mukta Kar, Arianne Hinds, Yasser Syed, Arturo Rodriguez, Wade Wan, Ajay Luthra, Lowell Winger, Clint Sheridan, Paul Haskell]

JCTVC-J0334 Adding a Level Restriction on Coding Tree Block Size [W. Wan, T. Hellman (Broadcom)]

### Beyond Main Profile

JCTVC-J0037 On still picture profile [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0152 Proposals on HEVC professional extensions [T. Suzuki, K. Sharman, N. Saunders, A. Tabatabai (Sony)]

## Source video test material

JCTVC-J0197 4:4:4 test sequence [K. Sugimoto (Mitsubishi)] [late]

## Functionalities

### Colour component sampling and higher bit-depth

JCTVC-J0078 AHG12: Non-4:2:0 formats syntax modifications [P. Andrivon, P. Bordes (Technicolor)]

JCTVC-J0079 AHG12: On beyond 8 bit-depth support in HEVC [P. Andrivon, P. Bordes (Technicolor)]

JCTVC-J0127 Integer Color Transforms and Resampling Filters for HEVC Applications [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

JCTVC-J0191 Extension of HM7 to Support Additional Chroma Formats [P. Silcock, K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-J0233 Syntax and semantics of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Tao Lin, Peijun Zhang, Shuhui Wang, Kailun Zhou, Xianyi Chen]

JCTVC-J0352 BD-rate performance vs. dictionary size and hash-table memory size in Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Peijun Zhang, Tao Lin, Xianyi Chen, Shuhui Wang, Kailun Zhou] [late]

JCTVC-J0353 R-D cost based effectiveness analysis of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Xianyi Chen, Tao Lin, Peijun Zhang, Shuhui Wang, Kailun Zhou] [late]

JCTVC-J0357 AHG12: 4:2:2/4:4:4 chroma format extension for HEVC Version 2 [K. Kawamura, T. Yoshino, S. Naito (KDDI Corp.)]

JCTVC-J0358 Chroma intra prediction based on residual luma samples in 4:2:2 chroma format [K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI Corp.)]

### Interlaced

JCTVC-J0258 Interlaced coding performance and Chroma consideration [Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse (ATEME)]

JCTVC-J0466 Performance of HEVC for Interlaced Video [A. Luthra, D. Baylon (Motorola Mobility)] [late]

## Deblocking filter

JCTVC-J0066 On Cross-Slice Deblocking Edge Ordering [P. Kapsenberg (Intel)]

JCTVC-J0090 AHG6: Transform Dependent Deblocking [G. Van der Auwera, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-J0418 AHG6: Cross-check of JCTVC-J0090 Transform Dependent Deblocking Strength [Shuo Lu (Sony)] [late] [miss]

JCTVC-J0091 AHG6: Chroma QP Offset and Deblocking [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-J0372 Cross-check of JCTVC-J0091 on Chroma QP offset and deblocking filter [J. Xu (Sony)] [late]

JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-J0403 AHG6: Cross-check of JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries [S. Lu, O. Nakagami (Sony)] [late]

JCTVC-J0445 Cross check of TI’s suppression of blocking artifacts at 32x32 transform boundaries [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0181 AHG6: On deblocking filter parameters [S. Lu, O. Nakagami, M. Ikeda, T. Suzuki (Sony)]

JCTVC-J0409 Crosscheck of JCTVC-J0181: AHG6: On deblocking filter parameters [D.-K. Kwon (TI)] [late]

JCTVC-J0186 On Deblocking Filter and DC Component of Quantization Matrices [K Sato (Sony)]

JCTVC-J0419 Crosscheck report of On Deblocking Filter and DC Component of Quantization Matrices (JCTVC-J0186) [M. Shima (Canon)] [late] [miss]

JCTVC-J0211 On the reference picture comparison for boundary strength [J. Kim, Hendry, B. Jeon (LG)]

JCTVC-J0398 AHG6: Crosscheck of the reference picture comparison for boundary strength in JCTVC-J0211 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0286 AHG6: Adaptive deblocking filtering [A. Norkin (Ericsson)] [late]

JCTVC-J0494 Crosscheck of JCTVC-J0286: AHG6: Reduction of block artifacts in HEVC for large blocks [D.-K. Kwon, M. Budagavi (TI)] [late] [miss]

JCTVC-J0343 Use of Chroma QP offsets in Deblocking [S. Kanumuri, G. J. Sullivan (Microsoft)]

## Non-deblocking loop filters

### Adaptive loop filter

JCTVC-J0036 ALF coefficient coding with a single k-table [K. Ugur (Nokia)]

JCTVC-J0048 AHG6: ALF with non-normative encoder-only improvements [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-J0253 AHG6: Cross-check for non-normative ALF improvements (JCTVC-J0048) [S. Esenlik, M. Narroschke (Panasonic)]

JCTVC-J0390 AHG6: Further cleanups and simplifications for the ALF in JCTVC-J0048 [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0440 Crosscheck of JCTVC-J0390: AHG6: Further cleanups and simplifications of the ALF in JCTVC-J0048 [D.-K. Kwon, M. Budagavi (TI)] [late]

JCTVC-J0049 AHG6: Comparison between ALF and bi-prediction MC [C.-Y. Chen, C.-Y. Tsai, Y.-C. Chang, C.-Y. Cheng, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0050 AHG6: ALF with modified padding process [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0325 Crosscheck of J0050: AHG6: ALF with modified padding process [M. Budagavi (TI)] [late]

JCTVC-J0144 AHG6: Hue/saturation-based chroma ALF design and LCU-based on/off control by encoder [T. Yamakage, T. Itoh, Takeshi Chujoh (Toshiba), C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-J0266 AHG6: Modification to loop filtering across slice boundaries [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

JCTVC-J0448 AHG6: Cross check of Modification to loop filtering across slice boundaries (JCTVC-J0266) [T. Ikai (Sharp)] [late] [miss]

JCTVC-J0288 On Loop Filter Disabling [G. Van der Auwera, R. Joshi, Y.-K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-J0320 Multicore-friendly ALF luma region coding [M. Tikekar, M. Budagavi, V. Sze (TI)]

JCTVC-J0337 Fix for ALF Padding Process [P. Chen, W. Wan (Broadcom)]

JCTVC-J0427 Crosscheck of JCTVC-J0337: Fix for ALF Padding Process [M. Budagavi (TI)] [late]

JCTVC-J0399 AHG6: Crosscheck of multicore-friendly ALF luma region coding in JCTVC-J0320 [C.-Y. Tsai, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0346 Unifying ALF coefficient coding with coeff_abs_level_remaining coding [J. Lou, Y. Yu, L. Wang (Motorola Mobility)]

JCTVC-J0493 Cross-check of ALF Coefficient Coding in JCTVC-J0346 [W.-S. Kim (TI)] [late]

Sample adaptive offset

JCTVC-J0031 Unification of band and edge offsets with respect to sign for SAO [K. Andersson, P. Wennersten, R. Sjöberg (Ericsson)]

JCTVC-J0041 AHG5/AHG6: On reducing context models for SAO merge syntax [E. Alshina, A. Alshin, J.H. Park, (Samsung), C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0323 AHG5/AHG6: Cross check report of reducing context models for SAO merge syntax (JCTVC-J0041) [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0043 AhG5: On bypass coding for SAO syntax elements [E. Alshina, A. Alshin, J.H. Park (Samsung)]

JCTVC-J0044 Encoder modification for SAO [E. Alshina, A. Alshin, J.H.Park, (Samsung)]

JCTVC-J0045 AhG6: On SAO type sharing between U and V components [E. Alshina, A. Alshin, J.H. Park, (Samsung), G. Laroche, C. Gisquet, P. Onno, (Canon)]

JCTVC-J0467 Verification of J0045 on SAO chroma processing [F. Bossen (Docomo Innovations)] [late]

JCTVC-J0046 AHG6: On left band position coding in SAO [E. Alshina, A. Alshin, J.H. Park, (Samsung)]

JCTVC-J0054 AhG6: Bypass bins grouping in SAO [J. Sole, I.S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-J0392 AHG6: Crosscheck of SAO bypass bins grouping in SAO in JCTVC-J0054 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0062 SAO CABAC Context Initialisation to Reduce Parallel Encoding Losses [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0455 Crosscheck of SAO CABAC Context Initialisation to Reduce Parallel Encoding Losses (JCTVC-J0062) [M. Coban (Qualcomm)] [late] [miss]

JCTVC-J0401 Crosscheck of SAO CABAC Context Initialisation in JCTVC-J0062 [Hendry, B. Jeon (LG)] [late] [miss]

JCTVC-J0065 AhG5/AhG6: On SAO type index coding [Y. H. Tan, C. Yeo (I2R)]

JCTVC-J0349 AHG5/AHG6: Cross-verification of JCTVC-J0065 on SAO type index coding [K. Chono, K. Tokumitsu (NEC)] [late]

JCTVC-J0087 AHG6: Independent luma and chroma SAO on/off control at slice level [M. Zhou (TI)]

JCTVC-J0051 AHG6: Crosscheck of independent luma and chroma SAO on/off control at slice level in JCTVC-J0087 [C.-M. Fu, Y.-W. Huang (MediaTek)]

JCTVC-J0097 Evaluation of picture-based SAO optimization in HM-7.0 [D.-K. Kwon, W.-S. Kim (TI)]

JCTVC-J0103 AHG6: Decoupling SAO LCU on/off from SAO type  [I. S. Chong, M. Karczewicz (Qualcomm), C.-W. Hsu, C.-M. Fu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0450 AhG6: Cross-check for decoupling SAO LCU on/off from SAO type (JCTVC-J0103) [E. Alshina (Samsung)] [late] [miss]

JCTVC-J0104 AHG6/AHG5: Fix and simplification for SAO type index [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm), C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0449 AhG5/6: Cross-check for SAO type coding using truncated unary and by-pass CABAC mode (JCTVC-J0104) [E.Alshina (Samsung)] [late] [miss]

JCTVC-J0106 AHG6/AHG5: SAO offset coding [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0387 AHG6/AHG5: Cross-check of JCTVC-J0106 (SAO offset coding) [T. Yamakage, T. Itoh (TOSHIBA)] [late]

JCTVC-J0132 AHG6: SAO One Unit Parameter Signalling [Y. Chiu, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai (Intel)]

JCTVC-J0139 AhG6: SAO Parameter Estimation Using Non-deblocked Pixels [W.-S. Kim (TI)]

JCTVC-J0391 AHG6: Crosscheck of SAO parameter estimation using non-deblocked pixels in JCTVC-J0139 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0140 AhG6: SAO Complexity Reduction with SAO LCU Flag Coding [W.-S. Kim, D.-K. Kwon (TI)]

JCTVC-J0324 AhG6: Cross check report of SAO Complexity Reduction with SAO LCU Flag Coding (JCTVC-J0140) [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0141 AhG6: SAO Offset Bypass Coding [W.-S. Kim, M. Budagavi, V. Sze (TI)]

JCTVC-J0148 AHG5: Bypass coding for SAO syntax elements [T. Matsunobu, K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-J0350 AHG5/AHG6: Cross-verification of JCTVC-J0148 on bypass coding for SAO syntax elements [K. Chono, K. Tokumitsu (NEC)] [late]

JCTVC-J0172 Cross-check of J0031 on SAO signs from Ericsson [P. Onno, G. Laroche, T. Poirier (Canon)] [late]

JCTVC-J0173 Cross-check of J0044 on SAO encoder modification from Samsung [G. Laroche, T. Poirier, P. Onno (Canon)] [late]

JCTVC-J0178 AHG5: On SAO syntax elements coding [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-J0406 Cross-check of SAO syntax elements coding (JCTVC-J0178) [J. Sole (Qualcomm)] [late]

JCTVC-J0179 On sao_merge_left_flag for effective Mx1 CTB coding [T. Ikai, T. Yamamoto, Y. Tasugi (Sharp)]

JCTVC-J0371 Cross-check of JCTVC-J0179 on sao_merge_left_flag for effective Mx1 CTB coding [J. Xu (Sony)] [late]

JCTVC-J0192 AHG6: Crosscheck of evaluation of picture-based SAO optimization in HM-7.0 in JCTVC-J0097 [C.-M. Fu, Y.-W. Huang (MediaTek)]

JCTVC-J0193 AHG5: Crosscheck of bypass coding for SAO syntax elements in JCTVC-J0043 [C.-M. Fu, Y.-W. Huang (MediaTek)]

JCTVC-J0207 Improved Type Coding for SAO [Sangoh Jeong, Seungwook Park, Byeongmoon Jeon (LGE)]

This contribution was initially accidentally uploaded as an "associated resource" instead of as the main contribution. This was corrected on 07-04.

JCTVC-J0394 Crosscheck of J0207: Improved type coding for SAO [D.-K. Kwon (TI)] [late]

JCTVC-J0213 AHG6: A threshold for SAO edge offset [T. Sugio, T. Matsunobu, T. Nishi (Panasonic)]

JCTVC-J0365 Cross-check of Threshold for SAO Edge Offset in JCTVC-J0213 [W.-S. Kim (TI)] [late]

JCTVC-J0268 AHG6: On SAO signalling [J. Xu, A. Tabatabai (Sony)]

JCTVC-J0374 AHG6: Cross-check of JCTVC-J0268: on SAO signalling [Koohyar Minoo (Motorola)] [late] [miss]

JCTVC-J0269 SAO on/off flag coding [J. Xu, A. Tabatabai (Sony)]

JCTVC-J0296 Cross-check of SAO on/off flags coding in JCTVC-J0269 [W.-S. Kim, D.-K. Kwon (TI)] [late]

JCTVC-J0316 New initialization value for SAO merge signals [I. S. Chong, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-J0317 Cross-Check of proposal J0141 of TI (AhG6: SAO Offset Bypass Coding) [C. Kim (Samsung)] [late]

JCTVC-J0347 AHG6/AHG5: Simplified SAO coding [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0422 Cross-check of AHG6/AHG5: Simplified SAO coding (J0347) by Qualcomm and Mediatek [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-J0355 AHG5, AHG6: Coding of SAO merge left and merge up flags [Koohyar Minoo, David Baylon (Motorola Mobility)]

JCTVC-J0379 Cross-check of JCTC-J0355 on coding of SAO merge left and merge up flags [C. Auyeung (Sony)] [late]

### Other

JCTVC-J0165 LCU-based framework with zero pixel line buffers for non-local means filter [M. Matsumura, S. Takamura, A. Shimizu (NTT)]

JCTVC-J0190 Cross-check of LCU-based framework for non-local means filter (JCTVC-J0165) [T. Yoshino, K. Kawamura, S. Naito (KDDI)]

## Block structures and partitioning

### General

JCTVC-J0133 TU Depth Clean-up [Y. Chiu, P. Kapsenberg, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai (Intel)]

It has been observed that there are two TU depth related issues in current HEVC text and software. Firstly, there’s no limitation to the values of maximum TU depths in SPS. Secondly, the derivation scheme of context index for split_transform_flag may result in unwanted context, which was reported in ticket #533. To clarify the usage of TU depth and ensure its correctness, this contribution proposes clean-up solutions for these issues.

First issue: Revisit together with J0335 (clarifications)

Second issue (ticket #533): There is an issue with CNU. In principle, the ticket could be solved as editorial issue (avoiding use of undefined context by replacing CNU through CNT), but what is suggested here is to combine the solution of the ticket with a reduction by one context.

Decision: Adopt the suggested solution on the second issue.

JCTVC-J0359 Cross-check for JCTVC-J0133 TU Depth Clean-up [X. Zhang, O.C. Au (HKUST)] [late]

JCTVC-J0360 Cross-check of TU Depth Clean-up (JCTVC-J0133) [J. Xu (Microsoft)] [late]

### NSQT

JCTVC-J0138 NSQT Simplification [X. Zheng, Y. Yuan, H. Yu, Y. He (??)]

This document provides a simplification of NSQT. Both software implementation and text modification are provided in this contribution. The proposed solution simplifies the non-square transform quadtree split process, and merges the luma non-square transform quadtree and chroma non-square transform quadtree. As a result, NSQT has become a lot easier to describe and implement. Experimental results show that the proposed solution does not have negative impact on coding performance. Both software encoding and decoding time are slightly reduced compared to HM7.0 anchor.

It is suggested to inhibit splitting of non-square transform blocks into square transform blocks at the last splitting stage.

Compared to the old method, class F shows small loss (0.1+%), otherwise approx. equal.

Gain of the new NSQT version versus RA main is about 0.4%, versus LDB main about 1% (not much different for HE 10).

Comments:

- Text still has some problems (and would need careful checking)

- From view point hardware implementation, it is a little bit better but does not solve the biggest problem (irregularity by branching from the square-shaped quadtree, computation of memory addresses etc.).

Still too complex to be considered in the main profile.

In general, this new version of NSQT is seen as a step in the right direction. Some discussion about whether the current NSQT should be removed entirely or be replaced by the new scheme. After this it was suggested to leave everything as it is in the DIS (considering that more urgency is on other issues)

Further study.

JCTVC-J0370 Cross-verification of JCTVC-J0138 on NSQT simplification [M. Zhou (TI)] [late]

JCTVC-J0415 Cross-verification of JCTVC-J0138 NSQT simplification [L. Guo (Qualcomm)] [late]

JCTVC-J0364 Implicit transform block split process for asymmetric partitions [X. Zheng (HiSilicon)]

This contribution provides an implicit TU split solution for asymmetric partitions when “QuadtreeTUMaxDepthInter” is set to 1. Experimental results show that the proposed solutions can contribute average coding gain from 0.3% to 1.3% at Main profile configurations. Both encoder and decoder complexity are same as HM7.0.

Two solutions are suggested:

- non-square transforms for asymmetric partitions (gain compared to RA main is about 0.6%, versus LDB main about 1.2%)

- additional implicit square transform split for asymmetric partition

Solution 1 appears more complex than the simplified NSQT of J0138

Solution 2 could be practical without too much implementation burden, but results are not available and it is unclear how many changes would be necessary to the text.

Revisit on solution 2 (if time allows and if reasonable gains can be shown)

JCTVC-J0473 Cross-check of J0364 on Implicit transform block split process for asymmetric partitions [E.François (Canon)] [late]

## Motion and mode coding

JCTVC-J0098 AHG5: Bypass bins for reference index coding [V. Seregin, J. Sole, X. Wang, M. Karczewicz (Qualcomm), V. Sze, M. Budagavi (TI)]

JCTVC-J0382 Cross-check report for bypass bins for reference index coding (JCTVC-J0098) [H. Sasai, K. Terada (Panasonic)] [late]

JCTVC-J0101 Splitting contexts for MVD coding [V. Seregin, J. Chen, X. Wang, M. Karczewicz (Qualcomm)]

JCTVC-J0167 Cross-check report of JCTVC-J0101 on Splitting contexts for MVD coding [T.Chujoh (Toshiba)] [late]

JCTVC-J0145 Simplification on spatial AMVP candidate derivation [Y. Lin, J. Zheng (HiSilicon)]

JCTVC-J0155 On MV prediction [J. Dong, Y. Ye (InterDigital)]

JCTVC-J0400 Crosscheck of simplification of the scaling process for MV prediction in JCTVC-J0155 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-J0170 On Temporal and Combined Merge motion vector predictors derivation for Merge/Skip mode [G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-J0410 Cross-check of J0170 on temporal and combined motion vector predictors derivation for merge/skip mode [T. Lee (Samsung)] [late]

JCTVC-J0176 AHG5: Reference index coding [C. Rosewarne, M. Maeda (Canon)]

JCTVC-J0489 Cross-check report for reference index coding (JCTVC-J0176) [S.H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-J0177 AHG5: Merge index coding [C. Rosewarne, M. Maeda (Canon)]

JCTVC-J0433 AHG5: Cross check of Canon’s Merge index coding (JCTVC-J0177) by Qualcomm [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0180 Zero merge candidate simplification [B. Li, H. Li (USTC), H. Yang (Huawei)]

JCTVC-J0188 Cross-check of J0180 on uni-directional zero merge candidate [Y. H. Tan, C. Yeo (I2R)]

JCTVC-J0203 implification on zero merge candidate derivation [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

JCTVC-J0443 Cross-check report of simplification on zero merge candidate derivation (JCTVC-J0203) [T. Sugio (Panasonic)] [late]

JCTVC-J0204 Dependency removal of temporal merge candidate and combined bi-predictive merge candidate derivation [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

JCTVC-J0381 Crosscheck of JCTVC-J0204: Dependency removal of temporal merge candidate and combined bi-predictive merge candidate derivation [K Sato (Sony)] [late]

JCTVC-J0218 Bi-pred merge restriction for small PUs [T. Lee, J. Park (Samsung)]

JCTVC-J0366 Cross-check of J0218 on Bi-pred merge restriction for small PUs [P. Onno (Canon)] [late]

JCTVC-J0219 On signalling the syntax of MVP flag [X. Zhang, Y. Shi, O.C. Au, F. Zou, C. Pang (HKUST)]

JCTVC-J0348 Cross-check for JCTVC-J0219 on signalling the syntax of MVP flag [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-J0225 Restrictions to the maximum motion vector range [Alistair Goudie (Imagination Technologies)]

JCTVC-J0240 Consistent coding of motion information for B slices with identical reference picture lists [J. Xu (Microsoft)]

JCTVC-J0397 Crosscheck of consistent coding of motion information for B slices with identical reference picture lists in JCTVC-J0240 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0278 AMP mode support for minimum CUs of size greater than 8x8 [M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

JCTVC-J0413 Crosscheck of AMP mode support for minimum CUs of size greater than 8x8 in JCTVC-J0278 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-J0297 AHG5:ref_idx coding [S.H. Kim, L. Kerofsky, A. Segall (Sharp)]

JCTVC-J0511 Cross-check of high throughput binarization for reference index coding (JCTVC-J0297) [J. Chen (Qualcomm)] [late]

JCTVC-J0312 Redundancy removal on InterDir syntax [C. Kim, T. Lee, E. Alshina (Samsung)]

JCTVC-J0315 AHG5: Context reduction for MVD coding [C. Kim, J. Kim, J.H. Park (Samsung)]

JCTVC-J0385 Cross-check of Context Reduction for MVD Coding in JCTVC-J0315 [W.-S. Kim (TI)] [late]

JCTVC-J0302 Restricted usage of motion vectors for long-term reference picture in motion vector prediction process [I.-K. Kim, Y. Park, J. H. Park (Samsung)]

JCTVC-J0341 Cross check of JCTVC-J0302 on Restricted usage of motion vectors for long-term reference picture in motion vector prediction process [Y. Takahashi, O. Nakagami, T. Suzuki (Sony)] [late]

### Hooks in scalability: Motion related

JCTVC-J0121 AHG10: Motion related hooks for HEVC multiview/3DV extension based on long-term reference pictures [Y. Chen, Y.-K. Wang, L. Zhang, V. Seregin (Qualcomm)]

JCTVC-J0510 AHG10: Mental cross check of JCTV-J0121 on motion related hooks for HEVC multiview/3DV extension [O. Bici, M. Hannuksela (Nokia)] [late]

JCTVC-J0122 AHG10: Hooks related to motion for the 3DV extension of HEVC [Y. Chen, Y.-K. Wang, L. Zhang (Qualcomm)]

JCTVC-J0224 AHG10: Hook for scalable extensions: Signalling TMVP reference index in slice header [O. Bici, M. Hannuksela, K. Ugur (Nokia)]

JCTVC-J0505 AHG10: Mental cross-check of JCTVC-J0224 [Y.Chen(Qualcomm)] [late]

JCTVC-J0461 AHG10: Mental cross-check of JCTVC-J0224 [Y. Chen (Qualcomm)] [late] [miss]

## High-level syntax and slice structure (108)

### NAL unit header (9)

J0063, J0231, J0250, J0112, J0113, and J0174 were all suggested to be related. Only the first four of these were discussed in the preceding AHG9 meeting.

JCTVC-J0063 AHG9: Syntax for NAL Packet Priority [Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He (InterDigital)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

Pictures in the same temporal level in a hierarchical-B structure can have different influence on error propagation and on decoded video quality. Currently in HEVC draft 7, the NAL unit header does not indicate packet priority within the same temporal layer. This contribution proposes two syntax options to indicate such priority of a NAL unit.

R. Sjoberg expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was remarked that the prioritization may depend on the loss concealment method.

It was noted that the nal_ref_flag is basically always currently equal to 1 except at the highest temporal layer.

"Method 1" would change nal_ref_flag to be a nal_priority_flag indicating relative priority within a temporal level.

"Method 2" would provide a priority_id in the AUD, which could carry more bits than the nal_ref_flag.

It was noted that there would be no normative purpose for the proposed priority_id – that it is just metadata – and could be an SEI message.

Some participants expressed some skepticism about the value of the proposed priority_id.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0231 On nal_ref_flag [T. K. Tan, Junya Takiue (NTT Docomo)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution seeks to clarify the purpose of the nal_ref_flag.  The nal_ref_flag does not seem to have any use in the decoding process apart from the final marking of the picture as reference or non-reference picture.

This contribution proposes to remove the nal_ref_flag syntax element from the NAL unit header and create 3 new NAL unit types for coded slices that can be either used for reference or not.

J0463 is reportedly a (late) cross-check.

It was suggested that an alternative would be to use the maximum temporal later value as a non-reference picture indication, rather than using the NUT or nal_ref_flag.

This (or something like it) seemed promising.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0250 Indication of non-reference pictures [R. Sjöberg, J. Samuelsson (Ericsson)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution proposes to change the semantics of nal_ref_flag so that in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer. This contribution claims that this makes it possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed and thus indicate whether that picture can safely be removed from the sub-stream without affecting the decoding of the remaining pictures in the sub-bitstream.

It was commented that this would make it more difficult for a "middle box" to identify pictures that can be dropped. Unless the highest temporal ID in the bitstream is known, the non-reference pictures cannot be dropped.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0112 AHG9: Various comments on HEVC draft 7 [Y.-K. Wang (Qualcomm)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan.)

This document proposes the following:

- Removal of nal_ref_flag, and push the saved bit to reserved_one_5bits to make it to reserved_one_6bits; this proposal is suggested to be ignored if the proposal in JCTVC-J0113 is adopted

- Change of temporal_id to temporal_id_plus1 (The DPB size in level definitions only support hierarchical coding structures with GOP size up to 16 (see document JCTVC-J0111 for analyses of DPB size requirements for different GOP sizes), i.e., typically up to 5 temporal layers are supported.) However, it was commented that there may be a desire in the future to use deeper temporal nesting.

- Change of the value 0 of nal_unit_type from "Unspecified" to "Reserved".

- An alleged editorial fix to the semantics of the extension syntax elements in VPS, SPS, PPS, APS and slice data. This was not agreed. However, it was agreed that some further clarification of the intended tolerance of decoders for reserved values would be desirable.

- Extending of the extension mechanism to all types of NAL units. The participants did not support this part of the proposal.

- To support SEI NAL units that may follow the first VCL NAL unit in the same access unit

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0469 AHG9: Mental cross-check of JCTVC-J0112 [T. C. Thang (UoA), Hendry (LG)] [late]

JCTVC-J0113 AHG10: High-level syntax hook for HEVC multi-standard extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

This is a follow-up proposal of JCTVC-I0355. A prior document G149 was also suggested as relevant background information.

In JCTVC-I0355, a high-level syntax hook for HEVC multi-standard scalable or 3DV extensions wherein the base layer or view is AVC compatible is proposed. It proposed to use the following principles for the syntax design:

- For the NAL unit header length in the multi-standard scalable or 3DV extensions to be the same as in the existing HEVC design;

- That there should be sufficient NAL unit types to be used by HEVC and its potential future extensions, ideally the same as in the existing HEVC design;

- For the AVC NAL units to be distinguishable from the NAL unit header itself.

A comment was given in response to JCTVC-I0355 that an AVC decoder would not be able to distinguish between an AVC NAL unit and an HEVC NAL unit.

In this proposal, an improved design is proposed to solve the above issue while the above three design principles are still followed.

The proposed change to the NAL unit header is to remove nal_ref_flag and re-arrange the syntax elements in the HEVC NAL unit header.

The proposal uses the NUT range from 16 to 31 from the AVC perspective.

A participant questioned the need to multiplex AVC within HEVC within the bitstream level – suggesting to depend on the system level to provide that capability.

Cross-checked in J0492.

Revisit.

JCTVC-J0492 AHG10: Mental cross-check of JCTVC-J0113 [M. M. Hannuksela (Nokia)] [late]

JCTVC-J0174 AHG 9 / AHG 10: On NAL unit header [T. Thang (UoA), J. Kang, H. Lee, J. Lee (ETRI), Hendry, B. Jeon (LG)]

This contribution discusses two items. First, it is suggested that the functionality of nal_ref_flag might be redundant as described in JCTVC-I0251 and JCTVC-I0355 so that the removal of the flag should be considered. To cover the flag functionality to differentiate reference and non-reference pictures, it is suggested to add a constraint to the semantics of temporal_id such that temporal_id of NAL units that contain slices of a non-reference picture must not be equal to 0.

It was commented that we could have a non-reference slice NUT.

Revisit that.

Second, it is asserted that if future extensions of HEVC also use the current fixed 2 bytes NAL unit header size, there is only 5 bits, which is the reserved_one_5bits, available to be used to describe layer identification. The contributor suggested that this might be too small when considering that the extension might cover not only scalability extension but also multiview extension. Furthermore, it might not be necessary to treat temporal identification from identification of other scalability / view types in the extensions of HEVC. Therefore, it is proposed to combine reserved_one_5bits and temporal_id and change its name to layer_id.

Revisit.

JCTVC-J0464 AHG9: Mental cross-check of JCTVC-J0174 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0463 AHG9: Mental cross-check of JCTVC-J0231 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0432 On NAL Unit Header and Video Parameter Set Design [B. Choi, J. Kim, J. Park (Samsung)] [late]

The NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable coding design with non-HEVC video coding standards. This proposal is originally prepared for JCT2 (HEVC 3D extension). However, it is also proposed in JCT because it contains some syntax change in the HEVC draft.

This proposal did not actually propose a change of the NUH of the base spec.

The VPS aspect of the contribution proposes to have an indicator of the type of standard associated with each layer. It was commented that this is a more elaborate scheme than what is expressed in our requirements and that it is something that could be done later if appropriate. Further study of this is encouraged.

JCTVC-J0239 AHG10 - Selective inter-layer prediction signaling for scalable extension [J. Xu (Microsoft)]

Proposes a flag to be included in the NAL unit header to indicate whether the current picture (layer representation) is used by layer representations from higher layers, similar as the discard_flag in the AVC based SVC, and change the 5-bit reserved_one_5bits to be 4-bit reserved_one_4bits.

Karsten: If a base layer is encoded first, and then add enhancement layers later.

Stephan: Instead of using one flag, we could use NAL unit types, equivalent to use of a fraction of bit. We should not reduce the 5 bits for layer_id for future extensions.

Further study was encouraged.

[YK to add more detailed notes.]

JCTVC-J0428 AHG10: Mental cross-check of Selective inter-layer prediction signalling (JCTVC-J0239) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

### Random access and adaptation (12)

#### Random access point (RAP) pictures (7)

JCTVC-J0107 AHG9: On RAP pictures [Y.-K. Wang, Y. Chen, R. J. Joshi, A. K. Ramasubramonian (Qualcomm)]

This document includes the following proposals related to RAP pictures (i.e., IDR, CRA and BLA pictures):

Topic 1: To include the support for handling a CRA picture as a BLA picture based on an indication through external means. Decision: Adopted.

Topic 2: To enable prediction from decodable leading pictures (non-TFD leading pictures) associated with a RAP picture by normal pictures associated with the same RAP picture, and by leading pictures associated with the next RAP picture (wherein leading pictures associated with a RAP picture are those pictures following the RAP picture in decoding order but preceding the RAP picture in output order, and normal pictures associated with a RAP picture are those pictures following a RAP picture in both decoding order and output order and preceding, in decoding order, the next RAP picture).

It was noted that contribution J0251 would eliminate non-TFD leading pictures.

Decision: The pictures that follow a RAP picture (including an IDR picture) in both decoding and output order cannot reference any leading picture.

Topic 3: To change the definition of RAP picture. No action.

Topic 4: To mandate the activation of VPS, SPS, PPS and APS at each BLA picture. No action needed.

Topic 5: To include a constraint to disallow output-order interleaving of non-TFD leading pictures with TFD pictures or pictures earlier than the same associated CRA or BLA picture in decoding order, and a constraint to disallow decoding-order interleaving of TFD pictures and following pictures associated with a RAP picture.

The spirit is that output order is as follows:

- pictures that precede the RAP in decoding order, then non-TFD leading pictures, then RAP, then following pictures

- TFD pictures must precede non-TFD leading pictures in output order

- but there is no relative output order constraint in regard to the order of TFD and pictures that precede the RAP in decoding order.

Decision: Agreed.

Regarding decoding order, all leading pictures associated with a RAP picture shall precede, in decoding order, all pictures that follow the RAP picture in output order. Decision: Agreed (consensus assessed by T. K. Tan).

Topic 6: To change  related to the inference of no_output_of_prior_pics_flag equal to 1.

It was remarked that a difference in language between Annex C and clause 7 is intentional, not an error, but it was agreed that some clarification might be beneficial if the current decoding conformance language is not sufficiently clear.

The second aspect proposed was to change "first IDR or BLA picture in the bitstream" to "first picture in the bitstream" in a few places relating to no_output_of_prior_pics_flag inference. Decision: Agreed.

Topic 7: To use one more NAL unit type to differentiate TFD & TLA pictures and non-TLA TFD pictures

No action taken on this aspect.

JCTVC-J0499 AHG9: A mental cross-check of JCTVC-J0107 (On RAP pictures) [M. M. Hannuksela (Nokia)] [late]

- JCTVC-J0345 Editorial modifications to HEVC text specification relating to reference picture sets and random access points [G. J. Sullivan, S. Kanumuri (Microsoft)]

Delegated to editors for consideration. (Any aspects that conflict with recorded decisions are not to be used.)

JCTVC-J0215 AHG 9: On NAL unit type [Hendry, B. Jeon (LG)]

JCTVC-J0482 Mental cross-check of JCTVC-J0215 On NAL unit type  [T. C. Thang (UoA)] [late]

JCTVC-J0229 AHG9: Comments and clarification on CRA, BLA and TFD pictures [TK Tan (NTT Docomo)]

JCTVC-J0462 AHG9: Mental cross-check of JCTVC-J0229 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0251 Restrictions on leading pictures of CRA and BLA [J. Samuelsson, R. Sjöberg (Ericsson)]

JCTVC-J0310 Revival of decodable backward predicted pictures that are output preceding a RAP picture [Arturo Rodriguez (Cisco), A. K Katti, H-Y Hwang]

JCTVC-J0344 Refinement of random access point support [S. Kanumuri, G. J. Sullivan (Microsoft)]

#### Splicing and editing (2)

JCTVC-J0108 AHG9: Splicing-friendly coding of some parameters [Y.-K. Wang, Y. Chen (Qualcomm)]

JCTVC-J0501 AHG9: Mental cross-check of JCTVC-J0108 (Splicing-friendly coding of some parameters) [M. M. Hannuksela (Nokia)] [late]

JCTVC-J0137 AHG9: New high-level syntax for simple HEVC stream editing [K Kazui, J Koyama, S Shimada, A Nakagawa ((Fujitsu)]

JCTVC-J0476 AHG9: Cross-check of JCTVC-J0137 [H. Aoki (NEC)] [late] [miss]

#### Temporal layer access (TLA) pictures (3)

JCTVC-J0156 AHG 10: Generalized definition of the TLA for scalable extension [C. K. Kim, Hendry, B. Jeon (LGE)]

JCTVC-J0246 On temporal layer access pictures [B. Choi, Y. Park, I. Kim, J. Kim, J. Park (Samsung)] [late]

JCTVC-J0305 AHG10: On Gradual Temporal Layer Access [S. Deshpande (Sharp)]

JCTVC-J0500 AHG10: Mental cross-check of JCTVC-J0305 (On Gradual Temporal Layer Access) [M. M. Hannuksela (Nokia)] [late]

### Slices and slice header parameters (16)

#### Picture order count (POC) (3)

JCTVC-J0084 AHG9: Restrict Picture Order Count to 40-bit [M. Zhou (TI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

At the 9th JCTVC meeting in Geneva, the dynamic range of picture order count (PicOrderCntVal) was increased from 32-bit to 64-bit. 64-bit PicOrderCntVal can support continuous 120 fps video recording for up to 4874.5 million years. However, on the decoder side there is certain complexity associated with carriage of 64-bit PicOrderCntVal. It is therefore recommended to restrict PicOrderCntVal to 40 bits, which can already support continuous 120 fps video recording for up to 290.5 years.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

At the previous meeting, we thought there was no real impact to the increase of the specified range.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0110 AHG9: On POC [R. L. Joshi, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan and J. Boyce.)

This document proposes the following:

- An editorial change to the definition of picture order count to discuss specific cases

- Removal of the function DifPicOrderCnt( ) or consistently using it in the specification, particularly for derivation processes for motion vectors and reference indices

- Removal of one of the POC-related constraints in Annex C.4

- An SEI message conveying additional POC information for intra pictures to enable derivation of output order for intra-picture-only trick mode playback

(Cross check not yet provided.)

Discussion of each item:

Topic 1. Editorial only. Something needs to be changed. Editors can consider this (and where this should go).

Topic 2. Notes that the POC difference computation function is not always used, and thus the constraint on POC difference is not applied everywhere that POC differences are computed. It may be desirable to impose the constraint when differences are computed for motion vector derivation, but perhaps not for RPS difference values (e.g. for an LTRP in the RPS). For further study.

Topic 3. About prevRefPic, it was commented that there seems to be an editing error, as our intent was to include prevRefPic in the set of pictures for which max and min POC counts are computed and the difference range constraint is imposed, but it was not put into the text that way. J. Boyce assessed the consensus that this was agreed and should be corrected.

The contribution suggested to include only prevRefPic and the current picture, and not to also include pictures in the DPB that are marked as used for short-term reference or waiting for output. J. Boyce assessed that there was no consensus to make this change.

A general comment was made that bitstream constraints are useful for early detection of bitstream errors.

Topic 4. Proposed new SEI message for intra pictures.

It was commented that the encoder could just send more POC LSBs to ensure coherence. It was responded that the presence of this SEI would provide an assurance that the issue was taken care of.

It was asked whether there could be a splicing problem with this – e.g., when discarding some GOPs so that a new bitstream starts at a CRA. It was suggested that sending the POC MSB difference rather than some POC MSB's value might be better.

It was asked how to detect an "intra picture".

It was commented that all RAP pictures are already in decoding order in the bitstream, so this may not be needed.

It was commented that non-RAP intra pictures would not be expected to ordinarily be used.

The AHG recommended this to be for further study with no action to be taken at this meeting.

JCTVC-J0248 On prevRefPic definition [J. Samuelsson, R. Sjöberg (Ericsson)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution proposes to revert the definition of prevRefPic to always be a picture in temporal layer 0, for the sake of simplicity and to correct a problem with TLA pictures having POC values that depend on the presence of prior inappropriate pictures.

Instead of using the previous reference picture that has a temporal_id equal to or less than the temporal_id of the current picture for prevRefPic, this document proposes to use the previous reference picture that has a temporal_id equal to 0.

(Cross check not yet provided.)

It was agreed that the TLA problem exists and needs to be corrected.

The AHG recommended adoption.

#### Slices (5)

JCTVC-J0083 AHG9: On slice header parsing overhead reduction [M. Zhou, M. Mody (TI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In the current HM7.0 design there is reportedly a large difference (over 100x) in slice header parsing overhead between the typical and worst case. It was asserted that the worst case slice header parsing can be beyond capability of real-time decoding in which slice header parsing is implemented in software. Weighted prediction tables and reference picture parameter sets (RPS) are parsing intensive parts of slice header. To reduce the worst case slice header parsing overhead, this contribution proposes the following two changes: 1) move RPS from slice header to APS, 2) reduce weighted prediction table cycle overhead by either constraining the size of the reference picture lists (option 1), or enabling signalling of default weighted prediction tables in APS and limiting the number of slice headers per picture which can override weighted prediction tables signalled in APS (option 2). The proposed methods can reportedly reduce the worst case slice parsing overhead by roughly 2x to 3x based on cycle estimate on ARM9.

It was remarked that slice header sharing (e.g. J0109) is related.

(Cross check not yet provided.)

It was commented that since the RPS is the same in all SHs, the decoder can just skip over those bits after parsing them from the first SH.

The weighted prediction syntax aspect needs additional discussion (e.g. "option 1").

JCTVC-J0109 AHG9: Header parameter set (HPS) [Y. Chen, Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan and J. Boyce.)

At the previous JCT-VC meeting, the proposal on slice header prediction in JCTVC-I0070 was discussed. It was noted in the meeting notes that using some kind of parameter set to enable slice header prediction seemed promising. Following such a direction, this document proposes a slice header prediction mechanism based on a so-called header parameter set (HPS), with two slightly different alternative approaches. In the first approach (single-AU HPS), an HPS would be required to be used only within one access unit. In the second approach (multi-AU HPS), an HPS may be used by multiple access units.

It is asserted that the proposed mechanism avoids the drawbacks in the JCTVC-I0070 design, the single-AU HPS approach is asserted to provide similar coding gains as reported in JCTVC-I0070, and the multi-AU HPS approach is asserted to provide opportunities for coding gains with increased complexity.

S. Wenger expressed his plan to submit a cross-check.

This was designed in a similar manner in concept to the previously-proposed APS partial update – multiple HPS IDs are proposed to be carried – one for each of several categories of syntax elements such as RPS selection, prediction weight selection, etc. A flag is proposed for single-slice encoding without using the scheme.

It was commented that the carrying of the multiple IDs seemed a bit complicated.

It was commented that the APS could perhaps be used for this.

The motivation is to provide coding efficiency improvement for multi-slice sharing of header data. There was discussion of loss resilience, but this did not seem to be a significant motivation.

With 6x6 CTB tiles and one slice per tile, the coding efficiency benefit was reportedly approximately 1.5% when reported in a prior contribution (I0070).

Aside from coding efficiency improvement, extensibility for SVC & 3D usage was suggested as a motivation.

It was commented that getting a better understanding of the coding efficiency impact would be needed.

Additional discussion needed in JCT-VC meeting.

JCTVC-J0216 AHG 9: Signalling slice index to detect lost slice earlier [Hendry, B. Jeon (LG)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution discusses the possibility of detecting lost slices earlier when multiple slices are used per picture. The following changes are proposed:

- Replacing the current syntax element first_slice_in_pic_flag (coded with u(1)) with slice_idx_in_pic (coded with ue(v)).

- Adding a flag last_slice_in_pic_flag (coded with u(1)).

Coding efficiency results without loss reportedly show that the proposed modification affects BD-rate on average 0.1%Y, 0.1%U, 0.1%V for all intra and random access cases (AI-HE10, AI-Main, RAHE10, RA-Main), and 0%Y, 0%U, 0%V for low delay B cases.

It was asked how it would be useful to know, e.g., that the 2nd and 3rd slices were lost (without knowing which CTBs were in those slices until parsing the 1st slice).

It was commented that if a back-channel was available, the slice ID number might be useful to send.

It was commented that the system layer would ordinarily provide a packet counter functionality.

No action on this recommended by the AHG.

(Cross check not yet provided.)

JCTVC-J0416 AHG 9: Cross-check of J0216 - Signalling slice index to detect lost slice earlier [A. K. Ramasubramonian (Qualcomm)] [late] [miss]

JCTVC-J0217 On dependent slices [T. Lee, J. Park (Samsung)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In the previous meeting, dependent slice support was adopted. According to the specification, a dependent slice cannot be used with entropy slice while it can be used with either WPP or tile within a set of pictures. Since entropy slice is regarded as similar tools to WPP or tile to enable parallel processing, it is proposed to enable dependent slice with entropy slice within a set of pictures for consistency. Or alternatively, it is proposed to restrict the usage of dependent slices to be used only if WPP is used.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was commented that entropy slices and dependent slices are somewhat overlapping in functionality, which is why their use is mutually exclusive.

Sub-picture ultra-low-delay was mentioned as a potential motivation for dependent slices (e.g. in combination with tiles or in non-WPP usage). Document J0264 was mentioned as providing further information.

As a syntax cleanup only, conditional coding of dependent_slice_enabled_flag was proposed in the PPS did seem to make sense, and was recommended for adoption by the AHG. No other action on this was recommended by the AHG.

JCTVC-J0255 AHG9: Slice Prefix for sub-picture and slice level HLS signalling [T. Schierl, V. George, R. Skupin, D. Marpe (HHI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In this document it was proposed to apply a functionality like SEI messages as well as additional high level syntax items, beyond the ones included in the NAL unit header, on a per slice level basis. Such messages were proposed as a "slice prefix NAL unit". Syntax and semantics of the slice prefix and slice-level/sub-picture SEI messages, use cases for low delay/sub-picture CPB operations, tile signalling and region of interest (ROI) signalling were described.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

Several potential sub-picture-level SEI messages were described.

It was commented that during the design of AVC the possibility was discussed to have slice-specific SEI messages or otherwise not require SEI messages to be at the beginning of the picture, but this was not done (for simplicity) because there was no clear need for it at the time.

It was commented that even today we still have reserved NUTs in AVC that could be used for such a purpose.

It was commented that we could just allow SEI NUT to follow the first VCL NALU of the picture, rather than using a different NUT for the slice level SEI than for the picture-level SEI.

Some participants commented that ultra-low-delay decode could benefit from definition of a timing SEI specification.

A participant suggested to also enable some SEI type of NALU that could follow after the relevant VCL NALU or the AU rather than precede it – e.g., for checksum data.

The general concept was supported by most participants in the AHG, but further discussion by the JCT-VC seemed needed.

#### Reference picture list (RPL) and weighted prediction (WP) parameters (8)

JCTVC-J0055 AHG9: Weighted Prediction Parameter Signalling [Yong He, Yan Ye (InterDigital)]

JCTVC-J0223 Cross-check of Weighted Prediction Parameter Signalling (JCTVC-J0055) [A. Tanizawa, T. Chujoh (Toshiba)] [late]

JCTVC-J0120 AHG9 & AHG13: Identical reference picture lists [Y. Chen, M. Coban, Y.-K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-J0504 AHG9 & AHG13: Cross-check of JCTVC-J0120: Identical reference picture lists [Philippe Bordes, Pierre Andrivon (Technicolor)] [late]

JCTVC-J0182 AHG 9 / AHG 13: On identical reference picture lists [Hendry, Y. Jeon, S. Park, B. Jeon (LG)]

JCTVC-J0221 AHG9: Clean-up of semantics and decoding process on weighted prediction [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-J0252 AHG9: Cross-check of JCTVC-J0221: Clean-up of semantics and decoding process on weighted prediction [P.Bordes, P.Andrivon (Technicolor)]

JCTVC-J0222 AHG9: Improved weighted prediction parameter signaling [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)]

JCTVC-J0295 Cross-check of improved weighted prediction parameter signalling (JCTVC-J0222) [Yong He (InterDigital)] [late]

JCTVC-J0503 AHG9: Simplification of weighted prediction signaling in PPS [Philippe Bordes, Pierre Andrivon (Technicolor)] [late]

### Reference picture set and reference picture list construction (9)

#### Reference picture set (8)

JCTVC-J0117 AHG13: On inter-RPS prediction [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0185 AHG13: On short_term_ref_pic_set [S Lu, K Sato (Sony)]

JCTVC-J0234 Inter-RPS complexity reduction [J. Ström, R. Sjöberg, J. Samuelsson (Ericsson)]

JCTVC-J0115 AHG13: Signalling of long-term reference pictures in the slice header [A. K. Ramasubramonian, Y.-K. Wang, R. L. Joshi, Y. Chen (Qualcomm)]

JCTVC-J0402 Crosscheck of Signalling of long-term reference pictures in the slice header in JCTVC-J0115 [Hendry, B. Jeon (LG)] [late] [miss]

JCTVC-J0116 AHG13: Signalling of long-term reference pictures in the SPS [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), C. S. Lim (Panasonic), S. Deshpande (Sharp)]

JCTVC-J0480 Crosscheck of long term reference pictures in SPS (JCTVC-J0116) [TK Tan (??)] [late] [miss]

JCTVC-J0118 AHG13: On semantics of delta_poc_msb_present_flag [A. K. Ramasubramonian, R. L. Joshi, Y.-K. Wang (Qualcomm)]

JCTVC-J0164 AHG 13: Simplified signalling of MSB cycle [Hendry, B. Jeon (LG)]

#### Reference picture list construction (2)

JCTVC-J0119 AHG13: On reference picture list modification [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0446 Cross-check report of AHG13: On reference picture list modification (JCTVC-J0119) [Sue M. T. Naing, Chong Soon Lim (Panasonic)] [late] [miss]

### Video parameter set (VPS) and sequence parameter set (SPS) (8)

JCTVC-J0074 AHG10 Hooks for Scalable Coding: Sequence Parameter Set Design [M. M. Hannuksela (Nokia)]

JCTVC-J0459 AHG10: Mental cross-check of JCTVC-J0074 [Y.-K. Wang (Qualcomm)] [late] [miss]

JCTVC-J0075 AHG10 Hooks for Scalable Coding: Video Parameter Set Design	[M. M. Hannuksela (Nokia)]

JCTVC-J0460 AHG10: Mental cross-check of JCTVC-J0075 [Y.-K. Wang (Qualcomm)] [late] [miss]

JCTVC-J0114 AHG10: On video parameter set [Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0502 AHG10: Mental cross-check of JCTVC-J0114 (Video parameter set HEVC base specification) [M. M. Hannuksela (Nokia)]

JCTVC-J0124 AHG10: On video parameter set for HEVC extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0196 AHG10: Proposed modification to video parameter set [K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-J0438 AHG10: Cross-check of JCTVC-J0196 [J. Xu (Microsoft)] [late]

JCTVC-J0257 AHG9/AHG10: Design of the Video Parameter Set [R. Skupin, V. George, T. Schierl]

JCTVC-J0484 AHG9/AHG10: Mental cross-check of JCTVC-J0257: Design of the Video Parameter Set [J. Boyce (Vidyo)] [late] [miss]

JCTVC-J0261 AHG9: Signaling of VPS Activation [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-J0481 Mental crosscheck for JCTVC-J0261 Signaling of VPS Activation [Hendry, B. Jeon (LG)] [late] [miss]

JCTVC-J0270 HEVC VUI Parameters with Extension Hooks  [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0487 Scalable Video Coding Signalling in VPS [A. Luthra (Motorola Mobility)] [late]

### Miscellaneous high-level syntax topics and syntax cleanups (12)

#### APS loss detection (1)

JCTVC-J0072 AHG9 High-Level Syntax: APS loss detection [M. M. Hannuksela (Nokia)]

JCTVC-J0457 AHG9: Mental cross-check of JCTVC-J0072 [Y.-K. Wang (Qualcomm)] [late]

#### Motion vector prediction related high-level syntax design (4)

JCTVC-J0071 High-level Syntax: Motion vector prediction issue for long-term reference picture [Y. Takahashi, O. Nakagami, T. Suzuki (Sony)]

JCTVC-J0356 Cross-verification on motion vector prediction issue for long-term reference picture (JCTVC-J0071) [I.-K. Kim (Samsung)] [late]

JCTVC-J0187 On temporal_mvp_enable_flag [K Sato (Sony)]

JCTVC-J0361 Cross-check report of JCTVC-J0187 on temporal_mvp_enable_flag [S.-C. Lim, H. Y. Kim, J. Lee (ETRI)] [late]

#### Multi-topic high-level syntax documents (1)

JCTVC-J0290 High layer syntax issues [C. Fogg (Harmonic), A. Wells (Ambarella)]

#### Syntax cleanups (6)

JCTVC-J0047 AHG6/AHG9: Syntax for APS ID [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0332 AHG6/AHG9: Cross check of Cleanups and fixes for APS (JCTVC-J0047) [T. Ikai (Sharp)] [late]

JCTVC-J0183 Syntax Issues [K. Sato (Sony)]

JCTVC-J0220 cu qp delta enabling syntax [T. Lee, J. Park (Samsung)]

JCTVC-J0273 A simple ordering issue for VUI parameters syntax [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0300 Slice Header Syntax Cleanup [Yue Yu, Jian Lou, Limin Wang(Motorola Mobility)]

JCTVC-J0495 Mental cross-check of JCTVC-J0300 [Y.-K. Wang (Qualcomm)] [late] [miss]

### High-level parallelism (18)

#### General (2)

JCTVC-J0123 AHG4: On tiles and wavefronts [M. Coban, Y.-K. Wang (Qualcomm)]

JCTVC-J0486 Mental cross-check of On tiles and wavefront parallel processing (JCTVC-J0123) [M. Horowitz (eBrisk)] [late]

JCTVC-J0249 Decoder parallelism indication [J. Samuelsson, R. Sjöberg]

JCTVC-J0479 Mental cross-check of JCTVC-J0249: Decoder parallelism indication [J. Boyce (Vidyo)] [late] [miss]

#### Tiles (8)

JCTVC-J0039 AHG4/AHG9: Syntax for restricting slices and tiles [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0471 Crosscheck of Syntax for restricting slices and tiles (JCTVC-J0039) [M. Coban, W.-J. Chien (Qualcomm)] [late]

JCTVC-J0042 AHG4/AHG9: Syntax for restricting tile numbers and tile sizes [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0412 Cross-check of “AHG4/AHG9: Syntax modifications for tile width constraint” (JCTVC-J0042) [M. Horowitz, S. Xu (eBrisk)] [late]

JCTVC-J0085 AHG9: On number of tile rows limit [M. Zhou(TI)]

JCTVC-J0088 AHG4: Enable parallel decoding with tiles [M. Zhou (TI)]

JCTVC-J0206 AHG 4: Asynchronous Tile Output [Hendry, B. Jeon (LG)]

JCTVC-J0423 Crosscheck of Asynchronous Tile Output (JCTVC-J0206) [G. Clare, F. Henry (Orange Labs] [late]

JCTVC-J0209 AHG 4: Constraint for slice and tile [Hendry, J. Kim, B. Jeon (LG)]

JCTVC-J0485 Mental cross-check of AHG 4: Constraint for slice and tile (JCTVC-J0209) [M. Horowitz (eBrisk)] [late]

JCTVC-J0235 On tile size restrictions in HEVC [A. Fuldseth (Cisco)]

JCTVC-J0383 A cross-check report for JCTVC-J0235 on tile size restriction in HEVC [K. Misra, K. Seung-Hwan, A. Segall (Sharp)] [late]

#### Wavefront parallel process (WPP) (4)

JCTVC-J0032 An HEVC transcoder converting non-parallel bitstreams to/from WPP [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0040 AHG4/AHG9: Syntax for restricting slices and WPP [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0472 Crosscheck of Syntax for restricting slices and WPP (JCTVC-J0040) [M. Coban, W.-J. Chien (Qualcomm)] [late]

JCTVC-J0279 CU skip and split CABAC context modification for flexible wavefront parallel parsing [M. Coban, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-J0425 AHG4: Improving parallelization efficiency of WPP using Overlapped Wavefront [M. Alvarez-Mesa, V. George, T. Schierl (HHI), C. C. Chi, B. Juurlink (TU Berlin)] [late]

#### Entry point signalling (5)

JCTVC-J0033 WPP entry points simplification [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0162 Crosscheck for JCTVC-J0033 [Hendry, B. Jeon (LG)]

JCTVC-J0081 AHG4: Signaling sub-stream entries in APS for parallel decoding [M. Zhou (TI)]

JCTVC-J0299 AHG4: On entry point coding [S.H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-J0475 Cross-check of JCTVC-J0299 - On entry point coding [A. Fuldseth (Cisco)] [late] [miss]

JCTVC-J0322 AHG4: Entry point signaling for wavefront substreams within tiles [K. Misra, K. Seung-Hwan, A. Segall (Sharp)]

### DPB size and other level related limits (9)

#### DPB size (6)

JCTVC-J0111 AHG9: On DPB size [R. L. Joshi, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-J0163 On DPBsize reduction [S. Lu, T. Suzuki (Sony)]

JCTVC-J0417 AHG9: Cross-check report for On DPBsize reduction (JCTVC-J0163) [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-J0199 AHG 9: Comments on DPB Size [Thomas Davies, Arild Fuldseth (Cisco)]

JCTVC-J0210 MaxDpbSize setting based on the picture size [A.Fujibayashi, T.K.Tan (NTT Docomo)]

JCTVC-J0283 Field Coding and MaxDpbSize constraints [D. Flynn, A. Gabriellini, M. Mrak, M. Naccari (BBC)]

JCTVC-J0367 On level considerations for interlaced content coding [Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse (ATEME)]

JCTVC-J0496 Variable MaxDpbSize based on maximum picture size and maximum number of reorder pictures [Arturo Rodriguez, A. K. Katti, H-Y Hwang (??)]  [late]

#### Other limits (4)

JCTVC-J0059 Suggested constraint on number of bits/bins per LCU [K. Chono (NEC)]

JCTVC-J0080 AHG9: On number of slices per picture limit [M. Zhou (TI)]

JCTVC-J0082 AHG9: On number of bits per LCU limit [M.Zhou, V. Sze (TI)]

JCTVC-J0442 Summary of discussion and input contributions on constraint on number of bits/bins per LCU [??] [late]

### Hypothetical reference decoder (HRD) (4)

JCTVC-J0136 AHG9: Improvement of HRD for sub-picture based operation [K Kazui, J Koyama, S Shimada, A Nakagawa (Fujitsu)]

JCTVC-J0272 Simplifications of HRD parameters for Temporal Scalability [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0306 AHG10: On Sub-picture Based CPB [S. Deshpande (Sharp)] [late]

JCTVC-J0465 AHG9: Mental cross-check of JCTVC-J0306 [Y.-K. Wang (Qualcomm)] [late] [miss]

JCTVC-J0444 Repeat picture output and HRD management [Arturo A. Rodriguez, A. K Katti, H-Y Hwang (??)] [late]

### VUI and SEI messages (9)

#### Video usability information (VUI) (1)

JCTVC-J0287 Additional sample_ratio_idc values for square samples [Arturo Rodriguez (Cisco)]

#### Frame packing arrangement (FPA) SEI message and related (4)

JCTVC-J0070 2D compatible frame packing arrangement SEI [O. Nakagami, T. Suzuki (Sony)]

JCTVC-J0073 AHG9 High-Level Syntax: Graceful Handling of Frame-Packed Pictures by Ignorant Decoders [M. M. Hannuksela (Nokia)]

JCTVC-J0491 AHG9: Mental Cross-check for JTCVC-0073 - Graceful Handling of Frame-Packed Pictures by Ignorant Decoders [S. Deshpande (Sharp)] [late]

JCTVC-J0458 AHG9: Mental cross-check of JCTVC-J0073 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0198 AHG9: Frame packing arrangement SEI message modification [M. Arena, P. Sunna (RAI)]

JCTVC-J0247 On frame packing arrangement SEI [B. Choi, C. Kim, J. Kim, J. Park (Samsung)]

#### Other SEI messages (4)

JCTVC-J0038 Exif data SEI message [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0456 AHG9: Mental cross-check of JCTVC-J0038 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0064 SEI messages for gradual decoding refresh [H. Aoki, K. Chono (NEC)]

JCTVC-J0483 AHG9: Mental cross-check of JCTVC-J0064 [K.Kazui (Fujitsu)] [late]

JCTVC-J0149 Signalling of Luminance Dynamic Range in Tone mapping information SEI [S. Hattori, T. Suzuki (Sony)]

JCTVC-J0245 SEI Message for profile and level signaling for temporal scalability and extensions [J. Boyce, D. Hong, W. Jang]

JCTVC-J0507 Crosscheck of JCTVC-J0245: SEI Message for profile and level signaling for temporal scalability and extensions [R. Sjöberg (Ericsson)] [late] [miss]

## Quantization

### QP prediction / delta QP coding

JCTVC-J0060 AHG5: Simplified cu_qp_delta coding [K. Chono (NEC)]

This contribution presents a simplified cu_qp_delta coding based on EG0 binarization. The simplified scheme maps a given cu_qp_delta into the unsigned value and represents it with the EG0 binary string. Only the first bin of the EG0 binary sequence is context-coded; the rest are bypass-coded. Thus it reduces the number of context-coded bins of cu_qp_delta in worst-case scenario from 11% to 2%. It is reportedly shown that the simplified scheme changes BD-rate results by 0.20% on average in AHG5 common test conditions.

The relatively large loss (compared to other proposals) is asserted to be due to change in mode decision.

JCTVC-J0429 AHG5: Cross-check of simplified cu_qp_delta coding with EG0 (JCTVC-J0060) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-J0089 AHG5: Bin reduction for delta QP coding [V. Sze, M. Budagavi (TI), V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

To improve the throughput of the CABAC, the number of context coded and bypass bins should be reduced.  Hardware is designed for the worst case, thus it is also important to reduce the worst case number of context coded bins.  In HM-7.0, cu_qp_delta accounts for 18% of the context coded bins in the worst case.  This is due to the fact that it is binarized with truncated unary and all the bins are context coded.  This contribution proposes using a combination of truncated unary and Exp-Golomb coding (TUEGk), similar to what is already used for motion vector difference.  Only the truncated unary portion is context coded and the Exp-Golomb portion is bypass coded. Furthermore, the bypass bins are grouped for throughput improvement; specifically, the sign bin is signaled after the magnitude bins. These modifications were implemented in HM-7.0 for TU with cMax=5 and Exp-Golomb, with k=0. Asserted benefits of this proposal include: 5x reduction of worst case context bins (from 26 to 5), ~2x reduction of total bin (from 27 to 15), grouping of bypass bins, simplified binarization process, and reduction in number of contexts by 1. The worst case number of bins for 8-bits and 10-bits are the same (0.06 context coded bins/pixel and 0.22 total bins/pixel).  Under the test conditions set by the AHG5 on entropy coding improvements, where cu_qp_delta is signaled for every 8x8 CU, and this proposal has overall BD-rate between -0.01 to 0.03.

Presentation not uploaded.

Main purpose of this change would be to prevent throughput problems in cases where QP is changed frequently (which is not usual but could hypothetically occur)

The test conditions with TM5 like rate control (as in previous quantization CE) were used

Several experts ask whether the results in terms of BD rate would still be similar when delta QP changes would be different from the current?

JCTVC-J0331 AHG5: Cross check of Bin reduction for delta QP coding (JCTVC-J0089) [T. Tsukuba (Sharp)] [late]

JCTVC-J0226 AhG5: Reduction of context bins and bypass bins grouping for cu_qp_delta [K. Terada, H. Sasai, T. Nishi (Panasonic)]

In the HEVC Draft 7, most of bins for the cu_qp_delta are arithmetic coded using the context models. This contribution proposes to use bypass coding instead of context based one for higher throughput and also proposes to concatenate bypass coded bins for parallel processing. The proposed modification was evaluated on top of HM7.0. Experimental result reportedly showed that the proposed modification can reduce context coded bins by 64% for cu_qp_delta and can reduce isolated bypass coded bins by 29% with less than 0.1% BD-rate loss.

Presentation not uploaded.

Different methods are investigated; proponent suggests the method “step2c” where everything is coded in bypass mode. Question: How does it compare versus direct usage of EG0 coding (method of JCTVC-J0060)? A: slightly better.

It is also commented that one more decision stage may be necessary.

JCTVC-J0328 AHG5: Cross-check of reduction of context bins and bypass bins grouping for cu_qp_delta (JCTVC-J0226) [V. Sze (TI)] [late]

Confirms that results match. Different versions are trading throughput versus efficiency.

JCTVC-J0298 AHG5:cu_qp_delta coding [S. H. Kim, L. Kerofsky, A. Segall (Sharp)]

In HM-7.0, cu_qp_delta requires up to 26 and 32 context coded bins in main profile and HE_10 configuration, respectively.  In order to improve the throughput efficiency, this contribution proposes a new efficient binarization method by combining truncated unary and fixed length coding (TUFLC).  Only a predefined number (cMax) of bins are context coded and the rest of the bins are bypass coded.  It is reported that the proposed method reduces the worst case number of context coded bins from 26 (or 32 in HE_10)bins to 4 bins. Moreover, it is reported that the total number of bins are reduced from 17 (or 33 in HE10) to 16 bins for all condition. The complexity reduction results in a BD-rate change from -0.02% to 0.02% on average in AHG5 common test conditions that is asserted to be negligible.

Comparably low loss (compared to other methods).

One benefit is that the length of the code is limited (unlike EG code).

JCTVC-J0506 Cross-check of high throughput binarization for cu_qp_delta coding (JCTVC-J0298) [J. Chen(Qualcomm)] [late] [miss]

Also related: One of the methods proposed in JCTVC-J0194 (see below)

Action on the proposals for modification of delta QP coding:

- It is generally desirable to decrease the possible maximum throughput at minimum loss in compression performance (J0089 and J0298 would give that)

- Structure of binarization should be consistent with schemes used for other syntax elements, e.g. MV differences

Decision: Adopt J0089 (Cmax=5), as it better fits the second criterion, and was already (similarly) proposed in the last meeting.

    - Quantization matrices

JCTVC-J0130 HVS Default Quantization matrices for 4x4 DCT/DST [C. Yeo, Y. H. Tan (I2R)]

This contribution proposes a set of default quantization matrices for 4x4 DCT/DST combinations derived from the same HVS based procedure used to obtain default quantization matrices for HEVC. It is claimed that the obtained default quantization matrices for the different combinations of DCT and DST share at least 9 out of 16 entries. It is advocated that this suggests that a single quantization weighting matrix may be sufficient for all combinations of 4x4 DCT/DST for intra luma coding.

JCTVC-J0275 Non-CE1: DST quantization matrix syntax for CE1 Simplification 2 [R. Cohen, A. Vetro (MERL)]

Core Experiment 1 (CE1) is testing two simplifications: Simplification 1 uses the 2-D DST for 4x4 luma Intra transform block, and Simplification 2 is the same except that the 2-D DCT is used when the Intra prediction mode is DC. The purpose of this contribution is to re-propose the syntax in JCTVC--I0419, which supports an additional scaling list for 4x4 luma Intra transform blocks, in case Simplification 2 of CE1 is adopted. This syntax supports two default scaling lists for 4x4 luma Intra transform blocks: one associated with the DCT and the other associated with the DST. It is reported that because Simplification 2 of CE1 does not apply the DCT and DST together in the same 4x4 transform block, it is not necessary to switch between the two scaling lists in a transform block.

Proposes the syntax in JCTVC-I0419, which supports a separate scaling list for 4x4 luma Intra transform blocks – one for the DCT residual case and one for the DST residual case – in the context of the case of "simplification 2" of CE1.

It was asked whether any visual benefit had been shown for this.

Argument ofThe statement made at the last meeting, that a switching of quant matrices dependent on intra mode at 4x4 block level is undesirable, is still valid.

Some doubt is raised whether usage of a separate quant matrix for 4x4 DST does provide a visually distinguishable benefit (the proposed matrices are quite similar between DCT and DST anyway)

The question is also raised how suitable the current DCT default matrices are,  as in practice most implementers would use own matrices with adaptation to the video material characteristics.

If CE1 simplification 1 would be adopted,  matrices do not need to be switched, but even in that case the current default matrix should be retained, as no evidence was shown (in terms of visual testing) that it is not appropriate for the 2D DST.

### Other

JCTVC-J0076 Improving HEVC compression efficiency by intensity dependant spatial quantisation [M. Naccari, M. Mrak, D. Flynn, A. Gabriellini (BBC)]

The properties of the human visual system can be exploited to improve the compression efficiency of the HEVC standard. In this proposal the pixel intensity masking of the human eye is used to apply coarser quantisation in darker and brighter image area. The proposed perceptual quantisation tool performs an Intensity Dependant Spatial Quantisation (IDSQ) similarly to the quantisation performed by the Intensity Dependant Quantisation (IDQ) tool presented at the 9th JCT-VC meeting in contribution I0257. However, the IDSQ tool proposed here is designed to make the inverse quantisation step at the decoder independent from the average pixel data which may introduce latency and pipelining refactoring during decoding. The proposed IDSQ has been implemented in the HM-7.0 and its performance is assessed by measuring the bitrate reduction with respect to the HM-7.0 codec. For the same perceptual quality level bitrate reductions of up to 25% are achieved and on average 3.4% across all tested points.

Most gain observed at low QP

In contrast to previous proposal, the adaptive scaling operation is done in the spatial domain, such that it is not necessary to wait with the transform until the prediction is available.

In the discussion, it is mentioned that this is likely to still have more latency than the current design and also requires additional buffers.

Further study, potentially interesting in the context of range extensions.

JCTVC-J0189 Cross check report of IDSQ (J0076) proposed by BBC [A Ichigaya (NHK)]

JCTVC-J0509 Cross-check: Subjective viewing on intensity dependant spatial quantisation (JCTVC-J0076) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-J0318 Consideration on Chroma QP Range Extension for HEVC version 1 and 2 [J. Xu, A. Tabatabai, K. Sato (Sony)]

No need to present, is covered by the decision on J0342.

JCTVC-J0342 Chroma QP range extension [G. J. Sullivan, S. Kanumuri, J. Xu, Y. Wu (Microsoft)]

This contribution proposes a way to extend the range of QP values used for chroma from [0, 39] to [0, 51]. The table used in the derivation of chroma QP values from luma QP is proposed to be modified to enable the extended range. In addition, the use of slice-level chroma QP offsets is discussed. The proposed scheme is asserted to have no significant impact on BD bit rate for common test conditions, as the modification mostly applies outside of the range of values that is tested in the common conditions. The results reportedly show a very small improvement for luma and a very small loss for chroma. Results are also reported for higher QP values than are used in the common conditions.

This contribution also proposes that offset range limits similar to those specified in AVC (ranging from −12 to +12) should be placed on the chroma QP offset values.

For the default case (offset 0), the ratio of luma versus chroma quant stepsize is suggested to be bouded to 2 (unlike 4 as it is currently)

No effect on current common conditions (except one small change at QPluma=34 which is done for the purpose of simpler implementation of the table).

Current HEVC spec does not have a limit on the offset values (whereas the software has the limit +/- 12 as suggested here)

Decision: Adopt all parts of the proposal.

JCTVC-J0373 Cross-check of JCTVC-J0342 on chroma QP range extension [J. Xu, A. Tabatabai (Sony)] [late] [miss]

The proposal is supported by cross-checkers

## Entropy coding

JCTVC-J0194 AHG5: Max exponential golomb code for reducing number of bins [K. Sugimoto, S. Sekiguchi, T. Murakami (Mitsubishi)]

This contribution proposes modifications to the CABAC binarization for ref_idx, cu_qp_delta and sao_offset. In HM-7.0, truncated unary binarization is used for these syntax, which introduces 15, 32 and 32 context coded bins in the worst case. To reduce the throughput of the CABAC, this contribution proposes using kth order Max Exp-Golomb coding. Max Exp-Golomb coding is similar to Exp-Golomb coding, but the longest code is one bit shorter than Exp-Golomb code by truncating the code using the maximum possible index.

The proposed method was implemented in HM-7.0, and it is reported that there is negligible coding efficiency impact with reducing the number of context coded bins per syntax element in the worst case down to 4, 5 and 5.

It is reported that bit rate increase in terms of BD-rate is 0.0% for ref_idx and 0.0-0.1% for sao_offset in CTC. The reported bit rate increase for cu_qp_delta is 0.2% in AHG5 test condition.

It is also proposed to apply 1st order Max Exp-Golomb code for mvd binarization. By signaling motion search range, maximum bins for mvd is also reduced.

Adopting this would mean that another binarization scheme would need to be defined, as EG0 cannot be replaced (is also used for transform coefficients)

Unlike EG0, this method might not have an implicit prefix/suffix error detection capability.

JCTVC-J0351 AHG5: Cross-verification of JCTVC-J0194 on max exponential golomb code for reducing number of bins [K. Chono (NEC)] [late]

JCTVC-J0195 AHG5: on CABAC initialization [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-J0208 AHG5: Bypass bins grouping on Intra mode coding [H. Sasai, K. Terada, T. Nishi (Panasonic)]

JCTVC-J0453 Cross-verification of JCTVC-J0208 : Bypass bins grouping on Intra mode coding [W.-J. Chien (Qualcomm)] [late]

## Transform coefficient coding

### Significance map coding

JCTVC-J0068 Refined significant map context derivation for large TU [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCTVC-J0200 Cross verification of Refined significant map context derivation for large TU (JCTVC-J0068) [H. Nakamura, T. Kumakura (JVC Kenwood)]

JCTVC-J0102 AHG5: Unification of context derivation for coefficient group flag [V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0396 Cross check of Qualcomm’s proposal JCTVC-J0102 [J. Kim (LGE)] [late] [miss]

duplicate?

JCTVC-J0451 Cross check of Qualcomm’s proposal JCTVC-J0102 [J. Kim (LGE)] [late]

JCTVC-J0308 Context index derivation method for level 0 significance map coding [K. Panusopone, W.-Y. Kung, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-J0439 Cross-verification of JCTVC-J0308 on Context index derivation method for level 0 significance map coding [X. Zheng (HiSilicon), H. Yu (Huawei)] [late] [miss]

JCTVC-J0354 Modification of context assignment for significance flag coding for large TUs [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0375 Cross-verification of Modification of context assignment for significance flag coding for large TUs (JCTVC-J0354) [R. Cohen (Mitsubishi)] [late]

### Last position coding

JCTVC-J0319 Further Unification of Luma/Chroma Context Derivation for Last Position Coding [L. Guo, M. Karczewicz, W.-J. Chien (Qualcomm)]

JCTVC-J0395 Cross check of Qualcomm’s proposal JCTVC-J0319 [J. Kim (LG)] [late]

### Coefficient level coding

JCTVC-J0095 AHG5: Pairwise Coding of Greater-than-1 Flags [N. Nguyen, D. He, G. Martin-Cocher (RIM)]

JCTVC-J0142 On coeff_abs_level_remaining coding [M. Budagavi, V. Sze (TI)]

JCTVC-J0378 Cross-check report for coeff_abs_level_remaining maximum codeword length reduction (JCTVC-J0142) [H. Sasai, K. Terada (Panasonic)] [late]

JCTVC-J0227 AhG5: Truncated unary binarization for coeff_abs_level_remaining [K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-J0454 Cross-verification of JCTVC-J0227:Truncated unary binarization for coeff_abs_level_remaining [W.-J. Chien (Qualcomm)] [late]

JCTVC-J0228 AhG5: Unification of transformed coefficient counter [K. Terada, H. Sasai, T. Nishi (Panasonic)]

JCTVC-J0362 Cross-verification on unification of transformed coefficient counter (JCTVC-J0228) [I.-K. Kim (Samsung)] [late]

JCTVC-J0408 AhG5: Crosscheck of JCTVC-J0228 [Y. Piao, J. H. Park (Samsung)] [late]

JCTVC-J0254 Position-based Context Derivation of coeff_abs_level_greater1_flag Coding [T. Ji, X. Yu, D. He, G. Martin-Cocher (RIM)]

JCTVC-J0488 Cross-verification of JCTVC-J0254: Position-based Context Derivation of coeff_abs_level_greater1_flag Coding [W.-J. Chien (Qualcomm)] [late]

JCTVC-J0424 Crosscheck of JCTVC-J0254 [Y. Piao, J. H. Park (Samsung)] [late] [miss]

JCTVC-J0285 Early by-pass mode switch of greater1_flag [J. Chen, W.-J. Chien, J. Sole, M. Karczewicz(Qualcomm)]

JCTVC-J0490 Cross-check report for early bypass mode switch of greater1_flag (JCTVC-J0285) [S. H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-J0304 Context assignment for parallel coefficient level coding [W.-J. Chien, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0498 Cross-check report for context assignment for parallel coefficient level coding (JCTVC-J0304) [H. Sasai, K. Terada (Panasonic)] [late] [miss]

### Scans

JCTVC-J0105 Simplification of coefficient scanning order [R. Cohen (Mitsubishi)]

JCTVC-J0135 Cross-verification of Simplification of coefficient scanning order (JCTVC-J0105) by BBC [M. Mrak, M. Naccari (BBC)]

JCTVC-J0150 Removal of zigzag scan from scaling list coding [M. Shima (Canon)]

JCTVC-J0205 Cross-check report on removal of zigzag scan from scaling list coding (JCTVC-J0150) [S.-C. Lim, H. Y. Kim, J. Lee (ETRI)]

JCTVC-J0256 Removal of the 8x2/2x8 coefficient groups [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-J0421 Cross-check of Removal of the 8x2/2x8 coefficient groups (J0256) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-J0281 Additional horizontal and vertical scan for transform coefficients [C. Auyeung (Sony)]

JCTVC-J0368 Cross-check of additional horizontal and vertical scans (JCTVC-J0281) [J. Sole (Qualcomm)] [late]

### Sign data hiding

JCTVC-J0067 Simplified decision for sign hiding [T. Ikai (Sharp)]

JCTVC-J0329 Cross-check of Simplified decision for sign hiding (JCTVC-J0067) [V. Sze (TI)] [late] [miss]

JCTVC-J0094 Simplification of multiple sign bit hiding criterion [J. Wang, D. He, X. Yu, G. Martin-Cocher (RIM)]

JCTVC-J0333 Cross-verification of Simplification of multiple sign bit hiding criterion (JCTVC-J0094) [Y. Yu, W. Wan (Broadcom Corp)] [late] [miss]

JCTVC-J0447 Crosscheck of Simplification of multiple sign bit hiding criterion (JCTVC-J0094) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0099 Sign data hiding simplification [J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-J0431 Cross-check of: "Sign data hiding simplification" (J0099) [A. Gabriellini, M. Mrak, M. Naccari (BBC)] [late]

JCTVC-J0377 Cross-check of JCTVC-J0099 on sign data hiding simplification [C. Auyeung (Sony)] [late]

JCTVC-J0274 Parity Inference [J. Wang, D. He, X. Yu, G. Martin-Cocher (RIM)]

JCTVC-J0405 Cross check of Parity Inference (JCTVC-J0274) [T. Ikai, T. Tsukuba (Sharp)] [late]

### CBF

JCTVC-J0157 Cross-check of luma CBF inference proposed by Microsoft [J. Kim (LGE)]

JCTVC-J0158 Cross-check of simplified CBF inference proposed by Microsoft [J. Kim (LGE)] [late]

JCTVC-J0161 Removal of the inference of luma CBF at the transform depth level zero in inter CU [J. Kim, B. Jeon (LGE)]

JCTVC-J0321 Cross-verification of JCTVC-J0161 Removal of the inference of luma CBF at the transform depth level zero in inter CU [L. Guo (Qualcomm)] [late]

JCTVC-J0241 Simplification on CBF inference and coding [J. Xu (Microsoft)]

JCTVC-J0303 Simplification on context derivation of cbf_luma syntax element [I.-K. Kim, J. H. Park (Samsung)]

JCTVC-J0376 Cross-check report for Simplification on context derivation of cbf_luma syntax element (JCTVC-J0303) [H. Sasai (Panasonic)] [late]

## Intra prediction and intra mode coding

### General

JCTVC-J0131 Motion estimation-based fast intra prediction(ME-FIP) [Xiaohai He, Guoyun Zhong, Yuan Li, Di Wu, Linbo Qing (Sichuan Univ.)]

No presenter available Wed. 18:25

JCTVC-J0271 Luma intra planar prediction filter [C. Auyeung (Sony)]

This contribution proposes to apply the luma intra DC prediction filter to filter the luma intra planar prediction. When the DC prediction filter is applied to both luma intra DC prediction and the luma intra planar prediction in HM7, both Intra Main and Intra HE10 resulted in BDR of -0.1%. When the DC prediction filter is disabled for DC prediction in HM7, both Intra Main and Intra HE10 resulted in BDR of 0.1% and BDR of -0.2% for Class F test sequences.

Concern is raised that this causes additional complexity in software implementation. Code optimized for DC cannot be re-used for planar.

Planar mode is already the most complex mode

JCTVC-J0301 Cross-verification of Luma intra planar prediction filter (JCTVC-J0271) [R. Cohen (Mitsubishi)]

### Chroma modes

JCTVC-J0052 Bug-fix of intra chroma DM mode for Intra_NxN with SCU size larger than 8x8 [T.-D. Chuang, Y.-W. Huang, Y.-H. Huang, T.-A. Lin, K.-B. Lee, C.-C. Ju, S. Lei (MediaTek)]

A mismatch between HM-7.0 software and HEVC text specification draft 7 is reported as follows. In the HM-7.0 software, an intra CU sends only one intra chroma prediction mode regardless of the PU partition mode. However, in the text specification (JCTVC-I1003_d4.doc), an intra CU sends one intra chroma prediction mode for each chroma PU, which is inconsistent with HM-7.0 software when NxN PU partition mode is selected. The inconsistency needs to be resolved.

Two possible solutions are reported here. Method 1 follows more or less what is in the software but includes a bug fix to let the four chroma PUs refer to the four different luma PUs when the intra CU is an SCU larger than 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.1-0.6% luma coding gains and -0.1-1.0% chroma coding gains when the SCU size is set to 16x16 in luma. Method 2 follows what is specified in text but includes a bug fix to send only one intra chroma prediction mode when the intra CU is an SCU equal to 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.0-0.4% luma coding gains and 0.6-2.4% chroma coding gains for when the SCU size is set to 16x16 in luma.

Matched software and text are provided for both methods. It is proposed to adopt one of the proposed methods as a bug-fix to the current HEVC.

Results are presented with SCU 16x16 as for 8x8 (common test conditions) no effect would be visible

What is suggested here is less a “bug fix” than a new proposal which suggests to derive different prediction modes for the case where several chroma PBs are contained in a CU. The decision of last meeting was to use only one chroma prediction mode (i.e. only one prediction direction etc. for all chroma PBs) within a CU. This was extensively discussed and it was confirmed that currently software and text are aligned.

No action.

JCTVC-J0146 Cross-check report of JCTVC-J0160 on LM parameter estimation [Y. Lin (HiSilicon)]

JCTVC-J0159 Cross check of MediaTek’s proposal JCTVC-J0052 [J. Kim (LGE)]

JCTVC-J0160 Parameter derivation for intra LM chroma prediction [J. Kim, B. Jeon (LGE)]

This contribution proposes a method to employ the samples around CUs (i.e. Coding blocks) instead of TUs(i.e. Transform blocks) in order to get parameters for intra LM chroma mode. It aims to reduce the number of calculation of parameters and corresponding calculation burden. It shows 0.0%, -0.7%, -0.2% BD-rate under the AIHE10 condition. It also shows 0.0%, -0.7%, -0.6% and 0.0%, 0.1%, -0.3% BD rate -0.3%, -0.2% BD-rates respectively under the RAHE10 and LBHE10 condition. It alters two words in the Text.

The initial computation becomes more complex (and may be subject to overflow in addition as the area is 2x as large at maximum); also introduces potential latency in first chroma block; on the other hand, the overall complexity is reduced as the process is only invoked once. For CU of size 8x8 (worst case) it does not help.

One expert mentions that the method may even have a slight complexity disadvantage due to the larger extension to CU size.

No action.

JCTVC-J0146 Cross-check report of JCTVC-J0160 on LM parameter estimation [Y. Lin (HiSilicon)]

## Transforms

JCTVC-J0267 On DST/DCT adaptation [J. Xu, C. Auyeung, A. Tabatabai (Sony)]

Extends the DCT/DST selection to chroma. Currently the chroma uses the DCT style transform always.

JCTVC-J0369 Non-CE 1: Cross-Check of JCTVC-J0267 [A Saxena, E Alshina, F Fernandes (Samsung)] [late]

JCTVC-J0452 Cross Check of JCTVC-J0267 Test 2 [G. Van der Auwera (Qualcomm)] [late]

  - Memory bandwidth reduction

JCTVC-J0086 AHG7: Disallow bi-predictive mode for 8x4 and 4x8 inter PUs [M. Zhou (TI)]

JCTVC-J0214 AHG7: Cross-check report of disallow bi-predictive mode for 8x4 and 4x8 inter PUs (JCTVC-J0086) [T. Sugio (Panasonic)]

JCTVC-J0166 An experimental comparison of memory bandwidth between P and B-slice coding [T.Chujoh (Toshiba)]

JCTVC-J0314 Cross-verification of "An experimental comparison of memory bandwidth between P and B-slice coding" (JCTVC-J0166) [S. Fukushima, H. Nakamura (JVC Kenwood)]

JCTVC-J0175 AHG7: Memory bandwidth limitation for higher level [K. Kondo, T. Suzuki (Sony), U. Motoharu, S. Fukushima (JVC KENWOOD Corporation)]

JCTVC-J0311 Cross check of memory bandwidth reduction for higher level (JCTVC-J0175) [T. Yamamoto (Sharp)]

## Alternative coding modes

A BoG (coordinated by K. Chono) was requested to review the contributions in this category.)

### I_PCM

JCTVC-J0153 Burst I_PCM Signalling [M. Coban, W.-J. Chien, J. Chon, M. Karczewicz (Qualcomm)]

JCTVC-J0411 Crosscheck of Burst IPCM Coding Signalling (JCTVC-J0153) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0277 Interaction between pcm_flag and cu_transquant_bypass_flag [R. Joshi, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

### Transform skipping

JCTVC-J0053 Residue scan for intra transform skip mode [J. An, L. Zhao, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0069 Constant coefficient context for intra transform skipping [T. Tsukuba, T. Yamamoto (Sharp)]

JCTVC-J0259 Cross-verification of Constant coefficient context for intra transform skipping (JCTVC-J0069) by BBC [M. Mrak, M. Naccari (BBC)] [late]

JCTVC-J0077 Transform Skipping for Inter Predicted Coding Units [A. Gabriellini, M. Mrak, D. Flynn, M. Naccari (BBC)]

JCTVC-J0092 Transform Skipping in the presence of Scaling Lists [D. Flynn, M. Mrak, M. Naccari, A. Gabriellini (BBC)]

JCTVC-J0093 Rotation of Residual Block for Transform Skipping [D. He, J. Wang, G. Martin-Cocher (RIM)]

JCTVC-J0100 Cross-check of transform skipping for inter prediction (JCTVC-J0077) [J. Sole, R. Joshi (Qualcomm)] [late]

JCTVC-J0171 On transform skip mode for chroma TUs [E. François, P. Onno, C. Gisquet, G. Laroche (Canon)]

JCTVC-J0380 Cross-check of JCTVC-J0171 on transform skip mode for chroma TUs	[C. Auyeung (Sony)] [late]

JCTVC-J0184 On Transform Skip [Y. Morigami, K Sato (Sony), T Sugio, H Sasai (Panasonic)]

JCTVC-J0201 QM bypass for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

JCTVC-J0420 Crosscheck report of QM bypass for transform skip mode (JCTVC-J0201) [M.Shima (Canon)] [late]

JCTVC-J0202 Coefficient scan for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

JCTVC-J0386 Cross check of Coefficient scan for transform skip mode (JCTVC-J0202) [T. Yamamoto (Sharp)] [late]

JCTVC-J0212 Modified scan method for intra transform skipping [T. Sugio, T. Nishi (Panasonic)]

JCTVC-J0327 Crosscheck of JCTVC-J0212: Modified scan method for intra transform skipping [M. Budagavi (TI)] [late]

JCTVC-J0237 Inter transform skipping [J. Xu (Microsoft)]

JCTVC-J0326 Crosscheck of JCTVC-J0237: Inter transform skipping [M. Budagavi (TI)] [late]

JCTVC-J0244 Cross-check of JCTVC-J0053 and JCTVC-J0093 [J. Xu (Microsoft)] [late]

JCTVC-J0265 Intra Transform Skipping: Smallest CU and Implicit Chroma [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-J0430 Cross-check of: "Intra Transform Skipping: Smallest CU and Implicit Chroma" (J0265) [A. Gabriellini, M. Mrak, M. Naccari (BBC)] [late]

JCTVC-J0309 Removal of last coefficient coding in transform skipping mode [K. Panusopone, Y. Yu, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-J0508 Cross-check of JCTVC-J0309 on removal of last significant coefficient position coding in transform skip [C. Yeo, Y. H. Tan (I2R)] [late] [miss]

JCTVC-J0313 Modified coefficient scan order mapping for transform skip mode [R. Cohen, A. Vetro (MERL)]

JCTVC-J0407 Cross-check of JCTVC-J0313 (Modified coefficient scan order mapping for transform skip mode) [C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-J0389 Simplification for intra transform skip mode [L. Zhao, J. An, Y.-W. Huang, S. Lei (MediaTek), D. Zhao (Harbin Institute of Technology)] [late]

JCTVC-J0393 Cross-check of Simplification for Intra Transform Skip Mode in JCTVC-J0389 [W.-S. Kim (TI)] [late]

JCTVC-J0468 Combination of JCTVC-J0069 and JCTVC-J0093 [T.Tsukuba, T.Yamamoto, T.Ikai (Sharp), D.He, J.Wang, G.Matin-Cocher (RIM), L.Guo, R.Joshi, M.Karczewicz (Qualcomm)] [late]

### Lossless compression

JCTVC-J0061 AHG11: Study report of HEVC lossless coding [K. Chono (NEC)]

JCTVC-J0168 AHG11: Syntax harmonisation of the I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

JCTVC-J0426 Crosscheck of Syntax harmonisation of the I_PCM and TransQuantBypass modes (JCTVC-J0168) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0169 AHG11: Loop filtering control for I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

JCTVC-J0436 AHG11: Study report of JCTVC-J0169 on loop filtering control for I_PCM and TransQuantBypass modes [K. Chono (NEC)] [late]

JCTVC-J0230 Improvement of HEVC lossless coding using transform coefficient coding [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

JCTVC-J0232 HEVC lossless coding for medical image compression [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

JCTVC-J0238 Lossless coding via transform skipping [J. Xu (Microsoft)]

JCTVC-J0363 AHG11: Cross-check of JCTVC-J0238 [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)] [late] [miss]

JCTVC-J0339 AHG11: Methods to Encode Lossless Coding Indication Flag [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

JCTVC-J0340 AHG11: New signalling mechanism for lossless coding [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

JCTVC-J0435 Lossless Coding and Loop Filter Control for Transform Skip [G. Van der Auwera, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0474 AHG11: evaluation of lambda settings for lossless coding [E.François, M.Shima, P.Onno, G.Laroche, T.Poirier (Canon)] [late]

### Texture coding

## Non-normative: Encoder optimization, decoder speed improvement, post filtering, loss concealment, rate control

JCTVC-J0057 Adaptive rate control for HEVC [J. Si, S. Ma, W. Gao (Peking Univ.), M. Yang (Huawei)]

JCTVC-J0126 Code for verifying the coding tree block raster, tile and z-scan processes [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-J0242 Encoding with fixed Lagrangian multipliers [J. Xu (Microsoft)] miss]

JCTVC-J0134 Cross-check of Encoding with Fixed Lagrangian Multipliers (JCTVC-J0242) [Y. Chiu, W. Zhang, ]J. Zhang (Intel) [late]

JCTVC-J0260 Source code bug fix on the RQ model equation [H. M. Choi, J. H. Nam, D. G. Sim (KWU)]

JCTVC-J0262 Initial QP set for rate control [H. M. Choi, J. H. Nam, D. G. Sim (KWU)]

JCTVC-J0263 Reports on improved RD performance of rate control [H. M. Choi, J. H. Nam, S. M. Yoo, D. G. Sim (KWU), Y.-H. Kim, J. H. Park (KETI)] [late]

## To be allocated | category not clear

JCTVC-J0143 AHG5: Crosscheck of redundancy removal on syntax in JCTVC-Jxxxx [J. Lee, S. Kim, S. Lee (Yonsei Univ.)] [late]

## Withdrawn and missing contributions

JCTVC-J0058 [Withdrawn]

JCTVC-J0294 [Withdrawn]

JCTVC-J0384 [Withdrawn]

JCTVC-J0404 [Withdrawn]

JCTVC-J0434 [Withdrawn]

JCTVC-J0441 [Withdrawn]

JCTVC-J0497 [Withdrawn]

# Plenary Discussions and BoG Reports

## Project development

JCTVC-J0470 HEVC Reference Software Manual [F. Bossen, D. Flynn, K. Suehring (SW Coordinators)] [late] [miss]

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XX Jul. 2012.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs

Because a draft design and HEVC test model (referred to as the HM) have now been established, group coordinated experiments are now referred to as "core experiments" rather than "tool experiments".

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-F900.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the HM 5.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-F9xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at the preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-F900.

## Software development (to be updated)

The software coordinator had already started integrating bug fixes on top of HM 3.3 software, and had strongly recommended for proponents of adopted proposals to re-implement them in HM 3.3 and test in this environment before integrating them into HM 4.x. All tools were planned to again be thoroughly tested after integration in HM 4.x.

HM 4.0 should be available within 3 weeks after the meeting, and will be used for CEs. HM 4.1 is planned to be available 3 weeks later.

## Subjective test plan for design

Plan a subjective test enabling results at the Feb meeting.

Plans for subjective testing: Preparation of visual comparison for the February meeting will need to be discussed in the next meeting. Initial idea: Use a set of rates e.g. as from CfP, and run HM and JM on same conditions. It may not be necessary to run all different constraint cases, RA may be sufficient.

Need to prepare JM & HM encodings, roughly as was done for the CfP.

# Establishment of ad hoc groups

JCTVC-J0280 Request to establish an AHG to reduce computation in residual coding [C. Auyeung, T. Suzuki (Sony)]

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-I1002 HEVC Test Model 7 (HM 7) Encoder Description. - Produce and finalize JCTVC-I1003 HEVC text specification Draft 7. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs) | N |
| Software development and HM software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCT-VC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 7.0 software version and the reference configuration encodings according to JCTVC-I1100 based on common conditions suitable for use in most core experiments (expected within 2 weeks after the meeting). - Prepare and deliver HM 7.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 7.0 version (expected within three weeks after the 7.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| High-level parallelism (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the implication of requiring the use of specific high-level parallelism tool(s) for very high resolution video to guarantee decoders can utilize parallel decoding. - Study the implication of having both tiles and WPP in one profile when the use of specific high-level parallelism tool(s) are required for very high resolution video. - Identify and work to resolve issues relating to the draft text description of existing high-level parallelism related tools and the associated HM implementation. - Identify and discuss additional issues relating to high-level parallelism. | M. Horowitz (chair), M. Coban, F. Henry, K. Kazui, A. Segall, W. Wan, S. Wenger, M. Zhou (vice-chairs) | N |
| Entropy Coding Improvements (AHG5) (jct-vc@lists.rwth-aachen.de) - Study the number of context coded bins of the current HM design across various syntax elements, especially in the worst-case scenario. - Investigate possible reductions to context coded bins; in particular, the reduction for reference index, delta QP, SAO offsets, and transform coefficient level coding. - Discuss and arrange test conditions and software. - Investigate the trade-off between coding efficiency and the context-coded bin reduction. - Study the CABAC initialization tables in the current HM design and their impact on test sequences both inside and outside the common conditions. - Investigate other possible improvements of the entropy coding in the current HM design. - Report the results of these studies to the JCT-VC and recommend solutions where feasible. | A. Segall (chair), C. Auyeung, K. Chono, G. Martin-Cocher, T. Nguyen, J. Sole, V. Sze, W. Wan (vice-chairs) | N |
| In-loop filtering (AHG6) (jct-vc@lists.rwth-aachen.de) - Clean up and stabilize the HM software, the draft text and the HM encoder description on non-deblocking in-loop filtering. - Study extending the adaptability range of the deblocking filter - Study harmonized control scheme of SAO and ALF. - Study SAO simplification helping efficient implementation. | T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin, P. Onno (vice-chairs) | N |
| Memory bandwidth restrictions in motion compensation (AHG7) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth in motion compensation. - Study memory bandwidth reduction method in the HM software and the draft text. - Study the impact of the memory bandwidth reduction techniques on complexity and coding efficiency. | T. Suzuki (chair), W. Wan, M. Zhou (vice-chairs) | N |
| Loss robustness (AHG8) (jct-vc@lists.rwth-aachen.de) - Create and/or maintain tools to test loss robustness including error patterns and a loss simulator. - Identify techniques and conditions for testing the loss robustness of the design. - Study the degree of loss robustness of the HM design and identify deficiencies. - Identify and study the interdependencies in the HM design in relation to loss robustness, and the potential consequences of these interdependencies. - Investigate solutions to improve loss robustness. - Investigate the trade-off between coding efficiency and loss robustness. | A. Rodriguez, P. Onno (co-chairs) | N |
| High-level syntax (AHG9) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs, including checking and fixing texts for SEI messages currently included by referring to the AVC specification. - Study the hypothetical reference decoder (HRD) syntax and operations, including making sure that the correct use of max_dec_pic_buffering[ i ] in the HDR text, as well as the text for bitstream conformance and decoder conformance. - Work towards simplification and general minor cleanup of the high-level syntax. - Assist in software development and text drafting for the high-level syntax in the HEVC design. | Y. K. Wang (chair), J. Boyce, Y. Chen, S. Deshpande, M. M. Hannuksela, M. Haque, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan, P. Wu (vice-chairs) | Y (one day before the July meeting) |
| Hooks for scalable coding (AHG10) (jct-vc@lists.rwth-aachen.de) - Investigate hooks that would be needed for support of bitstream scalability in HEVC syntax. | J. Boyce (chair), J. Kang, J. Samuelsson, W. Wan, Y.-K. Wang (vice-chairs) | N |
| Lossless coding (AHG11) (jct-vc@lists.rwth-aachen.de) - Complete the software using the adopted lossless coding signaling mechanism and integrate it into HM7.0. - Review the draft text on lossless coding, and suggest cleanup as appropriate. - Study and create test conditions for evaluating the efficiency of lossless signaling method. - Study and investigate possible improvements. | W. Gao (chair), K. Chono, F. Henry, J. Xu, M. Zhou, P. Topiwala (vice-chairs) | N |
| Support for range extensions (AHG12) (jct-vc@lists.rwth-aachen.de) - Examine modifications to the technical design that provide an increase in picture fidelity and support for non-4:2:0 or non-8-bit picture formats - Study aspects of the technical design and software that need modification to support non-4:2:0 chroma formats. - Study aspects of the current technical design and software that need modification to support bit-depths beyond 8 bit. - Assist and advise in the work of removing any implicit assumptions of 8-bit depth and 4:2:0 formatting from the current draft and software (where feasible, without introducing technical design changes). | D. Flynn (chair), D. Hoang, K. McCann, E. Francois, K. Sugimoto, P. Topiwala (vice-chairs) | N |
| Reference picture buffering and list construction (AHG13) (jct-vc@lists.rwth-aachen.de) - Provide source code that enables HM encoding of all test cases described in JCTVC-H0725 and produce anchor data. - Identify and work to resolve issues relating to the draft text description of reference picture handling and list construction, and the associated HM software functionality. - Study possible improvements related to reference picture buffering and list construction. - Study the loss resilience properties of reference picture handling and its support in the HM software (in coordination with the loss resilience AHG). | R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, Y.-K. Wang (vice-chairs) | N |
| Study on HEVC conformance requirements (AHG14) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Study to develop a potential set of HEVC conformance bitstreams | T. Suzuki, W. Wan (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-I1000 Meeting Report of 9th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Suehring, D. Flynn, F. Bossen, (software coordinators)]

Remains valid

JCTVC-I1002 High Efficiency Video Coding (HEVC) Test Model 7 (HM 7) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N XXXXX)

JCTVC-I1003 High Efficiency Video Coding (HEVC) text specification draft 7 [B. Bross (primary), W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand] (WG 11 N XXXXX)

JCTVC-I1004 Draft Disposition of Comments on.. [B. Bross, G. J. Sullivan, J.-R. Ohm, XXXX] (WG 11 N XXXXX)

JCTVC-I1100 Common HM test conditions and software reference configurations [F. Bossen]

Modifications include:

- SAO settings as from CE1.

- AMP on

- ALF conf. (RA mode as of I0603) in HE10, with current-picture optimization

- Transform skip off in main, on in HE10

Agreed.

Software HM7.0 availability (suitable for CTC coding efficiency experiments) two weeks after the end of the meeting. 7.1 three weeks later

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description, otherwise it cannot be claimed to be part of the CE

Document deadline of July 2012 meeting isJuly 2nd.

JCTVC-I110 Core Experiment 1: Intra transform mode dependency simplifications [K. Ugur, A. Saxena]

# Future meeting plans, expressions of thanks, and closing of the meeting

tbd

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 11–20 July 2012 under WG 11 auspices in Stockholm, SE.

- 10–19 October 2012 under WG 11 auspices in Shanghai, CN.

- 14–23 January 2013 under ITU-T auspices in Geneva, CH.

- 17-26 April 2013 under WG 11 auspices in Incheon, KR.

ITU was thanked for its excellent hosting of the 9th meeting of the JCT-VC and for providing the viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1753 hours on Monday 7 May 2012.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixth meeting of the JCT-VC, according to a sign-in sheet passed around during the meeting (approximately 255 in total), were as follows: