| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 13th Meeting: Incheon, KR, 18-26 Apr. 2013 | Document: JCTVC-M_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the 13th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Incheon, KR, 18-26 Apr. 2013 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its thirteenth meeting during 18-26 Apr. 2013 at the Songdo Conversia, Incheon, KR. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 18 Apr. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 Apr. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 10th HEVC Test Model (HM10) software and text and editing the 10th HEVC specification Draft (which was issued as ISO/IEC FDIS 23008-2 and submitted to ITU for consent on ITU-T Rec. H.265), review the results from two interim Core Experiments on range extensions (RCEx) and five Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Any Corrigendum work?

In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting: Draft 3 for HEVC conformance testing, draft 3 for HEVC range extensions, draft 2 of SHVC extensions and SHVC Test Model 2, and two documents specifying common test conditions and software reference configurations for experiments – one for HEVC range extension experiments, and one for scalable coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next five JCT-VC meetings are planned for 25 July – 2 Aug. 2013 under WG 11 auspices in Vienna, AT, 24 Oct. – 1 Nov 2013 under ITU-T auspices in Geneva, CH, 9–17 Jan. 2014 under WG 11 auspices in San José, US, 27 March – 4 Apr. 2014 in Valencia, ES, and 17–25 June 2014 in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its thirteenth meeting during 18-26 Apr. 2013 at the Songdo Conversia, Incheon, KR. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 900 hours on Thursday 18 Apr. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 Apr. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2013_04_M_Incheon/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 10th HEVC Test Model (HM10) software and text and editing the 10th HEVC specification Draft (which was issued as ISO/IEC FDIS 23008-2 and submitted to ITU for consent on ITU-T Rec. H.265), review the results from two interim Core Experiments on range extensions (RCEx) and five Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Any Corrigendum work?

See opening remarks and M0001 discussions.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 8 Apr. 2013.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 9 Apr. 2013 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-M0353 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "M0353+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late: (update)

- JCTVC-L0185 (a proposal relating to SHVC inter-layer motion prediction) [uploaded 01-09]

- ...

The following other profile/level proposal contributions were registered on time but were uploaded late:

- JCTVC-LXXXX (a profile/level proposal) [uploaded 01-xx]

- ...

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-L0096 (an information contribution showcasing an HEVC implementation) [uploaded 01-10]

- ...

The following cross-verification reports were registered on time but were uploaded late: JCTVC-L0066 [uploaded 01-09], JCTVC-L0081 [uploaded 01-12], JCTVC-L0082 [uploaded 01-12], JCTVC-L0091 [uploaded 01-09], JCTVC-L0092 [uploaded 01-09], JCTVC-L0093 [uploaded 01-10], JCTVC-L0094 [uploaded 01-11], JCTVC-L0123 [uploaded 01-11], JCTVC-L0124 [uploaded 01-11], JCTVC-L0141 [uploaded 01-10], JCTVC-L0142 [uploaded 01-10], JCTVC-L0144 [uploaded 01-10], JCTVC-L0145 [uploaded 01-10], ], JCTVC-L0172 [uploaded 01-11], ....

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-L0050, JCTVC-L0120, JCTVC-L0122, JCTVC-L0143, JCTVC-L0173, JCTVC-L0186, JCTVC-L0191, JCTVC-L0298, JCTVC-L0299, JCTVC-L0318, JCTVC-L0367, JCTVC-L0398, JCTVC-L0457.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-LXXXX (a ..., corrected by a late upload on 01-XX)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-L1000, the HEVC Test Model (HM) JCTVC-L1002, the Draft Specification JCTVC-L1003, the Conformance Draft JCTVC-L1004, the Draft Specification of Range Extensions JCTVC-L1005, the SHVC draft specification JCTVC-L1008 and SHVC test model 1 (SHM1) JCTVC-L1007 were approved. The HM reference software produced by the AHG on software development, the reference software versions for range extensions and SHVC, and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GRP: Generalized residual prediction.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TFD: Tagged for discard.

- Unit types:

  - CTB: code tree block (synonymous with LCU).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

The status of HEVC in ISO/IEC and ITU-T was noted. The FDIS 23008-2 had been submitted for ballot  in ISO/IEC. In ITU-T, the text was submitted for Consent and approved as Rec. H.265 on 2013-XX-XX.

The HEVC conformance testing and reference software specification had been submitted as ISO/IEC CD 23008-5. The ballot is 2013-07-14.

The range extensions draft 2 had been submitted as ISO/IEC 23008-2/PDAM1. The ballot is 2013-07-07.

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS / DAM ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

Goals: Progress of work on extensions, conformance & reference software (Study of), verification testing?

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2200, with coffee and lunch breaks as convenient.

- First day (Thu. 18 Apr.): 0900–2000 (approximately), morning plenary, Tracks A and B in afternoon.

- Second day (Fri. 19 Apr.): 0900–2000 (approximately), Tracks A and B (and BoGs, ...)

- ...

- Discussion of RExt starts on Sunday

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. (Note: allocation to tracks were subject to changes)

- AHG reports (17) Track P (section 2)

- Communication to and by parent bodies (0) Track P (section 3.1)

- Conformance testing development (1) Track P (section 3.2)

- Version 1 bug reports (1) Track P (section 3.3)

- Coding performance, implementation, and design analysis (5) Track P (section 3.4)

- Profile, level and constraint definitions (2) Track P (section 3.5)

- HEVC use cases (6) Track P (section 3.6)

- Source video test material (1) Track P (section 3.7)

- RExt CE1: Inter-component decorrelation (11) Track X (section 4.1)

- RExt CE2: Intra prediction for lossless coding (12) Track X (section 4.2)

- SHVC CE1:  Intra prediction improvements (32) Track A (section 5.1)

- SHVC CE2:  Inter-layer texture prediction (8) Track A (section 5.2)

- SHVC CE3:  Combined inter and inter-layer prediction (21) Track B (section 5.3)

- SHVC CE4:  Inter-layer filtering (19) Track A (section 5.4)

- SHVC CE5:  Inter-layer syntax prediction (HEVC base layer) (22) Track B (section 5.5)

- Non-CE RExt (57) Track X (section 6.1)

- Non-CE SHVC (87 CE related, 26 other) (section 6.2)

  - General (track P)

  - SCE1 related (22) (track A)

  - SCE2 related (2) (track A)

  - SCE3 related (30) (track B)

  - SCE4 related (21) (track A)

  - SCE5 related (12) (track B)

  - Motion/partition coding (2) (track B)

  - Modifications ref_idx (7) (track B)

  - Special up/downsampling (8) (track A)

  - Transforms (2) (track B)

  - Other scalable modalities (7) (track B)

- High-level syntax in RExt & single layer (5) Track X (section 6.3)

- High-level syntax in SHVC and 3D extensions (61) BoOG | joint with JCT-3V (section )
	BoG (M. M. Hannuksela) for initial review of non-3V HLS

- Interlaced scan and field-based video (0) Track X (section 6.5)

- Non-normative (2) Track X (section 6.6)

- Withdrawn and unclear category (7) (section 6.7)

- Plenary discussions and BoG reports (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Chroma format, CTC. (section 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: XX; Track A: XXX; Track B: XXX.

# AHG reports

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

M0001 / Opening plenary remarks.

Discussed verbally prior to availability.

All outputs of prior meeting delivered.

ITU-T approval completed 2013-04-XX. Text quality quite stable.

Editorial input to be submitted.

JCTVC-M0002 JCT-VC AHG report: HEVC Draft and Test Model editing (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs)]

Two editorial teams were formed to work on the two documents that were to be produced:

JCTVC-L1002 HEVC Test Model 10 (HM 10) Encoder Description

- Il-Koo Kim

- Ken McCann

- Kazuo Sugimoto

- Benjamin Bross

- Woo-Jin Han

JCTVC- L1003 HEVC text specification Draft 10 / FDIS & Consent text

- Benjamin Bross

- Woo-Jin Han

- Jens-Rainer Ohm

- Gary J. Sullivan

- Thomas Wiegand

Editing JCTVC-L1003 was assigned a higher priority than editing JCTVC-L1002.  The text of the final draft of JCTVC-K1003 (version 34) was submitted to ISO/IEC JTC1/SC29 for Final Draft International Standard ballot and to ITU-T SG16 for Consent.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of issues on the text of both documents.

Three successive versions of JCTVC-L1002 and 34 successive versions of JCTVC-L1003 were published by the Editing AHG following the 12th JCT-VC meeting.

The main changes in JCTVC-L1003, relative to the previous JCTVC-K1003, were listed in the AHG report.

The AHG recommended to:

- Approve the edited JCTVC-L1002 and JCTVC-L1003 documents as JCT-VC outputs. This was agreed.

- Encourage the use of the issue tracker to facilitate the reporting of issues with the text of either document.

- Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text.

JCTVC-M0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Suehring] [miss]

TBA by Sat/Sun

JCTVC-M0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan, C. Fogg]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The list of the candidate of the conformance bitstream and its volunteers are summarized in a table in the AHG report.

So far, 120 bitstreams had been collected. Most of them were updated to the HM10 syntax. However, 25 bitstreams are still based on older versions of the spec, e.g. HM9.1. Those bitstreams must be updated based on the final spec of the HEVC, as soon as possible. The problems were reported in 15 bitstreams for HM10 based bitstreams and those must be revised too.

The generated bitstreams are available at

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

The features and conformance point of each bitstream are summarized in an Excel sheet attached to the AHG report.

The MPEG parent body decided to standardize conformance and reference software as one single text. Therefore an integrated text was produced for that purpose (see M0327).

Some problem was identified in the reference software, and fixed in the HM10-dev branch.

An issue identified was how to have a consistent approach for providing MD5 checksums. It was suggested to produce guidelines to provide guidance on this matter.

JCTVC-M0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [C. Rosewarne]

Deferred.

JCTVC-M0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, C. Rosewarne, T. Suzuki]

Review deferred.

JCTVC-M0007 RExt Software development

Review deferred.

- JCTVC-M0008 JCT-VC AHG report: Screen content coding (AHG8) [H. Yu, M. Budagavi, B. Cohen, A. Duenas, T. Lin, J. Xu]

There were email discussions on RGB-to-YUV color format conversion for the screen content sequences, test conditions for RCE2, and general subjects of screen content coding.

We have a total of ten 4:4:4 screen content test sequences. Six of them came from Tongji Univ., three from Huawei & MERL, and one from BBC. All the sequences from Tongji Univ. and Huawei and MERL were captured in RGB with 8 bits per color component. The original YUV/YCbCr version of these 9 sequences was converted through different RGB-to-YUV color conversion methods, which resulted in confusion among the experts who were running tests with these sequences. It was also noted that unlike camera-captured content, screen content such as that generated by computer would typically use the full-amplitude range of 0 to 255 for R, G, and B. After some good discussions on the general topic of color format conversion, the group agreed to use the same calculation method given below in converting these screen capture RGB sequences to the YUV/YCbCr/ITU-709 color format:

t = 0.2126 * R + 0.7152 * G + 0.0722 * B;

Y = floor ( 16 + 219 * t / 255 + 0.5);

Cb = floor ( 128 + 112 * ( B − t )/ ( 255 * ( 1 − 0.0722 ) ) + 0.5);

Cr = floor ( 128 + 112 * ( R − t ) / ( 255 * ( 1 − 0.2126 ) ) + 0.5);

where t is a floating-point variable; R, G, B, Y, Cb, and Cr are integers; floor(x) is the largest integer of x, and single-precision floating-point processing is used in all calculations.

As a result of this discussion, six RGB sequences were re-converted to YUV and these new YUV files are now available for download at the server.

Per the last mandate of this ad hoc group, the test conditions for RCE2 were discussed and finalized jointly by the participants of RCE2 and AHG8. Below is a summary of the final test conditions:

- Total 20 test sequences: 10 screen content 4:4:4 RGB sequences; 4 ClassF sequences; 2 ClassB sequences (Kimono and Parkscene); 2 4:2:2 RExt sequences (EBUHorse and EBUWaterRocks); 2 4:4:4 YCbCr RExt sequences (BirdsInCage and EBURainFruits);

- Software: HM10.0-RExt2.0 (r3369) with a custom patch to use QP=0 in the tests.

- Coding structure: AI-Main, LB-Main, and RA-Main;

- Coding of RGB sequences: the RGB colour order was used in the input and output of the codec. However the RGB signal was re-ordered as GBR by the HM software for encoding and decoding.

Please see JCTVC-L1122 for the test descriptions and conditions in details.

Potential applications of screen content coding: wireless WiFi display, VDI, Remote desktop, etc.

The requirements of the relevant applications need clarification.

Comment from one participant on email reflector: “The new requirement is to compress discontinuous-tone contents that feature very sharp edges, uncomplicated shapes, and thin lines with few colors, even one-sample-wide single-color lines, etc. in subjectively lossless quality at ultra low bitrate with a compression ratio of 300:1 ~ 1000:1. The compression ratio of 300:1 to 1000:1 comes from the target to transfer the ultra-HD 1920x1200x60 fps resolution screen with a raw bit-rate of 1920x1200x60x24 = 3164Mbps over a transmission link of around 3-11Mbps reliable and guaranteed QoS bandwidth.”

Related contributions were identified. The "visually lossless" and "lossless" cases were noted.

A related MPEG ad hoc group on subjectively lossless screen content coding was noted to be having a meeting on Sunday morning April 21 (room 202, 0900–1300).

More coordination with parent bodies (MPEG AHG) necessary for application requirments: e.g. low delay for wireless displays; also bit rate constraints?

JCTVC-M0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) [M. M. Hannuksela, J. Boyce, Y.-K. Wang, T. Rusert, Y. Chen]

BoG (M. Hannuksela) activity was requested to sort out HL syntax contributions that only relate to SHVC; discuss those that relate to both SHVC and MV-HEVC jointly with JCT-3V.

Roughly 53 input contributions were identified as relating to AHG9.

JCTVC-M0010 JCT-VC AHG report: SHVC core experiments (AHG10) [X. Li, J. Boyce, P. Onno, Y. Ye]

Configurations for SHVC core experiments were discussed via JCT-VC main email reflector.

The configuration files of SHVC core experiments, the anchor data and the reporting sheets were released on Feb. 11th 2013.

It was suggested to include LD-B cases as mandatory tests via JCT-VC main email reflector, as this case has significantly better compression than LD-P.

In discussion of the AHG report, it was suggested to emphasize LB rather than LP in the future – except to use LP when testing interpolation filters (both for MC and spatial upsampling), as it may be easier to detect problems with the filtering when using LP. It was agreed to generally use LB in the future except in such cases where there is a rationale for using LP.

When generating the anchor of HM single layer coding and HM simulcast, both HM and SHM (with inter-layer prediction disabled) software may be used. However, the two software lead to slightly different bitstreams though identical PSNR. It is suggested to clearly define which software shall be used to generate the anchor for future meetings.

According to the MPEG “Requirements of the scalable enhancement of HEVC (w12956)”, different picture aspect ratios (PARs) across the layers shall be supported. The question on whether such cases shall be tested was raised via JCT-VC main email reflector. It was noted that some different resolution base layers could be made available for optional experiments.

JCTVC-M0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela]

The first version of JCTVC-L1007 (SHM 1) document was released on February 15, 2013. The syntax, semantics and decoding process of both reference index and textureBL frameworks were provided. An editor's update was released on March 27, 2013 to align with final version of HEVC base spec.

The first version of JCTVC-L1008 (SHVC WD 1) document, aligned with the MV-HEVC Draft Text 3 based on the decision made at 12th JCT-VC meeting in Geneva, was released on March 20, 2013. The document is generated by adding spatial and SNR scalable coding functionalities to MV-HEVC draft text document.

Some misalignments between text and software (e.g. upsampling filter phase) were reported.

The establishment of a bug tracking system for the SHVC work should be completed soon.

JCTVC-M0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, T.-D. Chuang, Y. He, D.-K. Kwon]

Software version SHM1.0 based on HM8.1 is used for experimentation and the most recent software version is SHM1.2 based on HM10. Alignment with MV-HEVC is to be discussed with JCT-3V.

The tools integrated into SHM1.0 were listed in the AHG report as follows:

- Chroma upsampling filter JCTVC-L0335

- Use cropped base layer picture with padding for inter-layer texture prediction JCTVC-L0178.

- IntraBL framework

- DST for 4x4 IntraBL JCTVC-L0067/ JCTVC-L0204

- cbf_root_flag JCTVC-L0437

- Motion hook enabled

- Intra hook disabled

- RefIdx framework

- Motion mapping JCTVC-L0336

- Bugfix JCTVC-L0167

- encoder-only zero MV JCTVC-L0051

- Encoder speedup method1 JCTVC-L0174

Experimental results of SHM1.0 against SMuC0.1.1 using SMuC0.1.1 common test conditions are summarized in tables in the AHG report.

SHM 1.1 is then based on SHM1.0 where base layer YUV and motion field metadata can be served as an input. It can be used for multi-standard scalability, when, for example, AVC codec is applied to code base layer. This version also includes following small fixes:

- Use cropped picture size for base layer MV scaling

- Use center pixels to get co-located base layer block

- Set the right picture size for the clipping

Experimental results of SHM1.1 against SHM1.0 using new common test conditions released by AHG10 are summarized in the AHG report.

SHM1.2 was developed by porting SHM1.1 on top of HM10 with the following high level syntax implementation:

- VPS extension

- RPL modification for including ILR picture into the RPL according to the common test conditions

- IDR alignment for an access unit

Experimental results of SHM1.2 against SHM1.0 using common test conditions released by AHG10 are summarized in the next two tables.

Coding results were reported to be generally as expected.

JCTVC-M0013 JCT-VC AHG report: SHVC upsampling and downsampling filters (AHG13) [Andrew Segall, Elena Alshina, Jianle Chen, Pankaj Topiwala]

There were approximately 10 identified input documents to the 12th meeting of the JCT-VC related to the mandates of the AhG.  They were listed in the AHG report.  Additionally, there were noted to be 37 input contributions related to SCE4, which also falls under the mandates of the AhG.

JCTVC-M0014 JCT-VC AHG report: Color Gamut Scalability (AHG14) [Andrew Segall, Alberto Duenas, D.-K. Kwon]

Two companies had informally indicated that they are collecting wide color gamut sequences for potential use by the JCTVC with the possibility of contributing the content between the 13th and 14th meeting.

There were noted to be three input documents (including one cross-check) to the meeting identified as related to the mandates of the AhG.

JCTVC-M0015 JCT-VC AHG report: SHVC hybrid codec scalability (AHG15) [J. Boyce, K. Kawamura]

The SHM 1.1 software release of March 12 supports an AVC base layer.

Reporting template and anchors for experiments using an AVC base layer were distributed on March 19.

Three contributions were noted to be related to the work of the AHG (M0076, M0254, and M0414).

JCTVC-M0016 JCT-VC AHG report: Single-Loop Scalability (AHG16) [M. Wien, J. Boyce, M. Budagavi, K. Mishra, K Ugur]

Two contributions were noted to be related to this AHG (M0176 and M0279).

For assessment and comparison of the complexity of Multi-Loop vs Single Loop decoding, the following measurements were performed (all BL+EL decoding)

- Entropy decoding

  - Count overall coded bins with context update

  - Count overall bypass coded bins

- Inter prediction

  - Count total number of pixels with full/half/quarter-sample MVs, uni- and bi-prediction

  - interBL prediction

- Intra prediction

  - Count number of intra predicted pixels

  - interBL

- Deblocking filtering

  - Count number of edge pixels with deblocking filter strength 1 or 2

- SAO

  - Count number of modified pixels

The results of this measurement are reported in JCTVC-M0176, with a cross-verification in JCTVC-M0279. The reported numbers shall provide a first insight on the complexity of multi/single loop decoding. Further complexity assessment work should be synchronized with AHG17.

Here, "single loop" means single-stage filtering and single inverse transform as well as single inter-prediction process.

JCTVC-M0017 JCT-VC AHG report: SHVC complexity assessment (AHG17) [M. Budagavi, E. Alshina, E. Francois, M. Karczewicz, A. Tabatabai, Y. Ye]

A complexity assessment module and spreadsheet was developed for use in SCE3 and SCE4 activity. L0440 provides the relevant patch and spreadsheet and also a brief description. The default complexity assessment template has been recently modified to correctly handle GRP (generalized residual prediction).

There are several non-SCE contributions that address complexity reduction and coding efficiency impact of the simplifications.

There were four email exchanges on JCTVC related to this AHG activity. These mainly dealt with issues that were discussed during the development of complexity assessment module.

During the development of the complexity assessment module, some issues were extensively discussed. These are highlighted below.

- Should up-sampling and MC operations/memory bandwidth be reported separately or should they be added and reported as a single number?

- What would be the scaler hardware architecture for SNR scalability in the current SHVC test model? 1) Would the scaler block be reused assuming scaling factor of 1 (i.e. integer pel positions in SNR case would be filtered with [... 0 0 1 0 0 ...] filter) or 2) Would BL picture pointer is used?

- What is the implication of JCTVC-L0178 cropping and padding adoption at last meeting on memory bandwidth for SNR scalability for RefIdx picture based processing? Can BL picture pointer be directly reused by EL decoder or will picture data copy of BL picture be needed to create reference picture for EL?

- Memory bandwidth for RefIdx with picture based processing is computed in the complexity assessment module assuming LCU by LCU up-sampling with LCU size = 64. What would be the optimal block size to use? Would it need to be matched with min LCU size? Currently, average complexity for RefIdx picture based approach is calculated using 64x64 block, whereas the worst case is calculated using 16x16 block.

- What scaling ratio do we consider for the worst spatial scalability case? Currently only ratios 2x and 1.5x are supported in SHM-1.0. But in the template, ratio 1x is used to compute the worst case numbers of spatial scalability (based on the hypothesis that in the future, SHVC will support any spatial ratio).

- Do we consider BL reference pictures as additional pictures to be stored in the EL DPB?

With regards to Issue 2, the following opinions were expressed by experts on the JCTVC email reflector:

- A scalar architecture would have input and output buffer plus interpolation logic. If EL and BL decoding is synchronized at frame-level in which BL reconstructed frame is normally in off-chip memory,  then the DMA could directly write the BL into the output buffer of the scaler in the SNR case. If such a capability does not exist for certain architecture, then a bypass mode should be added to the scaler logic so that the BL data (which is DMAed in to the scaler input buffer) can be copied from the input buffer to the output buffer of the scaler. If EL and BL decoding is synchronized at group of blocks level to reduce the bandwidth and latency for the EL decoding, the BL decoder should have capability to put the data directly in the input buffer of the scaler (avoiding going through off-chip memory). In the SNR case, the BL decoder could also write data directly into the output buffer of the scaler to bypass the scaler, or if such a capability does not exist, then a bypass mode should be added to the scaler logic to enable data copy for the input to the output buffer of the scaler. There is no memory bandwidth difference between the two options and whether a bypass mode is needed in the scaler purely depends on architecture choice.

- For SNR scalability, in EL one would not actually filter BL picture using a [0 0 1 0 0 ] filter.

- Frame buffer copy from BL to EL decoder would not be required. BL picture pointer can be passed instead.

The related contributions were identified in the report (M0086, M0088, M0192, several non-SCE contributions, and regarding item 3 above, M0180 and M0274).

# Project development, status, and guidance

## Communication to and by parent bodies

## Conformance test set development

JCTVC-M0327 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan] [late]

Detailed presentation not necessary. See notes for M0004.

## Version 1 bug reports

JCTVC-M0352 Report of issue in specifications of video and image transfer characteristics (gamma) functions [D. Singer, J. Zipnick, G. J. Sullivan]

## HEVC coding performance, implementation demonstrations and design analysis

### Coding performance

JCTVC-M0041 Updated results on HEVC still picture coding performance [K. Ugur, J. Lainema (Nokia)]

JCTVC-M0166 Comparison of Compression Performance of HEVC Draft 10 with AVC for UHD-1 material [P. Andrivon, P. Bordes, M. Arena, P. Sunna (??)]

JCTVC-M0234 Combined Scalable + Multiview HEVC coding results [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0346 Cross-check for Combined Scalable + Multiview HEVC coding results [E. Alshina, Y. Cho] [late]

JCTVC-M0329 Comparison of Compression Performance of HEVC Draft 10 with AVC High Profile [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

### Implementation demonstrations

### Design analysis

## Profile, level, and constraint definitions

JCTVC-M0094 Proposal of the profile/level for Range extensions [T. Suzuki, N. Saunders, S. Karl (Sony)] [late]

JCTVC-M0173 Removing a Level Restriction on Coding Tree Block Size [Eran Pinhasov, Shevach Riabtsev, Assaf Menachem]

## HEVC Use cases

JCTVC-M0172 Use cases and requirements for lossless and screen content coding [Thijs Vermeir (Barco)]

JCTVC-M0175 On the support for alpha channel in HEVC [M. Naccari, M. Mrak (BBC)]

JCTVC-M0190 Requirements for medical imaging applications [P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

JCTVC-M0255 Decoding a 10-bit HEVC sequence using an 8-bit decoder [D. Flynn, G. Martin-Cocher, D. He (RIM)]

JCTVC-M0315 AhG8: Requirements for wireless display applications [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-M0320 AHG8: More investigation on screen content coding [H. Yu (Huawei), X. Wang, J. Ye] [late]

JCTVC-M0429 Update on Cross-segment Decoding [Jiangtao Wen, Spencer Cheng (??)] [late] [miss]

## Source video test material

JCTVC-M0191 Medical imaging sequences for HEVC development [P. Amon, A. Hutter, U.-E. Martin (Siemens)]

JCTVC-M0431 YUV444 test sequences for screen content [Wenpeng Ding, Yunhui Shi, Baocai Yin (??)] [late]

# Core experiments in Range Extensions

## RCE1: Inter-component decorrelation methods

### RCE1 summary and general discussion

JCTVC-M0026 RCE1: Summary report of HEVC Range Extensions Core Experiment 1 on Inter component decorrelation methods [J. Sole, K. Kawamura, J. Kim]

### RCE1 primary contributions

JCTVC-M0048 RCE1: Adaptive Color Transforms for Range Extensions [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

JCTVC-M0384 RCE1: Crosscheck of JCTVC-M0048 on adaptive color transforms for range extensions [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0049 RCE1: Combined Chroma Tools For Range Extensions [P. Topiwala (FastVDO)], J. Kim (LGE), K. Kawamura (KDDI)]

JCTVC-M0097 RCE1: The performance of extended chroma mode for non 4:2:0 format [J. Kim (??)]

JCTVC-M0387 RCE1 Test 2.1: Cross-check of ‘In-loop color-space transformation of residual signals for range extensions’ by KDDI [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-M0410 RCE1: Results of Inter-plane intra coding of residual signals [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

JCTVC-M0411 RCE1: Results of in-loop color-space transformation of residual signals [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

### RCE1 cross checks

JCTVC-M0084 RCE1: Cross-verification of Test 1.1 - LM mode [S. Lee, C. Kim (Samsung)]

JCTVC-M0085 RCE1: Cross-verification of Test 1.2 - Inter-plane intra coding of residual signals (RM) [S. Lee, C. Kim (Samsung)]

JCTVC-M0367 RCE1: Cross-check of tests 3.1, 3.2 and 3.3 [J. Sole, W.-S. Kim (Qualcomm)] [late]

## RCE2: Intra prediction for lossless coding

### RCE2 summary and general discussion

JCTVC-M0027 RCE2: Summary report of HEVC Range Extensions Core Experiment 2 on Intra Prediction for Lossless Coding [W. Gao, M. Budagavi, P. Amon, S. Lee]

### RCE2 primary contributions

JCTVC-M0079 RCE2: Test 1 - Residual DPCM for HEVC lossless coding [S. Lee, I.-K. Kim, C. Kim (Samsung)]

JCTVC-M0056 RCE2: Experimental results on Test 3 and Test 4 [M. Zhou, M. Budagavi (TI)]

JCTVC-M0067 RCE2: Experimental Results for Test 5 [Y. H. Tan, C. Yeo (I2R)]

JCTVC-M0053 RCE2: Experimental results for Test 6 – combination of RDPCM and SWP for HEVC lossless coding [E. Wige (Universität Erlangen-Nürnberg), P. Amon (Siemens), S. Lee, I.-K. Kim, C. Kim (Samsung)]

JCTVC-M0052 RCE2: Sample-based weighted intra prediction for lossless coding [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

### RCE2 cross checks

JCTVC-M0317 RCE2: Cross-check of Test 1 (JCTVC-M0079) [W. Gao, J. Ye, H. Yu (??)] [late]

JCTVC-M0082 RCE2: Cross-verification of Test 2 - Sample-based Weighted Prediction [S. Lee, C. Kim (Samsung)]

JCTVC-M0083 RCE2: Cross-verification of Test 3 - Sample-based Angular Intra Prediction [S. Lee, C. Kim (Samsung)]

JCTVC-M0069 RCE2: Cross-check results for Test 4 [Y. H. Tan, C. Yeo (I2R)] [late]

JCTVC-M0318 RCE2: Cross-check of Test 5 (JCTVC-M0067) [W. Gao, J. Ye, H. Yu (??)] [late]

JCTVC-M0057 RCE2: Crosscheck of Test 6 (JCTVC-M0053) [M. Budagavi (TI)]

JCTVC-M0349 RCE2: Cross-check of Test 7 [R. Joshi, J. Sole (Qualcomm)] [late]

# Core experiments in SHVC

## SCE1: Intra prediction improvements

### SCE1 summary and general discussion

JCTVC-M0021 SCE1: Summary Report of Core Experiments on Intra Prediction Improvements in SHVC [A. Tabatabai, K. Rapaka, A. Saxena, S. Liu]

SCE 1.1 Intra Prediction Based on Reconstructed Base Layer

| Technique | Methods Description | Average of AI 2x and 1.5x |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | Enc | Dec |
| JCTVC-M0031 | DC Correction | −0.1% | 104% | 100% |
| JCTVC-M0095 | Unavailable Reference Samples Filling | −0.1% | 102% | 102% |
| JCTVC-L0036 JCTVC- L0099 | JCTVC-L0036 + JCTVC- L0099 | −0.2% | 106% | 103% |

No action on 1.1 category.

SCE 1.2 Intra Prediction Based on Differential Picture

| Technique | Methods Description | Average of AI 2x and 1.5x |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | Enc | Dec |
| JCTVC-M0032 | Gradient based intra prediction | −0.2% | 100% | 100% |
|  | JCTVC-L0036 + JCTVC-L0037 | −0.3% | 104% | 101% |
| JCTVC-M0324 | Difference Intra Prediction (JCTVC-L0222 + JCTVC-L0135) | −0.7% | 161% | 110% |
| JCTVC-L0140 / JCTVC-M0313 | Residual IntraBL+DC+ Planar | −0.3% | 98% | 107% |
| JCTVC-L0183 | Difference Intra Prediction | Withdrawn |  |  |
| JCTVC-L0140 + JCTVC-L0215 / JCTVC-M0306 | Residual Planar Intra Prediction (JCTVC-L0215 + JCTVC-L0222 + JCTVC-L0135) | −0.8% | 161% | 111% |
| JCTVC-M0331 | Difference Intra Prediction + NoIntraModefilteringof JCTVC-L0294 | −0.7% | 158% | 108% |
| JCTVC-M0328 | Simplified Two mode Difference Intra | −0.4% | 123% | 101% |

1.3: Inter-Layer Intra Mode prediction

| Technique | Methods Description | Average of AI 2x and 1.5x |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Y | Enc |  | Dec |
| JCTVC-M0282 | BL_NEIGHBOR | −0.2% | Timing not accurate |  |  |
|  | BL_NEIGHBOR MDCS_OFF | −0.1% | Timing not accurate |  |  |
| JCTVC-M0306 | L0239 with center coBL | 0.0% | 100% | 99% |  |
|  | L0239 with left_top coBL | 0.0% | 101% | 100% |  |
| JCTVC-M0326 | L0224 with parsing dependency | −0.2% | 101% | 100% |  |
|  | L0224 + MDCS_OFF no parsing dependency | −0.2% | 100% | 100% |  |
| JCTVC-M0194 | L0260(1) With parsing dependency | −0.2% | 101% | 100% |  |
|  | L0260(1) + MDCS_OFF no parsing dependency | −0.2% | 101% | 101% |  |
| JCTVC-L0260 (2) | center coBL intraBL for Neighbour | −0.2% | 103% | 100% |  |
|  | center coBL DC for Neighbour | −0.2% | 103% | 100% |  |
|  | left_top coBL intraBL for Neighbour | −0.2% | 102% | 100% |  |
|  | left_top coBL DC for Neighbour | −0.2% | 102% | 100% |  |
| JCTVC-M0158 | DIFF_INTRA_DIR_CODING | −0.2% | Timing not accurate |  |  |
|  | DIFF_INTRA_DIR_CODING with MDCS Off | −0.2% | Timing not accurate |  |  |
| JCTVC-M0063 | L0156 With parsing dependency | −0.2% | 102% | 100% |  |
|  | L0156 Without parsing dependency | −0.1% | 103% | 101% |  |
| JCTVC-M0103 | ILIntraMPMFix | −0.2% | 98% | 97% |  |
|  | L0113 +MDCS_OFF | 0.1% | 101% | 98% |  |
|  | MDCS Off+ ILIntraMPMFix | −0.1% | 100% | 98% |  |

SCE1 overall discussion:

The anchor here was IntraBL coding.

It was remarked that the techniques in this category (1.2 part) can actually provide gain even if the syntax is not changed, although there is more gain when the syntax is changed (to allow selection of either the existing or modified intra prediction modes).

Generally there seems to be less gain in 1.5x cases than 2.0x cases.

Overall the gains are not so large, and there is significant complexity impact. Revisit after looking at non-CE and inter cases.

### SCE1 primary contributions

JCTVC-M0031 SCE1: Results of test 1.1 on intra DC correction [J. Lainema, K. Ugur (Nokia)]

JCTVC-M0095 SCE1-1.1: Filling unavailable reference samples in intra prediction at enhancement layer [C. Kim, B. Jeon(LG)]

JCTVC-M0158 SCE1-1.3 Differential coding of enhancement layer intra mode [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-M0032 SCE1: Results of test 2.1 on gradient based intra prediction [J. Lainema, K. Ugur (Nokia)]

JCTVC-M0331 SCE1 Category 2.2 and 2.3: Difference Domain Intra Prediction [K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0324 SCE1 Category 2.2 and 2.6 : Difference domain Intra Prediction [K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm), J. Park, B. Jeon (LG), C. Auyeung, A. Tabatabai (Sony)]

JCTVC-M0328 SCE1 Category 2.3 : Simplified Difference Domain Intra Prediction [K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0306 SCE1 Category 2.4: Modifications of Prediction planar predictions in Difference Domain Intra Prediction [C. Auyeung, A. Tabatabai (Sony), J. Park, B. Jeon (LG), K. Rapaka, J. Cheun, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0313 SCE1 Category 2.4 and 2.5 : Modifications on textureRL, DC, planar predictions [J. Park, B. Jeon (LG), C. Auyeung, A. Tabltabai (Sony), K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0194 SCE1.3.1:Inter-Layer Intra Mode Prediction [M. Guo, S. Liu, S. Lei (MediaTek)]

JCTVC-M0326 SCE1 Category 3.3 and 3.2 : Inter-layer Mode prediction [K. Rapaka, J. Chen, V. Seregin, M. Karczewicz (Qualcomm), J. Kim, J. Park, B. Jeon (LG)

JCTVC-M0282 SCE1 3.5 Results of Inter-layer Intra MPM derivation for EL in SHVC [J. Xu, A. Tabatabai (Sony)]

JCTVC-M0100 SCE1 : 3.6 Inter-layer Intra Mode assignment for EL in SHVC [J. Kim (??)]

JCTVC-M0063 SCE1: Results of test 3.7 on inter-layer intra mode prediction [Z. Zhao, J. Ostermann (Leibniz Uni Hannover)]

JCTVC-M0103 SCE1: Results of test 3.8 on inter-layer intra mode prediction [J. Min, E. Alshina, T. Lee (Samsung)]

### SCE1 cross checks

JCTVC-M0121 SCE1: Cross-check result of test 1.1 on filling unavailable reference samples in intra prediction at enhancement layer (JCTVC-M0095) [H. Lee, J. Lee, J. W. Kang (ETRI)]

JCTVC-M0125 SCE1: Cross-check results of test 1.1 on intra DC correction (JCTVC-M0031) [C. Kim, B. Jeon (LG)]

JCTVC-M0244 SCE1: Cross check report for test 1.2.4 on residual intra planar prediction [M. Guo, S. Liu (MediaTek)] [late]

JCTVC-M0247 SCE1 : Cross-check of SCE1 2.4 [K. Rapaka (Qualcomm)] [late]

JCTVC-M0292 SCE1: Cross Check of SCE1 Category 2.4 and 2.5 (JCTVC-M0313) [S. Lu (Sony)]

JCTVC-M0421 SCE1: Crosscheck of 2.4 and 2.5 on Modification on textureRL, DC, Planar prediction (M0313) [J. Min, E. Alshina (Samsung)] [late]

JCTVC-M0064 SCE1: Crosscheck of Test 3.1 on inter-layer intra mode coding from MediaTek [Z. Zhao, Y. Liu (Leibniz Uni Hannover)]

JCTVC-M0245 SCE1 : Cross-check of SCE1 3.2 [K. Rapaka (Qualcomm)] [late]

JCTVC-M0243 SCE1: Cross check report for test 1.3.3 on inter-layer intra mode prediction [M. Guo, S. Liu (MediaTek)]

JCTVC-M0101 SCE1 : crosscheck for SCE1 3.4 [J. Kim (??)]

JCTVC-M0102 SCE1 : crosscheck for SCE1 3.5 [J. Kim (??)

JCTVC-M0280 SCE1: Cross-check of SCE1 3.6: Inter-layer Intra Mode assignment for EL in SHVC [J. Xu (Sony)]

JCTVC-M0422 SCE1: Crosscheck of test 4.2.2 and 4.2.6 on Difference Intra Prediction (M0324) [J. Min, E. Alshina (Samsung)] [late]

JCTVC-M0293 SCE1: Cross check of Leibniz Uni Hannover's proposal [S. Lu (Sony)]

JCTVC-M0296 SCE1 : crosscheck of Qualcomm's proposal [J. Park, B. Jeon (LG)] [late]

JCTVC-M0358 SCE1 : Crosscheck of simplified difference domain intra prediction (JCTVC-M0328) [J. Park, B. Jeon (LG)] [late]

JCTVC-M0159 SCE1 – Crosscheck of JCTVC-M0103: Inter layer intra mode prediction [D. Bugdayci, K. Ugur (Nokia)]

JCTVC-M0401 SCE1: Crosscheck of Test 4.2.2 and 4.2.3 on Difference Intra prediction by Qualcomm (M0331) [E. Alshina, J. Min (Samsung)] [late]

## SCE2: Inter-layer texture prediction signalling and deblocking

### SCE2 summary and general discussion

JCTVC-M0022 SCE2: Summary Report of SHVC Core Experiment on Inter-layer Texture Prediction Signaling and Deblocking [L. Guo, Y. He, D.-K. Kwon, J. Zan, J.-W. Kang]

SHVC Experiment 2 (SCE 2) includes two subtests which are related to inter-layer texture prediction. In subtest 2.1, methods related to inter-layer texture prediction flag signaling were investigated. In subtest 2.2, a method for BS setting of inter-layer texture prediction was evaluated. This report provides a summary of activities and results for this core experiment.

Subtest 2.1: Signaling for inter-layer texture prediction

| Test | Changes in Intra-BL flag coding |
| --- | --- |
| 2.1.1 (TI) | - In P and B slice, cu_skip_flag = 1 with an intraBL flag = 1 is used to indicate zero residue for IntraBL, and cu_skip_flag = 1 with intraBL flag = 0 indicates inter skip |
| 2.1.2 (TI) | - PU-level signaling - Conditionally adding Intra-BL prediction as a candidate in the merge list for non-2Nx2N PU |
| 2.1.3 (TI) | - Combination of 2.1.1 and 2.1.2 |

|  | Avg. LD-P/RA 2x |  |  | Avg. LD-P/RA 1.5x |  |  | Avg. LD-P/RA SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 2.1.1 | −0.1% | 0.1% | 0.1% | −0.2% | 0.2% | 0.1% | −0.0% | −0.0% | −0.0% |
| 2.1.2 | −0.3% | 0.0% | 0.1% | −0.3% | −0.2% | −0.0% | −0.2% | 0.1% | 0.3% |
| 2.1.3 | −0.4% | 0.1% | 0.2% | −0.4% | 0.0% | 0.1% | −0.3% | 0.1% | 0.3% |

|  | LD-B 2X |  |  | LD-B 1.5x |  |  | LD-B SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 2.1.1 | −0.1% | 0.1% | 0.1% | −0.3% | 0.3% | 0.3% | −0.1% | −0.0% | 0.0% |
| 2.1.2 | −0.3% | −0.1% | −0.1% | −0.3% | −0.3% | −0.2% | −0.2% | −0.2% | 0.1% |
| 2.1.3 | −0.4% | 0.0% | 0.0% | −0.6% | −0.2% | −0.0% | −0.3% | −0.2% | 0.1% |

The anchor here was IntraBL coding with CU-level selection of inter-layer texture prediction flag.

It was remarked that one difference between the prior proposed IntraBL and RefIdx approaches is that RefIdx operates at the PU level, whereas IntraBL has operated at the CU level. Here, the 2.1.3 approach provides a small gain (0.4%) with some added complexity relative to IntraBL, but in some sense there is a similar gain embedded in the RefIdx approach.

The suggestion is to keep in mind the 0.4% gain opportunity that can strengthen IntraBL when comparing it to RefIdx.

Subtests 2.2: Deblocking for inter-layer texture prediction

| Test |  |
| --- | --- |
| 2.2 (LG && Qualcomm) JCTVC-M0093 | - BS =1 for Intra-BL CU - Deblocking chroma for both Intra and Intra-BL |

|  | AI 2X |  |  | AI 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | Y | U | V |  |  |  |
| 2.2 | −0.1% | −0.2% | −0.2% | −0.1% | −0.3% | −0.1% |  |  |  |
|  | Avg. LD-P/RA 2x |  |  | Avg. LD-P/RA 1.5x |  |  | Avg. LD-P/RA SNR |  |  |
| Test | Y | U | V | Y | U | V | Y | U | V |
| 2.2 | 0.0% | −0.2% | −0.2% | −0.1% | −0.3% | −0.4% | 0.0% | −0.4% | −0.4% |

This has some increase in the complexity of the deblocking filter process.

The potential visual impact would be more important than the PSNR impact.

Does not seem likely to have significant benefit.

### SCE2 primary contributions

JCTVC-M0216 SCE2: Results of test 1.1 on skipped inter-layer texture prediction (ILTP) block signaling in Inter slice [D.-K Kwon, M. Budagavi (TI)]

JCTVC-M0217 SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0093 SCE2.2: Deblocking boundary strength modification for Intra BL [C. Kim, B. Jeon (LG), L. Guo, G. Van Der Auwera, J. Chen, M. Karczewicz (Qualcomm)]

### SCE2 cross checks

JCTVC-M0341 SCE2: Cross-check for test 1.1 on skipped inter-layer texture prediction (ILTP) block signaling in Inter slice [E. Alshina] [late]

JCTVC-M0054 Cross-check for SCE2: Results of test 1.2 and test 1.3 on inter-layer texture prediction (JCTVC-M0217) [Z. Ma, F. Fernandes] [late]

JCTVC-M0174 SCE2 Cross-check of CE2 Sub-test 2 [A. Norkin (Ericsson)] [late]

JCTVC-M0399 SCE1: Cross-check of SCE1 2.1 on gradient based intra prediction [K. Rapaka (Qualcomm)] [late]

## SCE3: Combined inter- and interlayer prediction

### SCE3 summary and general discussion

JCTVC-M0023 SCE3: Summary Report of SHVC Core Experiment on Combined Inter- and Interlayer Prediction [X. Li, E. Francois, P. Lai, D.-K. Kwon, A. Saxena]

Overview

| Test | Document | Crosschecking | Short description |
| --- | --- | --- | --- |
| 3.1 | JCTVC-M0119 (MediaTek) | Samsung (JCTVC-M0034) | Adaptive predictor compensation |
| 3.2 | JCTVC-M0294 (Qualcomm) | JCTVC-M0122 (ETRI) | Combined inter mode |
| 3.3 | JCTVC-M0260 (Qualcomm, Nokia, Canon) | JCTVC-M0060 (Intel) JCTVC-M0339 (Samsung) JCTVC-M0177 (RWTH-Aachen) JCTVC-M0108 (Huawei) | Generalized residual prediction |
| 3.4 | JCTVC-M0221 (MediaTek) | JCTVC-M0299 (LG) JCTVC-M0236 (Qualcomm) | Generalized combined and residue prediction |
| 3.5 | JCTVC-M0109 (Canon) | JCTVC-M0077 (Sharp) | Generalized residual prediction with MC at BL |
| 3.6 | JCTVC-M0073 (Sharp) | JCTVC-M0145 (Sony) | Generalized residual prediction with shorter MC filter |
| 3.7 | JCTVC-M0251 (LG, Vidyo) | JCTVC-M0394 (MediaTek) | Difference domain inter prediction |
| 3.8 | Withdrawn |  | RefIdx based differential coding |
| 3.9 | JCTVC-M0110 (Canon) | JCTVC-M0237 (Qualcomm) | Base Mode with Residual Prediction |

Results

| Test |  | Case | Aver BD-R Y | Config. | BD-R Y | BD-R U | BD-R V | Enc T. | Dec T. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | Adaptive predictor compensation | Case 1: medium complexity | -1.1% | RA | -0.5% | -2.7% | -3.2% | 104% | 100% |
|  |  |  |  | LD-P | -1.7% | -4.7% | -5.5% | 104% | 99% |
|  |  |  |  | LD-B | -0.6% | -2.8% | -3.2% | 103% | 100% |
| 3.2 | Combined inter mode | Case 1 | -1.1% | RA | -0.5% | -3.1% | -3.7% | 109% | 94% |
|  |  |  |  | LD-P | -1.7% | -5.6% | -6.5% | 112% | 93% |
|  |  |  |  | LD-B |  |  |  |  |  |
| 3.3 | Generalized residual prediction | Case 1: 3 weights, bi-linear interp., no GRP on chroma | -2.9% | RA | -1.7% | -5.5% | -6.5% | 119% | 100% |
|  |  |  |  | LD-P | -4.1% | -7.7% | -8.7% | 125% | 99% |
|  |  |  |  | LD-B | -2.7% | -6.7% | -7.6% | 116% | 101% |
|  |  | Case 2: 3 weights, bi-linear interp., 4-tap up-sample., block size constraint | -3.7% | RA | -2.0% | -3.7% | -3.9% | 119% | 126% |
|  |  |  |  | LD-P | -5.3% | -5.7% | -5.1% | 125% | 128% |
|  |  |  |  | LD-B | -3.1% | -4.6% | -4.5% | 116% | 127% |
|  |  | Case 3: 2 weights, bi-linear interp., 4-tap up-sample., block size constraint | -3.4% | RA | -2.0% | -3.9% | -4.3% | 114% | 126% |
|  |  |  |  | LD-P | -4.8% | -5.4% | -5.0% | 119% | 128% |
|  |  |  |  | LD-B | -2.9% | -4.7% | -4.7% | 111% | 127% |
|  |  | Case 4: 3 weights, bi-linear interp., 4-tap up-sample., block size constraint, no GRP on chroma | -3.2% | RA | -1.6% | -4.1% | -5.1% | 120% | 126% |
|  |  |  |  | LD-P | -4.8% | -6.7% | -8.0% | 126% | 127% |
|  |  |  |  | LD-B | -2.6% | -5.1% | -6.0% | 118% | 126% |
| 3.4 | Generalized combined and residue prediction | Case 1: Test1-3GCP On all partition sizes | -3.5% | RA | -2.4% | -4.5% | -4.8% | 127% | 107% |
|  |  |  |  | LD-P | -4.6% | -4.6% | -4.0% | 131% | 106% |
|  |  |  |  | LD-B | -4.4% | -6.0% | -5.7% | 124% | 111% |
|  |  | Case 2: Test2-3GCP On 2Nx2N only | -3.2% | RA | -2.2% | -4.1% | -4.4% | 121% | 108% |
|  |  |  |  | LD-P | -4.2% | -4.2% | -3.5% | 123% | 106% |
|  |  |  |  | LD-B | -4.1% | -5.4% | -5.1% | 118% | 111% |
| 3.5 | Generalized residual prediction with MC at BL | Case 1: One weighting mode (1) | -1.9% | RA | -1.5% | -3.4% | -3.8% | 111% | 106% |
|  |  |  |  | LD-P | -2.2% | -2.5% | -2.5% | 114% | 106% |
|  |  |  |  | LD-B | -2.3% | -3.2% | -3.3% | 111% | 106% |
|  |  | Case 2: Two weighting modes (0.5, 1) | -2.6% | RA | -1.6% | -3.5% | -4.0% | 119% | 104% |
|  |  |  |  | LD-P | -3.6% | -3.3% | -2.8% | 122% | 105% |
|  |  |  |  | LD-B | -2.6% | -3.6% | -3.7% | 119% | 106% |
| 3.6 | Generalized residual prediction with shorter MC filter | Case 1: | -2.6% | RA | -2.4% | -3.9% | -4.4% | 120% | 105% |
|  |  |  |  | LD-P | -2.8% | -3.3% | -3.3% | 117% | 104% |
|  |  |  |  | LD-B | -3.6% | -4.4% | -4.7% | 115% | 113% |
| 3.7 | Difference domain inter prediction | Case 1: Default | -2.1% | RA | -1.9% | -4.0% | -4.6% | 158% | 106% |
|  |  |  |  | LD-P | -2.2% | -3.3% | -3.5% | 154% | 107% |
|  |  |  |  | LD-B |  |  |  |  |  |
|  |  | Case 2: Default + bilinear interp. | -2.7% | RA | -2.1% | -4.3% | -4.7% | 158% | 107% |
|  |  |  |  | LD-P | -3.3% | -4.5% | -4.6% | 154% | 108% |
|  |  |  |  | LD-B |  |  |  |  |  |
|  |  | Case 3: Default + weighting 0.5 | -2.0% | RA | -1.3% | -1.9% | -1.9% | 155% | 107% |
|  |  |  |  | LD-P | -2.7% | -1.3% | -1.1% | 149% | 109% |
|  |  |  |  | LD-B |  |  |  |  |  |
|  |  | Case 4: Default + bilinear interp. + weighting 0.5 | -1.8% | RA | -1.2% | -2.0% | -1.9% | 155% | 107% |
|  |  |  |  | LD-P | -2.5% | -1.5% | -1.4% | 149% | 109% |
|  |  |  |  | LD-B |  |  |  |  |  |
| 3.9 | Base Mode with Residual Prediction | Case 1: Base mode with GRP 8x8 | -1.7% | RA | -1.3% | -1.9% | -2.4% | 103% | 107% |
|  |  |  |  | LD-P | -2.0% | -1.7% | -1.7% | 103% | 109% |
|  |  |  |  | LD-B | -1.8% | -1.7% | -1.8% |  |  |
|  |  | Case 2: Base mode with GRP 16x16 | -1.3% | RA | -1.1% | -2.1% | -2.4% | 102% | 106% |
|  |  |  |  | LD-P | -1.6% | -1.9% | -1.8% | 102% | 106% |
|  |  |  |  | LD-B | -1.4% | -2.1% | -2.1% |  |  |

Worst case enhancement-layer and upsampling complexity compared to single-layer high-res decoder

| Test | Case | Mul | Adds | MemBand (4x2) | MemBand (8x2) | Num Ref in Pred | Size of Look-up Tab | Add Pic Buffer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | Case 1 | 145% | 145% | 133% | 133% | 100% | 100% | 0% |
| 3.2 | Case 1 | 145% | 145% | 133% | 133% | 100% | 100% | 0% |
| 3.3 | Case 1 | 110% | 111% | 100% | 98% | 180% | 113% | 0% |
|  | Case 2 | 76% | 67% | 108% | 106% | 180% | 146% | 0% |
|  | Case 3 | 76% | 67% | 108% | 106% | 180% | 146% | 0% |
|  | Case 4 | 56% | 49% | 70% | 68% | 180% | 146% | 0% |
| 3.4 | Case 1 | 486% | 495% | 383% | 344% | 180% | 100% | 0% |
|  | Case 2 | 397% | 405% | 333% | 267% | 180% | 100% | 0% |
| 3.5 | Case 1 | 115% | 113% | 126% 99% spatial 3/2 | 126% 100% spatial 3/2 | 180% | 200% | 0% |
|  | Case 2 | 115% | 113% | 126% 99% spatial 3/2 | 126% 100% spatial 3/2 | 180% | 200% | 0% |
| 3.6 | Case 1 | 286% (B) 211% (P) | 295% (B) 221% (P) | 267% (B) 261% (P) | 267% (B) 233% (P) | 180% | 121% | 0% |
| 3.7 | Case 1 | 486% | 495% | 383% | 344% | 180% | 100% | 0% |
|  | Case 2 | 200% | 197% | 213% | 219% | 180% | 113% | 0% |
|  | Case 3 | 486% | 495% | 383% | 344% | 180% | 100% | 0% |
|  | Case 4 | 200% | 197% | 213% | 219% | 180% | 113% | 0% |
| 3.9 | Case 1 | 200% | 197% | 213% 179% spatial 3/2 | 219% 185% spatial 3/2 | 180% | 108% | 0% |
|  | Case 2 | 133% | 131% | 137% 116% spatial 3/2 | 137% 97% spatial 3/2 | 180% | 108% | 0% |

3.2: cascaded bi-pred (averaging of EL bipred with upsampled base)

3.3: Lower than HEVC simulcast in worst case, since bilinear interpolation is used for all EL motion comp (and also for the additional motion comp. in computing the residual prediction). On average, computations and memory accesses are still higher as shown in subsequent table.

Average complexity increase compared to SHM1

| Test | Case | Config. | 8b/8b | 64b/256b | 64b/512b | Mults | Adds |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | Case 1: medium complexity | RA | 103% | 103% | 103% | 99% | 99% |
|  |  | LD-P | 107% | 105% | 105% | 99% | 100% |
|  |  | LD-B | 105% | 104% | 104% | 98% | 99% |
| 3.2 | Case 1 | RA | 104% | 103% | 103% | 103% | 104% |
|  |  | LD-P | 109% | 107% | 107% | 109% | 111% |
|  |  | LD-B | 106% | 105% | 105% | 105% | 106% |
| 3.3 | Case 1: 3 weights, bi-linear interp., no GRP on chroma | RA | 112% | 111% | 112% | 112% | 112% |
|  |  | LD-P | 121% | 118% | 119% | 122% | 123% |
|  |  | LD-B | 119% | 116% | 118% | 120% | 120% |
|  | Case 2: 3 weights, bi-linear interp., 4-tap up-sample., block size constraint | RA | 120% | 120% | 120% | 114% | 111% |
|  |  | LD-P | 140% | 143% | 143% | 130% | 125% |
|  |  | LD-B | 130% | 130% | 130% | 121% | 117% |
|  | Case 3: 2 weights, bi-linear interp., 4-tap up-sample., block size constraint | RA | 118% | 119% | 118% | 113% | 111% |
|  |  | LD-P | 138% | 143% | 143% | 130% | 125% |
|  |  | LD-B | 127% | 128% | 128% | 120% | 117% |
|  | Case 4: 3 weights, bi-linear interp., 4-tap up-sample., block size constraint, no GRP on chroma | RA | 110% | 109% | 109% | 105% | 104% |
|  |  | LD-P | 120% | 119% | 120% | 113% | 109% |
|  |  | LD-B | 114% | 112% | 113% | 108% | 105% |
| 3.4 | Case 1: GCP for all blocks | RA | 147% | 148% | 149% | 161% | 165% |
|  |  | LD-P | 171% | 172% | 173% | 198% | 203% |
|  |  | LD-B | 179% | 180% | 181% | 209% | 215% |
|  | Case 2: GCP only for 2Nx2N blocks | RA | 147% | 148% | 149% | 161% | 165% |
|  |  | LD-P | 171% | 171% | 172% | 197% | 202% |
|  |  | LD-B | 180% | 181% | 183% | 210% | 216% |
| 3.5 | Case 1: One weighting mode (1) | RA | 129% | 132% | 133% | 118% | 121% |
|  |  | LD-P | 129% | 132% | 132% | 115% | 118% |
|  |  | LD-B | 140% | 144% | 145% | 125% | 128% |
|  | Case 2: Two weighting modes (0.5, 1) | RA | 132% | 136% | 137% | 117% | 119% |
|  |  | LD-P | 141% | 146% | 146% | 123% | 127% |
|  |  | LD-B | 145% | 150% | 152% | 123% | 127% |
| 3.6 | Case 1: | RA | 136% | 139% | 140% | 129% | 130% |
|  |  | LD-P | 155% | 158% | 159% | 148% | 151% |
|  |  | LD-B | 160% | 164% | 166% | 149% | 150% |
| 3.7 | Case 1: Default | RA | 133% | 133% | 133% | 114% | 116% |
|  |  | LD-P | 138% | 138% | 138% | 118% | 121% |
|  |  | LD-B |  |  |  |  |  |
|  | Case 2: Default + bilinear interp. | RA | 134% | 135% | 136% | 107% | 108% |
|  |  | LD-P | 148% | 148% | 150% | 114% | 115% |
|  |  | LD-B |  |  |  |  |  |
|  | Case 3: Default + weighting 0.5 | RA | 129% | 129% | 130% | 110% | 112% |
|  |  | LD-P | 146% | 144% | 145% | 116% | 119% |
|  |  | LD-B |  |  |  |  |  |
|  | Case 4: Default + bilinear interp. + weighting 0.5 | RA | 127% | 128% | 129% | 105% | 105% |
|  |  | LD-P | 142% | 140% | 142% | 108% | 109% |
|  |  | LD-B |  |  |  |  |  |
| 3.9 | Case 1: Base mode with GRP 8x8 | RA | 161% | 204% | 199% | 128% | 132% |
|  |  | LD-P | 179% | 229% | 222% | 139% | 145% |
|  |  | LD-B | 186% | 241% | 234% | 143% | 150% |
|  | Case 2: Base mode with GRP 16x16 | RA | 131% | 142% | 151% | 121% | 124% |
|  |  | LD-P | 137% | 148% | 159% | 127% | 131% |
|  |  | LD-B | 142% | 156% | 168% | 131% | 135% |

It was discussed how to interpret average number of computations/memory access – opinions expressed that it is related to power consumption. Worst case number is related to systems requirements in terms of computation and memory access.

The numbers reported in SCE3 only count the numbers of operation for motion comp and upsampling (which is different from the overall numbers reported in the context of the AHG).

The methodology for complexity assessment needs further improvements to make it more consistent across CEs (revisit).

Conclusion: Though some promising compression gains are observed, none of the methods investigated in SCE3 is attractive for adoption (regarding compression benefit vs. complexity), but further reduction of complexity is expected to be reported from non-CE contributions. It would be desirable to not further increase (or rather decrease) the overall computational complexity and memory bandwidth of SHM.

### SCE3 primary contributions

JCTVC-M0119 SCE3.1: Adaptive predictor compensation [T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei (MediaTek)]

Not planned for further CE

JCTVC-M0294 SCE3: Combined inter mode (test 3.2) [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

not planned for further CE

JCTVC-M0260 SCE3: Results of Test 3.3 on Generalized Residual Prediction [X. Li, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm), J. Lainema, K. Ugur (Nokia), C. Gisquet, F. Le Léannec, J. Taquet, E. François, G. Laroche, P. Onno (Canon)]

not planned for further CE

JCTVC-M0221 SCE3.4 Generalized Combined Prediction [P. Lai, S. Liu, T.-D. Chuang, Y.-W.Huang, S. Lei (MediaTek)]

not planned for further CE

JCTVC-M0109 SCE3: Experiment 3.5 on Simplification of Generalized Residual Inter-Layer Prediction for spatial scalability [E. François, J. Taquet, C. Gisquet, G. Laroche, P. Onno (Canon)]

Was presented. Combines motion comp and upsampling in computing the prediction of the residual. Compared to previous version that first did upsampling and then motion comp, the loss is about 0.2%. May not be combinable with arbitrary upsampling ratios.

Proponents suggest further investigation in CE

JCTVC-M0073 SCE3: Results of test 3.6 on Generalized Residual Prediction with shorter-tap MC filter [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution reports coding efficiency and complexity assessment results of SCE3.6: Generalized Residual Prediction with shorter-tap interpolation filter, where 2 or 4 tap filter is used depending on the slice type and colour component. The method is the same as JCTVC-L0265 and implemented on SHM1.0 with TE3 4.6.3(JCTVC-L0038) software. It is reported that the BD-rate (EL+BL) changes compared to SHM1.0 are -2.1%, -3.3%, -1.8%, -2.5%, -3.7%, -2.2%, -3.3%, -4.6% and -3.0% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR , LB 2x, LB 1.5x and LB SNR cases respectively.

Proponents suggest further investigation in CE in combination with other simplified GRP methods.

JCTVC-M0251 SCE3: 3.7 Inter prediction based on difference picture [W. Jang, J. Boyce, A. Abbas (Vidyo), J. Park, B. Jeon (LG)]

not planned for further CE

JCTVC-M0394 SCE3: Crosscheck of JCTVC-M0251 on result of SCE3.7 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0110 SCE3: Experiment 3.9 on Base Mode with Generalized Residual Prediction [E. François, J. Taquet, C. Gisquet, G. Laroche, P. Onno (Canon)]

This document presents the implementation of the Base Mode using Generalized Residual Prediction (GRP). In the Base Mode, the syntax of each NxN block of a CU is derived from the collocated BL CU. It is reported that the experiments have shown that using systematically GRP, instead of a switchable solution, provides the best coding performance. Average BDR gains of 1.3% Y, 1.9% U and 2.8% V for RA, of 2.0% Y, 1.7% U and 1.6% V for LDP, and of 1.9% Y, 1.7% U and 1.8 V for LDB are reported when N=8. Average BDR gains of 1.1% Y, 2.1% U and 2.4% V for RA, of 1.6% Y, 1.9% U and 1.8% V for LP, and of 1.4% Y, 2.1% U and 2.1% V for LDB are reported when N=16. The encoding and decoding runtime are reportedly slightly increased (by 2-3%). It is reported that the worst case complexity of the proposed tool (evaluated according to the methodology described in JCTVC-L0440) is in the same order as the SHM worst case complexity for the N=16 version, while for the N=8 version, multiplications, and memory access bandwidths worst cases are increased by around 60-65%.

Was presented.

According to the last of the 4 tables from the summary report, average complexity is clearly worse than SHM.

Proponents suggest further investigation in CE, provided that base mode is further considered.

### SCE3 cross checks

JCTVC-M0034 SCE 3: Cross-Check of test 3.1 (M-0119) in SCE 3 [A. Saxena, F. Fernandes (Samsung)]

JCTVC-M0122 SCE3: Cross-check result of test 3.2 on combined inter mode [H. Lee, J. Lee, J. W. Kang (ETRI)] [late]

JCTVC-M0108 SCE3: Cross-check results for Test 3.3 [X. Wei, J. Zan (Huawei)]

JCTVC-M0177 SCE3: Crosscheck of test 3.3 [Christian Feldmann, Mathias Wien (RWTH Aachen University)] [late]

JCTVC-M0339 SCE3: cross-check for SCE3: Results of Test 3.3 on Generalized Residual Prediction (case 2) [E. Alshina, A. Alshin] [late]

JCTVC-M0236 SCE3: Crosscheck of SCE 3.4 [X. Li (Qualcomm)] [late]

JCTVC-M0077 Cross-check on SCE3.5: Simplification of Generalized Residual Inter-Layer Prediction for spatial scalability [T. Tsukuba (Sharp)]

JCTVC-M0299 SCE3 : crosscheck of SCE3.5 GCP [J. Park, B. Jeon (LG)	] [late]

JCTVC-M0145 SCE3: Crosscheck of SCE3.3.6 [K. Sato(Sony)]

JCTVC-M0394 SCE3: Crosscheck of JCTVC-M0251 on result of SCE3.7 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-M0237 SCE3: Crosscheck of SCE 3.9 [X. Li (Qualcomm)] [late]

JCTVC-M0060 Cross check for SCE3 [W. Zhang, Y. Chiu (Intel)] [late]

## SCE4: Inter-layer filtering

### SCE4 summary and general discussion

JCTVC-M0024 SCE4: Summary Report of SHVC Core Experiment on inter-layer filtering [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park]

| Test | Technique Summary |
| --- | --- |
| 4.1.1 JCTVC-M0087 | - Low pass smoothing filter on integer luma samples Note: 5 taps filter is used instead of originally proposed 7 tap, and there was also some additional difference from what was planned. It was remarked that the proposal was substantially different than what was planned for the CE. - Only tested for SNR scalability - TextureRL framework: CU level on/off - RefIdx framework: always enabled |
| 4.1.2 JCTVC-M0058 | - Fixed 5x5 cross with a 3x3 square 2D non-separable filter - Low pass smoothing filter on integer luma samples - TextureRL framework: CU level on/off - Only apply to SNR scalability |
| 4.2.1 JCTVC-M0265 | - Picture level SAO - SAO type is derived from the high frequency component of the reconstructed samples - Offset of two SAO types are signaled and added to the reconstructed pixels simultaneously - SAO parameters are coded in the slice header - It was remarked that the original CE plan only applied this to luma, but the tested variant actually applied it to both luma and chroma. |
| 4.2.2 JCTVC-M0267 | - The adaptive up-sampling filter has same length, coefficient accuracy with the existing up-sampling filter - Filtering process is also applied to integer pixel position - Filter parameters are signaled at picture level and switchable at picture level |
| 4.2.3 JCTVC-M0195 | - Adaptive 5x5 cross with a 3x3 square 2D non-separable filter - Apply to reconstructed base layer picture before up-sampling process - Filter parameters are signaled at slice header - TextureRL framework: CU level on/off |
| 4.2.4 JCTVC-M0183 | - Enhance the chroma samples by using the surrounding luma samples - Apply to up-sampled base layer picture - Adding a offset to the chroma sample, the offset is obtained by using an adaptive high pass 4x3 filter with luma samples as input - Filter coefficients are derived at encoder side for each chroma plane of a picture - Picture level on/off, filter parameters are signaled at slice header |
| 4.2.5 JCTVC-M0055 | - 3x3 bilateral filter applies to the up-sampled base layer picture - Each filter coefficient (i = 0..8) is derived as follows - a weight w1[i] determined by the spatial position and derived by lookup table - a weight w2[i] derived by lookup table by using the sample value absolute difference between the current sample and the supporting sample - w[i] = w1[i]*w2[i] - Different table is trained for difference spatial scalability - Division is used as the final normalization - A lookup table method is additionally proposed to replace the final division operation |
| 4.2.6 JCTVC-M0213 | - 5x5 bilateral filter applies to the upsampled base layer picture - Each filter coefficient (i = 0..24) is derived as follows - a weight w1[i] determined by the spatial position and derived by lookup table - a weight w2[i] derived by lookup table by using the sample value absolute difference between the current sample and the supporting sample - w[i] = w1[i]*w2[i] - Division is used as the final normalization |

| Test |  | All Intra (2x, 1.5x) |  |  |  | RA, LD-P (2x, 1.5x) |  |  |  | RA, LD-P (SNR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | Cr&Cb | EncT | DecT | Y | Cr&Cb | EncT | DecT | Y | Cr&Cb | EncT | DecT |
| 4.1.1 | IBL |  |  |  |  |  |  |  |  | −2.1% | 0.0% | 102% | 103% |
|  | RefIdx |  |  |  |  |  |  |  |  | −1.4% | −0.7% | 100% | 127% |
| 4.1.2 | IBL |  |  |  |  |  |  |  |  | −2.1% | −0.2% | 103% | 114% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.1 | IBL | −0.2% | −0.5% | 99% | 113% | −0.6% | −1.1% | 99% | 129% | −1.8% | −1.2% | 99% | 129% |
|  | RefIdx | −0.2% | −0.4% | 101% | 116% | −0.6% | −0.9% | 100% | 131% | −1.8% | −1.5% | 100% | 130% |
| 4.2.2 | IBL | −0.2% | −0.6% | 115% | 107% | −0.5% | −0.6% | 106% | 102% | −2.3% | −0.9% | 106% | 129% |
|  | RefIdx | −0.2% | −0.5% | 102% | 104% | −0.5% | −0.6% | 103% | 105% | −2.3% | −1.3% | 100% | 133% |
| 4.2.3 | IBL | −0.6% | −0.2% | 110% | 99% | −0.5% | 0.1% | 103% | 81% | −2.4% | 0.1% | 103% | 101% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.4 | IBL | −0.8% | −7.6% | 102% | 105% | −0.2% | −8.2% | 101% | 108% | −0.3% | −6.2% | 100% | 109% |
|  | RefIdx | −0.8% | −8.3% | 104% | 105% | −0.3% | −8.7% | 101% | 109% | −0.3% | −6.8% | 101% | 109% |
| 4.2.5 | IBL | −0.5% | −0.9% | 112% | 117% | −0.6% | −0.7% | 104% | 107% | −0.7% | −0.6% | 103% | 107% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2.6 | IBL | −1.3% | −1.1% | 121% | 155% | −0.9% | −0.8% | 106% | 124% | −0.8% | −0.8% | 109% | 131% |
|  | RefIdx |  |  |  |  |  |  |  |  |  |  |  |  |

It was noted that the table reports only the average gain for 1.5x and 2x spatial scalability as a single number, which causes a loss of information since some techniques provide more gain in one of these cases than in the other.

The most gain is shown in SNR scalability cases.

The 4.2.1 SAO case is interesting but does not seem mature.

For 4.2.3, the non-fixed, non-separable operation does not seem desirable as-is. (Separable was not tested.) It was asked whether it was worth considering separable but non-fixed filtering. In the absence of some approach that is different in some other way, this seems unlikely to provide enough benefit to be desirable.

For SNR scalability, the fixed filters seem OK.

4.1.2 is non-separable, whereas 4.1.1 is separable, so 4.1.2 does not seem justifiable.

4.1.1 still on the table (only applies to SNR scalability). It was remarked that this has a significant relationship with pre-processing. Further discussion of 4.1.1 deferred to include review of non-CE crelated contributions.

4.2.2 is an adaptively-signalled upsampling filter (rather than fixed as in 4.1.1).

It was remarked that adaptively-signalled values may require (not yet proposed) normative encoding constraints or would have a dynamic range problem.

It was questioned how important the SNR scalability case really is for a multi-loop scalability design.

Regarding 4.2.4 (inter-component filtering using luma samples to enhance chroma). Memory bandwidth in the worst case was discussed and was reportedly manageable (e.g. in 5–15% increase range or less). Picture-based vs. block-based upsampling was discussed. A related non-CE contribution is also submitted. Revisit of this is recommended.

Regarding 4.2.5 and 4.2.6, these are bilateral filters with differing regions of support (3x3 and 5x5) and CU-level on-off switching. They provide more substantial gains than most. A related non-CE proposal (3x1 separable) has also been submitted. Further study of these is recommended.

### SCE4 primary contributions

JCTVC-M0058 SCE4.1.2: Inter-layer fixed refining filter [W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai, Y. Chiu (Intel)]

JCTVC-M0265 SCE4: Results of Test 4.2.1 on High Frequency Pass Inter Layer Sample Adaptive Offset Filter [W. Pu, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0267 SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter [W. Pu, J. Chan, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0195 SCE4.2.3: Inter-Layer Adaptive Filter on Reconstructed Base Layer Samples [M. Guo, S. Liu, S. Lei (MediaTek)]

JCTVC-M0183 SCE4: Results of test 4.2.4 on chroma enhancement for inter layer prediction [J. Dong, Y. Ye, Y. He (InterDigital)]

This contribution proposes to enhance the chroma planes for better inter-layer prediction, which is achieved by using the corresponding information from the luma plane. More specifically, for each chroma sample, an appropriate offset calculated based on the values of surrounding 3×4 luma samples is added. In the RefIdx framework, the average {Y, U, V} gains are {−0.8%, −7.5%, −9.1%}, {−0.3%, −9.2%, −9.7%}, {−0.2%, −6.4%, −7.2%}, and {−0.2%, −6.9%, −7.9%} for AI, RA, LDP, and LDB, respectively. Similar performance gain can be observed for the IntraBL framework. Compared with simulcast, the proposed chroma enhancement brings down the gap between luma gain and chroma gain significantly, which is large by only using SHM-1.0. The average memory access increase is from 0% to 4% for picture-based implementation, and 0% to 10% for block-based implementation. The average computational complexity (i.e., number of multiplications and additions) increase is from 14% to 37% for picture-based implementation, and 3% to 43% for block-based implementation. The proposed algorithm can be implemented using 16-bit integer arithmetic.

JCTVC-M0055 SCE4.2.5: Switchable De-ringing Filter for Inter-layer Prediction [Z.Ma, F. Fernandes]

JCTVC-M0213 SCE4.2.6: Adaptive up-sampling of base layer picture using bilateral filters [J. Zhao, K. Misra, A. Segall (Sharp)]

JCTVC-M0087 SCE4: De-noising filter for SNR scalability [E. Alshina, A.Alshin (Samsung)]

### SCE4 cross checks

JCTVC-M0185 SCE4: Cross-check results of test 4.1.1 on integer sample filtering for SNR scalability [J. Dong, Y. Ye (InterDigital)]

JCTVC-M0246 SCE4: Cross check report for test 4.1.2 on Inter-layer fixed refining filter [M. Guo, S. Liu (MediaTek)]

JCTVC-M0337 SCE4: cross-check for SCE4: Results of Test 4.2.1 on High Frequency Pass Inter Layer Sample Adaptive Offset Filter (JCTVC-M0265) [E. Alshina] [late]

JCTVC-M0278 SCE4: Cross check report for test 4.2.2 on upsampling filter [Kiran Misra, Andrew Segall (Sharp)] [late]

JCTVC-M0238 SCE4: Crosscheck of SCE 4.2.4 [X. Li (Qualcomm)] [late]

JCTVC-M0338 SCE4: cross-check for Chroma Enhancement (JCTVC-M0183) [E. Alshina] [late]

JCTVC-M0225 SCE4: Cross-check of SCE4.2.5 Switchable De-ringing Filter for Inter-layer Prediction (JCTVC-M0055) [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-M0239 SCE4: Crosscheck of SCE 4.2.6 [X. Li (Qualcomm)] [late]

JCTVC-M0373 Cross-check of SCE4.2.6 [X. Wei (Huawei)] [late]

JCTVC-M0061 Cross check for SCE4 [W. Zhang, Y. Chiu (Intel)] [late]

## SCE5: Inter-layer syntax prediction using HEVC base layer

### SCE5 summary and general discussion

JCTVC-M0025 SCE5: Summary report of SHVC core experiment on inter-layer syntax prediction using HEVC base layer [V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim, H. Yang]

This contribution summarizes the activities and test results performed in SCE5 on inter-layer syntax prediction using HEVC base layer. The tools in SCE5 were classified into four categories:

- 5.1 Inter-layer motion information prediction

  - Using base layer MV as a candidate in Merge and AMVP modes.

  - Inter-layer inferred prediction mode, where the enhancement layer is divided into sub-blocks and motion information associated with each sub-block is derived from the corresponding block in the base layer.

- 5.2  Motion data compression and memory reduction (memory measurement)

In this category, motion data compression of the base layer in relation to the base layer motion vector derivation is studied as well as a memory reduction associated with not using temporal MV candidate.

Overview of contributions:

| Test | Proposal | Cross-checking documents |
| --- | --- | --- |
| SCE5.1 | ## Inter-layer motion information prediction |  |
| 5.1.1 | JCTVC-M0295 (Qualcomm) “SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1)” | JCTVC-M0395 (MediaTek) |
| 5.1.2 | JCTVC-M0308 (Qualcomm) “SCE5: Results of Test 5.1.2 on Inter-layer Inferred Prediction Mode” | JCTVC-M0302 (LG) |
| 5.1.3 | withdrawn |  |
| 5.1.4 | JCTVC-M0413 (KDDI) “SCE5: Results of test 5.1.4 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer” | JCTVC-M0147 (Sony) |
| 5.1.5 | JCTVC-M0124 (MediaTek) “SCE5.1.5: Inter-layer motion information prediction” | JCTVC-M0149 (Sony) |
| 5.1.6 | JCTVC-M0160 (Nokia) “SCE5 – Results of 5.1.6 on inter-layer motion vector prediction” | JCTVC-M0300 (Qualcomm) |
| 5.1.7 | JCTVC-M0059 (Intel) “SCE5.1.7: Inter-layer motion data inheritance” | JCTVC-M0406 (KDDI) |
| 5.1.8 | JCTVC-M0289 (LG) “SCE5 : Inter-layer motion vector prediction in AMVP” | JCTVC-M0355 (Qualcomm) |
| 5.1.9 | withdrawn |  |
| 5.1.10 | JCTVC-M0047 (ETRI) “SCE5: Results of test 5.1.10 on inter-layer motion information prediction” | JCTVC-M0359 (Canon) |
| SCE5.2 | Motion data compression and memory reduction |  |
| 5.2.1 | JCTVC-M0283 (Sony, I2R) “SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP)” | JCTVC-M0128 (JVC Kenwood) |
| 5.2.2 | JCTVC-M0141 (Sony, LG) “SCE5: Result of SCE5.2.2” | JCTVC-M0392 (MediaTek) |
|  | JCTVC-M0142 (Sony) “SCE5: Buffer Size Estimation of SCE5.2.x” |  |

The tests results for both categories are summarised in the next table.

| Test | Short description | RA-2x | RA-1.5x | RA SNR | LP-2x | LP-1.5x | LP SNR | LB-2x | LB-1.5x | LB SNR | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1.1 (Qualcomm) | insert BL MV into the completed list | 0.25 | 0.19 | 0.29 | 0.26 | 0.21 | 0.32 |  |  |  | 100% | 100% |
| 5.1.2 (Qualcomm) | Inferred mode based on 8x8 sub-block | 0.03 | 0.00 | 0.03 | 0.01 | 0.01 | 0.02 |  |  |  | 101% | 103% |
| 5.1.3 |  | Withdrawn |  |  |  |  |  |  |  |  |  |  |
| 5.1.4 (KDDI) | 1. BL MV as a 3rd candidate | -0.03 | 0.17 |  | -0.22 | -0.06 |  |  |  |  | 100% | 100% |
|  | 2. MV refinement | -0.05 | 0.30 |  | -0.23 | -0.02 |  |  |  |  | 100% | 106% |
|  | 3. Test 1 + uncompressed MV | -0.84 | -0.19 |  | -0.89 | -0.35 |  |  |  |  | 100% | 100% |
| 5.1.5 (MediaTek) | Simplified pruning | 0.03 | 0.01 | 0.02 | 0.01 | 0.03 | 0.03 |  |  |  | 100% | 100% |
|  | 2 BL MVs with simplified pruning | -0.61 | -0.23 | -0.61 | -0.40 | -0.16 | -0.49 |  |  |  | 100% | 100% |
| 5.1.6 (Nokia) | 1. use TMVP scaling | -0.03 | 0.00 | 0.00 | -0.03 | 0.00 | 0.00 |  |  |  | - | - |
|  | 2. w/o BL MV pruning | 0.30 | 0.21 | 0.32 | 0.29 | 0.23 | 0.33 |  |  |  | - | - |
|  | 3. Test 1 + Test 2 | 0.27 | 0.21 | 0.32 | 0.27 | 0.23 | 0.33 |  |  |  | - | - |
| 5.1.7(Intel) | Inferred mode based on NxN | 0.09 | 0.08 | 0.12 | 0.09 | 0.08 | 0.11 | 0.09 | 0.06 | 0.10 | 103% | 98% |
| 5.1.8 (LG) | 1. BL MV for AMVP | -0.04 | -0.10 | -0.11 | -0.03 | -0.04 | -0.05 |  |  |  | 100% | 100% |
|  | 2. Test 1 + uncompressed MV | -1.26 | -0.64 | -0.63 | -1.07 | -0.47 | -0.69 |  |  |  | 100% | 100% |
| 5.1.9 |  | Withdrawn |  |  |  |  |  |  |  |  |  |  |
| 5.1.10 (ETRI) | 1. Simplified BL MV pruning | 0.08 | 0.05 | 0.07 | 0.09 | 0.07 | 0.08 |  |  |  | 100% | 100% |
|  | 2. BL MV for AMVP | 0.02 | -0.10 | -0.12 | 0.09 | 0.01 | -0.02 |  |  |  | 100% | 100% |
| 5.2.1 (I2R, Sony) | 1. disabling TMVP | 0.76 | 0.34 | 0.38 | 0.92 | 0.57 | 0.48 | 0.81 | 0.46 | 0.44 | 100% | 98% |
|  | 2. Test 1 + uncompressed MV | -0.49 | -0.20 | -0.17 | -0.27 | 0.06 | -0.25 |  |  |  | - | - |
|  | 3. Test 1 + 8x8 compressed MV | -0.15 | -0.12 | -0.11 | 0.13 | 0.15 | -0.14 |  |  |  | 101% | 100% |
| 5.2.2 (Sony, LG) | 1. uncompressed MV | -1.10 | -0.50 | -0.50 | -0.94 | -0.41 | -0.61 |  |  |  | 99% | 99% |
|  | 2. 8x8 compressed MV | -0.79 | -0.42 | -0.45 | -0.61 | -0.34 | -0.52 |  |  |  | 100% | 101% |

Note: Using uncompressed MV is known to give approx.  0.7% BR reduction.

5.1.2 and 5.1.7 (mode inferring from BL) do not provide benefit

Current anchor needs 5 additional comparisons (in worst case when all spatial candidates are available) in pruning for merge, compared to SL HEVC.

5.1.1 and 5.1.6 case 2 are avoiding additional pruning, but could end up with duplicate candidates (resulting in 0.2-0.3% BR increase)

For test 1: Prepare a table for each proposal, analyzing the number of additional or reduced operations (number of comparisons or if applicable, any other), compared to SHM. Revisit.

Test 2.2: Using 8x8 compressed BL MV (instead of 16x16 as in current SHM) gives around 0.5% bit rate reduction; 4x4 would give about 0.7%. JCTVC-M0142 provides analysis about additional memory.

Test 2.1: Disabling TMVP in the enhancement layer only has a loss of 0.7-0.8% BR reduction, whereas in a single layer configuration it should be 2% or more; disabling TMVP in enhancement layer and re-using the memory for better representation of the base layer MV (5.2.1 test 2) could lead to no performance loss, whereas saving MV memory for EL. Revisit 2.1/2.2 when more numbers about actual memory increase/saving are provided (see additional notes under JCTVC-M0142).

### SCE5 primary contributions

JCTVC-M0295 SCE5: Simplified base layer MV candidate for merge mode (test 5.1.1) [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-M0395 SCE5: Crosscheck of JCTVC-M0295 on result of SCE5.1.1 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0308 SCE5: Results of Test 5.1.2 on Inter-layer Inferred Prediction Mode [L. Guo, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-M0413 SCE5: Results of test 5.1.4 on inter-layer motion-vector prediction by the base-layer MV up-scaling and refinement using HEVC base layer [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

JCTVC-M0124 SCE5.1.5: Inter-layer motion information prediction [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-M0160 SCE5 – Results of 5.1.6 on inter-layer motion vector prediction [D. Bugdayci, K. Ugur (Nokia)]

`

JCTVC-M0059 SCE5.1.7: Inter-layer motion data inheritance [Z. Deng, W. Zhang, L. Xu, Y. Han, X. Cai, Y. Chiu (Intel)]

JCTVC-M0047 SCE5: Results of test 5.1.10 on inter-layer motion information prediction [J. Lee, H. Lee, J. W. Kang, J. S. Choi (ETRI)]

JCTVC-M0283 SCE5 5.2.1 on Effectiveness of temporal motion vector prediction (TMVP) [J. Xu, A. Tabatabai, K. Sato, S. Lu (Sony), Y. H. Tan, C. Yeo (I2R)]

JCTVC-M0141 SCE5: Result of SCE5.2.2 [K. Sato, J. Xu (Sony), J. Park, B Jeon(LG)]

JCTVC-M0392 SCE5: Crosscheck of JCTVC-M0141 on result of SCE5.2.2 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0142 SCE5: Buffer Size Estimation of SCE5.2.x [K. Sato, J. Xu(Sony)]

On the usage of motion data buffer for SHVC is being studied under SCE5.2.x.

It is proposed by SCE5.2.1 that temporal motion prediction be omitted at the enhancement layer to reduce the required buffer size, as the collocated base layer motion information takes part of TMVP in the enhancement layer.

It is proposed by SCE5.2.2 to postpone motion data compression after encoding/decoding of the enhancement layer, or 2-stage motion data compression for improving coding efficiency.

At the 12th JCT-VC meeting in Geneva, it was suggested to study on the buffer size requirements for SCE5.2.x proposals.

This contribution provides information on the buffer sizes required for each of SCE5.2.x proposals. Trade-off of buffer sizes and coding efficiency is also studied.

Presentation slides to be provided.

36 bits per MVx (16), MVy (16) and refidx (4), 40 byte aligned

In worst case (which is 1x scaling i.e. SNR scalability and RA, bi-pred) the memory increase relative to EL picture buffer would be approx. 40% in case of 4x4 (uncompressed), 10% in case of 8x8 compressed

It is mentioned by one expert that the memory for uncompressed could potentially be further reduced, as 4x8/8x4 only allow unipred; hypothetically it should not be larger than for 8x8 bi-pred, but additional signalling and logic would be necessary.

uncompressed case is too large memory increase

Revisit: Report actual additional memory for 8x8 case, as an example some level with 4K video

Another potential option for further investigation could also be to make the compression ratio dependent on the upscaling factor (e.g. 8x8 only for 2X case)

JCTVC-M0289 SCE5 : Inter-layer motion vector prediction in AMVP [J. Park, B. Jeon (LG)]

### SCE5 cross checks

JCTVC-M0302 crosscheck of SCE5.1.2 interBL [J. Park, B. Jeon (LG)] [late]

JCTVC-M0147 SCE5: Crosscheck of SCE5.1.4 [K. Sato(Sony)]

JCTVC-M0149 SCE5: Crosscheck of SCE5.1.5 [K. Sato(Sony)]

JCTVC-M0300 SCE5: Cross-verification of test 5.1.6 on inter-layer motion vector prediction [V. Seregin (Qualcomm)] [late]

JCTVC-M0406 SCE5: Cross-check of Test 5.1.7 on inter-layer motion data inheritance (JCTVC-M0059) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-M0355 SCE5 : Cross-check Report of SCE5.1.8 Test [L. Guo (Qualcomm)] [late] [miss]

JCTVC-M0359 SCE5: Cross-check of test 5.1.10 on inter-layer motion information prediction (JCTVC-M0047) [E. François (Canon)] [late]

JCTVC-M0128 SCE5: Cross-check of SCE5 Test 5.2.1 on Effectiveness of temporal motion vector prediction (JCTVC-M0283) [H. Nakamura (JVC Kenwood)]

# Non-CE Technical Contributions

## Range extensions

### General

JCTVC-M0178 AHG5: Range Extensions and High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-M0256 Suggested fixes for HEVC Range Extensions draft text [D. Flynn, J. Sole, T. Suzuki] [miss]

JCTVC-M0298 Bandwidth reduction for range extension [V. Seregin, X. Wang, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-M0345 Cross-check for Bandwidth reduction for range extension [E. Alshina, S. Lee, J. Min] [late]

JCTVC-M0335 AhG5: Offset Scaling in SAO for High Bit-depth Video Coding [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-M0405 AHG5: Crosscheck of JCTVC-M0335 on offset scaling in SAO for high bit-depth video coding [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-M0118 Removal of the offset quantization in SAO [C.-W. Hsu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-M0344 Cross-check for Removal of the offset quantization in SAO [S.Jeong, J.Min, E. Alshina] [late]

### RCE1 related (Inter-component decorrelation)

JCTVC-M0230 Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content [T. Nguyen, A. Khairat, D. Marpe (Fraunhofer HHI)]

JCTVC-M0418 Cross-check of adaptive inter-plane prediction for RGB content by HHI (JCTVC-M0230) [S.H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-M0304 Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding [S.H. Kim, A. Segall (Sharp)]

JCTVC-M0417 Cross-check of simplified level coding for transform skip and lossless coding by Sharp (JCTVC-M0304) [S. Lee, C. Kim (Samsung)] [late]

JCTVC-M0428 Cross-check of Simplified Level Coding for Transform Skip and Lossless Coding by Sharp (JCTVC-M0304) [?? (??)] [late] [miss]

JCTVC-M0072 AHG7: Modification of in-loop colour-space transformation [S. Matsuo, M. Matsumura, S. Takamura, H. Fujii, A. Shimizu (NTT)]

JCTVC-M0407 Cross-check of modified in-loop colour-space transformation (JCTVC-M0072) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-M0080 AHG7: Post filtering for colour-space transformation [M. Matsumura, S. Matsuo, S. Takamura, H. Fujii, A. Shimizu (NTT)]

JCTVC-M0408 Cross-check of post filter for colour-space transformation (JCTVC-M0080) [K. Kawamura, S. Naito (KDDI)] [late]

### RCE2 related (Prediction in lossless coding)

JCTVC-M0068 Non-RCE2: TU-level flag for DPCM [Y. H. Tan, C. Yeo (I2R)]

JCTVC-M0193 Improved sample-based weighted intra prediction for lossless coding [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

JCTVC-M0288 Non-RCE2: Extension of residual DPCM for lossless coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-M0430 non-RCE2: Cross-check of extension of residual DPCM for lossless coding (M0288) [Y. H. Tan, C. Yeo (I2R)] [late] [miss]

JCTVC-M0291 Non-RCE2: Modification of DC intra prediction mode for lossless coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

### Transforms and transform coefficient coding

JCTVC-M0098 AHG5: Independent chroma transform depth from luma transform depth for non 4:2:0 format [J. Kim (??)]

JCTVC-M0369 Cross-check report of separate chroma transform depth for non 4:2:0 format (JCTVC-M0098) [L. Guo(Qualcomm)] [late] [miss]

JCTVC-M0388 AHG5: Crosscheck of JCTVC-M0098 on independent chroma transform depth from luma transform depth for non 4:2:0 format [C.-Y. Chen, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0099 AHG5: using zero-depth chroma transform unit for 4:2:2 format [J. Kim (??)]

JCTVC-M0386 Cross-check of ‘AHG5: using zero-depth chroma transform unit for 4:2:2 format’ (M0099) by LGE [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-M0136 AHG5: 32x32 transform in chroma for 4:2:2 [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

JCTVC-M0378 AHG5: crosscheck of JCTVC-M0136 proposed by Canon [J. Kim (LG)] [late]

JCTVC-M0137 AHG5: Square transform deblocking for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

JCTVC-M0232 AHG5:Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 Chroma Extended Format [Z. Chen, S. Liu, S. Lei (MediaTek)]

JCTVC-M0368 Cross-check of Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 (JCTVC-M0232) [J. Sole (Qualcomm)] [late]

### Intra prediction

JCTVC-M0106 AHG 5: Reference sample filtering in intra prediction for extended color formats [J. Min, S. Lee, C. Kim (Samsung)]

JCTVC-M0380 crosscheck of JCTVC-M0106 proposed by Samsung [J. Kim (LG)] [late]

JCTVC-M0107 AHG5: Chroma intra prediction for extended color formats [J. Min, S. Jeong, S. Lee, C. Kim (Samsung)]

JCTVC-M0379 crosscheck of JCTVC-M0107 proposed by Samsung [J. Kim (LG)] [late]

JCTVC-M0116 Non-RCE1: Multiple LM chroma modes [C.-Y. Chen, C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-M0377 non RCE1 : crosscheck of JCTVC-M0116 proposed by MediaTek [J. Kim (LG)] [late]

JCTVC-M0127 AHG5: Unified intra prediction angles for 4:2:2 chroma format [H. Nakamura, M. Ueda, S. Fukushima, T. Kumakura (JVC Kenwood)]

JCTVC-M0372 AHG5: Cross-check on Unified intra prediction angles for 4:2:2 chroma format (JCTVC-M0127) [A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-M0383 Cross-check of proposal ‘AHG5: Unification of chroma intra prediction angles between 4:2:2 chroma format’ (M0127) by JVC Kenwood [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-M0138 AHG5: On chroma intra prediction for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

JCTVC-M0424 Cross-check of chroma intra prediction for 4:2:2 (JCTVC-M0138) [J. Sole (Qualcomm)] [late]

JCTVC-M0156 AHG5: Simplification of chroma intra prediction process for YUV4:2:2 coding [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

JCTVC-M0409 Cross-check of simplified chroma-intra-prediction process for YUV4:2:2 coding (JCTVC-M0156) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-M0412 AHG5: CU based chroma intra prediction with reduced reference [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late] [miss]

### Screen content

JCTVC-M0316 AHG8: Coefficient level cRiceParam updates for screen content coding [M. Budagavi (TI)]

JCTVC-M0323 Palette Mode for Screen Content Coding [L. Guo, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-M0381 crosscheck of JCTVC-M323 proposed by Qualcomm [J. Kim (LG)] [late] [miss]

JCTVC-M0330 Screen Content Coding with Multi-stage Base Color and Index Map Representation [W. Zhu (Beijing University of Technology), J. Xu (Microsoft), W. Ding (Beijing University of Technology)]

JCTVC-M0333 AhG8: Residue rotation and significance map context for screen content coding [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-M0427 AHG8: Cross-check of residue rotation and significance map context for screen content coding (JCTVC-M0333) [J. Wang, D. He (??)] [late]

JCTVC-M0348 AHG8: Quantitative measurement for subjectively lossless screen content coding [Tao Lin, Shuhui Wang, Kailun Zhou, Qiang Shen (Tongji U)]

JCTVC-M0350 AHG8: Video coding using Intra motion compensation [M. Budagavi, D.-K. Kwon (TI)]

JCTVC-M0416 Cross-check of intra motion compensation by TI (JCTVC-M0350) [S. Lee, C. Kim (Samsung)] [late]

JCTVC-M0351 AHG8: Residual DPCM for visually lossless coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-M0366 AhG8: Simplified update of the coefficient level Rice parameter [J. Sole, M. Karczewicz (Qualcomm)] [late]

JCTVC-M0382 crosscheck of JCTVC-M366 proposed by Qualcomm [J. Kim (LG)] [late]

### Other

JCTVC-M0111 AHG5: On chroma QP for HEVC Rext [E. François, C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-M0420 Cross-check of Chroma QP for HEVC RExt in JCTVC-M0111 [W.-S. Kim (Qualcomm)] [late]

JCTVC-M0229 AHG5: Backwards compatible enhancement of chroma format [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

JCTVC-M0398 Multiview and multiview-video-plus-depth support in SHM [A. Hallapuro, Y. Yan, D. Rusanovskyy, M. M.Hannuksela] [late]

## Scalable video coding

### General

JCTVC-M0086 AHG-17: complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast [A. Alshin, E. Alshina (Samsung)]

This contribution contains performance and complexity analysis of SHM1.0 both IntraBL and RefIdx frameworks, compared to single layer coding. Complexity assessment methodology developed for SCE3 &4 by AhG-17 was used. It is reported that memory access SHM1.0 in the worst case doesn’t exceed HEVC. In average for motion compensation test scenarios (RA and LD-P) the coding efficiency compare to HM8.1 simulcast is 18.5% for IntraBL framework and 18.0% for RefIdx framework in terms of Luma BD-rate. Average memory of SHM1.0 is lower than HM8.1 simulcast: 94–96% for IntraBL and 93–94% for RefIdx frameworks. In average for motion compensation test scenarios SHM1.0 IntraBL framework outperforms RefIdx by 0.7% with cost of 1–2% extra memory access in average.

Presentation to be uploaded.

The contribution conclusions were reported as follows:

- The average computational complexity and memory access of SHM1.0 is reportedly 4–6% lower than for HM8.1 simulcast single layer coding, while 18% Luma BD-rate reduction is achieved in motion compensation test scenarios.

- It was proposed that the motion vector in inter-layer predicted blocks in the RefIdx framework shall be normatively restricted to be 0, which was noted to already be a planned constraint. This restriction was asserted to guarantee that the worst case memory bandwidth of SHVC will not exceed HEVC limit.

- Complexity assessment using the AhG17 tool shows negligible difference between IntraBL and RefIdx frameworks (1–2% in motion compensation test scenarios), while the IntraBL branch outperforms the RefIdx framework 0.7% in terms of Luma BD-rate (0.6% in terms of Chroma BD BR).

The reported test data did not include LB case testing.

All results given here are reporting average results (not worst case).

Complexity comparison in this contribution is done against simulcast; in the discussion it is pointed out that from an application perspective, it would be more reasonable to compare vs. single layer corresponding to enhancement layer, as usually only the higher resolution would be decoded.

RefIDx was investigated once by assuming block-based processing (i.e. performing upsampling only in cases where it is needed), in which case there is no significant difference compared to textureBL. Results are also reported for the case when picture-based processing would be used (upsampling would always be necessary). In that case, the memory bandwidth is higher.

As analyzed, the use of whole-picture upsampling was not considered for the RefIdx approach, as this type of operation has higher memory bandwidth.

However, it was remarked that whole-picture processing can be used as an architecturally/conceptually simple way to construct an SHVC decoder when starting with a single-layer HEVC implementation.

It was noted that the complexity anchor for some comparisons was simulcast decoding of both simulcast streams, which does not necessarily really make sense.

JCTVC-M0182 AHG17: complexity analysis of SHM1.0 [J. Dong, Y. Ye, Y. He (Interdigital)]

SHM1.0 complexity assessment was done as part of the AHG17 activities, and the anchor results were released in the JCTVC-L0440 package. Using the SHM1.0 anchor results in the JCTVC-L0440 package ("r2" version), this contribution summarizes the complexity of the PU-based RefIdx, the Picture-based RefIdx, and the IntraBL implementations. It is reported that the PU-based RefIdx and IntraBL solutions have similar complexity characteristics, whereas the complexity characteristics of the Picture-based RefIdx solution are very different. It is also reported that the RefIdx solution offers the design flexibility and allows different applications to choose from block-based or picture based implementation based on its specific complexity considerations.

Decoder complexity: Compared to simulcast, both refidx and textureBL approaches have memory bandwidth of approx. 92-93% on average (refidx in PU-based operation). With picture-based refidx, approx. 102%. Number of computations around 94-95% for PU-based (both), 144% for picture-based refidx

No separate analysis is given in this contribution for the different cases (AI, RA, LD-P/B)

Worst-case complexity is identical to full simulcast decoding (base and enhancement) in PU-based processing (since spatial upsampling is less complex than MC bi-pred), and full simulcast plus upsampling filter for picture-based processing.

In worst case, with block-based upsampling, bipred with the RefIdx approach requires performing both upsampling and MC and then averaging the results together, whereas in the IntraBL approach this combination is not allowed. When this bipred case occurs, the MC part is higher or equal complexity than the upsampling part, and the total complexity is less than or equal to that of ordinary enhancement-layer temporal bipred.

The IntraBL as previously designed has a CU-based switch (whereas the similar switch point in RefIdx is the PU level). As previously designed, the coding efficiency difference is about 1% for LB, in favor of IntraBL. It was remarked that a modification of the switching point as proposed in M0220 can increase this to 2%.

JCTVC-M0076 Evaluation of IBP-like coding structure and non-HEVC base layer for hybrid standard scalability [T. Yamamoto (Sharp)]

JCTVC-M0242 AHG-17: Crosschecking of complexity and performance analysis of SHM1.0 compare to HM8.1 simulcast (JCTVC-M0086) [X. Li (Qualcomm)] [late]

### SCE1 related (intra prediction)

JCTVC-M0091 non SCE1: Low-pass filter for Combined Intra Prediction [A. Alshin, E. Alshina (Samsung)]

JCTVC-M0419 Cross-check of JCTVC-M0091 (Non SCE1-Combined Intra Prediction with low-pass filter) [J. Lainema (Nokia)] [late]

JCTVC-M0115 Non-SCE1: Simplification of remaining modes coding in SHVC [E. François, S.Shi, C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-M0184 Non SCE1: Cross-check for M0115 Simplification of remaining modes coding in SHVC [Y He (??)]

JCTVC-M0117 Non-SCE1: Weighted intra prediction [C. Kim, B. Jeon (LG)]

JCTVC-M0374 Non-SCE1: Cross-check for weighted intra prediction (JCTVC-M0117) [D.-K. Kwon (TI)] [late]

JCTVC-M0123 Non-SCE 1: Constrained intra prediction at enhancement layer [C. Kim, B. Jeon (LG)]

JCTVC-M0391 Non-SCE1: Crosscheck of JCTVC-M0123 on constrained intra prediction at enhancement layer [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0139 Non SCE1 : Inter-layer Intra Mode Prediction [Xuguang Zuo, Lu Yu (Zhejiang University)]

JCTVC-M0357 Non-SCE1: Cross-check of JCTVC-M0139 on Inter-layer Intra Mode Prediction [J. Xu (Sony)] [late]

JCTVC-M0196 Non-SCE 1: Simplified Intra Coding at Enhancement Layer for SHVC [M. Guo, S. Liu, S. Lei (MediaTek)]

JCTVC-M0285 Non-SCE1: Chroma-like coding of enhancement layer luma intra mode in SHVC [J. Xu, A. Tabatabai (Sony)]

JCTVC-M0364 Non-SCE1: Cross-check of chroma-like coding of enhancement layer luma intra mode in SHVC (JCTVC-M0285) [E. François (Canon)] [late]

JCTVC-M0290 Non-SCE1: Complexity reduction in intra prediction of enhancement layer in SHVC [C. Auyeung, J. Xu (Sony)]

JCTVC-M0376 Cross-check of complexity reduction in intra prediction of enhancement layer in SHVC (JCTVC-M0290) [D.-K. Kwon (TI)] [late]

JCTVC-M0385 Non-SCE1: Crosscheck of complexity reduction in intra prediction of enhancement layer in SHVC (JCTVC-M0290) [J. Park, B. Jeon (LG)] [late]

JCTVC-M0311 Non-SCE1: supplementary results on category 2 [J. Park, B. Jeon (LG), C. Auyeung, A. Tabltabai (Sony), K. Rapaka, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0397 Non-SCE1: Crosscheck of JCTVC-M0311 on supplementary results on SCE1 category 2 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0336 Non-SCE1 : Cross check of supplementary results on category 2 (M0311) [J. Min, E. Alshina (Samsung)] [late]

JCTVC-M0375 Non-SCE1: Cross-check for inter-layer intra mode prediction (JCTVC-M0312) [D.-K. Kwon (TI)] [late]

JCTVC-M0312 Non-SCE1: Inter-layer intra mode prediction [J. Min, E. Alshina (Samsung)]

JCTVC-M0303 crosscheck of Non-SCE1 (Samsung's) [J. Park, B. Jeon (LG)] [late]

JCTVC-M0305 crosscheck of Non-SCE1 (MediaTek's) [J. Park, B. Jeon (LG)] [late]

### SCE2 related (inter-layer texture prediction signalling)

JCTVC-M0075 On TextureRL flag context [T. Yamamoto (Sharp)]

JCTVC-M0249 Cross-check results of On TextureRL flag context (JCTVC-M0075) [Z. Ma, F. Fernandes (Samsung Electronics)] [late]

### SCE3 related (combined inter-picture and inter-layer prediction)

JCTVC-M0062 nonSCE3: Inter-layer residual prediction with motion prediction [W. Zhang, Z. Deng, L. Xu, Y. Han, X. Cai, Y. Chiu (Intel)]

This contribution presents an inter-layer residual prediction technology to improve the coding efficiency of SHVC. It utilizes the inter-coded residual of the collocated BL block to refine the inter prediction of the EL block. For spatial scalability, bilinear interpolation filter is performed to upsample the BL residual block. In addition, the proposed inter-layer residual prediction is enabled under the condition that inter-layer motion prediction is used. Specifically, if a 2Nx2N PU in EL is coded by inter merge mode and its motion candidate is from BL, the residual prediction is applied. No new syntax is added in the proposed implementation, thus no parsing dependent issue is introduced. Compared to the SHM1.0 anchor, the proposed inter-layer residual prediction achieves 0.94% BD-rate saving on average among the RA, LD-P and LD-B testing cases, with marginal increase of coding runtime and memory access bandwidth.

Residual prediction in “AVC/SVC” style is used unconditionally, whenever the base layer MV is used in merge, i.e. when base layer and enhancement layer motion comp can be considered to be consistent. Bilinear interpolation is used; no additional motion comp. loop.

Additional average complexity approx. 20% in memory accesses, 8% Mult, 4% Additions. No numbers for worst case.

Further study (CE)

Q: Is it necessary to store the entire residual picture when only the co-located base layer residual is accessed?

JCTVC-M0319 Cross-check of JCTVC-M0062 on inter-layer residual prediction with motion prediction [H. Yang (Huawei)]

JCTVC-M0071 Non-SCE3: Inferred GRP (IGRP) with reduced motion compensation [X. Wei, J. Zan (Huawei)]

This contribution presents an inferred GRP (IGRP) algorithm based on rqt_root_cbf flag to reduce complexity of generalized residual prediction (GRP). Test results report that average 13.0% of encoding time saving and 0.7% of decoding time saving are achieved. The average BD-rate losses are 0.2% (Y), 0.2% (U) and 0.2% (V). An additional test reports that extra motion compensation introduced by GRP is reduced by 11.7% on average.

Comparison is made against fast GRP from SCE3.3 case 4, against which the average complexity is slightly decreased. That indicates a slight simplification, due to the fact that weight 0 is chosen more frequently by the encoder, and no motion comp is performed in such a case. Worst case would be the same (as there is no guarantee that the encoder would always use the mode from which weight 0 is inferred.

SCE3.3 case 4 was on average around 12% higher in memory accesses than the anchor, which could be brought down to 10% by this proposal.

JCTVC-M0370 nonSCE3: Crosscheck for inferred GRP (IGRP) with reduced motion compensation (JCTVC-M0071) [W. Zhang, Y. Chiu (Intel)] [late] [miss]

JCTVC-M0074 Implicit derivation of weight factor for Generalized Residual Prediction [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

This contribution proposes implicit derivation of weight factor for Generalized Residual Prediction (GRP). A CU level flag is used to signal GRP and the weight factor for each PU is determined based on motion parameters. The method is implemented on SCE3.5 (JCTVC-M0109) software. It is reported that the BD-rate (EL+BL) changes compared to SHM1.0 are -1.4%, -1.5%, -1.8%, -2.8%, -2.7% ,-3.5%, -2.2%, -2.7% and -2.4% for RA 2x, RA 1.5x, RA SNR, LP 2x, LP 1.5x, LP SNR, LB 2x, LB 1.5x and LB SNR cases respectively. It is also reported that the proposed method achieves coding gain by 0.0% to -1.2% compared to SCE3.5 case 1 (one weight case: w=0, 1.0) without increase of encoding/decoding time. It is also reported that the proposed method reduces encoding time by 5.5% to 10.2% compared to SCE3.5 (two weight case: w=0, 0.5, 1.0) without significant coding loss (0.0% to 0.3%) except LP 2x, LP SNR and LB SNR cases (0.5% to 1.0%).

Presentation deck not uploaded.

Weight factor determined based on merge flag and inter_pred_idc.

Main benefit is for encoder speedup, but several experts expressed that this might be desirable in context of implementing multi-mode GRP with same complexity as single mode.

JCTVC-M0360 Cross-check on Implicit derivation of weight factor for Generalized Residual Prediction (JCTVC-M0074) [E. François (Canon)] [late]

JCTVC-M0078 Cross-check on GRP modification [T. Tsukuba (Sharp)] [late]

JCTVC-M0090 non SCE3: Low-pass filter for Combined Inter Prediction [A. Alshin, E. Alshina (Samsung)]

This contribution contains description of new prediction mode for SHVC which combines inter-layer prediction texture with enhancement layer motion compensation (MC) prediction. Application of 2D separable symmetrical 5-taps low-pass filter allows increasing influence of low frequencies from Inter layer prediction and preserving high frequencies of enhancement layer in resulting prediction. Proposed new combined prediction mode is enabled on CU level for Inter CUs. This additional mode provides average 1.7% (Luma) and 7,9% (Chroma ) BD-rate gain compare to SHM1.0 (IntraBL framework). If proposed combined mode is restricted for 2Nx2N Inter CUs and combination with bi-pred is not allowed for smallest CUs then performance gain is 1.5% (Luma) and 6,9% (Chroma) but the worst case memory band-width is within HEVC limit and encoding time overhead is 3% only.

Presentation deck not uploaded.

Approach: Average of BL and EL prediction, plus lowpass-filtered difference thereof. Padding is used to implement filtering within blocks. Compared to SCE3.2, the additional gain is approx. 0.5%, but complexity is also increased. Another version is presented which restricts usage to only 2Nx2N, and restricts bi-pred in 8x8 PUs, which has approximately the same average complexity as SCE3.2, and is still better by 0.25 %. JCTVC-M0092 uses a similar simplification on top of SCE3.2.

Average gain approx. 1.5% compared to SHM.

Q: Does the padding produce blocking artefacts at CU boundaries?

One expert points out that padding could also be used with GRP.

JCTVC-M0389 Non-SCE3: Crosscheck of JCTVC-M0090 on low-pass filter for combined inter prediction [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0092 Non SCE3: Simplified design for combined prediction (test 3.2) [E. Alshina, A. Alshin (Samsung)]

By restricting 8x8 bi-pred, the worst-case memory bandwidth for the enhancement layer is claimed to be within the margin of current SHM. Average gain approx. 1% compared to SHM.

JCTVC-M0301 Non-SCE3: Cross-verification of JCTVC-M0092 on simplified design for combined prediction [V. Seregin (Qualcomm)] [late]

JCTVC-M0132 Non-SCE3.1: Disabling adaptive predictor compensation for 8x8 bi-prediction [T.-D. Chuang, Y.-W. Huang, P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-M0252 Non-SCE 3: Cross-Check of M-0132 [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-M0143 Non-SCE3: Quantized GRP [K. Sato(Sony)]

JCTVC-M0154 Non-SCE3: Combination of Merge and GRP [Wenjing Zhu, Haitao Yang (Huawei)]

JCTVC-M0314 Cross check M0154 Combination of Merge and GRP [X Xiao (??)] [late]

JCTVC-M0155 Non-CE3: Enhanced inter layer reference picture for RefIdx based scalability [A. Aminlou, J. Lainema, K. Ugur, M. Hannuksela (Nokia)]

JCTVC-M0157 A study of Generalized Residual Predicition [Wenjing Zhu, Oscar Au, Wei Dai, Xingyu Zhang, Hong Zhang (HKUST)]

JCTVC-M0189 Non-SCE3: ILR enhancement with differential coding for RefIdx framework [Y. He, Y. Ye (InterDigital)]

JCTVC-M0362 Non-SCE3: Cross-check of ILR enhancement with differential coding for RefIdx framework (JCTVC-M0189) [E. François (Canon)] [late]

JCTVC-M0220 Non-SCE3: Uni-directional combined prediction (UCP) in Inter slice [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0354 Non-SCE3: Cross-verification of JCTVC-M0220 on uni-directional combined prediction [V. Seregin (Qualcomm)] [late] [miss]

JCTVC-M0222 Non-SCE3.4: Simplified Generalized Combined Prediction [P. Lai, S. Liu, T.-D. Chuang, Y.-W.Huang, S. Lei (MediaTek)]

JCTVC-M0426 Non-SCE3: Cross-check of Simplified Generalized Combined Prediction Test2 (M0222, test2) [Wonkap Jang, Adeel Abbas (??)] [late] [miss]

JCTVC-M0241 Non-SCE3: Crosscheck MTK's proposal [X. Li (Qualcomm)] [late]

JCTVC-M0275 Non-SCE3: Simplified Generalized Residual Prediction [X. Li, J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-M0226 Non-SCE3: Cross-check of Non-SCE3.3 Simplified Generalized Residual Prediction (JCTVC-M0275) [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-M0321 nonSCE3: Crosscheck for Simplified Generalized Residual Prediction (JCTVC-M0275) [W. Zhang, Y. Chiu (Intel)] [late]

JCTVC-M0297 Non-SCE3: Bandwidth reduction for combined inter mode [V. Seregin, M. Karczewicz (Qualcomm)]

JCTVC-M0396 Non-SCE3: Crosscheck of JCTVC-M0297 on bandwidth reduction for combined inter mode [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0347 Non-SCE3: Cross-check for bandwidth reduction for combined inter mode [E. Alshina] [late]

### SCE4 related (inter-layer filtering)

JCTVC-M0088 AHG-13,17: complexity and performance analysis of different length up-sampling filters in SHM1.0 [E. Alshina, A. Alshin (Samsung)]

JCTVC-M0262 Crosscheck: AHG-17: complexity and performance analysis of different length up-sampling filters in SHM1.0 [W. Pu (Qualcomm)] [late]

JCTVC-M0089 Non SCE4: simplified design of cross-color inter-layer filter (test 4.2.4) [E. Alshina, A. Alshin, Y. Cho (Samsung)]

JCTVC-M0186 Non-SCE4: cross-check for JCTVC-M0089 simplified design of cross-color inter-layer filter (test 4.2.4) [J. Dong, Y. Ye (InterDigital)] [miss]

JCTVC-M0114 Non-SCE4: On Interlayer SAO in SHVC [G. Laroche, P. Onno, J. Taquet, C. Gisquet, E. François (Canon)]

JCTVC-M0390 Non-SCE4: Crosscheck of JCTVC-M0114 on inter-layer SAO in SHVC [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0215 Non-SCE4: Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters [J. Zhao, K. Misra, A. Segall (Sharp)]

JCTVC-M0250 Cross-check results of Non-SCE4: Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters (JCTVC-M0215) [Z. Ma, F. Fernandes (Samsung Electronics)] [late]

JCTVC-M0223 Non-SCE4.1: Fixed Inter-layer Filter for SNR Scalability with Only One Non-unity Coefficient [X. Zhang, P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-M0371 nonSCE4: Crosscheck for Fixed Inter-layer Filter for SNR Scalability with Only One Non-unity Coefficient (JCTVC-M0223) [W. Zhang, Y. Chiu (Intel)] [late]

JCTVC-M0224 Non-SCE4.2: Inter-layer Fixed Directional Filtering [P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-M0423 Non-SCE4 : Crosscheck of inter-layer fixed directional filtering (JCTVC-M0224) [J. Park, B. Jeon (LG)] [late]

JCTVC-M0233 Non-SCE4: Inter-layer switchable upsampling filters for spatial scalability [Z. Chen, P. Lai, X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-M0400 Non-SCE4: crosscheck of JCTVC-M0233 on Inter-layer switchable upsampling filters for spatial scalability [J. Xu (Sony)] [late]

JCTVC-M0253 Non-SCE4: Simplification of chroma enhancement for inter layer reference picture generation [X. Li, J. Chen, W. Pu, M. Karczewicz (Qualcomm)]

JCTVC-M0187 Non-SCE4: cross-check for JCTVC-M0253 simplification of chroma enhancement for inter layer reference picture generation [J. Dong, Y. Ye (InterDigital)]

JCTVC-M0271 Non-SCE4.2.2: 6-Tap Adaptive Re-Sampling Filter [W. Pu, J. Chen, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0342 Non-SCE4 Cross-check for 6-Tap Adaptive Re-Sampling Filter (JCTVC-M0271) [E. Alshina] [late]

JCTVC-M0273 Non-SCE4: Switchable Filter on Integer Position [W. Pu, V. Seregin, J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, Alexander Alshin, Yongjin Cho (Samsung)]

JCTVC-M0402 Non-SCE4: Crosscheck of JCTVC-M0273 on switchable filter on integer position [X. Wei (Huawei)] [late] [miss]

JCTVC-M0340 AHG13: Cross-check for Low Complexity Resampling Filters For SHVC [E. Alshina] [late]

### SCE5 related (inter-layer syntax prediction)

JCTVC-M0066 Non-SCE5: On the effectiveness of temporal and multiple base layer co-located motion vector prediction candidates [Y. H. Tan, C. Yeo (I2R)]

JCTVC-M0404 Cross-check of JCTVC-M0066 on removing TMVP in enhancement layer [H. Yang (Huawei)] [late]

JCTVC-M0112 Non-SCE5: Position derivation for using Base Layer motion in SHVC [C. Gisquet, P. Onno, E. François, G. Laroche (Canon)]

JCTVC-M0353 Non-SCE5: Cross-verification of JCTVC-M0112 on position derivation for using Base Layer motion in SHVC [V. Seregin (Qualcomm)] [late]

JCTVC-M0113 Non-SCE5: Availability of Base Layer motion in SHVC [C. Gisquet, P. Onno, E. François, G. Laroche (Canon)]

JCTVC-M0120 On signaling the syntax ‘sps_inter_layer_mfm_enable_flag’ [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-M0133 Non-SCE5 : Simplified inter-layer MV scaling and sample position mapping [T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-M0144 Non-SCE5: Combined Tests of SCE5.1.5 and SCE5.2. [K. Sato(Sony)]

JCTVC-M0393 Non-SCE5: Crosscheck of JCTVC-M0144 on combined tests of SCE5.1.5 and SCE5.2.* [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

JCTVC-M0287 Non-SCE5 : Replacement of TMVP candidates with BL MV candidates [J. Park, B. Jeon]

JCTVC-M0325 Non-SCE5: On inter layer motion prediction in reference index based SHVC [J. Chen, V. Seregin, X. Li, M. Karczewicz (Qualcomm)]

JCTVC-M0284 Non-SCE 5: Crosscheck MTK's proposal [C. Kim, B. Jeon (LG)] [late]

JCTVC-M0286 Cross-check of Non-SCE5 [J. Xu (Sony)] [late]

### Motion/partition coding

JCTVC-M0070 Motion-based adaptive partition technique with an application on SHVC [J. Zan (Huawei)]

JCTVC-M0403 Cross-check of Motion-based adaptive partition technique with an application on SHVC (JCTVC-M0070) [A. Alshin, E. Alshina] [late] [miss]

### Modifications to ref_idx scheme

JCTVC-M0065 On collocated picture and low-delay checking for SHVC ref_idx framework [Y. Lin, X. Zheng, X. Chen, J. Zheng (HiSilicon)]

JCTVC-M0240 Non-SCE5: Crosschecking Hisilicon’s JCTVC-M0065 On collocated picture and low-delay checking for SHVC ref_idx framework [X. Li (Qualcomm)] [late]

JCTVC-M0135 On motion field compression in RefIdx mode [T.-D. Chuang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-M0361 Cross-check of motion field compression in RefIdx mode (JCTVC-M0135) [P. Onno (Canon)] [late]

JCTVC-M0192 Improved temporal motion vector prediction for reference index based SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

JCTVC-M0276 Cross-check results of JCTVC-M0192 [H. Lee, J. W. Kang, J. Lee (ETRI)] [late]

JCTVC-M0258 Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC [K. Misra, J. Zhao, A. Segall (Sharp)]

JCTVC-M0365 Cross-verification of Modified Motion Vector Signalling and Prediction Under Reference Index Based SHVC from Sharp [X. Xiu (InterDigital)] [late]

### Special Up-/downsampling filters

JCTVC-M0035 AHG13: Slice based upsampling filter for improved error resiliency [K. Ugur, M. M. Hannuksela (Nokia)]

JCTVC-M0051 AHG12: Low Complexity Resampling Filters For SHVC [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

JCTVC-M0188 Upsampling based on sampling grid information for aligned inter layer prediction [J. Dong, Y. Ye, Y. He (InterDigital)]

JCTVC-M0261 Crosscheck: Up-sampling based on sampling grid information for aligned inter layer prediction [W. Pu (Qualcomm)] [late]

JCTVC-M0231 AHG13: Signaling phase offset for upsampling in SHVC [K. Ugur, J. Lainema (Nokia)]

JCTVC-M0263 AHG13: SHVC Upsampling with phase offset adjustment [K. Minoo, D. Baylon, A. Luthra] [late]

JCTVC-M0322 Signaling of Phase Offset in Up-sampling Process [L. Guo, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-M0356 Cross-check for JCTVC-0322 signaling of phase offset in up-sampling process [J. Dong, Y. Ye (InterDigital)] [late] [miss]

JCTVC-M0425 About phase calculation and up-sampling filter coefficients in JCTVC-M0188 and JCTVC-M0322 [E. Alshina, A. Alshin] [late]

### Transforms in SHVC

JCTVC-M0033 On secondary transforms for Intra_BL residue [A. Saxena, F. Fernandes (Samsung)]

JCTVC-M0307 Cross-check report of On secondary transforms for Intra_BL residue (JCTVC-M0033) [L. Guo (Qualcomm)]

### Other scalable modalities

JCTVC-M0039 On lossless coding with SHVC [K. Ugur, H. Roodaki (Nokia)]

JCTVC-M0176 [AHG16] Analysis of Single-Loop SNR Scalability using Binary Residual Refinement Coding [Christian Feldmann, Fabian Jäger, Mathias Wien (RWTH Aachen University)]

JCTVC-M0279 AHG16: Cross check report for JCTVC-M0176 analysis of single-loop SNR scalability using binary residual refinement coding [Kiran Misra, Andrew Segall (Sharp)] [miss]

JCTVC-M0197 AHG14: Color Gamut Scalable Video Coding using 3D LUT [Philippe Bordes, Pierre Andrivon, Roshanak Zakizadeh (Technicolor)]

JCTVC-M0363 AHG14: Cross-check of Color Gamut Scalable Video Coding using 3D LUT (JCTVC-M0197) [E. François (Canon)] [late]

JCTVC-M0214 AHG9/ AHG14: On Color Gamut Scalable Video Coding [S. Deshpande, L. Kerofsky, A. Segall (Sharp)]

JCTVC-M0414 AHG15: Inter-layer motion-vector prediction using AVC base layer [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

## HL syntax for single-layer HEVC coding (5)

JCTVC-M0042 No display SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0146 VUI extension and SEI for chroma sampling filter [T. Chujoh (Tosiba), K. Kazui (Fujitsu Lab.), P. Topiwala, W. Dai, M. Krishnan (FastVDO LCC.)]

JCTVC-M0281 Additional experiment results for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [Y. Zhang, Y. Wu, S. Kanumuri, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft)]

JCTVC-M0126 Spatial substream definition [M. Arena (RAI), P. Sunna (RAI), G. Ballocca (Sisvel Technology)]

JCTVC-M0181 SEI message: independently decodable regions based on tiles [Y. Ye, Y. He, Y. He (InterDigital), X. Yang, P. Yue, Y. Zhang (Huawei), M. Horowitz (eBrisk Video)]

JCTVC-M0235 Motion-constrained tile sets SEI message [Y. Wu, G. J. Sullivan, Y. Zhang (Microsoft)]

## HL syntax in SHVC and 3D extensions (61)

### Generic high-level syntax issues (7)

JCTVC-M0045 High-level syntax modifications for HEVC extensions [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0046 High-level syntax modifications for SHVC [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0164 Layer identifier extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0201 MV-HEVC/SHVC HLS: On access unit definition [M. M. Hannuksela (Nokia)]

JCTVC-M0208 AHG9: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

JCTVC-M0264 AHG9: On some fundamental concepts in HEVC 3DV and scalable extensions [Y.-K. Wang, J. Chen, Y. Chen (Qualcomm)]

JCTVC-M0266 AHG9: On cross-layer alignments in HEVC 3DV and scalable extensions [Y.-K. Wang, A. K. Ramasubramonian, K. Rapaka, J. Chen, V. Seregin, Y. Chen (Qualcomm)]

### Random access and layer switching structures (6)

JCTVC-M0168 On random access point for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0170 Temporal layer management for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0199 On flexible stream switching [Patrick Lopez, Philippe Bordes, Franck Hiron (Technicolor)]

JCTVC-M0206 MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process [M. M. Hannuksela (Nokia)]

JCTVC-M0207 MV-HEVC/SHVC HLS: Step-wise layer access (STLA) pictures [M. M. Hannuksela (Nokia)]

JCTVC-M0096 AHG 9: On enhancement layer CRA [C. Kim, Hendry, B. Jeon (LGE)]

### Parameter sets (10)

JCTVC-M0153 Extension parameter set [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0163 On video parameter set extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0167 Parameter set property [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)] [late]

JCTVC-M0179 AHG9: APS for inter-layer processing signaling [Y He, J Dong, Y Ye (??)]

JCTVC-M0310 Cross-check of JCTVC-M0179 on APS for inter-layer processing signaling [H. Yang (Huawei)] [late]

JCTVC-M0211 AHG9: On Scaling List Signalling for SHVC [S. Liu, X. Zhang, S. Lei (MediaTek), K. Sato (Sony)]

JCTVC-M0268 AHG9: VPS and SPS designs in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

JCTVC-M0134 AHG9: Inter-layer SPS prediction for HEVC extensions [T. Rusert (Ericsson)]

JCTVC-M0140 AHG9: Inter-layer RPS Prediction [S. Lu, K. Sato(Sony)]

JCTVC-M0165 On parameter set prediction [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

### Signalling for inter-layer processing (7)

JCTVC-M0152 Slice level inter-layer prediction signalling [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0205 MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications [M. M. Hannuksela, K. Ugur (Nokia)]

JCTVC-M0129 AHG 9: Inter-layer Prediction Indication at Picture Level [Hendry, B. Jeon (LG), D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0151 AHG9: Inter-layer Prediction Signaling [K. Sato(Sony), T.-D. Chuang(MediaTek)]

JCTVC-M0203 AHG 9: Signalling Inter-layer Prediction Indication [Hendry (LG), M. M. Hannuksela, K. Ugur (Nokia), D.-K. Kwon (TI)]

JCTVC-M0209 AHG9: On Design for Signaling Inter-layer Prediction [S. Deshpande (Sharp)]

### Reference picture signalling and management (8)

JCTVC-M0081 AHG9: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

JCTVC-M0104 AHG 9: On order of inter-layer reference pictures in reference picture lists [Hendry, B. Jeon (LG)]

JCTVC-M0105 AHG 9: On position of inter-layer reference pictures in reference picture lists [Hendry, B. Jeon (LG)]

JCTVC-M0148 Signalling inter-layer reference picture positions [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0150 Inter-layer reference picture set initialization [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0161 Reference picture marking process for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0162 Unused reference picture management [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0269 AHG9: Signalling and derivation of inter-layer RPS for HEVC 3DV and scalable extensions [Y.-K. Wang, J. Chen, K. Rapaka, Y. Chen, X. Li, M. Karczewicz (Qualcomm)]

### Signalling of cropped inter-layer reference (3)

JCTVC-M0180 On base layer video output in SHVC [Y He, J Dong, Y He (??)]

JCTVC-M0219 Reference-layer cropping offsets signaling in SHVC [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0309 Signaling of extended spatial scalability for SHVC [A. K. Ramasubramonian, X. Li, Y. Chen, V. Seregin (Qualcomm)]

JCTVC-M0274 AHG9: On conformance-cropping-window-restricted inter-layer prediction [Y.-K. Wang, V. Seregin, J. Chen, X. Li, Y. Chen (Qualcomm)]

### Inter-layer slice header prediction (2)

JCTVC-M0171 Slice header design for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0218 Inter-layer slice header syntax elements prediction in SHVC and MV-HEVC [D.-K. Kwon, M. Budagavi (TI)]

### Parallel processing (4)

JCTVC-M0198 Independent tile upsampling for SHVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

JCTVC-M0200 Layer decoding delay indication for SHVC and 3D/MV-HEVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

JCTVC-M0202 Indication of tile boundary alignment [K. Suehring, R. Skupin, T. Schierl (Fraunhofer HHI)]

JCTVC-M0334 Parallel Processing Indications for Tiles in HEVC Extensions [K. Rapaka, W. Pu, X. Li, J. Chen, Y.-K. Wang, M. Karczewicz (Qualcomm)]

### Special functionalities (3)

JCTVC-M0040 AHG9: Using SHVC for adaptive resolution change and efficient trick mode [K. Ugur, H. Roodaki, M. M. Hannuksela (Nokia)]

JCTVC-M0204 Ultra-low delay multi-layer decoding by interleaving of decoding units in SHVC and 3D/MV-HEVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

JCTVC-M0277 A Use Case and High Level Syntax of SHVC: Scalability Based Region of Interest [C. Auyeung, J. Xu, O. Nakagami, A. Tabatabai (Sony)]

### Sub-bitstream extraction (2)

JCTVC-M0131 AHG 9: Sub-bitstream extraction for pictures not needed for inter-layer prediction [Hendry, B. Jeon (LG)]

JCTVC-M0272 AHG9: Multi-mode bitstream extraction in HEVC 3DV and scalable extensions [Y.-K. Wang, Y. Chen, A. K. Ramasubramonian (Qualcomm)]

### Hypothetical reference decoder (HRD) and DPB management (5)

JCTVC-M0169 Decoded picture buffer for HEVC extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0270 AHG9: On DPB operations in HEVC 3DV and scalable extensions [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCTVC-M0130 AHG 9: Signalling required DPB size for in layer set [Hendry, B. Jeon (LG), D.-K. Kwon, M. Budagavi (TI)]

JCTVC-M0210 AHG9: On Signaling DPB Parameters and DPB Operation [S. Deshpande (Sharp)]

JCTVC-M0254 Consideration of buffer management issues and layer management in HEVC scalability [Sam Narasimhan, Ajay Luthra]

### SEI messages (4)

JCTVC-M0042 No display SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0043 Layers present SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0044 Layers display info SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

JCTVC-M0212 AHG9: On Output Layer Sets Change Signaling [S. Deshpande (Sharp)]

## Interlaced scan and field-based video coding

## Non-normative: Encoder optimization, decoder speed improvement, post filtering, loss concealment, rate control

### Rate control

JCTVC-M0036 Adaptive bit allocation for R-lambda model rate control in HM [B. Li, H. Li, L. Li (USTC)]

JCTVC-M0037 Rate control by R-lambda model for SHVC [L. Li, B. Li, H. Li (USTC)]

JCTVC-M0415 Crosscheck for Rate control by R-lambda model for SHVC (JCTVC-M0037) [W. Zhang, Y. Chiu (Intel)] [late]

JCTVC-M0257 Intra Frame Rate Control Based on SATD [M. Karczewicz, X. Wang (Qualcomm)]

### Encoder optimization

JCTVC-M0259 Encoder Optimization for SHVC: Enhancement Layer Lambda Refinement in RA Configurations [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-M0343 Cross-check for Encoder Optimization for SHVC: Enhancement Layer Lambda Refinement in RA Configurations [E. Alshina] [late]

## Withdrawn and unclear allocation

JCTVC-M0038 [Withdrawn]

JCTVC-M0050 [Withdrawn]

JCTVC-M0248 [Withdrawn]

JCTVC-M0332 [Withdrawn]

JCTVC-M0227 [Withdrawn]

JCTVC-M0228 Cross-check Non-SCE [P. Lei (Samsung)] [miss]

# Plenary Discussions and BoG Reports

## Project development

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be 7 Jan. 2013.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-K1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the HM 9.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-K1100.

## Software development (to be updated)

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 9.0 should be available within 2 weeks after the meeting.

- Availability of the range extensions software is expected 1 week after HM 9.0.

- HM 9.1 is planned to be available 4 weeks after HM 9.0.

## Subjective verification test plan

Subjective verification is planned to be performed after finalization of standard. Contributions and planning toward the conduct of that testing should begin ASAP.

# Establishment of ad hoc groups (to be updated)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC draft and test model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-L1002 HEVC Test Model 10 (HM 10) Encoder Description. - Produce and finalize JCTVC-L1003 HEVC text specification Draft 10 / FDIS & Consent text. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCT-VC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 10.0 software version and the reference configuration encodings according to JCTVC-L1100 based on common conditions (expected within 1 week after the meeting). - Prepare and deliver HM 10.1 software and additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Study to develop a potential set of HEVC conformance bitstreams. | T. Suzuki (chair), C. Fogg, , W. Wan (vice-chairs) | N |
| HEVC range extensions development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats and bit depths beyond 8 bits. - Discuss and propose test conditions and test material for the development of the range extensions. - Study techniques for colour conversion and resampling and their relationship to non-4:2:0 chroma coding. | C. Rosewarne (chair) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-L1005 HEVC RExt draft text (HM 10-RExt-2). - Gather and address comments for refinement of the text. - Coordinate with AHG7. | J. Sole (primary), D. Flynn, C. Rosewarne, T. Suzuki | N |
| Range extensions software development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution to JCT-VC members. - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 10.0-RExt-2.0 software version and the reference configuration encodings according to JCTVC-L1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6. | D. Flynn, K. Sharman | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Consider needs for lossless coding and screen content coding support in the range extensions. - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for screen content coding. - Make recommendations for test conditions for screen content coding. - Coordination with finalization of the test conditions for RCE2 on lossless coding. | H. Yu (chair), M. Budagavi, R. Cohen, A. Duenas, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify commonality of multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study signalling of inter-layer prediction constraints by taking JCTVC-L0449 as a starting point. - Study methods for reduction of SPS redundancy. - Study SEI messages and VUI syntax designs. - Study the hypothetical reference decoder (HRD) syntax and operations and the related text for bitstream conformance and decoder conformance. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. | M. M. Hannuksela (chair), J. Boyce, Y.-K. Wang, T. Rusert, Y. Chen (vice-chairs) |  |
| SHVC core experiments (AHG10) (jct-vc@lists.rwth-aachen.de) - Establish configurations for SHVC core experiments - Generate anchors used for SHVC core experiments - Create reporting sheets for core experiments - Provide configuration data two weeks after software is available - Discuss and identify additional issues related to SHVC tool experiments | X. Li (chair), J. Boyce, P. Onno, Y. Ye. (vice-chairs) | N |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-L1007 SHVC Test Model 1 (SHM 1). - Produce and finalize JCTVC-L1008 SHVC text specification Draft 1. - Gather and address comments for refinement of these documents. - Coordinate with the SHVC Software (SHM) development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela Y. Ye, (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare software (based on HM8.1) for experimentation. - Provide software to CEs within two weeks after the meeting. - Bring software into alignment with HM 10 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), T. Chuang, Y. He, D. Kwon (vice-chairs) | N |
| SHVC upsampling and downsampling filters (AHG13) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability. - Study phase relationships between upsampling and downsampling filters. - Study resampling filters used in existing scalable systems. - Study resampling filters with ratios other than 1.5 and 2.0. - Discuss and identify additional issues related to upsampling or downsampling filters. | A. Segall (chair), E. Alshina. J. Chen, J. Dong, P. Topiwala, M. Zhou (vice-chairs) | N |
| Colour gamut scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut scalable coding. - Study the interaction of colour gamut and bit-depth scalability. - Study the interaction of colour gamut and spatial scalability. - Identify test sequences and test conditions. - Discuss and identify additional issues related to colour gamut scalability. | A. Segall, A. Duenas, and D.-K. Kwon (co-chairs) | N |
| Hybrid codec scalability (AHG15) (jct-vc@lists.rwth-aachen.de) - Coordinate with AHG10 on software and anchor support for AVC base layer - Study the rate distortion benefits, complexity impact, and practicality of using inter-layer syntax prediction from AVC base layer - Study and compare the rate distortion benefits of SHVC HEVC base layer coding tools for inter-layer texture prediction and inter-loop filtering on AVC base layer - Study signalling methods of AVC base layer properties in SHVC high-level syntax. | J. Boyce, K. Kawamura (co-chairs) | N |
| Single-loop scalability (AHG16) (jct-vc@lists.rwth-aachen.de) - Study and compare single-loop and multi-loop scalability (focus on SNR as the more demanding case). - Analyze and compare the memory bandwidth of single-loop and multi-loop scalability. - Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signalling inter-layer prediction restrictions on a multi-loop design. - Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability. | M. Wien (chair), J. Boyce, M. Budagavi, K. Misra, K. Ugur (co-chairs) | N |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them - Study restrictions to reduce memory bandwidth and complexity in scalable tools (e.g. restrictions on PU size, subpel accuracy, 1D vs. 2D interpolation filters, use of bi-predictive PUs, reduction in the number of EL reference pictures etc.) - Evaluate the impact of such restrictions on coding efficiency. | M. Budagavi (chair), E. Alshina, E. Francois, M. Karczewicz, A. Tabatabai, Y. Ye. (vice-chairs) | N |

# Output documents (to be updated)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-L1000 Meeting Report of 12th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Suehring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

Disposition of comments report

JCTVC-L1002 High Efficiency Video Coding (HEVC) Test Model 10 (HM 10) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N 13343) [2013-04-01]

JCTVC-L1003 High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent) [B. Bross (primary), W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand] (WG 11 N 13333) [2013-03-01]

JCTVC-L1004 HEVC Conformance Draft 2 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 13352 subsection for conformance, ISO/IEC CD) [2013-03-22]

JCTVC-L1005 HEVC Range Extensions Draft 2 [D. Flynn, J. Sole, T. Suzuki] (WG 11 N 13345, ISO/IEC PDAM) [2013-03-22]

JCTVC-L1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, K. Sharman] [2013-02-08]

JCTVC-L1007 SHVC Test Model 1 (SHM 1) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13350) [2013-02-15]

JCTVC-L1008 SHVC Working Draft 1 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13349) [2013-03-22]

JCTVC-L1009 Common SHM test conditions and software reference configurations [X. Li, J. Boyce, P. Onno, Y. Ye] [2013-02-08]

JCTVC-L1010 HEVC HM 10 Reference Software [F. Bossen, D. Flynn, K. Suehring] (WG 11 N 13352 subsection for reference software, ISO/IEC CD) [2013-03-22]

JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Alignment of the test configurations with the profile names is planned.)

- JCTVC-L1101 HEVC Scalable Extensions Core Experiment SCE1: Intra prediction improvements [A. Tabatabai, K. Rapaka, A. Saxena, S. Liu (TE Coordinators)]

- JCTVC-L1102 HEVC Scalable Extensions Core Experiment SCE2: Inter-layer texture prediction signalling and deblocking [L. Guo, Y. He, D. Kown, J. Zan, J. Kang (TE Coordinators)]

- JCTVC-L1103 HEVC Scalable Extensions Core Experiment SCE3: Combined inter- and interlayer prediction [X. Li, E. Francois, P. Lai, D. Kwon, A. Saxena (TE coordinators)]

- JCTVC-L1104 HEVC Scalable Extensions Core Experiment SCE4: Inter-layer filtering [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park (TE coordinators)]

- JCTVC-L1105 HEVC Scalable Extensions Core Experiment SCE5: Inter-layer syntax prediction using HEVC base layer [V. Seregin, P. Onno, S. Liu, E. Alshina, C. Kim, H. Yang (TE coordinators)]

JCTVC-L1101 through JCTVC-L1105 were pre-reviewed only in BoG activity. A two week finalization period was authorized.

JCTVC-L1121 HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods [J. Sole (primary), K. Kawamura, J. Kim]

JCTVC-L1122 HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding [W. Gao (primary), M. Zhou, P. Amon, S. Lee]

JCTVC-L1121 and JCTVC-L1122 were pre-reviewed only in BoG activity. A two week finalization period was authorized.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 25 July – 2 Aug. 2013 under WG 11 auspices in Vienna, AT.

- 24 Oct. – 1 Nov.2013 under ITU-T auspices in Geneva, CH.

NOTE – Some AHG meetings, e.g. on HLS, are likely to be held on 23 Oct. 2013.

- 9–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- 27 Mar. – 4 Apr. 2014 under WG 11 auspices in Valencia, ES.

- 17–25 June 2014 under ITU-T auspices in Geneva, CH.

The agreed document deadline for the July 2013 meeting is Monday XX July. Moreover, it was agreed to start the meeting with SHVC CE work, and not to address RExt or Version 1 topics at the meeting until XXXX.

XXX was thanked for the excellent hosting of the 13th meeting of the JCT-VC. XXX was thanked for providing viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately XXXX hours on Fri. 26 Feb. 2013.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the eleventh meeting of the JCT-VC, according to badge pick-up records produced by the ITU staff (approximately XXX in total), were as follows: