| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N_Notes_dF |
| --- | --- |

| Title: | Meeting report of the 14th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Vienna, AT, 25 July – 2 Aug. 2013 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourteenth meeting during 25 July – 2 Aug. 2013 at the Event Hotel Pyramide, Vösendorf, Vienna, AT. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 25 July 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1345 hours on Friday 02 Aug. 2013. Approximately 162 people attended the JCT-VC meeting, and approximately 350 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the thirteenth JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered, and a verification test plan was set up for HEVC version 1 performance testing.

In addition to 7 experiment plan descriptions (3 for HEVC range extensions and 4 for SHVC), the JCT-VC produced 10 other particularly important output documents from the meeting: the HEVC test model (HM) 12 encoder description, HM 12 reference software (issued as an ISO/IEC DIS), draft 4 for HEVC conformance testing  (issued as an ISO/IEC DIS), an HEVC Defect Report (for Version 1), draft 4 for HEVC range extensions, draft 3 of the SHVC extensions, the SHVC test model (SHM) 3, HEVC verification test plan draft 1, and two documents specifying common test conditions and software reference configurations for experiments (one for HEVC range extension experiments, and one for scalable coding experiments).

For the organization and planning of its future work, the JCT-VC established 22 "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 23 Oct. – 1 Nov. 2013 under ITU-T auspices in Geneva, CH, 9–17 Jan. 2014 under WG 11 auspices in San José, US, 27 March – 4 Apr. 2014 under WG 11 auspices in Valencia, ES, and 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourteenth meeting during 25 July–02 Aug. 2013 Event Hotel Pyramide, Vösendorf, Vienna, AT. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). Some particular discussions were chaired by other participants, as noted in this report.

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 25 July 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1345 hours on Friday 2 Aug. 2013. Approximately 162 people attended the JCT-VC meeting, and approximately 350 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2013_07_N_Vienna/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the thirteenth JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered, and a verification test plan was set up for HEVC version 1 performance testing.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 15 July 2013 (the Monday of the week preceding the meeting).

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 16 July 2013 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-N0295 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "N0295+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-N0115 (a proposal relating to color conversion) [uploaded 07-23]

- JCTVC-N0116 (a proposal relating to RGB 4:4:4 coding) [uploaded 07-18]

- JCTVC-N0133 (a proposal relating to FPA SEI message) [uploaded 07-18]

- JCTVC-N0134 (a proposal relating to FPA SEI message) [uploaded 07-18]

- JCTVC-N0135 (a proposal relating to HLS on maximum number of layers) [uploaded 07-19]

- JCTVC-N0167 (a proposal relating to transform skip coding) [uploaded 07-19]

- JCTVC-N0170 (a proposal relating to RCE1 results) [uploaded 07-19]

- JCTVC-N0264 (a proposal relating to VPS for multi-format) [uploaded 07-19]

- JCTVC-N0283 (a proposal relating to aspect ratio scalability) [uploaded 07-21]

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-N0179 (contribution on medical test sequences) [uploaded 07-30]

- JCTVC-N0284 (an editors’ contribution on conformance specification) [uploaded 07-22]

The following cross-verification reports were registered on time but were uploaded late: JCTVC-N0103 [uploaded 07-18], JCTVC-N0104 [uploaded 07-18], JCTVC-N0114 [uploaded 07-25], JCTVC-N0123 [uploaded 07-24], JCTVC-N0125 [uploaded 07-24], JCTVC-N0126 [uploaded 07-24], JCTVC-N0136 [uploaded 07-18], JCTVC-N0156 [uploaded 07-22], JCTVC-N0171 [uploaded 07-19], JCTVC-N0194 [uploaded 07-17], JCTVC-N0203 [uploaded 07-27], JCTVC-N0208 [uploaded 07-20], JCTVC-N0221 [uploaded 07-19], JCTVC-N0225 [uploaded 07-20], JCTVC-N0226 [uploaded 07-20], JCTVC-N0251 [uploaded 07-17], JCTVC-N0253 [uploaded 07-26], JCTVC-N0257 [uploaded 07-22], JCTVC-N0259 [uploaded 07-22], JCTVC-N0260 [uploaded 07-23], JCTVC-N0262 [uploaded 07-25], JCTVC-N0274 [uploaded 07-27], JCTVC-N0277 [uploaded 07-22], JCTVC-N0278 [uploaded 07-25], JCTVC-N0279 [uploaded 07-17], JCTVC-N0280 [uploaded 07-18], JCTVC-N0289 [uploaded 07-25], JCTVC-N0293 [uploaded 07-25].

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-N0068, JCTVC-N0166, JCTVC-N0317, JCTVC-N0324 (re-registered as JCTVC-N0378), JCTVC-N0345, JCTVC-N0347.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- This issue did not apply for documents of the 14th meeting

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-M1000, the HEVC Test Model (HM) JCTVC-M1002, the Conformance Draft JCTVC-M1004, the Reference Software Draft JCTVC-M1010, the Draft Specification of Range Extensions JCTVC-M1005, the SHVC draft specification JCTVC-M1008 and SHVC test model 2 (SHM2) JCTVC-M1007 were approved. The HM reference software produced by the AHG on software development, the reference software versions for range extensions and SHVC, and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. Additionally, development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC JTC 1.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GRP: Generalized residual prediction.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- Unit types:

  - CTB: code tree block (the luma or chroma array of a CTU).

  - CTU: code tree unit (synonymous with LCU).

  - CB: coding block (luma or chroma).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PB: prediction block

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: transform block

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a draft standard.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

The status of HEVC version 1 in ISO/IEC and ITU-T was noted. In ITU-T, the text was approved on 2013-04-13 and published as Rec. H.265 on 2013-06-07. The FDIS 23008-2 ballot period had ended on 2013-07-14, and formal publication was expected to follow shortly.

The HEVC conformance testing and reference software specification had been submitted as ISO/IEC CD 23008-5, but had been planned at the preceding meeting to be split into two separate parts for software (23008-5) and conformance (23008-8) at the DIS stage, which was reflected by issuing an ISO/IEC CD ballot for 23008-5 at the preceding meeting. The DIS ballot text was planned to be produced at this meeting.

ISO/IEC 23008-8 conformance ballot results were reported in the WG 11 document m29645.

ISO/IEC 23008-5 software ballot results were reported in the WG 11 document m29941.

The range extensions draft 2 had been submitted as ISO/IEC 23008-2/PDAM1. The ballot closing date was 2013-07-07, and ballot results were reported in the WG 11 document m29644.

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS / DAM ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

The goals of the meeting were reviewed. These included progress of work on HEVC extensions, conformance, and reference software ("Study of", in ISO/IEC). It was noted that plans for verification testing of HEVC version 1 should be established.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with topics of SHVC HLS on the first day, SHVC CE work and related contributions starting on the second day, and RExt and other topics starting on the third day. Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- First day (Thu. 25 July): 0900–2000 (approximately), with plenary in the morning and Tracks A and B in the afternoon (for general HLS and SVC-specific HLS)

- Second day (Fri. 26 July): 0900–2000 (approximately), start with plenary relating to AHGs for SVC CEs, then Tracks A and B (and BoG on HLS), with one track working on SCEs and the other on HLS.

- Third day (Sat. 27 July): 0900-1300 J. B. BoG & Track B (A. Segall), 1200-1315 Track A (GJS, AHGs and RExt), p.m. Track B (A. Segall) & ESS BoG., 14:45 Track A (GJS).

- Fourth day (Sun 28 July): 0900 Track B (A. Segall)

- Fifth day (Mon 29 July): Morning MPEG plenary,

  - Morning MPEG plenary

  - 1400: Continuation

- Sixth day (Tue 30 July):

- Seventh day (Wed 31 July):

  - Morning MPEG plenary

  - 1130: Continuation

  - Stopping time 1700 (for social event)

- Eighth day (Thu 1 Aug):

- Ninth day (Fri 2 Aug):

  - 0800 Morning wrap-up

  - 1300 End

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Substitute chairs and break-out-group coordinators were sometimes selected for particular topics as noted. (Note: allocation to tracks were subject to changes.)

- AHG reports (21) Track P (section 2)

- Conformance testing development (1) Track P (section 3.1)

- Version 1 bug reports and cleanup (4) Track P (section 3.2)

- Coding performance, implementation, and design analysis (3) Track P (section 3.3)

- Profile and level definitions (4) Track P (section 3.4)

- HEVC and RExt use cases (1) Track P (section 3.5)

- Source video test material (2) Track P (section 3.6)

- SHVC CE1: Support for additional resampling phase shifts (9) Track B (section 4.1)

- SHVC CE2: Combined inter-layer syntax prediction and motion data compression (15) Track B (section 4.2)

- SHVC CE3: Inter-layer filtering (15) Track B (section 4.3)

- RExt CE1: Inter-component decorrelation (6) Track A (section 5.1)

- RExt CE2: Prediction and coding techniques for transform skip blocks (31) Track A (section 5.2)
	BoG N0388 on combination of tools in RCE2 (J. Sole)

- RExt CE3: Intra coding methods for screen content (9) Track A (section 5.3)

- Non-CE RExt (56 CE related, 30 other) Track A (section 6.1)
	BoG N0384 on non-RCE2 contributions (R. Joshi, R. Cohen)

- Non-CE SHVC (20 CE related, 30 other) Track B (section 6.2)
	BoG N0387 on color gamut scalability (A. Segall)
	BoG N0375 on arbitrary scalability ratios (E. Francois)

- High-level syntax in RExt & single layer (4) Track A (section 6.3)

- High-level syntax in SHVC and 3D extensions (74) BoG | joint with JCT-3V (section 6.4)
	BoG N0373 on alignment, etc. (J. Boyce)

- High-level syntax in SHVC (10) BoG | Track A (section 6.5)
	J-BoG N0374 on SHVC/MV-HEVC HLS topics (J. Boyce)

- SEI messages (15) BoG N0374 | Track A (section 6.6)

- Non-normative (3) Track A (section 6.7)

- Withdrawn and unclear category (0) (section 6.8)

- Plenary discussions and BoG reports (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC. (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: 36; Track A: 108; Track B: 54; HLS J-BoG: 74.

# AHG reports

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

JCTVC-N0001 Ad hoc group report: JCT-VC project management [G. J. Sullivan, J.-R. Ohm]

(Reviewed Thu. 25th a.m. plenary)

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

The primary goals of the meeting were to review the work that was performed in the interim period since the 13th JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, and further consider on possible corrective actions. Other important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. In this context, three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx) had been conducted. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal.

The work of the JCT-VC overall had proceeded well and actively in the interim period. Active discussion had been carried out on the group email reflector (which had 1750 subscribers as of 2013-07-24), and the output documents from the preceding meeting had been produced.

Except as noted below, all report documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2013_04_M_Incheon/), particularly including the following:

- The meeting report (JCTVC-M1000) [Posted 2013-07-25]

- The HEVC software guidelines (JCTVC-H1001) [Not updated for several meeting cycles]

Note: The "H" document, in this case, remains valid as the latest approved software guidelines.

- The HM 11 encoder description (JCTVC-M1002) [Posted 2013-07-23]

- HEVC conformance specification Working Draft 3, submitted as ISO/IEC CD (JCTVC-M1004) [Posted 2013-05-10]

- HEVC range extensions Working Draft 3 (JCTVC-M1005) [First posted 2013-06-15, last updated 2013-06-21]

- HEVC range extensions common test conditions and software reference configurations (JCTVC-L1006). [The “L” document remained the most recent issued document number, although a small update was posted 2013-06-04.]

- SHVC Test Model 2 (JCTVC-M1007) [Posted 2013-06-06]

- SHVC Working Draft 2 (JCTVC-M1008) [First posted 2013-05-21, last updated 2013-06-14]

- Common SHM test conditions and software reference configurations (JCTVC-M1009) [Posted 2013-05-07, last updated 2012-05-23]

- HEVC HM11 Reference Software, submitted as ISO/IEC Study of CD (JCTVC-M1010) [Posted 2013-06-14]

- Description of HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts (JCTVC-M1101) [Posted 2013-05-18]

- Description of HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression (JCTVC-M1102) [Posted 2013-05-20 with final update 2013-06-24]

- Description of HEVC Scalable Extensions Core Experiment 3 (SCE3): Inter-layer filtering (JCTVC-M1103) [First posted 2013-05-18 with final update 2013-06-24]

- Description of HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods (JCTVC-M1121) [First posted 2013-05-17 with final update 2013-06-25]

- Description of HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks (JCTVC-M1122) [First posted 2013-04-26 with final update 2013-06-04]

- Description of HEVC Range Extensions Core Experiment 2 (RCE2): HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content (JCTVC-M1123) [First posted 2013-05-01 with final update 2013-05-29]

The current HM reference software manual has been continually updated along with the software itself, and a version-controlled copy is now included in the "doc" directory of the HM software repository. A PDF version has been produced and is included in the same location prior to each HM release.

The various ad hoc groups and the six core experiments had made progress, and various reports from those activities had been submitted.

The software for HM version 11.0 had been prepared and released approximately as scheduled. Progress was made to build the software for SHVC and Range Extensions on top of HM11 as well.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-11.0.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-11.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 330 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted. However, the relatively early upload deadline established for the current meeting (2013-07-15, ten days in advance of the meeting) has helped to enable advance study of most technical input material.

A preliminary basis for the document subject allocation and meeting notes for the 14th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted web site.

JCTVC-N0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs)]

(Reviewed Sat. 27 July a.m. Track A (GJS).)

At the 13th JCT-VC meeting, a document detailing the Editors' proposed corrections to HEVC version 1 was endorsed as representing the current state of development for errata corrections to the HEVC version 1 specification.  In addition, the 11th HEVC test model (HM11) was developed from the 10th HEVC test model (HM10).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents. A total of 10 issues with the HEVC version 1 specification were reported on the tracker following the 13th JCT-VC meeting. No issues were reported on the tracker with the HM10 Encoder Description during this period.

The HM11 Encoder Description was published as JCTVC-M1002.

Further editorial changes to HEVC version 1 have been proposed for consideration at the 14th JCT-VC meeting in N0041 and N0346.

It was reported that the HM encoder description is not getting adequate attention to bring it into appropriate condition for publication as part of a standard.

The coordinators indicated that they were waiting for input for the interlace handling non-normative enhancement that was planned to be integrated.

It was noted that the RExt software and SHVC software and 3V HTM need to track HM development to prevent code drift, and to be coordinated between RExt and SHVC and HTM.

JCTVC-N0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring]

(Reviewed Sat. 27 July p.m. Track A (GJS).)

A brief summary of activities related to each mandate is given below.

Development of the software was co-ordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN servers set up at HHI and the BBC, as announced on the JCT-VC email reflector.

The HM user manual has been updated and a version controlled copy is included in the doc directory of the repository. A PDF version has been produced and is included in the same location prior to each HM release.

Version 11.0 of the software was delivered to schedule and reference configuration encodings were provided according to the common test conditions through an ftp site hosted by the BBC.

ftp://ftp.kw.bbc.co.uk/hevc/hm-11.0-anchors/

Version 11.1 is still in development to be released during the 14th JCT-VC meeting. A number of bugs have been identified and fixed including an issue that allowed the encoder to generate non-conforming bitstreams.

There are a number of reported software bugs that should be fixed.

One non-normative change, which was pending from the 12th JCT-VC meeting, is still pending.

Improvements to the stability of the bug tracking system have been made which will allow rolling out additional bug trackers for SHVC and 3D-AVC.

Multiple versions of the HM software were produced and announced on the JCT-VC email reflector. The following sections give a brief summary of the changes made for each version. A detailed history of all changes made to the software can be viewed at https://hevc.hhi.fraunhofer.de/trac/hevc/timeline.

Released versions of the software are available on the SVN server at the following URL:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-11.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-11.0-dev).

Version 11.0: HM 11.0 has been released on June 17, 2013. It includes all the (non-normative) changes adopted at the 13th JCT-VC meeting. It also contains bug fixes and will print warnings if no or invalid profiles or levels are selected in the configuration settings. This release was announced on the email reflector.

The coding performance did not change compared to HM-10.0.

Version 11.1: HM 11.1 has not yet been released, but is expected during the 14th JCT-VC meeting. This version contains additional bug fixes.

One software only change related to field coding is still pending related to issues with the code quality (as reported at the last meeting).

Unless the release has been tagged, the development branch can be found under

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-11.0-dev

There are no performance changes expected for HM 11.1.

Recommendations included the following:

- Continue to develop reference software based on HM version 11.x and improve its quality.

- Remove macros introduced in previous HM versions before starting integration towards HM12.0 such as to make the software more readable

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bitstreams.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bistreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

Bitstreams that trigger bugs in the HM decoder are valuable – both for debugging the HM itself and for use as conformance bitstreams. LTRPs and usage of the output_flag were mentioned as topics that would be valuable to cover. See also JCTVC-N0004.

JCTVC-N0004 JCT-VC AHG Report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan]

(Reviewed Sat. 27 July a.m. Track A (GJS).)

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The list of the candidate of the conformance bitstream and its volunteers were summarized in the report.

So far, 128 bitstreams had been collected. Most of them were updated to the HM10 syntax. However, 5 bitstreams are still based on the older version of the spec. Those bitstreams must be updated based on the final spec of the HEVC, as soon as possible. The problems were reported in 16 bitstreams for HM10.1 based bitstreams and those must be revised too. The problems of bitstreams are summarized in the table below.

| Problems | Bitstreams |
| --- | --- |
| HM10.1 cannot decode (CRA/RPS issues, etc) | ENTP_A_LG_2, ENTP_B_LG_2, TSKIP_A_MS_2, AMP_D_HiSilicon_2, AMP_E_HiSilicon_2 |
| A.4.1 Violation | STRUCT_A_Samsung_4 |
| Profile/level indication | IPRED_C_Mitsubishi_2, TMVP_A_MS_2, SAO_C_Samsung_3, SAO_D_Samsung_3, ENTP_C_LG_3, CAINIT_A_Sharp_3, CAINIT_B_Sharp_3, CAINIT_C_Sharp_2, CAINIT_D_Sharp_2, CAINIT_E_Sharp_2, CAINIT_F_Sharp_2, CAINIT_G_Sharp_2, CAINIT_H_Sharp_2, AMP_F_HiSilicon_2, TSUNEQBD_A_MAIN10_Technicolor_1 |

The generated bitstreams are available at

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

The features and conformance point of each bitstream are summarized in the attached Excel sheet.

The desired features of bitstreams initially discussed are summarized in the report. The missing bitstreams/features are highlighted in yellow.

Most of the significant features are already covered by the existing bitstreams, however, stress bitstreams, corner case bitstreams are still missing. Those are useful to ensure interoperability and therefore missing bitstreams should be collected. The plan, volunteers and logistics, to collect such bitstreams should be discussed in during Vienna meeting.

The following relevant contribution was submitted: JCTVC-M0284 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan]

The AHG recommended

- to discuss the plan to collect missing bitstreams

- to continue to collect more bitstreams, especially corner case bitstreams

It was suggested to issue a "call for additional conformance bitstreams".

- JCTVC-N0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [M. Naccari, C. Rosewarne]

(Reviewed Sat. 27 July a.m. Track A (GJS).)

No significant email discussion.

A revision of the prior output document JCTVC-L1006 was uploaded, defining the test conditions as agreed at the 12th JCT-VC meeting for Range extensions development, with summary averages across colour spaces removed.

Related contributions as of 25 July were identified as follows:

- REXT/MV-HEVC/SHVC HLS: auxiliary picture layers (JCTVC-N0063)

- AHG 5: On support for alpha channel in HEVC (JCTVC-N0077)

- AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions (JCTVC-N0116)

- HLS: Frame packing arrangement SEI message for bit depth extension (JCTVC-N0133)

- AHG5: Square transform deblocking for 4:2:2 (JCTVC-N0138)

- AHG5: on chroma QP for HEVC RExt (JCTVC-N0141)

- On Mode Dependent Intra Smoothing for Range Extension (JCTVC-N0143)

- AHG 5 and 18: Internal Precision for High Bit Depths (JCTVC-N0188)

- AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths (JCTVC-N0189)

- AHG 5 and 18: Entropy Coding Throughput for High Bit Depths (JCTVC-N0190)

- AHG 5 and 18: Profiles for Range Extensions (JCTVC-N0191)

- AHG 5: 32x32 Scaling List Derivation for Chroma (JCTVC-N0192)

- SAO extension for higher bit-depth coding (JCTVC-N0201)

- In-loop Chroma Enhancement for HEVC Range Extensions (JCTVC-N0223)

- AHG5: Modified SAO for range extensions (JCTVC-N0246)

- AhG5: Memory Bandwidth Reduction for HEVC RExt (JCTVC-N0261)

- AhG5: Deblocking Filter in 4:4:4 Chroma Format (JCTVC-N0263)

- Updated proposal with software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams (JCTVC-N0270)

- RExt: Fidelity adaptive coding mode (JCTVC-N0292)

- Signalling of chroma sampling filter (JCTVC-N0309)

- A proposal on HEVC 4:2:2 profile (JCTVC-N0312)

JCTVC-N0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, C. Rosewarne, T. Suzuki]

(Reviewed Sat. 27 July a.m. Track A (GJS).)

The High Efficiency Video Coding (HEVC) Range Extensions test model was developed following the decisions taken at the 13th JCT-VC meeting in Incheon, KR (April 2013).

Two versions of the draft 3 specification text JCTVC-M1005 were published by the Editing AhG following the 13th JCT-VC meeting in Incheon. Both versions were based upon JCTVC-L1003_v34.

One item had not been integrated into the text

Changes in JCTVC-M1005 relative to the previous version were as follows (with issue tracker number, where appropriate):

- Integrated modified IntraPredModeC derivation for 4:2:2 (JCTVC-M0127)

- Integrated horizontal and vertical intra DPCM for transquant bypass (JCTVC-M0056)

- Integrated the no display SEI message (JCTVC-M0042)

- Integrated the motion-constrained tile sets SEI message (JCTVC-M0235)

Note that integration of SAO offset scaling adjustment for higher bitdepths has been deferred (JCTVC-M0335).

In regard to the SAO offset scaling issue, the JCT-VC confirmed that this should not be integrated, based on our current understanding of the technical situation (pending contributions indicating otherwise).

JCTVC-N0007 JCT-VC AHG report: Range extensions software (AHG 7) [K. Sharman, D. Flynn]

(Reviewed Sat. 27 July a.m. Track A)

The HM10.0_RExt2.0 software was upgraded to HM10.1_RExt3.0_rc1, and released on May 3rd. This release included all of the adopted tool changes except for the two new SEI messages. During this process, the reported gains of one of the proposed changes were seen to be benefiting from a correction to an existing bug in the SAO module, relating to bit depths beyond 10. After discussions on the reflector, the tool was disabled (but not removed) and the bug fix was corrected. It was indicated that further discussions regarding the tool would be continued at the next committee meeting.

The software was formally released (and tagged in the SVN repository) as HM10.1_RExt3.0 on May 20th, with the gains relative to HM10.0_RExt2.0 shown in a table in the report.

The changes in the results are:

- For 4:2:2 material, due to the change to use an mapping table for 4:2:2 for intra prediction, which also affects MDCS.

- For RGB material, due to the bug fix in SAO (this only affects the 12-bit sequence)

- For random access and low delay test conditions, due to changes between HM10.0 and HM10.1 causing a slight degradation.

In addition, a lossless coding tool has been added, which does not affect the standard lossy test conditions. This tool is only used when the development profile ‘rextdev’ is specified. It has been indicated to the community that this development profile should be used for all Range Extension related activities.

- JCTVC-N0008 JCT-VC AHG report: Screen content coding (AHG8) [H. Yu, R. Cohen, A. Duenas, T. Lin, J. Xu]

(Reviewed Sat. 27 July a.m. Track A (GJS).)

There was a number of emails sent in the interim period relating to AHG8. Most of them were posted on the reflector before May 17, i.e. before the submission deadline for RCE2 and RCE3 documents. These emails made comments and suggestions to the test materials and test conditions for RCE2 and RCE3.

A couple of emails were posted recently and made general suggestions and comments about performance evaluation and coding tool testing.

Per the last mandate of this ad hoc group, the test conditions for RCE2 and RCE3 were discussed and finalized jointly by the participants of RCE2 and RCE3 and AHG8. A few changes were made to the test sequences this time. In particular, the group decided to add the RGB and YUV 4:4:4 versions of the SlideShow sequences in the tests. As a result, for the first time, some results are available for reporting the HEVC performance on coding the same content but in different video formats, i.e. RGB, YUV 4:4:4, and YUV 4:2:0.

A summary of the resulting test conditions was tabulated in the report.

There were some comments and questions posted on the reflector on the general aspects of testing process. Below are some of these comments and questions and suggestions

- Question 1: Is subjective viewing sufficient for measuring visually lossless quality?

[Comment]: For professional applications (e.g., engineering work done in a cloud computing or remote-desktop platform), I believe that subjective viewing only is not sufficient for measuring visually lossless quality. We really need to establish a strict objective metric to measure visually perfect losslessness, which means that in the coding process, not only no noise should be added, but also no original “noise” or pixels even a single pixel in the picture should be visually altered. Subjective viewing can be used to evaluate the effectiveness of the objective metric.

- Question 2: Can we just use the same subjective test method that was used in the development of the HEVC Main profile? Do we need to revise it for the case of screen content coding?

- Question 3: Should we consider some additional objective quality metrics? For example, some of the proposed new tools do not use transform and may introduce distortion directly to pixels. For these kind of tools, PSNR may not be a good quality metric any more. Perhaps, a constraint to the percentage of the maximum difference over the original pixel value, say less than 2%, can also be used as additional objective metric.

[Comment]: Yes. The difference percentage (equivalent to SNR, instead of PSNR) should be a useful metrics, especially considering "Weber-Fechner law" about human perception, which states "the just-noticeable difference" between two stimuli is proportional to the magnitude of the stimuli and subjective sensation is proportional to the logarithm of the stimulus intensity.

- Question 4: On the test material, should we add new 4:4:4 sequences for the mixed content where camera-captured natural video, such as sequences in ClassD, is imbedded inside the screen content after first being up-sampled to 4:4:4 color sampling format? Furthermore, should we even use some pre-encoded ClassD sequences to make the new mixed content?

- Question 5: On the test conditions, since some of the proposed new coding tools introduce distortion without using the existing HEVC quantization process, should we add more test points, in addition to those QP points, that can reflect different levels of distortion resulted from these new tools?

[Comment]: Since visually perfect losslessness is strongly desired for some applications while mathematically lossless coding may not achieve desired bitrate, I think qp = 1, 4, 7, 10 may be a good set of values to test those tools targeting professional or prosumer (professional-consumer) quality visually lossless applications.

Relevant non-CE contributions were listed, including JCTVC-N0116 (m29739), JCTVC-N0148 (m29774), JCTVC-N0294 (m29966), JCTVC-N0115 (m29738), JCTVC-N0183 (m29814), JCTVC-N0231 (m29862).

However, the inputs listed are generally about coding technology, not methodology.

Further requirements establishment remains under consideration at the parent body level.

Forced-choice for perceptual loss was suggested as one test methodology for perceptual losslessness.

- JCTVC-N0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9)[M. M. Hannuksela, J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang]

(Reviewed Thu. 25th plenary)

This report summarized the activities of the ad-hoc group of high-level syntax for HEVC extensions (AHG9) between the 13th JCT-VC meeting and the 14th JCT-VC meeting and proposed a categorization of the related input contributions.

There had not been many discussions related to the mandates of this ad-hoc group over the reflector between the 13th and 14th JCT-VC meetings.

A change-marked version of the circulated JCTVC-N_Notes_d0 document suggested a refined categorization of AHG9-related contributions to facilitate their review at the meeting.

JCTVC-N0010 JCT-VC AHG report: SHVC core experiments (AHG10) [X. Li, J. Boyce, P. Onno, Y. Ye]

(Reviewed Fri. 26th a.m. plenary)

The configuration files of SHVC core experiments, the anchor data and the reporting sheets were released on May 18, 2013 as attachments to JCTVC-M1009.

The following contribution was noted to be related: JCTVC-N0163 (AHG14: Wide Color Gamut Test Material Creation – Test sequences for color gamut scalability).

JCTVC-N0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela]

(Reviewed Fri. 26th a.m. plenary)

The SHVC working draft 2 and SHVC Test Model 2 text were developed from the SHVC Working Draft 1 and SHVC Test Model 1 text according to the decisions made at the 13th JCT-VC meeting in Incheon, KR (18-26 Apr. 2013). The main activities performed by AHG11 were as follows:

- JCTVC-M1007 (SHM 2) Test Model 2 document was published following the 13th JCV-VC meeting. The document retains the specification text of textureRL (IntraBL) framework and the specification text of reference index framework was moved to Working Draft 2.

- Three versions of JCTVC-M1008 (SHVC WD 2) were published following the 13th JCV-VC meeting. The major changes compared to working draft JCTVC-L1008 were:

  - Adding the decoding process of reference index framework

  - Incorporation of all adopted proposals in the 13th JCTVC meeting

  - Editorial improvement

- An editorial improvement on SHVC Draft Text 2 has been submitted as the input of the 14th JCTVC meeting as JCTVC-N0242, and was aligned with the MV-HEVC editor’s input document JCT3V-E0100. The primary improvement was restructuring of the Annexes related MV-HEVC and SHVC text as follows:

  - Annex F includes the syntax, semantics and decoding processes common to the HEVC layered extensions

  - Annex G includes syntax, semantics and decoding processes for the multi-view extension

  - Annex H includes syntax, semantics and decoding processes for the scalable extension

The AHG recommended to:

- Adopt JCTVC-N0242, the editorial improvements on SHVC Draft Text 2, as the starting point for SHVC Working Draft 3 editing.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG.

- Continue to improve the overall editorial quality of the SHVC Working Draft and Test Model documents.

It was planned to issue PDAM from ISO/IEC (internally to be referred to as Draft 3) – do not use “Working Draft” for the JCT-VC drafts henceforth to avoid confusion with the stage of ISO/IEC work referred to by that name.

JCTVC-N0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He, T.-D. Chuang, D.-K. Kwon]

(Reviewed Fri. 26th a.m. plenary)

The current latest software version SHM2.1 contains all the items adopted last meeting by exception of the following:

- Picture marking part related to the F.8.1.2.1 section from JCTVC-L0188

- Marking of sub-layer non-reference pictures as "unused for reference" from proposal 1 of JCTVC-M0209

- When discardable_flag is equal to 1, a picture is immediately marked as “unused for reference” after its decoding from JCTVC-M0162

The last two items are dependent on the first one, and the proponent of the first item was contacted regarding software integration help.

It was remarked that thes topics may not be especially important to the behaviour. However, it should be ready for integration soon.

Software version SHM2.0, based on HM10.1, was released according to the schedule for CE experimentation.

The following items have been integrated in this version:

- Software is aligned with HM10.1

- IntraBL:

  - M0124 simplified pruning

  - M0075 IntraBL context

- M0133 MV scaling and resampling position calculation

- M0274 use decoded picture for inter-layer prediction

- M0259 lambda refinement (encoder only optimization)

- M0268 Signalling of profile and output layers in VPS

- M0309 signal offsets to specify the relative spatial alignment of the base and enhancement layers

- M0458 inter-layer RPS

SHM2.0 performance relative to the SHM1.0 based on CTC with the “reference index” framework was summarized and more details can be found in an accompanying Excel table in the report.

Overall, there was a small (beneficial) difference in compression relative to SHM 1.0.

SHM2.0avc is another version based on SHM2.0 where the assert lines were updated in the upsampling function. This version was released along with AVC base configuration files.

Another software version SHM2.1 based on HM11 was released on July 22, 2013 before the JCT-VC meeting start.

In this software version, almost all remained adopted items were integrated and a multilayer coding with multiple dependent reference layers was implemented to address the decision about MV-HEVC alignment made for JCTVC-M0343.

In this version, configuration files were also updated and the parameters associated with the layers were moved to a separate configuration file named layers.cfg.

SHM2.1 performance relative to the SHM2.0 based on CTC with the “reference index” framework is summarized in the report (with very little difference shown between their performance) and more details can be found in an accompanying excel table in the report.

The AHG recommended to develop reference software based on SHM-2.1 and improve its quality.

An informative sequence downsampling tool, used to generate SHVC downsampled test sequences, was suggested to be useful to generate a reference layer for non-CTC test sequences, and the suggested to plan to include it into the SHM software package as a separate project with the next software release. In the discussion, it was suggested to potentially also integrate this into the encoder capability. However, some participants suggested that keeping it as a stand-alone tool may be preferable.

It was reported that it would be helpful if a bug tracker could be used for software development. K. Suehring indicated the ability to quickly set up a bug tracker.

JCTVC-N0013 JCT-VC AHG report: SHVC Inter-Layer Filtering (AHG13) [A. Segall, E. Alshina, J. Chen, J. Dong, P. Topiwala, M. Zhou]

(Reviewed Fri. 26th a.m. plenary)

There had not been discussions related to the mandates of this ad-hoc group over the reflector between the 13th JCT-VC meeting and the 14th JCT-VC meeting. However, there was significant activity in the area within the SCE1 and SCE3 core experiments. There were approximately 45 contributions identified as being related to the mandates of this AhG.

JCTVC-N0014 JCT-VC AHG report: Colour gamut scalability (AHG14) [A. Segall, P. Bordes, J. Dong, A. Duenas, L. Guo, D. K. Kwon]

(Reviewed Fri. 26th a.m. plenary)

The AhG used the JCT-VC reflector for all discussions. A kick-off messages was sent on July 11, 2013, and there were approximately 8 email messages related to the mandates of this ad-hoc group over the reflector between the 13th and 14th JCT-VC meetings.

Potential test sequences were provided by Technicolor and announced on the reflector on July 11, 2013. The sequences were made available on the following FTP site, though multiple experts identified problems in downloading the data.

FTPserver: ftp-renn.thmulti.com

login:     imx-wp3

password:  HEVC2011wp

Examples from the sequences were shown in the report.

There were five contributions related to the topic of wide color gamut scalability that were identified in the AHG report: N0146, N0163, N0168, N0218, and N0271 – with cross-checks in N0274, N0278, and N0339. N0168 reported some results with the new test sequences.

It was remarked that software availability for colour gamut scalability will be needed to enable coordinated work. A couple of participants indicated that this would not be difficult to provide.

It was agreed to establish a BoG (A. Segall), to investigate sequences and test conditions, and review relevant contributions.

JCTVC-N0015 JCT-VC AHG report: SHVC hybrid codec scalability (AHG15) [J. Boyce, K. Kawamura]

(Reviewed Thu. 25th p.m. Track A (GS).)

Here, "hybrid scalability" refers to the use of non-HEVC base layers.

The SHM 2.0 software release of May 15 supports an AVC base layer.

Reporting template and anchors for experiments using an AVC base layer were distributed on May 23 as attachments to JCTVC-M1009.

Two relevant contributions were noted: JCTVC-N0050 and JCTVC-N0211.

The VPS extension design in the SHVC WD includes an avc_base_layer_flag syntax element to indicate an AVC base layer.  An editorial comment in the document reads:

[Ed. (YK): For possible support of base layer of other codecs, e.g. MPEG-2, a flag is not sufficient.]

Ajay Luthra raised the question of whether some reserved fields should be added now to the VPS to support an MPEG-2 base layer in the future. It was noted that a vps_extension2_flag syntax element is already present in the design.

It was agreed that there does not seem to be a clear need to identify what non-HEVC encoding was used to generate a base layer unless the base layer is encapsulated within the same video elementary stream.

It was noted that access unit specification along with definition of base layer decoding order and output order (as related to enhancement layer decoding order and output order) is needed. And the HRD flow relationship between the base and enhancement layers. Our plan has been that the access unit structure, decoding order and output order are locked together for the base and enhancement layers.

If we do decide to specify encapsulation of the base layer within the video elementary stream, we should have a type code syntax element capable of indicating multiple base layer coding types (e.g. AVC, MPEG-2, and other values reserved for future use).

It was noted that an encapsulation method is not currently in our draft text.

This was further discussed Thu p.m. to determine whether to change the current HEVC-vs-AVC base layer indication or add the type indication.

Encapsulation is for further study in AHG; no type code was planned to be specified for now.

JCTVC-N0016 JCT-VC AHG report: Single-Loop Scalability (AHG16) [M. Wien, J. Boyce, M. Budagavi, K. Mishra, K. Ugur]

(Reviewed Fri. 26th a.m. plenary)

There was no activity for the AhG on the reflector. Five contributions were noted to be related: N0129, N0161, N0186, N0187, and N0202 – with cross-checks in N0203, N0297, N0352.

Document JCTVC-N0129 proposes modifications to the VPS. A new syntax element indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

Document JCTVC-N0161 proposes to apply the key-picture concept of SVC in the multi-loop approach of SHVC. Thereby, reconstructed enhancement layer (EL) pictures are used for inter prediction in base layer (BL) pictures. The scheme improves the compression efficiency in the EL. The induced drift if reconstructing only the BL has to be further studied. A similar approach is included in contribution JCTVC-N0202.

Document JCTVC-N0186 proposes a single-loop decoding approach for SHVC. The BL motion information is applied to the EL reference pictures for inter prediction and the BL residual is added in addition to the EL residual. Constrained intra prediction is applied in the base layer. The scheme enables single-loop decoding and does not induce drift into the reconstructed BL pictures. The reported simulation results are verified in contribution JCTVC-N0203.

Document JCTVC-N0187 investigates the performance of multi-loop decoding using the hybrid inter-layer prediction which is proposed in JCTVC-N0186 for SNR scalability. The multi-loop decoding results are obtained by using both the inter-layer prediction (ILP) picture and the hybrid inter-layer prediction (H-ILP) picture for EL prediction. The experimental results show that H-ILP picture can further improve the EL coding efficiency for multi-loop decoding based scheme.

Document JCTVC-N0202 proposes a single-loop decoding approach for SHVC using the key-picture concept as applied in SVC. The reconstructed EL pictures are used for inter prediction in BL non-key-pictures. BL motion vector coding is applied without reference to EL syntax elements. The proposed scheme induces drift to the reconstructed BL pictures if only the BL is reconstructed. The drift is controlled by periodic insertion of key-pictures (at tid=0). The contribution presents results with multi-loop decoding as well as with single-loop decoding. In the latter case, constrained intra prediction is applied in the base layer. The induced drift if reconstructing only the BL has to be further studied. A similar multi-loop approach is proposed in contribution JCTVC-N0161. The reported simulation results are verified in contribution JCTVC-N0297.

JCTVC-N0017 JCT-VC AHG report: SHVC complexity assessment (AHG17) [M. Budagavi, E. Alshina, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai]

(Reviewed Fri. 26th a.m. plenary)

A complexity assessment module and spreadsheet was developed for use in SCE1 and SCE3 activity. JCTVC-N0150 provides the relevant patch and spreadsheet and also a brief description (which had been circulated to CE participants). A prior version had been made available in the prior JCTVC-M0455 BoG report.

It was noted that there are several SCE and non-SCE contributions that address complexity reduction and coding efficiency impact of the simplifications.

Contribution JCTVC-N0150 presents a performance and complexity analysis of SHM2.0 (RefIdx framework) compared to HEVC single-layer coding.

The following open issues were identified:

- For the SNR scalability case, because of the scaled reference layer offsets (M0309) adoption at the last meeting, the latest version of the complexity assessment module for average memory access estimation (July 15 version) assumes that the inter-layer reference frame is created by a copy of the reconstructed BL picture (1 tap filtering). For the worst case complexity assessment, both implementations are considered (pointer to BL and copy of reconstructed BL picture).

- Complexity is measured using two models: PU based inter-layer processing ("up-sample on the fly") and picture-based inter-layer processing. Keeping both complexity measure doubles amount of work for testers and cross-checkers. It is preferable to have only one complexity measure.

There are several SCE and non-SCE related documents that are related to complexity reduction.

- JCTVC-N0018 JCT-VC AHG report: high bit-rate and bit-depth operating points (AHG18) [K. Sharman, E. Francois, H.-Y. Kim]

(Reviewed Sat. 27 July p.m. Track A (GJS).)

There was no email reflector activity. Relevant contributions were identified, including JCTVC-N0142, JCTVC-N0179, JCTVC-N0188, JCTVC-N0189, JCTVC-N0190, JCTVC-N0191, JCTVC-N0275.

Source material was identified as a key need.

JCTVC-N0019 JCT-VC AHG report: Verification test preparation (AHG19) [T. K. Tan (NTT Docomo), V. Baroncini (FUB), W. Wan (Broadcom), M. Karczewicz (Qualcomm), J. Wen (Tsinghua Uni)]

(Reviewed Sat. 27 July p.m. Track A (GJS).)

The following test conditions were proposed in the kick off message for the activities and discussed on the reflector.

- Four sequences in each of the following resolutions (480p, 720p 1080p and 4K), perhaps 5 for 720p and 480p.

  - Comment: At least one sequence with film grain and other types of noise (e.g. compression noise after a sequences goes through MPEG-2 and/or AVC compression) that is present in some real world sequences.

- Bit depths of 8 bits for 480p, 720p and 1080p, 10 bits for 4K. (so we test both Main and Main 10)

  - Comment: It will be worthwhile to test 4k at 8 bits as well

- To decide on the following parameters once we select the sequences

  - Random access and/or low delay coding options be used depending on the nature of the content that we select.

  - This would cover the streaming/storage and communications use cases, respectively.

  - The bitrate selected should roughly correspond to the bitrate that is used in the industry but adjusted to cover the whole MOS range as much as possible.

    - Four bit rate points per sequences. Bit rates selection is the most important point for the good outcome of the test.

- To use the HM (latest) for the HEVC and JM18.5 for the AVC encoding.

- To use encoding parameters close to the CC conditions, including fixed QP.

Regarding identification of Test Sequences: Some sequences were available on the JCTVC FTP site, however many were found to be unsuitable. There are no 720p or 480p sequences available.

Most of these sequences were found to be unsuitable for performing subjective testing because of one or more of the following reasons:

- Too easy to code.

- Static background with very little moving objects.

- Blur and do not have any high-resolution spatial details.

- Contain too much noise.

Some test sequences were collected and encoding with the HM and JM software were done. This will be available for screening during the JCTVC meeting.

“Aligning” lambda parameters that are used by HM and JM: Comment: One comment on test conditions, which might not be at this point so important, but should also one try to "align" the lambda parameters that are used between the two software?  The HM for example uses a different lambda weight factor (QPfactor) for each picture thus providing additional control and flexibility on RD versus what is done in the JM with fixed lambda parameters. In fact for both low delay B and random access it seems that the "default" implicit lambda computation for B slices is used in the JM, which was "designed" (if it can be called that) mainly for use with non-reference B slices. This can have a severe impact on performance (both subjectively and objectively). Has this been looked at at all?

Verification of multiple tiles: One member commented that as the parallel processing using tiles is one of the features of the Main and Main10 profiles of HEVC, should we also test 4k with, e.g. four independent tiles with no cross tile processing to test the performance when four encoders are used in parallel.

Another member replied that when you test four independent tiles with no cross-tile processing, this needs to be evaluated subjectively too because without cross boundary loop filtering it is very likely that you'll see the tile boundaries. However, as a general comment I think we should not inflate this first verification test by adding the parallel tools because I would then also be interested in testing the 4K performance with wavefronts and so on.

The first member argued that the verification testing will involve subjective viewing. Unlike tiles, as I understand, wavefront does not impact in a significant way the video quality. So, wavefront specific verification testing may not be high on the list and may not be needed. However, as you also mention, that may not be the case with tiles. The visual quality may depend significantly on some other constraints (on design / architecture) in the presence of tiles. So, I think, it will be important to verify the performance with tiles (as it is with the presence or absence of B pictures) under different conditions (e.g., as you mention, with or without cross tile deblocking and/or others) so that if their presence makes a significant impact on architecture and/or quality, it will be good to verify and understand that. And, that may also provide some helpful guidance to the applications standards bodies and others.

The AHG recommended:

- JCT-VC participants to contribute test sequences suitable as subjective test material covering all resolutions of interest.

- To adopt and refine the proposed test conditions.

- To draft a video verification test plan.

Some problems with test sequences were discussed, in regard to identifying sequences that provide discrimination between codecs, e.g., not being too "easy", and not being dominated by noise. Test sequences are a critical need.

(Further discussion Mon. 29th plenary)

There was a suggestion to solicit non-reference encodings.

If submitted, it was suggested that we should include both reference and non-reference encodings in the test.

JCTVC-N0020 JCT-VC AHG report: Chroma format scalability (AHG20) [A. Segall, A. Duenas, K. Ugur]

(Reviewed Sat. 27 July p.m. Track A (GJS).)

There was an email message on the reflector on July 12, 2013 summarizing the further developments regarding the earlier proposal JCTVC-M0229. There was no further activity for the AhG on the reflector.

One contribution was noted to be relevant: JCTVC-N0145. See notes on that contribution.

- JCTVC-N0021 JCT-VC AHG report: Best-effort decoding with reduced decoding complexity (AHG 21) [D. Flynn, J. Sole]

(Reviewed Sat. 27 July p.m. Track A (GJS).)

Motivations for such techniques were discussed on the JCT-VC mailing list, with comments made about minimum quality levels, controlling the use of such features, the relationship to scalability and a use case for still picture decoding.

A contribution on the topic, providing use cases, a summary of an implementation of such a technique and providing additional options as to how to craft the specification is provided in JCTVC-N0291. Similar input has also been submitted at the parent body level on possible use cases.

# Project development, status, and guidance

## Conformance test set development

JCTVC-N0284 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan] [late]

Further improvement was encouraged.

## Version 1 bug reports and cleanup

JCTVC-N0041 Editors' proposed corrections to HEVC version 1 [B. Bross, G. J. Sullivan, Y.-K. Wang]

(Discussed in Track A Tue. 30th (GJS).)

These corrections seemed uncontroversial.

JCTVC-N0094 HEVC version 1 (corrigendum): Derivation of CPB removal time [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

Discussed Thu 1st (GJS):

This document reports that the derivation of nominal CPB removal times of access units in the specification of HEVC version 1 has a bug, more specifically related to the derivation of the variable AuCpbRemovalDelayVal, and proposes a method to fix the issue.

The problem is about how we specified the intended modulo operation of the CPB removal delay.

Decision (BF): Add this to our defect report output document.

JCTVC-N0346 Proposed editorial changes for maximum number of slices and tiles per picture limit [M. Zhou, B. Heng, W. Wan (Broadcom)] [late]

Discussed Thu 1st (GJS):

It is asserted that, based on the calculations specified in the current HEVC version 1 text, the maximum number of slices per picture could be 0 for high levels with small enough pictures. A similar issue is reported in the maximum number of tiles per picture limit.

The issue seems to be an obvious oversight.

Discussed Thu 1st (GJS): Decision (BF): Ensure that at least one slice (and tile) is allowed.

JCTVC-N0378 Clarifications on HEVC NALU order [Jean Le Feuvre (Telecom ParisTech), Cyril Concolato (Telecom ParisTech), Mickael Raulet (IETR/INSA Rennes)] [late]

(Reviewed in Track A Tue. 30th (GJS).)

Previously registered as N0324 (also late).

Contains two elements:

- Regarding first_slice_segment_in_pic_flag, the contribution reported that there is an issue with picture boundary detection – esp. for consecutive IDR pictures. The flag can be lost. Also, the end of one picture may not be detected until receiving the first slice of the next picture. Historically, idr_pic_id was in drafts of HEVC but was removed prior to finalization. It was noted that we don't really expect consecutive IDR pictures to be used commonly and don't encourage them to be used – since CRA pictures are expected to be more appropriate for all-intra use. Also we have a temporal sub-layer zero SEI message that can be helpful if used. Many systems have provisions to assist with this topic (e.g., RTP timestamps, PTSs, AU framing in ISOBMFF, RTP marker bits, MPEG-2 TS PES). It was suggested to add some informative note(s) to discuss the issue and discourage consecutive IDR usage. Decision: Agreed.

- Regarding parallel processing, the contribution noted that it is possible to decode tiles and slices in parallel, but we require slices (and tiles) to be in a specific order. It was remarked that this is an old and well-known issue, and the current spec content was carefully negotiated intentionally. Supporting arbitrary order in a decoder is non-trivial. Removing this constraint would require defining a new profile, which is unlikely to be justified. Unless it is known that a decoder can handle out-of-order NALUs, it may be necessary for encoders or receivers to buffer and order NALUs in the specified conforming order.

## Implementation demonstrations

JCTVC-N0313 4EVER HEVC demonstrations during Roland Garros tournament [S. Kervadec (Orange Labs), M. Raulet (INSA), J. Le Feuvre (Telecom Paris Tech), J. Vieron (Ateme), M. Clare (Orange Labs)] [late]

This contribution described an end-to-end live HD HEVC-based delivery chain that was demonstrated by the 4EVER project during the French Open Tennis Tournament Roland-Garros.

Several MPEG/ITU technologies were reportedly showcased during these demonstrations. Demonstrations were shown during the second week of the tournament, from June 1st, to June 9th.

Three HEVC-based HD End-to-End Live delivery chains were demonstrated: IPTV chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered using UDP multicast transport over an Orange IPTV managed network; an Internet chain, based on HEVC/MPEG-4 AAC/ISOBMFF/MPEG DASH, delivered using HTTP over best-effort Internet network, through two different (nominal/backup) HTTP servers; a DVB-T2 chain, based on HEVC/MPEG-4 AAC/MPEG-2 TS, delivered over DVB-T2 modulation.

These demonstrations were reportedly operated in the same conditions as any commercial audiovisual service, the IPTV and Internet Live feeds being broadcasted at the same time they were produced on the courts, and being accessible through any Orange access (although requiring a dedicated HEVC decoder).

No action was requested; the presenter not present Thu pm.

JCTVC-N0276 A hardware oriented implementation of HEVC encoding [Ryoji Hashimoto, Seiji Mochizuki, Kenichi Iwata (Renesas)]

Reviewed Thu 1 a.m. (GJS).

This contribution presents a hardware oriented implementation of HEVC encoding, which makes control logic in encoder easier. A hardware encoder can extend input images to multiple of CTB size by utilizing the conformance window syntax to simplify control logic. However, extension of images requests more bits to encode, especially with large CTB size. To minimize overhead of bits in extended area, truncation of coefficients in quantization and optimization of TU partitioning and process are adopted. As a result, adopted methods in developed hardware can reportedly save 6.3% the BD rate in maximum.

JCTVC-N0043 On software complexity: real time and parallel SHVC video decoder [W. Hamidouche, M. Raulet, O. Déforges (IETR/INSA)]

(Contributor indicated that detailed presentation was not necessary – please see contribution submission.)

This contribution describes an open source SHVC software decoder implementing the reference index based SHVC design. The wavefront parallel processing approach is used to perform the video decoding of both the base layer and the enhancement one in parallel.

Experimental results carried out on a laptop fitted with a core Intel i7 processor reportedly show that the demonstrated software decoder achieves the decoding of 1280×720 base layer and 1920×1080 enhancement layer video sequences at 25 fps when using four concurrent threads.

## Profile and level definitions and other requirements related topics

Joint discussions

Contributions in this area were initially discussed in a joint meeting with MPEG Requirements and VCEG at 4 p.m. Tue. 30th (GJS & J. Ostermann). At that time it was suggested for JCT-VC to identify a somewhat-reduced set of profiles relative to those proposed in JCTVC-N0191 for specification action at the current meeting, and for the remainder to be considered in further study.

The subject was then further discussed in JCT-VC on Wed. 31st (GJS).

Tentative conclusions for the output draft of this meeting were identified as follows:

- Separate colour plane coding should be prohibited.

- Aux pics (if specified) should not be profiled (at the moment).

- Define the following new profiles:

  - 4:4:4/10, 12

  - 4:2:2/10, 12

  - 4:2:0/12 (note that 8 and 10 bit profiles have already been specified)

- Use one profile_idc for the syntax family, and use constraint flags for subset profiles within that.

It was remarked that intra-only is also very desirable to consider.

The subject was further discussed again in a joint discussion Thu. 1st at noon (GJS & J. Ostermann).

Additional aspects were agreed as follows:

- The 4:2:2 and 4:4:4 cases should have 1.25 and 1.5 times the bit rate capability and half the minimum compression ratio MinCr requirement

- The maximum CPB size should not be increased for the 4:2:2 and 4:4:4 cases – so, in these cases, the maximum CPB size will be less than a 1 second buffer at the max bit rate.

- Separate colour plane mode should be prohibited.

The values will be studied and may be refined in the final version.

The naming of the profiles was left to the discretion of the editors.

Decision: Agreed as described above.

Other topics of joint discussion included the following:

- JCTVC-N0050 (see section 6.5.1) also has aspects related to profile/level (for use of an AVC base layer in an SHVC video context). This was also the topic of a contribution to VCEG as VCEG-AV13. There remains an interest in enabling hybrid codec scalability, although some specifics need to be sorted out.

- There is keen interest in further work on screen content coding (SCC) as a focus of development (versus designing a "generic" video coding standard intended to handle a broad range of applications). The WG 11 parent body developing a plan to issue a Call for Proposals for new work on the subject. Members of both parent bodies expressed an interest in this work, and potentially in conducting it jointly.

- Work has been progressing in the WG 11 parent body regarding the potential benefit for modification of HEVC for improving the coding efficiency of coding interlaced video, and the study will be continued.

- The potential use of the "tiled" frame packing arrangement with HEVC for stereoscopic video coding has been under discussion at the parent body level, and a suggestion under consideration in WG 11 was to specify such an indication in a separate SEI message rather than adding it to the existing frame packing arrangement SEI message.

- A contribution VCEG-AV14 had been submitted to VCEG and been discussed, regarding still picture coding with HEVC. The response was as follows:

  - Decision: It was agreed to move forward toward specification of a "level 8.5" with unlimited picture size, for which decoding is optional but normatively specified if decoded.

  - Other aspects, including tiling constraints, are for further study.

- "Best effort" decoding with degraded quality remains under investigation for potential action. See contribution N0291 and related notes.

- 3D and ultra-multiview video coding has been investigated in some recent work.

- Input was received by the parent bodies regarding using a different sort of color representation for coded video content than has typically been used in past practice. This has sometimes been referred to as the coding of XYZ video content. A relevant contribution to VCEG was VCEG-AV11.

- 3GPP is evaluating the possibility of using HEVC and may request the specification of another tier/level of HEVC. Contribution N0237 was related to this and VCEG document VCEG-AV12.

JCTVC-N0178 HEVC profiles for medical imaging applications [P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

This document proposes HEVC profiles for medical imaging applications in the context of the Range Extensions currently standardized in the JCT-VC. The main requirements addressed by these proposed profiles are high bit-depth compression, lossless compression and signed coding. This results in the four proposed profiles High 12 Intra 4:0:0, High 12 4:0:0, High 16 Intra 4:0:0, and High 16 4:0:0.

JCTVC-N0191 AHG 5 and 18: Profiles for Range Extensions [K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony)]

A set of profiles are presented that propose to address the uses envisioned for HEVC Range Extensions. These proposed profiles are further refined, with the view to simplify encoder implementation and increase interoperability. This contribution proposes the following.

- For JCT-VC (with its parent bodies) to specify all conformance points required by industry to avoid de-facto profiles defined outside this standardization body.

- Two categories of profiles proposed to be defined:

  - Main xxx profiles: No new RExt tools are included (intended for consumer applications)

  - High xxx profiles: Include new RExt tools and higher input bit-depths

- Consideration of tools for non-camera captured content

- Specification of a 4:2:0 12 bit profile

JCTVC-N0312 A proposal on HEVC 4:2:2 profile [S. Sekiguchi, A. Minezawa, H. Sakate (Mitsubishi)] [late]

This contribution proposes an HEVC profile supporting 4:2:2 chroma format. The proposed profile is an extension to the Main 10 profile.

JCTVC-N0237 HEVC version 1: Level 2.2 for support of WVGA@30fps [Y.-K. Wang (Qualcomm)]

(Reviewed at parent level Tue. 30th)

No immediate action was taken on this – parent bodies will liaise with 3GPP and determine appropriate action in future study.

## HEVC and RExt use cases (requirements related)

JCTVC-N0291 [AHG21] Best-effort decoding of 10-bit sequences: use cases, requirements and specification methods [D. Flynn, G. Martin-Cocher, D. He (RIM)]

(Further discussed with presentation Fri. 2nd.)

HEVC, like some other well-known codecs, contains profiles that accommodate the decoding of non-8-bit video sequences. In particular, HEVC contains a 10-bit profile that specifies 10-bit coded video sequences. In JCTVC-M0255, it was demonstrated that it is possible to re-purpose an existing 8-bit decoder design to decode 10-bit bitstreams. It was also reported that management of rounding in the decoder (e.g. round to even) can reduce the accumulation of decoder error to a lesser amount than might otherwise be expected.

This contribution proposes a candidate requirements basis and use case information, summarizes the previous work, and suggests methods of text specification.

The method was suggested to work better at low fidelity than at high fidelity.

The proposal suggested also considering adding data within the bitstream to indicate some intention to enable this usage.

Prior discussion

- The contributor suggested to define a "degraded bit depth decoding profile" with a normatively specified decoding process.

- A participant commented that there are various ways to perform this – and opined we shouldn't try to specify it since we can't specify them all.

- It was agreed to study to determine whether there is one method or a small number that is adequate/best (AHG activity); then decide later what to do with that information.

Further suggestions were:

- Consider informative description as a "suggested" functionality (non-normative).

- Provide encoder advice (don't use long inter-prediction chains, use slices to reset intra prediction error).

In the described method, the processing is generally performed as if the decoding process used 10 b, but the reference picture storage and reference samples for intra prediction are 8 b, and then the result is rounded after adding prediction and residual, before applying deblocking and SAO, and parts of deblocking and SAO are adjusted to compensate for the differing granularity.

## Source video test material

See also JCTVC-N0163.

JCTVC-N0179 Selected medical imaging sequences for HEVC development [P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens)] [late]

Discussed Thu 1st (GJS):

This contribution proposes new test sequences from the medical domain for the development of HEVC, especially for the specification of range extensions. The proposed set contains 8-bit, 12-bit, and 16-bit monochrome image data as well as 8-bit RGB content.

Sequences were not available yet. Anticipated copyright terms would allow only for use for the committee's development of standards. It was suggested for the contributor to allow use for promotion of the standards and research and publication of snapshots in academic publications as well.

JCTVC-N0294 AHG8: 4:4:4 game content sequences for HEVC Range Extensions development [R. Cohen (MERL), F. Liu, C. S. Ping, N.-M. Cheung, Y. Chau, S.-K. Yeung (Singapore Univ. of Technology and Design)]

Discussed Thu 1st (GJS):

This document proposes two 4:4:4 game content sequences for use in HEVC Range Extensions development. The primary application represented by these sequences is multiplayer mobile cloud gaming. The format of these 10-second RGB sequences is 1280x720, 30fps, 8 bits per component. The sequences were written directly to files by the content-generation software.

These test sequences were proposed for addition to SCC test set (which otherwise has 10 sequences).

Suggested as being too easy: CAD waveform, PCB layout, CG Twist tunnel. It was agreed to drop all three of these from the usual tests – keep them available but drop them from usual testing.

The contributor said that non-anti-aliased versions would also be provided (but we will focus on the anti-aliased ones for the test set).

Other frame rates and resolutions may be possible to generate.

These test sequences were available. Members were encouraged to experiment. We are likely to add to our SCC test set at next meeting.

# Core experiments in SHVC

## SCE1: Support for additional resampling phase shifts

### SCE1 summary and general discussion

(Reviewed in Track B Fri 26th (JO).)

JCTVC-N0031 SCE1: Summary Report of SHVC Core Experiment on support for additional re-sampling phase shifts [E. Alshina, X. Li, J. Dong]

This document summarizes the activities and test results performed in SCE3 on support for additional re-sampling filters. Two problems were investigated: enhancement and base layer picture misalignment and relative Luma and Chroma samples misalignment. Solving of both problems requires modification of re-sampling process beyond SHM2.0: additional re-sampling filters are needed. Two types of solutions were studied: fixed predetermined re-sampling filters with additional information about filters choice signalling (so-called category 1 tests) and variable coefficients (so-called category 1 tests).  Tools have been evaluated on the test conditions defined in document JCTVC-M1101.

Tools listed in the table below were evaluated in this CE. 4 related non-SCE contributions were submitted for this meeting. Non-SCE1 contributions are marked with * in the table below.

Table: List of tested tools and cross-checks availability.

| Tests | Title | Tester | Cross-checkers |
| --- | --- | --- | --- |
| Category 1: fixed coefficients of re-sampling filters |  |  |  |
| About relative El and BL pictures displacement |  |  |  |
| 1.1 | Performance with default parameter values | Inter Digital JCTVC-N0182 | Qualcomm |
| 1.2 | The accuracy of down-sampler filter phase | Inter Digital, Qualcomm, Nokia JCTVC-N0182 | Arris JCTVC-N0225 |
| 1.2* | Non-SCE1: Results of test 1.2 on sampling offset signalling with accurate interpolation filter | Samsung JCTVC-N0149 | None |
| 1.2* | Non SCE1: On handling resampling phase offsets with fixed filters | Arris JCTVC-N0272 |  |
| About relative Luma and Chroma samples position |  |  |  |
| 1.3 | Accurate Chroma position alignment | Qualcomm JCTVC-N0045 | Arris JCTVC-N0226 |
| Category 2: variable coefficients of re-sampling filters |  |  |  |
| 2.1 | Phase Compensation by signalling filter coefficients on sequence level | Qualcomm JCTVC-N0046 | Samsung JCTVC-N0308 |
| 2.2 | Phase Compensation by signalling filter coefficients at PPS with sample shift | Arris JCTVC-N0078 | Inter Digital JCTVC-N0193 |
| General comments on re-sampling process |  |  |  |
| * | Non-SCE1: Dynamic range control of intermediate data in re-sampling process | Qualcomm JCTVC-N0214 | Samsung JCTVC-N0218 |
| * | Non-SCE1: On arbitrary spatial ratio scalability in SHVC | Qualcomm, Samsung, Nokia JCTVC-N0219 | MediaTek JCTVC-N0315 |

Following tables show the average data of AI, LDB, LDP and RA configurations.

For the following table, simulations were performed where a phase shift occurs between BL and EL in downsampling, where SHM can only support zero phase shift due to its filters.

Performance and complexity summary for tests on EL and BL pictures alignment.

| Test |  | x2 |  |  |  |  | x1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s/w |  | BD-rate |  | Memory |  |  | BD-rate |  | Memory |  |
|  | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| SHM2.0 | 0.00 | 0.00 | 0.00 | 100% | 100% | 0.00 | 0.00 | 0.00 | 100% | 100% |
| Category 1: fixed coefficients of re-sampling filters |  |  |  |  |  |  |  |  |  |  |
| N0182_16 | -6.65 | -5.21 | -5.11 | 97% | 97% | -6.29 | -5.86 | -5.51 | 97% | 98% |
| N0149_16 |  |  |  |  |  | -6.26 | -5.81 | -5.49 | 97% | 98% |
| N0182_8 |  |  |  |  |  | -5.71 | -5.29 | -4.94 | 98% | 98% |
| N0149_8 |  |  |  |  |  | -5.96 | -5.53 | -5.16 | 98% | 98% |
| N0182_4 |  |  |  |  |  | -3.85 | -3.70 | -3.43 | 99% | 99% |
| N0149_4 |  |  |  |  |  | -3.97 | -3.86 | -3.57 | 99% | 99% |
| Category 2: variable coefficients of re-sampling filters |  |  |  |  |  |  |  |  |  |  |
| N0046(Seq) | -6.62 | -5.18 | -5.08 | 97% | 97% | -6.26 | -5.82 | -5.49 | 97% | 98% |
| N0046(Pic) | -6.84 | -5.62 | -5.45 | 97% | 97% | -6.91 | -6.68 | -6.23 | 97% | 97% |
| N0078 | -6.65 | -5.21 | -5.11 | - | - | -6.88 | -6.41 | -5.99 | - | - |

From these results, it becomes evident that if a different phase would be used, the current SHM would suffer.

The phase offset in downsampling was taken similar as in SVC, approximately 1/6 for 1.5X. No other phase shifts were tested. Upsampling used rounding to 1/4, 1/8, 1/16, and arbitrary/adaptive

Series of tests with 1/4…1/16 accuracy for down-sampling filter phase shift signalling allows to identify relationship between performance and bits needed for down-sampling filter phase shift signalling. Graph on Fig. 2 shows dependency between size of side information signalling (horizontal axis) and gain achieved in non-ctc content (vertical axis). Blue dots show performance of tests with fixed coefficients; red dots – for adaptive filter solutions. No significant gain was observed beyond 1/8 sample precision.

In the last meeting, it was reported that SVC (which uses ½ pel phase shift, e.g. in 2x scalability) had a compression benefit from doing this. However, SVC does not allow signalling the downsampling phase.

However, the following table shows that a deviation from the zero position in downsampling (as currently in SHM) is still the best solution in terms of performance

Performance and complexity summary vs HM10.1 single layer.

| Test | Test |  | x2 |  |  |  |  | x1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| content | s/w | BD-rate |  |  | Memory |  | BD-rate |  |  | Memory |  |
|  |  | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| ctc | SHM2.0 | 21.8 | 31.3 | 31.3 | 105% | 155% | 18.6 | 26.1 | 27.5 | 110% | 160% |
| non-ctc | N0182_16 | 22.2 | 31.5 | 31.7 | 106% | 155% | 19.4 | 26.8 | 28.1 | 112% | 160% |
| non-ctc | N0149_16 |  |  |  |  |  | 19.5 | 26.9 | 28.1 | 112% | 160% |
| non-ctc | N0182_8 |  |  |  |  |  | 20.2 | 27.6 | 28.8 | 112% | 160% |
| non-ctc | N0149_8 |  |  |  |  |  | 19.9 | 27.2 | 28.5 | 112% | 160% |
| non-ctc | N0182_4 |  |  |  |  |  | 22.6 | 29.7 | 30.8 | 113% | 161% |
| non-ctc | N0149_4 |  |  |  |  |  | 22.4 | 29.5 | 30.6 | 113% | 161% |
| ctc | N0045 | 21.8 | 31.5 | 31.7 | 105% | 155% | 18.6 | 26.2 | 27.6 | 112% | 159% |
| non-ctc | N0046(Seq) | 22.3 | 31.6 | 31.8 | 106% | 155% | 19.5 | 26.8 | 28.1 | 112% | 160% |
| non-ctc | N0046(Pic) | 22.0 | 31.0 | 31.3 | 105% | 155% | 18.7 | 25.8 | 27.2 | 112% | 159% |
| non-ctc | N0078 | 22.2 | 31.5 | 31.7 | - | - | 18.7 | 26.1 | 27.4 | - | - |

From these results, it is reportedly evident that using another than zero phase position in downsampling would give benefit in terms of compression. In the discussion, the following applications are mentioned:

- upsampling from even/odd fields (i.e. vertical downsampled even/odd fields as base layer)

- using sequences that were differently downsampled beforehand (question: Is it a big effort to down-sample again with zero phase shift as SHM uses)

- transcoding of SVC streams

More evidence required included:

- which phase shifts would be required for such applications

- what actual downsamplers would be used

- what the benefit in terms of device (encoder/transcoder) complexity and performance would be.

Does the definition of arbitrary phase come for free? Definitely not, as it requires implementation of additional filters at the decoder, signalling of side information. Defining the different phases is not per se necessary from the viewpoint of standardization (as it does not give benefit from the viewpoint of compression performance and memory usage), it would rather be beneficial for encoding devices.

More evidence required about the need and benefits for these applications (current CE conditions did not have such applications in mind, could better be done in AHG)

A more broad range of phase shifts of downsampling should be tested.

Filters for all phases would be needed for arbitrary spatial scalability ratios, that would be included, the arbitrary phase could be implemented with almost no additional cost (is a matter of profiles). No study of arbitrary upsampling ratio performed so far.

Further study on these aspects is necessary before any decision could be taken.

Another test was conducted to investigate performance and complexity summary for tests on relative Luma and Chroma samples alignment.

Current CTC assumes that the 4:2:0 chroma samples are horizontally aligned with each second luma sample, and vertically between a pair of luma samples (which is not known).

a

b

c

d

e

f

SHM2.0+ctc

The following results shows the performance when b is used in downsampling, and a in upsampling (worst case would be a to e). Therefore, slight loss occurs, but this is almost negligible.

Further study was encouraged for:

- Use cases a through f for downsampling of the 4:2:0 sequences, and investigate whether any visible difference appears in the chroma of the base layer between those cases. 5 of these 6 cases will be wrong. If no difference is visible (with extreme sequence e.g. screen content) it does not matter which downsampling phase is used in SHVC.

- Test other extreme cases e.g. a) downsampling and e) upsampling, where it could be assumed that for optimizing the enhancement layer, the same phase should be used in down and upsampling.

- If i) comes true, downsampling and upsampling both assuming a) might be better than the current approach (which would reduce the number of necessary upsampling phases in chroma)

May be tested only in AI.

| Test |  | x2 |  |  |  |  | x1.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Memory |  | BD-rate |  |  | Memory |  |
| s/w | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| SHM2.0 | 0.00 | 0.00 | 0.00 | 100% | 100% | 0.0 | 0.0 | 0.0 | 100% | 100% |
| N0045 | 0.05 | 0.13 | 0.23 | 100% | 100% | 0.03 | 0.10 | 0.05 | 100% | 100% |

### SCE1 primary contributions

JCTVC-N0045 SCE1: Results of Test 1.1.3 on accurate chroma position alignment [X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), J. Dong, Y. Ye, Y. He (InterDigital), K. Ugur, J. Lainema (Nokia)]

JCTVC-N0046 SCE1: Results of Test 1.2.1 on Adaptive Re-Sampling Filter [X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0078 SCE1: Test 2.2 report [K. Minoo, D. Baylon]

JCTVC-N0182 SCE1: Results of test 1.1 and 1.2 on sampling offset signalling [J. Dong, Y. Ye, Y. He (InterDigital), X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), K. Ugur, J. Lainema (Nokia)]

JCTVC-N0308 SCE1: Verification of test 2.1 on picture and sequence level adaptive re-sampling filter [E. Alshina, A. Alshin (Samsung)] [late]

### SCE1 cross checks

JCTVC-N0193 SCE1: Cross-check for Test 2.2 on phase compensation by signalling filter coefficients at PPS with sample shift [J. Dong, Y. Ye (InterDigital)]

JCTVC-N0225 SCE1: Cross-check for Test1.2, The accuracy of signaled up-sampling phase offset [K. Minoo, D. Baylon (Arris)]  [late]

JCTVC-N0226 SCE1: Cross-check for Test1.3, accurate Chroma position alignment [K. Minoo, D. Baylon (Arris)] [late]

## SCE2: Combination of inter-layer syntax prediction and motion data compression

### SCE2 summary and general discussion

(Reviewed in Track B Fri 26th (JRO).)

JCTVC-N0032 SCE2: Summary report of SHVC core experiment on combination of inter-layer syntax prediction and motion data compression [Christophe Gisquet, Kazushi Sato]

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| Combination |  |  |  |
| 1.1 | 2.1 + 2.2 | Canon and Sony | Sharp |
| 1.2 | 2.1 + 2.3 | Canon and Sharp | Sony |
| 1.3 | 2.2 + 2.3 | Sony and Sharp | LG |
| 1.4 | 2.1 + 2.2 + 2.3 | Canon, Sony and Sharp | LG |
| Single Tools |  |  |  |
| 2.1 | M0112 | Canon | Sony |
| 2.2 | M0141 | Sony | Sharp |
| 2.3 | M0258 | Sharp | ETRI |

| Test | Proposal | Cross-check |
| --- | --- | --- |
| 1.1 | JCTVC-N0239 (Sony/Canon), “SCE2: Result of Test 1.1” | JCTVC-N0257 (Sharp) |
| 1.2 | JCTVC-N0302 (Canon/Sharp), “SCE2: Result of Test 1.2” | JCTVC-N0240 (Sony) |
| 1.3 | JCTVC-N0255 (Sony/Sharp), “SEC2 test 1.3 results” | JCTVC-N0300 (LG) |
| 1.4 | JCTVC-N0241 (Sony/Canon/Sharp), “SCE2: Result of Test 1.4” | JCTVC-N0301 (LG) |
| 2.1 | JCTVC-N0139 (Canon), “SCE2: Results on test 2.1” | JCTVC-N0243 (Sony) |
| 2.2 | JCTVC-N0245 (Sony), “SCE2: Result of Test 2.2” | JCTVC-N0259 (Sharp) |
| 2.3 | JCTVC-N0252 (Sharp), “SCE2 test 2.3 motion buffer modification results” | JCTVC-N0122 (ETRI) |

Three basic methods were investigated as follows:

2.1: Use alternative positions of collocated bL MV

The collocated position of the base layer is calculated as;

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where originally R=4, S=4 , subsampling is 4:1

- Tests 1.1, 1.4 (combination with 2.2): R=-2, S=3 (subsampling is 2:1);

- Test 1.2 (combination with 2.3): R=4, S=4 (compression is 4:1, i.e. unchanged)

Gives gain mostly for 2X scalability, overall the gain is marginal

In terms of complexity impact, 2.1 should be roughly identical to SHM (two additional additions), no additional memory accesses or usage. It is just re-defining the position from the BL MV memory that is used to generate the motion field mapping.

The question was raised whether it could happen that an MV is accessed which is outside of the BL MV memory. It is confirmed that the specification text includes a clipping operation preventing this.

Decision: Adopt (2.1 = JCTVC-N0139).

2.2: 2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

2.2 would require additional memory. In the analysis made by the last meeting the additional MV memory was assessed to be increased by 40% (SNR scalability), 10-15% (spatial scalability). Analysis for on-the-fly computation (as suggested by AHG17 and confirmed by JCT-VC to be the common ground for complexity analysis) is not available yet, but likely lower. Provide analysis for on-the-fly computation.

Further analysis was provided and discussion on this issue was performed in Track B on Sunday afternoon (JRO). “On-the-fly” computation in the case of MV would mean that the MV memory corresponding to the EL’s BL reference is written by the time when the base layer MV is decoded. This would in worst case require an additional memory for MV at the EL resolution, regardless whether uncompressed or compressed BL MV are used. However, in a “HLS only implementation” concept, a bigger problem could arise that uncompressed BL MV are only available on the chip and are not accessible.

This was discussed in plenary on Mon. 29th.

Estimated benefit of accessing internal MV memory is 0.7% for 2x scalability; less for 1.5x scalability, nothing for SNR scalability.

Several experts expressed the opinion that it is undesirable to require access to the uncompressed MV memory, as this would require alteration of the BL decoder . It was agreed not to access the finer granularity motion vectors. No action or further investigation was planned on 2.2.

It was also noted that the memory for MV compression is not counted (and there is no specification about limits) in the DPB, as it is usually around 3%. Therefore, several experts suggested whether it would be reasonable to re-consider using the uncompressed MV field (see JCTVC-M0142 et al.). Analysis for memory consumption assuming on-the-fly computation should also be provided for that case.

It was also asked whether the two-stage compression (2:1->4:1) provides the same 4:1 compressed MV field as the base spec. It was later confirmed that this is the case.

Gives gain mostly for 2X scalability; overall the gain is marginal.

For frame-based implementation and worst-case SNR scalability, the MV memory requirements would need to be understood.

2.3: For the motion field mapped from the base layer the motion information is replaced with motion information from a candidate enhancement layer motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer. In particular, the motion information includes - prediction mode, motion vector, reference picture POC, reference picture used as long term status.

The replacement process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

To be provided: Analysis about worst-case computation and memory accesses. JCTVC-N0233 is an extension of the (N0252) method which is said to be simpler, should also be included in this analysis. Further

A high-level analysis was shown in Track B on Sunday afternoon, which showed that different items need to be checked (some at picture level, some at 16x16 block level), but did not contain sufficient detail to understand the worst-case complexity of additional comparisons.

Gives gain for 2X and 1.5X cases (a bit less for the latter)

Further worst case complexity analysis was presented Thursday evening:

- 2 checks at slice level

- 4 checks at block level, each for L0 and L1

Checking for POC differences over the whole reference picture list could be time consuming at the block level, but could be shifted to slice level (15 checks then) and use lookup at the block level.

No gain was shown for AI and LD cases; however a smart encoder could potentially switch the tool off for these cases.

Currently we do not use joint optimization of base and enhancement layer – how would the tool perform in that case?

Further study was encouraged – it would be desirable to also achieve gain in the LD case.

Specification text for 2.2 and 2.3 was not available initially. It was presented in Track B Sunday afternoon for both proposals. The text change for 2.2 is minimal (change in 2 shift operations when determining the collocated MV), whereas the text change for 2.3 reflects the additional checks that are necessary on various syntax elements.

As a general remark, 2.2 and 2.3 provide interesting gains, which should be considered against the additional complexity with the additional information requested

Results:

|  | RA_2x | RA_1.5x | RA_SNR | LDB_2x | LDB_1.5x | LDB_SNR |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1.1 | -0.44% | -0.07% | 0.00% | -0.41% | -0.05% | 0.00% |
| Test 1.2 | -0.49% | -0.18% | -0.16% | -0.11% | 0.02% | 0.00% |
| Test 1.3 | -0.75% | -0.22% | -0.11% | -0.41% | -0.01% | 0.08% |
| Test 1.4 | -0.75% | -0.25% | -0.17% | -0.41% | -0.05% | 0.00% |
| Test 2.1 | -0.15% | 0.01% | 0.00% | -0.11% | 0.02% | 0.00% |
| Test 2.2 | -0.44% | -0.04% | 0.05% | -0.41% | -0.01% | 0.08% |
| Test 2.3 | -0.40% | -0.19% | -0.17% | 0.00% | 0.00% | 0.00% |

Average, also including optional LD P:

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Test 1.1 | -0.15% | -0.19% | -0.22% | -0.16% |
| Test 1.2 | -0.13% | -0.03% | -0.02% | -0.11% |
| Test 1.3 | -0.21% | -0.14% | -0.17% | -0.19% |
| Test 1.4 | -0.24% | -0.19% | -0.23% | -0.23% |
| Test 2.1 | -0.04% | -0.02% | -0.01% | -0.03% |
| Test 2.2 | -0.12% | -0.14% | -0.15% | -0.12% |
| Test 2.3 | -0.10% | -0.02% | -0.02% | -0.08% |

Further discussion N0252 / N0233 Thu p.m. (GS):

Blending of EL & BL motion data

0.5% improvement reported on RA2x.

Complexity analysis presented in revision of N0252. Four checks for each list at 16×16 block level, more checks (~15) at slice level. Negligible runtime perf impact.

Gain is only in RA. The feature was actually disabled in the LD case, as it was not useful there.

It was commented that this requires an extra motion field buffer in the SNR scalabiity case.

The topic was agreed to be for further study.

### SCE2 primary contributions

JCTVC-N0139 SCE2: Results on test 2.1 [C. Gisquet, P. Onno, E. François, G. Laroche (Canon)]

JCTVC-N0239 SCE2: Result of Test 1.1 [C. Gisquet (Canon), K. Sato, J. Xu (Sony)]

JCTVC-N0241 SCE2: Result of Test 1.4 [C. Gisquet (Canon), K. Sato, J. Xu (Sony), K. Misra (Sharp)]

JCTVC-N0245 SCE2: Result of Test 2.2 [K. Sato, J. Xu (Sony)]

JCTVC-N0252 SCE2 test 2.3 motion buffer modification results [K. Misra, A. Segall, J. Zhao (Sharp)]

JCTVC-N0255 SCE2 test 1.3 results [K. Misra, A. Segall (Sharp), K. Sato, J. Xu (Sony)]

JCTVC-N0302 SCE2: Results on combination test 1.2 [C. Gisquet, K. Misra, A. Segall (Sharp)] [late]

### SCE2 cross checks

JCTVC-N0122 SCE2: Cross-check of test2.3 on motion field buffer update [J. Lee, H. Lee, J. W. Kang (ETRI)]

JCTVC-N0240 SCE2: Crosscheck Result of Test 1.2 [K. Sato (Sony)]

JCTVC-N0243 SCE2: Crosscheck Result of Test 2.1 [K. Sato (Sony)]

JCTVC-N0257 SCE2 Cross check report of test 1.1 [K. Misra, A. Segall (Sharp)] [late]

JCTVC-N0259 SCE2 Cross check report of test 2.2 [K. Misra, A. Segall (Sharp)] [late]

JCTVC-N0300 Crosscheck of SCE 2 test 1.4 [C. Kim, B. Jeon (LGE)] [late]

JCTVC-N0301 Crosscheck of SCE 2 test 1.3 [C. Kim, B. Jeon (LGE)] [late]

## SCE3: Inter-layer filtering

### SCE3 summary and general discussion

(Reviewed in Track B Fri 26th (JRO).)

JCTVC-N0033 SCE3: Summary Report of SHVC Core Experiment on Inter-layer Filtering [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong]

This document summarizes the activities and test results performed in SCE3 on inter-layer filtering. Six tools have been evaluated on the test conditions defined in document JCTVC-M1103.

| Test | Methods | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- |
| 3.1 | Switchable alternative inter-layer filter | JCTVC-N0151 (Qualcomm, Samsung) | JCTVC-N0194 (Interdigital) JCTVC-N0343 (Nokia) |
| 3.2 | Adaptive re-sampling filter | JCTVC-N0047 (Qualcomm) | JCTVC-N0303 (Samsung) |
| 3.3 | Interlayer SAO filtering | JCTVC-N0140 (Canon) | JCTVC-N0304 (Samsung) |
| 3.4.1 | Chroma enhancement for inter-layer prediction | JCTVC-N0184 (Interdigital) | JCTVC-N0251 (Sony) |
| 3.4.2 | Simplified cross-color inter-layer prediction | JCTVC-N0152 (Samsung) | JCTVC-N0208 (Qualcomm) |
| 3.5.1 | Separable Bilateral inter-layer filter | JCTVC-N0220 (Sharp) | JCTVC-N0305 (Samsung) |

| Test | Spatial scalability |  |  |  |  |  | SNR scalability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  |
|  | Luma | Chroma | Avg. | Worst | Enc | Dec | Luma | Chroma | Avg. | Worst | Enc | Dec |
| 3.1Pic | −0.1% | 0.0% | 100% | 100% | 100% | 97% | −1.5% | −0.2% | 100% | 100% | 100% | 120% |
| 3.1PU | −0.2% | 0.1% | 137% | 128% | 107% | 123% | −1.9% | −0.1% | 137% | 109% | 103% | 135% |
| 3.2AUF | −0.3% | −0.5% | 100% | 100% | 103% | 100% | −2.0% | −1.4% | 99% | 100% | 101% | 133% |
| 3.3ILSAO | −0.5% | −0.5% | 100% | 105% | 101% | 104% | −1.6% | −1.3% | 99% | 101% | 100% | 112% |
| 3.4.1Chr1 | −0.4% | −8.4% | 100% | 100% | 105% | 109% | −0.3% | −6.5% | 100% | 100% | 107% | 119% |
| 3.4.2Chr2 | −0.4% | −8.2% | 101% | 100% | 99% | 113% | −0.3% | −6.5% | 103% | 100% | 100% | 116% |
| 3.5.1BLF | −0.4% | −0.0% | 143% | 128% | 106% | 136% |  |  |  |  |  |  |

Coding performance, memory access and running time summary of SCE3 tests

| Test | Spatial scalability |  |  |  |  |  | SNR scalability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Access Avg. |  | Memory Access worst |  | BD-rate |  | Memory Access Avg. |  | Memory Access worst |  |
|  | Luma | Chroma | PU | Pic | PU | Pic | Luma | Chroma | PU | Pic | PU | Pic |
| SHM2.0 |  |  | 108% | 157% | 145% | 160% |  |  | 98% | 160% | 200% | 218% |
| 3.1Pic | −0.1% | 0.0% | 107% | 157% | 145% | 160% | −1.5% | −0.2% | 98% | 161% | 200% | 219% |
| 3.1PU | −0.2% | 0.0% | 106% | 218% | 145% | 204% | −1.9% | −0.1% | 97% | 220% | 200% | 237% |
| 3.2AUF | −0.3% | −0.5% | 107% | 157% | 145% | 160% | −2.0% | −1.4% | 97% | 163% | 200% | 220% |
| 3.3ILSAO | −0.5% | −0.5% | 110% | 157% | 145% | 160% | −1.6% | −1.3% | 101% | 159% | 200% | 220% |
| 3.4.1Chr1 | −0.4% | −8.4% | 108% | 158% | 145% | 160% | −0.3% | −6.5% | 99% | 160% | 200% | 219% |
| 3.4.2Chr2 | −0.4% | −8.2% | 109% | 159% | 145% | 160% | −0.3% | −6.5% | 99% | 164% | 200% | 219% |
| 3.5.1BLF | −0.4% | −0.0% | 108% | 228% | 145% | 204% |  |  |  |  |  |  |

Memory access summary using HEVC high layer as anchor

| Test | Technique Summary |
| --- | --- |
| 3.1 JCTVC-N0151 Switchable alternative inter-layer filter | - Low pass smoothing filter on integer Luma samples (5 taps) - Picture level switch on/off - Pu level switch: Both filtered and unfiltered pictures are used as reference - Alternative filter for all phases in Luma resampling process - Picture level switch on/off - Pu level switch: for spatial case, two upsampled inter layer pictures are used as reference |
| 3.2 JCTVC-M0047 Adaptive re-sampling filter | - The adaptive up-sampling filter has same length and coefficient accuracy with the existing up-sampling filter - Filtering process is also applied to integer pixel position - Picture level switch on/off - Filter parameters are signaled at picture level |
| 3.3 JCTVC-N0140 Interlayer SAO filtering | - Edge index is determined by 5 samples instead of 3 samples in the original SAO - Inter layer SAO parameters are coded at slice level. |
| 3.4.1 JCTVC-N0184 Chroma enhancement for inter-layer prediction | - Enhance the Chroma samples by adding a offset which is derived by using the surrounding Luma samples - For each Chroma sample in the interlayer reference picture, applies an adaptive high pass filter to surrounding 4x3 Luma samples of the upsampled base layer picture to derive the offset - The adaptive high pass filter is derived at encoder side for each Chroma plane of a picture - Picture level on/off, filter parameters are signaled at slice header, |
| 3.4.2 JCTVC-N0152 Simplified cross-color inter-layer prediction | - A variant of test3.4.1. The main differences between of test 3.4.1 and test 3.4.2 are - The input of high pass filter of 3.4.1 is the Luma samples of the upsampled base layer picture. - The input of high pass filter of 3.4.2 is the Luma samples of the base layer picture |
| 3.5.1 JCTVC-N0220 Separable Bilateral inter-layer filter | - 3x1 kernel bilateral filter is applied separately for both horizontal and vertical direction - A lookup table is used to replace the final division operation - Both default upsampled picture and bilateral filtered picture are used as interlayer reference pictures |

3.1: Proponents would suggest picture-based on/off (5-tap fixed filter) for SNR scalability only.

Picture-based optimization may introduce additional delay (which applies to most proposals here)

3.2: Both luma and chroma are filtered, filters for all subsampling phases (depending on scalability ratio) are sent at picture level, and on/off information. 7-tap filter for SNR scalability luma, 3-tap for chroma. Zero phase position is also filtered, which means that it increases the complexity versus SHM by more operations, and need to re-load filter coefficients.

3.3: For determining edge index, a second derivative (from 5 samples) is computed, applied after upsampling. The approach is picture-based, not LCU based as the version 1 SAO.

3.4.2 (simplification of 3.4.1, no more latency at the decoder). Coefficients determined by LMS optimization, once per picture. Filter is non-separable, 4 bits per coefficient (11 coefficients transmitted) and scaling factor 11 bits. Operation applied to the up-sampled chroma, roughly 1.5x computations compared to current up-sampling of chroma. Less gain for SNR scalability

3.5 Not operated for SNR scalability. The high increase in decoder run time (36%) is due to the implementation where two upsampling processes are run to write two reference pictures (one with conventional upsampling, and one with conventional upsampling with bilateral filter integrated). In a real implementation, this would be made switchable on PU basis. Bilateral filter requires 9 mul and 15 add in addition to conventional upsampler.

3.1 with PU based adaptation (which is not suggested by proponents) and 3.5 put an additional filtered picture in the reference picture memory, which is assessed to be the reason for the decoder runtime increase.

Gains for some of the proposals are mostly coming from people on street. One expert mentions that de-noising before encoding might also help in that case.

For spatial scalability:

- all proposals add some complexity, in either filtering the integer pel position, or adding an additional filter stage. Typical average luma gains 0.1-0.5%.

- all proposals (except 3.5 and 3.1 PU based) achieve this gain currently by optimizing parameters per picture – unclear whether that gain would be retained if true low latency is required (similar discussions were conducted in the context of version 1 SAO and ALF)

- 3.1 and 3.5 PU based use an additional reference picture for LD-P and AI, or replace one of the duplicate upsampled BL references that current SHM uses. Some of the gain might also be achieved by using one more temporal reference picture, or using the additional encoder complexity for more sophisticated search.

- 3.4 has remarkable gain on chroma, and additional complexity only for chroma upsampling (Note there is a non-CE contribution JCTVC-N0229 that reports even more gain)

None from the proposals from the CE gives sufficient benefit regarding the additional complexity

Several experts expressed the opinion that 3.4 looks most interesting, the new version does not have the additional decoder delay any more. Was further discussed in Track B in context of non-CE contributions on Sun. 28 (JRO). No further action.

For SNR scalability:

- 3.1, 3.2 and 3.3 show gain in the range of 1.5-2 %.

- Additional complexity: 20-30% increase overall decoder runtime; memory access <5%

- Gains are non-equally distributed over set of sequences (without people on street gain would be 0.7-1.3%; without BQ terrace ...??)  - main effect seems to be on noisy sequences

Gain is not justifying additional complexity – no action, no continuation on aspect of filtering SNR scalability reference from the current CE proposals.

(Further Track B discussion Sat. July 27 (AS).)

Further discussion of 3.4 (N0152):

- One expert expressed support to adopt.

- Comment: Requires additional process step and complicates upsample design.

- Suggestion to consider encode solutions to reduce latency.

- Comment: Cross-color introduces additional constraints on design.

- Clarification: Are coefficients fixed? No, variable.

- Comment: Similar to a 12-tap ALF (Agreed).

- Comment: Preferred to use a single channel filter for SNR applications.

- Comment: Gains do not justify complexity.

- Comment: Appears to be a combination of ALF and cross-channel filtering.  Both were considered (and removed) from the HEVC base specification.

- Comment: Continuation of above … gains of original ALF and cross-channel proposals were high in HEVC base specification.

- Comment: The goal of the “refIdx” approach is a clean design.  Cross-channel solutions make the inter-layer prediction quite complex.

No action.

### SCE3 primary contributions

JCTVC-N0047 SCE3: Results of Test 3.2 on Adaptive Re-Sampling Filter [W. Pu, X. Li, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0140 SCE3: Results of test 3.3 on Interlayer SAO filtering for SHVC [G. Laroche, J. Taquet, P. Onno (Canon)]

JCTVC-N0151 SCE3: Results of test 3.1 on switchable alternative inter-layer filter [P. Wei, V. Seregin, J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung)]

JCTVC-N0152 SCE3: Results of test 3.4.2 on simplified cross-color inter-layer inter layer prediction [E. Alshina, A. Alshin, Y. Cho (Samsung), J. Dong, Y. Ye, Y. He (InterDigital)]

JCTVC-N0184 SCE3: Results of test 3.4.1 on chroma enhancement for inter layer prediction [J. Dong, Y. Ye, Y. He (InterDigital)]

JCTVC-N0220 SCE3: Results of test.3.5.1 on adaptive up-sampling of base layer picture using simplified separable bilateral filters [J. Zhao, K. Misra, A. Segall (Sharp)]

JCTVC-N0303 SCE3: Verification of test 3.2 on adaptive re-sampling filter [E. Alshina, A. Alshin (Samsung)] [late]

JCTVC-N0304 SCE3: Verification of test 3.3 on high frequency pass inter-layer SAO [E. Alshina, A. Alshin (Samsung)] [late]

### SCE3 cross checks

JCTVC-N0194 SCE3: Cross-check for Test 3.1 on switchable alternative inter-layer filter [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-N0208 SCE3: Crosscheck of test 3.4.2 on simplified cross-color inter-layer inter layer prediction [X. Li] [late]

JCTVC-N0221 SCE3: Cross-check of test 3.1 on switchable alternative filters (all phases) [J. Zhao, A. Segall (Sharp)] [late]

JCTVC-N0251 SCE3: cross-check of Test 3.4.1 on chroma enhancement for inter layer prediction [J. Xu (Sony)] [late]

JCTVC-N0305 SCE3: Cross-check for test 3.5 (bi-lateral inter-layer filter) [A. Alshin, E. Alshina (Samsung)] [late]

JCTVC-N0343 SCE3: Cross-check of SCE3 Test3.1 non-switchable inter-layer filter [K. Ugur (Nokia)] [late]

# Core experiments in Range Extensions

## RCE1: Inter-component decorrelation methods

### RCE1 summary and general discussion

JCTVC-N0034 RCE1: Summary Report of HEVC Range Extensions Core Experiment 1 on Inter-Component Decorrelation Methods [T. Nguyen, J. Sole, J. Kim]

(Discussed Sat. 27 July p.m. Track A (GJS).)

All testing for this CE is for 4:4:4 content.

Experiment 2 and 7 were withdrawn by the proponent and there was no cross check result for experiment 6. A short description of the experiments is given in the following.

- Experiment 1: LM Chroma (LMC): TU-based parameter derivation

- Experiment 3: In-loop residual prediction (ILP): TU-based signalling

- Experiment 4: LM Chroma with alternative reference sample set (Provided implementation uses even instead of odd positions from the above samples as reference in 4:2:2 sub-sampled content for parameter derivation)

- Experiment 5: ILP: LMC-like variants, is derived from the left and above neighbours, is calculated using luma reconstructed and chroma unquantized residuals, is transmitted in the bitstream (encoder decision)

- Experiment 6: Combination of LMC (Experiment 1) and ILP (Experiment 5)

Detailed results were provided in the CE summary report and are not duplicated here.

Some observations on the results were reported as:

- Experiment 1: Similar results for 4:4:4 and 4:2:2 YCbCr sequences relative to the previous meeting cycle is observed. However, the performance on screen content is constantly higher than for natural content. A degeneration of the improvement is observed for inter-predicted GOP structures.

- Experiment 3: The results are mainly the same as reported by the proponent for RGB content. There are mostly no gain or loss for 4:4:4 YCbCr sequences. In inter-predicted GOP structures, the performance for the first component measured in both colour spaces, i.e., RGB and YCbCr, is higher than in the intra-only configuration.

- Experiment 4: The results for 4:4:4 content is exactly the same as in experiment 1. A negligible loss can be observed for 4:2:2 sub-sampled content.

- Experiment 5: The results show an improvement for mainly 4:4:4 YCbCr sequences while the performance for RGB content is dropped relative to the scheme in experiment 3.

- Experiment 6: Combination of both schemes lead to small improvements for YCbCr relative to experiment 1 and small improvements for RGB content relative to experiment 5. Still the performance for inter-predicted configurations is lower than the performance of the scheme in experiment 3. The combination results in higher performance for screen content.

One participant observed that the bit allocation between the luma and chroma components of the ILP approach (Experiment 3) is mainly a result of the chroma QP offset (QpOffset). The ILP scheme increases the bit depth of the chroma components internally by one for RGB content due to the residual characteristic of the prediction signal. This lead to a higher quantization parameter for the chroma components. The same effect can be achieved by setting the QpOffset for the chroma components equal to 6.

After repeating the test with the QP offset set to 6, the following were observed:

- Experiment 1: The performance for YCbCr is improved relative to experiment 1 for RGB content and the bit allocation between the components are similar to that in experiment 3.

- Experiment 3: Similar results can be observed relative to the original experiment 3.

In terms of complexity analysis, there are two main approach on which the different techniques rely on. The first approach is LM-based and backward-driven and the second approach is forward-driven.

- Backward-driven: The LM-based techniques require a parameter derivation at both decoder and encoder. The complexity is increased consequently at both encoder and decoder.

- Forward-driven: The appropriate mode is transmitted in the bit stream. Encoder complexity is increased but decoder complexity is mostly maintained.

The encoder run time can be limited for forward-driven schemes by applying an early skip technique. Please note that the reported run times are not always reliable because the simulations were run on different computers with different CPU, HDD, RAM etc.

Overall conclusions provided in summary report:

- LM-based (Experiment 1, 4, 5):

  - Somewhat more complexity than forward-driven schemes.

  - Achieve coding improvement for YCbCr sequences (max. 2% and about 1% for inter-predicted GOP structures).

- Forward-driven (Experiment 3):

  - Higher coding improvement in the context of colour space transformation.

  - Especially for screen content RGB.

- ILP and LMC:

  - ILP results in higher coding improvement for inter-predicted GOP structures relative to LMC in the context of colour space transformations.

  - LMC results in coding improvement for YCbCr sequences while ILP doesn’t show any effects.

- Combined techniques:

  - Combining LM-based ILP and LMC results in best performance for RGB screen content.

  - The improvement for YCbCr sequences is maintained and the performance of ILP on RGB content can be achieved.

- QpOffset for chroma:

  - A QpOffset for chroma results in higher YCbCr performance for all schemes

  - For the anchor itself, only a bit allocation effect is observed

Simplified conclusions from RCE report presentation:

- ILP shows no gain/loss for YCbCr sequences.

- Similar results for LMC relative to previous meeting cycles – small modification for 4:2:2 only.

- LM-based approaches achieve gain for YCbCr sequences () but less performance for RGB.

- ILP can be applied for inter.

- Additional bit depth of ILP unnecessary.

Some comments during the presentation were as follows:

- It was asked about the interaction with other techniques when coding screen content.

- For RGB, it was commented that external conversion to YUV is an alternative.

- There was some questioning of whether PSNR is measured in the appropriate space – whether PSNR should be measured in RGB domain or in YUV domain?

- Should deblocking be adjusted to account for the QP offset effect? That hadn't been explored.

- It was suggested to consider using both of the first two components to predict the third. However, another participant commented that the B component is usually noisy, and this usually doesn't help.

Overall conclusions from the JCT-VC discussion:

- For YCbCr natural content, there is gain for LM approaches (but this has substantial complexity for both encoder and decoder).

- LM chroma gain for YCbCr is about 2% for intra, 1% for RA, 0.2% for LD.

- LM is only for intra, so its benefit is primarily for the AI case. ILP is not just for all-intra.

- For RGB content, the non-LM approaches are better (this one requires mode selection in the encoder, but is not so difficult in the decoder). However, has been agreed that RGB is not so high priority in our current work, unless we change our mind about that, this diminishes the potential interest in the ILP scheme.

- YCbCr has a bit allocation effect. External YCbCr conversion has a similar effect as ILP; any additional observed gain is primarily from the chroma QP offset effect.

- There are two closely-related non-RCE contributions: N0223 and N0266. One allows negative slope values for LM chroma, and shows gain for YCbCr.

See the discussion of the related non-RCE contributions.

### RCE1 primary contributions

JCTVC-N0170 RCE1: Description and Results for Experiment 3, 5, and 6 [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-N0227 RCE1: EXP1/4 The performance of extended chroma mode for non 4:2:0 format [J. Kim, B.Jeon (LGE)]

### RCE1 cross checks

JCTVC-N0114 RCE1: Cross-check on Experiment 5 [A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-N0171 RCE1: Cross Check Results for Experiment 1 and 4 [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-N0298 RCE1: Cross-check on Experiment 6 [Wei Pu, Woo-Shik Kim (Qualcomm)] [late]

## RCE2: Prediction and coding techniques for transform-skip blocks

### RCE2 summary and general discussion

JCTVC-N0035 RCE2: Summary report on HEVC Range Extensions Core Experiment on prediction and coding techniques for transform-skip and transform-bypass blocks [R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari]

(Reviewed in Track A Sun. 28th p.m. (GJS).)

This is a summary report on HEVC Range Extensions Core Experiment 2 on prediction and coding techniques for transform-skip and transform-bypass blocks. The core experiment investigated prediction and coding techniques for transform-skip and transform-bypass blocks. The core experiment consisted of subtests that investigated update of the Rice parameter for level coding, rotation, flipping or other scan changes and replacement of specific intra prediction modes. Performance of the proposed methods as well as their combinations was evaluated for lossy and lossless configurations based on the test conditions and sequences described in JCTVC-M1122.

The CE contains:

- Subtest A: Rice parameter modifications (3 methods).

- Subtest B: Rotation, flipping, or scan changes.

- Subtest C: Replacement of specific intra prediction modes (RDPCM for intra for lossy, RDPCM for inter for lossy and lossless, replacing DC mode with something else, and sample-adaptive weighted prediction in place of the planar mode).

- Subtest D: Combinations within subtest C.

In subtest A, the focus is on performance for lossless (and near-lossless), intra, and screen content. Especially lossless. A.3 seems to perform a little bit better than A.2. A.1 has higher complexity than A.2 and A.3 (but more gain). The complexity impact (esp. for A.2 and A.3) is relatively low.

It was remarked that Rice parameter changes can be especially helpful with higher bit depth. It was noted that we could have profile-based restrictions – e.g., for CABAC changes to only be in effect for profiles with higher bit depths.

It we take action on this, it would probably be A.3.

For subtest B, there are three variants.

In subtest B, the focus is on performance for screen content for lossy coding (and it also helps on lossless). B.2 is simpler than the other two (but has less gain).

The lossy case is only at 4x4.

It is also suggested to consider only applying the modification to smaller blocks (perhaps the same block size for which lossy coding applies).

It was suggest to focus on the variant with the extra context.

Lossless:

Techniques B.1 and B.3 do better here than technique B.2 – both for lossy and lossless.

Lossy:

It was agreed that some action should be taken on this topic.

It was noted that it is proposed to enable transform skip for non-4x4 block sizes, and it is proposed to enable RDPCM for lossy coding. Some participants suggested considering the interactions.

Subtest C considers RDPCM for intra for lossy, RDPCM for inter for lossy and lossless, replacing DC mode with something else, and sample-adaptive weighted prediction in place of the planar mode).

Regarding CU versus TU level operation, it was suggested that CU level is easier for the encoder. However, TS is a TU level decision and it was suggested to keep the decision at the TU level.

C.1 is only for inter, both for lossy and lossless. C.2 extends our current RDPCM to the lossy case (for intra). So these are essentially independent. Combining C.1 and C.2 is subtest D.1.

C.1 shows benefit (in YUV) only for SC – and only for LD, because it is only for inter, and the bits are allocated mostly to intra. There might be gain if applied to other block sizes – but 4x4 only for now.

C.2 is also only beneficial for SC.

If C.2 is in the design, C.1 might have no added complexity other than syntax flag parsing on some architectures.

Decision: Adopt C.2. Also adopt C.1. (This was pending that C.1 is not the only change to CABAC parsing, and it isn't.)

(Further discussion Monday 29th p.m. (GJS).)

Decision: Need an enable flag for each feature. There should be an enable/disable flag at the SPS level for RDPCM – one flag for intra and one for inter.

It was noted that RDPCM disables sign data hiding for the blocks that use that mode.

C.3 has a 4-pixel neighbourhood average with focus on lossless (more complex), replacing the planar mode.

This is interesting, but no action taken (since this is only for lossless and has significant complexity).

C.4 is a DC mode replacement, using a parallelized neighborhood-based average. It is both for lossless and lossy, and somewhat higher complexity than RDPCM (esp. for encoder in lossy case).

Primarily for screen content.

On top of C.2, the gain of C.4 is less – about 1% in AI for YUV 444.

No action due to reduced benefit relative to added complexity when considered on top of C.2.

In N0207, B.3 (with extra context) was tested on top of C2+C1, and the gain was mostly (but not completely) additive.

In N0222, C2 + C1 + A.3 + B.1 + TS8x8 + N0181 (Rice param change) was tested.

Discussion of combinations to test and further discussion relative to C2, for Class F and YUV 444 (Discussed Thu 1st (GJS)):

- + C1 = D1 (RDPCM stuff, already had these results):

- + A.3 (Rice parameter stuff) Conclusion: Essentially additive.

- + special context Conclusion: Essentially additive.

- B variations (mirroring/flipping)

  - B.1 rotation of residue (lesser of the more complex two)

  - B.2 swapping of vert/horiz scans (simplest)

  - B.3 mirroring of the scan (most complex)

JCTVC-N0388 BoG on combination of tools in RCE2 [J. Sole]

During the discussion of RCE2 proposals, it was requested to analyze the BD-rate gains obtained by several RCE2 tools when tested on the top of each other. The BoG generated the software to conduct these tests and provided the requested set of results.

Decision: D1 + context + B.1 (4×4 only).

Defer A.3 to further study in a CE (with other techniques).

### RCE2 primary contributions

JCTVC-N0044 RCE2 Test B.1: Residue rotation and significance map context [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0052 RCE2 subtest C.2: Extension of residual DPCM to lossy coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0053 RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0071 RCE2: Simplified sample-based weighted prediction (Test C.3) [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

JCTVC-N0073 RCE2: Description and report for Test D.3 [E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens), R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0074 RCE2: Experimental results for Test C.1 [M. Naccari, M. Mrak (BBC)]

JCTVC-N0083 RCE2 subtest D.1: Extension of intra and inter residual DPCM to lossy coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm), M. Naccari, M. Mrak (BBC)]

JCTVC-N0096 RCE2: Test B.2: Swapping of scan order for transform skip/bypass [R. Cohen, A. Vetro (MERL)]

JCTVC-N0101 RCE2 Test A.3: Simplified update of the coefficient level Rice parameter [J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0175 RCE2 Test B.3: Mirroring of Coefficients and significance map context [R. Weerakkody, M. Mrak (BBC)]

JCTVC-N0207 RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) [R. Weerakkody, M. Mrak, M.Naccari (BBC)]

JCTVC-N0228 RCE2: Results for Test A.2 Rice parameter update [M. Budagavi (TI)]

JCTVC-N0268 RCE2 Test A.1: Rice parameter signalling for transform-skip blocks [S. H. Kim, A. Segall (Sharp)]

JCTVC-N0354 Non-RCE2: Results for combination of Mirroring of non-transformed residual blocks (Test B.3) with other methods [R. Weerakkody, M. Mrak, M. Naccari (BBC)] [late]

### RCE2 cross checks

JCTVC-N0076 RCE2: Cross-check for Test B.2 [M. Naccari, M. Mrak (BBC)]

JCTVC-N0097 RCE2: Cross-verification of JCTVC-N0044, Test B.1: Residue rotation and significance map context [R. Cohen (MERL)]

JCTVC-N0098 RCE2: Cross-verification of JCTVC-N0073, Description and report for Test D.3 [R. Cohen (MERL)]

JCTVC-N0099 RCE2: Cross-verification report for subtest B.3 (N0175) on mirroring of coefficients [R. Joshi (Qualcomm)]

JCTVC-N0103 RCE2: Cross-verification of Test C.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-N0105 RCE2: Cross-check of Test A.2 on coefficient level Rice parameter update (JCTVC-N0228) [J. Sole (Qualcomm)]

JCTVC-N0123 RCE2: Cross-check of Test A.2 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0136 RCE2: Cross-check of Sub-test D.1 (JCTVC-N0083) by the BBC and Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-N0156 RCE2: Crosscheck for Test C.3 [W. Zhang, Y. Chiu (Intel)] [late]

JCTVC-N0164 RCE2: Cross-check results for Test C.4 (JCTVC-N0053) [E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens)]

JCTVC-N0328 Cross-check report for RCE2 C2 [J. Ye, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0329 Cross-check report for RCE2 A1 [J. Ye, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0330 Cross-check report for RCE2 B3 [J. Ye, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0337 RCE2: Cross-check report for Test A3 [S.-H. Kim, A. Segall (Sharp)] [late]

JCTVC-N0370 A Cross-Check report of N-0354 [H. Chen, A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-N0365 Cross-check for JCTVC-N0207: RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) [K. Sharman, N. Saunders, J. Gamei (Sony)] [late]

## RCE3: Intra coding methods for screen content

JCTVC-N0036 RCE3: Summary Report of HEVC Range Extensions Core Experiment 3 on Intra Coding Methods for Screen Content [L. Guo]

(Reviewed Mon. 29th, Track A (David Flynn).)

A summary of RCE3 on Intra coding methods for Screen Content for HEVC Range Extensions is reported. Three methods have been evaluated based on the CE description in JCTVC-M1123.

The CE performed tests, covering two subject areas:

- Palette coding (two methods, tests one and two), where for each block, a dictionary (palette) of pixel values is transmitted, and the block consists of indices into the dictionary.

- Intra motion compensation (an intra picture block copying operation), with two subtests three and four that respectively disable and enable limiting the vertical vector to within the current CTU.

No combination tests have been performed.

All methods tested only offer benefits for screen content material, with either no gain or minor losses for non-screen content.

Complexity assessments are reported for one method (Test 1).

For the palette coding methods, the main difference between the two methods is the size of the dictionary, the use of a pixel prediction method and the entropy coding method.  Test 2 seems to report higher gains than test 1, but has a higher runtime cost.

### RCE3 summary and general discussion

### RCE3 primary contributions

JCTVC-N0205 RCE3: Results of test 3.3 on Intra motion compensation [D.-K. Kwon, M. Budagavi (TI)]

(Reviewed in Track A Tue. 30th (DF).)

Applications such as wireless displays, automotive infotainment, remote desktop, remote gaming, cloud computing etc. are becoming popular. Video in these applications often has mixed content consisting of natural video, text, graphics etc. In text and graphics regions, patterns (e.g. text characters, icons, lines etc.) can repeat within a picture. This contribution proposes a CU-level intra motion compensation tool to remove this redundancy to reportedly achieve coding gain. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is signalled. The proposed method is tested with two different search ranges in the encoder. In the first test, horizontal and vertical motions are limited to a range of 0–63. In the second test, the vertical motion is further limited so that displaced block does not go beyond LCU boundary. The following bit rate savings under RCE3 common conditions for lossless coding are reported:

- First test - Class F: (2.5%/1.4%/0.7%), SC RGB 444: (25.2%/19.9%/16.6%), SC YUV 444: (20.7%/18.9%/17.0%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- Second test - Class F: (2.3%/1.4%/0.6%), SC RGB 444: (21.5%/17.0%/14.8%), SC YUV 444: (18.5%/16.6%/16.1%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

The following average luma BD-Rate savings under RCE3 common conditions for lossy coding are reported:

First test - AI-MT: 19.0%, AI-HT: 18.4%, AI-SHT: 17.9%, RA-MT: 15.7%, RA-HT: 15.2%,  LDB-MT: 12.0% and LDB-HT: 11.7%.

Second test - AI-MT: 16.8%, AI-HT: 16.3%, AI-SHT: 15.9%, RA-MT: 13.8%, RA-HT: 13.4%,  LDB-MT: 10.5% and LDB-HT: 10.0%.

The method tested is an intra picture block copying operation that, on a CU basis, permits copying a block of reference samples using a 1D integer vector in either the horizontal or vertical direction.  Vector ranges are 0 to 63, offset by the current CU width, and are coded with a 6-bit magnitude and direction.  For chroma, the luma vector corresponds to half-pel sampling, with interpolation performed using the current chroma inter interpolation filter.

A variant is tested that limits the vertical range of the vector such that all reference samples reside within the current CTU.

The mode is signalled prior to the current CU pred_mode flag, completely bypassing the current intra syntax.  There is no NxN split for the smallest CU size.  The transform tree is not affected.

An example of redundancy exploited was provided in the presentation, although figurative it would require a 2D vector.  An expert wondered if 2D vectors would these be more appropriate.

It was also remarked that it would be helpful to provide a vector map overlay showing the mode utilisation and vector lengths.

Class F result averaging may be pessimistic, since some class F sequences contain very little screen content, rather they just contain some small captions.

JCTVC-N0247 RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding [L. Guo, M. Karczewicz, J. Sole (Qualcomm)]

(Reviewed in Track A Tue. 30th (DF).)

This contribution reports the results of RCE3 test 3.1 on palette coding for screen content.

The method tested is a palette coding mode that bypasses the HEVC intra prediction, transform and quantisation, where a palette is signalled at the start of each CU, and then individual pixel values are signalled by indexing the palette.  To reduce the palette overhead, each palette may be inherited from a previous CU.  The indexed pixel values are signalled using three modes: 1) where a run of pixels are copied from the row above, 2) where a run of pixels have the same palette index, and 3) when no entry exists in the palette, a PCM sample value may be sent.

Lossless has no benefit in class B, a small benefit in class F (1.3% intra), screen content RGB (25%), YUV (11.6%) for a 113% encoder runtime cost.

Lossy: As QP decreases, the gains are increasing.  For Class F (0.4% – 1.6%), SC RGB (14% – 25%) C YUV(3.7% – 9.3%).  Similar gains are reported for Random Access.  For Low delay, gains are diminished by about 3/4.

Concern was expressed about the level of parallelism at the encoder for deriving the palette.  First the palette must be determined and then the values mapped to the palette indices.

A cross-checker reported that encoder complexity needs to be limited in the application areas that this proposal addresses.  In the current implementation, the palette is formed by calculating a histogram of the current pixel values, and a sorting operation may be costly.  There needs to be consideration of the palette size, signalling and encoder complexity.

JCTVC-N0287 RCE3 Test 2: Multi-stage Base Color and Index Map [W. Zhu (BJUT), J. Xu (Microsoft), W. Ding (BJUT)]

(Reviewed in Track A Tue. 30th (DF).)

This contribution presents the description and results of RCE3 test 2. In the test, a new mode for screen content coding is used. In the new mode, each intra block is represented by several base colors and an index map for each pixel to indicate which base color is used as the reconstructed value for that specific pixel. The coding of an index uses neighbouring indices as predictions. Compared with the HM10.1_RExt3.1 anchor, on average 15.8%, 41.1% and 26.0% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the All Intra HE Main-tier case; 11.3%, 35.1% and 21% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Random Access HE Main-tier case; 7.7%, 29.6% and 15.7% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Low Delay HE Main-tier case.

Compared to N0247:

- A prediction of the pixel index values is formed on a pixel-by-pixel basis, where the prediction direction for the current pixel is estimated using previously reconstructed neighbouring samples.

- Predictors and index values are CABAC coded

Predicted pictures do not use the palette mode.

The palette is formed on a CU basis and never copied from a previous block.  The palette size may vary, and to find the optimum palette size, a search is performed starting from a small number of entries and iteratively increases.  An early termination method is used to reduce the search time.

Chroma palette size is related to sampling format.

This technique offers higher gains than N0247 but with a greater run time penalty.

Concern was expressed regarding the encoder and how to make it more parallel.  It was commented that an RDO search is required, since using a simpler metric would be difficult.  More study may be required to see how such a mode search fits into implementation budgets.

There is a decoder parallelism concern regarding the throughput of the pixel-by-pixel prediction.  Previous methods with serial decoding loops have been discouraged in the past.

### RCE3 cross checks

JCTVC-N0104 RCE3: Cross-verification of Test 3.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-N0125 RCE3: Cross-check of Test 3.2 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0126 RCE3: Cross-check of Test 3.3 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0326 Cross-check report for RCE 3.1 [X. Wang, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0327 Cross-check report for RCE 3.3 [X. Wang, Z. Ma, M. Xu (Huawei)] [late]

# Non-CE Technical Contributions

## Range extensions

### RCE1 related (inter-component decorrelation)

JCTVC-N0266 Non RCE1: Inter Color Component Residual Prediction [W. Pu, W.-S. Kim, C. Chen, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

(Reviewed in Track A (GJS) 27th p.m.)

In the 13th JCT-VC meeting, seven experiments were included in RCE1 to study inter-component decorrelation methods. This proposal presented an asserted improvement of RCE1 Experiment 3. In the proposed method, the chroma residual is predicted using the scaled luma residual signal. The scaling factor is signalled for each TU. Compared to Experiment 3 of RCE1, a wider range of scaling values are allowed in order to improve the prediction performance. A flag is signalled to switch on/off the method. In case of intra coding, the method is allowed only when DM mode is used as chroma prediction mode. For common coding conditions, the average BD-rate of the method is reportedly -20.2%, 2.1%, -1.1% for Y, U, V, respectively. For the screen content sequences, the BD-rate is reportedly -23.0%, -13.7%, -13.8% for Y, U, V, respectively.

This is an ILP technique. For RGB coding, using only positive coefficients is best. However, for YCbCr, negative are suggested to be allowed. The proposal increases the alpha range accordingly. The alpha does not need to be computed at the decoder side – it is signalled directly. Thus there is lower complexity for the decoder than for a method in which the decoder computes the correlation.

The encoder uses calculation, not exhaustive testing, so the encoder search is not a big issue.

A QP offset can be used or not.

Significantly improved gain was reported for YUV 4:4:4: AI/RA/LB gain was reported as 1.5%/0.5%/0.3% for luma, and 6-8% for chroma. For screen content, the corresponding gains were 6.9%/6.0%/5.9% for luma, and 8-10% for chroma.

The results were cross-checked.

The signalling is at the TU level. Thus, tere is substantial overhead for signalling. The proponent however indicates that TU level operation is best. He has tested others, but found TU level to be best.

To convert luma resolution to chroma resolution for 4:2:2, the (encoder and) decoder decimates the signal.

The technique has been tested for 4:2:2 (and 4:4:4), but not 4:2:0.

The sent information includes, for u and v, the magnitude and sign of alpha.

Some participants suggested strong consideration for use of this technique in a profile supporting 4:4:4 and were leaning toward adoption. Deeper study was certainly supported. However, a couple of participants thought the technique was too complex for the provided benefit (esp. for the encoder).

One participant remarked saying there are other better things we can to in the case of SCC.

The perceptual effects have not been investigated.

The proponent said that when using YCbCr coding, the QP offset idea discussed in the context of RCE1 should be applied.

It was remarked that there is a need to confirm the timescale of the RExt work. Further study in a CE was suggested.

JCTVC-N0366 Cross-check for JCTVC-N0266: Non-RCE1: Inter Color Component Residual Prediction [K. Sharman, N. Saunders, J. Gamei (Sony)] [late]

JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions [J. Dong, Y. Ye, Y. He (InterDigital)]

(Reviewed Sun. 28th p.m. Track A (GJS).)

This contribution proposes an in-loop chroma enhancement filtering scheme to HEVC Range Extensions, which is performed after SAO and before the reconstructed picture is added into the DPB. It aims at enhancing the chroma planes of a reconstructed picture, which, if used as a reference picture, also improves the accuracy of future MCP for chroma components. Specifically, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the chroma enhancement filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. Experimental results based on common test condition JCTVC-L1006 show that the average {Y, U, V} gain over three color formats (i.e., RGB 4:4:4, YUV 4:4:4, YUV 4:2:2) is {0.0%, -3.0%, -4.9%}, {0.0%, -1.4%, -3.2%}, {0.0%, -0.6%, -1.7%}, {0.2%, -6.0%, -8.4%}, {0.0%, -3.6%, -6.8%}, {-0.1%, -4.6%, -6.7%}, and {-0.2%, -2.7%, -5.1%} for AI-MT, AI-HT, AI-SHT, RA-MT, RA-HT, LB-MT, and LB_HT, respectively.

The original idea of this chroma enhancement filtering scheme was proposed to SHVC to enhance the chroma planes of an ILR picture (JCTVC-L0059 and JCTVC-M0183), and in this contribution is extended to the HEVC Range Extensions single layer coding as an in-loop chroma enhancement filtering scheme.

It was noted that this could be applied as a post-filter, and this was the topic addressed in JCTVC-N0224.

However, no comparison was provided of the effectiveness of the technique to the same test sequences when applying in-loop versus out-of-loop processing.

It was also noted that Wiener filtering within a single component rather than across components (as done in this contribution) can provide a benefit, and would be less complex from the decoder perspective. However, no comparison was provided of the effectiveness of the technique in such a manner.

It was also noted that ALF could be considered somewhat similar in spirit.

The tested technique was constrained to be a high-pass filter. It was asked whether this constraint harms performance, and commented that it does not.

Line buffering was commented to be an issue.

Subjective viewing was suggested if this is to be studied further.

It was noted that in the RA Main Tier case, there was luma degradation (0.6%) that seemed significant enough to potentially offset the chroma improvement (6%).

The technique was not tested on screen content.

Further study was encouraged.

JCTVC-N0359 Non-RCE1: Cross-check of JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions [L. Guo (Qualcomm)] [late]

- JCTVC-N0368 Non-RCE1: Chroma intra prediction with mode-dependent reduced reference [K. Kawamura, T. Yoshino, S. Naito (KDDI)] [late]

(Reviewed Sun. 28th p.m. Track A (GJS).)

This contribution proposes the two chroma prediction method with reduced references which predicts chroma samples by using linear combination of luma samples for non 4:2:0 format. When a block size is large, a load of the parameter derivation process for each transform unit is reduced by using limited reference samples. One method utilizes fixed reduction pattern, while the other utilizes luma-intra mode-dependent reduced pattern. The Y BD-rate gains of HE Main / High / Super-High tiers are 1.8% / 1.5% / 1.0% for all intra conditions of YUV4:4:4. Compared with the top of TU-based chroma prediction in JCTVC-N0227, the Y BD-rate loss is less than 0.1% for whole AI conditions while the number of reference samples are limited.

The result for natural content YCbCr is reportedly similar to LMC.

The asserted benefit is to reduce the complexity of the coefficient derivation (in both encoder and decoder) relative to TU-based LMC, by the reduced reference usage.

### RCE2 related (prediction and coding for transform skip)

Discussed in Non-RCE2 BoG (R. Joshi)

JCTVC-N0384 BoG report on non-RCE2 contributions [R. Joshi, R. Cohen]

N0042: Restriction of RDPCM block size: No action.

N0075: RDPCM reset: No action.

N0100: For further study.

N0288: Extending TS to non-4×4 block sizes and N0167: Always allow TS at minimum block size.

- Suggestion to not necessarily use it in CTC encoding. Suggestion for encoder to indicate the max TU size at which it is used (at same level of syntax as TS on/off). Rotation only at 4×4. Single context for TS regardless of block size. No change to default QM. Decision: Adopt as described.

N0079 and N0176 and N0177 Sample based intra prediction

- Consider only 45 degree diags (modes 2, 18, 34)

- Consider lossless/lossy only if the same processing

- For further study in a CE.

N0080 lossless neighbouring filtering

- Decision: Add flag to disable all intra pred smoothing at SPS level. (Turn on the flag when doing lossless CTC tests).

N0113: Cross RDPCM: For further study.

N0183: Nearest-neighbour intra pred

- Further study in a CE

JCTVC-N0137 Non-RCE2: Golomb-Rice parameter initialization for transform-skip and transquant-bypass modes [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

JCTVC-N0350 Non-RCE2: Cross-verification of JCTVC-N0137, Golomb-rice parameter initialization for transform-skip and transquant-bypass modes [R. Cohen (MERL)] [late]

JCTVC-N0181 Non-RCE2: Rice Parameter Initialization [M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

JCTVC-N0232 Non-RCE2: Rice parameter update method [J. Min, S. Lee, C. Kim (Samsung)]

JCTVC-N0325 Non-RCE2: Cross-verification of JCTVC-N0232 Rice parameter update method [L. Guo (Qualcomm)] [late]

JCTVC-N0281 Non-RCE2 Rice parameter extension for transform-skip blocks [S. H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-N0333 Non-RCE2 Cross-check of N0281 (Rice parameter extension for transform-skip blocks) [J. Min, S. Lee (Samsung)] [late]

JCTVC-N0042 Non-RCE2: Restriction on the Residual DPCM block size [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0279 Cross-check of JCTVC-N0042 on Restriction on the Residual DPCM block size [E. François (Canon)] [late]

JCTVC-N0072 RCE2-related: Variants of simplified sample-based weighted prediction [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

- JCTVC-N0364 Non-RCE2: A cross-verification report of N0072 [R. Joshi (Qualcomm)] [late]

JCTVC-N0075 Non-RCE2: Complexity reduction for inter residual DPCM in lossless coding [M. Naccari, M. Mrak (BBC)]

JCTVC-N0293 Cross-check of complexity reduction for inter residual DPCM in lossless coding (JCTVC-N0075) [J. Xu (Microsoft)] [late]

JCTVC-N0079 Non-RCE2: Simplified sample based intra prediction for lossless coding [J.Zhu, W. Zheng, K. Kazui (Fujitsu)]

JCTVC-N0367 Non-RCE2: cross-check of simplified sample based intra prediction for lossless coding (JCTVC-N0079) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-N0080 Non-RCE2: Skip of neighbouring samples filtering in intra prediction for lossless coding [J. Zhu, K. Kazui (Fujitsu)]

JCTVC-N0100 Non-RCE2: Unified lossless residual coding [Y. H. Tan, C. Yeo (I2R)]

JCTVC-N0321 Cross-check for JCTVC-N0100: Non-RCE2: Unified lossless residual coding [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0176 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossless coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

JCTVC-N0319 Crosscheck of sample adaptive intra prediction for oblique modes in lossless coding (JCTVC-N0176) [D.-K. Kwon (TI)] [late]

JCTVC-N0177 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossy coding [A. Saxena, H. Chen, F. Fernandes (Samsung)]

JCTVC-N0363 Non-RCE2: A cross-verification report of N0177 [R. Joshi (Qualcomm)] [late]

- JCTVC-N0386 Cross-check for JCTVC-N0177: Non-RCE2: On sample adaptive intra prediction for oblique modes in lossy coding [M. Naccari, M. Mrak, R. Weerakkody (BBC)]

JCTVC-N0222 Non-RCE2: Results for combination of methods [J. Sole, R. Joshi, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-N0353 Cross-check for JCTVC-N0222, Non-RCE2: Results for combination of methods [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0258 Non-RCE2: Extension of TU-Based Inter RDPCM [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0342 Cross-check of JCTVC-N0258 on TU-Based inter RDPCM extension [H. Yang (Huawei)] [late]

JCTVC-N0288 Non-RCE2: Transform skip on large TUs [X. Peng, J. Xu (Microsoft), L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0335 Cross-check for JCTVC-N0288: Non-RCE2: Transform skip on large TU [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0167 Transform skip based on minimum TU size [Kwanghyun Won, Seungha Yang, Byeungwoo Jeon (SKKU)] [late]

JCTVC-N0289 Cross-check of transform skip based on minimum TU size (JCTVC-N0167) [J. Xu (Microsoft)] [late]

JCTVC-N0113 Cross Residual DPCM for HEVC lossless coding [Yung-Lyul Lee, Sung-Wook Hong]

### RCE3 related (intra coding for screen content)

JCTVC-N0169 Non-RCE3:Template-based palette prediction [Wenjing Zhu, Haitao Yang (Huawei)]

(Reviewed in Track A Tue. 30th (DF).)

This contribution proposes a template-based palette prediction method. In contrast with existing palette coding scheme where all palette elements of the current block are directly coded, an element in the palette of the current CU can be predicted from an element in the palette template with the proposed palette prediction method. The palette template is initialized in the beginning of a slice, and is updated every time when a CU with palette coding mode is processed. The proposed method is compared against the existing palette coding scheme MBCIM in the performance evaluation. Simulation results show that compared to RCE3.2, on average 1.3%, 2.6% and 2.2% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the All Intra HE Main-tier case; 1.1%, 2.7% and 1.8% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Random Access HE Main-tier case; 0.8%, 2.6% and 2.0% bit-saving are achieved for class F, SC RGB 444 and SC YUV 444 sequences in the Low Delay HE Main-tier case.

Results presented are a delta to N0287.

The scheme is a palette prediction scheme, using a cache of eight entries and a least-recently-used eviction policy. The update occurs once per CU. The cache (or template) is used to predict the contents of the palette for the next CU, and a difference coding method is employed.

JCTVC-N0351 Cross-check of template-based palette prediction (JCTVC-N0169) [J. Xu (Microsoft)] [late]

JCTVC-N0206 Non-RCE3: Intra motion compensation with variable length intra MV coding [D.-K. Kwon, M. Budagavi (TI)]

(Reviewed in Track A Tue. 30th (DF).)

The CU-level intra motion compensation method of RCE 3.3.3 (JCTVC-N0205) uses fixed length coding for intra motion vectors. In this contribution, it is proposed to use variable length coding for the intra motion vector. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is allowed for a CU. The horizontal motion is limited so that a displaced block does not go beyond two previous LCUs and the vertical motion is limited so that a displaced block does not go beyond the current LCU boundary. Therefore, the proposed intra MC method requires only two left LCUs to be stored additionally. Since intra motion vector is variable length coded, it is possible to use larger horizontal search range than fixed length coding. Intra motion is binarized by using the 3rd order Exp-Golomb code and each bin is bypassed coded. The proposed method is compared with HM RExt-3.0 anchor and RCE 3.3.3 Method 2, which requires the same additional memory for intra MC. The proposed method with 1 left LCU reference is also compared with HM-RExt-3.0 anchor. The following bit rate savings under RCE3 common conditions for lossless coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - Class F: (3.1%/1.9%/0.8%), SC RGB 444: (24.0%/18.9%/16.7%), SC YUV 444: (21.6%/19.6%/19.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - Class F: (0.9%/0.5%/0.2%), SC RGB 444: (3.4%/2.6%/2.7%), SC YUV 444: (4.4%/4.0%/4.2%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

- 1 Left LCU compared to HM RExt-3.0 anchor - Class F: (2.5%/1.5%/0.7%), SC RGB 444: (22.1%/17.5%/15.3%), SC YUV 444: (19.3%/17.4%/16.8%), Class B and RangeExt: (0.0%/0.0%/0.0%) for AI/RA/LDB.

The following average luma BD-Rate savings under RCE3 common conditions for lossy coding are reported:

- 2 Left LCUs compared to HM RExt-3.0 anchor - AI-MT: 19.3%, AI-HT: 18.7%, AI-SHT: 18.2%, RA-MT: 15.8%, RA-HT: 15.3%, LDB-MT: 12.5% and LDB-HT: 12.2%.

- 2 Left LCUs compared to RCE 3.3.3 Method 2 - AI-MT: 3.6%, AI-HT: 3.4%, AI-SHT: 3.3%, RA-MT: 2.8%, RA-HT: 2.7%, LDB-MT: 2.7% and LDB-HT: 2.4%.

- 1 Left LCU compared to HM RExt-3.0 anchor - AI-MT: 17.5%, AI-HT: 17.0%, AI-SHT: 16.5%, RA-MT: 14.3%, RA-HT: 13.9%, LDB-MT: 11.0% and LDB-HT: 10.9%.

The proposal extends the range of the horizontal and vertical block copying vectors to the limit of the neighbouring CTU boundary, offering a coding benefit compared to RCE3.3 option 2 (which has a fixed horizontal search range and a limited vertical range).  The horizontal search range, for 64x64 CTUs, may be up to 119 for the right-most 8x8 CUs in a CTU.  Since the larger range is not supported by the original coding method, a replacement binarisation is proposed using exp-golomb codes.

The entropy coding change on its own shows negligible coding gain.  The reported gains come from the extended search range.  The encoder implementation used will incur a run time penalty, but no run times were presented. (Estimated at ~130% for intra against anchor).

It was commented that normally when sending motion vectors, prediction is performed and the MV difference is transmitted.  In this case (and RCE3 tests 1 and 2) there are no MV predictors.

JCTVC-N0348 Non-RCE3: Cross-check of Intra Motion Compensation in JCTVC-N0206 [W.-S. Kim (Qualcomm)] [late]

JCTVC-N0235 Non-RCE3: base color merging for MBCIM [J. Xu, A. Tabatabai (Sony)]

(Reviewed in Track A Tue. 30th (DF).)

Merging of base color in MBCIM in RCE3 is proposed to improve coding efficiency. Experimental results state that compared to MBCIM, there are -1.5% for SC RGB sequences and -0.5% for SC YUV 444 sequences in lossless coding on average; -0.3% for class F sequences at all tiers, -1.6% for SC RGB sequences at all tiers and -0.9% for SC YUV 444 sequences at all tiers in lossy coding on average.

Since N0287 provides no palette adaptation mechanism, this contribution proposes implementing merging flags similar to those of SAO, where a palette of a CU may be inherited from a neighbouring block to the left or above.  A new syntax element is required to signal the merge and direction.

Details of the merge estimation process were not described, but the presenter believes that the decision is based on an estimation of similarity between the two derived palettes.

No visual tests have been performed so far and concern was expressed that there may be observable subjective losses due to the palette merging.  Reference was made to JCTVC-N0169 which permits the palette to be partially predicted.

JCTVC-N0323 Non-RCE3: Cross-check of JCTVC-N0235 base color merging for MBCIM [L.Guo (Qualcomm)] [late]

JCTVC-N0249 Non-RCE3: Modified Palette Mode for Screen Content Coding [L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

(Reviewed in Track A Tue. 30th (DF).)

This contribution describes a modified palette-based coding method. The modifications compared to the palette-based coding described in RCE3 Test 3.1 are: (1) “pixel mode” is removed and all the pixel values are converted to palette indices for encoding; (2) the possible error (from pixel values to palette indices) is encoded using the HEVC residue coding method; and (3) the palette index and the “run” are shared by all the 3 color components.

The proponent prefers the N0249 method to their earlier proposed N0247 that was tested in RCE3.

In this proposal, a single palette is shared between all components, where each entry consists of a triplet of sample values, one for each component.  A single map is transmitted rather than one per component.  For non-4:4:4 chroma formats, some palette entries contain null chroma values if that triplet does not correspond to a luma and chroma tuple owing to the subsampling.

The copy-above mode is not available for the first row of samples in the CU, and the copying is performed prior to adding the residual.  No per-pixel prediction is performed.

Lossless: AI: SC YUV 27%, RGB 38% @ 134% (an increase from 113% of N0247)

Delta to N0247, SC YUV 16pp, 13pp, losses in class F (1.2pp).

Lossy, AI YUV 444: 21%, RGB 41%, class F 0.6% (AI-main) @123%

RA: 20%, 35%, 0.3%. (ClassB / RExt seeing losses, probably due to the signalling overhead)

LD: 15%, 31%, 0.1% @ 113%

Results are improved over N0247, however the encoder complexity is also increased.

JCTVC-N0332 Non-RCE3: Cross-check of N0249 (Modified Palette Mode for Screen Content Coding) [J. Min, S. Lee (Samsung)] [late]

JCTVC-N0254 Non-RCE3: Pipeline Friendly Intra Motion Compensation [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

(Reviewed in Track A Tue. 30th (DF).)

This document addresses pipelining issues for the intra motion compensation method. It is asserted that restricting the search region to the left CTU or to the rightmost 4 columns of the left CTU makes intra motion compensation more suitable for pipelining. It is proposed to remove the interpolation filter in intra motion compensation to reportedly improve memory bandwidth and pipelining. Finally, intra vectors are binarized with an exponential-Golomb code to allow the signalling of any position within the search area. The BD-rate of each of these modifications is reported. The average BD-rate impact of removing the intra interpolation filter is 0.1% for lossless and 0.3% for lossy, respectively.

Three tests are performed:

- Interpolation filter (on/off)
Lossless: no impact
Non-lossless: 0.3% Intra, 0.2% RA

- Exp-Golomb
Lossless: AI, 0.3 -- 1.4 % gain
Non-lossless: 0.6--2% Intra, 0.4--1.5 RA, 0.2--1.5%LD

- Search area restrictions(2CTU)
Final results against anchor
Lossless: 3%--24%
Lossy: 9.5%--28% (Intra)

With search restrictions:

- (Search area restriction(1CTU))
~0.5--2pp loss
~1.5--3pp loss

- (Search area restriction(4col))
1.8--9pp loss
~5--9pp loss

It is observed that there isn't a significant runtime change between using a four column search and 2CTU search.

The contributor proposes to disable the interpolation filter, to use exp-golomb coding, and to limit the search range to either a maximum of the left 4 columns or the left CTU boundary.

Are there any visual effects from the loss of the interpolation filter on SCC content?  The contributor points out that various filters have been shown to be detrimental to this type of content.

JCTVC-N0360 Cross-check of JCTVC-N0254 Table 10 and 13 on pipeline friendly Intra motion compensation [J. Xu (Sony)] [late]

JCTVC-N0376 Non-RCE3:Cross-check of Table 11 and 14 from N0254 ( Pipeline Friendly Intra Motion Compensation) [J. Min, S. Lee (Samsung)] [late]

JCTVC-N0377 Non-RCE3: Cross-check of N0254 on Table11 and Table14 (Pipeline Friendly Intra Motion Compensation) [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0256 Non-RCE3: 2-D MV Supported Intra Motion Compensation [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

(Reviewed in Track A Tue. 30th (DF).)

The intra motion compensation method in HEVC range extension core experiment 3 uses 1-D motion vectors binarized with a fixed length code. In this proposal, the intra motion compensation method is extended to support 2-D motion vectors. It is asserted that 2-D motion vectors provide better alignment with the HEVC inter method. The 2-D motion vectors are binarized with an exponential-Golomb code. In a tested variant, motion vectors are predicted prior to coding. Results are also reported for the combination of 2-D intra motion compensation method with the method of JCTVC-N0254, which uses a reduced prediction area and no interpolation filter.

Based upon N0254, ie, no interpolation filter, and using the range restriction

- 2-D MVs

- Motion vectors

- An Exponential Golomb code (different from N0254)

- MV coding:

  - MV predictor, non-zero difference, + expg + sign.  The MV predictor introduces a line buffer.

  - No predictor + coding as used for MVD in HEVC

Also investigates encoder options:

- Fast encoder for 2D search, including early skips and vector refinement, but exact details are not known.

- Using 1D search but 2D signalling

2-D MV with 1CTU against N0254:

- Lossless: 2.1 -- 5.7 % (Intra) with relative 222% runtime.

- Non-lossless: 3.5--7.1% (Intra) @ 177%

A fast search can significantly reduce the runtimes, e.g. 130% in All Intra lossless case, without loss of performance (at most 0.1--0.2%). (Similar for lossy).

In Intra, the final runtime increase is similar to other methods.

2D signalling with 1D vectors works has negligible loss against the 1D case.

MV prediction provides 0.3--0.8% gains in lossless all intra, 1.2--2.7% for non-lossless.

NB, some sequences such as waveform are perfectly exploited by the 1D system, whereas other sequences provide higher gains.

Search is only performed on luma without chroma.

Are there additional gains using the two left CUs with all of these enhancements?  This was not investigated, although there will be a buffering penalty.

Deblocking: do different discontinuities occur compared to normal inter prediction. Has this been investigated?

JCTVC-N0340 Non-RCE3: Cross-check of N0256 (Intra Motion Compensation with 2-D MVs) [J. Min, E. Alshina (Samsung)] [late]

- JCTVC-N0382 Non-RCE3: Cross-check of JCTVC-N0256 (Intra MC with 2D MVs) on Table 17 and Table 18 [X. Wang, Z. Ma (Huawei)]

JCTVC-N0285 Non-RCE3: Intra motion compensation for screen contents [J. Min, M. W Park, S. Lee, C. Kim (Samsung)]

(Reviewed in Track A Tue. 30th (DF).)

This proposal presents a motion compensation method for intra coding of screen content. Exhaustive search for finding matched patterns in the decoded area leads to significant increase in the encoding complexity. The proposed method uses only 4 positions in the decoded area for motion compensation candidates. The proposed method provides gains of -0.4%, 0.0%,-10.2%, -7.6% and 0.0% for classF, classB, SC RGB ,SC YUV and Range Extension classes in AI lossless test conditions.

Could this be applied as an encoder restriction to one of the earlier methods?  This method effectively does so in combination with a coding efficiency proposal to reduce the signalling overhead of short vectors.

Do pcb-layout and cad-waveform distort the results?  There may be some effect, but not too large.

The vertical search range is based on method 1 of RCE3.3, ie, will access the top CTU.

Proponents like limiting the vector range to access data in the current CTU (with four columns).

JCTVC-N0341 Non-RCE3: Crosscheck for JCTVC-N0285 Intra motion compensation for screen contents [C. Pang (Qualcomm)] [late]

JCTVC-N0231 AHG 8: Intra mode coding for screen contents [J. Min, S. Lee, C. Kim (Samsung)]

(Reviewed in Track A Tue. 30th (DF).)

This contribution presents an intra mode coding method for screen contents. 3 MPMs (Most Probable Mode) are set without referring intra modes of neighbouring prediction units. The proposed method provides performance gains of -0.7% for both screen content sequences with RGB and YUV formats respectively for AI lossless test conditions. For AI lossy test conditions, gains of -1.0 and -0.2 % are observed for screen content sequences with RGB and YUV formats.

Uses fixed predictors for the most probable mode in intra, leading to gains.  However there are losses associated with class B sequences.

The contributor suggests that this may be an encoder side simplification (as this adds some cost to the decoder) and that a separate screen content profile might wish to use such a method.

Further study was suggested.

JCTVC-N0322 AHG 8 Cross-check for JCTVC-N0231: Intra mode coding for screen contents [M. Naccari, M. Mrak (BBC)] [late]

Discussion

(Discussion in Track A Tue. 30th (DF).)

To aid the decision process, before examining the merits of intra picture block copying and palette coding, consideration was given as to what may be desirable individual refinements to both methods.

Discussion (intra picture block copying): Hypothetical action to take: Take 2D-MV, without prediction (to remove line buffer), chroma interpolation disabled (needs visual check), search range restriction set to 1CTU (but should not be limited at this time).

Further study is required on how to reduce the search range. Is an N-step search possible?

It was also remarked that decoder statistics, such as a histogram of mode use and vector lengths may also be useful to analyse the mode.  Similarly, a visual representation may provide some insight into the mode's behaviour and utility for particular types of content.

Further discussed on Thu 1st (DF).

Text for the intra picture block copying scheme had not been reviewed.

If the intra MC scheme is adopted, does it mean we're not adopting the palette scheme? Software was suggested to be provided. It was remarked that further study is needed. It was remarked that both this and the palette scheme have significant complexity and the gains are not additive. It was remarked that we may not have sufficient knowledge of the principles of operation of this. A participant said that intra MC has a significant complexity increase in terms of memory requirements and need to have a motion prediction process available in intra operation, and that the palette scheme has less of a memory impact – from the decoder side both are OK, palette side is rather unknown esp. for encoder since the technique is new and unknown – although the gain is significant and may justify the benefit. It seems likely that we would adopt something in this area. The intra MC proponent indicated that they had privately done some informal subjective viewing and found the technique beneficial. Some concern was expressed about encoder complexity for the intra MC scheme.

Decision: Adopt (as described above as "Hypothetical action to take", with constraint as noted below).

Further discussed Fri. (GJS): Text had been uploaded in a revision.

It was commented that reference region availability needed to be assured by an appropriate constraint. Agreed.

Further study on interaction with constrained intra prediction was encouraged.

Regarding palette schemes:

|  | N0247 (RCE3.1) | N0249 | N0287 (RCE3.2) |
| --- | --- | --- | --- |
| Palette size | per-component fixed (4*1-value) | single (shared) fixed (4*3-tuples) | per-component variable (2–16) |
| Index planes | 3 | 1 | 3 |
| Bypasses residual coding | Yes | No | Yes |
| Intra plane prediction | No | No | Yes (Serial) |
| Palette prediction | Inherit left (replacement) | Inherit left (replacement) | None |
| AI-lossy (BD-rate %) (F / SCYUV / SCRGB) | 0.4–1.6 / 3–9 / 14–24 | 0.5–1 / 21–27 / 41 | 15–10 / 26 / 41–44 |
| Runtime | 110% | 123% | 142 |
| AI-lossless (%) (F / SCYUV / SCRGB) | 1 / 11 / 25 | 0.1 / 27 / 38 | 0.6 / 12 / 30 |
| Runtime | 113% | 134% | 112 |

Modifications to N0287, (Results are presented using N0287 as a reference).

|  | N0169 | N0235 |
| --- | --- | --- |
| Palette prediction | Predicted using LRUcache | Inherit above¦left (replacement) |
| AI-lossy (BD-rate %) (F / SCYUV / SCRGB) Reference: N0287 | 1.3–1.5 / 2.2–2.8 / 2.6–3.4 | 0.4 / 0.9 / 1.5–1.8 |
| Runtime | 100% |  |
| AI-lossless (%) (F / SCYUV / SCRGB) Reference: N0287 | 0.1 / 2.7 / 2.7 | 0 / 0.6 / 1.5 |
| Runtime | 100% |  |

There seem to be two distinct operating points (high complexity with higher gains, and lower complexity with lower gains).  Is there some trade-off possible in the operating points?

Comparing to intra block copying situation, the hypothetical decision makes the base proposal more flexible, without loss of performance to simpler systems. Is such flexibility possible with these systems to allow encoder trade-offs?

What should be studied in the next meeting cycle?

Merging the palette inheritance modifications into the schemes. Both N0235 and N0169 could be applied to all of the palette techniques. Possibly a CE activity to investigate the merits of this?

Comments regarding the time line were expressed.

AHG to investigate the performance and operating points available for each palette method.

Combination: An expert reported that the gains of one of the palette coding methods plus the intra block copying method is not additive, and that the gains of the combination may be around 5%.

### Transforms and transform coefficient coding

JCTVC-N0138 AHG5: Square transform deblocking for 4:2:2 [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

(Reviewed Thu 1st (DF).)

At the 12th JCT-VC meeting, square transforms were adopted for the rectangular blocks present in the chroma channels when the 4:2:2 chroma format is used.  This adoption results in boundaries between the pairs of square transforms that are not deblocked.  It is asserted that having transform boundaries in the design that are not deblocked is inconsistent with the 4:2:0 design and could lead to deblocking artefacts.  This contribution introduces deblocking to the boundaries between the square transforms in chroma when the 4:2:2 chroma format is in use.

There is a very minor impact in BD-rate terms.

From a selection of pictures presented, there are some differences, although the scale of the change is possibly more related to mode decision rather than deblocking.

Proposes introducing a deblocking edge between the pair of square leaf chroma CTUs in the 4:2:2 system.

Cross-checker confirmed the results, code and text. This could be considered as a bug fix or alignment of the design.  No visual comparison was performed.

A comment was made that this may be no need to change this now, since it will be very difficult to see any visual effect.  If content is found that demonstrates an issue, this could be reconsidered later.

Look for effects on intra, as this won't have any mode-decision changes.

No action.

- JCTVC-N0381 Cross-check for JCTVC-N0138 AHG5: Square transform deblocking for 4:2:2 [M. Naccari, M. Mrak (BBC)]

No action.

JCTVC-N0192 AHG 5: 32x32 Scaling List Derivation for Chroma [K. Sharman, N. Saunders, J. Gamei (Sony)]

(Review Thu 1st (DF).)

The document details the method currently used in HEVC Range Extensions to derive 32x32 scaling lists for chroma. An alternative derivation is presented using source scaling lists that are expected to be more closely correlated.

The current draft text does not include a provision for 32x32 chroma scaling lists, however the software model currently uses the 32x32 luma list for chroma. This may be sub-optimal. Proposes using either a process to convert from the 16x16 (8x8) chroma matrix, or to permit sending a 32x32 matrix in the SPS/PPS extension data.

No opinion expressed on the two approaches.

If RExt is making use of the PPS extension, is there any harm in doing so?

Decision: Adopt derivation process (16x16).

### Intra prediction

JCTVC-N0143 On Mode Dependent Intra Smoothing for Range Extension [G. Laroche, C. Gisquet, T. Poirier (Canon)]

(Reviewed Thu 1st (DF).)

In 4:2:0 we currently MDIS on luma but not chroma. The current draft for 4:4:4 applies the same processing to chroma as done on luma. This proposes changing the MDIS for both luma and chroma, applying more smoothing.

This contribution proposes a modification of the Mode Dependant Intra Smoothing (MDIS) of HEVC for Range Extension. The modification consists in simplifying the MDIS usage by using the same minimum condition for TU sizes greater than 4x4 when the input sequence color format is 4:4:4. In average for RExt Intra configurations, it is reported that the proposed modification gives -0.3% (Y), -0.1% (U), -0.2% (V) BDR gain.

Proposes making MDIS less mode-dependent, by (except for 4x4 which keeps the current behaviour -- ie, not used) enabling intra smoothing for 8x8, 16x16, 32x32 for all intra modes except DC, Horizontal and Vertical.

NB, this isn't a simplification for a decoder (as it will have to support both systems).

Comment: this doesn't increase the worst case complexity and may be a good idea.

This would apply only to 4:4:4.

This does not modify the post prediction filtering.

It was pointed out that real systems may not have an actual table.

This was generally supported for adoption in this discussion.

Further discussion Thu pm.

No test results were provided for screen content. It was remarked that increasing the filtering may be harmful to screen content.

The need to change luma processing was questioned. It was suggested that it is desirable to not need to change processing elements relative to version 1.

The proponent said that there is a subjective benefit, but this had not been evaluated formally or confirmed. It was remarked that there would need to be a significant visual benefit to justify a change.

For further study.

JCTVC-N0349 Cross-check of Mode Dependent Intra Smoothing in JCTVC-N0143 [W.-S. Kim (Qualcomm)] [late]

JCTVC-N0183 Non-RCE 2: Enhanced angular intra prediction for screen content coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

Handled in non-RCE2 BoG.

JCTVC-N0358 Cross-check for JCTVC-N0183 Non-RCE2: Enhanced angular intra prediction for screen content coding	[M. Naccari, M. Mrak (BBC)] [late]

### High bit depth

JCTVC-N0142 AHG18: On 16-bits support for Range Extensions [E. François, J. Taquet (Canon)]

(Reviewed Wed. p.m. Track A (GJS).)

This contribution relates to Intra-coding of high bit-depth (16-bits) monochrome content in HEVC RExt. The goal is to address the sources of errors due to internal accuracy limitations without explicitly extending the bit-depth of the internal operations in the HEVC design. The proposed solution re-uses the HEVC 4:4:4 or 4:2:2 8-bit design to code 16-bit 4:0:0 content without extending the bit-depth of the internal register for the transform, the quantization and the entropy coding processes. The input 16-bit monochrome picture is first converted into a color picture of lower bit-depth (typically 8 bits per component) by splitting the MSBs and LSBs of the 16-bit samples into luma and chroma components. Then the color picture is coded, with specific adaptations related to the quantization in order to take into account the range difference between the MSBs and LSBs. The performance of the proposed concept is illustrated on preliminary results.

The interaction of quantization in the decomposition was discussed. The MSB image should be lossless for the LSBs to be useful.

It was suggested that something like this could become an SEI message, with backward compatibility for 8 bit decoder. However, the proponent indicated that some modification of quantization may be needed.

Further study was encouraged.

JCTVC-N0188 AHG 5 and 18: Internal Precision for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

(Reviewed Wed. a.m. Track A (GJS).)

This is an analysis contribution, with suggestions of how to modify test conditions and reference software.

The document discusses the current capabilities of HEVC Range Extensions when operating at low (including negative) QPs, and proposes that such low QPs will be needed if the codec is to truly support high bit depths, as required by the mandate of the Range Extensions amendment. Possible sources of errors that may be caused by internal accuracy limitations currently present in HEVC are explored. It is claimed that some changes to those accuracies can mitigate the errors and thereby extend the operating range of HEVC.

Previous contribution N0178.

Suggests a target MSE of 1 (48 dB @ 8 b, 60 dB @ 10 b, 72 dB @ 12 b, 84 dB @ 14 b, 96 dB @ 16 b).

Tested the HM with RDOQ off, transform skip off.

Parts of RDO operate in integer precision, which causes problems.

It was remarked that the forward transform is not the precise inverse of the inverse transform, and a proper inverse should be tested. This was not tested by the contributor.

The contributor suggested that the current design may be adequate for 12 b video, but is unlikely to be adequate beyond 12 b.

Proposes transform dynamic range of bit depth + 7 bits (signed) beyond 12 b video.

It was also remarked that perhaps the largest transform block sizes do not necessarily need to support full-dynamic-range error signal inputs.

Decision (BF): At this point, for up to 12 b depth, no change. For MC, 12 b has 4 b downshift. Set downshift to 4b for > 12b. For transform, we have a coeff level range limit and a clip after coeff reconstruction, and a clip after the 1st stage inverse transform. These are 16 b (signed). Set to bit depth + 7 (signed) for profiles supporting bit depths beyond 12 bits. Have a SPS extension flag to control which rule applies (extended range/precision or not). One flag controls both at once. We have a downshift after the first stage inverse transform of 7 bit, which we won't change now.

JCTVC-N0369 AHG 5 and 18: Cross-check of Internal Precision for High Bit Depths (JCTVC-N0188) by Sony [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-N0189 AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

(Reviewed Wed. a.m. Track A (GJS).)

Four systems are presented that are described as requiring only small changes to the current entropy coding scheme, while reportedly yielding a substantial BD-rate saving at 12-bit and higher.

Out of the four systems, the proposed method is Golomb-Rice Parameter modification with auto-adaptation, which is indicated to give efficiency improvements of -30.3% at 96 dB (16-bit operating point), -21.5% at 84dB (14-bit operating point), -8.3% at 72 dB (12-bit operating point), -0.7% at 60 dB (10-bit operating point) and no significant change at 48 dB (8-bit operating point) relative to the proposed higher internal accuracy system described in JCTVC-N0188. Improvements are not observed for positive QPs, however, this is not the intended operating range for this high bit rate/high bit depth tool.

In addition, the system has been trialled under AHG8 lossless test conditions, with reported intra BD-rate changes of  1.6%, 0.0%, -16.4%, -8.3%, -2.4% for Classes F, B, ScreenContent RGB, ScreenContent 4:4:4 and a limited subset of the Range Extensions sequences respectively; inter results are similar.

Proposes transform dynamic range of bit depth + 6 bits.

Proposes a Rice parameter change and a number of LSBs that are always coded in bypass mode, with adaptive computation of that number of LSBs in the decoder.

It was commented that this produces a different set of binarization codewords that need to be handled in CABAC.

It was commented that it may be desirable to keep the existing binarization set and instead change the rules about which binarization is applied.

Only changes of the Rice parameter were considered rather than other changes. It was commented that the statistics underlying the scheme may not fit well anymore with this method.

It was remarked that N0181 has some relationship to what is proposed here, focuses on screen content (not high bit depth).

It was remarked that whatever changes are being considered for 8 b SCC should be rationalized relative to changes for increased bit depth.

Test N0181 (adjusted as necessary for bit depth) and this in CE.

JCTVC-N0338 Cross-check report for JCTVC-N0189: AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths [S.-H. Kim, A. Segall (Sharp)] [late]

JCTVC-N0380 Cross-check for JCTVC-N0189: AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths [M. Naccari, M. Mrak (BBC)]

JCTVC-N0190 AHG 5 and 18: Entropy Coding Throughput for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

(Reviewed Wed. a.m. Track A (GJS).)

A method is presented for aligning the CABAC process prior to coding bypass data, reportedly allowing easy, simultaneous decoding of multiple bypass bins; the number of CABAC encoded bins is described as being bounded to 25 bins per coefficient group.

The cost of alignment has been reduced through refinements presented in Section 3.1 and further reduced at low operating points via conditional application. The losses due to the alignment are reported to be 0.4%-0.5% for the intra Range Extensions test conditions. Losses of 0.2% and less are reported at the more negative QPs at which this throughput tool is targeted, described as an unnoticeable loss when used in conjunction with the entropy coding compression efficiency tool described in JCTVC-N0189.

Focuses on throughput. Software will be uploaded in a revision of the contribution.

Request AHG to study – likely to adopt at next meeting if no better approach is identified.

JCTVC-N0336 Cross check AHG 5 and 18: Entropy Coding Throughput for High Bit Depths [Wei Pu, Woo-Shik Kim] [late]

JCTVC-N0201 SAO extension for higher bit-depth coding [Alexis Tourapis (Apple)]

(Reviewed Wed. p.m. Track A (GJS).)

The sample adaptive offset (SAO) process was one of the new coding tools introduced in the HEVC video coding standard because of its perceived improvements in objective as well as subjective quality. The Edge Offset mode of SAO in particular, tries to establish distortion that may have been introduced during encoding around edge boundaries, and then attempts to correct this distortion through the addition of a “gradient-dependent” offset. However, it is suggested this process may be significantly impacted when resolution and bit-depth are increased, due to the likely increase of noise in the underlying material. Noisy neighbouring samples may be misclassified as edges and a less than optimal offset may be applied on all the samples as classified. This contribution proposes the use of a quantization step during the edge classification process that it is asserted can provide further flexibility and enhance the performance of the SAO process in the presence of noise.

The contribution suggests sending a threshold parameter at the slice level, e.g. as a bit shift value.

It was suggested that a fixed-value parameter depending on the bit depth may be preferable.

The contributor said that our test sequences may not have as much noise as some video encountered in applications.

The contributor suggested that the subjective benefit might be better than the objective benefit, although this had not been checked.

The measured benefit was very small (basically no measurable benefit).

It was remarked that SAO typically works best in P slices.

Related to N0246. Further study is encouraged.

JCTVC-N0253 Cross-check of JCTVC-N0201 on SAO extension for higher bit-depth coding [J. Xu, A. Tabatabai (Sony)] [late]

JCTVC-N0246 AHG5: Modified SAO for range extensions [S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

(Reviewed Wed. p.m. Track A (GJS).)

This proposal attempts to improve the coding efficiency of the existing SAO coding tool for HEVC range extensions. The proposed method includes a new set of syntax elements such that the SAO coding tool can be effectively adapted to different sample bit depths, coding bitrates and sampling formats while making no significant modifications to the existing SAO method in the current HEVC. The proposed method is backward compatible to the current HEVC SAO tool when the new syntax elements are set to the default values. The experimental results on the RExt test sequences under the common test conditions reportedly show overall luma BD-rate savings 0.2%, 0.4%, 0.7% and chroma BD rate savings 0.4%, 1.2%, 1.2% for AI Main-tier, RA Main-tier, and LB Main-tier, respectively.

Part of this is similar in concept to N0201.

A second element is to extend the range of maximum offset values by using a shift parameter sent at the slice level.

A third element of the proposal is to select a binarization mode for the offset value, as the offset values can tend to be large.

The encoder operated by trying different values.

Further study is encouraged.

JCTVC-N0331 AHG 5: Cross-check of N0246 (Modified SAO for range extensions) [J. Min, E. Alshina (Samsung)] [late]

JCTVC-N0275 AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than equal to 14 [S. H. Kim, K. Misra, A. Segall (Sharp)]

(Reviewed Wed. p.m. Track A (GJS).)

In the current transform-skip design the least significant bits of residual information are discarded when the source bit depth is greater than or equal to 14. This unnecessarily restricts the transform-skip mode to operating at high fidelity to the original signal. This contribution proposes a modified scaling factor for transform-skip blocks to support high fidelity coding of residuals for source bit depths 14 and higher. The scaling factor is modified for only transform-skip blocks and no change is made for transform blocks.

Evaluating the proposed approach with HM-10.1+RExt3.1 anchor for 14-bit source data at QP=-36, -32, -28, -24 reportedly results in average luma BD rate of -8.5% and -7.1% for all-intra and random-access configurations respectively.

Decision (BF): When in high-precision operating mode (see flag above), apply Min( 7, dbShift ) instead of just 7.

JCTVC-N0379 Cross-check for JCTVC-N0275: AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14 [K. Sharman, N. Saunders, J. Gamei (Sony)]

### Lossless and screen content coding related contributions

JCTVC-N0115 On RGB to YCbCr conversion for screen contents [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)] [late]

(Reviewed Wed. p.m. Track A (GJS).)

Information document.

This contribution evaluates PSNR of color converted RGB contents in order to investigate the effect of the color space conversion from RGB to YCbCr and back to RGB. From the result, PSNR of screen contents is lower than that of RExt sequences on average, especially B and R signals. Due to this information loss, the coding performance of YCbCr coding does not reach that of direct RGB coding especially for screen contents at high bit rate.

The screen content seemed to have more conversion error, perhaps because it uses more saturated colours.

A relationship with the prior contribution JVT-I017 was noted.

### Other

See also N0145 regarding chroma format.

JCTVC-N0141 AHG5: On chroma QP for HEVC Rext [E. François, C. Gisquet, G. Laroche, P. Onno (Canon)]

(Reviewed Thu 1st a.m. (GJS).)

This contribution relates to the chroma QP in HEVC RExt. In the current HEVC RExt draft, one chroma QP table, linking the chroma QP (QPC) to the luma QP (QPY), is specified for each one of the three color formats 4:2:0, 4:2:2 and 4:4:4. This contribution contains three proposals.

- Proposal 1 consists in adding a syntax element in the PPS or SPS in case of 4:2:2 or 4:4:4 content, indicating which table is selected. This reportedly gives more flexibility to control the chroma QP depending on the content.

- Proposal 2 consists in removing the 4:2:2 table. This change reportedly simplifies the design, since only the 4:2:0 table, already present in the HEVC V1 specification, and the 4:4:4 table, based on a straightforward QPC derivation, are kept.

- Proposal 3 combines proposals 1 and 2 by removing the 4:2:2 table and adding a syntax element to select the table among the 2 available tables in case of 4:2:2 or 4:4:4 content.

The coding efficiency impact of removing the 4:2:2 table is reportedly small, mainly resulting in a slightly different balance between the luma and chroma quality. The proposed added syntax element is reported to provide a simple mechanism to easily and flexibly control this repartition.

It was commented that it is not clear that the BD analysis in this contribution is the best way to measure the performance.

It was commented that 4:2:2 is unlikely to be used much with low fidelity encoding, so the high-QP behaviour of the 4:2:2 chroma relationship is not so important. However, another participant indicated that this is sometimes used as a "preview" made.

Also we might expect more professional encoders to not simply use default QP relationship.

The same thing had been proposed at the previous meeting.

Decision (Simpl.): Replace 4:2:2 QPc table with 4:4:4 table.

(No action on Prop. 1.)

JCTVC-N0295 AhG5: Cross-check of Chroma QP for HEVC RExt in JCTVC-N0141 [W.-S. Kim (Qualcomm)] [late]

JCTVC-N0116 AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)] [late]

(Reviewed Thu 1st a.m. (GJS).)

In this contribution, as the base coding architecture of RGB coding to be studied in RExt, a RGB video coding scheme using architecture of HEVC multi-view extensions is proposed. The proposed scheme applies multi-view video coding architecture to encode each of RGB color planes as a view source of multi-view video. Coding performance of the proposed scheme has been evaluated using MV-HEVC software under RExt common test configurations. It is reportedly confirmed that the average BD-rate gains of the proposed scheme relative to RExt3.0 are 9%-25% and 11%-27% for described PSNR-GBR and PSNR-GBRm based measurements, respectively. With regard to screen contents, the result demonstrates that the proposed scheme achieves up to 33% average coding gain compared with RExt3.0. It is also reported that further coding improvement of the proposed scheme relative to RExt3.0 is obtained under the condition of applying QP offset for B and R plane.

It was remarked that this is an interesting scheme. It was also remarked that weighted prediction could be used in this context for improved performance (not tested by the contributor).

Currently we don't plan to allow separate colour plane mode in currently-planned RExt profiles.

It was remarked that, in some sense, it might be possible to use this already (within a profile that supports 4:0:0 pictures) as a way to interpret 4:0:0 coded pictures, if the decoder is aware of what is happening.

Further study is encouraged.

JCTVC-N0261 AhG5: Memory Bandwidth Reduction for HEVC Rext [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

(Reviewed Thu 1st a.m. (GJS).)

HEVC Range Extensions 4:4:4 potential applications include consumer applications, so the complexity increase due to the 4:4:4 processing has to be taken into account. The scope of this contribution is the memory bandwidth increase for 4:4:4 motion estimation and compensation, which is reportedly around 37% more than the 4:2:0 case for 4×2 memory blocks. Specifically, in this contribution, a restriction of bi-directional prediction for 8×8 chroma components is studied, while the luma component still can have bi-directional prediction. This modification reportedly increases 4:4:4 bandwidth by 8%, as opposed to the current 37% with respect to 4:2:0. The impact on coding performance is reportedly 0.2% and 0.4% luma BD-rate loss for RA and LB, respectively, in YUV 4:4:4, and about 0.8% for chroma. In addition, restriction of motion interpolation is also studied. When the vertical motion interpolation is disallowed for chroma 8×8 PU, the bandwidth is increased by 13% with respect to 4:2:0 with the coding efficiency loss of 0.1% and 0.2% for RA and LB, respectively, in YUV 4:4:4.

Simply disallowing bipred 8×8 resulted in 1.0% and 0.6% luma BD rate loss, and about the same impact for chroma.

A prior contribution M0298 had been submitted to the previous meeting on the topic.

Test results and a modified variant for weighted prediction was also described.

It was remarked that we should try to avoid using different processing for the different components, and that the asserted memory bandwidth reduction would not be experienced when using interleaved YUV storage arrays.

The memory writing bandwidth was not measured here – only read access. Including the write accesses would substantially reduce the benefit as measured relative to the larger total. A participant remarked that including the write accesses was requested at the previous meeting.

It was suggested to simply keep in mind the potential to use the bipred block size constraint approach when finalizing RExt, but not to act now since the 4:4:4 design is not yet final.

JCTVC-N0310 Cross check of Memory Bandwidth Reduction for HEVC RExt (JCTVC-N0261) [G. Laroche (Canon)] [late]

JCTVC-N0263 AhG5: Deblocking Filter in 4:4:4 Chroma Format [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

(Reviewed Thu 1st (DF).)

In this contribution, three methods are proposed to improve the performance of deblocking filter of chroma components in 4:4:4 format. In the first method, the luma deblocking filter is applied to the chroma components. In the second method, the luma deblocking filter is applied to the chroma components without the strong filter. In the third method, the chroma filter is used when the boundary strength is larger than 0. Experimental results report visual quality improvements as well as coding efficiency improvements of 0.2/2.2/2.2 and 0.4/2.2/1.8 (Y/U/V BD-rate (%)) for RA and LB Main-tier in Test 1, 0.2/1.8/1.9 and 0.4/1.9/1.7 in Test 2, respectively.

Proponent suggests a CE for subjective testing.

Some subjective testing was performed using method1 (luma filter), indicating a preference for this filtering.

Comment: Method1 was the most complicated method, is such effort needed for chroma?

Proponent suggests that method two may be a better compromise.

It was suggested that some subjective viewing could be performed during the meeting.

Would this make similar changes to 4:2:2?

How does this interact with 4:2:0 when performed using the RExt profiles?  Proponent suggests that the profile should behave in two different ways according to the chroma format.

Further discussed Fri 2nd after performing some subjective viewing to decide if any investigation should happen during the next meeting cycle – e.g., AHG/CE.

It seemed that some subjective benefit was observed, although there were difficulties with the available equipment.

It was remarked that 4:4:4 has problems with chroma deblocking, since chroma is treated with a weak filter that is controlled by luma. Further study in AHG5 of deblocking issues was requested.

JCTVC-N0320 Crosscheck of JCTVC-N0263 on deblocking filter in 4:4:4 chroma format [D.-K. Kwon (TI)] [late]

JCTVC-N0292 RExt: Fidelity adaptive coding mode [D. Flynn, N. Nguyen, D. He (RIM)]

(Reviewed Fri 2nd (GJS).)

The current HEVC range extensions design for 4:4:4 chroma formats, reportedly exhibit different rate-distortion behaviour for upconverted 4:2:0 sequences compared to the 4:2:0 design. This contribution proposes a mode which sends an additional chroma QP offset in the PPS and applies this QP offset on a CU basis. When this offset is applied in a CU, the TU size for chroma within that CU is restricted to a minimum size of 8x8 (disabling the Intra-NxN split mode for chroma in 8x8 CUs).

A (non-RDO based naïve mode estimator) that examines source picture activity is provided to demonstrate control of the mode in both native 4:4:4 environments and for upconverted 4:2:0 sources.

The design is asserted to be capable of preserving the current 4:4:4 performance on native 4:4:4 sequences, while also being capable of providing the effect of a fixed QP offset and simulating the 4:2:0 behaviour on upconverted 4:2:0 sequences. It is suggested that the in-loop nature of the design provides a desirable intermediate operating point between these two extremes that is dependent upon the nature of the content.

In the tested scheme the idea is that chroma QP would be set bigger in regions of low-frequency chrome (which might be upsampled 4:2:0 regions).

There was significant interest in the idea.

It was remarked that the complexity impact seems minimal.

How to test for true benefit?

Some possible variants:

- Does the offset need to be signed? (No.)

- Separate offsets for each chroma component?

- If separate for each chroma component, control by one flag or two?

- At what level of syntax would it be best to put a switch?

- The coupling with chroma block size constraint

Further study was encouraged – add to mandate of RExt AHG (AHG5).

## SHVC

### General

JCTVC-N0150 AhG17: complexity analysis of SHM2.0 [E. Alshina, A. Alshin (Samsung)]

(Reviewed Fri. 26th a.m. plenary)

This contribution contains a performance and complexity analysis of SHM2.0 (RefIdx framework) compare to HEVC single layer coding. A complexity assessment methodology developed by AhG-17 was used. Complexity was evaluated assuming a two-layer scalable system. Two different implementations were tested: picture-based and PU based interlayer processing were studied. It was reported that memory access for SHM2.0 in the worst case is 200% for PU-based inter-layer processing and 218% for picture-based inter-layer processing, respectively, compared to single layer HEVC. On average, memory access compared to single layer HEVC is 106–111% for spatial scalability tests and 97–98% for SNR scalability tests, respectively, assuming PU-based inter-layer processing and 152–163% assuming picture-based inter-layer processing.

Memory access assessment results are reported for scalable 2 layers SHM2.0 system relatively to single layer HEVC decoder, with results reported as follows:

- In the worst case memory usage (WCMU) by scalable codec is characterized as follows:

  - The WCMU is twice higher compare to single layer decoder (assuming PU-based inter-layer processing) – which is equivalent to two-layer simulcast decoding complexity;

  - The WCMU is 2.2 times higher compare to single layer decoder (assuming Picture-based inter-layer processing) – which noticeably exceeds two-layer simulcast decoding complexity.

- Actual memory access estimated using SHM2.0 bit-streams was as follows:

  - About 3% higher compare to single layer decoder if PU-based inter-layer processing is implemented;

  - About 50% higher compare to single layer decoder if picture-based inter-layer processing is implemented.

Discussion included the following:

- Two different implementations: Picture based, PU based. Picture based is more memory consuming, whereas PU based computes BL->EL prediction on the fly.

- In case of SNR scalability, picture based could be roughly 220% worst case memory access compared to single layer. However, measurements from the actual test set show that the actual increase in memory access compared to single layer is much less (only 103% in some case).

- Question: Is number of operations different? No deep analysis was done on this, only number of multiplications and additions were counted.

- It is suggested to consider PU based implementation in software, to possibly make measurements of encoder/decoder run time consistent.

The contributor suggested to emphasize measurement of PU-based operation complexity, and simply keep in mind that the alternative approach of picture-based operation is available with about 50% higher memory access. This was agreed by the group.

JCTVC-N0242 Editorial improvements on SHVC Draft Text 2 [J. Chen, J. Boyce, Y. Ye, M. Hannuksela]

(Reviewed Fri. 26th a.m. plenary)

This text contains some restructuring of the prior draft content.

See also notes for AHG report N0011.

It was remarked that:

- This submitted text is an improvement in the structuring of the specification and should be the starting point of Draft 3 editing (PDAM in ISO/IEC process).

- Ultimately, text should be coordinated across projects and should be developed relative to official published editions by (both) the parent bodies.

- Generally, our ultimate goal is to produce complete texts rather delta documents.

- The editors have broad discretion regarding structuring of the text.

- Integration of some content into the main body of the standard rather than keeping it in annexes may be desirable (but care must be taken not to introduce errors in the specification of version 1 technical content).

JCTVC-N0375 BoG report on arbitrary scalability ratios support in SHVC [E. François]

(Outcome reviewed in plenary Mon. 29th.)

The BoG on arbitrary scalability ratios support addresses the following topics.

- Definition of test conditions (test material, downsampler) for the upcoming investigation,

- Selection of an initial set of upsampling filter coefficients as a comparison point.

The BoG met first on 27 July. The status of the current discussions as of Mon 29th was as follows:

- It is preferred to use for test condidions one (or a limited set of) spatial scalability ratio(s) enabling to test as many different phases as possible. Further analysis is being made to identify such ratio(s).

- It is preferred to use the JSVM downsampler with phase offset set to 0. Further analysis of the JSVM downsampler is being made to evaluate the amount of changes required in the software.

- It is recommended to select a set of upsampling filter coefficients which ensures the SHM behaviour unchanged for common test conditions.

The BoG met again on July 29. The resulting recommendations are listed in the document.

The BoG came up with the following recommendations:

To set-up an SCE on arbitrary spatial scalability ratio.

Test conditions:

- Spatial scalability ratios

  - Use the existing 1.5x and 2x spatial ratios

  - Use the following additional ratios, depending on sequences resolution:

    - Class A – horizontal: 2560 / 1456 – vertical: 1600 / 912

    - Class B – horizontal: 1080 / 616 – vertical: 1920 / 1096

- Downsampler

  - Use JSVM downsampler (with phase offset set to 0)

  - Generate new anchors for 1.5x, 2x and the new arbitrary ratios to be tested

  - Base layer sequences to be uploaded on the site used for HEVC, with downsampler source code and md5

    - Software developped by Qualcomm, sent to Samsung who will generate and upload the Base Layer sequences + md5 + software package

    - When – software 2 weeks after meeting ends, sequences and md5 data 3 weeks after meeting ends

- The detailed test conditions will be refined off-line for the CE (if there is a CE) definition

Initial set of upsampling filter coefficients as a comparison point:

- The filters for currently defined phase positions are not changed

- Use as the starting point for the missing positions the following filters :

  - Pick-up coefficients from N0219

  - except for luma 6/16 phases, picked-up from N0273

- These filters will be used to generate the anchor as a comparison point

The recommendations listed above were agreed upon, and open issues were further discussed in JCT plenary on Monday evening.

- Which software for filters coefficients?

  - Put in the SHM software, enabled by macro, such that the same filters can also be used e.g. for related investigations of AHG

- What will be investigated in the CE ?

  - The two proposals N0219 and N0273 related to arbitrary spatial scalability

### SCE1 related (resampling phase)

JCTVC-N0149 Non-SCE1: Results of test 1.2 on sampling offset signalling with accurate interpolation filter [E. Alshina, A. Alshin (Samsung)]

An up-sampling scheme achieving aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift, was studied in SCE1. In order to allow this feature support four additional interpolation filters for Luma and one Chroma interpolation filter would need to be added to SHVCM. For 2 from 4 additional Luma interpolation filters more accurate coefficients design was presented in Incheon meeting. This contribution presents test results. Test is identical to test 1.2, just for 2 fractional positions interpolation filter with lower deviation from identical frequency response is used. If down-sampler phase shift is known and signalled with 1/16 accuracy then there is no performance improvement from more accurate filter design. But if down-sampler phase shift is known (signalled) with 1/8 accuracy then more accurate filter design provides 0,3% Luma BD-rate gain. More accurate filter design reduces the performance gap between 1/16 and 1/8 accuracy for down-sampler phase shift signalling by half with no additional cost.

No need for presentation – was mentioned in CE report as variant of CE test

JCTVC-N0214 Non-SCE1: Dynamic range control of intermediate data in re-sampling process [J. Chen, X. Li, M. Karczewicz (Qualcomm)]

(Initial discussion Track B (JRO).)

This contribution proposes to apply the same scheme used in motion compensation interpolation filter to limit the dynamic range of intermediate data of 2D separable interpolation filtering in SHVC resampling process within 16-bit accuracy.

With current CTC (only 8 bit data), this rounding would not have an effect.

Several experts supported this idea as it is consistent with the approach of motion comp. The change is simple and could be adopted during this meeting.

One expert requests that experimental results are reported on 10-bit data.

(Further discussion Thu 1st (GS).)

Ran tests (10 bit sequences from the RExt test set, clarified conditions with the requesting expert); confirmed that concern was addressed.

Does not affect operation with 8 b base layer.

Decision: Adopt.

JCTVC-N0219 Non-SCE1: On arbitrary spatial ratio scalability in SHVC [J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung), K. Ugur (Nokia)]

This contribution proposes to support arbitrary spatial ratio between base and enhancement layers. It is asserted that the complexity impact of supporting arbitrary spatial ratio in SHVC is marginal. It is also reported that the 1/8 pixel phase accuracy shows noticed coding performance drop when comparing to 1/16 pixel phase accuracy.

Drop in performance in 1.5X scalability by only using 1/8 pel rounding around 1% (BR increase).

Profile issue: If the intention is to have only one scalable profile, would there be need to include the arbitrary ratio? Would not be useful to have a separate profile only for arbitrary scalability

One domain that may require arbitrary ratio  is the variety of mobile phones, tablets etc.

Several experts support that having this from the beginning (considering that few additional filters do not have major complexity impact) is desirable

Open questions that require further investigation:

- Which ratio of scalability factors? Lower limit >1? Upper limit?

- Same factor for horizontal/vertical?

Ratio might be signalled by picture size and scaled ref offset (i.e. cropping area).

BoG on arbitrary scalability ratio (Chair: E. Francois):

- to define test conditions (test material, downsampler) for the upcoming investigation,

- select an initial set of filter coefficients as a comparison point.

Possible sets of filter coefficients are suggested in 219 and 273.

JCTVC-N0315 Cross-check of JCTVC-N0219: Non-SCE1 On arbitrary spatial ratio scalability in SHVC [P. Lai, S. Liu, S. Lei (Mediatek)][late]

JCTVC-N0218 AhG14: On bit-depth scalability support [E. Alshina, A. Alshin (Samsung)]

This contribution gives preliminary information about the performance of a scalable system which combines spatial and bit-depth scalability. A comparison was done against single layer encoding of higher resolution and bit-depth layer. SHM2.0 results for class A in ×2 scalability tests are 11.3% (AI)/ 21.7% (RA) / 32.0%(LBD)/ 30.4%(LDP) BD-rate drop compare to single layer HM10.1 (“main” configuration). Similar test with 10 bits content of class A resolution gives only 0.7% (AI)/ 2.4% (RA) / 9.7 % (LDB)/ 6.0% (LDP) BD-rate drop compare to single layer HM10.1 (“main10” configuration) if spatial and bit-depth scalability are combined. So, noticeable performance improvement can be achieved by combination of spatial and bit-depth scalability.

The basic idea is to retain the effect of interpolation filters, i.e. round the filter output to the bit depth of the enhancement layer.

Question whether downsampling in combination with the 8-bit rounding was done in a reasonable way. Typical systems perform dithering to avoid banding effects.

BD rate comparison made in the document may not be fully consistent, as the SHM result was using 8 bit PSNR measurement, and the modified version used 10 bit PSNR. Would be more  consistent to use 10 bit PSNR for both, and fill “10” into the SHM 8 bit output.

JCTVC-N0146 AHG14: On resampling & color gamut scalability [K. Ugur, A. Aminlou (Nokia)]

JCTVC-M0214 proposed a method for increasing the bit-depth of enhancement layer to achieve color gamut scalability. An important use-case mentioned in JCTVC-M0214 is the enhancement layer increasing both the spatial resolution and bit-depth of the base layer signal and this is achieved by using a cascaded process, where a resampling filter is utilized first on base layer reconstructed picture and then the bit-depth of the upsampled picture is increased. It is argued that this results in redundant computations and this contribution instead proposes to change the resampling process to achieve spatial resolution and bit-depth increase.

The second version of this document presents experimental results for the proposal. When compared to the method in JCTVC-M0213 keeping the high precision operations and performing upsampling and bit-depth increase jointly achieves a BD-rate on average -0.22%, -0.33%, -0.43% for main tier, high tier and super-high tier.

Similar approach as N0218, but all PSNR measured in 10 bits.

Question to be clarified (issue for plenary discussion and seek advice from parent bodies):

- If SHVC profile only would support 8 bit in both BL and EL, no need to do this

- If additionally a 10+ bit would be defined, 8-to-10 bit scalability could easily be supported by an approach as suggested in N0148 and N0218

- It is inconsistent in the current requirements document that support for color gamut scalability is included, but bit-depth scalability is not mentioned.

- Need to start thinking about SHVC profile(s), and seek industry input about the needs.

- Relations with range extensions also to be clarified.

JCTVC-N0272 Non SCE1: On handling resampling phase offsets with fixed filters [K. Minoo, D. Baylon, A. Luthra (Arris)]

This document discusses issues with resampling using fixed filters, and reports the results of a comparison of fixed filters with different phase resolution tested in SCE1 Test 1.2 with that proposed in SCE1 Test 2.2. For the sequences tested with BL sequences generated with a downsampling offset, the results reportedly show that even with 1/16 phase resolution accuracy, there is a loss in luma performance of 0.6% for 1.5x scalability compared to the adaptive filter signalling proposed in SCE1 Test 2.2.  In order to compensate for limited filter phase precision computation using fixed filters, it is proposed here that offsets be signalled per filter to improve performance when fixed filters are used.

No need for presentation in track B – was discussed in the context of SCE1, this could become relevant after the definition of filters for arbitrary upsampling ratio.

Delegated to BoG on arbitrary scalability ratios to consider whether the contribution contains relevant input for their work.

JCTVC-N0273 On the selection of fixed filters for upsampling [K. Minoo, D. Baylon (Arris)]

Discussed in BoG on arbitrary scalability ratios.

### SCE2 related (inter-layer syntax prediction and motion compression)

JCTVC-N0233 Non-SCE2: scaled MV in motion field buffer update [J. Xu, A. Tabatabai, K. Sato (Sony)]

(Reviewed in Track B Sun. 28th (JRO).)

MV scaling in motion field buffer update is proposed to keep the coding gain and avoid extra reference pictures introduced by JCTVC-N0252. Simulation results state that BD-rate numbers of proposed approach are close to those of JCTVC-N0252. Combine test results state that there are negligible changes in BD-rate.

The advantage compared to N0252 is the omission of the “virtual” motion field in the reference list and perform scaling of an available entry instead.

See further disposition under N0252.

JCTVC-N0262 Cross check report of JCTVC-N0233 [K. Misra, A. Segall (Sharp)] [late]

### SCE3 related (inter-layer filtering)

JCTVC-N0061 Non-SCE3.3: Inter-layer interpolation-based SAO filtering for SHVC [Alexey Filippov, Vasily Rufitskiy (Huawei)]

(Reviewed Sat. 27 Track B (AS).)

This contribution proposes adaptive restriction of EO offsets for the inter-layer SAO filter that was initially proposed in JCTVC-L0234 and JCTVC-M0114. The proposed modification of JCTVC-M0114 is based on restricting EO offsets for pixels. The restrictions are calculated using the values of the same neighbouring pixels that are used for determining the edge index for the Edge Offset (EO). Coding gain measurement performed over all SHM2.0 configurations reveals 0.7% (Y), 0.6% (U), 0.6% (V) average BDR gain on mandatory configurations. For the LD-P SNR configuration, BDR gain can reach up to 2.7% (Y), 1.8% (U), 1.7% (V).

Complexity results reported by processing the first 32 frames of each sequence.

Comment; Similar technique considered in HEVC design.  Conclusion then was that complexity did not justify the gain.

Response that proposed design provides lower complexity.

Comment: Possible benefit in subjective quality.

Further study was encouraged.

JCTVC-N0344 Non-SCE3.3: Crosscheck of JCTVC-N0061 on Inter-layer interpolation-based SAO filtering for SHVC [X. Li (Qualcomm)] [late]

JCTVC-N0070 Non-SCE3: Inter-layer prediction modes based on base layer sharpness filter [M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei)]

(Reviewed Sat. 27 Track B (AS).)

A method for predicting higher resolution layer images from lower resolution layer images is proposed. The algorithm applies a sharpness filter to to the low resolution frame.  Simulation results show that the proposed method provides 1.7% and 1.0% BD rate savings on average for AI-2x and AI-1.5x, respectively, compared with anchors.   Class A test sequences show 2.9% BD rate saving. Encoding times are 113.3% and 111.2%, and decoding times are 117.7% and 116.0%.

The results provided use the same parameters for all sequences.

The proponent reports that future study includes adaptation of parameters and reduction of complexity

Comment: The coding gains are interesting.

Further study in a CE was suggested.

JCTVC-N0229 Non-SCE3: Region based Inter-layer Cross-Color Filtering [X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin, Y. Cho (Samsung)]

(Reviewed Sat. 27 Track B (AS).)

In this proposal, a region based inter-layer cross-color filtering is proposed. With this method, each chroma component in an enhancement picture is equally split into 1, 4, 16 regions and one set of cross-color filter parameters is signaled for each region. It is asserted that the proposed region based filtering significantly improves the coding performance by providing better local adaptation. It is reported that 1.4%, 11.1%, and 22.0% BD-rate reduction of Y, U, and V components were obtained on average for AI cases. For inter cases (RA, LD-B and LD-P), 0.4%, 11.0%, and 20.1% BD-rate reduction of Y, U, and V components were achieved on average.

Adaptive partitioning of inter-layer partitioned multiple.

Asserted to be lower complexity than previous proposal on cross-color filtering (N0152).

Removes some tap locations from the calculation.

Comment; The proposed method reduces delay.

Comment: Are there issues with boundaries of the region based processing?

Comment: Is the quad-tree structure fixed?

The current implementation does require multi-pass encoding.

There was a request for subjective viewing.

It was noted that the partitioning may introduce encoder delay and asked whether it is possible to do LCU based application?

Concern was expressed about partitioning granularity (motivating by SAO design discussions in HEVC).

There was a suggestion to study further in core experiment.

Further study in a CE was suggested.

JCTVC-N0311 Cross check of Region based Inter-layer Cross-Color Filtering (JCTVC-N0229) [E. François (Canon)] [late]

(Reviewed Sat. 27 Track B (AS).)

The results matched and the proposal was confirmed.

JCTVC-N0250 Non-SCE3.3: Modified Interlayer SAO with highpass processing [S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

(Reviewed Sat. 27 Track B (AS).)

This proposal attempts to further reduce the complexity for inter-layer SAO processing studied in SCE3.3. It is achieved by reusing the current method employed in the base-layer HEVC for sample classification for the two diagonal EO classes. The method proposed in SCE3.3 is only applied to the horizontal and vertical classes. Experimental results report that the proposed algorithm achieves average overall BD rate savings 0.6%, 0.7%, and 0.8% for Y, U, and V color components, respectively, compared with SHM-2.0 and has no BD rate loss compared with the current IL-SAO in SCE3.3 under the mandatory common test conditions.

The proposal shows improvement compared to the 3.3 test in the CE.

No action was taken on this.

JCTVC-N0307 non-SCE3: Cross-check for inter-layer SAO design from MediaTek [E. Alshina, A. Alshin (Samsung)] [late]

### Sampling position

JCTVC-N0111 Sample position mapping with already defined scaling factor [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

(Reviewed Sat. 27 Track B (AS).)

In SHVC, scaling factors are used together with the current and reference layer pictures sizes for the resampling process. This contribution proposes to use only scaling factors for this purpose.

Comment: Obvious bugfix

Comment: Division that is replaced is not needed in implementation of 2x and 1.5x.  Perhaps editorial for 2x and 1.5x.

Comment: Seems reasonable

Comment: Mainly affects arbitrary scaling ratios, if the group decides to adopt arbitrary scaling ratios.

Decision: Adopt.

JCTVC-N0318 Cross check JCTVC-N0111 Sample position mapping with already defined scaling factor [Y. He (InterDigital)] [late]

JCTVC-N0248 Support of Field Coding for Signalling of Chroma Phase for Upsampling [K. Sato (Sony)]

(Reviewed Sat. 27 Track B (AS).)

It is proposed by that chroma sampling position be transmitted for the purpose of up-sampling for spatial scalability. The proposed syntax is in sps_extension() and only one phase can be specified. However, if a field structure is applied, chroma phase for the top field picture may be different from the one for the bottom field picture within a sequence.  This contribution proposes to modify the syntax to allow up to 2 phases for up-sampling to support field coding.

The proposal is to signal the sampling grid information for both top and bottom field.

It is similar to M0465/N0045.

No simulation results were provided.

It was commented that using interlaced material should be considered for this study.

It was recommended to coordinate further study with consideration of N0045.

JCTVC-N0283 Aspect ratio scalability based on SHM-2.0 [Y. Liu, J. Ostermann (Leibniz Uni Hannover)] [late]

(Reviewed Sat. 27 Track B (AS).)

An approach to support different aspect ratios is proposed.  The proposal assumes that the baselayer represents a vertical out take of the enhancement layer.  It is reported that the approach achieves a luma BD-rate reduction of 6.5% compared to simulcast and an average of 2.0% reduction of encoding time.

Question: Does the approach support changing window on a picture by picture basis? A: Yes.

Comment: Perhaps related to JCTVC-N0089

Comment: Picture level adaptation may make motion field mapping less efficient

It was unclear if there is a strong need for picture based adaptation. Additional information gathering was encouraged as appropriate.

JCTVC-N0334 Derivation of picture and slice level information for resampled interlayer reference picture [J. Chen, V. Seregin, X. Li, K. Rapaka, M. Karczewicz (Qualcomm)] [late]

(Reviewed Sat. 27 Track B (AS).)

This contribution proposes specification text of derivation process for picture and slice level information of resampled interlayer reference picture.

Consideration of this was deferred to when all HLS and SHVC interested parties were available. It was then discussed in an HLS BoG. See BoG report N0374 and related notes.

### Up-/downsampling filters

JCTVC-N0055 On resampling process for outside-bounds samples [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

(Reviewed in Track B Sun 28th. (JRO).)

This contribution proposes to modify the padding process in luma and chroma resampling to use the padding for reference-layer picture boundary instead of the padding for target-layer ROI boundary (scaled reference layer offset boundary). It is asserted that the modification unify padding process for the picture boundary and the ROI boundary. The method is implemented on SHM2.0. It is reported that no BD-rate changes are observed for all common test conditions (AI 2x, AI 1.5x, RA 2x, RA 1.5x RA SNR, LB 2x, LB 1.5x, LB SNR cases).

In addition, additional tests were done, in which scaled reference layer offsets were set to positive values (scaled_ref_layer_left_offset = 20, scaled_ref_layer_right_offset = 20, scaled_ref_layer_top_offset = 22, scaled_ref_layer_bottom_offset = 22). We also found a bug on SHM2.0 that the length of left-side padding area on a resampled reference layer was not correct and fixed the bug (SHM2.0+bugfix).

It is reported that BD-rate gains (EL only) of the proposal compared to SHM2.0+bugfix are 0.00%, -0.01% and -0.01% for AI 2x, RA 2x and LB 2x, respectively. Test packages for additional tests are also provided.

The proposal suggests to first perform padding of base layer for the scaled reference offset area, and then perform upsampling. Currently, first upsampling is performed, and then padding of boundary samples for the upsampled picture. This requires also padding of samples at the boundary of the base layer picture which are needed for upsampling filter. However, the scaled reference offset area would be larger. Therefore, if applied straightforward for the whole scaled area it would be more complex, but clipping might be performed for cases where the input to the filter would be identical samples. One argument of the proponents is that the “base layer padding” is more convenient for block based implementation.

The two methods of padding (current and proposed) would give different results for samples close to the boundary of the base layer picture.

If implemented efficiently, it may mean to partially re-invoke the current method (after [4 pixels x spatial ratio] the filter output from the padded base layer would be identical, such that the filtered result could be padded in the enhancement layer). Further study and evidence about the complexity benefit would be required.

Decision (SW): The bug fix appears necessary and should be made with the SW coordinator (no effect on the text).

JCTVC-N0086 Independent tile upsampling for SHVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

(Reviewed Sun. 28 Track B (AS).)

This contribution proposes a constraint for the spatial scalable upsampling filter at the border of tiles in reference layers. A syntax element is proposed to enable the constraint. If the syntax element is enabled, it is proposed that no pixels shall be used for upsampling that lie outside the tile that the collocated reference layer pel lies in. This document is based on JCTVC-M0198v2 and JCTVC-M0464 which contains a text proposal for the upsampling filter process. The base documents were discussed at the Incheon meeting in the joint BoG on high-level syntax. The plenary decided that this is a low-level change and people concerned with low-level tools were not aware of this discussion. Thus we are proposing the concept again for consideration in the appropriate group.

Remarked that N0159, N087 is also related to this proposal. N087 is a tile constraint SEI message.

It was reported N0159 is the same as N0086 except that the signalling is the SPS instead of the PPS.

It was asserted that the PPS may be desirable because the tiles_enabled_flag is in the PPS.

The presenter reported that text is available.

It was commented that an encoder layer constraint may be more desireable.  However, we do not currently know the performance of this solution.

Concern was expressed on the addition of a tile boundary check in the inter-layer prediction process.

It was expressed that the tile locations may be easier to determine, compared to slice based constraint, in an application due to how the tile information is signaled.

It was commented that a decoder change may not be sufficiently justified at this time.  An additional expert supported this statement.

The solution of this problem was recommended to be an encoder constraint and indication. HLS experts were asked to consider and define solution. See BoG report and related notes.

JCTVC-N0144 AHG13: Slice based upsampling filter for improved error resiliency [K. Ugur, M. M. Hannuksela (Nokia)]

(Reviewed Sat. 27 Track B (AS).)

In spatial scalability, base layer samples are upsampled to enhancement layer resolution and used as reference. If the base layer picture is coded with slices, sub-pixel samples close to the border of the slice are calculated using the integer samples from another slice. Similar to filtering across slice-boundaries, it is asserted that this increases error propagation and reduces the error resiliency when SHVC is used within error-prone environments. Similar to restricting filtering across slice boundaries, this contribution proposes an optional restriction on upsampling, so that sub-pixel samples never use integer samples from another slice. It is noted that the proposed functionality is already present SVC using the constrained_intra_resampling_flag syntax element.

Comment: This requires the enhancement layer to have knowledge of the location of slices within a baselayer

Comment: This may require an additional slice boundary (of the baselayer)  check per block

Comment: Concern expressed that the boundary of the slice may be not be rectangular

Comment: One participant expressed support for a decoder constraint to restrict inter-layer prediction

(Further discussed Sun. 28 Track B (AS).)

This was discussed further after reviewing JCTVC-N0086.

One participant commented that the concern about boundary checks for the previous proposal were also valid for this proposal.

Recommend solution of this problem with an encoder constraint and indication. HLS experts were asked to consider and define solution.

JCTVC-N0153 On adaptive re-sampling filter coefficients coding [E. Alshina, A. Alshin (Samsung)]

(Reviewed Sat. 27 Track B (AS).)

In SCE1 and SCE3 adaptive re-sampling filter solutions for SHVC were studied. In this contribution coding method for content adaptive re-sampling filter coefficients is suggested. The benefit of proposed method is no overflow of 16-bits intermediate buffer is guaranteed.

Information document on how to manage the dynamic range of an adaptive upsampling filter

Currently, the SHVC design does not include adaptive upsampling filter

No action was taken on this.

JCTVC-N0265 AHG13: Complexity reduction of the up-sampling process [K. Andersson, R. Sjöberg (Ericsson)]

(Reviewed Sat. 27 Track B (AS).)

It is asserted that multi-loop scalable coding has a significant impact on average decoder complexity. To make it possible for applications to trade off performance versus complexity it is proposed to enable use of 4-tap filters in addition to the current 8-tap filters for luma up-sampling.

It is reported that deploying 4-tap filters for luma up-sampling results in a BDR loss of 0.9% for luma with a 20% reduction in average number of multiplications and additions for MC and block based up-sampling.

For coding with B or P pictures, it is reported that the BDR loss is 0.9% for 2x scalability and 0.4% for 1.5x scalability with a 10%/20% reduction on average of multiplications and additions for MC and up-sampling for respectively PU and picture based up-sampling. It is reported that the BDR loss for intra coding is a BDR loss of 1.9% for 2x scalability and 1.1% for 1.5x scalability with more than 40% reduction on average of multiplications and additions for both PU based and picture based up-sampling.

The proposal also reports the use of  8-tap luma up-sampling filters for intra slices and proposed 4-tap luma up-sampling filters for other slices. This result is an average BDR loss of 0.4% for random access compared to SHM-2.0.

Comment: Possible to use 6-taps instead of 4-taps to balance loss with complexity reduction.

Comment: Mixture of 4-taps and 8-taps would require support for two upsamplers.

Comment: 4-tap only may be more interesting than the combination of 4-tap and 8-tap.

Comment: 4-tap filters are defined for chroma – possible to use the same filters for chroma and luma.

Current chroma filters were considered but provide more loss in coding efficiency.

May be preferred to use existing chroma filters for further complexity reduction.

Comment: Worst case complexity defined by bi-prediction motion compensation.

Comment: Would be good to quantify the performance of the filter within the context of the still picture profile.

Further study was encouraged.

JCTVC-N0280 AHG13: cross-check of JCTVC-N0265 on Complexity reduction of the up-sampling process [E. François (Canon)] [late]

### Residual prediction

JCTVC-N0106 Generalized residual prediction with motion vector clipping for SHVC [K. Kim, H. Jo, J. Ryu, D. Sim, S.-J. Oh (KWU)]

(Reviewed Sat. 27 Track B (AS).)

This contribution proposes a method to reduce the computational complexity of generalized residual prediction (GRP) for spatial scalability. GRP uses a motion vector (MV) of enhancement layer (EL) to reconstruct the residual signal at up-sampled base layer (BL). In the GRP, it is asserted that the up-sampled BL should be interpolated again in the cases of the accuracy of MV is quarter-pel or half-pel. However, since the up-sampled BL has low cut-off frequency in compared to EL, interpolation of the up-sampled BL may not create any higher frequency components. Therefore, to reduce computational complexity of GRP, the proposed method removes the interpolation step of the up-sampled BL and the residual is constructed with integer motion vectors. With the proposed GRP method, encoding time of SHVC increases by 11% and BD-rate decreases by 0.4%, compared with SHM2.0-based IntraBL.

Comment: Results are interesting and seem to suggest that half pel interpolation is promisingComment: Information greatly appreciated.

No action was taken due to the decision to focus on the RefIdx approach.

JCTVC-N0204 ILR enhancement with differential coding for SHVC reference index framework [Y. He, Y. Ye, X. Xiu (InterDigital)]

(Reviewed Sat. 27 Track B (AS).)

This proposal describes inter-layer reference (ILR) enhancement with differential coding for SHVC reference index (RefIdx) framework. In SHVC reference index framework, the base layer reconstructed picture (after upsampling if needed) is used as an additional reference for enhancement layer coding. In this contribution, the ILR is further enhanced by adding weighted differential signal from the temporal domain to restore high frequency information. The differential signal is generated by motion compensation in the temporal domain with the motion field from the base layer picture. Compared to the SHM-2.0 RefIdx anchor, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-2.3%, -6.6%, -7.4%}, {-2.9%, -7.0%, -7.6%} and {-3.6%, -6.9%, -7.3%} for RA, LD-B, and LD-P if uncompressed motion field from base layer picture is used, respectively. The results with 8x8 and 16x16 sized compressed motion field are also reported.

JCTVC-N0277 Cross-check of JCTVC-N0204 on ILR enhancement with differential coding for SHVC reference index framework [E. François (Canon)] [late]

JCTVC-N0234 Low-Complexity Generated Inter-layer Reference for SHVC [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

(Reviewed Sat. 27 Track B (AS).)

In this proposal, a second inter-layer reference is generated based on previously coded pictures by a GRP (generalized residual prediction) like method. To reduce computational cost and memory bandwidth requirement, sub-pixel motion compensation interpolation is avoided by rounding motion vectors to the closest integer-pixel positions. It is reported that 0.8%, 1.3% and 0.7% luma BD-rate reduction was obtained on average for RA, LD-B and LD-P cases, respectively.

Integer pel motion vectors for both luma and chroma are used

Compressed motion vector field is used

Complexity reduction acknowledged and gains impressive

Multiple participants expressed skepticism that the technology would be adopted after study in a CE due to current complexity

The topic was suggested to be for further study with N0204.

This was further discussed in Track B on Sun 28th (AS) and agreed to be for further study (not in CE).

JCTVC-N0314 Cross-check of JCTVC-N0234 on Low-Complexity Generated Inter-layer Reference [P. Lai, S. Liu, S. Lei (MediaTek)] [late]

### Key picture concept and single-loop decoding

JCTVC-N0161 Using decoded pictures from higher layer as references in SHVC [K. Rapaka, J. Chen, M. Karczewicz (Qualcomm)]

(Reviewed in Track B Sun. 28th (AS).)

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document proposes

- Signalling a flag in VPS to specify that immediate higher layer picture is used as reference for lower layer

- The concept of key picture to reduce error drift for base layer decoding

When the SHM 2.0 anchor is modified to enable temporal scalability, the proposed method reports an average of -2.5%, -3.1% and -3.2% luma BD-rate reductions (EL+ BL) for RA-SNR, LD-P and LD-B SNR scalability cases respectively. It is also reported that over SHM 2.0 anchor an average of -2.3%, 0.2% and 1.4% luma BD-rate reductions (EL+ BL) is obtained for RA-SNR, LD-P and LD-B SNR scalability cases respectively.

Further, this contribution proposes encoder only constraints to facilitate a single-loop decoding design for SHVC SNR scalability. Based on the key picture framework, separate encoder only constraints for key and non-key pictures are proposed to avoid the need of motion compensation in the reference layer. A flag is signaled in VUI to indicate the presence of such bitstream constraints. It is also reported that over SHM 2.0 anchor (with temporal scalability) an average of 3.1%, 6.3% and 4.0% luma BD-rate reductions (EL+ BL) is obtained for RA-SNR, LD-P and LD-B SNR scalability cases respectively.

It was reported that this contribution is similar to JCTVC-N0202.

The base layer is required to have SAO and deblocking and disabled.  It was suggested to study the visual quality impact of this design.

Results were reported using a XLS table that is not the same as commonly reported to the group. Additional results were requested, and the proponent agreed to provide such results.  Additionally, it was asserted that the results in JCTVC-N0202 are reported relative to the CTC anchor.

There was discussion that an organized study would be necessary to identify correct and appropriate methods for reporting results for a single loop configuration.

JCTVC-N0352 Cross-check of JCTVC-N0161 from Qualcomm on using decoded pictures from higher layer as references in SHVC [J. Xu, C. Auyeung (Sony)] [late]

JCTVC-N0186 AHG16: Single-loop decoding of SNR scalability for refIdx based SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

(Reviewed in Track B Sun. 28th (AS).)

This contribution describes a single-loop decoding scheme for SNR scalability of reference index based SHVC. In the current reference index based SHVC, the reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. This implies that multiple motion compensation (MC) operations have to be performed for all dependent layers. In this contribution, a single-loop decoding scheme is achieved by introducing an alternative ILP picture by applying BL motion information on EL temporal reference pictures. In addition, BL residue is added to the alternative ILP picture for quality improvement. As both BL motion information and BL residue could be obtained without the full reconstruction of BL picture, the single-loop decoding requirement is fulfilled by replacing the conventional ILP picture with the alternative ILP picture for EL prediction. Compared to the existing single-loop decoding approaches studied in AHG16, the proposed scheme does not require any low-level changes to single-layer HEVC.

Experimental results shows that compared to SHM 2.0 reference index based multi-loop decoding scheme, the proposed sing-loop decoding scheme can reportedly reduce the average memory access by 34% for block-based implementation and 32% for picture-based implementation, with average BD-rate increase of 1.3% and 1.5% for RA and LDB configurations respectively. In addition, the average decoding time is reduced by 25%. If the combined prediction between the alternative ILP picture and EL temporal reference picture is disallowed for EL prediction, the average decoding time reduction is increased to 28% and the worst case of memory access is the same as single-layer HEVC with BD-rate increase of 2.2% and 3.5%.

The proposal introduces a new form of inter-layer prediction into the SHVC design.  It applies base layer motion to an enhancement layer prediction.  Moreover, it adds the baselayer residual to this prediction.

It was asserted that the method doesn’t introduce drift in the baselayer, which may differentiate it from other proposals in the area.

Request to establish a CE and additionally study the software in AhG.

It was suggested that since the proposal requires access to the baselayer residual information, which requires a low level change in many existing implementations.

Concern was expressed by multiple participants that this proposal is not consistent with a “high level change only” design.

It was remarked that a multiple loop decoder decoding the proposed single loop bit-stream would require higher memory bandwidth than our current SHVC design.

It was requested to study the constraint of not using bi-directional prediction in the baselayer in the existing SHVC design  It was remarked that this may have been previously studied.

It was remarked that the baselayer was changed by requiring constrained intra prediction.

It was asked why there was a large chroma gain.  Currently, there is not a clear understanding.

JCTVC-N0187 Investigation on using H-ILP picture for refIdx based SHVC [X. Xiu, Y. Yan, Y. He (InterDigital)]

(Reviewed in Track B Sun. 28th (AS).)

The reference index based solution of current SHVC Test Mode (SHM-2.0) is built upon a multi-loop decoding scheme, where reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. One hybrid inter-layer prediction (H-ILP) picture, which is generated using the motion information and the residue information of BL picture and the texture of EL temporal reference picture, is proposed to replace the conventional ILP picture for the single-loop decoding of SNR scalability of SHVC in JCTVC-N0186. This contribution investigates the performance of H-ILP picture based multi-loop decoding by using both ILP picture and H-ILP picture as inter-layer reference pictures for EL prediction. As a part of the investigation, bilinear filter is applied to up-sample the residue of BL picture for the generation of H-ILP picture in the spatial scalability cases. Our investigation shows that H-ILP picture can further improve the performance of SHVC reference index based framework by providing 2.1%, 2.4% and 1.6% BD-rate savings on average for RA, LDB and LDP respectively.

The document was provided for information.

The proposal reports the results of using the inter-layer prediction from N0186 in a multi-loop design.  This includes extension of the method to spatial scalability.

JCTVC-N0202 AHG16: Key picture concept and single loop decoding [C. Feldmann, F. Jäger, M. Wien (RWTH Aachen Univ.)]

(Reviewed in Track B Sun. 28th (AS).)

In this document a performance analysis of the key picture concept in SHVC is presented. The scheme has similarity with MGS as used in SVC. A multi-loop and a single-loop variant are presented. In the multi-loop variant, the enhancement layer performance is reportedly increased by 3.1% BD rate on average. Due to the induced drift, the quality of the reconstructed base layer is reportedly reduced by 4.5% BD rate.

As a modification of the scheme, a variant is presented that adds restrictions to the encoder in order to allow for single loop decoding of the enhancement layer. The enhancement layer performance is reportedly comparable to the multi-loop approach of SHM while the quality of the base layer is reportedly reduced by 0.23 dB BD PSNR on average.

In the proposal, non-key pictures are allowed to reference enhancement layer pictures.

Results show a gain of 3.1% for the overall system.  At the same time, the base layer coding efficiency is reduced by 4.5%

The method was also extended to single loop decoding.  Constrained intra prediction was enabled, the unfiltered BL frame was used for predicting the EL.  Results are provided for disabling the filters (SAO and de-blocking) for the BL shows a loss of 1.6% and a loss of the baselayer of 13.9%.  Enabling the filters for the BL, but using the unfiltered result for EL prediction, reduces the loss due to drift to 6.7%

Low delay results were not provided, and it was suggested that the topic of low delay needs discussion.  It was discussed that the approach in N0161 may be relevant.

It was suggested that the group should visually study the impact of base layer drift in the design.

It was suggested that the area should be further investigated

In terms of categorization, we appear to have two different concepts here.  The first is to use a key picture concept, which allows (or requires) drift within the base layer.  The second is an SVC-style single loop structure.

Test conditions would require modification of CTC.

One suggestion for an anchor would be to enable the use of max_tid_il_ref_pics_plus1

A test should consider the impact on the base layer

A test should measure and report the change in memory bandwidth and complexity.

It was planned to establish a CE on the topic relating to N0161, N0186, N0202.

JCTVC-N0297 AHG16: cross-verification of JCTVC-N0202 on Key picture concept and single loop decoding [X. Xiu (InterDigital)] [late]

JCTVC-N0203 AHG16: Cross-Check of JCTVC-N0186 Single-loop decoding of SNR scalability for refIdx based SHVC [C. Feldmann, M. Wien (RWTH Aachen Univ.)] [late]

### Colour and bit-depth scalability

Most documents in this category were reviewed in the BoG JCTVC-N0387 (A. Segall).

JCTVC-N0387 BoG report on Color Gamut Scalability [A. Segall]

Reviewed Thu p.m. The BoG met on July 27, 2013, and August 1, 2013. Additionally, the BoG conducted a viewing session on July 31, 2013.

Suggested plan going forward:

- Continue AhG on color gamut scalability with additional mandates above.

- Create CE for organized testing of proposals for color gamut scalability.  To include JCTVC-N0146, JCTVC-N0168, JCTVC-N0218, JCTVC-N0271, and JCTVC-M0214.

See BoG report for agreed plans.

JCTVC-N0145 AHG20: Backwards compatible enhancement of chroma format [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

(Initial review chaired by AS.)

This contribution proposes a backwards compatible chroma format enhancement method, where base layer codes the 4:2:0 with HEVC version 1 and enhancement layer codes 4:4:4 U, V colour planes separately, using the functionality indicated with separate_colour_plane_flag in HEVC. In other words, the high and low resolution chroma components are simulcasted. This idea was first introduced in JCTVC-M0229 and in this contribution additional experimental results are presented analyzing the effect of separate_colour_plane_flag. Furthermore, software implementing the idea is planned to be attached to this contribution as a revision prior to the meeting.

Version 1 of this contribution includes detailed experimental results showing that the proposed backwards compatible enhancement is achieved with using around 39% less bits over simulcast. When compared to single layer coding of 4:4:4, the scalability is achieved with around 11.3% penalty in coding efficiency.

(From initial review, chaired by AS) The contribution does not have a direct comparison with spatial scalable coding, but estimated around 10% loss relative to that. Proponent roughly estimates that about 3% loss is coming from using separate color planes and the other 7% from not using the base layer to predict the enhancement layer.

Further review in plenary Monday 29:

- Base and enhancement: Luma is shared, chroma is simulcasted.

- Gain over simulcast due to share of luma average 36%, loss over single layer 14% for RExt 4:4:4 camera-captured content.

- Options suggested by proponent: Code chroma full res as auxiliary pictures, or include in RExt (technially equivalent or very similar to first option) or include in SHVC in some way (perhaps with predictive coding of the 4:4:4 chroma MVs and/or texture)

It is pointed out that the coding as auxiliary pictures would be more complex, due to longer interpolation filters (RExt uses 4-tap interpol. filters for chroma even in 4:4:4 case)

The chroma information from 4:2:0 would still need to be parsed.

In RExt, the color planes would be coded as independent monochrome components.

In SHVC, motion information is sent separate for the chroma components.

For the 4:2:2 case, the auxiliary coded planes would be half width.

When used in RExt: Would it be a separate mode to conventional 4:4:4? Probably, i.e. a 4:4:4 decoder would be required to decode both types.

There was reluctance among some participants to require 4:4:4 profile decoders to decode this type of 4:4:4 content in addition to decoding an ordinary 4:4:4 encoding. One suggestion was to impose such a requirement but with lower-resolution decoding capability for this type of encoded data.

The functionality is assessed as useful, but should not be put as a burden on every decoder. Therefore, the version of defining colour planes as auxiliary picture type would be the desirable way out of the three suggested solutions.

It was remarked that this may have some relationship to N0116.

The proponent suggests expressing the syntax using the auxiliary coded picture syntax, with Cb and Cr basically being additional auxiliary picture types in addition to alpha and pre-multiplied alpha.

The consequences for profile definition requires further study. It is for further study to determine whether to specify decoder conformance in a profile.

This was further discussed on Friday and agreed to be for further study in the AHGs on SHVC HLS and RExt.

It was noted that auxilliary pictures have existed outside the scope of conformance and could hypothetically be specified on a different schedule because of that.

JCTVC-N0163 AHG14: Wide Color Gamut Test Material Creation [Pierre Andrivon, Philippe Bordes (Technicolor)]

JCTVC-N0168 AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results [Philippe Bordes, Pierre Andrivon, Franck Hiron (Technicolor)]

JCTVC-N0274 AHG14: Cross-checking of JCTVC-N0168 from Technicolor on color gamut scalable video coding using 3D LUT [C. Auyeung (Sony)] [late]

JCTVC-N0278 AHG14: cross-check of JCTVC-N0168 on Color Gamut Scalable Video Coding using 3D LUT: New Results [P. Onno (Canon)] [late]

JCTVC-N0271 AHG14: Color gamut scalable video coding with piecewise linear predictions [C. Auyeung (Sony)]

JCTVC-N0339 Cross-check report for JCTVC-N0271: AHG14: Color gamut scalable video coding with piecewise linear predictions [S.-H. Kim, A. Segall (Sharp)] [late]

## HL syntax for range extensions and single-layer HEVC coding

JCTVC-N0155 HLS: Thumbnail Support in HEVC [C. Wang, C. Wang, W. Zhang, Y. Chiu (Intel)]

This contribution proposes a modification to the syntax of current HEVC in order to embed thumbnail video into HEVC bitstreams. A thumbnail video, which is a sequence of snapshots with smaller resolution, is required in many video applications to give a preview of the video content. To support this feature, three modifications are applied to the high level syntax of HEVC. Firstly, a new VCL NAL unit type is proposed to contain the coded thumbnail picture. Secondly, a new SEI message is added to transmit auxiliary information, e.g. the period of presentation of the thumbnails. Finally, a reserved bit in profile_tier_level() structure is changed into a flag to signal the on/off of the thumbnail video in a bitstream. These modifications reportedly do not violate the conformance requirement of current HEVC, and provide a flexible method to enclose the thumbnail information into regular HEVC bitstream.

Using an all-intra VCL NALU was proposed.

Thumbnail alignment to POC was suggested.

It was commented that auxiliary pictures seems like a good alternative approach, perhaps accompanied by SEI.

Further study was encouraged, particularly focused on the auxiliary pictures approach.

JCTVC-N0269 HLS: Non-significant slice segments with tiles for single layer HEVC extensions [C. Auyeung (Sony)]

The motion-constrained tile sets SEI message in Range Extensions text specification: Draft 3 can be used to signal that a CVS is comprised of one or more regions of interest in the form of independently decodable motion-constrained tile sets. A user can interactively select and decode a motion constrained tile set without decoding the other tiles. This contribution proposes syntax and semantics that support the extraction of independently decodable motion-constrained tile sets to form a new bitstream without transcoding at the CTU level for streaming of the regions of interest in applications such as interactive UHDTV application, dynamic high-quality zoom-in application, interactive on-demand, e-learning, smart surveillance, and etc.

A prior relevant idea was presented in M0277 and M0046.

Proponent describes replacing regions outside an ROI set of tiles with blank/filler coded picture regions.

As proposed, this would require a new profile.

It was remarked that alternatives to non-backward-compatible syntax may be feasible for this. An SEI message can indicate that some slices would decode as empty.

An analogy was drawn with the pseudo-monochrome indicator at the SPS/VUI level.

Further study is encouraged.

JCTVC-N0063 REXT/MV-HEVC/SHVC HLS: Auxiliary picture layers [M. M. Hannuksela (Nokia)]

This contribution proposes to specify new scalability dimension in the VPS extension syntax for a layer carrying auxiliary pictures. It is asserted that the proposed approach:

- Enables to indicate CPB HRD parameters separately for the primary pictures (i.e., the base layer) and for the entire bitstream;

- Enables to the bitstream extraction process to extract the bitstream containing the primary pictures only;

- Can be used in systems functionalities, such as session negotiation, as the presence of auxiliary pictures is indicated through the VPS; and

- Does not require any new VCL NAL unit type(s).

Revision 1 discusses the following options on syntax structures for including the type and characteristics of auxiliary pictures. Options 1 and 2 are relevant regardless of whether auxiliary pictures are carried on specific layers (as proposed in this contribution) or within new VCL NAL unit type(s). Option 3 is suggested in this contribution for the proposed auxiliary picture layer design.

- Indicating the type and characteristics of auxiliary pictures in the SPS extension.

- Indicating the type and characteristics of auxiliary pictures in an SEI message.

- Indicating the type of auxiliary pictures in the VPS extension and the characteristics in an SEI message.

Revision 2 includes specification text for chroma enhancement auxiliary pictures proposed in JCTVC-N0145.

It was remarked that the scheme is quite interesting and seems like a good idea. It was agreed to consider this as a starting point for strong consideration and further development in AHGs on SHVC HLS and RExt. The proponent indicated that software could be provided and this was encouraged.

JCTVC-N0077 AHG 5: On support for alpha channel in HEVC [M. Naccari, M. Mrak (BBC)]

Alpha channel signals are used in professional (studio) video coding applications and their usage may become popular also for frame composition at the receiver side. Briefly, the alpha channel complements the information contained in the main bitstream by providing, for each image pixel, its degree of transparency. To support the coding and embedding of alpha channel in the range extensions of the HEVC standard, this contribution introduces the concept of auxiliary picture. An auxiliary picture is a picture which is sent together with the primary coded picture and is compressed and managed as a monochrome picture coded with the same coding tools specified in the syntax of the HEVC standard. In this document, proposed definitions, syntax and semantics for the proposed auxiliary pictures are introduced first and then different options, to address different application requirements, are proposed and discussed.

The contribution proposes the use of LTRPs for repetition, and proposes thresholding values for opaque/transparent determination. The topic was agreed to be for further study in AHGs on SHVC HLS and RExt.

## HL syntax in SHVC and 3D extensions

### Generic HLS issues

(Reviewed Thu 25th plenary)

JCTVC-N0374 BoG report on SHVC/MV-HEVC HLS topics [J. Boyce]

Joint BoG with JCT-3V.

(Initially reviewed Sun 28th Track A (GJS).)

The BoG met 27 July to review the following contributions:

- 6.4.4.5 Reference picture list construction
JCTVC-N0082, JCTVC-N0095, JCTVC-N0216, JCTVC-N0316

- 6.4.3.3 Efficient parameter set parameters signalling
JCTVC-N0162, JCTVC-N0200, JCTVC-N0212

- 6.6.2 Layer presence and dependency change SEI messages
JCTVC-N0173, JCTVC-N0174

The BoG recommended the following:

- Adopt JCTVC-N0082, initialization process of reference picture lists for HEVC extensions.

- Adopt JCTVC-N0371, scaling list prediction in SPS and PPS (harmonization of JCTVC-N0162 and JCTVC-N0200 variant 3).

- Layer dependency change SEI message (which originated from JVT-S080) be removed from specification (since a new VPS could be sent to change the layer dependency). If the SEI message does remain, to adopt JCTVC-N0174 (with some editorial improvements).

There was some questioning about the 3rd recommendation. However, there was not a clear objection to the recommendation.

Decision: Agreed to these BoG recommendations (assuming 3V confirmation).

The BoG then met again to further discuss initialization of reference picture lists: Revision to JCT3V-D0220, and harmonization of JCTVC-N0095 and JCTVC-N0216, with experimental results compared to JCTVC-N0082

The BoG met 28 July to review the following contributions:

- 6.4.4.3 General inter-layer RPS signalling and derivation

  - N0059, N0081, N0118, N0154, N0195, N0217, N0131

- Portions of 6.4.3.1 General

  - N0130, N0195, N0217

- 6.4.4.6 Management of resampled or filtered inter-layer reference pictures

  - N0128, N0282, N0056

Review of the outcome was conducted in plenary Mon. 29th.

Decision: The JCT-VC endorsed the BoG recommendations of the following:

- Adopt from JCTVC-N0057 second proposal, change decoding process to add the condition that when SamplePredEnabledFlag equal to 1, don't include picture in the motion pred ref list.

- Adopt a condition on signalling inter_layer_pred_layer_idc[ i ], to avoid sending when NumDirectRefLayers equals NumActiveRefLayerPics, and instead infer values.  From JCTVC-N0081, JCTVC-N0195 proposal 1, JCTVC-N0154, and JCTVC-N0217.

- Adopt an Inter Layer Reference Picture (ILRP) presence flag in the VPS, conditioning the presence of ILRP syntax elements in the slice segment header, similar to JCTVC-N0195 proposal 2.

- Add a constraint when splitting_flag is used, that the sum of the lengths be less than or equal to 6, from JCTVC-N0195 5th proposal.

The BoG had also recommended to change order of sorting of inter-layer reference pictures to be in descending rather than ascending order, from JCTVC-N0131 Proposal 4. However, this recommendation was not endorsed by the JCT-VC. This is just a matter of default order. The objection was that the change might have a negative impact on multiview. The change seemed more logical from the scalability perspective, but after further discussion, seemed unlikely to really be needed in the scalability case. The situation does not occur in the spatial scalability case. In the SNR scalability case it was remarked that there would ordinarily be a need to have multiple inter-layer reference pictures in the list, and that this only affects the default order, which can be changed by the encoder if suboptimal in some situation.

Decision (Ed): The JCT-VC endorsed the BoG recommendation that the editors consider the following:

- A text bug fix identified in JCTVC-N0059, in which motion resampling is currently only invoked when alt_collocated_idc flag is equal to 1, while it should be invoked whenever inter-layer motion prediction is performed and the current and reference layer differ in resolution.

- Add an editorial note for SHVC encoders to avoid use of TMVP when the inter-layer reference picture is the only one in the list.

The BoG suggested to the Track that:

- JCTVC-N0128 Proposal 2 be reviewed in the track, as the topic was difficult to address in the BoG. This topic is further discussed above, and AHG study was planned on HRD-related topics.

- The Track further discuss the concept of sending IL RPS candidates in VPS with slice indication, as in JCTVC-N0118and JCTVC-N0081. Regarding JCTVC-N0118, it was commented that this is just about saving bits. Another participant indicated that this was not for use in common cases. The amount of potential savings was unclear, so the topic was agreed to be for further study.

The BoG met again, to further discuss JCTVC-N0217 Proposals 4.1.1 and 4.1.2.

The BoG also met again on 30 July to review the following contributions

- 6.4.4.5 Reference picture list construction (4)

  - Revisit of JCTVC-N0316

- 6.4.4.2 Sub-layer related inter-layer prediction signalling (4)

  - Revisits of JCTVC-N0120, N0109, N0196

- 6.4.5 Tiles and parallel processing (4)

  - JCTVC-N0158,  JCTVC-N0160,  JCTVC-N0199

Decision: The JCT-VC endorsed the BoG recommendations of the following:

- Adopt a presence flag for signalling of max_tid_il_ref_pics_plus1[i], from JCTVC-N0120.

- Move num_ilp_restricted_ref_layers and related offset delay syntax elements from SPS VUI to VPS VUI, and change to a num_ilp_restricted_ref_layers flag per direct dependent layer for each layer, from JCTVC-N0160.

- Modify calculation of xRef[ ] and yRev[ ] variables in semantics for offset delay syntax element semantics, from JCTVC-N0160

- Adopt JCTVC-N0199 proposal 2 variant 2 (also in JCTVC-N0160) to move tile boundaries alignment flag from the SPS VUI to the VPS VUI, and also signal the flag per direct dependent layer for each layer, from JCTVC-N0199.

The BoG had also recommended to add a constraint to not use tiles in a layer and wavefronts in a different layer, when one layer is a direct dependent of the other layer, from JCTVC-N0158. These features cannot be mixed in the same CVS in v1. However, several participants said that there might be advantages of allowing the mixture in the multi-layer case, since the base layer and enhancement layer may target substantially different purposes – e.g., using tiles in an enhancement layer and wavefronts in a base layer. The proponent said that it could be a burden to need to be able to process both in the same bitstream. After discussion, the JCT-VC did not agree to impose this constraint.

The BoG met again on 31 July to cover the following topics:

- 6.6.1 Motion and prediction constrained SEI messages (5)

  - JCTVC-N0088, JCTVC-N0117, JCTVC-N0236

  - Revisits of JCTVC-N0069,  JCTVC-N0159

- 6.2.5 Sampling position

  - Revisit of JCTVC-N0334

Decision: The JCT-VC endorsed the BoG recommendation of the following:

- Add inter-layer prediction tile constraint indication to the motion constrained tile information SEI message. Exact syntax to be provided in new contribution JCTVC-N0383, which harmonizes JCTVC-N0069, JCTVC-N0087, JCTVC-N0088,  and JCTVC-N0236.

The BoG met again on 1 August to review the following:

- 6.4.1 Generic HLS issues

  - JCTVC-N0355, JCTVC-N0356, JCTVC-N0357, JCTVC-N0135, JCTVC-N0065, JCTVC-N0244

- 6.4.2 Random access, layer switching structures and cross-layer alignment of pictures types

  - JCTVC-N0066, JCTVC-N0147

- 6.5.1 SHVC Generic HLS issues

  - JCTVC-N0108

- 6.6.1 Motion and prediction constrained SEI messages

  - JCTVC-N0383

Decision: The JCT-VC endorsed the BoG recommendation of the following:

- Adopt JCTVC-N0244, to use a reserved slice header bit for a POC reset flag, plus signal POC LSB in enhancement layer IRAP pictures from JCTVC-N0065, to maintain POC alignment between layers when IRAP pictures are not aligned.

  - Clarifying note: The bit is not necessary to be present when alignment is established per item 3 below.

- Adopt JCTVC-N0066 version 2 layer-wise start up decoding process.

- Add a flag in VPS extension to indicate if all IRAP pictures are aligned in set of dependent layers, from JCTVC-N0147.

- Add explicit constraint that sample resampling may be done once per enhancement layer picture, and motion field resampling may be done once per enhancement layer picture, from JCTVC-N0108.

  - The BoG noted that the SHVC track may also wish to discuss whether sample prediction and motion field prediction from different reference layers can be performed when coding an enhancement layer picture in the Scalable Main profile.

  - Refinement agreed by JCT-VC: Establish a profile constraint not to allow sample prediction from a different inter-layer reference picture than motion field prediction.

- Adopt JCTVC-N0383 inter-layer predication constraint SEI message, plus modification to motion constrained tile set SEI message to align syntax.

JCTVC-N0135 MV-HEVC/SHVC HLS: Extended maximum number of layers [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)] [late]

(Reviewed Thu. 25th plenary)

This is a follow-up proposal of JCTVC-M0164. To support having more than 64 layers, proposes to use three of the reserved bits in slice headers for extra bits of nuh_layer_id. Approximately 500 layers can be represented with extra 3 bits of nuh_layer_id.

Primarily motivated by "super multiview" application for displays with many views.

Question: Would such an application use one layer per view?

Suggestion to put additional bits into slice header, further investigation necessary whether this is a good place. The current proposal suggests 15 additional bits for layer_id, which according to some experts' opinion might be excessive.

It was remarked that the proposed syntax requires parsing the PPS before being able to access the bits of the extended layer ID, which may be undesirable.

It was noted that we have substantial syntax freedom for non-base layers, although we are constrained by compatibility for layer ID zero.

Near-term profiles likely will not need to support many layers, but it is important to provide extensibility in terms of the number of layers. Some way of allowing extension is highly desirable. Compatibility with existing decoders would be desirable, e.g. for decoding a subset of views.

Another possibility would be to assign one value of layer_id as “reserved for extension”.

Further offline discussion about best way to achieve this

Not clear whether there is any need for immediate action w.r.t. the MV-HEVC draft – likely not.

It was remarked that JCT3V-E0092, JCT3V-E0223 and JCT3V-E0224 are related (not submitted as JCT-VC contributions), where it is suggested to put additional bits into parameter sets. These proposals will also be registered as JCT-VC docs, and were discussed in the context of generic HLS issues.

See BoG report N0374 and related notes.

JCTVC-N0355 / JCT3V-E0092 3D/MV-HEVC HLS: Extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, T. Schierl (FhG HHI)] [late]

(Reviewed Thu. 25th plenary)

This contribution proposes an extension mechanism for layer identifiers to increase the number of supported layers in MV-HEVC and 3D-HEVC. The range of nuh_layer_id is extended by an additional syntax element within the NAL units. The concept of so-called layer clusters allows using the existing extraction processes to select groups of related layers as proposed during the 4th meeting. The syntax has been modified slightly to ensure a backward compatible base layer and to be aligned with MV-HEVC Draft 4.

Again focused on "super multiview" with many views.

Proposes to signal, in VPS, a number of extra bits of layer ID.

It was commented that just having reserved fields might be adequate from a syntax perspective.

It was commented that some form of syntax involving "if( layer_id != 0)" branch in the syntax could be used.

It was commented that using layer ID equal to 63 as an escape code indication could be an alternative way to deal with the layer ID range.

Having an extended NUH for use with some particular profile was also discussed.

It was remarked that having a view-subset decoding capability for a lower-capability decoder is desirable. The proponent suggested that having a "clustering" of views to indicate which subset to decode is also desirable.

It was generaly agreed that support for some extensibility to more views with subset capabilities would be desirable, but we don't want to burden "mainstream" decoders with significant extra work to accomplish that, and we don't want the standard to contain purely "hypothetical syntax" that is normatively forbidden to be used.

See BoG report N0374 and related notes.

JCTVC-N0244 / JCT3V-E0075 MV-HEVC/SHVC HLS: Cross-layer POC alignment [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

(Reviewed Thu. 25th plenary)

In this proposal, a mechanism is proposed in order to ensure that the POC values all pictures of each access unit are the same even when it is allowed that access units for some pictures are IRAP pictures with NoRaslOutputFlag equal to 1 while others are not. Draft text was provided.

The contribution proposes a "poc_reset_flag" syntax flag.

When set to 1, the flag changes the POC of the previously-decoded pictures of the same layer in the DPB, by subtracting an offset from their POC values. It was remarked that this has loss resilience implications (when the picture which causes the POC reset is lost), and further study of that aspect was encouraged (e.g. to add some SEI message to improve the detection and handling of lost pictures).

Base layer decoder would work different from a single-layer (version 1) decoder (which is likely not critical).

The handling of long term pictures was discussed, and this was claimed to be solved.

See BoG report N0374 and related notes.

JCTVC-N0356 / JCT3V-E0223 3D/MV-HEVC HLS: Dependency signalling for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

Related to JCTVC-N0355 / JCT3V-E0092, considered in JCT-3V.

See also BoG report N0374 and related notes.

JCTVC-N0357 / JCT3V-E0224 3D/MV-HEVC HLS: Flexible layer clustering for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

Related to JCTVC-N0355 / JCT3V-E0092, considered in JCT-3V.

See also BoG report N0374 and related notes.

JCTVC-N0267 / JCT3V-E0087 MV-HEVC/SHVC HLS: On changing of the highest layer ID across AUs and multi-mode bitstream extraction [Y.-K. Wang, Y. Chen (Qualcomm)]

(Reviewed Thu. 25th plenary)

This document discusses allowing changing of the highest value of nuh_layer_id across AUs within a CVS (which is currently allowed, due to an adoption intended to allow ARC), and proposes a multi-mode bitstream extraction process. The contribution raises a number of issues relating to this topic.

Related to N0110.

Question: Does “higher layer” usually mean equal or higher resolution? Currently, yes. However, in the case of using the “single_layer_for_non_irap_flag” it might also make sense to allow prediction of lower resolution from higher resolution, which would require defining decimation filters for prediction.

See BoG report N0374 and related notes.

### Random access, layer switching structures and cross-layer alignment of pictures types

(Reviewed Fri. 26th a.m. Track A (GJS).)

See BoG report N0373 (J. Boyce) relating to N0244, N0065, N0084, N0121, N0066, N0090, N0147, and N0195 item 4.

Item 4 of JCTVC-N0195 is related to this agenda category: A restriction on the alignment of IDR and BLA pictures within the same access unit is proposed to be relaxed.

Current text: When the nal_unit_type value nalUnitTypeA is equal to IDR_W_DLP, IDR_N_LP, BLA_W_LP, BLA_W_DLP or BLA_N_LP for a coded picture, the nal_unit_type value shall be equal to nalUnitTypeA for all VCL NAL units of all coded pictures of the same access unit.

Proposal 4a: When the nal_unit_type value nalUnitTypeA is equal to IDR_W_DLP, IDR_N_LP, BLA_W_LP, BLA_W_DLP or BLA_N_LP for a coded picture within a particular access unit belonging to a layer with nuh_layer_id value equal to nuhLayerIdA and has NumDirectRefLayers[ nuhLayerIdA ] equal to 0, all other coded pictures within the same access unit shall have nal_unit_type equal to nalUnitTypeA when they belong to a layer which has nuhLayerIdA as a direct reference layer.

Proposal 4b: When a coded picture within an access unit belonging to a layer with nuh_layer_id value equal to nuhLayerIdA is an IDR picture and has NumDirectRefLayers[ nuhLayerIdA ] equal to 0, all other coded pictures within the same access unit whose layer has nuhLayerIdA as a direct reference layer shall be IDR pictures.

Proposal 4c: When a coded picture within an access unit is an IDR picture and has nuh_layer_id value equal to 0 or has NumDirectRefLayers[nuh_layer_id] equal to 0, all other coded pictures within the same access unit shall be IDR pictures.

JCTVC-N0373 / JCT3V-E0306 BoG report on random access and cross-layer alignment of pictures types [J. Boyce]

Joint BoG with JCT-3V.

(Reviewed Sun 28th Track A (GJS).)

BoG concerned N0244, N0065, N0084, N0121, N0066, N0090, N0147, and N0195 item 4.

- IDRs will not be required to be aligned; to work out how to handle POC alignment between layers

  - Current spec requires POC value alignment

  - Alternative approach is layer-specific POC with alignment of POC differences

- Specification of a layer-wise startup of the decoding process is needed

- For TSA/STSA, if EL picture is a TSA/STSA, the corresponding base layer should also be a TSA/STSA

- It was asked whether IDR/BLA in base layer but not in EL, the IDR in the BL causes marking of the EL pics as unused for reference (in other layers)? No, but need to figure out how/whether this is expressed in the text. (This is different than temporal sub-layer handling.) It was remarked that there may need to be some need to check/fix activation rules.

The track agreed with the BoG recommendations (some details remained open to be worked out).

The BoG recommended further discussion of the following:

- A POC alignment solution when IRAPs not aligned, harmonization of JCTVC-N0244 and JCTVC-N0065, or alternatively to consider relaxing the POC alignment requirement and only require that POC deltas be aligned

- Layer-wise startup of decoding process design, harmonization of JCTVC-N0066 and JCTVC-N0090.

See also JCTVC-N0374 and related notes.

JCTVC-N0065 / JCT3V-E0051 MV-HEVC/SHVC HLS: On IDR picture constraints [M. M. Hannuksela (Nokia)]

In the current draft, IDR is required at all layers if present at any layer. The contribution is to enable a layer switching mechanism.

It is asserted in this contribution that it would be beneficial to enable activation of layer SPSs at access units where some but not all layers contain an IDR picture for example to:

- Provide the encoder the flexibility to change coding modes controlled by syntax elements in the SPS separately for the enhancement layer than for the base layer, but not require the encoder to code an IDR picture across all layers when new active layer SPS is taken into use.

- Enable changing the spatial resolution of the enhancement layer, for example to reflect the resolution of the source pictures for encoding, without a need to code an IDR picture across all layers.

This contribution proposes to relax the constraint on having IDR pictures present on all layers of an access unit as follows:

- When an IDR picture has nuh_layer_id equal to 0, all other pictures in the same access unit shall be IDR pictures.

- IDR pictures with nuh_layer_id greater than 0 may be present in access unit where the picture with nuh_layer_id equal to 0 is a non-IDR picture.

The proposal is reportedly conceptually the same as alternative 1 of JCTVC-M207r1.

The proposed syntax is:

if( nuh_layer_id > 0  | |  
		( nal_unit_type  !=  IDR_W_RADL  &&  nal_unit_type  !=  IDR_N_LP ) )
			slice_pic_order_cnt_lsb
		if( nal_unit_type  !=  IDR_W_RADL  &&  nal_unit_type  !=  IDR_N_LP ) {
			short_term_ref_pic_set_sps_flag
			…

In this proposal, there is no description of how POC should be handled, which must be different than what is in the current draft text. POC is an issue. This issue is related to N0244.

It was noted that a related contribution of the previous meeting M0207 also had another approach, which included introducing a layer-switching picture type.

Additional work is needed to specify how POC would work with this proposal.

See also BoG report N0374 and related notes.

JCTVC-N0084 / JCT3V-E0056 MV-HEVC/SHVC HLS: On various cross-layer alignments [Y.-K. Wang, A. K. Ramasubramonian, J. Chen, Hendry (Qualcomm)]

This document proposes to require cross-layer alignment of leading pictures, TSA/STSA pictures, IRAP picture types, and "GOP structures".

On cross-layer alignment of leading pictures, proposes that "For any two IRAP pictures picA and picB in an AU, let layerA and layerB be the two layers containing picA and picB, respectively, when there exists a picture picC that is in layerA and is a leading picture of picA and there exists a picD that is in layerB and is in the same AU as picC, picD shall be a leading picture of picB."

It was remarked that if the IRAP pictures in different layers are not aligned, it is not clear why there should be a constraint on leading pictures associated with such non-aligned IRAP pictures. This aspect seems to require further thought.

On cross-layer alignment of TSA and STSA pictures, the following is proposed:

- When one picture in an access unit has nal_unit_type equal to TSA_N or TSA_R, any other picture in the same access unit shall have nal_unit_type equal to TSA_N or TSA_R.

- When one picture in an access unit has nal_unit_type equal to STSA_N or STSA_R, any other picture in the same access unit shall have nal_unit_type equal to STSA_N or STSA_R.

It was remarked that the potential relationship between (non-aligned) IRAP in some layer and TSA/STSA in some other layer should be considered, as both of these picture types provide switching points. This aspect seems to require further thought.

On cross-layer alignment of IRAP picture types, the following was proposed:

- When one IRAP picture in an access unit has nal_unit_type equal to IDR_N_LP, any other IRAP picture in the same access unit shall have nal_unit_type equal to IDR_N_LP.

- When one IRAP picture in an access unit has nal_unit_type equal to IDR_W_RADL any other IRAP picture in the same access unit shall have nal_unit_type equal to IDR_W_RADL.

- When one IRAP picture in an access unit has nal_unit_type equal to BLA_N_LP and has nuh_layer_id equal to layerId, any other IRAP picture in the same access unit that has nuh_layer_id less than layerId shall have nal_unit_type equal to BLA_N_LP or CRA_NUT.

- When one IRAP picture in an access unit has nal_unit_type equal to BLA_W_LP or BLA_W_RADL and has nuh_layer_id equal to layerId, any other IRAP picture in the same access unit that has nuh_layer_id less than layerId shall have nal_unit_type equal to BLA_W_LP, BLA_W_RADL, or CRA_NUT.

Note: When one IRAP picture in an access unit has nal_unit_type equal to CRA_NUT, any other IRAP picture in the same access unit must have nal_unit_type equal to CRA_NUT, BLA_W_LP, BLA_W_RADL, or BLA_N_LP.

It was noted that SVC has a constraint that the highest dependency ID (roughly equiv. to layer ID) must be the same for all AUs in the CVS. We do not have that restriction currently in SHVC and MV-HEVC, to allow dynamic-resolution up-conversion (among other possibilities).

See BoG report.

On cross-layer alignment of "GOP structures", the contribution proposes adding the following definitions of key picture and non-key picture:

- Key picture: a picture for which there is no other picture in the same layer that precedes the picture in decoding order and follows the picture in output order.

- Non-key picture: a picture that follows another picture in the same layer in decoding order and precedes the another picture in output order.

and proposes a constraint to require cross-layer alignment of key pictures, as follows:

- When a picture of one layer in an AU is a key picture, all pictures of other layers in the same AU shall be key pictures.

It was commented that this constraint might affect "SHVC as a simulcast mux" usage. It was remarked that fully adaptive GOP variation in different views/layers may be somewhat disallowed by the AU definition already.

It was asked whether such a constraint is really necessary – is really achieving anything.

The topic was agreed to be for further study.

JCTVC-N0121 / JCT3V-E0107 MV-HEVC/SHVC HLS: Random access of multiple layers [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

The contribution proposes:

- Definitions of AU, IRAP AU, and CVS for HEVC layered-extension.

- Constraints of IRAP pictures and definitions of IDR/CRA/BLA access units.

- Constraints and Definitions of TSA/STSA and RASL/RADL access unit.

- A suggested (asserted to be minor) correction of long-term picture definition.

The proposals of definitions are just editorial, but they depend on the behaviour that we plan to enable. Remarks about these definitions included the following:

- It was remarked that the proposed change of definition of AU may neglect back-to-back IDR pictures and may not be necessary.

- For the proposed definition of IRAP AU, there may be a conflict with another proposal – the contribution assumes alignment of IRAP positions, which is not required for CRA IRAPs.

- For the proposed definition of CVS, again there is a need to determine the cross-layer alignment requirements.

- For the proposed definition of IDR and BLA AUs, the suggested interpretation is in line with the current specification intent. However, there are proposals to change this.

For CRA AUs, the contribution proposes to require cross-layer alignment of CRA pictures. This aspect is not just editorial and is different than the current text. A similar alignment constraint is proposed for TSA, STSA pictures.

For RADL and RASL the proposal is only editorial for establishing a definition – not for establishing a new constraint.

Regarding long-term reference pictures, the proposal is just editorial – whether to call something "long term" or not – not a matter of how to use the pictures.

See BoG report N0374 and related notes.

JCTVC-N0066 / JCT3V-E052 MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process [M. M. Hannuksela (Nokia)]

It is asserted that MV-HEVC and SHVC drafts do not allow starting the decoding process from a CRA picture (with nuh_layer_id equal to 0 and a particular POC value), when some of the pictures in the same access unit and with nuh_layer_id greater than 0 are non-IRAP pictures. It is proposed to allow such decoding operation with the following modifications:

- Unavailable pictures with nuh_layer_id greater than 0 are generated for the reference pictures of the first picture in decoding order with that nuh_layer_id value.

- Enhancement layer pictures are output starting from an IRAP picture in that enhancement layer, when all reference layers of that enhancement layer have been initialized similarly with an IRAP picture in the reference layers.

The proposal is reported to be conceptually the same as in JCTVC-M206r1 but the specification text has been updated to be based on to the latest MV-HEVC specification text (JCT3V-D1004).

The proposal is to actually allow the bitstream to start at such a point or could have a BLA (or CRA treated as BLA) with such "step-wise layer recovery behaviour", not just to enable decoders to voluntarily random access to such a point.

In this proposal, there is no description of how POC should be handled, which must be different than what is in the current draft text. POC is an issue. This issue is related to N0244.

It was remarked that this is conceptually aligned with the idea of allowing a version 1 picture to start with a CRA or BLA.

It was agreed that the concept of the proposal is supported in principle, assuming the details can be worked out without too much difficulty.

Additional work is needed to specify how POC would work with this proposal.

See BoG report N0374 and related notes.

JCTVC-N0090 / JCT3V-E0058 MV-HEVC/SHVC HLS: Cross-layer non-alignment of IRAP pictures [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm)]

This document discusses cross-layer non-alignment of IRAP pictures (i.e. not requiring IRAP pictures to be cross-layer aligned) and the necessary changes needed to support it. A new definition for IRAP access unit is proposed: an access unit is an IRAP AU if it contains an IRAP picture with nuh_layer_id equal to 0. Two new NAL unit types are defined to identify cross-layer random access skipped (CL-RAS) pictures that would not be decodable when random accessing from certain IRAP AUs. A modification to the generation of unavailable pictures is proposed to specify the decoding process of such CL-RAS pictures.

The proposal is similar in concept to N0066; see notes for N0066.

Some POC-related aspects seem not yet fully resolved. The proponent suggested for POC alignment to be achieved based on N0244.

JCTVC-N0124 / JCT3V-E0108 MV-HEVC/SHVC HLS: Random layer access [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

The concept of random layer access and random layer access pictures are proposed. The random layer access is to access and successfully decode specific pictures with nuh_layer_id greater than 0 without decoding pictures in lower layers. Two random layer access picture types are proposed. The first one is the single random layer access (SRLA) picture, which has no dependency from any picture in lower layers and can be successfully decoded without interlayer prediction. The other one is the clean random layer access picture (CRLA). A CRLA picture with nuh_layer_id equal to k has no dependency from any picture with nuh_layer_id less than k, and a picture with nuh_layer_id greater than k also has no dependency from any picture with nuh_layer_id less than k. The random layer access is suggested to be useful for fast accessing of specific pictures in specific layers to enable a trick mode play or single picture decoding.

The form of "random access" proposed here does not include the ability to decode pictures in subsequent AUs. It only provides the ability to decode a specific picture in a specific layer and the pictures in other layers within the AU that depend on that layer.

Basically, it is a picture with no inter-layer and no (temporal) inter prediction.

The proposal is to use a NUT for this type of picture.

It was suggested that if such an indicator is needed, to consider using the AUD for this indication rather than a NUT. The group did not see a strong need for the proposed functionality.

No action was taken on this.

JCTVC-N0130 / JCT3V-E0112 MV-HEVC/SHVC HLS: On temporal sub-layer management [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Items 1 to 4 of this contribution seem relevant to this agenda category (item 4 withdrawn per below).

In HEVC layered-extension, several constraints are proposed to allow different frame rates for different layers. In addition, if frame rates for non-base layers are greater than a frame rate of a base layer, the maximum number of sub-layers in non-base layers can be greater than the maximum number of sub-layers in the base layer. Then, it is asserted that signalling the increased number of sub-layers, profile-tier-level information and sub-layer ordering information for additional sub-layers of non-base layer in video parameter set extension is needed. Elements of the proposal included.

- Definition of sub-access unit in which nuh_layer_ids of all VCL NAL units are not equal to 0 is suggested to explain the following proposals.

- The TemporalId values of all pictures in an access unit or a sub-AU shall be the same.

- The TemporalId value of a picture in a sub-AU shall be greater than the TemporalId value of a picture in an AU that has a picture with nuh_layer_id equal to 0.

- A picture belonging to a sub-AU shall be a TSA picture, an STSA picture or a trailing picture. The proponent withdrew this aspect of the proposal.

- The increased number of sub-layers, profile-tier-level information and sub-layer ordering information for additional sub-layers of non-base layer are signalled in the video parameter set extension.

- sps_max_sub_layers_minus1 and profile_tier_level( 1, sps_max_sub_layers_minus1 ) are signaled in an SPS with nuh_layer_id greater than 0 to allow different frame rates for each layer.

Some aspects seemed editorial or already seemed agreed. It did not seem necessary to have the "sub-AU" definition.

The contribution proposes to require the use of temporal sub-layering whenever the picture rate is higher in a higher layer. It was remarked that this constraint might make it easier to detect AU boundaries. However, it was remarked that this would prevent some inter-picture referencing structures that would otherwise be desirable (per previous HM compression analysis) and that the HM CTC does not use temporal layering. So this constraint is undesirable unless really necessary.

It was remarked that we should try, if possible, to allow lower frame rates in higher layers – e.g., for low-frame-rate spatial enhancement of a higher frame rate base layer bitstream. We believe this is not disallowed currently.

Regarding item 5, the proposal is to define the current VPS max sub-layers parameter as relevant only to version 1 sub-layers. Currently we define that parameter to refer to all layers in the bitstream. It was also remarked that the SPS contains a similar parameter relevant to the base layer that seems to accomplish the intended goal of identifying the number of temporal sub-layers in the base layer. Thus no action seems necessary on that item.

Regarding item 6, it was remarked that there is an ability to send profile/tier/level for various operating points in the VPS that may suffice. No action was taken on this.

JCTVC-N0147 / JCT3V-E0085 MV-HEVC/SHVC HLS: On restriction and indication of cross-layer IRAP picture distribution [J. Chen, Y.-K. Wang, K. Rapaka, A.-K. Ramasubramonian, Hendry (Qualcomm)]

This document proposed to have restriction of cross-layer IRAP picture distribution and sequence level indication of cross-layer IRAP pictures alignment.

The contribution proposes to have a restriction of cross-layer IRAP pictures distribution structure; only one of the following IRAP pictures distribution patterns is allowed in any CVS to avoid cases described as useless in the contribution.

- IRAP pictures are cross-layer aligned, that is, when a picture of one layer in an AU is an IRAP picture, all other pictures in the same AU are IRAP pictures.

- Lower layers have more IRAP pictures such that when a picture of a current layer in an AU is an IRAP picture, all pictures in the same AU of the layers, which are specified as direct dependent layers of the current layer in VPS shall also be IRAP pictures.

- Higher layers have more IRAP pictures such that when a picture of a current layer in an AU is an IRAP picture, all pictures in the same AU of the layers, for which the current layer is specified as its direct dependent layer in VPS shall also be an IRAP picture.

Furthermore, this contribution proposes to signal an indication of IRAP distribution pattern in the VPS extension since it’s beneficial for system entities to know the information.

It was remarked that making this SEI/VUI would be adequate, since it does not affect the decoding process. However, the VPS would make this information available at a high level for session negotiation, and we may want to require its presence – we would not ordinarily require SEI to be present.

It was questioned whether we are confident that we really want this kind of constraint. For example, non-alignment could be desirable to avoid excessive bit-rate fluctuations. Also non-alignment could be desirable for flexibility in the "simulcast mux" case.

The contribution also proposes a VPS-level flag to indicate when picture types are fully aligned across layers. It was remarked that the usefulness of the flag would depend on the alignments constraints that we choose to impose.

See BoG report N0374 and related notes.

### Parameter sets

#### General

(Much of this category was assigned to a BoG for review.)

Items 5 to 6 of JCTVC-N0130 seem relevant to this agenda category.

Items 3 and 5 of JCTVC-N0195 are related to this agenda category:

3.	A bitstream constraint related to values for syntax elements direct_dependency_flag[i][j] and max_one_active_ref_layer_flag is proposed.

5.	A bitstream constraint related to the values of syntax elements splitting_flag and dimension_id_len_minus1[i] is proposed.

Part of JCTVC-N0217 is related to this agenda category.

JCTVC-N0085 / JCT3V-E0057 MV-HEVC/SHVC HLS: On parameter sets [Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm)]

(Presented Fri. 26th p.m. Track A (GJS).)

This document includes various proposals and discussions related to parameter sets. Firstly, suggestions and discussions for several general topics are presented. Secondly, some specific technical proposals on vps_extension_offset semantics, signalling of scalability dimension identifier and view identifier, signalling of timing and HRD information in VUI, and signalling of bitrate and picture rate for operation points in VPS are proposed. Lastly, pure editorial improvements for the current MV-HEVC specification are provided. The proposed changes are included in the attachment of this document, with changes marked in relative to JCT3V-D1004v4.

- General:

  - Add a restriction "The value of nuh_layer_id of a VPS NAL unit shall be equal to 0." (for bitstreams conforming to specified proposals, and decoder shall ignore VPS NALUs with other values of nuh_layer_id. Decision: Agreed.

  - To establish that SPS/PPS IDs with different values of nuh_layer_id share the same "value space" such that different layers may share the same SPS/PPS. It is proposed to let them share the same value space. Decision: Agreed.

  - (for discussion) VUI includes information such as sample aspect ratio, over scan, source video format (PAL/NTSC etc., sample value range, source color format), field/frame information, bitstream restrictions (including cross-layer bitstream restrictions). Most of such information, including cross-layer bitstream restrictions, is really not layer-specific and is the same for all layers. Thus it is asserted to be awkward to not have such VUI information signalled in the VPS that naturally applies to all layers. It is suggested to have a general discussion on this, to decide whether something should be done to enable signalling of the above-mentioned VUI information in the VPS. No specific proposal was provided to address this issue. The size of the VPS should be minimized, to enable its use in session negotiation and stream-level signalling. This was agreed to be for further study.

- Semantics of vps_extension_offset: It is proposed to clearly specify that emulation prevention bytes are counted. Decision (Ed.): Agreed

- Signalling of scalability dimension ID and view ID: See BoG report.

- No timing and HRD information in VUI for SPS with nuh_layer_id > 0: Remark: Make it optional? Note that there is a related contribution JCTVC-N0049. Decision (cleanup): Require the flag in the SPS VUI to indicate that this data is not present.

- Signalling of bit rate and picture rate information for session negotiation. Remark: Could this be in SEI? Hypothetically, it could be, but is very-high-level information. Remark: Should we have a section of the VPS extension data that is clearly identified as being for metadata purposes such as we did for VUI at the SPS level. (But we want to make sure the VPS doesn't get bloated.) In principle, it is agreed that we would like to define a VUI-like VPS section that has this in it and put this in it. Revised version of document provided with details.

- Editorial changes – delegated to editors for consideration.

Further discussion for text review Fri. noon. (GJS) after N0085-v2 upload (Section 7):

It was commented that, editorially, the byte alignement should be move out of the syntax structure to the higher level syntax structure (only editorial). This was agreed.

It was commented that the syntax should provide the possibility for data preceding the VPS VUI and the currently-specified content of the vps extension. This was agreed.

Decision: Adopted Section 7 as agreed to be modified.

It was suggested to consider key-length-value coding as in SEI message for the syntax structuring. This is for further study.

JCTVC-N0129 MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

(Presented Fri. 26th p.m. Track A (GJS).)

The syntax and semantics modification related to the syntax element single_layer_for_non_irap_flag and a new syntax inter_layer_prediction_disabled_for_non_irap_flag are proposed for single loop decoding of non-IRAP pictures in HEVC multi-layered extensions. When single_layer_for_non_irap_flag is equal to 1, IRAP access units or pictures may have multiple layers while non-IRAP access units or pictures shall have a single layer, in the proposed text. In addition, the syntax elements that can be inferred without signalling when single_layer_for_non_irap_flag is equal to 1 are proposed to be optionally signalled according to the value of single_layer_for_non_irap_flag. The proposed syntax inter_layer_prediction_disabled_for_non_irap_flag indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

Items 1 and 2 are related to this agenda category:

- The semantics modification of single_layer_for_non_irap_flag to generalize the functionality. In the proposed text, more than two layers are allowed in IRAP access units.

- The syntax elements max_tid_il_ref_pics_plus1[ i ] and default_one_target_output_layer_flag are optionally signalled according to the value of single_layer_for_non_irap_flag.

Regarding item 1, it was questioned whether the more-than-two layers approach is important to try to support. With more than two layers, there would be extra decoding work needed to handle more than two layers. Using the scheme effectively on the server side eems to require more than just bitstream extraction, as the value of a flag must be changed relative to a source bitstream that contains "simulcast with IRAP layer selection". The intent of the flag was more to support ARC rather than trick mode operation. This is for further study.

Item 2 proposes some syntax structure optimization to avoid sending some syntax elements at the VPS level that can be inferred from single_layer_for_non_irap_flag. An inference rule would be needed for when syntax elements are not present. It was remarked that SVC-specific syntax elements should be grouped and this violates that convention. It was also remarked that this is a minor syntax cleanup which does not seem necessary to worry about at this stage. No action.

Item 3 is also syntax structure optimization, but in the slice header.

Later resolved in BoG. See BoG report N0374 and related notes.

JCTVC-N0165 On VPS extension [Y. Cho, B. Choi, M. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

(Presented Fri. 26th p.m. Track A (GJS).)

This contribution proposes restructuring of the current design of the video parameter set extension. Also, a semantics change of default_one_target_output_layer_flag is proposed.

The first part of the proposal concerns grouping syntax elements of the VPS extension according to their function in regard to SHVC, MV-HEVC, or shared, or used for combinations of scalability types.

The categorization applied in the contribution was questioned, and it was suggested that really most syntax elements have shared uses. The proponent, to some extent, was trying to establish some constraints on permitted uses in the way the categorization was performed. These constraints were not explicitly discussed or described in the contribution.

It also does not seem very high priority to make the syntax especially clean at this point in the process, as syntax is not necessarily stable yet and we have other higher priority topics.

The semantics of default_one_target_output_layer_flag was also proposed to be changed.

It was remarked that it is important to note that the semantics are expressed in terms of the "default output layer sets".

It is proposed to change "the highest layer" to "the highest DependencyId".

Current semantics: "default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers."

Proposed semantics: "default_one_target_output_layer_flag equal to 1 specifies that only the layer with the highest DependencyId in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all multiview layers with the highest DependencyId in each of the default output layer sets are target output layers. When NumScalabilityTypes is 1, the value of default_one_target_output_layer_flag is inferred to be 0 for multiview scalability, and 1 for spatial/SNR scalability."

Part of the concern of the proponent is about combinations of scalability types (which may not be defined in the near term, but are envisioned as future possibilities to specify).

It was suggested to consider instead changing the flag to an indicator, and keep the current meaning for two values and prohibit the use of all other values at this time. This is because there may be additional types of scalability in the future in addition to view scalability and spatial/SNR scalability.

It was commented that there is no outright bug, although there may be a lack of flexibility.

We have not really tried to establish a hypothetical combined scalability syntax.

This was agreed to be for further study.

#### Signalling of representation format

JCTVC-N0092 / JCT3V-E0060 MV-HEVC/SHVC HLS: Representation format information in VPS [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), J. Boyce (Vidyo), S. Deshpande (Sharp)]

(Reviewed in Track A Tue. a.m. (GJS).)

This document proposes signalling of representation format information – including spatial resolution, bit depth and chroma format – in the VPS for session negotiation purposes. In addition, a mechanism is proposed to enable update of the representation format information in the SPS.

Revision 1 of this document includes a flag to specify that the number of representation formats is equal to the number of layers, and the representation format for each layer would be associated with a particular format structure. The changes with respect to the original document are presented.

Allows SPS sharing with different picture formats, which we could not previously do.

It was remarked that FLC coding of bit depth as proposed only supports up to 15 bit depth since u(3) coded delta, adjust to be u(4).

Decision: Adopt (with the u(4) adjustment).

JCTVC-N0238 On Source Representation Information Signalling in VPS [S. Deshpande (Sharp)]

(Reviewed in Track A Tue. a.m. (GJS).)

This document proposes syntax and semantics for signalling bit-depth, spatial resolution, color chromaticity and color format related information regarding layers in VPS extension.  It is asserted that this is useful for session negotiation.

Some aspect of the proposal are covered by the action taken on N0092.

Basically the remaining question is whether to add "video signal type" information at the VPS level (in VUI-like section or elsewhere).

Something like this may be desirable, but it requires further study.

JCTVC-N0264 / JCT3V-E0116 HLS: MV-HEVC/SHVC HLS: VPS extension for multi-format [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)] [late]

(Reviewed in Track A Tue. a.m. (GJS).)

This is a follow-up proposal of JCTVC-L0132. In the proposal, the spatial resolutions for spatial scalability, the bit-depth/chroma format/color description for range extension, and view information for multiview extension are signalled in VPS. Providing different video format of different layers is suggested to be beneficial for setting-up inter-layer format conversion for inter-layer prediction, as well as session negotiation when a channel is linked.

Some parts of the proposal have the same concepts (and nearly identical syntax) as N0238 and N0092. See notes for those contributions.

The remaining aspect is for view scalability. See section 6.4.3.4.

#### Efficient parameter set parameters signalling (3)

See BoG report N0374 and related notes.

JCTVC-N0162 MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions [Martin Pettersson, Thomas Rusert]

JCTVC-N0200 On Scaling List Data Signalling [S. Deshpande (Sharp), S. Liu (MediaTek), S. Lei (MediaTek), K. Sato (Sony)]

JCTVC-N0212 SHVC HLS: On Inter Layer Parameter Set [Y. He, Y. Ye, Y. He (InterDigital)]

JCTVC-N0371 MV-HEVC/SHVC HLS: On Scaling List Data Signalling [S. Deshpande (Sharp), M. Pettersson (Ericsson), S. Liu (MediaTek), T. Suzuki (Sony)] [late]

#### ViewId signalling

See also JCTVC-N0264 / JCT3V-E0116.

Item 3 of JCTVC-N0085 is related to this agenda category. However, it is a matter of view scalability rather than SHVC; see JCT-3V report.

JCTVC-N0051 MV-HEVC/SHVC HLS: ViewId and view position index [J. Boyce (Vidyo)]

A matter of view scalability rather than SHVC; see JCT-3V report.

JCTVC-N0067 MV-HEVC/SHVC HLS: on associating ViewId with nuh_layer_id and camera position [M. M. Hannuksela (Nokia), L. Chen (USTC)]

A matter of view scalability rather than SHVC; see JCT-3V report.

JCTVC-N0299 MV-HEVC/SHVC HLS: On use of splitting_flag with flexible coding order [Andrey Norkin, Thomas Rusert (Ericsson)] [late]

Primarily a matter of view scalability rather than SHVC (although does affect the SHVC VPS extension semantics mapping of dimension ID to dependency ID); see JCT-3V report.

### Signalling for inter-layer dependency and inter-layer prediction reference

#### Sequence-level inter-layer dependency signalling

(Reviewed in Track B (chaired by JRO) on Thu 25th p.m.)

JCTVC-N0058 MV-HEVC/SHVC HLS: On dependency type [T. Ikai, T. Uchiumi (Sharp)]

The contribution presents a direct_dependency_type representation which is assertively more effective for many cases. In the proposed representation, the case in which direct_dependency_type is equal to 0, represent sample and motion dependency.  With the proposed change, direct_dependency_type can be exempted by setting direct_dep_type_len equal to 0 for the case sample and motion dependency is used for all layers.

The proposal would add another type “no dependency” with dependency_id=3, and shift the “motion+sample” from id=3 to the new id=0.

The benefit in terms of bit rate saving would be minor in current test conditions, but it is claimed that the benefit might be higher with more layers.

One expert mentions that “no dependency” can already be signalled differently in the current spec.

No action was taken on this.

JCTVC-N0132 MV-HEVC/SHVC HLS: On interlayer prediction type [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

In order to enable an independent configuration of each inter-layer dependency type and a bit-efficient signalling, a bit-mask prediction_type_mask[ i ] and direct_prediction_type_flag[ i ][ j ][ k ] are signalled in VPS extension similar to scalability_mask[ i ]. In addition, a proposed flag motion_only_decoding_flag indicates that only the motion vector related data are needed for inter-layer prediction without the full decoding of pixel data. It removes unnecessary decoding process of unused decoded pixel data.

The proposal would save bit rate in case of sequences where inter-layer motion prediction is not used at all (not in current CTC)

Further, a flag is proposed at slice level to indicate that only motion dependency is used. This is intended for a case that only motion information from the base layer is used for the inter-layer prediction. No inter-picture prediction would be performed for this case (i.e. also no TMVP, which is not fully clear from the semantics description in the contribution). One expert suggests that this flag is rather a kind of indication metadata that could be put into an SEI message (does not have impact on normative decoding process).

Adds more flexibility, but benefit not obvious, and the suggested mask makes the parsing slightly more complicated.

No action was taken on the prediction_type_mask; further study was suggested on the motion_only_decoding_flag as an SEI message. Further study was also suggested regarding whether this could have a possible impact on saving DPB memory.

- JCTVC-N0383 Indication of inter-layer and motion constrained prediction constraints [K. Ugur, M. M. Hannuksela (Nokia), K. Suehring, R. Skupin, Y. Sanchez (FhG HHI), K. Rapaka, J. Chen (Qualcomm), C. Auyeung, S. Hattori (Sony)]

See BoG report N0374 and related notes.

#### Sub-layer related inter-layer prediction signalling (4)

(Reviewed in Track B (chaired by JRO) on Thu 25th p.m.)

JCTVC-N0060 MV-HEVC/SHVC HLS: TemporalID alignment and inter-layer prediction restriction [T. Ikai, Y. Yamamoto (Sharp)]

The contribution proposes to introduce a flag inter_layer_tid_alignment_flag in VPS to indicate if TemporalID is aligned across layers.  It is also proposed that if the inter_layer_tid_alignment_flag is 1 and max_tid_il_ref_pics_plus1[0] is less than 7, max_tid_il_ref_pics_plus1[0] is used for common value of max_tid_il_ref_pics_plus1 across layers and inter-layer related signalling in slice segmentation header is not sent if the TemporalID of a layer is larger than the common max value.

Additionally in case the above proposal (Option1) is not agreed, it is proposed as Option2 to include a syntax general max_tid_il_ref_pics_plus1 for common value of max_tid_il_ref_pics_plus1

Option 1: In case of base layer restriction on tid, inherit same restriction over all layers (saves the sending of the max_tid flag, and some other syntax in slice header for dependent layers)

Question: What is the intention of this?

It would restrict the encoder flexibility; bit rate saving likely not large.

The semantics for deriving the parameters in the contribution seems to be incomplete.

Option 2: Max_tid is optionally inherited to the enhancement layers, syntax in slice header is inherited similar as in option 1.

No action was taken on this.

JCTVC-N0120 MV-HEVC/SHVC HLS: Signalling for Inter-layer prediction indication [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

In the previous meeting, the method that controlling the use of inter-layer prediction based on the temporal sub-layer at sequence level was adopted in SHVC / MV-HEVC draft text. This contribution propose a present flag indicating whether the use of inter-layer prediction is controlled based on temporal sub-layer or not.

Two alternatives: Common signalling of max_tid_..._present_flag for all layers, or individually for each layer

Main intent is bit rate saving (3 bits per layer at sequence level).

The decoder operation is becoming slightly more complex, as it needs to be checked whether the flag is present or not, and whether the information is inferred or to be parsed (this also applies to N0060 option 2)

Several experts expressed support for JCTVC-N0120 “alternative 1”, common signalling of the “present flag” for all layers, and max_tid=7 is assigned for all layers. However, the semantics part of the text in the contribution seems to require more investigation – new version to be uploaded. See BoG report N0374 and related notes.

JCTVC-N0109 MV-HEVC/SHVC HLS: Signalling for sub-layer dependency [V. Seregin, Y.-K. Wang (Qualcomm)]

This contribution proposes to further classify the sequence-level inter-layer dependency, which is currently derived from the syntax element direct_dependency_flag[ i ][ j ], to be sub-layer specific, by utilizing information carried by the syntax element max_sublayer_for_ilp_plus1[ i ]. Specifically, the value of NumDirectRefLayers that is currently defined for each layer is defined for each sub-layer of each layer, thus becoming a two-dimension array. The sub-layer classified variable is then applied in reference picture marking and picture-level inter-layer reference picture signalling, in order to mark certain pictures as "unused for reference" earlier and release the picture buffer for storing other decoded pictures or to avoid sending of unnecessary bits in slice headers for signalling of pictures used for inter-layer prediction by each picture.

Main intentions: Earlier identification of pictures as “unused for reference” (by changing the derivation process, also increasing the storage for NumDirectRefLayers); saving of bit rate.

Max additional memory would be 64x6 bytes at sequence level.

No clear evidence was provided on how large the benefits would be (e.g. a case of dependency conditions where the earlier identification of “unused for reference” would save DPB memory, reduction of bits). Perform offline analysis, update input contribution. See BoG report N0374 and related notes.

JCTVC-N0196 On Sub-layer Non-reference Pictures Indication for Inter-layer Prediction [S. Deshpande (Sharp)]

This document proposes syntax and semantics in VPS for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction. A bitstream conformance constraint is proposed based on this indication which gets used in the decoding process for inter-layer reference picture set. Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

In r1 revision some changes to the proposed specification text are made with no change to the proposed design.

The goal is similar with N0109 – DPB memory saving. N0196 further allows earlier identification of sub-layer non-reference pictures not used for inter-layer reference. Explicit signalling is used. As a possible intention, this could also be used as a bit-stream restriction to limit worst-case decoder complexity.

Variant 1 only performs signalling for highest tid, variant 2 performs signalling for each tid up to maximum.

One expert mentioned that a similar benefit could already be achieved by encoders appropriately using temporal scalability (unless the number of 7 sub-layers would not be sufficient).

This example however only shows a case where the benefit can be drawn from temporal scalability. Provide more examples where it becomes evident that the additional syntax is necessary. See BoG report N0374 and related notes.

#### General inter-layer RPS signalling and derivation

The following item of the JCTVC-N0057 is relevant for the agenda item: RPS includes only pictures whose direct_dependency_type shows sample prediction dependency.

The following aspect (item 3) of JCTVC-N0129 is related to this agenda item: Signalling of inter-RPS syntax can be skipped when single_layer_for_non_irap_flag is equal to 1 and the current picture is not an IRAP picture.

See BoG report N0374 and related notes.

JCTVC-N0059 MV-HEVC/SHVC HLS: On slice segment header extension [T. Ikai, T. Uchiumi (Sharp)]

The aspect for slice-based inter-layer prediction signalling is relevant for this agenda item.

JCTVC-N0081 MV-HEVC/SHVC HLS: On inter-layer prediction related syntax [J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony)]

The following aspect of JCTVC-N0107 is related to this agenda item: This contribution proposes removal of the slice header syntax element inter_layer_sample_pred_only_flag.

JCTVC-N0118 MV-HEVC/SHVC HLS: On Inter layer Prediction Signalling [J. Chen, Y. Chen, Hendry, Y.-K. Wang, K. Rapaka (Qualcomm)]

JCTVC-N0154 MV-HEVC/SHVC HLS: On signalling of inter-layer RPS in slice segment header [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-N0195 / JCT3V-E0078 Comments On SHVC and MV-HEVC [S. Deshpande (Sharp)]

Items 1 and 2 of this contribution are related to this agenda category:

- In slice segment header signalling if NumActiveRefLayerPics is equal to NumDirectRefLayers[nuh_layer_id] then the inter_layer_pred_idc[i] syntax elements are not signalled as they can be inferred.

- A gating flag is proposed for signalling syntax elements related to inter-layer prediction in slice segment header.

JCTVC-N0217 MV-HEVC/SHVC HLS: On SHVC High Level Syntax [Y. He, X. Xiu, Y. Ye, Y. He (Interdigital)]

See BoG report N0374 and related notes.

JCTVC-N0131 MV-HEVC/SHVC HLS: On interlayer reference picture set [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

#### Signalling of TMVP and collocated pictures (8)

(Reviewed Thu. 25th p.m. in Track B (JRO).)

JCTVC-N0057 MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic [T. Ikai, T. Uchiumi (Sharp)]

In the current spec, any inter-view prediction pictures are included in RPS irrelevant to dependency type and the RPS includes motion only dependent pictures. Because motion only dependent pictures are not used as the reference samples, the ref_idx which used to indicate the picture of the reference sample is not efficient due to the non-used picture existence in the reference picture list.  Alternative colPic indication syntax for temporal motion vector derivation by layer_id has been introduced to support the motion only dependent picture which might be not included in the reference picture list, so there is inefficiency due to inclusion of motion only dependent picture in RPS. However this issue has not yet been addressed.

Similarly, any inter-view prediction pictures with motion dependency are included as candidates for the temporal picture indicated by the alternative colPic indication. That means the colPic can be selected by either the conventional syntax based on ref_idx or the alternative syntax based on layer_id.  Because of this redundancy, the layer_id base indication is inefficient. This proposal proposes that 1) RPS doesn’t include motion only dependent picture and 2) alternative colPic indication is only used in the case that the colPic is motion only dependent picture (i.e. not included in RPS).

The intention is to save memory for the samples of pictures where only motion-related inter-layer dependency is active. However, the current version of the contribution does not fully specify the handling of RPS. Also not clearly specified that TMVP would not be used.

It is also mentioned that the current design of putting all pictures into RPS has been made out of several reasons, including error resilience in case of losses.

General impact on consistency of the spec not fully clear.

Further study was suggested on the first part (RPS not to include reference pictures that are only used for motion prediction).

The second part was presented in the context of JCTVC-N0059. See notes for BoG report N0374.

Some aspects of JCTVC-N0059 are relevant for this agenda item (on collocated pictures).

JCTVC-N0064 MV-HEVC/SHVC HLS: on storage of motion fields [M. M. Hannuksela (Nokia)]

(Initial review Thu. 25th p.m. in Track B (JRO).)

The proposal consists of two parts:

- collocated_picture_constraint_flag in the SPS extension indicating, when equal to 1, that no TMVP is used for pictures within the same layer.

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

Basic idea is using a new flags to indicate that TMVP is not used for subsequent pictures within the same layer. This allows a decoder to infer whether storage of MV is necessary or not (current version 1 TMVP disable flag does not care about inter-frame or inter-layer usage).

No impact on normative decoding process. Could also be defined as an SEI message.

(Note: The current SPS flag disabling TMVP in version 1 has impact on decoding, e.g. MV prediction).

The current spec does not normatively specify the usage of memory for the MV data. Therefore, a better place for this would be an SEI message.

(Further discussed in Track A Tue. 30th a.m. (GJS).)

Question: How much data would be saved in a decoder by this knowledge? Answer: Perhaps a few percent – a motion field at 16x16 resolution. Then why bother with it?

If there was a profile constraint rather than just metadata, it might be more useful from the decoder perspective, as it would make decoders easier to make. If it is just metadata, a decoder would be required to still operate when the usage is not constrained.

This was agreed to be for further study.

JCTVC-N0112 MV-HEVC/SHVC HLS: High-level syntax for temporal motion vector prediction [Hiroya Nakamura, Motoharu Ueda, Hideki Takehara, Shigeru Fukushima (JVC Kenwood)]

This contribution proposes to add a syntax element(s) to distinguish the following conditions for temporal motion vector prediction in sequence level along with alt_collocated_indication_flag in slice level.

- no temporal motion vector prediction

- temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

- temporal motion vector prediction from only reference layer

- temporal motion vector prediction from only the same layer

The suggested change at sequence level is similar to N0064, but can additionally differentiate the cases whether TMVP is used only over the temporal sequence of the same layer, or both (which can however anyway be inferred from NumActiveMotionPredRefLayers syntax element in case of N0064); generally N0064 as SEI message would be preferred.

Another suggestion in N0112 is a change at slice level, signalling the alt_collocated_xx elements only for case b), whereas currently it is for cases b) and c) (in cases a and c, it is suggested to be inferred). This may only have minor impact on bit rate savings. It is also mentioned that during the last meeting a discussion (on JCTVC-M0457) was performed whether those syntax elements in slice header might imply a low-level change, since the elements would be used at PU level. See also below on N0107 etc.

JCTVC-N0102 MV-HEVC/SHVC HLS: On alternative collocated picture [Y. Lin (HiSilicon), J. Zan (Huawei)]

This contribution discusses issue on using alternative collocated picture which was adopted in last meeting. TMVP derivation process in HEVC specification exploits not only a collocated picture but also a reference picture list indicator (i.e. collocated_from_l0_flag) which is used in collocated MV derivation. However the adopted method of using alternative collocated picture only signals the collocated picture, but not the reference picture list indicator. Therefore it seems not work well. Two solutions on the issue are proposed. The first solution proposes to avoid usage of the reference picture list indicator in collocated MV derivation by changing the corresponding condition checking. In this way the collocated MV can be derived regardless of the reference picture list indicator when alt_collocated_indication_flag is enabled. The second solution is to additionally signal the reference picture list indictor for the alternative collocated picture.

It is agreed that the current solution around alt_collocated_indication_flag has a problem that when the low-delay condition is not true there is no means to choose between list 0 and list 1, whereas the collocated_ref_layer_idx is always put into list 0.

The first suggested solution is a potential low-level change. Beyond the current WD, which changes 8.5.3.2.7, it is further suggested to change 8.5.3.2.8 (adding a condition about alt_collocated_indication_flag in the derivation process of collocated MV at PU level).

The second suggested solution (adding collocated_from_l0_flag for B slices in case of inter-layer) is asserted to solve the problem.

JCTVC-N0107 MV-HEVC/SHVC HLS: On collocated picture indication and inter_layer_sample_pred_only_flag [V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm)]

(Initial review Thu. 25th p.m. in Track B (JRO).)

This contribution proposes removal of the slice header syntax elements alt_collocated_indication_flag, collocated_ref_layer_idx, and inter_layer_sample_pred_only_flag, and the corresponding decoding processes. The first two syntax elements involve low-level decoding process changes, including the temporal motion vector prediction, which are not allowed for MV-HEVC and SHVC. The last syntax element is used to enable avoiding inclusion of intra-layer pictures into reference picture lists, which is already enabled in RPS signalling by setting those pictures to be not used for reference by the current picture.

About alt_collocated_indication_flag, collocated_ref_layer_idx:

The dispute on whether the inclusion of the syntax elements in slice header would imply a low-level change or not is still open.

One expert mentions that using the existing collocated_ref_idx (as suggested in N0107) instead of collocated_ref_layer_idx would not have an implication on the length of the ref pic list, unless any change would be made to exclude pictures which are used only for motion prediction from the ref pic list.

About inter_layer_sample_pred_only_flag: It is suggested to achieve the same functionality by setting used_by_curr_pic_flag, used_by_curr_pic_s0_flag, and used_by_curr_pic_s1_flag equal to 0 for each entry of a reference picture set in reference picture set signalling. This is asserted to be correct, and suggested to be embraced by several experts. N0081 suggests something similar.

(Further discussed in Track A Tue. 30th a.m. (GJS).)

We don't really know how much benefit there is from the alt_collocated_indication_flag, collocated_ref_layer_idx. The current draft is a bit mixed up in regard to this aspect.

It was suggested that N0185 is related.

There seems to be no need for inter_layer_sample_pred_only_flag in our current design.

Decision: Remove the slice header syntax elements alt_collocated_indication_flag, collocated_ref_layer_idx, and inter_layer_sample_pred_only_flag.

JCTVC-N0119 MV-HEVC/SHVC HLS: On collocated picture indication [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

In SHVC/MV-HEVC draft text, reference layer picture can be used as a collocated picture for temporal motion vector prediction. For this, there are two syntax elements ‘alt_collocated_indication_flag’ and ‘collocated_ref_layer_idx’ in the slice segment header. However, when ‘alt_collocated_indication_flag’ is equal to 1 and current slice is B slice, since current draft text does not specify the syntax element ‘collocated_from_l0_flag’, it is not possbile to know whether a collocated picture is derived from reference picture list 0 or reference picture list 1. Also the motion vector of collocated prediction block cannot be derived without low-level changes. This contribution proposes two alternatives to solve these problems.

First solution similar to N0102 (adding collocated_from_l0_flag for B slices in case of inter-layer references), but puts this syntax element prior to the alt_collocated_indication_flag; second solution infers the collocated_from_l0_flag.Proponents themselves indicate that first solution would be more consistent.

JCTVC-N0185 On low-delay flag checking process of SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital), Y. Lin, X. Zheng, X. Chen (HiSilicon)]

(Initial review Thu. 25th p.m. in Track B (JRO).)

In this contribution, the low-delay checking process in SHVC Test Model (SHM2.0) is modified to reportedly improve the efficiency of temporal motion vector prediction (TMVP) for enhancement layer (EL) coding. More specifically, the low-delay flag is set to true if the inter-layer prediction (ILP) picture is used as the co-located picture for EL TMVP derivation, such that the motion vector (MV) of the co-located prediction unit (PU) always comes from the same reference picture list of the target MV of the current PU for better TMVP prediction. The proposed modification is a slice-level change as the low-delay flag is determined per slice and referred to by all the PUs of a coded picture. Experimental results show that the proposed change reportedly achieves 0.4%, 0.4% and 0.5% BD-rate savings on average for 2x, 1.5x and SNR scalability in RA configuration, compared to the anchors of SHM2.0.

Suggestion to use the LD flag to differentiate between l0 and l1 candidate in case of inter-layer reference. A similar approach was suggested in JCTVC-M0065, which had been commented as follows:

“The low-delay flag is determined at the block level, therefore it would not be applicable to the refidx approach. However, whereas HEVC spec defines it this way, the conditions would not change for all blocks of the slice. It may be implementation specific, whether this can be asserted as a low-level change or not. Further evidence should be provided.”

Further discussion was necessary on meaning of “low-level change” in the overall context of the alt_collocated_indication_flag, affecting potential decisions on N0185, N0119, N0112, N0102. Alternatively, the solution suggested in N0107 could solve the problem in a reasonable way.

(Further discussed in Track A Tue. 30th a.m. (GJS).)

The concern was expressed that the spec describes the modified part of the process as a low-level operation, and if it is implemented that way, there would need to be a low-level change made within the decoding process (although this could be implemented as a high-level check, and the reference software does a high-level check).

Really, from the spec perspective, the decoder can do whatever it wants as long as it produces correct picture output. The spec does not specify how a decoder actually operates internally.

One participant indicated that this change would not be compatible with using their current product to construct a HLS-only-modified SHVC decoder. Others indicated that this might be true of other implementations as well, but they were not sure and might want to have time to check to find out.

The intent is to not change the decoding process at all – at least without strong justification.

As an SHVC proposal, we were not inclined to take action on this.

JCTVC-N0260 Cross check report of JCTVC-N0185 on low-delay flag checking process of SHVC [K. Misra, A. Segall (Sharp)] [late]

#### Reference picture list construction

See BoG report N0374 and related notes.

JCTVC-N0082 MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

JCTVC-N0361 Cross-check of JCTVC-N0082/JCT3V-E0055: On initialization process of reference picture lists for HEVC extensions [A. K. Ramasubramonian (Qualcomm)] [late]

JCTVC-N0095 MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, L. Zhang (Qualcomm)]

JCTVC-N0216 MV-HEVC/SHVC HLS: On Reference Picture List Modification [Y He, X. Xiu, Y Ye (InterDigital)]

JCTVC-N0316 MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction [Andrey Norkin, Usman Hakeem (Ericsson)] [late]

JCTVC-N0362 Cross-check of JCTVC-N0316/JCT3V-E0239: Initial inter-layer reference picture list construction [A. K. Ramasubramonian (Qualcomm)] [late]

#### Management of resampled or filtered inter-layer reference pictures

See BoG report N0374 and related notes.

JCTVC-N0128 MV-HEVC/SHVC HLS: Reference picture marking and picture removal [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

See also section 6.4.6.3.

JCTVC-N0282 MV-HEVC/SHVC HLS: On handling of filtered inter-layer reference [P. Lai, S. Liu, S. Lei (MediaTek)]

### Tiles and parallel processing

See BoG report N0374 and related notes.

JCTVC-N0158 MV-HEVC/SHVC HLS: Bitstream restrictions on tiles and wavefronts across layers [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, J. Chen (Qualcomm)]

See BoG report N0374 and related notes.

JCTVC-N0159 MV-HEVC/SHVC HLS: Parallel Processing Indications for Tiles in HEVC Extensions [K. Rapaka, X. Li, J. Chen, W. Pu, Y.-K. Wang, M. Karczewicz (Qualcomm)]

(Reviewed Sun. 28 Track B (AS).)

Initially reviewed in Track B as it was related to another Track B proposal.

It was asserted that HEVC supports tile based coding to enable parallel processing. In this contribution, some problems are discussed related to parallel processing of tiles across layers and two methods are proposed to address the problems and to enable more friendly parallel processing of tiles across layers. The first method proposes an indication of an encoder constraint on inter-layer prediction for the samples of the enhancement layer picture that lie across the tile boundaries. The second method proposes a tile based up-sampling. The third method proposes an indication if inter-layer prediction is used for a particular tile in the enhancement layer picture.

It was reported that the difference between the first and third method is that the third method supports indicating the constraint on a tile by tile basis.

It was remarked that this problem may be conceptually similar to the motion constrained SEI message.

See also BoG report N0374 and related notes.

JCTVC-N0160 MV-HEVC/SHVC HLS: On signalling of offset delay parameters and tile alignment [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

See BoG report N0374 and related notes.

JCTVC-N0199 On Tile Alignment [S. Deshpande (Sharp), K. Misra (Sharp)]

See BoG report N0374 and related notes.

JCTVC-N0069 and JCTVC-N0087 (in SEI category) are also related.

### Hypothetical reference decoder (HRD) and DPB management

This section not reviewed in BoG N0374.

Later discussed Thu 1st (GJS).

#### General principles of HRD and DPB operation

Some aspects of JCTVC-N0172 relate to this agenda category.

Further study in AHG work was planned.

See also JCTVC-N0290.

JCTVC-N0048 Extensions to support layer addition and removal, access unit structure and changes to HRD model in scalable HEVC [S. Narasimhan, A. Luthra (Arris)]

(Reviewed Sun. 28th a.m. Track A (GJS).)

The current VPS structure in HEVC is proposed to be changed to signal removal or addition of layers. This contribution recommends addition of a syntax element in VPS to signal presence or absence of layers that are not included in the VPS so that the VPS does not need to be altered at re-multiplexing or re-distribution points (see JCTVC-K0206). In addition, this contribution suggests a change to the access unit structure in SHVC.

The contribution also suggests alignment of the systems use case (which requires base layer and enhancement layer combinations to be transmitted in separate streams) with extensions to the HRD model that are asserted to be needed to align the HRD and STD models.

In the discussion, the following comments were recorded:

- The contribution considers sending BL and EL separately, then reassembling. It also considers sending only the BL to some decoders.

- The contribution proposes potentially structuring an AU so that the NALUs of each layer are clustered together.

- It was commented that it is now allowed for PSs (including VPSs) and SEI NALUs to be interleaved between VCL NALUs, so the envisioned AU structure is already OK in principle. We may need to check how AUD works.

- The number of layers actually present is also allowed to be less than the maximum indicated value (and for layers to appear and disappear and reappear).

Further study in an AHG was suggested.

JCTVC-N0049 Consideration of buffer management issues and layer management in HEVC scalability [S. Narasimhan, A. Luthra (Arris), K. Sato (Sony Corp), A. Tabatabai (Sony Electronics)]

(Reviewed Sun. 28th a.m. Track A (GJS).)

It was reported that the buffer management in AVC based scalability (SVC and MVC) required an extension to system STD buffer model and introduced an additional layer of complexity to re-purposing and re-distribution equipment. The extensions required management of both the base layer buffer and buffer with base and enhancement layer/layers at the same time in both transmission and decoding equipment. This contribution suggests two options to reduce the buffer model complexity in HEVC based scalability.

A "scalability information" SEI message was described in the proposal, in a similar spirit as from SVC.

Layer-specific HRD information was proposed as part of this SEI.

Some VPS extension syntax was also described as an alternative to the SEI approach.

However, the scheme was not fully worked out in all detail.

In the discussion, the following comments were recorded:

- It was commented that the layer-specific HRD envisioned in this contribution would require a substantial amount of work to define appropriately. Our current draft uses a layer set combined HRD operation.

- At the moment, we only have a "concept-level" understanding of what should be done to specify layer-specific HRD operation.

- The majority of information that was carried in the previous scalability information SEI message is now carried in the VPS in the current SHVC design.

- We should decide whether we want to specify (additionally or alternatively or as a replacement) a layer-specific HRD model.

- There were mixed opinions about the desirability of retaining the current combined model.

- Another contribution, N0290, considers the combined-vs-separate HRD model issue in the context of ultra-low-delay.

See also notes for N0048.

It was agreed to plan for an AHG on HRD (incl. DPB).

JCTVC-N0093 / JCT3V-E0061 MV-HEVC/SHVC HLS On DPB operations [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

(Reviewed Sun. 28th a.m. Track A (GJS).)

This proposal presents several methods to change the specification of picture-based removal of decoded pictures from the DPB. A set of target output layers is used to specify the operation point. The DPB is partitioned into sub-DPBs based on spatial resolution, bit depth, and color format. The sub-DPB sizes are signalled in the VPS for the various output layer sets. The management of the DPB is proposed changed to operate on the sub-DPBs.

In revision 1 of this proposal, aspects related to parsing dependency and DPB-related parameters (reorder and latency) as described in N0091/E0059 are included in the attachment document. The revised attachment document also modifies the picture output process in the DPB to be dependent on the reorder and latency parameters of the layer that has the highest layer ID amount the set of target output layers.

It is proposed to associate an operating point with a target output layer set and a temporal ID, especially for MV-HEVC, but proposed to be generically defined. We have something like this in the prior MVC specification. The VPS would identify the set of output layer sets, and an index (by external means or default) would identify the selected target output layer set.

In the current spec, each layer has a conceptually separate DPB, but no means by which to identify the capacity of the DPB (as with MaxDpbSize / max_dec_pic_buffering) for output layer sets.

It is proposed for all layers that have the same resolution, chroma format and bit depth to share the same "sub-DPB". MVC operated this way, but never needed more than one combination of resolution, chroma format and bit depth.

One participant indicated a preference to instead have each layer have its own separate DPB without sharing of DPB capacity across layers.

Some action is needed (at least eventually).

Regarding bumping, the contribution considers max latency and max reordering, and how these parameters should work with layers. It suggests that these parameters should perhaps be associated with output layer sets.

It may matter whether we should assume that the highest layer has the highest frame rate. However, we have agreed that we don't want to require that. A mentioned possibility is to take parameters from the layer that has the highest frame rate (if that can be identified).

On Thu 1st p.m., a new revision of the contribution was discussed with a joint starting point suggestion.

Further study in AHG work was suggested.

JCTVC-N0172 MV-HEVC/SHVC HLS: Layer-wise DPB operation and size indications [M. M. Hannuksela (Nokia)]

Further study in AHG work was suggested.

JCTVC-N0198 On DPB Operation [S. Deshpande (Sharp)]

Further study in AHG work was suggested.

#### DPB parameter signalling

JCTVC-N0056 MV-HEVC/SHVC HLS: On inter-layer reference picture output marking [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

Was discussed in BoG. See BoG report N0374.

JCTVC-N0091 MV-HEVC/SHVC HLS: DPB-related parameters in SPS and VPS [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

Further study in AHG work was suggested.

JCTVC-N0127 MV-HEVC/SHVC HLS: On decoded picture buffer [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Further study in AHG work was suggested.

JCTVC-N0157 MV-HEVC/SHVC HLS: On signalling of sps_max_sub_layers_minus1 [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

Further study in AHG work was suggested.

JCTVC-N0197 On Signalling DPB Parameters in VPS [S. Deshpande (Sharp)]

Further study in AHG work was suggested.

#### Other HRD related aspects

(Discussed Thu 1st (GS).)

The following aspect of JCTVC-N0128 is related to this agenda item: A picture removal process from a decoded picture buffer (DPB) is proposed. When the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied. The purpose of the second item is to avoid the removal of inter-layer reference pictures before inter-layer prediction.

In the discussion of this aspect of JCTVC-N0128, it was discussed whether no_output_of_prior_pics_flag and the similar variable can be different in different layers. It seemed agreed that the flag effect should be layer-specific. The current picture should obviously not be discarded, and the value 0 for setting of fullness in the contribution seems potentially incorrect (e.g. if there are other-layer pictures retained).

Further study in AHG work was suggested.

JCTVC-N0062 MV-HEVC/SHVC HLS: Access unit boundary detection [M. M. Hannuksela (Nokia)]

(Thu 1st (GS).)

It has reportedly been confirmed in the previous JCT-VC meeting that it is desirable to allow access units where the base layer picture is not present – for example to enable a base layer @ 30 Hz and a spatial or quality enhancement layer @ 60 Hz.

If there is no NAL unit that starts a new access unit (e.g. an access unit delimiter) present and also if there is no base layer picture present in the access unit, it is asserted that HEVC v1 decoders may consider the following coded enhancement layer pictures as a part of the previous access unit, while SHVC/MV-HEVC decoders are intended to consider them as part of a new access unit. Consequently, as the CPB operates on access unit basis, it is asserted that the HRD parameters may become ambiguous and may be interpreted differently by HEVC v1 decoders and SHVC/MV-HEVC decoders.

It is proposed to require the presence of the access unit delimiter NAL unit when there is no base layer picture present in the access unit.

Regarding how v.1 decoder sees the enhancement layers.

Further study in AHG work was suggested.

JCTVC-N0110 SHVC HLS: Earlier DPB clearing for adaptive resolution change [V. Seregin, Y. Chen, Y.-K. Wang (Qualcomm)]

(Thu 1st (GS).)

Further study in AHG work was suggested.

JCTVC-N0290 Ultra-low delay with SHVC, MV-HEVC and 3D-HEVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (HHI)]

(Thu 1st (GS).)

Plan further study in AHG.

Also, regarding DU order, proposes to allow an EL DU to precede a BL DU if it does not use the region of the BL picture contained in the BL DU for reference.

This was proposed previously.

It was remarked that this seems necessary to enable scalability to function properly in ultra-low-delay operation.

A decoder that is not designed to be able to decode part of picture and then decode part of some other layer, then come back and decode more of the base layer would need to have buffering to re-order the DUs into whole-picture order.

The concept was generally agreed in spirit, but some participants were unsure that all issues had been fully thought through. We were inclined to adopt, but it was felt that some further study is needed to confirm that this does not introduce problems. Further study was needed to confirm, with an intent to adopt if not found problematic.

## HL syntax in SHVC

### Generic HLS issues

(This agenda category discussed Thu 25th p.m. in Track A (GJS).)

JCTVC-N0108 SHVC HLS: Avoid resampling process for pictures used only for inter-layer motion prediction [V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm)]

In SHVC, the picture and the motion field of an inter-layer reference pictures may both need to be resampled for the prediction of a current picture. However, it is not always necessary to invoke them jointly since there might be cases when that it is not required to add the resampled picture from the reference layer as a reference picture of the current picture. This contribution proposes skipping the invocation of the resampling process for the inter-layer reference pictures used only for inter-layer motion prediction.

The change is basically editorial, except for the fact that we limit complexity with a constraint on how much processing is allowed to be invoked.

The idea of having motion resampling separated from texture resampling was adopted at the previous meeting. It was suggested that the idea of this was to allow sample prediction without motion prediction or vice-versa, but perhaps not to allow sample prediction from a different picture than motion prediction. It was questioned whether there is actual value in using motion prediction without sample prediction.

See BoG report N0374 and related notes.

JCTVC-N0213 SHVC HLS: On Resampling Buffer Indication [Y. He, Y. Ye (InterDigital)]

An indicator signalling of two types of ILP, sample and motion, was proposed in JCTVC-M0457 and adopted into SHVC WD2. The motivation is to indicate whether a reference layer is used for sample prediction, motion prediction, or both, and the resampling operation and the corresponding buffer could be allocated in advance. However, for SNR scalability, the resampling operation and the corresponding buffer allocation could be bypassed even though the sample and/or motion prediction are carried out for a reference layer. In this contribution, an extra resampling buffer indicator is proposed to indicate if the corresponding resampling buffer is required or not in advance.

The proponent noted that for SNR scalability it is not necessary for the decoder to allocate a resampling buffer, and proposed to add a flag to indicate such a case.

It was remarked that the decoder can directly observe the picture size and offset values to determine what to do without needing a flag. The flag would enable determining the resampling requirement at the VPS level rather than the SPS level; however, buffer allocation requires using information at the SPS level already.

It was remarked that some of the relevant parameters are proposed to be moved up from the SPS to the VPS in N0092. N0215 is also noted to be related.

No action was planned, pending whether a need is identified for this indication after consideration of other contributions.

JCTVC-N0215 SHVC HLS: On SNR Scalability Indication [Y. He, Y. Ye (InterDigital)]

This contribution proposes to signal SNR scalability for the needs of re-sampling process, inter-layer filters and certain applications. It is asserted that it would be beneficial to differentiate SNR scalability from spatial scalability in high level syntax so that the encoder and decoder operations can be configured and/or initialized according to the relevant coding tools to be supported.

It was remarked that the decoder can directly observe the picture size and offset values to determine whether the scalability is SNR scalability without needing a flag.

It has not yet been determined whether there are different coding tools for SNR and spatial scalability.

It was remarked that we basically need to first determine of these more fundamental questions before worrying about this indication.

No action was planned, pending whether a need is identified for this indication after consideration of other contributions.

JCTVC-N0209 SHVC HLS: SHVC Skipped Picture Indication [J. Boyce (Vidyo), X. Xiu, Y He, Y Ye (InterDigital)]

It is proposed to add a skip_picture_flag syntax element in the SHVC slice segment header to indicate if a picture is skipped, and to define a normative skipped picture decoding process.  The proposals are asserted to be similar to SVC's slice_skip_flag and associated decoding process, but apply to an entire picture rather than a single slice.

The contribution proposed not performing deblocking for skipped pictures. It was remarked that the motion compensation process will generate blocking artefacts, so deblocking should be performed (or explicitly indicated whether it is to be performed or not).

It was remarked that it may be desirable to establish slice boundaries even in the case of skipping. In the prior SVC design, skipping was a slice-level operation rather than a picture-level operation, and slice boundaries are established.

It was suggested to check whether temporal motion prediction from the enhancement layer might perform better than using the base layer motion for a skipping operation.

It seemed agreed that having some kind of skipping capability is desirable. However, further study is needed to determine whether that should operate at the slice level or picture level (or perhaps tile level), and exactly how it should operate.

JCTVC-N0210 SHVC HLS: Adaptation Picture Set for SHVC [Y. He, J. Dong, Y. Ye (InterDigital)]

This contribution is a follow-up of JCTVC-M0179. It is proposed to use Adaptation Parameter Set (APS) to carry picture level parameter information to support scalable extension of HEVC. The syntax elements of inter-layer processing, such as inter-layer filtering coefficients, sample prediction and motion prediction syntax elements are proposed to be signaled in APS instead of slice header to save bits and keep the existing slice header syntax intact. The APS syntax, semantic and simulation results are provided in this contribution.

An example use is the inter_layer_information() syntax.

It was remarked that

- We should first determine that there is a need for the syntax that would benefit from being sent using an APS, rather than trying to decide to have an APS first than the figure out what to put in it.

- Some coding efficiency impact should be measured or calculated to determine the extent of the potential need.

No action was planned, pending whether a need is identified after consideration of other contributions.

JCTVC-N0296 Cross-check of JCTVC-N0210 on APS for SHVC [H. Yang (Huawei)] [late]

Cross-check based on SCE 3.4.1 chroma enhancement filter parameters.

### Signalling of cropped inter-layer reference

(This agenda category discussed Thu 25th p.m. in Track A (GJS).)

JCTVC-N0054 Signalling and restriction for scaled reference layer offsets [T. Tsukuba, T. Yamamoto (Sharp)]

This contribution proposes a modified signalling and restriction regarding scaled reference layer offsets. On the modified signalling, a presence flag indicating whether to signal scaled reference layer offsets parameters is added, and a present flag for scaled reference layer offsets is added for each direct reference layer. On the restriction, a semantic restriction is proposed to restrict inter layer sample prediction outside or across the bounds specified by scaled reference layer offsets.

It is asserted that the signalling modification is beneficial to improve the coding efficiency and to simplify decoder implementation by removing additional boundary operation for scaled reference layer offsets.

The proposal suggests to remove num_scaled_ref_layer_offsets, as it is asserted that this syntax element is redundant with VPS information.

The contribution proposed the following changes to the syntax:

- Removing num_scaled_ref_layer_offsets.

- Adding scaled_ref_layer_offset_params_present_flag.

- Adding scaled_ref_layer_offset_present_flag for each direct reference layer.

Measurement data was not provided regarding the loss from disallowing prediction off the edge of the reference layer. The group was rather skeptical about the desirability of disallowing the extrapolation case.

It was remarked that removing num_scaled_ref_layer_offsets introduces an undesirable dependency of the parsing of SPS on VPS.

It was remarked that there is already a way to not send offsets for all layers.

No action was taken on this.

JCTVC-N0089 MV-HEVC/SHVC HLS: On signalling of scaled reference offset [J. Chen, A.-K. Ramasubramonian, Y.-K. Wang, Y. Chen, X. Li (Qualcomm)]

This contribution proposes moving the scaled reference offsets syntax from SPS extension to VPS extension. In addition, similar to SVC, it is proposed for the scaled reference offsets to be further signalled in the slice header to have picture-level adaptation.

The motivation for this proposal depends somewhat on N0092 / E0060.

Regarding allowing picture-level adaptation of the offset values, this might add decoder complexity and it did not seem very likely that encoders would want to use that feature.

It was agreed to keep the scaled reference offsets at the SPS (extension) level. No action was taken on this.

### Hybrid scalability

(This agenda category discussed Thu 25th p.m. in Track A (GJS).)

JCTVC-N0050 / JCT3V-E0037 Specification text to support AVC base layer in HEVC layered extensions [J. Boyce (Vidyo), K. Kawamura (KDDI)]

This contribution proposes draft specification text to support an AVC base layer for SHVC, and proposes a new Hybrid Scalable Main profile. The proposed draft text changes provided are based upon the SHVC editor’s draft input contribution JCTVC-N0242, and fit within one additional page of specification text.  While specifically proposed for SHVC, the changes could also apply to MV-HEVC and other HEVC layered extensions. Draft text was provided. The proposal included syntax, encapsulation format, decoding process, and a draft profile.

Version 2 of the contribution adds an author, and specifies that avc_base_layer_flag shall be equal to 1 in the proposed Hybrid Scalable Main profile.

It was noted that the N12956 MPEG requirements document has a 'should' regarding AVC base layer support.

For the prior drafted profiles, it was proposed to disallow using a non-HEVC base layer.

In the discussion of the proposal, it was asked whether we really want to use encapsulation within an HEVC elementary stream.

Do we want MPEG-2? (This would need some working out of what is equivalent to a NAL unit and a POC.)

It was noted that the proposed encapsulation would be opaque to existing AVC decoders, such that an extraction process would be needed to pull out the AVC base layer for consumption by such a decoder.

If instead of specifying encapsulation within an HEVC elementary stream, we depend on the system to provide the base layer by external means, the base layer could be compatible with an existing non-HEVC decoder.

If we don't define the encapsulation, the scope of our profile would be only the enhancement layer capability.

The proponent indicated that since AVC is so structurally similar, it would be easier to specify how HEVC works with AVC than for other base layers.

It was remarked that it seems cleaner not to have cross-specification base layer normative referencing within the HEVC standard. That should be done in some other specification(s), if done.

This contribution was also discussed jointly and was closely related to the VCEG contribution VCEG-AV13.

Further study was encouraged to determine the right way to construct such a profile.

JCTVC-N0211 SHVC HLS: On AVC Base Layer GOP Structure Indication [Y. He, Y. Ye, Y. He (InterDigital)]

It is proposed to add an identical GOP structure indicator for the AVC base scalability to indicate if the AVC base layer GOP structure is the same as the GOP structure of the enhancement layers. The benefit of such indicator is to inform the decoder if all the pictures associated with the same output time can be decoded consecutively or not, and further facilitate the decoding initialization and memory allocation.

It was agreed that our current plan is that the GOP structure should always be aligned, at least in the HEVC base layer case. This is implied by our current AU definition. This should be editorially clarified if necessary.

If we have an external specification the provides base layer pictures, perhaps that specification could determine its own rules. But this is our current plan within the scope of HEVC.

See also notes for N0050 and N0015.

## SEI messages

### Motion and prediction constrained SEI messages

JCTVC-N0069 Motion and inter-layer prediction constrained SEI message [K. Ugur, M. M. Hannuksela (Nokia)]

(Reviewed Thu. 25th p.m. in Track A (GJS).)

One potential application of SHVC is to enhance the quality of a region of interest (ROI) of the base layer picture with the enhancement layer. In such use, as only a certain region of the picture is enhanced, it would be useful to indicate the details of this operation to avoid redundant decoding of certain regions and minimize complexity. It is asserted that tiles, and more specifically motion-constrained tiles, can be used to support the ROI quality enhancement use case. This contribution proposes some modifications to the motion-constrained tile sets SEI message with the aim of supporting ROI enhancement.

Two scenarios:

- Enhancing an ROI, leaving some non-ROI area the same as the base layer (except perhaps for resampling).

- Enhancing an ROI without using BL texture, leaving some non-ROI area the same as the base layer. (A further constraint relative to the first scenario.)

The second scenario is related to N0117 and N0236.

Proposes, by adding additional syntax elements for each tile set rectangle, to:

- Indicate "skipped tile sets" where the EL area is identical to the (possibly upsamplesd) BL.

- Indicate no-inter-layer-prediction tile sets.

In discussion, it was asked what is the purpose of the layer ID syntax element and its relationship with scalable nesting and the layer ID in the NUH.

It was remarked that N0159 is also related.

See BoG report N0374 and associated notes.

JCTVC-N0087 Inter-layer constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

(Reviewed in Track B Sun. 28th (AS).)

This contribution proposes an SEI message that indicates inter-layer decoding independence between tiles. The syntax is aligned with the (temporal) motion constrained tile sets SEI message. In addition to region of interest based decoding, a decoder can use the information provided by the SEI message for load assignment, e.g. parallel decoding of tiles.

Uses the structure of the motion constrained tile sets SEI message to signal that tiles are independent for inter-layer prediction.

In addition, the proposal also contains a flag to signal that all the tiles are independent.

This is similar to N0159 (proposal 3).

The proposal is also applicable to multi-view coding.

It was commented that there may be a conflict between this application and region of interest signalling.

Others asserted that the proposed method could co-exist with the signalling of region of interest tile sets.

It was commented that region of interest and parallelisation, which is the goal of this proposal, are different applications.

One expert commented that the relationship between motion constrained tile sets and inter-layer constrained tile sets should be discussed.

See also BoG report N0374 and related notes.

JCTVC-N0088 Extension of (temporal) motion constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

See BoG report N0374 and related notes.

JCTVC-N0117 HLS: Extensions to Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami, T. Suzuki (Sony)]

See BoG report N0374 and related notes.

JCTVC-N0236 HLS: Extensions of motion-constrained tile sets SEI message for interactivity [C. Auyeung, J. Xu (Sony)]

See BoG report N0374 and related notes.

### Layer presence and dependency change SEI messages

JCTVC-N0173 MV-HEVC/SHVC HLS: On Layers Present SEI message [J. W. Kang (ETRI), J. Lee, H. Lee, J. S. Choi, T. C. Thang (UoA)]

See BoG report N0374 and related notes.

JCTVC-N0174 MV-HEVC/SHVC HLS: On Layer Dependency Change SEI message [T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI)]

See BoG report N0374 and related notes.

### Frame packing SEI messages

JCTVC-N0133 HLS: Frame packing arrangement SEI message for bit depth extension [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)] [late]

A change of the frame packing arrangement (FPA) SEI message is proposed for delivering extended bit-depth data up to 14 bits in nominally 8bit bitstreams. Two layered bitstreams are carried with FPA SEI messages. One constituent picture can be decoded compatibly as an ordinary 8 bit image, or can be decoded an extended bits (10~14bits) image with a supplementary picture data, which are differential data between the original data and the coded 8 bits data, in a scalable manner.

Further study was encouraged.

JCTVC-N0134 HLS: Frame packing arrangement SEI message for full resolution 3D [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)] [late]

A change of the frame packing arrangement (FPA) SEI message is proposed. The proposal is to add a flag, which indicates whether the current video sequence is the "primary half resolution 3D data" or is complementary (secondary) data. It is asserted that the full resolution 3D video as well as the half resolution 3D video can be delivered in a scalable manner with the proposed FPA SEI message.

Further study was encouraged.

JCTVC-N0270 Updated proposal with software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [Y. Zhang, Y. Wu, S. Kanumuri, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft)]

(Reviewed Fri. 2nd a.m. (JRO).)

This contribution proposes a method to extend the use of the frame packing arrangement SEI message to represent 4:4:4 content in nominally 4:2:0 bitstreams. The contribution is an update of the prior contributions JCTVC-K0240, JCTVC-L0316 and JCTVC-M0281 that provides software and specification text with a graphical illustration (as reported to be requested during discussion at the previous meeting). With the proposed method, it is reported that one constituent frame (e.g. in a top-bottom packing or alternating-frame coding scheme) can be decoded compatibly as an ordinary 4:2:0 image, or can be supplemented with the data from another constituent frame to form a complete 4:4:4 image representation. It is proposed to include support for the additional scheme into the frame packing arrangement SEI message in both AVC and HEVC, to facilitate deployment of systems using this method. Since 4:2:0 is the most widely supported format in products, it is asserted that having an effective way of conveying 4:4:4 content through such decoders can provide the substantial benefit of enabling widespread near-term deployment of 4:4:4 capabilities (especially for screen content coding). The proposed method operates by packing the samples of a 4:4:4 frame into two 4:2:0 frames and encoding the two 4:2:0 frames as the constituent frames of a frame packing arrangement. The semantics of 'content_interpretation_type' are extended to signal this packing arrangement. The proposed scheme is asserted to be of high practical value for applications involving screen content. Relative to native 4:4:4 encoding, the proposed scheme can provide the advantage of compatibility with the ordinary 4:2:0 decoding process that is expected to be more widely supported in decoding products. It is reported that the attached software is capable of handling the frame-packing and frame-unpacking processes and can be used in conjunction with any 4:2:0 codec.

Basically the same as previously proposed. Update includes software and improved text.

Three types of 4:2:0 low-resolution (new interpretation types 3, 4, and 5):

without filtering, two informatively describing downsampling and reconstruction: conventional filters; lifting based. Phase position as from annex E.

Question: Necessary to specify the filters precisely? Lifting filters could be implemented via conventional?

Lifting filters would allow (almost) perfect reconstruction (for PR, it would require higher bit depth)

One expert supports the proposal.

Using existing frame packing, but adding interpretation type.

The group initially agreed to adopt this for the SHVC draft, but only interpretation types 3 and 4, without specifying anti alias and upsampling filters.

However, after further discussion, it was agreed to defer the planned action to consideration at the next meeting.

### Color enhancement SEI messages

JCTVC-N0148 AhG8: Guided Image Filtering for Screen Content Coding [T. Vermeir (Barco), J. De Cock, G. Van Wallendael, S. Van Leuven (Ghent Univ. - iMinds)]

(Reviewed Thu 1st a.m. (GJS).)

Downsampling the chroma components, as used for coding in HEVC Main profile (4:2:0), generates visual artifacts for screen content. This contribution provides the results of experiments with a guided image post-processing filter to reduce these artifacts after decoding. This filtering step is asserted to restore edges in the chroma component of screen-content, which have been affected due to the original 4:2:0 sub-sampling. The luma component is used as a guide to enhance the chroma components. By analyzing a local histogram per pixel, the radius of the guided image filter is reconfigured. Tests conducted on several sequences reportedly show that the PSNR of the Cb and Cr components for lossless compressed streams is increased by 0.63 dB and 0.99 dB on average, respectively.

The presented results are based on on-going experiments, therefore also future work is proposed.

This is described as a post-process, not something that happens within the prediction loop.

The subjective quality improvement is suggested to exceed the measured PSNR benefit.

Hypothetically, this could be supported using an SEI message.

The contributor suggested that this could also be used in chroma format scalability.

The reported quality improvement seems substantial.

Further study was encouraged.

JCTVC-N0180 Color Mapping SEI message [Philippe Bordes, Pierre Andrivon, Patrick Lopez, Franck Hiron (Technicolor)]

This contribution proposes a colour mapping SEI message to support colour mapping post-processing on reconstructed frames. The colour mapping function is coded using a 3D Colour LUT.

It was remarked that this seems closely related to new parent-level inputs under substantial consideration – e.g. input from studios on XYZ and input submitted for CICP consideration.

It may also be related to the colour gamut scalability, in the sense that it provides a way to provide compatible service targeting alternative displays.

It was commented that VUI (SPS or VPS VUI) could be considered here as an alternative way of expressing the information. The contributor suggested that this might be too much data to carry at the VPS level. It was further remarked that perhaps there could be a combination of some small indicator at a high level with details provided at a lower syntax level.

Further study was encouraged.

JCTVC-N0224 SEI message: post filters to enhance the chroma planes [J. Dong, Y. Ye, Y. He (InterDigital)]

This contribution proposes an SEI message carrying post filter coefficients, in order to enhance the quality of the reconstructed chroma planes. On the decoder side, a reconstructed chroma pixel is enhanced by adding an appropriate offset obtained by applying the received post filters, which usually have high-pass characteristics, on the surrounding luma pixels. By doing this, the chroma edges lost during compression are well restored using the high frequency components from the corresponding luma plane. The proposed SEI message is implemented in HM-10.1 and HEVC RExt. software “HM-10.1+RExt-3.0”. Compared with HM-10.1 anchor and HEVC RExt. anchor, significant chroma PSNR improvement, e.g., up to 1.5 dB, can be observed in a wide range of sequences, including Class A, Class B, screen contest sequences, and YUV sequences with higher bit-depth (10-bit) and color sampling ratios (4:4:4 and 4:2:2).

It was noted that N0224 is a post-filter approach corresponding to the same idea described in N0223.

Some variant was also considered in the SHVC context in JCTVC-M0183 and in a CE contribution to this meeting.

We also have the post-filter hint SEI message.

This is in the area of post-filtering enhancement, and hypothetically we know there are other things we could do, e.g. ALF as a post-filter with block-level adaptivity. We have not had other recent inputs on post-filtering enhancement information.

Significant PSNR gain in the chroma domain was reported (up to 1.5 dB, esp. at lower bit rates in the range of 39 dB PSNR).

It was asked whether a post-filter scheme that does not have the cross-component characteristic would provide similar amounts of gain. The contributor responded that the high-pass nature of this filter might provide visually superior results.

It was remarked that there is interest in defining post-filtering enhancement information and that it would be desirable to study the general area in order to try to identify the best possible schemes for such technology.

Further study was encouraged for the scheme, and for other post-filtering methods as well.

JCTVC-N0309 Signalling of chroma sampling filter [K. Kazui (Fujitsu), T. Chujoh (Toshiba)] [late]

JCTVC-M0146 provided a framework to send recommended up-sampling and down-sampling filter coefficients for chroma format conversion by using VUI and SEI message. This follow-up contribution proposes a modified signalling scheme of chroma sampling filter taking into consideration the discussion on JCTVC-M0146 from the last meeting.

Two variations were provided – VUI or SEI.

A first contribution was submitted in October on the subject.

Only one specific type of filtering indication was SMPTE RP2050-1 (with guidelines in SMPTE EG2050) for vertical and JPEG 2000 5/3. It was suggested that providing the JPEG 2000 values would not take much text and would avoid a need to find the information elsewhere.

Regarding rounding, the prop refers to SMPTE EG, and it was suggested that there is rounding advice in JPEG 2000.

Suggested to be sent (only) when the encoding is 4:2:0 or 4:2:2. This should be clarified.

Editorial improvement seems needed for the draft text.

SEI rather than VUI suggested, e.g. due to the size.

The amount of text is substantial and had not been reviewed in detail.

It was agreed to plan to adopt at the next meeting, given adequate editorial improvement, good candidate reference software being provided (perhaps with external tool, probably usable with HM), and no new issues discovered.

### Other SEI messages

JCTVC-N0286 Time code SEI message [C. Fogg, B. Lewis, J. Lassahn, T. Prins (Elmental)]

This proposal requests that some clock_timestamp elements from AVC be copied into a new HEVC high level syntax construct, such as an SEI message, that can accompany each coded picture in the elementary video bitstream. Time code is informative, and has no impact on the decoding, HRD, or any output processes within the scope of the HEVC specification. Although several user_data() conventions for time code are used in exchange within specific applications, the MPEG-2 GOP header and AVC pic_timing() SEI message have reportedly served as universal embedded time codes that are relied upon in practice by several encoders across applications to synthesize systems timing information, in particular the PTS and DASH timeline. Time code is also passed through the elementary stream for various uses downstream. The authors of this proposal suggest that, especially for the benefit of transcoding operations from AVC to HEVC, a simplified SEI time code be re-introduced into the next HEVC draft. The most essential information is suggested to be the {Hours, Minutes, Seconds, Picture Count} parameters commonly known as “SMPTE 12M”. An alternative syntax adapted from AVC is also provided that includes more of its complicated features, should some unknown users rely on drop frame counts and/or need more than 24 hours of time range indication. It is proposed that time code be signalled only for coded pictures, and therefore any repeat pictures (indicated by pic_struct and DeltaToDivisor in HEVC Table E-6) be implicitly derived, rather than explicitly signalled as is done via the AVC clock_timestamp[i] array.

The presenter was not available Fri 2nd a.m. The contribution is available for study.

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control

### Encoder optimization

JCTVC-N0372 HM Software modifications for interlaced coding [Zineb Agyo, Jerome Vieron, Jean Marc Thiesse (Ateme)] [late]

This contribution provides extensions to the HM software in order to handle field based coding.

An HM 11.0 based patch was made for the reference software in order to introduce the field coding features previously presented in a study of field based coding. Furthermore, SEI messages are now handled in this patch version so the decoder can correctly display interlaced content.

Thu 1 (GJS): The presenter was not available. The software enhancement to the HM was delegated to the software coordinator for consideration.

### Software development

JCTVC-N0230 SAO software cleanup and non-normative encoder-only bug-fixes [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, a significant software cleanup and three non-normative encoder-only bug-fixes for sample adaptive offset (SAO) on top of HM-11.0 are proposed. In comparison with HM-11.0 plus the three bug-fixes under common test conditions, the new encoder reportedly generates exactly the same bitstreams, and the new decoder outputs exactly the same decoded videos. No run time increase is reported. Legacy code for picture quadtree SAO, adaptation parameter set (APS), and fine granularity slices are removed, and thus the readability is reportedly significantly improved. The number of SAO code lines is reported to be reduced by half. It is also reported that the bug-fixes cause very minor BD-rate changes on average. Corresponding software packages on top of rev. 538 of HTM-DEV-0.3-dev2, SHM-2.0, and RExt-3.0 are all provided..

The was delegated to the software coordinator for consideration.

JCTVC-N0306 Cross-check for HM11.0 SAO code clean-up and encoder bug-fix [E. Alshina, A. Alshin (Samsung)] [late]

## Withdrawn and unclear allocation

See section 1.4.2.

# Plenary Discussions and BoG Reports

## Project development coordination

Joint discussions were held with the WG 11 hosting parent body and with ITU-T Q6/16 VCEG. See section 3.4.

## BoGs

JCTVC-N0373 / JCT3V-E0306 BoG report on random access and cross-layer alignment of pictures types [J. Boyce]

See section 6.4.2.

JCTVC-N0374 BoG report on SHVC/MV-HEVC HLS topics [J. Boyce]

See section 6.4.1.

JCTVC-N0375 BoG report on Arbitrary Scalability Ratios support in SHVC [E. François]

See section 6.2.1.

JCTVC-N0384 BoG report on non-RCE2 contributions [R. Joshi, R. Cohen]

See section 6.1.2.

JCTVC-N0387 BoG report on Color Gamut Scalability [A. Segall]

See section 6.2.9.

JCTVC-N0388 BoG on combination of tools in RCE2 [J. Sole]

See section 5.2.1.

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the Monday of the week preceding the meeting (14 Oct 2013).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

The CE plan should be final at the same time as corresponding software, except for SCEs 1 & 4 due to test sequence issues.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

Note that regardless of preliminary plans for CEs discussed earlier in the meeting (and possibly noted in this report), any such plans were not considered binding on final CE plans as reviewed in the closing plenary.

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

No particular changes were noted w.r.t. prior CTC (with the small update posted 2013-06-04).

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-L1100.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 12.0 and SHM 3.0 should be available within 2 weeks after the meeting.

- HM 12.0+RExt should be available within 1 week after HM 12.0 availability.

## Subjective verification test plan

A proposed draft verification test plan was submitted as follows.

JCTVC-N0385 Draft HEVC verification test plan [T.K. Tan, V. Baroncini]

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1002 HEVC Test Model 12 (HM 12) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 12.0 software version (by 2013-08-04 for ISO/IEC DIS ballot) and the reference configuration encodings according to JCTVC-L1100 common conditions (expected within two weeks after the meeting). - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Study to develop a potential set of HEVC conformance bitstreams. | T. Suzuki, W. Wan (co-chairs) | N |
| HEVC range extensions development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats, bit depths beyond 8 bits, and auxiliary/alpha channel coding, in coordination with AHG7 and AHG18. - Perform memory bandwidth analysis of the range extensions technical design and its proposed modifications. - Evaluate blocking artifacts in 4:4:4 chroma format coding and set up a plan for subjective testing of associated techniques at the next meeting. - Discuss and propose test conditions and test material for the development of the range extensions. - Study techniques for colour conversion and resampling and their relationship to non-4:2:0 chroma coding. | M. Naccari, C. Rosewarne (co-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1005 HEVC RExt draft text. - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary), D. Flynn, C. Rosewarne, T. Suzuki | N |
| Range extensions software development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 11.0-RExt-4.0 software version and the reference configuration encodings according to JCTVC-N1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for testing screen content coding performance. - Make recommendations for test conditions for screen content coding. - Coordinate for finalization of the test conditions for CEs relating to lossless and screen content coding. | H. Yu (chair), R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Study the high level syntax for auxiliary pictures with consideration of JCTVC-N0063/JCT3V-E0049 as well as signalling for carrying alpha channel and 4:4:4 chroma enhancement as auxiliary pictures. - Study JCTVC-N0355/JCT3V-E0092 and other mechanisms to extend the number of layers (beyond 64) in comparison with reserving nuh_layer_id value of 63 to provide an extension, in terms of functionality supported and signalling efficiency. - Coordinate efforts with JCT-3V AHG7 on high-level syntax issues in relation to 3D extensions. | M. M. Hannuksela (chair), J. Boyce, Y. Chen, S. Deshpande, A. Norkin, Y.-K. Wang, (vice-chairs) | N |
| SHVC core experiments (AHG10) (jct-vc@lists.rwth-aachen.de) - Establish configurations, generate anchors, and create reporting sheets for SHVC CEs. - Provide configuration data to be used in SHVC CEs (within one week after SHM 3.0 software is available). - Discuss and identify additional issues related to SHVC CEs. | X. Li (chair), J. Boyce, P. Onno, X. Xiu (vice-chairs) | N |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1007 SHVC Test Model 3 (SHM 3) text. - Produce and finalize JCTVC-N1008 SHVC text specification Draft 3. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 3.0 software (based on HM 11) for experimentation. - Provide software to CEs within two weeks after the meeting. - Remove IntraBL framework from SHM software. - Bring software into alignment with HM 12 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), T. Chuang, Y. He, D. Kwon (vice-chairs) | N |
| SHVC inter-layer filtering (AHG13) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability. - Study the effect of taking into account chroma position alignment during resampling processing. - Identify alternative candidate resampling filters and distribute associated downsampled test material within three weeks after the meeting. - Study resampling filters with ratios other than 1.5 and 2.0 (and 1.0). - Discuss and identify additional issues related to inter-layer filtering. | E. Alshina (chair), J. Chen, J. Dong, A. Segall P. Topiwala (vice-chairs) | N |
| Colour gamut scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut scalable coding. - Study the interaction of colour gamut scalability with bit-depth and spatial scalability. - Identify test sequences and test conditions. - Discuss and identify additional issues related to colour gamut scalability. | A. Duenas, A. Segall (co-chairs), P. Bordes, J. Dong, D.-K. Kwon, X. Li (vice-chairs) | N |
| Hybrid codec scalability (AHG15) (jct-vc@lists.rwth-aachen.de) - Coordinate with AHG12 on software development and anchor support for AVC base layer. - Study methods for base layer codec identification and picture alignment, including use of encapsulation, and identification of external means. - Study profile and level specification methods for hybrid codec scalability. | J. Boyce, K. Kawamura (co-chairs) | N |
| Single-loop scalability (AHG16) (jct-vc@lists.rwth-aachen.de) - Study and compare single-loop and multi-loop SNR scalability. - Analyze and compare the memory bandwidth of single-loop and multi-loop scalability and the use of "key pictures". - Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signalling inter-layer prediction restrictions on a multi-loop design. - Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability. | M. Wien (chair), M. Budagavi, K. Misra, K. Rapaka, K. Ugur, X. Xiu (co-chairs) | N |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them. - Study restrictions to reduce the complexity of scalable tools and evaluate the impact of such restrictions on coding efficiency. - Prepare a report analyzing performance and complexity of single-layer, simulcast, and scalable coding configurations for 1.0, 1.5, and 2.0 and arbitrary scalability ratio (e.g. ~1.75) cases. - Coordinate with SCE developments and provide a template to be used for complexity analysis of SHM3.0, single-layer coding, and SCE techniques. | E. Alshina (chair), M. Budagavi, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai (vice-chairs) | N |
| High bit-rate & bit-depth operating points (AHG18) (jct-vc@lists.rwth-aachen.de) - Study the accuracy needed for internal processing elements to support bit depths up to 16-bits per sample. - Study relationship to lossless coding capability. - Verify rate-distortion optimization behaviour for high bit rates and high bit depths. - Study entropy coding operation and throughput at high bit rates and high bit depths and potential needs for associated design modification. - Identify test sequences and test conditions for testing high bit rate and high bit depth coding behaviour. - Prepare software implementation for technical investigation of new features intended for high bit rates and high bit depths. - Study coding performance at high bit-rate and high bit depth operating points and investigate the benefit over existing standards. | K. Sharman (chair), E. François, H.-Y. Kim (vice-chairs) | N |
| Verification test preparation (AHG19) (jct-vc@lists.rwth-aachen.de) - Identify test conditions and test sequences for verification of HEVC compression capability, including consideration of use case scenarios for random access and low delay operation. - Identify bit rates, picture resolutions and bit depths appropriate for HEVC verification. - Identify appropriate encoder usage for comparison of HEVC and AVC compression capability. - Communicate with subjective test coordinator to ensure test design validity. - Generate and collect candidate HEVC and AVC encoded bitstreams for HEVC verification testing. - Review and edit the HEVC verification test plan JCTVC-N1011. | T. K. Tan, V. Baroncini (co-chairs), M. Karczewicz, M. Mrak, W. Wan, J. Wen (vice-chairs) | N |
| Multi-layer hypothetical reference decoder (AHG20) (jct-vc@lists.rwth-aachen.de) - Study the issues related to layer-specific and combined multi-layer HRD operation (including DPB and CPB), including the proposed . - Identify layer-specific or combined multi-layer HRD operations that need to be addressed at the system and/or video coding layer. - Study issues and implications of either or both HRD mechanisms when adding or removing layers at re-distribution points. - Study issues and implications of layer-specific buffer operation in relevant application contexts. - Study high-level syntax needs for HRD information, e.g. in SEI, SPS VUI, or VPS VUI. | K. Suehring, A. Tabatabai (co-chairs), S. Deshpande, M. M. Hannuksela, J. Kang, A. K. Ramasubramonian (vice-chairs) | N |
| Best-effort decoding with reduced decoding complexity (AHG21) (jct-vc@lists.rwth-aachen.de) - Identify use cases and key functional elements for application of reduced-complexity decoding processes for decoding of bitstreams encoded for HEVC and its extensions - Analyze tradeoffs for optimization of video quality in the application of reduced-complexity decoding processes - Particularly study characteristics of design and optimization for decoding processes with reduced bit depth - Consider and analyze the impact of inter-picture dependencies and cross-region spatial prediction dependencies in the application of reduced-complexity decoding processes - Study potential approaches to describe/specify the provision of best-effort decoding capabilities in the HEVC text specification | D. Flynn, J. Sole, M. Zhou (co-chairs) | N |
| Test sequence material (AHG22) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for HEVC development. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG8 on screen content coding and AHG19 for verification test development. | T. Suzuki, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |

# Output documents and end-of-meeting actions

For preparation of outputs of SHVC test model text and software, it was agreed to not include the prior TextureRL/IntraBL technology in the outputs of this meeting.

Regarding the HM encoder description – the plan has been to improve this in AHG work toward publishing it as part of the Reference Software standard. Considering the state of work on that topic, it remains TBD whether this would be in v.1 of the RS standard or be added later, and substantial further work would be needed to bring this description to an appropriate level of maturity.

Regarding issues in conformance test set development, the situation has been improving. profile_idc and level_idc need to be correct, some HM 9 bitstreams had not been updated, some planned bitstreams were missing – many bitstreams need updates for corrections of these issues and others. Coverage still needs improvement. Various combinations of tiles & slices & loop filtering control parameters, SEI messages, and "corner cases" were discussed as needs in further development for conformance testing. However, the work was generally moving forward with some significant progress reported. It was discussed whether we should issue a "Call for HEVC conformance bitstreams" as an output document or meeting resolution. After discussion, this was not done (although additional work on the conformance test set is certainly needed and encouraged).

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

The following disposition of comments reports were prepared for WG 11 purposes:

- WG 11 N13771 Disposition of Comments on ISO/IEC CD 23008-5

- WG 11 N13773 Disposition of Comments on ISO/IEC CD 23008-8

JCTVC-N1000 Meeting Report of 14th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-N1002 High Efficiency Video Coding (HEVC) Test Model 12 (HM 12) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N 13770) [2013-09-30]

JCTVC-N1003 High Efficiency Video Coding (HEVC) Defect Report [Y.-K. Wang, G. J. Sullivan, B. Bross] (WG 11 N 13837) [2013-08-31]

JCTVC-N1004 HEVC Conformance Draft 4 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 13774 ISO/IEC DIS) [2013-08-07]

JCTVC-N1005 HEVC Range Extensions Draft 4 [D. Flynn, J. Sole, T. Suzuki] (WG 11 N 13763, ISO/IEC DAM) [2013-08-07]

JCTVC-N1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman] [2013-08-16] (2 weeks)

The editors were instructed to describe the SCC test sequences, and were also requested to shorten the length of sequences for operation if the results don't change significantly.

JCTVC-N1007 SHVC Test Model 3 (SHM 3) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13766) [2013-09-15]

The editors were instructed for this to include an encoder description but not retain the TextureRL technology.

JCTVC-N1008 SHVC Draft 3 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13765) [2013-09-15]

JCTVC-N1009 Common SHM test conditions and software reference configurations [X. Li, J. Boyce, P. Onno, Y. Ye] [2013-08-23] (1 week after the SHM 3 software)

The editors were instructed to include updated anchor results.

JCTVC-N1010 HEVC HM 12 Reference Software [F. Bossen, D. Flynn, K. Sühring] (WG 11 N 13575 ISO/IEC DAM) [2013-08-07]

JCTVC-N1011 HEVC verification test plan draft 1 [T.K. Tan, V. Baroncini] (WG 11 N 13838) [2013-08-02]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting discussions, such plans were not considered binding on final CE plans as reviewed in the closing plenary.

- JCTVC-N1101 HEVC Scalable Extensions Core Experiment SCE1: Arbitrary scalability ratio support [E. Francois (primary), E. Alshina, J. Chen (CE Coordinators)]

- JCTVC-N1102 HEVC Scalable Extensions Core Experiment SCE2: Key pictures and single-loop decoding [M. Wien, K. Rapaka, X. Xiu (CE Coordinators)]

- JCTVC-N1103 HEVC Scalable Extensions Core Experiment SCE3: Inter-layer filtering [J. Chen, E. Alshina, J. Dong, M. Sychev (CE coordinators)]

- JCTVC-N1104 HEVC Scalable Extensions Core Experiment SCE4: Colour Gamut and Bit Depth Scalability [A. Segall, P. Bordes, C. Auyeung, X. Li, A. Duenas]

JCTVC-N1101 through JCTVC-N1104 were reviewed in non-final form in closing plenary. A three week finalization period was authorized. Most aspects of review other than intended technologies were deferred.

- JCTVC-N1121 HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods [W.-S. Kim, T. Nguyen (CE coordinators)]

- JCTVC-N1122 HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update [C. Rosewarne, J. Sole, K. Sharman, S.-H. Kim (CE coordinators)]

- JCTVC-N1123 HEVC Range Extensions Core Experiment 3 (RCE3) Intra prediction techniques [A. Saxena, D. Kwon, M. Naccari, C. Pang (CE coordinators)]

Both lossless and lossy.

JCTVC-N1121 through JCTVC-N1123 were reviewed in non-final form in closing plenary. A three week finalization period was authorized. Most aspects of review other than intended technologies were deferred.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 23 Oct. – 1 Nov. 2013 under ITU-T auspices in Geneva, CH.

- 9–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- 27 Mar. – 4 Apr. 2014 under WG 11 auspices in Valencia, ES.

- 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP.

The agreed document deadline for the October 2013 meeting is the Monday of the week preceding the meeting (14 Oct 2013). No restrictions were planned to be imposed on the scheduling of agenda items within that meeting.

Kenzler Conference Management was thanked for the excellent hosting of the 14th meeting of the JCT-VC. Alpen-Adria University of Klagenfurt was thanked for providing viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1345 hours on Fri. 2 Aug. 2013.

# Annex A to JCT-VC report:
List of documents

| JCT-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JCTVC-N0001 | m30456 | 2013-07-25 02:46:03 | 2013-07-25 02:46:47 | 2013-07-25 02:46:47 | JCT-VC AHG report: JCT-VC project management | G. J. Sullivan, J.-R. Ohm |
| JCTVC-N0002 | m30413 | 2013-07-23 12:13:58 | 2013-07-25 09:27:57 | 2013-07-25 09:27:57 | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) | B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs) |
| JCTVC-N0003 | m30429 | 2013-07-24 00:48:28 | 2013-07-27 10:59:15 | 2013-07-27 11:04:57 | JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) | F. Bossen, D. Flynn, K. Sühring |
| JCTVC-N0004 | m30411 | 2013-07-23 11:20:06 | 2013-07-23 11:20:43 | 2013-07-23 11:20:43 | JCT-VC AHG report: HEVC conformance test development (AHG4) | T. Suzuki, W. Wan |
| JCTVC-N0005 | m30450 | 2013-07-24 21:18:26 | 2013-07-25 10:13:00 | 2013-07-25 10:13:00 | JCT-VC AHG report: HEVC range extensions development (AHG5) | M. Naccari, C. Rosewarne |
| JCTVC-N0006 | m30404 | 2013-07-23 09:08:05 | 2013-07-23 09:32:01 | 2013-07-23 09:32:01 | JCT-VC AHG report: Range extensions draft text (AHG6) | J. Sole, D. Flynn, C. Rosewarne, T. Suzuki |
| JCTVC-N0007 | m30524 | 2013-07-27 08:32:42 | 2013-07-27 10:02:11 | 2013-07-27 10:02:11 | JCT-VC AHG report: Range extensions software (AHG 7) | K. Sharman, D. Flynn |
| JCTVC-N0008 | m30461 | 2013-07-25 10:13:44 | 2013-07-25 10:15:23 | 2013-07-26 09:50:34 | JCT-VC AHG report: Screen content coding (AHG8) | H. Yu, R. Cohen, A. Duenas, T. Lin, J. Xu |
| JCTVC-N0009 | m30459 | 2013-07-25 08:59:18 | 2013-07-25 09:00:48 | 2013-07-25 09:00:48 | JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) | M. M. Hannuksela, J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang |
| JCTVC-N0010 | m30166 | 2013-07-19 23:36:33 | 2013-07-24 14:53:02 | 2013-07-24 14:53:02 | JCT-VC AHG report: SHVC core experiments (AHG10) | X. Li, J. Boyce, P. Onno, Y. Ye, |
| JCTVC-N0011 | m30178 | 2013-07-20 20:58:49 | 2013-07-23 08:38:28 | 2013-07-23 08:38:28 | JCT-VC AHG report: SHVC text editing (AHG11) | J. Chen, J. Boyce, Y. Ye, M. Hannuksela |
| JCTVC-N0012 | m30360 | 2013-07-23 00:34:03 | 2013-07-24 16:19:33 | 2013-07-24 16:19:33 | JCT-VC AHG report: SHVC software development (AHG12) | V. Seregin, Y. He, T.-D. Chuang, D.-K. Kwon |
| JCTVC-N0013 | m30497 | 2013-07-26 09:40:40 | 2013-07-26 09:44:44 | 2013-07-26 09:44:44 | JCT-VC AHG report: SHVC Inter-Layer Filtering (AHG13) | A. Segall, E. Alshina, J. Chen, J. Dong, P. Topiwala, M. Zhou |
| JCTVC-N0014 | m30498 | 2013-07-26 09:41:49 | 2013-07-26 09:46:00 | 2013-07-26 09:46:00 | JCT-VC AHG report: Colour gamut scalability (AHG14) | A. Segall, P. Bordes, J. Dong, A. Duenas, L. Guo, D. K. Kwon |
| JCTVC-N0015 | m30339 | 2013-07-22 22:54:00 | 2013-07-23 20:24:46 | 2013-07-23 20:24:46 | JCT-VC AHG report: SHVC hybrid codec scalability (AHG15) | J. Boyce, K. Kawamura |
| JCTVC-N0016 | m30281 | 2013-07-22 17:24:50 | 2013-07-24 13:12:35 | 2013-07-26 10:24:59 | JCT-VC AHG report: Single-Loop Scalability (AHG16) | M. Wien, J. Boyce, M. Budagavi, K. Mishra, K. Ugur |
| JCTVC-N0017 | m30441 | 2013-07-24 14:59:26 | 2013-07-24 15:03:19 | 2013-07-26 10:08:40 | JCT-VC AHG report: SHVC complexity assessment (AHG17) | M. Budagavi, E. Alshina, J. Dong, E. FranÃ§ois, J. Kang, X. Li, A. Tabatabai |
| JCTVC-N0018 | m30523 | 2013-07-27 08:29:30 | 2013-07-27 08:31:14 | 2013-07-27 08:31:14 | JCT-VC AHG report: High bit-rate and bit-depth operating points (AHG18) | K. Sharman, E. Francois, H.-Y. Kim |
| JCTVC-N0019 | m30401 | 2013-07-23 09:02:10 | 2013-07-24 14:06:07 | 2013-07-24 14:06:07 | JCT-VC AHG report: Verification test preparation (AHG19) | T.K. Tan (NTT DOCOMO), V. Baroncini (FUB), W. Wan (Broadcom), M. Karczewicz (Qualcomm), J. Wen (Tsinghua Uni) |
| JCTVC-N0020 | m30496 | 2013-07-26 09:34:27 | 2013-07-26 09:35:00 | 2013-07-26 09:35:00 | JCT-VC AHG report: Chroma format scalability (AHG20) | A. Segall, A. Duenas, K. Ugur |
| JCTVC-N0021 | m30540 | 2013-07-27 15:48:41 | 2013-07-27 15:49:24 | 2013-07-27 15:49:24 | JCT-VC AHG report: Best-effort decoding with reduced decoding complexity (AHG 21) | D. Flynn, J. Sole |
| JCTVC-N0031 | m30397 | 2013-07-23 08:43:29 | 2013-07-24 01:06:13 | 2013-07-26 07:37:21 | SCE1: Summary report of SHVC Core Experiment on support for additional re-sampling phase shifts | E. Alshina, X. Li, J. Dong |
| JCTVC-N0032 | m30416 | 2013-07-23 19:03:34 | 2013-07-24 09:29:19 | 2013-07-24 09:29:19 | SCE2: Summary report of SHVC core experiment on combination of inter-layer syntax prediction and motion data compression | C. Gisquet (Canon), K. Sato (Sony) |
| JCTVC-N0033 | m30179 | 2013-07-20 21:02:37 | 2013-07-24 23:38:15 | 2013-07-24 23:38:15 | SCE3: Summary report of SHVC Core Experiment on Inter-layer Filtering | J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong |
| JCTVC-N0034 | m30518 | 2013-07-27 00:17:42 | 2013-07-27 00:21:31 | 2013-07-27 00:21:31 | RCE1: Summary Report of HEVC Range Extensions Core Experiment 1 on Inter-Component Decorrelation Methods | T. Nguyen, J. Sole, J. Kim, |
| JCTVC-N0035 | m30417 | 2013-07-23 20:19:23 | 2013-07-25 08:04:53 | 2013-07-25 08:04:53 | RCE2: Summary report on HEVC Range Extensions Core Experiment on prediction and coding techniques for transform-skip and transform-bypass blocks | R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari |
| JCTVC-N0036 | m30481 | 2013-07-25 20:29:39 | 2013-07-25 20:30:01 | 2013-07-25 20:30:01 | RCE3: Summary report of HEVC Range Extensions Core Experiment 3 on Intra Coding Methods for Screen Content | L. Guo |
| JCTVC-N0041 | m29563 | 2013-06-14 12:03:50 | 2013-07-16 01:51:01 | 2013-07-16 01:51:01 | Editors' proposed corrections to HEVC version 1 | B. Bross, G. J. Sullivan, Y.-K. Wang |
| JCTVC-N0042 | m29564 | 2013-06-21 20:32:53 | 2013-06-25 20:17:46 | 2013-06-25 20:17:46 | Non-RCE2: Restriction on the Residual DPCM block size | J. Sole, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-N0043 | m29616 | 2013-07-09 11:24:49 | 2013-07-09 22:06:50 | 2013-07-09 22:06:50 | On software complexity: real time and parallel SHVC video decoder | W. Hamidouche, M. Raulet, O. DÃ©forges (IETR/INSA Rennes) |
| JCTVC-N0044 | m29619 | 2013-07-09 22:56:27 | 2013-07-09 23:18:40 | 2013-07-09 23:18:40 | RCE2 Test B.1: Residue rotation and significance map context | J. Sole, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-N0045 | m29623 | 2013-07-11 00:50:17 | 2013-07-16 02:59:45 | 2013-07-16 02:59:45 | SCE1: Results of Test 1.1.3 on accurate chroma position alignment | X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), J. Dong, Y. Ye, Y. He (InterDigital), K. Ugur, J. Lainema (Nokia) |
| JCTVC-N0046 | m29624 | 2013-07-11 00:58:49 | 2013-07-16 07:37:49 | 2013-07-22 23:20:06 | SCE1: Results of Test 1.2.1 on Adaptive Re-Sampling Filter | X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-N0047 | m29625 | 2013-07-11 01:58:23 | 2013-07-16 07:36:34 | 2013-07-22 23:11:54 | SCE3: Results of Test 3.2 on Adaptive Re-Sampling Filter | W. Pu, X. Li, J. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-N0048 | m29626 | 2013-07-11 17:46:24 | 2013-07-12 23:35:19 | 2013-07-12 23:35:19 | Extensions to support layer addition and removal, access unit structure and changes to HRD model in scalable HEVC | S. Narasimhan, A. Luthra (Arris) |
| JCTVC-N0049 | m29627 | 2013-07-11 17:50:50 | 2013-07-12 23:35:56 | 2013-07-24 22:59:46 | Consideration of buffer management issues and layer management in HEVC scalability | S. Narasimhan, A. Luthra (Arris), K.Sato (Sony Corp), A. Tabatabai (Sony Electronics Inc) |
| JCTVC-N0050 | m29628 | 2013-07-11 20:39:28 | 2013-07-16 16:41:02 | 2013-07-25 12:08:51 | Specification text to support AVC base layer in HEVC layered extensions | J. Boyce (Vidyo), K. Kawamura (KDDI) |
| JCTVC-N0051 | m29629 | 2013-07-11 20:39:30 | 2013-07-11 23:17:11 | 2013-07-25 12:09:23 | MV-HEVC/SHVC HLS: ViewId and view position index | J. Boyce (Vidyo) |
| JCTVC-N0052 | m29632 | 2013-07-12 00:15:54 | 2013-07-16 08:34:02 | 2013-07-16 08:34:02 | RCE2 subtest C.2: Extension of residual DPCM to lossy coding | R. Joshi, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0053 | m29633 | 2013-07-12 00:19:12 | 2013-07-16 08:35:54 | 2013-07-16 08:35:54 | RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks | R. Joshi, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0054 | m29651 | 2013-07-12 07:45:26 | 2013-07-12 08:05:32 | 2013-07-25 15:22:34 | Signalling and restriction for scaled reference layer offsets | T. Tsukuba, T. Yamamoto (Sharp) |
| JCTVC-N0055 | m29652 | 2013-07-12 07:45:30 | 2013-07-12 08:06:33 | 2013-07-24 02:49:36 | On resampling process for outside-bounds samples | T. Tsukuba, T. Yamamoto, T. Ikai (Sharp) |
| JCTVC-N0056 | m29653 | 2013-07-12 07:45:32 | 2013-07-12 08:59:34 | 2013-07-27 10:52:43 | MV-HEVC/SHVC HLS: On inter-layer reference picture output marking | T. Yamamoto, T. Tsukuba, T. Ikai (Sharp) |
| JCTVC-N0057 | m29654 | 2013-07-12 07:51:51 | 2013-07-12 09:00:29 | 2013-07-28 17:04:36 | MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic | T. Ikai, T. Uchiumi (Sharp) |
| JCTVC-N0058 | m29655 | 2013-07-12 07:51:55 | 2013-07-12 09:00:55 | 2013-07-12 09:00:55 | MV-HEVC/SHVC HLS: On dependency type | T. Ikai, T. Uchiumi (Sharp) |
| JCTVC-N0059 | m29656 | 2013-07-12 07:51:59 | 2013-07-12 09:01:15 | 2013-07-25 13:54:53 | MV-HEVC/SHVC HLS: On slice segment header extension | T. Ikai, T. Uchiumi (Sharp) |
| JCTVC-N0060 | m29657 | 2013-07-12 07:52:03 | 2013-07-12 09:01:34 | 2013-07-25 13:55:28 | MV-HEVC/SHVC HLS: TemporalID alignment and inter-layer prediction restriction | T. Ikai, Y. Yamamoto (Sharp) |
| JCTVC-N0061 | m29658 | 2013-07-12 10:47:34 | 2013-07-15 00:16:36 | 2013-07-25 21:59:29 | Non-SCE3.3: Inter-layer interpolation-based SAO filtering for SHVC | A. Filippov, V. Rufitskiy (Huawei) |
| JCTVC-N0062 | m29659 | 2013-07-12 14:02:06 | 2013-07-12 14:34:04 | 2013-07-12 14:34:04 | MV-HEVC/SHVC HLS: access unit boundary detection | M. M. Hannuksela (Nokia) |
| JCTVC-N0063 | m29661 | 2013-07-12 14:06:19 | 2013-07-12 14:34:35 | 2013-08-02 08:15:03 | REXT/MV-HEVC/SHVC HLS: auxiliary picture layers | M. M. Hannuksela (Nokia) |
| JCTVC-N0064 | m29663 | 2013-07-12 14:12:36 | 2013-07-12 14:35:07 | 2013-07-29 14:02:07 | MV-HEVC/SHVC HLS: on storage of motion fields | M. M. Hannuksela (Nokia) |
| JCTVC-N0065 | m29665 | 2013-07-12 14:15:29 | 2013-07-12 14:35:41 | 2013-08-01 13:18:38 | MV-HEVC/SHVC HLS: On IDR picture constraints | M. M. Hannuksela (Nokia) |
| JCTVC-N0066 | m29667 | 2013-07-12 14:18:33 | 2013-07-12 14:36:10 | 2013-08-01 13:03:15 | MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process | M. M. Hannuksela (Nokia) |
| JCTVC-N0067 | m29669 | 2013-07-12 14:22:29 | 2013-07-15 18:37:59 | 2013-07-26 18:02:29 | MV-HEVC/SHVC HLS: on associating ViewId with nuh_layer_id and camera position | M. M. Hannuksela (Nokia), L. Chen (USTC) |
| JCTVC-N0068 | m29672 | 2013-07-12 14:58:10 | 2013-07-12 15:17:09 | 2013-07-12 15:17:09 | Inter-layer prediction modes based on base layer sharpness filter | Maxim Sychev, Valery Anisimovskiy, Sergey Ikonin |
| JCTVC-N0069 | m29673 | 2013-07-12 15:20:28 | 2013-07-12 15:22:57 | 2013-07-30 12:27:18 | Motion and inter-layer prediction constrained SEI message | K. Ugur, M. M. Hannuksela (Nokia) |
| JCTVC-N0070 | m29674 | 2013-07-12 15:31:39 | 2013-07-12 15:39:41 | 2013-07-26 19:38:05 | Non-SCE3: Inter-layer prediction modes based on base layer sharpness filter | M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei) |
| JCTVC-N0071 | m29675 | 2013-07-12 16:31:41 | 2013-07-15 18:07:30 | 2013-07-26 19:32:31 | RCE2: Simplified sample-based weighted prediction (Test C.3) | P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg) |
| JCTVC-N0072 | m29676 | 2013-07-12 16:32:13 | 2013-07-15 18:08:04 | 2013-07-26 19:33:10 | RCE2-related: Variants of simplified sample-based weighted prediction | P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg) |
| JCTVC-N0073 | m29677 | 2013-07-12 16:32:52 | 2013-07-15 18:08:50 | 2013-07-15 18:08:50 | RCE2: Description and report for Test D.3 | E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens), R. Joshi, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0074 | m29678 | 2013-07-12 17:24:55 | 2013-07-12 17:33:51 | 2013-07-27 09:42:31 | RCE2: Experimental results for Test C.1 | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0075 | m29679 | 2013-07-12 17:56:06 | 2013-07-12 18:01:54 | 2013-07-27 09:54:39 | Non-RCE2: Complexity reduction for inter residual DPCM in lossless coding | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0076 | m29680 | 2013-07-12 18:07:26 | 2013-07-12 18:09:36 | 2013-07-12 18:09:36 | RCE2: Cross-check for JCTVC-N0096, Test B.2: Swapping of scan order for transform skip/bypass | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0077 | m29681 | 2013-07-12 18:13:14 | 2013-07-12 18:17:16 | 2013-07-27 09:57:04 | AHG 5: On support for alpha channel in HEVC | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0078 | m29682 | 2013-07-13 03:35:52 | 2013-07-13 08:22:35 | 2013-07-26 10:59:33 | SCE1: Test 2.2 report | K. Minoo, D. Baylon (Arris) |
| JCTVC-N0079 | m29683 | 2013-07-14 04:23:34 | 2013-07-16 08:32:20 | 2013-07-27 22:09:41 | Non-RCE2: Simplified sample based intra prediction for lossless coding | J. Zhu, W. Zheng, K. Kazui (Fujitsu) |
| JCTVC-N0080 | m29684 | 2013-07-14 07:09:00 | 2013-07-16 08:32:44 | 2013-07-27 21:59:17 | Non-RCE2: Skip of neighbouring samples filtering in intra prediction for lossless coding | J. Zhu, K. Kazui (Fujitsu) |
| JCTVC-N0081 | m29685 | 2013-07-14 12:06:51 | 2013-07-14 12:21:47 | 2013-07-26 06:38:57 | MV-HEVC/SHVC HLS: On inter-layer prediction related syntax | J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony) |
| JCTVC-N0082 | m29686 | 2013-07-14 12:08:07 | 2013-07-14 12:32:19 | 2013-07-26 07:16:57 | MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions | O. Nakagami, T. Suzuki (Sony) |
| JCTVC-N0083 | m29694 | 2013-07-14 20:39:49 | 2013-07-16 08:37:08 | 2013-08-02 11:06:21 | RCE2 subtest D.1: Extension of intra and inter residual DPCM to lossy coding | R. Joshi, J. Sole, M. Karczewicz (Qualcomm), M. Naccari, M. Mrak (BBC) |
| JCTVC-N0084 | m29695 | 2013-07-14 21:28:24 | 2013-07-16 02:53:31 | 2013-07-16 02:53:31 | MV-HEVC/SHVC HLS: On various cross-layer alignments | Y.-K. Wang, A. K. Ramasubramonian, J. Chen, Hendry (Qualcomm) |
| JCTVC-N0085 | m29696 | 2013-07-14 21:31:02 | 2013-07-16 02:54:05 | 2013-07-29 17:49:38 | MV-HEVC/SHVC HLS: On parameter sets | Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm) |
| JCTVC-N0086 | m29699 | 2013-07-14 23:34:18 | 2013-07-16 18:13:27 | 2013-07-16 18:13:27 | Independent tile upsampling for SHVC | K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI) |
| JCTVC-N0087 | m29700 | 2013-07-14 23:34:58 | 2013-07-16 12:30:45 | 2013-07-16 12:30:45 | Inter-layer constrained tile sets SEI message | K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI) |
| JCTVC-N0088 | m29701 | 2013-07-14 23:35:42 | 2013-07-16 12:31:21 | 2013-07-16 12:31:21 | Extension of (temporal) motion constrained tile sets SEI message | K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI) |
| JCTVC-N0089 | m29702 | 2013-07-15 00:44:43 | 2013-07-16 10:02:01 | 2013-07-16 10:02:01 | MV-HEVC/SHVC HLS: On signalling of scaled reference offset | J. Chen, A.-K. Ramasubramonian, Y.-K. Wang, Y. Chen, X. Li (Qualcomm) |
| JCTVC-N0090 | m29703 | 2013-07-15 01:21:35 | 2013-07-16 11:18:25 | 2013-07-16 11:18:25 | MV-HEVC/SHVC HLS: Cross-layer non-alignment of IRAP pictures | A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm) |
| JCTVC-N0091 | m29704 | 2013-07-15 01:29:29 | 2013-07-16 11:10:21 | 2013-07-16 11:10:21 | MV-HEVC/SHVC HLS: DPB-related parameters in SPS and VPS | A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCTVC-N0092 | m29705 | 2013-07-15 01:33:01 | 2013-07-16 11:04:34 | 2013-07-28 10:01:51 | MV-HEVC/SHVC HLS: Representation format information in VPS | A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), J. Boyce (Vidyo), S. Deshpande (Sharp) |
| JCTVC-N0093 | m29709 | 2013-07-15 01:43:26 | 2013-07-16 11:39:33 | 2013-08-02 00:50:35 | MV-HEVC/SHVC HLS On DPB operations | A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia), S. Deshpande (Sharp) |
| JCTVC-N0094 | m29711 | 2013-07-15 01:45:29 | 2013-07-16 10:55:39 | 2013-07-29 19:13:48 | HEVC version 1 (corrigendum): Derivation of CPB removal time | A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCTVC-N0095 | m29712 | 2013-07-15 01:49:27 | 2013-07-16 13:49:50 | 2013-07-16 13:49:50 | MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization | A. K. Ramasubramonian, Y. Chen, L. Zhang (Qualcomm) |
| JCTVC-N0096 | m29714 | 2013-07-15 02:10:53 | 2013-07-15 23:37:08 | 2013-07-27 19:27:04 | RCE2: Test B.2: Swapping of scan order for transform skip/bypass | R. Cohen, A. Vetro (MERL) |
| JCTVC-N0097 | m29715 | 2013-07-15 02:11:00 | 2013-07-15 23:37:22 | 2013-07-15 23:37:22 | RCE2: Cross-verification of JCTVC-N0044, Test B.1: Residue rotation and significance map context | R. Cohen (MERL) |
| JCTVC-N0098 | m29716 | 2013-07-15 02:11:06 | 2013-07-15 23:37:35 | 2013-07-15 23:37:35 | RCE2: Cross-verification of JCTVC-N0073, Description and report for Test D.3 | R. Cohen (MERL) |
| JCTVC-N0099 | m29717 | 2013-07-15 02:52:26 | 2013-07-16 18:10:07 | 2013-07-16 18:10:07 | RCE2: Cross-verification report for subtest B.3 (N0175) on mirroring of coefficients | R. Joshi (Qualcomm) |
| JCTVC-N0100 | m29718 | 2013-07-15 03:01:50 | 2013-07-15 18:40:12 | 2013-07-28 09:46:53 | Non-RCE2: Unified lossless residual coding | Y. H. Tan, C. Yeo (I2R) |
| JCTVC-N0101 | m29719 | 2013-07-15 04:00:18 | 2013-07-16 08:35:19 | 2013-07-16 08:35:19 | RCE2 Test A.3: Simplified update of the coefficient level Rice parameter | J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0102 | m29720 | 2013-07-15 04:24:14 | 2013-07-15 10:20:13 | 2013-07-15 10:20:13 | MV-HEVC/SHVC HLS: On alternative collocated picture | Y. Lin (HiSilicon), J. Zan (Huawei) |
| JCTVC-N0103 | m29722 | 2013-07-15 04:32:11 | 2013-07-18 08:50:07 | 2013-07-18 08:50:07 | RCE2: Cross-verification of Test C.1 | S. Lee, C. Kim (Samsung) |
| JCTVC-N0104 | m29723 | 2013-07-15 04:33:57 | 2013-07-18 09:59:13 | 2013-07-18 09:59:13 | RCE3: Cross-verification of Test 3.1 | S. Lee, C. Kim (Samsung) |
| JCTVC-N0105 | m29724 | 2013-07-15 05:24:06 | 2013-07-16 08:36:39 | 2013-07-16 08:36:39 | RCE2: Cross-check of Test A.2 on coefficient level Rice parameter update (JCTVC-N0228) | J. Sole (Qualcomm) |
| JCTVC-N0106 | m29726 | 2013-07-15 06:06:06 | 2013-07-15 13:50:47 | 2013-07-22 08:04:19 | Generalized residual prediction with motion vector clipping for SHVC | K. Kim, H. Jo, J. Ryu, D. Sim, S.-J. Oh (KWU) |
| JCTVC-N0107 | m29727 | 2013-07-15 06:33:59 | 2013-07-16 09:11:41 | 2013-07-16 09:11:41 | MV-HEVC/SHVC HLS: On collocated picture indication and inter_layer_sample_pred_only_flag | V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-N0108 | m29728 | 2013-07-15 06:34:12 | 2013-07-16 09:05:00 | 2013-08-01 20:50:29 | SHVC HLS: Avoid resampling process for pictures used only for inter-layer motion prediction | V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-N0109 | m29729 | 2013-07-15 06:34:30 | 2013-07-16 09:13:51 | 2013-07-30 11:25:15 | MV-HEVC/SHVC HLS: Signalling for sub-layer dependency | V. Seregin, Y.-K. Wang (Qualcomm) |
| JCTVC-N0110 | m29730 | 2013-07-15 06:34:46 | 2013-07-16 09:07:39 | 2013-07-16 09:07:39 | SHVC HLS: Earlier DPB clearing for adaptive resolution change | V. Seregin, Y. Chen, Y.-K. Wang (Qualcomm) |
| JCTVC-N0111 | m29731 | 2013-07-15 06:35:00 | 2013-07-16 08:58:14 | 2013-07-16 08:58:14 | Sample position mapping with already defined scalling factor | V. Seregin, J. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-N0112 | m29732 | 2013-07-15 06:35:47 | 2013-07-16 07:50:59 | 2013-07-29 13:02:30 | MV-HEVC/SHVC HLS: High-level syntax for temporal motion vector prediction | H. Nakamura, M. Ueda, H. Takehara, S. Fukushima (JVC Kenwood) |
| JCTVC-N0113 | m29736 | 2013-07-15 06:55:07 | 2013-07-15 12:00:51 | 2013-07-28 15:43:12 | Cross Residual DPCM for HEVC lossless coding | Y.-L. Lee, S.-W. Hong (Sejong Univ.) |
| JCTVC-N0114 | m29737 | 2013-07-15 07:01:00 | 2013-07-25 05:16:33 | 2013-07-25 05:16:33 | RCE1: Cross-check on Experiment 5 | A. Minezawa, S. Sekiguchi (Mitsubishi) |
| JCTVC-N0115 | m29738 | 2013-07-15 07:03:23 | 2013-07-23 02:45:51 | 2013-07-31 15:44:28 | AHG5/AHG8: On RGB to YCbCr conversion for screen contents | A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi) |
| JCTVC-N0116 | m29739 | 2013-07-15 07:03:36 | 2013-07-18 15:14:35 | 2013-07-28 16:27:29 | AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions | A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi) |
| JCTVC-N0117 | m29740 | 2013-07-15 07:33:31 | 2013-07-15 07:38:40 | 2013-07-25 12:09:12 | HLS: Extensions to Motion-constrained tile sets SEI message | S. Hattori, O. Nakagami, T. Suzuki (Sony) |
| JCTVC-N0118 | m29744 | 2013-07-15 07:46:01 | 2013-07-16 10:04:39 | 2013-07-29 13:57:04 | MV-HEVC/SHVC HLS: On Inter layer Prediction Signalling | J. Chen, Y. Chen, Hendry, Y.-K. Wang, K. Rapaka (Qualcomm), O. Nakagami, T. Suzuki (Sony) |
| JCTVC-N0119 | m29745 | 2013-07-15 08:35:09 | 2013-07-15 16:41:38 | 2013-07-26 08:59:29 | MV-HEVC/SHVC HLS:On collocated picture indication | H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI) |
| JCTVC-N0120 | m29746 | 2013-07-15 08:39:52 | 2013-07-15 16:42:27 | 2013-07-26 09:25:39 | MV-HEVC/SHVC HLS:Signalling for Inter-layer prediction indication | H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI) |
| JCTVC-N0121 | m29747 | 2013-07-15 08:45:25 | 2013-07-16 09:11:16 | 2013-07-23 13:17:19 | MV-HEVC/SHVC HLS: Random access of multiple layers | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0122 | m29748 | 2013-07-15 08:45:51 | 2013-07-15 14:31:24 | 2013-07-15 14:31:24 | SCE2: Cross-check of test2.3 on motion field buffer update | J. Lee, H. Lee, J. W. Kang (ETRI) |
| JCTVC-N0123 | m29749 | 2013-07-15 09:02:44 | 2013-07-24 15:12:10 | 2013-07-24 15:12:15 | RCE2: Cross-check of Test A.2 | X. Wei, J. Zan (Huawei) |
| JCTVC-N0124 | m29750 | 2013-07-15 09:05:02 | 2013-07-16 09:11:43 | 2013-07-23 13:18:00 | MV-HEVC/SHVC HLS: Random layer access | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0125 | m29751 | 2013-07-15 09:05:59 | 2013-07-24 15:13:08 | 2013-07-24 15:13:08 | RCE3: Cross-check of Test 3.2 | X. Wei, J. Zan (Huawei) |
| JCTVC-N0126 | m29752 | 2013-07-15 09:06:18 | 2013-07-24 15:13:28 | 2013-07-24 15:13:28 | RCE3: Cross-check of Test 3.3 | X. Wei, J. Zan (Huawei) |
| JCTVC-N0127 | m29753 | 2013-07-15 09:06:46 | 2013-07-16 09:12:14 | 2013-07-23 08:40:56 | MV-HEVC/SHVC HLS: On decoded picture buffer | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0128 | m29754 | 2013-07-15 09:09:33 | 2013-07-16 09:12:40 | 2013-07-18 06:19:27 | MV-HEVC/SHVC HLS: Reference picture marking and picture removal | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0129 | m29755 | 2013-07-15 09:11:14 | 2013-07-16 09:13:06 | 2013-07-22 15:38:36 | MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0130 | m29756 | 2013-07-15 09:12:19 | 2013-07-16 09:13:39 | 2013-07-22 18:48:55 | MV-HEVC/SHVC HLS: On temporal sub-layer management | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0131 | m29757 | 2013-07-15 09:13:30 | 2013-07-16 12:10:28 | 2013-08-01 15:59:14 | MV-HEVC/SHVC HLS: On interlayer reference picture set | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0132 | m29758 | 2013-07-15 09:14:25 | 2013-07-16 09:14:33 | 2013-07-23 08:42:00 | MV-HEVC/SHVC HLS: On interlayer prediction type | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0133 | m29759 | 2013-07-15 09:15:45 | 2013-07-18 14:03:05 | 2013-07-19 06:19:54 | HLS: Frame packing arragement SEI message for bit depth extension | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0134 | m29760 | 2013-07-15 09:17:23 | 2013-07-18 14:03:36 | 2013-07-19 06:20:21 | HLS: Frame packing arragement SEI message for full resolution 3D | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0135 | m29761 | 2013-07-15 09:18:54 | 2013-07-19 08:48:04 | 2013-07-25 17:14:41 | MV-HEVC/SHVC HLS: Extension of maximum number of layers | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0136 | m29762 | 2013-07-15 09:44:57 | 2013-07-18 09:15:40 | 2013-07-18 09:15:40 | RCE2: Cross-check of Sub-test D.1 (JCTVC-N0083) by the BBC and Qualcomm | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-N0137 | m29763 | 2013-07-15 09:47:04 | 2013-07-15 13:57:44 | 2013-07-15 13:57:44 | Non-RCE2: Golomb-rice parameter initialization for transform-skip and transquant-bypass modes | V. Kolesnikov, C. Rosewarne, M. Maeda (Canon) |
| JCTVC-N0138 | m29764 | 2013-07-15 09:48:24 | 2013-07-15 13:58:20 | 2013-07-27 10:08:47 | AHG5: Square transform deblocking for 4:2:2 | C. Rosewarne, V. Kolesnikov, M. Maeda (Canon) |
| JCTVC-N0139 | m29765 | 2013-07-15 09:59:32 | 2013-07-15 18:19:36 | 2013-07-15 18:19:36 | SCE2: Results on test 2.1 | C. Gisquet, P. Onno, E. Francois, G. Laroche (Canon) |
| JCTVC-N0140 | m29766 | 2013-07-15 09:59:49 | 2013-07-15 18:17:31 | 2013-07-22 11:53:19 | SCE3: Results of test 3.3 on Interlayer SAO filtering for SHVC | G. Laroche, J. Taquet, P. Onno (Canon) |
| JCTVC-N0141 | m29767 | 2013-07-15 10:00:11 | 2013-07-15 18:16:24 | 2013-07-27 16:02:30 | AHG5: on chroma QP for HEVC RExt | E. Francois, C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-N0142 | m29768 | 2013-07-15 10:02:16 | 2013-07-15 18:57:58 | 2013-07-23 14:47:27 | AHG18: On 16-bits support for Range Extensions | E. Francois, J. Taquet, P. Onno, G. Laroche, C. Gisquet (Canon) |
| JCTVC-N0143 | m29769 | 2013-07-15 10:06:38 | 2013-07-15 18:15:46 | 2013-07-25 14:17:22 | On Mode Dependent Intra Smoothing for Range Extension | G. Laroche, C. Gisquet, T. Poirier (Canon) |
| JCTVC-N0144 | m29770 | 2013-07-15 10:18:59 | 2013-07-15 15:15:16 | 2013-07-15 15:15:16 | AHG13: Slice based upsampling filter for improved error resiliency | K. Ugur, M. M. Hannuksela (Nokia) |
| JCTVC-N0145 | m29771 | 2013-07-15 10:20:11 | 2013-07-15 17:23:02 | 2013-07-29 14:36:29 | AHG20: Backwards compatible enhancement of chroma format | K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia) |
| JCTVC-N0146 | m29772 | 2013-07-15 10:21:09 | 2013-07-15 17:22:14 | 2013-07-26 15:51:30 | AHG14: On resampling & color gamut scalability | K. Ugur, A. Aminlou (Nokia) |
| JCTVC-N0147 | m29773 | 2013-07-15 10:26:52 | 2013-07-16 10:06:14 | 2013-07-16 10:06:14 | MV-HEVC/SHVC HLS: On restriction and indication of cross-layer IRAP picture distribution | J. Chen, Y.-K. Wang, K. Rapaka, A.-K. Ramasubramonian, Hendry (Qualcomm) |
| JCTVC-N0148 | m29774 | 2013-07-15 11:00:51 | 2013-07-15 18:00:59 | 2013-07-25 16:10:12 | AhG8: Guided Image Filtering for Screen Content Coding | T. Vermeir (Barco), J. De Cock, G. Van Wallendael, S. Van Leuven (Ghent Univ. - iMinds) |
| JCTVC-N0149 | m29776 | 2013-07-15 12:07:12 | 2013-07-16 12:18:02 | 2013-07-16 12:18:02 | Non-SCE1: Results of test 1.2 on sampling offset signalling with accurate interpolation filter | E.Alshina, A.Alshin (Samsung) |
| JCTVC-N0150 | m29777 | 2013-07-15 12:07:42 | 2013-07-16 12:17:00 | 2013-07-26 07:42:11 | AhG17: complexity analysis of SHM2.0 | E.Alshina, A.Alshin (Samsung) |
| JCTVC-N0151 | m29778 | 2013-07-15 12:08:08 | 2013-07-16 12:25:16 | 2013-07-26 07:40:38 | SCE3: Results of test 3.1 on switchable alternative inter-layer filter | P. Wei, V. Seregin, J. Chen, X. Li, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin (Samsung) |
| JCTVC-N0152 | m29779 | 2013-07-15 12:08:28 | 2013-07-16 12:36:41 | 2013-07-24 00:32:13 | SCE3: Results of test 3.4.2 on simplified cross-color inter-layer inter layer prediction | E.Alshina, A.Alshin, Y.Cho (Samsung), J.Dong, Y.Ye, Y.He (InterDigital) |
| JCTVC-N0153 | m29780 | 2013-07-15 12:08:48 | 2013-07-16 12:46:23 | 2013-07-16 12:46:23 | On adaptive re-sampling filter coefficients coding | E. Alshina, A. Alshin (Samsung) |
| JCTVC-N0154 | m29781 | 2013-07-15 12:36:00 | 2013-07-15 14:20:47 | 2013-08-01 10:37:48 | MV-HEVC/SHVC HLS: On signalling of inter-layer RPS in slice segment header | J.W. Kang, H. Lee, J. Lee, J.S. Choi (ETRI) |
| JCTVC-N0155 | m29782 | 2013-07-15 12:46:51 | 2013-07-16 07:22:53 | 2013-07-23 06:50:14 | HLS: Thumbnail Support In HEVC | C. Wang, C. Wang, W. Zhang, Y. Chiu (Intel) |
| JCTVC-N0156 | m29783 | 2013-07-15 12:47:55 | 2013-07-22 06:59:19 | 2013-07-22 06:59:19 | RCE2: Crosscheck for test C.3 on simplified sample-based weighted prediction (JCTVC-N0071) | W. Zhang, Y. Chiu (Intel) |
| JCTVC-N0157 | m29784 | 2013-07-15 12:52:47 | 2013-07-15 14:21:21 | 2013-08-01 10:57:38 | MV-HEVC/SHVC HLS: On signalling of sps_max_sub_layers_minus1 | J.W. Kang, H. Lee, J. Lee, J.S. Choi (ETRI) |
| JCTVC-N0158 | m29785 | 2013-07-15 13:34:38 | 2013-07-16 09:17:08 | 2013-07-16 09:17:08 | MV-HEVC/SHVC HLS: Bitstream restrictions on tiles and wavefronts across layers | K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, J. Chen (Qualcomm) |
| JCTVC-N0159 | m29786 | 2013-07-15 13:36:08 | 2013-07-16 09:21:36 | 2013-07-16 09:21:36 | MV-HEVC/SHVC HLS: Parallel Processing Indications for Tiles in HEVC Extensions | K. Rapaka, X. Li, J. Chen, W. Pu, Y.-K. Wang, M. Karczewicz (Qualcomm) |
| JCTVC-N0160 | m29787 | 2013-07-15 13:37:22 | 2013-07-16 09:34:28 | 2013-07-31 17:13:29 | MV-HEVC/SHVC HLS: On signalling of offset delay parameters and tile alignment | K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-N0161 | m29788 | 2013-07-15 13:37:49 | 2013-07-16 11:39:01 | 2013-07-27 00:33:18 | Using decoded pictures from higher layer as references in SHVC | K. Rapaka, J. Chen, M. Karczewicz (Qualcomm) |
| JCTVC-N0162 | m29789 | 2013-07-15 13:51:05 | 2013-07-15 17:21:53 | 2013-07-15 17:21:53 | MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions | M. Pettersson, T. Rusert (Ericsson) |
| JCTVC-N0163 | m29790 | 2013-07-15 13:52:36 | 2013-07-15 13:56:41 | 2013-07-19 19:01:10 | AHG14: Wide Color Gamut Test Material Creation | P. Andrivon, P. Bordes (Technicolor) |
| JCTVC-N0164 | m29791 | 2013-07-15 13:59:46 | 2013-07-15 18:09:52 | 2013-07-15 18:09:52 | RCE2: Cross-check results for Test C.4 (JCTVC-N0053) | E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens) |
| JCTVC-N0165 | m29794 | 2013-07-15 15:30:47 | 2013-07-16 07:49:42 | 2013-07-23 07:17:51 | On VPS extension | Y. Cho, B. Choi, M. Park, J. Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0166 | m29795 | 2013-07-15 15:31:23 | 2013-07-16 07:43:33 | 2013-07-18 11:34:14 | Signalling of inter-layer filter selection |  |
| JCTVC-N0167 | m29796 | 2013-07-15 15:45:05 | 2013-07-19 17:30:47 | 2013-07-30 09:09:35 | Transform skip based on minimum TU size | K. Won, S. Yang, B. Jeon (SKKU) |
| JCTVC-N0168 | m29797 | 2013-07-15 16:23:11 | 2013-07-15 18:53:30 | 2013-07-24 10:28:54 | AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results | P. Bordes, P. Andrivon, F. Hiron (Technicolor) |
| JCTVC-N0169 | m29798 | 2013-07-15 16:41:20 | 2013-07-15 17:29:47 | 2013-07-30 16:24:45 | Non-RCE3:Template-based palette prediction | W. Zhu, H. Yang (Huawei) |
| JCTVC-N0170 | m29800 | 2013-07-15 16:58:08 | 2013-07-19 14:32:36 | 2013-07-19 14:32:36 | RCE1: Description and Results for Experiment 3, 5, and 6 | T. Nguyen (Fraunhofer HHI) |
| JCTVC-N0171 | m29801 | 2013-07-15 16:58:36 | 2013-07-19 16:02:43 | 2013-07-19 16:02:43 | RCE1: Cross Check Results for Experiment 1 and 4 | T. Nguyen (Fraunhofer HHI) |
| JCTVC-N0172 | m29802 | 2013-07-15 17:20:14 | 2013-07-15 17:28:08 | 2013-07-15 17:28:08 | MV-HEVC/SHVC HLS: Layer-wise DPB operation and size indications | M. M. Hannuksela (Nokia) |
| JCTVC-N0173 | m29804 | 2013-07-15 17:39:32 | 2013-07-15 19:10:52 | 2013-07-16 09:14:49 | MV-HEVC/SHVC HLS: On Layers Present SEI message | J. W. Kang (ETRI), J. Lee, H. Lee, J. S. Choi, T. C. Thang (UoA) |
| JCTVC-N0174 | m29805 | 2013-07-15 17:42:47 | 2013-07-15 19:11:26 | 2013-07-16 09:16:44 | MV-HEVC/SHVC HLS: On Layer Dependency Change SEI message | T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI) |
| JCTVC-N0175 | m29806 | 2013-07-15 17:44:09 | 2013-07-16 00:21:29 | 2013-07-26 23:48:34 | RCE2 Test B.3: Mirroring of Coefficients and significance map context | R. Weerakkody, M. Mrak (BBC) |
| JCTVC-N0176 | m29807 | 2013-07-15 17:45:39 | 2013-07-16 01:05:51 | 2013-07-31 10:34:21 | Non-RCE2: On sample adaptive intra prediction for oblique modes in lossless coding | H. Chen, A. Saxena, F. Fernandes (Samsung) |
| JCTVC-N0177 | m29808 | 2013-07-15 17:46:28 | 2013-07-16 19:06:47 | 2013-07-31 10:38:35 | Non-RCE2: On sample adaptive intra prediction for oblique modes in lossy coding | A. Saxena, H. Chen, F. Fernandes (Samsung) |
| JCTVC-N0178 | m29809 | 2013-07-15 17:48:05 | 2013-07-15 18:12:47 | 2013-07-16 11:26:50 | HEVC profiles for medical imaging applications | P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens) |
| JCTVC-N0179 | m29810 | 2013-07-15 17:48:19 | 2013-07-30 12:19:30 | 2013-07-30 12:19:30 | Selected medical imaging sequences for HEVC development | P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens) |
| JCTVC-N0180 | m29811 | 2013-07-15 18:11:09 | 2013-07-15 18:14:49 | 2013-07-24 14:23:39 | Color Mapping SEI message | P. Bordes, P. Andrivon, P. Lopez, F. Hiron (Technicolor) |
| JCTVC-N0181 | m29812 | 2013-07-15 18:19:48 | 2013-07-16 10:16:13 | 2013-07-24 06:54:48 | Non-RCE2: Rice Parameter Initialization | M. Karczewicz, J. Sole, R. Joshi (Qualcomm) |
| JCTVC-N0182 | m29813 | 2013-07-15 18:31:03 | 2013-07-15 19:16:46 | 2013-07-15 19:16:46 | SCE1: Results of test 1.1 and 1.2 on sampling offset signalling | J. Dong, Y. Ye, Y. He (InterDigital), X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), K. Ugur, J. Lainema (Nokia) |
| JCTVC-N0183 | m29814 | 2013-07-15 18:31:13 | 2013-07-16 07:45:35 | 2013-07-28 11:37:07 | Non-RCE2: Enhanced angular intra prediction for screen content coding | H. Chen, A. Saxena, F. Fernandes (Samsung) |
| JCTVC-N0184 | m29815 | 2013-07-15 18:32:01 | 2013-07-15 21:12:17 | 2013-07-26 00:55:07 | SCE3: Results of test 3.4.1 on chroma enhancement for inter layer prediction | J. Dong, Y. Ye, Y. He (InterDigital) |
| JCTVC-N0185 | m29816 | 2013-07-15 18:56:42 | 2013-07-16 00:19:37 | 2013-08-01 15:00:00 | On low-delay flag checking process of SHVC | X. Xiu, Y. Ye, Y. He, Y. He (InterDigital), Y. Lin, X. Zheng, X. Chen (HiSilicon) |
| JCTVC-N0186 | m29817 | 2013-07-15 19:00:53 | 2013-07-16 00:14:26 | 2013-08-01 15:04:49 | AHG16: Single-loop decoding of SNR scalability for refIdx based SHVC | X. Xiu, Y. Ye, Y. He, Y. He (InterDigital) |
| JCTVC-N0187 | m29818 | 2013-07-15 19:06:54 | 2013-07-16 01:01:18 | 2013-08-01 15:11:16 | Investigation on using H-ILP picture for refIdx based SHVC | X. Xiu, Y. Yan, Y. He (InterDigital) |
| JCTVC-N0188 | m29819 | 2013-07-15 19:11:28 | 2013-07-16 00:10:58 | 2013-07-29 14:31:54 | AHG 5 and 18: Internal Precision for High Bit Depths | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0189 | m29820 | 2013-07-15 19:13:03 | 2013-07-16 00:11:26 | 2013-07-26 16:13:50 | AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0190 | m29821 | 2013-07-15 19:13:58 | 2013-07-16 00:11:56 | 2013-07-26 16:15:14 | AHG 5 and 18: Entropy Coding Throughput for High Bit Depths | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0191 | m29822 | 2013-07-15 19:15:23 | 2013-07-16 00:12:26 | 2013-07-31 08:55:31 | AHG 5 and 18: Profiles for Range Extensions | K. Sharman, N. Saunders, J. Gamei, T. Suzuki (Sony) |
| JCTVC-N0192 | m29823 | 2013-07-15 19:16:23 | 2013-07-16 00:12:54 | 2013-07-26 16:16:15 | AHG 5: 32x32 Scaling List Derivation for Chroma | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0193 | m29824 | 2013-07-15 21:13:26 | 2013-07-16 20:03:21 | 2013-07-16 20:03:21 | SCE1: Cross-check for Test 2.2 on phase compensation by signalling filter coefficients at PPS with sample shift | J. Dong, Y. Ye (InterDigital) |
| JCTVC-N0194 | m29825 | 2013-07-15 21:14:10 | 2013-07-17 02:42:13 | 2013-07-17 02:42:13 | SCE3: Cross-check for Test 3.1 on switchable alternative inter-layer filter | J. Dong, Y. Ye (InterDigital) |
| JCTVC-N0195 | m29826 | 2013-07-15 21:15:58 | 2013-07-16 07:53:37 | 2013-07-30 07:48:32 | Comments On SHVC and MV-HEVC | S. Deshpande (Sharp) |
| JCTVC-N0196 | m29827 | 2013-07-15 21:17:25 | 2013-07-16 07:54:10 | 2013-07-18 19:04:37 | On Sub-layer Non-reference Pictures Indication for Inter-layer Prediction | S. Deshpande (Sharp) |
| JCTVC-N0197 | m29828 | 2013-07-15 21:18:15 | 2013-07-16 07:54:36 | 2013-07-16 07:54:36 | On Signalling DPB Parameters in VPS | S. Deshpande (Sharp) |
| JCTVC-N0198 | m29829 | 2013-07-15 21:18:50 | 2013-07-16 07:55:11 | 2013-07-16 07:55:11 | On DPB Operation | S. Deshpande (Sharp) |
| JCTVC-N0199 | m29830 | 2013-07-15 21:19:32 | 2013-07-16 07:55:42 | 2013-07-31 10:29:10 | On Tile Alignment | S. Deshpande (Sharp), K. Misra (Sharp) |
| JCTVC-N0200 | m29831 | 2013-07-15 21:19:33 | 2013-07-16 07:56:10 | 2013-07-27 11:37:03 | On Scaling List Data Signalling | S. Deshpande (Sharp), S. Liu (MediaTek), S. Lei (MediaTek), K. Sato (Sony) |
| JCTVC-N0201 | m29832 | 2013-07-15 22:32:49 | 2013-07-16 08:39:46 | 2013-07-26 10:47:19 | SAO extension for higher bit-depth coding | A. Tourapis (Apple) |
| JCTVC-N0202 | m29833 | 2013-07-16 00:02:34 | 2013-07-16 01:02:02 | 2013-08-01 11:29:14 | AHG16: Key picture concept and single loop decoding | C. Feldmann, F. Jäger, M. Wien (RWTH Aachen Univ.) |
| JCTVC-N0203 | m29834 | 2013-07-16 00:11:01 | 2013-07-27 09:40:47 | 2013-07-27 09:40:47 | AHG16: Cross-Check of JCTVC-N0186 Single-loop decoding of SNR scalability for refIdx based SHVC | C. Feldmann, M. Wien (RWTH Aachen Univ.) |
| JCTVC-N0204 | m29835 | 2013-07-16 00:13:46 | 2013-07-16 00:27:40 | 2013-07-23 23:12:59 | ILR enhancement with differential coding for SHVC reference index framework | Y. He, Y. Ye, X. Xiu (InterDigital) |
| JCTVC-N0205 | m29836 | 2013-07-16 00:14:13 | 2013-07-16 00:22:46 | 2013-07-27 12:39:20 | RCE3: Results of test 3.3 on Intra motion compensation | D.-K. Kwon, M. Budagavi (TI) |
| JCTVC-N0206 | m29837 | 2013-07-16 00:16:24 | 2013-07-16 02:56:50 | 2013-08-01 11:18:07 | Non-RCE3: Intra motion compensation with variable length intra MV coding | D.-K. Kwon, M. Budagavi (TI) |
| JCTVC-N0207 | m29838 | 2013-07-16 00:19:39 | 2013-07-16 17:56:35 | 2013-07-16 17:56:35 | RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) | R. Weerakkody, M. Mrak, M. Naccari (BBC) |
| JCTVC-N0208 | m29839 | 2013-07-16 00:24:50 | 2013-07-20 00:26:44 | 2013-07-20 00:26:44 | SCE3: Crosscheck of test 3.4.2 on simplified cross-color inter-layer prediction (JCTVC-N0152) | X. Li (Qualcomm) |
| JCTVC-N0209 | m29840 | 2013-07-16 00:27:11 | 2013-07-16 01:08:00 | 2013-07-25 14:40:59 | SHVC HLS: SHVC Skipped Picture Indication | J. Boyce (Vidyo), X. Xiu, Y. He, Y. Ye (InterDigital) |
| JCTVC-N0210 | m29841 | 2013-07-16 00:27:34 | 2013-07-16 01:08:22 | 2013-07-18 23:10:53 | SHVC HLS: Adaptation Parameter Set for SHVC | Y. He, J. Dong, Y. Ye (InterDigital) |
| JCTVC-N0211 | m29842 | 2013-07-16 00:28:13 | 2013-07-16 01:08:43 | 2013-07-24 21:55:19 | SHVC HLS: On AVC Base Layer GOP Structure Indication | Y. He, Y. Ye, Y. He (InterDigital) |
| JCTVC-N0212 | m29843 | 2013-07-16 00:28:39 | 2013-07-16 01:09:00 | 2013-07-25 09:15:12 | SHVC HLS: On Inter Layer Parameter Set | Y. He, Y. Ye, Y. He (InterDigital) |
| JCTVC-N0213 | m29844 | 2013-07-16 00:28:57 | 2013-07-16 01:09:16 | 2013-07-16 01:09:16 | SHVC HLS: On Resampling Buffer Indication | Y. He, Y. Ye (InterDigital) |
| JCTVC-N0214 | m29845 | 2013-07-16 00:29:05 | 2013-07-16 07:32:16 | 2013-07-29 17:37:37 | Non-SCE1: Dynamic range control of intermediate data in re-sampling process | J. Chen, X. Li, M. Karczewicz (Qualcommm) |
| JCTVC-N0215 | m29846 | 2013-07-16 00:29:21 | 2013-07-16 01:09:36 | 2013-07-25 09:15:45 | SHVC HLS: On SNR Scalability Indication | Y. He, Y. Ye (InterDigital) |
| JCTVC-N0216 | m29847 | 2013-07-16 00:29:46 | 2013-07-16 07:27:51 | 2013-07-25 09:16:11 | MV-HEVC/SHVC HLS: On Reference Picture List Modification | Y He, X. Xiu, Y Ye (InterDigital) |
| JCTVC-N0217 | m29848 | 2013-07-16 00:30:08 | 2013-07-16 07:28:11 | 2013-07-25 09:16:26 | MV-HEVC/SHVC HLS: On SHVC High Level Syntax | Y. He, X. Xiu, Y. Ye, Y. He (Interdigital) |
| JCTVC-N0218 | m29849 | 2013-07-16 00:48:55 | 2013-07-16 12:56:58 | 2013-07-29 12:43:31 | AhG14: On bit-depth scalability support | E.Alshina, A.Alshin (Samsung) |
| JCTVC-N0219 | m29850 | 2013-07-16 00:49:11 | 2013-07-16 12:00:22 | 2013-07-16 12:00:22 | Non-SCE1: On arbitrary spatial ratio scalability in SHVC | J.Chen, X.Li, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin (Samsung), K.Ugur (Nokia) |
| JCTVC-N0220 | m29851 | 2013-07-16 01:10:30 | 2013-07-16 02:36:59 | 2013-07-18 20:23:48 | SCE3: Results of test.3.5.1 on adaptive up-sampling of base layer picture using simplified separable bilateral filters | J. Zhao, K. Misra, A. Segall (Sharp) |
| JCTVC-N0221 | m29852 | 2013-07-16 01:14:36 | 2013-07-19 19:28:13 | 2013-07-19 19:28:13 | SCE3: Cross-check of test 3.1 on switchable alternative filters (all phases) | J. Zhao, A. Segall (Sharp) |
| JCTVC-N0222 | m29853 | 2013-07-16 01:19:41 | 2013-07-16 11:22:00 | 2013-07-23 23:37:55 | Non-RCE2: Results for combination of methods | J. Sole, R. Joshi, L. Guo, M. Karczewicz (Qualcomm) |
| JCTVC-N0223 | m29854 | 2013-07-16 01:23:17 | 2013-07-16 02:51:54 | 2013-07-25 19:00:27 | Non-RCE1: In-loop Chroma Enhancement for HEVC Range Extensions | J. Dong, Y. Ye, Y. He (InterDigital) |
| JCTVC-N0224 | m29855 | 2013-07-16 01:24:03 | 2013-07-16 02:02:53 | 2013-08-01 20:50:24 | SEI message: post filters to enhance the chroma planes | J. Dong, Y. Ye, Y. He (InterDigital) |
| JCTVC-N0225 | m29856 | 2013-07-16 01:46:31 | 2013-07-20 10:38:14 | 2013-07-20 10:38:14 | SCE1: Cross-check for Test1.2, The accuracy of signaled up-sampling phase offset | K. Minoo, D. Baylon (Arris) |
| JCTVC-N0226 | m29857 | 2013-07-16 01:48:34 | 2013-07-20 10:39:08 | 2013-07-26 11:02:13 | SCE1: Cross-check for Test1.3, accurate Chroma position alignment | K. Minoo, D. Baylon (Arris) |
| JCTVC-N0227 | m29858 | 2013-07-16 01:57:34 | 2013-07-16 02:47:49 | 2013-07-16 02:47:49 | RCE1: EXP1/4 The performance of extended chroma mode for non 4:2:0 format | J. Kim, B. Jeon (LG) |
| JCTVC-N0228 | m29859 | 2013-07-16 02:03:00 | 2013-07-16 02:22:35 | 2013-07-16 02:22:35 | RCE2: Results for Test A.2 Rice parameter update | M. Budagavi (TI) |
| JCTVC-N0229 | m29860 | 2013-07-16 02:05:10 | 2013-07-16 07:42:28 | 2013-07-27 09:20:10 | Non-SCE3: Region based Inter-layer Cross-Color Filtering | X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin, Y. Cho (Samsung) |
| JCTVC-N0230 | m29861 | 2013-07-16 02:12:14 | 2013-07-16 08:34:26 | 2013-07-26 08:49:56 | SAO software cleanup and non-normative encoder-only bug-fixes | C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-N0231 | m29862 | 2013-07-16 02:43:32 | 2013-07-16 04:58:49 | 2013-07-23 09:57:51 | AHG 8: Intra mode coding for screen contents | J.Min, S.Lee, C.Kim (Samsung ) |
| JCTVC-N0232 | m29864 | 2013-07-16 02:45:35 | 2013-07-16 04:59:36 | 2013-07-29 03:53:34 | Non-RCE2: Rice parameter update method | J. Min, S. Lee, C. Kim (Samsung) |
| JCTVC-N0233 | m29867 | 2013-07-16 02:53:00 | 2013-07-16 07:35:57 | 2013-07-16 07:35:57 | Non-SCE2: scaled MV in motion field buffer update | J. Xu, A. Tabatabai, K. Sato (Sony) |
| JCTVC-N0234 | m29868 | 2013-07-16 02:54:33 | 2013-07-16 07:40:24 | 2013-07-16 07:40:24 | Low-Complexity Generated Inter-layer Reference for SHVC | X. Li, J. Chen, M. Karczewicz (Qualcomm), |
| JCTVC-N0235 | m29869 | 2013-07-16 02:56:33 | 2013-07-16 07:37:03 | 2013-07-16 07:37:03 | Non-RCE3: base color merging for MBCIM | J. Xu, A. Tabatabai (Sony) |
| JCTVC-N0236 | m29871 | 2013-07-16 02:57:58 | 2013-07-16 08:41:38 | 2013-07-25 01:00:10 | HLS: Extensions of motion-constrained tile sets SEI message for interactivity | C. Auyeung, J. Xu (Sony) |
| JCTVC-N0237 | m29872 | 2013-07-16 02:58:35 | 2013-07-16 08:29:58 | 2013-07-16 08:29:58 | HEVC version 1: Level 2.2 for support of WVGA@30fps | Y.-K. Wang (Qualcomm) |
| JCTVC-N0238 | m29874 | 2013-07-16 05:01:35 | 2013-07-16 08:28:49 | 2013-07-16 08:28:49 | On Source Representation Information Signalling in VPS | S. Deshpande (Sharp) |
| JCTVC-N0239 | m29875 | 2013-07-16 05:02:14 | 2013-07-16 07:39:13 | 2013-07-19 07:30:29 | SCE2: Result of Test 1.1 | C. Gisquet (Canon), K. Sato, J. Xu (Sony) |
| JCTVC-N0240 | m29876 | 2013-07-16 05:03:34 | 2013-07-16 07:40:13 | 2013-07-19 07:31:30 | SCE2: Crosscheck Result of Test 1.2 | K. Sato (Sony) |
| JCTVC-N0241 | m29877 | 2013-07-16 05:06:07 | 2013-07-16 07:41:08 | 2013-07-19 07:32:26 | SCE2: Result of Test 1.4 | C. Gisquet (Canon), K. Sato, J. Xu (Sony), K. Misra (Sharp) |
| JCTVC-N0242 | m29878 | 2013-07-16 05:06:57 | 2013-07-16 07:28:47 | 2013-07-16 07:28:47 | Editorial improvements on SHVC Draft Text 2 | J. Chen, J. Boyce, Y. Ye, M. Hannuksela |
| JCTVC-N0243 | m29879 | 2013-07-16 05:07:50 | 2013-07-16 07:42:03 | 2013-07-19 07:33:29 | SCE2: Crosscheck Result of Test 2.1 | K. Sato (Sony) |
| JCTVC-N0244 | m29880 | 2013-07-16 05:09:03 | 2013-07-16 10:49:29 | 2013-07-16 10:49:29 | MV-HEVC/SHVC HLS: Cross-layer POC alignment | Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-N0245 | m29882 | 2013-07-16 05:11:04 | 2013-07-16 07:42:40 | 2013-07-29 03:48:31 | SCE2: Result of Test 2.2 | K. Sato, J. Xu (Sony) |
| JCTVC-N0246 | m29885 | 2013-07-16 07:30:20 | 2013-07-16 09:15:08 | 2013-07-25 09:11:59 | AHG5: Modified SAO for range extensions | S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-N0247 | m29886 | 2013-07-16 07:30:58 | 2013-07-16 07:32:49 | 2013-07-16 07:32:49 | RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding | L. Guo, M. Karczewicz, J. Sole (Qualcomm) |
| JCTVC-N0248 | m29887 | 2013-07-16 07:31:13 | 2013-07-16 07:44:01 | 2013-07-19 06:36:26 | Support of Field Coding for Signalling of Chroma Phase for Upsampling | K. Sato, A. Cheung (Sony) |
| JCTVC-N0249 | m29890 | 2013-07-16 07:35:17 | 2013-07-16 09:47:28 | 2013-07-16 09:47:28 | Non-RCE3: Modified Palette Mode for Screen Content Coding | L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm) |
| JCTVC-N0250 | m29892 | 2013-07-16 07:36:12 | 2013-07-16 08:46:01 | 2013-07-25 10:17:23 | Non-SCE3.3: Modified Interlayer SAO with highpass processing | S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-N0251 | m29894 | 2013-07-16 07:37:54 | 2013-07-17 22:55:08 | 2013-07-17 22:55:08 | SCE3 : cross-check of Test 3.4.1 on chroma enhancement for inter layer prediction | J. Xu (Sony) |
| JCTVC-N0252 | m29896 | 2013-07-16 07:39:16 | 2013-07-16 08:26:58 | 2013-08-01 10:41:30 | SCE2 test 2.3 motion buffer modification results | K. Misra, A. Segall, J. Zhao (Sharp) |
| JCTVC-N0253 | m29897 | 2013-07-16 07:39:21 | 2013-07-26 22:22:48 | 2013-07-26 22:22:48 | Cross-check of JCTVC-N0201 on SAO extension for higher bit-depth coding | J. Xu, A. Tabatabai (Sony) |
| JCTVC-N0254 | m29898 | 2013-07-16 07:40:09 | 2013-07-16 11:10:32 | 2013-08-02 00:39:07 | Non-RCE3: Pipeline Friendly Intra Motion Compensation | C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi (Qualcomm) |
| JCTVC-N0255 | m29900 | 2013-07-16 07:43:09 | 2013-07-16 09:03:25 | 2013-07-16 09:03:25 | SCE2 test 1.3 results | K. Misra, A. Segall (Sharp), K. Sato, J. Xu (Sony) |
| JCTVC-N0256 | m29901 | 2013-07-16 07:43:09 | 2013-07-16 11:13:44 | 2013-08-02 02:50:18 | Non-RCE3: Intra Motion Compensation with 2-D MVs | C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi (Qualcomm) |
| JCTVC-N0257 | m29902 | 2013-07-16 07:45:25 | 2013-07-22 23:56:35 | 2013-07-22 23:56:35 | SCE2 Cross check report of test 1.1 | K. Misra, A. Segall (Sharp) |
| JCTVC-N0258 | m29903 | 2013-07-16 07:45:34 | 2013-07-16 11:15:40 | 2013-07-28 15:46:37 | Non-RCE2: Extension of TU-Based Inter RDPCM | C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-N0259 | m29905 | 2013-07-16 07:46:20 | 2013-07-22 23:57:03 | 2013-07-22 23:57:03 | SCE2 Cross check report of test 2.2 | K. Misra, A. Segall (Sharp) |
| JCTVC-N0260 | m29906 | 2013-07-16 07:47:20 | 2013-07-23 22:25:20 | 2013-07-23 22:25:20 | Cross check report of JCTVC-N0185 on low-delay flag checking process of SHVC | K. Misra, A. Segall (Sharp) |
| JCTVC-N0261 | m29907 | 2013-07-16 07:47:59 | 2013-07-16 09:04:08 | 2013-07-30 17:12:52 | AhG5: Memory Bandwidth Reduction for HEVC RExt | W.-S. Kim, V. Seregin, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0262 | m29908 | 2013-07-16 07:48:11 | 2013-07-25 00:08:54 | 2013-07-25 00:08:54 | Cross check report of JCTVC-N0233 scaled MV in motion field buffer update | K. Misra, A. Segall (Sharp) |
| JCTVC-N0263 | m29909 | 2013-07-16 07:49:51 | 2013-07-16 09:04:45 | 2013-07-31 12:41:22 | AhG5: Deblocking Filter in 4:4:4 Chroma Format | W.-S. Kim, W. Pu, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0264 | m29910 | 2013-07-16 07:50:53 | 2013-07-19 08:17:19 | 2013-07-19 08:17:19 | MV-HEVC/SHVC HLS: VPS extension for multi-format | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) |
| JCTVC-N0265 | m29911 | 2013-07-16 07:53:13 | 2013-07-16 08:32:55 | 2013-07-27 17:43:31 | AHG13: Complexity reduction of the up-sampling process | K. Andersson, R. Sjöberg (Ericsson) |
| JCTVC-N0266 | m29914 | 2013-07-16 07:56:20 | 2013-07-16 10:35:15 | 2013-07-30 11:41:28 | Non RCE1: Inter Color Component Residual Prediction | W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0267 | m29915 | 2013-07-16 08:31:21 | 2013-07-16 09:02:37 | 2013-07-16 09:02:37 | MV-HEVC/SHVC HLS: On changing of the highest layer ID across AUs and multi-mode bitstream extraction | Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-N0268 | m29917 | 2013-07-16 08:34:10 | 2013-07-16 08:39:03 | 2013-07-27 15:05:58 | RCE2 Test A.1: Rice parameter signalling for transform-skip blocks | S.H. Kim, A. Segall (Sharp) |
| JCTVC-N0269 | m29919 | 2013-07-16 08:44:59 | 2013-07-16 08:49:04 | 2013-07-25 01:01:39 | HLS: Non-significant slice segments with tiles for single layer HEVC extensions | C. Auyeung (Sony) |
| JCTVC-N0270 | m29921 | 2013-07-16 08:49:59 | 2013-07-16 19:13:29 | 2013-07-16 19:13:29 | Updated proposal with software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams | Y. Zhang, Y. Wu, S. Kanumuri, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft) |
| JCTVC-N0271 | m29922 | 2013-07-16 08:52:16 | 2013-07-16 08:55:05 | 2013-07-27 17:46:52 | AHG14: Color gamut scalable video coding with piecewise linear predictions predictions and shift-offset model | C. Auyeung, K. Sato (Sony) |
| JCTVC-N0272 | m29923 | 2013-07-16 08:57:33 | 2013-07-16 23:55:42 | 2013-07-26 11:06:24 | Non SCE1: On handling resampling phase offsets with fixed filters | K. Minoo, D. Baylon, A. Luthra (Arris) |
| JCTVC-N0273 | m29924 | 2013-07-16 08:59:34 | 2013-07-16 23:58:45 | 2013-07-23 09:12:48 | On the selection of fixed filters for upsampling | K. Minoo, D. Baylon (Arris) |
| JCTVC-N0274 | m29925 | 2013-07-16 08:59:57 | 2013-07-27 02:52:55 | 2013-07-27 02:52:55 | AHG14: Cross-checking of JCTVC-N0168 from Technicolor on color gamut scalable video coding using 3D LUT | C. Auyeung (Sony) |
| JCTVC-N0275 | m29926 | 2013-07-16 09:11:02 | 2013-07-16 09:12:59 | 2013-07-31 14:19:58 | AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14 | S.H. Kim, K. Misra, A. Segall (Sharp) |
| JCTVC-N0276 | m29927 | 2013-07-16 09:25:57 | 2013-07-16 12:15:57 | 2013-07-23 12:21:48 | A hardware oriented implementation of HEVC encoding | R. Hashimoto, S. Mochizuki, K. Iwata (Renesas) |
| JCTVC-N0277 | m29928 | 2013-07-16 09:35:29 | 2013-07-22 09:46:45 | 2013-07-24 08:22:56 | Cross-check of JCTVC-N0204 on ILR enhancement with differential coding for SHVC reference index framework | E. Francois (Canon) |
| JCTVC-N0278 | m29929 | 2013-07-16 09:35:39 | 2013-07-25 16:14:50 | 2013-07-25 16:14:50 | AHG14: cross-check of JCTVC-N0168 on Color Gamut Scalable Video Coding using 3D LUT: New Results | P. Onno (Canon) |
| JCTVC-N0279 | m29930 | 2013-07-16 09:35:48 | 2013-07-17 14:24:31 | 2013-07-17 14:24:31 | Cross-check of JCTVC-N0042 on Restriction on the Residual DPCM block size | E. Francois (Canon) |
| JCTVC-N0280 | m29931 | 2013-07-16 09:35:58 | 2013-07-18 10:03:29 | 2013-07-25 17:40:39 | AHG13: cross-check of JCTVC-N0265 on Complexity reduction of the up-sampling process | E. Francois (Canon) |
| JCTVC-N0281 | m29932 | 2013-07-16 09:39:03 | 2013-07-16 09:41:05 | 2013-07-27 16:13:27 | Non-RCE2 Rice parameter extension for transform-skip blocks | S.H. Kim, K. Misra, A. Segall (Sharp) |
| JCTVC-N0282 | m29943 | 2013-07-16 10:46:57 | 2013-07-16 21:02:15 | 2013-07-23 00:06:01 | MV-HEVC/SHVC HLS: On handling alternative filtered inter-layer references | P. Lai, S. Liu, S. Lei (MediaTek) |
| JCTVC-N0283 | m29944 | 2013-07-16 11:17:13 | 2013-07-21 09:12:11 | 2013-07-24 21:37:36 | Aspect ratio scalability based on SHM-2.0 | Y. Liu, J. Ostermann (Leibniz Uni Hannover) |
| JCTVC-N0284 | m29945 | 2013-07-16 11:23:46 | 2013-07-22 04:23:31 | 2013-08-01 19:23:19 | Editor's proposed draft text of HEVC conformance testing | T. Suzuki, G. Sullivan, W. Wan |
| JCTVC-N0285 | m29947 | 2013-07-16 12:01:32 | 2013-07-16 13:45:22 | 2013-07-23 09:59:34 | Non-RCE3 : Intra motion compensation for screen contents | J. Min, M.W Park, S. Lee, C. Kim (Samsung) |
| JCTVC-N0286 | m29953 | 2013-07-16 14:10:18 | 2013-07-16 14:27:36 | 2013-07-16 14:27:36 | Time code SEI message | C. Fogg, B. Lewis, J. Lassahn, T. Prins |
| JCTVC-N0287 | m29954 | 2013-07-16 15:08:59 | 2013-07-16 15:58:55 | 2013-07-16 15:58:55 | RCE3 Test 2: Multi-stage Base Color and Index Map | W. Zhu (BJUT), J. Xu (Microsoft), W. Ding (BJUT) |
| JCTVC-N0288 | m29955 | 2013-07-16 15:09:46 | 2013-07-16 18:31:30 | 2013-07-30 12:45:02 | Non-RCE2: Transform skip on large TUs | X. Peng, J. Xu (Microsoft), L. Guo, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-N0289 | m29956 | 2013-07-16 15:10:47 | 2013-07-25 15:35:21 | 2013-07-25 15:35:21 | Cross-check of transform skip based on minimum TU size (JCTVC-N0167) | J. Xu, B. Li (Microsoft) |
| JCTVC-N0290 | m29957 | 2013-07-16 15:56:12 | 2013-07-16 21:51:23 | 2013-07-28 10:05:49 | Ultra-low delay with SHVC, MV-HEVC and 3D-HEVC | R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI) |
| JCTVC-N0291 | m29958 | 2013-07-16 16:24:40 | 2013-07-16 23:26:27 | 2013-07-16 23:26:27 | [AHG21] Best-effort decoding of 10-bit sequences: use cases, requirements and specification methods | D. Flynn, G. Martin-Cocher, D. He |
| JCTVC-N0292 | m29959 | 2013-07-16 16:24:45 | 2013-07-16 17:26:05 | 2013-07-16 17:26:05 | RExt: Fidelity adaptive coding mode | D. Flynn, N. Nguyen, D. He |
| JCTVC-N0293 | m29964 | 2013-07-16 18:54:04 | 2013-07-25 15:35:55 | 2013-07-25 15:35:55 | Cross-check of complexity reduction for inter residual DPCM in lossless coding (JCTVC-N0075) | J. Xu (Microsoft) |
| JCTVC-N0294 | m29966 | 2013-07-16 21:16:27 | 2013-07-16 21:30:10 | 2013-07-26 10:10:51 | AHG8: 4:4:4 game content sequences for HEVC Range Extensions development | R. Cohen (MERL), F. Liu, C. S. Ping, N.-M. Cheung, Y. Chau, S.-K. Yeung (Singapore Univ. of Technology and Design) |
| JCTVC-N0295 | m29968 | 2013-07-17 03:19:05 | 2013-07-17 03:21:59 | 2013-07-17 03:21:59 | AhG5: Cross-check of Chroma QP for HEVC RExt in JCTVC-N0141 | W.-S. Kim (Qualcomm) |
| JCTVC-N0296 | m29969 | 2013-07-17 04:00:06 | 2013-07-21 07:31:36 | 2013-07-21 07:31:36 | Cross-check of JCTVC-N0210 on APS for SHVC | H. Yang (Huawei) |
| JCTVC-N0297 | m29980 | 2013-07-17 19:25:50 | 2013-07-24 15:20:30 | 2013-07-24 15:20:30 | AHG16: cross-verification of JCTVC-N0202 on Key picture concept and single loop decoding | X. Xiu (InterDigital) |
| JCTVC-N0298 | m29982 | 2013-07-17 21:07:19 | 2013-07-28 05:35:19 | 2013-07-31 20:43:39 | RCE1: Cross-check on Experiment 3 | W. Pu, W.-S. Kim (Qualcomm) |
| JCTVC-N0299 | m29984 | 2013-07-18 00:13:32 | 2013-07-18 00:25:25 | 2013-07-18 00:25:25 | MV-HEVC/SHVC HLS: On use of splitting_flag with flexible coding order | A. Norkin, T. Rusert (Ericsson) |
| JCTVC-N0300 | m29987 | 2013-07-18 02:39:48 | 2013-07-18 03:14:25 | 2013-07-18 03:14:25 | Crosscheck of SCE 2 test 1.4 | C. Kim, B. Jeon |
| JCTVC-N0301 | m29992 | 2013-07-18 02:57:24 | 2013-07-18 03:15:00 | 2013-07-18 03:15:04 | Crosscheck of SCE 2 test 1.3 | C.Kim, B. Jeon |
| JCTVC-N0302 | m30006 | 2013-07-18 08:56:33 | 2013-07-18 09:01:15 | 2013-07-18 09:01:15 | SCE2: Results on combination test 1.2 | C. Gisquet, K. Misra, A. Segall |
| JCTVC-N0303 | m30008 | 2013-07-18 12:31:50 | 2013-07-19 10:04:02 | 2013-07-19 10:04:02 | SCE3: Verification of test 3.2 on adaptive re-sampling filter | E. Alshina, A. Alshin (Samsung) |
| JCTVC-N0304 | m30009 | 2013-07-18 12:32:13 | 2013-07-19 10:06:16 | 2013-07-23 11:19:11 | SCE3: Verification of test 3.3 on high frequency pass inter-layer SAO | E. Alshina, A. Alshin (Samsung) |
| JCTVC-N0305 | m30010 | 2013-07-18 12:32:34 | 2013-07-19 10:22:36 | 2013-07-19 10:22:36 | SCE3: Cross-check for test 3.5 (bi-lateral inter-layer filter) | A. Alshin, E. Alshina (Samsung) |
| JCTVC-N0306 | m30011 | 2013-07-18 12:32:53 | 2013-07-19 10:24:32 | 2013-07-19 10:24:32 | Cross-check for HM11.0 SAO code clean-up and encoder bug-fix | E. Alshina, A. Alshin (Samsung) |
| JCTVC-N0307 | m30012 | 2013-07-18 12:33:15 | 2013-07-25 08:07:14 | 2013-07-25 08:07:14 | non-SCE3: Cross-check for inter-layer SAO design from MediaTek |  |
| JCTVC-N0308 | m30013 | 2013-07-18 12:33:34 | 2013-07-19 10:25:14 | 2013-07-19 10:25:14 | SCE1: Verification of test 2.1 on picture and sequence level adaptive re-sampling filter | E.Alshina, A.Alshin (Samsung) |
| JCTVC-N0309 | m30091 | 2013-07-19 08:30:15 | 2013-07-22 13:05:12 | 2013-07-26 20:02:24 | Signalling of chroma sampling filter | K. Kazui (Fujitsu), T. Chujoh (Toshiba) |
| JCTVC-N0310 | m30098 | 2013-07-19 09:09:37 | 2013-07-25 23:30:03 | 2013-07-25 23:30:03 | Cross check of Memory Bandwidth Reduction for HEVC RExt (JCTVC-N0261). | G. Laroche (Canon) |
| JCTVC-N0311 | m30099 | 2013-07-19 09:11:58 | 2013-07-23 13:50:39 | 2013-07-23 13:50:39 | Cross check of JCTVC-N0229 on Region based Inter-layer Cross-Color Filtering | E. Francois (Canon) |
| JCTVC-N0312 | m30132 | 2013-07-19 14:51:14 | 2013-07-23 02:47:33 | 2013-08-01 12:08:25 | A proposal on HEVC 4:2:2 profile | S. Sekiguchi, A. Minezawa, H. Sakate (Mitsubishi), K. Kazui (Fujitsu) |
| JCTVC-N0313 | m30138 | 2013-07-19 15:49:28 | 2013-07-19 15:52:57 | 2013-07-30 23:29:47 | 4EVER HEVC demonstrations during Roland Garros tournament | S. Kervadec (Orange Labs), M. Raulet (IETR/INSA Rennes), J. Le Feuvre (Telecom ParisTech), J. Vieron (ATEME), M. Clare (Orange Labs) on behalf of the 4EVER project |
| JCTVC-N0314 | m30148 | 2013-07-19 19:19:51 | 2013-07-23 01:44:17 | 2013-07-23 01:44:17 | Cross-check of JCTVC-N0234 on Low-Complexity Generated Inter-layer Reference | P. Lai, S. Liu, S. Lei (MediaTek) |
| JCTVC-N0315 | m30149 | 2013-07-19 19:21:10 | 2013-07-24 21:08:55 | 2013-07-24 21:08:55 | Non-SCE1: Cross-check of JCTVC-N0219 On arbitrary spatial ratio scalability in SHVC | P. Lai, S. Liu, S. Lei (MediaTek) |
| JCTVC-N0316 | m30159 | 2013-07-19 21:51:51 | 2013-07-20 01:56:53 | 2013-08-01 16:40:44 | MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction | A. Norkin, U. Hakeem, T. Rusert (Ericsson) |
| JCTVC-N0317 | m30165 | 2013-07-19 23:30:30 |  |  | Non-SCE1: Crosscheck of JCTVC-N0149 on sampling offset signalling with accurate interpolation filter | X. Li (Qualcomm) |
| JCTVC-N0318 | m30168 | 2013-07-20 00:48:22 | 2013-07-21 23:56:58 | 2013-07-21 23:56:58 | Cross check JCTVC-N0111 Sample position mapping with already defined scalling factor | Y. He (InterDigital) |
| JCTVC-N0319 | m30194 | 2013-07-21 19:43:24 | 2013-07-21 20:31:31 | 2013-07-24 14:09:43 | Crosscheck of sample adaptive intra prediction for oblique modes in lossless coding (JCTVC-N0176) | D.-K. Kwon (TI) |
| JCTVC-N0320 | m30195 | 2013-07-21 19:45:08 | 2013-07-22 02:27:23 | 2013-07-22 02:27:23 | AHG5: Crosscheck of JCTVC-N0263 on deblocking filter in 4:4:4 chroma format | D.-K. Kwon (TI) |
| JCTVC-N0321 | m30230 | 2013-07-22 12:04:35 | 2013-07-22 12:08:24 | 2013-07-22 12:08:24 | Cross-check for JCTVC-N0100: Non-RCE2: Unified lossless residual coding | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0322 | m30239 | 2013-07-22 12:25:58 | 2013-07-22 12:29:48 | 2013-07-22 12:29:48 | AHG 8 Cross-check for JCTVC-N0231: Intra mode coding for screen contents | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0323 | m30304 | 2013-07-22 18:56:32 | 2013-07-25 03:05:23 | 2013-07-25 03:05:23 | Non-RCE3: Cross-check of JCTVC-N0235 base color merging for MBCIM | L. Guo (Qualcomm) |
| JCTVC-N0325 | m30306 | 2013-07-22 19:06:44 | 2013-07-25 03:36:02 | 2013-07-29 22:02:15 | Non-RCE2: Cross-verification of JCTVC-N0232 Rice parameter update method | L. Guo (Qualcomm) |
| JCTVC-N0326 | m30363 | 2013-07-23 02:01:26 | 2013-07-23 02:09:09 | 2013-07-23 02:09:09 | Cross-check report for RCE 3.1 | X. Wang, Z. Ma, M. Xu (Huawei) |
| JCTVC-N0327 | m30364 | 2013-07-23 02:03:34 | 2013-07-23 02:10:51 | 2013-07-23 02:10:51 | Cross-check report for RCE 3.3 | X. Wang, Z. Ma, M. Xu (Huawei) |
| JCTVC-N0328 | m30365 | 2013-07-23 02:14:31 | 2013-07-23 02:39:45 | 2013-07-23 02:39:45 | Cross-check report for RCE2 C2 | J. Ye, Z. Ma, M. Xu (Huawei) |
| JCTVC-N0329 | m30366 | 2013-07-23 02:15:15 | 2013-07-23 02:41:08 | 2013-07-23 02:41:08 | Cross-check report for RCE2 A1 | J. Ye, Z. Ma, M. Xu (Huawei) |
| JCTVC-N0330 | m30369 | 2013-07-23 02:41:23 | 2013-07-23 02:43:13 | 2013-07-23 02:43:13 | Cross-check report for RCE2 B3 | J. Ye, Z. Ma, M. Xu (Huawei) |
| JCTVC-N0331 | m30379 | 2013-07-23 04:26:48 | 2013-07-25 02:40:52 | 2013-07-25 08:44:41 | AHG 5: Cross-check of N0246 (Modified SAO for range extensions) | J. Min, E. Alshina (Samsung) |
| JCTVC-N0332 | m30380 | 2013-07-23 04:29:42 | 2013-07-25 02:59:33 | 2013-07-25 02:59:33 | Non-RCE3: Cross-check of N0249 (Modified Palette Mode for Screen Content Coding) | J. Min, S. Lee (Samsung) |
| JCTVC-N0333 | m30381 | 2013-07-23 04:31:26 | 2013-07-25 03:17:02 | 2013-07-25 03:38:36 | Non-RCE2 Cross-check of N0281 (Rice parameter extension for transform-skip blocks) | J. Min, S. Lee (Samsung) |
| JCTVC-N0334 | m30412 | 2013-07-23 12:13:49 | 2013-07-23 22:56:01 | 2013-07-28 16:32:56 | Derivation of picture and slice level information for resampled interlayer reference picture | J. Chen, V. Seregin, X. Li, K. Rapaka, M. Karczewicz (Qualcomm) |
| JCTVC-N0335 | m30414 | 2013-07-23 12:49:44 | 2013-07-23 12:51:44 | 2013-07-23 12:51:44 | Cross-check for JCTVC-N0288: Non-RCE2: Transform skip on large TU | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0336 | m30418 | 2013-07-23 20:22:06 | 2013-07-29 08:18:33 | 2013-07-29 20:02:33 | Cross-check AHG 5: Entropy Coding Throughput for High Bit Depths | W. Pu, W.-S. Kim (Qualcomm) |
| JCTVC-N0337 | m30426 | 2013-07-23 22:59:02 | 2013-07-23 23:31:31 | 2013-07-23 23:31:31 | RCE2: Cross-check report for Test A3 | S.-H. Kim, A. Segall (Sharp) |
| JCTVC-N0338 | m30427 | 2013-07-23 23:01:34 | 2013-07-30 22:46:53 | 2013-07-30 22:46:53 | Cross-check report for JCTVC-N0189 : AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths | S.-H. Kim, A. Segall (Sharp) |
| JCTVC-N0339 | m30428 | 2013-07-23 23:03:20 | 2013-07-29 19:12:38 | 2013-07-29 19:12:38 | Cross-check report for JCTVC-N0271 : AHG14: Color gamut scalable video coding with piecewise linear predictions | S.-H. Kim, A. Segall (Sharp) |
| JCTVC-N0340 | m30431 | 2013-07-24 03:26:11 | 2013-07-26 03:36:49 | 2013-07-26 03:36:49 | Non-RCE3: Cross-check of N0256(Intra Motion Compensation with 2-D MVs) | J. Min, E. Alshina (Samsung) |
| JCTVC-N0341 | m30434 | 2013-07-24 05:56:53 | 2013-07-24 18:36:34 | 2013-07-24 18:36:34 | Non-RCE3 : Crosscheck for JCTVC-N0285 Intra motion compensation for screen contents | C. Pang (Qualcomm) |
| JCTVC-N0342 | m30435 | 2013-07-24 09:06:16 | 2013-07-27 08:08:04 | 2013-07-27 08:08:04 | Cross-check of JCTVC-N0258 on TU-Based inter RDPCM extension | H. Yang (Huawei) |
| JCTVC-N0343 | m30439 | 2013-07-24 13:32:28 | 2013-07-26 17:40:41 | 2013-07-26 17:40:41 | SCE3: Cross-check of SCE3 Test3.1 non-switchable inter-layer filter | K. Ugur (Nokia) |
| JCTVC-N0344 | m30442 | 2013-07-24 15:00:27 | 2013-07-27 09:22:01 | 2013-07-27 09:22:01 | Non-SCE3.3: Crosscheck of JCTVC-N0061 on Inter-layer interpolation-based SAO filtering for SHVC | X. Li (Qualcomm) |
| JCTVC-N0345 | m30447 | 2013-07-24 19:39:18 |  |  | A downsampler supporting arbitrary spatial ratios | J. Dong, Y. Ye (InterDigital) |
| JCTVC-N0346 | m30449 | 2013-07-24 20:58:44 | 2013-07-24 22:05:38 | 2013-07-24 22:05:38 | Proposed editorial changes for maximum number of slices and tiles per picture limit | M. Zhou, B. Heng, W. Wan (Broadcom) |
| JCTVC-N0347 | m30453 | 2013-07-25 00:13:34 |  |  | Crosscheck report of JCTVC-N0145 backwards compatible enhancement of chroma format | K. Misra, A. Segall (Sharp) |
| JCTVC-N0348 | m30454 | 2013-07-25 02:35:17 | 2013-07-26 09:17:03 | 2013-07-26 09:17:03 | Non-RCE3: Cross-check of Intra Motion Compensation in JCTVC-N0206 | W.-S. Kim (Qualcomm) |
| JCTVC-N0349 | m30455 | 2013-07-25 02:35:59 | 2013-07-26 07:39:48 | 2013-07-26 07:39:48 | Cross-check of Mode Dependent Intra Smoothing in JCTVC-N0143 | W.-S. Kim (Qualcomm) |
| JCTVC-N0350 | m30458 | 2013-07-25 03:24:33 | 2013-07-25 04:07:34 | 2013-07-26 09:56:28 | Non-RCE2: Cross-verification of JCTVC-N0137, Golomb-rice parameter initialization for transform-skip and transquant-bypass modes | R. Cohen (MERL) |
| JCTVC-N0351 | m30464 | 2013-07-25 10:40:19 | 2013-07-27 11:58:06 | 2013-07-27 11:58:06 | Cross-check of template-based palette prediction (JCTVC-N0169) | J. Xu (Microsoft) |
| JCTVC-N0352 | m30467 | 2013-07-25 11:30:42 | 2013-07-26 22:23:16 | 2013-07-26 22:23:16 | Cross-check of JCTVC-N0161 from Qualcomm on using decoded pictures from higher layer as references in SHVC | J. Xu, C. Auyeung (Sony) |
| JCTVC-N0353 | m30469 | 2013-07-25 12:31:42 | 2013-07-25 12:38:15 | 2013-07-25 12:38:15 | Cross-check for JCTVC-N0222, Non-RCE2: Results for combination of methods | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0354 | m30472 | 2013-07-25 14:48:24 | 2013-07-27 18:21:21 | 2013-07-28 23:28:29 | Non-RCE2: Results for combination of Mirroring of non-transformed residual blocks (Test B.3) with other methods | R.Weerakkody, M.Mrak, M.Naccari (BBC) |
| JCTVC-N0355 | m30473 | 2013-07-25 15:10:38 | 2013-07-25 15:16:17 | 2013-07-30 09:34:48 | 3D/MV-HEVC HLS: Extending the supported number of layers | K. Sühring, G. Tech, R. Skupin, T. Schierl (FhG HHI), B. Choi (Samsung) |
| JCTVC-N0356 | m30474 | 2013-07-25 15:51:32 | 2013-07-25 16:07:53 | 2013-07-25 16:07:53 | 3D/MV-HEVC HLS: Dependency signalling for extending the supported number of layers | G. Tech, K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI) |
| JCTVC-N0357 | m30475 | 2013-07-25 15:51:46 | 2013-07-25 16:08:17 | 2013-07-25 16:08:17 | 3D/MV-HEVC HLS: Flexible layer clustering for extending the supported number of layers | G. Tech, K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI) |
| JCTVC-N0358 | m30479 | 2013-07-25 17:51:32 | 2013-07-25 17:54:15 | 2013-07-25 17:54:15 | Cross-check for JCTVC-N0183 Non-RCE2: Enhanced angular intra prediction for screen content coding | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0359 | m30482 | 2013-07-25 21:04:25 | 2013-07-25 22:01:35 | 2013-07-25 22:01:35 | Non-RCE1: Cross-check of JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions | L. Guo (Qualcomm) |
| JCTVC-N0360 | m30483 | 2013-07-26 00:00:19 | 2013-07-27 03:53:59 | 2013-07-27 03:53:59 | Cross-check of JCTVC-N0254 Table 10 and 13 on pipeline friendly Intra motion compensation | J. Xu (Sony) |
| JCTVC-N0361 | m30488 | 2013-07-26 03:38:10 | 2013-07-26 03:44:39 | 2013-07-26 03:44:39 | Cross-check of JCTVC-N0082/JCT3V-E0055: On initialization process of reference picture lists for HEVC extensions | A. K. Ramasubramonian (Qualcomm) |
| JCTVC-N0362 | m30490 | 2013-07-26 03:45:34 | 2013-07-26 03:49:13 | 2013-08-01 17:17:25 | Cross-check of JCTVC-N0316/JCT3V-E0239: Initial inter-layer reference picture list construction | A. K. Ramasubramonian (Qualcomm) |
| JCTVC-N0363 | m30511 | 2013-07-26 17:58:55 | 2013-07-28 11:04:22 | 2013-07-28 11:04:22 | Non-RCE2: Cross-verification report for sample adaptive intra prediction for oblique modes in lossy coding (JCTVC-N0177) | R. Joshi (Qualcomm) |
| JCTVC-N0364 | m30512 | 2013-07-26 18:00:18 | 2013-07-29 19:59:45 | 2013-07-29 19:59:45 | RCE2-related: Cross-verification report for variants of simplified sample-based weighted prediction (JCTVC-N0072) | R. Joshi (Qualcomm) |
| JCTVC-N0365 | m30529 | 2013-07-27 09:20:00 | 2013-07-27 18:00:07 | 2013-07-27 18:00:07 | Cross-check for JCTVC-N0207: RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0366 | m30530 | 2013-07-27 09:21:34 | 2013-07-27 18:47:48 | 2013-07-27 18:47:48 | Cross-check for JCTVC-N0266: Non-RCE1: Inter Color Component Residual Prediction | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0367 | m30532 | 2013-07-27 09:58:01 | 2013-07-27 10:00:43 | 2013-07-27 10:00:43 | Non-RCE2: cross-check of simplified sample based intra prediction for lossless coding (JCTVC-N0079) | K. Kawamura, S. Naito (KDDI) |
| JCTVC-N0368 | m30533 | 2013-07-27 10:01:32 | 2013-07-27 12:31:06 | 2013-07-28 14:43:46 | Non-RCE1: Chroma intra prediction with mode-dependent reduced reference | K. Kawamura, T. Yoshino, S. Naito (KDDI) |
| JCTVC-N0369 | m30535 | 2013-07-27 12:46:05 | 2013-07-29 14:44:40 | 2013-07-29 14:44:40 | AHG 5 and 18: Cross-check of Internal Precision for High Bit Depths (JCTVC-N0188) by Sony | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-N0370 | m30536 | 2013-07-27 13:07:57 | 2013-07-28 11:13:07 | 2013-07-29 00:04:03 | A Cross-Check report for N-0354 | H. Chen, A. Saxena, F. Fernandes (Samsung) |
| JCTVC-N0371 | m30544 | 2013-07-27 19:19:11 | 2013-07-28 00:56:21 | 2013-07-28 00:56:21 | MV-HEVC/SHVC HLS: On Scaling List Data Signalling | S. Deshpande (Sharp), M. Pettersson (Ericsson), S. Liu (MediaTek), T. Suzuki (Sony) |
| JCTVC-N0372 | m30545 | 2013-07-27 21:15:46 | 2013-07-27 21:30:44 | 2013-08-01 12:45:33 | HM Software modifications for interlaced coding | Z. Agyo, J. Vieron, J. M. Thiesse |
| JCTVC-N0373 | m30546 | 2013-07-27 22:57:55 | 2013-07-27 23:16:19 | 2013-07-27 23:16:19 | BoG report on random access and cross-layer alignment of pictures types | J. Boyce |
| JCTVC-N0374 | m30548 | 2013-07-27 23:17:59 | 2013-07-27 23:44:39 | 2013-08-01 18:08:11 | BoG report on SHVC/MV-HEVC HLS topics | J. Boyce |
| JCTVC-N0375 | m30553 | 2013-07-28 08:30:29 | 2013-07-28 08:31:55 | 2013-07-29 19:06:55 | BoG report on arbitrary scalability ratios support in SHVC | E. Francois |
| JCTVC-N0376 | m30591 | 2013-07-29 04:25:56 | 2013-07-29 07:32:07 | 2013-07-30 11:43:49 | Non-RCE3:Cross-check of Table 11 and 14 from N0254 ( Pipeline Friendly Intra Motion Compensation) | J. Min, S. Lee (Samsung) |
| JCTVC-N0377 | m30592 | 2013-07-29 06:19:08 | 2013-07-30 14:03:17 | 2013-07-30 14:03:17 | Non-RCE3: Cross-check of N0254 on Table11 and Table14 (Pipeline Friendly Intra Motion Compensation) | X. Wei, J. Zan (Huawei) |
| JCTVC-N0378 | m30303 | 2013-07-22 18:55:30 | 2013-07-24 20:34:02 | 2013-07-30 14:20:00 | Clarifications on HEVC NALU order | J. Le Feuvre, C. Concolato (Telecom ParisTech), M. Raulet (IETR/INSA Rennes) |
| JCTVC-N0379 | m30603 | 2013-07-30 20:01:48 | 2013-07-31 09:17:16 | 2013-07-31 09:17:16 | Cross-check for JCTVC-N0275: AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than or equal to 14 | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-N0380 | m30618 | 2013-07-31 12:38:33 | 2013-07-31 13:21:07 | 2013-07-31 13:21:07 | Cross-check for JCTVC-N0189: AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths | M. Naccari, M. Mrak (BBC) |
| JCTVC-N0381 | m30621 | 2013-07-31 15:54:43 | 2013-07-31 16:05:31 | 2013-08-01 20:51:07 | Cross-check for JCTVC-N0138 AHG5: Square transform deblocking for 4:2:2 | M. Naccari, M. Mrak |
| JCTVC-N0382 | m30624 | 2013-08-01 08:47:40 | 2013-08-01 09:07:53 | 2013-08-01 09:07:53 | Non-RCE3: Cross-check of JCTVC-N0256 (Intra MC with 2D MVs) on Table 17 and Table 18 | X. Wang, Z. Ma (Huawei), |
| JCTVC-N0383 | m30626 | 2013-08-01 09:25:47 | 2013-08-01 16:36:14 | 2013-08-01 16:36:14 | Indication of inter-layer and motion constrained prediction constraints | K. Ugur, M. M. Hannuksela (Nokia), K. Suehring, R. Skupin, Y. Sanchez (FhG HHI), K. Rapaka, J. Chen (Qualcomm), C. Auyeung, S. Hattori (Sony) |
| JCTVC-N0384 | m30633 | 2013-08-01 12:28:35 | 2013-08-02 12:52:31 | 2013-08-02 12:52:31 | BoG report on non-RCE2 contributions | R. Joshi, R. Cohen |
| JCTVC-N0385 | m30634 | 2013-08-01 12:45:22 | 2013-08-01 12:50:21 | 2013-08-01 12:50:21 | Draft HEVC verification test plan | T.K. Tan, V. Baroncini |
| JCTVC-N0386 | m30635 | 2013-08-01 14:46:01 | 2013-08-01 14:48:51 | 2013-08-01 14:48:51 | Cross-check for JCTVC-N0177: Non-RCE2: On sample adaptive intra prediction for oblique modes in lossy coding | M. Naccari, M. Mrak, R. Weerakkody (BBC) |
| JCTVC-N0387 | m30637 | 2013-08-01 15:48:15 | 2013-08-01 15:51:10 | 2013-08-01 15:51:10 | BoG report on Color Gamut Scalability | A. Segall |
| JCTVC-N0388 | m30641 | 2013-08-01 22:14:27 | 2013-08-01 22:17:20 | 2013-08-02 15:12:59 | BoG on combination of tools in RCE2 | J. Sole |
| JCTVC-N1000 | m30664 | 2013-08-10 00:41:03 |  |  | Meeting Report of 14th JCT-VC Meeting | G. J. Sullivan, J.-R. Ohm |
| JCTVC-N1002 | m30665 | 2013-08-10 00:43:35 |  |  | High Efficiency Video Coding (HEVC) Test Model 12 (HM 12) Encoder Description | K. McCann, B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan |
| JCTVC-N1003 | m30666 | 2013-08-10 00:46:00 | 2013-09-27 19:58:52 | 2013-09-27 19:58:52 | High Efficiency Video Coding (HEVC) Defect Report | Y.-K. Wang, G. J. Sullivan, B. Bross |
| JCTVC-N1004 | m30667 | 2013-08-10 00:47:47 | 2013-09-03 12:58:55 | 2013-09-03 12:58:55 | HEVC Conformance Draft 4 | T. Suzuki, W. Wan, G. J. Sullivan |
| JCTVC-N1005 | m30658 | 2013-08-07 14:37:43 | 2013-08-07 14:38:28 | 2013-08-08 02:39:11 | Range Extensions Draft 4 | D. Flynn, J. Sole, T. Suzuki |
| JCTVC-N1006 | m30668 | 2013-08-10 00:49:31 | 2013-08-16 07:33:59 | 2013-08-16 07:33:59 | Common test conditions and software reference configurations for HEVC range extensions | D. Flynn, K. Sharman, C. Rosewarne |
| JCTVC-N1007 | m30669 | 2013-08-10 00:51:22 | 2013-09-15 20:25:08 | 2013-09-15 20:25:08 | Scalable High Efficiency Video Coding Test Model 3 (SHM 3) | J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela |
| JCTVC-N1008 | m30670 | 2013-08-10 00:52:59 | 2013-08-20 00:40:40 | 2013-09-16 20:21:45 | Scalable High Efficiency Video Coding Draft 3 | J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela |
| JCTVC-N1009 | m30671 | 2013-08-10 00:54:18 | 2013-08-28 17:49:03 | 2013-08-28 17:49:03 | Common SHM test conditions and software reference configurations | X. Li, J. Boyce, P. Onno, Y. Ye |
| JCTVC-N1010 | m30672 | 2013-08-10 00:56:23 |  |  | High Efficiency Video Coding Test Moel 12 (HM 12) Reference Software | F. Bossen, D. Flynn, K. SÃ¼hring |
| JCTVC-N1011 | m30649 | 2013-08-02 11:46:25 | 2013-08-03 05:11:22 | 2013-08-03 05:11:22 | HEVC verification test plan draft 1 | T.K. Tan, V. Baroncini |
| JCTVC-N1101 | m30673 | 2013-08-10 00:59:04 | 2013-08-12 12:09:22 | 2013-08-15 20:29:42 | HEVC Scalable Extensions Core Experiment SCE1: Arbitrary scalability ratio support | E. Francois, E. Alshina, J. Chen |
| JCTVC-N1102 | m30661 | 2013-08-08 17:28:33 | 2013-08-08 17:34:03 | 2013-08-08 17:34:03 | HEVC Scalable Extension Core Experiment SCE2: Key pictures and single-loop decoding | M. Wien, K. Rapaka, X. Xiu |
| JCTVC-N1103 | m30653 | 2013-08-02 15:08:45 | 2013-08-02 17:34:23 | 2013-08-29 02:12:40 | HEVC Scalable Extensions Core Experiment 3 (SCE3): Inter-layer filtering | J. Chen, E. Alshina, J. Dong, M. Sychev |
| JCTVC-N1104 | m30755 | 2013-09-28 11:13:52 | 2013-09-28 11:15:19 | 2013-09-28 11:15:19 | HEVC Scalable Extensions Core Experiment 4 (SCE4): Color Gamut and Bit-Depth Scalability | Andrew Segall, Philippe Bordes, Cheung Auyeung, Xiang Li, Elena Alshina |
| JCTVC-N1121 | m30660 | 2013-08-08 02:12:45 | 2013-08-09 01:57:36 | 2013-09-20 18:43:48 | HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods | W.-S. Kim, T. Nguyen |
| JCTVC-N1122 | m30651 | 2013-08-02 11:52:19 | 2013-08-02 11:56:00 | 2013-09-06 02:03:47 | HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods | C. Rosewarne, J. Sole, K. Sharman, S.-H Kim |
| JCTVC-N1123 | m30652 | 2013-08-02 12:38:40 | 2013-08-02 12:40:19 | 2013-09-18 00:55:52 | HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques | A. Saxena, D. Kwon, M. Naccari, C. Pang |

# Annex B to JCT-VC report:
List of meeting participants

The participants of the thirteenth meeting of the JCT-VC, according to a sign-in sheet circuated during the meeting (approximately 162 people in total), were as follows:

- Elena Alshina (Samsung Electronics)

- Peter Amon (Siemens AG)

- Kenneth Andersson (LM Ericsson)

- Ichiro Ando (Nikon)

- Kohtaro Asai (Mitsubishi Electric)

- Cheung Auyeung (Sony)

- Gun Bang (ETRI)

- Guillaume Barroux (Fujitsu)

- Philippe Bordes (Technicolor)

- Frank Bossen (Docomo Innovations)

- Jill Boyce (Vidyo)

- Benjamin Bross (Fraunhofer HHI)

- Madhukar Budagavi (Texas Instruments)

- Done Bugdayci (Nokia)

- Yu-Lin Chang (MediaTek USA Inc.)

- Chun-Chi Chen (NCTU/ITRI)

- Jianle Chen (Qualcomm)

- Peisong Chen (Broadcom)

- Yi-Wen Chen (MediaTek)

- Yongjin Cho (Samsung Electronics Co., Ltd)

- Yoonsik Choe (Yonsei Univ.)

- Byeongdoo Choi (Samsung Electronics)

- Jangwon Choi (Yonsei University)

- Keiichi Chono (NEC)

- Takeshi Chujoh (Toshiba)

- Robert Cohen (Mitsubishi Electric Res. Labs)

- Davy De Schryver (Cisco Systems)

- Sachin Deshpande (Sharp)

- Alexey Dolgoborodov (Vanguard Software Solutions)

- Alberto Duenas (NGcodec)

- MyungJin Eom (Samsung Electronics)

- Christian Feldmann (RWTH Aachen University)

- Felix Fernandes (Samsung Telecom. America)

- Alexey Filippov (Huawei)

- David Flynn (Research In Motion)

- Chad Fogg (Harmonic)

- Edouard François (Canon CRF)

- Arild Fuldseth (Cisco Systems Norway)

- Christophe Gisquet (Canon Research)

- Dan Grois (Fraunhofer HHI)

- Miska Hannuksela (Nokia)

- Ryoji Hashimoto (Renesas Electronics)

- Shinobu Hattori (Sony)

- Dake He (Blackberry)

- Yong He (InterDigital Commun.)

- Félix Henry (Orange Labs)

- Jin Heo (LG Electronics)

- Hideo Hoshuyama (Nikon)

- Yu-Wen Huang (MediaTek USA Inc.)

- Walt Husak (Dolby Labs)

- Atsuro Ichigaya (NHK)

- Tomohiro Ikai (Sharp)

- Byeongmoon Jeon (LG Electronics)

- Byeungwoo Jeon (Sungkyunkwan Univ. (SKKU))

- Rajan Joshi (Qualcomm)

- Jung Won Kang (ETRI)

- Mukta L. Kar (Cable Television Labs)

- Kei Kawamura (KDDI)

- Kimihiko Kazui (Fujitsu Labs)

- Hui Yong Kim (ETRI)

- Woo-Shik Kim (Qualcomm)

- Peter Kuhn (European Patent Office)

- Do-Kyoung Kwon (Texas Instruments)

- Jani Lainema (Nokia)

- Bae-Keun Lee (KT)

- Gyoyoon Lee (LG Electronics)

- Hahyun Lee (ETRI)

- Jae Yung Lee (Sejong Univ.)

- Jinho Lee (ETRI)

- Sunil Lee (Samsung Electronics)

- Yung-Lyul Lee (Sejong Univ.)

- Shawmin Lei (MediaTek USA)

- Ming Li (ZTE)

- Xiang Li (Qualcomm Technologies)

- Chongsoon Lim (Panasonic)

- Jian-Liang Lin (MediaTek USA)

- Youyao Lin (Tencent)

- Shan Liu (MediaTek USA)

- Xiaoyu Liu (Tencent)

- Yiqun Liu (Leibniz Univ. Hannover)

- Ajay Luthra (Arris)

- Zhan Ma (Huawei)

- Gaëlle Martin-Cocher (Research in Motion)

- Shohei Matsuo (NTT)

- Ken McCann (Zetacast / Samsung)

- Akira Minezawa (Mitsubishi Electric)

- Koohyar Minoo (Arris)

- Marta Mrak (BBC R&D)

- Tokumichi Murakami (Mitsubishi Electric)

- Matteo Naccari (BBC R&D)

- Takayuki Nakachi (NTT)

- Ohji Nakagami (Sony)

- Hiroya Nakamura (JVC Kenwood)

- Junghak Nam (Kwangwoon Univ.)

- Sam Narasimhan (Motorola)

- Nguyen Nguyen (Research in Motion)

- Tung Nguyen (Fraunhofer HHI)

- Takahiro Nishi (Panasonic)

- Andrey Norkin (Ericsson AB)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Wen-Hsiao Peng (NCTU/ITRI)

- Martin Pettersson (Ericsson)

- Adarsh Krishnan Ramasubramonian (Qualcomm Technologies)

- Krishnakanth Rapaka (Qualcomm Technologies)

- Justin Ridge (Nokia)

- Christopher Rosewarne (CiSRA / Canon Inc)

- Dmytro Rusanovskyy ()

- Yago Sanchez (Fraunhofer HHI)

- Hisao Sasai (Panasonic)

- Nicholas Saunders (Sony Europe Broad. & Prof. Res. Labs)

- Ankur Saxena (Samsung Telecom. America)

- Thomas Schierl (Fraunhofer HHI)

- Shun-ichi Sekiguchi (Mitsubishi Electric)

- Vadim Seregin (Qualcomm)

- Karl Sharman (Sony Europe Broad. & Prof. Res. Labs)

- Masato Shima (Canon)

- Shinya Shimizu (NTT)

- John Sievers (LifeSize Commun.)

- Rickard Sjöberg (Ericsson)

- Robert Skupin (Fraunhofer HHI)

- Joel Sole (Qualcomm Technologies)

- Kazuo Sugimoto (Mitsubishi Electric)

- Shiori Sugimoto (NTT)

- Karsten Sühring (Fraunhofer HHI)

- Gary Sullivan (Microsoft)

- Teruhiko Suzuki (Sony)

- Maxim Sycher (Huawei)

- Yasser Syed (Comcast)

- Ali Tabatabai (Sony)

- Seishi Takamura (NTT)

- Thiow Keng Tan (NTT Docomo)

- Yih Han Tan (Inst. Infocomm Res. (I2R))

- Biatek Thibaud (TDF)

- Herbert Thoma (Fraunhofer IIS)

- Alexandros Tourapis (Apple)

- Yi-Shin Tung (ITRI USA / MStar Semiconductor)

- Kemal Ugur (Nokia)

- Geert Van der Auwera (Qualcomm Technologies)

- Glenn Van Wallendael (Ghent University - iMinds)

- Thijs Vermeir (Barco)

- Anthony Vetro (Mitsubishi Electric Res. Labs)

- Ye-Kui Wang (Qualcomm Technologies)

- Hamiclouche Wassim (IETR/INSA)

- Stephan Wenger (Vidyo)

- Thomas Wiegand (Fraunhofer HHI)

- Mathias Wien (RWTH Aachen Univ.)

- Ping Wu (ZTE UK)

- Xiaoyu Xiu (InterDigital Commun.)

- Jizheng Xu (Microsoft)

- Tomoyuki Yamamoto (Sharp)

- Haitao Yang (Huawei)

- Yan Ye (InterDigital Commun.)

- Chuohao Yeo (Inst. Infocomm Res. (I2R))

- Haoping Yu (Huawei USA)

- Jinwen Zan (Huawei)

- Louis (Lei) Zhang (AMD)

- Wen Zhang (Tencent)

- Wenhao Zhang (Intel)

- Jianhua Zheng (Huawei)

- Yunfei Zheng (Apple)

- Minhua Zhou (Broadcom)

- Wenjing Zhu (Hong Kong Univ. Sci. & Tech.)