| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O_Notes_d0 |
| --- | --- |

| Title: | Meeting report of the 15th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 23 Oct. – 1 Nov. 2013 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fifteenth meeting during 23 Oct. – 1 Nov. 2013 at the headquarters of the World Meteorological Organization (for the first three days) and at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 1000 hours on Wednesday 23 Oct. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 01 Nov. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the fourteenth JCT-VC meeting in producing the 12th HEVC Test Model (HM12) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and four Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered, and a verification test plan was set up for HEVC version 1 performance testing.

(update) In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting: HEVC test model (HM) 12, HM 12 reference software (issued as (ISO/IEC DIS), draft 4 for HEVC conformance testing  (issued  as ISO/IEC DIS), HEVC Defect Report (for Version 1), draft 4 for HEVC range extensions, draft 3 of SHVC extensions and SHVC Test Model 3, verification test plan draft 1, and two documents specifying common test conditions and software reference configurations for experiments – one for HEVC range extension experiments, and one for scalable coding experiments.

For the organization and planning of its future work, the JCT-VC established 22 "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 9–17 Jan. 2014 under WG 11 auspices in San José, US, 27 March – 4 Apr. 2014 under WG 11 auspices in Valencia, ES, 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP, and XXXX.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fifteenth meeting during 23 Oct. – 1 Nov.2013 at the headquarters of the World Meteorological Organization (for the first three days) and at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 1000 hours on Wednesday 23 Oct. 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 1 Nov. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2013_10_O_Geneva/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the fourteenth JCT-VC meeting in producing the 12th HEVC Test Model (HM12) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and four Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered, and a verification test plan was set up for HEVC version 1 performance testing.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 14 Oct. 2013.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 15 Oct. 2013 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-O0277 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "O0277+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-O0XXX (a proposal relating to XXX) [uploaded XX-XX]

- ...

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-O0XXX (contribution on XXX) [uploaded XX-XX]

- ...

The following cross-verification reports were registered on time but were uploaded late: JCTVC-O0XXX [uploaded XX-XX], ....

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-O0XXX,...7.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-O0258 (a ..., corrected by a late upload on XX-XX)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-N1000, the HEVC Test Model (HM) JCTVC-N1002, the Defect Report JCTVC-N1003, the Conformance Draft JCTVC-N1004, the Reference Software Draft JCTVC-N1010, the Draft Specification of Range Extensions JCTVC-N1005, the SHVC draft specification JCTVC-N1008, the SHVC test model 2 (SHM2) JCTVC-N1007, the common test conditions for RExt (JCTVC-N1006) and SHVC (JCTVC-N1009), and the HEVC verification test plan JCTVC-N1011 were approved. The HM reference software produced by the AHG on software development, the reference software versions for range extensions and SHVC, and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GRP: Generalized residual prediction.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TFD: Tagged for discard.

- Unit types:

  - CTB: code tree block (synonymous with LCU).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

The status of HEVC version 1 in ISO/IEC and ITU-T was noted. The FDIS 23008-2 had been submitted for ballot in ISO/IEC, which closed by July 2014. Publication is under preparation.. In ITU-T, the text was published as Rec. H.265 on 2013-XX-XX.

The HEVC reference software and conformance testing specifications had been submitted as ISO/IEC DIS 23008-5 and DIS 23008-8, respectively.  The ballot closing dates are expected to be in time for issuing the FDIS documents by January 2014.

The range extensions draft 4 had been submitted as ISO/IEC 23008-2/DAM1. The ballot closing dates are expected to be in time for issuing the FDAM document by January 2014.

The scalable extensions draft 3 had been submitted as ISO/IEC 23008-2/PDAM3. The ballot closing dates are expected to be in time for issuing the DAM document by January 2014.

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS / DAM ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

Goals: Progress of work on extensions, conformance & reference software (Study of), verification testing?

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on SHVC HLS, SHVC and RExt CE work and related contributions during the first few days.. Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- First day (Wed. 23 Oct.): 1000–1800 (approximately), plenary in WMO, track A and B as well as breakout work to continue in ITU after 1800

- Second day (Thu. 24 Oct.): 900–1800 (approximately), plenary in WMO, track A and B as well as breakout work to continue in ITU after 1800

- Third day (Fri. 25 Oct.): 0900-1300 (approximately), plenary in WMO, no meeting 1400-1800, track A and B as well as breakout work to continue in ITU after 1800

- Fourth day (Sat. 26 Oct.): ...

## Contribution topic overview (track allocation t.b.d.)

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm (and partially by N.N.). Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. (Note: allocation to tracks were subject to changes)

- AHG reports (XX) Track P (section 2)

- Communication to and by parent bodies (XX) Track P (section 3.1)

- Conformance testing development (XX) Track P (section 3.2)

- Version 1 bug reports and cleanup (XX) Track P (section 3.3)

- Coding performance, implementation, and design analysis (XX) Track P (section 3.4)

- Profile and level definitions (XX) Track P (section 3.6)

- HEVC and RExt use cases (XX) Track P (section 3.7)

- Source video test material (XX) Track P (section 3.8)

- SHVC CE1: Arbitrary scalability ratio support (XX) Track X (section 4.1)

- SHVC CE2: Key pictures and single loop decoding (XX) Track X (section 4.2)

- SHVC CE3: Inter-layer filtering (XX) Track X (section 4.3)

- SHVC CE4: Color gamut and bit-depth scalability (XX) Track X (section 4.4)

- RExt CE1: Inter-component decorrelation (XX) Track X (section 5.1)

- RExt CE2: Rice parameter initialization and update (XX) Track X (section 5.2)

- RExt CE3: Intra prediction techniques (XX) Track X (section 5.3)

- Non-CE RExt (XX CE related, XX other) Track X (section 6.1)

- Non-CE SHVC (XX CE related, XX other) Track X (section 6.2)

- High-level syntax in RExt & single layer (XX) Track X (section 6.3)

- High-level syntax in SHVC and 3D extensions (XX) BoG | joint with JCT-3V (section 6.4)
	BoG alignment, etc. (J. Boyce)

- High-level syntax in SHVC (XX) BoG | Track X (section 6.5)

- VUI and SEI messages (XX) BoG | Track X (section 6.6)

- Non-normative (XX) Track X (section 6.7)

- Withdrawn and unclear category (XX) (section 6.8)

- Plenary discussions and BoG reports (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC. (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: XX; Track A: XXX; Track B: XXX.

# AHG reports (22)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

JCTVC-O0001 JCT-VC AHG report: JCT-VC project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

JCTVC-O0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs)] [miss]

JCTVC-O0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan]

JCTVC-O0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [M. Naccari, C. Rosewarne] [miss]

JCTVC-O0010 JCT-VC AHG report: SHVC core experiments (AHG10) [X. Li, J. Boyce, P. Onno, X. Xiu] [miss]

JCTVC-O0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela] [miss]

JCTVC-O0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He, T.-D. Chuang, D.-K. Kwon] [miss]

JCTVC-O0013 JCT-VC AHG report: SHVC inter-layer filtering (AHG13) [E. Alshina, J. Chen, J. Dong, A. Segall, P. Topiwala]

JCTVC-O0017 JCT-VC AHG report: SHVC complexity assessment (AHG17) [E. Alshina, M. Budagavi, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai]

# Project development, status, and guidance (34)

## Communication to and by parent bodies (0)

## Conformance test set development (2)

JCTVC-O0084 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan]

JCTVC-O0104 Corner case conformance streams [C. Fogg (Harmonic)]

## Version 1 bug reports and cleanup (1)

JCTVC-O0054 Editors' proposed corrections to HEVC version 1 [Y.-K. Wang, G. J. Sullivan, B. Bross]

## RExt text improvements (3)

JCTVC-O0042 Proposed Editorial Improvements to High efficiency video coding (HEVC) Range Extensions Text Specification Draft 4 [D. Flynn (BlackBerry)] [miss]

JCTVC-O0133 Text fixes for inter RDPCM in HEVC-Rext [M. Naccari, M. Mrak (BBC)]

JCTVC-O0192 AHG5/AHG6/AHG9: Text changes for HEVC Range Extensions [P. Lai, S. Liu, S. Lei (MediaTek)]

## HEVC coding performance, implementation demonstrations and design analysis (13)

### Version 1 verification test (1)

JCTVC-O0267 Proposed updates to HEVC verification test plan draft 1 [T.K. Tan (NTT DOCOMO)]

### RExt coding performance (4)

JCTVC-O0101 HDR coding results [C. Fogg (Harmonic)]

JCTVC-O0148 RExt: Testing results on 12-bit 4:2:0 Traffic Sequence [M. Zhou, P. Chen (Broadcom)]

JCTVC-O0184 Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-O0269 AhG8: Using qp=0, 4, 8, 12 for visually lossless coding experiments and results from SC coding by packed pixel Pseudo-2D-matching integrated with HM12.0RE4.0 [Tao Lin, Xianyi Chen, Peijun Zhang, Liping Zhao (Tongji Univ.)]

### RExt design aspects (6)

JCTVC-O0088 AhG5: Memory Bandwidth Reduction for HEVC Rext [W.-S. Kim, V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0318 Cross check of Memory Bandwidth Reduction for HEVC RExt (JCTVC-O0088) [G. Laroche (Canon)] [late] [miss]

JCTVC-O0221 RExt: On motion compensation memory bandwidth constraint [M. Zhou (Broadcom)]

JCTVC-O0309 Cross-check report of RExt: On motion compensation memory bandwidth constraint (JCTVC-O0221) [O. Nakagami, T. Suzuki (Sony)] [late] [miss]

JCTVC-O0233 Interpolation with 16 Bit Restriction on HEVC Range Extension [W. Pu, J. Chen, W.-S. Kim, M. Karczewicz, J. Sole, L. Guo (Qualcomm)]

JCTVC-O0235 Motion Compensation Interpolation with 16 Bit Intermediate Buffer for HEVC Range Extension [W. Pu, J. Chen, M. Karczewicz, W.-S. Kim, J. Sole, L. Guo (Qualcomm)]

### SHVC coding performance (2)

JCTVC-O0097 AHG10/AHG12: On SHVC common test conditions [Y. He, Y. Ye (InterDigital), V. Seregin (Qualcomm)]

JCTVC-O0243 Cross-check report of AHG10/AHG12: On SHM Common Test Conditions (JCTVC-O0097) [O. Nakagami (Sony)]

### SHVC design aspects (1)

JCTVC-O0115 Pipeline and parallel architecture for the SHVC decoder [W. Hamidouche, M. Raulet, O. Deforges]

### Implementation demonstrations (2)

JCTVC-O0165 Real time SHVC software decoding with multi-threaded parallel processing [S. Gudumasu, Y. He, Y. Ye, Y. He (InterDigital)]

JCTVC-O0321 HEVC Interlaced coding - application software proposal [Zineb Agyo, Jérôme Vieron, Jean-Marc Thiesse, Pierre Larbier] [late] [miss]

## Profile and level definitions (requirements related) (8)

### RExt profiles (5)

JCTVC-O0082 AHG 5 and 18: Profiles for Range Extensions [T. Suzuki (Sony)]

JCTVC-O0105 Profiles for extended precision [C. Fogg (Harmonic), J. Helman (Movielabs), B. Mandel (Universal)]

JCTVC-O0126 Request for an 8 bit 4:4:4 profile [A. Fuldseth, G. Bjøntegaard (Cisco Systems)]

JCTVC-O0144 AHG5 and AHG8: Recommended profiling of range extension coding tools [S. Lee, E. Alshina, C. Kim (Samsung)] [late]

JCTVC-O0288 Proposal on Main 4:2:2 10 profile [A. Minezawa, S. Sekiguchi, H. Sakate (Mitsubishi)] [late]

### SHVC profiles (3)

JCTVC-O0094 AHG9: On SHVC main profile constraint [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

JCTVC-O0103 Intra-only SHVC rescaling [C. Fogg (Harmonic)]

JCTVC-O0253 On constraints of the Scalable Main profile [M. M. Hannuksela, K. Ugur (Nokia)]

## HEVC and RExt use cases (requirements related) (0)

## Source video test material (7)

JCTVC-O0069 AHG5 and AHG18: 16-bit Test Sequences for Range Extensions [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0172 AHG22: Sequences with medical mixed content and medical visual content with bit depth beyond 10 bits for HEVC development [P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens)]

JCTVC-O0173 Coding of medical mixed content and medical visual content with bit depth beyond 10 bits [P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

JCTVC-O0222 New test sequences for Screen Content coding [Alexis Tourapis, David Singer, Krasimir Kolarov (Apple)] [miss]

JCTVC-O0256 AHG8: New 4:4:4 test sequences with screen content [H. Yu, W. Wang, X. Wang, J. Ye, Z. Ma (Huawei)]

JCTVC-O0258 AHG8: Results from coding 4:4:4 screen content sequences [H. Yu, Z. Ma, J. Ye, X. Wang, W. Wang (Huawei)] [late]

[initial version rejected as placeholder]

JCTVC-O0268 AhG8: New 4:4:4 screen content test sequence [Tao Lin, Kailun Zhou (Tongji Univ.)]

# Core experiments in SHVC (24)

## SCE1: Arbitrary scalability ratio support (5)

### SCE1 summary and general discussion

JCTVC-O0031 SCE1: Summary report of SHVC Core Experiment on support for arbitrary scalability ratio [E. François, E. Alshina, J. Chen]

### SCE1 primary contributions

JCTVC-O0052 SCE1: Results of Test 1.2 on selection of fixed filters for upsampling [K. Minoo, D. Baylon (ARRIS)]

JCTVC-O0076 SCE1: Performance and complexity of test 1.1 [E.Alshina, A.Alshin (Samsung), J.Chen, X.Li, M.Karczewicz (Qualcomm)]

### SCE1 cross checks

JCTVC-O0124 SCE1: Cross-check of JCTVC-O0076 on alternative 6/16 upsampling filter [P. Onno (Canon)]

JCTVC-O0281 SCE1: Verification of Test 1.2 results [E. Alshina (Samsung)] [late]

## SCE2: Key pictures and single loop decoding (7)

### SCE2 summary and general discussion

JCTVC-O0032 SCE2 Summary Report [Mathias Wien, Krishna Rapaka, Xiaoyu Xiu] [miss]

### SCE2 primary contributions

JCTVC-O0145 [SCE2] Key picture concept and single loop decoding [Christian Feldmann, Mathias Wien (RWTH Aachen University)]

JCTVC-O0162 SCE2: Single-loop decoding based SNR scalability for SHVC [X. Xiu, Y. Ye, Y.-W. He, Y. He (InterDigital)]

JCTVC-O0227 SCE2 : On key pictures in SHVC [K. Rapaka, J. Chen, V. Seregin, Y.-K. Wang, M. Karczewicz (Qualcomm)]

### SCE2 cross checks

JCTVC-O0146 [SCE2] Cross check for Qualcomm JCTVC-O0227 [Christian Feldmann, Mathias Wien (RWTH Aachen University)] [miss]

JCTVC-O0169 SCE2: Cross-verification of key picture concept and single loop decoding from Aachen [X. Xiu, Y. Ye (InterDigital)] [miss]

JCTVC-O0231 SCE2 : Cross check for Interdigital JCTVC-O0162 [K. Rapaka (Qualcomm)] [miss]

## SCE3: Inter-layer filtering (5)

### SCE3 summary and general discussion

JCTVC-O0033 SCE3: Summary report of SHVC Core Experiment on Inter-layer Filtering [J. Chen, E. Alshina, J. Dong, M. Sychev]

### SCE3 primary contributions

JCTVC-O0078 SCE3: performance and complexity test for cross-color inter-layer filter [X.Li, J.Chen, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin(Samsung), J.Dong, Yan Ye, Yuwen He (InterDigital)]

JCTVC-O0163 SCE3: Inter-layer prediction modes based on base layer sharpness filter [M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei)]

### SCE3 cross checks

JCTVC-O0151 SCE3: Cross-Check of test 3.1 region based inter-layer cross-color filtering (N-0229) in SCE 3 [V. Anisimovskiy, M. Sychev (Huawei)]

JCTVC-O0282 SCE3: Verification of performance and complexity assessment for sharpening inter-layer filter [E. Alshina (Samsung)] [late]

## SCE4: Color gamut and bit depth scalability (9)

### SCE4 summary and general discussion

### SCE4 primary contributions

JCTVC-O0159 SCE4: Results on 5.3-test1 and 5.3-test2 [P. Bordes (Technicolor)]

JCTVC-O0194 SCE4: Test 5.1 results on bit-depth and color-gamut scalability [A. Aminlou, K. Ugur, M. M. Hannuksela (Nokia), E. Alshina, A. Alshin (Samsung)]

JCTVC-O0196 SCE4: Results of test 5.4-model1 on piecewise linear color space predictor [C. Auyeung (Sony)]

JCTVC-O0201 SCE4 Test 5.2: Color prediction with Gain-Offset model [Jie Zhao, Sachin Deshpande, Kiran Misra, Seung-Hwan Kim (Sharp)]

### SCE4 cross checks

JCTVC-O0130 SCE4: Cross-check of 5.3-test1 [J. Lee, H. Lee, J. W. Kang (ETRI)] [late]

JCTVC-O0210 SCE4: Cross-check results of test 5.1 on joint upsampling and shift from Nokia and Samsung [C. Auyeung (Sony)] [miss]

JCTVC-O0241 SCE4: Crosscheck of Test 5.2 on Color prediction with Gain-Offset model [X. Li (Qualcomm)] [miss]

JCTVC-O0242 SCE4: Crosscheck of Test 5.4 Model1 on piecewise linear color space predictor [X. Li (Qualcomm)] [miss]

JCTVC-O0261 SCE4: Cross-check of test 5.3 color prediction with 3D LUT (10bit base case) [Jie Zhao, Seung-Hwan Kim (Sharp)]

# Core experiments in Range Extensions (28)

## RCE1: Inter-component decorrelation methods (4)

### RCE1 summary and general discussion

### RCE1 primary contributions

JCTVC-O0202 RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4 [W. Pu, W.-S. Kim, J. Chen, J. Sole, M. Karczewicz (Qualcomm)]

### RCE1 cross checks

JCTVC-O0086 RCE1: Cross-check on Experiment 1 [A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-O0131 RCE1: Cross Check Results for Experiment 3 and 4 [T. Nguyen (Fraunhofer HHI)]

JCTVC-O0264 RCE1: cross-check results of test 2 in inter color component residual prediction (JCTVC-O0202) [K. Kawamura, S. Naito (KDDI)]

## RCE2: Rice parameter initialization and update (15)

### RCE2 summary and general discussion

JCTVC-O0036 RCE2: Summary report on HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialisation and update methods [C. Rosewarne, J. Sole, K. Sharman, S.-H. Kim] [miss]

### RCE2 primary contributions

JCTVC-O0065 RCE2: C2 - Entropy Coding Compression Efficiency for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0114 RCE2: Test B2. Golomb-rice parameter initialization for transform-skip and transquant-bypass modes [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

JCTVC-O0206 RCE2 Test A1: Simplified update of the coefficient level Rice parameter [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0219 RCE2: Results of Test C1 on Rice Parameter Initialization [M. Karczewicz, L. Guo, J. Sole, R. Joshi (Qualcomm)]

JCTVC-O0239 RCE2: Results of Test D1 on Rice Parameter Initialization [M. Karczewicz, L. Guo, J. Sole, R. Joshi (Qualcomm), K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0246 RCE2: Test A2. Rice parameter signaling for transform-skip blocks [S.-H Kim, K. Misra, A. Segall (Sharp)]

JCTVC-O0247 RCE2: Test B1. Rice parameter signaling for transform-skip blocks [S.-H. Kim, K. Misra, A. Segall (Sharp)]

### RCE2 cross checks

JCTVC-O0240 Cross-check of ‘RCE2 Test A1: Simplified update of the coefficient level Rice parameter’ (JCTVC-O0206) by Qualcomm [C. Rosewarne, M. Maeda (Canon)]

JCTVC-O0250 Cross-check of ‘RCE2: Test A2. Rice parameter signaling for transform-skip blocks’ (JCTVC-O0246) by Sharp Labs [C. Rosewarne, M. Maeda (Canon)]

JCTVC-O0257 Cross-check of 'RCE2: Results of Test D1 on Rice Parameter Initialization' (JCTVC-O0239) by Qualcomm and Sony [C. Rosewarne, M. Maeda (Canon)] [miss]

JCTVC-O0283 RCE2: Verification of test C1 on Rice parameter initialization [E. Alshina (Samsung)] [late]

JCTVC-O0290 Cross-check of ‘RCE2: Test B2. Golomb-rice parameter initialization for transform-skip and transquant-bypass modes [S.-H. Kim, K.Misra (Sharp)] [late]

JCTVC-O0294 RCE2: Cross-check of Test B1 JCTVC-O0247 Rice parameter signaling for transform-skip blocks [L. Guo (Qualcomm)] [late]

JCTVC-O0295 RCE2: Cross-check of Test C2 JCTVC-O0065 Entropy Coding Compression Efficiency for High Bit Depths [L. Guo (Qualcomm)] [late]

## RCE3: Intra prediction techniques (13)

### RCE3 summary and general discussion

JCTVC-O0037 RCE3: Summary report of HEVC Range Extensions Core Experiment 3 on Intra Prediction techniques [A. Saxena, D. Kwon, M. Naccari, C. Pang]

### RCE3 primary contributions

JCTVC-O0080 RCE3: Results of Test A.1 on sample based intra prediction for lossless coding [J Zhu, W Zheng, K.Kazui(Fujitsu)]

JCTVC-O0047 RCE 3: On sample adaptive intra prediction for oblique modes in lossless coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

JCTVC-O0048 RCE 3: On sample adaptive intra prediction for oblique modes in lossy coding [A. Saxena, H. Chen, F. Fernandes (Samsung)] [late]

JCTVC-O0049 RCE 3: Nearest-neighbor intra prediction for screen content video coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

JCTVC-O0051 RCE 3: Results of Experiment B.1 [A. Saxena, H. Chen, F. Fernandes (Samsung)]

### RCE3 cross checks

JCTVC-O0050 RCE 3: Cross-Check of Tool A.1 from Fujitsu [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-O0081 RCE3: Cross-check of Test A.2.4, A.2.5, and B.1.2 from Samsung [J Zhu, W Zheng, K Kazui (Fujitsu)] [miss]

JCTVC-O0278 RCE3: Crosscheck of JCTVC-O0047 on sample adaptive intra prediction for oblique modes in lossless coding [D.-K. Kwon (TI)] [late]

JCTVC-O0280 RCE3: Cross check of Test A.3 (Nearest-neighbor intra prediction for screen content video coding) [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-O0203 RCE 3: Cross-Check of Tool A.1 from Fujitsu [Z. Ma, J. Ye, H. Yu (Huawei)] [late]

JCTVC-O0204 RCE 3: Cross-check of Results of Experiment B.1 from Samsung [Z. Ma, J. Ye, H. Yu (Huawei)] [miss]

JCTVC-O0293 RCE3: Cross-check of Test A.2.5 (JCTVC-O0048) SAIP for oblique modes in lossy coding [P. Lai, S. Liu  (MediaTek)] [late] [miss]

# Non-CE Technical Contributions (161)

## Range extensions (64)

### General (0)

### RCE1 related (inter-component decorrelation) (4)

JCTVC-O0150 Non-RCE1: Extended Adaptive Inter-Component Prediction [A. Khairat, T. Nguyen, M. Siekmann, D. Marpe (Fraunhofer HHI)]

JCTVC-O0320 Cross checking of Non-RCE1: Extended Adaptive Inter-Component Prediction [W. Pu (Qualcomm)] [late] [miss]

JCTVC-O0263 Non-RCE1: Inter colour-component residual coefficients prediction [K. Kawamura, S. Naito (KDDI)]

JCTVC-O0319 Cross checking of Non-RCE1: Inter colour-component residual coefficients prediction [W. Pu (Qualcomm)] [late] [miss]

### RCE2 related (transform coefficient coding) (8)

JCTVC-O0248 Non-RCE2: TU-based rice parameter initialization for transform-skip blocks [S.-H. Kim, K. Misra, A. Segall (Sharp)] [miss]

JCTVC-O0129 Non-RCE2 and AHG18: Increase in the maximum value of Rice parameter for high bit-depth support [S. Lee, J. Min, E. Alshina, C. Kim (Samsung)]

JCTVC-O0312 Cross-check of JCTVC-O0129 [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-O0207 AhG5 and AhG18: Bypass bins alignment depending on the Rice parameter [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0208 AhG5 and AhG18: Bypass of the coefficient level flags for higher throughput [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0313 Cross-check of JCTVC-O0208 [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-O0209 AHG5 and AHG18: Bypass of the significance and coefficient level flags for higher throughput [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0301 Cross-check of JCTVC-O0209 on AHG5 and AHG18: Bypass of the significance and coefficient level flags for higher throughput [S.-H. Kim (Sharp)] [late]

### RCE3 related (intra prediction methods) (5)

JCTVC-O0181 Non-RCE3: Implicit derivation for adaptively turning filtering off in intra prediction [J. Kang, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0292 cross-check O0181 Method 2 [?? (??)] [late] [miss]

JCTVC-O0308 A cross-check report for JCTVC-O0181 [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-O0087 Non-RCE3: Unified lossless residual coding [Y. H. Tan, C. Yeo (I2R)]

JCTVC-O0147 RExt: Proposed changes in the horizontal and vertical gradient filtering control for intra prediction [M. Zhou (Broadcom)]

### Intra block copy (35)

JCTVC-O0102 AHG5: Block size restriction of intra block copy [S. Lee, C. Park, E. Alshina, C. Kim (Samsung)]

JCTVC-O0296 AHG5: Cross-check of JCTVC-O0102 for block size restriction of intra block copy [L. Guo (Qualcomm)] [late] [miss]

JCTVC-O0112 AHG5: Extension of intra block copy [S. Lee, E. Alshina, C. Kim (Samsung)]

JCTVC-O0307 Cross check for JCTVC-O0112: Extension of intra block copy [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-O0113 AHG5: On context modelling for intra block copy mode [C. Rosewarne, M. Maeda (Canon)]

JCTVC-O0122 AHG5: Vector prediction for Intra Block Copy [G. Laroche, T. Poirier, C. Gisquet (Canon)]

JCTVC-O0123 AHG5: Vector transformation for Intra Block Copy [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-O0154 AhG5: Displacement vector signaling for intra block copying [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0155 AhG5: Constrained intra prediction for intra block copying [C. Pang, J. Chen, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0156 AhG5: Fast encoder search and search region restriction for intra block copying [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0303 AHG5: Crosscheck of JCTVC-O0156 on fast encoder search and search region restriction for intra block copying [D.-K. Kwon (TI)] [late] [miss]

JCTVC-O0157 AhG5: Intra block copying with padding [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm), T. Lin, S. Wang (Tongji University)]

JCTVC-O0311 Cross-check of JCTVC-O0157 [J. Xu (Sony)] [late] [miss]

JCTVC-O0158 AhG5: Context derivation method for intra_bc_flag coding [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-O0286 Cross-check of JCTVC-O0158 method 2 [J. Xu (Sony)] [late] [miss]

JCTVC-O0170 AHG8: Use of inter RDPCM for blocks using intra block copy mode [R. Joshi, C. Pang, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0205 AHG8: Pseudo-PU-based Intra Block Copy [C.-C. Chen, T.-S. Chang, R.-L. Liao, W.-H. Peng, H.-M. Hang (NCTU/ITRI), C.-L. Lin, F.-D. Jou (ITRI)] [late]

JCTVC-O0232 On intra block copying in RExt [J. Xu, A. Tabatabai, O. Nakagami (Sony)]

JCTVC-O0297 Cross-check of JCTVC-O0232 on Intra Block Copying in Rext [L. Guo (Qualcomm)] [late] [miss]

JCTVC-O0244 RExt: Signaling Motion Vector Range for Intra Block Copying [L. Guo, C. Pang, J. Chen, J. Sole, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm)]

JCTVC-O0245 AHG5: Fast encoding using early skipping of Intra block copy (IntraBC) search [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-O0289 A cross-check report for JCTVC-O0245: Method 1+Method 3 [G. Jin, A. Saxena, F. Fernandes] [late] [miss]

JCTVC-O0277 RExt: On Intra Block Copy mode [G. Jin, A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-O0305 Crosscheck of JCTVC-O0277 on intra block copy motion vector coding [D.-K. Kwon (TI)] [late] [miss]

JCTVC-O0074 AhG5: Intra block copy within one LCU [E.Alshina, A.Alshin, S.Lee (Samsung)]

JCTVC-O0183 On Intra BC mode [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-O0306 A cross-check report for JCTVC-O0183 [A. Saxena, E. Alshina, F. Fernandes (Samsung)] [late]

JCTVC-O0323 Crosscheck of JCTVC-O0183 (On IntraBC mode) Section 2.4 about BC vector coding [Z. Ma, J. Ye, H. Yu (Huawei)] [late] [miss]

JCTVC-O0186 On residual rotation for Inter and Intra BC modes [X. Peng, B. Li, J. Xu (Microsoft)]

JCTVC-O0053 RExt: On transform selection for Intra-BlockCopy blocks [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

JCTVC-O0073 AhG5: On context modelling simplification for Intra_bc_flag coding [E.Alshina, A.Alshin (Samsung)]

JCTVC-O0167 AHG5 : Intra Motion Vector Coding [C. Park, S. Lee, C. Kim (Samsung)]

JCTVC-O0300 Cross-check of JCTVC-O0167 on AHG5:Intra Motion Vector Coding [S.-H. Kim (Sharp)] [late]

JCTVC-O0315 AhG5: Cross-check for prediction of displacement vector in intra block copying [E. Alshina, A. Alshin] [late] [miss]

JCTVC-O0316 AhG5: Cross-check for constrain intra design for Intra block copy [E. Alshina, A. Alshin] [late] [miss]

### Residual DPCM (9)

JCTVC-O0066 AHG5: Unifying DPCM [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0134 On residual DPCM design unification for lossy coding in HEVC-Rext [M. Naccari, M. Mrak (BBC)]

JCTVC-O0310 Cross-check of JCTVC-O0134 on residual DPCM design unification [S. Lee, C. Park, C. Kim (Samsung)] [late] [miss]

JCTVC-O0171 Order of rotation, inter RDPCM and transform-skip [R. Joshi, J. Kang, C. Pang, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0178 Explicit signalling for intra RDPCM [J. Kang, R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0317 Cross check for JCTVC-O0178: Explicit signalling for intra RDPCM [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-O0185 RDPCM operation unification and cleanup [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-O0291 Cross-check of JCTVC-O0185 RDPCM operation unification [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-O0193 AHG5: Unification of processing order of RDPCM [P. Lai, S. Liu, S. Lei (MediaTek)]

### Transform skip (3)

JCTVC-O0067 Transform-skip and Scaling Lists [K. Sharman, N. Saunders, K. Sato, J. Gamei (Sony)]

JCTVC-O0237 Transform-skip and scaling lists [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0238 Sign coding for transform skipped blocks [J. Wang, D. He, N. Hu, D. Flynn (BlackBerry)]

### High bit depth (7)

JCTVC-O0046 AHG5 and AHG18: Entropy Coding Throughput for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0068 AHG5 and AHG18: Transform Matrix Precision for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-O0090 AhG 5 and 18: High Bit-depth Coding Using Auxiliary Picture [W.-S. Kim, W. Pu, J. Chen, Y.-K. Wang, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0091 AhG 5 and 18: Sample Adaptive Offset on High Bit-depth [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0236 CABAC Bypass Throughput Improvement for High Bit Depths [W. Pu (Qualcomm)] [miss]

JCTVC-O0259 AHG18: On Supporting Higher Bit Depth With Transform Skip Only Changes [K. Misra, S.-H. Kim (Sharp)]

JCTVC-O0314 Cross-check of JCTVC-O0259 on supporting higher bit-depth with transform skip only changes [S. Lee, C. Park, C. Kim (Samsung)] [late] [miss]

### Alpha channel (1)

JCTVC-O0132 AHG5/AHG9: On support for alpha channel in HEVC [M. Naccari, M. Mrak (BBC)]

### Lossless and screen content coding related contributions (7)

JCTVC-O0070 Fixed MPM set in intra mode coding for screen contents [X. Xu, S. Liu (MediaTek), J. Min, S. Lee, E. Alshina, C. Kim (Samsung)]

JCTVC-O0304 Crosscheck of JCTVC-O0070 on fixed MPM set in intra mode coding for screen contents [D.-K. Kwon (TI)] [late] [miss]

JCTVC-O0083 HW complexity analysis on screen content coding tools [T. Suzuki (Sony)] [late]

JCTVC-O0085 AHG5/AHG8: Improvement of MV-HEVC-based RGB coding for screen contents [A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

JCTVC-O0182 AHG8: Major-color-based screen content coding [X. Guo, B. Li, J. Xu, Y. Lu, S. Li (Microsoft)]

JCTVC-O0302 Crosscheck of JCTVC-O0182 [S.-T. Hsiang (MediaTek)] [late] [miss]

JCTVC-O0228 AHG9: Signaling lossless slices [S.-T. Hsiang, Y.-W. Huang, S. Lei (MediaTek)]

### Other (10)

JCTVC-O0044 RExt: CU-adaptive chroma QP offsets [D. Flynn, N. Nguyen, D. He, G. Martin-Cocher (BlackBerry), A. Tourapis, G. Cote, D. Singer (Apple)]

JCTVC-O0230 Chroma Deblocking [Alexis Tourapis (Apple), David Flynn (BlackBerry), David Singer (Apple)] [late]

JCTVC-O0043 Best-effort decoding of 10-bit sequences [D. Flynn (BlackBerry)]

JCTVC-O0276 Cross-check of JCTVC-O0044 on RExt: CU-adaptive chroma QP offsets [J. Xu, A. Tabatabai(Sony)] [miss]

JCTVC-O0045 RExt: minimum chroma TU size restriction for low-fidelity coding mode [D. Flynn, N. Nguyen, D. He (BlackBerry)]

JCTVC-O0089 AhG5: Deblocking Filter in 4:4:4 Chroma Format [W.-S. Kim, W. Pu, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-O0279 AHG5: Crosscheck of JCTVC-O0089 on deblocking filter in 4:4:4 chroma format [D.-K. Kwon (TI)] [late]

JCTVC-O0218 Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1 [L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

JCTVC-O0299 Cross-check of JCTVC-O0218 [J. Xu (Sony)] [late] [miss]

JCTVC-O0270 RExt: Comment on Fidelity Adaptive video coding mode [K. Sato (Sony)]

## SHVC (20)

### General (0)

### SCE1 related (arbitrary scalability ratio) (0)

### SCE2 related (key picture concept and single-loop decoding) (2)

JCTVC-O0127 Transcoder-friendly scalable coding [Kenneth Andersson, Thomas Rusert, Rickard Sjöberg, Jonatan Samuelsson (Ericsson)]

JCTVC-O0322 Cross-check of JCTVC-O0127: Transcoder-friendly scalable coding [D. Bugdayci (Nokia)] [late] [miss]

### SCE3 related (inter-layer filtering) (4)

JCTVC-O0187 Non-SCE3: Unified inter-layer filtering for SHVC [M. Sychev (Huawei)] [miss]

JCTVC-O0189 non-SCE3: Combined inter-layer prediction: sharpness and region-based cross-color filters [M. Sychev, V. Anisimovskiy (Huawei)]

JCTVC-O0191 non-SCE3: Reduced complexity for inter-layer sharpness prediction mode [M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei)]

JCTVC-O0285 non-SCE3: Verification for simplified design of sharpening inter-layer filter [E. Alshina (Samsung)] [late] [miss]

### SCE4 related (color gamut and bit depth scalability) (6)

JCTVC-O0160 Non-SCE4: Cross-Check of InterDigital (JCTVC-O0161) [P. Bordes (Technicolor)] [late]

JCTVC-O0161 Non-SCE4/AHG14: Combined bit-depth and color gamut conversion with 3D LUT for SHVC color gamut scalability [Y. He, Y. Ye, J. Dong (InterDigital)]

JCTVC-O0180 Non-SCE4: Weighted Prediction Based Color Gamut Scalability [X. Li, V. Seregin, J. Chen, K. Rapaka, Y. Chen, M. Karczewicz (Qualcomm)]

JCTVC-O0195 Non-SCE4: Picture and region adaptive gain-offset prediction for color space scalability [C. Auyeung (Sony)]

JCTVC-O0298 Non-SCE4: Cross-check of JCTVC-O0195 “Picture and region adaptive gain-offset prediction for color space scalability” [J. Zhao, S. H. Kim (Sharp)] [late]

JCTVC-O0284 Non-SCE4: Verification of on Weighted Prediction Based Color Gamut Scalability [E. Alshina (Samsung)] [late]

### Up-/downsampling process (6)

JCTVC-O0071 AhG13: Performance analysis of scalable systems with different down-samplers [X.Li, J.Chen, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin (Samsung), J.Dong, Y.Ye (InterDigital), E.Francois (Technicolor)]

JCTVC-O0072 AhG13: On Chroma accurate position alignment during re-sampling [E.Alshina, A.Alshin (Samsung)]

JCTVC-O0215 On phase alignment of up-sampling process in SHVC [J. Chen, L. Guo, X. Li, S. Fan, M. Karczewicz (Qualcomm)]

JCTVC-O0274 Cross-check report for JCTVC-O0215 on phase alignment of up-sampling process in SHVC [Jie Dong, Yan Ye (InterDigital)]

JCTVC-O0272 SHVC: Upsampling with shorter-tap filters [K. Sato (Sony)

JCTVC-O0287 Cross check of JCTVC-O0272 [T. Yamamoto (Sharp)] [late] [miss]

### Inter-layer information derivation (3)

JCTVC-O0121 SHVC: On Inter Layer Reference frame and motion derivation [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-O0168 On derivation of slice information and motion information for inter-layer reference picture in SHVC [X. Xiu, Y. Ye, Y. He, Y.-W. He (InterDigital)]

JCTVC-O0216 On slice level information derivation and motion field mapping for resampled interlayer reference picture [J. Chen, V. Seregin, X. Li, K. Rapaka, M. Karczewicz (Qualcomm)]

### Residual prediction (1)

JCTVC-O0107 Low-complexity generalized residual prediction for SHVC [Kyeonghye Kim, Jiwoo Ryu, Donggyu Sim (KWU)]

### Other (0)

## HL syntax for range extensions and single-layer HEVC coding (2)

JCTVC-O0041 REXT/MV-HEVC/SHVC HLS: auxiliary picture layers [M. M. Hannuksela (Nokia)]

JCTVC-O0135 MV-HEVC/SHVC HLS: Carriage of auxiliary pictures [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

## HL syntax in SHVC and 3D extensions (56)

### Generic HLS issues (8)

JCTVC-O0096 AHG9: On high level syntax [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

JCTVC-O0110 MV-HEVC/SHVC HLS: On picture output flag marking process [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0116 Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

JCTVC-O0119 On Highest Temporal Sub-layer [S. Deshpande (Sharp)]

JCTVC-O0153 MV-HEVC/SHVC HLS: on output layer sets [M. M. Hannuksela (Nokia)]

JCTVC-O0137 MV-HEVC/SHVC HLS: Extended layer identifier [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0200 3D/MV-HEVC HLS: Study and proposal of methods for extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

JCTVC-O0223 MV-HEVC/SHVC HLS: Comments on latest MV-HEVC and SHVC draft specs [K. Rapaka, Y. Chen, A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late]

### POC alignment and derivation (7)

JCTVC-O0117 On POC Alignment [S. Deshpande (Sharp)]

JCTVC-O0128 MV-HEVC/SHVC HLS: On poc reset for long-term reference pictures [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-O0140 MV-HEVC/SHVC HLS: Alignment of picture order counts [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0176 HLS: Error robust POC alignment [R. Sjöberg, J. Samuelsson, R. Yu (Ericsson)]

JCTVC-O0211 MV-HEVC/SHVC HLS: On cross-layer POC alignment for layer-wise startup [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-O0213 On picture order count [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang, Hendry (Qualcomm)]

JCTVC-O0275 MV-HEVC/SHVC HLS: on POC value derivation [M. M. Hannuksela (Nokia)]

### Random access and layer switching structures (4)

JCTVC-O0139 MV-HEVC/SHVC HLS: Layer-wise initialization and parameter set activation [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0149 MV-HEVC/SHVC HLS: on splicing and layer-wise start-up of the decoding [M. M. Hannuksela (Nokia)]

JCTVC-O0212 MV-HEVC/SHVC HLS: On CL-RAS pictures [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-O0220 MV-HEVC/SHVC HLS: On first decodable CRA picture of the layer-wise startup [J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

### Parameter sets (12)

JCTVC-O0058 MV-HEVC/SHVC HLS: On profile, tier, and level information [T. Tsukuba, T. Ikai, T. Yamamoto (Sharp)]

JCTVC-O0059 MV-HEVC/SHVC HLS: On sharing parameter sets across layers [T. Tsukuba, T. Ikai, T. Yamamoto (Sharp)]

JCTVC-O0092 MV-HEVC/SHVC HLS: On nuh_layer_id of SPS and PPS [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

JCTVC-O0109 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0111 MV-HEVC/SHVC HLS: On VPS extension syntax element arrangement [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0118 On Video Signal Information in VPS [S. Deshpande (Sharp)]

JCTVC-O0125 SHVC/MV-HEVC HLS: On use of splitting_flag with combinations of scalability dimensions [A. Norkin, T. Rusert (Ericsson)]

JCTVC-O0141 MV-HEVC/SHVC HLS: Clean-ups of parameter sets [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0142 Conditional SPS extension syntax for RExt, SHVC, and MV-HEVC [J. Boyce (Vidyo)]

JCTVC-O0179 MV-HEVC/SHVC HLS: Improvements of parameter sets [Jung Won Kang (ETRI), Jinho Lee, Hahyun Lee, Jin Soo Choi, Truong Cong Thang (UoA)]

JCTVC-O0214 On parameter sets [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, V. Seregin (Qualcomm)]

JCTVC-O0252 SHVC / MV-HEVC HLS: On signalling of representation format [Hendry, Y.-K Wang, Y. Chen (Qualcomm)]

### Inter-layer dependency signaling and derivation (7)

JCTVC-O0061 MV-HEVC/SHVC HLS: Alternative colPic indication [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

JCTVC-O0093 AHG9: On inter layer dependency [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

JCTVC-O0120 On Inter-layer Reference Picture Set [S. Deshpande (Sharp)]

JCTVC-O0138 MV-HEVC/SHVC HLS: On inter-layer dependency signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0225 MV-HEVC/SHVC HLS: On inter-layer RPS derivation and sub-layer inter-layer dependency [K. Rapaka, Y.-K. Wang, J. Chen, V. Seregin, Hendry, M. Karczewicz (Qualcomm)]

JCTVC-O0254 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [Hendry, Y. Chen, Y.-K Wang (Qualcomm)] [late]

JCTVC-O0271 SHVC: On direct_dependency_flag [K. Sato (Sony)]

### Reference picture list construction (2)

JCTVC-O0060 MV-HEVC/SHVC HLS: reference picture list construction for motion only dependent picture [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

JCTVC-O0174 MV-HEVC/SHVC HLS: Reference picture set/list derivation independent of ViewId [M. M. Hannuksela (Nokia)]

### Hypothetical reference decoder (HRD) (5)

JCTVC-O0136 MV-HEVC/SHVC HLS: Signalling decoded picture buffer size [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-O0217 MV-HEVC/SHVC HLS: Sub-DPB based DPB operations [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), M. M. Hannuksela (Nokia), S. Deshpande (Sharp Labs)]

JCTVC-O0164 MV-HEVC/SHVC HLS / JCT-VC AHG20: Multi-layer HRD operation [M. M. Hannuksela (Nokia)]

JCTVC-O0234 Multilayer HRD Management [S. Narasimhan, A. Luthra (Arrisi), S. Hattori, K. Sato, T. Suzuki, A. Tabatabai (Sony)]

JCTVC-O0266 MV-HEVC/SHVC HLS: On flushing of decoded pictures from the DPB based on NoOutputOfPriorPicsFlag [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

### HLS for hybrid scalability (3)

JCTVC-O0143 Specification text and profile for SHVC with AVC base layer [J. Boyce (Vidyo)]

JCTVC-O0166 MV-HEVC/SHVC HLS: On non-HEVC base layer [M. M. Hannuksela (Nokia)]

JCTVC-O0190 AHG15: AVC and HEVC encapsulation for hybrid codec scalability [J. Samuelsson, J. Enhorn, R. Sjöberg (Ericsson)]

### Miscellaneous HLS topics (8)

JCTVC-O0056 MV-HEVC/SHVC HLS: On conversion to ROI-capable multi-layer bitstream [T. Yamamoto, T. Ikai, T. Tsukuba (Sharp)]

JCTVC-O0057 MV-HEVC/SHVC HLS: On support of different luma CTB sizes for different layers [T. Yamamoto, T. Ikai, T. Tsukuba (Sharp)]

JCTVC-O0062 On independent layer [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)] [late]

JCTVC-O0175 MV-HEVC/SHVC HLS: Indications related to single-loop decoding [M. M. Hannuksela, H. Roodaki (Nokia)]

JCTVC-O0188 AHG20: Ultra Low Delay for SHVC, MV-HEVC and 3D-HEVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

JCTVC-O0260 Inter Prediction Signalling and Picture Marking [K. Misra, S. Deshpande (Sharp)]

JCTVC-O0262 MV-HEVC/SHVC HLS: decoding operation in the case of missing reference pictures [M. M. Hannuksela (Nokia)]

JCTVC-O0273 MV-HEVC/SHVC HLS: On multi-mode bitstream extraction [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

## HL syntax in SHVC (6)

### Generic HLS issues (1)

JCTVC-O0251 SHVC HLS: On adaptive resolution change based on single_layer_for_non_irap_flag [V. Seregin, Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

### Slice/picture skipping (4)

JCTVC-O0055 MV-HEVC/SHVC HLS: Skipped slice and use case [T. Yamamoto, T. Ikai, T. Tsukuba (Sharp)]

JCTVC-O0095 AHG9: Skipped slice signaling [Y. He, Y. Ye, X. Xiu, Y. He (InterDigital)]

JCTVC-O0199 SHVC skip pictures [J. Samuelsson, J. Enhorn (Ericsson)]

JCTVC-O0265 MV-HEVC/SHVC HLS: On signaling of enhancement layer skip picture [J. Chen, K. Rapaka, Y.-K. Wang, V. Seregin, X. Li, M. Karczewicz (Qualcomm)]

### Signalling of cropped inter-layer reference (1)

JCTVC-O0098 AHG9: On signaling of scaled reference layer offsets [K. Ugur, M. M. Hannuksela (Nokia)]

## VUI and SEI messages (12)

### VUI (2)

JCTVC-O0106 VUI color description set for XYZ [C. Fogg (Harmonic)]

JCTVC-O0226 MV-HEVC/SHVC HLS: On early indication of parallel processing tools in HEVC extensions [K. Rapaka, Hendry, Y.-K. Wang (Qualcomm)]

### Motion and prediction constrained SEI messages (2)

JCTVC-O0063 HLS: Extensions to Temporal Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami, T.Suzuki (Sony)]

JCTVC-O0255 SHVC / MV-HEVC HLS: On motion and inter-layer constrained tile set SEI messages [Hendry, K. Rapaka, Y.-K Wang (Qualcomm)]

### Frame packing SEI messages (2)

JCTVC-O0198 Additional experiments and software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [S. Reddy, S. Kanumuri, Y. Wu, G. J. Sullivan, H. S. Malvar]

JCTVC-O0249 On frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [D. Bugdayci, K. Ugur, M.M. Hannuksela (Nokia)]

### Other SEI messages (6)

JCTVC-O0064 HLS: SEI message for transfer function information [S. Hattori, T. Suzuki (Sony)]

JCTVC-O0079 Proposed text of Chroma sampling filter hint SEI [K Kazui (Fujitsu), T Chujoh (Toshiba)]

JCTVC-O0099 time code from AVC pic_timing() SEI for HEVC [C. Fogg (Harmonic)]

JCTVC-O0177 MV-HEVC/SHVC HLS: On Layers Not Present SEI message [Jung Won Kang (ETRI), Jinho Lee, Hahyun Lee, Jin Soo Choi, Truong Cong Thang (UoA)]

JCTVC-O0197 HLS: Non-significant tile set for tiled streaming with single layer HEVC extensions [C. Auyeung (Sony)]

JCTVC-O0224 MV-HEVC/SHVC HLS: On Signalling of random accessibility for IRAP pictures in non-IRAP AUs [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, Y. Chen, J. Chen, M. Karczewicz (Qualcomm)]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (0)

### Rate control

### Encoder optimization

### Software development

## Withdrawn and unclear allocation (1 : not counting withdrawn)

JCTVC-O0075 [Withdrawn]

JCTVC-O0077 [Withdrawn]

JCTVC-O0108 [Withdrawn]

JCTVC-O0152 [Withdrawn]

JCTVC-O0229 [Withdrawn]

JCTVC-O0100 field/frame sequence coding [C. Fogg (Harmonic)]

# Plenary Discussions and BoG Reports

[Add notes from joint & parent discussions.]

## Project development

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XX Oct 2013.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

CE plan final at same time as corresponding software except for SCE1 & 4 due to test sequence issues.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 12.0 and SHM 2.0 should be available within 2 weeks after the meeting.

- HM 12.0+RExt should be available within 1 week after HM 12.0 availability.

## Subjective verification test plan

JCTVC-N0385 Draft HEVC verification test plan [T.K. Tan, V. Baroncini]

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1002 HEVC Test Model 12 (HM 12) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 12.0 software version (by 2013-08-XX for ISO/IEC DIS ballot) and the reference configuration encodings according to JCTVC-L1100 common conditions (expected within two weeks after the meeting). - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Study to develop a potential set of HEVC conformance bitstreams. | T. Suzuki, W. Wan (co-chairs) | N |
| HEVC range extensions development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats, bit depths beyond 8 bits, and auxiliary/alpha channel coding, in coordination with AHG7 and AHG18. - Perform memory bandwidth analysis of the range extensions technical design and its proposed modifications. - Evaluate blocking artifacts in 4:4:4 chroma format coding and set up a plan for subjective testing of associated techniques at the next meeting. - Discuss and propose test conditions and test material for the development of the range extensions. - Study techniques for colour conversion and resampling and their relationship to non-4:2:0 chroma coding. | M. Naccari, C. Rosewarne (co-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1005 HEVC RExt draft text (HM 10-RExt-3). - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary), D. Flynn, C. Rosewarne, T. Suzuki | N |
| Range extensions software development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 11.0-RExt-3.0 software version and the reference configuration encodings according to JCTVC-N1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for testing screen content coding performance. - Make recommendations for test conditions for screen content coding. - Coordinate for finalization of the test conditions for CEs relating to lossless and screen content coding. | H. Yu (chair), R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Study the high level syntax for auxiliary pictures with consideration of JCTVC-N0063/JCT3V-E0049 as well as signalling for carrying alpha channel and 4:4:4 chroma enhancement as auxiliary pictures. - Study JCTVC-N0355/JCT3V-E0092 and other mechanisms to extend the number of layers (beyond 64) in comparison with reserving nuh_layer_id value of 63 to provide an extension, in terms of functionality supported and signalling efficiency. - Coordinate efforts with JCT-3V AHG7 on high-level syntax issues in relation to 3D extensions. | M. M. Hannuksela (chair), J. Boyce, Y. Chen, S. Deshpande, A. Norkin, Y.-K. Wang, (vice-chairs) | N |
| SHVC core experiments (AHG10) (jct-vc@lists.rwth-aachen.de) - Establish configurations, generate anchors, and create reporting sheets for SHVC CEs. - Provide configuration data to be used in SHVC CEs (within one week after SHM 3.0 software is available). - Discuss and identify additional issues related to SHVC CEs. | X. Li (chair), J. Boyce, P. Onno, X. Xiu (vice-chairs) | N |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-N1007 SHVC Test Model 3 (SHM 3) text. - Produce and finalize JCTVC-N1008 SHVC text specification Draft 3. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 3.0 software (based on HM 11) for experimentation. - Provide software to CEs within two weeks after the meeting. - Remove IntraBL framework from SHM software. - Bring software into alignment with HM 12 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), T. Chuang, Y. He, D. Kwon (vice-chairs) | N |
| SHVC inter-layer filtering (AHG13) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability. - Study the effect of taking into account chroma position alignment during resampling processing. - Identify alternative candidate resampling filters and distribute associated downsampled test material within three weeks after the meeting. - Study resampling filters with ratios other than 1.5 and 2.0 (and 1.0). - Discuss and identify additional issues related to inter-layer filtering. | E. Alshina (chair), J. Chen, J. Dong, A. Segall P. Topiwala (vice-chairs) | N |
| Colour gamut scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut scalable coding. - Study the interaction of colour gamut scalability with bit-depth and spatial scalability. - Identify test sequences and test conditions. - Discuss and identify additional issues related to colour gamut scalability. | A. Duenas, A. Segall (co-chairs), P. Bordes, J. Dong, D.-K. Kwon, X. Li (vice-chairs) | N |
| Hybrid codec scalability (AHG15) (jct-vc@lists.rwth-aachen.de) - Coordinate with AHG12 on software development and anchor support for AVC base layer. - Study methods for base layer codec identification and picture alignment, including use of encapsulation, and identification of external means. - Study profile and level specification methods for hybrid codec scalability. | J. Boyce, K. Kawamura (co-chairs) | N |
| Single-loop scalability (AHG16) (jct-vc@lists.rwth-aachen.de) - Study and compare single-loop and multi-loop SNR scalability. - Analyze and compare the memory bandwidth of single-loop and multi-loop scalability and the use of "key pictures". - Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signalling inter-layer prediction restrictions on a multi-loop design. - Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability. | M. Wien (chair), M. Budagavi, K. Misra, K. Rapaka, K. Ugur, X. Xiu (co-chairs) | N |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them. - Study restrictions to reduce the complexity of scalable tools and evaluate the impact of such restrictions on coding efficiency. - Prepare a report analyzing performance and complexity of single-layer, simulcast, and scalable coding configurations for 1.0, 1.5, and 2.0 and arbitrary scalability ratio (e.g. ~1.75) cases. - Coordinate with SCE developments and provide a template to be used for complexity analysis of SHM3.0, single-layer coding, and SCE techniques. | E. Alshina (chair), M. Budagavi, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai (vice-chairs) | N |
| High bit-rate & bit-depth operating points (AHG18) (jct-vc@lists.rwth-aachen.de) - Study the accuracy needed for internal processing elements to support bit depths up to 16-bits per sample. - Study relationship to lossless coding capability. - Verify rate-distortion optimization behaviour for high bit rates and high bit depths. - Study entropy coding operation and throughput at high bit rates and high bit depths and potential needs for associated design modification. - Identify test sequences and test conditions for testing high bit rate and high bit depth coding behaviour. - Prepare software implementation for technical investigation of new features intended for high bit rates and high bit depths. - Study coding performance at high bit-rate and high bit depth operating points and investigate the benefit over existing standards. | K. Sharman (chair), E. François, H.-Y. Kim (vice-chairs) | N |
| Verification test preparation (AHG19) (jct-vc@lists.rwth-aachen.de) - Identify test conditions and test sequences for verification of HEVC compression capability, including consideration of use case scenarios for random access and low delay operation. - Identify bit rates, picture resolutions and bit depths appropriate for HEVC verification. - Identify appropriate encoder usage for comparison of HEVC and AVC compression capability. - Communicate with subjective test coordinator to ensure test design validity. - Generate and collect candidate HEVC and AVC encoded bitstreams for HEVC verification testing. - Review and edit the HEVC verification test plan JCTVC-N1011. | T. K. Tan, V. Baroncini (co-chairs), M. Karczewicz, M. Mrak, W. Wan, J. Wen (vice-chairs) | N |
| Multi-layer hypothetical reference decoder (AHG20) (jct-vc@lists.rwth-aachen.de) - Study the issues related to layer-specific and combined multi-layer HRD operation (including DPB and CPB), including the proposed . - Identify layer-specific or combined multi-layer HRD operations that need to be addressed at the system and/or video coding layer. - Study issues and implications of either or both HRD mechanisms when adding or removing layers at re-distribution points. - Study issues and implications of layer-specific buffer operation in relevant application contexts. - Study high-level syntax needs for HRD information, e.g. in SEI, SPS VUI, or VPS VUI. | K. Suehring, A. Tabatabai (co-chairs), S. Deshpande, M. M. Hannuksela, J. Kang, A. K. Ramasubramonian (vice-chairs) | N |
| Best-effort decoding with reduced decoding complexity (AHG21) (jct-vc@lists.rwth-aachen.de) - Identify use cases and key functional elements for application of reduced-complexity decoding processes for decoding of bitstreams encoded for HEVC and its extensions - Analyze tradeoffs for optimization of video quality in the application of reduced-complexity decoding processes - Particularly study characteristics of design and optimization for decoding processes with reduced bit depth - Consider and analyze the impact of inter-picture dependencies and cross-region spatial prediction dependencies in the application of reduced-complexity decoding processes - Study potential approaches to describe/specify the provision of best-effort decoding capabilities in the HEVC text specification | D. Flynn, J. Sole, M. Zhou (co-chairs) | N |
| Test sequence material (AHG22) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for HEVC development. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG8 on screen content coding and AHG19 for verification test development. | T. Suzuki, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |

# Output documents (to be updated)

Remove intra BL from SHVC test model text and software. agreed.

"Call for HEVC conformance bitstreams"?  resolution  perhaps not (discretionary)

Check/update delivery dates.

Planning of HM encoder description – plan to improve this in AHG work toward publishing as part of the RS standard. TBD whether this would be in v.1 of the RS standard or added later.

Issues in conformance: The situation has been improving. profile_idc and level_idc need to be correct, some HM 9 bitstreams not updated, some planned bitstreams missing, many bitstreams need updates for corrections of these issues and others. Coverage still needs improvement. Various combinations of tiles & slices & loop filtering control parameters, SEI messages, "corner cases".

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-N1000 Meeting Report of 14th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

Disposition of comments reports:

WG 11 N13771 Disposition of Comments on ISO/IEC CD 23008-5

WG 11 N13773 Disposition of Comments on ISO/IEC CD 23008-8

JCTVC-N1002 High Efficiency Video Coding (HEVC) Test Model 12 (HM 12) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N 13574) [2013-07-15]

JCTVC-N1003 High Efficiency Video Coding (HEVC) Defect Report) [Y.-K. Wang, G. J. Sullivan, B. Bross] (WG 11 N 13837) [2013-08-31]

JCTVC-N1004 HEVC Conformance Draft 4 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 13774 ISO/IEC DIS) [2013-05-10]

JCTVC-N1005 HEVC Range Extensions Draft 4 [D. Flynn, J. Sole, T. Suzuki] (WG 11 N 13569, ISO/IEC DAM) [2013-06-15]

JCTVC-N1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman] [2013-02-08] (2 week)

describe scc test sequences, consider shortening the length of sequences for operation (shorten if results don't change significantly)

JCTVC-N1007 SHVC Test Model 3 (SHM 3) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13571) [2013-06-15]

Will contain encoder descript and will not retain TextureRL.

JCTVC-N1008 SHVC Draft 3 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13570, ISO/IEC PDAM) [2013-06-15]

JCTVC-N1009 Common SHM test conditions and software reference configurations [X. Li, J. Boyce, P. Onno, Y. Ye] [2013-04-26] (1 week after software)

update to include updated anchor results

JCTVC-N1010 HEVC HM 12 Reference Software [F. Bossen, D. Flynn, K. Sühring] (WG 11 N 13575 ISO/IEC DAM) [2013-05-10]

JCTVC-N1011 HEVC verification test plan draft 1 (WG 11 N 13838) [T.K. Tan, V. Baroncini] [2013-08-02]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting were not considered binding on final CE plans as reviewed in closing plenary.

- JCTVC-N1101 HEVC Scalable Extensions Core Experiment SCE1: Arbitrary scalability ratio support [E. Francois (primary), E. Alshina, J. Chen (CE Coordinators)]

- JCTVC-N1102 HEVC Scalable Extensions Core Experiment SCE2: Key pictures and single-loop decoding [M. Wien, Rapaka, xiu (CE Coordinators)]

- JCTVC-N1103 HEVC Scalable Extensions Core Experiment SCE3: Inter-layer filtering [J. Chen, E. Alshina, J. Dong, M. Sychev (CE coordinators)]

- JCTVC-N1104 HEVC Scalable Extensions Core Experiment SCE4: Colour Gamut and Bit Depth Scalability [A. Segall, P. Bordes, C. Auyeung, X. Li, A. Duenas]

JCTVC-N1101 through JCTVC-N1104 were reviewed in non-final form in closing plenary. A three week finalization period was authorized. Most aspects of review deferred other than intended technologies.

- JCTVC-N1121 HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods [W.-S. Kim, T. Nguyen (CE coordinators)]

- JCTVC-N1122 HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update [C. Rosewarne, J. Sole, K. Sharman, S.-H. Kim (CE coordinators)]

- JCTVC-N1123 HEVC Range Extensions Core Experiment 3 (RCE3) Intra prediction techniques [A. Saxena, D. Kwon, M. Naccari, C. Pang (CE coordinators)]

Both lossless and lossy.

JCTVC-N1121 through JCTVC-N1123 were reviewed in non-final form in closing plenary. A three week finalization period was authorized. Most aspects of review deferred other than intended technologies.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 9–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- 27 Mar. – 4 Apr. 2014 under WG 11 auspices in Valencia, ES.

- 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP.

- ...

The agreed document deadline for the October 2013 meeting is Monday of the week preceding the meeting (14 Oct). No restrictions were planned to be imposed on the scheduling of agenda items within that meeting.

Kenzler Conference Management was thanked for the excellent hosting of the 14th meeting of the JCT-VC. Alpen-Adria University of Klagenfurt was thanked for providing viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1345 hours on Fri. 2 Aug. 2013.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the thirteenth meeting of the JCT-VC, according to a sign-in sheet circuated during the meeting (approximately 161 people in total), were as follows: