# Draft text changes

In 8.4.4

….

–	When the derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouringluma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ 0 ], yCb + bvIntra[ 1 ] ) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in subclause 6.4.1 is invoked with ( xCurr, yCurr ) set equal to ( xCb + nCbS - 1, yCb + nCbS - 1 ) and the neighbouringluma location ( xNbY, yNbY ) set equal to ( xCb + bvIntra[ 0 ] + nCbS - 1 , yCb + bvIntra[ 1 ] + nCbS - 1) as inputs, the output shall be equal to TRUE.

–	One or both of the following conditions shall be true:

–	bvIntra[ 0 ] + nCbS<=  0

–	bvIntra[ 1 ] + nCbS<=  0

In 8.4.5.2.7

….

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-63)

bv[ 1 ] = bvIntra[ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-64)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-65)

If xRefCmp >= xTbCmp and yRefCmp >= yTbCmp, then (xRefCmp, yRefCmp) is modified as follows:

- If (-bv[0]) < (-bv[1])

(xRefCmp, yRefCmp) = (xRefCmp + bv[0], yTbCmp + bv[1]).

–	Otherwise

(xRefCmp, yRefCmp) = (xTbCmp + bv[0], yRefCmp + bv[1]).

- Each sample at the location ( xRefCmp, yRefCmp )is assigned to predSamples[ x ][ y ].