| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 Mar. – 4 Apr. 2014 | Document: JCTVC-Q_Notes_dD |
| --- | --- |

| Title: | Meeting report of the 17th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Valencia, ES, 27 Mar. – 4 Apr. 2014 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventeenth meeting during 27 Mar. – 4 Apr. 2014 at the Palacio de Congresos de Valencia, Valencia, ES. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 27 Mar. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1230 hours on Friday 4 Apr. 2014. Approximately 126 people attended the JCT-VC meeting, and approximately 250 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the sixteenth JCT-VC meeting in producing the 14th HEVC Test Model (HM14) software and text, the range extensions (RExt) Draft 6 with associated test model and software, the Scalable HEVC (SHVC) extensions Draft 5 with associated test model and software, review the results from one interim Core Experiments on RExt (RCE1) and one Core Experiment on SHVC (SCE1), review the responses on the joint call for proposals on screen content coding tools, and review other technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC, RExt, and screen content coding. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were considered, and the results of the verification test that had been set up for HEVC version 1 performance testing were reviewed.

In addition to experiment plan descriptions, the JCT-VC produced 13 other particularly important output documents from the meeting:

- For HEVC version 1, the HEVC test model (HM) 15, HEVC Defect Report draft 4 (for Version 1), HEVC conformance testing draft 7, and the HEVC version 1 verification test report;

- For HEVC range extensions (RExt), the HEVC range extensions draft 7 (final specification draft submitted for ISO/IEC approval), RExt test model 7, and RExt conformance draft 1;

- For HEVC scalable extensions (SHVC), the SHVC extensions draft 6 and SHVC Test Model 6, and a document specifying common test conditions and software reference configurations for SHVC experiments;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

For the organization and planning of its future work, the JCT-VC established 17 "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP, during 17–24 October 2014 under WG 11 auspices in Strasbourg, FR, during 10–18 Feb. 2015 under ITU-T auspices in Geneva, CH, and during 18–26 June 2015 under WG 11 auspices in Warsaw, PL.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its seventeenth meeting during 27 Mar. – 4 Apr. 2014 at the Palacio de Congresos de Valencia, Valencia, ES. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 900 hours on Thursday 27 Mar. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1230 hours on Friday 4 Apr. 2014. Approximately 126 people attended the JCT-VC meeting, and approximately 250 input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2014_03_Q_Valencia/.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the fifteenth JCT-VC meeting in producing the 14th HEVC Test Model (HM14) software and text, the range extensions (RExt) Draft 6 with associated test model and software, the Scalable HEVC (SHVC) extensions Draft 5 with associated test model and software, review the results from one interim Core Experiments on RExt (RCE1) and one Core Experiments on SHVC (SCE1), review the responses on the joint call for proposals on screen content coding tools, and review other technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC, RExt, and screen content coding. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were considered, and the results of the verification test that had been set up for HEVC version 1 performance testing were reviewed.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report (as of the time of preparation of this report in late-June 2014).

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 17 Mar. 2014.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 18 Mar. 2014 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-Q0173 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "P0173+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-Q0094 (a proposal on software for palette mode) [uploaded 03-19],

- JCTVC-Q0117 (a proposal on pic struct element) [uploaded 03-19],

- JCTVC-Q0119 (a proposal on scaled reference layer offset) [uploaded 03-19],

- JCTVC-Q0120 (a proposal on phase offset) [uploaded 03-19],

- JCTVC-Q0122 (a proposal on reference sample locations) [uploaded 03-19],

- JCTVC-Q0154 (a proposal on HLS for picture flushing and DPB) [uploaded 03-20].

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-Q0087 (an information document about coding results) [uploaded 04-04],

- JCTVC-Q0136 (a document discussing test conditions for colour gamut scalability) [uploaded 03-27].

The following cross-verification reports were registered on time but were uploaded late: JCTVC-Q0097 [uploaded 03-28], JCTVC-Q0098 [uploaded 03-26], JCTVC-Q0099 [uploaded 03-25], JCTVC-Q0123 [uploaded 03-25], JCTVC-Q0125 [uploaded 03-26], JCTVC-Q0143 [uploaded 03-26], JCTVC-Q0144 [uploaded 03-26], JCTVC-Q0150 [uploaded 03-27], JCTVC-Q0151 [uploaded 03-27], JCTVC-Q0152 [uploaded 04-02], JCTVC-Q0153 [uploaded 03-27], JCTVC-Q0171 [uploaded 04-02], JCTVC-Q0172 [uploaded 04-02].

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-Q0058, JCTVC-Q0121, JCTVC-Q0126, JCTVC-Q0233.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution document was rejected as a "placeholder" without any significant content and was not corrected until after the upload deadline:

- JCTVC-Q0137 (a proposal on adaptive SAO offset scaling, corrected by a late upload on 03-20)

- JCTVC-Q0140 (a performance report on encoder-only changes for SCC, corrected by a late upload on 03-19)

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-P1000, the HEVC Test Model (HM) JCTVC-P1002 (which still needs further attention to reach the quality we would like it to have), the Defect Report JCTVC-P1003, the Conformance Draft JCTVC-P1004, the Draft Specification of Range Extensions JCTVC-P1005, the RExt Test Model JCTVC-P1013, the SHVC draft specification JCTVC-P1008, the SHVC test model JCTVC-P1007, the common test conditions for RExt (JCTVC-P1006) and SHVC (JCTVC-P1009), and the HEVC verification test plan JCTVC-P1011 were approved. The HM reference software and the reference software versions for range extensions and SHVC, were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Review of contributions submitted to the CfP on screen content coding

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-acess skip.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC).

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- OLS: Output layer set

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Rancom-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC).

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a draft standard.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing

- Block and unit names:

  - CTB: Coding tree block (luma or chroma).

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU)

  - CB: Coding block (luma or chroma).

  - CU: Coding unit (containing both luma and chroma).

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma)

  - PU: Prediction unit (containing both luma and chroma), with four shape possibilities for non-AMP cases.

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN: Having two areas that each have the full width and half the height of the CU.

    - Nx2N: Having two areas that each have half the width and the full height of the CU.

    - NxN: Having four areas that each have half the width and half the height of the CU.

  - TB: Transform block (luma or chroma).

  - TU: Transform unit (containing both luma and chroma).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

Opening remarks included:

- Remarks on lateness of documents.

- Remarks on the number of documents.

Primary topic areas were noted as follows:

- HLS

- SHVC

- RExt

- Corrigenda items and preparation of new edition (how to progress RExt to final approval ballot in ISO/IEC)

- Screen content coding CfP results

Other

- Verification testing had been completed

Unfinished (or less-than-optimally finished) deliverables

- HEVC conformance (& Draft DoCR)

- RExt draft finalization

- HM test model text

Two main tracks were followed for most meeting discussions:

- HLS

- RExt & SCC

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on SHVC HLS, SHVC and RExt CE work and related contributions during the first few days. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Thu. 27 Mar., 1st day 0900–2000

  - JCT-VC opening and review of AHG reports [JRO & GJS] 0900–1315 (MP1)

  - SCC CfP response presentations of technical proposals [JRO] 1445–2000 (MP1)

  - HLS BoG [J. Boyce] 1445–2000 (Aud. 3B)

- Fri. 28 Mar., 2nd day 0800–1900

  - RExt [JRO] 0800–1300, 1630–1900 (MP1)

  - HLS BoG [J. Boyce] 0900–1300, 1430–1900 (3B), esp.:

    - 7.5.2 IRAP alignment and POC derivation

    - 7.5.3 RPS signalling and derivation

    - 7.5.5 Parameter sets

  - SCC CfP Summarization BoG [H. Yu. R. Cohen, R. Joshi] 0800–1300 (3A)

  - SCC CfP results review [JRO] 1430–1600 (MP1)

  - SCE1 and related non-CE SHVC contributions [GJS] 1430–1900 (3A)

- Sat. 29 Mar., 3rd day 0800–2000

  - Parallel activity: JCT-3V first day 0900–2000 (3A+B)

  - Non-CfP SCC or RExt [R. Cohen] 0830–1330, 1500–1730 (MP1)

  - HLS [Y.-K. Wang] 0900–1330, 1500–1630 (S2)

  - SHVC position calculation [J. Boyce] 0800–1330 (S1)

  - JCT-VC review of Non-CfP SCC / RExt and SHVC position calculation [GJS] 1600–2000 (MP1)

- Sun. 30 Mar., 4th day 0900–2000 [Summer time change noted]

  - JCT-VC review of SHVC position calculation and HLS [GJS] 0900–1200 (Aud. 2)

  - JCT-VC remainders [GJS] 1200–1400, 1330–1800

  - RExt BoG [D. Flynn & C. Rosewarne] 1830–2000

  - HLS BoG [J. Boyce] 1830–2000

- Mon. 31 Mar., 5th day 1430–2100

  - Parent level: WG 11 plenary 0900–1400

  - Parent level: VCEG plenary 1400–1600

  - Parent level: Joint meeting 1600–1800 (SCC &c)

  - JCT-VC HLS 1815–2100 (GJS) (3B)

  - JCT-VC RExt 1815–2100 (JRO) (Aud. 2)

- Tue. 1 Apr., 6th day 0800–2100

  - JCT-VC RExt and next steps for SCC (GJS & JRO) 0800–1000+ (Aud. 2)

  - SHVC position calculation [J. Boyce] 0800–1000 (Room A)

  - HLS [J. Boyce] 0900–1100 (Room A)

  - SCM [R. Cohen] (Aud. 2)

  - RExt editorial review [GJS] 1000 (Foyer Aud. 2B)

  - RExt colour VUI [GJS] 1200 (Foyer Aud. 2B)

  - Parent level: Joint meeting 1600–1800 (various topics)

  - RExt editorial review [DF] 1800–2100 (3A)

- Wed. 2 Apr., 7th day 1130–2100

  - Parent level: WG 11 plenary 0900–1100

  - JCT-VC review of side activity, RExt profiling details 1130−1300 (Aud. 2)

  - Parent level: VCEG plenary 1300–1400 (Aud. 2)

  - SCC 1500−1700 BoG (Room 3A)

  - HLS 1500−1800 (Aud. 2)

  - JCT-VC remainders 1800-2100 (Aud. 2)

    - SHVC CGS phase 2.1 vs. 2.2

    - HEVC v1 Conformance Q0219

    - SHVC conformance (To be part of conformance AHG – volunteers: J. Boyce, Y. Ye, T. Suzuki, A. Ramasubramonian) & ref soft

    - Verification test follow-up (Q0019, Q0204 to be the basis of output doc)

    - RExt conformance Q0249

    - RExt coding features Q0051

  - RExt finalization and editorial review BoG 1900−2100

  - Parent level: ISO/IEC editor guidance 1700–1800 (Room 5)

- Thu. 3 Apr., 8th day 0800–2100

  - Parent level: Joint meeting 1500–1700 (various topics)

  - BoG position & phase adj SHVC Room A 8am

  - BoG HLS Room A 1000

  - BoG SCC Room 3A 8am-1000

  - JCT-VC & JCT-3V joint HLS review 1700-1800 Aud. 2

  - JCT-VC Remainders 0800-1500 Aud. 2

    - 0800 Misc

    - 1000 Still picture

    - 1030 SCC Review

    - AHG plans

    - RExt DoCR (IT NB: FPA)

    - Rectangular region FPA

    - 1700 HLS BoG and revisits review Q0223

    - Shared DPB further discussion

    - 1900 SHVC position & phase adjustment BoG review

    - CE planning (SCC, CGS WP?, 12 b SHVC?)

    - Output doc planning

    - Notes on some discussions completed

      - RExt, CABAC initialization and CBF

      - Version 1 defects MinCR (confirmed Wed p.m. (GJS)), Q0111

      - SCC Project desc.

      - Profiles – see joint meeting notes in section 8.1.

        - CPB size effect for all-intra profiles

        - Bit rates for high-throughput profile

        - 12 bit profile Q0206

- Fri. 4 Apr., 9th day 0800–1230

  - Parent level: WG 11 plenary 1400–2200

  - JCT-VC closing plenary 0800–1230

    - AHG plans

    - Output docs

    - WG11 DoCRs, NBs, resolutions

    - CE plans

    - Q0256 check

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks was subject to changes)

- AHG reports (19) Track P (section 2)

- Communication to and by parent bodies (0) Track P (section 3.1)

- Conformance testing development (0) Track P (section 3.2)

- Version 1 bug reports and cleanup (1) Track P (section 3.3)

- Coding performance, implementation, and design analysis (5) Track P (section 3.4)

- Profile and level definitions (7) Track P/Joint with Parent Bodies (section 3.5)

- Use cases (4) Track P/Joint with Parent Bodies (section 3.7)

- Source video test material (2) Track P (section 3.8)

- SHVC CE1: Colour gamut and bit depth scalability (9) Track A (section 4)

- RExt CE1: Adaptive motion vector precision (5) Track B (section 5)

- CfP on screen content coding (12) Track P (section 6)

- Non-CE RExt and related contributions (37) BoG | Track B (section 7.1)

- Non-CE SHVC (17) BoG | Track A (section 7.2)

- Non-CfP SCC (31) BoG (section 7.3)

- High-level syntax common issues in RExt, 3D, SHVC, single layer (7) BoG | Track A (section 7.4)

- High-level syntax in SHVC and 3D extensions (33) BoG | Track A | joint with JCT-3V (section 7.5)

- VUI and SEI messages (9) BoG | Track A (section 7.6)

- Non-normative (0) (section 7.7)

- Plenary discussions and BoG reports (0) Track P (section 8)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 9, 10, and 11)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports (19)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Reviewed Thu 27th a.m. except as noted otherwise.)

JCTVC-Q0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm (AHG chairs)]

In the interim period since the 16th JCT-VC meeting, the 14th HEVC Test Model (HM14) software and text had been produced; one interim Core Experiment on range extensions (RCE1) and one Core Experiment on scalable extensions (SCE1) were run. In preparation of the 17th meeting, progress was made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions was also a significant goal. Needs for corrections to version 1 were considered, and a verification test was further executed for HEVC version 1 performance testing. Furthermore, a joint call for proposals on screen content coding tools had been issued by the parent bodies, and 7 complete proposals were received.

The work of the JCT-VC overall had proceeded well and actively in the interim period. Active discussion had been carried out on the group email reflector (which had 1753 subscribers as of 2014-03-26), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2014_01_P_Sanjose/), particularly including the following:

- The meeting report (JCTVC-P1000) [Posted 2014-03-26]

- The HM 14 encoder description (JCTVC-P1002) [Posted 2014-03-26]

- The HEVC (version 1) defect report draft 3 (JCTVC-P1003) [Posted 2013-02-08]

- The HEVC conformance specification Draft 6, to be submitted as Preliminary Draft of ISO/IEC FDIS 23008-8 (JCTVC-P1004) [First posted 2013-03-27]

- HEVC range extensions Draft 6, to be submitted as I Preliminary Draft of ISO/IEC FDAM (JCTVC-P1005) [First posted 2014-02-19, but additional update was necessary – final update 2014-04-03]

- HEVC range extensions common test conditions and software reference configurations (JCTVC-P1006) [First posted 2014-01-24, last updated 2014-02-05]

- SHVC Test Model 5 (JCTVC-P1007) [Posted 2014-02-28]

- SHVC Draft 5, submitted as ISO/IEC DAM (JCTVC-P1008) [First posted 2014-01-19, last updated 2014-01-22]

- Common SHM test conditions and software reference configurations (JCTVC-P1009) [Posted 2014-02-08]

- HEVC verification test plan draft 3 (JCTVC-P1011) [Posted 2014-01-17]

- Range Extensions Test Model 6 encoder description (JCTVC-P1013) [First posted 2014-01-19, last updated 2014-02-23]

- Description of HEVC Scalable Extensions Core Experiment 1 (SCE1): Colour gamut and bit depth scalability (JCTVC-P1101) [First posted 2014-01-18 with final update 2014-01-22]

- Description of HEVC Range Extensions Core Experiment 1 (RCE1): Adaptive motion vector precision (JCTVC-P1121) [First posted 2014-02-01 with final update 2014-02-06]

The software for HM version 13.0 had been released just after the last meeting (Jan 22). Software for SHVC and Range Extensions were built on top of HM13. HM 14.0 had been prepared but not yet released.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-13.0.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-13.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 200 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted.

A preliminary basis for the document subject allocation and meeting notes for the 17th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-Q0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (AHG co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 16th meeting in San José (9–17 January 2014) and the 17th meeting in Valencia (27 March – 4 April 2014).

At the 16th JCT-VC meeting, documents JCTVC-O1002 and JCTVC-O1003 were approved as JCT-VC output documents. In addition, document JCTVC-P0064, detailing some defect corrections for HEVC version 1 (esp. modulo CPB removal delay wrapping), was adopted.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents. A single issue with version 1 of the HEVC specification (JCTVC L1003 and JCTVC-O1003) was reported on the tracker following the 16th JCT-VC meeting. No issues with the HM13 Encoder Description in JCTVC-O1002 were reported on the tracker during this period.

The HM14 Encoder Description was published as JCTVC-P1002.

The HEVC Defect Report Draft 3 was published as JCTVC-P1003.

The recommendations of the HEVC test model editing and errata reporting AHG were for JCT-VC to:

- Approve the HM14 Encoder Description in document JCTVC-P1002 as JCT-VC output.

- Approve the HEVC Defect Report Draft 3 as JCT-VC output.

- Determine whether or not an HM15 Encoder Description should be created, given the absence of any input documents or issues raised via the tracker.

- Encourage the use of the issue tracker to report issues with the text of both the HEVC v1 specification and the Encoder Description.

- Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to any mismatches between software and text.

- Consider further proposals for editorial improvements to HEVC v1.

JCTVC-Q0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring (AHG chairs)]

This report summarizes the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 16th and 17th JCT-VC meetings. Activities focused on fixing bugs.

A brief summary of activities is given below.

- Development of the software was coordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN servers set up at HHI and the BBC, as announced on the JCT-VC email reflector.

- The HM user manual had been updated and a version controlled copy is included in the doc directory of the repository. A PDF version had been produced and was included in the same location prior to each HM release.

- Version 14.0 was still in development to be released during the 17th JCT-VC meeting. A number of bugs have been identified and fixed.

- There were a number of reported software bugs that should be fixed.

Multiple versions of the HM software were produced and announced on the JCT-VC email reflector. The following sections give a brief summary of the changes made for each version. A detailed history of all changes made to the software can be viewed at https://hevc.hhi.fraunhofer.de/trac/hevc/timeline.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-11.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-11.0-dev).

HM 13.0 had been released on January 22, 2013.

The coding performance did not change compared to HM-12.0.

HM 13.0 was the basis for CE, RExt and SHVC software development.

HM 14.0 had not yet been released, but was expected during the 17th JCT-VC meeting. This version contains additional bug fixes as well as improved handling of encoding field sequences. It should also be noted that the submission of additional conformance bitstreams had helped fixing a number of bugs and implementing features that were previously not available in HM. Notable additions included:

- Additional bug fixes

- Rules pertaining to picture output

- Processing of filler data

- Improved field coding (incl. recovery point SEI messages)

HM 14.0 was expected to correctly decode all conformance bitstreams submitted to date.

Unless the release has been tagged, the development branch can be found under

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-13.0-dev

There were no performance changes expected for HM 14.0.

Recommendations:

- Continue to develop reference software based on HM version 14.0 and improve its quality.

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bitstreams.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bistreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

Regarding the potential to issue an ISO/IEC FDIS at this meeting, it was agreed to finalize this in July rather than now for synchronization with the ITU-T approval process.

JCTVC-Q0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan (AHG chairs)]

This AHG was established to develop conformance specification of HEVC

The ftp site at ITU-T was used to exchange bitstreams. The ftp site for downloading bitstreams is,

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

The generated bitstreams are available at

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_conformance/

The features and conformance point of each bitstream are summarized in the attached Excel sheet.

12 bitstreams were revised – the known problems in those were fixed (EXT_A_ericsson_4, NUT_A_ericsson_5, RPS_C_ericsson_5, RPS_D_ericsson_6, IPRED_A_docomo_2, MVDL1ZERO_A_docomo_4, RAP_A_docomo_5, RPS_F_docomo_2, RPS_A_docomo_5, ENTP_A_QUALCOMM_1, ENTP_B_Qualcomm_1, ENTP_C_Qualcomm_1).

13 new bitstreams were generated.

Some requested bitstreams were not yet generated. These were listed in the report, along with volunteers for most cases.

JCTVC-Q0219 was an input document from editors.

Regarding the potential to issue an ISO/IEC FDIS at this meeting, it was agreed to finalize this in July rather than now for synchronization with the ITU-T approval process.

The need was identified to discuss plans for extensions conformance testing.

JCTVC-Q0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [C. Rosewarne, M. Naccari (AHG chairs)]

This report summarizes the HEVC range extensions development Ad-Hoc Group (AHG5) between the 16th JCT-VC meeting in in San José, CA, USA (January 2014) and the 17th JCT-VC meeting in Valencia, ES, (March-April 2014).

After a kick-off message issued on the 27th January no further emails were sent to the reflector regarding AHG5.

Contribution JCTVC-P1006 was uploaded, defining the test conditions as agreed at the 16th JCT-VC meeting for Range extensions development. This contribution also includes the spread sheet templates to be used in RExt Core Experiments (RCEs) at this meeting for AHG5, 8 and 18 related experiments.

Related contributions to the meeting were listed and categorized in the report.

The AHG recommended:

- To review all related contributions.

- To include appropriate methods into HEVC range extensions WD and HM range extensions software.

JCTVC-Q0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, G. J. Sullivan, T. Suzuki (AHG chairs)]

This document reports on the work of the JCT-VC ad hoc group on Range extensions draft text (AHG6) between the 16th JCT-VC meeting in San Jose, USA (January 2014) and the 17th JCT-VC meeting in Valencia, ES (March/April 2014).

The High Efficiency Video Coding (HEVC) Range Extensions test model was developed following the actions taken at the 16th JCT-VC meeting in San Jose, US (January 2014).

One version of JCTVC-P1005 was published by the Editing AhG following the 16th JCT-VC meeting in San Jose, which is based upon JCTVC-L1003_v34.

Changes in JCTVC-P1005 relative to the previous version was as follows:

- Fixed loosened constraint on scaling list scaling_list_pred_matrix_id_delta [#1231]

- Fixed incorrect coordinate scaling in §8.4.5.1, §8.7.3.2, §8.7.2.5.8 [#1234, #1236, #1238]

- Fixed incorrect cbf check for 4:2:2 in transform unit syntax [#1235]

- Added inferrence rule for delta_chroma_log2_weight_denom for monochrome [#1237]

- Fixed typos in §8.4.1

- Integrated extra context for log2_res_scale_abs_plus1 (JCTVC-P0154)

- Integrated PPS signalling of SAO offset scale values (JCTVC-P0222)

- Integrated Luma-Chroma scaling for unequal component bitdepths (JCTVC-P0066)

- Integrated change to horizontal BV range limit to be CTU size independent (uses 64) (mtgNotes)

- Integrated change of intraBV vector initialization (JCTVC-P0304)

- Integrated categorized Rice parameter initialization (JCTVC-P0199)

- Integrated modifications to temporal motion constrained tile sets SEI message (JCTVC-P0051, JCTVC-P0172)

- Integrated (without diagram) knee function information SEI message (JCTVC-P0050)

- Integrated IntraBC NxN, 2NxN, Nx2N modes (JCTVC-P0180)

- Integrated CABAC bypass alignment for high rates (JCTVC-P0060)

- Integrated VUI for SMPTE 2084 and XYZ (JCTVC-P0084)

- Integrated mastering display colour volume SEI message (JCTVC-P0084)

- Integrated PPS extension mechanism (JCTVC-P0166)

- Renamed luma-chroma prediction to cross-component prediction

Not yet integrated:

- PPS extension mechanism

- Profiles

- Some reported issues in the tracker

The recommendations of the HEVC Range extensions draft text AHG were to:

- Approve the edited JCTVC-P1005 document as JCT-VC output

- Encourage the use of the issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc/) to facilitate the reporting of issues with the text

- Resolve all text issues logged in trac

- Compare the HEVC Range Extensions document with the HEVC Range Extensions software and resolve any discrepancies that may exist, in collaboration with the Range extensions software development (AHG7)

- Ensure that properly drafted candidate text for the HEVC Range Extensions text specification is available prior to introducing changes to the specification

- Review, before the end of the meeting, all adoptions for integration in the output text

- Recommned that the meeting notes include suffcient details of adoptions based on recommendations of BoGs

- Continue to improve the overall editorial quality of the draft text specification, to allow it to proceed to FDAM ballot following this meeting

The following needs for further discussion were identified:

- Further improvement of draft

- DoCR

- PPS extension signalling

- FDAM process issue – possibilities:

  - Delta documents for each ballot, new edition subsequently (e.g. October)

  - FDIS for RExt, delta for July docs

  - FDIS for all in July

JCTVC-Q0007 JCT-VC AHG report: Range extensions software (AHG7) [K. Sharman, D. Flynn (AHG chairs)]

This report summarizes the activities of Ad Hoc Group 7 on support for range extensions between the 16th and 17th JCT-VC meetings.

Following the end of the previous meeting on Friday 17th January, The HM12.0_RExt5.1 software was upgraded to HM13.0_RExt5.1 (reflecting the changes made to the main HM branch) and then to HM13.0_RExt5.2 (macro removal), with both revisions released on Tuesday 28th January. All were tested according to the HM and RExt6 test conditions.

On Thursday 30th January, the development code for RExt6.0 (RExt6.0_rc1) with changes for all tools was announced, and a request was made for proponents to verify their tools. A few small changes were subsequently made, including a bug fix to the previously adopted cross-component prediction inter search algorithm (see JCTVC-Q0076). This then lead to the full release of RExt6.0 on Wednesday 5th February.

The changes made were due to the following topics:

- Renaming of cross-component decorrelation to cross-component prediction.

- O0099		– Time Code SEI

- RCE1-B3a	– Entropy coding alignment for 16-bit profiles

- RCE2-A1	– Golomb-Rice Coding Initialization

- RCE3-D2	– NxN, 2NxN, Nx2N intra block copy (with PU-level overlap prohibited)

- Mtg notes	– Intra-Block-Copy search area always 64 wide.

- P0044		– Tiers and levels.

- P0066		– Bit depth correction for cross-component-prediction

- P0154-Method1	– Add additional context for cross-component prediction alpha signalling

- P0166		– Align PPS extension syntax with SPS extension syntax

- P0222+notes	– SAO programmable shift values to be sent in PPS

- P0304 (RCE3)	– Change of initial Intra-Block-Copy predictor to (-W,0)

On Monday 17th March, the following tools were also added, alongside some bug-fixes for 4:2:2 processing. It is hoped that this will become RExt6.1, following requested feedback from the community.

- O0044		– CU-adaptive chroma QP offsets

- The encoder scheme currently defined does not employ a rate-distortion search.

- O0079		– Chroma sampling filter hint SEI message

- P0050		– Knee function SEI message

- P0084		– Master Display Colour Volume SEI message

The following were still to be integrated:

- O0043		– Best-effort decoding

- Motion constrained tile sets SEI message

  - N0383		– Motion constrained tile sets SEI message

  - P0051		– Extensions to temporal motion constrained tile sets SEI message

  - P0172-F	– Further extensions to temporal motion constrained tile sets SEI message

Some significant gains in compression performance were shown (see tables in report) – esp. for:

- SCC cases, primarily due to NxN and 2NxN/Nx2N

- High bit depth, primarily due to Rice adaptation rule change

Comparison to AVC was also provided for 14 b and 12 b. (There were some problems running the AVC JM in these cases.)

The following contribution relates to the problem of having multiple encoder search algorithms within RExt:

- JCTVC-Q0076 Unifying HM and RExt Inter-Prediction Search [K. Sharman, N. Saunders and J. Gamei]

Recommendations made by the AHG:

- Continue to develop reference software based on HM and improve its quality.

- Update encoder and documentation to be consistent with the current usage.

- Remove macros introduced in previous HM versions before starting integration towards RExt7.0 such as to make the software more readable.

- Continue to identify bugs and discrepancies with text, and address them.

- Test reference software more extensively outside of common test conditions.

JCTVC-Q0008 JCT-VC AHG report: Screen content coding (AHG8) [H. Yu, R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu (AHG chairs)]

This report summarizes the activities of the JCT-VC ad hoc group on screen content coding (AHG8) between the JCT-VC 16th meeting in San José, USA, and the 17th meeting in Valencia, ES.

There had been much effort in studying and developing new coding tools for screen content during this short interim period. So far, a total of 33 SCC related contributions had been submitted to this meeting, and 7 of them were registered and submitted as SCC CfP response proposals. It was reported that, basically, the tools discussed in these proposals can be classified into the following areas: Intra-BC, in-loop filtering process, colour palette/table, colour transform, inter colour prediction, string match, inter picture coding, etc. A list of contributions was given in the AHG report.

It was recommended to

- Present contributions and create CEs to further study these new tools.

- Schedule viewing sessions, whenever necessary and possible, to further evaluate and assess the subjective quality of these new coding tools.

JCTVC-Q0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) [M. M. Hannuksela, J. Boyce, Y. Chen, S. Deshpande, J. Samuelsson, Y.-K. Wang, P. Wu (AHG chairs)]

This report summarizes the activities of the ad-hoc group on high-level syntax for HEVC extensions (AHG9) between the 16th JCT-VC meeting and the 17th JCT-VC meeting.

No coordinated AHG activity took place. No emails were exchanged over the JCT-VC reflector.

The number of related input contributions was approximately 43, and these were listed in the report.

- JCTVC-Q0010 JCT-VC AHG report: Palette-based coding (AHG10) [W. Pu, P. Onno, X. Guo, P. Lai (AHG chairs)]

Discussed Sat 29th (JRO) or Sun 30th (D. Flynn & C. Rosewarne as BoG).

Contribution JCTVC-Q0063 falls into the category of “Improvement on palette prediction”.

In the current AHG10 suggested software, the palette items in the last palette coded CU are used as predictors to predict the current CU’s palette. Proposals in the category suggest to use more palette predictors from more than one previous coded CUs.

The following contributions fall to the category of “Improvement on index and run-length coding”:

- In the current AHG10 reference software, run-length code is used with three run modes, i.e. copy left run mode, copy top run mode, and escape mode. JCTVC-Q0064-66 in this category suggest to includes more run modes and improve run-length / index coding.

- In JCTVC-Q0047, bug fixes for RExt6.0 based AHG10 suggested software was reported. It also reported encoder only improvements and improved palette index coding.

- In JCTVC-Q0138, a MATCH mode is designed based on a line-matching above copy method to code colour indices.

- JCTVC-Q0169 and JCTVC-Q0174 presented results of adaptive scanning and modified copy above run mode.

The following contributions fall to the category of “Tool combinations”:

- Proposals in the category combine more than one palette coding tools to improve palette based coding.

- In JCTVC-Q0069, combination of JCTVC-Q0063, JCTVC-Q0064, JCTVC-Q0065, JCTV-Q0066 was reported.

- In JCTVC-Q0071, three tools to improve coding efficiency of palette based coding of screen content including (i) long-term palette prediction, (ii) restricted run-length coding and (iii) adaptive scanning of palette coded CUs are evaluated.

- In JCTVC-Q0083, a triplet palette mode combining the triple palette and component wise palette was reported. This proposal also reported improved index coding methods.

In JCTVC-Q0094, RExt6.0 based AHG10 suggested software was reported.

JCTVC-Q0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang (AHG chairs)]

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 16th JCT-VC meeting in San Jose, US, (9 – 17 Jan. 2014) and 17th JCT-VC meeting in Valencia, Spain, (27 March – 4 April 2014).

The editorial team worked on two documents: JCTVC-P1007 (SHVC Test Model 5 text) and JCTVC-P1008 (SHVC draft 5). Editing JCTVC-P1008 was assigned a higher priority than editing JCTVC-P1007.

One version of JCTVC-P1007 and four versions of JCTVC-P1008 were produced by the editing AHG following the 16th JCT-VC meeting in San Jose. The ISO/IEC SHVC DAM document was generated based on the final version of JCTVC-P1008.

The main changes in JCTVC-P1008, relative to the previous JCTVC-O1008 (SHVC Draft 4), are:

- Incorporation of all common SHVC and MV-HEVC HLS normative adoptions at the 16th JCTVC meeting

- Integration of all SHVC-specific normative adoptions at the 16th JCTVC meeting, including:

  - (JCTVC-P0312): vertical phase adjustment in resampling process for field-to-frame scalability

  - (Scalable Main 10 profile): Profile for up to 10 bits in each layer

  - (RL and EL bit depth restriction): EL bit depth shall not be smaller than RL bit depth (when the EL depends on the BL)

- Editorial improvements and fixes

- Fixes of bug tracker tickets.

The JCTVC-P1007 Test Model 5 document mainly contains the general descriptions of SHVC framework, texture data resampling process and motion field mapping process. The main change to the previous JCTVC-O1007 (SHM4) is the inclusion of field-to-frame scalability.

The AHG recommended to:

- Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the overall editorial quality of the SHVC Draft and Test Model documents.

- Request that proponents provide mature text for integration into the SHVC draft.

JCTVC-Q0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He (AHG chairs)]

This report summarizes activities of the AHG12 on SHVC software development between 16th and 17th JCT-VC meetings.

The current latest software version was SHM-5.1, containing almost all the items adopted last meeting, however some work was still needed on POC derivation related to JCTVC-P0041 and JCTVC-P0056.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

Two software versions had been released by AHG12, integration details and performance summary are given in the next subsections. In the document, only HEVC base layer results are provided and AVC base layer data can be found in the accompanying Excel tables. Performance results are reportedly consistent with the adopted techniques.

The performance was essentially unchanged relative to prior SHM software (basically only HLS changes).

Recommendations:

- Continue to develop reference software based on SHM-5.1 and improve its quality.

- Resolve identified software and working draft mismatches.

JCTVC-Q0013 JCT-VC AHG report: SHVC inter-layer filtering (AHG13) [E. Alshina, J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (AHG chairs)]

This document reports on the work of the JCT-VC AHG on SHVC inter-layer filtering (AHG13) between the 16th JCT-VC meeting in San Jose, US, (9 – 17 Jan. 2014) and 17th JCT-VC meeting in Valencia, Spain, (27 March – 4 April 2014) and, and lists the related input documents.

There had not been discussions related to the mandates of this ad-hoc group over the reflector between the 16th JCT-VC meeting and the 17th JCT-VC meeting. However, there was significant activity in input contributions for this meeting.

It was reported that AhG13 related activity could be classified in 4 major topics:

- Phase shift in re-sampling process adjustment;

- Additional inter-layer filtering;

- Resampling process refinement;

- Bug fix in inter-layer filtering for bit-depth scalability.

Several contributions fall to the category "Phase shift in re-sampling process adjustment".

- Current SHVC draft distinguishes 2 types relative displacement between base and enhancement pictures introduced by down-sampling process which (zero-phase shift or central position alignment). In re-sampling process Chroma sample in 4:2:0 format is assumed to be half-pel vertically displaced relatively to Luma sample.

- A solution which handles interlaced-to-progressive scalability was adopted at JCTVC 16th meeting. Re-sampling process distinguishes 3 cases in current SHVC draft spec: picture to picture, top filed –picture and bottom-field to picture conversion. Phase shift during re-sampling is derived from system of flags indicating one of mentioned above situation.

- Proposals in this category suggest reorganizing the syntax for re-sampling process phase adjustment in order to cover wider variety of combinations. In contributions from Arris and Nokia, phase offsets are derived from system of flags. In Qualcomm and InterDigital contributions, phase offsets are implicitly signalled.

- It was asserted that the resampling process in the current SHVC draft is not well enough suited for interlaced-to-progressive scalability when the base layer contains coded frames. But no performance or subjective test results were reported in order to confirm this statement.

Several contributions fall to the category "Additional inter-layer filtering":

- Proposals in this category were tested using SCE1 settings: base and enhancement layers have different colour gamut. Colour-conversion methods introduce additional processing step during inter-layer reference picture construction executed before or after re-sampling process.

- The highest performance in SCE1 tests (~8% average BD-rate gain) was demonstrated by inter-layer cross-colour filter with coefficients depending on triplet (Y,Cb,Cr) position in colour space. Roughly speaking, this gain is composed from ~5% gain which comes from splitting the colour space into multiple regions (up to 8x8x8) and another ~3% comes from cross-colour inter-layer filtering [JCTVC-Q0072]. More details about these tools, comparison and complexity analysis can be found in AhG14 and SCE1 reports.

- Denoising for inter-layer reference previously was proposed for SNR scalability only for colour-gamut scalability content shows BD-rate gain both for spatial ratio ×1 and ×2. Average gain for inter-layer reference denoising is reportedly ~1% and is additive with colour-gamut conversion tools.

- About 1.5% BD-rate gain could be achieved with encoder modifications for weighted prediction utilized in SCE1 anchor for inter-layer colour-gamut conversion.

Several contributions fall to the category "Re-sampling process refinement".

- In an Ericsson contribution, a strange performance drop was demonstrated in case of 540p to 720p conversion (spatial ratio 4:3). Proposed solution adds one SPS level flag which affects Scaled Reference Offsets and Scaling factor derivation.

- In an Arris contribution, the shift value in reference sample position derivation is modified. Shift becomes dependent on reference layer picture size.

- In a Sharp contribution, a scaling factor and reference sample position derivation for region-of-interest extraction is proposed. It is a re-proposal of a contribution from 15th and 16th JCTVC meetings responding to the request for clarification of application case.

Several contributions fall to the category "Bug fix in inter-layer filtering for bit-depth scalability".

- This is s/w bug-fix for bit-depth conversion. Bit-depth conversion between layers is incorporated with re-sampling process. In case of spatial ratio ×1 it was simply not implemented. The bug fix reportedly provides more than 30% BD-rate gain.

Recommendations:

- Review related contributions

- Fix bit-depth conversion for ×1 case in reference s/w.

JCTVC-Q0014 JCT-VC AHG report: Colour gamut and bit depth scalability (AHG14) [P. Andrivon, A. Duenas, E. Alshina, S. Deshpande, X. Li, K. Ugur, Y. Ye (AHG chairs)]

This report summarizes the activities of the ad-hoc group of colour gamut and bit depth scalability (AHG14) between the 16th JCT-VC meeting and the 17th JCT-VC meeting.

The AhG used the JCT-VC reflector for all discussions. A kick-off message was sent on January 30th, 2014. There were limited activity (typos, cross-checking volunteering, SHM bug-fixes) related to the SCE1 over the SCE1 participants mailing list between the 16th JCT-VC meeting and the 17th JCT-VC meeting.

As of the 25th of March 2014, there were 14 contributions related to colour gamut and bit-depth scalability.

Two contributions are primary SCE1 proposals (JCTVC-Q0048 and JCTVC-Q0072). There are 4 non-SCE1 contributions mainly built on top of existing SCE1 technology proposals. One contribution proposes to extend test conditions (JCTVC-Q0136).

An SCE1 report (JCTVC-Q0021) provides details on complexity and performance of CGS Inter-Layer Prediction (ILP) mechanism proposals.

The AhG recommended to:

- Review all documents related to colour gamut and bit-depth scalability.

- Analyze SCE1 related contributions on CGS ILP tools performance and complexity.

JCTVC-Q0015 JCT-VC AHG report: Hybrid codec scalability (AHG15) [G. J. Sullivan (AHG chair)]

This document reports on the work of the JCT-VC ad hoc group on hybrid codec scalability, including a report of the teleconferences held and the progress made during the interim period since the preceding meeting. (The reader of this report is assumed to be familiar with various jargon and abbreviations used in JCT-VC; recent meeting reports and the draft specification text for HEVC and its SHVC and MV-HEVC extensions can be consulted for definitions of terms.)

The AHG held 5 teleconferences of 1–2 hours duration each, with substantial discussions of the issues. Approximately 10–20 people participated in each call – each of which was announced with at least 7 days of advance notice. The calls were held at different times of day to ensure reasonable opportunities for participation from different time zones.

The AHG reached agreement to propose an approach to hybrid codec scalability that is specified in a draft text document submitted with the AHG report.

The following needs for further work were identified:

- Editors' notes are included in the accompanying draft text (prefixed by the string "[Ed.") to identify open issues for further study.

- Further work is needed for reference software development to fully support the proposed scheme. A basic form of external means for providing a base layer is supported in the software; however, the software assumes that there is always a base layer picture for each access unit. Support for the planned signalling and syntax needs to be implemented.

The AHG noted that, due to the way the document repository web site works, two contributions to the interim work had been registered as inputs of the previous meeting, although they are really new inputs that arrived after that meeting ended.

- JCTVC-P0315 Comments on JCTVC-P0184v3 ("Support of AVC base layer in SHVC") [M. M. Hannuksela (Nokia)]

- JCTVC-P0316 Comments on Hybrid Scalability [S. Deshpande (Sharp)]

However, the above-listed documents were considered in the work of the AHG and do not appear to require further review at the Valencia meeting, as updated versions of those contributions have been provided as new input to the Valencia meeting.

The following contributions registered for the Valencia meeting were considered by the AHG.

- JCTVC-Q0041 AHG 15: Comments on Hybrid Scalability [S. Deshpande (Sharp)]
(an update of JCTVC-P0316)

- JCTVC-Q0042 AHG 15: Support of hybrid scalability [Y.-K. Wang, J. Chen, Y. Chen, Hendry, A. K. Ramasubramonian (Qualcomm)]
(an update of JCTVC-P0315)

The following additional new contribution is noted to be relevant to the work of the AHG:

- JCTVC-Q0188 Alternative AVC base layer HRD parameters for HEVC hybrid codec scalability [M. M. Hannuksela (Nokia)]

JCTVC-Q0016 JCT-VC AHG report: Test sequence material (AHG16) [T. Suzuki, R. Cohen, T. K. Tan, S. Wenger (AHG chairs)]

The AHG provided an updated list of available test sequences. Two relevant additional contributions (JCTVC-Q0087 and JCTVC-Q0088) were noted to have been submitted to the meeting.

See also the AHG 19 report JCTVC-Q0019 and contribution JCTVC-Q0204, which discusses additional test sequences used for verification testing.

JCTVC-Q0017 JCT-VC AHG report: Adaptive motion precision (AHG17) [J. Xu, X. Li (AHG chairs)]

This report summarizes the activities of the ad-hoc group of adaptive motion precision (AHG1) between the 16th JCT-VC meeting and the 17th JCT-VC meeting.

The AhG used the JCT-VC reflector for all discussions. A kick-off message was sent on January 29th, 2014. There were limited activity related to RCE1 over RCE1 participant mailing list between the 16th JCT-VC meeting and the 17th JCT-VC meeting.

Related contributions were listed. The AhG recommended to review the related documents.

JCTVC-Q0018 JCT-VC AHG report: high bit-rate and bit-depth operating points (AHG18) [K. Sharman, R. Joshi, H-Y. Kim (AHG chairs)]

This report summarizes the activities of the Ad Hoc Group on high bit-rate and bit-depth operating points between the 16th JCT-VC meeting held in San José, USA and the 17th JCT-VC meeting held in Valencia, Spain.

The AHG used the JCT-VC reflector for discussion, although there was very little activity for this AHG.

The test conditions now include lossless coding. The test conditions and benchmarks were included alongside the other RExt test conditions in JCTVC-P1006. JCTVC-Q0007 includes a summary of the changes due to adoptions at the previous meeting.

Related contributions were listed.

A viewing of some HDR sequences was planned to take place during the Valencia JCT-VC meeting.

The AHG recommended studying the contributions related to the AHG.

JCTVC-Q0019 JCT-VC AHG report: Verification test preparation and testing (AHG19) [T. K. Tan, V. Baroncini, M. Mrak, M. Karczewicz, W. Wan, J. Wen (AHG chairs)]

Several e-mail messages were exchanged among the members conducting the tests. The following tasks were accomplished within the mandate of the ad hoc group.

- The test sequences for Book and BT709Birthday that were originally encoded in 10 bits with HEVC Main 10 and AVC High 10, were re-encoded in 8bits with HEVC Main and AVC High as it was decided not to proceed with the 10 bits display.

- The University of West Scotland (UWS) was added as a test site replacing FUB and EPFL.

- Subjective tests were successfully conducted at BBC and UWS.

- Results were analyzed and input document JCTVC-Q0204 was drafted.

The AHG recommended:

- To review the results in JCTVC-Q0204 at the JCT-VC meeting in Valencia.

- To publish the results in a publicly available document.

- To request permission where necessary to make the original video sequences available publicly and allow for them to be used in publications.

# Project development, status, and guidance (24)

## Communication to and by parent bodies (0)

See section 8.1.

## Conformance test set development (1)

JCTVC-Q0219 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, W. Wan, G. Sullivan] [late]

This document is a proposed draft 7 of conformance test of HEVC.

- JCTVC-Q0249 Proposal of the RExt conformance test development plan [T. Suzuki, O. Nakagami (Sony)]

HEVC RExt is due for finalization. It is necessary to ensure the interoperability of the standard and conformance testing is one tool for accomplishing that. It is an urgent issue to confirm the spec by bitstream exchange, etc for product development. Therefore this contribution proposes to start RExt conformance activity at the Valencia meeting. This contribution provides an initial idea on conformance testing as a starting point of the discussion.

## Version 1 bug reports and cleanup (1)

The previously planned handling of MinCR for version 1 defect purposes, in particular, was supported by Japan NB input and was confirmed Wed p.m. (GJS).

JCTVC-Q0111 Errata report: Parsing issue for picture timing SEI message [Y. Wu, L. Zhu, G. J. Sullivan, F. Kyslov, S. Sadhwani (Microsoft)]

Discussed Wed. (MMH). Further discussed Thu. (YKW)

This contribution asserts that there can be a parsing problem for the picture timing (PT) SEI message under certain circumstances, and suggests corrective actions to provide additional information in the standard and to prohibit a particular pathological combination. One discussed case is when CpbDpbDelaysPresentFlag is equal to 1 (and thus PT SEI messages are present) but NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0 (and thus buffering period SEI message are not present). It is proposed to disallow this combination, as it is suggested that this combination does not seem particularly sensible or to represent any real application scenario and was not fully considered in the development of the standard. The second discussed case is when frame_field_info_present_flag is equal to 1 and CpbDpbDelaysPresentFlag is equal to 0, and it is proposed to add information to the standard to assist decoders in parsing PT SEI messages and to discourage encoders from generating bitstreams that could cause parsing problems.

Decision: Disallow CpbDpbDelaysPresentFlag being equal to 1 when NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0.

It was remarked that this modification may not require sequential ordering of APS SEI preceding BP SEI preceding PT SEI. Further study of the potential remaining parsing issues for BP and PT SEI was encouraged. It was also remarked that these SEI messages and possibly some others also may require delayed parsing.

Editorial action item: Editors are also requested to review and add clarification as necessary regarding concept of POC being increasing in output order within each CVS.

## HEVC coding performance, implementation demonstrations and design analysis (5)

### Version 1 verification test (1)

JCTVC-Q0204 HEVC verification test results [T. K. Tan (NTT Docomo), M. Mrak (BBC), V. Baroncini (FUB), N. Ramzan (UWS)]

This document reports on the results for the HEVC video verification test.

The purpose of the HEVC verification test is to confirm that the objective of achieving a substantially greater bit-rate reduction over the AVC High Profile has been met by the HEVC standard.

A subjective evaluation was conducted comparing the HEVC Main profile to the AVC High profile. Twenty video sequences with resolutions ranging from 480p to UHD were tested (at 4 different bit-rates / quality levels).

Analysis of the subjective test results reportedly show that in 86% of the test points, HEVC and AVC have the same quality, even though the bit-rates of AVC are greater than or equal to approximately double the HEVC bit-rates.

The bit-rate savings are similar for the different resolutions tested, with slightly more savings for sequences with higher resolutions. The average bit-rate savings for test sequences with UHD, 1080p, 720p and 480p resolutions are estimated at approximately 64%, 62%, 56% and 52%, respectively.

The results confirmed that the HEVC Main profile achieves the same subjective quality as AVC High profile while requiring on average approximately 59% less bit-rate. A summary of measured bit rate savings is illustrated below.

It was agreed to issue a final report of the verification test effort as an output document, based on this document.

### Still picture coding (1)

JCTVC-Q0229 HEVC still picture coding performance evaluation [D. Nicholson, C. Larabi, A. Descampe] [late]

Discussed Thu a.m. (GJS).

This document presents a coding performance evaluation of HEVC for coding still pictures. JPEG reference test image set, available in 4:2:0, in 4:4:4 as well as in RGB 4:4:4 were used for the evaluation. All Intra configuration was used and for 4:4:4 and RGB 4:4:4 content, HEVC Rext was used (HM13.0 + RExt 6.0), and results compared to JPEG 2000 (using OpenJPEG v2.1 release candidate).

The objective of this document was reported to be to present coding performance of HEVC for coding still pictures, done in cooperation with some JPEG experts with the objective of verifying the performance of HEVC for coding still pictures.

In the tables below, values indicate the bit rate of JPEG 2000 relative to HEVC, such that positive numbers indicate a benefit for HEVC and negative numbers indicate a benefit for JPEG 2000.

Results generally

RGB 4:4:4

YCC 4:4:4

YCC 4:2:0

The results reportedly validated the performance of HEVC for coding still pictures, both for 4:2:0 and 4:4:4 8bit images. The following actions were recommended by the contributors:

- To propose a 4:4:4 Still image profile based upon one of the 4:4:4 RExt profiles. (See notes elsewhere.)

- To set-up subjective tests to complete these verification tests (see below)

- To continue tests with more than 8bit material

- To continue tests with video material (including more than 8bits, using HEVC verification test material as well as RExt video sequences) instead of still pictures.

- To create HEVC still picture compliance (V1 and RExt) test set, potentially by using the JPEG Still picture test set, as for HEVC Still Picture profile, as included in HEVC V1, compliance material is not available.

The contributors indicated that subjective testing was planned and would be reported in the future.

The contributors volunteered to contribute to the development of the HEVC conformance test set with contribution of still pictures.

### SHVC performance and design aspects (2)

JCTVC-Q0046 Complexity analysis of an optimized SHVC decoder [W. Hamidouche, M. Raulet, O. Deforges (INSA)]

Discussed Tues p.m. (A. Duenas).

This contribution provides a complexity assessment of an optimized SHVC decoder. The SHVC decoder is based on a real time and parallel HEVC decoder developed under the open source "OpenHEVC" project. This contribution reported that the main time-consuming operations introduced in the SHVC standard include the up-sampling of the lower layer picture and the up-scaling of its motion vectors (MVs), which are optimized in single instruction multiple data (SIMD) operations.

The optimized SHVC decoder reportedly archives a real time decoding of 1080p50 enhancement layer on single core of an Intel i7 processor running at 2.6 GHz. The SHVC decoder decoding two layers reportedly introduces an average 50%–80% extra complexity relative to a single layer HEVC decoder.

The up-sampling and up-scaling operations, in the case of spatial scalability and inter coding configuration, reportedly represent around 16% of the total decoding time. The low level optimizations, together with a hybrid parallelism approach, reportedly enable the decoding of a 1600p enhancement layer at 88 frames per second on a 4-core i7 processor.

It was reported that the parallelism was achieved by a combination of picture-level parallelism and wavefront parallel processing. Results were also presented by using only frame level or wavefront parallel processing or both at the same time, as shown in the following table, wherein n is the number of threads allocated for the wavefront parallelism and m is the number of frames decoded in parallel for frame based parallelism.

| Configurations |  |  | Decoding configurations (n, m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (1, 1) | (4, 1) | (1, 4) | (2, 2) |
| Class B | Speedup | x2 | 1 | 2.99 | 2.02 | 3.24 |
|  |  | x1.5 | 1 | 3.13 | 2.21 | 3.36 |
|  |  | SNR | 1 | 2.90 | 2.50 | 3.34 |
|  |  | HEVC | 1 | 3.10 | 2.64 | 3.05 |
|  | Decoding frame rate (fps) | x2 | 52 | 156 | 107 | 169 |
|  |  | x1.5 | 44 | 138 | 99 | 149 |
|  |  | SNR | 41 | 124 | 108 | 138 |
|  |  | HEVC | 72 | 226 | 195 | 224 |
|  | Decoding time per frame (ms) | x2 | 20.54 | 6.88 | 24.04 | 16.08 |
|  |  | x1.5 | 24.11 | 7.76 | 27.82 | 19.27 |
|  |  | SNR | 29.47 | 10.28 | 33.36 | 24.81 |
|  |  | HEVC | 16.07 | 5.20 | 24.05 | 10.66 |
| Class A | Speedup | x2 | 1 | 3.6 | 2.07 | 3.81 |
|  |  | SNR | 1 | 3.49 | 2.47 | 3.67 |
|  |  | HEVC | 1 | 3.49 | 2.83 | 3.22 |
|  | Decoding frame rate (fps) | x2 | 23 | 84 | 47 | 88 |
|  |  | SNR | 17 | 62 | 43 | 65 |
|  |  | HEVC | 33 | 120 | 96 | 110 |
|  | Decoding time per frame (ms) | x2 | 47.85 | 13.65 | 55.00 | 32.65 |
|  |  | SNR | 63.92 | 18.76 | 73.67 | 50.95 |
|  |  | HEVC | 34.56 | 10.25 | 48.65 | 21.90 |

It was reported that it was difficult to achieve more than 3x speed up when using 4 cores.

It was reported that the usage of WPP allowed a simpler combination of picture level parallelism with other types of parallelism and that this was the reason this decoder implementation was selected. Of course, if the encoder does not use WPP encoding, the reported benefits of WPP would not be available.

It was reported that the decoder is capable of correctly decoding all of the HEVC version one conformance bitstreams.

A demonstration of this decoder decoding several scalable bitstreams was presented, with a demonstration of switching between displaying the base-layer decoded information or the combination of the multiple layers.

JCTVC-Q0050 4K real time streaming with SHVC decoder and GPAC player [W. Hamidouche, M. Raulet, J. Le Feuvre (INSA)]

Discussed Tues p.m. (A. Duenas).

This contribution reported a 4K end-to-end video streaming demonstration using an optimized SHVC decoder under the "OpenHEVC/FFmpeg" library and the "GPAC" player. The SHVC decoder was integrated in the OpenHEVC/FFmpeg library, enabling this open source library to support the decoding of a conforming SHVC bitstream. The GPAC player reportedly implements tools to encapsulate conforming SHVC bitstreams into the mp4 file format.

At the server side, the GPAC player reportedly broadcasts the mp4 content in MPEG2-TS. At the receiver side, the GPAC player feeds the openHEVC/FFmpeg SHVC decoder with the received video packets. The FFmpeg library with the SHVC decoder decodes the requested SHVC video layers and outputs YUV pictures corresponding to the highest decoded layer for rendering by the GPAC layer. This demonstration also included, at the receiver side, an interactive interface to switch between the decoded SHVC layers: HD and 4K for x2 spatial scalability.

A demonstration of this streaming system switch in real time between the base layer for HD resolution and the enhancement layer for 4K resolution was performed, and additional information about how to replicate this system was presented in the contribution.

### Interlace (1)

JCTVC-Q0118 Interlace coding in HEVC v.1 [A. Tourapis, D. Singer (Apple)]

This contribution was provided as information for further study. It was not requested for detailed presentation.

## Profile and level definitions (requirements related) (7)

### General (1)

JCTVC-Q0089 Profiles [C. Fogg, J. Helman (Movielabs)]

A table is requested in HEVC draft Annex A that summarizes the bitstream element restrictions for each profile. It is suggested that this would clarify the definition of profiles added since the publication of HEVC version 1 that lack a unique code point.

Editor action item: Delegated to the editors for consideration.

Discussion comments were recorded as follows:

- high_precision_offsets_enabled_flag can be 0 or 1 in Main 12

- separate_colour_plane_flag does not exist for Main and Main 10 and Main 12

- general_max_422chroma_constraint_flag must be 1 for Main 12

### RExt profiles and levels (5)

JCTVC-Q0051 AHG5: Recommended profiling of range extension coding tools [S. Lee, C. Park, E. Alshina, C. Kim (Samsung), K. McCann (Zetacast)]

First discussed at joint parent-level meeting.

Discussed Wed 2 April p.m. (GJS).

This contribution provided recommendations on the process that should be used in the profiling of range extension coding tools. It was suggested that the profiling for it should be based on an assessment of the trade-off between coding gain and complexity of the additional range extension coding tools, evaluated under the common test conditions of HEVC range extensions.

The contribution asserted that the results indicate that cross component decorrelation was the only additional coding tool in the draft RExt text that provides a beneficial trade-off between performance and complexity with what the contribution calls generic video content at less than 16 bits. It proposed that the Main 4:4:4 10/12 profiles should include cross component decorrelation, as well as CU-adaptive chroma QP offset for encoder-side rate control in 4:4:4, but not the other coding tools present in the draft text, which were asserted to provide a beneficial performance/complexity trade-off only with screen content. In addition, it suggested to include Golomb-Rice parameter adaptation and CABAC bit alignment in the 4:4:4 16 b all-intra profile, but not the Main 4:4:4 10/12 profiles.

The contribution also suggested that screen content coding (SCC) 4:4:4 Profiles should be developed based on the evaluation results of the responses to the Call for Proposals for SCC. It suggested that all of the coding tools in the draft RExt text should be included in the initial test model for that activity.

The presentation deck was not available, and differed from the prior contribution content. It was requested to be uploaded.

In the presentation, the potential interactions between different features was discussed. It is possible that adding or removing individual features would change the measured performance of the other features (and the performance measurements would depende on the particular selected test set, of course, and some features may have benefits not best characterized by PSNR measures – e.g., chroma QP adjustment flexibility is not motivated by PSNR maximization).

From the recommendation of proponent as presented, the only remaining difference suggested, relative to the draft text produced from the preceding meeting, was a recommendation to remove CU adaptive chroma QP offset in 4:2:2 profiles. This is a feature that is difficult to evaluate in experiments, due to its perceptual optimization motivation. Ease of transcode to 4:2:0 was mentioned by the contributor as a motivation for desiring not to have a feature supported in 4:2:2 profiles that is not supported in 4:2:0 profiles.

Some participants indicated a strong interest in having the QP adjustment functionality in 4:2:2 usage.

(Not evaluated, but recommended to be included in 4:4:4 profiles by the contributor, was large-block TS. Another difference in some profiles was the weighted prediction modification, which was not discussed as a distinct feature in the contribution.)

JCTVC-Q0113 Request for an HEVC 4:4:4 8 bit profile [G. Martin-Cocher (BlackBerry), P. Onno, C. Rosewarne (Canon), A. Fuldseth (Cisco), R. Sjöberg (Ericsson), A. Duenas (NGCodec), J. Sole, M. Karczewicz (Qualcomm), M. Budagavi (TI)]

This document proposes to define a 4:4:4 8 bit profile as part of Amd1 of HEVC, which was proposed to include coding tools designed for 4:4:4 colour sampling as well as tools that are useful for coding "mixed content".

It was asserted that the following consumer applications would make use of this profile:

- wireless display

- video conferencing with screen sharing

- compression and transmission of computer generated sequences

- mixed content scenarios

It was asserted that computer generated content predominantly uses the 4:4:4 8 bit format, and that mixed content will also use a 4:4:4 8 bit format. It was asserted that a corresponding HEVC profile needs to be defined, and not postponed to a later amendment.

It was further asserted that some tools, previously identified as screen content tools, are in fact performing pretty well on natural sequences (e.g. block copying) and/or on mixed content (e.g. transform skip). It was asserted that those tools have been so far mischaracterized as screen-content-only tools.

It was proposed that the coding tools currently defined in the HEVC Rext study text be included in the proposed 4:4:4 8 bit profile.

JCTVC-Q0133 Comments on HEVC 4:4:4 8 bit profiles [A. Tourapis, D. Singer (Apple)]

This contribution suggests certain possibilities for the creation of 4:4:4 8 bit profiles as an outcome of the Screen Content CfP and expresses an opinion on which avenue is preferred to be followed depending on this outcome.

The contributors recommended that the outcome of the Screen Content CfP be studied carefully, and that the committee defines now either:

- the only expected 4:4:4 8-bit profile that is planned to be specified, or

- the initial member of a pair of expected profiles which would have a sensible, nested, relationship.

JCTVC-Q0186 AHG5: Super-high Tier Specification Targeted at the Intra 16-bit 4:4:4 Profile [K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony)] [late]

At the 16th JCT-VC meeting in San José, the use of a new super tier was adopted, which could be used in combination with the Intra 16-bit 4:4:4 profile. Following directives from the parent bodies where the previously adopted super-tier was dropped from the currently defined profiles, this contribution proposes that a “High Intra 16-bit 4:4:4” profile is adopted, enabling the compression ratio defined by MinCR to be approached for all levels as a sequence average when using intra-only coding.

JCTVC-Q0212 AHG5: Objective and subjective evaluations of cross-component decorrelation in RExt6.0 for range extensions profile [K. Kawamura, S. Naito (KDDI)] [late]

This contribution reports objective and subjective performance of the cross-component decorrelation tool in RExt6.0 for the general content (camera captured content). This contribution provides an update on the results from the previous report in JCTVC-P0099. The cross-component decorrelation feature was asserted to provide a substantial benefit, and was proposed for inclusion in appropriate RExt profiles.

- JCTVC-Q0251 On HEVC 4:4:4 profiles with intra block copying [J. Sole] [late]

This contribution provides a summary of the complexity and performance of intra block copying and proposes to include this tool in the 4:4:4 profiles for HEVC Amd1.

### SHVC profiles and levels (3)

JCTVC-Q0145 MV-HEVC/SHVC HLS: On level definitions [Y.-K. Wang, K. Rapaka, J. Chen, Hendry, A. K. Ramasubramonian (Qualcomm)]

Discussed Wed 2 April (GJS).

This document reports and discusses asserted issues associated with the current level definitions in SHVC, and proposes a different approach to define levels and decoder capabilities as well as other changes to solve the above issues. It is asserted that the proposed approach for level definitions also applies to MV-HEVC and that the level limits are specified in way that is scalable to the number of layers in bitstreams.

As a part of the proposed level definitions, a decoder capability is specified in a way where the level is associated with a scale, which is needed together with the level to express the decoder capability. Two categories of conforming decoders are specified. Category I decoders can decode some bitstreams conforming to a higher level of the decoder but with the number of layers being smaller than the scale associated with the level of the decoder. Category II decoders can only decode bitstreams conforming to the same or a lower level. Example Category I decoders are those that are implemented using one existing single-layer HEVC decoder by reusing the hardware core as is without changes or implemented without reusing the hardware cores of existing single-layer HEVC decoders. Example Category II decoders are those that are implemented using more than one existing single-layer HEVC decoder by reusing the hardware cores as they are without change.

The current level definition of SHVC has the following issues (note that the attached Excel sheets contain an analysis that would help understand some of the following issues):

- If more than 4 SNR layers of 720p resolution (or equivalent number of luma pixels e.g. more layers with combined spatial and SNR layers) are needed, then Level 5 or above would have to be used. Consequently, the value of CtbSizeY shall be equal to 32 or 64 (per item e of the general limits). For resolution like 720p or lower, this restriction disallows the use of smaller coding tree block sizes such as 16x16 and can therefore result in sub-optimal coding efficiency.

- If an SHVC decoder is implemented in a way that consists of 4 cores of HEVC Level 3.1 decoders, and is able to decode 4 SNR layers of 720p, per the current level definition, it would have to be said to be conforming to Level 4 or above. Consequently, the decoder needs to be able to decode any Level 4 bitstreams. However, with only high-level syntax changes (i.e. no changes to the hardware cores), such a decoder would not be able to decode an SHVC Level 4 bitstream with 2 SNR layers of 1080p resolution.

- If an SHVC decoder is not implemented by reusing multiple existing HEVC decoder cores but is implemented in a way that can decode both a single-layer HEVC bitstream of 1080p and a two-layer SHVC bitstream of 720p, according to the current level definition it would labeled as a Level 3.1. However, then the expression of the other capability is missing.

- If an SHVC decoder is implemented in a way that consists of 4 cores of HEVC 3.1 decoders to be able to decode 4 SNR layers of 720p, per the current level definition, it would be said to be conforming to Level 4 or above. Consequently, each enhancement-layer picture can have more than 3 tile rows and more than 3 tile columns, e.g. 5 tile rows and 5 tile columns, each tile with width of 256 luma samples and height of 144 luma samples. However, this would go beyond the Level 3.1 limits and consequently the decoder cores for decoding the enhancement layers would have problems.

  - All items in subclause A.4.1 of HEVC text are currently specified to be applied to each layer. However, some of these items are not directly applicable to each layer.

  - For item d on DPB size, the SPS syntax element is not applicable for enhancement layers. Also, the DPB in the latest SHVC text is a shared-sub-DPB design, thus item d cannot be directly applied to each layer.

- For items h and i on CPB size, for bitstream-specific CPB operations, the parameter cannot be applied to each layer.

- Bitstream-specific restrictions on CPB size (by items h and i in subclause A.4.1 of HEVC text) are needed. However, the items h and i in subclause A.4.1 of HEVC text cannot be directly applied on bitstream level, because if directly applied, the same CPB size limit for single-layer bitstreams would also be the limit for multi-layer bitstreams. This is not scalable to the number of layers and would only allow for low picture quality when there are many layers.

- The restrictions by items b, c, d, g, h, i, and j in subclause A.4.2 of HEVC text are currently specified to be layer-specific only. However, bitstream-specific restriction by these items should be specified, regardless of whether their layer-specific counterparts are specified.

This contribution proposes a different approach to define levels and decoder capabilities as well as other changes to solve the above issues. Though in the above the problems are described based on the current SHVC level definitions, some of the problems also apply to MV-HEVC, and the proposed approach and most other changes also apply to MV-HEVC.

The proposal is summarized as follows:

- A decoder capability is specified in a way where the level is associated with a scale, which is needed together with the level to express the decoder capability.

- Two categories of conforming decoders are specified. Category I decoders can decode some bitstreams conforming to a higher level but with the number of layers being smaller than the scale associated with the level of the decoder, while Category II decoders can only decode bitstreams conforming to the same or a lower level (provided that the profile and tier of the bitstream are the same as that of the decoder, respectively). Example Category I decoders are those that are implemented using one existing single-layer HEVC decoder by reusing the hardware core as is without changes or implemented without reusing the hardware cores of existing single-layer HEVC decoders. Example Category II decoders are those that are implemented using more than one existing single-layer HEVC decoder by reusing the hardware cores as they are without change.

- Two DPB size-related restrictions are proposed, one for each layer, and one for each sub-DPB, specified using the DPB related parameters in the VPS.

- Bitstream-specific restrictions on CPB size are specified in a way that is scalable to the number of layers and allow for high picture quality when there are many layers.

- Bitstream-specific restrictions corresponding to items b, c, d, g, h, i, and j in subclause A.4.2 of HEVC text are specified in a way that is that is scalable to the number of layers.

The text changes in relative to the latest SHVC and MV-HEVC draft texts are provided in the attachments. The attached slides show some example scenarios for illustrating of some of the above constraints.

In the discussion, a suggestion was to define a per-layer profile/level specification rather than a specification that bundles the requirements of multiple layers. This would sort of make the "external means" base layer approach become the normal approach, where the requirements of each layer are separate. In a sense, this is already constructable – but not as a single bitstream package.

Further AHG study was agreed to be needed on bitstream constraint specifications.

JCTVC-Q0103 MV-HEVC/SHVC HLS: On DPB Profile Level Limits [S. Deshpande (Sharp)]

This document proposes per-layer and shared sub-DPB capacity level limits for SHVC and MV-HEVC profiles. Maximum DPB size level limit constraints for per-layer sub-DPB parameters max_vps_layer_dec_pic_buff_minus1[ ][ ][ ] and for (possibly) shared sub-DPB parameters max_vps_dec_pic_buffering_minus1[ ][ ][ ] for SHVC scalable main and scalable main 10 and MV-HEVC stereo main profiles are proposed.

Further AHG study was agreed to be needed on bitstream constraint specifications.

JCTVC-Q0206 Proposal to support 12 bit video in SHVC [T. Suzuki (Sony)] [late]

This document proposed to investigate 4:2:0 12 bit support in SHVC. Currently bit depth scalability in SHVC is limited up to 10 bit. However, in UHDTV, for example ITU-R BT.2020, 4K video is defined for both 10 bit and 12 bit. In addition, 4:2:0 12 bit profile is defined in RExt and the profile is used for consumer application in near future. Therefore, in SHVC bit depth scalability profile in SHVC, 12 bit video should be allowed for both BL and EL. This contribution proposes to allow to use 4:2:0 12 bit video in bit depth scalability profiles in SHVC.

## RExt editing status

Starting with P1005-v2, the RExt editorial status was reviewed Tues 1000-1330 (GJS)

- Profile & level

  - Question: Should we ever require cabac_bypass_alignment_enabled_flag to be 1? If yes, should we require decoders to decode bitstreams in which it is 0 (e.g. in lower levels)?

Decision: Main 4:4:4 16 Intra and all other profiles in this meeting's output draft will require the feature to be disabled. The planned new profile to be finalized in July as part of "SHVC" will have an additional profile with higher bit rate capabilities that will require that feature to be enabled and will not require decoders to decode bitstreams in which the feature is disabled.

  - Question: Table A-3: Scale by 1.25 and 1.5 instead of 1.5 and 2 for bit rate and CPB size adjustments for 4:2:2 and 4:4:4? No action needed – 1.5 and 2 confirmed.

  - Question: Change mcts_tier_flag[ i ] and max_mcts_tier_flag as u(1) to mcts_tier_value[ i ] and max_mcts_tier_value as ue(v) for future proofing (although technically OK for currently specified profiles). Decision: Agreed.

  - Question: constraint on sps_extension_7bits seems to be wrong.
Decision (Ed.): Constraint in Annex A is mistaken and should be removed.

  - Question: constraint forbidding presence of extension of SPS for Main is unnecessary and should be removed (and may inadvertently cause non-conforming bitstreams to exist). Decision (Ed.): Remove that constraint.

  - Question: PPS extension drafted incorrectly regarding using extension flag 0 when it isn't present. Should either have inferred value 0 or should be nested inside if statement that establishes its presence. Decision (Ed.): Agreed (the same editorial alternatives exist for the SPS and PPS extension and it would be editorially preferable to use the same method of describing it in both cases).

  - Question: SHVC and MV-HEVC drafts conflict with RExt and do not allow extension bits other than extension bit 7 to be used. Decision (Ed.): Fix that.

  - SEI message specification maturity was evaluated, and agreed to be categorized as follows:

    - Seem mature – proceed in RExt:

      - Colour volume

      - No display

      - Timecode

    - Plan to move from RExt draft to SHVC draft:

      - MCTS

      - Knee function

      - Chroma resampling filter

  - Colour VUI – see notes on Q0084.

  - No special need for review was identified for the following topics:

    - Need to update payloadType list.

    - Main 4:4:4 16 Intra should have no other "tool constraints"

    - MinCR handling – seems OK.

    - Editorially, bracket the constraint flags to clarify and simplify semantics.

    - Should we allow additional flag combinations to be used? (No: it is more future-proof to disallow them for now.)

    - For hypothetical auxiliary pictures (not in RExt), there is no 8b monochrome profile. This aspect was decided to be changed later, as recorded in sections 7.5.1 and 8.1.

- In additional further discussion near the end of the meeting (GJS), it was noted that CABAC initialization and CBF had been identified by the editors as areas of potential deficiencies in the text. Decision (Ed.): Generally speaking, discrepancies in these aspects should be resolved by studying the software in case of doubt.

## HEVC, SHVC and RExt use cases (requirements related) (4)

To be dealt with at parent level – outside the scope of current JCT-VC work. Not reviewed.

JCTVC-Q0085 HDR/WCG workflow [B. Mandel (Universal)]

JCTVC-Q0190 Evaluation of distortion metrics on HDR video content [E. François, P. Lopez, F. Le Léannec, S. Lasserre (Technicolor)] [late]

JCTVC-Q0191 New HDR video coding results [E. François, S. Lasserre, F. Le Léannec (Technicolor)] [late]

JCTVC-Q0192 Insights and open questions on HDR/WCG video coding [D. Singer, A. Tourapis] [late]

## Source video test material (4)

The test sequences described below may be valuable for testing RExt coding performance. Detailed review of these contributions was not requested. Demonstration viewing was conducted for some of these as side activity during the meeting.

JCTVC-Q0087 Technicolor clip results [B. Mandel (Universal), C. Fogg, J. Helman (Movielabs)] [late]

JCTVC-Q0088 StEM and Telescope HDR/WCG test sequences [J. Helman, C. Fogg (Movielabs)]

JCTVC-Q0228 DCI StEM Content Description [J. Helman (Movielabs), W. Husak (Dolby)] [late]

- JCTVC-Q0232 HDR/WCG StEM and Telescope clips encoded as Y'DzDx and BT.2020 9 [B. Mandel (Universal), C. Fogg, J. Helman (MovieLabs)] [late]

# Core experiment in SHVC (10)

## SCE1: Colour gamut and bit depth scalability (10)

### SCE1 summary and general discussion (1)

JCTVC-Q0021 SCE1: Summary Report of Colour Gamut and Bit Depth Scalability [P. Andrivon, A. Duenas, E. Alshina, Y. Ye, K. Ugur, X. Li]

Discussed 2nd day p.m. (GJS).

This document summarizes the activities and test results performed in SCE1 on Colour Gamut and Bit Depth Scalability. The evaluated tools and the test conditions are defined in document JCTVC-P1101.

Basically, there were two groups of proposals.

- 1.1, 1.2, 1.3, 1.4 = various gain/offset models

- 2.1, 2.2 = 3D LUT mapping 8x2x2 – 8 uniform segments for luma, and 2 uniform segments for each colour channel, (a, b, c, d)  Y' = aY * Y + bY * U + cY * V + dY, U' = aU * Y + bU * U + cU * V + dU, V' = aV * Y + bV * U + cV * V + dV.

1.1 and 1.3 have thousands of table entries, which is generally agreed to be excessive.

1.4 requires a lot of computations and does not have great performance.

1.2 does not have a significant performance benefit relative to the anchor.

2.1 and 2.2 have 384 bytes of table data each, and not too much computational load, and operate in the lower-resolution domain. For each, a table would be sent in PPS and optionally updated in SH. It was asserted that the table content would be unlikely to be totally stable within a CVS of significant length. There was discussion of whether slice-level update is necessary or not – likely not seemed to be the tentative conclusion.

It was suggested that the 2.x proposals each contain a combination of techniques, and that some of the elements of these (esp. entropy coding part) may not be necessary/justified.

The difference between 2.1 and 2.2 is the use of spatial phase positioning alignment for chroma in 2.1, based on the assumption of the default positioning of the chroma samples. The gain for this is in the 0.31–1.0% range. The overall gain is 6–10% (as a percentage of the total bit rate).

The overall conclusion was that a 2.x approach or something close to it would not be overly burdensome to include and has adequate gain to justify its complexity, and should thus be adopted.

But it is for a particular application space, so should it be in a "generic" scalable profile for all profiles with a certain bit depth capability, or should it have its own profile?

Between 2.1 and 2.2, the gain difference is about 0.6%, with some extra complexity associated with the alignment adjustment due to look-ahead aspect of the position adjustment processing. There were mixed opinions about that.

A non-CE proposal Q0129 was trying to find a new trade-off.

Five sequences; two colour grades each, different resolutions, two bit depths for BT.709.

Gain was roughly 8% relative to WP for the highest-performing methods, 3% for the lowest-performing (1.2).

Further review was held in the evening of Wed. 2 April (GJS). Regarding the selection between the 2.1 and 2.2 variants, there was discussion of whether method 2.2 was simpler, and it was estimated that method 2.1 has about 0.6% gain over method 2.2.

Decision: Adopt method 2.1, with se(v) coding.

### SCE1 primary contributions (2)

JCTVC-Q0048 SCE1: Colour gamut scalability with asymmetric 3D LUT [X. Li, J. Chen, M. Karczewicz (Qualcomm), Y. He, Y. Ye, J. Dong (InterDigital), P. Bordes, P. Andrivon, E. Francois, F. Hiron (Technicolor)]

JCTVC-Q0072 SCE1: Colour gamut scalability using gain-offset models [A. Aminlou, K. Ugur, M. M. Hannuksela (Nokia)]

### SCE1 cross checks (7)

JCTVC-Q0059 SCE1: Cross-check of JCTVC-Q0048 [A. Aminlou, K. Ugur (Nokia)]

JCTVC-Q0097 SCE1: Crosscheck report on colour gamut scalability using gain-offset models (JCTVC-0072) [X. Li (Qualcomm)] [late]

JCTVC-Q0123 SCE1: Crosscheck for Colour gamut scalability using gain-offset models (JCTVC-0072) Test 2 [K. Minoo (Arris)] [late]

JCTVC-Q0143 Crosscheck report of SCE1 test on colour gamut scalability using 8x8x8 regions and matrix mapping (JCTVC-Q0072) [K. Misra (Sharp)] [late]

JCTVC-Q0144 Crosscheck report of SCE1 test on asymmetric 3D-LUT with phase alignment filter disabled (JCTVC-Q0048) [K. Misra (Sharp)] [late]

JCTVC-Q0196 SCE1: Crosscheck result of Test 1.3 [K. Sato (Sony)] [late]

JCTVC-Q0222 SCE1: Cross-check of 3D-LUT parameter coding of JCTVC-Q0048 [K. Ugur, A. Aminlou (Nokia)] [late]

# Core experiment in Range Extensions (5)

## RCE1: Adaptive motion vector precision (5)

### RCE1 summary and general discussion (1)

JCTVC-Q0022 RCE1: Summary Report of adaptive motion precision [X. Li, J. Xu, Y. Zhou]

Discussed 2nd day (Fri) (JRO).

The tests were conducted according to the RCE1 description JCTVC-P1121. The results are summarized as follows.

RA-MT

|  | JCTVC-Q0155 PPS/slice flag with full search |  |  | JCTVC-Q0155 PPS/slice flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -1.0% | -1.1% | -0.9% | -0.7% | -0.8% | -1.1% | -1.2% | -1.2% | -0.6% | -0.7% | -0.8% |
| Class B | -0.3% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% |
| RGB 4:4:4 SC | -0.7% | -0.7% | -0.8% | 0.0% | 0.0% | 0.0% | -0.9% | -0.9% | -0.9% | -0.6% | -0.6% | -0.6% |
| RGB 4:4:4 Animation | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% | -1.0% | -1.3% | -1.4% | -0.6% | -0.8% | -0.9% |
| YCbCr 4:4:4 SC | -0.7% | -0.6% | -0.6% | 0.1% | 0.1% | 0.1% | -0.6% | -0.5% | -0.5% | -0.4% | -0.3% | -0.3% |
| YCbCr 4:4:4 Animation | -0.5% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | -0.9% | -1.4% | -1.5% | -0.3% | -0.7% | -0.7% |
| RangeExt | -0.3% | -0.2% | -0.3% | 0.0% | 0.0% | -0.1% | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |
| RGB 4:4:4 SC (Optional) | -2.4% | -2.4% | -2.4% | -2.0% | -2.0% | -2.0% | -1.8% | -1.7% | -1.8% | -0.5% | -0.5% | -0.5% |
| YCbCr 4:4:4 SC (Optional) | -2.6% | -2.3% | -2.2% | -2.0% | -1.7% | -1.6% | -1.7% | -1.5% | -1.5% | -0.4% | -0.4% | -0.3% |
| Enc Time[%] | 197% |  |  | 97% |  |  | 142% |  |  | 147% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  | 101% |  |  |

RA-HT

|  | JCTVC-Q0155 PPS/slice flag with full search |  |  | JCTVC-Q0155 PPS/slice flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.0% | -0.9% | -0.9% | -0.8% | -0.7% | -0.7% | -1.2% | -1.3% | -1.3% | -0.7% | -0.8% | -0.9% |
| Class B | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.4% | -0.3% | -0.2% | -0.2% | -0.1% | -0.1% |
| RGB 4:4:4 SC | -0.8% | -0.8% | -0.8% | 0.0% | 0.0% | 0.0% | -1.2% | -1.2% | -1.2% | -0.8% | -0.7% | -0.8% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -1.3% | -1.6% | -1.6% | -0.9% | -1.1% | -1.1% |
| YCbCr 4:4:4 SC | -0.6% | -0.6% | -0.6% | 0.3% | 0.3% | 0.3% | -0.7% | -0.7% | -0.7% | -0.5% | -0.5% | -0.5% |
| YCbCr 4:4:4 Animation | -0.1% | -0.3% | -0.2% | 0.0% | 0.0% | 0.0% | -0.9% | -1.4% | -1.4% | -0.4% | -0.9% | -0.8% |
| RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.3% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -2.3% | -2.2% | -2.2% | -2.0% | -1.9% | -1.9% | -1.8% | -1.7% | -1.7% | -0.7% | -0.6% | -0.6% |
| YCbCr 4:4:4 SC (Optional) | -2.8% | -2.6% | -2.5% | -2.2% | -2.0% | -1.9% | -1.9% | -1.7% | -1.7% | -0.6% | -0.6% | -0.5% |
| Enc Time[%] | 198% |  |  | 98% |  |  | 142% |  |  | 147% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  | 96% |  |  | 101% |  |  |

LB-MT

|  | JCTVC-Q0155 PPS/slice flag with full search |  |  | JCTVC-Q0155 PPS/slice flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.5% | -1.3% | -1.5% | -0.9% | -0.6% | -0.6% | -1.1% | -0.5% | -1.0% | -0.9% | -0.4% | -0.7% |
| Class B | -0.4% | 0.1% | 0.2% | 0.2% | 0.2% | 0.4% | -0.1% | 0.1% | 0.4% | -0.2% | -0.6% | -0.4% |
| RGB 4:4:4 SC | -1.0% | -1.1% | -1.1% | 0.0% | 0.0% | 0.0% | -1.1% | -1.1% | -1.2% | -0.7% | -0.8% | -0.8% |
| RGB 4:4:4 Animation | -0.3% | -0.4% | -0.5% | 0.0% | 0.0% | 0.0% | -2.0% | -2.1% | -2.0% | -1.3% | -1.4% | -1.4% |
| YCbCr 4:4:4 SC | -1.2% | -1.2% | -1.3% | -0.1% | -0.1% | -0.1% | -0.7% | -0.8% | -1.0% | -0.4% | -0.5% | -0.5% |
| YCbCr 4:4:4 Animation | -0.2% | -0.5% | -0.3% | 0.0% | 0.0% | 0.0% | -0.5% | -1.4% | -1.0% | -0.4% | -1.0% | -0.9% |
| RangeExt | -0.2% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -5.0% | -5.0% | -4.9% | -3.3% | -3.3% | -3.3% | -3.0% | -3.0% | -3.0% | -1.8% | -1.8% | -1.7% |
| YCbCr 4:4:4 SC (Optional) | -5.5% | -5.3% | -5.2% | -2.0% | -1.9% | -1.8% | -3.0% | -2.9% | -2.8% | -1.2% | -1.1% | -0.9% |
| Enc Time[%] | 197% |  |  | 97% |  |  | 142% |  |  | 148% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 93% |  |  | 100% |  |  |

LB-HT

|  | JCTVC-Q0155 PPS/slice flag with full search |  |  | JCTVC-Q0155 PPS/slice flag with fast search |  |  | JCTVC-Q0049 CU flag |  |  | JCTVC-Q0049 Enc Only |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.3% | -1.1% | -1.2% | -0.7% | -0.5% | -0.5% | -1.1% | -0.9% | -1.1% | -0.9% | -0.7% | -0.8% |
| Class B | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | -0.3% | -0.1% | -0.1% | -0.3% | -0.4% | -0.3% |
| RGB 4:4:4 SC | -1.2% | -1.2% | -1.2% | 0.0% | 0.0% | 0.0% | -1.5% | -1.5% | -1.5% | -1.0% | -1.1% | -1.1% |
| RGB 4:4:4 Animation | -0.3% | -0.3% | -0.4% | 0.0% | 0.0% | 0.0% | -2.6% | -2.6% | -2.4% | -1.5% | -1.5% | -1.5% |
| YCbCr 4:4:4 SC | -1.3% | -1.2% | -1.3% | -0.1% | -0.1% | -0.1% | -1.1% | -1.1% | -1.2% | -0.7% | -0.7% | -0.8% |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.8% | -1.8% | -1.4% | -0.5% | -1.1% | -1.0% |
| RangeExt | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.3% | -0.2% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -4.1% | -4.1% | -4.0% | -2.9% | -2.9% | -2.9% | -1.7% | -1.7% | -1.7% | -1.7% | -1.7% | -1.7% |
| YCbCr 4:4:4 SC (Optional) | -5.1% | -5.1% | -4.9% | -2.1% | -2.2% | -2.0% | -2.7% | -2.7% | -2.6% | -1.0% | -0.9% | -0.8% |
| Enc Time[%] | 198% |  |  | 97% |  |  | 143% |  |  | 149% |  |  |
| Dec Time[%] | 95% |  |  | 94% |  |  | 94% |  |  | 100% |  |  |

RA-lossless

|  | JCTVC-Q0155 PPS/slice flag with full search |  | JCTVC-Q0155 PPS/slice flag with fast search |  | JCTVC-Q0049 CU flag |  | JCTVC-Q0049 Enc Only |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Aver | Total | Aver | Total | Aver | Total | Aver |
| Class F | -0.1% | -0.2% | -0.1% | 0.0% | -0.5% | -0.5% | -0.3% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.5% | -1.1% | 0.0% | 0.0% | -1.3% | -0.9% | -1.2% | -0.9% |
| RGB 4:4:4 Animation | -0.2% | -0.2% | 0.0% | 0.0% | -0.8% | -0.9% | -0.7% | -0.8% |
| YCbCr 4:4:4 SC | -0.8% | -0.7% | 0.0% | 0.0% | -0.9% | -0.9% | -0.7% | -0.7% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.5% | -0.4% | -0.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.8% | -1.4% | -1.7% | -0.7% | -1.4% | -0.5% | -0.5% | -0.2% |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.7% | -1.6% | -0.6% | -1.5% | -1.4% | -0.6% | -1.0% |
| Enc Time[%] | 195% |  | 98% |  | 137% |  | 142% |  |
| Dec Time[%] | 95% |  | 95% |  | 92% |  | 98% |  |

LB-lossless

|  | JCTVC-Q0155 PPS/slice flag with full search |  | JCTVC-Q0155 PPS/slice flag with fast search |  | JCTVC-Q0049 CU flag |  | JCTVC-Q0049 Enc Only |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | Aver | Total | Aver | Total | Aver | Total | Aver |
| Class F | -0.1% | -0.1% | 0.0% | 0.0% | -0.4% | -0.4% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.4% | -1.6% | -0.7% | -0.2% | -1.6% | -1.7% | -1.3% | -1.4% |
| RGB 4:4:4 Animation | -0.2% | -0.3% | 0.0% | 0.0% | -0.8% | -0.9% | -0.6% | -0.7% |
| YCbCr 4:4:4 SC | -0.8% | -0.9% | 0.0% | 0.0% | -0.7% | -0.7% | -0.6% | -0.6% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.5% | -0.4% | -0.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -1.9% | -0.3% | -1.8% | 0.2% | -1.5% | 0.4% | -0.5% | 1.0% |
| YCbCr 4:4:4 SC (Optional) | -2.0% | -3.3% | -1.8% | -0.6% | -1.5% | -2.6% | -0.6% | -1.8% |
| Enc Time[%] | 193% |  | 95% |  | 138% |  | 141% |  |
| Dec Time[%] | 93% |  | 94% |  | 91% |  | 95% |  |

It was pointed out that BD calculation for optional sequences may not be reliable due to non-convex behaviour of the anchor RD curves in some sequences of the set.

It is also mentioned that Q0067 and Q0092 are non-CE proposals which show more benefit

Generally, the integer pel restriction is mostly beneficial for screen content, and therefore any changes below slice level should be considered in the screen content activity.

Also the slice-level proposal requires a change in TMVP and AMVP scaling (integer rounding necessary)

Some gain is claimed for class B (mostly Kimono), but from the results above this can already be achieved by non-normative means.

Some other proposals with non-normative changes (Q0077, Q0147) indicate that even higher gain may be possible in a non-normative way by improving motion search.

The question was raised whether any study exists about the usefulness of sub-pel motion comp in high resolution (e.g. 8K) content. This is not the case.

No action was taken for RExt – further investigation was encouraged in the context of screen content coding.

### RCE1 primary contributions (2)

JCTVC-Q0049 RCE1: Adaptive MV Precision [X. Li, J. Sole, M. Karczewicz (Qualcomm)]

Discussed 2nd day (Fri) (JRO).

It is proposed to signal a flag at CU level to indicate whether all PUs in the CU have integer-precision MVs. When the flag is on, MV predictors of non-skip and non-merge PUs are rounded to integer precision and MV differences are signaled in integer precision. For PUs in merge or skip mode, MVs are rounded only before motion compensation so that fractional MVs are saved and can be used as MV predictors for neighboring PUs. When the flag is off, sub-pixel motion compensation is allowed.

JCTVC-Q0155 RCE1: Subtest 1 - Motion Vector Resolution Control for Screen Content Coding [Y. Zhou, J. Xu, G. J. Sullivan, B. Lin (Microsoft)

Discussed 2nd day (Fri) (JRO).

It is proposed to add a high-level indicator (enabling flag at the PPS, and switching at slice header level) to indicate the resolution for interpretation of the motion vectors.

Before encoding a slice, encoder may decide the motion vector resolution based on historical statistical data, knowledge of its type of application, multi-pass analysis, or some other such technique. If the encoder decides to use full pixel motion only, the fractional pixel search is skipped. If a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value.

At decoder side, if the motion vectors are indicated to be at full pixel resolution and a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value. Motion vector differences are simply interpreted as integer offsets rather than 1/4-sample offsets. All other decoding processes remain same. The parsing process (below the header level) is unchanged. When the motion vectors are coded at full-sample precision and the video uses 4:2:2 or 4:2:0 sampling, the chroma motion vectors can be derived in the usual manner, which will produce 1/2-sample chroma motion displacements. Alternatively, it may be worth considering also rounding the chroma motion.

The fast search version decides based on the first 5 frames of a sequences whether integer MC works better, and then continues only using integer pel.

### RCE1 cross checks (2)

JCTVC-Q0099 RCE1: Crosscheck report on adaptive MV precision by Microsoft [X. Li (Qualcomm)] [late]

JCTVC-Q0156 RCE1: cross-check report of subtest 2 [Y. Zhou, J. Xu (Microsoft)]

# CfP on screen content coding (14)

## General

General issues for the screen content coding (SCC) joint CfP were discussed on the afternoon of the 1st day (Thu) (JRO), and each primary response was briefly reviewed.

The following overall issues were noted regarding experiment conditions:

- The "Slide show" sequence had too many frames – so it was sped up for viewing

- Lossless testing should use "cost mode = lossless" (not mentioning this in the CfP was an oversight, although most people noticed the problem)

- There was discussion of maximum PSNR and BD metric issues, with various opinions about how to address this.

Side activity in a BoG Q0239 of proposing parties (coordinated by H. Yu. R. Cohen, and R. Joshi) was established to conduct the following activities:

- Discuss the problems that occurred in computing BD rates, and align the different ways it was done

- Tabulate encoding/decoding times

- Report back with all proposals in comparison, provide RD curves

- Provide a survey of the proposals (tools used in the proposals)

It was agreed that for this first report, the clipped PSNR values should be used, if the unclipped values are not available for some of the proposals. In cases where this gives invalid results, the following options should be considered

- Exclude these cases

- Use BD SNR instead

- Add/subtract a small value to avoid zero division in the Excel sheet's computation.

It was agreed to also be reported how many cases are affected.

JCTVC-Q0239 BoG report on summary of objective performance and tools for SCC CfP responses [R. Joshi, R. Cohen, H. Yu]

The following information was provided by this BoG:

- A summary of B-D rates for lossy conditions excluding ‘desktop’, ‘console’ and ‘webBrowsing’

- A summary of compression ratio gains for lossless conditions for all sequences

- A summary of tools used in the CfP responses

- The rate-distortion plots for AI, RA, and LB test conditions for all sequences and rate points

Discussion of this BoG activity was held on the 2nd day (Fri) (JRO).

A BoG was held on Friday March 28 to provide a summary of objective performance of screen content coding CfP responses.

During the BoG meeting there was extensive discussion regarding the problems in the calculation of BD-rates. The main discussion points were:

- Whether clipping the PSNR to 58.92 dB was appropriate or some other clip value or unclipped values should be used for calculating the BD-rates. It was concluded that using unclipped PSNR values was unworkable when there was even a single lossless point.

- Some proponents had lossless coding at two or more rate points. In one case, all the rate-points were coded losslessly. This presented a challenge for the calculation of BD-rates. Some modifications such as adding small offsets to clipped PSNR values corresponding to lossless points were suggested. But, upon further testing, the BD-rate calculations were found to be unreliable in problem cases.

- Another suggestion was to use BD-PSNR. There was some concern that this measure had been very rarely used in JCT-VC. So it would be harder to interpret, and its behavior had not been well-tested under different conditions. A concern was also expressed that due to PSNR clipping, this may favor proposals with lower rates than the anchor rates.

- Another suggestion was to calculate PSNR difference assuming that the proposal and anchor bit-rates were the same and also list the percentage difference in rates. But there was objection that averaging the difference in PSNRs across rate points was meaningless.

- Another suggestion was to take the lossless results out of the lossy group, as lossless coding, in general, has never been evaluated in the RD space.

After an interim report was presented to the JCT-VC committee along with the rate-distortion plots, the committee advised the BoG to provide BD-rates for lossy conditions excluding the sequences ‘desktop’, ‘console’ and ‘webBrowsing’, which were the sequences that were most affected by the presence of lossless frames under lossy coding conditions. It was noted, however, that these screen-content sequences were the ones that exhibited the most improvement over the anchors when coded, so any BD-rate average tables that exclude these sequences should not be used alone when evaluating the proposals or tools. Performance on the three excluded sequences should be included in those considerations.

The following categories were selected for classification of elements found in proposals

- Block-based IntraBC

  - Partition, range, search

  - Block-based IntraBC: Modes

  - Block-based IntraBC: Block Vector coding

- Line-based Intra Copy

- Palette

  - Coding of the palette/colour table

  - Coding of colour indices

- String matching

- Combined IntraBC, Palette, and/or string-matching

- Cross-component

- Colour-space transform

- Loop-filtering

- Rice parameter initialization

- Other tools

- Encoder-only changes

The following table was presented as a result of the BoG's survey of proposals.

|  |  | Q0031 |  | Q0032 |  | Q0033 |  | Q0034 |  | Q0035 |  | Q0036 |  | Q0037 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Qualcomm |  | NCTU / ITRI |  | MediaTek |  | Huawei |  | Microsoft |  | Mitsubishi |  | InterDigital |  |
| s/w base |  | RExt6.0 |  | RExt6.0 |  | RExt5.1 |  | RExt5.1 |  | RExt5.1 |  | RExt6.0 |  | RExt5.1 |  |
| Block-based IntraBC - Partition, range, search | • Partition: No 2NxN, Nx2N • Range: Full picture • Search: Hash-based (16 bits) |  | • Range: Cur/left CTU, as anchor |  | • Partition: PU-based (RExt 6.0) • Range: Extended search range 2 L/A/R |  | • Range: Cur/left CTU, as anchor |  | • Partition: PU-based (RExt 6.0) • Range: Full picture • Search: hash-based, 2D Dictionary |  | • Range: Cur/left CTU, as anchor |  | • Partition: PU-based (RExt 6.0) • Range: Cur/left CTU, as anchor |  |  |
| Block-based IntraBC - Modes |  |  |  |  |  |  |  |  |  | • IntraBC-skip: 2Nx2N, no residue • IntraBC-merge: 2Nx2N, left/above • IntraBC-flip: PU-flag vertical flip |  |  |  |  |  |
| Block-based IntraBC - BV Coding |  | • Initialized with (-2w,0) • Left/Above BVs as predictors • Change to BV binarization |  | • Initialized with (−w, 0), (−2w,0) • Two most-recent BVs as predictors |  |  |  |  |  |  |  |  |  | • Initialized with (−w, 0) (Rext6.0) |  |
| Line-based Intra Copy |  |  |  | Line-based IBC • Partition: Within a PU, HOR or VER lines • Search Range: Cur/left CTU, as anchor • BVd Coding: Same as Block-based IBC |  | Line-based IBC • Partition: 2Nx1 or 1x2N lines • Search range: Cur/Above, Cur/Left • BVd Coding: 1D, all positive |  |  |  |  |  |  |  |  |  |
| Palette - Coding of the palette/colour table |  | • P0303+ • Flag to indicate escape mode. |  | • P0108+ • Representation: Component-wise, Nmax = 15 • Table: Palette propagation and palette merge Left/Above |  | • P0108+ • Representation: Triplet, Nmax = 64 • Table: Palette share from LAST |  | • Representation: Triplet, Nmax = 128 • Colour table merge Left/Above • Table: Inter-table colour pred, intra-table DPCM |  | • P0108+ • Table: Palette sharing |  | • P303 |  | • P303+ • Representative colour dictionary for palette prediction • Table: Entire palette copy from dictionary: "palette-skip" |  |
| Palette - coding of colour indices |  | • P0303 |  | • P0108+ • Index Line modes: Hor, Ver, Ver-Abv, Normal • Index Normal: 2 half-lines. Copy i-th above line, or Four neighbor index pred |  | • P0108+ • Index: Predictive coding of line modes • Index Normal: Four neighbor index pred, Transition Copy with 2 candidates |  | • Index scan: Hor/Ver scans • Index: 1-D string search for index • Residual coding |  | • P0108+ • Index Normal: Four neighbor index pred • Transition Copy |  |  |  | • P303+ • Escape as palette_idx 1 • Index: Transition Copy • Index mapping: -1 for some cases • Escape colour prediction |  |
| String matching |  | 1D dictionary for lossless ONLY (L0303) • CTU-flag, full pixel matching • (offset, length) • offset, predictive coded using 8 last • length 1~64x64. EG • Search range is whole frame |  |  |  |  |  |  |  | 1D dictionary • CU-flag, full pixel matching • Hor/ver scan • Mode 1: maintain dictionary, similar to Lempel-Ziv, size: 1<<18, level 5 - (offset, length) • Mode 2: All rec_samples - (offset_x, offset_y, length) • Search range is whole frame |  |  |  |  |  |
| Combined IntraBC, Palette, and/or string-matching |  |  |  | • Combined IBC-palette mode • Some pixels (signalled as a given palette index) use IBC, others use palette |  |  |  | • String-matching used to code colour indices |  |  |  |  |  |  |  |
| Cross-component |  |  |  |  |  | • RExt6.0 + modified alpha coding for RGB |  | • LMChroma from HM8.1, except no luma downsampling |  |  |  | • Switch between RExt CCP (residual domain) or LMchroma for Intra/IntraBC (Y to U; Y or U to V) |  |  |  |
| Colour-space transform |  | • CU-level adaptive • One for lossy, one for lossless • For lossy: not normalized, QP+8, bit-depth + 2 • Intra: On predicted / orig block • Inter/IntraBC: On residual (cbf=1) |  |  |  |  |  |  |  | • CU-level adaptive • Only for Intra mode with GBR sequences • GBR to YCoCg; component reorder: RGB, BGR • On predicted / orig block |  |  |  | • CU-level adaptive • GBR to YCoCg (lossy) or YCoCg-R (reversible, for lossless) • For lossless, bit-depth + 1 • ONLY to Inter/IntraBC residue (cbf=1) |  |
| Loop-filtering |  | Modified deblocking: • PPS, deblock chroma as luma • BS = 3, conditions: large TU, intra / 2Nx2N, gradient • BS = 3, extended to 7 on each side |  |  |  |  |  |  |  |  |  | • Histogram Correction - As alternative mode of SAO - Each CUT, ranges w1, w2, w3, w4 around top 4 histogram peaks p, and "correct" values between p-w, p+w mapped to p |  | • No-deblocking for palette CU |  |
| Rice parameter init. |  |  |  |  |  |  |  |  |  |  |  |  |  | • As in Rext 6.0 |  |
| Other tools |  | • Explicit RDPCM on Intra / Inter / IntraBC • Deflickering - SPS flag: AI + high QP - CU flag, if ON, spatial neighbor mark UNAVAILABLE |  | • Adaptive MV precision (additional ME) |  | • Single colour mode - One value for entire CU - Candidate values from neighboring index • Disable boundary filter IntraDC,hor,ver |  |  |  |  |  | • Independent Uniform Pred Mode - One value for entire CU - Candidate values by analyzing CTU in slice |  | • For 4:4:4 lossy: 8-tap interpolation filters for all 3 colour components |  |
| Enc-only changes |  | • LD: Scene-change detection: CRA • Modified ME algorithm • Fast AI mode selection |  |  |  |  |  |  |  | • Modified inter-ME algorithm - hash based (16-bit) for all frames in DPB and for certain CU sizes; early termination; starting point |  |  |  |  |  |

The following possible areas of experiments were initially discussed on Friday and later tentatively planned to be executed as Core Experiments (SCCE) by the JCT-VC plenary on Tuesday 0800-1000 (chaired by JRO) with following coordinators:

- SCCE1 Intra BC extensions (J. Sole)

- SCCE2 Line based intra copy (C. C. Chen)

- SCCE3 Palette mode (Y.W. Huang)

- SCCE4 String matching for sample coding (Y. Chen)

- SCCE5 Inter-component prediction and adaptive colour transforms (X. Xiu)

Possible basis for experimentation: RExt 7.0 (could include new motion estimation for screen content, that was adopted from Q0147; it is report on performance of hash based search from Q0035 approximately12% for RA against RExt6, approximately 8.7% for LDB average over the entire screen content set)

Possible AHGs were identified:

- AHG on encoder optimization for screen content coding

- AHG on loop filter for screen content coding

- AHG on colour spaces for screen content coding

- String matching as generic method is used for different purposes and should also be studied in an AHG

RD plots (with clipped PSNR values) were also presented. It is asserted that the BD rate computation for the cases where PSNRs were clipped are invalid. The following sequences are affected

- Desktop

- Console

- Web browsing

The BoG was asked to prepare an average BD rate comparison excluding these three sequences. It should be reported as a result of the CfP that for these sequences the gain compared to the anchors were so large that even the lossless range was reached for the higher rate points, and BD rate computation was not possible.

JCTVC-Q0236 BoG report for Non-CfP SCC and related documents [R. Cohen]

A BoG discussion was held on Saturday March 29 to go over the Non-CfP screen content coding and related technical contributions.

The reviewed documents included both new proposals and proposals related to one or more tools also included in the formal responses to the SCC CfP. Improvements to, and combinations of, existing tools were also proposed. Most, but not all, of the proposals were related to tools proposed in response to the SCC CfP, or to SCC-related tools proposed at earlier meetings. Many of these tools are not in the current HEVC Range Extensions Draft Text, and because they were related to the SCC Joint CfP responses, no formal recommendations were made by the BoG to adopt any specific proposal into RExt.

Many documents proposed identical or very similar tools or combinations of tools.The outcomes of most discussions on these documents were that the proposals should be considered when defining common software platforms or common conditions for subsequent CEs, TEs, or AHG experiments. This happened for several cases, including Palette mode colour table generation, palette mode colour index coding, SAO improvements, single-colour CU coding, string-matching/dictionary coding, and related encoder improvements.

In addition to discussing specific proposals, methodologies for evaluating them were also discussed. There was discussion of and consensus on using a higher, resolution-dependent PSNR limit than was used in the SCC CfP for clipping per-frame PSNR values, because the current clipping value of 58.92 dB caused problems with both computing BD-Rate and plotting PSNR vs. Rate curves. There was also some discussion on how to evaluate SAO tools in a way that isn’t affected by the encoder’s RD-optimized mode decision process. That would be an issue more for RA and LB coding constraints as compared to AI. Issues related to reporting average PSNR values across RGB and YUV results were also discussed.

The general recommendation from the BoG for most of these proposals was that they be considered when defining common software platforms or common conditions for related CEs, TEs, or AHG experiments. Further details can be found in the document and discussion summaries included in the BoG report. Some convergence prior to the end of the meeting was suggested to be desirable, as opposed to waiting for the post-meeting editing period for CEs/TEs.

JCTVC-Q0244 BoG report on screen content coding reference model (SCM) [R. Cohen]

This document contains the meeting report from the BoG on screen content coding reference model.

Discussed Thu a.m. (GJS)

BoG meetings were held on April 1, 2, and 3 to discuss and recommend a reference model for screen content coding activities. The decision had been reached in JCT-VC to use full frame IBC as the reference for comparison in all experiments. However, it was reported that other experiments can show additionally that they provide benefit when the option of RExt6 (2 CTU) configuration of IBC is only used.

The scope of the BoG activity was to work to establish the reference model (SCM) which is an extension from RExt6.0 with:

- Additional non-normative tools for motion search and whether they should be hash based for SCC;

- Full-frame IBC (with option for 2 CTU range) with an algorithm for encoder search; quantizer modification.

The aspect was identified as the full frame IBC in encoder and decoder and determining how the search is done.

Options for the SCM were identified.

The BoG Recommendation – endorsed by JCT-VC was to use "Option 6" (Intra hash from Qualcomm, Inter hash from Microsoft, JCTVC-Q0248) if updated results are verified and consistent with those shown above, and if consensus obtained from JCT-VC participants; otherwise to use Option 2a (Intra hash from Qualcomm, no hash for Inter, JCTVC-Q0243). For the verification, non-proponent(s) will complete a crosscheck of JCTVC-Q0248 within 2 weeks after the meeting (so far, MediaTek had volunteered).

The BoG Recommendation was also to establish CE or AHG study on inter/intra hash topics.

Regarding test conditions:

- The BoG recommendation was to remove SocialNetworkMap from the test set. In JCT-VC discussion, it was suggested and agreed to also reduce the length of "flyinggraphics text" – to use the first half of the sequence.

- Regarding QP values, using 22, 27, 32, 37 was agreed.

- The BoG recommended to include a add a camera-captured category consisting of EBURainFruit, and Kimono for SCC CTC purposes.

It was suggested to give a mandate to an AHG to facilitate the testing of combinations of proposed coding tools.

It was agreed that SCM 0.x is to be based on RExt 6, with version 1.0 based on RExt 7, and additional point releases as appropriate. CEs were agreed to be based on version 1.0.

## Primary responses (7)

JCTVC-Q0031 Description of screen content coding technology proposal by Qualcomm [J. Chen, Y. Chen, T. Hsieh, R. Joshi, M. Karczewicz, W.-S. Kim, X. Li, C. Pang, W. Pu, K. Rapaka, J. Sole, L. Zhang, F. Zou (Qualcomm)]

Discussed 1st day (Thu) p.m. (JRO).

This document presents the details of Qualcomm's response to the Joint Call for Proposals for Coding of Screen Content issued by MPEG and ITU-T. The proposed solution is developed based on the HEVC Range Extensions Draft 6 and the HM13.0-RExt6.0 software. The main changes with respect to the base are the following:

- Extension of the current tools: full-frame search intra BC (using left and above block as prediction for displacement), deblocking filter (luma process used for chroma, new Bs3 which accesses 8 samples at both sides of boundary, used in case of 16x16 TU) and explicit RDPCM

- New tools: palette and colour transform (CU based adaptive), colour transform is modified YCoCg, two different for lossless/lossy

- Encoder modifications and subjective enhancement for screen content: Improved motion estimation, uniform quantization in RDPCM, include chroma in RD selections, scene change detection for low delay, reduce max RQT depth to 2, disable IBC with 2NxN.

- 1D dictionary method (related to JCTVC-L0303), matching area is the whole reconstructed frame, hash table used for fast matching

- De-flickering method in all intra (enforcing the same prediction mode which basically is an encoder selection), combined with disabling prediction from neighboring CUs (normative)

The proposed screen content video codec reportedly achieves BD-rate savings summarized as follows:

- For RGB sequences, G-component, lossy condition:

  - text & graphics with motion, 1080p: (AI) 52.7%  (RA) 45.4%  (LB) 41.7%

  - text & graphics with motion,720p: (AI) 39.9%  (RA) 42.2%  (LB) 39.7%

  - mixed content, 1440p: (AI) 35.4%  (RA) 42.3%  (LB) 48.3%

  - mixed content, 1080p: (AI) 31.1%  (RA) 36.6%  (LB) 38.1%

  - animation, 720p: (AI) 23.4%  (RA) 26.5%  (LB) 28.4%

- For YUV sequences, Y-component, lossy condition:

  - text & graphics with motion, 1080p: (AI) 47.5%  (RA) 36.9%  (LB) 30.7%

  - text & graphics with motion,720p: (AI) 27.7%  (RA) 29.5%  (LB) 26.0%

  - mixed content, 1440p: (AI) 21.8%  (RA) 22.8%  (LB) 29.5%

  - mixed content, 1080p: (AI) 20.2%  (RA) 20.6%  (LB) 20.9%

  - animation, 720p: (AI) 0.6%  (RA) 3.8%  (LB) 6.1%

It is observed that the bit rate savings for lossy and lossless case are very similar.

The results above include unclipped PSNR values for two sequences where the BD rate computation failed due to equal PSNR when clipping at neighbored rate points.

Encoding time increase lossy 175% in AI, 115% in RA, 128% in LB

Encoding time lossless: 238%/178%/172%

Decoding time lossy 79%/116%/111%

Decoding time lossless 78%/104%/97%

Note: These time measurements are compared to RExt 5.1, whereas RExt 6.0 already increases encoding time by approximately 50%.

JCTVC-Q0032 Description of screen content coding technology proposal by NCTU and ITRI International [C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, Y.-J. Chang, C.-H. Hung, C.-C. Lin, J.-S. Tu, E.-C. Ke, J.-Y. Kao, C.-L. Lin, F.-D. Jou, (NCTU/ITRI)]

Discussed 1st day (Thu) p.m. (JRO).

This document describes the technologies, jointly proposed by NCTU and ITRI International, in response to the joint call for proposals for coding of screen content. It extends the HEVC RExt (JCTVC-P01005) with three additional types of intra modes (palette coding, combined palette coding and intra block copy, and line-based intra block copy), adaptive motion vector precision, and two block vector coding schemes (block vector initialization and second block vector predictor). The average G BD-rate reduction for RGB sequences in lossy coding was reported as follows:

- text & graphics with motion, 1080p: (AI) 28.6%,	(RA) 17.3%,	(LB) 13.7%;

- text & graphics with motion,720p  : (AI) 14.9%,	(RA) 12.3%,	(LB) 10.4%;

- mixed content, 1440p            : (AI) 9.5%,	(RA) 7.7%,	(LB) 10.0%;

- mixed content, 1080p            : (AI) 11.1%,	(RA) 8.7%,	(LB) 8.0%;

- animation, 720p                : (AI) 0.8%,	(RA) 1.5%,	(LB) 2.3%.

The average Y BD-rate reduction for YUV sequences is reported as follows:

- text & graphics with motion, 1080p : (AI) 26.3%,	(RA) 15.6%,	(LB) 10.5%;

- text & graphics with motion,720p  : (AI) 12.7%,	(RA) 10.6%,	(LB) 8.2%;

- mixed content, 1440p            : (AI) 9.2%,	(RA) 6.6%,	(LB) 7.6%;

- mixed content, 1080p            : (AI) 10.5%,	(RA) 7.8%,	(LB) 6.5%;

- animation, 720p                : (AI) 0.6%,	(RA) 0.6%,	(LB) 0.8%.

The average runtime relative to the HM-12.1+RExt-5.1 anchors is reported as follows:

- encoding time                  : (AI) 169%,	(RA) 122%,	(LB) 121%;

- decoding time                  : (AI) 103%,	(RA) 86%,	(LB) 115%.

Based on HM‐13.0+RExt‐6.0 with

- IntraBC Extensions

  - Line‐based IntraBC (only from current and left CTU)

  - Pingpong BV predictor (P0217)

- Palette Coding (based on P0108)

  - Major colour merging (P0152)

  - Sub‐row copy above mode

- Combined IntraBC and Palette Coding

- Adaptive MV Precision (P0283)

Additional results are presented with full frame IBC, which indicate that significant additional gain is possible (e.g. 39.3% for AI t&gmotion 1080).

JCTVC-Q0033 Description of screen content coding technology proposal by MediaTek [P. Lai, T.-D. Chuang, Y.-C. Sun, X. Xu, J. Ye, S.-T. Hsiang, Y.-W. Chen, K. Zhang, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

Discussed 1st day (Thu) p.m. (JRO).

The goal of this proposal is to provide screen content coding technologies on top of the HEVC standard. In order to achieve this goal, a number of coding tools are proposed. These include intra block copying (IntraBC) with extended search range, triplet palette mode, line-based (2N×1/1×2N) intra copying, modified coding of inter-component residual prediction, single colour mode. With all the proposed tools enabled, the proposed screen content video codec reportedly achieves BD-rate savings summarized as the following:

For RGB sequences, G-component, lossy condition:

- text & graphics with motion, 1080p: (AI) 46.0%  (RA) 31.4%  (LB) 26.0%;

- text & graphics with motion,720p: (AI) 28.6%  (RA) 23.7% 	(LB) 20.3%;

- mixed content, 1440p      	: (AI) 20.2%,	(RA) 14.1%, 	(LB) 13.0%;

- mixed content, 1080p      	: (AI) 18.8%,	(RA) 13.1%, 	(LB) 8.7%;

- animation, 720p            	: (AI) 1.7%, 	(RA) 1.3%, 	(LB) 1.1%.

For YUV sequences, Y-component, lossy condition:

- text & graphics with motion, 1080p: (AI) 44.1%,	(RA) 27.4%,	(LB) 21.2%;

- text & graphics with motion,720p: (AI) 25.1%,	(RA) 20.0%, 	(LB) 14.7%;

- mixed content, 1440p           : (AI) 20.0%,	(RA) 14.0%, 	(LB) 11.5%;

- mixed content, 1080p           : (AI) 17.9%,	(RA) 13.4%, 	(LB) 8.1%;

- animation, 720p                : (AI) 0.3%, 	(RA) 0.1%, 	(LB) 0.3%.

The average encoding time for the proposed encoder compared against the anchor RExt 5.1 are 314%, 151%, 141% for lossy AI, RA, and LD configurations, and 472%, 175%, 163% for lossless AI, RA, and LD configurations, respectively. The corresponding average decoding time compared against anchor are 81%, 99%, and 101% for lossy and 87%, 93%, and 96% for lossless, respectively.

The high encoding times are likely caused by extending IBC search range and line-based copy (no fast algorithm e.g. hash based search used).

Software base: HM12.1-RExt5.1 (anchor)

Coding tools on top of anchor

- PU-based IntraBC (RExt6.0) with extended search range (12 previous CTUs left and above)

- Line-based Intra copying

- Triplet palette mode (3 colours, various aspects in coding of palette table and sample indices)

- Single colour mode (reconstruct entire block with one colour based on candidates from boundary)

- Modified alpha parameter coding for inter-component residual coding (On top of RExt6.0)

- Intra boundary filter disabling

It is also mentioned that line based copy utilizes similar redundancy as the 1D dictionary from Q0031.

JCTVC-Q0034 Description of screen content coding technology proposal by Huawei Technologies (USA) [Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu (Huawei)]

Discussed 1st day (Thu) p.m. (JRO).

This contribution introduces an additional "colour table and index map coding mode" in intra frames, on top of the HEVC Range Extensions (RExt) Draft 5 [2], for coding of screen content. Additionally, a luma-based chroma intra prediction method is added to further exploit correlations between colour components. The proposed solution is implemented in the HEVC range extension reference software HM12.1+RExt-5.1 [3] (RExt5.1). The simulation results have shown an average bit-rate reduction over the CfP anchors for lossless coding by 14.0%, 8.9%, and 7.8% for All Intra (AI), Low-Delay with B-picture (LB), and Random Access (RA), respectively. For the lossy coding mode, BD rate reduction of 10.3% for AI, 7.9% for RA, and 5.3% for LB have been observed. The complexity is measured using the encoding and decoding times. However, due to the heterogeneous computing nodes utilized, the running times presented in the test results may not accurately reflect the relative complexity.

Colour table processing (for three components) has some commonality with the triplet colour table of Q0033. Merge (use table from left or above CU at CTU boundary); adaptive index map scanning.

1D string search from line buffer gives around 1% gain.

Search is constrained to current CU for all elements of the proposal.

LM Chroma does give only small benefit (less than 1% on average).

Encoder/decoder run times are asserted to be not reliable.

JCTVC-Q0035 Description of screen content coding technology proposal by Microsoft [B. Li, J. Xu, F. Wu, X. Guo, G. J. Sullivan (Microsoft)]

Discussed 1st day (Thu) p.m. (JRO).

In this proposal, hash based search, 1-D dictionary mode, adaptive colour space coding, modifications to intra BC mode, and palette mode, etc. are introduced to improve the coding efficiency and reduce the coding complexity for screen contents. Experimental results reportedly show that for lossy coding, using lower bit rates than the anchor, the proposed scheme achieves 12.47 dB, 9.67 dB, and 9.12 dB on Y-component (or G-component if the input is of RGB format) PSNR improvement on average for AI, RA, and LB respectively. The encoding time for RA and LB are reported to be less than 90% of that of the anchor. The decoding time is similar to the anchor (saving about 10% for AI case). For lossless coding, about 29.9%, 24.0% and 23.0% bit rate savings are reportedly achieved for AI, RA and LB respectively. Moreover, different trade-off points between the encoding complexity and coding efficiency can be achieved; e.g., the proposed scheme with a low complexity setting reportedly shows about 3x faster encoding than the anchor, with average bit-rate savings of 19.5% and 18.6% for RA and LB lossless coding, respectively.

Intra BC: Skip, merge, flip vertical, full frame with 2D dictionary (hash based search)

1D dictionary which copies a string of samples from a reconstructed area (follows 2D structure of current PB but string is variable in length), using another hash based search. Horizontal/vertical possible.

Adaptive colour space coding GBR/YCoCg/RGB/BGR, only applied to RGB.

Hash based search also applied for motion estimation in inter mode (would likely not work for camera content due to noise), also considering chroma

Based on RExt 5.1, but some elements aligned with RExt 6

No BD rates reported, since some numbers were not meaningful (e.g. 0% BD rate was computed in a case where PSNR range was non-overlapping)

Encoding time for AI 290/370% for lossy/lossless; RA and LDB are reduced in encoding time compared to anchor (likely due to hash based search in motion estimation). Without the hash table for motion estimation, encoding times for RA/LD might also be more significantly increased.

Additional memory for storing hash table at encoder for motion comp could be non-negligible.

JCTVC-Q0036 Description of screen content coding technology proposal by Mitsubishi Electric Corporation [R. Cohen, A. Minezawa, X. Zhang, K. Miyazawa, A. Vetro, S. Sekiguchi, K. Sugimoto, T. Murakami (Mitsubishi Electric)]

Discussed 1st day (Thu) p.m. (JRO).

This document presents specifications of a new video coding algorithm developed for submission as a response to the Joint Call for Proposals for Coding of Screen Content issued by ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q6. The proposed algorithm builds upon HEVC Range Extensions Draft 6 by adding several coding tools targeting objective and subjective performance for coding screen content video. These new tools include block-level inter-component prediction, a histogram correction mode for Sample Adaptive Offset, an independent uniform prediction mode, and a palette mode. Specific examples of subjective improvements are presented. For objective performance, for lossy test conditions, average gains over the anchor for various content types are up to 27% for the G component of RGB sequences and up to 26% for the Y component of YCbCr, for All Intra conditions. For Random Access conditions, the corresponding averages were up to 17% for G and 14% for Y, and for Low Delay conditions, the averages were up to 10.2% for G and 8.1% for Y. The gains on individual sequences were up to 32%, 29% and 19% for the G component for AI, RA, and LD conditions respectively, and up to 31%, 25% and 11% for the Y component. For lossless conditions, average bit-rate savings for RGB sequences were up to 32%, 28% and 28% for AI, RA, and LD conditions respectively, and for YCbCr sequences average bit-rate savings were up to 30%, 20% and 17%. Maximum bit-rate savings were up to 43%, 40% and 40% for RGB AI, RA, and LB conditions, and 46%, 32% and 24% for YCbCr.

Based on RExt 6.0. Additional tools:

- Inter-component prediction (based on linear model from co-located block)

- Histogram Correction mode for SAO

- Independent uniform prediction (using an explicit colour table signaled at the slice header)

- Palette mode (from P0303)

Visual examples are given for the histogram correction mode. It is asked wheter these were using the same coding mode and bit rate – this may not be the case.

Encoder time AI lossy 300% (with follow-up contribution showing approximately 250%), decoder time 87%.

RA/LD 150/140% encoder, 88/111% decoder (decoder times may not be reliable).

JCTVC-Q0037 Description of screen content coding technology proposal by InterDigital [X. Xiu, C.-M. Tsai, Y. He, Y. Ye (InterDigital)]

Discussed 1st day (Thu) p.m. (JRO).

This proposal uses two main technologies, namely improved palette coding and adaptive residue colour space conversion, based on the current framework of HEVC Range Extensions.

Compared to the CfP anchors, for lossy coding, the proposed solution achieves the average {G, B, R} BD-rate reductions of {16.3%, 15.9%, 15.8%}, {13.4%, 13.2%, 13.1%} and {13.8%, 13.6%, 13.4%} for AI, RA and LD, respectively, in RGB coding, and the average luma BD-rate reductions of 13.2%, 9.2% and 7% for AI, RA and LD, respectively, in YCbCr coding. For lossless coding, the average bit-rate savings of the proposed solution are 14.8%, 16.2% and 16.7% for AI, RA and LD, respectively, in RGB coding, and 13.8%, 10.2% and 9.3% for AI, RA and LD, respectively, in YCbCr coding.

The performance improvement for screen content sequences (video sequences in the category "text & graphics with motion") is significantly higher. For lossy coding, the proposed solution achieves average {G, B, R} BD-rate reductions of {28.9%, 28.4%, 28.3%}, {21.7%, 21.1%, 21.3%} and {18.2%, 17.5%, 17.8%} for AI, RA and LD, respectively, in RGB coding, and average luma BD-rate reductions of 22.7%, 15.7% and 10.3% for AI, RA and LD, respectively, in YCbCr coding. For lossless coding, the bit-rate savings of the proposed solution are 27.8%, 25.8% and 25.9% for AI, RA and LD, respectively, in RGB coding, and are 28%, 21.5% and 19.4% for AI, RA and LD, respectively, in YCbCr coding.

Basis is RExt 5.1, additionally IBC with 2NxN, and Rice parameter modification of RExt 6.0.

Palette mode based on the AHG10 framework, with improved palette table prediction, and table skip mode. Re-sorting of indices by "Burrows-Wheeler transform" for achieving longer run-lengths. Transition mode from JCTVC-P0115 for table sizes >14, but not used with BWT.

Adaptive conversion RGB / YCoCg

Encoding lossy 284/174/163% for AI/RA/LD

Encoding 324/185/167% for AI/RA/LD

Decoding times were around 85% of anchor for all cases; numbers may not be fully reliable.

It was not exactly known what the benefit of the different elements of the proposal were.

## Related contributions (7)

JCTVC-Q0052 Low-complexity encoders for JCTVC-Q0035 [B. Li, J. Xu (Microsoft)]

Reviewed in BoG Q0236 on 3rd day (RC).

JCTVC-Q0053 On BD-Rate results [B. Li, J. Xu (Microsoft)]

Reviewed in BoG Q0236 on 3rd day (RC).

There appeared to be consensus to change how to handle very high PSNR measurements.

JCTVC-Q0193 Combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036 [R. Cohen, X. Zhang, A. Vetro, K. Sugimoto (MERL), A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi Electric), Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu (Huawei Technologies (USA)] [late]

Reviewed 3rd day (Sat.) p.m. (GJS).

This document proposes a combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and and JCTVC-Q0176. Using HEVC Range Extensions Draft 6 and the HM-13.0_RExt-6.0 software as a foundation, this combined proposal adds a colour table and index map coding mode, block-level inter-component prediction, a histogram correction mode for SAO, and an independent uniform prediction mode. Using the same test conditions as specified in the Joint Call for Proposals for Coding of Screen Content, with HM 12.1_RExt 5.1 coded bitstreams as anchors, average improvements in BD-Rate for various content types under the lossy All Intra (AI) configuration are reportedly up to 39% for RGB sequences and up to 34% for the Y component of YCbCr. For Random Access (RA) conditions, the corresponding averages were up to 26% for RGB and 21% for YCbCr, and for Low Delay (LB) conditions, the averages were up to 18% for RGB and 13% for YCbCr. For lossless conditions , averaged bit rate reduction is up to 45%, 46% for AI RGB and AI YCbCr, 39% and 36% for RA RGB and RA YCbCr, and 38% and 32% for LB RGB and LB YCbCr, respectively..

Combined proposal for Mitsubishi and MERL (Q0036 and Q0176), combining as tabulated below.

| JCTVC-Q0034/JCTVC-Q0176 | JCTVC-Q0036 | Combined proposal |
| --- | --- | --- |
| Colour table and index map mode | Palette mode | Colour table and index map mode |
| Inter-component prediction | Inter-component prediction | Inter-component prediction |
|  | Histogram Correction for SAO | Histogram Correction for SAO |
|  | Independent Uniform Prediction | Independent Uniform Prediction |
| Also includes RExt 5.1 tools | Also includes RExt 6.0 tools | Also includes RExt 6.0 tools |

Reporting that for a combined complexity about the same as in one of these proposals, an additional gain is reported. For example, for the "RGB text & graphics with motion at 1080p" sequences, about 9% improvement was reported for AI, RA and LB.

This seems to be good information for consideration regarding how to assess the potential shown in CfP response proposals and potential ways to move forward into collaborative work.

JCTVC-Q0194 Cross-check of JCTVC-Q0034 [X. Zhang, R. Cohen (MERL), K. Miyazawa (Mitsubishi Electric)] [late]

JCTVC-Q0213 Fix for adaptive colour space coding in JCTVC-Q0035 [B. Li, J. Xu (Microsoft)] [late]

Reviewed in BoG Q0236 on 3rd day (Sat.) (RC).

For further consideration in the context of future adaptive colour space work.

JCTVC-Q0235 Cross-check report on Description of SCC technology proposal by MERL (JCTVC-Q0036) [X. Wang, H. Yang, M. Xu (Huawei)] [late]

JCTVC-Q0238 Encoding time reduction for Independent Uniform Prediction in SCC CfP response JCTVC-Q0036 [X. Zhang, R. Cohen] [late]

- JCTVC-Q0242 Proposed common software base for future screen content coding development [B. Li, J. Xu, G. J. Sullivan (Microsoft)] [late]

- JCTVC-Q0243 Software for the Screen Content Coding Model [K. Rapaka, J. Sole, M. Karczewicz] [late]

JCTVC-Q0250 Cross-check of JCTVC-Q0243 Software for the Screen Content Coding Model and JCTVC-Q0248 Software for SCM with hash-based motion search [P. Lai, X. Xu, S. Liu, Y.-C. Sun, T.-D. Chuang (MediaTek)] [late] [miss]

JCTVC-Q0245 Hash-based motion search [B. Li, J. Xu (Microsoft)] [late]

JCTVC-Q0248 Software for SCM with hash-based motion search [K. Rapaka (Qualcomm), J. Xu (Microsoft)] [late]

JCTVC-Q0252 Hash-based intraBC search [B. Li, J. Xu (Microsoft)] [late]

# Non-CE Technical Contributions (147)

## Range extensions (41)

### General (1)

JCTVC-Q0076 Unifying HM and RExt Inter-Prediction Search [K. Sharman, N. Saunders, J. Gamei (Sony)]

Discussed Sat 29th (JRO) or Sun 30th (D. Flynn & C. Rosewarne as BoG).

This contribution examines the encoder inter search algorithms used in RExt and HM. It is observed that, as a result of the adoption of the cross-component prediction tool at an earlier JCTVC meeting, two inter residual estimation algorithms now exist within the Range Extensions software model: one that is compatible with HM, and one that is not.

It is proposed that RExt and HM are harmonised by applying a patch to HM.

Tests using HM common test conditions are reported to show a BD-rate change for luma between 0.1% and -0.1%, and for chroma between 0.3% and -0.5% (Classes A-E), with no significant average change, indicating that the modified search has no significiant overall effect. Averaged BD-rate changes using the AHG5 RExt common test conditions are reported to be 0.1% to -0.3%, indicating no significant effect. Averaged BD rate changes using the AHG8 RExt lossless test conditions are observed to lie within the 0.0% to -2.9% range, indicating equal or superior performance of the modified RExt search algorithm across all test sequences.

Follow-up of P0059. The performance of that contribution was inconsistent in lossy/lossless conditions, which is asserted to be due to a bug found in the meantime. Question: Relation with Q0147? This tackles a different aspect (pattern in diamond search), whereas Q0076 unifies the RD optimization in mode selection between HM and RExt.

This helps to keep the two software bases aligned. Several experts expressed support.

Decision (SW): Adopt.

### RCE1 related (4)

JCTVC-Q0067 Non-RCE1: On MV resolution and motion vector predictor number [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Discussed 2nd day (Fri) (JRO).

The HEVC Range Extensions Core Experiment 1 is dedicated to the adaptive Motion vector precision method. In this RCE1, 2 methods to signal the motion vector precision are investigated. One signals the MV precision at slice level and the second one at CU level. Both methods modify the Merge and Inter/AMVP modes. In this contribution, the Motion vector precision is signaled at PU level for Inter mode only. Moreover, when the precision is set equal to the full-pel, the number of predictors for Inter/AMVP is reduced to 1. Several complexity compromises are reported. The proposed modification can reach 2.1% gain over RExt6.0 for RA/ LDB configurations. When the encoding complexity is reduced to 110% the average gain for all RA/LDB configurations is 1.7%.

In the first experiment, Test 1, the adaptive motion vector resolution for Inter/AMVP PU (MV res) is combined with 1 motion vector predictor for full-pel resolution (1 Pred). The second one, Test 2, is the same as Test 1 with the proposed fast estimation. Test 3 is the same as Test 2 without 1 predictor for full-pel resolution. Test 4 is the fast estimation without change of the Rext6.0 syntax.

|  | MV RES Inter/ AMVP PU |  | 1 Pred |  | Fast Estimation |  | Average BDR |  | Average BDR +optional |  | Enc Time |  | Dec Time |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test 1 |  | ✓ |  | ✓ |  |  |  | -1.6% |  | -2.1% |  | 120% |  | 95% |  |
| Test 2 |  | ✓ |  | ✓ |  | ✓ |  | -1.1% |  | -1.7% |  | 110% |  | 94% |  |
| Test 3 |  | ✓ |  |  |  | ✓ |  | -0.7% |  | -1.1% |  | 110% |  | 98% |  |
| Test 4 |  |  |  |  |  | ✓ |  | -0.1% |  | -0.1% |  | 110% |  | 99% |  |
| RCE1 Test2 MV1 |  |  |  |  |  |  |  | -0.6% |  | -0.7 |  | 148% |  | 101% |  |
| RCE1 Test2 MV2 |  |  |  |  |  |  |  | -0.9% |  | -1.2% |  | 142% |  | 95% |  |

Switching is done at PU level (whereas Q0049 of RCE1 signals at CU level)

Test 1 and Test 2 use only one MV in AMVP (instead of 2) in case of integer pel

Test 3 is using two MV in AMVP.

Test 4 is non-normative test for integer pel restriction.

Average gain of approximately 0.4% average is reported on the AHG5 test set (which consists of camera captured content). However, Test 4 (non-normative) is not reported for this test set.

Main gain is achieved for screen content, and this proposal would require more significant changes in core parts of AMVP and CABAC parsing. Further study was planned in the context of screen content coding activity.

JCTVC-Q0098 Non-RCE1: Crosscheck report on MV resolution and motion vector predictor number (JCTVC-Q0067) [X. Li (Qualcomm)] [late]

JCTVC-Q0092 Non-RCE1: Simplification of RCE1 Test2 [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

Discussed 2nd day (Fri) (JRO).

A CU-level adaptive MV precision method was proposed at the 16th JCT-VC meeting and is studied in RCE1 Test2 [1], where a CU-level flag is signaled to indicate whether all PUs in the CU have integer-precision MVs. If the flag is 1, the MV precision is in integer precision. The MV predictors of PUs are rounded to integer precision and MV differences are signaled in integer precision. Otherwise, the MV precision is in quarter precision. The MV coding is the same with HEVC version 1 and the sub-pixel motion compensation is allowed. In RCE1 Test2, the adaptive MV precision is applied on all inter CU. The context formation of the CU-level flag requires the information of the coded adaptive MV precision flag of the upper CU, which results in a requirement of a line buffer for this flag. The adaptive MV precision is also found to be less efficient for merge mode.

Gains are reported relative to RCE1 test 2 (Q0049) and RExt 6.0

Lossy coding results and lossless coding results of the proposed single context adaptive MV precision flag coding using RCE1 Test2 as the anchor and implementation basis are shown in Table 1. The results reportedly show 0.2% and 0.1% loss for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier.

Lossy coding results and lossless coding results of the proposed single context adaptive MV precision flag coding and disabling adaptive MV precision for merge mode using RCE1 Test2 as the anchor and implementation basis are shown in Table 2, where 0.2% and 0.6%BD-rate savings are shown for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. The encoding times are reduced by 9% and 10% for RA-Main-Tier and LB-Main-Tier, respectively.

Table 3 shows the lossy coding results and lossless coding results of the proposed methods using HM-13.0+RExt-6.0 as the anchor. It is reported to achieve 0.9% and 1.3% BD-rate savings for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively.

Signaling at CU level as in Q0049, but adaptive motion precision is not used in merge, therefore the flag is not present if all PUs are in merge mode.

Further study was planned in the context of screen content coding activity.

JCTVC-Q0173 Non-RCE1: Crosscheck report on simplification of RCE1 test2 (JCTVC-Q0092) [X. Li (Qualcomm)] [late]

### Implementation aspects of high bit rate and high bit depth (4)

JCTVC-Q0044 AhG18: On SAO quant-bits coding [E. Alshina, A. Alshin (Samsung)]

Discussed 2nd day (Fri) (JRO).

Quantized SAO parameters are signalled in the bitstream according to the JCTVC 16th meeting outcome, but there is no recommendation about the choice for these parameters. This contribution suggests for settings of SAO quantization depending on processing bit-depth  and slice QP, which reportedly improves SAO performance by 1.6% (LD) and 0.2%(AI) for 16 bit coding and 1.1%(LD) for 12 bit coding. At the same time, this contribution suggests to move some SAO control syntax from PPS to the slice header.

Some concern was expressed that the slice header may not be the best position for these parameters, since they require a significant amount of data. The PPS seems to be the best place, as the same offset parameters are likely used for several slices (depending on encoder decisions).

Decision (SW): Adopt the non-normative part (proposed setting of offset value for 12 bits & beyond).

Note: The syntax still requires editorial improvement. This is a known issue.

(Editorial action item.)

JCTVC-Q0073 AHG18: Worst-case Escape Code Length Mitigation [K. Sharman, N. Saunders, J. Gamei (Sony)]

Discussed 2nd day (Fri) (JRO).

This contribution identifies the worst-case code length that may occur when coding an escape code for a coefficient as 46. The contribution proposes an alternative scheme to the exponential-Golomb part of escape coding that claims to reduce the worst-case escape code length to 32, which is the same as the worst-case escape code length in HEVC version 1. All-tier BD-rate changes of 0.0% are reported for AHG18 lossy and lossless common test conditions.

Simplification of previous P0061, the new proposal would make the coding also identical with v1 scheme for smaller bit depths, which allows simplification of a RExt decoder.

Decision: Adopt (Q0073&Q0131).

JCTVC-Q0185 Cross-check of JCTVC-Q0073 AHG18: Worst-case Escape Code Length Mitigation by Sony [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-Q0131 AHG18: Limiting the worst-case length for coeff_abs_level_remaining syntax element to 32 bits [M. Karczewicz, R. Joshi (Qualcomm)]

Discussed 2nd day (Fri) (JRO).

This contribution proposes a modification of the binarization for the coeff_abs_level_remaining syntax element. The number of prefix bits is restricted depending upon MAX_TR_DYNAMIC_RANGE and a truncated unary representation is used for the prefix. The suffix bits are suitably modified for the highest suffix. It is asserted that with this modification, the length of the coeff_abs_level_remaining syntax in the worst-case is limited to 32. The impact on BD-rate performance is reported to be negligible (less than 0.02%) for AHG18 common test conditions.

Identical with Q0073.

### Intra block copy (26)

JCTVC-Q0062 AhG5: On the displacement vector prediction scheme for Intra Block Copy [P. Onno, G. Laroche, T. Poirier, C. Gisquet (Canon)]

Discussed 2nd day (Fri) (JRO).

This contribution presents a modification of the displacement vector prediction scheme for the Intra Block Copy method. In the current design of IBC, the displacement vector predictor corresponds to the latest displacement vector used for the last CU coded with the IBC mode. In this contribution, the displacement vector predictor scheme is changed to take into account the new 2NxN, Nx2N and NxN IBC partitions adopted at the last meeting. In average, it is reported that the new proposed prediction method based on the three last displacement vectors gives -0.3% gain for the AI and -0.2% for the RA/LB configurations. It is also reported that the proposed scheme shows some gains for the Lossless case as well.

Instead of using the vector from last CU coded in IBC as predictor, it is proposed to use three candidates, and a syntax element which signals the selection.

Search is not modified relative to current RExt. The new syntax element is bypass coded, using 1 bit CW for the last CU, and 2 bit for the other two.

Gain is only achieved in screen content.

Some concerns are expressed whether the additional complexity (parsing, memory for storing two additional vectors) is justified by the relatively small coding improvement.

Q0134 and Q0114 are similar.

See further notes under Q0114.

JCTVC-Q0214 Cross-check of JCTVC-Q0062 [L. Zhu, J. Xu (Microsoft)] [late]

JCTVC-Q0134 Ping-pong block vector predictor for intra block copy [L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft)]

Discussed 2nd day (Fri) (JRO).

A "ping-pong" approach with an encoder-selectable predictor for block vector (BV) prediction in the intra block copy (BC) mode is described. This approach reportedly provides some coding gain (approximately 1.2% and 1.0% average benefit for the tested YUV and RGB screen content coding cases, and generally little impact on performance for other cases). The scheme is similar to that proposed previously in JCTVC-P0217, although it is reportedly improved by only applying the update of the two cached values at the CU level.

No benefit for camera captured content.

Two predictor candidates which are the last two PUs where IBC was used, provided that they are different. A kind of FIFO approach is used, replacing the oldest predictor in the buffer if the new IBC vector is new, i.e. coded with non-zero difference.

In the first version of the document, bypass coding of the flag is used. The second version (uploaded 03-28) uses context based coding, asserted to give 0.2-0.3% additional gain. Version 3 provides syntax.

Version 1 only provides results for all intra.

One expert points out that syntax-wise this would unify AMVP and IBC vector coding (provided that the order of syntax elements is changed relative to the syntax proposal in version 3).

Encoding time is increased to 107% in AI - this measurement is reliable according to the cross-check. Clarify how the search was modified and how that impacts the result.

Further discussed later – see further notes under Q0114.

JCTVC-Q0180 Crosscheck of ping-pong block vector predictor for intra block copy (JCTVC-Q0134) [P. Onno (Canon)] [late]

JCTVC-Q0246 Cross-check of JCTVC-Q0134-v4 Solution 3 on ping-pong block vector predictor for intra block copy [P. Lai, S. Liu (MediaTek)] [late] [miss]

JCTVC-Q0075 AHG5: Intra-block-copy in Non-4:4:4 Formats [K. Sharman, N. Saunders, J. Gamei (Sony)]

Discussed 2nd day (Fri) (JRO).

This contribution details two issues encountered when using intra-block-copy for non-2Nx2N PUs in non-4:4:4 chroma formats, both of which stem from the adopted practice of merging the PUs where the chroma PU dimensions would be smaller than 4x4. A rule is proposed to be added to HEVC Range Extensions that would prevent these issues from occurring and also reduce computational complexity in the decoder. The application of this rule is reported to have no effect on coding efficiency.

The proposed rule would impose encoder constraints.

For class F, an average bit rate increase of 0.1%-0.2% is observed.

By the last meeting, an adoption was made based on P0180 that merges chroma PBs in cases that would be smaller than 4x4. However, the current software implementation still would use 2x2 blocks in cases where clipping is performed to prevent an access beyond the CTU boundary. The specification text is also not correct in this regard.

Generally, this proposal was supported by several experts as fixing a known problem. Editor (D. Flynn) to check for consistency before it is adopted.

Further discussed Thu noon (GJS).

Decision: It was agreed to remove the clipping of the BV values and otherwise change the software to match the text and establish constraints in the text to prohibit out-of-range references. Further study was encouraged to determine whether some different approach is more appropriate in the longer term.

JCTVC-Q0210 Cross-check of JCTVC-Q0075 on Intra-block-copy in Non-4:4:4 Formats [C. Pang (Qualcomm)] [late]

JCTVC-Q0080 Block vector prediction for intra block copy [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

Discussed 2nd day (Fri) (JRO).

In the current HEVC Range Extensions Draft 6, the block vector (BV) predictor for each intra block copying (IBC) PU is set to the BV of the previous IBC PU in general. This leads to cases where the BV predictor comes from a block that is not adjacent to the current block, even if the adjacent block is IBC coded. For example, for an IBC CU with N×N partitioning, the BV predictor for its lower-left PU (3rd PU in coding order) comes from its upper-right PU (2nd PU in coding order). This contribution proposes modifications to the derivation of BV predictor, such that the aforementioned scenario is avoided. Experimental results reportedly show that the proposed method can achieve 0.1%, 0.1%, 0.2%, 0.1%, and 0.3% BD-rate savings respectively for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI Main-Tier condition. For RA Main-Tier, the BD-rate savings are 0.1%, 0.2%, 0.0%, 0.1%, and 0.2%. For LD Main-Tier, the BD-rate savings are 0.1%, -0.2%, 0.1%, 1.7%, and 0.3%. The test results under the test conditions of Joint Call for Proposals for coding of screen content are also provided in this contribution.

Benefit in terms of compression was negligible – no action.

JCTVC-Q0226 Cross-check of block vector prediction for intra block copy in JCTVC-Q0080 [W.-S. Kim (Qualcomm)] [late]

JCTVC-Q0082 Symmetric intra block copy [K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)]

Discussed 2nd day (Fri) (JRO).

In the current HEVC range extensions draft specification, intra block copy (IBC) was adopted to consider reduplicated patterns in a picture. Besides reduplication, symmetry is often observed in natural or screen-content pictures. Symmetric intra block copy is proposed to consider symmetric patterns in a picture by flipping the reference block. Experimental results reportedly show that the proposed method can achieve 0.8%, 0.9%, 0.9%, 1.2%, and 2.6% BD-rate reductions respectively for Class F, RGB SC, YUV SC, optional RGB SC, and optional YUV SC sequences in AI Main-Tier configurations under the common test condition for HEVC range extensions. The proposed method can also reportedly achieve 1.8%, 0.7%, 1.1%, 0.4%, 2.0%, 0.9%, 1.1%, and 0.4% BD-rate savings respectively for RGB, text & graphics with motion, 1080p / RGB, text & graphics with motion,720p / RGB, mixed content, 1440p/ RGB, mixed content, 1080p/ YUV, text & graphics with motion, 1080p/ YUV, text & graphics with motion,720p/ YUV, mixed content, 1440p/ YUV, mixed content, 1080p classes in AI configurations under the test condition of the call for proposals for coding of screen content.

Horizontal/vertical flipping is dependent on vectors

Question is raised what the gain would be when only horizontal flipping is used

Not for RExt – further study in CE of IBC for screen content.

JCTVC-Q0202 Cross-check for Symmetric intra block copy (JCTVC-Q0082) [?? (??)] [late]

JCTVC-Q0230 Cross-check of symmetric intra block copy (JCTVC-Q0082) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-Q0095 AHG8: Coding the prediction differences of the intra BC vectors [S.-T. Hsiang, S. Lei (MediaTek)]

Discussed 2nd day (Fri) (JRO).

This contribution develops a new method for coding the block vector prediction difference associated with the Intra block copy (BC) coding mode. The proposed method re-uses the existing binarization scheme employed for coding the motion vector difference in the current HEVC. However, an improved context modeling scheme has been developed for entropy coding the resulting bin string. It is reported to achieve 1.9%, 1.4%, and 1.6% Luma BD-rate savings for YCbCr 4:4:4 SC sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively, of the AHG8 CTCs.

The proposal increases the number of contexts to 15 (currently it is 2).

Number of context coded bins is also increased.

Would give up the unified MV coding between AMVP in inter and the IBC. This is undesirable.

Not for RExt – not clear if it is still give gain in combination with other extensions of screen content.

No action at this point.

The contribution also includes a second proposal to limit the range of BVD components to [-127,127] to enable storage in 1 byte. However, due to some other restrictions in the spec, it is believed de facto the value range is anyway restricted to even smaller values.

Further discussed Mon 31st (JRO) after offline clarification with editors.

The currently specified constraint is [-128, 128], which is not correct. The maximum range of horizontal differences could be [-176..+184]. The specification of these constraints would however be redundant with the already existing constraints on maximum vector ranges.

Decision (Ed.): The constraint should be removed (action for editors, depending on question of IBC inclusion in RExt).

- JCTVC-Q0231 Crosscheck of JCTVC-Q0095 on Coding the prediction differences of the intra BC vectors [C. Pang (Qualcomm)] [late]

JCTVC-Q0179 AHG8: Crosscheck of Coding the prediction differences of the intra BC vectors (JCTVC-Q0095) [P. Onno (Canon)] [late]

JCTVC-Q0114 Block vector predictor for Intra block copy [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

In this document, block vector prediction method for Intra block copy is proposed. In the proposed method, either spatial neighboring block vector or default vector is used as block vector predictor. Reported average BD-rates are −1.4%, −1.0% and −0.7% for YUV 4:4:4 SC sequences in AI MT, RA MT and LB MT, respectively.

In the current design, the BV predictor is the previously reconstructed block vector in decoding order from within the current block. This contribution modifies the current BV prediction, by allowing an encoder to choose which of its two neighbouring predictors (from within the CTU) are used. A syntax element is added to signal this selection.

This contribution is similar in some respects to the proposal Q0134, but requires a line buffer within the CTU to allow selection between left and top predictor.

It was suggested that previously the committee was not looking at changes that did not offer gains in the RExt content. This proposal does not offer any gains in non-SCC content.

It was noted that the supplied specification text is not of sufficient quality, only presenting a syntax element without any decoding process (or updates to context tables).

Further discussion Monday 31st evening (chaired by JRO)

Proponents of Q0062, Q0114, Q0134 will prepare a report about the average performance with same data sets (excluding optional).

Note: Generally, the gains by these improved prediction methods appear relatively low compared to the gains that are reported by extended compensation ranges. It is not clear whether the benefit would still be existing with ranges larger than current and left CTU.

Further investigation of the three methods in IBC experiment (SCC).

JCTVC-Q0207 Crosscheck of JCTVC-Q0114 on Block vector prediction method for Intra block copy [J. Xu (Sony)] [late]

JCTVC-Q0127 Simplification on block vector prediction for intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

In this contribution, the block vector prediction for PU based intra block copy (IntraBC) is proposed to change from using the block vector (BV) of last coded IntraBC PU to using the BV of last coded IntraBC CU. The proposed change can remove the dependency of BV derivation between PUs when one CU consists of multiple PUs. The simulation results show coding performance changes relative to RExt-6.0 anchor are in the range of -0.1% (gain) to 0.2% (loss) for mandatory sequences, and -0.6% (gain) to 0.2% (loss) for optional sequences.

This contribution modifies the intraBC BV to predict all PUs within a CU from the last PU of the previous CU. The contributor asserts that there is a reduction in the critical path for the serial computation of the final BVs within a CU. However, this path may not be very significant.

No support was expressed for this change.

JCTVC-Q0172 Cross-check of JCTVC-Q0127 [J. Xu (Microsoft)] [late]

JCTVC-Q0132 On unification of intra block copy and inter-picture motion compensation [X. Xu, S. Liu, S. Lei (MediaTek)]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

In this contribution, the intra block copy operation (IntraBC) for a prediction unit is modified from its current form (in RExt6) to align with a motion compensated PU in inter slice. AMVP is used for PU level BV prediction and inter PU syntax is used for IntraBC signaling. The simulation results using AMVP for IntraBC reported average -0.3%, 0.0% and -0.3% BD-rate change (YCbCr444SC main-tier) compared with RExt6 anchor. The simulation results using inter PU signaling reported average 1.7%, 1.8% and -2.1% BD-rate change (YCbCr444SC main-tier) compared with RExt6 anchor. The IntraBC related syntax is simplified.

This contribution attempts to unify the intraBC BV prediction to utilise the existing inter signalling methods. It is provided in two stages:

    - Using AMVP gives a 0.3%.

    - Using Inter's prediction_unit() syntax, skip/merge. This has a ~2% AI loss, most likely due to the inability to use 4x4 in this mode.

NB, use of AMVP will require extra line buffers

NB, this implementation didn't use the merge mode

NB, AMVP wasn't tested in this configuration

It was noted that the provided specification text inadiquately describes the method.

Due to the above, it would be inappropriate to adopt this at this stage of the project (FDIS/FDAM)

JCTVC-Q0237 Crosscheck of JCTVC-Q0132 on AMVP for BV prediction and PU syntax for IntraBC [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU)] [late]

JCTVC-Q0135 AMP for the Intra BC prediction [L. Zhu, J. Xu, Y. Wu, G. J. Sullivan, S. Sankuratri, B. A. Kumar (Microsoft)]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

This document reports the results of applying asymmetric motion partitioning (AMP) for Intra block copy (BC) prediction. The addition of the AMP mode is asserted to improve harmonization of the Intra BC design with what is used for inter-picture prediction, and it is noted that since AMP uses only two partitions instead of four, it may have lower decoding complexity than the NxN case. Two sets of results with different motion estimation algorithms to speed up AMP in the Intra BC mode are provided. The first one uses the existing fast motion estimation for the Intra BC mode. The second one uses a modified hybrid motion estimation for the Intra BC mode.

This contribution does not include any specification text (it is effectively similar to the AMVP text from above).

Disabled for 8x8 CUs

The gains are small compared to encoder only modifications.

It was suggested that we shouldn't be looking at coding efficiency proposals for RExt at this point in time. This proposal doesn't seem to simplify anything.

"Not just about coding efficiency, but alignment to intra, and encoder complexity. Somehow it is more aligned".

No action.

JCTVC-Q0184 Cross-check of JCTVC-Q0135 AMP for Intra BC prediction [P. Lai, X. Xu, S. Liu (MediaTek)] [late]

JCTVC-Q0139 Intra block copy with larger search region [C. Pang, J. Sole, T. Hsieh, M. Karczewicz (Qualcomm)]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

This contribution provides results of the objective compression performance for Intra block copy with a larger search region than 64 columns on the left of the current CTU. When the search region is the entire frame, BD-rate improvements of 19.1%, 16.5% and 14.8% are reported for YUV 4:4:4 SC sequences in AI-MT, RA-MT and LB-MT, respectively.

Three options using a hash-based search for 8x8:

    - Full frame, ~19% AI Luma

    - Full frame, but remove an inter reference frame

    - 3x5 CTU, ~12% AI Luma

4x4 blocks are not being searched within the extended area.

Contributor suggests that the memory bandwidth is still less than inter.

(In the follow-up discussion, doubt was raised about this, and it might require more analysis. May also depend on implementation specific memory/cache structures.)

Extending the area requires writing out an aditional copy of the picture (prior to deblocking) if the internal cache is not large enough to store the additional area.

How would this compare to using a different search using say two CTUs to the left and up to four rows of the above CTU.

It was suggested from some private testing, that extending the search area to 1x1, 2x2,...5x5 offers gains initially.

Question: how does affect just doing this on an I-frame? and keeping the current limit for B as it currently is.

Comment: how does this relate to encode

Follow-up discussion on Mon. 31st evening (chaired by JRO).

Further results were presented where for the case of inter coded pictures the IBC range was restricted to the current and left CTU. This reduces the performance in RA conf. for 4:4:4 SC RGB from approximately 16% to 10% bit rate reduction.

Further consideration in the context of SCC experiments, not a candidate for RExt.

- What are the different ranges of IBC to be tested

- What is the range to be defined for “comparison reference” – 2 CTU or larger? This shall be brought to JCT plenary

- Aspects of memory bandwidth & Memory/cache size need to be analysed as well

Follow-up discussion in JCT-VC plenary Tue 8:00. Possible approaches:

- RExt 6. This would eventually have the disadvantage that other tools (in other experiments) show benefit that they would no longer show when IBC allows larger displacement. A possible option to resolve this would be to test other tools in combination with two IBC configurations (restricted and full frame)

- Use full frame IBC as reference

- Use something in between. This would have different options: Only CTUs from left plus 4 lines above, which would be asserted as hardware friendly, but likely not give similar compression benefit; something like 3x3, 3x4, 3x5, which would give already significant benefit.

- Decision is made to use full frame IBC as reference for comparison in all experiments. However, other experiments can show additionally that they provide benefit when the option of RExt6 (2 CTU) is only used.

BoG (R. Cohen):

- Define the reference model (SCM) which is an extension from RExt6.0 with additional non-normative tools for motion search, full frame IBC with an algorithm for encoder search, quantizer modification.

JCTVC-Q0140 AHG8: Performance of encoder and parameter only changes for Screen Content Coding [J. Sole, C. Pang, L. Zhang, K. Rapaka, M. Karczewicz (Qualcomm)] [late]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

This contribution provides results of the objective compression performance of some encoder and parameter modifications of HEVC RExt Profile for screen content coding. BD-rate improvements of 31.3% and 26.7% are reported for the RGB 4:4:4 SC and the YUV 4:4:4 SC classes.

1.	Search area for intra BC is increased with respect to the reference: full frame search and hash-table at the encoder side are used as described in Q0139.

2.	Transform skip size is increased with respect to the reference: a maximum transform-skip size of 8×8 is used (instead of 4). This had been reported in JCTVC-N0288, where a gain for SC sequences of 1.5% (RGB444) and 1.3% (YUV444) AI had been reported.

3.	Refinement inter-frame motion estimation for screen content coding as in [Q0147].

4.	Use of uniform quantization for RDPCM as in [Q0148].

5.	Refinement of intra block copying search by considering the chroma components as in [Q0175].

This document provides further information relating to the previous contribution Q0139.

3-5 have been adopted as non-normative tools in software (see under Q0147, Q0148, Q0175)

About 2: Transform skip 8x8 is reported to increase the encoder run time by approximately 15% (likely a problem with fast search of RQT). This would require some update of encoder software to justify the gain.

About1: See above under Q0139.

JCTVC-Q0220 Cross-check of 'Intra block copy with larger search region' (JCTVC-Q0139) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-Q0175 Intra block copy with encoder search using chroma component [C. Pang, J. Sole, M. Karczewicz (Qualcomm)] [late]

(Chaired by D. Flynn and C. Rosewarne Sun 30th p.m. as part of a RExt BoG)

This contribution proposes an encoder search method for Intra block copy. In the proposed method, both luma and chroma components are used in the encoder search for Intra block copy in a final step. Reported average BD-rates are -1.3%, -0.8% and -0.9% for YUV 4:4:4 SC sequences in AI-MT, RA-MT and LB-MT, respectively.

(Further discussion chaired by JRO Mon 31st evening)

Less gain for lossless.

This could be beneficial in SCC development when comparing the restricted (2 CTU) search range versus extended search range. Several expressed however the opinion that when the search range is extended, testing only 4 candidates may not be sufficient.

Decision (SW): Adopt in RExt SW with option to enable/disable.

- JCTVC-Q0221 Cross-check of 'Intra block copy with encoder search using chroma component' (JCTVC-Q0175) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

### Other (6)

JCTVC-Q0070 Consistent usage of intra boundary filter disabling [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

Discussed 2nd day (Fri) (JRO).

In the current HEVC range extensions, the horizontal and vertical gradient filters are disabled when implicit_rdpcm_enabled_flag and cu_transquant_bypass_flag are both equal to 1, from JCTVC-O0147. In addition to the gradient filters, the DC filter is another boundary filter to smooth the intra block boundary. However, the variable disableIntraBoundaryFilter only controls the gradient filters. Therefore, this contribution firstly proposes method 1 to consistently utilize of boundary-filter disabling in lossless coding by disabling and enabling the gradient filters and DC filter together. Considering the video characteristics that some videos do not need to smooth the boundary in intra prediction, in method 2, the boundary filters are also suggested to be optionally disabled for lossy coding. Experimental results reportedly show the consistent usage of intra boundary filters (method 1) does not change the bit-rate noticeably and can achieve up to 0.2% bit-savings for AHG8 lossless coding conditions.

Method 1 (lossless): No obvious benefit, no action.

Method 2 (lossy): Only beneficial (bit rate reduction) for screen content, bit rate for natural video is slightly increased. No action.

JCTVC-Q0201 Cross-check for Consistent usage of intra boundary filter disabling (JCTVC-Q0070) [E. Alshina, A. Alshin (Samsung)] [late]

JCTVC-Q0128 Fix for Strong Intra Smoothing in RExt [J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony)]

Discussed 2nd day (Fri) (JRO).

This contribution identifies the mismatch between RExt text draft and RExt software on Strong Intra Smoothing. Solution is proposed to resolve this issue. Simulation results demonstrate that proposed solution can maintain coding efficiency of current reference software.

Results show that allowing SIS for chroma when chroma format is 4:4:4 will degrade the coding performance of current reference software under common test conditions.

During the RExt development, the application of SIS to chroma components had been included in the text, but it was never implemented in software. This contribution shows that the usage of SIS for chroma is not beneficial.

Decision (BF): Adopt. Change the RExt spec. text such that it is aligned with the software.

JCTVC-Q0209 Cross-check of JCTVC-Q0128 on Fix for Strong Intra Smoothing in Rext [C. Pang (Qualcomm)] [late]

JCTVC-Q0148 Quantization rounding for RDPCM [F. Zou (Qualcomm)]

Discussed 2nd day (Fri) (JRO).

In this proposal, a uniform quantization is proposed for residue differential pulse code modulation (RDPCM) blocks. The proposed scheme is implemented on HM-13.0+RExt-6.0, and the simulation results demonstrate the proposed simple encoder change result in 0.2%, 0.4%, 0.8% BD-rate savings on average for AI, RA and LB main tiers respectively, up to 3.6% BD-savings for the RGB 4:4:4 SC LB high tier.

Encoder only change (replacing the 1/3 dead zone quantization in RDPCM by uniform rounding quantization).

Decision (SW): Adopt this non-normative proposal.

JCTVC-Q0181 Crosscheck of quantization rounding for RDPCM (JCTVC-Q0148) [C. Gisquet (Canon)] [late]

## SHVC (20)

### General (2)

JCTVC-Q0130 AHG12: Mismatch of SHVC draft 5 and SHM-5.0 software in bit-depth scalability with 1x scalability [C. Auyeung (Sony)]

Discussed 2nd day (Fri) p.m. (GJS).

This contribution reports that when bit-depth scalability is enabled with 1x scalability, SHM-5.0 software does not match the description of SHVC working draft 5. This contribution proposes two alternate fixes to the SHM-5.0 software for bit-depth and 1x scalability. Both bug fixes resulted in the same luma BDR of AI_1x and RA_1x of −28% and −17.9%, respectively, with the SCE1 BT.709 8 bits and 10 bits test sequences as inputs.

It was reported that a software change had already been made to make the software conform to the text.

JCTVC-Q0198 Cross-check for bit-depth conversion bug fix in SHM reference s/w (JCTVC-Q0130) [E. Alshina, A. Alshin (Samsung)] [late]

### SCE1 related (colour gamut and bit depth scalability) (7)

JCTVC-Q0129 Non-SCE1: Reduction of the number of colour space regions in SCE1 test 1.1 [C. Auyeung, K. Sato (Sony)]

Discussed 2nd day (Fri) p.m. (GJS).

In SCE1 test 1.1, Nokia proposed to divide the YUV colour space uniformly into NYxNCbxNCr=8x8x8 regions, and for each region, use a YUV matrix based mapping to derive the mapped pixel values. Nokia reported that the luma BDR of AI_1x, AI_2x, RA_1x, and RA_2x were −8.1%, −8.5%, −9.7%, −6.4% respectively. This contribution proposes to reduce the number of regions by 75% by dividing the regions non-uniformly (roughly logarithmically in the Cb and Cr domain) into 8x4x4 regions to reduce the memory requirements and the computation for regression analysis. This reportedly results in corresponding BD BR impacts of −8.6%, −8.8%, −9.9%, −6.5%. In addition, if the colour space is divided into the proposed 4x4x4 non-uniform regions, the BD BR became −9.1%, −8.8%, −9.5%, −5.8% correspondingly.

The reduced sizes are still larger than the 2.x designs tested in the CE. An extra stage of table lookup is needed to identify the partition in which the pixel lies.

The resulting table size is still larger than in the 2.x designs, and the scheme, as tested, is being applied after upsampling rather than before upsampling.

However, the non-uniform partitioning concept could, in principle, also be applied to the 2.x designs – e.g. with a table size not larger than in 2.x or performing the mapping prior to upsampling.

Further study (possible CE – proponent was asked to check during meeting whether it seeems to help or not).

JCTVC-Q0199 Non-SCE1: Cross-check for reduction of the number of colour space regions in SCE1 test 1.1 (JCTVC-Q0129) [E. Alshina, A. Alshin (Samsung)] [late]

JCTVC-Q0043 Non-SCE1: Denoising of inter-layer reference [E. Alshina, A. Alshin (Samsung)]

Discussed 2nd day (Fri) p.m. (GJS).

This contribution presents test results for denoising of inter-layer reference frame for colour-gamut scalability. Denoising is achieved by filtering "zero-phase" positions by 2D separable FIR filter consistent with SHVC re-sampling filter. Two variants are tested: non-switchable and picture based on/off denoising of inter-layer reference. Under SCE1 test conditions algorithm shows in average 1.5% (×1) and 0.3% (×2) BD-BR gain for non-switchable version and 1.6% (×1) and 0.6% (×2) BD-BR gain for a non-switchable version. It is reportedly demonstrated that gain is additive with performance improvement provided by other tools for colour-gamut scalability in SCE1 and non SCE1 contributions.

The proponent suggested that this may not be necessary for a generic profile for which inter-layer processing is not otherwise needed in the SNR case, but should be considered when bit depth or colour adjustment operations are being applied for inter-layer referencing.

It was remarked that denoising as a preprocessing stage is also a possibility.

It was remarked that the gain would be larger if low-delay P testing was included, as that case showed the highest gain for the concept when previously proposed (e.g., JCTVC-M0273, Incheon, KR, 18–26 Apr. 2013).

Considering the late stage of development of SHVC, the possibility of prefiltering, the limited amount of gain measured, and the limited prior interest when the technique was previously proposed, no action was taken on this.

JCTVC-Q0056 Non-SCE1: Encoder improvements for weighted prediction [A. Aminlou, K. Ugur (Nokia), E. Alshina (Samsung)]

Discussed 2nd day (Fri) p.m. (GJS).

This contribution proposes several encoder improvements on weighted prediction so that it suits better to colour gamut scalability. The encoder classifies pixels of inter-layer reference frame into two classes and finds separate weight and offset parameters for each class. Then, the inter-layer reference pictures are placed in both lists and different weight parameters are signalled for each inter-layer reference picture. Then encoder selects the best mode and reference picture for each block based on rate-distortion cost. The simulation results show that this technique brings average BD-rates of −1.6% (AI×1), −1.2% (AI×2), −2.1% (RA×1) and −1.1% (RA×2) under SCE1 test conditions.

This is an encoder-only compression technique modification.

More elaborate application of the same concepts (e.g., how to segment the colour regions and whether and how to apply bipred and how many inter-layer references to use) could provide further gain.

The concept could also potentially reduce the size of the tables needed in a combined approach or could potentially provide additional gain relative to a particular table size, since this could be applied

Further study was encouraged (possibly in a CE).

JCTVC-Q0227 Crosscheck report of JCTVC-Q0056 on encoder improvements for weighted prediction [K. Misra (Sharp)] [late]

JCTVC-Q0136 AHG14: On methods and test conditions for colour gamut scalability [K. Minoo, A. Luthra, D. Baylon (Arris)] [late]

Discussed 2nd day (Fri) p.m. (GJS).

This contribution proposes the following modifications to the test conditions of future core experiments regarding colour gamut scalability:

- Include test sequences for which the colour grading is performed at the lowest spatial resolution.

- Include a test condition where the base layer is in the same colour gamut as enhanced layer.

In discussion, the second point was highlighted as bringing up some interesting implications and opportunities for optimizations (e.g., non-normative encoding techniques).

Regarding the proposed methods in SCE1, this contribution asserts that none of the proposed methods captures the local nature of colour transform function for a class of use cases in which tone-mapping and colour grading are performed based on local characteristics of the scene. To qualify a proposed scheme for such use cases, it is also proposed to:

- Include test sequences representing global and local tone mapping and colour grading scenarios.

The contributor remarked that removing the ability to send colour adjustment information at a slice level may preclude the ability to use this functionality for local region adaptivity.

JCTVC-Q0141 Non-SCE1: Coding of colour gamut prediction coefficients [K. Misra, S.-H. Kim, A. Segall (Sharp)]

Discussed 2nd day (Fri) p.m. (GJS).

This contribution proposes an alternate technique for coding of colour gamut prediction coefficients presented in JCTVC-Q0072 (category 1.1 of SCE1). The purpose of the contribution is to propose improved coding performance relative to the prior proposal.

The performance of the proposed approach was evaluated with SCE1 test conditions as specified in JCTVC-P1101. The average luma BD BR improvement of the proposed approach is reported to be:

- Configuration 1, JCTVC-Q0072: 9.0% (All intra 1x), 9.1% (All intra 2x), 9.8% (Random access 1x) and 6.5% (Random access 2x)

- Configuration 3, JCTVC-Q0072: 5.7% (All intra 1x), 5.5% (All intra 2x), 6.9% (Random access 1x) and 4.3% (Random access 2x).

Relative to the proposal JCTVC-Q0072, the reported benefit was ranging from 0.1–0.9%.

### Upsampling process, phase offset (9)

See notes on BoG report JCTVC-Q0234 for consideration of contributions in this area.

- JCTVC-Q0234 BoG report on SHVC upsampling process and phase offset [J. Boyce]

The BoG met on 3rd meeting day (29 March). The outcome was discussed in JCT-VC later on 29 March and in the morning of 30 March (GJS).

The BoG recommended that the following activities take place:

- Discussion within the track of whether the phase adjustment should use explicit signalling of phases vs. the current method which derives phases based upon flags for interlace indication and cross-layer phase position? Mixed opinions were expressed in review. It was noted that addressing the needs for the next topic listed below may affect the desired outcome on this.

- Side activity among proponents to consider a general solution for explicit signaling of related parameters to address multiple issues, including 540p->720p, ROI scalability, field-to-frame, and other phase adjustment issues, and further discussion to be held during meeting. The 540p->720p case could be handled by using the conformance cropping window to derive the scaling factor.

The BoG planned to meet again (Tues) to further discuss the following

- Any new contributions based upon side activity discussion

- Phase adjustment issues.

- Improving coding efficiency for 540p -> 720p scalability (since if this is treated with an inaccurate scaling ratio, the coding efficiency loss reported in Q0104 is substantial – e.g., 9%).

A subsequent review of (-v3) BoG results was conducted on Thurdsay at 1900.

The BoG recommended adoption of the following:

- JCTVC-Q0200, use the cropping window parameters of the reference layer in the calculation of ScalingFactor and the starting phase position.

- JCTVC-Q0120, introduce a calculated delta (which is calculated once per picture) for use in the reference sample location calculation.

The following was recorded fro the review of the BoG report:

- Q0254 seemed rather complicated.

- Q0203 was a much lateer submission than would ordinarily be indicated by its document number, as a previous document registration was re-used for a new proposal in response to activity during the meeting. (This practice is highly discouraged.) It seemed more consistent with the direction given to the BoG to study general-purpose signalling. It includes both a method of evaluating technology and a specific technical approach with formulas for a general calculation scheme. However, it was not fully mature for action. Further study of both the evaluation methodology and the technical proposal in Q0203was encouraged.

- In the meantime, some small changes to the existing design were suggested for current adoption, and the JCT-VC endorsed these suggested changes.

  - Q0200 was also a re-used document registration number. It was nevertheless recommended and agreed for adoption.

  - Q0120 improves precision. It was also recommended and agreed for adoption.

The BoG also recommended, and the JCT-VC endorsed, the following plans and topics for further study activity:

- To study in AHG13, including several (e.g., 3) conference calls, a generalized phase signaling solution, taking into consideration JCTVC-Q0203 and JCTVC-Q0254. At least 7 days notice was agreed for announcing the date & time of these calls, and the scheduling would be conducted to apropriately balance the time zone requirements of the various participants.

- JCTVC-Q0203 proposes using 4 syntax elements per picture to signal general-purpose phase (X and Y, luma and chroma), and an evaluation methodology for accuracy of re-sampling. The design goals are consistent with the direction given to the BoG to study general-purpose phase signaling, but it was available too late this meeting for action.

- JCTVC-Q0254 was co-authored by 5 proponent companies and considers several use cases, and proposes that the 3 current flags be replaced by 9 syntax elements and formulas to utilize those syntax elements (scaled_ref_layer_frac_factor_minus1, scaled_ref_layer_left_phase_offset, scaled_ref_layer_top_phase_offset, scaled_ref_layer_horizontal_chroma_position, scaled_ref_layer_vertical_chroma_position, ref_layer_left_phase_offset, ref_layer_top_phase_offset, ref_layer_horizontal_chroma_position, ref_layer_vertical_chroma_position), with possible addition of 4 more syntax elements for more flexible ROI scalability (ref_layer_left_offset, ref_layer_top_offset, ref_layer_right_offset, ref_layer_bottom_offset).

JCTVC-Q0107 SHVC HLS: On picture level resampling phase filter selection [Y. Ye, Y. He, Y. He (Interdigital)]

JCTVC-Q0119 Increased resolution for scaled reference layer offset [K. Minoo, D. Baylon, A. Luthra (Arris)] [late]

JCTVC-Q0120 Re-sampling using existing phase offset flag signaling [K. Minoo, D. Baylon (Arris)] [late]

JCTVC-Q0216 Cross-check for modified phase offset calculation in JCTVC-Q0120 [J. Chen (Qualcomm)] [late]

JCTVC-Q0122 Increased precision for deriving the reference layer sample locations [K. Minoo, D. Baylon (Arris)] [late]

JCTVC-Q0197 Cross-check for increased precision for deriving the reference layer sample location [E. Alshina (Samsung)] [late]

Crosscheck of Q0122; includes a counter-proposal. However, the counter-proposal aspect was withdrawn in a revised version of the contribution.

JCTVC-Q0159 AHG 13: Sub-region extraction – position calculation and comparison of different approaches [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

JCTVC-Q0168 On phase offset for resampling process in SHVC [J. Chen, K. Rapaka, M. Karczewicz (Qualcomm)]

JCTVC-Q0187 Resampling of reference layer frames in interlaced-to-progressive scalability [M. M. Hannuksela, K. Ugur (Nokia)] [late]

JCTVC-Q0104 Fix for the computation of scaling factors used in inter-layer prediction [K. Andersson, J. Samuelsson (Ericsson)]

JCTVC-Q0215 Cross-check for refined scaling factor calculation in JCTVC-Q0104 [J. Chen (Qualcomm)] [late]

- JCTVC-Q0254 Report of side activity on reference position derivation [K. Minoo, Y. Ye, J. Chen, T. Yamamoto, K. Andersson]

(Although registered as a "report", this contribution appeared to be a technical proposal submitted by a group of proponents rather than a BoG report.)

- JCTVC-Q0200 Re-sampling process modification considering cropping information [E. Alshina (Samsung), J. Chen (Qualcomm) ] [late]

Note: The document number was initially registered as cross-check on something, and later re-allocated to a different topic.

JCTVC-Q0203 AhG13: Methodology for evaluation an accuracy of re-sampling process [E. Alshina (Samsung)]

## Screen content coding (37)

### Palette mode (20)

JCTVC-Q0047 AHG10: Simplification of Palette Based Coding [W. Pu, R. Joshi, M. Karczewicz, F. Zou, J. Sole (Qualcomm)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0178 AHG10: Crosscheck of Simplification of Palette Based Coding (JCTVC-Q0047) [C. Gisquet (Canon)] [late]

JCTVC-Q0063 AhG10: Palette predictor stuffing [C. Gisquet, G. Laroche, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0150 Crosscheck AhG10: Palette predictor stuffing (JCTVC-Q0063) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0064 AhG10: Palette index coding [C. Gisquet, G. Laroche, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0151 Crosscheck AhG10: Palette index coding (JCTVC-Q0064) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0065 AhG10: Transition copy mode for Palette mode [C. Gisquet, G. Laroche, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0152 Crosscheck AhG10: Transition copy mode for palette mode (JCTVC-Q0065) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0066 AhG10: Run coding for palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0153 Crosscheck AhG10: Run coding for palette mode (JCTVC-Q0066) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0069 AhG10: Combination of Palette coding tools [C. Gisquet, G. Laroche, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0224 Crosscheck AhG10: Combination of Palette coding tools (JCTVC-Q0069) [W. Pu, R. Joshi (Qualcomm)] [late]

JCTVC-Q0071 AHG10: Improvements on palette coding [D. Bugdayci, J. Lainema, K. Ugur, M. M. Hannuksela (Nokia)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0083 AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198 [Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0205 AHG10: Cross-check report for JCTVC-Q0083 X. Guo (Microsoft) [late]

JCTVC-Q0094 Suggested software for the AHG10 Palette Based Coding based RExt6.0 [W. Pu (Qualcomm), X. Guo (Microsoft), P. Onno (Canon), P. Lai (MediaTek), J. Xu (Sony)] [late]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

This was a porting of P0303 AHG10 software from RExt 5.1 to RExt 6.0 basis.

For further consideration in the context of potential future palette work.

JCTVC-Q0169 AHG10: Adaptive Scan Order on Palette Based Coding [J. Zhu, J. Ye, K. Kazui (Fujitsu)

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0138 AHG10: Line-matching above copy method [Y.-J. Chang, C.-L. Lin, C.-H. Hung (ITRI), C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0218 Crosscheck of JCTVC-Q0138 on line-matching above copy method [Y.-C. Sun, Y.-W. Huang (MediaTek)] [late]

JCTVC-Q0174 AHG10: Modified copy above mode for palette based coding [J. Zhu, J. Ye, K. Kazui (Fujitsu)] [late]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0225 Improvements to palette based coding [W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)] [late]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future palette work.

JCTVC-Q0240 Crosscheck of JCTVC-Q0225 on AHG10: Improvements to Palette Based Coding [J. Xu (Sony)] [late]

### Other (17)

JCTVC-Q0068 AhG8: On Sample Adaptive Band Value [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

For further consideration in the context of potential future SAO work.

This contribution brought up an issue of how to compare filtering techniques (both for SAO and deblocking – somewhat previously discussed) where the filtering affects subsequent mode decisions in the prediction loop and it becomes hard to separate filtering effects from the perturbation of the mode decision process.

JCTVC-Q0208 Crosscheck of JCTVC-Q0068 on AHG8: On Sample Adaptive Band Value [J. Xu, A. Tabatabai (Sony)] [late]

JCTVC-Q0077 AHG8: Effectiveness of the HM Inter Search for Screen Content [K. Sharman, N. Saunders, J. Gamei (Sony)]

Discussed 2nd day (Fri) (JRO).

This contribution examines the reasons for the effectiveness of intra block-copy when using inter-predictive coding, noting that intra block-copy is effective for source material with a high degree of spatial repetitiveness, such as screen content. It is asserted that the current HM inter-prediction search is poorly suited for predicting such content and that substantial improvement may be made purely by changing the search algorithm.

It is recommended that the inter search algorithm be considered before accepting any further new tools based solely on their inter performance, and that some test material be re-evaluated for its suitability for testing new tools.

It is also pointed out that some of the screen content sequences (e.g. web browsing, ppt) seem to be very easy to compress.

No specific proposal on alternative motion estimation was provided. Adoption of Q0147 seems to be an appropriate start to solve the problem.

JCTVC-Q0093 Single colour intra mode for screen content coding [Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

Was included in a SCC CfP response.

For further consideration in the context of potential future SCC (possibly palette-related) work.

JCTVC-Q0171 Cross-check of JCTVC-Q0093 [J. Xu (Microsoft)] [late]

JCTVC-Q0112 In-loop colour-space transform [L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

Includes report of gain for in-loop versus out-of-loop operation of the colour transform.

For further consideration in the context of potential SCC work.

JCTVC-Q0124 Improvements on 1D dictionary coding [J. Ye, S. Liu, S. Lei (MediaTek), X. Chen, L. Zhao, T. Lin (Tongji)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

Was not in the organization's formal CfP response.

For further consideration in the context of potential SCC 1D dictionary work.

JCTVC-Q0125 Crosscheck for JCTVC-Q0124 Improvements on 1D dictionary coding mode [R. Cohen (MERL)] [late]

JCTVC-Q0149 1D Dictionary Coding [F. Zou (Qualcomm)]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

The technique also part of a formal CfP response proposal (Q0031) when that proposal was operating in lossless mode. (Not cross-checked.)

For further consideration in the context of potential SCC 1D dictionary work.

JCTVC-Q0137 AHG 8: Adaptive Scaling of SAO offset [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)] [late]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236. Further discussed in review of the BoG on 3rd day (GJS).

Not screen content specific – more related to RExt as it is, and its design consistency.

RExt, as it is, limits this technique to higher than 10 bit depth operation.

Some participants questioned the value due to the small reported gain and the need to continue to support the prior type of operation for version 1. No visual improvement was reported. Not a definite problem identified. No action.

JCTVC-Q0217 Crosscheck of JCTVC-Q0137 on adaptive scaling of SAO offsets [X. Zhang, Y.-W. Huang (MediaTek)] [late]

JCTVC-Q0147 AHG8: On fast inter search method for screen content coding [K. Rapaka, J. Sole, L. Zhang, M. Karczewicz (Qualcomm)]

Discussed 2nd day (Fri) (JRO).

This contribution proposes a fast inter search method for screen content coding to better capture the spatial and temporal characteristics exhibited by screen content sequences like sharp edges, high spatial frequencies and non-monotonic error surface. It is asserted that the current HM’s inter prediction search is poorly suited for predicting such content and that substantial improvement may be made purely by changing the search algorithm. The following three modifications to inter search are proposed a) Multi stage approximated error cost computation with early exit at each stage b) Modified initial search method with a fixed diamond search traversing uniformly across the initial search space c) Modified early skip detection based on residual signals merge MV only. The proposed modifications are encoder only and do not affect decoder operation. For the lossy coding, the proposed inter search under SCC CfP test conditions show BD-rate gain of 8% and 9.2% for 1080p text and graphic sequences under random access and low delay configurations respectively.

Results are on CFP test conditions and AHG8 test conditions. This indicates that different strategies of motion search may be necessary for screen content than for natural video. Since the gains of approximately 3% average over all sequences are significant, this should be carefully considered when assessing coding tools for screen content.

Decision (SW): Adopt as non-default option to next version of RExt software.

JCTVC-Q0241 Cross-check of JCTVC-Q0147 "On fast inter search method for screen content coding" [K. Sharman, N. Saunders, J. Gamei (Sony)] [late]

JCTVC-Q0176 AHG8: String match in coding of screen content [W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu (Huawei)] [late]

Reviewed in BoG Q0236 on 3rd day (RC). See notes on BoG report JCTVC-Q0236.

2D string match of palette colour indices. (Not used in CfP response.)

## HL syntax common issues for range extensions, 3D, SHVC, and single-layer HEVC coding (7)

### Auxiliary pictures (3)

JCTVC-Q0081 RExt/SHVC HLS (AHG5 and 9): On reuse of alpha planes in auxiliary pictures [M. Naccari, M. Mrak (BBC)]

Discussed Sun 30th (GJS).

Alpha channel signals are supported in HEVC using auxiliary pictures, and their associated information is conveyed using the SEI message proposed in JCTVC-P0123. The syntax supports several functionalities such as values for transparent and opaque samples, whether pre-multiplied alpha planes are received etc. The proposal discusses the case when not all primary pictures have an associated channel. The contribution proposes to extend the syntax of the alpha_channel_info SEI message by adding a flag specifying the persistence of alpha planes within the coded video sequence.

It was asked why there would not be a presumption that the alpha channel content would simply persist, without needing the flag to indicate it. Decision (Ed.): Delegated to editors to clarify, as necessary, that the alpha channel content persists until cancelled or updated in output order.

It was asked whether it is really necessary to require a primary picture in the access unit for every alpha picture – e.g., to change the transparency of a picture without changing its texture content. Decision: Remove the constraint that an alpha picture must be accompanied by a primary picture.

JCTVC-Q0161 Layer association of auxiliary pictures [T. Ikai (Sharp)]

See BoG Q0223 and related notes.

JCTVC-Q0162 SHVC/MV-HEVC HLS: Scaled offset information of auxiliary pictures [T. Ikai (Sharp)]

See BoG Q0223 and related notes.

### Other (4)

JCTVC-Q0078 MV-HEVC/SHVC HLS: On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers [M. M. Hannuksela (Nokia)]

Initially discussed in BoG Q0223 (JB), then discussed 30 March in JCT-VC (GJS).

The proposal includes the following aspects:

- Additional layer sets can be specified in VPS extension in two value ranges of layer set indices:

  - A first range of indices for additional layer sets when an external base layer is in use. The sub-bitstream extraction process is appended so that it can be applied to additional layer sets where an external base layer is in use.

  - A second range of indices for additional independent layer sets (which may be converted to a new bitstream). These additional layer sets are not required to generate conforming bitstreams with the sub-bitstream extraction process of clause 10.

A loop of hrd_parameters( ) syntax structures may be provided for the additional layer sets where an external base layer is in use.

- The syntax of SPS and PPS with nuh_layer_id greater than 0 is modified so that the SPS and PPS syntax of HEVC version 1 can be used also when nuh_layer_id is that of the independent non-base layers. This is asserted to make it possible to rewrite independent non-base layers to standalone bitstreams by changing the nuh_layer_id of SPSs and PPSs for the independent non-base layers.

- A non-base layer subtree extraction process is specified for converting one or more non-base layer subtrees to a standalone conforming bitstream.

  - Profile-tier-level combinations specified for additional independent layer sets apply to the output bitstream of the non-base layer subtree extraction process:

  - Profile-tier-level combinations indicated in the VPS for the output layer sets of additional layer sets apply to the output bitstream of the non-base layer subtree extraction process.

- The profile-tier-level combination indicated in the SPS for a non-base independent layer applies to the base layer of an output bitstream of the non-base layer subtree extraction process.

- It is allowed to include auxiliary picture layers for output layer sets that are indicated to be conforming to the MV-HEVC and SHVC profile specifications (currently Stereo Main, Scalable Main, Scalable Main 10), but auxiliary picture layers are not taken into account in the profile and level constraints.

- Additionally, complementary aspects numbered 1, 2, 3a to 3d, and 4 are proposed in Section 2.7 of the contribution.

In discussion, a participant remarked that we could just require the decoding capability of auxiliary pictures to be within the scope of a profile/level constraint of some defined profile, rather than treating them as extra data that is not required to be decoded.

For further study in AHG.

JCTVC-Q0106 RExt HLS: on lossless coding [X. Xiu, Y. He, Y. Ye (Interdigital)]

Discussed 30 March in JCT-VC (GJS).

Range Extensions draft 6 uses cu_transquant_bypass_flag to indicate the use of transquant bypass coding at the CU level. This contribution proposes to be able to indicate at a higher level that all CUs use this coding type, and to skip the associated lower-level syntax when this is indicated. It also proposes to establish some other syntax constraints to prohibit "useless" syntax when this is indicated (e.g., deblocking control parameters).

There is essentially no coding efficiency benefit for this.

It was asked whether it seems appropriate to expect lossless coding to be dominant enough in usage to establish a special way to indicate it at a high level.

One participant expressed support for this – e.g., with monochrome coding.

Another participant suggested that if we want such a thing, it could be a constraint indicator– e.g., to define a special profile that would not be required to obey bit rate constraints. Using an SEI message was also mentioned by another participant as a possibility.

It was suggested that further study may be beneficial – e.g., in relation to future work on SCC.

JCTVC-Q0115 REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

JCTVC-Q0116 RExt HLS: Picture referencing across CRA pictures [R. Sjöberg, J. Samuelsson, Y. Wang (Ericsson)]

See Q0223 BoG report and related notes.

## HL syntax in SHVC and 3D extensions (33)

### General HLS issues (7)

JCTVC-Q0223 BoG report on High Level Syntax [J. Boyce]

The BoG met 27, 28, and 29 March, and results were reviewed by JCT-VC on 30 March (GJS).

Decision: The BoG recommended, and the JCT-VC endorsed, the following actions:

- JCTVC-Q0108

  - Proposal #1: Remove restriction on alignment of TSA and STSA with dependent layers

  - Proposal #2: Allow STSA pics of non-base layers to have TemporalId equal to 0

- JCTVC-Q0158 Proposal #2: Change range of values for HighestTid in Annex C to reduce number of encoder conformance tests

- JCTVC-Q0146

  - Proposal 1.1: Change inference rule in semantics of poc_msb_val_present_flag

  - Proposal 1.4, Infer a value of 0 when poc_reset_info_present_flag is not present

- JCTVC-Q0060 Proposal 2: Add a condition to the derivation of refLayerPicIdc of (TemporalId = = 0)

- JCTVC-Q0165

  - Proposal 1.a), add a constraint to disallow an empty output layer set

  - Proposal 2. change condition for presence of num_add_output_layer_sets to avoid sending it when there is only one layer set.

- JCTVC-Q0195

  - Proposal 1: restructure syntax table for sending of rep_format_idx_present_flag and vps_num_ref_formats_minus1

  - Proposal 2: add a semantic constraint on the value of update_ref_format_flag (no need for update if there is only one possible value)

- JCTVC-Q0177

  - Proposal 1: correction of inference rule for sps_temporal_id_nesting_flag when it is not present

  - Proposal 2: clarification of description of end of sequence NAL unit

- JCTVC-Q0183, numerous small clean-ups on SEI messages (with further text review)

- JCTVC-Q0078, Items 3 b, 3 c and 4, clarifying which pictures in an output layer sets are applied the values of general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag and general_frame_only_constraint_flag.

Decision (Ed.): The BoG suggested, and the JCT-VC endorsed, delegating the following to the editors:

- Several editorial issues regarding HRD operation from JCTVC-Q0055. The editors were asked to review the text modifications with the contribution author.

- Editorial improvements are needed for the association of NAL units within an access unit and access unit boundary detection. Language in JCTVC-0091 may be useful to study.

- JCTVC-Q0146, proposal 1.2, consider adding a NOTE and check if usage of HandleCraAsBlaFlag and EOS behavior is clearly described, and proposal 1.3 specification bug fix

- JCTVC-Q0054 proposal #2, bug fix in the semantics of vps_profile_present_flag[ ]

- JCTVC-Q0116, consider adding a note regarding how random accessibility is affected by the recovery point SEI message

The BoG recommended the following activities to take place:

- To discuss JCTVC-Q0166 in track, "On scaled reference layer offset"

A high-level question raised by this is whether to allow changes of the region of the reference layer that is enhanced by the EL – in regard to both the positioning and the resampling ratio.

It was remarked that the per-picture adjustment flexibility needed for the interlace phase adjustment already imposes some of the same kind of requirement on the decoder as this flexibility would.

It was agreed in principle that it would be desirable to enable this functionality – although details need to be worked out and examined for suitability.

See also Q0119

See further notes below.

- To discuss JVTVC-Q0170 in track, "Resampling need for a scalable layer"

This contribution proposes a two-bit indicator per layer to indicate whether resampling or position adjustment is needed or not for decoding an EL. An alternative indicator could be provided as scalability mask information. It was remarked that bit depth difference is also not indicated. No action was taken on this.

- To discuss JCTVC-Q0090 in track, "Redundant picture SEI message"

Further study was encouraged on this topic, but no immediate action was planned. This would be for consideration beyond the scope of the current phase of work.

- To discuss JCTVC-Q0105 in track, "On temporal enhancement layers and diagonal inter-layer prediction". See notes on that contribution.

- To discuss aspects of JCTVC-Q0078 in track, "On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers". See notes on that contribution. See additional notes below.

- To discuss JCTVC-Q0115 in track, "On indication of decoding process". This was related to Q0078. See additional notes on JCTVC-Q0078; further study planned.

Some additional items remained to be addressed or were identified as out of scope for this BoG.

- Items that could be covered if further discussion would be held in BoG: Q0161, Q0162, Q0164, Q0167. These topics were considered in further discussions held later in the BoG.

- Other items: Q0081 (see notes below), Q0106 (see notes below), Q0041 (discussed in AHG 15 prior to meeting; result incorporated into AHG proposed text), Q0042 (discussed in AHG 15 prior to meeting; result incorporated into AHG proposed text), Q0188 (see notes below), Q0110 (further study in AHG later planned), Q0084 (see notes below), Q0183 (see notes above and in discussion of Q0078), Q0045 (see notes below), Q0096 (see notes below), Q0074 (see notes below), Q0086 (see notes below).

See additional notes immediately below.

Discussion of HLS BoG JCTVC-Q0223 in JCT-VC Wed p.m. (GJS & JB)

Decision: Remove DPB sharing and processes that mark inter-layer reference pictures as "unused for reference".

Y.-K. W. volunteered to arrange software for sub-bitstream extraction.

Should we normatively define a rewriting process? It was remarked that this is not something that affects decoders – it is purely a hypothetical process to specify conformance. Decision: Yes.

Should auxiliary pictures be required to conform to some defined profiles/level. Decision: Yes, it needs to be possible to indicate what are the characteristics of these pictures – what syntax they conform to, what bit rates, etc.

Q0078 proposes that the profile indication in the SPS of an aux pic is the profile that would apply after rewriting. Decision: Agreed.

Should we define a Monochrome 8 bit profile, so that monochrome (e.g. alpha) auxiliary pictures have something to conform to? Decision: Agreed.

On output layer sets:

- if there is an output layer set that doesn't include the base layer, the bitstream is rewritten and the same output layer set is required to be present for the rewritten bitstream, which contains a base layer. Decision: Agreed.

- if an aux picture is part of an output layer set but is not part of a target output layer, the buffer flow includes the aux picture but the decoding process requirements do not. Decision: Agreed.

- For currently specified profiles, if the target output layers include aux pics, the decoding process requirements include decoding the aux pics – however, we would not allow that in currently-specified profiles? See additional notes below; further study planned in AHG.

Association between auxiliary and primary pictures:

- Can an aux pic be associated with more than one primary? Yes.

- Vice versa? Yes.

- How does a decoder know which aux picture(s) to use? That's subject to definition by SEI or external means.

- Should there be a default relationship? For further study.

See additional notes immediately below.

Joint JCT-VC and JCT-3V discussion of HLS BoG report Q0223 Thu 1700 (GJS)

The "-v7" version of the BoG report was presented to a joint session of JCT-VC and JCT-3V.

This joint session began with rapid review of the items previously reviewed in JCT-VC.

The reported aspects were endorsed except as otherwise noted.

The hybrid scalability text in the AHG15 report was approved.

Sub-bitstream extraction capability will be added to reference software.

A normative rewriting process is to be defined, so that independent non-base layers can be rewritten to be decodable by non-layered profiles (i.e.,. Main, Main 10).

Regarding Auxiliary picture concepts:

- The profile indication in the SPS of an aux pic is the profile that would apply after rewriting.

- When an output layer set that doesn't include the base layer, the bitstream is rewritten and the same output layer set is required to be present for the rewritten bitstream, which contains a base layer

- When an aux picture is part of an output layer set but is not part of a target output layer, the buffer flow includes the aux picture but the decoding process requirements do not.

- An auxiliary picture can be associated with more than one primary picture.

- More than one auxiliary picture of a particular type may be associated with the same primary picture.

Regarding whether a separate profile is needed for the case where the base layer is provided by external means, there was discussion about that. Further study was encouraged.

There should be a constraint on the size of the pictures provided by external means and of their representation format.

For MV-HEVC, don't remove the restriction to prohibit the external base layer (at least for now). Otherwise, the MV-HEVC draft will include the same specification for hybrid operation as for SHVC.

The constraint specification and capability indication AHG and HLS AHG should be jointly conducted or at least closely coordinated between JCT-VC and JCT-3V, as much of this work is common for the two layered coding extensions.

Regarding sub-DPB sharing, some participants indicated that they thought the identified problem was solvable and asked for that topic to be reopened. The suggested potential savings was up to one reference picture per enhancement layer (under certain conditions). A new contribution Q0255 had been provided to address the topic. Due to the late stage and not necessarily large potential savings in typical use, the decision reached earlier in the meeting was not changed.

Regarding the Q0078 requirement to send an additional VPS in an SEI message (to be used by the rewritten bitstream). It was agreed that the output bitstream of the rewriting process would be allowed to produce a bitstream that is otherwise conforming but does not contain a VPS for the case of single-layer rewritten bitstreams. Further study was encouraged to determine whether this solution is fully adequate. To clarify the decision on not including requirement for decoding process for aux pictures, it is only an editorial matter whether something is described in the body and prohibited in all profiles, or prohibited in general – the latter is probably better on this topic (until a future version needs to allow it in some profile).

Regarding the Q0247 field/frame indication proposal, it was agreed to adopt the SEI message but not the extra SPS flags. Further study was encouraged to determine whether the solution is fully adequate.

Q0166 considers per-picture spatial offsets, proposing to remove the spatial offsets from the VPS and put them in the SPS and SH. (It was in the SPS and SH also in SVC.) This was agreed to be for further study including in AHG 13.

JCTVC-Q0055 MV-HEVC/SHVC HLS: Comments on general decoding process and selection of CPB operation in the HRD operation [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

See Q0223 BoG report and related notes.

JCTVC-Q0091 MV-HEVC/SHVC HLS: On access unit definition and allowing different decoding orders in different layer trees [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

JCTVC-Q0105 MV-HEVC/SHVC HLS: On temporal enhancement layers and diagonal inter-layer prediction [M. M. Hannuksela (Nokia)]

First discussed in BoG Q0223 (JB).

Then discussed in JCT-VC (GJS) 30 March a.m.

This contribution asserts that diagonal inter-layer prediction would be useful in the following use cases:

- An enhancement layer provides a temporal enhancement, possibly along with spatial or quality enhancement, relative to the base layer, where the picture rate ratio is non-dyadic, e.g. a 24-Hz base layer, a 30-Hz first enhancement layer, and a 120-Hz second enhancement layer.

In discussion, there was some questioning of what should be done for this use case for professionally produced content, as it was suggested that exposure times should be different for presentation at different frame rates, so that even when there is temporal aligment between BL and EL pictures, some coded difference may be needed in the EL to prevent temporal artefacts. The display of very sharp pictures at very low frame rate is undesirable, as is the display of blurry pictures at high frame rate.

- An SHVC-coded temporal enhancement layer is provided for an AVC or HEVC base layer.

See the comments above regarding potential temporal artefacts also for this case.

- Interlaced-to-progressive scalability, when the base layer contains coded frames. This case would require more than just diagonal referencing. This seemed to bring up larger issues for which additional study would be needed, probably beyond the scope of the current SHVC phase of work.

It was mentioned during discussion that ARC is another potential case. However, VUI flags have been drafted to address that usage, and it was suggested that the switching points for that scenario are less frequent and therefore perhaps less in need of special handling.

It was proposed to enable the use of diagonal inter-layer reference pictures as summarized in the following:

- A gating flag in SPS multilayer extension would indicate whether diagonal inter-layer prediction is enabled in the slice header level.

- A short_term_ref_pic_set( ) syntax structure could be included in the slice header for a direct reference layer and would be referred to as the reference-layer short-term RPS. The reference-layer short-term RPS syntax structure would specify the pictures from the direct reference layer that are included in the initial reference picture list(s) of the current picture, but causes no change to the marking of the pictures.

- The decoding process for reference picture lists construction would be modified to include reference pictures from the reference-layer short-term RPS syntax structure for the current picture.

The following changes were made in revision 1 of the contribution:

- The short_term_ref_pic_set( ) syntax structure is used as requested in the first review in the of the contribution during the meeting (instead of using a specific reference-layer short-term RPS syntax structure).

- The use of diagonal inter-layer prediction with hybrid codec scalability was enabled.

It was proposed that, when the scheme is activated, a flag for each direct reference layer would be sent to indicate whether it is a diagonally referenced layer.

As proposed, only one reference layer could be used for "diagonal" referencing – although this was a design choice that would not be necessary to make the concept work.

In discussion, it was noted that the proposal would enable referencing multiple pictures from the reference layer rather than the current design in which only one picture from each reference layer can be referenced. The referenced pictures would not need to be "neighbouring" pictures either temporally or in terms of decoding order.

It was asked whether there is any anticipated impact on TMVP or scaling. The proponent indicated that the diagonally referenced pictures would be treated as long-term referenced pictures (for the purpose of decoding the current picture only). This aspect was suggested by the proponent to not be necessary and was identified as an open issue – the pictures in the other layer could instead be treated as ordinary short-term reference pictures. The safer option may be to treat them as long-term reference pictures.

The proposal was to also remove the constraint (under non-resampling operation) that the motion vectors used to reference the reference layer must be equal to 0. It was remarked that this could have complexity implications. The proponent indicated that limiting the motion vector values to 0 would somewhat hurt compression performance.

Without the scheme, similar functionality could be achieved by creating (possibly non-output) "skipped pictures" in the EL that correspond to the access units of the BL, but this could require a higher decoder level of capability.

It was agreed that we need to decide how important it is to have the zero MV value constraint.

It was noted that pictures that are not ordinarily used as reference pictures in the base layer would become reference pictures for the purposes of decoding the EL, and this would constrain the allowed position of the EL pictures in decoding order and the ability to reference certain pictures, as the EL must appear before the marking of any such RL picture as unused for reference in the reference layer.

It was remarked that the zero MV constraint is important for memory bandwidth requirements on the decoding process.

Unless this functionality is important enough to justify a profile that does not impose the zero-MV constraint, we can't adopt the proposed scheme. Thus, no action was taken on the proposal.

JCTVC-Q0108 MV-HEVC/SHVC HLS: On TSA and STSA pictures [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

JCTVC-Q0109 MV-HEVC/SHVC HLS: On TemporalId constraints [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

JCTVC-Q0158 MV-HEVC/SHVC HLS: On Highest TemporalId [S. Deshpande (Sharp)]

See Q0223 BoG report and related notes.

JCTVC-Q0163 MV-HEVC/SHVC HLS: On decoding non-output/non-reference layers [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

See Q0223 BoG report and related notes.

### IRAP alignment and POC derivation (2)

JCTVC-Q0057 MV-HEVC/SHVC HLS: Comments on IRAP alignments and POC value derivation [B. Choi, Y. Cho, M. W. Park, J. Y. Lee, S. Lee, C. Kim (Samsung)]

See Q0223 BoG. report and related notes

JCTVC-Q0146 MV-HEVC/SHVC HLS: On picture order count and related [A. K. Ramasubramonian, Hendry, Y.-K. Wang]

See Q0223 BoG report and related notes.

### RPS signalling and derivation (3)

JCTVC-Q0060 MV-HEVC/SHVC HLS : On inter-layer RPS signalling and derivation [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

See Q0223 BoG report and related notes.

JCTVC-Q0079 MV-HEVC/SHVC HLS: max_tid_il_ref_pics_plus1 in inter-layer RPS syntax and semantics [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

JCTVC-Q0100 MV-HEVC/SHVC HLS: Constraints for Reference Picture Set Parameters [S. Deshpande (Sharp)]

See Q0223 BoG report and related notes.

### HLS for hybrid scalability (3)

General

Text was proposed with the AHG 15 report

The following needs for further work were identified by the AHG:

- Editors' notes are included in the accompanying draft text (prefixed by the string "[Ed.") to identify open issues for further study.

- Further work is needed for reference software development to fully support the proposed scheme. A basic form of external means for providing a base layer is supported in the software; however, the software assumes that there is always a base layer picture for each access unit. Support for the planned signalling and syntax needs to be implemented.

Decision: The proposed text was endorsed, with non-editorial open issues considered as follows:

- Regarding whether, in F.7.3, to send sub_layers_vps_max_minus1[ i ] and max_tid_il_ref_pics_plus1[ i ][ j ] for the externally-provided base layer, it was agreed that these should be sent. M. M. Hannuksela agreed to provide the semantics.

- Regarding the question of "Further study whether all the six specified types of IRAP NAL unit types should be defined instead of just three, and whether codec specific definitions, e.g. when the base layer is AVC, are needed, such as defined in P0184v2_attachment", no action appeared necessary.

- Would the addition of the support of external-means-provided base layer affect the level definitions? This aspect to be resolved by further study in AHG, including review of Q0145.

- It was suggested that we may not necessarily define separate profiles for support of external-means-provided base layer, since we do not have separate profiles for other type of external means signalling. If this is agreed, then the addition of the hybrid profiles should be removed and the added restriction on vps_base_layer_internal_flag for the non-hybrid profiles should also be removed. Pending an identification of a need to do otherwise, we will assume there is no need to specify the hybrid case as a separate profile.

JCTVC-Q0041 AHG 15: Comments on Hybrid Scalability [S. Deshpande (Sharp)]

Discussed in AHG 15 prior to meeting; result incorporated into AHG proposed text.

JCTVC-Q0042 AHG 15: Support of hybrid scalability [Y.-K. Wang, J. Chen, Y. Chen, Hendry, A. K. Ramasubramonian (Qualcomm)]

Discussed in AHG 15 prior to meeting; result incorporated into AHG proposed text.

JCTVC-Q0188 Alternative AVC base layer HRD parameters for HEVC hybrid codec scalability [M. M. Hannuksela (Nokia)] [late]

Discussed 30 March (GJS).

Also submitted to parent bodies; submitted to JCT-VC for information.

It is asserted that the base layer (BL) hypothetical reference decoder (HRD) parameters are intended for single-layer use and might not suit the consumption of the base layer for hybrid codec scalability. It is proposed to initiate work to specify a mechanism to convey alternative HRD parameters for an AVC base layer, which suit hybrid codec scalability. As these HRD parameters concern the base layer, it is suggested to specify the parameters as part of the AVC specification.

The suggestion is to provide HRD parameters for which the pictures needed for reference by the EL are ensured to have been decoded prior to that use and are retained within the DPB until that use.

The contribution also suggests considering whether there is a need to synchronize the output of the BL as a single-layer output and the output of a scalable decoding process.

This contribution is intended for consideration by the parent bodies.

### Parameter sets (5)

JCTVC-Q0054 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung)]

See Q0223 BoG report and related notes.

JCTVC-Q0061 MV-HEVC/SHVC HLS: On max_tid_il_ref_pics_plus1 in the VSP extension [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

See Q0223 BoG report and related notes.

JCTVC-Q0110 Support for out-of-band signalling in VPS to enable future layer additions [S. Narasimhan, A. Luthra (Arris)]

See Q0223 BoG report and related notes.

JCTVC-Q0165 MV-HEVC/SHVC HLS: Clean up for output layer set [T. Tsukuba, T. Yamamoto, T. Ikai, S. Deshpande (Sharp)]

See Q0223 BoG report and related notes.

JCTVC-Q0211 MV-HEVC/SHVC HLS: On HighestTid and MaxSubLayersInLayerSetMinus1 [T. Ikai (Sharp)] [late]

See Q0223 BoG report and related notes.

### HRD related (5)

JCTVC-Q0101 MV-HEVC/SHVC HLS: On Bitstream Partition Buffer [S. Deshpande (Sharp)]

Discussed 31 March p.m. (GJS).

In this document various constraints are proposed for bitstream partition buffer related parameters. Additionally signalling modifications are proposed for bitstream partition buffer HRD parameters.

In r1 revision a variant bitstream conformance constraint is added for Sub-proposal 12 and Sub-proposal 13.

- Sub-proposal 1: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in VPS VUI, the signalled index of hrd_parameters( ) structure for a bitstream partition is within a valid range of hrd_parameters( ) structures signalled in the vps_vui_bsp_hrd_parameters() syntax structure.

- Sub-proposal 2: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in bitstream partition HRD parameters SEI message, the signalled index of hrd_parameters( ) structure for a bitstream partition is within a valid range of hrd_parameters() structures signalled in the bsp_hrd() SEI message structure.

- Sub-proposal 3: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in VPS VUI, the signalled index of a delivery schedule within hrd_parameters( ) structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

- Sub-proposal 4: A bitstream restriction which makes sure that when HRD parameters are signalled for bitstream partition in bitstream partition HRD parameters SEI message, the signalled index of a delivery schedule within hrd_parameters( ) structure is within a valid range of delivery schedules signalled in the sub_layer_hrd_parameters structure in that hrd_parameters() structure.

- Sub-proposal 5: A bitstream restriction which makes sure that when only one bitstream partition is signalled for a layer set in vps_vui_bsp_hrd_parameters( ) the bitstream partition does not exactly match the corresponding layer set.

- Sub-proposal 6: A bitstream restriction which makes sure that when only one bitstream partition is signalled for a layer set in bitstream partition HRD parameters SEI message the bitstream partition does not exactly match the corresponding layer set.

- Sub-proposal 7: Since there is a limited range of values that can be assigned to bsp_comb_hrd_idx[ h ][ i ][ j ]  syntax element in VPS VUI HRD parameters - vps_vui_bsp_hrd_parameters() syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- Sub-proposal 8: Since there is a limited range of values that can be assigned to sei_bsp_comb_hrd_idx[ lsIdx ][ i ][ j ] syntax element in bitstream partition HRD parameters SEI message syntax and that it is typically not likely that some index values are taken more often than others it is proposed to use u(v) coding for this syntax element instead of ue(v) coding currently used.

- Sub-proposal 9: A modification of the syntax and semantics for bitstream partition HRD parameters SEI message is proposed. Also a bitstream constraint related to hybrid scalability is proposed, consistent with agreements in AHG15 meetings, JCTVC-Q0015 AHG report and specification draft text output from AHG15.

- Sub-proposal 10: With the current signalling of bitstream partition specific HRD parameters in vps_vui_bsp_hrd_parameters( ), it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner. Two options are proposed to prevent including a layer in a layer set in multiple bitstream partitions.

- Sub-proposal 11: With the current signalling of bitstream partition specific HRD parameters in bitstream partition HRD parameters SEI message it is possible to include a layer in a layer set in multiple bitstream partitions. It is asserted that there may not be any benefit of doing partitioning in this manner. Two options are proposed to prevent including a layer in a layer set in multiple bitstream partitions.

- Sub-proposal 12: It is proposed that each bitstream partition of a layer set must be distinct. i.e. any two bitstream partitions of a layer set can not be identical. It is proposed that a bitstream partition signalled in VPS VUI bitstream partition HRD parameters for a layer set can not exactly match another bitstream partition for the same layer set.

It is noted that if Sub-proposal 10 is adopted then Sub-proposal 12 is not necessary.

- Sub-proposal 13: It is proposed that each bitstream partition of a layer set must be distinct. i.e. any two bitstream partitions of a layer set can not be identical. It is proposed that a bitstream partition signalled in bitstream partition HRD parameters SEI message for a layer set can not exactly match another bitstream partition for the same layer set.

It was noted that if sub-proposal 11 is adopted then sub-proposal 13 is not necessary.

In discussion of sub-proposal 5, it was noted that when the base layer is externally specified, the sub-bitstream extraction process cannot produce a conforming bitstream, and the text needs to allow that.

In discussion of sub-proposals 10 and 11, option 2 (a contraint-only approach) was preferred. Sub-proposals 12 and 13 therefore did not need to be considered.

Decision (BF/Cleanup): Adopt (sub-proposals 1–11, refined as described).

JCTVC-Q0182 MV-HEVC/SHVC: On bitstream partition buffering [M. M. Hannuksela, A. Hallapuro (Nokia)] [late]

Discussed 31 March p.m. (GJS).

The contribution proposes a number of items that are asserted to be fix bugs or editorial clarifications related to the indications for bitstream partition buffering.

- Sub-proposal 1

  - Problem: The sequence-level bitstream partition HRD parameters can be indicated through the vps_vui_bsp_hrd_parameters( ) syntax structure and through the bitstream partition HRD parameters SEI message. No constraints have been specified whether both can be present. Furthermore, when both are present, no constraints have been specified on the contents of these syntax structures, particularly if the contents have to be semantically identical.

  - Proposal: Allow the vps_vui_bsp_hrd_parameters( ) syntax structure and the bitstream partition HRD parameters SEI message to be present. Specify that when both structures are present, their contents shall be semantically identical. The following text is proposed: "When both the bitstream partition HRD parameters SEI message and the vps_vui_bsp_hrd_parameters( ) syntax structure in the active VPS are present, the contents of the SEI message shall be semantically identical to the contents of the vps_vui_bsp_hrd_parameters( ) syntax structure of the active VPS."

  - Alternative: It could be reconsidered whether both the VPS VUI and the SEI signalling are needed. In discussion, the alternative was not preferred.

- Sub-proposal 2

  - Problem: The syntax element initial_cpb_removal_delay_length_minus1 of the hrd_parameters( ) syntax structure is used to derive the length of the u(v)-coded syntax elements nal_initial_arrival_delay[ i ] and vcl_initial_arrival_delay[ i ] in the bitstream partition initial arrival time SEI message. The hrd_parameters( ) syntax structure can be included in the SPS VUI or in the VPS where it is associated with a layer set. Different SPSs may be active in different layers. It is ambiguous which SPS VUI to use or whether to use the VPS to obtain the correct hrd_parameters( ) syntax structure for concluding the lengths for these syntax elements.

  - Proposal: Add length signaling of the u(v)-coded syntax elements nal_initial_arrival_delay[ i ] and vcl_initial_arrival_delay[ i ] in the vps_vui_bsp_hrd_parameters( ) syntax structure and in the bitstream partition HRD parameters SEI message.

  - Alternative 1: Clarify that the VPS hrd_parameters( ) syntax structure that applies to the layer set which is associated with the bitstream partition initial arrival time SEI message is used to determine the lengths of the nal_initial_arrival_delay[ i ] and vcl_initial_arrival_delay[ i ] syntax elements. In discussion of sub-proposal 2, the "alternative 1" approach was preferred.

  - Alternative 2: An approach would be to require the initial_cpb_removal_delay_length_minus1 to the same in all hrd_parameters( ) syntax structures of the active SPSs and in the active VPS.

- Sub-proposal 3

  - Problem: The vps_vui_bsp_hrd_parameters( ) syntax structure includes the num_bsp_sched_combinations syntax element, which specifies the number of schedule combinations for a bitstream partition. Respectively, the bitstream partition HRD parameters SEI message includes the sei_num_bsp_sched_combinations_minus1 syntax element. The proponents could not see a reason to allow zero schedule combinations like the num_bsp_sched_combinations syntax element of the vps_vui_bsp_hrd_parameters( ) syntax structure allows.

  - Proposal: Change num_bsp_sched_combinations to num_bsp_sched_combinations_minus1 in the vps_vui_bsp_hrd_parameters( ) syntax structure.

- Sub-proposal 4 (editorial)

  - Layer set 0, which consists of the base layer only, cannot be partitioned. A clarification is proposed that it is disallowed to have a bitstream partition HRD parameters SEI message or a bitstream partition nesting SEI message being contained in a scalable nesting SEI message, which specifies the nested SEI messages to apply to the layer set 0.

- Sub-proposal 5 (editorial)

  - A clarification is proposed that, similarly to other HRD related SEI messages, also the bitstream partition initial arrival time SEI message is allowed to be conveyed through external means.

Decision (BF/Cleanup/Ed): Adopted (such that we use the main proposal for sub-proposal 1, and alternative 1 for sub-proposal 2).

JCTVC-Q0102 MV-HEVC/SHVC HLS: Comments on HEVC Extensions [S. Deshpande (Sharp)]

Discussed 31 March p.m. (GJS).

Only sub-proposals 1 and 2 belong to this category.

- Sub-proposal 1: A modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed.

- Sub-proposal 2: A semantics bug fix is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix. In discussion, the first option was preferred.

Decision (BF/Cleanup/Ed.): Adopt. (The need for sub-proposal 1 is conditioned on the specification of a shared sub-DPB.)

It was suggested that also the separate_colour_plane_flag should affect inference of NoOutputOfPriorPicsFlag. Decision (Ed.): Agreed (affects RExt text).

JCTVC-Q0154 MV-HEVC/SHVC HLS: On picture flushing and DPB parameters [A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm)] [late]

Discussed 31 March p.m. (GJS).

This document proposes a change to the picture flushing operation and the inference of DPB-related parameters. The behaviour of NoOutputOfPriorPicsFlag for controlling output of pictures is proposed to be made applicable across all layers, and removal of pictures is proposed to be decoupled from their output such that the removal is not controlled by NoOutputOfPriorPicsFlag. It is asserted that this design can avoid problems of exceeding the buffer size limit in certain bitstreams. The second change proposes to infer the value of DPB-related parameters in the VPS from the corresponding parameters in SPS for the 0-th output layer set.

Further discussed 2 April p.m. (GJS).

It was noted that this imposes some requirements on EL decoders that are not found in BL-only decoders.

Decision: Adopted (some details discussed further in BoG).

JCTVC-Q0157 MV-HEVC/SHVC HLS: On DPB - to share or not to share; that is the question [A. K. Ramasubramonian, Hendry, Y.-K. Wang, Y. Chen (Qualcomm)]

Discussed 30 March p.m. (GJS).

A sub-DPB in an SHVC/MV-HEVC decoder is shared by all layers that have the same representation format indicated in the VPS. This document asserts that sharing of sub-DPBs may result in decoded picture buffer overflow when bitstream down-switching or bitstream thinning (discarding of individual or a set of discardable pictures) occurs. Several possible solutions, with none asserted to be seemingly satisfying, are presented. Another discussion of the question on whether to share or not to share seems needed, and a final decision on this is suggested to be made at this meeting.

Basically, only a picture in a particular layer can mark previously decoded pictures in that same layer as unused for reference (except for sub-layer non-reference pictures not used for inter-layer prediction).

It was agreed that problem had been identified; side activity was ongoing to determine whether there is a reasonably simple and clean solution. Unless such a solution is found, we will simply prohibit the sharing.

JCTVC-Q0255 MV-HEVC/SHVC HLS: On sharing of sub-DPBs [A. K. Ramasubramonian, Hendry (Qualcomm), S. Deshpande (Sharp)] [late]

See joint VC/3V HLS meeting notes. Due to the late stage and not necessarily large potential savings in typical use, the decision reached earlier in the meeting to not allow sharing of sub-DPBs was not changed.

### Miscellaneous HLS topics (8)

JCTVC-Q0142 MV-HEVC/SHVC HLS: On extraction of independent non-base layer [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

See Q0223 BoG report and related notes.

JCTVC-Q0102 MV-HEVC/SHVC HLS: Comments on HEVC Extensions [S. Deshpande (Sharp)]

Only proposal 3 belongs to this category.

See Q0223 BoG report and related notes.

JCTVC-Q0160 SHVC/MV-HEVC HLS: On alternative output layer flag [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

See Q0223 BoG report and related notes.

JCTVC-Q0166 MV-HEVC/SHVC HLS: On scaled reference layer offset [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

See Q0223 BoG report and related notes.

JCTVC-Q0170 SHVC HLS: Resampling need for a scalable layer [K. Andersson, J. Samuelsson, R. Sjöberg, J. Ström]

See Q0223 BoG report and related notes.

JCTVC-Q0177 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [Hendry, A. K. Ramasubramonian, Y.-K. Wang, V. Seregin (Qualcomm)] [late]

See Q0223 BoG report and related notes.

JCTVC-Q0189 MV-HEVC/SHVC HLS: On slice_temporal_mvp_enabled_flag [M. M. Hannuksela (Nokia)] [late]

See Q0223 BoG report and related notes.

JCTVC-Q0195 MV-HEVC/SHVC HLS: On representation format signaling [S. Hattori, O. Nakagami, T. Suzuki (Sony)] [late]

See Q0223 BoG report and related notes.

## SEI and VUI (9)

### General (1)

JCTVC-Q0183 MV-HEVC/SHVC HLS: SEI message cleanups [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)] [late]

See Q0223 BoG report and related notes.

JCTVC-Q0247 MV-HEVC/SHVC HLS: On frame-field related indications (follow-up of parts of JCTVC-Q0183/JCT3V-H0082 and JCTVC-Q0078/JCT3V-H0026) [M. M. Hannuksela (Nokia)]

See Q0223 BoG report and related notes.

### Overlay-related SEI (2)

JCTVC-Q0045 MV-HEVC/SHVC HLS: Proposal for an SEI message on selectable overlays [N. Stefanoski, O. Wang, A. Smolic (DRZ), T. Szypulski (ESPN)]

Discussed 31 March p.m. (GJS).

In this document, an SEI message is proposed to realize the functionality of selectable overlays. selectable overlays is a functionality which enables viewers to turn on and off single overlay elements, like score bars, information panels, news tickers, etc. on their display device. An SEI message is proposed that describes overlays with three different categories of auxiliary pictures, representing overlay content, overlay label, and overlay alpha information. The syntax is identical to the second option that is proposed in JCTVC-Q0096. This proposal is based on the input documents JCTVC-P0092, JCTVC-P0135, and JCTVC-O0358 and intends to harmonize the designs of the SEI messages proposed in these documents.

The selectable overlays functionality was first proposed in JCTVC-O0358 at the 15th JCT-VC meeting (there this functionality was called optional overlays). Auxiliary picture types have been adopted for which the interpretation can be specified by an SEI message. It was proposed to use that approach for the selectable overlays functionality with help of auxiliary pictures. At the last JCT-VC meeting (16th JCT-VC-Meeting in San Jose), two proposals, i.e. JCTVC-P0092 and JCTVC-P0135, were presented that proposed the selectable overlays functionality using auxiliary pictures. When discussed previously, it was recorded that "At the concept level, it was agreed to plan to support such a capability. Further study was needed to work out details."

General question: What, in general, is the association between auxiliary pictures and primary pictures?

String coding using UTF-8 with null termination was suggested per the scalability information SEI message in AVC (H.263 refers to ISO/IEC 10646 for UTF-8, AVC has a similar reference).

It was noted that ISOBMFF has strings with a string type defined and could be studied as an example of how to convey and specify them.

It was remarked, and agreed, that this is not essential to delivering SVC or other major projects, and could be considered on a slower schedule if it does not mature sufficiently rapidly.

Roughly the same as proposed in Q0096.

Decision: Adopt (modified for variable-length strings).

JCTVC-Q0096 MV-HEVC/SHVC HLS: Overlay info SEI message [J. Boyce, S. Wenger (Vidyo)]

Discussed 31 March p.m. (GJS).

A new SEI message is proposed to support overlay pictures with individually controllable overlay elements using three categories of auxiliary pictures, representing overlay content, overlay label, and overlay alpha. The proposed SEI message describes the overlays, by indicating the layer_id values of the various aux pic layers, and providing overlay label mapping parameters. Two options are proposed regarding AuxId values. In the first option, a single AuxId value is proposed to be allocated to overlay pictures. In the second option, AuxId values from the already allocated “Unspecified” range are used.  The syntax in the second option is identical to that proposed in JCTVC-Q0045. Other related contributions are JCTVC-P0135, JCTVC-O0358, and JCTVC-P0092.

In the v2 version of this document, editorial improvements have been made to the semantics of overlay_content_layer_id[ i ].

The argument for allocating a special aux type code to be used with overlays would be to be able to identify at the VPS level (rather than from an SEI message) whether overlays are (or may be) present.

Closely related to JCTVC-Q0045, see notes on that contribution.

### Colour-related SEI and VUI (3)

JCTVC-Q0074 SEI message for Colour Mapping Information [P. Andrivon, P. Bordes, E. François (Technicolor)]

Discussed Tues pm (GJS).

The proposal is to provide information about how to convert the decoded pictures from the colour space in which the decoded pictures are represented to a different colour space – e.g., for use on an alternative display. For example, if the decoded pictures are in the Rec. BT.2020 colour space, the proposal is to provide guidance to perform a better mapping to a more limited colour space such as BT.709 – e.g., performing clipping.

The concept was previously discussed in JCTVC-N0180, JCTVC-O0363, JCTVC-P0126.

The model is asserted to be compatible with built-in functionality in CE devices.

It is stated that software is supplied to identify proposed model parameters. Implementation is provided in HM-13.0+RExt-6.0 encoders and decoders. When the proposed colour mapping information SEI message is present, colour mapping is applied to the decoded output pictures.

The proposal is asserted to be similar in concept (and in syntax and semantics) to the current tone mapping SEI message type 3 but extended to support one LUT per channel instead of just one LUT that applies to all three channels, and a matrix is also proposed for cross-component transformation. At the decoder side, the LUT would applied first, followed by the matrix (in the opposite order ordinarily used, e.g. for YUV to RGB to linear conversion).

At the previous meeting, some concerns were expressed about editorial quality and persistence aspects.

Quote from previous meeting: "Some additional study was requested by some participants, although there was some support for the proposal (at least in concept). No particular problems were identified with the proposal."

Q0183 was related regarding persistence and was asserted to have been taken into account.

It was remarked that the syntax elements pre_lut_num_pivots_minus1[ c ] and post_lut_num_pivots_minus1[ c ] were proposed to have a very large range of values. The proponent indicated that their testing was done with 33 pivot points. A suggestion was 64 * 2^(bit depth – 8).

The proposed syntax was somewhat similar to the prior tone mapping information SEI message. The distinction made here is that the proposed information is more about colour space transformation than about dynamic range transformation.

The proponent indicated that RP 177 is a relevant SMPTE recommendation, but the proponent indicated that its use would result in clipping.

Further discussed 2 April p.m. (GJS).

It was suggested to adopt this into the SHVC draft; however, a final review is anticipated to occur at the next meeting to determine whether it is fully mature or not at that time for final approval (or should wait for a next amendment/revision). Decision: Agreed.

JCTVC-Q0086 Mastering Display Colour Volume SEI [C. Fogg, J. Helman (Movielabs)]

Discussed Tues noon (GJS).

Proposes adding precision.

Decision: Adopted (RExt). (Confirmed 2 April p.m. GJS.)

Editorial issues:

- SMPTE 2086 is an FCD ST and should be referred to that way.

- Needs to mention that x, y is defined by ISO 11664-1

- Check boldface in syntax

- "quantized_CE_xy_coordinate" expression needs cleanup

- 216 should be 216.

JCTVC-Q0084 VUI entries for YZX and digital cinema EOTF [C. Fogg, J. Helman (Movielabs)]

Discussed Tues noon (GJS).

This contribution suggests changes to address feedback received since the previous meeting in San Jose (January 2014) on the XYZ colour description information in HEVC Annex E (video usability information, a.k.a. VUI). In the matrix_coeffs semantics, it is proposed that the YZX mapping from real values to integer follow convention for YCbCr and GBR signals for both video_full_range_flag=0 and video_full_range_flag=1. A transfer_characteristics code point for digital cinema EOTF (with gamma 2.6) is also requested.

Technical actions:

- Requests to add another EOTF (SMPTE ST 428-1:2006)

- SMPTE FCD ST 2084 is non-final, but seems to be past the stage of technical changes or significant risk of non-approval; refer to it as such (esp. since not a normative reference)

Editorial actions:

- Missing multiplication symbols, missing subscript formatting

- Missing bibliography entries

- 2085 should be 2084 in bibliography

- Disconnect of variable names

- Just use 0 for matrix coefficients if no matrix is needed (and editorially modify the description of what that means)

Decision: Adopted (RExt). (Confirmed 2 April p.m. GJS.)

### Other SEI (3)

JCTVC-Q0090 HEVC/MV-HEVC/SHVC HLS: Redundant picture SEI message [M. Sychev, V. Stepin, S. Ikonin (Huawei)]

Reviewed in BoG, see Q0223 and related notes.

JCTVC-Q0164 MV-HEVC/SHVC HLS: On sub-bitstream property SEI [T. Ikai, T. Yamamoto, T. Tsukuba (Sharp)]

Reviewed in BoG, see Q0223 and related notes.

JCTVC-Q0167 MV-HEVC/SHVC HLS: On layers not present SEI [T. Ikai (Sharp)]

Reviewed in BoG, see Q0223 and related notes.

- JCTVC-Q0253 MV-HEVC/SHVC HLS: On semantics of layers not present SEI message [A. K. Ramasubramonian (Qualcomm)] [late]

Reviewed in BoG, see Q0223 and related notes.

JCTVC-Q0117 Extension of the pic_struct element in HEVC [A. Tourapis, D. Singer (Apple), A. Duenas (NGCodec), G. Martin-Cocher (BlackBerry)] [late]

Discussed Thu 3 April (GJS).

This contribution updates a previous contribution JCTVC-P0261 proposing to extend the pic_struct element that is present in the picture timing SEI message of HEVC to handle some interlace content applications. In particular, this extension proposes to enable indication of a method of carriage of interlaced video material using a "progressive-like" carriage mechanism. A previous document P0082 discussed the same topic.

The contributor indicated that the document was essentially provided as information about how this could be done, without necessarily saying that the contributor themselves planned to do this or was aware that any significant community would be likely to want to be able to indicate such usage.

It was remarked that also the same indication could be provided by other means (e.g. a user data SEI message or system-level signal), and noted that only a few reserved values of pic_struct exist. Only one value would remain reserved for future use if this would be adopted (as-is).

However, another participant indicated that some previous system had used a similar indication, and another participant indicated that this could be used for "PSF" applications to avoid a one-field delay when sending PSF video.

It was suggested that a new SEI message may be a more appropriate way to indicate this than extending the capability of an existing SEI message. The functionality was agreed to be desirable. A single bit would appear to suffice for the indicator. The value of field_seq_flag would be required to be zero when the new SEI message is present.

Decision: Adopt as new 1-bit SEI message with field_seq_flag required to be zero (in SHVC, editor delegated to produce the exact text for the syntax consisting of a one-bit flag with the meaning proposed).

- JCTVC-Q0256 Rectangular Region Frame Packing Arrangement [G. Balocca (Sisvel)] [late]

Q0256 was a late contribution providing modified proposed text relative to the WG 11 IT NB ballot input, responding to comments made in joint review on Thu. It was reviewed in closing plenary on Fri. a.m. (GJS).

A participant questioned whether the suggested name was the best choice (although not preferring "FPA version 2") and suggested giving the editors the discretion to select an alternative name (with follow-up consultation with the proponent).

It was suggested to remove the reserved byte and type code and aspect ratio flag and shorten the content interpretation type to 2 bits with no intent to extend later.

A participant commented that if this new message is present, there should be some indication that the video should not be displayed in the normal fashion. One suggestion was to define a type code in the old SEI message that would indicate that some packing existed but was not defined in that SEI message. It was noted that, in the SPS, there is a general_nonpacked_constraint_flag syntax element that may be a way to address this issue.

It was questioned how to deal with the backward compatibility issue of decoders that may understand the existing FPA but not the new one. It was noted that the proposed text suggested for encoders to use the "default display window" to manage that case. It was suggested to mandate rather than advise that the "default display window", but the contributor suggested that the encoder may have valid reasons for choosing not to do that (e.g., not wanting to provide a lower-resolution 2D viewable picture as an acceptable alternative to 3D viewing). It was also noted that decoders may ignore the default display window.

It was remarked that the frame0_self_contained_flag was removed in the modified syntax, but would actually be useful.

It was suggested to clarify that both types of FPA should not be allowed in the same CVS. This was agreed.

It was suggested that it could be desirable to be able to cancel the new FPA using the cancellation signal specified by the old FPA SEI message.

It was suggested that document availability was somewhat unfortunate in this case; however, there was a need to proceed, and the text seemed generally adequate at this stage.

Decision: Include in RExt (prohibit both types in the same CVS, remove the proposed reserved byte and type code and aspect ratio flag and shortening the content interpretation type to 2 bits with no intent to extend later, using general_nonpacked_constraint_flag syntax element).

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (0)

See notes on JCTVC-Q0077 (effectiveness of the HM Inter Search for Screen Content) and JCTVC-Q0147 (fast inter search method for screen content coding) and JCTVC-Q0148 (RExt Quantization rounding for RDPCM) and JCTVC-Q0056 (weighted prediction parameter optimization).

# Plenary Discussions and BoG Reports

## Project development

Joint meetings with the VCEG and MPEG parent bodies were held at the following times:

- Mon. 31 March 1600–1800

- Tue. 1 April 1600–1800

- Thu. 3 April 1500–1630

Topics discussed and outcomes of these joint meetings are recorded as follows:

- For HEVC version 1, the following is reported:

  - The verification testing of HEVC version 1 had been completed with good results, and a report of the outcome would be issued by the JCT-VC and its parent bodies.

  - Regarding, version 1 defect correction, the previous plan to correct the defect in the MinCR specification for the current high tier was confirmed (a relevant parent-level contribution was m33346).

- Regarding the current range extensions work and the future screen content coding (SCC) extensions, various views were expressed in input contributions to the parent bodies on these projects and the appropriate relationship between them and the particular configurations of coding capabilities that should be specified in profiles and levels currently and in the near future (e.g., M32734 (summary of voting), M33347, M32712, JCTVC-Q0113 VCEG-AX12 / M32845, JCTVC-Q0133 / M32868, M33413, JCTVC-Q0051 / M32728, VCEG-AX14 / M33338, M33100, M33337, JCTVC-Q0212 / M33363, M32820, JCTVC-Q0186, M33048). The impact of this discussion on the planning for SCC and the range extensions is reflected below, especially in regard to the specification of a 4:4:4 8 b profile and the "intra block copy" coding feature.

- For the screen content coding (SCC) CfP outcome, the following is reported:

  - The input report (VCEG-AX13 / M33474) of the formal subjective assessment of the submissions in response to the CfP was noted, along with the objective measurements, coding tools analysis, and other information gathered from the study of the submissions themselves (JCTVC-Q0239).

  - It was agreed that the responses to the joint CfP indicated that the HEVC standard should be extended to include additional coding capabilities for SCC and that additional profiles should be developed to include such technology.

  - It is anticipated that such profiles will include typical HEVC capabilities for coding camera-captured content as well as additional SCC-oriented coding features

  - The technical work for development of such SCC extensions should be conducted in the JCT-VC.

  - The development timeline was agreed such that the completion of the standardization process for the SCC extensions was estimated to occur around the end of 2015 or beginning of 2016, with ISO/IEC CD ballot likely to be issued in February 2015.

- For the range extensions, the following is reported:

  - Decision: Issue a complete text containing range extensions and errata corrections (in integrated form) as an ISO/IEC FDIS as an output of the current meeting.

  - Decision: In consideration of new contributions for RExt and SCC indicating a potential lack of adequate maturity, some implementation difficulty, and a potental lack of adequate benefit on a sufficiently broad range of video content, the "intra block copy" coding feature will not be included in the RExt profile specification currently being finalized.

  - Decision: For 4:4:4 profiles:

    - Define 8b and 16 b profiles

    - Define corresponding simple subsets as still picture profiles

    - Remove the highest tier previously drafted for the 4:4:4 Intra 16 profile from the text to be produced

    - Define an all-intra 16 b profile with CABAC bypass alignment required to be enabled by encoders and with decoders not required to decode bitstreams in which CABAC bypass alignment is disabled. The maximum bit rates in each tier of the additional profile will be 12 times the bit rates in the corresponding tier of the 4:4:4 Intra 16 profile (and there will be two tiers, as in other profiles). The specification of this profile will be finalized at the meeting of July 2014 rather than at the current (Valencia, April 2014) meeting.

NOTE - Post-meeting observation: Since decoders for this profile would not be required to decode bitstreams from any previously specified profiles, some other features should probably also be given the same treatment as CABAC bypass alignment in this new profile. This would include the modified Rice parameter adaptation rule, high-precision transform processing, and features that could be characterized as bug-fixes for version 1, such as extended-precision weighted prediction, the context for coding the transform skip flag, the rotation of transform skip blocks, large block-size transform skip. It seems less clear what should be done regarding RDPCM and intra smoothing disabling, but suggested by editors GJS and DF that decoders should support both modes for these.

  - Decision: For all-intra profiles, the CPB size constraint will correspond to 0.5 seconds of the maximum bit rate.

  - For the all-intra profiles, in the parent-level input contribution M33048, it had been suggested to make the "in loop" filtering processes mandatory for these profiles. After discussion, it was agreed to keep these processes optional in such profiles (although the picture hash SEI message should assume these processes had been applied).

  - Decision: Regarding the proposed specification of an SEI message to signal a rectangular frame packing arrangement (e.g., JCTVC-Q0256 and IT NB input in parent-level contribution M32734), various aspects of the proposed text were discussed, and it was ultimately agreed to include specification of such an SEI message in the RExt extensions specification. More detailed notes on the subject are found in the section of this report concerning JCTVC-Q0256.

  - Regarding colour-related aspects for the RExt specification (e.g., SEI and VUI), the actions and plans recorded elsewhere in this report on those topics were reviewed and confirmed as appropriate.

- For the scalability extensions, the following is reported:

  - Various issues were discussed, particularly including JCT-VC and parent-level inputs M32735 (summary of voting), JCTVC-Q0188 / VCEG-AX11 / M33043, M33412, and plans and considerations relating to ROI scalability and the use of an interlaced base layer.

  - Parent-level input M33171 was noted in regard to a proposed requirement for chroma format scalability, and it was agreed that this should be considered a longer-term issue not for immediate action.

  - It was noted that a tentative conclusion had been reached in JCT-VC HLS study that the specification of a Monochrome 8 bit profile is needed for purposes of achieving adequate clarity of the specification of decoding requirement capabilities, so that monochrome (e.g. alpha) auxiliary pictures have something specified to conform to. This need was confirmed.

  - Regarding the 12 b profile proposed in Q0206 and WG 11 parent body document M33427, it was noted that 12 b operation has not been tested in JCT-VC. It was agreed that the subject should be further studied (in AHG 14) and a decision should be reached on the subject at the next meeting.

- For 3D extensions, various topics were discussed. Such topics not directly relevant to JCT-VC are recorded in the JCT-3V report.

- Regarding future work on HDR (high dynamic range) and WCG (wide colour gamut), e.g., the parent-level input contribution m32771, it was noted that MPEG is planning further study in this area and expected to determine in October 2014 or February 2015 whether significant new standardization work is needed for this.

- Regarding the study of interlaced video coding with HEVC and the potential for further enhancing the performance of HEVC on interlaced video content by making core technology modifications specifically to address needs for coding interlaced video (e.g., including parent-level inputs M33407 and M33476), viewing of video content was conducted by the MPEG Requirements subgroup, and it was determined that the evidence was insufficient to identify a need for new technology development (e.g., new profiles) for this purpose. However, it would be desirable to conduct verification testing to confirm the compression capability improvement that already exists for interlaced video content for use of HEVC versus its predecessors. It was remarked that some study in this area had already been done (e.g., by a prior "4EVER" project). A mandate was given for a JCT-VC ad hoc group to study what has already been done in this area and identify what further is needed for performing such a verification test of HEVC capability.

- Regarding still image coding using HEVC, the recent reports of the substantial effectiveness of HEVC for this purpose were noted (e.g., JCTVC-Q0229). Further study on the subject was noted to be planned – particularly in the MPEG parent body – and interest in the subject was also noted by VCEG.

## BoGs

JCTVC-Q0223 BoG report on High Level Syntax [J. Boyce]

See section 7.5.1.

- JCTVC-Q0234 BoG report on SHVC upsampling process and phase offset [J. Boyce]

See section 7.2.3.

JCTVC-Q0236 BoG report for Non-CfP SCC and related documents [R. Cohen]

See section 6.1.

JCTVC-Q0239 BoG report on summary of objective performance and tools for SCC CfP responses [R. Joshi, R. Cohen, H. Yu]

See section 6.1.

JCTVC-Q0244 BoG report on screen content coding reference model (SCM) [R. Cohen]

See section 6.1.

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These have traditionally fallen into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

The CE plan is to be considered final at the same time as corresponding software.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

No particular changes were noted w.r.t. prior CTC. The general agreed common conditions for single-layer coding efficiency experiments remained as described in the prior output document JCTVC-L1100.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- SHM 6.0, based on HM 14.0, should be available within 2 weeks after the meeting.

- HM 14.0+RExt7.0 should also be available within 2 weeks after the meeting.

- HM 14.0+RExt7.0+SCM 1.0 should be available within 2 weeks after HM 14.0+RExt7.0.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1002 HEVC Test Model 15 (HM 15) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 15.0 software version and the reference configuration encodings according to JCTVC-L1100 common conditions. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC v.1, RExt and SHVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye (vice-chairs) | N |
| Verification test preparation (AHG5) (jct-vc@lists.rwth-aachen.de) - Complete the HEVC verification test report. - Make preparations for verification testing of HEVC for interlaced video content. - Make preparations for verification testing of HEVC range extensions. | V. Baroncini (chair), M. Karczewicz, N. Ramzan, C. Rosewarne, K. Sharman, T. K. Tan, J.-M. Thiesse, W. Wan, J. Wen (vice-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1005 HEVC RExt draft text. - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary), D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan, T. Suzuki | N |
| Range extensions software and test model development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Prepare and deliver HM 14.0-RExt-7.0 software version and the reference configuration encodings according to JCTVC-P1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Update HEVC Range extensions test model 7 encoder description (Q1013). - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and the performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for testing screen content coding performance, in coordination with AHG16. - Make recommendations for test conditions for screen content coding. - Coordinate with SCCEs on combination experiments. - Finalize the common test conditions SCCEs. | R. Cohen and H. Yu (co-chairs), , A. Duenas, T. Lin, S. Liu, J. Sole, J. Xu, Y. Ye (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Study mechanisms to indicate decoding capabilities/requirements for auxiliary picture layers - Investigate mechanisms for picture referencing across CRA pictures - Study and compare mechanisms for enabling temporal enhancement layers and diagonal inter-layer prediction - Study the operation of the Hypothetical Reference Decoder for multi-layer operation. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Coordinate efforts with JCT-3V AHG6 on high-level syntax issues in relation to 3D extensions. | J. Boyce and M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) | N |
| Layered coding constraint specifications and capability indications (AHG10) (jct-vc@lists.rwth-aachen.de) - Study how constraints on bitstream syntax affect specifications of bitstream conformance and decoder conformance for layered coding. - Study indications of bitstream properties and decoding process requirements for layered coding. - Identify potential side-effects of constraint specifications and capability indications and recommend alternative approaches. - Identify potential needs for modifications and additional indications of constraints and decoding process requirements for layered coding. - Coordinate with AHG 9 on high-level syntax for constraint specifications and capability indications for layered coding. | G. Sullivan (chair) | Tel. |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1007 SHVC Test Model 6 (SHM 6) text. - Produce and finalize JCTVC-Q1008 SHVC text specification Draft 6. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 6.0 software (based on HM 14) for experimentation. - Provide software to CEs within two weeks after the meeting. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, (co-chairs) | N |
| SHVC inter-layer resampling process (AHG13) (jct-vc@lists.rwth-aachen.de) - Develop methodology for evaluating the accuracy of resampling processing - Study generic solutions for phase adjustment in resampling process - Study re-sampling process modification related to extracting regions of interest. - Discuss and identify additional issues related to resampling filtering. | E. Alshina (chair), J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (vice-chairs) | Tel. |
| Colour gamut and bit depth scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut and bit depth scalable coding. - Study the interaction of colour gamut scalability with bit-depth and spatial scalability. - Identify test sequences and test conditions, in coordination with AHG16. - Study the complexity of various methods for colour gamut scalable coding - Discuss and identify additional issues related to colour gamut scalability. | P. Andrivon, A. Duenas, (co-chairs), C. Auyeung, E. Alshina, S. Deshpande, X. Li, K. Ugur, Y. Ye (vice-chairs) | N |
| SCC extensions test model text editing (AHG15) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions test model 1.0 text. - Gather and address comments for refinement of the test model. - Coordinate with AHG [SCC extensions software development] to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, Z. Ma, Y. Ye | N |
| Test sequence material (AHG16) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for HEVC development. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 on range extensions development, AHG8 on screen content coding, and AHG14 on colour gamut scalability, AHG18 on high bit rate and high bit depth operation. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |
| SCC extensions software development (AHG17) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM SCM software and its distribution. - Prepare and deliver HM 14.0-SCM-1.0 software version and the reference configuration encodings according to JCTVC-Q1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG15 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-Q1000 Meeting Report of 17th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm] [2014-06-20] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-Q1002 High Efficiency Video Coding (HEVC) Test Model 15 (HM 15) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] [2014-06-20] (near next meeting)

JCTVC-Q1003 High Efficiency Video Coding (HEVC) Edition 1 Defect Report Draft 4 [Y.-K. Wang, G. J. Sullivan, B. Bross] [2014-05-30]

JCTVC-Q1004 HEVC Conformance Draft 7 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 14426 Preliminary draft of ISO/IEC FDIS) [2014-05-16]

JCTVC-Q1005 HEVC Range Extensions Draft Text 7 [D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. Sullivan, T. Suzuki] (WG 11 N 14414, ISO/IEC FDIS) [2014-05-30]

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman]

JCTVC-Q1007 SHVC Test Model 6 (SHM 6) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 14419) [2014-06-06]

JCTVC-Q1008 SHVC Draft Text 6 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang] (WG 11 N 14416 ISO/IEC Study of DAM3) [2014-06-06]

JCTVC-Q1009 Common SHM test conditions and software reference configurations [V. Seregin, Y. He] [2014-05-02] (1 week after software)

Update to include updated anchor results and config files and include colour gamut scalability tests.

Remains valid – not updated JCTVC-O1010 Guidelines for conformance test bitstream preparation [T. Suzuki, W. Wan]

JCTVC-Q1011 Report on HEVC compression performance verification test (WG 11 N 14420) [T. K. Tan, M. Mrak, V. Baroncini, N. Ramzan] [2014-04-25]

JCTVC-Q1012 Range extensions conformance draft 1 (WG 11 N 14427) [T. Suzuki, K. Kazui (Editors)] [2014-05-30]

JCTVC-Q1013 HEVC Range Extensions Test Model 7 Encoder Description [M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan] (WG 11 N 14418) [2014-06-06]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting were not considered binding on final CE plans as reviewed in closing plenary.

JCTVC-Q1014 Screen content coding test model 1 (SCM 1) (WG 11 N 14421) [R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye] [2014-04-25]

JCTVC-Q1015 Common conditions for screen content coding tests [H. Yu, R. Cohen, K. Rapaka, J. Xu] [2014-05-02]

JCTVC-Q1101 HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability [A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li (CE coordinators)] [2014-04-25] (three weeks)

3 weeks for finalization of CE description. Some details of planned test to be worked out to provide a more precise defintion of the experiment.

To test normative and non-normative alternatives.

JCTVC-Q1101 was reviewed in non-final form in closing plenary. A three week finalization period was authorized for the plan document.

JCTVC-Q1121 HEVC Screen Content Core Experiment 1 (SCCE1): Intra block copy [J. Sole, S. Liu (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1121 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1122 HEVC Screen Content Core Experiment 2 (SCCE2): Line-based intra copy [C.-C. Chen, X. Xu, L. Zhang (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1122 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1123 HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode [Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1123 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1124 HEVC Screen Content Core Experiment 4 (SCCE4): String matching for sample coding [Y. Chen, J. Xu (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1124 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1125 HEVC Screen Content Core Experiment 5 (SCCE5): Inter-component prediction and adaptive colour transform [X. Xiu, J. Chen (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1125 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 30 June (3:45 p.m.) – 9 July 2014 under ITU-T auspices in Sapporo, JP.

- 17–24 Oct. 2014 under WG 11 auspices in Strasbourg, FR.

- 10–18 Feb. 2015 under ITU-T auspices in Geneva, CH.

- 18–26 June 2015 under WG 11 auspices in Warsaw, PL.

The agreed document deadline for the 18th JCT-VC meeting is 20 June. Plans for scheduling of agenda items within that meeting TBA.

UPC, UPM and Kenzler Conference Management were thanked for the excellent hosting of the 17th meeting of the JCT-VC.

The JCT-VC meeting was closed at approximately 12:30 hours on Fri. 4 Apr. 2014.

# Annex A to JCT-VC report:
List of documents

| JCT-VC number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JCTVC-Q0001 | m33387 | 2014-03-26 21:13:06 | 2014-03-27 05:54:34 | 2014-03-27 05:54:34 | JCT-VC AHG report: Project management (AHG1) | G. J. Sullivan, J.-R. Ohm (AHG chairs) |
| JCTVC-Q0002 | m33377 | 2014-03-26 08:59:46 | 2014-03-26 23:27:05 | 2014-03-26 23:27:05 | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) | B. Bross, K. McCann (AHG co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang (AHG vice-chairs) |
| JCTVC-Q0003 | m33415 | 2014-03-27 16:32:07 | 2014-03-27 16:33:50 | 2014-03-27 16:35:12 | JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) | F. Bossen, D. Flynn, K. Sühring (AHG chairs) |
| JCTVC-Q0004 | m33402 | 2014-03-27 08:40:54 | 2014-03-27 10:32:04 | 2014-03-28 16:24:00 | JCT-VC AHG report: HEVC conformance test development (AHG4) | T. Suzuki, W. Wan (AHG chairs) |
| JCTVC-Q0005 | m33403 | 2014-03-27 08:44:09 | 2014-03-27 09:04:43 | 2014-03-27 09:04:43 | JCT-VC AHG report: HEVC range extensions development (AHG5) | C. Rosewarne, M. Naccari (AHG chairs) |
| JCTVC-Q0006 | m33389 | 2014-03-26 23:28:31 | 2014-03-27 10:10:45 | 2014-03-27 10:10:45 | JCT-VC AHG report: Range extensions draft text (AHG6) | J. Sole, D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan, T. Suzuki (AHG chairs) |
| JCTVC-Q0007 | m33167 | 2014-03-24 16:14:53 | 2014-03-27 09:10:11 | 2014-03-27 09:10:11 | JCT-VC AHG report: Range extensions software (AHG7) | K. Sharman, D. Flynn (AHG chairs) |
| JCTVC-Q0008 | m33408 | 2014-03-27 09:25:09 | 2014-03-27 09:25:45 | 2014-03-27 09:25:45 | JCT-VC AHG report: Screen content coding (AHG8) | H. Yu, R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu (AHG chairs) |
| JCTVC-Q0009 | m33378 | 2014-03-26 08:59:46 | 2014-03-26 09:00:09 | 2014-03-26 09:00:09 | JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) | M. M. Hannuksela, J. Boyce, Y. Chen, S. Deshpande, J. Samuelsson, Y.-K. Wang, P. Wu (AHG chairs) |
| JCTVC-Q0010 | m33460 | 2014-03-29 16:11:52 | 2014-03-29 16:16:13 | 2014-03-29 16:16:13 | JCT-VC AHG report: Palette-based coding (AHG10) | W. Pu, P. Onno, X. Guo, P. Lai (AHG chairs) |
| JCTVC-Q0011 | m33390 | 2014-03-27 00:13:35 | 2014-03-27 00:14:16 | 2014-03-27 00:14:16 | JCT-VC AHG report: SHVC text editing (AHG11) | J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang (AHG chairs) |
| JCTVC-Q0012 | m33278 | 2014-03-24 23:05:43 | 2014-03-26 05:14:53 | 2014-03-26 05:14:53 | JCT-VC AHG report: SHVC software development (AHG12) | V. Seregin, Y. He (AHG chairs) |
| JCTVC-Q0013 | m33101 | 2014-03-24 07:10:55 | 2014-03-25 04:35:15 | 2014-03-27 11:27:28 | JCT-VC AHG report: SHVC inter-layer filtering (AHG13) | E. Alshina, J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (AHG chairs) |
| JCTVC-Q0014 | m33179 | 2014-03-24 16:57:15 | 2014-03-27 08:13:56 | 2014-03-27 11:53:20 | JCT-VC AHG report: Colour gamut and bit depth scalability (AHG14) | P. Andrivon, A. Duenas, E. Alshina, S. Deshpande, X. Li, K. Ugur, Y. Ye (AHG chairs) |
| JCTVC-Q0015 | m32717 | 2014-03-14 05:50:58 | 2014-03-14 05:52:02 | 2014-03-27 05:27:04 | JCT-VC AHG report: Hybrid codec scalability (AHG15) | G. J. Sullivan (AHG chair) |
| JCTVC-Q0016 | m33409 | 2014-03-27 09:57:40 | 2014-03-27 12:31:12 | 2014-03-27 12:31:12 | JCT-VC AHG report: Test sequence material (AHG16) | T. Suzuki, R. Choen, T. K. Tan, S. Wenger (AHG chairs) |
| JCTVC-Q0017 | m32992 | 2014-03-20 21:37:52 | 2014-03-25 18:52:08 | 2014-03-25 18:52:08 | JCT-VC AHG report: Adaptive motion precision (AHG17) | J. Xu, X. Li (AHG chairs) |
| JCTVC-Q0018 | m33169 | 2014-03-24 16:17:09 | 2014-03-27 09:10:30 | 2014-03-27 09:10:30 | JCT-VC AHG report: high bit-rate and bit-depth operating points (AHG18) | K. Sharman, R. Joshi, H-Y. Kim (AHG chairs) |
| JCTVC-Q0019 | m33414 | 2014-03-27 11:56:45 | 2014-03-27 11:57:53 | 2014-03-27 12:01:25 | JCT-VC AHG report: Verification test preparation and testing (AHG19) | T. K. Tan, V. Baroncini, M. Mrak, M. Karczewicz, W. Wan, J. Wen (AHG chairs) |
| JCTVC-Q0021 | m33180 | 2014-03-24 17:02:30 | 2014-03-25 09:58:42 | 2014-03-28 14:35:54 | SCE1: Summary report of CE on colour gamut and bit depth scalability | P. Andrivon, A. Duenas, E. Alshina, Y. Ye, K. Ugur, X. Li (CE coordinators) |
| JCTVC-Q0022 | m33235 | 2014-03-24 19:30:09 | 2014-03-25 21:17:35 | 2014-03-25 21:17:35 | RCE1: Summary report of CE on adaptive motion precision | X. Li, J. Xu, Y. Zhou (CE coordinators) |
| JCTVC-Q0031 | m32877 | 2014-03-18 03:12:37 | 2014-03-18 11:03:42 | 2014-03-28 08:59:32 | Description of screen content coding technology proposal by Qualcomm | J. Chen, Y. Chen, T. Hsieh, R. Joshi, M. Karczewicz, W.-S. Kim, X. Li, C. Pang, W. Pu, K. Rapaka, J. Sole, L. Zhang, F. Zou (Qualcomm) |
| JCTVC-Q0032 | m32858 | 2014-03-17 23:44:26 | 2014-03-17 23:53:39 | 2014-03-27 14:43:55 | Description of screen content coding technology proposal by NCTU and ITRI International | C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, Y.-J. Chang, C.-H. Hung, C.-C. Lin, J.-S. Tu, E.-C. Ke, J.-Y. Kao, C.-L. Lin, F.-D. Jou, F.-C. Chen (NCTU/ITRI) |
| JCTVC-Q0033 | m32905 | 2014-03-18 06:59:15 | 2014-03-18 08:27:07 | 2014-03-26 14:12:05 | Description of screen content coding technology proposal by MediaTek | P. Lai, T.-D. Chuang, Y.-C. Sun, X. Xu, J. Ye, S.-T. Hsiang, Y.-W. Chen, K. Zhang, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-Q0034 | m32878 | 2014-03-18 03:29:45 | 2014-03-18 05:12:19 | 2014-03-27 16:29:08 | Description of screen content coding technology proposal by Huawei Technologies (USA) | Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu (Huawei) |
| JCTVC-Q0035 | m32882 | 2014-03-18 04:54:19 | 2014-03-18 19:24:04 | 2014-03-27 15:52:03 | Description of screen content coding technology proposal by Microsoft | B. Li, J. Xu, F. Wu, X. Guo, G. J. Sullivan (Microsoft) |
| JCTVC-Q0036 | m32852 | 2014-03-17 23:21:54 | 2014-03-18 05:29:52 | 2014-03-28 01:14:19 | Description of screen content coding technology proposal by Mitsubishi Electric Corporation | R. Cohen, A. Minezawa, X. Zhang, K. Miyazawa, A. Vetro, S. Sekiguchi, K. Sugimoto, T. Murakami (Mitsubishi Electric) |
| JCTVC-Q0037 | m32880 | 2014-03-18 04:23:54 | 2014-03-18 07:54:16 | 2014-03-29 03:46:37 | Description of screen content coding technology proposal by InterDigital | X. Xiu, C.-M. Tsai, Y. He, Y. Ye (InterDigital) |
| JCTVC-Q0041 | m32709 | 2014-03-06 00:24:09 | 2014-03-06 00:27:52 | 2014-03-06 00:27:52 | AHG 15: Comments on Hybrid Scalability | S. Deshpande (Sharp) |
| JCTVC-Q0042 | m32710 | 2014-03-06 18:12:02 | 2014-03-06 20:12:32 | 2014-03-06 20:12:32 | AHG 15: Support of hybrid scalability | Y.-K. Wang, J. Chen, Y. Chen, Hendry, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-Q0043 | m32718 | 2014-03-14 13:35:56 | 2014-03-15 09:29:48 | 2014-03-24 09:07:18 | Non-SCE1: De-noising of inter-layer reference | E. Alshina, A. Alshin (Samsung) |
| JCTVC-Q0044 | m32719 | 2014-03-14 13:36:38 | 2014-03-15 09:30:13 | 2014-03-15 09:30:20 | AhG18: On SAO quant-bits coding | E. Alshina, A. Alshin (Samsung) |
| JCTVC-Q0045 | m32720 | 2014-03-14 17:37:08 | 2014-03-14 17:53:25 | 2014-04-02 17:28:29 | MV-HEVC/SHVC HLS: Proposal for an SEI message on selectable overlays | N. Stefanoski, O. Wang, A. Smolic (DRZ), T. Szypulski (ESPN) |
| JCTVC-Q0046 | m32722 | 2014-03-14 21:09:39 | 2014-03-14 21:10:58 | 2014-03-30 09:54:41 | Complexity analysis of an optimized SHVC decoder | W. Hamidouche, M. Raulet, O. Deforges |
| JCTVC-Q0047 | m32723 | 2014-03-15 02:13:57 | 2014-03-18 09:33:22 | 2014-03-28 06:40:14 | AHG10: Simplification of Palette Based Coding | W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole (Qualcomm) |
| JCTVC-Q0048 | m32724 | 2014-03-15 04:35:06 | 2014-03-17 19:28:18 | 2014-04-02 18:18:42 | SCE1: Colour gamut scalability with asymmetric 3D LUT | X. Li, J. Chen, M. Karczewicz (Qualcomm), Y. He, Y. Ye, J. Dong (InterDigital), P. Bordes, P. Andrivon, E. Francois, F. Hiron (Technicolor) |
| JCTVC-Q0049 | m32725 | 2014-03-15 22:35:42 | 2014-03-17 20:55:28 | 2014-03-17 20:55:28 | RCE1: Adaptive MV Precision | X. Li, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0050 | m32726 | 2014-03-16 22:17:57 | 2014-03-16 22:20:03 | 2014-03-16 22:20:03 | 4K real time streaming with SHVC decoder and GPAC player | W. Hamidouche, M. Raulet, J. Le Feuvre (IETR/INSA) |
| JCTVC-Q0051 | m32728 | 2014-03-17 03:47:57 | 2014-03-17 10:53:30 | 2014-04-03 08:22:03 | AHG5: Recommended profiling of range extension coding tools | S. Lee, C. Park, E. Alshina, C. Kim (Samsung), K. McCann (Zetacast) |
| JCTVC-Q0052 | m32729 | 2014-03-17 05:58:12 | 2014-03-18 19:27:46 | 2014-03-18 19:27:46 | Low complexity encoders for JCTVC-Q0035 | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0053 | m32730 | 2014-03-17 05:59:01 | 2014-03-18 19:28:13 | 2014-03-30 22:15:47 | On the interpretation of screen content coding results | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0054 | m32767 | 2014-03-17 08:18:06 | 2014-03-17 08:57:25 | 2014-03-17 08:57:25 | MV-HEVC/SHVC HLS: VPS extension clean-up | Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung) |
| JCTVC-Q0055 | m32768 | 2014-03-17 08:24:16 | 2014-03-17 08:57:52 | 2014-03-17 08:57:52 | MV-HEVC/SHVC HLS: Comments on general decoding process and selection of CPB operation in the HRD operation | Y. Cho, B. Choi, M. W. Park, J. Y. Lee (Samsung) |
| JCTVC-Q0056 | m32772 | 2014-03-17 09:16:48 | 2014-03-17 12:27:03 | 2014-03-17 12:27:03 | Non-SCE1: Encoder improvements for weighted prediction | A. Aminlou, K. Ugur (Nokia), E. Alshina (Samsung) |
| JCTVC-Q0057 | m32773 | 2014-03-17 12:20:38 | 2014-03-18 01:31:22 | 2014-03-28 15:02:28 | MV-HEVC/SHVC HLS: Comments on IRAP alignment and POC value derivation | B. Choi, Y. Cho, M.W. Park, J.Y. Lee, S. Lee, C. Kim (Samsung) |
| JCTVC-Q0058 | m32774 | 2014-03-17 12:20:48 |  |  | Withdrawn |  |
| JCTVC-Q0059 | m32775 | 2014-03-17 14:00:37 | 2014-03-17 16:22:05 | 2014-03-17 16:22:05 | SCE1: Cross-check of JCTVC-Q0048 | A. Aminlou, K. Ugur (Nokia) |
| JCTVC-Q0060 | m32776 | 2014-03-17 15:25:50 | 2014-03-17 15:51:05 | 2014-03-28 09:18:56 | MV-HEVC/SHVC HLS: On inter-layer RPS signalling and derivation | H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI) |
| JCTVC-Q0061 | m32777 | 2014-03-17 15:27:09 | 2014-03-17 15:51:26 | 2014-03-17 15:51:26 | MV-HEVC/SHVC HLS: On max_tid_il_ref_pics_plus1 in the VPS extension | H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI) |
| JCTVC-Q0062 | m32780 | 2014-03-17 15:37:59 | 2014-03-17 19:11:49 | 2014-03-29 15:59:41 | AhG5: On the displacement vector prediction scheme for Intra Block Copy | P. Onno, G.Laroche, T. Poirier, C. Gisquet (Canon) |
| JCTVC-Q0063 | m32781 | 2014-03-17 15:38:12 | 2014-03-17 18:51:51 | 2014-03-29 09:11:51 | AhG10: Palette predictor stuffing | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-Q0064 | m32782 | 2014-03-17 15:38:22 | 2014-03-17 18:52:21 | 2014-03-29 16:06:13 | AhG10: Palette index coding | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-Q0065 | m32783 | 2014-03-17 15:38:35 | 2014-03-17 18:53:05 | 2014-03-29 16:03:12 | AhG10: Transition copy mode for Palette mode | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-Q0066 | m32784 | 2014-03-17 15:38:50 | 2014-03-17 18:51:44 | 2014-03-17 18:51:44 | AhG10: Run coding for palette mode | G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon) |
| JCTVC-Q0067 | m32785 | 2014-03-17 15:39:05 | 2014-03-17 22:16:41 | 2014-03-28 08:33:51 | Non-RCE1: On MV resolution and motion vector predictor number | G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon) |
| JCTVC-Q0068 | m32786 | 2014-03-17 15:39:13 | 2014-03-17 18:56:55 | 2014-03-17 18:56:55 | AhG8: On Sample Adaptive Band Value | G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon) |
| JCTVC-Q0069 | m32787 | 2014-03-17 15:39:24 | 2014-03-17 18:54:39 | 2014-03-29 15:56:46 | AhG10: Combination of Palette coding tools | C. Gisquet, G. Laroche, P. Onno (Canon) |
| JCTVC-Q0070 | m32788 | 2014-03-17 15:55:03 | 2014-03-17 17:14:07 | 2014-03-26 15:15:08 | Consistent usage of intra boundary filter disabling | X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek) |
| JCTVC-Q0071 | m32789 | 2014-03-17 16:23:21 | 2014-03-18 14:55:58 | 2014-03-28 12:03:10 | AHG10: Improvements on palette coding | D. Bugdayci, J. Lainema, K. Ugur, M.M. Hannuksela (Nokia) |
| JCTVC-Q0072 | m32790 | 2014-03-17 16:24:32 | 2014-03-17 16:51:53 | 2014-03-17 16:51:53 | SCE1: Colour gamut scalability using gain-offset models | A. Aminlou, K Ugur, M. M. Hannuksela (Nokia) |
| JCTVC-Q0073 | m32791 | 2014-03-17 16:48:17 | 2014-03-17 16:56:54 | 2014-03-24 12:26:22 | AHG18: Worst-case Escape Code Length Mitigation | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-Q0074 | m32792 | 2014-03-17 16:49:43 | 2014-03-17 16:53:07 | 2014-04-02 16:37:20 | SEI message for Colour Mapping Information | P. Andrivon, P. Bordes, E. Franois (Technicolor) |
| JCTVC-Q0075 | m32793 | 2014-03-17 16:49:46 | 2014-03-17 16:57:14 | 2014-03-24 12:26:46 | AHG5: Intra-block-copy in Non-4:4:4 Formats | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-Q0076 | m32794 | 2014-03-17 16:51:35 | 2014-03-17 16:57:36 | 2014-03-17 16:57:36 | Unifying HM and RExt Inter-Prediction Search | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-Q0077 | m32795 | 2014-03-17 16:53:15 | 2014-03-17 16:57:50 | 2014-03-18 12:57:48 | AHG8: Effectiveness of the HM Inter Search for Screen Content | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-Q0078 | m32797 | 2014-03-17 17:06:32 | 2014-03-18 12:04:53 | 2014-04-04 08:22:21 | MV-HEVC/SHVC HLS: On additional layer sets, rewriting of simulcast layers, and profile-tier-level indication for auxiliary picture layers | M. M. Hannuksela (Nokia) |
| JCTVC-Q0079 | m32799 | 2014-03-17 17:12:47 | 2014-03-18 12:05:52 | 2014-03-18 12:05:52 | MV-HEVC/SHVC HLS: max_tid_il_ref_pics_plus1 in inter-layer RPS syntax and semantics | M. M. Hannuksela (Nokia) |
| JCTVC-Q0080 | m32800 | 2014-03-17 17:21:54 | 2014-03-17 17:33:25 | 2014-03-26 15:16:40 | Block vector prediction for intra block copy | X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek) |
| JCTVC-Q0081 | m32803 | 2014-03-17 17:28:06 | 2014-03-17 17:31:39 | 2014-03-17 17:31:39 | RExt/SHVC HLS (AHG5 and 9): On reuse of alpha planes in auxiliary pictures | M. Naccari, M. Mrak (BBC) |
| JCTVC-Q0082 | m32808 | 2014-03-17 17:36:18 | 2014-03-17 17:45:47 | 2014-03-27 09:23:41 | Symmetric intra block copy | K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek) |
| JCTVC-Q0083 | m32809 | 2014-03-17 17:50:15 | 2014-03-17 17:57:08 | 2014-03-27 12:33:04 | AHG10: A triplet palette mode combining JCTVC-P0108 and JCTVC-P0198 | Y.-C. Sun, T.-D. Chuang, P. Lai, S.-T. Hsiang, Y.-W. Chen, X. Zhang, S. Liu, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-Q0084 | m32801 | 2014-03-17 17:24:55 | 2014-03-18 06:11:19 | 2014-04-01 11:56:20 | VUI entries for YZX and digital cinema EOTF | C. Fogg, J. Helman (Movielabs) |
| JCTVC-Q0085 | m32804 | 2014-03-17 17:28:10 | 2014-03-18 11:23:46 | 2014-04-14 20:50:23 | High Dynamic Range / Wide Colour Gamut workflow | B. Mandel (Universal Pictures), C. Fogg, J. Helman (MovieLabs) |
| JCTVC-Q0086 | m32802 | 2014-03-17 17:26:18 | 2014-03-18 06:11:41 | 2014-04-01 12:00:01 | Mastering Display Colour Volume SEI | C. Fogg, J. Helman (Movielabs) |
| JCTVC-Q0087 | m32805 | 2014-03-17 17:30:01 | 2014-04-04 14:34:32 | 2014-04-04 14:34:32 | Technicolor HDR clips encoded as Y’DzDx and BT.2020 Y’CbCr | B. Mandel (Universal), C. Fogg, J. Helman (MovieLabs) |
| JCTVC-Q0088 | m32806 | 2014-03-17 17:31:07 | 2014-03-18 06:27:45 | 2014-03-29 18:01:52 | HDR/WCG StEM encoded with Y'DzDx | J. Helman, C. Fogg (MovieLabs), B. Mandel (Universal) |
| JCTVC-Q0089 | m32807 | 2014-03-17 17:34:11 | 2014-03-18 06:13:26 | 2014-03-18 06:13:26 | Profile tables | C. Fogg, J. Helman (Movielabs) |
| JCTVC-Q0090 | m32810 | 2014-03-17 17:52:11 | 2014-03-18 07:53:35 | 2014-03-25 16:42:07 | HEVC/MV-HEVC/SHVC HLS: Redundant picture SEI message | M. Sychev, V. Stepin, S. Ikonin (Huawei) |
| JCTVC-Q0091 | m32811 | 2014-03-17 17:56:18 | 2014-03-18 12:07:14 | 2014-03-18 12:07:14 | MV-HEVC/SHVC HLS: On access unit definition and allowing different decoding orders in different layer trees | M. M. Hannuksela (Nokia) |
| JCTVC-Q0092 | m32814 | 2014-03-17 18:06:41 | 2014-03-17 18:17:02 | 2014-03-26 15:29:00 | Non-RCE1: Simplification of RCE1 Test2 | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-Q0093 | m32815 | 2014-03-17 18:20:09 | 2014-03-17 18:37:07 | 2014-03-28 05:15:54 | Single colour intra mode for screen content coding | Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek) |
| JCTVC-Q0094 | m32816 | 2014-03-17 18:30:20 | 2014-03-19 00:29:20 | 2014-03-19 00:29:20 | AHG10: Suggested Software for Palette Coding based on RExt6.0 | W. Pu (Qualcomm), X. Guo (Microsoft), P. Onno (Canon), P. Lai (MediaTek), J. Xu (Sony) |
| JCTVC-Q0095 | m32817 | 2014-03-17 18:41:28 | 2014-03-17 18:47:17 | 2014-03-28 12:12:36 | AHG8: Coding the prediction differences of the intra BC vectors | S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek) |
| JCTVC-Q0096 | m32818 | 2014-03-17 18:54:51 | 2014-03-17 20:39:19 | 2014-04-02 20:42:38 | MV-HEVC/SHVC HLS: Overlay info SEI message | J. Boyce, S. Wenger (Vidyo) |
| JCTVC-Q0097 | m32822 | 2014-03-17 19:46:41 | 2014-03-28 10:16:18 | 2014-03-28 10:16:18 | SCE1: Crosscheck report on colour gamut scalability using gain-offset models (JCTVC-0072) | X. Li (Qualcomm) |
| JCTVC-Q0098 | m32823 | 2014-03-17 19:49:08 | 2014-03-26 22:48:28 | 2014-03-26 22:48:28 | Non-RCE1: Crosscheck report on MV resolution and motion vector predictor number (JCTVC-Q0067) | X. Li (Qualcomm) |
| JCTVC-Q0099 | m32824 | 2014-03-17 19:50:22 | 2014-03-25 21:32:57 | 2014-03-25 21:32:57 | RCE1: Crosscheck report on Motion Vector Resolution Control (JCTVC-Q0155) | X. Li (Qualcomm) |
| JCTVC-Q0100 | m32825 | 2014-03-17 20:14:02 | 2014-03-18 07:08:35 | 2014-04-03 11:25:03 | MV-HEVC/SHVC HLS: Constraints for Reference Picture Set Parameters | S. Deshpande (Sharp) |
| JCTVC-Q0101 | m32826 | 2014-03-17 20:15:08 | 2014-03-18 07:08:53 | 2014-03-19 18:17:14 | MV-HEVC/SHVC HLS: On Bitstream Partition Buffer | S. Deshpande (Sharp) |
| JCTVC-Q0102 | m32827 | 2014-03-17 20:16:17 | 2014-03-18 07:09:34 | 2014-03-29 15:40:40 | MV-HEVC/SHVC HLS: Comments on HEVC Extensions | S. Deshpande (Sharp) |
| JCTVC-Q0103 | m32828 | 2014-03-17 20:17:46 | 2014-03-18 07:10:03 | 2014-03-18 07:10:03 | MV-HEVC/SHVC HLS: On DPB Profile Level Limits | S. Deshpande (Sharp) |
| JCTVC-Q0104 | m32833 | 2014-03-17 20:56:10 | 2014-03-17 21:33:56 | 2014-03-29 11:22:10 | Fix for the computation of scaling factors used in inter-layer prediction | K. Andersson, J. Samuelsson (Ericsson) |
| JCTVC-Q0105 | m32834 | 2014-03-17 21:15:04 | 2014-03-18 14:50:42 | 2014-03-30 09:54:34 | MV-HEVC/SHVC HLS: On temporal enhancement layers and diagonal inter-layer prediction | M. M. Hannuksela (Nokia) |
| JCTVC-Q0106 | m32836 | 2014-03-17 21:18:28 | 2014-03-17 21:21:18 | 2014-03-27 02:56:37 | Rext HLS: on lossless coding | X. Xiu, Y. He, Y. Ye (InterDigital) |
| JCTVC-Q0107 | m32837 | 2014-03-17 21:22:51 | 2014-03-18 05:57:10 | 2014-03-27 02:59:14 | SHVC HLS: On picture level resampling phase filter selection | Y. Ye, Y. He, Y. He (InterDigital) |
| JCTVC-Q0108 | m32838 | 2014-03-17 21:24:07 | 2014-03-18 12:07:58 | 2014-03-18 12:07:58 | MV-HEVC/SHVC HLS: On TSA and STSA pictures | M. M. Hannuksela (Nokia) |
| JCTVC-Q0109 | m32840 | 2014-03-17 21:34:17 | 2014-03-18 12:09:38 | 2014-03-18 12:09:38 | MV-HEVC/SHVC HLS: On TemporalId constraints | M. M. Hannuksela (Nokia) |
| JCTVC-Q0110 | m32842 | 2014-03-17 21:34:59 | 2014-03-17 21:36:43 | 2014-03-17 21:36:43 | Support for out-of-band signaling in VPS to enable future layer additions | S. Narasimhan, A.Luthra (Arris) |
| JCTVC-Q0111 | m32843 | 2014-03-17 21:48:34 | 2014-03-17 22:34:56 | 2014-03-17 22:34:56 | Errata report: Parsing issue for picture timing SEI message | Y. Wu, L. Zhu, G. J. Sullivan, F. Kyslov, S. Sadhwani (Microsoft) |
| JCTVC-Q0112 | m32844 | 2014-03-17 21:49:46 | 2014-03-18 10:54:39 | 2014-03-20 22:06:34 | AhG8: In-loop colour-space transform | L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0113 | m32845 | 2014-03-17 22:09:38 | 2014-03-18 01:10:50 | 2014-03-30 18:19:32 | Request for an HEVC 4:4:4 8 bit profile as well as other bit depth profiles | M. Mrak (BBC), G. Martin-cocher (BlackBerry), P. Onno, C. Rosewarne (Canon), A. Fuldseth (Cisco), R. Sjöberg (Ericsson), A. Duenas (NG Codec), J. Sole, M. Karczewicz (Qualcomm), R. Haszhimoto (Renesas), M. Budagavi (TI), |
| JCTVC-Q0114 | m32846 | 2014-03-17 22:26:31 | 2014-03-18 10:02:53 | 2014-03-31 16:14:52 | Block vector prediction method for Intra block copy | C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm) |
| JCTVC-Q0115 | m32847 | 2014-03-17 23:08:12 | 2014-03-18 12:11:08 | 2014-03-18 12:11:08 | REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process | M. M. Hannuksela (Nokia) |
| JCTVC-Q0116 | m32848 | 2014-03-17 23:08:13 | 2014-03-18 13:33:38 | 2014-03-27 18:32:18 | RExt HLS: Picture referencing across CRA pictures | R. Sjöberg, J. Samuelsson, Y. Wang (Ericsson) |
| JCTVC-Q0117 | m32850 | 2014-03-17 23:18:25 | 2014-03-19 02:49:05 | 2014-03-19 02:49:05 | Extension of the pic_struct element in HEVC | A. M. Tourapis, D. Singer (Apple), A. Duenas (NGCodec), G. Martin-Cocher (Blackberry) |
| JCTVC-Q0118 | m32851 | 2014-03-17 23:19:27 | 2014-03-18 23:53:10 | 2014-03-18 23:53:10 | Interlace coding in HEVC v.1 | A.M. Tourapis, D. Singer (Apple) |
| JCTVC-Q0119 | m32853 | 2014-03-17 23:40:54 | 2014-03-19 01:55:55 | 2014-03-27 07:27:42 | Increased resolution for scaled reference layer offset | K. Minoo, D. Baylon, A. Luthra (Arris) |
| JCTVC-Q0120 | m32854 | 2014-03-17 23:41:49 | 2014-03-19 01:56:24 | 2014-03-27 07:29:18 | Re-sampling using existing phase offset flag signaling | K. Minoo, D. Baylon (Arris) |
| JCTVC-Q0121 | m32855 | 2014-03-17 23:42:07 |  |  | Withdrawn |  |
| JCTVC-Q0122 | m32856 | 2014-03-17 23:42:27 | 2014-03-19 01:56:47 | 2014-03-29 12:48:37 | Increased precision for deriving the reference layer sample locations | K. Minoo, D. Baylon (Arris) |
| JCTVC-Q0123 | m32857 | 2014-03-17 23:43:14 | 2014-03-25 20:00:30 | 2014-03-25 20:00:30 | SCE1: Crosscheck for Colour gamut scalability using gain-offset models (JCTVC-0072) Test 2 | K. Minoo (Arris) |
| JCTVC-Q0124 | m32859 | 2014-03-18 00:00:43 | 2014-03-18 01:37:01 | 2014-03-30 09:51:12 | Improvements on 1D dictionary coding | J. Ye, S. Liu, S. Lei (MediaTek), X. Chen, L. Zhao, T. Lin (Tongji) |
| JCTVC-Q0125 | m32860 | 2014-03-18 00:00:54 | 2014-03-26 20:41:47 | 2014-03-28 23:03:51 | Crosscheck for JCTVC-Q0124 Improvements on 1D dictionary coding mode | R. Cohen (MERL) |
| JCTVC-Q0126 | m32861 | 2014-03-18 00:01:19 |  |  | Withdrawn |  |
| JCTVC-Q0127 | m32862 | 2014-03-18 00:02:03 | 2014-03-18 04:47:13 | 2014-03-27 00:43:34 | Simplification on block vector prediction for intra block copy | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-Q0128 | m32863 | 2014-03-18 00:02:46 | 2014-03-18 05:52:57 | 2014-03-28 10:40:43 | Fix for Strong Intra Smoothing in RExt | J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony) |
| JCTVC-Q0129 | m32864 | 2014-03-18 00:05:58 | 2014-03-18 08:00:44 | 2014-03-18 08:00:44 | Non-SCE1: Reduction of the number of colour space regions in SCE1 test 1.1 | C. Auyeung, K. Sato (Sony) |
| JCTVC-Q0130 | m32865 | 2014-03-18 00:12:08 | 2014-03-18 07:56:57 | 2014-03-20 21:28:57 | AHG12: Mismatch of SHVC draft 5 and SHM-5.0 software in bit-depth scalability with 1x scalability | C. Auyeung (Sony) |
| JCTVC-Q0131 | m32866 | 2014-03-18 00:18:40 | 2014-03-18 06:18:29 | 2014-03-26 20:33:12 | AHG18: Limiting the worst-case length for coeff_abs_level_remaining syntax element to 32 bits | M. Karczewicz, R. Joshi (Qualcomm) |
| JCTVC-Q0132 | m32867 | 2014-03-18 00:31:06 | 2014-03-18 05:08:48 | 2014-04-03 19:30:34 | On unification of intra block copy and inter-picture motion compensation | X. Xu, S. Liu, S. Lei (MediaTek) |
| JCTVC-Q0133 | m32868 | 2014-03-18 00:44:19 | 2014-03-18 23:49:36 | 2014-03-18 23:49:36 | Comments on HEVC 4:4:4 8 bit profiles | A.M. Tourapis, D. Singer (Apple) |
| JCTVC-Q0134 | m32872 | 2014-03-18 01:53:22 | 2014-03-18 23:08:36 | 2014-04-02 04:23:06 | Ping-pong block vector predictor for intra block copy | L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A Kumar (Microsoft) |
| JCTVC-Q0135 | m32873 | 2014-03-18 01:56:51 | 2014-03-18 23:10:34 | 2014-03-18 23:10:34 | AMP for Intra BC prediction | L. Zhu, J. Xu, Y. Wu, G. J. Sullivan, S. Sankuratri, B. A Kumar (Microsoft) |
| JCTVC-Q0136 | m32874 | 2014-03-18 02:11:39 | 2014-03-27 00:36:07 | 2014-03-27 00:36:07 | AHG14: On methods and test conditions for colour gamut scalability | K. Minoo, A. Luthra, D. Baylon (Arris) |
| JCTVC-Q0137 | m32875 | 2014-03-18 02:11:53 | 2014-03-18 07:18:41 | 2014-03-24 17:38:42 | AhG 8: Adaptive Scaling of SAO offset | W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0138 | m32876 | 2014-03-18 02:58:36 | 2014-03-18 05:06:18 | 2014-03-27 07:40:19 | AHG10: Line-matching above copy method | Yao-Jen Chang, Chun-Lung Lin, Chao-Hsiung Hung (ITRI), Chun-Chi Chen, Wen-Hsiao Peng, Hsueh-Ming Hang (NCTU) |
| JCTVC-Q0139 | m32879 | 2014-03-18 04:08:08 | 2014-03-18 07:37:43 | 2014-03-29 16:30:13 | Intra block copy with larger search region | C. Pang, J. Sole, T. Hsieh, M. Karczewicz (Qualcomm) |
| JCTVC-Q0140 | m32881 | 2014-03-18 04:37:44 | 2014-03-18 10:05:15 | 2014-03-29 16:30:57 | AhG8: Performance of encoder and parameter only changes for Screen Content Coding | J. Sole, C. Pang, L. Zhang, K. Rapaka, M. Karczewicz (Qualcomm) |
| JCTVC-Q0141 | m32883 | 2014-03-18 05:00:57 | 2014-03-18 05:08:18 | 2014-03-27 00:56:45 | Non-SCE1: Coding of colour gamut prediction coefficients | K. Misra, S.-H. Kim, A. Segall (Sharp) |
| JCTVC-Q0142 | m32884 | 2014-03-18 05:01:21 | 2014-03-18 10:55:58 | 2014-04-03 16:17:46 | MV-HEVC/SHVC HLS: On extraction of independent non-base layer | Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm) |
| JCTVC-Q0143 | m32885 | 2014-03-18 05:01:50 | 2014-03-26 17:37:11 | 2014-03-26 17:37:11 | Crosscheck report of SCE1 test on colour gamut scalability using 8x8x8 regions and matrix mapping (JCTVC-Q0072) | K. Misra (Sharp) |
| JCTVC-Q0144 | m32886 | 2014-03-18 05:02:15 | 2014-03-26 17:36:26 | 2014-03-26 17:36:26 | Crosscheck report of SCE1 test on asymmetric 3D-LUT with phase alignment filter disabled (JCTVC-Q0048) | K. Misra (Sharp) |
| JCTVC-Q0145 | m32887 | 2014-03-18 05:05:19 | 2014-03-18 12:03:36 | 2014-03-18 12:03:36 | MV-HEVC/SHVC HLS: On level definitions | Y.-K. Wang, K. Rapaka, J. Chen, Hendry, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-Q0146 | m32888 | 2014-03-18 05:05:36 | 2014-03-18 12:20:46 | 2014-03-18 12:20:46 | MV-HEVC/SHVC HLS: On picture order count and related | A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm) |
| JCTVC-Q0147 | m32892 | 2014-03-18 05:09:52 | 2014-03-18 20:51:49 | 2014-03-30 13:27:44 | AhG8: On fast inter search method for screen content coding | K. Rapaka, J. Sole, L. Zhang, M. Karczewicz (Qualcomm) |
| JCTVC-Q0148 | m32893 | 2014-03-18 05:26:42 | 2014-03-18 22:36:25 | 2014-03-27 19:31:58 | Quantization rounding for RDPCM | F. Zou, R. Joshi, J. Sole, C. Pang, M. Karczewicz (Qualcomm) |
| JCTVC-Q0149 | m32894 | 2014-03-18 05:27:09 | 2014-03-18 21:44:47 | 2014-03-31 19:32:10 | Pixel-based 1D Dictionary Coding | F. Zou, Y. Chen, C. Pang, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0150 | m32895 | 2014-03-18 06:17:07 | 2014-03-27 22:31:06 | 2014-03-27 22:31:06 | Crosscheck AhG10: Palette predictor stuffing (JCTVC-Q0063) | W. Pu, R. Joshi (Qualcomm) |
| JCTVC-Q0151 | m32896 | 2014-03-18 06:18:28 | 2014-03-27 22:30:12 | 2014-03-27 22:40:04 | Crosscheck AhG10: Palette index coding (JCTVC-Q0064) | W. Pu, R. Joshi (Qualcomm) |
| JCTVC-Q0152 | m32897 | 2014-03-18 06:20:01 | 2014-04-02 03:32:33 | 2014-04-02 03:32:33 | Crosscheck AhG10: Transition copy mode for Palette mode (JCTVC-Q0065) | W. Pu, R. Joshi (Qualcomm) |
| JCTVC-Q0153 | m32898 | 2014-03-18 06:21:18 | 2014-03-27 22:45:07 | 2014-03-27 22:45:07 | Crosscheck AhG10: Run coding for palette mode (JCTVC-Q0066) | W. Pu, R. Joshi (Qualcomm) |
| JCTVC-Q0154 | m32899 | 2014-03-18 06:25:21 | 2014-03-20 10:27:41 | 2014-03-20 10:27:41 | MV-HEVC/SHVC HLS: On picture flushing and DPB parameters | A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm) |
| JCTVC-Q0155 | m32901 | 2014-03-18 06:35:22 | 2014-03-18 23:36:37 | 2014-03-26 14:27:32 | RCE1: Subtest 1 - Motion Vector Resolution Control | Y. Zhou, J. Xu, G. J. Sullivan, B. Lin (Microsoft) |
| JCTVC-Q0156 | m32902 | 2014-03-18 06:35:44 | 2014-03-18 23:28:37 | 2014-03-18 23:28:37 | RCE1: cross-check report of subtest 2 | Y. Zhou, J. Xu (Microsoft) |
| JCTVC-Q0157 | m32903 | 2014-03-18 06:49:20 | 2014-03-18 11:56:03 | 2014-03-18 11:56:03 | MV-HEVC/SHVC HLS: On DPB - to share or not to share, that is the question | A. K. Ramasubramonian, Hendry, Y.-K. Wang, Y. Chen (Qualcomm) |
| JCTVC-Q0158 | m32906 | 2014-03-18 07:02:51 | 2014-03-18 07:10:32 | 2014-03-18 07:10:32 | MV-HEVC/SHVC HLS: On Highest TemporalId | S. Deshpande (Sharp) |
| JCTVC-Q0159 | m32908 | 2014-03-18 08:01:41 | 2014-03-18 09:16:26 | 2014-03-29 10:57:43 | AHG 13: Sub-region extraction â€“ position calculation and comparison of different approaches | T. Yamamoto, T. Tsukuba, T. Ikai (Sharp) |
| JCTVC-Q0160 | m32909 | 2014-03-18 08:01:44 | 2014-03-18 09:17:44 | 2014-04-03 07:53:01 | SHVC/MV-HEVC HLS: On alternative output layer flag | T. Yamamoto, T. Tsukuba, T. Ikai (Sharp) |
| JCTVC-Q0161 | m32910 | 2014-03-18 08:01:46 | 2014-03-18 09:33:18 | 2014-03-29 08:05:52 | Layer association of auxiliary pictures | T. Ikai (Sharp) |
| JCTVC-Q0162 | m32911 | 2014-03-18 08:01:48 | 2014-03-18 09:35:31 | 2014-03-29 08:06:36 | SHVC/MV-HEVC HLS: Scaled offset information of auxiliary pictures | T. Ikai (Sharp) |
| JCTVC-Q0163 | m32912 | 2014-03-18 08:01:50 | 2014-03-18 09:26:17 | 2014-03-27 18:08:30 | MV-HEVC/SHVC HLS: On decoding non-output/non-reference layers | T. Tsukuba, T. Yamamoto, T. Ikai (Sharp) |
| JCTVC-Q0164 | m32913 | 2014-03-18 08:01:52 | 2014-03-18 09:37:51 | 2014-03-18 09:37:51 | MV-HEVC/SHVC HLS: On sub-bitstream property SEI | T. Ikai, T. Yamamoto, T. Tsukuba(Sharp) |
| JCTVC-Q0165 | m32914 | 2014-03-18 08:01:54 | 2014-03-18 09:21:03 | 2014-03-28 10:43:29 | MV-HEVC/SHVC HLS: Clean up for output layer set | T. Tsukuba, T. Yamamoto, T. Ikai, S. Deshpande(Sharp) |
| JCTVC-Q0166 | m32915 | 2014-03-18 08:01:57 | 2014-03-18 09:20:14 | 2014-04-02 15:42:17 | MV-HEVC/SHVC HLS: On scaled reference layer offset | T. Yamamoto, T. Tsukuba, T. Ikai (Sharp), A. K. Ramasubramonian (Qualcomm) |
| JCTVC-Q0167 | m32916 | 2014-03-18 08:01:59 | 2014-03-18 09:42:22 | 2014-03-18 09:42:22 | MV-HEVC/SHVC HLS: On layers not present SEI | T. Ikai (Sharp) |
| JCTVC-Q0168 | m32917 | 2014-03-18 08:09:46 | 2014-03-18 10:04:58 | 2014-03-18 10:04:58 | On phase offset for resampling process in SHVC | J. Chen, K. Rapaka, M. Karczewicz (Qualcomm) |
| JCTVC-Q0169 | m32928 | 2014-03-18 10:42:44 | 2014-03-18 10:47:54 | 2014-03-29 11:15:10 | AHG10: Adaptive Scan Order on Palette Based Coding | J. Zhu, J. Ye, K. Kazui (Fujitsu) |
| JCTVC-Q0170 | m32929 | 2014-03-18 13:34:37 | 2014-03-18 13:39:23 | 2014-03-18 13:39:23 | SHVC HLS: Resampling need for a scalable layer | K. Andersson, J. Samuelsson, R. Sjöberg, J. Ström (Ericsson) |
| JCTVC-Q0171 | m32931 | 2014-03-18 23:38:14 | 2014-04-02 00:24:36 | 2014-04-02 00:24:36 | Cross-check of JCTVC-Q0093 | J. Xu (Microsoft) |
| JCTVC-Q0172 | m32932 | 2014-03-18 23:39:01 | 2014-04-02 00:24:12 | 2014-04-02 04:37:44 | Cross-check of JCTVC-Q0127 | J. Xu (Microsoft) |
| JCTVC-Q0173 | m32933 | 2014-03-19 00:21:18 | 2014-03-26 22:57:47 | 2014-03-26 22:57:47 | Non-RCE1: Crosscheck report on simplification of RCE1 test2 (JCTVC-Q0092) | X. Li (Qualcomm) |
| JCTVC-Q0174 | m32935 | 2014-03-19 01:51:49 | 2014-03-19 08:40:33 | 2014-03-29 13:23:20 | AHG10: Modified copy above mode for palette based coding | J. Zhu, J. Ye, K. Kazui (Fujitsu) |
| JCTVC-Q0175 | m32936 | 2014-03-19 01:54:29 | 2014-03-19 23:47:09 | 2014-03-28 18:08:11 | Intra block copy with encoder search using chroma component | C. Pang, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0176 | m32937 | 2014-03-19 05:08:27 | 2014-03-24 01:08:52 | 2014-03-25 05:15:43 | AHG8: String match in coding of screen content | W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu (Huawei) |
| JCTVC-Q0177 | m32938 | 2014-03-19 06:41:57 | 2014-03-19 09:12:02 | 2014-03-19 09:12:02 | MV-HEVC/SHVC HLS: Miscellaneous HLS topics | Hendry, A. K. Ramasubramonian, Y.-K. Wang, V. Seregin (Qualcomm) |
| JCTVC-Q0178 | m32947 | 2014-03-19 15:14:53 | 2014-03-30 10:58:56 | 2014-03-30 10:58:56 | AHG10: Crosscheck of Simplification of Palette Based Coding (JCTVC-Q0047) | C. Gisquet (Canon) |
| JCTVC-Q0179 | m32948 | 2014-03-19 15:15:10 | 2014-03-25 16:29:02 | 2014-03-25 16:29:02 | AHG8: Crosscheck of Coding the prediction differences of the intra BC vectors (JCTVC-Q0095) | P. Onno (Canon) |
| JCTVC-Q0180 | m32949 | 2014-03-19 15:15:27 | 2014-03-28 22:32:34 | 2014-03-28 22:32:34 | Crosscheck of ping-pong block vector predictor for intra block copy (JCTVC-Q0134) | P. Onno (Canon) |
| JCTVC-Q0181 | m32950 | 2014-03-19 15:15:41 | 2014-03-26 10:52:10 | 2014-03-26 10:52:10 | Crosscheck of quantization rounding for RDPCM (JCTVC-Q0148) | C. Gisquet (Canon) |
| JCTVC-Q0182 | m32986 | 2014-03-20 16:37:49 | 2014-03-20 16:43:14 | 2014-03-21 13:09:10 | MV-HEVC/SHVC: On bitstream partition buffering | M. M. Hannuksela, A. Hallapuro (Nokia) |
| JCTVC-Q0183 | m32988 | 2014-03-20 17:09:52 | 2014-03-25 03:20:35 | 2014-03-25 03:20:35 | MV-HEVC/SHVC HLS: SEI message cleanups | Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm) |
| JCTVC-Q0184 | m32991 | 2014-03-20 18:45:19 | 2014-03-29 17:12:23 | 2014-03-29 17:12:23 | Cross-check of JCTVC-Q0135 AMP for Intra BC prediction | P. Lai, X. Xu, S. Liu (MediaTek) |
| JCTVC-Q0185 | m32993 | 2014-03-21 03:15:23 | 2014-03-28 11:21:54 | 2014-03-28 11:21:54 | Cross-check of 'AHG18: Worst-case Escape Code Length Mitigation' (JCTVC-Q0073) by Sony | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-Q0186 | m33038 | 2014-03-21 11:46:28 | 2014-03-21 11:48:19 | 2014-04-03 08:22:16 | AHG5: Super-high Tier Specification Targeted at the Intra 16-bit 4:4:4 Profile | K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony) |
| JCTVC-Q0187 | m33042 | 2014-03-21 14:02:16 | 2014-03-21 17:42:58 | 2014-03-21 17:42:58 | Resampling of reference layer frames in interlaced-to-progressive scalability | M. M. Hannuksela, K. Ugur (Nokia) |
| JCTVC-Q0188 | m33044 | 2014-03-21 14:33:39 | 2014-03-21 14:41:43 | 2014-03-21 14:41:43 | Alternative AVC base layer HRD parameters for HEVC hybrid codec scalability | M. M. Hannuksela (Nokia) |
| JCTVC-Q0189 | m33058 | 2014-03-21 21:46:39 | 2014-03-21 21:50:22 | 2014-03-30 18:26:49 | MV-HEVC/SHVC HLS: On slice_temporal_mvp_enabled_flag | M. M. Hannuksela (Nokia) |
| JCTVC-Q0190 | m33077 | 2014-03-23 14:22:32 | 2014-03-23 22:05:47 | 2014-04-09 17:39:27 | Evaluation of distortion metrics on HDR video content | E. Franois, P. Lopez, F. Le Léannec, S. Lasserre (Technicolor) |
| JCTVC-Q0191 | m33078 | 2014-03-23 14:24:22 | 2014-03-24 18:36:43 | 2014-03-25 18:51:01 | New HDR video coding results | E. Franois, S. Lasserre, F. Le Léannec (Technicolor) |
| JCTVC-Q0192 | m33240 | 2014-03-24 20:52:05 | 2014-03-24 20:53:27 | 2014-03-24 20:53:27 | Insights and open questions on HDR/WCG video coding | D. Singer, A.M. Tourapis (Apple) |
| JCTVC-Q0193 | m33246 | 2014-03-24 21:36:26 | 2014-03-24 21:41:16 | 2014-03-24 21:41:16 | Combination of screen content coding proposals JCTVC-Q0034/JCTVC-Q0176 and JCTVC-Q0036 | R. Cohen, X. Zhang, A. Vetro, K. Sugimoto (MERL), A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi Electric), Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu (Huawei Technologies (USA) |
| JCTVC-Q0194 | m33276 | 2014-03-24 22:57:21 | 2014-03-29 04:17:42 | 2014-03-31 04:12:04 | Cross-check of JCTVC-Q0034: Description of screen content coding technology proposal by Huawei Technologies (USA) | X. Zhang, R. Cohen (MERL), K. Miyazawa, A. Minezawa (Mitsubishi Electric) |
| JCTVC-Q0195 | m33299 | 2014-03-25 01:17:10 | 2014-03-25 01:21:40 | 2014-03-25 01:21:40 | MV-HEVC/SHVC HLS: On representation format signaling | S. Hattori, O. Nakagami, T. Suzuki (Sony) |
| JCTVC-Q0196 | m33302 | 2014-03-25 02:18:34 | 2014-03-25 02:27:05 | 2014-03-25 02:27:05 | SCE1: Crosscheck result of Test 1.3 | K. Sato (Sony) |
| JCTVC-Q0197 | m33304 | 2014-03-25 02:26:59 | 2014-03-25 03:13:46 | 2014-03-28 08:39:47 | Cross-check for increased precision for deriving the reference layer sample location | E. Alshina (Samsung) |
| JCTVC-Q0198 | m33305 | 2014-03-25 02:29:02 | 2014-03-25 04:04:53 | 2014-03-25 04:04:53 | Cross-check for bit-depth conversion bug fix in SHM reference s/w (JCTVC-Q0130) | E. Alshina, A. Alshin (Samsung) |
| JCTVC-Q0199 | m33306 | 2014-03-25 02:30:48 | 2014-03-27 04:11:17 | 2014-03-27 04:11:17 | Non-SCE1: Cross-check for reduction of the number of colour space regions in SCE1 test 1.1 (JCTVC-Q0129) | E. Alshina, A. Alshin (Samsung) |
| JCTVC-Q0200 | m33307 | 2014-03-25 02:30:50 | 2014-03-31 16:31:43 | 2014-04-01 12:45:36 | Re-sampling process modification considering cropping information | E. Alshina (Samsung), J. Chen (Qualcomm) |
| JCTVC-Q0201 | m33308 | 2014-03-25 02:32:10 | 2014-03-27 04:21:48 | 2014-03-27 04:21:48 | Cross-check for Consistent usage of intra boundary filter disabling (JCTVC-Q0070) | E. Alshina, A. Alshin (Samsung) |
| JCTVC-Q0202 | m33310 | 2014-03-25 03:10:50 | 2014-04-03 11:38:16 | 2014-04-03 11:38:16 | Cross-check for Symmetric intra block copy (JCTVC-Q0082) | E. Alshina (Samsung) |
| JCTVC-Q0203 | m33311 | 2014-03-25 03:10:52 | 2014-04-03 11:37:41 | 2014-04-04 08:53:57 | AhG13: Methodology for evaluation an accuracy of re-sampling process | E. Alshina (Samsung) |
| JCTVC-Q0204 | m33340 | 2014-03-25 09:17:38 | 2014-03-27 16:05:46 | 2014-04-01 14:30:53 | HEVC verification test results | T.K. Tan, M. Mrak, V. Baroncini, N. Ramzan (Test coordinators) |
| JCTVC-Q0205 | m33343 | 2014-03-25 10:26:41 | 2014-03-28 06:22:45 | 2014-03-28 06:22:45 | AHG10: Cross-check report for JCTVC-Q0083 | X. Guo (Microsoft) |
| JCTVC-Q0206 | m33345 | 2014-03-25 11:37:26 | 2014-03-28 18:36:42 | 2014-04-01 14:33:55 | Proposal to support 12 bit video in SHVC | T. Suzuki, C. Auyeung, K. Sato (Sony) |
| JCTVC-Q0207 | m33353 | 2014-03-25 18:55:46 | 2014-03-25 19:58:41 | 2014-03-25 19:58:41 | Crosscheck of JCTVC-Q0114 on Block vector prediction method for Intra block copy | J. Xu (Sony) |
| JCTVC-Q0208 | m33354 | 2014-03-25 18:57:25 | 2014-03-28 00:20:29 | 2014-03-28 00:20:29 | Crosscheck of JCTVC-Q0068 on AHG8: On Sample Adaptive Band Value | J. Xu, A. Tabatabai (Sony) |
| JCTVC-Q0209 | m33355 | 2014-03-25 21:26:57 | 2014-03-28 04:00:41 | 2014-03-28 04:00:41 | Cross-check of JCTVC-Q0128 on Fix for Strong Intra Smoothing in RExt | C. Pang (Qualcomm) |
| JCTVC-Q0210 | m33357 | 2014-03-25 22:24:02 | 2014-03-27 00:55:48 | 2014-03-27 00:55:48 | Cross-check of JCTVC-Q0075 on Intra-block-copy in Non-4:4:4 Formats | C. Pang (Qualcomm) |
| JCTVC-Q0211 | m33358 | 2014-03-26 01:08:25 | 2014-03-27 02:20:38 | 2014-03-27 02:20:38 | MV-HEVC/SHVC HLS: On HighestTid and MaxSubLayersInLayerSetMinus1 | T. Ikai (Sharp) |
| JCTVC-Q0212 | m33363 | 2014-03-26 01:57:09 | 2014-03-26 01:59:41 | 2014-03-28 10:14:28 | AHG5: Objective and subjective evaluations of cross-component decorrelation in RExt6.0 for range extensions profile | K. Kawamura, S. Naito (KDDI) |
| JCTVC-Q0213 | m33385 | 2014-03-26 18:54:52 | 2014-03-26 22:10:58 | 2014-03-29 11:13:26 | Fix for adaptive colour space coding in JCTVC-Q0035 | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0214 | m33386 | 2014-03-26 18:58:42 | 2014-03-28 00:25:13 | 2014-03-28 00:25:13 | Cross-check of JCTVC-Q0062 | L. Zhu, J. Xu (Microsoft) |
| JCTVC-Q0215 | m33391 | 2014-03-27 00:43:30 | 2014-03-28 16:06:36 | 2014-03-28 16:06:36 | Cross-check for refined scaling factor calculation in JCTVC-Q0104 | J. Chen (Qualcomm) |
| JCTVC-Q0216 | m33392 | 2014-03-27 01:07:40 | 2014-03-28 16:21:18 | 2014-03-28 16:21:18 | Cross-check for modified phase offset calculation in JCTVC-Q0120 | J. Chen (Qualcomm) |
| JCTVC-Q0217 | m33400 | 2014-03-27 06:30:48 | 2014-03-27 07:19:20 | 2014-03-27 07:19:20 | Crosscheck of JCTVC-Q0137 on adaptive scaling of SAO offsets | X. Zhang, Y.-W. Huang (MediaTek) |
| JCTVC-Q0218 | m33401 | 2014-03-27 06:32:22 | 2014-03-29 15:53:19 | 2014-03-29 15:53:19 | Crosscheck of JCTVC-Q0138 on line-matching above copy method | Y.-C. Sun, Y.-W. Huang (MediaTek) |
| JCTVC-Q0219 | m33404 | 2014-03-27 09:08:19 | 2014-03-28 16:24:30 | 2014-03-28 16:24:30 | Editor's proposed draft text of HEVC conformance testing | T. Suzuki, W. Wan, G. Sullivan (Editors) |
| JCTVC-Q0220 | m33405 | 2014-03-27 09:13:05 | 2014-03-30 13:07:32 | 2014-03-30 13:07:32 | Cross-check of 'Intra block copy with larger search region' (JCTVC-Q0139) by Qualcomm | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-Q0221 | m33406 | 2014-03-27 09:17:50 | 2014-03-30 13:09:22 | 2014-03-30 13:09:22 | Cross-check of â€˜Intra block copy with encoder search using chroma component' (JCTVC-Q0175) by Qualcomm | C. Rosewarne, M. Maeda (Canon) |
| JCTVC-Q0222 | m33410 | 2014-03-27 10:17:58 | 2014-03-27 12:20:03 | 2014-03-27 12:20:03 | SCE1: Cross-check of 3D-LUT parameter coding of JCTVC-Q0048 | K. Ugur, A. Aminlou (Nokia) |
| JCTVC-Q0223 | m33418 | 2014-03-27 22:03:47 | 2014-03-27 23:14:22 | 2014-04-03 15:26:39 | BoG report on High Level Syntax | J. Boyce, Y.-K. Wang |
| JCTVC-Q0224 | m33419 | 2014-03-27 22:46:44 | 2014-04-02 03:19:07 | 2014-04-02 03:28:18 | Crosscheck AhG10: Combination of Palette coding tools (JCTVC-Q0069) | W. Pu, R. Joshi (Qualcomm) |
| JCTVC-Q0225 | m33424 | 2014-03-28 01:15:01 | 2014-03-28 07:32:18 | 2014-03-28 07:32:18 | Improvements to palette based coding | W. Pu, F. Zou, R. Joshi, M. Karczewicz, J. Sole (Qualcomm) |
| JCTVC-Q0226 | m33425 | 2014-03-28 06:53:27 | 2014-03-31 19:29:32 | 2014-03-31 19:29:32 | Cross-check of block vector prediction for intra block copy in JCTVC-Q0080 | W.-S. Kim (Qualcomm) |
| JCTVC-Q0227 | m33430 | 2014-03-28 16:07:45 | 2014-03-30 18:25:37 | 2014-03-30 18:25:37 | Crosscheck report of JCTVC-Q0056 on encoder improvements for weighted prediction | K. Misra (Sharp) |
| JCTVC-Q0228 | m33446 | 2014-03-29 07:12:47 | 2014-03-29 08:57:08 | 2014-03-29 08:57:08 | DCI HDR & WCG StEM Content Description | J. Helman (MovieLabs), W. Husak (Dolby) |
| JCTVC-Q0229 | m33433 | 2014-03-28 19:10:21 | 2014-03-28 19:15:00 | 2014-04-02 01:13:39 | HEVC still picture coding performance evaluation | Didier Nicholson, Chaker Larabi, Antonin Descampe |
| JCTVC-Q0230 | m33436 | 2014-03-28 20:34:44 | 2014-03-28 20:47:10 | 2014-03-28 20:47:10 | Cross-check of symmetric intra block copy (JCTVC-Q0082) | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0231 | m33443 | 2014-03-29 00:36:27 | 2014-03-29 00:45:37 | 2014-03-29 00:45:37 | Crosscheck of JCTVC-Q0095 on Coding the prediction differences of the intra BC vectors | C. Pang (Qualcomm) |
| JCTVC-Q0232 | m33448 | 2014-03-29 08:49:51 | 2014-04-04 14:35:17 | 2014-04-04 14:35:17 | HDR/WCG StEM and Telescope clips encoded as Y'DzDx and BT.2020 | B. Mandel (Universal), C. Fogg, J. Helman (MovieLabs) |
| JCTVC-Q0233 | m33449 | 2014-03-29 08:49:52 |  |  | Withdrawn |  |
| JCTVC-Q0234 | m33459 | 2014-03-29 13:51:38 | 2014-03-29 15:51:44 | 2014-04-03 16:06:41 | BoG report on SHVC upsampling process and phase offset | J. Boyce (BoG coordinator) |
| JCTVC-Q0235 | m33461 | 2014-03-29 16:38:38 | 2014-03-29 16:46:24 | 2014-03-29 16:46:24 | Cross-check report on Description of SCC technology proposal by MERL (JCTVC-Q0036) | X. Wang, H. Yang, M. Xu (Huawei) |
| JCTVC-Q0236 | m33463 | 2014-03-30 03:22:14 | 2014-03-30 03:36:25 | 2014-03-31 08:51:25 | BoG report for Non-CfP SCC and related documents | R. Cohen (BoG coordinator) |
| JCTVC-Q0237 | m33475 | 2014-03-30 16:42:36 | 2014-04-02 21:36:53 | 2014-04-10 05:34:25 | Crosscheck of JCTVC-Q0132 on AMVP for BV prediction and PU syntax for IntraBC | R.-L. Liao, C.-C. Chen, W.-H. Peng (NCTU), J.-S. Tu (ITRI) |
| JCTVC-Q0238 | m33477 | 2014-03-30 22:36:23 | 2014-03-31 08:05:39 | 2014-03-31 08:05:39 | Encoding time reduction for Independent Uniform Prediction in SCC CfP response JCTVC-Q0036 | X. Zhang, R. Cohen |
| JCTVC-Q0239 | m33480 | 2014-03-31 11:34:43 | 2014-03-31 16:26:22 | 2014-03-31 16:26:22 | BoG report on summary of objective performance and tools for SCC CfP responses | R. Joshi, R. Cohen, H. Yu |
| JCTVC-Q0240 | m33483 | 2014-03-31 19:18:50 | 2014-04-03 20:30:22 | 2014-04-03 20:30:22 | Crosscheck of JCTVC-Q0225 on AHG10: Improvements to Palette Based Coding | J. Xu (Sony) |
| JCTVC-Q0241 | m33484 | 2014-03-31 20:21:07 | 2014-03-31 20:23:11 | 2014-03-31 20:23:11 | Cross-check of JCTVC-Q0147 "On fast inter search method for screen content coding" | K. Sharman, N. Saunders, J. Gamei (Sony) |
| JCTVC-Q0242 | m33488 | 2014-04-01 08:44:25 | 2014-04-01 08:55:56 | 2014-04-01 08:55:56 | Proposed common software base for future screen content coding development | B. Li, J. Xu, G. J. Sullivan (Microsoft) |
| JCTVC-Q0243 | m33492 | 2014-04-01 10:30:58 | 2014-04-01 10:32:49 | 2014-04-09 17:18:21 | Software for the Screen Content Coding Model | K. Rapaka, C. Pang, J. Sole, M. Karczewicz (Qualcomm) |
| JCTVC-Q0244 | m33493 | 2014-04-01 12:33:38 | 2014-04-01 12:35:49 | 2014-04-03 12:17:39 | BoG report on screen content coding reference model (SCM) | R. Cohen (BoG coordinator) |
| JCTVC-Q0245 | m33500 | 2014-04-01 22:50:58 | 2014-04-01 23:20:17 | 2014-04-01 23:20:17 | Hash-based motion search | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0246 | m33501 | 2014-04-02 01:17:26 | 2014-04-02 23:07:40 | 2014-04-02 23:07:40 | Cross-check of JCTVC-Q0134 Solution 3 on ping-pong block vector predictor for intra block copy | P. Lai, S. Liu (MediaTek) |
| JCTVC-Q0247 | m33505 | 2014-04-02 11:07:33 | 2014-04-02 11:14:51 | 2014-04-02 11:14:51 | MV-HEVC/SHVC HLS: On frame-field related indications (follow-up of parts of JCTVC-Q0183/JCT3V-H0082 and JCTVC-Q0078/JCT3V-H0026) | M. M. Hannuksela (Nokia) |
| JCTVC-Q0248 | m33508 | 2014-04-02 15:08:44 | 2014-04-02 15:12:32 | 2014-04-10 07:37:06 | Software for SCM with hash-based motion search | K. Rapaka (Qualcomm), J. Xu (Microsoft) |
| JCTVC-Q0249 | m33509 | 2014-04-02 15:40:55 | 2014-04-02 15:44:01 | 2014-04-02 15:44:01 | Proposal of the RExt conformance test development plan | T. Suzuki, O. Nakagami (Sony) |
| JCTVC-Q0250 | m33510 | 2014-04-02 15:58:09 | 2014-04-04 11:00:01 | 2014-04-16 01:57:18 | Cross-check of JCTVC-Q0243 Software for the Screen Content Coding Model and JCTVC-Q0248 Software for SCM with hash-based motion search | P. Lai, X. Xu, S. Liu, Y.-C. Sun, T.-Z. Chuang (MediaTek) |
| JCTVC-Q0251 | m33511 | 2014-04-02 16:50:26 | 2014-04-02 17:15:33 | 2014-04-02 17:15:33 | On HEVC 4:4:4 profiles with intra block copying | J. Sole (Qualcomm) |
| JCTVC-Q0252 | m33512 | 2014-04-02 18:02:04 | 2014-04-02 18:07:41 | 2014-04-03 08:23:25 | Hash-based intraBC search | B. Li, J. Xu (Microsoft) |
| JCTVC-Q0253 | m33514 | 2014-04-03 06:36:06 | 2014-04-03 06:43:22 | 2014-04-03 06:43:22 | MV-HEVC/SHVC HLS: On semantics of layers not present SEI message | A. K. Ramasubramonian (Qualcomm) |
| JCTVC-Q0254 | m33516 | 2014-04-03 09:54:22 | 2014-04-03 12:44:39 | 2014-04-03 12:44:39 | Report of side activity on reference position derivation | K. Minoo (Arris), Y. Ye (InterDigital), J. Chen (Qualcomm), T. Yamamoto (Sharp), K. Andersson (Ericsson) |
| JCTVC-Q0255 | m33523 | 2014-04-03 17:10:49 | 2014-04-03 17:17:27 | 2014-04-03 17:17:27 | MV-HEVC/SHVC HLS: On sharing of sub-DPBs | A. K. Ramasubramonian, Hendry (Qualcomm), S. Deshpande (Sharp) |
| JCTVC-Q0256 | m33524 | 2014-04-03 19:18:03 | 2014-04-03 19:21:30 | 2014-04-03 23:02:02 | Rectangular Region Frame Packing Arrangement | G. Balocca (Sisvel) |
| JCTVC-Q1000 | m33551 | 2014-04-18 22:01:51 |  |  | Meeting report of the 17th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Valencia, ES, 27 Mar. â€“ 4 Apr. 2014 | G. J. Sullivan, J.-R. Ohm |
| JCTVC-Q1002 | m33552 | 2014-04-18 22:06:13 | 2014-06-22 19:19:04 | 2014-06-22 19:19:04 | High Efficiency Video Coding (HEVC) Test Model 15 (HM 15) Encoder Description | K. McCann, B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan |
| JCTVC-Q1003 | m33553 | 2014-04-18 22:08:02 | 2014-05-28 07:09:48 | 2014-05-28 07:09:48 | High Efficiency Video Coding (HEVC) Edition 1 Defect Report Draft 4 | Y.-K. Wang, G. J. Sullivan, B. Bross |
| JCTVC-Q1004 | m33548 | 2014-04-06 19:07:18 | 2014-05-25 22:31:14 | 2014-05-25 22:31:14 | HEVC Conformance Draft 7 | T. Suzuki, G. Sullivan, W. Wan (Editors) |
| JCTVC-Q1005 | m33545 | 2014-04-06 11:15:42 | 2014-04-06 11:20:22 | 2014-06-19 08:00:13 | High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 7 | D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, J. Sole, G. J. Sullivan, T. Suzuki (Editors) |
| JCTVC-Q1007 | m33554 | 2014-04-18 22:09:41 | 2014-06-07 09:25:59 | 2014-06-07 09:25:59 | SHVC Test Model 6 (SHM 6) | J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela |
| JCTVC-Q1008 | m33550 | 2014-04-15 02:10:21 | 2014-04-15 02:12:01 | 2014-06-21 00:58:54 | High efficiency video coding (HEVC) scalable extension Draft 6 | J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang |
| JCTVC-Q1009 | m33555 | 2014-04-18 22:10:49 | 2014-04-29 22:51:06 | 2014-04-29 22:51:06 | Common SHM test conditions and software reference configurations | V. Seregin, Y. He |
| JCTVC-Q1011 | m33518 | 2014-04-03 15:11:29 | 2014-05-18 02:34:10 | 2014-05-18 02:34:10 | HEVC verification test report | T. K. Tan, M. Mrak, V. Baroncini (Test coordinators) |
| JCTVC-Q1012 | m33549 | 2014-04-06 19:10:47 |  |  | Range extensions conformance draft 1 | T. Suzuki, K. Kazui (Editors) |
| JCTVC-Q1013 | m33546 | 2014-04-06 11:17:10 | 2014-06-11 15:06:02 | 2014-06-11 15:06:02 | HEVC Range Extensions Test Model 7 Encoder Description | C. Rosewarne, K. Sharman, M. Naccari, G. Sullivan (Editors) |
| JCTVC-Q1014 | m33556 | 2014-04-18 22:13:32 | 2014-04-28 22:34:03 | 2014-04-28 22:34:03 | Screen content coding test model 1 (SCM 1) | R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye |
| JCTVC-Q1015 | m33557 | 2014-04-18 22:14:52 | 2014-05-02 03:56:00 | 2014-05-07 20:08:50 | Common conditions for screen content coding tests | H. Yu, R. Cohen, K. Rapaka, J. Xu |
| JCTVC-Q1101 | m33558 | 2014-04-18 22:20:14 | 2014-04-29 00:37:41 | 2014-04-29 00:37:41 | HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability | A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li |
| JCTVC-Q1121 | m33544 | 2014-04-05 13:58:06 | 2014-04-05 20:38:48 | 2014-04-18 22:44:37 | HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions | J. Sole, S. Liu (CE coordinators) |
| JCTVC-Q1122 | m33543 | 2014-04-05 00:00:41 | 2014-04-08 00:57:15 | 2014-04-19 06:26:48 | HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line-based Intra Copy | C.-C. Chen, X. Xu, L. Zhang (CE coordinators) |
| JCTVC-Q1123 | m33547 | 2014-04-06 11:31:02 | 2014-04-18 08:52:12 | 2014-04-18 08:52:12 | HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode | Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma (CE coordinators) |
| JCTVC-Q1124 | m33542 | 2014-04-04 21:14:54 | 2014-04-04 21:18:05 | 2014-04-21 19:01:47 | HEVC Screen Content Coding Core Experiment 4 (SCCE4): String matching for sample coding | Y. Chen, J. Xu (CE coordinators) |
| JCTVC-Q1125 | m33534 | 2014-04-04 18:27:56 | 2014-04-04 18:32:24 | 2014-04-18 22:54:12 | HEVC Screen Content Coding Core Experiment 5 (SCCE5): Inter-component prediction and adaptive colour transforms | X. Xiu, J. Chen (CE coordinators) |

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixteenth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting (approximately 126 people in total), were as follows:

- Elena Alshina (Samsung Electronics)

- Peter Amon (Siemens AG)

- Kenneth Andersson (LM Ericsson)

- Pierre Andrivon (Technicolor)

- Jicheng Au (MediaTek)

- Cheung Auyeung (Sony)

- Giovanni Ballocca (Sisvel Tech)

- Gun Bang (ETRI)

- Guillaume Barroux (Fujitsu)

- Frank Bossen (NTT Docomo)

- Jill Boyce (Vidyo)

- Done Bugdayci Sansli (Tampere University of Technology)

- Tsui-Shan Chang (NCTU/ITRI)

- Chun-Chi Chen (NCTU/ITRI)

- Jianle Chen (Qualcomm)

- Peisong Chen (Broadcom)

- Weizhong Chen (Huawei)

- Xu Chen (Huawei Technologies)

- Ying Chen (Qualcomm)

- Yi-Wen Chen (MediaTek)

- Yongjin Cho (Samsung Electronics)

- Keiichi Chono (NEC)

- Tzu-Der Chuang (MediaTek)

- Takeshi Chujoh (Toshiba)

- Robert Cohen (Mitsubishi Electric Res. Labs)

- Sachin Deshpande (Sharp)

- Alexey Dolgoborodov (Vanguard Software Solutions)

- Alberto Duenas (NGCodec)

- David Flynn (Blackberry)

- Chad Fogg (Harmonic)

- Edouard François (Technicolor)

- Arild Fuldseth (Cisco Systems Norway)

- Dan Grois (Fraunhofer HHI)

- Miska Hannuksela (Nokia)

- Ryoji Hashimoto (Renesas Electronics)

- Shinobu Hattori (Sony)

- James Helman (MovieLabs)

- Fnu Hendry (Qualcomm)

- Yu-Wen Huang (MediaTek USA Inc.)

- Atsuro Ichigaya (Japan Broadcasting Corporation (NHK))

- Tomohiro Ikai (Sharp)

- Byeungwoo Jeon (Sungkyunkwan Univ. (SKKU))

- Jung Won Kang (ETRI (Electronics and Telecommunications Research Institute))

- Marta Karczewicz (Qualcomm Technologies)

- Kei Kawamura (KDDI)

- Kimihiko Kazui (Fujitsu Labs)

- Dae Yeon Kim (Chips & Media)

- Hui Yong Kim (ETRI)

- Seung Hwan Kim (Sharp)

- Peter Kuhn (European Patent Office)

- PoLin (Wang) Lai (Mediatek USA)

- Jani Lainema (Nokia)

- Guillaume Laroche (Canon Research)

- Sebastien Lasserre (Technicolor)

- Thorsten Laude (Leibniz Univ. Hannover)

- Bae-Keun Lee (KT)

- Hahyun Lee (Electronics and Telecommunications Research Institue (ETRI))

- Sukho Lee (ETRI)

- Sunil Lee (Samsung Electronics)

- Shawmin Lei (MediaTek USA)

- Ming Li (ZTE)

- Xiang Li (Qualcomm Technologies)

- Chong Soon Lim (Panasonic)

- Ching-Chieh Lin (ITRI International)

- Jian-Liang Lin (MediaTek Inc.)

- Tao Lin (Tongji University)

- Shan Liu (MediaTek USA)

- Ajay Luthra (Arris)

- Zhan Ma (Huawei)

- Vinod Kumar Malamal Vadakital (Nokia)

- Gaëlle Martin-Cocher (Blackberry)

- Shohei Matsuo (NTT)

- Ken McCann (Zetacast / Samsung)

- Akira Minezawa (Mitsubishi Electric)

- Koohyar Minoo (Arris)

- Kazuyuki Miyazawa (Mitsubishi Electric)

- Matteo Naccari (BBC R&D)

- Ohji Nakagami (Sony corporation)

- Takahiro Nishi (Panasonic)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Patrice Onno (Canon Research Centre France)

- Chanyul Park (Samsung Electronics)

- Wen-Hsiao Peng (ITRI International/NCTU)

- Adarsh Krishnan Ramasubramonian (Qualcomm Technologies)

- Krishnakanth Rapaka (Qualcomm Technologies)

- Justin Ridge (Nokia Oyj)

- Christopher Rosewarne (CiSRA / Canon Inc)

- Damian Ruiz (Univ. Politec. de Valencia)

- Jesus Sampedro (Polycom)

- Nicholas Saunders (Sony Europe Broad. & Prof. Res. Labs)

- Karl Sharman (Sony Europe Broad. & Prof. Res. Labs)

- Masato Shima (Canon)

- Shinya Shimizu (NTT)

- Rickard Sjöberg (Ericsson AB)

- Joel Sole (Qualcomm Technologies)

- Nikolce Stefanoski (Disney Research Zurich)

- Karsten Sühring (Fraunhofer HHI)

- Gary Sullivan (Microsoft)

- Huifang Sun (Mitsubishi Electric Res. Labs)

- Teruhiko Suzuki (Sony)

- Maxim Sychev (Huawei Technologies)

- Ali Tabatabai (Sony)

- Thiow Keng Tan (NTT Docomo)

- Gerhard Tech (Fraunhofer HHI)

- Pankaj Topiwala (FastVDO)

- Alexandros Tourapis (Apple)

- Yi-Shin Tung (ITRI USA / MStar Semiconductor)

- Kemal Ugur (Nokia)

- Glenn Van Wallendael (Ghent University - iMinds)

- Anthony Vetro (Mitsubishi Electric Res. Labs)

- Wei Wang (Huawei)

- Ye-Kui Wang (Qualcomm Technologies)

- Stephan Wenger (Vidyo)

- Thomas Wiegand (Fraunhofer HHI)

- Ping Wu (ZTE UK)

- Xiaoyu Xiu (InterDigital Commun.)

- Jizheng Xu (Microsoft)

- Tomoyuki Yamamoto (Sharp)

- Haitao Yang (Huawei Technologies)

- Yan Ye (InterDigital Commun.)

- Peng Yin (Dolby Labs)

- Haoping Yu (Huawei USA)

- Xiaozhen Zheng (Huawei Technologies)

- Yunfei Zheng (Apple)

- Minhua Zhou (Broadcom)

- Jianqing Zhu (Fujitsu R&D Center)