| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0085 |
| --- | --- |

| Title: | High Dynamic Range / Wide Color Gamut workflow |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bill Mandel 100 Universal City Plaza Universal City, CA 91608 USA | Email: | bill.mandel@nbcuni.com |
| Source: | Universal Pictures |  |  |

_____________________________

# Abstract

This contribution describes the High Dynamic Range / Wide Color Gamut (HDR/WCG) workflow process applied in the simulation results provided in JCTVC-Q0087 and JCTVC-Q0088 for Main10 and Main12 coded video.  The video container aspects of the encoding are intended to be conveyed in the video characteristcs information of VUI (HEVC Annex E) , such as YCbCr BT.2020 and XYZ primary color space derivations, in combination with PQ and BT.1886 Electro-Optic Transfer Functions (EOTF).

# Introduction

The basic workflow process consist of:  (1) format conversion from source file to HEVC encoder input (2) signalling of metadata into HEVC VUI  (3) encoding HEVC with HDR/WCG-friendly parameters.

The format conversion process consists of a series of mappings, from camera-specific “scene referred” to “display referred” as described in [1,2].

.bin

# Source code

The following sources are necessary to follow the recipe above.

## OpenEXR

The platform operating system running the experiment may already include OpenEXR version 1.7 or higher.  If the local copy of OpenEXR is updated from an earlier install, some tools such as ImageMagick and image viewers that depend upon OpenEXR may need to be recompiled, otherwise they may fail to open EXR files after the update.

https://github.com/openexr/openexr/releases

It is recommended to test and install IlmBase after unpacking OpenEXR source ---- then build, test and install OpenEXR. Some tests may fail due to computation precision differences. This issue is currently being investigated by the github project maintainers (see project notes for explanations), and should not be a concern for the purposes of following this workflow.

## ACES

The latest development branch of ctl contains the format conversion tools used in this workflow.  Only the ctlrender tool is necessary to run .ctl scripts.

https://github.com/ampas/CTL/releases

Follow the build instruction on:

https://github.com/ampas/CTL

The ACES scripts for such steps needed for RRT and ODT are available here. V0.7.1 was tested. These are useful to automatically manipulate wide dynamic / gamut range files through a standard process if no traditional manual color grading or output conversion step for reference monitor viewing is available.

https://github.com/ampas/aces-dev/releases

## TIFF Conversion tools

The following tools (copyright Bill Mandel, Universal Pictures) are provided to convert the display referred images into the raw container that the HM HEVC reference encoder can exchange.

### Tiff to YUV container

This is the primary tool to convert an array of ODT TIFF files to a .yuv planar file of concatenated images that HEVC reference encoder HM expects:

To compile:

g++ -O3 tiff2ydzdx.cpp -o tiff2ydzdx -ltiff

Use:

tiff2ydzdx [tiff file] [options]

Command line options: BOX, FULL, 709, 2020, HD1920, HD960, B10, B14, Y100, Y500

- Color formula (default is Y'DzDx) use 709 or 2020 respectively for others

- Bit Depth (default is 12 bits) use B10 for 10 bits

- Resolution (default is 3840x2160) options are HD1920 or HD960. Resolution is achieved by center cutting if the image is bigger than the resolution.

- (e.g. tiff2ydzdx Frame.tif 2020 B10 would subsample and append Frame.tif using a 2020 formula and 10 bits onto YDzDx.yuv)

### YUV container back to TIFF

This tool converts, for example, the decoded output of HEVC back to individual TIFF files for quality analysis.

To compile:

g++ -O3 ydzdx2tiff.cpp -o ydzdx2tiff –ltiff

Use:

ydzdx2tiff  [tiff file] [options]

Command line options: BOX, FULL, 709, 2020, HD1920, HD960, Y100, Y500, B10, B14

- Color formula (default is Y'DzDx) use 709 or 2020 respectively for others

- Bit Depth (default is 12 bits) use B10 for 10 bits

Resolution (default is 3840x2160) options are HD1920 or HD960. 
(e.g. ydzdx2tiff Frame.tif 2020 B10 would subsample and append Frame.tif using a 2020 formula and 10 bits onto YDzDx.yuv)

### Compare source to decoded

This tool generates a difference image between two TIFF images. For example: input to encoder vs. output of decoder.

To compile:

g++ -O3 tifcmp.cpp -o tifcmp –ltiff

Use:

tifcmp [first tiff image] [second tiff image] [options]

Command line options:  B10, B16,  -g [gain],  -o [offset],

- Bit Depth (default is 12 bits) use B10 for 10 bits or B16 for the whole tiff files to be compared.

- Default is the difference in each channel is coded into an 8-bit sRGB TIFF.

- Gain -g and offset -o options provided to allow for creation of a difference image centered around gray. (e.g.  tifcmp f1.tif f2.tif -g 2.0 -o 128 would double the difference between the files and add to 128 before writing into the 8-bit TIFF)

## Included tools and scripts

The following source files are attached to the .zip archive (/src subdirectory) of this document:

Tools:

Tiff2ydzdx.cpp

Ydzx2tiff.cpp

Tifcmp.cpp

ctlrender scrips:

PQ.ctl

odt_PQ10k.ctl

odt_PQ10k2020.ctl

INVPQ10k-2-XYZ.ctl

INVPQ10k2020-2-XYZ.ctl

Otd_rec709_full_500nits.ctl

Scale10k.ctl

Scale10k.ctl

XYZ2ACES.ctl

Example process scripts

encodeHD.sh

2020-2-709-500.sh

## HEVC reference encoder

A copy of the HEVC HM reference coder is available here:

svn co https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0/

cd ./HM-13.0+RExt-6.0/build/linux

make all_highbitdepth

## Other tools

LibTiff:  needed to compile the tiff2yuv containers.

ImageMagick, GNU Plot, Octave

# Steps for converting .exr source to HEVC HM encoder input

## Converting .exr to BT.2020 PQ-EOTF

The following serial procedures convert the .exr master files into the BT.2020 PQ-EOTF container format passed as a .yuv planar input file into the HEVC HM reference encoder.

### Step 1: ICES to ACES RRT

Ctlrender can combine several conversion steps with one command line chain, each initiated with –ctl. For illustration purposes, these steps shall be shown serially.

For each .exr source frame, run the RRT to convert the master file in ICES space to ACES

ctlrender -force -verbose –ctl rrt.ctl  input.exr -format tiff16 rrt_output.tif

Note that in the current release of ctlrender, the .ctl scripts in aces-dev-0.7.1/transforms/ctl need to be in the path (for example, copied in the current execution directory) for rrt.ctl to work.

Alternatively, if starting with a S-gamut log file such as FruitStall [5], combine the first step of S-log to ACES, followed by the RRT:

ctlrender -verbose -force –ctl S-gamut_to_ACES.ctl -ctl rrt.ctl  -format tiff16 rrt_output.tiff

### Step 2: RRT to 2020 PQ ODT

ctlrender  -verbose -ctl odt_PQ10k2020.ctl rrt_output.tiff -format tiff16 odt_output.tiff

### Step 3: ODT to encoder input

For 10-bit:

tiff2ydzdx odt_output.tiff  2020 B10

For 12-bit

tiff2ydzdx odt_output.tiff  2020 B12

Currently, the tiff2ydzdx program always writes a file called YDzDx.yuv regardless of whether the container is BT.2020 or YDzDx or BT.709.

## Converting .exr to YDxDx-color PQ-OETF

For YDzDx PQ-EOTF, the steps are similar to the BT.2020 PQ example above, but the odt_PQ10k.ctl script is called that is designed for XYZ color space. Here, we combine both the RRT and ODT steps into the same command line (serial order is important).

ctlrender -force -verbose –ctl rrt.ctl -ctl odt_PQ10k.ctl input.exr -format tiff16 odt_output.tiff

The default values of tiff2yzdx are all that is needed:

tiff2ydzdx odt_output.tiff

## .exr to BT.709

As with BT.2020 PQ example above, the steps are similar for BT.709:

ctlrender -force -verbose –ctl rrt.ctl -ctl odt_rec709_full_500bits.ctl input.exr -format tiff16 odt_output.tiff

The above .ctl aims to fit 500 nit max display luminance into an eventual 10-bit .yuv container..

…and the final format conversion step:

tiff2ydzdx odt_output.tiff 709  B10

..or, for 12-bits:

tiff2ydzdx odt_output.tiff 709  B12

# HEVC HM encoder parameters

An example encoder setting:

TAppEncoderStatic -c main12.cfg -i YDzDx.yuv -wdt 1920 -hgt 1080 -fr 25 -f $frames -fs $fs --InputBitDepth=12 --ExtendedPrecision=0 --InternalBitDepth=12 --InputChromaFormat=420 -vui 1 --VideoFullRange=0 --Profile=main-RExt --Tier=main --Level=4.1  -b str$qp.bin -o rec$qp.yuv

Note: main12.cfg is the standard encoder_random_access_main10.cfg. BitDepth in the .cfg is overridden by the –InternalBitDepth=12 command line argument as per Main12 profile.

# Decode and post-process

[ to be added later ]

# References

[1] E. Giorgianni, T. Madden, M. Kriss (2009) “Digital Color Management: Encoding Solutions”, Wiley-Blackwell, ISBN 978-0-470-51244-9

[2] J. Morovic, (2008) “Color Gamut Mapping”, Wiley, ISBN 978-0470030325

[3] R. Mantiuk, K. Myszkowski, H-P. Seidel, “Lossy Compression of High Dynamic Range Images and Video”

[4] R. Mantiuk, K. Myszkowski, H-P. Seidel, “High Dynamic Range Image and Video Compression – Human Visual Performance.”

[5] K. Sharman, N. Saunders, “16-bit Test Sequences for Range Extensions”,  JCTVC-O0069.

# Patent rights declaration(s)

Universal Pictures does not have any current or pending patent rights relating to the technology described in this contribution.