| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R_Notes_d2 |
| --- | --- |

| Title: | Meeting report of the 18th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Sapporo, JP, 30 June – 9 July 2014 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighteenth meeting during 30 June – 9 July 2014 at the Sapporo Convention Center, Sapporo, JP. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 1600 hours on Monday 30 June 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 9 July 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the seventeenth JCT-VC meeting in producing the 15th HEVC Test Model (HM15) software and text, the version 1 conformance, the range extensions (RExt) Draft 7 (to be issued as new edition along with version 1 corrigenda items) with associated test model, software and conformance, the Scalable HEVC (SHVC) extensions Draft 6 with associated test model and software, review the results from one interim Core Experiments on SHVC (SCE1) and five Core Experiments on Screen Content Coding (SCCE1), review the input documents on common high level syntax for SHVC and 3D extensions, and review other technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC and screen content coding tools. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 and RExt were also considered.

In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting (to be updated):

- For HEVC version 1, the HEVC test model (HM) 15, HEVC Defect Report draft 4 (for Version 1), HEVC conformance testing draft 7, and the HEVC version 1 verification test report;

- For HEVC range extensions (RExt), the HEVC range extensions draft 7 (final specification draft submitted for ISO/IEC approval), RExt test model 7, and RExt conformance draft 1;

- For HEVC scalable extensions (SHVC), the SHVC extensions draft 6 and SHVC Test Model 6, and a document specifying common test conditions and software reference configurations for SHVC experiments;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 17–24 October 2014 under WG 11 auspices in Strasbourg, FR, during 10–18 Feb. 2015 under ITU-T auspices in Geneva, CH, during 18–26 June 2015 under WG 11 auspices in Warsaw, PL, and during XX–XX Oct. 2015 under ITU-T auspices in XXXX, XX.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its eighteenth meeting during 30 June – 9 July 2014 at the Sapporo Convention Center, Sapporo, JP. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 1600 hours on Monday 30 June 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 9 July 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2014_06_R_Sapporo/.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the seventeenth JCT-VC meeting in producing the 15th HEVC Test Model (HM15) software and text, the version 1 conformance, the range extensions (RExt) Draft 7 (to be issued as new edition along with version 1 corrigenda items) with associated test model, software and conformance, the Scalable HEVC (SHVC) extensions Draft 6 with associated test model and software, review the results from one interim Core Experiments on SHVC (SCE1) and five Core Experiments on Screen Content Coding (SCCE1), review the input documents on common high level syntax for SHVC and 3D extensions, and review other technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC and screen content coding tools. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 and RExt were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 20 June 2014.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Saturday 21 June 2014 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-R0242 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "R0242+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-R0XXX (a proposal on XXX) [uploaded 06-XX],

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-R0XXX (an information document about XXX) [uploaded 06-XX],

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-R0XXX [uploaded 06-XX], ….

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-R0XXX, ….

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution document was rejected as a "placeholder" without any significant content and was not corrected until after the upload deadline:

- JCTVC-R0XXX (a proposal on XXX, corrected by a late upload on 06-XX)

- …

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-Q1000, the HEVC Test Model 15 (HM15) JCTVC-Q1002 (which however is almost identical with HM14 and considerably lacks quality of encoder description), the Defect Report JCTVC-Q1003, the Conformance Draft JCTVC-Q1004, the Draft Specification of Range Extensions JCTVC-Q1005, the RExt Test Model JCTVC-Q1013 and Conformance Draft JCTVC-Q1012, the SHVC draft specification JCTVC-Q1008, the SHVC test model JCTVC-Q1007, the SCC test model JCTVC-Q1014, the common test conditions for SHVC (JCTVC-O1009) and SCC (JCTVC-Q1015), and the HEVC version 1 verification test report JCTVC-Q1011 were approved. The HM reference software and the reference software versions for range extensions, SHVC and SCC were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below (some additions are needed for SCC):

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-acess skip.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC).

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- OLS: Output layer set

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Rancom-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC).

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a draft standard.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing

- Block and unit names:

  - CTB: Coding tree block (luma or chroma).

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU)

  - CB: Coding block (luma or chroma).

  - CU: Coding unit (containing both luma and chroma).

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma)

  - PU: Prediction unit (containing both luma and chroma), with four shape possibilities for non-AMP cases.

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN: Having two areas that each have the full width and half the height of the CU.

    - Nx2N: Having two areas that each have half the width and the full height of the CU.

    - NxN: Having four areas that each have half the width and half the height of the CU.

  - TB: Transform block (luma or chroma).

  - TU: Transform unit (containing both luma and chroma).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

Opening remarks included:

- Remarks on lateness of documents.

- Remarks on the number of documents.

- Publication of RExt – suggested plan to issue new edition replacement FDIS including SHVC and MV-HEVC, since prior FDIS ballot not yet begun

Primary topic areas were noted as follows:

- HLS

- SHVC

- RExt and pseudo-RExt – little input (R0128)

- Corrigenda items, preparation of new edition (how to progress RExt to final approval ballot in ISO/IEC) – no input

- Screen content coding

Other

- Verification test for interlaced video – no input

- Reference software and conformance, RExt & SHVC

Unfinished (or less-than-optimally finished) deliverables

- HM test model text

Deliverables from this meeting

- SHVC FDIS / FDAM / Consent

- HEVC conformance FDIS / Consent

- HEVC reference software FDIS / Consent

Two main tracks were followed for most meeting discussions:

- HLS

- SCC

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on SHVC HLS, SHVC and RExt CE work and related contributions during the first few days. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Mon. 30 June, 1st day 1600–2030

  - JCT-VC opening and review of AHG reports [JRO & GJS]

  - …

- Tue. 1 July, 2nd day 0900–2000 (2145)

  - JRO: SCCEs

  - GJS: Phase, CGS, HLS

  - VCEG Q6/16 3Q (1400–1515)

  - …

  - Welcome reception 1720–1920

  - …

- Wed. 2 July, 3rd day 0845–2000 (2145)

  - …

- Thu. 3 July, 4th day 0845–2000 (2145)

  - JCT-3V 0900–…

  - VCEG Q6/16 2Q (1045–1200) TBD

- Fri. 4 July, 5th day 0845–2000 (2145)

  - …

- Sat. 5 July, 6th day 0845–2000 (2145)

  - …

- Sun. 6 July, 7th day 0845–2000 (2145)

  - …

- Mon. 7 July, 8th day 0845–2000 (2145)

  - VCEG Q6/16 2Q (1045–1200)

  - VCEG Q6/16 4Q (1545–1700)

  - …

- Tue. 8 July, 9th day 0845–2000 (2145)

  - VCEG Q6/16 4Q (1545–1700)

  - …

- Wed. 9 July, 10th day 0845–1300

  - VCEG Q6/16 2Q (1045–1200) ??

  - …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks was subject to changes)

- AHG reports (17) Track P (section 2)

- Coding performance, implementation, and design analysis (1) Track P (section 3.4)

- Profile and level definitions (2) Track P/Joint with Parent Bodies (section 3.5)

- Source video test material (2) Track P (section 3.7)

- SHVC CE1: Colour gamut and bit depth scalability (8) Track A (section 4)

- SCC CE1: Intra block copy (31) Track B (section 5.1)

- SCC CE2: Line-based intra bock copy (5) Track B (section 5.2)

- SCC CE3: Palette mode (50) Track B (section 5.3)

- SCC CE4: String matching (10) Track B (section 5.4)

- SCC CE5: Inter-component prediction and adaptive colour transform (11) Track B (section 5.5)

- Non-CE SHVC (11) Track A (section 6.1)

- Non-CE SCC (108) Track B (section 6.2)
CE1 related 40, CE2 related 4, CE3 related 43, CE4 related 3, CE5 related 7, other 11

- High-level syntax common issues in RExt, 3D, SHVC, single layer (1) BoG | Track A (section 6.3)

- High-level syntax in SHVC and 3D extensions (30) BoG | Track A | joint with JCT-3V (section 6.4)

- VUI and SEI messages (11) BoG | Track A (section 6.5)

- Non-normative (1) (section 6.6)

- Plenary discussions and BoG reports (0) Track P (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 9, 10, and 11)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports (17)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Reviewed Mon 30th p.m., jointly chaired by GJS & JRO, except as noted otherwise.)

JCTVC-R0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 17th JCT-VC meeting, the 15th HEVC Test Model (HM15) software and text had been produced, one Core Experiment on scalable extensions (SCE1) and five Core Experiments on screen content coding tools (SCCE1…5) were run. In preparation of the 18th meeting, progress was made towards definitions of Scalable HEVC (SHVC) extensions, while the specification text for range extensions into higher bit depths and non-4:2:0 colour sampling was finalized (integrated into text for new edition along with miscellaneous version 1 corrigenda items). Furthermore, an initial test model (and software) for screen content coding tools was set up. Advancing the work on development of conformance and reference software for HEVC and its extensions was also a significant goal. Further needs for corrections to version 1 were considered, and after finalization, the report on the verification test for HEVC version 1 performance testing was finalized.

The work of the JCT-VC overall had proceeded well and actively in the interim period with an increased number of input documents to the current meeting. Active discussion had been carried out on the group email reflector (which had 1751 subscribers as of 2014-06-30), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2014_03_Q_Valencia/ ), particularly including the following:

- The meeting report (JCTVC-Q1000) [Posted 2014-06-30]

- The HM 15 encoder description (JCTVC-Q1002) [Posted 2014-06-22] (which however is almost identical with HM14 and considerably lacks quality of encoder description)

- The HEVC (version 1) defect report draft 4, to be submitted as integrated text with ISO/IEC FDIS 23008-2:201X 2nd edition (JCTVC-Q1003) [Posted 2014-05-28]

- The HEVC conformance specification Draft 7, to be submitted as Preliminary Draft of ISO/IEC FDIS 23008-8 (JCTVC-Q1004) [Posted 2014-05-25]

- HEVC range extensions Draft 7, as integrated text for ISO/IEC FDIS 23008-2:201X 2nd edition (JCTVC-Q1005) [First posted 2014-04-0, last updated 2014-06-19]

- SHVC Test Model 6 (JCTVC-Q1007) [Posted 2014-06-07]

- SHVC Draft 6, submitted as ISO/IEC Study of DAM (JCTVC-Q1008) [First posted 2014-04-15, last updated 2014-06-21]

- Common SHM test conditions and software reference configurations (JCTVC-Q1009) [Posted 2014-04-29]

- HEVC verification test report (JCTVC-Q1011) [Posted 2014-05-18]

- Range Extensions conformance draft (JCTVC-Q1012) [Posted 2014-06-24]

- Range Extensions Test Model 7 encoder description (JCTVC-Q1013) [Posted 2014-06-11]

- Screen Content Coding Test Model 1 (JCTVC-Q1014) [Posted 2014-04-28]

- Common SCC test conditions and software reference configurations (JCTVC-Q1015) [First posted 2014-05-02, last updated 2014-05-07]

- Description of HEVC Scalable Extensions Core Experiment 1 (SCE1): Color gamut and bit depth scalability (JCTVC-Q1101) [Posted 2014-04-29]

- Description of HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra block copy (JCTVC-Q1121) [First posted 2014-04-05 with final update 2014-04-18]

- Description of HEVC Screen Content Coding Core Experiment 2 (SCCE2): Line based intra copy (JCTVC-Q1122) [First posted 2014-04-08 with final update 2014-04-19]

- Description of HEVC Screen Content Coding Core Experiment 3 (SCCE3): Palette mode (JCTVC-Q1123) [Posted 2014-04-18]

- Description of HEVC Screen Content Coding Core Experiment 4 (SCCE4): String matching for sample coding (JCTVC-Q1124) [First posted 2014-04-04 with final update 2014-04-21]

- Description of HEVC Screen Content Coding Core Experiment 5 (SCCE5): Inter-component prediction and adaptive colour transform (JCTVC-Q1121) [First posted 2014-04-04 with final update 2014-04-18]

The various ad hoc groups and the six core experiments had made progress, and various reports from those activities had been submitted.

The different software modules (HM, RExt, SHVC and Screen Content) had been prepared and released approximately as scheduled.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-15.0.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-15.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 300 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted, even though most were cross-check documents.

A preliminary basis for the document subject allocation and meeting notes for the 18th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-R0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (AHG co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 17th meeting in Valencia (27 March – 4 April 2014) and the 18th meeting in Sapporo (30 June – 9 July 2014).

Documents JCTVC-P1002 and JCTVC-P1003 were approved as JCT-VC output documents at the 17th JCT-VC meeting.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.  No issues with either the HEVC specification (JCTVC L1003 and JCTVC-P1003) or the HM14 Encoder Description (JCTVC-P1002) were reported on the tracker during the period of this report.

The HM15 Encoder Description was published as JCTVC-Q1002.

The HEVC Edition 1 Defect Report Draft 4 was published as JCTVC-Q1003.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Approve the HM15 Encoder Description in document JCTVC-Q1002 as JCT-VC output

- Approve the HEVC Edition 1 Defect Report Draft 4 in document JCTVC-Q1003 as JCT-VC output

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description

- Identify and resolve any issues relating to mismatches between software and text

- Consider how best to organize future work on editorial improvements to the Encoder documentation for both HEVC version 1 and the Range Extensions

JCTVC-R0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3)

This report summarises the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 17th and 18th JCT-VC meetings. Activities focused on fixing bugs.

The mandates given to the AhG are as follows:

- Coordinate development of the HM software and its distribution.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver HM 15.0 software version and the reference configuration encodings according to JCTVC-L1100 common conditions.

- Prepare and deliver additional "dot" version software releases and software branches as appropriate.

- Suggest configuration files for additional testing of tools.

- Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text.

A brief summary of activities related to each mandate is given below.

- Development of the software was co-ordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN servers set up at HHI and the BBC, as announced on the JCT-VC email reflector.

- The HM user manual has been updated and a version controlled copy is included in the doc directory of the repository. A PDF version has been produced and is included in the same location prior to each HM release.

- Version 15.0 is still in development to be released during the 18th JCT-VC meeting. A number of bugs have been identified and fixed.

- There are a number of reported software bugs that should be fixed.

A detailed history of all changes made to the software can be viewed at https://hevc.hhi.fraunhofer.de/trac/hevc/timeline.

Released versions of the software are available on the SVN server at the following URL: 
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-11.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-11.0-dev).

HM 14.0 had been released on April 3, 2014.

The coding performance did not change compared to HM-13.0.

HM 15.0 had not yet been released, but is expected during the 18th JCT-VC meeting. This version contains additional bug fixes and cleanups.

Unless the release has been tagged, the development branch can be found under
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-14.0-dev

There were no performance changes expected for HM 15.0.

Recommendations of the AHG were to:

- Continue to develop reference software based on HM version 15.0 and improve its quality.

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bitstreams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bistreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

HEVC conformance test development (AHG4) [miss]

TBP

Verification test preparation (AHG5) [miss]

TBP

Some progress on range extensions; perhaps not for interlace

JCTVC-R0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan, T. Suzuki (AHG chairs)]

This document reports on the work of the JCT-VC ad hoc group on Range extensions draft text (AHG6) between the 17th JCT-VC meeting in Valencia, ES (March-April 2014) and the 18th JCT-VC meeting in Sapporo, JP (June-July 2014).

The High Efficiency Video Coding (HEVC) Range Extensions test model was developed and finalized following the decisions taken at the 17h JCT-VC meeting in Valencia, ES (March-April 2014).

Nine versions of JCTVC-Q1005 were published by the Editing AhG following the 17th JCT-VC meeting in Valencia, which were based upon JCTVC-L1003_v34 and JCTVC-Q1003_v1. First 7 drafts were based upon JCTVC-L1003_v34. The text was rebased upon Q1003_v1 in the last two versions. The text of JCTVC-Q1005 (revision 9) was submitted to ISO/IEC JTC 1/SC 29 for Final Draft International Standard (not yet issued, pending further guidance).

A detailed description of the topics addressed in the editing work was provided in the report.

The recommendations of the HEVC Range extensions draft text AHG are to:

- Approve the edited JCTVC-Q1005 document as JCT-VC output

- Encourage the use of the issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc/) to facilitate the reporting of issues with the text

- Resolve all text issues logged in trac

- Compare the HEVC Range Extensions document with the HEVC Range Extensions software and resolve any discrepancies that may exist, in collaboration with the Range extensions software development (AHG7)

JCTVC-R0007 JCT-VC AHG report: Range extensions software (AHG7) [K. Sharman, D. Flynn (AHG chairs)]

This report summarizes the activities of Ad Hoc Group 7 on support for range extensions between the 17th and 18th JCT-VC meetings.

There was a significant effort to produce the various revisions in a timely manner, given that the screen-content coding model (SCCM) is dependent on the RExt model, which itself is dependent on the upgrades of the main HM model. This resulted in the upgrade of the HM13.0_RExt6.2 software to HM14.0_RExt6.2 on the final day (Friday 4th April) of the Valencia meeting.

On Wednesday 9th April, HM14.0_RExt7.0_rc1 was released.

This was followed two days later (11th April) with HM14.0_RExt7.0_rc2.

This revision was tagged on the SVN HHI repository as HM14.0_RExt7.0; however, on inspection of some of the simulation results, a bug was identified in one of the patches (#1281).

On Thursday 24th April, HM14.0_RExt7.1_rc1 was uploaded to the repository, which eventually became HM14.0_RExt7.1 on Tuesday 29th April.

On the same day, HM14.0_RExt7.2 was released, in which the SCC hooks and the intra block copy tool were removed. The changes between RExt7.1 and RExt7.2 were not merged into the SCCM software branch.

More recently, effort has been made in clearing some minor discrepancies between the text and software, including creating valid profile constraint flag combinations in the encoder, and providing an alternative method of specifying the RExt profile from the command line.

The items reported as currently outstanding were:

- Q0256: Segmented rectangular frame packing arrangement SEI

- Increase flexibility for the time code SEI message

- Macro removal

Recommendations were to:

- Update encoder and documentation to be consistent with the current usage.

- Continue to identify bugs and discrepancies with text, and address them.

- Test reference software more extensively.

An accompanying Excel sheet included a comparison of coding performance between software versions.

JCTVC-R0008 JCT-VC AHG report: Screen content coding (AHG8)

This report summarizes the activities of the JCT-VC ad hoc group on screen content coding (AHG8) between the JCT-VC 17th meeting in Valencia, ES, and the 18th meeting in Sapporo, Japan.

Many email messages were posted during the early weeks of this interim period, discussing issues related to total PSNR calculation in the case when some frames in a sequence are encoded without any loss. Following the suggestions, we have added MSE-based PSNR values in the test results reporting template, and suggested to the SCM coordinators that the PSNR values for lossless coded frames should be resolution and bit-depth dependent, as suggested in JCTVC-Q0053.

Based on the comments, suggestions, and agreements through email discussions, and in collaboration with the SCM and SCCE coordinators, the following content was included in JCTVC-Q1015:

“To facilitate the evaluation of the test results, sequence MSE results and MSE-based PSNR values are added to the test results reporting templates. Therefore, the following settings must be applied to the encoder:

- MSEBasedSequencePSNR=1

- PrintFrameMSE=1

- PrintSequenceMSE=1 (Note: this setting has to be entered at the command line for the SCM-1.0 software, as it is not included in the encoder configuration files mentioned above.)”

The rest of JCTVC-Q1015 is basically the same as the test conditions for the CfP. Below is a summary of coding modes, configurations, encoding formats and QP values:

| Coding modes: | Lossy Mathematically lossless |
| --- | --- |
| Configurations: | All intra (AI) Random access (RA) Low-delay B (LB) |
| Encoding formats: | R’G’B’ 4:4:4 Y’CbCr 4:4:4 |
| QP values: | Lossy: 22, 27, 32, 37 Lossless: 0 |

Most effort has been focused on the 5 technical areas that are under testing in SCCE1 to 5. SCCE coordinators did good job to keep CE participants well informed with the test plan, and most CE participants have made their deliverables on schedule.

The contributions that are related to the general subject of screen content coding were listed and categorized in the report.

JCTVC-R0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) [J. Boyce, M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu]

This report summarizes the activities of the ad-hoc group of high-level syntax for HEVC extensions (AHG9) between the 17th JCT-VC meeting and the 18th JCT-VC meeting.

Contribution JCTVC-R0049 was particularly noted. It proposes allowing a VPS with a non-zero nuh_layer_id to be delivered to the decoder in addition to a "base" VPS with nuh_layer_id equal to 0, both with the same vps_video_parameter_set_id value. The VPS with a non-zero nuh_layer_id is required to be a semantic superset of the base VPS and may for example include information about additional layers. The VPS with the highest nuh_layer_id value among the VPSs with the same vps_video_parameter_set_id value is activated.

No other coordinated AHG activity took place, and no other emails were exchanged over the JCT-VC reflector.

There are about 40 input contributions on the topic areas of AHG9.

JCTVC-R0010 JCT-VC AHG report: Layered coding constraint specifications and capability indications (AHG10) [G. J. Sullivan (coordinator)]

This document reports on the work of the JCT-VC and JCT-3V ad hoc group on layered coding constraint specifications and capability indications, including a report of the teleconferences held and the progress made during the interim period since the preceding JCT-VC and JCT-3V meetings. (The reader of this report is assumed to be familiar with various jargon and abbreviations used in JCT-VC and JCT-3V; recent meeting reports and the draft specification text for HEVC and its SHVC and MV-HEVC extensions can be consulted for definitions of terms.)

The AHG held 9 teleconferences of 2.5–3.5 hours duration each, with substantial discussions of the issues. Approximately 10–20 people participated in each call – each of which was announced with at least 7 days of advance notice. The calls were held at different times of day to ensure reasonable opportunities for participation from different time zones (with the last few calls held during the mornings in the U.S. West Coast time zone by unanimous agreement, as this was a convenient time for all participants).

The AHG reached agreement on a number of conclusions and recommendations.

The AHG concluded the following, according to its understanding:

- That decoder conformance requirements (versus bitstream conformance requirements) had not been specified in the current draft and should be added by the editors. Also, the profile_idc values had not been assigned and should be added. Our understanding of the intent is that Scalable Main and Stereo Main decoders should be capable of decoding Main profile bitstreams (at least when the base layer is not provided by external means).

- Regarding the handling of auxiliary pictures and target output layers of output layer sets:

  - When the target output layers of an output layer set do not include the auxiliary pictures, the bits of the auxiliary pictures in the extracted bitstream subset would be counted in the CPB buffer flow, but the decoding of the auxiliary pictures would not be part of the associated decoding process requirements.

  - When the target output layers of an output layer set do include the auxiliary pictures, the profile specification for (partitions of) the output layer set would need to include decoding process requirements for decoding those pictures. Unless some way is found to specify the necessary decoding process capabilities (e.g., by a profile definition that includes such capabilities), the specification would need to disallow having target output layers of an output layer set that are auxiliary pictures.

  - It appears that layer sets can include any layers, and output_layer_flag[ i ][ j ] can be set without constraints (except for the requirement that each output layer set contain at least one target output layer). This appears to make it possible to have other (auxiliary or non-auxiliary) pictures that are included in an output layer set but are not target output layers of the output layer set and are not in layers that are needed for decoding the target output layers (e.g., an independently coded layer). It was suggested that the profile/tier/level information for such an output layer set should include all the layers in the output layer set in the CPB operation but not include the "unnecessary" layers in its decoding process requirements. In principle, it was suggested to be desirable to support this possibility (esp. for the future extensibility purpose), and suggested that we should conduct further study to ensure that the specification is consistent with this design intent. However, there was no clear agreement that this needed to be supported, pending further study.

- Regarding the case where a base layer is provided by external means, it still seems possible that we may not need a separate profile for the external base layer case, if the specification is drafted appropriately, as the question of whether the base layer is provided within the bitstream or by external means could be a system configuration question rather than a profiling issue. However, this question seemed to remain in need of further consideration.

- Regarding DPB specification (incl. JCTVC-Q0103 / JCT3V-H0034) level limit constraints related to maximum DPB size, the VPS-level DPB needs to have constraints as well as the SPS level having such constraints (each layer in the TargetDecLayerIdList shall obey profile-specific level limit in A.4.1 d, which applies to sps_max_dec_pic_buffering_minus1[ HighestTid ]).

The AHG reached agreement on the following recommendations, for which either detailed text changes or editor's notes are included in the attached AHG output text:

- The text should clearly specify the following:

  - Values of general_profile_idc for indicating SHVC and MV-HEVC profiles.

  - Decoding requirements specification needs to require decoding (an editorial oversight)

  - If the capabilities of one profile are "nested" within the capabilities of another profile, decoders should be required to recognize that and decode the bitstream.

- Disallow an auxiliary picture layer to be indicated as a target output layer (unless there is a profile defined that contains the required decoding capability for the auxiliary picture layer) for Scalable Main and Stereo Main profile OLSs (depending on the rewriting process to handle that, as applicable).

- Make the PPS syntax HEVCv1 compatible, and enable the syntax of SPS with nuh_layer_id > 0 to be either HEVCv1 compatible or not.

- Simplify the bitstream rewriting process for independent non-base layers (part of JCTVC-R0042).

- Specify independent non-base layer decoding for profiles specified in Annex A. For example, the following phrasing could be used "When expressing the capabilities of a decoder for a profile specified in Annex A, the capability of decoding an independent non-base layer should be expressed."

- Allow a bitstream to be considered conforming without containing a base layer (i.e., without containing pictures of layer 0), and indicate such a bitstream using the vps_reserved_one_bit (with the syntax element name and semantics changed).

- Remove the bitstream partition HRD parameters SEI message.

- Specify signalling for indication of decoder capability as follows:

  - Each output layer set is associated with one or more partitioning schemes, and for each partitioning scheme, one or more bitstream partitions are signalled, and each bitstream partition contains one or more layers.

  - Each bitstream partition is associated with a set of profile, tier, and level. Level limits are basically bitstream partition specific.

  - A decoder specification conceptually based on a number of single-layer decoder cores without changing the existing hardware cores (but with addition of resampling and firmware changes) expresses its decoding capability by expressing the list of PTLs (profile, tier, and level syntax structures) of the single-layer decoder cores, each of which is suitable for decoding a bitstream partition.

  - In the semantics for partition signalling, when the number of partitions in the partitioning scheme of an OLS is greater than 1, the number of layers in each partition is required to be less than or equal to 4 (as a profile or semantics constraint). Decoders should not be required to decode partitions that contain more than 4 layers, and an OLS should not be required to have a PTL (profile, tier, and level syntax structure) that applies to the entire OLS.

  - Picture size related constraints and level limits should be established, based on the partitioning.

The following needs for further study or work were identified:

- Whether the bitstream rewriting process is needed, and how to specify/indicate the "plus" capability of HEVCv1 and RExt profiles (where the "plus" capability refers to the decoding capability required for decoding of an independent non-base layer that is not rewritten to be a base layer).

- Whether there is a need to specify resampling capability distinctly as part of profile/tier/level or VPS-level syntax.

- Currently, for bitstream partitions, instead of using the different delivery schedules signalled in an hrd_parameters( ) syntax structure as for layer sets in HEVCv1, a combination of delivery schedules from different hrd_parameters( ) syntax structures are used. Should the same way as for layer sets be used for bitstream partitions?

- For the bitstream partition nesting SEI message syntax, would it be desirable to enable more than one OLS index, more than one partitioning scheme index, and perhaps more than one BSP index, which could reduce the number of such SEI messages needed in the bitstream?

- Wording improvement is needed for item b of the profile specific level limits, to correctly implement the intent to consider output times of pictures in different AUs.

- Should the level limits on the number of tiles (or, more precisely, the rate of tiles) be layer specific (i.e. picture based) instead of bitstream partition specific? (see JCTVC-R0046)

- Contribution JCTVC-R0053 / JCT3V-I0029 (On invocation of a correct decoding process for the current picture) should be considered.

The following observations and questions were agreed by the AHG to be raised for discussion at JCT-VC / JCT-3V and the parent bodies:

- It is noted that the drafted Scalable Main profile does not support monochrome decoding

- It is also noted that 8-deep layering dependency is present in the current SHVC profile specifications.

- It is noted that the proposed modified signalling scheme for indication of decoder capability (as described above) differs in a substantial way from the content of the current draft specification. However, no other method had been identified to express decoding capabilities in a way that would support a substantial number of layers together with the intent to enable the construction of decoders from existing single-layer implementations (without a need to restructure the associated low-level computing resources in a fundamentally different way or to grossly-overprovision the resource requirements). The need for this design selection should be checked and confirmed by the parent bodies.

- Should the number of layers supported by a decoder be part of the profile/tier/level specification or can this be something chosen by individual decoder designers versus decoders just being required to decode an unlimited number of layers? For example, should there be a profile that restricts the number of layers to only 2 or 3?

- Whether there are some "simple" cases (such as Stereo Main or a case where each layer is a separate partition) in which decoder capabilities can be expressed as a single PTL rather than a "list of PTLs".

- Is it desirable to use general_profile_space equal to 1 for independent non-base layers for profiles specified in Annex A (HEVCv1 and RExt profiles), or is it adequate for this to be handled outside of the profile-tier-level syntax structure (e.g., based only on layer ID and a VPS indication)?

- Should some additional adjustment be made of the syntax of independent non-base layers in regard, e.g., to VPS repetition and handling of end of sequence and end of bitstream NAL units?

The following contributions registered for the Sapporo meeting were considered by the AHG.

- JCTVC-R0041 / JCT3V-I0023

- JCTVC-R0042 / JCT3V-I0021

- JCTVC-R0043 / JCT3V-I0022

- JCTVC-R0044 / JCT3V-I0024

- JCTVC-R0046 / JCT3V-I0022

- JCTVC-R0052 / JCT3V-I0028

- JCTVC-R0054 / JCT3V-I0030

The following additional contributions were noted to be relevant to the work of the AHG:

- JCTVC-R0053 / JCT3V-I0029 "AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture" [M. M. Hannuksela (Nokia)]

(This only includes contributions submitted up to the time of preparation of the report, which was substantially prior to the meeting.)

In dicussion, a participant asked whether it may be useful to define some sort of “optional” or “best effort” decoding in case of multi-layer concept.

Further discussion of the outcome of this AHG was agreed to be needed during the meeting (with some issues to be raised at the parent body level as noted). Revisit for that.

JCTVC-R0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. J. Sullivan, Y.-K. Wang (AHG chairs)]

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 17th JCT-VC meeting in Valencia, Spain, (27 March – 4 April 2014) and 18th JCT-VC meeting in Sapporo, Japan, (30 June – 9 July 2014).

The editorial team worked on two documents: JCTVC-Q1007 (SHVC Test Model 6 text) [1] and JCTVC-Q1008 (SHVC draft 6) [2]. Editing JCTVC-Q1008 was assigned a higher priority than editing JCTVC-Q1007.

One version of JCTVC-Q1007 and five versions of JCTVC-Q1008 were published by the editing AHG following the 17th JCT-VC meeting in Valencia.

The main changes in JCTVC-Q1008, relative to the previous JCTVC-P1008 (SHVC Draft 5), are:

- Incorporation of all common SHVC and MV-HEVC HLS normative adoptions at the 17th JCTVC meeting.

- Integration of all SHVC-specific normative adoptions at the 17th JCTVC meeting.

- Integration of five single layer SEI messages, including three SEI messages deferred from Rext specification.

- Fixes of bug tracker tickets, editorial improvements and fixes.

JCTVC-Q1007 Test Model 6 document mainly contains the general descriptions of SHVC framework, texture data resampling process, motion field mapping process and colour mapping process. The main change to the previous JCTVC-P1007 (SHM5) is the inclusion of a description of the colour mapping process for colour gamut scalability.

Issues reported included:

- Specification for colour gamut scalability: Colour gamut scalability based on 3D Look-Up-Table (LUT) mapping process was adopted at the Valencia meeting, and it was decided to limit the table size up to 8x2x2. In the current specificaton (and the accompanying SHM software), a syntax element, named 'cm_octant_depth', is used to control the table size. This syntax element shall be less than or equal to 1 to limit the table size to 8x2x2 at most, so that 1 bit is enough for it. However, at this moment, the SHVC specification (and the accompanying SHM software) was drafted in a way that the syntax element cm_octant_depth is 2 bits long and the value is restricted to be 0 and 1. The AHG11 had received concern about the text as well as the accompanying software. It was asserted that the text specification was drafted more general than what’s adopted since the decoding process supports the prohibited value of the syntax. And it has been asserted that this excess generality makes the text and the software more complicated than the case of 1 bit syntax representation. The editors suggest the JCTVC discuss on whether to keep the 2 bits representation of the syntax element 'cm_octant_depth' and the decoding process of the prohibited value. The contribution JCTVC-R0179 is related to this issue. Revisit.

- Specification for colour resampling hint SEI message: It is asserted that the information delivered by the syntax element 'minimum_degradation_flag' can be identified from filter coefficient values at the decoder. The editors and original proponent suggest the JCTVC discuss the necessity of this flag at Sapporo meeting and consider removal of this flag from SHVC draft. Decision: Remove the flag.

The AHG recommended to:

- Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the overall editorial quality of the SHVC Draft and Test Model documents.

- Request that proponents provide mature text for integration into the SHVC draft.

- To go through all editor’s notes and identify those that need the JCT-VC to discuss whether technical changes are needed to for them to be addressed. For example: [Ed. Are the tiles in an MCTS required to be in separate NAL units from the tiles that are not in the MCTS]. Revisit.

JCTVC-R0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He (AHG chairs)]

This report summarizes activities of the AHG12 on SHVC software development between 17th and 18th JCT-VC meetings.

The current latest software version is SHM-6.1 contains almost all the items adopted last meeting, however some work is still needed on

- POC derivation related to JCTVC-P0056 and layer-specific prediction structures JCTVC-Q0108.

In addition, the following items remained to be integrated:

- JCTVC-Q0154: on picture flushing and DPB parameters

- AHG15: vps_base_layer_internal_flag

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

Two software versions had been released by AHG12, integration details and performance summary are given in the next subsections. In the document, only HEVC base layer results are provided and AVC base layer data can be found in the accompanying excel tables. Performance results are consistent with the adopted techniques.

Software version SHM6.0 based on HM14 was released within a couple of weeks after the previous meeting.

In SHM-6.0, a functionality of having layer-specific CTB structure was implemented with macro LAYER_CTB, it allows using different CTB settings across layers. This functionality is in experimental stage and the macro is disabled by default.

A listed set of additional items had been integrated on top of SHM6.0.

The following development plan and recommendations were made:

- Continue to develop reference software based on SHM-6.1 and improve its quality.

- Resolve identified software and working draft mismatches.

- Remove input of motion field metadata for non-HEVC base layer. It was used for CE experimentation in the beginning of SHVC development.

It was remarked that the SHVC CTC does not include colour gamut mapping. Decision (CTC): It was agreed to add these as optional tests in the CTC.

JCTVC-R0013 JCT-VC AHG report: SHVC inter-layer re-sampling process (AHG13) [E. Alshina, J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (AHG coordinators)]

This document reports on the work of the JCT-VC AHG on SHVC inter-layer re-sampling process (AHG13) between the 17th JCT-VC meeting in Valencia, Spain, (27 March – 4 April 2014) and 18th JCT-VC meeting in Sapporo, Japan, (30 June – 9 July 2014), and lists the related input documents. Four teleconferences (about 2 hours each) were conducted between the 17th and the 18th JCT-VC meetings.

Draft text was proposed by the AHG. This includes consideration of the following:

- Current draft text explicitly uses reference layer conformance window parameters during re-sampling process. But actual reference picture size might be different from conformance window (for example, if region of interest extraction is considered) or conformance window information can un- available (if non-HEVC reference layer is used). It is recommended to replace conformance window parameters in re-sampling process by reference layer offsets as was proposed several times before [JCTVC-P0049, JCTVC-Q0159].

- Current draft text derives phases and delta parameters for re-sampling process from system of 3 flags [JCTVC-P0312, JCTVC-Q0120] allowing more freedom in vertical re-sampling process variants rather than horizontal. In order to make phase adjustment more generic it is recommended to combine phase and delta parameters (since they never used separately) and signal them in a bitstream for Luma and Chroma both for vertical and horizontal resampling process [JCTVC-Q0203]. The suggested change replaces the "system of 3 flags" in the specification.

- In current draft text parameters used in resampling process are signalled in VPS, SPS and slice header. Signaling of all related parameters in PPS (as was suggested in [JCTVC-Q0107, JCTVC-Q0168, JCTVC-Q0254]) was agreed to be cleaner. At the same time PPS signalling allows enough freedom for resampling parameters adaptation.

- It was mentioned that non-4:2:0 colour format support is a desirable feature. Even if SHVC is supposed to support only 4:2:0 colour formats, the specification text should not contain any restriction preventing easy extension of the spec for non 4:2:0 color format support during future standards development. In the discussion, it seemed that this was mostly an editorial matter and should not be difficult to describe (and that the extra generality seemed desirable, including having different chroma formats in base and enhancement layers). However, it was suggested that some technical adjustment may be needed for CGS with a 4:2:2 base layer. Decision (Ed.): It was agreed to use the same scheme for handling 4:2:2 as what was done for 4:2:0.

Several issues reportedly still remained unclear. These were outlined as follows, with discussion and actions recorded from further Track A discussion on Tues 1st a.m. (GJS).

- In "Picture parameter set RBSP semantics" the ranges for scaled reference layer offsets, reference layer offsets and resampling phases need to be specified. Decision: Upper bound of left (top) offset plus right (bottom) offset must be less than the picture width (height), in BL and EL, respectively. Also limit the se(v) range to −2^31 to 2^31−2 (actually based on the maximum length of the sc(v) codeword).

For the Phase range for luma from 0 to 31 (5 bit FLC) and for chroma −16 to 47 (offset by 16 and use 6-bit FLC). (Also a default resampling relationship if the phase data is not present for a reference layer, as is already in the AHG recommended text.) Revisit to check.

- In "Derivation process for inter-layer reference pictures" the derivation for unequalPictureSizeFlag may need to be adjusted in order to skip re-sampling process when not needed. Decision: Allow a phase-only adjustment between the RL and EL (with spatial ratio equal to 1), but count this as a resampling operation.

- The value of resampling offset may need to be restricted to be zero for non-spatial scalability, i.e. when spatial ratio is equal to 1 so that the results of direct copy and using interpoation process are identical. This is resolved by the action noted above.

- Clipping in "Resampling process of luma sample values" and "Luma interpolation process" put the restriction that pixels on a reference layer picture accessed in those processes are restricted within "the reference layer offset window extended with filter-tap length". Need to discuss and determine if this is reasonable restriction. See notes on R0220 regarding this.

- Current syntax signal 1) scaled reference layer offsets, 2) reference layer offsets, and 3) resample phases in independent loops. In two layers case, it could result in signaling the same 6 bit layer id syntax three times. It is suggested to discuss whether we should keep this design or signalling the parameters associated with the same reference layer in one loop. See notes on R0220 regarding this.

Related contributions were identified as R0209 (providing explanation of the recommendation of the AHG), R0220 (regarding the fourth and fifth topics in the above list and one additional aspect), and R0312 (a cross-check).

Also, in the discussion, it was remarked that Q0122 had proposed a drift reduction for position calculation (similar to what was done in SVC) and suggested that the modification for this would be small and that this should be further discussed.

A BoG (E. Alshina) was asked to consider the remaining issues and check the recorded actions.

JCTVC-R0014 JCT-VC AHG report: Colour gamut and bit depth scalability (AHG14) [P. Andrivon, A. Duenas (AHG co-chairs), C. Auyeung, E. Alshina, S. Deshpande, X. Li, K. Ugur, Y. Ye (AHG vice-chairs)]

Reviewed in Track A (GJS) 2nd day (Tue) p.m.

This report summarizes the activities of the ad-hoc group of color gamut and bit depth scalability (AHG14) between the 17th JCT-VC meeting and the 18th JCT-VC meeting.

There were very limited activity (cross-check, volunteering…)  related to the SCE1 over the SCE1 participants mailing list.

On the 27th of June 2014, there were 15 contributions related to colour gamut and bit-depth scalability.

Two contributions are primary SCE1 proposals (JCTVC-R0239 and JCTVC-R0163). There are 8 non-SCE1 contributions (propositions or bugfixes) mainly built on top of SHM-6. SCE1 report is document JCTVC-R0021.

The AhG recommends to:

- Review all documents related to colour gamut and bit-depth scalability.

- Analyze SCE1 related.

JCTVC-R0015 JCT-VC AHG report: SCC extensions test model text editing (AHG15) [J. Xu, R.Joshi (AHG co-chairs), R. Cohen, S. Liu, Z. Ma, Y. Ye (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on SCC extensions test model text editing (AHG15) between the 17th JCT-VC meeting in Valencia, (27th March to 4th April, 2014) and the 18th meeting in Sapporo (30th June to 9th July, 2014).

The first version of the screen content coding test model (SCM 1) document JCTVC-Q1014 was released on 28th April, 2014.

The AHG recommended to:

- Approve the JCTVC-Q1014 as the JCT-VC output document

- Establish an SCC working draft 1 soon (e.g., now)

- Continue to improve and refine the SCC test model document and resolve any discrepancies that may exist, in collaboration with the SCC extensions software development AHG

Test sequence material (AHG16) [miss]

TBP.

JCTVC-R0017 JCT-VC AHG report: SCC extensions software development (AHG17) [K. Rapaka, B. Li (AHG co-chairs), R. Cohen, T.-D. Chuang, X. Xiu (AHG vice-chairs)]

This report summarizes the activities of Ad Hoc Group 17 on screen content extensions software (SCM) development that have taken place between the 17th and 18th JCT-VC meetings. These activities included porting all the changes adopted as the starting point for SCM at the 17th JCT-VC meeting and updating configuration setting as per JCTVC-Q1015. As a result, primary software for screen content extensions development is now available.

Multiple versions of the HM SCM software were produced and announced on the JCT-VC email reflector.

HM14.0_RExt7.0_SCM1.0 was tagged on the SVN HHI repository on April 25th. This release includes:

- All the changes adopted at the 17th JCT-VC meeting integrated over HM14.0_RExt7.0 as the starting point of screen content extension software.

- The configurations files were aligned with JCTVC-Q1015.

- The bug fix for HM RExt ticket#1281.

The release was announced on the email reflector. The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.0+SCM-1.0/

The performance of this version against HM14.0_RExt7.0 (without IBC) was described according to the common test conditions in Q1015.

HM14.0_RExt7.1_SCM1.1 was tagged on the SVN HHI repository on June 26th. This release includes:

- SCM source base updated to HM14.0_RExt7.1.

- Removal of macro related to bug fix for ticket #1281.

- Bug fixes and clean-ups:

- Remove redundant inter search points in selective search method.

- Build issue under XCode development environment.

- Memory leak issue in HM SCC ticket #1297.

HM14.0_RExt7.2_SCM1.1 was also tagged on the SVN HHI repository on June 26th that updates SCM source to HM14.0_RExt7.2.

The coding performance of HM14.0_RExt7.1_SCM1.1 and HM14.0_RExt7.2_SCM1.1 did not change compared to HM14.0_RExt7.0_SCM1.0.

The releases were announced on the email reflector. The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.1+SCM-1.1/

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-14.0+RExt-7.2+SCM-1.1/

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-2.0, version SCC-1.0 (RExt-7.0).

Recommendations

- Continue to develop reference software based on HM14.0_RExt7.2_SCM1.1 and improve its quality.

- Remove macros introduced in previous versions before starting integration towards SCM-2.0 such as to make the software more readable.

# Project development, status, and guidance (XX)

## Communication to and by parent bodies (0)

See section 8.1.

## Conformance test set development (0)

## Version 1 and RExt bug reports and cleanup (0)

## HEVC coding performance, implementation demonstrations and design analysis (2)

### RExt performance/verification test (0)

### Still picture coding (0)

### SHVC performance and design aspects (0)

### SCC performance and design aspects (1)

JCTVC-R0101 Comparison of Compression Performance of HEVC Range Extensions Draft 7 and HEVC Screen Coding Extensions Draft 1 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

## Profile and level definitions (requirements related) (2)

### General (0)

### RExt profiles and levels (1)

JCTVC-R0128 High 4:4:4 16 Intra profile specification [K. Sharman, N. Saunders, J. Gamei (Sony)]

### SHVC profiles and levels (1)

JCTVC-R0158 Modifications to Scalable Main profile and general and profile-specific level limits [J. Boyce (Vidyo)]

## HEVC, SHVC and RExt use cases (requirements related) (0)

To be dealt with at parent level – outside the scope of current JCT-VC work. Not reviewed.

## Source video test material (2)

JCTVC-R0214 Video sequence conversion tools [C. Fogg (MovieLabs)]

JCTVC-R0216 Compression performance [C. Fogg (MovieLabs)] [miss]

# Core experiment in SHVC (8)

## SCE1: Colour gamut and bit depth scalability (8)

### SCE1 summary and general discussion (1)

JCTVC-R0021 SCE1: Summary Report on Color Gamut Scalability [A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li, K. Ugur, S-H. Kim (CE coordinators)]

Reviewed in Track A (GJS) on 2nd day (Tue) p.m.

This document summarizes the activities and test results performed in SCE1 on Colour Gamut and Bit Depth Scalability. The evaluated tools and the test conditions are defined in document JCTVC-Q1101.

Section 1 of the CE considered encoder-only modifications for use of weighted prediction, as alternatives to the table-driven colour gamut mapping function. It contained the following sub-experiments:

1.1	1 ILP in both reference lists, 2 classes of samples in WP parameters selection

1.2	2 ILPs in both reference lists, 4 classes of samples of samples in WP parameters selection

1.3	4 ILPs in both reference lists, 8 classes of samples of samples in WP parameters selection

1.4	8 ILPs in both reference lists, 16 classes of samples of samples in WP parameters selection

The results of section 1 tests continue to show a substantial benefit for the CGS mapping function, roughly on the order previously identified (perhaps 1% less when a fancier alternative was tried).

It was remarked that fancy WP usage could also potentially be applied with the CGS function (as currently specified), potentially providing that 1% on top of the tested CGS case.

So the basic conclusion seems to be that the presence of the CGS mapping function continues to appear justified.

Section 2 of the CE considered a claimed complexity reduction for the colour gamut mapping function (with the same table size, but different equations).

Some coding efficiency benefit was shown for the proposed alternative to the current CGS mapping function as tested in section 2 of the CE (2x AI 0.6%, 2x RA 0.2%, 1x AI 0.7%, 1x RA 0.5%).

There are also some non-CE contributions related to section 2 of the CE.

Revisit after review of non-CE input.

### SCE1 primary contributions (2)

JCTVC-R0163 SCE1: Test 5.2 results on color gamut scalability by matrix mapping [C. Auyeung (Sony), K. Misra, S. H. Kim, A. Segall (Sharp Lab)]

JCTVC-R0239 SCE1: Test 5.1 results on improved weighted prediction for color gamut scalability [D. Bugdayci, A. Aminlou, K. Ugur, M. M. Hannuksela (Nokia)]

### SCE1 cross checks (5)

JCTVC-R0077 Cross-checking SCE1 - Test 5.1.4 [P. Bordes (Technicolor)]

JCTVC-R0148 SCE1: Crosscheck report of SCE 1 Test 1.1 [X. Li (Qualcomm)] [late]

JCTVC-R0149 SCE1: Crosscheck report of SCE 1 Test 2.1 [X. Li (Qualcomm)] [late]

JCTVC-R0173 SCE1: crosscheck report of SCE1 test 1.1 [Y. He, Y. Ye (InterDigital)]

JCTVC-R0206 SCE1: Cross-check of Test-5.1.2 (4ILR) [C. Auyeung (Sony)]

# Core experiments in SCC (XX)

## SCCE1: Intra block copy (31)

### SCCE1 summary and general discussion (1)

(chaired by JRO, 07-01 morning/afternoon)

JCTVC-R0031 Summary report of HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions [J. Sole, S. Liu (CE coordinators)]

Test A: IBC Search Area

Tested several search areas (JCTVC-R0141, JCTVC-R0184):

- 2 CTUs (left and current)

- 3 CTUs (current CTU + 2 left CTU)

- 4 CTUs (current CTU + 3 left CTU)

- 4 CTUs (current CTU + 3 left CTU) + 4 rows above

- Surrounding the current CTU:  2×3 CTUs

- Surrounding the current CTU:  3×5 CTUs

- Full frame for intra frames and local search (2 CTUs) for inter frames

- Full frame (anchor)

BD-rates with respect to full-frame search.

|  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 34.7% | 25.5% | 21.3% | 19.2% | 20.1% | 11.4% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 22.6% | 15.8% | 12.9% | 12.0% | 14.2% | 8.1% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 15.1% | 11.7% | 10.2% | 9.4% | 8.6% | 5.2% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 20.3% | 17.0% | 15.9% | 14.5% | 10.5% | 4.7% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 35.2% | 25.9% | 21.7% | 19.6% | 20.5% | 11.5% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 22.4% | 15.7% | 12.9% | 12.0% | 14.1% | 8.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 16.5% | 12.9% | 11.3% | 10.3% | 9.3% | 5.5% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 21.8% | 18.4% | 17.3% | 15.9% | 11.3% | 5.1% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% | 109% | 134% | 123% | 95% | 96% | 103% | 100% |
| Dec Time[%] | 105% | 106% | 112% | 101% | 104% | 103% | 103% | 100% |
|  | Random Access |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 17.5% | 12.9% | 10.7% | 9.8% | 10.5% | 6.0% | 7.7% | 0.0% |
| RGB, text & graphics with motion,720p | 15.2% | 10.8% | 8.7% | 8.1% | 9.8% | 5.6% | 1.3% | 0.0% |
| RGB, mixed content, 1440p | 8.3% | 6.6% | 5.9% | 5.4% | 4.3% | 2.7% | 2.1% | 0.0% |
| RGB, mixed content, 1080p | 12.0% | 10.1% | 9.5% | 8.8% | 6.7% | 3.0% | 2.4% | 0.0% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 16.9% | 12.5% | 10.4% | 9.6% | 10.0% | 5.6% | 6.9% | 0.0% |
| YUV, text & graphics with motion,720p | 15.1% | 10.6% | 8.6% | 8.1% | 9.9% | 5.7% | 1.2% | 0.0% |
| YUV, mixed content, 1440p | 10.0% | 8.0% | 7.3% | 6.7% | 5.3% | 3.2% | 2.1% | 0.0% |
| YUV, mixed content, 1080p | 14.4% | 12.3% | 11.6% | 10.9% | 7.8% | 3.4% | 2.5% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 104% | 100% | 103% | 101% | 95% | 93% | 100% |
| Dec Time[%] | 104% | 105% | 101% | 100% | 102% | 96% | 96% | 100% |
|  | Low Delay |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 11.3% | 8.2% | 6.9% | 6.3% | 6.5% | 3.4% | 9.4% | 0.0% |
| RGB, text & graphics with motion,720p | 5.8% | 4.2% | 3.6% | 3.4% | 4.1% | 2.6% | 3.9% | 0.0% |
| RGB, mixed content, 1440p | 4.4% | 3.8% | 3.6% | 3.2% | 2.5% | 1.2% | 2.0% | 0.0% |
| RGB, mixed content, 1080p | 4.4% | 3.5% | 3.3% | 3.2% | 2.6% | 1.3% | 3.1% | 0.0% |
| RGB, Animation, 720p | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.1% | 7.5% | 6.3% | 5.8% | 5.8% | 3.0% | 8.3% | 0.0% |
| YUV, text & graphics with motion,720p | 5.0% | 3.6% | 3.2% | 3.0% | 3.6% | 2.3% | 3.3% | 0.0% |
| YUV, mixed content, 1440p | 5.3% | 4.3% | 4.2% | 3.9% | 2.5% | 1.5% | 1.9% | 0.0% |
| YUV, mixed content, 1080p | 5.8% | 5.0% | 4.8% | 4.3% | 3.3% | 1.9% | 2.9% | 0.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 105% | 101% | 98% | 93% | 93% | 93% | 100% |
| Dec Time[%] | 102% | 105% | 100% | 95% | 101% | 95% | 96% | 100% |

Assuming that the memory block size is 4×2 for both luma and chroma, then the average memory access is summarized as follows when the alignment size is 64 and burst size is 256.

|  | All Intra |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 62.1% | - | - | - | 101.7% | 100.9% | 100.0% | 100.0% |
| RGB, text & graphics with motion,720p | 56.5% | - | - | - | 101.2% | 100.6% | 100.0% | 100.0% |
| RGB, mixed content, 1440p | 55.3% | - | - | - | 99.6% | 99.3% | 100.0% | 100.0% |
| RGB, mixed content, 1080p | 57.5% | - | - | - | 100.9% | 100.1% | 100.0% | 100.0% |
| RGB, Animation, 720p | 52.4% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
| RGB, camera captured, 1080p | 50.7% | - | - | - | 100.1% | 100.0% | 100.0% | 100.0% |
| YUV, text & graphics with motion, 1080p | 61.3% | - | - | - | 101.6% | 100.9% | 100.0% | 100.0% |
| YUV, text & graphics with motion,720p | 55.6% | - | - | - | 101.1% | 100.6% | 100.0% | 100.0% |
| YUV, mixed content, 1440p | 54.2% | - | - | - | 99.5% | 99.2% | 100.0% | 100.0% |
| YUV, mixed content, 1080p | 56.7% | - | - | - | 100.8% | 100.1% | 100.0% | 100.0% |
| YUV, Animation, 720p | 51.3% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
| YUV, camera captured, 1080p | 50.2% | - | - | - | 100.0% | 100.0% | 100.0% | 100.0% |
|  | Random Access |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 72.1% | - | - | - | 100.4% | 100.2% | 72.7% | 100.0% |
| RGB, text & graphics with motion,720p | 71.9% | - | - | - | 100.0% | 100.1% | 72.9% | 100.0% |
| RGB, mixed content, 1440p | 71.7% | - | - | - | 100.0% | 99.8% | 72.5% | 100.0% |
| RGB, mixed content, 1080p | 72.1% | - | - | - | 99.9% | 99.9% | 72.5% | 100.0% |
| RGB, Animation, 720p | 77.0% | - | - | - | 100.0% | 100.0% | 78.0% | 100.0% |
| RGB, camera captured, 1080p | 77.8% | - | - | - | 100.0% | 100.0% | 78.6% | 100.0% |
| YUV, text & graphics with motion, 1080p | 72.2% | - | - | - | 100.3% | 100.2% | 72.7% | 100.0% |
| YUV, text & graphics with motion,720p | 72.0% | - | - | - | 100.1% | 100.0% | 73.0% | 100.0% |
| YUV, mixed content, 1440p | 72.2% | - | - | - | 100.0% | 100.0% | 72.7% | 100.0% |
| YUV, mixed content, 1080p | 72.2% | - | - | - | 100.0% | 100.0% | 72.7% | 100.0% |
| YUV, Animation, 720p | 76.6% | - | - | - | 100.0% | 100.0% | 77.5% | 100.0% |
| YUV, camera captured, 1080p | 77.6% | - | - | - | 100.0% | 100.0% | 78.4% | 100.0% |
|  | Low Delay |  |  |  |  |  |  |  |
|  | 2C | 3C | 4C | 4C+4 | 2x3C | 3x5C | FuII+2C | Full |
| RGB, text & graphics with motion, 1080p | 69.7% | - | - | - | 100.5% | 100.3% | 69.8% | 100.0% |
| RGB, text & graphics with motion,720p | 67.6% | - | - | - | 100.0% | 100.0% | 67.7% | 100.0% |
| RGB, mixed content, 1440p | 68.0% | - | - | - | 100.1% | 100.0% | 68.2% | 100.0% |
| RGB, mixed content, 1080p | 68.4% | - | - | - | 100.0% | 100.0% | 68.5% | 100.0% |
| RGB, Animation, 720p | 76.0% | - | - | - | 100.0% | 100.0% | 76.0% | 100.0% |
| RGB, camera captured, 1080p | 78.2% | - | - | - | 100.0% | 100.0% | 78.3% | 100.0% |
| YUV, text & graphics with motion, 1080p | 69.8% | - | - | - | 100.4% | 100.2% | 69.8% | 100.0% |
| YUV, text & graphics with motion,720p | 67.5% | - | - | - | 100.0% | 100.0% | 67.6% | 100.0% |
| YUV, mixed content, 1440p | 67.8% | - | - | - | 100.1% | 100.1% | 68.0% | 100.0% |
| YUV, mixed content, 1080p | 68.1% | - | - | - | 100.0% | 100.0% | 68.1% | 100.0% |
| YUV, Animation, 720p | 75.1% | - | - | - | 99.7% | 99.9% | 75.3% | 100.0% |
| YUV, camera captured, 1080p | 77.3% | - | - | - | 100.0% | 100.0% | 77.6% | 100.0% |

No cache considered in memory access results (with enough on-chip memory, 2x3 and 3x5 configurations might need less memory access)

Presentation of 188: Worst case memory bandwidth increases by appox. 10% due to the fact that both filtered and unfiltered references need to be written. Worst case read access is still with 8x8 bipred inter. This means that the increase of worst case is only effective for inter coded frames.

Presentation of 269: Investigation shows that approx. 15-20% of IBC blocks are selected outside of 2x4 CTU area with current reference. However, for the encoder search more than 90% are from farther distances. It is argued that this is not practical for real-time and low-latency coding.

The requirements of SCC are not explicitly requesting for low-complexity encoding (which an encoder could do when penalizing compression) and sub-picture low-delay requirements. Therefore, low decoder complexity should be main target

Proponents of 269 request to investigate a restricted configuration additionally in Core Experiments.

Revisit: Further discuss whether two IBC test configurations (full frame and restricted access) should be defined. The restricted case would likely be an additional case to be tested in specific CEs, but if it is used, it should be identical (so far, CE3 and CE5 have additionally used the 2 CTU restriction)

Test 2: IBC modes

- Test 2.1. IBC as inter PU (JCTVC-R0190)

Several aspects of IBC are modified to align it with inter predicted CU and PU syntax including: signaling of the block vector (BV) predictor, deblocking filter, DCT, and IBC related syntax moved from coding_unit to prediction_unit and harmonized with inter PU syntax. IntraBC mode is treated as an inter mode with inter_pred_idc equals to 0 (L0). 	4x4 partition mode is turn on or off for IntraBC.

- Test 2.2. Skip mode (JCTVC-R0097)

IBC skip mode is a 2N×2N partition IBC mode without residue. A flag is signaled for each CU to indicate the usage of the IBC skip mode.

- Test 2.3. Merge mode (JCTVC-R0097)

For IBC merge mode, the BV used for the current CU uses the BV in the left  or the above block. No BV difference is signaled for IBC merge mode. Applied only to 2N×2N partitions. A flag per CU indicates the usage. Another flag is signaled to indicate the BV used for the current CU.

- Test 2.4. AMP mode (JCTVC-R0241)

AMP for IBC introduces the motion partitions of 2N×nU, 2N×nD, nL×2N, and nR×2N into the IBC design.

Note: Test 2.3 should be considered along with the BV prediction methods

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -3.9% | -1.3% | 0.0% | -7.2% | -0.8% |
| RGB, text & graphics with motion,720p | -2.0% | -0.7% | 0.1% | -4.0% | -0.5% |
| RGB, mixed content, 1440p | -1.6% | -0.8% | 0.0% | -2.9% | -0.5% |
| RGB, mixed content, 1080p | -1.9% | -0.7% | 0.0% | -2.9% | -0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.6% | -2.1% | 0.0% | -7.4% | -1.1% |
| YUV, text & graphics with motion,720p | -2.4% | -1.3% | 0.0% | -4.3% | -0.7% |
| YUV, mixed content, 1440p | -1.8% | -1.0% | 0.0% | -3.1% | -0.5% |
| YUV, mixed content, 1080p | -2.1% | -1.0% | 0.0% | -3.2% | -0.6% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 91% | 98% | 110% | 139% |
| Dec Time[%] | 98% | 100% | 100% | 98% | 119% |
|  | Random Access |  |  |  |  |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -2.0% | -0.9% | 0.0% | -4.3% | -0.6% |
| RGB, text & graphics with motion,720p | -1.5% | -0.5% | 0.0% | -2.7% | -0.6% |
| RGB, mixed content, 1440p | -1.2% | -0.6% | 0.1% | -1.8% | -0.5% |
| RGB, mixed content, 1080p | -1.3% | -0.4% | 0.0% | -1.9% | -0.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.8% | -0.7% | -0.1% | -4.4% | -0.8% |
| YUV, text & graphics with motion,720p | -1.5% | -0.6% | 0.0% | -2.8% | -0.5% |
| YUV, mixed content, 1440p | -0.9% | -0.3% | 0.1% | -1.9% | -0.3% |
| YUV, mixed content, 1080p | -1.3% | -0.7% | 0.0% | -2.3% | -0.5% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 97% | 98% | 99% | 112% |
| Dec Time[%] | 97% | 98% | 100% | 97% | 115% |

|  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.1% | 0.0% | -3.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.9% | -0.3% | -0.1% | -1.0% | -0.4% |
| RGB, mixed content, 1440p | -0.7% | -0.5% | 0.0% | -1.0% | -0.4% |
| RGB, mixed content, 1080p | -0.9% | -0.7% | -0.2% | -1.3% | -0.4% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -0.8% | -0.1% | -3.2% | -0.6% |
| YUV, text & graphics with motion,720p | -0.5% | -0.1% | 0.0% | -1.2% | -0.2% |
| YUV, mixed content, 1440p | -0.3% | -0.3% | 0.0% | -1.2% | -0.4% |
| YUV, mixed content, 1080p | -0.5% | -0.1% | 0.3% | -0.8% | 0.2% |
| YUV, Animation, 720p | 0.2% | 0.1% | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.2% | 0.2% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 97% | 96% | 98% | 99% | 108% |
| Dec Time[%] | 99% | 97% | 99% | 98% | 119% |

|  | BV Memory Requirements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.1 w. 4x4 | 2.1 w/o 4x4 | 2.2 | 2.3 | 2.4 |
| Line Buffer | No | No | Yes | Yes | No |
| #vectors to be stored | Additional 24 for AI and 0 for RA and LD | Additional 24 for AI and 0 for RA and LD | - | Additional 24 for AI and 0 for RA and LD | Additional 24 for AI and 0 for RA and LD |

2.1 is integrating IBC into inter, i.e. signalling by refidx at PU (decision below CU), but disallowing AMP, disallowing bipred, no interpolation, no merge, no skip

Benefit of 2.1 could come by PU-level decision and capability to predict IBC vector from the block above. Usage of refidx for signalling is likely not giving benefit. For intra pictures, this also means that they need to implement and “inter” mode, which however can only be IBC.

2.1 also changes the effect of de-blocking such that the boundary between adjacent IBC blocks that have same vectors are not de-blocked.

2.2 does not provide benefit

2.3 should rather be considered under cat.3 (prediction)

2.4 is using AMP (beyond NxN, 2NxN, Nx2N and 2Nx2N currently used)

5.1 is combination of 2.1, 2.3, 2.4.

Agreement:

- IBC is a “third mode” different from both intra and inter prediction. In terms of prediction reference, it uses the same as intra, whereas in terms of memory access mechanism, coding of vector etc. it has some commonality with inter (but without the capability to simply re-use, because merge list may need to be modified, interpolation not necessary …)

- Refidx signalling is not asserted to be beneficial w.r.t. compression.

- PU–level decision? Would branch between intra and inter/IBC at CU level, and further branch between inter pred and IBC at PU level. More information was requested about the benefit of doing this, i.e. results of experiment 2.1 without using the vector from block above as predictors for the current BV. Revisit.

Test 3A: IBC Block Vector Prediction

- Test 3.1: BV prediction (JCTVC-R0185)

The block vector prediction method of IBC is modified to use either an spatial neighboring BV or default vector is used as BV predictor. Up to two BV from those in spatial neighboring blocks as used in HEVC are used to form a BV candidate list and one BV is selected.

- Test 3.2: Ping-Pong BV prediction (JCTVC-R0240)

Block vector prediction values are maintained as state information. For each BV to be sent, a flag is sent to select between them for the BV prediction value.

- Test 3.3: BV prediction (JCTVC-R0081)

The BV predictor is selected among a predictors list that includes the three last decoded BVs. The selected predictor is indicated through a syntax element. Additional fixed predictor values are added when the full list does not contain three elements.

- Test 3.4 AMVP prediction (JCTVC-R0061)

The block vector prediction method of IBC is modified to use either an spatial neighboring BV or default vector is used as BV predictor. A flag is used to signal which of the predictors is used. When there is only one unique candidate BV available, (-2*CU_width, 0) is used for the second candidate. If both candidates are not available, (-2*CU_width, 0) and (-CU_width, 0) are used.

Note: Encoders have different levels of optimization, which makes difficult to compare the algorithms. Same encoder results in table below.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.0% | -1.6% | -3.8% |
| RGB, text & graphics with motion,720p | -0.8% | -1.2% | -0.9% | -1.9% |
| RGB, mixed content, 1440p | -1.0% | -0.8% | -0.7% | -1.4% |
| RGB, mixed content, 1080p | -1.4% | -0.8% | -0.8% | -1.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.8% | -2.3% | -1.8% | -4.5% |
| YUV, text & graphics with motion,720p | -1.1% | -1.4% | -1.1% | -2.4% |
| YUV, mixed content, 1440p | -1.3% | -1.0% | -0.8% | -1.7% |
| YUV, mixed content, 1080p | -1.5% | -1.0% | -0.9% | -2.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% | 107% | 102% | 101% |
| Dec Time[%] | 104% | 101% | 100% | 99% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.6% | -1.2% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.3% | -0.8% |
| RGB, mixed content, 1440p | -0.8% | -0.6% | -0.5% | -1.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.9% | -1.1% | -0.8% | -1.7% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.8% | -0.7% | -1.5% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% | -0.6% | -0.9% |
| YUV, mixed content, 1440p | -1.1% | -0.6% | -0.4% | -1.3% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | 0.2% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | -0.6% | -0.6% | -0.5% | -1.0% |
| Enc Time[%] | 104% | 99% | 103% | 99% |
| Dec Time[%] | 107% | 98% | 99% | 102% |

|  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.2% | -0.4% |
| RGB, text & graphics with motion,720p | -0.3% | 0.1% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | -0.3% | -0.8% | -0.5% | -0.7% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | -0.3% | -0.4% |
| YUV, text & graphics with motion, 1080p | -0.2% | -0.7% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | 0.1% | -0.4% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | -0.2% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.2% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 104% | 98% |
| Dec Time[%] | 104% | 101% | 99% | 100% |

Results with following encoder settings for 3.1, 3.3 and 3.4 (would also be applicable to 3.2, but no results were provided on this by proponents):

- Software bug fix (could also be applied to SCM1.0)

- BV costs are estimated by considering all predictors

- Allow the use of predictors to be the BV of current IBC block (that means guarantee that the predictor used for coding is in the estimation list)

- BV predictor is RD estimated even if the predictor is outside the 2 CTBs

Test 3.1 provides further results with full RD check at BV predictor position 3.1*.

Test 3.3 provides further results with full RD check of BV predictor and testing neighboring left and above BVs: test 3.3*.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p | -7.3% | -6.5% |  | -7.2% | -9.7% | -8.9% |
| RGB, text & graphics with motion,720p | -4.2% | -3.8% |  | -4.2% | -5.3% | -5.3% |
| RGB, mixed content, 1440p | -2.9% | -2.6% |  | -2.9% | -3.7% | -3.7% |
| RGB, mixed content, 1080p | -3.5% | -3.1% |  | -3.5% | -4.2% | -4.2% |
| RGB, Animation, 720p | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -7.7% | -6.7% |  | -7.7% | -10.5% | -9.2% |
| YUV, text & graphics with motion,720p | -4.7% | -4.4% |  | -4.7% | -5.9% | -6.0% |
| YUV, mixed content, 1440p | -3.3% | -3.1% |  | -3.3% | -4.1% | -4.2% |
| YUV, mixed content, 1080p | -3.7% | -3.5% |  | -3.8% | -4.8% | -4.6% |
| YUV, Animation, 720p | -0.1% | -0.2% |  | -0.2% | -0.1% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 101% |  | 96% | 113% | 113% |
| Dec Time[%] | 98% | 98% |  | 98% | 96% | 96% |
|  |  |  | Random Access |  |  |  |
|  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p | -3.3% | -3.6% |  | -3.3% | -5.4% | -6.0% |
| RGB, text & graphics with motion,720p | -2.7% | -2.5% |  | -2.7% | -3.5% | -3.5% |
| RGB, mixed content, 1440p | -1.6% | -1.6% |  | -1.7% | -2.1% | -2.2% |
| RGB, mixed content, 1080p | -2.1% | -2.0% |  | -2.2% | -2.7% | -2.8% |
| RGB, Animation, 720p | -0.1% | -0.1% |  | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.1% | -3.3% |  | -3.2% | -5.6% | -6.1% |
| YUV, text & graphics with motion,720p | -3.0% | -2.7% |  | -2.9% | -3.9% | -3.8% |
| YUV, mixed content, 1440p | -1.7% | -1.8% |  | -1.9% | -2.3% | -2.7% |
| YUV, mixed content, 1080p | -2.4% | -2.4% |  | -2.6% | -3.2% | -3.3% |
| YUV, Animation, 720p | 0.0% | -0.1% |  | -0.2% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 102% |  | 96% | 106% | 106% |
| Dec Time[%] | 99% | 99% |  | 98% | 103% | 91% |
|  |  |  | Low Delay |  |  |  |
|  | 3.1 | 3.3 |  | 3.4 | 3.1* | 3.3* |
| RGB, text & graphics with motion, 1080p | -1.8% | -2.6% |  | -1.8% | -3.6% | -4.9% |
| RGB, text & graphics with motion,720p | -1.0% | -1.1% |  | -1.0% | -1.5% | -1.6% |
| RGB, mixed content, 1440p | -0.7% | -0.8% |  | -0.9% | -1.1% | -1.3% |
| RGB, mixed content, 1080p | -0.8% | -1.4% |  | -0.8% | -1.1% | -1.4% |
| RGB, Animation, 720p | 0.0% | 0.0% |  | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% |  | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.6% | -2.2% |  | -1.6% | -3.7% | -4.9% |
| YUV, text & graphics with motion,720p | -1.0% | -1.2% |  | -1.0% | -1.6% | -1.7% |
| YUV, mixed content, 1440p | -0.7% | -0.8% |  | -0.7% | -1.3% | -1.6% |
| YUV, mixed content, 1080p | -0.5% | -1.0% |  | -0.7% | -1.0% | -1.5% |
| YUV, Animation, 720p | 0.0% | -0.1% |  | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 102% |  | 95% | 103% | 105% |
| Dec Time[%] | 101% | 102% |  | 96% | 106% | 95% |

|  | BV Memory Requirements |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 | 3.4 |
| Line Buffer | No | No | No | No |
| #vectors to be stored | Additional 23 for AI and 0 for RA and LD | 2 | 3 | Additional 24 for AI and 0 for RA and LD |

Usage of increasing the number of candidates for prediction gives clear benefit. A late contribution (309) proposes a unification of 3.1, 3.2 and 3.4. In contrast to them, 3.3 uses three candidates, which however does not give benefit for AI, but provides gain for RA and LD configurations.

Agreement: Some improved IBC vector prediction mechanism should be included in new test model, revisit in context of R309.

Test 3B: IBC Block Vector Binarization

- Test 3.5: BV difference coding (JCTVC-R0133)

A modified context modeling method for coding IBC block vector difference (BVD) is tested. Additional contexts are assigned for coding the prefix bins used for representing the absolute value of the BVD component.

- Test 3.6: BV binarization (JCTVC-R0186)

BVD binarization is modified: first, a flag is signaled to indicate whether the difference is zero. When BV difference is not zero, Exp-Golomb code of order 3 is used.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx rd | 3.6 |
| RGB, text & graphics with motion, 1080p | -3.2% | -3.0% | -2.3% |
| RGB, text & graphics with motion,720p | -2.0% | -1.9% | -1.3% |
| RGB, mixed content, 1440p | -1.0% | -0.9% | -0.8% |
| RGB, mixed content, 1080p | -1.3% | -1.2% | -1.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.0% | -3.7% | -2.9% |
| YUV, text & graphics with motion,720p | -2.5% | -2.3% | -1.6% |
| YUV, mixed content, 1440p | -1.3% | -1.3% | -1.0% |
| YUV, mixed content, 1080p | -1.5% | -1.4% | -1.2% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 97% | 101% |
| Dec Time[%] | 102% | 102% | 103% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx rd | 3.6 |
| RGB, text & graphics with motion, 1080p | -1.8% | -1.7% | -1.4% |
| RGB, text & graphics with motion,720p | -1.3% | -1.3% | -1.0% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.5% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.6% |
| RGB, Animation, 720p | -0.3% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -2.0% | -1.8% | -1.5% |
| YUV, text & graphics with motion,720p | -1.6% | -1.6% | -1.2% |
| YUV, mixed content, 1440p | -0.8% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -1.1% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 97% | 102% |
| Dec Time[%] | 106% | 98% | 103% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | 3.5 | 3.5 ctx r.d. | 3.6 |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.4% | -1.1% |
| RGB, text & graphics with motion,720p | -0.9% | -0.8% | -0.6% |
| RGB, mixed content, 1440p | -0.7% | -0.5% | -0.6% |
| RGB, mixed content, 1080p | -0.4% | -0.9% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.2% |
| YUV, text & graphics with motion,720p | -0.9% | -0.8% | -0.5% |
| YUV, mixed content, 1440p | -0.4% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.1% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 96% | 99% |
| Dec Time[%] | 104% | 98% | 102% |

|  | Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  | SCM | 3.5 | 3.5 ctx rd | 3.6 |
| # regular (context coded) bins per BV | 4 | 15 | 14 | 2 |
| # additional contexts | 0 | 17 | 4 | 2 |
| Binarization as in HEVC | Yes | Yes, except the prefix bins are regular bins |  | No |

3.5 would increase the worst number of context coded bins per coefficient, as 14 CC bins could occur per 4x4 block. This is undesirable

Side activity (K. Rapaka): Analyse new contributions (123, 181, 088, 074) wrt performance, complexityin comparison to 3.6 (as far as IBC (BV) binarization is concerned). Revisit. Some of these are also touching inter binarization, which should be discussed separately as it would modify existing HEVC technology specifically for screen content.

Test 4: Block copying modifications

- Test 4.1: Symmetric intra block copy (JCTVC-R0116)

Symmetric intra block copy (SIBC) is tested. When SIBC is applied, the reference block is flipped horizontally or vertically before it is used to predict the current block. SIBC is only allowed for IBC with block size 2Nx2N. A flag is signaled to indicate whether the current block chooses SIBC or normal IBC. If used, another flag indicates horizontal or vertical SIBC. When horizontal SIBC is on, BVDy is not signaled and BVy is inferred to be 0. When vertical SIBC is on, BVDx is not signaled and BVx is inferred to be 0.

- Test 4.2: Block flipping (JCTVC-R0097)

Vertical flipping is enabled for the CU using Intra BC prediction. One flag is signaled to distinguish flipping or not for every PU using Intra BC prediction (except the CU in Intra BC merge mode).

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.9% | -0.7% | -1.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.4% | -0.7% | -0.8% | -0.1% |
| RGB, mixed content, 1440p | -0.8% | -0.3% | -1.0% | -0.1% |
| RGB, mixed content, 1080p | -0.7% | -0.2% | -0.8% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -0.6% | -1.2% | -0.6% |
| YUV, text & graphics with motion,720p | -0.5% | -0.8% | -0.9% | -0.5% |
| YUV, mixed content, 1440p | -0.8% | -0.4% | -1.0% | -0.2% |
| YUV, mixed content, 1080p | -0.7% | -0.1% | -0.8% | 0.0% |
| YUV, Animation, 720p | -0.1% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 102% | 102% | 105% |
| Dec Time[%] | 100% | 100% | 99% | 102% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.9% | -0.3% |
| RGB, text & graphics with motion,720p | -0.3% | -0.5% | -0.6% | -0.2% |
| RGB, mixed content, 1440p | -0.4% | -0.2% | -0.4% | 0.0% |
| RGB, mixed content, 1080p | -0.3% | 0.0% | -0.4% | 0.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.4% | -0.7% | -0.2% |
| YUV, text & graphics with motion,720p | -0.3% | -0.6% | -0.6% | -0.3% |
| YUV, mixed content, 1440p | -0.4% | 0.0% | -0.4% | 0.1% |
| YUV, mixed content, 1080p | -0.4% | -0.1% | -0.5% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 99% | 99% |
| Dec Time[%] | 100% | 99% | 99% | 101% |

|  | Low Delay |  |  |  |
| --- | --- | --- | --- | --- |
|  | 4.1 hor. | 4.1 ver. | 4.1 h.+v. | 4.2 |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.5% | -0.1% |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.4% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.3% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.5% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.5% | 0.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% | 0.3% |
| YUV, mixed content, 1440p | -0.2% | 0.1% | -0.3% | -0.1% |
| YUV, mixed content, 1080p | 0.3% | 0.0% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 99% | 98% | 99% |
| Dec Time[%] | 101% | 99% | 99% | 101% |

The gain is relatively low compared to other proposed methods in the IBC domain. The flipping would require some additional decoder operations at the level of samples (re-ordering of the arrangement by which they are stored in the reference buffer). No action at this point, consider continuation of CE depending on new ideas in non-CE category.

Test 5: Combinations

- Test 5.1: Combination of tests 2.1 + 2.3 + 2.4 (JCTVC-R0191)

In this test, the methods that are used in tests 2.1, 2.3 and 2.4 are tested in combination. Test A is 2.1 + 2.3 and test B is 2.1 + 2.3 + 2.4.

- Test 5.2: Combination of tests 3.1 + 3.6 (JCTVC-R0187)

In this test, the BV prediction method in test 3.1 and the BV coding method in test 3.6 are combined.

Note: enc. refers to the integration of the common encoder described for tests 3.1 and 3.3. enc* indicates the encoder with full RD in 3.1.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -9.3% | -9.9% | -4.4% | -8.6% | -10.9% |
| RGB, text & graphics with motion,720p | -5.2% | -5.7% | -2.1% | -5.0% | -6.2% |
| RGB, mixed content, 1440p | -4.2% | -4.6% | -1.7% | -3.4% | -4.2% |
| RGB, mixed content, 1080p | -4.5% | -4.8% | -2.1% | -4.1% | -4.8% |
| RGB, Animation, 720p | -0.3% | -0.4% | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -9.7% | -10.6% | -5.1% | -9.5% | -12.0% |
| YUV, text & graphics with motion,720p | -5.7% | -6.2% | -2.4% | -5.7% | -6.9% |
| YUV, mixed content, 1440p | -4.1% | -4.5% | -2.1% | -4.0% | -4.8% |
| YUV, mixed content, 1080p | -4.6% | -5.0% | -2.4% | -4.5% | -5.5% |
| YUV, Animation, 720p | -0.2% | -0.5% | 0.0% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% | 166% | 100% | 102% | 115% |
| Dec Time[%] | 94% | 95% | 104% | 103% | 97% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -5.8% | -6.4% | -2.1% | -4.3% | -6.3% |
| RGB, text & graphics with motion,720p | -3.8% | -4.2% | -1.3% | -3.2% | -4.0% |
| RGB, mixed content, 1440p | -2.6% | -3.0% | -1.0% | -2.0% | -2.6% |
| RGB, mixed content, 1080p | -3.0% | -3.2% | -1.3% | -2.6% | -3.1% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -5.6% | -6.2% | -2.3% | -4.3% | -6.7% |
| YUV, text & graphics with motion,720p | -3.8% | -4.3% | -1.5% | -3.7% | -4.5% |
| YUV, mixed content, 1440p | -2.4% | -2.6% | -1.3% | -2.5% | -2.9% |
| YUV, mixed content, 1080p | -3.2% | -3.5% | -1.7% | -3.1% | -3.8% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 116% | 97% | 102% | 105% |
| Dec Time[%] | 99% | 101% | 100% | 100% | 98% |

|  | Low Delay |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 5.1A | 5.1B | 5.2 | 5.2 enc | 5.2 enc* |
| RGB, text & graphics with motion, 1080p | -4.9% | -5.4% | -1.4% | -2.7% | -4.6% |
| RGB, text & graphics with motion,720p | -2.1% | -2.2% | -0.8% | -1.4% | -2.0% |
| RGB, mixed content, 1440p | -2.0% | -2.2% | -0.4% | -1.0% | -1.4% |
| RGB, mixed content, 1080p | -1.7% | -2.0% | -0.6% | -1.4% | -1.9% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.2% | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.4% | -4.9% | -1.3% | -2.5% | -4.7% |
| YUV, text & graphics with motion,720p | -1.4% | -1.9% | -0.7% | -1.4% | -1.9% |
| YUV, mixed content, 1440p | -1.4% | -1.5% | -0.8% | -1.2% | -1.9% |
| YUV, mixed content, 1080p | -1.6% | -1.8% | -0.5% | -0.9% | -1.6% |
| YUV, Animation, 720p | 0.3% | 0.1% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.3% | 0.3% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 112% | 102% | 103% | 102% |
| Dec Time[%] | 101% | 101% | 97% | 102% | 105% |

5.1: Extends the “refidx” method of 2.1 by merge mode of 2.3 (2 merge candidates) and asymmetric partitioning of 2.4. R0100 is a non-CE proposal that does something similar but more identical with conventional inter prediction. Revisit in context of discussing R0100: Does usage of refidx have benefits?

### SCCE1 primary contributions (14)

JCTVC-R0061 SCCE1 Test 3.4: IntraBC BV prediction [X. Xu, S. Liu (MediaTek)]

JCTVC-R0081 SCCE1: Test 3.3 - BV prediction using the three last predictors [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-R0097 SCCE1: Result of Test 2.2 2.3 and 4.2 [B. Li, J. Xu (Microsoft)]

JCTVC-R0116 SCCE1 Test 4.1: Symmetric intra block copy [K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)]

JCTVC-R0133 SCCE1: Test 3.5 - Block vector difference coding [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-R0141 SCCE1: Result of Test 1.2 for IBC with constrained buffers from left CTUs [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei Technologies)]

JCTVC-R0184 SCCE1: Test 1.1 – Intra block copy with different search areas [C. Pang, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0185 SCCE1: Test 3.1 – Block vector prediction method for Intra block copy [C. Pang, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0186 SCCE1: Test 3.6 – Block vector coding for Intra block copy [C. Pang, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0187 SCCE1: Test 5.2 – Combination of Test 3.1 and Test 3.6 [C. Pang, Y. Chen, K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0190 SCCE1 Test 2.1: IntraBC coded as Inter PU [X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-R0191 SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4 [X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-R0240 SCCE1 subtest 3.2: Ping-Pong block vector predictor test [L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft)]

JCTVC-R0241 SCCE1 subtest 2.4: AMP for Intra BC prediction [L. Zhu, J. Xu, G. J. Sullivan, Y. Wu, S. Sankuratri, B. A. Kumar (Microsoft)]

### SCCE1 cross checks (16)

JCTVC-R0090 SCCE1: Cross-check of Test 3.2 [P. Onno (Canon)] [late]

JCTVC-R0107 SCCE1: Cross-check of test 3.3 (JCTVC-R0081) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0108 SCCE1: Cross-check of test 4.1 (JCTVC-R0116) [B. Li, J. Xu (Microsoft)]

JCTVC-R0129 Crosscheck of SCCE1 Test 2.1 on IBC as Inter PU [R.-L. Liao, C.-C. Chen, W. H. Peng, H.-M. Hang (NCTU/ITRI)]

JCTVC-R0138 SCCE1: Crosscheck of SCCE1 Test 3.6 - BVD binarization and coding [S.-T. Hsiang, T.-D. Chuang (MediaTek)]

JCTVC-R0171 SCCE1: Crosscheck report of SCCE1 test 2.3 Merge mode [Y. He, Y. Ye (InterDigital)]

JCTVC-R0192 Cross check of SCCE1 Test 3.1: Block vector prediction method for Intra block copy [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-R0193 Cross check of SCCE1 Test 5.2: combination of 3.1 and 3.6 [X. Xu, S.-T. Hsiang (MediaTek)] [late]

JCTVC-R0195 Cross check of SCCE1 Test 2.2: Intra block copy skip mode [C. Pang (Qualcomm)]

JCTVC-R0196 Cross check of SCCE1 Test 3.5: BVD coding for Intra block copy [C. Pang (Qualcomm)]

JCTVC-R0211 SCCE1: Cross-verification of test1.2 results [M. Zhou (Broadcom)]

JCTVC-R0244 Cross check of ‘SCCE1: Test 1.1 – Intra block copy with different search areas’ (JCTVC-R0184) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0245 Cross check of ‘SCCE1 subtest 2.4: AMP for Intra BC prediction’ (JCTVC-R0241) by Microsoft [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0246 Cross check of ‘SCCE1 Test 5.1: Combination tests of 2.1, 2.3 and 2.4’ (JCTVC-R0191) by Mediatek [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0248 SCCE1: Cross-check results of Test 3.4 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

JCTVC-R0273 Crosscheck for SCCE1 Test 4.2 in JCTVC-R0097 [K. Zhang (MediaTek)] [late]

## SCCE2: Line-based intra copy (5)

### SCCE2 summary and general discussion (1)

(chaired by JRO, 07-01 afternoon/evening)

JCTVC-R0032 SCCE2: Summary Report of HEVC Screen Content Coding Core Experiment 2 on Line-based Intra Copy [C.-C. Chen, X. Xu, L. Zhang (CE coordinators)]

Test B: Line-based Intra Copy with 2-D BV and PU-level Splitting

- Proposal: (JCTVC-R0131)

- X-checker: (JCTVC-R0183)

- Description: It is proposed to performing intra copy for lines at PU level with 2-D BV for each line. In addition, a flag is present at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -7.1% | -7.1% | -7.1% | -4.1% | -4.1% | -4.2% | -2.6% | -2.6% | -2.5% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.1% | -3.1% | -2.4% | -2.2% | -2.4% | -1.5% | -1.4% | -1.4% |
| RGB, mixed content, 1440p | -2.9% | -2.9% | -2.9% | -2.2% | -2.0% | -2.1% | -1.3% | -1.4% | -1.2% |
| RGB, mixed content, 1080p | -3.6% | -3.8% | -3.9% | -2.6% | -2.8% | -2.7% | -1.2% | -1.3% | -1.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.7% | -6.6% | -6.6% | -3.7% | -4.0% | -3.9% | -2.2% | -2.3% | -2.4% |
| YUV, text & graphics w/ motion,720p | -2.6% | -2.9% | -3.4% | -2.0% | -2.3% | -2.5% | -0.7% | -1.3% | -1.5% |
| YUV, mixed content, 1440p | -2.3% | -2.7% | -2.9% | -1.4% | -2.1% | -2.3% | -0.6% | -1.6% | -1.9% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% | -2.4% | -3.4% | -3.1% | -0.7% | -2.4% | -0.8% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 115% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 102% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 6.6% | 4.3% | 10.4% | 4.1% | 1.7% | 7.6% | 3.0% | 1.3% | 5.0% |
| RGB, text & graphics w/ motion,720p | 1.8% | 0.5% | 3.3% | 1.0% | 0.3% | 2.8% | 0.7% | 0.2% | 1.9% |
| RGB, mixed content, 1440p | 1.0% | 0.6% | 1.4% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 6.6% | 4.3% | 10.3% | 4.2% | 2.0% | 7.7% | 3.1% | 1.5% | 5.3% |
| YUV, text & graphics w/ motion,720p | 2.1% | 0.4% | 4.8% | 1.3% | 0.3% | 4.1% | 0.9% | 0.2% | 2.7% |
| YUV, mixed content, 1440p | 1.0% | 0.5% | 1.4% | 0.2% | 0.1% | 0.3% | 0.1% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 122% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |

Test C: Test B + Constrained PU-level Splitting

- Proposal: (JCTVC-R0131)

- X-checker: (JCTVC-R0249)

- Description: This test imposes an additional constraint on Test B, which requires that all the PUs within a CU use the same form of line splitting, row-wise or column-wise. Only one flag needs to be signaled at the CU level to indicate which mode is in use.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -7.0% | -7.0% | -6.9% | -4.1% | -4.0% | -4.0% | -2.5% | -2.4% | -2.3% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.2% | -3.2% | -2.3% | -2.2% | -2.4% | -1.5% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -2.8% | -2.7% | -2.8% | -2.1% | -1.9% | -1.9% | -1.1% | -1.3% | -1.1% |
| RGB, mixed content, 1080p | -3.7% | -3.8% | -3.8% | -2.6% | -2.7% | -2.7% | -1.3% | -1.3% | -1.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.6% | -6.5% | -6.5% | -3.6% | -3.9% | -3.9% | -2.2% | -2.4% | -2.5% |
| YUV, text & graphics w/ motion,720p | -2.5% | -3.0% | -3.3% | -2.1% | -2.6% | -2.6% | -0.9% | -1.1% | -1.8% |
| YUV, mixed content, 1440p | -2.2% | -2.7% | -2.9% | -1.3% | -2.4% | -2.1% | -0.7% | -1.7% | -1.8% |
| YUV, mixed content, 1080p | -3.2% | -3.9% | -3.7% | -2.4% | -3.1% | -3.0% | -0.5% | -2.0% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 113% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  | 103% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 6.5% | 4.3% | 10.2% | 4.0% | 1.7% | 7.4% | 3.0% | 1.3% | 4.9% |
| RGB, text & graphics w/ motion,720p | 1.7% | 0.5% | 3.2% | 1.0% | 0.3% | 2.7% | 0.7% | 0.2% | 1.8% |
| RGB, mixed content, 1440p | 1.0% | 0.6% | 1.4% | 0.2% | 0.1% | 0.4% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 6.5% | 4.3% | 9.9% | 4.1% | 2.0% | 7.5% | 3.1% | 1.6% | 5.1% |
| YUV, text & graphics w/ motion,720p | 2.1% | 0.4% | 4.6% | 1.3% | 0.3% | 3.9% | 0.9% | 0.2% | 2.6% |
| YUV, mixed content, 1440p | 1.0% | 0.5% | 1.4% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.3% | 1.3% | 1.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 121% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 104% |  |  | 102% |  |  |

Test D: Test C + Block Vector Merging

- Proposal: (JCTVC-R0131)

- X-checker: (JCTVC-R0194)

- Description: To reduce BV overhead for Test C, this test reuses one previously decoded BV for representing the BV of the current line, and no block vector difference needs to be signaled. A flag is present for each line to switch adaptively between BV merging and BV difference coding.

- Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -6.9% | -6.9% | -6.9% | -3.9% | -3.9% | -3.9% | -2.3% | -2.3% | -2.1% |
| RGB, text & graphics w/ motion,720p | -3.2% | -3.2% | -3.3% | -2.4% | -2.3% | -2.3% | -1.6% | -1.4% | -1.5% |
| RGB, mixed content, 1440p | -2.7% | -2.6% | -2.7% | -1.9% | -1.8% | -1.9% | -1.1% | -1.1% | -1.0% |
| RGB, mixed content, 1080p | -3.6% | -3.6% | -3.7% | -2.6% | -2.6% | -2.7% | -1.2% | -1.3% | -1.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.6% | -6.6% | -6.5% | -3.6% | -3.8% | -3.8% | -2.1% | -2.3% | -2.3% |
| YUV, text & graphics w/ motion,720p | -2.7% | -3.0% | -3.3% | -2.0% | -2.4% | -2.5% | -0.8% | -1.2% | -1.8% |
| YUV, mixed content, 1440p | -2.1% | -2.6% | -2.8% | -1.3% | -2.1% | -2.2% | -0.6% | -1.4% | -1.5% |
| YUV, mixed content, 1080p | -3.0% | -3.7% | -3.6% | -2.3% | -3.1% | -3.1% | -0.6% | -2.2% | -0.7% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | 0.1% | -0.1% | -0.4% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 115% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  | 101% |  |  |

- Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 7.3% | 4.9% | 11.2% | 4.6% | 2.0% | 8.3% | 3.4% | 1.4% | 5.6% |
| RGB, text & graphics w/ motion,720p | 2.0% | 0.5% | 3.8% | 1.2% | 0.3% | 3.2% | 0.8% | 0.3% | 2.1% |
| RGB, mixed content, 1440p | 1.1% | 0.6% | 1.6% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| RGB, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 7.4% | 5.0% | 11.1% | 4.7% | 2.3% | 8.5% | 3.6% | 1.8% | 5.9% |
| YUV, text & graphics w/ motion,720p | 2.4% | 0.5% | 5.5% | 1.5% | 0.3% | 4.7% | 1.0% | 0.2% | 2.9% |
| YUV, mixed content, 1440p | 1.1% | 0.5% | 1.6% | 0.2% | 0.1% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.4% | 1.4% | 1.4% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 117% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 102% |  |  | 102% |  |  |

Line base intra copy invoked at CU level (line means either horizontal or vertical 1D structure). Coding of displacement (2D) is done as in IBC, i.e. differential with reference to previous value. The structure can only be outside the current CU

Test B: signalling about using horizontal or vertical structure is done at PU level

Test C: signalling about using horizontal or vertical structure is done at CU level

Test D: Test C with modified signalling, where a flag indicates that the previous vector is not changed

The implemented syntax would basically allow full-frame range, but the encoder restricts the search in all cases within 2 CTU. There are also some further constraints on restricting the search range to horizontal or vertical-only. Further results about full-frame search are reported in non-CE proposals (R0132).

No analysis performed w.r.t. worst case memory bandwidth (which may also be different for the horizontal and vertical modes, depending on memory access arrangement).

Worst case number of context-coded should also be studied. It could happen that a new displacement vector needs to be signalled for each 1x4 or 4x1 structure within a 4x4 PU.

Presentation of R0132: This allows any 2D displacement within current picture. However, most of the additional gain is already available when searching only 2 CTUs over all possible 2D displacements.

CE2 to be continued

- Investigate benefit in comparison to restricted and full-frame IBC

- Investigate implications w.r.t. worst case memory bandwidth and CABAC throughput in the same fashion as it is done for IBC. Furthermore, investigate possible impact on decoder processing pipeline, particularly w.r.t. the new contribution R0281.

### SCCE2 primary contributions (1)

JCTVC-R0131 SCCE2: Line-based Intra Copy with 2-D BV and PU-level Splitting (Test B), and Its Variants with Constrained PU-level Splitting (Test C) and BV Merging (Test D) [T.-S. Chang, C.-C. Chen, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU), T. Lin, L. Zhao (Tongji Univ.), C.-C. Lin, F.-D. Jou (ITRI)]

### SCCE2 cross checks (3)

JCTVC-R0183 Cross check of SCCE2 Test-B: Line-based Intra Copy with 2-D BV and PU-level Splitting (JCTVC-R0131) [K. Rapaka (Qualcomm)] [late]

JCTVC-R0194 Cross check of SCCE2 Test D: Line-based Intra Copy with 2-D BV and PU-level Splitting and BV Merging [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-R0249 SCCE2: Cross-check of Test C [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

## SCCE3: Palette mode (50)

### SCCE3 summary and general discussion (1)

JCTVC-R0033 SCCE3: Summary report of CE on palette mode [Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma (CE coordinators)]

### SCCE3 primary contributions (23)

JCTVC-R0045 SCCE3: Test B.14 – Restricted run coding [J. Lainema, K. Ugur, D. Bugdayci (Nokia)]

JCTVC-R0047 SCCE3: Test A.9 – Long-term palette prediction [D. B. Sansli, J. Lainema, K. Ugur (Nokia)]

JCTVC-R0048 SCCE3: Test B.15 – Scanning indicator for the palette coded CUs [D. B. Sansli, J. Lainema, K. Ugur (Nokia)]

JCTVC-R0057 SCCE3 Test B.4: Adaptive color index map scan [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, X. Zhang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-R0058 SCCE3 Test D.1: Single color intra mode for screen content coding [Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-R0063 SCCE3: Test A.8 – Improvements on Palette Prediction Vector Signaling [M. Karczewicz, W. Pu, V. Seregin, R. Joshi, J. Sole (Qualcomm)]

JCTVC-R0064 SCCE3: Test A.7 – Encoder-Improved Palette Generation [W. Pu, V. Seregin, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-R0065 SCCE3: Test B.12 – Binarization of Escape Sample and Palette Index [W. Pu, V. Seregin, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-R0066 SCCE3: Test B.13 – Flag to Indicate Escape and Index Coding [W. Pu, R. Joshi, M. Karczewicz, J. Sole, V. Seregin (Qualcomm)]

JCTVC-R0067 SCCE3: Test C.4 – Combination of Tests A.3, A.7, A.8, B.12, and B.13 [W. Pu, V. Seregin, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-R0119 SCCE3 Test A.2: Palette coding and palette predictor updates [Y.-C. Sun, P. Lai, T.-D. Chuang, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-R0120 SCCE3 Test B.2: Transition copy mode [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-R0121 SCCE3 Test B.1: Line-based color index map coding [T.-D. Chuang, Y.-C. Sun, S.-T. Hsiang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-R0122 SCCE3 Test C.1: Combination of SCCE3 Test A.2 and Test B.1 [T.-D. Chuang, Y.-C. Sun, P. Lai, S.-T. Hsiang, X. Zhang, Y.-W. Chen, Y.-W. Huang, S. Lei (Mediatek)]

JCTVC-R0142 SCCE3 Test A.4: color table signaling with merge, sharing and DPCM [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)]

JCTVC-R0143 SCCE3 Test B.8: String Search for Color Index Map Coding [Z. Ma, W. Wang, M. Xu, H. Yu (Huawei Technologies)]

JCTVC-R0144 SCCE3 Test C.3: Combined Color Table and Index Map Coding of A.4 and B.8 [Z. Ma, W. Wang, M. Xu, H. Yu (Huawei Technologies)]

JCTVC-R0166 SCCE3: Test A.5 - Palette table prediction [Y. He, X. Xiu, Y. Ye, C.-M. Tsai (InterDigital)]

JCTVC-R0167 SCCE3: Test A.6 - Palette table generation [Y. He, X. Xiu, Y. Ye, C.-M. Tsai (InterDigital)]

JCTVC-R0168 SCCE3: Test B.9 - BWT-based index grouping [C.-M. Tsai, Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-R0169 SCCE3: Test B.10 - Palette index value mapping [C.-M. Tsai, Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-R0170 SCCE3: Test B.11 - Escape color prediction [C.-M. Tsai, Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-R0200 SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode [X. Zhang, R. Cohen (MERL)]

### SCCE3 cross checks (26)

JCTVC-R0069 Crosscheck SCCE3: Test B.5 – Palette Index Coding [W. Pu (Qualcomm)] [late]

JCTVC-R0070 Crosscheck SCCE3: Test B.10 – Palette Index Value Mapping [W. Pu (Qualcomm)] [late]

JCTVC-R0091 SCCE3: Cross-check of Test A.2 [C. Gisquet (Canon)] [late]

JCTVC-R0092 SCCE3: Cross-check of Test A.5 [C. Gisquet (Canon)] [late]

JCTVC-R0093 SCCE3: Cross-check of Test A.9 [C. Gisquet (Canon)] [late]

JCTVC-R0094 SCCE3: Cross-check of Test B.9 [P. Onno (Canon)] [late]

JCTVC-R0095 SCCE3: Cross-check of Test C.1 [P. Onno (Canon)] [late]

JCTVC-R0127 Cross check of ‘SCCE3: Test B.12 – Binarization of Escape Sample and Palette Index’ (JCTVC-R0065) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0160 Crosscheck of SCCE3 Test D.1 Single Color Mode [X. Zhang, R. Cohen (MERL)]

JCTVC-R0172 SCCE3: crosscheck report of SCCE3 test A.7 encoder-improved palette generation [Y. He, Y. Ye (InterDigital)]

JCTVC-R0205 Cross-check SCCE3: Test A.3 - Palette Stuffing [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0207 Cross-check SCCE3: Test A.8 - Improvements on palette prediction vector signaling [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0210 Cross-check SCCE3: Test B.7 - Run coding [J.-Y. Kao, X. Xiu, Y. He, Y. Yan (InterDigital)]

JCTVC-R0247 Cross check of ‘SCCE3: Test B.11 - Escape color prediction’ (JCTVC-R0170) by Interdigital [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0250 SCCE3: Cross-check results of Test A.6 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

JCTVC-R0251 SCCE3: Cross-check results of Test B.1 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

JCTVC-R0252 SCCE3: Cross-check results of Test B.2 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

JCTVC-R0265 SCCE3: crosscheck report of SCCE3 test D.2 on independent uniform prediction [Y.-W. Chen, Y.-W. Huang (MediaTek)] [late]

JCTVC-R0275 SCCE3: Cross-check of test B.6 (JCTVC-R0084) [C.-H. Hung, C.-L. Lin, Y.-J. Chang, C.-C. Lin, J.-S. Tu (ITRI)] [late]

JCTVC-R0294 Crosscheck of SCCE3 Test C.2 [T.-D. Chuang (MediaTek)] [late]

JCTVC-R0295 Crosscheck of SCCE3 Test C.3 [T.-D. Chuang (MediaTek)] [late]

JCTVC-R0296 Crosscheck of SCCE3 Test C.4 [T.-D. Chuang (MediaTek)] [late]

JCTVC-R0297 Crosscheck of SCCE3 Test A.4 [Y.-C. Sun (MediaTek)] [late]

JCTVC-R0298 SCCE3: Cross-check of Test B.4 (JCTVC-R0057) [X. Guo (Microsoft)] [late]

JCTVC-R0299 SCCE3: Cross-check of Test B.13 (JCTVC-R0066) [X. Guo (Microsoft)] [late]

JCTVC-R0302 SCCE3: Cross-check of JCTVC-R0143 on SCCE3 Test B.8 [F. Zou (Qualcomm)] [late] [miss]

## SCCE4: String matching for sample coding (10)

### SCCE4 summary and general discussion (1)

(chaired by JRO, 07-01 evening)

JCTVC-R0034 SCCE4: Summary Report on string matching for sample coding [Y. Chen, J. Xu]

JCTVC-R0098

The 1-D dictionary is enabled at the CU level with both horizontal scanning and vertical scans. Per-pixel matching is used for string matching. There are two types of 1-D dictionary mode, line based 1-D matching and shape based 2-D matching.

In the first 1D matching mode, the previous coded pixels using 1-D dictionary mode are kept in the dictionary (which is dynamically updated). In this mode, prediction mode and direct mode are allowed. If a matching is found, an offset (the offset relative to the position of the current pixel in the dictionary) and a matching length are signaled, otherwise the pixel value is signaled directly.  The above two cases are called prediction mode and escape mode respectively.

In the second 2D matching mode, all the previously reconstructed pixels can be used for prediction. Similar to 1D matching mode, 2D matching mode has prediction mode and escape mode. If a matching is found, a 2D MV and a matching length are signaled, otherwise, the pixel value is also signaled directly. Note that in 2D matching, when the current region starts a new row or column, the pixel used for prediction also starts a new row or column, as shown in the following figure. There is no additional memory requirement at the decoder side.

JCTVC-R0140

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and left CTU. The proposed method utilizes horizontal scan only and per-pixel matching. The proposed method utilizes 2D matching mode thus can share the same reconstruction buffer as in intra BC, with up to 2CTU search range. It supports prediction mode and escape mode. The previously coded pixels within the current CU are allowed to be chosen as reference pixels.

JCTVC-R0060

1D dictionary is enabled at the CU level, but based on sample matching: instead of matching three components of a full pixel, sample matching can take one sample of a pixel and then either stop or go to the next sample of the same pixel. The proposed method utilizes horizontal scan only and is based on 1D matching mode. It supports also prediction and escape mode. In the 1D matching mode, 1-D distance (pos) is used to represent 2-D displacement with the basic unit of sample instead of pixel. The maximum matching string length is extended from 273 to 12288. It is found that the matching string length has unusual characteristic that many special and large values such as multiples of 3 and multiples of the current CU width occur much more frequently than many other small values. Based on the characteristics, a mapping of the string length values to two matching parameters (pos and len) is introduced to improve the coding efficiency for length values. A histogram-based qp-dependent color quantization scheme is used as a pre-processing step of the input pixels to enable lossy match at the encoder side.

JCTVC-R0145

1D dictionary is enabled at the CU level but with a search range constrained to the current CTU and three left CTUs. The proposed method utilizes horizontal scan and per-pixel matching. The 1D string matching can be based on either 1D matching mode or 2D matching mode. Compared to other CE proposals, different approaches for both prediction mode and escape mode are used. In the prediction mode, residual can be signaled with the normal HEVC residue coding. In the escape mode, if the color value can be found in an adaptive color table, the index to it rather than the color value itself is coded, otherwise, the color value is coded and the adaptive color table is updated based on the color value. The adaptive table is derived at the decoder unless if it contains only one entry. In that case, the adaptive table is signaled.

The experimental results of the proposed methods are presented in Table 1. All the simulations are conducted on top of SCM1.0, and SCC common test conditions are used. The BD-rates are provided for the lossy configuration.

Table 1 Lossy results in full frame intra BC

Some properties of CE proposals are compared in Error: Reference source not found and Error: Reference source not found. As shown in these tables, commonalities may be identified among the CE proposals. The major difference of the proposed method, in terms of coding efficiency, encoder, decoder memory and complexity may be mainly due to different encoder optimizations.

Comparison of the key technical aspects of the CE proposals (1/2)

| Proposal | Scan | Min. run | Matching modes | Search Range | Res. Coding | Additional dec. Buffer | Lossy enc. Match |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0098/3.1 | V&H | 1 pixel | 1D/2D | Full frame | No | 1M pixels | No |
| R0140/3.2 | H | 1 pixel | 2D | 2 CTU | No | None | No |
| R0060/3.3 | H | 1 sample | 1D | Full frame | No | 4K/3~4M/3 pixels | Yes |
| R0145/3.4 | H | 1 pixel | 1D/2D | 3 or 4?CTU | Yes | 3x64x64 pixels | No |

Comparison of the key technical aspects of the CE proposals (2/2)

| Proposal | Enc. Hash Table Size (Bytes) | Eff. Highlight | Enc. Time AI, Other | Dec. time | Miscellaneous |
| --- | --- | --- | --- | --- | --- |
| R0098/3.1 | 160 M B (1080p) | -21.3% | 150%, 110% | 100% | 2D matching from pixels outside the current CU |
| R0140/3.2 | 64 K (2 CTUs) | -11.7% | 190%, 115% | 100% | 2D matching from any decoded pixels of the current CU |
| R0060/3.3 | 16K~ 16M (configurable) | -14.1% | 140%, 110% | 105% | Mapped table for offset/length coding |
| R0145/3.4 | 144 K (3 CTUs) | -11.4% | 105%, 105% | 120%~180% | Adaptive color table for escape pixel |

Tests “1” and “2” were originally planned but did not turn out to be successful.

For “2D matching” (R0098, R0140, R0145), the differences compared to IBC are that 1) the minimum size is 1x1 sample, and 2) some of the proposals (except R0145) do not encode residual. For “1D matching” (R060, R0145), some additional decoder processing (re-organsation of samples in the dictionary) is necessary.

The different proposals show significant gains (10-20% for AI in screen content classes), depending on restrictions imposed.

It however appears that the complexity is significantly increased (worst case memory access and CABAC throughput clearly higher than in current HEVC).

Analysis is missing about complexity aspect (particularly w.r.t. worst case memory access, CABAC throughput and organization of the dictionary at the decoder). Breakout activity (Y. Chen, J. Xu) to make an initial assessment on this for the different proposals. Revisit.

The side activity should also investigate the new proposals (R0137, R0255) in the complexity investigation. R0137 might have impact on CABAC throughput (unknown whether it becomes worse), R0255 on worst case memory accesses (claims to become better).

### SCCE4 primary contributions (4)

JCTVC-R0060 SCCE4: Results of Test 3.3 [L. Zhao and some other authors from Tongji University and MediaTek who did not inform the chairs about renaming an existing document]

JCTVC-R0098 SCCE4: Result of Test 3.1 [B. Li, J. Xu (Microsoft)]

JCTVC-R0140 SCCE4: Test 3.2 Pixel based 1D dictionary coding [F. Zou, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0145 SCCE4: Result of Test 4.3.4 for string search for sample coding [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei Technologies)]

### SCCE4 cross checks (5)

JCTVC-R0109 SCCE4: Cross-check of test 3.3 (JCTVC-R0060) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0110 SCCE4: Cross-check of test 3.4 (JCTVC-R0145) [B. Li, J. Xu (Microsoft)]

JCTVC-R0130 Crosscheck of SCCE4 Test 3.1 on Combined Normal 1-D Dictionary Mode and Reconstruction Based 1-D Dictionary Mode [R.-L. Liao, C.-C. Chen, T.-S. Chang, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

JCTVC-R0197 SCCE4: Crosscheck results of Test 3.2 [J. Ye, S. Liu (MediaTek)] [late]

JCTVC-R0253 SCCE4: Cross-check result of Test 3.1 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

## SCCE5: Inter-component prediction and adaptive colour transform (11)

### SCCE5 summary and general discussion (1)

JCTVC-R0035 SCCE5: Summary report of HEVC Screen Content Coding Core Experiment 5 on Inter-component prediction and adaptive color transforms [X. Xiu, J. Chen (CE coordinators)]

### SCCE5 primary contributions (6)

JCTVC-R0072 SCCE5 3.1.2: Extended inter-component prediction (JCTVC-Q0036) [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-R0099 SCCE5: Result of Test 3.2.2 [B. Li, J. Xu (Microsoft)]

JCTVC-R0115 SCCE5 Test 3.1.3: Adaptive alpha-parameter coding in cross-component prediction [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

JCTVC-R0146 SCCE5: Result of Test 3.3.1 for modified LMChroma [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)]

JCTVC-R0278 Crosscheck for SCCE5 Test 3.1.1 in JCTVC-R0146 [X. Zhang (MediaTek)] [late]

JCTVC-R0147 SCCE5 Test 3.2.1: In-loop color-space transform [L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm), X. Xiu, Y. He, Y. Ye (InterDigital)]

### SCCE5 cross checks (4)

JCTVC-R0073 SCCE5: Cross-check on subtest 3.2.1 with disabling SAME_BD_YCOCG [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-R0111 SCCE5: Cross-check of combination test in SCCE5 (JCTVC-R0147) [B. Li, J. Xu (Microsoft)]

JCTVC-R0177 SCCE5: Crosscheck report of SCCE5 test 3.1.2 Extended inter-component prediction [L. Zhang (Qualcomm)] [late]

JCTVC-R0254 SCCE5: Cross-check result of Test 3.2.2 [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei Technologies)] [late]

# Non-CE Technical Contributions (XX)

## SHVC (11)

### General (1)

JCTVC-R0164 Bug fix in SHVC draft text [P. Bordes (Technicolor), Y. Ye (InterDigital)]

### SCE1 related (colour gamut and bit depth scalability) (7)

Note that R0157 also contains one aspect regarding CGS.

JCTVC-R0151 Non-SCE1: Improved CGS partitioning on top of SHM-6 [X. Li, J. Chen, M. Karczewicz (Qualcomm), C. Auyeung (Sony), K. Misra, S.-H. Kim, A. Segall (Sharp), P. Bordes, P. Andrivon, E. François, F. Hiron (Technicolor)]

Discussed in Track A (GJS) on 2nd day (Tue) p.m.

Presenter was asked to upload presentation.

Relative to the current CGS function (with uniform partitioning of chroma) this proposal (which has non-uniform threshold positioning to segment each chroma compontent instead of always using mid-grey as the threshold).

The CGS LUT coefficient values (which do not have a range limit in the method tested in section 2 of the SCE) are restricted to be no more than 12 bits.

In addition, the CGS LUT coefficient coding proposed in JCTVC-R0163 is also enabled in this proposal.

luma gains were reported as: 1x AI 1.2%, 2x AI 0.9%, 1x RA 1.1%, 2xRA 0.4% (somewhat more in chroma).

The proposal requires one extra comparisons per chroma sample. However, there is a reduction in the number of additions in the mapping function.

Decision: Adopted.

JCTVC-R0300 Non-SCE1: improved residual coefficient coding for 3D LUT [Y. Ye, Y. He (InterDigital)] [late]

Discussed in Track A (GJS) on 2nd day (Tue) p.m.

This contribution proposes a change to the residual coefficient coding method proposed in JCTVC-R0151. It is asserted that having a more frequently updated 3D LUT causes the overall 3D LUT signaling overhead of JCTVC-R0151 to increase by 14% compared to SHM6.0 (at the PPS level, because more PPSs are sent, not because the amount of bits per PPS is increased). It is reported that the proposed change can reduce the overall signaling overhead of 3D LUT in JCTVC-R0151 by 8%. With the proposed change, the signalling overhead of 3D LUT using JCTVC-R0151 is reduced and reportedly becomes comparable to that of SHM6.0.

The contribution proposes to signal, within the PPS mapping information, as a 2-bit syntax element, the number of bits of precision used for the FLC part of the residual coefficients of the mapping function.

The coding efficiency is is reportedly essentially unaffected.

Decision: Adopted.

JCTVC-R0310 Non-SCE1: Crosscheck report of improved residual coefficient coding for 3D LUT (JCTVC-R0300) [X. Li (Qualcomm)] [late]

JCTVC-R0150 On CGS Signaling Improvement and Constraint [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

TBP.

JCTVC-R0256 Cross check of non-SCE1: Improved CGS partitioning on top of SHM-6 (JCTVC-R0151) [Y. Ye (InterDigital)] [late]

JCTVC-R0179 Non-SCE1: 3D LUT with larger sizes [Y. Ye, Y. He (InterDigital)]

JCTVC-R0242 Cross-Checking of Non-SCE1 JCTVC-R0179 [P. Bordes (Technicolor)] [late]

    - Upsampling process, phase offset (3)

See also AHG report R0013.

JCTVC-R0209 AHG13: Re-sampling process with generic phase [E. Alshina (Samsung)]

Reviewed in Track A (GJS) on 2nd day (Tue.) a.m.

This document explains the resampling process with generic phase (versus phase and delta as in the current draft) proposed by AHG13. Normative changes related to this contribution are part of recommendation of AHG13 where the "system of 3 flags" for resampling process phase derivation is replaced by explicit signaling for phases. This document describes proper settings of resampling phases for different use cases and estimates potential effect on accuracy of resampling process.

Editor action item: It was suggested to include, if feasible, some informative text about how to set the phase syntax element values (left to the discretion of the editors).

JCTVC-R0312 AHG13 Crosscheck for Re-sampling process with generic phase (JCTVC-R0209) [K. Minoo (Arris Group)] [late]

JCTVC-R0220 AHG13: On reference position calculation [T. Yamamoto, T. Takeshi, T. Ikai (Sharp)]

Reviewed in Track A (GJS) on 2nd day (Tue.) a.m.

In this contribution, three modifications relating to reference position calculation are proposed. Proposal 1 suggests signalling scaled reference layer offsets, reference layer offsets, and resample phases associated with the common reference layer in a single loop. Proposal 2 suggests the variables representing reference layer offsets to be derived using the prediction by the corresponding scaled reference layer offset. Proposal 3 suggests removing clipping in reference position calculation so that whole reference layer picture can be accessed.

The current Q1008 SHVC text uses the boundary clipping region in the reference layer as the boundary of the coded picture region, not the boundary of the reference region (and the "reference region" is not a concept found in the Q1008 text). The proposal does not suggest changing that. In the discussion, one participant indicated that for some uses it could be preferable to clip to the boundary of the reference region (e.g., if the region outside of that boundary is black bars or an independently coded region). However, several other participants suggested that changing this would add significant extra complexity and thus would be undesirable – so no action was taken on that.

The proposal would make the position calculation for resampling purposes in the area outside the EL scaled ref layer region similar to the way motion data is resampled – which is performed the same way without regard to the boundaries of the reference region (and the "reference region" is not a concept found in the Q1008 text). This differs from the AHG13 output text, but was identified as an open issue in the AHG13 report. Decision: Remove the clip to the boundary of the EL scaled ref layer region.

From AHG 13 report notes: "Current syntax signal 1) scaled reference layer offsets, 2) reference layer offsets, and 3) resample phases in independent loops. In two layers case, it could result in signaling the same 6 bit layer id syntax three times. It is suggested to discuss whether we should keep this design or signalling the parameters associated with the same reference layer in one loop."

The proposal suggests to combine three loops in the PPS extension and use presence flags within the combined loop. In discussion, it was suggested to use 3 presence flags rather than two, so there is also a flag for the scaled reference layer offset information. This was agreed. Decision: Adopt this aspect as described (with three flags).

As a final element, the proposal suggests to predict the reference layer offset values from the scaled reference layer offset values. It was commented that the solution does not seem adequately generic for different applications and may be getting too complicated and fancy. No action was taken on this.

## SCC (102)

### SCCE1 related (intra block copy) (40)

JCTVC-R0050 Non-SCCE1: Intra block copy masking [J. Lainema, K. Ugur, M. M. Hannuksela (Nokia)]

JCTVC-R0056 Restriction of CU sizes for Intra Block Copy [J. H. Ma, H. H. Jo, M. Ismail, Y. J. Ahn, W. Lim, D. G. Sim (KWU)]

JCTVC-R0074 Non-SCCE1: Adaptive switching between differential and direct coding for intra block copy vectors [K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)]

JCTVC-R0315 Cross-check of JCTVC-R0074 on adaptive switching between differential and direct coding for intra block copy vectors [C. Pang (Qualcomm)] [late]

JCTVC-R0088 Non-SCCE1: Vector residual coding for Inter and IBC modes [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-R0287 Crosscheck of JCTVC-R0088 [S.-T. Hsiang, T.-D. Chuang (MediaTek)] [late] [miss]

JCTVC-R0089 Non-SCCE1: IBC BV throughput issue [C. Gisquet, G. Laroche, T. Poirier, P. Onno (Canon)]

JCTVC-R0307 Cross-check Non-SCCE1: IBC BV throughput issue (JCTVC-R0089) [X. Xiu, Y. Ye (InterDigital)] [late]

JCTVC-R0096 Non-SCCE1: Intra block copy with modified motion vector predictor [Dong Jiang, Xuerong Zhang, Yaqiang Wu (Lenovo)]

JCTVC-R0100 Non-SCCE1: Unification of intra BC and inter modes [B. Li, J. Xu (Microsoft)]

JCTVC-R0258 Cross-check of JCTVC-R0100 on intraBC and inter unification [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-R0117 Non-SCCE1: Segmental prediction for intra block copy [K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)]

JCTVC-R0288 Cross-check of Non-SCCE1: Segmental prediction for intra block copy (JCTVC-R0117) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0118 Modified Deblocking Filter for Intra Block Copy (IBC) [M. Ismail, H. H. Jo, J. H. Ma, W. Lim, D. G. Sim (KWU)]

JCTVC-R0123 Non-SCCE1: On block vector coding [C. Rosewarne, M. Maeda (Canon)]

JCTVC-R0324 Crosscheck of JCTVC-R0123 on Non-SCCE1: On block vector coding [S. H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-R0126 On deblocking for intra block copy [C. Rosewarne, M. Maeda (Canon)]

JCTVC-R0285 Crosscheck of JCTVC-R0126 on deblocking for intra block copy [C. Pang (Qualcomm)] [late]

JCTVC-R0134 Non-SCCE1: Unified BVD & MVD coding using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-R0283 Cross-check of JCTVC-R0134 on Unified BVD & MVD coding using a universal entropy coding scheme [P. Onno (Canon)] [late] [miss]

JCTVC-R0162 Non-SCCE1: Intra block copy hash search enhancement [Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-R0212 Crosscheck of JCTVC-R0162 on Intra block copy hash search enhancement [C. Pang (Qualcomm)] [late]

JCTVC-R0165 Non-SCCE1: Improved intra block copy coding with block vector derivation [Y. He, Y. Ye, X. Xiu (InterDigital)]

JCTVC-R0289 Cross-check of Non-SCCE1: Improved intra block copy coding with block vector derivation (JCTVC-R0165) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0181 Non-SCCE1: Block vector coding for Intra block copy [M. Karczewicz, K. Rapaka, C. Pang, J. Sole (Qualcomm)]

JCTVC-R0182 Non-SCCE1: On Intra block vector difference redundancy removal [K. Rapaka, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0188 Non-SCCE1: Memory bandwidth reduction for Intra block copy [C. Pang, K. Rapaka, J. Sole, X. Li, T. Hsieh, M. Karczewicz (Qualcomm)]

JCTVC-R0320 Crosscheck of JCTVC-R0188 on Non-SCCE1: Memory bandwidth reduction for Intra block copy (Method 1) [K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-R0189 Non-SCCE1: Intra block copy deblocking [C. Pang, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0262 Cross-check of 'Non-SCCE1: Intra block copy deblocking' (JCTVC-R0189) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0201 Block vector predictor selection for intra block copy [S. Yang, H. Shim, D. Lee, B. Jeon (SKKU)]

JCTVC-R0203 Non-SCCE1: Default vector selection for Intra block copy BV prediction [X. Xu, T.-D. Chuang, S. Liu, J. Ye, S. Lei (MediaTek)]

JCTVC-R0243 Cross-check of JCTVC-R0203 Non-SCCE1: Default vector selection for Intra block copy BV prediction, simulation B [R. Cohen (MERL)] [late] [miss]

JCTVC-R0204 Non-SCCE1: PU Intra block copy with flipping mode [X. Xu, S. Liu, J. Ye, K. Zhang, S. Lei (MediaTek)]

JCTVC-R0291 Cross-check of Non-SCCE1: PU Intra block copy with flipping mode (JCTVC-R0204) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0208 Non-SCCE1: Additional test results on intra block copy (IBC) [M. Zhou (Broadcom)]

JCTVC-R0301 Non-SCCE1: Cross-check of Additional results on intra block copy (IBC) (JCTVC-R0208) [Z. Ma, W. Wang, F. Duanmu, M. Xu, H. Yu (Huawei R&D USA)] [late]

JCTVC-R0218 trType for Intra Block Copy for screen content coding [X. Zhang, R. Cohen (MERL)]

JCTVC-R0269 Non-SCCE1: Analysis of Full Frame IBC Block Vector Distribution [F. Duanmu, Z. Ma, M. Xu, W. Wang, H. Yu (Huawei R&D USA)] [late]

JCTVC-R0309 Non-SCCE1: Combination of JCTVC-R0185 and JCTVC-R0203 [C. Pang, J. Sole, Y. Chen, M. Karczewicz (Qualcomm), X. Xu, S. Liu, T.-D. Chuang, S. Lei (MediaTek)] [late]

### SCCE2 related (line-based intra copy) (4)

(handled in context of SCCE2, 07-01 afternoon/evening)

JCTVC-R0132 Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C [C.-C. Chen, T.-S. Chang, R.-L. Liao, C.-W. Kuo, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Juo (NCTU/ITRI))]

See notes under SCCE2 summary

JCTVC-R0264 Cross-check of JCTVC-R0132: Non-SCCE2: Line-based Intra Copy with Unconstrained 2-D BV Coding for Test C [J. Ye, X. Xu (MediaTek)] [late] [miss]

JCTVC-R0281 Non-SCCE2: An improvement on Intra Line Copy of SCCE2 [L. Liping, X. Chen, T. Lin] [late]

This contribution proposes an improvement on Intra Line Copy (Line-based Intra Copy) technique described in the Screen Content Core Experiment 2 (JCTVC-Q1122). The improvement is to allow a current line to copy a reference line inside the same CU as the current line or partially overlapped with the current line. Experiment results report -1.8%, -0.8%, and -0.4% BD-rate savings for YUV TGM 1080p sequences under AI, RA, and LB, respectively, using SCCE2 TestD as the anchor.

Less gain in lossless coding.

It is reported that in contrast to the current line-based copy, this modification also allows usage of closer lines from same CU and same PU.

Question: Does encoder and decoder use the same prediction in this case? They do, since the residual is not added yet to the prediction.

The approach definitely requires additional processing and may also impact on latency/pipelining. More analysis necessary about complexity/memory accesses.

Further study in ongoing CE

JCTVC-R0292 Crosscheck of JCTVC-R0281: An improvement on Intra Line Copy of SCCE2 [R.-L Liao, C.-C Chen, T.-S Chang, W.-H Peng, H.-M Hang (NCTU/ITRI)] [late] [miss]

### SCCE3 related (palette mode) (43)

JCTVC-R0051 Non-SCCE3: Adaptive escape scheme for palette mode [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

JCTVC-R0075 Non-SCCE3: Encoder modification of palette mode for escape pixels [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

JCTVC-R0263 Cross-check of 'Non-SCCE3: Encoder modification of palette mode for escape pixels' (JCTVC-R0075) by ITRI [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0076 Non-SCCE3: Run-copy coding method for two-color index map of palette CU [Y.-J. Chang, C.-L. Lin, C.-H. Hung, C.-C. Lin, J.-S. Tu (ITRI)]

JCTVC-R0068 Crosscheck Non-SCCE3: Run-copy coding method for two-color index map of palette CU [W. Pu (Qualcomm)] [late]

JCTVC-R0078 Non-SCCE3: Modified copy above mode for palette based coding [J. Zhu, Z. Xu, J. Ye (Fujitsu R&D Center)]

JCTVC-R0079 Non-SCCE3: Skip palette size for palette based coding [J. Zhu, J. Ye (Fujitsu R&D Center)]

JCTVC-R0080 Non-SCCE3: Modified Escaped pixel mode in palette based coding [J. Zhu, J. Ye (Fujitsu R&D Center)]

JCTVC-R0112 Non-SCCE3: Bi-color intra mode for screen content coding [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, C.-H. Hung (ITRI)] [late]

JCTVC-R0113 Non-SCCE3: Improved Palette Index Coding with Contextualization [T. Laude (Univ. Hannover)]

JCTVC-R0135 Non-SCCE3: Palette index coding using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-R0305 Cross-check of JCTVC-R0135 on Palette index coding using a universal entropy coding scheme (Non-SCCE3) [Y. He, Y. Yan (InterDigital)] [late]

JCTVC-R0136 Non-SCCE3: Run coding of the palette index map using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-R0323 Crosscheck of JCTVC-R0136 Non-SCCE3: Run coding of the palette index map using a universal entropy coding scheme [X. Zhang, R. Cohen (MERL)] [late] [miss]

JCTVC-R0161 Non-SCCE3: Simplified palette size coding [J. Zhao, S. K. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-R0322 Cross check of ‘Non-SCCE3: Simplified palette size coding’ (JCTVC-R0161) by Sharp [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-R0174 non-SCCE3: Contexts for coding index runs [R. Joshi, M. Karczewicz, W. Pu, V. Seregin, J. Sole (Qualcomm)]

JCTVC-R0316 Cross check of ‘non-SCCE3: Contexts for coding index runs’ (JCTVC-R0174) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-R0175 non-SCCE3: Additional results for subtest C.4 [R. Joshi, V. Seregin, W. Pu, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-R0308 Cross-check of JCTVC-R0175: non-SCCE3: Additional results for subtest C.4 [S. H. Kim, A. Segall (Sharp)] [late]

JCTVC-R0198 Non-SCCE3 Test D.1 Modified single color mode [P. Lai, S. Liu, Y.-W. Chen, S. Lei (MediaTek)]

JCTVC-R0222 Cross-check of JCTVC-R0198 Non-SCCE3 Test D.1 Single Color Mode, Method 1 [R. Cohen, X. Zhang (MERL)]

JCTVC-R0202 Non-SCCE3: Copy from previous row mode for palette coding [F. Zou, R. Joshi, J. Sole, M. Karczewicz, W. Pu, V. Seregin (Qualcomm)]

JCTVC-R0213 Non-SCCE3: Palette and deblocking [J. Sole, C. Pang, W. Pu, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]

JCTVC-R0319 Cross check of ‘Non-SCCE3: Palette and deblocking’ (JCTVC-R0213) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-R0215 Non-SCCE3: WD text fixes for palette [J. Sole, R. Joshi, V. Seregin, W. Pu, M. Karczewicz (Qualcomm)]

(Question: How can WD text be fixed for something that is not included?)

JCTVC-R0318 Crosscheck of JCTVC-R0215 on WD text fixes for palette [Y.-C. Sun, Y.-W. Huang, S. Lei (MediaTek)] [late]

JCTVC-R0228 Non-SCCE3: Run-length coding for palette predictor [V. Seregin, M. Karczewicz, W. Pu, R. Joshi, J. Sole (Qualcomm)]

JCTVC-R0270 Cross-check of JCTVC-R0228 on Run-length coding for palette predictor (Non-SCCE3) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0229 Non-SCCE3: Palette predictor resetting [V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)]

JCTVC-R0271 Cross-check of JCTVC-R0229 on Palette predictor resetting (non-SCCE3) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0232 Non-SCCE3: high throughput palette coding [S. H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

JCTVC-R0233 Modifications to palette coding for tiles/slices/dependent slices/wavefronts [K. Misra, S.-H. Kim, J. Zhao, A. Segall (Sharp)]

JCTVC-R0234 Palette coding and scaling lists [K. Misra, S.-H. Kim, J. Zhao, A. Segall (Sharp)]

JCTVC-R0237 Non-SCCE3: Delta QP signalling in palette mode [J. Sole, R. Joshi, W. Pu, V. Seregin, M. Karczewicz (Qualcomm)]

JCTVC-R0266 Non-SCCE3: Palette Table Prediction by the Nearest Palette Table [C.-H. Hung, C.-L. Lin, Y.-J. Chang, C.-C. Lin, J.-S. Tu (ITRI)] [late]

JCTVC-R0286 Cross-check of JCTVC-R0266: Non-SCCE3: Palette Table Prediction by the Nearest Palette Table [S. H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-R0268 Non-SCCE3: Results of SCCE3 with support of 2x4 CTUs as reference buffer [Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei R&D USA)] [late]

JCTVC-R0277 Non-SCCE3: Cross-check of JCTVC-R0268 SCCE3 with support of 2x4 CTUs as reference buffer [?? (Mediatek??)] [late] [miss]

JCTVC-R0304 Non-SCCE3: Improvements for SCCE3 Test C3 [Z. Ma, W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei R&D USA)] [late]

JCTVC-R0306 SCCE3 base Software Bugfix Report [?? (Qualcomm)] [late] [miss]

JCTVC-R0311 Non-SCCE3: Combination of palette tools [Y. He, X. Xiu, Y. Ye (InterDigital), R. Joshi, V. Seregin, W. Pu, M. Karczewicz, J. Sole (Qualcomm)] [late]

JCTVC-R0317 Non-SCCE3: memory reduction for palette mode software [C. Gisquet, G. Laroche, P. Onno (Canon), T.-D. Chuang (MediaTek)]

### SCCE4 related (string matching for sample coding) (3)

(handled in context of SCCE4, 07-01 afternoon/evening)

JCTVC-R0137 Non-SCCE4: Unified method for coding string matching syntax using a universal entropy coding scheme [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

This contribution develops a unified method for coding new syntax elements employed by string matching coding. The proposed method utilizes a universal entropy coding scheme representing a syntax value by sending its most significant bit (MSB) index and refinement bits. It is reported to achieve 4.8%, 2.9%, and 2.2% average Luma BD-rate savings compared with the results of the SCCE4 Test3.1 for YUV, text & graphics with motion, 1080p sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively, of the screen content coding (SCC) CTCs. It is also reported that the proposed universal entropy coding scheme is experimentally shown to be efficient for coding other new syntax elements used by the new SCC coding tools and can thus further support coding some other new SCC syntax elements by a unified entropy coding scheme.

Perform intial complexity analysis in context of side activity (see further notes under SCCE4 summary).

JCTVC-R0303 Non-SCCE4: Cross-check of JCTVC-R0137 [F. Zou (Qualcomm)] [late] [miss]

JCTVC-R0225 Non-SCCE4: Constrained run for 1D dictionary coding [F. Zou, Y. Chen, J. Sole, M. Karczewicz (Qualcomm)] [late]

This proposal presents preliminary results by constraining the current 1D dictionary (string match) methods investigated under the current SCCE4. It is reported that the current 1D dictionary methods may require much more times of memory access from different locations of the reference area compared to e.g., Intra BC. Therefore, it could be beneficial to address more pixels for each string match. In this proposal, the matching length is constrained to be equal or larger than a predefined threshold T, e.g., 4. The simulation results show that the constraint as implemented based on SCCE4 proposals JCTVC-R0098 and JCTVC-R0140 leads to noticeable bitrate increase. It is suggested to have discussions on 1) whether the proposed constraint is needed, if yes, 2) in which extent such a constraint should apply, e.g., what is a suitable number of T? and 3) in which level of reference region such a constraint does not need to apply, e.g., should the constraint for a match from pixels within the same CU and that for a match from pixels outside the current CTU be different?

Further discussion in BoG (see under SCCE4 summary).

### SCCE5 related (inter-component prediction and adaptive colour transform) (7)

JCTVC-R0178 Non-SCCE5: On index signaling of cross component prediction [L. Zhang, M. Karczewicz, J. Chen, J. Sole (Qualcomm)]

JCTVC-R0284 Cross-check of R0178 Non-SCCE5: On index signaling of cross component prediction by Qualcomm [X. Zhang, R. Cohen (MERL)] [late] [miss]

JCTVC-R0180 Non-SCCE5: Deblocking filter for in-loop color-space transform [L. Zhang, J. Chen, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-R0290 Cross-check of Non-SCCE5: Deblocking filter for in-loop color-space transform (JCTVC-R0180) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-R0219 Improvement of cross-component prediction [X. Zhang, R. Cohen (MERL)]

JCTVC-R0199 Cross-check of JCTVC-R0218 on Transform by MERL [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-R0321 Non-SCCE5: Modified alpha-parameter sets in cross-component prediction for SCC [K. Kawamura, S. Naito (KDDI)] [late]

### Other (11)

JCTVC-R0087 On Sample Adaptive Band Value for Screen Content Coding [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-R0102 Fix and improvement for hash based inter search [B. Li, J. Xu (Microsoft)]

JCTVC-R0259 Cross-check of JCTVC-R0102 Fix and improvement for hash based inter search [X. Xu, S. Liu (MediaTek)] [late] [miss]

JCTVC-R0104 On HAD calculation in the lossless RDO process [B. Li, J. Xu (Microsoft)]

JCTVC-R0260 Cross-check of JCTVC-R0104 on HAD calculation in the lossless RDO process [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0105 On motion estimation start point [B. Li, J. Xu (Microsoft)]

JCTVC-R0261 Cross-check of JCTVC-R0105 on motion estimation start point [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-R0106 Adaptive motion vector resolution for screen content [B. Li, J. Xu, G. J. Sullivan, Y. Zhou, B. Lin (Microsoft)]

JCTVC-R0114 Optionally disabling the usage of the intra boundary filters for SCC [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

JCTVC-R0282 Cross-check of JCTVC-R0114 on disabling the usage of the intra boundary filters for SCC [P. Onno (Canon)] [late] [miss]

JCTVC-R0314 On intra chroma boundary filtering [B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen (MERL)] [late]

## HL syntax common issues for range extensions, 3D, SHVC, and single-layer HEVC coding (1)

### Auxiliary pictures (1)

JCTVC-R0062 On monochrome coding (concerning multi-layer profiles and auxiliary pictures) [M. M. Hannuksela (Nokia)]

### Other (0)

## HL syntax in SHVC and 3D extensions (30)

### General HLS issues and cleanups (12)

JCTVC-R0155 MV-HEVC/SHVC HLS: Comments on SHVC and MV-HEVC [S. Deshpande (Sharp)]

Proposal 1 belongs to this agenda category.

Proposal 2 relates to the sub-bitstream property SEI message and hence relates to agenda category 6.5.2.

JCTVC-R0157 Miscellaneous questions and clarifications for SHVC [J. Boyce (Vidyo)]

TBP except as noted.

Reviewed in Track A (GJS) on 2nd day (Tue.) a.m.

Contains one aspect commenting on CGS.

The colour mapping 3D LUT is signaled in the PPS. Each 3D LUT may require up to 384*12 bits.  There may be up to 64 picture parameter sets. In the worst case, a significant amount of 3D LUT storage is required, 64 * 384 * 12 bits.

It is proposed to allow only 8 picture parameter sets containing 3D LUTs, in order to limit the storage requirement.

Proposed language: "When pps_pic_parameter_set_id greater than or equal to 8, colour_mapping_enabled_flag shall be equal to 0."

In the discussion, an alternative was suggested, being that no more than 8 can have the mapping data, but that they do not need to be the PPS ID values from 0 to 7. However, it seemed difficult to determine how to specify this, considering that we ordinarily don't refer to non-active parameter sets.

Decision: Adopt (this aspect).

JCTVC-R0224 MV-HEVC/SHVC HLS: Clarifications to the specification of end of sequence (EOS) NAL unit [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCTVC-R0071 MV-HEVC/SHVC HLS: On cross-layer impacts of IRAP and EOS [M. M. Hannuksela (Nokia)]

JCTVC-R0226 MV-HEVC/SHVC HLS: On indication of the use of temporal motion vector prediction [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-R0227 MV-HEVC/SHVC HLS: Miscellaneous cleanups [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm)]

JCTVC-R0230 MV-HEVC/SHVC HLS: Unavailable and useless stuff [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)]

JCTVC-R0238 MV-HEVC/SHVC HLS: Editorial cleanups [A. K. Ramasubramonian, Y.-K. Wang, Hendry (Qualcomm)]

JCTVC-R0272 HEVCv1/MV-HEVC/SHVC HLS: Conformance cleanups [Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)] [late]

JCTVC-R0276 MV-HEVC/SHVC HLS: On sub-bitstream extraction [M. M. Hannuksela (Nokia)] [late]

JCTVC-R0236 MV-HEVC/SHVC HLS: On access unit boundary detection [M. M. Hannuksela (Nokia)]

JCTVC-R0279 MV-HEVC/SHVC HLS: More miscellaneous cleanups [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)] [late]

### Layered decoding constraints and capability (AHG10 related) (11)

JCTVC-R0158 is directly related to proposals in this category too.

JCTVC-R0041 Use of general_profile_space for auxiliary picture profile indication [J. Boyce (Vidyo)]

JCTVC-R0054 AHG10: Comments on JCTVC-R0041/JCT3V-I0023 [M. M. Hannuksela (Nokia)]

JCTVC-R0042 AHG10: Profile indication for additional layer sets [M. M. Hannuksela (Nokia)]

JCTVC-R0043 SHVC/MV-HEVC level definitions and related decoder capability requirements [A. K. Ramasubramonian, Y.-K. Wang, K. Rapaka, Hendry (Qualcomm)]

JCTVC-R0044 AHG10: On decoder capability indication [M. M. Hannuksela (Nokia)]

JCTVC-R0046 AHG10: Comments on JCTVC-R0043/JCT3V-I0022 [M. M. Hannuksela (Nokia)]

JCTVC-R0052 Picture size related constraints and level limits [S. Deshpande (Sharp), Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), M. M. Hannuksela (Nokia)]

JCTVC-R0053 AHG10 and MV-HEVC/SHVC HLS: On invocation of a correct decoding process for the current picture [M. M. Hannuksela (Nokia)]

JCTVC-R0125 MV-HEVC/SHVC HLS: On layer set and bitstream extraction [B. Choi, C. Kim (Samsung)] [late]

JCTVC-R0235 AHG10: Processing of bitstreams without an available base layer [M. M. Hannuksela (Nokia)]

JCTVC-R0280 MV-HEVC/SHVC HLS: Bitstream conformance restrictions in subclause C.4 [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)] [late]

### IRAP alignment and POC derivation (1)

JCTVC-R0223 MV-HEVC/SHVC HLS: On POC [Hendry, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

### HLS for hybrid scalability (2)

JCTVC-R0124 MV-HEVC/SHVC HLS: On hybrid codec scalability [B. Choi, C. Kim (Samsung)] [late]

JCTVC-R0153 MV-HEVC/SHVC HLS: On External Base Layer [S. Deshpande (Sharp)]

### Parameter sets (1)

JCTVC-R0049 MV-HEVC/SHVC HLS: VPSs with nuh_layer_id greater than 0 [M. M. Hannuksela (Nokia)]

### HRD related (1)

JCTVC-R0231 MV-HEVC/SHVC: Signalling and use of HRD parameters for bitstream partitions [Y.-K. Wang, Hendry, A. K. Ramasubramonian (Qualcomm), S. Deshpande (Sharp), M. M. Hannuksela (Nokia)] [late]

### Miscellaneous HLS topics (2)

JCTVC-R0154 MV-HEVC/SHVC HLS: On Alternative Output Layer and Default Output Layer Indication [S. Deshpande (Sharp)]

JCTVC-R0156 MV-HEVC/SHVC HLS: Move conformance cropping window parameters to rep_format( ) syntax structure of VPS extension [J. Boyce, D. Hong (Vidyo)]

## SEI and VUI (11)

### General (3)

JCTVC-R0217 SEI comments [C. Fogg (MovieLabs)]

JCTVC-R0221 HEVCv1/MV-HEVC/SHVC HLS: On extensibility of nesting SEI messages [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCTVC-R0255 MV-HEVC/SHVC HLS: On persistence of SEI messages [M. M. Hannuksela (Nokia)] [late]

### Existing SEI messages or VUI indications (2)

JCTVC-R0257 Language indication for Overlay information SEI message [K. Suehring (Fraunhofer HHI)] [late]

JCTVC-R0274 MV-HEVC/SHVC HLS: On active parameter sets SEI message [M. M. Hannuksela (Nokia)] [late]

### New SEI/VUI proposals (6)

JCTVC-R0059 HLS: Dependent RAP indication SEI message [Martin Pettersson, Jonatan Samuelsson, Rickard Sjöberg, Ruoyang Yu, Jacob Ström (Ericsson)]

JCTVC-R0159 HEVC/MV-HEVC/SHVC HLS: SEI message recommended by MMT for improvement of HEVC packet loss resilience [M. Sychev, S. Ikonin (Huawei)]

JCTVC-R0139 Modulation channel SEI message [F. Le Leannec, S. Lasserre, E. Francois (Technicolor)]

JCTVC-R0293 Crosscheck of JCTVC-R0139: Modulation channel SEI message [O. Nakagami, T. Suzuki (Sony)] [late]

JCTVC-R0325 Cross-check of JCTVC-R0139 (Modulation channel information SEI message) [Patrick Boissonade (Orange), Pierrick Philippe (Orange)] [late] [miss]

JCTVC-R0267 Usage of modulation channel for high bit-depth signal encoding [F. Le Léannec, S. Lasserre, E. Francois (Technicolor)] [late]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (XX)

JCTVC-R0103 On referencing structure supporting temporal scalability [B. Li, J. Xu (Microsoft)]

# Unclear, withdrawn (2)

JCTVC-R0055 Withdrawn

JCTVC-R0152 Withdrawn

JCTVC-R0313 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

(joint meetings)

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the Friday of the week preceding the meeting (3 Jan).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs (to be updated)

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

CE plan final at same time as corresponding software except for SCE1 & 4 due to test sequence issues.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 12.0 and SHM 2.0 should be available within 2 weeks after the meeting. [To be fixed]

- HM 12.0+RExt should be available within 1 week after HM 12.0 availability.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1002 HEVC Test Model 15 (HM 15) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 15.0 software version and the reference configuration encodings according to JCTVC-L1100 common conditions. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC v.1, RExt and SHVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye (vice-chairs) | N |
| Verification test preparation (AHG5) (jct-vc@lists.rwth-aachen.de) - Complete the HEVC verification test report. - Make preparations for verification testing of HEVC for interlaced video content. - Make preparations for verification testing of HEVC range extensions. | V. Baroncini (chair), M. Karczewicz, N. Ramzan, C. Rosewarne, K. Sharman, T. K. Tan, J.-M. Thiesse, W. Wan, J. Wen (vice-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1005 HEVC RExt draft text. - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary), D. Flynn, M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan, T. Suzuki | N |
| Range extensions software and test model development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Prepare and deliver HM 14.0-RExt-7.0 software version and the reference configuration encodings according to JCTVC-P1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Update HEVC Range extensions test model 7 encoder description (Q1013). - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and the performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for testing screen content coding performance, in coordination with AHG16. - Make recommendations for test conditions for screen content coding. - Coordinate with SCCEs on combination experiments. - Finalize the common test conditions SCCEs. | R. Cohen and H. Yu (co-chairs), A. Duenas, T. Lin, S. Liu, J. Sole, J. Xu, Y. Ye (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Study mechanisms to indicate decoding capabilities/requirements for auxiliary picture layers - Investigate mechanisms for picture referencing across CRA pictures - Study and compare mechanisms for enabling temporal enhancement layers and diagonal inter-layer prediction - Study the operation of the Hypothetical Reference Decoder for multi-layer operation. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Coordinate efforts with JCT-3V AHG6 on high-level syntax issues in relation to 3D extensions. | J. Boyce and M. M. Hannuksela (co-chairs), Y. Chen, S. Deshpande, J. Samuelsson, G. Tech, Y.-K. Wang, P. Wu (vice-chairs) | N |
| Layered coding constraint specifications and capability indications (AHG10) (jct-vc@lists.rwth-aachen.de) - Study how constraints on bitstream syntax affect specifications of bitstream conformance and decoder conformance for layered coding. - Study indications of bitstream properties and decoding process requirements for layered coding. - Identify potential side-effects of constraint specifications and capability indications and recommend alternative approaches. - Identify potential needs for modifications and additional indications of constraints and decoding process requirements for layered coding. - Coordinate with AHG 9 on high-level syntax for constraint specifications and capability indications for layered coding. | G. Sullivan (chair) | Tel. |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-Q1007 SHVC Test Model 6 (SHM 6) text. - Produce and finalize JCTVC-Q1008 SHVC text specification Draft 6. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 6.0 software (based on HM 14) for experimentation. - Provide software to CEs within two weeks after the meeting. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, (co-chairs) | N |
| SHVC inter-layer resampling process (AHG13) (jct-vc@lists.rwth-aachen.de) - Develop methodology for evaluating the accuracy of resampling processing - Study generic solutions for phase adjustment in resampling process - Study re-sampling process modification related to extracting regions of interest. - Discuss and identify additional issues related to resampling filtering. | E. Alshina (chair), J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (vice-chairs) | Tel. |
| Colour gamut and bit depth scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut and bit depth scalable coding. - Study the interaction of colour gamut scalability with bit-depth and spatial scalability. - Identify test sequences and test conditions, in coordination with AHG16. - Study the complexity of various methods for colour gamut scalable coding - Discuss and identify additional issues related to colour gamut scalability. | P. Andrivon, A. Duenas, (co-chairs), C. Auyeung, E. Alshina, S. Deshpande, X. Li, K. Ugur, Y. Ye (vice-chairs) | N |
| SCC extensions test model text editing (AHG15) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions test model 1.0 text. - Gather and address comments for refinement of the test model. - Coordinate with AHG [SCC extensions software development] to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, Z. Ma, Y. Ye | N |
| Test sequence material (AHG16) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for HEVC development. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 on range extensions development, AHG8 on screen content coding, and AHG14 on colour gamut scalability, AHG18 on high bit rate and high bit depth operation. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |
| SCC extensions software development (AHG17) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM SCM software and its distribution. - Prepare and deliver HM 14.0-SCM-1.0 software version and the reference configuration encodings according to JCTVC-Q1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG15 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu (vice-chairs) | N |

# Output documents (to be updated)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-Q1000 Meeting Report of 17th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm] [2014-06-20] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-Q1002 High Efficiency Video Coding (HEVC) Test Model 15 (HM 15) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] [2014-06-20] (near next meeting)

JCTVC-Q1003 High Efficiency Video Coding (HEVC) Edition 1 Defect Report Draft 4 [Y.-K. Wang, G. J. Sullivan, B. Bross] [2014-05-30]

JCTVC-Q1004 HEVC Conformance Draft 7 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 14426 Preliminary draft of ISO/IEC FDIS) [2014-05-16]

JCTVC-Q1005 HEVC Range Extensions Draft Text 7 [D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. Sullivan, T. Suzuki] (WG 11 N 14414, ISO/IEC FDIS) [2014-05-30]

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman]

JCTVC-Q1007 SHVC Test Model 6 (SHM 6) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 14419) [2014-06-06]

JCTVC-Q1008 SHVC Draft Text 6 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang] (WG 11 N 14416 ISO/IEC Study of DAM3) [2014-06-06]

JCTVC-Q1009 Common SHM test conditions and software reference configurations [V. Seregin, Y. He] [2014-05-02] (1 week after software)

Update to include updated anchor results and config files.

Include colour gamut scalability tests.

Remains valid – not updated JCTVC-O1010 Guidelines for conformance test bitstream preparation [T. Suzuki, W. Wan]

JCTVC-Q1011 Report on HEVC compression performance verification test (WG 11 N 14420) [T. K. Tan, M. Mrak, V. Baroncini, N. Ramzan] [2014-04-25]

JCTVC-Q1012 Range extensions conformance draft 1 (WG 11 N 14427) [T. Suzuki, K. Kazui (Editors)] [2014-05-30]

JCTVC-Q1013 HEVC Range Extensions Test Model 7 Encoder Description [M. Naccari, C. Rosewarne, K. Sharman, G. Sullivan] (WG 11 N 14418) [2014-06-06]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting were not considered binding on final CE plans as reviewed in closing plenary.

JCTVC-Q1014 Screen content coding test model 1 (SCM 1) (WG 11 N 14421) [R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye] [2014-04-25]

JCTVC-Q1015 Common conditions for screen content coding tests [H. Yu, R. Cohen, K. Rapaka, J. Xu] [2014-05-02]

JCTVC-Q1101 HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability [A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li (CE coordinators)] [2014-04-25] (three weeks)

3 weeks for finalization of CE description. Some details of planned test to be worked out to provide a more precise defintion of the experiment.

To test normative and non-normative alternatives.

JCTVC-Q1101 was reviewed in non-final form in closing plenary. A three week finalization period was authorized for the plan document.

JCTVC-Q1121 HEVC Screen Content Core Experiment 1 (SCCE1): Intra block copy [J. Sole, S. Liu (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1121 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1122 HEVC Screen Content Core Experiment 2 (SCCE2): Line-based intra copy [C.-C. Chen, X. Xu, L. Zhang (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1122 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1123 HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode [Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1123 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1124 HEVC Screen Content Core Experiment 4 (SCCE4): String matching for sample coding [Y. Chen, J. Xu (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1124 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

JCTVC-Q1125 HEVC Screen Content Core Experiment 5 (SCCE5): Inter-component prediction and adaptive colour transform [X. Xiu, J. Chen (CE coordinators)] [2014-04-18] (two weeks)

JCTVC-Q1125 was reviewed in non-final form in closing plenary. A two week finalization period was authorized for the plan document.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 17–24 Oct. 2014 under WG 11 auspices in Strasbourg, FR.

- 10–18 Feb. 2015 under ITU-T auspices in Geneva, CH.

- 18–26 June 2015 under WG 11 auspices in Warsaw, PL.

- XX–XX Oct. 2015 under ITU-T auspices in XXXX, XX.

The agreed document deadline for the 19th JCT-VC meeting is XX Oct. Plans for scheduling of agenda items within that meeting TBA.

XXX were thanked for the excellent hosting of the 18th meeting of the JCT-VC.

The JCT-VC meeting was closed at approximately XXXX hours on Wed. 9 July 2014.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the eighteenth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting (approximately XXX people in total), were as follows: