#### Draft text specification for CE6 tests A5 and A6

#### Draft text specification for CE6 test A5

| Table 9-43 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |  |
| ... |  |  |  |  |  |  |  |
| palette_run_type_flag | 0, 1 (subclause 9.3.4.2.2) | na | Na | na | Na | Na |  |
| palette_index | bypass | bypass | bypass | bypass | bypass | bypass |  |
| palette_run | 0, 1, 2, 3 (subclause 9.3.4.2.8) | 4, 5 (subclause 9.3.4.2.8) | 6, 7 (subclause 9.3.4.2.8) | bypass | bypass | bypass |  |
| palette_all_zeros_in_group | bypass | na | na | na | na | na |  |
| ... |  |  |  |  |  |  |  |

9.3.4.2.8	Derivation process of ctxInc for the syntax element palette_run

Inputs to this process are binIdx, palette_index and palette_run_type_flag.

Output of this process is the variable ctxInc.

Table x: Specification of ctxInc for palette_run

| binIdx | palette_run_type_flag | palette_index | ctxInc |
| --- | --- | --- | --- |
| 0 | 1 |  | 0 |
|  | 0 | 0 | 1 |
|  |  | 1,2 | 2 |
|  |  | greater than 2 | 3 |
| 1 | 1 |  | 4 |
|  | 0 |  | 5 |
| 2 | 1 |  | 6 |
|  | 0 |  | 7 |

#### Draft text specification for CE6 test A6

#### 9.3.3.13	Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run, a colour component index cIdx, scanPos, and nCbS * nCbS.

Output of this process is the binarization of palette_run as specified in Table 9-46.

Table 46 – Binarization for palette_run

| palette_run | Codeword |
| --- | --- |
| 0 | '0' |
| >0 | Prefix = '1', Suffix = 0-th order TEGk code symbolVal = palette_run – 1, cTEGParam = nCbS * nCbS – 2 – scanPos |

#### 9.3.3.x k-th order Truncated Exp-Golomb (TEGk) binarization process

Inputs to this process is a request for an TEGk binarization of symbolVal and a parameter cTEGParam.

Output of this process is the TEGk binarization associating each value symbolVal with a corresponding bin string.

The bin string of the TEGk binarization process for each value symbolVal is specified as follows, where each call of the function put( X ), with X being equal to 0 or 1, adds the binary value X at the end of the bin string:

stopLoop = 0
do
	if(symbolVal >=  ( 1  <<  k ) ) {
		put( 1 )
		symbolVal = symbolVal − ( 1  <<  k ) 
		cTEGParam = cTEGParam − ( 1  <<  k ) 
		k++
	} else {
		gapVal = cTEGParam − ( 1  <<  k )
		if(gapVal >= 0) {		 (9-yy)
			put( 0 )
			while( k− − )
				put( (symbolVal >>  k)  &  1 )
			stopLoop = 1
		} else {
			9.3.3.6 TB binarization, synVal = symbolVal, cMax = gapVal
			stopLoop = 1
		}
	}
while( !stopLoop )

#### Draft text specification for combination of tests A5 and A6

The draft text specifications for tests A5 and A6 can be combined in a straightforward manner since they affect different parts of the specification.