WD changes for JCTVC-S0177 based on JCTVC-R1005-v3:

### Decoding process for intra blocks

#### General decoding process for intra blocks

…

–	Otherwise (splitFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( cIdx > 0  &&  ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The variable yTbOffset is set equal to blkIdx * nTbS.

- The variable yTbOffsetY is set equal to yTbOffset * SubHeightC.

- When controlParaACT is not equal to 2, the variable residualDpcm is derived as follows:

  - If all of the following conditions are true, residualDpcm is set equal to 1.

  - implicit_rdpcm_enabled_flag is equal to 1.

  - either transform_skip_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ] is equal to 1, or cu_transquant_bypass_flag is equal to 1.

  - either predModeIntra is equal to 10, or predModeIntra is equal to 26.

  - Otherwise, residualDpcm is set equal to explicit_rdpcm_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ].

- When controlParaACT is not equal to 1, depending upon the value of predModeIntraBc, the following applies:

–	When predModeIntraBc is equal to 0, the general intra sample prediction process as specified in subclause 8.4.4.2.1 is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1), the intra block copying process as specified in subclause 8.4.4.2.7 is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable trafoDepth, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples and a 1x(2*nTbs-1) array predSampleDiff.

- When controlParaACT is not equal to 2, the scaling and transformation process as specified in subclause 8.6.2 is invoked with the luma location ( xTbY, yTbY + yTbOffsetY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When controlParaACT is not equal to 2 and residualDpcm is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause 8.6.5 is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- When controlParaACT is not equal to 2 and cross_component_prediction_enabled_flag is equal to 1, ChromaArrayType is equal to 3, and cIdx is not equal to 0, the residual modification process for transform blocks using cross-component prediction as specified in subclause 8.6.6 is invoked with the current luma transform block location ( xTbY, yTbY ), the variable nTbS, the variable cIdx, the (nTbS)x(nTbS) array rY set equal to the corresponding luma residual sample array resSamples of the current transform block, the 1x(2*nTbs-1) array b set equal to array predSampleDiff, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- When controlParaACT is not equal to 1, the picture construction process prior to in-loop filtering for a colour component as specified in subclause 8.6.7 is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

…

8.4.5.2.5	Specification of intra prediction mode INTRA_DCInputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are

–	the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1,

–	the offset values predSampleDiff[m], with m = 0..2*nTbS – 2.

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1, are derived by the following ordered steps:

- A variable dcVal is derived as follows:

dcVal = 	(8-41)

where k = Log2( nTbS ).

- Depending on the value of the colour component index cIdx, the following applies:

- If cIdx is equal to 0 and nTbS is less than 32, the following applies:

predSamples[ 0 ][ 0 ] = ( p[ −1 ][ 0 ] + 2 * dcVal + p[ 0 ][ −1 ] + 2 )  >>  2	(8-42)

predSamples[ x ][ 0 ] = ( p[ x ][ −1 ] + 3 * dcVal + 2 )  >>  2, with x = 1..nTbS − 1	(8-43)

predSamples[ 0 ][ y ] = ( p[ −1 ][ y ] + 3 * dcVal + 2 )  >>  2, with y = 1..nTbS − 1	(8-44)

predSamples[ x ][ y ] = dcVal, with x, y = 1..nTbS − 1	(8-45)

- Otherwise, the prediction samples predSamples[ x ][ y ] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x, y = 0..nTbS − 1	(8-46)

- The offset values predSampleDiff[ m ] are derived as follows:

predSampleDiff[ m ] = predSamples[ x ][ y ] − dcVal, with x=0, y= 0..nTbS – 1 and x= 1..nTbS – 1, y=0 for m = nTbS – 1 – y + x.

        - Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are

–	the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1,

–	the offset values predSampleDiff[m], with m = 0..2*nTbS – 2.

…

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is greater than or equal to 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8-47)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-48)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8-49)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-50)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-51)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-52)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-53)

  - When If predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, then the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-54)

and the offset values predSampleDiff[ m ] are derived as follows:

predSampleDiff[  m ] = predSamples[ 0 ][ y ] − p[ x ][ −1 ] , with x=0, y= 0..nTbS – 1 for m = nTbS – 1 – y + x.

predSampleDiff[ m ] = 0 , with m = nTbS..2*nTbS – 2.

–	Otherwise the offset values predSampleDiff[ m ] are derived as follows:

predSampleDiff[ m ] = 0 , with m = 0..2*nTbS – 2.

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8-55)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-56)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8-57)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-58)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-59)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-60)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-61)

- When If predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32, and disableIntraBoundaryFilter is equal to 0, then the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-62)

and the offset values predSampleDiff[ m ] are derived as follows:

predSampleDiff[  m ] = predSamples[ x ][ 0 ] − p[ −1 ][ y ] , with x=0..nTbS – 1, y= 0 for m = nTbS – 1 – y + x.

predSampleDiff[ m ] = 0 , with m = 0..nTbS – 2.

–	Otherwise the offset values predSampleDiff[ m ] are derived as follows:

predSampleDiff[ m ] = 0 , with m = 0..2*nTbS – 2.

### 8.6.6  Residual modification process for transform blocks using cross-component prediction

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array of luma residual samples rY with elements rY[ x ][ y ],

–	a 1x(2*nTbS-1) array of prediction offset samples b with elements b[m].

–	a cu_residual_act_flag[ xTbY ][ yTbY ]

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

Output of this process is the modified (nTbS)x(nTbS) array r of residual samples.

The (nTbS)x(nTbS) array of residual samples r with  x = 10..nTbS − 1, y = 10..nTbS − 1  is modified as follows:

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-294)
			( ( rY[ x ][ y ]  <<  BitDepthC )  >>  BitDepthY ) )  >>  3

r with x = 0, y = 0..nTbS − 1 and  r with x = 1..nTbS − 1 , y = 0 is modified as follows:

if cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 0, then

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-299)
			(  ( ( rY[ x ][ y ] + b[ nTbS – 1 − y + x ])  <<  BitDepthC )  >>  BitDepthY ) )  >>  3

otherwise,

if cIdx is equal to 1, then

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-300)
			(  ( ( rY[ x ][ y ] + b[ nTbS – 1 − y + x ] >> 2)  <<  BitDepthC )  >>  BitDepthY ) )  >>  3

otherwise, if cIdx is equal to 2, then

r[ x ][ y ] += ( ResScaleVal[ cIdx ][ xTbY ][ yTbY ] *	(8-301)
			(  ( ( rY[ x ][ y ] + b[ nTbS – 1 − y + x ] >> 2)  <<  BitDepthC )  >>  BitDepthY ) )  >>  3 - b[ nTbS – 1 − y + x ] >> 2