| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S0178 |
| --- | --- |

| Title: | Non-CE6: Improved binarization and signaling of index coding for transition copy mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Feng Zou Rajan Joshi Vadim Seregin Wei Pu 5775 Morehouse Drive San Diego, CA, 92122 | Tel: Email: | +1-858-845-1115 martak@qti.qualcomm.com fzou@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents a CU level flag to indicate the usage of transition copy mode for the palette coding. In addition, two binarization methods depending on the palette size are proposed to code the palette index. The simulation results report that the proposed method achieves 0.2% and 0.3% BR-rate savings for 720p text and graphics RGB and YUV respectively against the CE6 Test C.2 in the full frame intra BC test condition.

# Introduction

In the 18th JCT-VC meeting in Sapporo, CE6 [1] was established to study different palette representation methods. Test C.2 in CE6 proposed the transition copy mode for palette coding, in which a separate palette mode is introduced. During palette index coding, if the current pixel does not use transition mode, index adjustment is made to reduce the dynamic range of the available candidate palette indices. And truncated binary codeword is used to code the palette index candidates.

# Background on palette index coding in SCM2.0

In SCM2.0 reference software, the pixel wise palette index is coded using truncated binary codeword. And the maximum range for the truncated binary codeword depends on the neighboring pixel modes. The following is an example from the encoder point of view. Specifically, if the current pixel is not the first pixel within the current block in scanning order and the last coded pixel in scanning order is a non-escape pixel, the following procedures are applied to code the current pixel palette index T.

- The maxSymbol is set equal to the current block palette size if there is no escape pixel in the current block or the current block palette size plus 1 if the current block may have escape pixel

- If the last coded neighboring pixel in scanning order is “index” mode, the current pixel cannot be equal to the last coded neighboring pixel’s index. Therefore, a variable Ref is set equal to the index value of the last coded neighboring pixel.

  - maxSymbol is set equal to (maxSymbol-1)

  - If the current index is larger than Ref, the current index T is set equal to (T-1)

- Otherwise, if the current pixel is at the non-first row, and the last coded neighboring pixel in scanning order is “copy above” mode and the pixel above the current pixel is non-escape, the current pixel index cannot be equal to index of the above pixel. Therefore, Ref is set equal to the index value of the above pixel.

  - maxSymbol is set equal to (maxSymbol-1)

  - If the current index is larger than Ref, the current index T is set equal to (T-1)

# Proposed CU level use_trans_flag

It is proposed to adaptively enable the usage of transition mode using a flag use_trans_flag at the Coding Unit (CU) level. For instance, if use_trans_flag is equal to 1, the current block uses the transition copy mode. In this case, the transition signalling is enabled for the current block. Otherwise, (use_trans_flag is equal to 0), the traditional signaling (as done in SCM2.0) is applied. In summary, introducing this CU level flag can reduce the overhead cost associated with signaling the transition modes for those blocks that do not use transition mode. At the encoder, a rate estimation is used to determine whether the current block uses transition mode or not.

# Proposed binarization methods for index coding

A novel binarization method is proposed to code the palette index depending on the adjusted index range maxSymbol. Specifically, if adjusted maxSymbol is smaller than a predefined threshold Thr, truncated binary code is used to code the palette index where the first symbol in the truncated binary codeword represents the transition prediction index. Otherwise, the combination of a flag (indicating whether the symbol to be coded equals to the transition prediction index) and truncated binary code (if the symbol to be coded does not equal to the transition prediction index) are used to signal the current palette index. Note that transition prediction index is denoted as MPM in this proposal for easy explanation.

The following is an encoder example as shown in Figure 1, where Ref (see definition in Section 2) is equal to 4, MPM is equal to 2, and maxSymbol is equal to 7. After the index adjustment as mentioned in Section 2.1.5, the index dynamic range is [0, 5] with maxSymbolMinus1 being equal to 6. And then the following procedures are applied.

Figure 1. Index adjustment given Ref value

- If the adjusted maxSymbolMinus1 is smaller than a predefine threshold Thr =4 as an example, an MPM shift adjustment is proposed such that the first symbol 0 represents MPM index while the original indices smaller than MPM are shifted by +1 as shown in Figure 2. This novel binarization is used to code the current palette index given maxSymbolMinus1 being equal to 6.

Figure 2. MPM shift operation

- Otherwise, the adjusted maxSymbolMinus1 is equal or larger than the predefine threshold Thr =4, the combination of an index prediction flag MPM_pred_flag and truncated binary code is used to signal the current palette index. Specifically, the following procedure is applied:

  - If the current index is equal to MPM, then MPM_pred_flag is set equal to 1;

  - Otherwise, the current index is not equal to MPM, similar procedure as done in Figure 2 is applied to shift MPM to first place. And then MPM is removed from the candidate list, and the other indices are adjusted by minus 1. maxSymbolMinus2 is set equal to maxSymbolMinus2 - 1, as shown in Figure 3.

Figure 3. MPM removal based on Figure 2.

Note that the proposed binarization method disables the palette mode signaling (Spoint), but rather signals the usage of transition mode in an index prediction manner.

# Simulation results

The proposed scheme is implemented on SCC common software and tested using the common test condition defined in [2]. Table 1 demonstrates the coding performance under full frame Intra BC test conditions with the proposed binarization method for index coding.

Table 1: RD performance against SCM2.0 anchor

|  |  | CE6 Test C.2 |  | Proposed index coding |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  | All Intra |  |  |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.2% | -1.2% | -1.1% | -1.2% | -1.2% |
| RGB, text & graphics with motion,720p | -0.8% | -0.8% | -0.8% | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.2% | -1.4% | -1.4% | -1.2% | -1.5% | -1.5% |
| YUV, text & graphics with motion,720p | -0.4% | -0.8% | -1.0% | -0.7% | -0.9% | -1.0% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.4% | -0.2% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusions

This proposal presents a CU level flag to indicate the usage of transition copy mode for the palette coding and two binarization methods to code the palette index. The simulation results report that the proposed method achieves 0.2% and 0.3% BR-rate savings for 720p text and graphics RGB and YUV respectively against the CE6 Test C.2 in the full frame intra BC test condition.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]. Y. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu and Z. Ma, “Description of Core Experiment 6 (CE6): Improvements of palette mode”, JCTVC-R1106.

[2]. H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-R1015, Sapporo, Japan, June 2014.