| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 23rd Meeting: San Diego, US, 19–26 Feb. 2016 | Document: JCTVC-W_Notes_d3 |
| --- | --- |

| Title: | Meeting report of the 23rd meeting of the Joint Collaborative Team on Video Coding (JCT-VC), San Diego, US, 19–26 Feb. 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-third meeting during 19–26 Feb 2016 at the San Diego Marriott La Jolla in San Diego, US. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 February 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 February 2016. Approximately XX people attended the JCT-VC meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-second JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 4;

- For the format range extensions (RExt), the RExt reference software draft 4;

- For the scalable extensions (SHVC), the SHVC reference software draft 3, conformance testing draft 4, SHVC test model 11 (SHM 11), and verification test plan for SHVC;

- For the HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 6 (SCM 6) and SCC draft text 5.

The other most important goals were to review the results from eight Core Experiments in High Dynamic Range (HDR) Video Coding (a work that had been allocated to JCT-VC by the parent bodies from the beginning of the 23rd meeting), and review other technical input documents. Finalizing the specification of screen content coding tools (integrated in the preparation for version 4) and making progress towards HDR support in HEVC were the most important topics of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Evaluating results of SHVC verification tests was conducted, and possible needs for corrections to version 3 (to be integrated in version 4) were also considered.

In addition to X new experiment plan description in high dynamic range video coding, the JCT-VC produced XX other particularly important output documents from the meeting (update):

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 4;

- For the format range extensions (RExt), the RExt reference software draft 4;

- For the scalable extensions (SHVC), the SHVC reference software draft 3, conformance testing draft 4, SHVC test model 11 (SHM 11), and verification test plan for SHVC;

- For the HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 6 (SCM 6) and SCC draft text 5.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for Thu. 26 May – Wed. 1 June 2016 under ITU-T auspices in Geneva, CH, Fri. 14 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN, XXXX and XXXX.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-third meeting during 19–26 Feb 2016 at the San Diego Marriott La Jolla in San Diego, US. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 February 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 February 2016. Approximately XX people attended the JCT-VC meeting, and approximately XX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

- 22nd "V" meeting (Geneva, 2015-10)		XX people, XX input documents

- 23rd "W" meeting (San Diego, 2016-02)		XX people, XX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2016_02_W_SanDiego/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-second JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 4;

- For the format range extensions (RExt), the RExt reference software draft 4;

- For the scalable extensions (SHVC), the SHVC reference software draft 3, conformance testing draft 4, SHVC test model 11 (SHM 11), and verification test plan for SHVC;

- For the HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 6 (SCM 6) and SCC draft text 5.

The other most important goals were to review the results from eight Core Experiments in High Dynamic Range (HDR) Video Coding (a work that had been allocated to JCT-VC by the parent bodies from the beginning of the 23rd meeting), and review other technical input documents. Finalizing the specification of screen content coding tools (integrated in the preparation for version 4) and making progress towards HDR support in HEVC were the most important topics of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Evaluating results of SHVC verification tests was conducted, and possible needs for corrections to version 3 (to be integrated in version 4) were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 9 Feb 2016.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 10 Feb 2016 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-W0108 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "W0108+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- (This case did not occur at this meeting)

The following technical design proposal contributions were both registered late and uploaded late:

- JCTVC-W0XXX (a proposal document from XXX relating to XXX) [uploaded 02-XX]

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-W0XXX (a proposal on XXX) [uploaded 02-XX]

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-W0XXX [uploaded 02-XX], …

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-W0036, JCTVC-W0065, JCTVC-W0082.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents (both crosscheck reports) were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline: JCTVC-W0064, JCTVC-W0070.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-V1000, the improved HEVC Test Model 16 (HM16) JCTVC-V1002, the RExt Reference Software Draft 4 JCTVC-V1011, the SHVC test model 11 (SHM11) JCTVC-V1007, the SHVC Conformance Testing Draft 4 JCTVC-V1008, the SHVC Reference Software Draft 3 JCTVC-V1013, the SHVC Verification Test Plan JCTVC-v1004, the Screen Content Coding (SCC) Draft Text 5 JCTVC-V1005 (integrated into draft of HEVC version 4), and the SCC test model 6 JCTVC-V1014, were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE).

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment.

- SCCE: Screen content core experiment.

- SCM: Screen coding model.

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a term for a draft standard, especially one prior to its first ballot in the ISO/IEC approval process, although the term is sometimes used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting. However, the rapporteur of Q6/16 (VCEG) reported that ITU-R SG6 had replied to the query sent to them about the chroma positioning in 4:2:0 sampling for ITU-R BT.2020. The reply confirmed that the co-sited positioning (both horizontally and vertically) was intentional.

[Note additional LS.]

## Opening remarks

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

Primary topic areas were noted as follows:

- Screen content coding

  - Input from editors (incl. forgotten/open technical aspects such as combined wavefronts & tiles)

  - Input on WP with current picture referencing

  - Input on DPB handling with CPR

  - Non-SCC aspects (VUI/SEI, scalable RExt profiles, high-throughput profiles, errata)

- HDR: New work allocated to JCT-VC effective at this meeting

  - There has been prior work on parent bodies (e.g., eight CEs of MPEG, AHGs of MPEG & VCEG, and a January meeting of MPEG AHG)

  - A significant development is the report of new non-normative techniques for improved "anchor" encoding with ordinary HEVC (see esp. CE1 of MPEG)

  - That can be part of the development of the planned "best practices" / "good practices" document development, which is a planned deliverable (see CE1 of MPEG and AHG report of VCEG)

  - The top priority is to confirm that enhanced signal processing extension can provide an adequate improvement over such an anchor to justify a new extension

    - How to measure the benefit achieved?

    - Should organize some viewing during this meeting and discuss metrics.

    - Non-normative vs. normative for decoding post-processing also to be considered

  - There is new relevant liaison input (e.g. from ITU-R WP6C and SMPTE).

  - Backward compatibility is an additional issue to consider.

    - Multiple dimensions to the issue

    - Highest priority is best HDR quality (regardless of BC)

    - There have been liaison inputs requesting backward compatibility

    - Some ways to achieve BC that are already supported (or soon will be):

      - Simply coding HDR video (e.g. with HLG or PQ) and letting an SDR decoder receive it and display it as SDR

      - Scalable coding

      - Perhaps using some existing SEI messages (tone mapping or CRI)

    - Brings the need to consider relative importance of the SDR and HDR and which flavours of SDR and HDR can be supported

    - Two flavors of BC have been proposed:

      - Decoders that do not use new extension data, for "bitstream backward compatibility"

      - Using new extension data for "display backward compatibility"

    - Potential for needing multiple profiles / multiple extensions depending on the use case

    - Need to properly define requirement aspects and priorities

      - Prior requirements expressions: MPEG Feb 2015 output, VCEG docs (add specific references - check C.853 and the Feb 2015 report for VCEG)

      - There are approaches that have been studied in CEs to address improvement over existing BC solutions

      - Requirement issues need parent-level discussion, clarification, and joint agreement for BC issues, before spending effort on that in JCT-VC

- Corrigenda items for version 3 (see, e.g., the AHGx and AHGy reports and SCC editor input)

- Verification testing for SHVC

- Reference software and conformance, RExt & SHVC

  - RExt reference software was done at the last meeting, and conformance is scheduled for FDAM at this meeting

  - SHVC conformance is scheduled for FDAM (and also for MV-HEVC and 3D-HEVC from JCT-3V, but does not seem editorially adequate (a couple of bugs also identified)

- Test model texts and software manuals

- Common test conditions for coding efficiency experiments (hasn't been under active development here recently; being considered more elsewhere)

Status of deliverables (all delivered)

Key deliverables from this meeting [update]

- FDAM for RExt conformance (draft 7)

- Verification test report for SHVC

- SCC specification Draft 6 (FDIS)

- SHVC software Draft 4 (FDAM)

- SHVC conformance Draft 5 (FDAM?? - likely to postpone one meeting cycle)

- SCC Reference software (??)

- SCC Conformance (??)

- SCC verification testing

- HDR outputs

  - Suggested practices draft

  - CEs

  - Test model

- New HM(?), SHM(?), SCM

Two main tracks (?) were followed for most meeting discussions: [not really]

- Track A (GJS): …, etc.

- Track B (JRO): …, etc.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on Screen Content Coding CE work and related contributions during the first few days. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 19 Feb., 1st day

  - 0900–1400, 1530–2100 [add detail]

- Sat. 20 Feb., 2nd day

  - Did not meet except BoG in morning.

  - 1400–2000 [add detail]

- Sun. 21 Feb., 3rd day

  - 0900–xx [add detail]

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks to be clarified if necessary)

- AHG reports (12+2 HDR) Track P (section 2)

- Documents of HDR AHG in Vancouver (6) (for information) (section 3)

- Project development status (6) Track P (section 4)

- HDR CE1: Optimization without HEVC specification change (1) Track X (section 5.1)

- HDR CE2: 4:2:0 YCbCr NCL fixed point (13) Track X (section 5.2)

- HDR CE3: Objective/subjective metrics (3) Track X (section 5.3)

- HDR CE4: Consumer monitor testing (1) Track X (section 5.4)

- HDR CE5: Colour transforms and sampling filters (6) Track X (section 5.5)

- HDR CE6: Non-normative post processing (10) Track X (section 5.6)

- HDR CE7: Hybrid Log Gamma investigation (4) Track X (section 5.7)

- HDR CE8: Viewable SDR testing (1) Track X (section 5.8)

- SCC coding tools (5) Track X (section 6.1)

- HDR coding (34) Track X (section )

  - CE1 related (11)

  - CE2 related (7)

  - CE3 related (1)

  - CE6 related (1)

  - CE7 related (6)

  - Other (8)

- High-level syntax (0) Track X (section 6.2)

- VUI and SEI messages (5) Track X (section 6.3)

- Non-normative encoder optimization (9) Track X (section 6.4)

- Withdrawn (5) (section 7)

- Plenary discussions and BoG reports (xx) Track P (section 8)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 9, 10, and 11)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports (12+2)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Friday 19th, 1145–14:00 and 1530–1800, except as noted.)

- JCTVC-W0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

[Add notes]

- JCTVC-W0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne, M. Naccari, J.-R. Ohm, K. Sharman, G. Sullivan, Y.-K. Wang]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 22nd meeting in Geneva, CH (October 2015) and the 23rd meeting in San Diego, USA (February 2016).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

WD tickets in tracker for text specification

- #1439 (IRAP constraint for Main 4:4:4 Intra profile)

- #1440 (Wavefront+tiles) are noted. Also noted are WD tickets #1435-1437 (editorial in nature).

Editor action item: These should be addressed in the SCC output.

The ‘High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 4 of Encoder Description’ was published as JCTVC-V1002. This document represented a refinement of the previous HM16 Update 2 of the Encoder Description document (JCTVC-U1002). The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Review the list of bug fixes collected for HEVC Edition 2, and include all confirmed bug fixes, including the outcome of the above items, if any, into a JCT-VC output document for the purpose of HEVC Edition 2 defect reporting.

JCTVC-W0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring, K. Sharman]

This report summarises the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 22nd and 23rd JCT-VC meetings. Activities focused on integration of software adoptions and software maintenance, i.e. code tidying and fixing bugs.

The one proposal that was adopted at the last meeting regarding modifications to rate control (JCTVC-V0078), and one proposal that was adopted at the 21st meeting on the Alternative Transfer Characteristics SEI (JCTVC-U0033) were added to the HM development code base.

In addition, some minor bug fixes and cleanups were addressed. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector, and http://hevc.info has been updated.

Version 16.8 is due to be released during the meeting.

There are a number of reported software bugs that should be fixed.

HM16.8 is due to be released during the meeting. It includes:

- JCTVC-V0078: modification of the rate control lower bounds. Rate control is not used in common test conditions.

- JCTVC-U0033: add support for Alternative Transfer Characteristics SEI message (with syntax element naming taken from the current SCC draft text, V1005).

- Bug reports were closed, although only one of these related to a bug within the code (relating to the Mastering Display Colour Volume SEI).

None of the changes affect the coding results of the common test conditions set out in JCTVC-L1100 or JCTVC-P1006, and therefore these have not been re-published.

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA (Tickets #1096, #1101, #1333, #1334, #1346)

- Rate-control and QP selection – numerous problems with multiple slices (Tickets #1314, #1338, #1339)

- Field-coding (Tickets #1145, #1153)

- Decoder picture buffer (Tickets #1277, #1286, #1287, #1304)

- NoOutputOfPriorPicture processing (Tickets #1335, #1336, #1393)

- Additional decoder checks (Tickets #1367, #1383)

However, a patch has been generated that adds some conformance checks. It is being considered for potential inclusion in a future release.

As described to the community at the last three JCT-VC meetings, alterations to remove the unused software hierarchy in the entropy coding sections of the code, and to remove terms such as CAVLC is being considered. However, this will now need to also consider the impact on the JEM branch.

The possibility of merging branches has been considered, in particular merging of SHM (Scalable coding HEVC Model) into HM, although further discussions are required. Currently, within the JCT-VC and JCT-3V, there are the HM, the SCM (for SCC), the SHM (for scalable) and the HTM (3D and multi-view) software models. There is also the JVET’s JEM branch for exploration of future technologies. The biggest problem to merging these branches is the clash in the overlapping functionality in SHM and HTM – for example, the handling of multiple levels is not consistent between the two. By careful consideration of these two codebases, it would be possible to incrementally modify HM so that the SHM and HTM branches gradually aligned, although it is also very important to minimise the impact on the core HM code base as this also impacts the other branches. The work appears to be significant and would require close coordination between the software coordinators.

Recommendations:

- Continue to develop reference software based on HM version 16.7/8 and improve its quality.

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Continue to investigate the merging of branches with the other software coordinators.

In discussion, it was suggested that harmonization of HM with SCM is probably the highest priority, since SCC features should be included (or at least testable) in combination with future video work.

JCTVC-W0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, J. Boyce, R. Joshi, K. Kazui, A. Ramasubramonian, Y. Ye]

Review deferred due to AHG chair late arrival.

JCTVC-W0005 JCT-VC AHG report: SHVC verification testing (AHG5) [V. Baroncini, Y.-K. Wang, Y. Ye (co-chairs)]

The main activities of this AHG included the following:

- Finalized all rate points, encoded all test sequences using SHM10.0 reference software, and delivered all bitstreams, sequences, and MD5SUM values for subjective evaluations.

- Conducted verification test following the finalized SHVC verification test plan.

- Produced a preliminary report of the test for review and finalization in JCTVC-W0095.

The AHG recommended to review the preliminary verification test report JCTVC-W0095.

The test coordinator (V. Baroncini) remarked that the refinements of the selection of test content proved beneficial, so the test was able to be effective and provide a good discrimination covering a wide range of coded video quality.

JCTVC-W0006 JCT-VC AHG report: SCC coding performance analysis (AHG6) [H. Yu, R. Cohen, A. Duenas, K. Rapaka, J. Xu, X. Xu (AHG chairs)]

This report summarizes the activities of the JCT-VC ad hoc group on SCC coding performance analysis (AHG6) between the JCT-VC 22nd meeting in Geneva, Switzerland, and the 23rd meeting in San Diego, USA.

A kick-off message for AHG 6 was sent out on Dec. 1, 2015.

As decided in the last meeting, the test conditions described in JCTVC-U1015 remained valid during this meeting cycle. To facilitate new simulations that would be done with the new reference software SCM-6.0, the new test-results reporting templates with SCM-6.0 anchor data were uploaded in JCTVC-U1015-v4 on Dec. 2, 2015. These templates were also distributed via the AHG6 kick-off email.

One relevant contribution was noted: JCTVC-W0104.

It is recommended to continue to evaluate the coding performance of the draft SCC coding features in comparison with the existing HEVC tools in the Main profile and range extensions.

JCTVC-W0007 JCT-VC AHG report: SCC extensions text editing (AHG7) [R. Joshi, J. Xu (AHG co-chairs), Y. Ye, S. Liu, G. Sullivan, R. Cohen (AHG vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on SCC extensions text editing (AHG7) between the 22nd JCT-VC meeting in Geneva, Switzerland (October 2015).and the 23nd JCT-VC meeting in San Diego, USA (February 2016).

The fifth specification text draft (JCTVC-V1005) for the High Efficiency Video Coding Screen Content Coding (HEVC SCC) extensions was produced by the editing ad hoc group as an output document following the decisions taken at the 22nd JCT-VC meeting in Geneva, Switzerland (October 2015).

The text of JCTVC-V1005 was submitted to ISO/IEC JTC1/SC29 as an output document WG11 N15776 Study Text of ISO/IEC DIS 23008-2:201X 3rd Edition

The following is a list of changes with respect to JCTVC-U1005-v1:

- Added constraints on the range of palette escape samples to disallow nonsense values (JCTVC-V0041)

- Allowed zero size for palette initialization in PPS, so palette initialization can be explicitly disabled at the PPS level (JCTVC-V0042)

- Integrated restriction for maximum palette predictor size (JCTVC-V0043)

- Modified the formula for computing PaletteMaxRun including consideration of copy_above_indices_for_final_run_flag (JCTVC-V0065)

- Removed special treatments of IBC as different from inter (w.r.t. usage for predicting intra regions and TMVP disabling) (JCTVC-V0066)

- Moved the SPS level IBC mode check aspect to the PPS level; MV & referencing picture based memory bandwidth constraint rather than AMVR based; if the decoder detects the prohibited case, the decoding process will convert so that list 0 uniprediction is used (and the converted motion data is stored) (JCTVC-V0048)

- When the merge candidate references the current picture, round it to an integer value (JCTVC-V0049)

- When conversion to uniprediction is performed due to 8x8 biprediction, only do that conversion if TwoVersionsOfCurrDecPicFlag is equal to 1 (JCTVC-V0056)

- About sps_max_num_reorder_pics and sps_max_dec_pic_buffering_minus1 when IBC is used (JCTVC-V0050)

- About sps_max_dec_pic_buffering_minus1 when IBC is used (JCTVC-V0057)

- Editorial improvements in palette run coding (JCTVC-V0060)

- Decoding process for intra_boundary_filtering_disabled_flag (previously missing)

- (HEVC_ERRATA): Added a constraint that the reference layer active SPS (specified by sps_scaling_list_ref_layer_id or pps_scaling_list_ref_layer_id) shall have scaling_list_enabled_flag = 1 (JCTVC-V0011)

- Generalized Constant and Non-Constant Luminance Code Points (JCTVC-V0035)

- Clarification of colour description semantics (especially for transfer_characteristics) (JCTVC-V0036)

- New High Throughput Profiles for HEVC (JCTVC-V0039)

- HEVC corrigendum: On parsing of bitstream partition nesting SEI message (JCTVC-V0062)

- Fixes to colour remapping information SEI message (JCTVC-V0064)

- New HEVC scalable format range extension profiles (JCTVC-V0098)

- (HEVC_ERRATA): Replaced undefined variable "Log2MaxTrafoSize" with "MaxTbLog2SizeY" (Ticket #1363)

The screen content coding test model 6 (SCM 6) (document JCTVC-V1014) was released on 10th February 2016. Its main changes were modification of the restriction on the use of 8×8 bi-prediction with IBC, conversion of 8×8 bi-prediction to uni-prediction when the restriction is in effect, removing special treatments of IBC as different from inter when using constrained intra prediction, rounding merge candidates to integers when they reference the current picture, and allowing zero size for palette predictor initialization in the PPS.

JCTVC-V0096 proposes editorial improvements to address the feedback and comments related to the SCC draft text 5. It also summarizes known open issues.

The recommendations of the HEVC SCC extension draft text AHG were to:

- Approve the documents JCTVC-V1005 and JCTVC-V1014 as JCT-VC outputs (agreed, although it was noted that some issues remained, such as wavefront-tile combination)

- Address the comments and feedback on SCC extensions text specification as appropriate

- Compare the HEVC SCC extensions document with the HEVC SCC extensions software and resolve any discrepancies that may exist, in collaboration with the SCC extension software development (AHG8)

JCTVC-W0008 JCT-VC AHG report: SCC extensions software development (AHG8) [K. Rapaka, B. Li (AHG co-chairs), R. Cohen, T.-D. Chuang, X. Xiu, M. Xu (AHG vice-chairs)]

This report summarizes the activities of Ad Hoc Group 8 on screen content extensions software (SCM) development that have taken place between the JCT-VC 22nd meeting in Geneva, Switzerland, and the 23rd meeting in San Diego, USA.

Multiple versions of the HM SCM software were produced and announced on the JCT-VC email reflector. The integration details and performance summary of these revisions are provided in the next subsections. The performance results of software revisions were observed to be consistent with the adopted techniques.

HM-16.7_SCM-6.0 was announced on the email reflector on November 23rd 2015. The software was tagged as https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.7+SCM-6.0/ .

HM-16.7_SCM-6.0 incorporates following adoptions/bug fixes:

- JCTVC-V0034: Palette encoder improvement for 4:2:0.

- JCTVC-V0040: Enable by default the method in U0095 on fast encoder for ACT and intra.

- JCTVC-V0041: Constrain the range of escape values.

- JCTVC-V0042: Allow zero size palette in PPS.

- JCTVC-V0043: Restriction for maximum palette predictor size.

- JCTVC-V0048: Relax 8x8 bi-bred restriction based on mv's and temporal referencing.

- JCTVC-V0049: Round merge MVs when ref picture is curr pic.

- JCTVC-V0056: Relax 8x8 bi-bred restriction based on the value of TwoVersionsOfCurrDecPicFlag.

- JCTVC-V0065: Modified formula for computing PaletteMaxRun including consideration of copy_above_indices_for_final_run_flag.

- JCTVC-V0066: Remove special treatments of IBC as different from inter in the case of CIP.

- Bug fix for delta QP for palette mode.

JCTVC-V0057/ JCTVC-U0181 (storage of unfiltered decoded picture in DPB) are disabled for now in the SCM and are planned to be enabled in the future releases after software issues involved are fixed. (These do not impact CTC).

The performance HM-16.7_SCM-6.0 compared to HM-16.6_SCM-5.2 was described according to the common test conditions in JCTVC-U1015. For the lossy 444 configuration, it is reported that this version provides BD-rate reduction of 0.0%, 0.2% and 0.3% for RGB 1080p & 720p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 0.0%, 0.1% and 0.2% for YUV 1080p & 720p text and graphics category in AI/RA/LB configuration, respectively.

For the lossy 4:2:0 configuration, it is reported that this version provides BD-rate reduction of 3.2%, 2.5% and 1.6% for YUV 1080p & 720p text and graphics category in AI/RA/LB configurations, respectively.

The improved 4:2:0 performance was suggested to be due to non-normative palette mode encoder improvement specifically for 4:2:0 (from document JCTVC-V0034, with better handling of the fact that some chroma is coded but discarded in the 4:2:0 case).

HM-16.6_SCM-5.3, HM-16.6_SCM-5.4 were tagged on HHI Server on November 02nd, 2015 and can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/

The changes over HM-16.6_SCM5.2 are

- Following tickets were fixed: #1411, #1417, #1418, #1419, #1420, #1421, #1422.

- Macro Removals related to SCM 5.2

- Merge to HM 16.7

- Misc. Cleanups/ fixes for memory leaks

It was reported that there was no noticeable change in performance under common test configuration due to above integrations.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-7.0, version SCC-6.0 (HM16.7).

Following tickets were closed during the meeting cycle: #1411, #1417, #1418, #1419, #1420, #1421, and #1422. . Currently there are no open tickets.

Recommendations

- Continue to develop reference software based on HM16.7_SCM6.0 and improve its quality.

- Remove macros introduced in previous versions before starting integration towards SCM-6.x/SCM-7.0 such as to make the software more readable.

- Continue merging with later HM versions.

JCTVC-W0009 JCT-VC AHG report: Complexity of SCC extensions (AHG9) [A. Duenas, M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, W. Xiu (co-chairs)]

This report summarizes the activities of the JCT-VC ad hoc group on Complexity of SCC extensions (AHG9)) between the JCT-VC 22nd meeting in Geneva, Switzerland, and the 23rd meeting in San Diego, USA.

No coordinated AhG activity took place on the JCT-VC reflector between the 22nd JCT-VC meeting in Geneva, Switzerland (October 2015). and the 23rd JCT-VC meeting in San Diego, USA (February 2016).

Related contributions:

- Encoder-only contributions

  - JCTVC-W0042: SCC encoder improvement.

  - JCTVC-W0075: Palette encoder improvements for the 4:2:0 chroma format and lossless.

- Aspects that affect normative requirements

  - JCTVC-W0076: Comments on alignment of SCC text with multi-view and scalable

  - JCTVC-W0077: Bug fix for DPB operations when current picture is a reference picture.

  - JCTVC-W0129: SCC Level Limits Based on Chroma Format.

The AhG recommended to review the contributions related to this topic.

JCTVC-W0010 JCT-VC AHG report: Test sequence material (AHG10) [T. Suzuki, V. Baroncini, R. Cohen, T. K. Tan, S. Wenger, H. Yu]

Review deferred due to AHG chair late arrival.

JCTVC-W0011 JCT-VC AHG report: SHVC test model editing (AHG11) [G. Barroux, J. Boyce, J. Chen, M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye]

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 22th JCT-VC meeting in Geneva, Switzerland (15–21 October 2015) and the 23th JCT-VC meeting in San Diego, USA (19–26 February 2016).

During this period, the editorial team worked on the Scalable HEVC (SHVC) Test Model text to provide the corresponding text for the newly added profiles. Six related profiles are currently defined in the draft SHVC specification:

- Scalable Main (previously specified)

- Scalable Main10 (previously specified)

- Scalable Monochrome (newly added, planned to be final at this meeting)

- Scalable Monochrome12 (newly added, planned to be final at this meeting)

- Scalable Monochrome16 (newly added, planned to be final at this meeting)

- Scalable Main 4:4:4 (newly added, planned to be final at this meeting)

JCTVC-W0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, H. Yong, G. Barroux]

This report summarizes activities of the AHG12 on SHVC software development between 22th and 23th JCT-VC meetings.

The latest software version is SHM-11.0.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

The latest version is SHM-11.0 and it was released with JCTVC-V1013 for DAM.

SHM-11.0 is based on HM-16.7 and includes newly adopted profiles for scalable monochrome and 4:4:4 chroma colour format coding. The following software improvements have been additionally made:

- Fixes for tickets ## 90, 93, 94, 96, 97

- Reduce amount of software changes relative to HM for potential software merging

After adoption of the new profiles, SHM software supports higher than 10 bitdepth input. For such high bitdepth, macro RExt__HIGH_BIT_DEPTH_SUPPORT should be enabled for better performance.

Anchor data and templates have been generated based on common test conditions JCTVC-Q1009 and attached to this report.

Development plan and recommendations

- Continue to develop reference software based on SHM-11.0 and improve its quality.

- Fix open tickets.

JCTVC-W0013 VCEG Ad hoc group report on HDR Video Coding (VCEG AHG5) [J. Boyce, E. Alshina, J. Samuelsson (AHG chairs)]

This document reports on the work of the VCEG ad hoc group on HDR Video Coding. The first version of the document contained the mandates, a short introduction and the notes from the teleconference meeting. In the second version of the document, text was added related to email discussions, contributions and recommendations.

During 2015 there were several input contributions to VCEG and JCT-VC reporting on, discussing and encouraging the experts to investigate coding of HDR video in a configuration that is sometimes called HDR10 (meaning that the representation format is 10 bit, 4:2:0, non-constant luminance, Y’CbCr, SMPTE ST 2084 EOTF, ITU-R BT.2020 colour primaries and, when compressed, is using the Main10 Profile of HEVC). Examples of such contributions are VCEG-C853, JCTVC-U0045, JCTVC-V0052, JCTVC-V0053 and VCEG-C1031. In the discussions related to VCEG-C1031 it was agreed that VCEG would start working towards documenting best practice techniques for using the existing HEVC standard for compression of HDR video.

A kick-off message was sent on December 9, 2015, listing the mandates of ad hoc group 5 and encouraging emails related to the ad hoc group to include both the VCEG reflector, vceg-experts@yahoogroups.com, and the JCT-VC reflector, jct-vc@lists.rwth-aachen.de.

In relation to software development, it was reported on December 9, 2015 that several improvements and extensions to HDRTools had been included in the gitlab repository.

A suggestion for a starting point for documenting best practice techniques was circulated on December 10, 2015. The discussion on the email reflectors where primarily related to mandate 4. In total approximately 50 emails were exchanged.

A teleconference was held on February 8, 22.00-23.59 CET with attendance of approximately 30 experts.

The purpose of the teleconference was to continue the work the request to “Study best practices techniques for pre-processing, encoding, post-processing, and metadata for handling HDR video content using the existing HEVC standard, and collect such information toward documenting such practices in a technical paper or supplement” and to try to resolve some of the open issues that had been discussed via email, in preparation for the upcoming JCT-VC meeting.

There was a question if also other operating points besides HDR10 should be included in this work. In JCT-VC review, it was agreed to no necessarily limit the scope in that manner.

The agreement that VCEG’s and MPEG’s work on HDR should be performed jointly in JCT-VC was noted.

Approximately 80 HDR related input contributions had been submitted to JCT-VC (including CE reports and crosschecks).

Summary of issues discussed in the teleconference (see the report for details about the discussion in the teleconference about each of these):

- Should we aim for one or multiple documents? The call agreed to start focusing on one document but with the different parts as separated as possible.

- Is the scope clear and is the concept of a "generalized hypothetical reference viewing environment" useful? The call agreed to use the generalized hypothetical reference viewing environment for the time being and to further investigate the effect of the reference viewing environment.

  - In JCT-VC review, it was commented that we need better clarity of what such an environment is, especially if we recommend certain practices that target the viewing experience in that environment, and should make sure that a benefit is provided in that environment with those practices.

  - In JCT-VC review, it was noted that the new ITU-R BT.[HDR] document describes a reference viewing environment for critical viewing

- Should SEI messages be included? (Which ones? Only signalling, or also how to apply them?) The call agreed to put SEI messages (the mastering display colour volume SEI message and the content light level SEI message) and VUI parameters in an appendix.

  - In JCT-VC review, it was suggested not to limit the scope to just two such SEI messages, and just the ones most supported elsewhere (e.g. by HDMI), and to survey what other SEI message might be appropriate for HDR.

- At what level of detail should encoding and decoding processes be described? (Only HEVC, or also AVC?) The call agreed to include recommendations for encoding with HEVC and also with AVC (but with secondary priority).

- Should pre- and/or post-filtering (such as denoising etc) be included/considered? The call agreed that denoising filters, smoothing filters etc. should not be considered to be in scope, but that colour transform and chroma resampling etc. should be considered to be in scope. The type of filters that are in scope and the type of filters that are not in scope should be explicitly defined in the document.

  - In JCT-VC review, it was suggested to at least mention such processing and describe why further information has not been provided, as a scope clarification, and perhaps to provide cautionary remarks if some practice that has been used for SDR needs special consideration because of HDR issues.

- How to handle the multitude of different processing platforms and target applications? The call agreed to consider creating categories and evaluate if the process should be different for the different categories. For those processes where alternatives are motivated these should be included and it should be described under what circumstances it might be beneficial to use them.

- Should there be insights and motivations for the processes? The call agreed to include motivation and insights related to the described technologies.

  - In JCT-VC review, it was commented that the proposed draft seems a bit too much like a specification of a single prescribed thing to do rather than providing general informative and educational comments, and this should be addressed.

- What experiments would we like to perform?

  - In JCT-VC review, it was said that this remained an open issue and various things should be considered; tests should be used to verify that the suggestions in the document are beneficial.

- How to proceed with the document? The call agreed to include the circulated document as an attachment to the AHG report.

  - In JCT-VC review, it was agreed that additional information could and should be included in the TR beyond what has been drafted.

Attached to this AHG report was the straw-man suggestion for the technical paper that was made available on the VCEG email reflector and JCT-VC email reflector on December 10, 2015 with modifications as suggested on December 17, 2015.

The ad hoc group recommended to create a break out group to continue the work and the discussion related to developing the TR.

To be done at this meeting:

- Review the draft of the TR

- Consider relevant input contributions

JCTVC-W0014 Report of MPEG AHG on HDR and WCG Video Coding [C. Fogg, E. Francois, W. Husak, A. Luthra]

This document provides the report of the MPEG AHG on HDR and WCG Video Coding.

An email reflector xyz@lists.uni-klu.ac.at was set up and hosted by Alpen-Adria-Universität Klagenfurt and industry experts were invited to join via http://lists.uni-klu.ac.at/mailman/listinfo/xyz and participate in this activity. As of February 3, 2016 there were 459 email addresses on this reflector’s list. Several emails were exchanged on various topics related to the mandates. Eight separate sub-reflectors were also set up associated with eight categories of the Core Experiments identified in Geneva (N15622).

We should clarify which reflectors will be used by JCT-VC in future work.

The AHG had one face-to-face meeting in Vancouver from Jan 12-14, 2016, with attendance of about 30 people (according to the registration record – there may have been somewhat more that were not recorded). As this activity of HDR/WCG video coding was planned to be made a part of JCTVC starting from February 19, 2016, the 2nd face to face meeting of the AHG, tentatively planned to start on February 19, 2016, was not held.

Documents reviewed at the AHG F2F meeting

| JCT-VC | MPEG | Source | Title |
| --- | --- | --- | --- |
| W0032 | m37539 | InterDigital | Encoder optimization for HDR/WCG coding |
| W0031 | m37536 | CE2 chairs | Description of the reshaper parameters derivation process in ETM reference software |
| W0024 | m37537 | CE4 chairs | Report of HDR CE4 |
| W0025 | m37538 | CE5 chairs | Report of HDR CE5 |
| W0034 | m37542 | Philips | Report on CE6 |
| W0033 | m37541 | BBC | Partial results for Core Experiment |
| W0101 | m37535 | BBC | Hybrid Log-Gamma HDR |
| W0035 | m37543 | Arris | Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7) |
| ? | m37541 | BBC | Partial results for Core Experiment 7 |
| ? | ? | Technicolor | SDR quality evaluation of HLG |
| W0036 | m37602 | Philips | Report on CE7 xCheck and Viewing |

Substantial detail about the AHG meeting was provided in the AHG report.

CEs:

- HDR CE1: Optimization without HEVC specification change

- HDR CE2: Post-decoding signal proc with 4:2:0 10 b YCbCr NCL fixed point output for HDR video coding

  - Setting 0: Improving HDR coding beyond what CE1 could do (may or may not be BC), see W0084/W0085

  - Setting 1: Backward compatibility by one method for chroma (may also improve HDR)

  - Setting 2: Backward compatibility by another method for chroma (may also improve HDR)

- HDR CE3: Objective/subjective metrics

- HDR CE4: Consumer monitor testing

- HDR CE5: Alternative colour transforms and 4:4:44:2:0 resampling filters

- HDR CE6: Using other processing not included in CE2 (e.g. 4:4:4 domain processing)

- HDR CE7: Hybrid log gamma investigation

- HDR CE8: Viewable SDR testing [single-layer BC]

CE2 ETM experiments can be categorized as follows:

| Mode | Use case | Luma | Chroma mode | Mode |
| --- | --- | --- | --- | --- |
| 0 | Single-layer HDR | 8 piecewise polynomial segments. | Intra plane | 1 piece-wise linear segment each for Cb, Cr. |
| 1 | Single-layer SDR backward compatible. |  | Cross plane | 8 piece-wise linear segments each for Cb, Cr. |
| 2 |  |  |  | 8 piece-wise linear segments shared by Cb, Cr. |

(All modes are now completely automatic. This was not the case before.)

Setting 0 was said to be better performing for HDR quality than "setting 1".

First check "CE 1 v3.2" vs. one of the automatic "CE 2 setting 0" variants.

# Documents of the MPEG AHG meeting in Vancouver (6)

- See JCTVC-W0014 for details of prior discussion of these contributions – further review was not considered necessary

JCTVC-W0031 Description of the reshaper parameters derivation process in ETM reference software [K. Minoo (Arris), T. Lu, P. Yin (Dolby), L. Kerofsky (InterDigital), D. Rusanovskyy (Qualcomm), E. Francois (Technicolor)]

JCTVC-W0032 Encoder optimization for HDR/WCG coding [Y. He, Y. Ye, L. Kerofsky (InterDigital)]

W0033 (WG11 number M37541) were overwrittten by another doc

W0034 (WG11 number M37542) were overwrittten by another doc

(put headers here, fix numbering problem)

JCTVC-W0035 Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7) [A. Luthra, D. Baylon, K. Minoo, Y. Yu, Z. Gu]

JCTVC-W0037 Hybrid Log-Gamma HDR [A. Cotton, M. Naccari (BBC)]

# Project development, status, and guidance (6)

## Corrigenda items (0)

See the AHG 2 report.

## Profile/level definitions (1)

JCTVC-W0129 SCC Level Limits Based on Chroma Format [S. Deshpande, S.-H. Kim (Sharp)] [late]

Discussed Sunday 21st 1700 GJS.

The current calculation in HEVC SCC Draft 5 for MaxDpbSize does not account for different chroma subsampling. This proposal describes extending the current design to support using higher MaxDpbSize for lower chroma format values. In this document MaxDpbSize calculation which accounts for memory use due to chroma components is proposed for Screen-Extended Main 10 4:4:4 profile.

A similar concept was previously proposed in V0037.

It was asked how common the use case might be.

It was remarked that the only SCC profiles that support monochrome are 4:4:4 profiles.

Consider in joint meeting.

## Conformance test set development (0)

See the AHG report JCTVC-W0004 and outputs JCTVC-W10XX for SHVC.

## SCC text development (2)

Discussed Sunday 1430 GJS.

JCTVC-W0096 Proposed editorial improvements to HEVC Screen Content Coding Draft Text 5 [R. Joshi, G. Sullivan, J. Xu, Y. Ye, S. Liu, Y.-K. Wang, G. Tech] [miss] [late]

This document summarizes the proposed editorial improvements to HEVC Screen Context Coding Draft Text 4. It also identifies known issues that should be addressed in version 6 of the draft text. The accompanying documents, JCTVC-V1005-W0096.doc and JCTVC-V0031_fixes_annexes_F_G_v2, contain the proposed changes with revision marks relative to JCTVC-V1005-v1.

- Proposed editorial improvements related to adaptive colour transform (editorial – OK)

- Wavefronts & tiles should be allowed to be used together in Screen-Extended 4:4:4 profiles Decision (BF): OK (was an overlooked item from the previous meeting) – the editors were asked to check the wording of the CPR region constraint

- Small editorial items – OK

  - Ticket #1440 – editorial clarification

  - Ticket #1435: Changed "residual_adaptive_colour_transform_flag" to "residual_adaptive_colour_transform_enabled_flag" in subclause 8.4.1.

  - Ticket #1434 and #1436: Introduced the variable ControlParaAct as an input to subclause 8.4.4.1: General decoding for intra blocks.

  - Ticket #1437 small luma/chroma copy-paste error

  - In subclause I.7.4.3.3.7, "Picture parameter set 3D extension semantics", the variable MaxNumLayersMinus1 is used without a definition.

  - In subclauses G.11.2.1 and H.11.2.1, an equation number is missing.

  - Ticket #1431: In subclause 7.3.6.3, pred_weight_table( ), which is invoked from slice_segment_header( ), has dependencies on RefPicList0 and PicOrderCntVal, variables that (according to the first sentence of subclause 8.3.2) are only computed "after decoding of a slice header".

  - Ticket #1433: Missing CABAC information.

  - Ticket #1439: IRAP constraint for Main 4:4:4 Intra profile (forgotten mention of one profile in a list)

- New High Throughput profiles require support for 4:2:2 (esp. the Screen-Extended ones) Tentative decision (self-consistency/BF): Do not support in the Screen-Extended ones

- Monochrome is not supported in Screen-Extended Main and Screen-Extended Main 10 profiles - no action on that

- Decoder conformance for Scalable Range Extensions – Revisit

- Should an Annex A SCC decoder be required to have INBLD capability? That doesn't really seem necessary or appropriate – no action on that.

- Should the SCC HT profiles be called HT profiles? Editors have discretion to consider some terminology restructuring, but no clear approach was identified in the discussion that seemed better than the current text in that regard.

- Multiview & SHVC aspect Deferred.

NB comments & disposition

Two technical comments from Finland (bug fixes of prior scalability-related aspects): Deferred.

Other NB comments seem to be on issues that have been resolved.

## HEVC coding performance, implementation demonstrations and design analysis (1)

### HM performance (0)

### RExt performance (0)

### SHVC performance/verification test (1)

JCTVC-W0095 SHVC verification test results [Y. He, Y. Ye (InterDigital), Hendry, Y. K. Wang (Qualcomm), V. Baroncini (FUB)]

### SCC performance, design aspects and test conditions (1)

JCTVC-W0104 Comparison of Compression Performance of HEVC Screen Content Coding Extensions Test Model 6 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

Presented on Sunday 21st at 1730 (Rajan).

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC Screen Content Coding (SCC) Test Model 6 (SCM 6) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, JCTVC-R0101, JCTVC-S0084, JCTVC-T0042, JCTVC-U0051, and JCTVC-V0033 – updating the results by using the latest available reference software (JM-19.0, HM-16.7+SCM-6.0), profile and test model designs, and SCC common test conditions (CTC) test sequences. The overall results indicate that for screen content CTC sequences, the HEVC SCC Test Model 6 improves quite substantially over JM-19.0. For example, for RGB text and graphics with motion (TGM) 1080p&720 sequences, HEVC SCC Test Model 6 saves 86%, 81%, and 78% bits for AI, RA and LB lossy coding over JM-19.0, respectively (the corresponding numbers are also 86%, 81% and 78% in JCTVC-V0033, which compares HM-16.6+SCM-5.2 with JM-19.0).

Results are also provided for non-CTC sequences (used during RExt development). For some simple sequences such as waveform, the BD-rate saving were over 90%.

## Systems interfacing (0)

## Source video test material (1)

JCTVC-W0083 SJTU 4k test sequences evaluation report [N. Kim, S. Jeon, H. Shim (SKKU)] [miss] [late]

Was intended to go to JVET – will be withdrawn.

# Core experiments in HDR (39)

## HDR CE1: Optimization without HEVC specification change (2)

Initially discussed Fri 1745 GJS & JRO

### CE1 summary and general discussion (1)

JCTVC-W0021 Report on HDR CE1 [Jacob Ström, Joel Sole, Yuwen He]

This document provides a description of work done in Core Experiment (CE1). The goal of this CE was to identify and investigate methods for the optimization of the HEVC Main 10 coding of High Dynamic Range (HDR) and Wide Colour Gamut (WCG) content without any specification changes. During the course of the CE1, several ways to improve the anchors have been found and incorporated into the anchor generation process. At the interim meeting in Vancouver meeting anchor v3.2 was selected as the anchor to be used.

The anchor generation process has gone through a number of versions summarized in Table 1.

| Final code released | Version number | Comment | Date of decision to use as current anchor |
| --- | --- | --- | --- |
|  | v1.0 | CfE anchor |  |
| Oct 30, 2015 | v2.0 | CE1.a + CE1.b + HM16.7 + VUI/SEI parameter changes + chroma position change + integer QP | Oct 30 2015 |
| Nov 10, 2015 | v2.1 | New sequences, split sequences, new rate points |  |
| Nov 21, 2015 | v3.0 | Average-luma controlled adaptive QP | Dec 1, 2015 |
| Dec 6, 2015 | v3.1 | Bugfix1 (luma dQP LUT metadata file reading) |  |
| Jan 5, 2016 | v3.2 | Bugfix2 (luma dQP signaling) + integer QP | Jan 12, 2016 |

On Jan 5, 2016, Interdigital shared two anchor improvement proposals to the reflector based on m37227. The software was based on v3.0 of the anchors and contained two tests; test1 was anchor v3.0 with added deblocking optimization and chroma QP offset adjustment, and test 2 was anchor v3.0 with deblocking optimization only.

During the interim meeting, anchors 3.2 were shown on the SIM2 display and compared against v2.1. It was concluded that the new anchors (v3.2) were a substantial improvement over v2.1.

A viewing was also held between Interdigital’s test1 and anchor v3.2. The group decided to select anchor v3.2, which became the current anchor.

Comment: this QP control is deterministic; similar in spirit to fixed-QP. Let's not get into the territory of full rate control.

It was commented that reference software for "best practices", e.g. as a branch to the HM and the HDRtools software package, should be developed (and probably approved as standard reference software).

Further discussed Saturday GJS & JRO 1800

For software distribution:

- The CE1 anchor will be made a branch of the HM

- The HDRtools package will made accessible via a login available to all JCT-VC members

### CE1 primary contributions (0)

### CE1 cross checks (1)

JCTVC-W0108 Cross-check report on HDR/WCG CE1 new anchor generation [D. Jun, J. Lee, J. W. Kang] [late]

## HDR CE2: HDR CE2: 4:2:0 YCbCr NCL fixed point for HDR video coding (13)

Initially discussed Fri 1800 GJS & JRO

### CE2 summary and general discussion (1)

JCTVC-W0022 Report of HDR Core Experiment 2 [D. Rusanovskyy (Qualcomm), E. Francois (Technicolor), L. Kerofsky (InterDigital), T. Lu (Dolby), K. Minoo (Arris)]

The exploratory experiment 2 (CE2) was established at the 113th MPEG meeting (N15800) with investigate HDR video coding technologies operating in 4:2:0 YCbCr NCL fixed point domain. Detailed CE2 description and planned sub-tests was provided in ISO/IEC JTC1/SC29/WG11 document N15795. There are 6 cross-checked input contribution reporting results of 10 sub-tests and 5 CE2 related contributions.

As a result of CE2 activity all given mandates have been accomplished in time. The Exploration Test Model (ETM) with automated encoder side algorithms was released on 12/19/2015, SW description and simulation results has been presented on 01/12/2016.

Preliminary CE2 report was presented in January 2016, F2F meeting. The CE2 description was updated based on the outcome of the F2F meeting. This document reports CE2 activity which followed the F2F meeting.

Planned CE2 sub-tests

| Planed sub-test | Proponents |
| --- | --- |
| CE2.a-1 luma ATF with LCS (m37245) | Qualcomm |
| CE2.a-2 reshaping (m37267) | Dolby |
| CE2.a-3 adaptive transfer function (m37091) | Arris |
| CE2.b-1 Cross-channel modulation (m37088 and m37285) | Technicolor |
| CE2.b-2 SDR backward compatibility study (m37092) | Arris |
| CE2.c Chroma QP offset (m37179) | Qualcomm, Arris |
| CE2.d DeltaQP adjustment for Luma | Arris |
| CE2.e-1 Enhancement of ETM | Qualcomm |
| CE2.e-2 harmonization of luma and chroma reshaping (m37332) | Arris |
| CE2.e-3 automatic selection of ETM parameters | Interdigital |

A revised CE description will be uploaded as part of the report.

| JCTVC doc | Title | Source | Sub-test |
| --- | --- | --- | --- |
| W0022 | HDR CE2 report (This document) | Coordinators |  |
| W0033 | HDR CE2: report of CE2.b-1 experiment (reshaper setting 1) | Technicolor | CE2.b-1 |
| W0084 | HDR CE2: CE2.a-2, CE2.c, CE2.d and CE2.e-3 | Dolby, InterDigital | CE2.a-2, CE2.c, CE2.d, C2.e-3 |
| W0093 | HDR CE2: Report of CE2.a-3, CE2.c and CE2.d experiments (for reshaping setting 2) | Arris | CE2a-3, CE2.c, CE2.d |
| W0094 | HDR CE2: Report of CE2.b-2, CE2.c and CE2.d experiments (for reshaping setting 2) | Arris | CE2.b-2, CE2.c, CE2.d |
| W0097 | HDR CE2: CE2.c-Chroma QP offset study report | Qualcomm | CE2.c |
| W0101 | HDR CE2: Report on CE2.a-1 LCS | Qualcomm | CE2.a |
| W0068 | CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile | ZTE |  |
| W0071 | CE2-related: Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile | ZTE |  |
| W0085 | HDR CE2 related: Further Improvement of JCTVC-W0084 | Dolby, InterDigital |  |
| W0089 | HDR CE2-related: some experiments on reshaping with input SDR | Technicolor |  |
| W0100 | HDR CE2-related: Results for combination of CE1 (anchor 3.2) and CE2 | Qualcomm |  |
| W0112 | HDR CE2: cross-check report of JCTVC-W0084 | Technicolor |  |
| W0113 | HDR CE2: cross-check report of JCTVC-W0101 | Technicolor |  |
| W0093 | HDR CE2: cross-check report of JCTVC-W0033 | Dolby |  |
| W0094 | HDR CE2: cross-check report of JCTVC-W0097 | Dolby |  |
| W0101 | HDR CE2 related: cross-check report of linear luma re-shaper | Samsung |  |
| W0128 | HDR CE2: Cross-check of JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3) | Qualcomm |  |
| W0130 | Crosscheck for further improvement of HDR CE2 (JCTVC-W0085) | Intel |  |

It was asked whether there were changes happening in CE2 after the stabilization of CE1 that could have been applied to CE1, and whether there had been sufficient time to study what had been happening. However, it was commented that some or all of the changes that took place were not really the development of new ideas but rather application of things that had taken place in the CE 1 context.

In principle, CE1 itself could be a moving target (e.g. further improvement of the table mapping of brightness to QP) but its evolution was stopped by a desire to deliver it on a deadline.

It was agreed that our initial test vs. CE 1 should consider only one of the two CE 2 variants since they had been stabilized earlier: W0084 and W0097.

Some participants had received Jan 27 software ETM r1 (which was not the W0084 software) and have studied it and had it ready to use. Samsung had checked this.

Some proponents suggested Feb 1 W0084 CE2 software instead, as it was delivered as the software to be used for the CE. This had been cross-checked by Technicolor (although they had reported a rate allocation issue - see notes elsewhere).

It was agreed we'll use the Feb 1 software.

Some concern about the bit allocation was expressed - see notes elsewhere. We can investigate that question afterwards.

We have two codecs * 15 test sequences * 4 bit rates = 120 total possible

Trim the number of test sequences to avoid ones that are too short  about 8

Use the three lowest bit rates.

2*6*3=36

Dry run Sat a.m.; test Sat 6 p.m. or by lunch Sunday.

See discussion of W0112.

Make sure the PC graphics card sync rate is 50 Hz if the display is 50 Hz.

Use:

- Balloon [guy lines]

- Market [walls, canopy, clothing]

- EBU06Starting (play slowed to 50; source is 100) [lines of tracks, problems in dark]

- Garage Exit [power line at the end]

- Showgirl [face area, background dark region blocking & noise]

- Sunrise [dark areas, clouds in sky, trees with blocking]

V. Baroncini to coordinate.

| Reported sub-tests | Proponents | Cross-checker | Notes | Report Doc. | Cross-check report |
| --- | --- | --- | --- | --- | --- |
| CE2.a-1 luma ATF with LCS (m37245) | Qualcomm | Technicolor |  | W0101 | W0113 |
| CE2.a-2 reshaping (m37267) Jointly with CE2.a-2, CE2.c, CE2.d, CE2.e-3 | Dolby, Interdigital | Qualcomm Technicolor |  |  |  |
| CE2.e-3 automatic selection of ETM parameters |  |  | Merged with CE2.a-2 | W0084 | W0128 W0112 |
| CE2.c Chroma QP offset (m37179) |  |  | Merged with CE2.a-2 |  |  |
| CE2.d DeltaQP adjustment for Luma |  |  | Merged with CE2.a-2 |  |  |
| CE2.b-1 Cross-channel modulation (m37088 and m37285) | Technicolor | Dolby |  | W0033 | W0093 |
| CE2.a-3 adaptive transfer function (m37091) | Arris |  | Combined with CE2.c and CE2.d | W0093 |  |
| CE2.b-2 SDR backward compatibility study (m37092) | Arris |  | Combined with CE2.c and CE2.d | W0094 |  |
| CE2.c Chroma QP offset (m37179) | Qualcomm | Dolby |  | W0097 | W0094 |
| CE2.e-1 Enhancement of ETM | Qualcomm | Arris |  | Withdrawn | W0130 |
| CE2.e-2 harmonization of luma and chroma reshaping (m37332) | Arris |  |  | Withdrawn |  |

CE2 sub-tests summary:

W0033:	CE2.b-1 experiment (reshaper setting 1)

Backward compatible configuration of ETM, reshaper setting 1 (cross component YCbCr processing)

Algorithms of luma and chroma reshapers derivation

No changes to the syntax or decoding process of the ETM reported

These experiments relate to SDR backward compatible configuration, and focus on reshape setting 1 that involves cross-plane colour reshaping. The experiments only involve the reshaping algorithm, and does not require any change in the current ETM syntax and inverse reshaping process. The modified reshaper tuning algorithm for reshape setting 1 is proposed to be adopted in the ETM. It is reported that the proposed changes improves the reshaped SDR content quality (in terms of visual rendering of contrast and colours), while preserving the compression performance.

W0084:	Combined solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3

Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

Algorithm for luma reshaper derivation and automatic update mechanism

HM encoder optimization

DeltaQP adjustment for luma and

ChromaQP offset adjustment for chroma for local control

Change to deblocking filter param derivation

No changes to the syntax or decoding process of the ETM reported

This proposal reports a combination of CE2 subtests: CE2.a-2 on luma forward reshaping improvement, CE2.c on chromaQPOffset, CE2.d on DeltaQP adjustment for luma, and CE2.e-3 on automatic selection of ETM parameters. The experiment is to test non-normative luma reshaping improvement as well as joint optimization of reshaper and encoder. The encoder optimization methods include chromaQPOffset, luma DeltaQP adjustment, and deblocking filter parameter selection.

W0093 HDR CE2: Report of CE2.a-3, CE2.c and CE2.d experiments (for reshaping setting 2)

Backward compatible configuration of ETM, reshaper setting 2 (cross component YCbCr processing)

Algorithms for Adaptive Transfer Function (ATF) derivation for luma and chroma

HM encoder optimization

DeltaQP adjustment for luma

No changes to the syntax or decoding process of the ETM reported

This document reports the results of experiments for CE2.a-3 in conjunction with encoding optimization techniques which have been studied under CE2.c and CE2.d categories. The combined experiment was conducted to measure the performance of an alternative transfer function (instead of conventional PQ TF) as well as joint optimization of the reshaper and encoder, specifically when the reshaping setting is in "setting 2". Informal subjective evaluations were conducted for both the reconstructed SDR and HDR sequences. It is asserted that compared to the version 3.2 of anchor, the proposed approach provides similar perceptual quality of the reconstructed HDR signal while supporting SDR backward compatibility.

W0094 HDR CE2: Report of CE2.b-2, CE2.c and CE2.d experiments (for reshaping setting 2)

Backward compatible configuration of ETM, reshaper setting 2 (cross component YCbCr processing)

Algorithms for Adaptive Transfer Function (ATF) derivation for luma and chroma

HM encoder optimization

DeltaQP adjustment for luma

ChromaQP offset adjustment for chroma

No changes to the syntax or decoding process of the ETM reported

This document reports a combination of CE2 subtests: CE2.b-2 on SDR backward compatibility, CE2.c on chromaQPOffset, and CE2.d on DeltaQP adjustment for luma. The experiment was conducted to study bitstream SDR backward compatibility as well as joint optimization of the reshaper and encoder. Subjective evaluation was conducted on a SIM2. It is asserted that compared to anchor v3.2, the proposed joint optimization of reshaper and encoder provides similar perceptual quality while supporting SDR backward compatibility.

W0097 HDR CE2: CE2.c-Chroma QP offset study report

Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

Algorithm for chroma ATF derivation

Tested in 2 frameworks:

Reference ETM software

Software of the W0084 (alternative luma reshaper)

HM encoder optimization:

CE1 anchor 3.2 default settings

No changes to the syntax or decoding process of the ETM reported

This document reports results of CE2.c sub-test which studies harmonization of chroma QP offset utilized in CE1 anchor and CrCb processing of the ETM. As a result of the study, modification of encoder side algorithm is proposed to the ETM. Proposed in this document solution employs an independent multirange ATF (reshaper) for each colour component. Proposed solution provides objective gain as well as improvement in visual quality over reference ETM design as well as over the CE1 anchor. Proposed multirange ATF was also integrated and tested in software of W0084.

W0101 HDR CE2: Report on CE2.a-1 Luma based Chroma Scaling

Non backward compatible configuration of ETM, reshaper setting 1 (cross-component YCbCr processing)

Algorithm for chroma ATF derivation

No changes to the syntax or decoding process of the ETM reported

This document reports the results of the study of CE2.a-1 on luma-based chroma scaling (LCS). The chroma sample values are scaled by values that are dependent on the co-sited luma sample values, and the scale values are computed based on a function that increases with luma values. The document reports objective evaluation on the tested configuration.

CE2 – related summary:

W0068	CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile

Non-ETM framework

The perceptual quantization (PQ) for HDR video coding has perceptual uniformity in the luminance range with the modest bit-depth. However, the dynamic range of HDR video is not fully utilized in PQ, especially PQ for chrominance channels. Thus, there exist the wasted dynamic ranges in PQ which cause detail loss and colour distortions. In this contribution, adaptive quantization-based HDR video coding is proposed. Adaptive mapping of sample values with bit depths of 16 bit and 10 bit is established to perform conversion based on cumulative distribution function (CDF), i.e. adaptive quantization, instead of linear mapping. First, CDF is derived from the histogram of each picture. Then, adaptive quantization is performed based on CDF. The metadata for adaptive conversion are coded for performing the inverse conversion at destination. Experimental results demonstrate that the proposed method achieves average 1.7 dB gains in tPSNR XYZ and 2 dB gains in PSNR_DE over Anchor. Subjective comparisons show that the proposed method preserves image details while reducing colour distortion.

W0071	CE2-related: Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile

Non-ETM framework

Proposed is an adaptive transform function based perceptual quantizer (PQ) for HDR video, called adaptive PQ. Adaptive PQ is to solve the problem that PQ is not adaptive to HDR content because of its using a fixed mapping function from luminance to luma. By introducing a ratio factor derived from the luminance information of HDR content, adaptive PQ maps luminance to luma adaptively according to the content. Experimental results demonstrate that adaptive PQ achieves average bit-rate reductions of 3.51% in tPSNR XYZ and 5.51% in PSNR_DE over PQ.

W0085 HDR CE2 related: Further Improvement of JCTVC-W0084

This proposal reports a further improvement of JCTVC-W0084. The improvement is solely on luma DeltaQP adjustment.

W0089 HDR CE2-related: some experiments on reshaping with input SDR

Backward compatible configuration of ETM, reshaper setting 1 (cross component YCbCr processing)

Algorithms of luma and chroma reshapers derivation

No changes to the syntax or decoding process of the ETM reported

This document reports preliminary experiments on ETM with dual SDR/HDR grading. In this configuration, a graded SDR version as well as the usual graded HDR version are given as inputs to the reshaper. The reshaping process uses the input SDR as reference and generates reshaping data to match as much as possible the reshaped input HDR to the input SDR. In the experiments, reshape setting 1 configuration is used (cross-plane chroma reshaping). It is reported that these preliminary experiments tend to show that ETM models are able to properly reproduce an SDR intent.

W0100 Results for combination of CE1 (anchor 3.2) and CE2

Non backward compatible configuration of ETM, reshaper setting 0 (independent component YCbCr processing)

HM encoder optimization

CE1 default anchor settings

Additional DeltaQP adjustment for luma

Additional ChromaQP offset adjustment for chroma

No changes to the syntax or decoding process of the ETM reported

This document reports results of combining the software of anchor 3.2 that is the outcome of CE1 and the ETM that is the outcome of CE2. Results for the direct combination of the CE1 and CE2 software are provided. It is noted that CE1 and CE2 algorithms have some degree of overlap, so the direct combination could be improved by a proper adjustment of the parameters. Results obtained by adjusting the parameters are also provided.

### CE2 primary contributions (6)

JCTVC-W0033 HDR CE2: report of CE2.b-1 experiment (reshaper setting 1) [E. Francois, Y. Olivier]

Setting 1: This (as used) targets backward compatibility. Deferred.

JCTVC-W0084 HDR CE2: CE2.a-2, CE2.c, CE2.d and CE2.e-3 [T. Lu, F. Pu, P. Yin, T. Chen, W. Husak (Dolby), Y. He, L. Kerofsky, Y. Ye (InterDigital)]

Discussed Sunday 21st 1010–1130 (GJS & JRO).

This proposal reports a combination of CE2 subtests: CE2.a-2 on luma forward reshaping improvement, CE2.c on chromaQPOffset, CE2.d on DeltaQP adjustment for luma, and CE2.e-3 on automatic selection of ETM parameters. The experiment is to test non-normative luma reshaping improvement as well as joint optimization of reshaper and encoder. The encoder optimization methods include chromaQPOffset, luma DeltaQP adjustment, and deblocking filter parameter selection. Subjective evaluation was conducted on both Pulsar and SIM2. It is asserted that compared to Anchor v3.2, the proposed joint optimization of reshaper and encoder provides visible subjective quality improvements for many of the test clips, primarily in the form of more texture details.

The ST 2084 luminance range is divided into a number of pieces and codewords are allocated into each piece based on image statistics computed on the fly.

The remapper is determined automatically and without look-ahead. Updating is done when a change of image content range or content characteristics is detected. It is sent with each IDR, even when not changed.

For the version of Feb1 (used in visual comparison), one improvement is that no update is made when unnecessary (the first fully automatic version ETMr1 made an update at least once per second, based on the characteristics of the new IDR picture). This however only saves little data rate. Other changes are more fundamental. The reshaper function of ETMr1 is a simple power function, which is changed to become more flexible based on the alpha parameter in 0084. 0084 also uses adaptive chroma QP offset and luma QP adaptation similar to CE1. Furthermore, deblocking is modified. The encoder selects tc and beta for minimum distortion between original and reconstructed. This method is equivalent to the method that had been investigated in CE1 (but not used there per decision of the Vancouver meeting).

In the discussion, it was asked what would be done if two different image areas are included in the same picture (e.g., by an editing operation, or different areas of the same scene having different characteristics). The scheme applies only one remapper to the entire picture and would thus need to apply a single remapping to all regions within a picture.

In principle, the same thing could be done to SDR video if desirable. However, the encoding scheme used here is targeted for ST 2084.

One experts mentioned that similar adjustment as by the suggested reshaping curve could also be used to further improve the QP adaptation of CE1.

The alpha parameter which adjusts the reshaper characteristics is determined based on percentage of dark and bright areas. alpha equal 1 would retain the original PQ codeword. The actual reshaping is done based on the representation of the ATM (piecewise linear etc.)

For the local QP adaptation, the change is less aggressive than in CE1, as it only compensates locally what the globally optimized reshaper is not able to do. The subsequent contribution 0085 makes this further dependent on reshaper characteristics, whereas W0084 is reshaper agnostic.

Another modification is made by adjusting the QP offset over hierarchy layers that was made. This explains the shift to first frame as observed by crosschecker, but some concern is raised about such strategy in visual comparison.

JCTVC-W0093 HDR CE2: Report of CE2.a-3, CE2.c and CE2.d experiments (for reshaping setting 2) [Y. Yu, Z. Gu, K. Minoo, D. Baylon, A. Luthra (Arris)]

Setting 2: This (as used) targets backward compatibility. Deferred.

JCTVC-W0094 HDR CE2: Report of CE2.b-2, CE2.c and CE2.d experiments (for reshaping setting 2) [Z. Gu, K. Minoo, Y. Yu, D. Baylon, A. Luthra (Arris)]

Setting 2: This (as used) targets backward compatibility. Deferred.

JCTVC-W0097 HDR CE2: CE2.c-Chroma QP offset study report [D. Rusanovskyy, J. Sole, A. K. Ramasubramonian, D. Bugdayci, M. Karczewicz (Qualcomm)]

Discussed Sunday 21st 1145–1230 (GJS & JRO).

This document reports results of CE2.c sub-test which studies modification of the chroma QP offset proposed in m37179 and used in CrCb processing in the HDR/WCG ETM. As a result of the study, of modification of the encoder side algorithm is proposed to the ETM. The proposal in this document employs an independent multirange "adaptive transfer function" (ATF) (a.k.a. reshaper, remapper) for each color component. It reportedly provides objective gain as well as improvement in visual quality over the reference ETM design as well as over the CE1 anchor.

This is in "setting 0" of the ETM – i.e., it does not include cross-component processing and does not have spatial effects. It uses a multi-segment TF, whereas the CE2 ETM encoder uses just a single segment (one offset and scaling) for each chroma component. This uses 4 segments. The ETM decoder software supports 32 segments.

Viewing needed after CE1 vs. CE2 test.

Powerpoint deck is missing.

JCTVC-W0101 HDR CE2: Report on CE2.a-1 LCS [A. K. Ramasubramonian, J. Sole, D. Rusanovskyy, D. Bugdayci, M. Karczewicz (Qualcomm)]

Discussed Sunday 21st 1230–1245 (GJS & JRO).

This document reports the results of the study of CE2.a-1 on luma-based chroma scaling (LCS). The chroma sample values are scaled by values that are dependent on the co-sited luma sample values, and the scale values are computed based on a function that increases with luma values. The document reports objective evaluation on the tested configuration.

As used, when the luma value increases, the chroma gain increases.

Powerpoint deck is missing.

The cross-checker reports that in regard of visual quality this is equivalent to W0084. This may be explainable by the fact that W0101 uses the luma reshaper of ETMr1, whereas W0084 has an improved method of determining the luma reshaper. Could potentially be combined on top of 0084.

The cross-checker said this did not seem visually better than W0084, although further study in a CE for a future combination may be desirable.

JCTVC-W0102 Withdrawn

### CE2 cross checks (6)

Discussed Fri 19:00 (GJS & JRO).

JCTVC-W0112 HDR CE2: crosscheck of CE2.a-2, CE2.c, CE2.d and CE2.e-3 (JCTVC-W0084) [C. Chevance, Y. Olivier (Technicolor)] [late]

This document reports on cross-checking of core experiment HDR CE2.a-2, CE2.c, CE2.d and CE2.e-3, described in document JCTVC-W0084. It is reported that objective results are perfectly matching. It is also reported that the overall visual quality compared to the (latest) anchors is improved. Some comments on the differences of bitrate repartition compared to the anchors are also made.

According to HDR CE2 plan, subjective viewing was conducted, on SIM2 display. The evaluation was performed in priority in video mode, then in still picture mode to assess more specific details.

Compared to the CE1 - V3.2 anchors, the visual quality is reportedly better on the following sequences:

- BalloonFestival : Sharper on mountain, better on rope

- BikeSparklers (cut1, cut2) : Sharper everywhere (roof, ground)

- Hurdles : Racetrack sharper

- Starting : sharper on Grass, red line, roof, cameras, rope and pylon

- Market : wall and tower

Compared to the CE1 - V3.2 anchors, the visual quality is reportedly slightly better on the following sequences (mostly when viewing in still picture mode, the difference being rather difficult to catch in video mode):

- Showgirl (face)

- Stem_MagicHour (cut1, cut2,cut3) : Slightly sharper

- Stem_WramNight (cut1, cut2) : Slightly sharper

- garageExit : faces and ropes

Finally, an equivalent quality with the anchors is reported on:

- FireEater

- SunRise

In conclusion the visual quality is globally better than HDR CE1 - v3.2 anchors.

An analysis of the bitrate repartition has also been made to check the differences compared to the anchors.

In particular it seems that more bits are spent on intra pictures with the proposal. One possible explanation is that the proposed reshaping by itself results in distributing more bits in intra pictures, because of a larger use of codewords range, which is then compensated by a lower cost spent for inter pictures. The improvement of the intra frames quality generally leads to a quality improvement on the entire sequence, especially for sequences with low amplitude global motion. The bitrate repartition actually strongly depends on the automatic reshaping function and on the way the codewords are redistributed by the reshaping.

More bits were devoted to the I picture than for the anchor. The bit cost difference with anchors, per frame, is diagrammed below.

There was concern expressed about whether a visual improvement relative to the anchor might be related to the bit allocation effect observed above.

JCTVC-W0113 HDR CE2: crosscheck of CE2.a-1 LCS (JCTVC-W0101) [C. Chevance, Y. Olivier (Technicolor)] [late]

JCTVC-W0121 HDR CE2: Cross-check report of JCTVC-W0033 [T. Lu, F. Pu, P. Yin (Dolby)] [miss] [late]

JCTVC-W0122 HDR CE2: Cross-check report of JCTVC-W0097 [T. Lu, F. Pu, P. Yin (Dolby)] [miss] [late]

JCTVC-W0128 HDR CE2: Cross-check of JCTVC-W0084 (combines solution for CE2.a-2, CE2.c, CE2.d and CE2.e-3) [D. Rusanovskyy (Qualcomm)] [miss] [late]

JCTVC-W0131 HDR CE2: cross-check report of CE2.a-3, CE2.c and CE2.d experiments (JCTVC-W0093) [C. Chevance, Y. Olivier (Technicolor)]

## HDR CE3: Objective/subjective metrics (3)

Initially discussed Fri 1930 GJS & JRO

### CE3 summary and general discussion (1)

JCTVC-W0023 Report of HDR Core Experiment 3 [Vittorio Baroncini, Louis Kerofsky, Dmytro Rusanovskyy]

The exploratory experiment 3 (CE3) was established at the 113th MPEG meeting (N15800) with the purpose of understanding various proposed objective video quality metrics. Two categories of objective metrics were previously identified based on sensitivity to luma or colour distortions.  Software for additional objective metrics was collected. The details of the experiments that were conducted is provided in ISO/IEC JTC1/SC29/WG11 N15457

Currently in MPEG, investigation an objective metric for HDR/WCG coding which correlates well with subjective results is absent. Separate classes of metrics corresponding to “luminance only” and “colour” specific metrics have been proposed. This CE served to examine the performance of these metrics. The lack of subjective data hiders the process of examining objective metrics. It was realized the subjective data recorded along with the CfE responses could be leveraged to help understand the objective metrics. The lab in Rome had access to the CfE response source data as well as subjective evaluations. They graciously offered to assist with leveraging this data understand the objective metrics.

Four luminance specific objective metrics and four colour specific objective metrics were identified of interest. Software for these objective metric calculation was collected. The original plan was for executables only to be provided for the HDR-VDP2 and HDR-VQM metrics. Software for the other metrics was delivered as an extension to the HDRTools package. With exception of the HDR-VDP2, software for these algorithms was distributed following the process developed to share CE software via mpeg ftp.

An update on progress with the generation of objective data and corresponding subjective results was given at the interim meeting held January 12-14 in Vancouver Canada. Concern was raised about both processing speed and exact details of the metics under examinatoin. Desire for publicly available C code for metrics was an outcome of this dicussion. Several highlights of the discssion are repeated below.

- Samsung has kindly publicly released C++ code for the HDR-VQM metric (for HDRtools software package).

- The Dolby VDP2 implementation has been verified by Arris. A request was made to Dolby to examine release of their C++ implementation ASAP.

- EPFL has completed a subjective evaluation of the CFE anchors and will have an input contribution for the San Diego meeting. EPFL will release the MOS scores for independent cross correlation to the objective metrics.

- UBC has indicated VIF C++ package would be released by the San Diego meeting.

- UBC also has MOS scores that they are willing to release.

Based on discussion at this interim meeting, several contributions on objective metrics are expected at the San Diego meeting. Review of these contributions is expected to be a major source of disucssion. The following contributions to the San Diego meeting were identified as of interest to the objective metric topic of CE3:

- JCTVC-W0041/ m37625 "HDR-VQM Reference Code and its Usage“. Source: Samsung, Describes structure and use of the SW provided for the HDR-VQM objective metric.

- JCTVC-W0090 "HDR CE3: Results of subjective evaluations conducted with the DSIS method”. Source EPFL. Describes additional subjective testing done on HDR/WCG CfE responses. Replaces previous paired comparison method with DSIS method. Notes results may be used to evaluate objective metrics.

- JCTVC-W0091 "HDR CE3: Benchmarking of objective metrics for HDR video quality assessment”. Source EPFL. Provides evaluation of sample set of objective metrics using updated subjective test results. A subset of metrics listed in CE-3 were considered. Objective luminance metrics examined include:: tPSNR-Y, HDR-VDP-2, HDR-VQM, avLumaPSNR. Objective colour metrics examined include: avColourPSNR and PSNR-DE100. The VIF metric was also included in this study.

- JCTVC-W0115 "VIF Code for HDR Tools” (currently unavailable on document repository as of 2/18/2016)

The following items need to be addressed in San Diego:

- Examine the HDR-VQM metric C++ code kindly provided by Samsung, trying to fix the problems encountered (cfg info for a correct execution; analysis of the speed and of the results).

- Ask the Proponents to the CfE to provide the decoded files in .exr format (the HDR-VQM metric works on those). They could provide or the .exr files or the bitstreams and the scripts to decode them (so headache for the CE3 people in decoding stuff, please!!).

- Get bitstreams of the new Anchors and run subjective assessment and metric on those; FUB is volunteering to assess the new Anchors; we need volunteers to run the other metrics.

VQM behaviour

### CE3 primary contributions (2)

JCTVC-W0090 HDR CE3: Results of subjective evaluations conducted with the DSIS method [M. Rerabek, P. Hanhart, T. Ebrahimi (EPFL)] [late]

JCTVC-W0091 HDR CE3: Benchmarking of objective metrics for HDR video quality assessment [P. Hanhart, T. Ebrahimi (EPFL)] [late]

### CE3 cross checks (0)

## HDR CE4: Consumer monitor testing (1)

Initially discussed Fri 2000 GJS & JRO

### CE4 summary and general discussion (1)

JCTVC-W0024 Report of HDR CE4 [P. Topiwala (FastVDO), E. Alshina (Samsung), A. Smolic (Disney), S. Lee (Qualcomm)]

HDR CE4 had the mandate to investigate the potential suitability of consumer HDR televisions for use in our investigations of HDR/WCG video compression. The CE4 Description is included below as an Appendix for the convenience of the JCT-VC. It is remarked that this CE is not a traditional contest between proposals, but rather an open and collaborative investigation, more like an AHG. Moreover, due to device testing, most of the work of the CE can be performed mainly at meetings.

Legacy receivers have been in the range of 100 nits, and supporting 8-bit video with the BT.709 colour space. HDR can be viewed as video in the 1000+ nits range, supporting 10+ bits/sample, and potentially supporting the BT.2020 colour space. The work in the ISO/MPEG AhG on HDR/WCG has been exclusively using two monitors for all subjective assessments: (1) the Dolby Pulsar monitor, which is a private model internal to Dolby and unavailable; and (2) the Sim2, which at approximately $40K dollars is a reference monitor that is unlikely to reach consumers soon. Nevertheless, the work of the AhG aims to provide a coding technology for consumer services to the home. The only way to assess the value of any design for suitability for consumer services is therefore to actually test on devices that may be deployed in the home. We include in this category OLED reference monitors for test purposes.

Volunteer Disney Research, using proprietary algorithmic methods developed at their labs, created some 1k nit regraded version of a couple of test sequences. These sequences have been uploaded to the MPEG Repository, under \Explorations\HDR\1k Nit grading.

Volunteer Samsung produced some test output sequences by using the latest source codes produced by CE1 and CE2, with a view to compare them, both on professional monitors and consumer devices.

The interim meeting of the HDR activity in Vancouver afforded some viewing of both the sequences produced by Samsung, as well as the 1k Nit data produced by Disney. The devices available at the Telus labs included the Samsung JS9500. A number of impressionistic observations were made in Vancouver in a non-systematic, non-scientific way. It was noted that the 1k Nit data appeared not to be a close match to the 4k nit originals (they appeared more washed out). In fact, Disney reports that it was part of the intent in the regrading to make the 1k nit data paler, and somewhat less colourful, but closer to a cinematic look. If there are other preferences on how the regraded data should appear, it would be meaningful to collect such opinion if possible. It was also found that the original 4k nit data, as shown on the 1k nit max JS9500 device, appeared somewhat washed out and “non-HDRish”. These observations required further review of the test set up. Upon investigation, E. Alshina concluded that the way the video was fed to the device did not indicate how to perform the necessary display adaptation, resulting in brightness clippin and therefore suboptimal performance. So these experiments will need to be repeated.

E. Alshina of Samsung brought coded sequences to Vancouver according to CE1 and CE2, using the latest available source codes (e.g, v3.2 for CE1), for comparison on the consumer class JS9500, and general impressions were gathered in a non-scientific viewing. It seemed to be general opinion that on the whole both seemed to be about equal in quality, while there was some opinion that CE1 was better (opinion the other way seemed less common).

It was anticipated that activities similar to those at the Vancouver meeting will be conducted in San Diego, potentially as a breakout group. It was also anticipated to propose continuing this activity beyond this San Diego meeting.

It was agreed to have a BoG (P. Topiwala) to try to arrange a similar test of CE 1 and CE 2 results using a consumer monitor to help determine the ability to test with a consumer device.

It was suggested to try to measure and study the response characteristics of some consumer monitor(s) and professional monitor(s) and determine what characteristics exist and how this may effect viewing results.

### CE4 primary contributions (0)

### CE4 cross checks (0)

## HDR CE5: Colour transforms and sampling filters (6)

Initially discussed Saturday 1400 GJS & JRO.

### CE5 summary and general discussion (1)

JCTVC-W0025 Report of HDR CE5 [P. Topiwala (FastVDO), R. Brondijk (Philips), J. Sole (Qualcomm), A. Smolic (Disney)]

HDR CE5 had the mandate to investigate the impact of alternative colour transforms and resampling filters as pre- and post-processing steps on the efficiency of HDR/WCG coding. In addition, there was a mandatory transfer function, as well as an optional one, to incorporate in the processing chain. Several proposals were submitted under this CE, investigating these parameters.

A number of proposed tests were set up in this CE, stemming from the work presented at the Geneva meeting. Three tests were performed – a Y′′u′′v′′ colour space proposed in W0069, a Y′FbFr lifting-based colour space proposed in m37065, and constant-luminance colour space from BT.2020 (see W0099) with different ordering of the application of the transformation matrix and different scaling of the chroma – each tested with some alternative resampling filter. The Y′FbFr proposal used a significantly different resampling filter than used in the anchor. The source code for these experiments were released and uploaded to the MPEG repository, under /Explorations/HDR/CE5/. As of this time, the following documents are registered.

- JCTVC-W0069, HDR CE5: Report of Core Experiment CE5.3.1, R. Brondijk et al (Philips)

- JCTVC-W0055, HDR CE5: Report of CE5.3.2, W. Dai et al (FastVDO)

- JCTVC-W0070, CE5-related, xCheck of CE5.3.2

- JCTVC-W0080, Crosscheck of HDR CE5.3.1 (JCTVC-W0069).

- JCTVC-W0099, HDR CE5 test 3: Constant Luminance results.

The cross-checkers did not assess the proposals for visual improvement – only looked for serious visual problems.

One point that was raised on the reflector was by Tomohiro Ikai of Sharp. It is stated in the CE5 description that the colour transforms and resampling filters are the only tools under test; no other parameters were to be varied. That is, to follow CE1 directly in other matters. (One exception was already envisioned… that the Transfer Function could optionally be allowed to be different from the ST 2084 TF, as an additional test.) Tomohiro Ikai observed that as we vary the colour space instead of the usual Y′CbCr, a certain qp offset value may also have to be modified for optimal performance. So it was suggested to allow that adjustment. We reached a decision in the CE to allow for that variation, but as an additional test.

The suggestion was that if gain is shown, we might liaise with other organizations to determine their degree of interest in an alternative colour space.

It was asked whether the upsampling after decoding would need to be designed to match the downsampling.

It was suggested that we should be cautious about considering the idea of defining new colour spaces.

No immediate action is expected from the outcome of the informal viewing.Visual testing was suggested to be done. This was agreed, but to be lower priority than other visual testing for this meeting.

Informal viewing should be performed later during week to demonstrate the claimed benefit versus the new CE1 anchors; it would however not be possible to identify whether benefit would be due to filters or other colour transforms. Sampling filters could also be used with CE1 as optional pre-/postprocessing.

### CE5 primary contributions (3)

JCTVC-W0055 HDR CE5: Report of Experiment 5.3.2 [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

JCTVC-W0069 HDR CE5: Report on Core Experiment CE5.3.1 [R. Brondijk, W. de Haan, J. Stessen (Philips)]

JCTVC-W0099 HDR CE5 test 3: Constant Luminance results [J. Sole, D. Rusanovskyy, A. Ramasubramonian, D. Bugdayci, M. Karczewicz (Qualcomm)]

### CE5 cross checks (2)

JCTVC-W0070 CE5-related: xCheck of CE5.3.2 [R. Brondijk, R. Goris (Philips)] [miss/placeholder]

JCTVC-W0080 Crosscheck of HDR CE5.3.1 (JCTVC-W0069) [J. Samuelsson, M. Pettersson, J. Strom (Ericsson)] [late]

## HDR CE6: Non-normative post processing (10)

Initially discussed Saturday 1500 GJS & JRO.

### CE6 summary and general discussion (1)

JCTVC-W0026 Report of HDR Core Experiment 6 [R. Brondijk, S. Lasserre, D. Rusanovskyy, Y. He]

This was originally planned for processing that is performed, after decoding, in the 4:4:4 space. However, some proposal in the CE used output in the 4:2:0 space.

| CE Test | Contribution | Summary | Proponent | Cross Checker |
| --- | --- | --- | --- | --- |
| 4.1 | JCTVC-W0103 | Technology based on m37064: Study of reshaper in 4:4:4 Dynamic range adjustment with signalling of parameters for HDR reconstruction through SEI message Has concept of decoder-side content DR adaptation for the coding space, re-expanded to destination DR | Qualcomm | Philips JCTVC-W0109 BBC JCTVC-W0125 |
| 4.2 | JCTVC-W0034 | Colour enhancement (m37072): modifying decoded chroma (in 4:2:0 domain) based on luma signal No SDR concept | InterDigital | Samsung JCTVC-W0123 |
| 4.3 | JCTVC-W0063 | Dynamic range adaptation and re-saturation (m36266) in 4:2:0 Version 1: HM 16.7 fixed QP Version 2: Using CE 1 adaptive QP Has concept of decoder-side content DR adaptation for the coding space, re-expanded to destination DR | Philips | Technicolor (version 1) JCTVC-W0114 Qualcom (version 2) |
| 4.6a | JCTVC-W0063 | Technologies based upon m37285: m36266, using an SEI message described in m37285. Similar to 4.3, except signalling. Dynamic range adaptation, based on application of 1D-mapping functions to the YCbCr and/or RGB components, and colour correction consisting of the modulation of the CbCr and/or RGB components samples by a 1D scaling function depending on the Y component sample and signalled as presented in m37245. Has concept of decoder-side content DR adaptation for the coding space, re-expanded to destination DR | Philips | FastVDO JCTVC-W0073 |
| 4.6b | JCTVC-W0059 | Technologies based upon m37285: the test model, with usage of an SEI message. Has concept of decoder-side content DR adaptation for the coding space, re-expanded to destination DR | Technicolor | Philips JCTVC-W0064 |

The "concept of decoder-side content DR adaptation" is designed to be used for adapting the content for display with a different HDR display DR, different environment light level, user-adjusted light level, or SDR display DR.

The bitstream VUI could indicate an SDR space or some other space for default interpretation in the absence of adaptation metadata.

It was asked how this capability could be demonstrated – and the coordinator responded that the adaptation to a different display could be demonstrated using various displays.

SMPTE 2094 (see incoming SMPTE LS, m38082, VCEG-BAXX) is being developed to provide some display adaptation metadata. The nominal purpose of SMPTE 2094 is, at least primarily, to enable adaptation of a video signal to a smaller colour volume (e.g., lower dynamic range).

W0063 is different.

Revisit.

### CE6 primary contributions (4)

JCTVC-W0034 CE-6 test4.2: Colour enhancement [Y. He, L. Kerofsky, Y. Ye (InterDigital)]

JCTVC-W0059 HDR CE6: report of CE6-4.6b experiment (ETM using SEI message) [E. Francois, C. Chevance, Y. Olivier (Technicolor)]

JCTVC-W0063 HDR CE6: Core Experiments 4.3 and 4.6a: Description of the Philips CE system in 4:2:0 and with automatic reshaper parameter derivation [W. de Haan, R. Brondijk, R. Goris, R. van der Vleuten (Philips)]

JCTVC-W0103 HDR CE6: Test 4.1 Reshaper from m37064 [D. B. Sansli, A. K. Ramasubramonian, D. Rusanovskyy, J. Sole, M. Karczewicz (Qualcomm)]

### CE6 cross checks (5)

JCTVC-W0064 HDR CE6-related: Cross Check of CE6.46b [R. Brondijk, W. de Haan (Philips)] [miss/placeholder]

JCTVC-W0073 HDR CE6: Cross-check of 6.46a [W. Dai, M. Krishnan, P. Topiwala (FastVDO)] [late]

JCTVC-W0109 CE6-related: xCheck of CE6 4.1 [Robert Brondijk (Philips)] [miss] [late]

JCTVC-W0114 HDR CE6: crosscheck of CE6.4.3 (JCTVC-W0063) [C. Chenvance, Y. Olivier (Technicolor)] [late]

JCTVC-W0123 HDR CE6: cross-check report of test4.2: Colour enhancement (JCTVC-W0034) [E.Alshina (Samsung)] [late]

JCTVC-W0125 CE6-related: Cross check report for Test 4.1 Reshaper from m37064 [M. Naccari (BBC)] [late]

## HDR CE7: Hybrid Log Gamma investigation (4)

Initially discussed Saturday 1645 GJS & JRO.

### CE7 summary and general discussion (1)

JCTVC-W0027 Report of HDR/WCG CE 7 on investigating the visual quality of HLG generated HDR and SDR video [A. Luthra (Arris), E. Francois (Technicolor), L. van de Kerkhof (Philips)]

This document provides the report of the Core Experiment 7 investigating the visual quality of Hybrid Log-Gamma (HLG) generated HDR and SDR video.

The exploratory experiment 7 (CE7) was established at the 113th MPEG meeting (N15800) with the purpose of understanding various techniques used to generate video using Hybrid Log-Gamma (HLG) transfer function and investigating the visual quality of the video.

Hybrid Log-Gamma (HLG) Transfer Function (TF) is designed to possibly provide backward compatibility with the "legacy" SDR systems. At this stage, the "legacy" systems are taken to be the SDR systems not needing colour gamut conversion (i.e., for example, not needing BT.2020 to BT.709 colour gamut conversion) but needing only the dynamic range conversion. If the legacy SDR TV cannot process the BT.2020 container then it is assumed the BT.2020 container is removed externally and the video is provided with appropriate format. The details of the experiments that were conducted is provided in ISO/IEC JTC1/SC29/WG11 N15800.

The initial version of N15800 lacked specific details. Those details were provided and uploaded as the second version of N15800. Various issues associated with HLG were discussed on the email reflector as well as during the HDR/WCG AHG's face-to-face meeting in Vancouver in Jan 2016.

An overview of the motivation for development of HLG was given to the group by BBC during the Vancouver meeting and uploaded as ISO/IEC JTC1/SC29/WG11 m37535 (JCTVC-W0037). There was substantial discussion on the impact and interactions of various elements of the OOTF. The utility of modifying the system gamma to change the peak brightness was discussed. BBC reported on a subjective test where viewers modified the system gamma by a factor in order to adapt the signal from one peak brightness to another. The value is dependent on the source and target peak brightness values. It was observed that only 2–3% of TV programs go through a reference display environment and only one of 24 in-house studios surveyed by BBC followed BT.2035 lighting environment recommended practice.

BBC was requested to send ITU-R WP6C contribution on system gamma design for monitors to the MPEG/JCT HDR ad hoc group.

Arris presented a document during the Vancouver meeting on the backward compatibility of HLG. The document was uploaded to the MPEG and JCT repositories as ISO/IEC JTC1/SC29/WG11 m37543 (JCTVC-W0035) respectively. Arris presented images that showed CE7.1b looked noticeably washed out and suggested focusing on CE7.1a. CE7.1a uses gamma processing in the YUV domain and CE7.1b uses gamma processing in the RGB domain. It was also reported that there was a change in hue when the HLG processed video is shown on a legacy SDR TV. The amount of change in hue was observed to be a function of the brightness of the object. Its impact was that as an object moved from less bright area to brighter area, the displayed colour changed. It was further investigated. To be able to test it in more controllable environment, Arris generated colour bars that showed the similar behavior of change in the hue depending on the brightness of the object as seen in other video sequences. The EXR file consisting of those colour bars were also provided on the CE7 reflector. The results are reported in ISO/IEC JTC1/SC29/WG11 m37543 (JCTVC-W0035).

A demo for this will be arranged and announced on our email reflector (target 1100 Sunday if feasible).

Technicolor presented an informal document during the Vancouver meeting on the backward compatibility of HLG – specifically whether the compatibility is content dependent. The document was later completed and uploaded as document JCTVC-W0110. The test, done using BT.709 container, was similar to BBC’s where viewers were asked match SDR and HDR by adjusting the system gamma. Technicolor showed selected ranges from gamma 1.2 to 1.6. Technicolor reported that that the system gamma value for an optimal SDR rendering is content dependent, and not only dependent on the peak brightness and viewing environment. Bright content would require smaller gamma values (close to 1.2) while dark content would need large gamma values (above 1.4). In the discussions, BBC stated that gamma 1.6 is no longer the recommended value. The second round of test done using BT.2020 container confirmed the previous observations.

Philips presented a document (ISO/IEC JTC1/SC29/WG11 m37602) during the interim meeting describing their crosscheck of CE7.1 and CE7.2. The document confirmed the work of Technicolor and Arris. For 7.1.b, the SDRs look washed out and there are noticeable colour tone changes. The 7.1.a SDRs look better than those of 7.1.b, however their colours are often too saturated (reds are too red, greens too green and blues too blue). The compression results vary: for some sequences, 7.1.b has fewer issues and for others 7.1.a have fewer issues. At the lower gammas of CE 7.2 the oversaturation is less noticeable, and the errors in luminance and colour are smaller. Cross-checking of 7.2 experiments are provided by BBC in JCTVC-W0043.

It was also noted during the Vancouver meeting that the conversion from SDR 2020 to 709 could be achieved by HDRTools with a 2 step approach: conversion from SDR2020 to EXR2020, then conversion of EXR2020 to SDR709. Philips made a fix to enable direct conversion of SDR2020 to SDR709. There are still ongoing discussions on the reflector to clarify how the conversion from SDR2020 to SDR709 has to be applied. An input document (JCTVC-W0046) is provided as one of the suggested starting point, with the target of providing a recommendation by the end of the JCTVC meeting, for further visual quality evaluations of the SDR produced by the various studied solutions (see CE8 report for in-depth coverage of the topic of SDR backward compatiblity systems evaluation).

Contributions:

- Results report

  - JCTVC-W0061/m37695 - CE7: Results Core Experiment 7.1 test a and b, M. Naccari, A. Cotton, M. Pindoria

  - JCTVC-W0079/m37717 - HDR CE7-related: Additional results for Experiment 7.2a, M. Naccari, M. Pindoria, A. Cotton (BBC)

- Comments/Observations

  - JCTVC-W0035/m37543 - Some observations on visual quality of Hybrid Log-Gamma (HLG) TF processed video (CE7), A. Luthra, D. Baylon, K. Minoo, Y. Yu, Z. Gu (Arris)

  - JCTVC-W0110/m37771 - HDR CE7: Comments on visual quality and compression performance of HLG for SDR backward compatible HDR coding system, E. Francois, Y. Olivier, C. Chevance (Technicolor)

  - JCTVC-W0119/m37979 - Some considerations on hue shifts observed in HLG backward compatible video, M. Pindoria, M. Naccari, T. Borer, A. Cotton (BBC)

- Cross-checking

  - JCTVC-W0043/m37630 - CE7: Cross-check report for Experiment 7.2, M. Naccari, M. Pindoria (BBC)

  - JCTVC-W0067/m37702 - HDR CE7: xCheck 1a 1b and Core Experiment 2a and 2b, Robert Brondijk, Wiebe de Haan, Rene van der Vleuten, Rocco Goris

- Others

  - JCTVC-W0046 /m37680 - Recommended conversion process of YCbCr 4:2:0 10b SDR content from BT.709 to BT.2020 colour gamut, E. Francois, K. Minoo, R. van de Vleuten, A. Tourapis

Full presentation of all related documents may not be necessary.

### CE7 primary contributions (1)

JCTVC-W0061 CE7: Results Core Experiment 7.1 test a and b [M. Naccari, A. Cotton, M. Pindoria (BBC)]

### CE7 cross checks (2)

JCTVC-W0043 CE7: Cross-check report for Experiment 7.2 [M. Naccari, M. Pindoria (BBC)]

JCTVC-W0067 HDR CE7: xCheck 1a 1b and Core Experiment 2a and 2b [R. Brondijk, W. de Haan, R. van der Vleuten, R. Goris (Philips)]

## HDR CE8: Viewable SDR testing (1)

### CE8 summary and general discussion (1)

[miss]

Deferred.

### CE8 primary contributions (0)

### CE8 cross checks (0)

# Non-CE Technical Contributions (xx)

## SCC coding tools (2)

JCTVC-W0076 Comments on alignment of SCC text with multi-view and scalable [C. Gisquet, G. Laroche, P. Onno (Canon)]

Discussed Sunday 21st 1630 GJS.

At the 21st JCT-VC meeting in June 2015, disabling of weighted prediction parameters syntax for CPR was adopted, by checking whether a reference picture has the same POC as the current frame. It is asserted that the text specifications modifications have consequences outside of the SCC extension.

The problem was confirmed. A suggested fix in the discussion was to check for equality of the layer ID.

Revisit after offline drafting to check draft text for that.

JCTVC-W0077 Bug fix for DPB operations when current picture is a reference picture [X. Xu, S. Liu, S. Lei (MediaTek)]

Discussed Sunday 21st 1645 GJS.

In the current HEVC SCC specification, the current decoded picture prior to the loop filtering operations is used as a reference picture. This reference picture is also put and managed in the decoded picture buffer (DPB), together with other decoded pictures. An asserted bug fix is proposed in this contribution for the DPB operation such that before the start of decoding current picture, one empty picture buffer is created when TwoVersionsOfCurrDecPicFlag is equal to 0; alternatively, two empty picture buffers are created when TwoVersionsOfCurrDecPicFlag is equal to 1.

A participant said they believed the current text is not broken. Revisit after offline study of the issue.

HDR coding (35)

### CE1 related (9)

JCTVC-W0039 Luma delta QP adjustment based on video statistical information [J. Kim, J. Lee, E. Alshina, Y. Park (Samsung)]

Initially discussed Saturday 1800 GJS & JRO.

This contribution is targeting to performance improvement of high dynamic range video compression using existing HEVC tools. The contribution asserts that the performance can be improved by adjusting the luma QP based on the video statistical properties. Depending on the average of the luma components in a picture and the luma block average and variance, the proposed algorithm adjusts the delta QP for each CTU. The performance of the algorithm was evaluated with the same method of "SuperAnchor v3.2" and reportedly shows consistently better compression performance.

Additional criteria included the brightness of entire picture and the variance of luma at the CTU level. By this, in some cases the adjustment is disabled either at the frame or CTU level. It was claimed that some improvement in dark areas is visible relative to the new anchors of CE1. Several experts expressed the opinion that the idea is interesting.

It is also suggested by another expert that taking into account colour qp offset for dark areas would be another option to give improvement.It was asked what scope of techniques we should consider:

- This looks at whole-frame luma, which seems fine (note that CE2 also includes whole-frame analysis)

- It also looks at local variance, which was previously discouraged. It was remarked that the variance is only used to disable an adjustment that would otherwise be made based on a local average. It was also commented that the spirit is to compensate for what is done by the colour transfer function.

The presentation deck was requested to be uploaded.

We probably want to include this in a CE or go ahead and adopt it as the new anchor. Revisit to determine which. Viewing was also requested.

JCTVC-W0081 Crosscheck of Luma delta QP adjustment based on video statistical information (JCTVC-W0039) [J. Samuelsson, J. Ström, P. Hermansson (Ericsson)] [late]

JCTVC-W0054 AHG on HDR and WCG: Average Luma Controlled Adaptive dQP [A. Segall, J. Zhao (Sharp), J. Strom, M. Pettersson, K. Andersson (Ericsson)]

Initially discussed Saturday 1830 GJS & JRO.

This document describes the average luma-controlled adaptive dQP method that is part the anchor generating process since anchor v3.0, as outlined in JCTVC-W0021/m37605. The main idea behind the method has previously been described partly in m37439, but this document provides implementation details that have previously only been communicated over email and/or as part of the software packages associated with anchors v3.0, v3.1 and v3.2.

It was also commented that the spirit is to compensate for what is done by the colour transfer function (in the way that its effect differs from that for SDR).

Some possible modifications discussed:

- It was noted that lambda can also be used to provide some amount of intermediate points between QP values.

- It was noted that chroma is not considered in this technique.

- Different block sizes for the adaptation

The information about anchor encoding technique should be included in the CTC encoding algorithm adjustment, and perhaps in the draft TR.

JCTVC-W0056 CE1-related: LUT-based luma sample adjustment [C. Rosewarne, V. Kolesnikov (Canon)]

Discussed Saturday 1900 GJS & JRO

The ‘Common test conditions for HDR/WCG video coding experiments’ defines an HDR anchor (at version 3.2) that includes a method known as ‘luma sample adjustment’. This method compensates for a shift in luminance that occurs e.g. when downsampling from 4:4:4 to 4:2:0 using non-constant luminance and YCbCr. The method performs an iterative search to determine each luma sample values, which is asserted to be overly complex for real time implementations. This contribution shows a method for performing ‘luma sample adjustment’ that replaces an iterative search with a LUT-based approximation whereby a second-order model is applied to predict the adjusted luma sample, with the coefficients for the model obtained from the LUT. tPSNR values when no luma sample adjustment is applied are 52.2 dB, 62.6 dB and 43.3 dB in X, Y and Z channels, respectively. The corresponding values for the v3.2 anchor are 53.8 dB, 69.8 dB and 43.1 dB. With the LUT-based method, the values are 53.3 dB, 64.4 dB and 43.0 dB.

It was agreed to study this in an AHG on luma adjustment for chroma subsampling methods.

JCTVC-W0052 Enhanced Luma Adjustment Methods [A. M. Tourapis, Y. Su, D. Singer (Apple Inc)]

Discussed Saturday 1915 GJS & JRO

Changes are proposed for the luma adjustment method that is currently used for the creation of the anchor video material used in the JCT-VC HDR/WCG video standardization development process. These changes reportedly result in considerable speedups in the conversion process, with little if any impact in quality. Alternative implementations that try to emphasize other video quality aspects and not only luminance, are also presented.

The contribution also proposes changes to micrograding for XYZ and RGB accuracy optimization by multi-component optimization.

It was agreed to replace the anchor generation process with the bounded luma micrograding method using a LUT (10*106 elements).

It was agreed to study this in an AHG the RGB and XYZ aspects.

JCTVC-W0107 Closed form HDR 4:2:0 chroma subsampling (HDR CE1 and AHG5 related) [Andrey Norkin (Netflix)]

Discussed Saturday 2000 GJS & JRO

Two algorithms are proposed for removal of colour artifacts in saturated colours of HDR video that appear in non-constant luminance Y′CbCr 4:2:0 colour subsampling. Both proposed algorithms perform calculations in one step, which reportedly results in lower complexity compared to the luma micro-grading algorithm that is currently used for HDR anchor generation. Algorithm 1 is reported to produce higher average PSNR and DE1000 numbers than the current HDR anchor, wheres showing smaller numbers on tPSNR and L0100 measures. The performance of algorithm 2 across the studied metrics is claimed to resemble performance of the current HDR anchor. It is also claimed that both algorithms improve the subjective quality of the sequences with the described artifacts, whereas algorithm 1 avoids certain colour shifts possible in the anchor.

Regarding the subjective quality comparison, a participant remarked that viewing the result using tone mapping may cause artifacts that do not occur with viewing on an HDR monitor.

A participant suggested that since this message has a performance loss, it should be considered as a complexity reduction method to document and study but not to replace the anchor method with it.

It was also suggested that this could be used in an iterative method to speed it up.

It was agreed to study this in an AHG.

JCTVC-W0111 Cross-check of JCTVC-W0107: Closed form HDR 4:2:0 chroma subsampling [A. Tourapis (Apple)] [late]

JCTVC-W0051 Enhanced Filtering and Interpolation Methods for Video Signals [A. M. Tourapis, Y. Su, D. Singer (Apple Inc)]

Discussed Sunday 21st 0900 GJS & JRO

New adaptive filtering mechanisms for the 4:4:4 to 4:2:0 and 4:2:0 to 4:4:4 conversion processes are introduced in HDRTools. Two classes of filters are introduced. A parallel multi-filter scheme that tries to determine the “best” filter, given a criterion for each column/row/segment, and schemes that adaptively select the length of the filters based on local colour variations/edges.

Currently, the JCT-VC HDR/WCG activity employs relatively simplistic filtering mechanisms when converting 4:4:4 video signals to 4:2:0 and vice versa. In particular , a 3 tap linear filter, with coefficients [1 6 1]/8, is employed for the down-conversion process, while for up-conversion the Lanczos2 4 tap filter with coefficients [-1 9 9 -1]/16 is used. Both of these filters and the down-conversion and up-conversion processes, were applied on fixed precision data.

In this contribution, several adaptive filtering mechanisms are presented that can reportedly result in better performance, both objectively and subjectively, compared to the filters currently used by the JCT-VC HDR/WCG activity. The implementation of these filters is also combinable with other methods such as the luma adjustment method and its refinements.

Two schemes were described:

- Filtering optimization using parallel filters: Instead of utilizing a single filter for filtering an image, multiple filters could be used in parallel and the one determined as best, given a particular criterion, could be selected for filtering an area during down-conversion.

- Edge adaptive/colour variation adaptive filtering methods: This is done by performing a neighborhood analysis and, using predefined classification conditions, selecting the most appropriate filter for filtering this sample. HDRTools currently supports three different schemes for neighborhood analysis. In the first scheme, starting from the longest filter and for each filter that is eligible for selection, an analysis of the local neighborhood mean is performed for each sample to be filtered. The local neighborhood is defined based on the length of the filter. If the distance of any sample within this neighborhood from the local neighborhood mean exceeds a certain tolerance value, then this filter is disallowed for this sample, and the next filter is evaluated. At the last stage, and if all other earlier filters were disallowed, a 3 tap filter is considered instead.

A "min/max" filtering scheme is also supported in the HDRtools.

A number of other techniques could certainly be studied in further work.

The degree to which the effect is preserved in visual quality through the encoding-decoding process was discussed, and is not necessarily clear.

It was agreed to study this in an AHG.

JCTVC-W0066 CE1-related: Optimization of HEVC Main 10 coding in HDR video Based on Disorderly Concealment Effect [C. Jung, S. Yu, Q. Lin (Xidian Univ.), M. Li, P. Wu (ZTE)]

Discussed Sunday 21st 0945 GJS & JRO

Proposed is an optimization of coding HDR video based on the "disorderly concealment effect" using an HEVC encoder. Perceptual block merging is implemented on HEVC Main 10 Profile-based HDR video coding. Firstly, luminance adaptation (LA) for quantized HDR video is calculated based on the Barten contrast sensitivity function (CSF) model. Then a "free-energy"-based just-noticeable-difference (FEJND) map is derived to represent the disorderly concealment effect for each picture. Finally, a perceptual Lagrange multiplier is derived using the average FEJND values of the coding unit (CU) and picture. Experimental results reportedly demonstrate that the proposed method achieves average bit-rate reductions of 5.23% in tPSNR XYZ and 8.02% in PSNR_DE over Anchor, while producing block partitioning merging results correlated with human visual perception.

The method is compared to HM 16.2.

The technique has also been applied to SDR as well.

The encoder runtime effect is approximately 2–3x, and thus substantial.

This is viewed as a general encoding technique that could be applied in various contexts, not necessarily related to the techniques.

The presented results were for an LD configuration; study on RA seemed more desirable.

No immediate action was requested on this, and further study was encouraged.

### CE2 related (9)

JCTVC-W0040 Adaptive Gamut Expansion for Chroma Components [A. Dsouza, A. Aishwarya, K. Pachauri (Samsung)]

Discussed Sunday 21st 1245–1300 GJS & JRO.

This proposal describes a method for content adaptive gamut expansion of chroma components before quantization in order to minimize the noise. The document also proposes the addition of an additional SEI message to be used for post processing. Objective gains of 14% at scaling factor of 1.4 in deltaE BD-Rate is reported for the HDR anchor sequences.

Unclear why this is not equivalent with adjusting chroma QP (since the scaling is linear, this should be the case), and why additional signaling & scaling steps would be needed.

Could also be seen as a simplified (chroma only) reshaper.

The contribution raises the question of, if a reshaper is used, how complicated of a reshaping function should be supported (number of segments, nonlinearity, etc.) It was agreed that this should be studied in any further reshaping study e.g., in CE.

JCTVC-W0068 CE2-related: Adaptive Quantization-Based HDR video Coding with HEVC Main 10 Profile [C. Jung, Q. Fu, G. Yang (Xidian Univ.), M. Li, P. Wu (ZTE)]

TBP.

JCTVC-W0071 CE2-related : Adaptive PQ: Adaptive Perceptual Quantizer for HDR video Coding with HEVC Main 10 Profile [C. Jung, S. Yu, P. Ke (Xidian Univ.), M. Li, P. Wu (ZTE)]

TBP.

JCTVC-W0085 HDR CE2 related: Further Improvement of JCTVC-W0084 [T. Lu, F. Pu, P. Yin, T. Chen, W. Husak (Dolby), Y. He, L. Kerofsky, Y. Ye (InterDigital)] [late]

See summary in section 5.2.1. Further discussed Sunday 21st 1010 (GJS & JRO). See notes under W0084. Further study in CE.

JCTVC-W0120 Cross-check of JCTVC-W0085 [J. Lee, J. W. Kang, D. Jun, H. Ko (ETRI)] [late]

JCTVC-W0130 Crosscheck for further improvement of HDR CE2 (JCTVC-W0085) [Y. Wang, W. Zhang, Y. Chiu (Intel)] [miss] [late]

JCTVC-W0089 HDR CE2-related: some experiments on reshaping with input SDR [Y. Olivier, E. Francois, C. Chevance (Technicolor)] [late]

Targets backward comp.

JCTVC-W0092 Description of the Exploratory Test Model (ETM) for HDR/WCG extension of HEVC [K. Minoo (Arris), T. Lu, P. Yin (Dolby), L. Kerofsky (InterDigital), D. Rusanovskyy (Qualcomm), E. François (Technicolor)]

Description of ETMr0, no need for detailed presentation.

JCTVC-W0100 HDR CE2-related: Results for combination of CE1 (anchor 3.2) and CE2 [J. Sole, A. K. Ramasubramonian, D. Rusanovskyy, D. Bugdayci, M. Karczewicz (Qualcomm)]

TBP.

### CE3 related (1)

JCTVC-W0041 HDR-VQM Reference Code and its Usage [K Pachauri, S Sahota (Samsung)]

### CE4 related (0)

### CE5 related (0)

### CE6 related (1)

JCTVC-W0060 Re-shaper syntax extension for HDR CE6 [H. M. Oh, J.-Y. Suh (LGE)]

### CE7 related (6)

JCTVC-W0045 Hybrid Log Gamma observations [C. Fogg (Movielabs)] [late]

JCTVC-W0079 HDR CE7-related: Additional results for Experiment 7.2a [M. Naccari, M. Pindoria, A. Cotton (BBC)]

JCTVC-W0088 SDR backward compatibility requirements for HEVC HDR extension [L. van de Kerkhof, W. de Haan (Philips), E. Francois (Technicolor)]

JCTVC-W0110 HDR CE7: Comments on visual quality and compression performance of HLG for SDR backward compatible HDR coding system [E. Francois, Y. Olivier, C. Chevance (Technicolor)] [late]

JCTVC-W0106 Evaluation of Backward-compatible HDR Transmission Pipelines [M. Azimi, R. Boitard, M. T. Pourazad, P. Nasiopoulos (UBC)] [miss] [late]

JCTVC-W0119 Some considerations on hue shifts observed in HLG backward compatible video [M. Pindoria, M. Naccari, T. Borer, A. Cotton (BBC)] [late]

### CE8 related (0)

### Other (9)

JCTVC-W0047 ICtCp testing [C. Fogg (Movielabs)]

JCTVC-W0048 HDR-10 status update [C. Fogg (Movielabs)]

JCTVC-W0049 Best practices for HDR10 [C. Fogg (Movielabs)] [miss] [late]

JCTVC-W0050 Noise reduction for HDR video [C. Fogg (Movielabs)] [miss] [late]

JCTVC-W0072 Highly efficient HDR video compression [A. Chalmers, J. Hatchett, T. B. Rogers, K. Debattista (Univ. Warwick)] [late]

JCTVC-W0074 CIECAM02 based Orthogonal Colour Space Transformation for HDR/WCG Video [Y. Kwak, Y. Baek (UNIST), J. Lee, J. W. Kang, H.-Y. Kim (ETRI)]

(Presenter to be available Tues & Wed.)

JCTVC-W0087 Description of Colour Graded SDR content for HDR/WCG Test Sequences [P. J. Warren, S. M. Ruggieri, W. Husak, T. Lu, P. Yin, F. Pu (Dolby)]

JCTVC-W0105 Evaluation of Chroma Subsampling for High Dynamic Range Video Compression [R. Boitard, M. T. Pourazad, P. Nasiopoulos (UBC)] [late]

JCTVC-W0126 Preliminary study of filtering performance in HDR10 pipeline [K Pachauri, Aishwarya (??)] [late]

## HL syntax (0)

No contributions on general matters of high-level syntax were specifically noted, although various contributions include consideration of high-level syntax issues relating to specific features.

## SEI and VUI (5)

JCTVC-W0044 BT.HDR and its implications for VUI [C. Fogg (Movielabs)] [more authors]

JCTVC-W0057 Content colour gamut SEI message [H. M. Oh, J. Choi, J.-Y. Suh]

JCTVC-W0058 Video usability information signaling for SDR backward compatibility [H. M. Oh, J.-Y. Suh (LGE)]

JCTVC-W0086 Indication of SMPTE 2094-10 metadata in HEVC [R. Yeung, S. Qu, P. Yin, T. Lu, T. Chen, W. Husak (Dolby)]

JCTVC-W0098 Effective Colour Volume SEI [A. Tourapis, Y. Su, D. Singer (Apple Inc.)]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (9)

### General (3)

JCTVC-W0062 Non-normative HM encoder improvements [K. Andersson, P. Wennersten, R. Sjöberg, J. Samuelsson, J. Ström, P. Hermansson, M. Pettersson (Ericsson)]

Presented on Sunday 21st at 1850 (Rajan).

This contribution reports that a fix to the misalignment between QP and lambda improves the BD rate for luma by 1.8% on average for RA and 1.7% for LD using the common test conditions. The fix in combination with extension to a hierarchy of length 16 for random access is reported to improve the BD rate for luma by 7.1% on average using the common test conditions. To verify that a longer hierarchy does not decrease the performance for difficult to encode content, 4 difficult sequences were also tested. An average improvement in luma BD rate of 4.8% is reported for this additional test set. Results for HDR content is also presented. It is also reported that subjective quality improvements have been seen by the authors. The contribution proposes that both the fix to the misalignment between QP and lambda and the extension to a hierarchy of 16 pictures are included in the reference software for HEVC and used in the common test conditions. Software is provided in the contribution.

A subjective viewing using Samsung TV is planned at this meeting. A related document is JVET B0039.

Proposed to adjust the alignment between lambda and QP. Would be an encoder only change.

Decision (SW): Adopt the QP and lambda alignment to the HM encoder SW.

Proposed increase in the hierarchy would require a larger DPB size than HEVC if the resolution was the max for the level. Will add a very long delay. Encoders might not actually use the larger hierarchy, so this might not represent expected real world conditions.

Proponent mentioned that the GOP size could be extended to 32 providing higher gains. Appropriate intra period adjustment would be necessary for both GOP size of 16 and 32.

Revisit this aspect.

JCTVC-W0117 Cross-check of Non-normative HM encoder improvements (JCTVC-W0062) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-W0038 HEVC encoder optimization [Y. He, Y. Ye, L. Kerofsky (InterDigital)]

Presented on Sunday 21st at 1900 (Rajan).

This proposal proposes two non-normative encoder optimization methods for HEVC encoders: deblocking filter parameter selection, and chroma quantization parameter adjustment. Compared to HEVC HM-16.7 anchor, average luma BD rate saving of the deblocking filter parameter selection method is reported to be0.2% for AI, 0.4% for RA, 0.3% for LDB, and 0.5% for LDP. Average chroma BD rate saving of the chroma QP offset adjustment method is reported to be 11.6% for RA, 14.2% for LDB, and 13.9% for LDP, with a small loss for the luma component.

These two optimization methods are also applicable to HDR/WCG coding. It is asserted that on SIM2 display, compared to the HDR/WCG anchors, quality improvements can be observed for low to medium bit rates for a range of HDR video content, mainly in the form of more details and fewer blocking artifacts.

For the 2nd aspect: For setting 1, there were some losses for luma (0.8% - 0.9%) and large improvements for chroma. A participant commented that for setting 1, you could shift some bits from luma to chroma to get such results. The proponent commented that setting 2 had no loss for luma and > 3% gains for chroma. Another participant asked whether similar gains would be possible by redistributing bits by changing the QP hierarchy. This may not be possible at this granularity.

Another participant asked whether the methods were for subjective quality improvement. The proponent commented that with aspect 1 they observed some blocking artifact improvement at low bit-rates.

Informal subjective study was encouraged to verify that there were subjective gains (or no artifacts).

Revisit.

### SCC (4)

JCTVC-W0075 Palette encoder improvements for the 4:2:0 chroma format and lossless [C. Gisquet, G. Laroche, P. Onno (Canon)]

Discussed Sunday 1615 GJS.

At the 22nd JCT-VC meeting in October 2015, the proponents' non-normative modification for palette lossy coding of non-4:4:4 content was adopted. It was asserted then that the fact superfluous chroma samples are coded was not properly taken into account, resulting in suboptimal coding. It is asserted that similar considerations affect lossless encoding. In addition, it is proposed that the encoder tracks whether an entry is only associated with pixels with superfluous chroma samples. Finally, it is asserted that various palette evaluations need not be run, the removal of which is asserted to result in encoder runtime improvements. It is reported that the non-normative changes provide, for 4:2:0 content, up to 1.1%, 0.5% and 0.5% BDR gains, for runtimes of 90%, 94% and 95%, for respectively the AI, RA and LDB lossless configurations.

Substantial speed-up was shown, and some coding efficiency improvement for lossess.

Software impact? One main function is modified, perhaps 50 lines modified, and the proponent said the impact was not so large.

Decision (SW): Adopt.

JCTVC-W0127 Cross-check of JCTVC-W0075 on palette encoder improvements for lossless mode [V. Seregin (Qualcomm)] [miss] [late]

No problem was identified by the cross-checker.

JCTVC-W0078 Bottom-up hash value calculation and validity check for SCC [W. Xiao (Xidian Univ.), B. Li, J. Xu (Microsoft)]

Presented on Sunday 21st at 1750 (Rajan).

This document presents a bottom-up hash value calculation and validity check method in hash table generation, where large blocks’ results can reuse small blocks’. Such a design can reduce the redundant computation and then decrease encoding time in SCM. Experimental results show that the encoding time is reduced by 7% and 12% on average for RA and LB, respectively while the BD-rate performance almost same as the anchor.

The hash calculation was changed. Now the higher block size hash value can be derived from the lower block size hash values. This results in the 2/3 reduction in computations. For validity testing, no change in the formula but hierarchical implementation.

It was asked whether the breakdown of the run-time savings was know between has calculation and validity testing. That was not tested separately.

Currently has is applied only to 2Nx2N. A participant commented that this could be extended to asymmetric motion partitions.

A participant asked whether the hash used in current picture reference could be improved in a similar manner. The hash functions for current picture reference and inter prediction are different. It is not clear whether this could be extended to current picture reference hash. The presenter commented that they may investigate this further.

The software patch has been released in a revision. Crosschecker was not present.

Decision (SW): Adopt.

JCTVC-W0118 Crosscheck for bottom-up hash value calculation and validity check for SCC (JCTVC-W0078) [W. Zhang (Intel)] [late]

JCTVC-W0042 SCC encoder improvement [Y.-J. Chang, P.-H. Lin, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

Presented on Sunday 21st at 1815 (Rajan).

To reduce the SCM encoding time in 444 chroma format, JCTVC-U0095 proposed to disallow the TU splitting processes for the intra prediction mode enabling adaptive colour transform when the CU sizes are 64x64 and 32x32. The proposed method was adopted into SCM anchor in the last Geneva meeting for substantial encoder improvement in 444 chroma format. However, the TU splitting processes for the intra prediction mode in non-444 chroma formats are also a huge computational cost. This contribution proposes to apply the same method which disallows TU splitting processes to the intra prediction mode in non-444 chroma formats. Compared to the SCM6.0 anchor, it is reported that the proposed encoder can save 3% encoding time in average with 0.1 gain on the AI-lossy conditions in 420 chroma format. This contribution also re-tests the performance of the adopted method in JCTVC-U0095 when it is applied to different sizes of CUs for the intra prediction mode enabling adaptive colour transform in 444 chroma format.

The speedup is applied only to non-4:4:4 chroma formats. If CU size is 32x32 or higher, checking of RQT is disabled (no TU split). A participant commented if this method is adopted, SCM would behave differently than HM for 4:2:0 case when all the SCC and RExt tools are disabled, which was thought to be undesirable. Another participant commented that this method would need to be tested on variety of camera-captured content.

Supplemental results were presented for method in U0095 for other block size (16x16 and 8x8). There were some losses 0.1% for 16x16 for some classes, 0.2% for 8x8 for some classes. The encoding time reduction was 1-3% for 16x16 and ~5% for 8x8.

The proponent suggested that a config parameter be added to control the block size at which the U0095 method is applied.

No action.

JCTVC-W0116 Cross-check of SCC encoder improvement (JCTVC-W0042) [B. Li, J. Xu (Microsoft)] [late]

### HDR (3)

JCTVC-W0046 Recommended conversion process of YCbCr 4:2:0 10b SDR content from BT.709 to BT.2020 colour gamut [E. Francois, K. Minoo, R. van de Vleuten, A. Tourapis]

JCTVC-W0053 HDRTools: Extensions and Improvements [A. M. Tourapis, Y. Su, D. Singer (Apple Inc), C. Fogg (Movielabs)]

JCTVC-W0115 VIF Code for HDR Tools [H. Tohidypour, M. Azimi, M. T. Pourazad, P. Nasiopoulos (UBC)] [late]

# Withdrawn, unclear allocation (5)

JCTVC-W0036 Withdrawn

JCTVC-W0065 Withdrawn

JCTVC-W0082 Withdrawn

JCTVC-W0102 Withdrawn

JCTVC-W0124 Withdrawn

# Plenary Discussions, Joint Meetings, BoG Reports, and Summary of Actions Taken

## General

Topics for general discussion at the plenary level:

- …

## Project development

Joint meetings are discussed in this section of this report.

## BoGs

There were no break-out groups established at this meeting, and hence no BoG reports were submitted.

## List of actions taken affecting the draft HEVC specification

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the HEVC specification. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

- …

# Project planning

## WD drafting and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the XXday of the week preceding the meeting (xx May 2016).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments have been planned. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-L1100.

The general timeline agreed for CEs of this meeting was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

A deadline of four weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-S11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Other than the addition of a Chinese text editing sequence to the TGM category of the CTC (see the notes for JCTVC-U0150 (later uploaded as JCTVC-U0188) and the AHG report JCTVC-U0010), no particular changes were noted w.r.t. the prior CTC. update?

## Software development (update)

Software coordinators were asked to work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 16.6 available prior to the meeting.

- SCM 5.0 (based on HM 16.6 or newer) should be available within 3 weeks after the meeting.

- SHM 10.x U1013 (DAM, based on HM 16.2 or newer) should be available within 5 weeks after the meeting.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-V1002 HEVC Test Model 16 (HM 16) Update 4 of Encoder Description - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 16.x software versions and the reference configuration encodings according to JCTVC-L1100 and JCTVC-P1006 common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), K. Sharman (vice-chair) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Prepare and deliver the JCTVC-V1008 SHVC conformance draft 4 specification. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt, SHVC, and SCC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, R. Joshi, K. Kazui, A. K. Ramasubramonian, W. Wan, Y. Ye (vice-chairs) | N |
| SHVC verification testing (AHG5) (jct-vc@lists.rwth-aachen.de) - Edit the final SHVC verification test plan description. - Make logistic arrangements and conduct the SHVC verification test. - Prepare and submit a preliminary report of the test for review and finalization. | V. Baroncini, Y.-K. Wang, Y. Ye (co-chairs) | N |
| SCC coding performance analysis (AHG6) (jct-vc@lists.rwth-aachen.de) - Study test conditions and coding performance analysis methods for SCC coding performance. - Analyze the coding performance of the draft SCC coding features | H. Yu (chair), R. Cohen, A. Duenas, X. Xu, K. Rapaka, J. Xu (vice-chairs) | N |
| SCC extensions text editing (AHG7) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions draft 5 and test model 6 text. - Gather and address comments for refinement of the test model text. - Coordinate with AHG8 to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, G. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SCM software and its distribution. - Prepare and deliver HM 16.x-SCM-6.0 software version and the reference configuration encodings according to JCTVC-U1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu (vice-chairs) | N |
| Complexity of SCC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Analyze complexity characteristics of SCC coding methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, parallelism, pixel processing, chroma position interpolation, and other aspects of complexity as appropriate. - Quantify and compare the average and worse case throughput (context-coded as well as bypass bins) for SCC coding methods. - Study latency and parallelism implications of SCC coding techniques, considering multicore and single-core architectures. - Identify criteria to determine the hardware implementability of the key hardware modules. - Identify bottlenecks in the current design with regard to implementation complexity. | A. Duenas (chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (vice-chairs) | N |
| Test sequence material (AHG10) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 regarding interlaced video and range extensions development, and AHG6 regarding screen content coding. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger, H. Yu (vice-chairs) | N |
| SHVC test model editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-V1007 SHVC Test Model 11 (SHM 11) text. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), G. Barroux, J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 11.x software (based on HM 16.x) for experimentation. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, G. Barroux (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JCTVC-V1000 Meeting Report of the 22nd JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2016-01-31] (near next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

Draft TR

JCTVC-V1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 4 of Encoder Description [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 15777) [2016-01-31] (near next meeting)

No document JCTVC-V1003 (formerly RExt & I/L verif testing)

JCTVC-V1004 SHVC verification test plan [Y. Ye, V. Baroncini, Y.-K. Wang (editors)] (WG 11 N 15782) [2016-01-08]

JCTVC-V1005 HEVC Screen Content Coding Draft Text 5 [R. Joshi, S. Liu, G. J. Sullivan, Y.-K. Wang, J. Xu, Y. Ye (editors)] (WG 11 N 15776 Study of ISO/IEC DIS 23008-2:201X 3rd Edition) [2015-12-11] (7 weeks)

Basic elements (no changes of features):

- IBC

- Adaptive colour transform

- Palette mode

- Adaptive MV resolution

- Intra boundary filtering disabling

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)]

JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778) [2016-01-31] (near next meeting)

JCTVC-V1008 SHVC Conformance Testing Draft 4 [J. Boyce, A. K. Ramasubramonian (editors)] (WG 11 N 15789 Study of ISO/IEC 23008-8/DAM 3) [2015-12-11] (7 weeks)

Remains valid – not updated JCTVC-Q1009 Common SHM Test Conditions and Software Reference Configurations [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-V1011 Reference Software for HEVC Format Range Extensions Draft 4 [K. Sharman, D. Flynn, K. Sühring, T. Suzuki (editors)] (WG 11 N 15784 ISO/IEC 23008-5/FDAM1) [2015-11-18] (4 weeks)

Remains valid – not updated: JCTVC-U1012 Conformance Testing for Improved HEVC Version 1 Testing and Format Range Extensions Profiles Draft 5 (WG 11 N 15449 ISO/IEC 23008-8/DAM2) [T. Suzuki, K. Kazui (editors)]

JCTVC-V1013 Reference software for Scalable HEVC (SHVC) Extensions Draft 3 (WG 11 N 15787 Study of ISO/IEC 23008-5/DAM3) [Y. He, V. Seregin (editors)] [2015-12-11] (7 weeks)

JCTVC-V1014 Screen Content Coding Test Model 6 Encoder Description (SCM 6) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 15779) [2016-01-31] (near next meeting)

Remains valid – not updated: JCTVC-U1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)]

Remains valid – not updated: JCTVC-L1100 Common Test Conditions and Software Reference Configurations for HM [F. Bossen (editor)]

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Thu. 26 May – Wed. 1 June 2016, 24th meeting under ITU-T auspices in Geneva, CH.

- Fri. 14 – Fri. 21 Oct. 2016, 25th meeting under WG 11 auspices in Chengdu, CN.

- …

The agreed document deadline for the 24th JCT-VC meeting is xxday xx May 2016. Plans for scheduling of agenda items within that meeting remain TBA.

Companies expressing a likely willingness to contribute to SHVC verification test funding (to some degree TBD):

- InterDigital

- Microsoft

- Qualcomm

(more such contributors are encouraged)

ITU was thanked for the excellent hosting of the 22nd meeting of the JCT-VC. [appreciation to those assisting with equipment.]

The JCT-VC meeting was closed at approximately 1100 hours on Wed. 21 Oct 2015.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the twenty-second meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately XX people in total), were as follows:

- …

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.