| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 24th Meeting: Geneva, CH, 26 May – 1 June 2016 | Document: JCTVC-X_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the 24th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Geneva, CH, 26 May – 1 June 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-fourth meeting during 26 May – 01 June 2016 at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 1000 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 01 June 2016. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-third JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 5;

- For the format range extensions (RExt), conformance testing draft 6 (including improved HEVC version 1 testing);

- For the scalable extensions (SHVC), the SHVC reference software draft 4, conformance testing draft 5, and a verification test report;

- For the HEVC screen content coding (SCC) extensions, a draft text for version 4 of HEVC including draft text 6 of the text of SCC extensions, the HEVC screen content coding test model 7 (SCM 7), reference software draft 1, and conformance testing draft 1.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 1, a technical report text of conversion and coding practices for HDR/WCG video draft 1, a verification test plan for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

The other most important goals were to review the work on High Dynamic Range (HDR) video coding, and review other technical input documents. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC and Screen Content Coding) was also a significant goal. Preparation of SCC verification tests was started, and possible needs for corrections to the prior HEVC specification text were also considered.

The JCT-VC produced XX particularly important output documents from the meeting (update):

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 5;

- For the format range extensions (RExt), conformance testing draft 6 (including improved HEVC version 1 testing);

- For the scalable extensions (SHVC), the SHVC reference software draft 4, conformance testing draft 5, and a verification test report;

- For the HEVC screen content coding (SCC) extensions, a draft text for version 4 of HEVC including draft text 6 of the text of SCC extensions, the HEVC screen content coding test model 7 (SCM 7), reference software draft 1, and conformance testing draft 1.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 1, a technical report text of conversion and coding practices for HDR/WCG video draft 1, a verification test plan for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for Fri. 14 – Fri. 21 Oct. 2016 under WG 11 auspices in Chengdu, CN, during Thu. 12 – Wed. 18 Jan. 2017 under ITU-T auspices in Geneva, CH, during Fri. 31 Mar. – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU, and during ….

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-fourth meeting during 26 May – 01 June 2016 at the at the ITU-T premises in Geneva, CH. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 1000 hours on Thursday 26 May 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Wednesday 1 June 2016. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

- 22nd "V" meeting (Geneva, 2015-10)		155 people, 75 input documents

- 23rd "W" meeting (San Diego, 2016-02)		159 people, 125 input documents

- 24th "X" meeting (Geneva, 2016-05/06)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2016_05_X_Geneva/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-third JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 5;

- For the format range extensions (RExt), conformance testing draft 6 (including improved HEVC version 1 testing);

- For the scalable extensions (SHVC), the SHVC reference software draft 4, conformance testing draft 5, and a verification test report;

- For the HEVC screen content coding (SCC) extensions, a draft text for version 4 of HEVC including draft text 6 of the text of SCC extensions, the HEVC screen content coding test model 7 (SCM 7), reference software draft 1, and conformance testing draft 1.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 1, a technical report text of conversion and coding practices for HDR/WCG video draft 1, a verification test plan for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

The other most important goals were to review the work on High Dynamic Range (HDR) video coding, and review other technical input documents. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC and Screen Content Coding) was also a significant goal. Preparation of SCC verification tests was started, and possible needs for corrections to the prior HEVC specification text were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 16 May 2016.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 17 May 2016 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-X0068 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). No break-out activity reports were generated during this meeting.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-X00XX (a proposal of …) [uploaded 05-XX]

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JCTVC-X00XX (a proposal of …) [uploaded 05-XX]

- …

The following other documents were registered on time but were uploaded late:

- JCTVC-X00XX (an information document on …) [uploaded 05-XX]

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-X00XX [uploaded 05-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-X0031, JCTVC-X0032, …

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents (both crosscheck reports) were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline: JCTVC-X00XX [improved on 05-XX], … .

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload, along with a record of uploading times.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-W1000, the improved HEVC Test Model 16 (HM16) JCTVC-W1002, the RExt and improved version 1 Conformance Testing Draft 6 (merged into a new edition) JCTVC-W1012, the SHVC Conformance Testing Draft 5 JCTVC-W1008, the SHVC Reference Software Draft 4 JCTVC-W1013, the SHVC Verification Test Report JCTVC-W1004, the Screen Content Coding (SCC) Draft Text 6 JCTVC-W1005 (integrated into a new edition of HEVC, version 4), the SCC Reference Software Draft 1 JCTVC-W1011, and the SCC test model 7 JCTVC-W1014, the SCC Conformance Testing Draft 1 JCTVC-W1016, and the document Conversion and Coding Practices for HDR/WCG Video JCTVC-W1017, were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group was initially asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable basic description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities (as applicable)

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages, and subscribers must respond adequately to basic inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the JCT-VC email reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the headers and registrations of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: May refer either to block copy (see CPR or IBC) or backward compatibility. In the case of backward compatibility, this often refers to what is more formally called forward compatibility.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (MV used for intra BC prediction, not a term used in the standard).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE).

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector sometimes called a block vector, in a manner basically the same as motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element of AVC or HEVC).

- EL: Enhancement layer.

- ET: Encoding time.

- ETM: Experimental test model (design and software used for prior HDR/WCG coding experiments in MPEG).

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (esp. 8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (esp. 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment.

- SCCE: Screen content core experiment.

- SCM: Screen coding model.

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a term for a draft standard, especially one prior to its first ballot in the ISO/IEC approval process, although the term is sometimes used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Liaison activity

The JCT-VC did not directly send or receive formal liaison communications at this meeting. However, the following information had been conveyed in liaison communication at the parent-body level:

…

## Opening remarks

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

Primary topic areas were noted as follows:

- Screen content coding

  - Software

  - Conformance

  - Verification testing

- HDR

  - Verification testing

  - Development of TR

  - ICTCP support

  - Possible other SEI & VUI

  - Possible work on backward-compatibility (pending requirements)

  - Reference software

- Corrigenda items for version 4

- SHVC proposed additional verification testing

- Frame repetition signalling

- Test model texts and software manuals

- Common test conditions for coding efficiency experiments (hasn't been under active development here recently; being considered more elsewhere)

Status of deliverables:

- SCC text

- SCC reference software

- RExt & v1 conformance

- Corresponding WG 11 outputs

- Check meeting report

Key deliverables initially planned from this meeting:

- SCC specification Draft 6 (FDIS)

- FDAM for RExt conformance (draft 6)

- Possible additional verification test plan for SHVC

- SHVC software Draft 4 (FDAM)

- SHVC conformance Draft 5 (FDAM)

- SCC Reference software [Consent]

- SCC Conformance

- SCC verification testing plan (postponed)

- HDR outputs

  - Suggested practices draft

  - HDR verification test report

  - CEs?

  - ICTCP support

- New HM, SHM, SCM document versions

A single meeting track was followed for most meeting discussions.

## Scheduling of discussions

Scheduling: Generally, meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on Screen Content Coding CE work and related contributions during the first few days. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Thu. 26 May, 1st day

  - 1000–1300, Opening remarks, status review, AHG report review for AHGs 1–3 and 5–8 (GJS & JRO)

  - 1530–1700, AHGs 9, 12, 13, 14 (GJS)

  - 1700–1800 X0033 HDR/WCG Verification test (GJS)

- Fri. 27 May, 2nd day

  - 0900–1045 Best practices for HDR coding (section 5.1.1) (JRO)

  - 1115–1300 X0033 HDR/WCG Verification test (GJS)

  - 1400 BoG?

  - 1615 Continuation (GJS)

- Sat. 28 May, 3rd day

  - 0900–1xxxx

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized as follows. Some plenary sessions were chaired by both co-chairmen, and others by only one. Chairing of other discussions is noted for particular topics.

- AHG reports (14) (section 2)

- Project development status (7) (section 3)

- Core experiments (0) (section 4)

- HDR coding (34) (section 5.1)

  - Best practices (4)

  - CRI usage (4)

  - Luma sample adjustment (3)

  - ICtCp (4)

  - SDR backward compatibility (6)

  - Sampling/subsampling (5)

  - Other (2)

- High-level syntax (1) (section 5.2)

- VUI and SEI messages (7) (section )

- Non-normative, encoder optimization (1) (section 5.4)

- Withdrawn (4) (section 6)

- Plenary discussions (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 8, 9, and 11)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

## Topics discussed in final wrap-up at the end of the meeting

Notes on potential remainders near the end of the meeting:

- …

- Output preparations (see section 11 for full list)

  - …

- Plans

  - AHGs

  - CEs

  - Reflectors (jct-vc) & sites (test sequence location to be listed in CTC doc) to be used in future work

  - Meeting dates (Thu - Wed)

  - Doc deadline (Mon 10 days prior)

There were no requests to present any "TBP" contributions in the closing plenary.

# AHG reports (14)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Thursday 26th, 10:00–14:00, except as noted.)

JCTVC-X0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

[Add notes]

JCTVC-X0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne, M. Naccari, J.-R. Ohm, K. Sharman, G. Sullivan, Y.-K. Wang]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 23rd meeting in San Diego, USA (February 2016) and the 24th meeting in Geneva, CH (May/June 2016).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The ‘High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 5 of Encoder Description’ was published as JCTVC-W1002. This document represented a refinement of the previous HM16 Update 4 of the Encoder Description document (JCTVC-V1002). The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Review the list of bug fixes collected for HEVC Edition 3, and include all confirmed bug fixes, including the outcome of the above items, if any, into a JCT-VC output document for the purpose of HEVC Edition 3 defect reporting.

-

JCTVC-X0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring, K. Sharman]

This report summarizes the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 23rd and 24th JCT-VC meetings. Activities focused on integration of software adoptions and software maintenance, i.e. code tidying and fixing bugs.

A brief summary of activities related to each mandate is given below.

- The two proposals that were adopted at the last meeting (JCTVC-W0038 and JCTVC-W0062 [lambda/QP aspect only]), and one proposal that was adopted at the 21st meeting (JCTVC-U0128) were added to the HM development code base.

- In addition, some minor bug fixes and cleanups were addressed. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector, and http://hevc.info has been updated.

- Version 16.10 is due to be released during the meeting.

- There are a number of reported software bugs that should be fixed.

Versions developed

HM16.8 was tagged on 23 February during the previous meeting. The changes can be found in JCTVC-W0003.

HM16.9 was tagged on 21 March. It included the following modifications:

- JCTVC-W0038 “HEVC encoder optimization” – in particular the addition of chroma QP offsets to GOP table and an adaptive deblocking filter method.

- JCTVC-W0062 “Non-normative HM encoder improvements”. This tool is currently disabled by default (see below).

The addition of functions that have been added to JVET’s JEM software model, to help with benchmarking. The functions are SAOResetEncoderStateAfterIRAP and TemporalSubsampleRatio.

None of the changes affect the coding results of the common test conditions set out in JCTVC-L1100 or JCTVC-P1006, and therefore these have not been re-published.  However, note that when JCTVC-W0062 is enabled, changes would be observed.

HM16.10 is expected to be released during the meeting. It should include:

- Bug fixes (including the fixing of interlace which was inadvertently broken by a previous adoption), comment fixes and code tidies.

- JCTVC-U0128 “An HEVC SEI Message for Green Metadata”.

- Reduced encoder memory footprint (see below).

Currently none of the changes should affect the coding results of the common test conditions set out in JCTVC-L1100 or JCTVC-P1006.

Lambda and QP control

During the integration of JCTVC-W0062, the software coordinators became concerned about the mechanism that was being used to select the QP. As a general rule, if a setting is controlled by the user, it is used as specified.

However, for the inter configurations, the user specifies a base QP for the sequence, and also QP offsets for each of the GOP entries. In addition, lambda adjustments are specified, both in the GOP table and as separate parameters. With JCTVC-W0062, it was noticed that by modifying the lambda, the QP can become disconnected from the optimal QP, and therefore the QP should be modified to compensate for the lambda change.

The result is that the QPs observed during the normal CTCs are adjusted. This can be seen in a table provided in the AHG report, along with the values used when the similar but optional “RecalculateQPAccordingToLambda” function, described in JCTVC-J0242 (“Encoding with fixed Lagrangian multipliers”), is enabled.

From a control point of view, the coordinators felt that having settings that were subsequently changed was potentially confusing for the user and it was hoped that an alternative approach could be found.

In addition, the presence of so many different methods to specify lambda and QP should be studied to see if there could be more concise and transparent control methods.

Finally, it was unclear what would happen to the “RecalculateQPAccordingToLambda” optional tool once JCTVC-W0062 was enabled.

It is noted that the proponents of JCTVC-W0062 had provided an alternative parameterization, described in JCTVC-X0038.

See notes for X0038.

Encoder memory footprint

It was observed that the HM encoder incurred a large memory footprint. This problem is exacerbated by analysis of 4K images and the use of 16-frame GOPs, an aspect of other branches of the HM software.

For example, when encoding Traffic (2560x1600) with the 16-frame GOP structure used by JVET (and proposed previously in JCTVC-W0062 and again in JCTVC-X0038), memory requirements peak at 2482 MiB. For 4K sequences, memory requirements are approximately 2.1 times higher; i.e. ~ 5361 MiB. For derived branches, such as JEM, memory utilization is even higher.

The reason was found to be because all memory is allocated for all the processing at the outset. This means that for a 16 frame GOP simulation, up to 32 TComPic objects will be present, all of which have memory allocated for all encoding decisions.

However:

- 15 of the frames will only actually contain source image data.

  - These are frames that have been buffered up prior to the complete GOP encoding.

- 1 of the frames will be being processed.

  - This will therefore need source data and encoding decision data.

- Up to 16 frames will be kept as part of the reference picture list structure.

  - These technically only require the reconstructed picture data and other information present in the DPB.

A patch was developed that reduced the memory usage by ~60% for the above example, reducing Traffic, GOP-16 from 2482 MiB to just 1020 MiB. It achieves this by allocating memory only when it is needed. It also introduces a DPB structure into which side information is placed for reference frame usage.

Note that there is not such a significant problem in the decoder, as the decoder only allocates data for pictures when they are decoded, and destroys the data afterwards (as the decoder also has to cope with video format changes, array sizes are potentially dynamic, unlike in the HM encoder).

The patch does not have a noticeable effect on the run-time; it has no impact on coding efficiency.

Decision (SW): It was agreed to include this fix in the HM.

Remaining issues

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA: Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices: Tickets #1314, #1338, #1339.

- Field-coding: Tickets #1145, #1153.

- Decoder picture buffer: Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing: Tickets #1335, #1336, #1393.

- Additional decoder checks: Tickets #1367, #1383.

However, a patch has been generated that adds some conformance checks. It is being considered for potential inclusion in a future release.

As described to the community at the last four JCT-VC meetings, alterations to remove the unused software hierarchy in the entropy coding sections of the code, and to remove terms such as CAVLC is being considered. However, this will now need to also consider the impact on the JEM branch.

Recommendations

- Continue to develop reference software based on HM version 16.9/10 and improve its quality.

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Continue to investigate the merging of branches with the other software coordinators.

JCTVC-X0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, J. Boyce, R. Joshi, K. Kazui, A. Ramasubramonian, W. Wan, Y. Ye] [miss]

Deferred TBP.

JCTVC-X0005 JCT-VC AHG report: SHVC verification test reporting (AHG5) [Vittorio Baroncini, Ye-Kui Wang, Yan Ye]

This AHG was established to finalize the SHVC verification test report. The mandate of this AHG is:

Edit and produce the final SHVC verification test report.

Activities of this AHG

Main activity of this AHG included:

Produced the final SHVC verification test report in document JCTVC-W1004. The report was uploaded on March 18th.

There are two input contributions related to SHVC verification test at this meeting:

- JCTVC-X0046, “Draft supplemental SHVC verification test plan”

- JCTVC-X0058, “Cross check of JCTVC-X0046: Draft supplemental SHVC verification test plan”

The AHG recommended to review and discuss these at this meeting.

JCTVC-X0006 JCT-VC AHG report: SCC verification testing (AHG6) [H. Yu, R. Cohen, A. Duenas, K. Rapaka, J. Xu, X. Xu]

This report summarizes the activities of the JCT-VC ad hoc group on SCC verification testing (AHG6) between the JCT-VC 23rd meeting in San Diego, USA, and the 24th meeting in Geneva, Switzerland.

A kick-off message for AHG 6 was sent out on March 11, 2016.

A draft test plan JCTVC-X0074 is submitted and proposes the following test conditions:

- Software: SCM-8.0, HM-16.9, JM-19.0

- Test material: all sequences in the SCC common test condition JCTVC-U1015

- 8 test cases: 4:4:4 lossless, 4:2:0 lossless, 4:4:4 AI/RA/LB, and 4:2:0 AI/RA/LB

- Test points: 4 QPs for SCM, i.e. QP=22, 27, 32, and 37.

Open issues identified in the AHG report for further discussion:

- Time line?

- Subjective test?

- Matching bit-rates or QP values for HM and JM?

The AHG recommended to

- approve the last version of U1015, uploaded on Dec. 2, 2015. It has the new test-results reporting templates with SCM-6.0 anchor data.

- discuss the open issues in X0074, perhaps in BoG, and provide an update on bitstream generation, subjective testing, and time-line.

The discussion of the AHG report included the following comments:

- It was suggested to use some test sequences for verification testing that were not used in the SCC CTC.

- It was commented that bit depths should be considered in the test planning. It was commented that the design process primarily focused on 8 bit screen content (and the only screen content we have was captured as 8 bit content), so that should be the primary tested scenario.

- It was commented that HDR could be considered. Another participant commented that this was not in the scope of the design process.

- Areas of potential difficulty included identifying content and determining how to do subjective quality testing for screen content video. Side activity during this meeting to work on those aspects was encouraged (coordinated by H. Yu and V. Baroncini).

- In terms of scheduling, it was suggested to finalize the test plan by the next meeting and conduct the test by the next meeting after that.

-

JCTVC-X0007 JCT-VC AHG report: SCC extensions text editing (AHG7) [R. Joshi, J. Xu (AHG co-chairs), Y. Ye, S. Liu, G. Sullivan, R. Cohen (AHG vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on SCC extensions text editing (AHG7) between the 23rd JCT-VC meeting in San Diego, USA (February 2016).and the 24th JCT-VC meeting in Geneva, Switzerland (May 2016).

The sixth specification text draft (JCTVC-W1005) for the High Efficiency Video Coding Screen Content Coding (HEVC SCC) extensions was produced by the editing ad hoc group as an output document following the decisions taken at the 23rd JCT-VC meeting in San Diego, USA (February 2016).

Three versions of JCTVC-W1005 were produced. The text of JCTVC-W1005 (version 3) [note upload time] is considered final for the ISO/IEC FDIS ballot text.

The following is a list of changes with respect to JCTVC-V1005:

- Alignment with SHVC, MV-HEVC and 3D-HEVC in the syntax for disabling weighted prediction when current picture reference (CPR) is enabled, by using both layer ID and POC (JCTVC-W0076)

- Decoding process invocation for an independent non-base layer in an additional layer set containing more than one layer (FINB comment #1)

- Relaxed constraint on the presence of layers not present SEI messages (FINB comment #3)

- Changed the instances of VpsMaxLatencyPictures[][] to MaxVpsLatencyPictures[][], as there are also instances of VpsMaxLatencyPictures[] (JCTVC-V0031)

- Ignore the undefined RPS when deriving NumPicTotalCurr in IDR pictures (JCTVC-V0031)

- Setting of the initial value of NumActiveRefLayerPics0 and NumActiveRefLayerPics1 equal to 0 in clause F.8.1.4 (JCTVC-V0031)

- Added an update of the constraint, specified in clause 8.3.2, that requires no entry in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr when one or more of three conditions are true into clause F.8.3.2 to take FirstPicInLayerDecodedFlag into account. (JCTVC-V0031)

- Clarified the semantics of poc_reset_period_id and added a constraint to explicitly express that the value of poc_reset_period_id shall be the same for all pictures in an access unit. (JCTVC-V0031)

- Name change for chromaticity-derived colour matrix coefficient specification for clarity, precision, and objectivity

- Renamed variables pps_act_*_qp_offset to PpsActQpOffset* to follow editorial conventions. Added a note regarding the default values for qp offsets (JCTVC-W0096)

- Enabled simultaneous use of wavefronts and tiles for Screen-Extended Main 4:4:4 and Screen-Extended Main 4:4:4 10 profiles. (Geneva October 2015 meeting notes)

- Modified the semantics of entry_point_offset_minus1 (ticket #1440)

- Miscellaneous tickets (ticket #1435, ticket #1437, ticket #1439, ticket #1441)

- Added a constraint that when sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0, the value of TwoVersionsOfCurrDecPicFlag shall be equal to 0

- Corrected the constraint that requires slice_type to be equal to 2 to be "When sps_max_dec_pic_buffering_minus1[ TemporalId ] is equal to 0, nuh_layer_id is equal to 0, and pps_curr_pic_ref_enabled_flag is equal to 0, slice_type shall be equal to 2."

- On a constraint on the value of num_long_term_pics, a correction was also made from using sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ] to sps_max_dec_pic_buffering_minus1[ TemporalId ], as this is a slice segment header level constraint

- Added a note to clause 8.1.3 trying to clearly describe the difference cases of the current decoded picture depending on the values of TwoVersionsOfCurrDecPicFlag and pps_curr_pic_ref_enabled_flag, and markings of the different versions of the current decoded picture

- Updated, in clause C.5.2.2, the condition for invocation of the bumping process by taking into account the value of TwoVersionsOfCurrDecPicFlag (JCTVC-W0077)

- Clarification of the semantics of the colour remapping information SEI message semantics in a less restrictive way than had been planned at the Geneva October 2015 meeting (per meeting notes of editorial review at San Diego February 2016 meeting)

- Fixed subscript typo for ST 2084 transfer function, missing space for SMPTE ST 428-1, and factor-of-12 scaling, full-range scaling, and nominal range for ARIB STD-B67 transfer function (JCTVC-W0044)

- Updated, in clause F.13.5.2.2, the condition for invocation of the bumping process by taking into account the value of TwoVersionsOfCurrDecPicFlag, to be aligned with the bumping process invocation in clause C.5.2.2

- Aligned the DPB handling process in clause F.8.1.5 with that in clause 8.1.3

- Removed the definition and initialization of CurrPicInList0Flag and CurrPicInList1Flag, which are not used anymore

- Corrected the condition in the equation for including the current picture into RefPicList0 when reference picture list modification is not in use

- Updated the reference picture list construction process in clause F.8.3.4 to enable current picture reference

- Updated the constraint on the value of NumPicTotalCurr in clause F.8.3.2 to take into consideration of pps_curr_pic_ref_enabled_flag, for alignment with the corresponding constraint in clause 8.3.2

- Updated in clause F.7.4.7.2 the equation for deriving NumPicTotalCurr taking in account pps_curr_pic_ref_enabled_flag, for alignment with the corresponding equation in clause 7.4.7.2

- Added in clause F.8.3.4 a constraint on RefPicList0 and RefPicList1 not to contain entries that refer to a picture other than the current picture for an independent non-base layer, for alignment with clause 8.3.4

- Replaced the decoder capability requirement description in clause H.11.1.2 with changes to clause F.11.2

- Added the derivation of sub-bitstreams subBitstream and baseBitstream to a new clause F.11.3 and replaced each of the descriptions of the same derivation in clauses G.11.1.1, H.11.1.1, H.11.1.2, and I.11.1.1with a reference to clause F.11.3

- Added missing CABAC information related to the palette mode in clause 9.3 (ticket 1433)

- Added fix for SHVC text and test model ticket 111

- Clarified the bitstream conformance condition on the picture referred to by the collocated_ref_idx so that the condition applies only when slice_temporal_mvp_enabled_flag is equal to 1 and the slice type is not 2. This allows for pictures having only the current picture in the reference list, which was intended to be allowed but had been accidentally disallowed by the previously drafted language.

- Changed the payloadType values for the alternative transfer characteristics information SEI message and the ambient viewing environment SEI message from 182 and 183 to 147 and 148 to maintain the intended logical groupings of SEI messages

- Added the coded region completion SEI message to the suffix part of the syntax table as intended by the contributions and meeting notes (already present in the prefix part).

The screen content coding test model 7 (SCM 7) (document JCTVC-W1014) was released on 23rd May 2016. The main changes from SCM 6 to SCM 7 are the introduction of bottom-up hash value calculation and validity check method for hash-based inter search

AHG recommendations:

- The recommendations of the HEVC SCC extension draft text AHG are to:

- Approve the documents JCTVC-W1005 and JCTVC-W1014 as JCT-VC outputs

- Address the comments and feedback on SCC extensions text specification as appropriate

- Compare the HEVC SCC extensions document with the HEVC SCC extensions software and resolve any discrepancies that may exist, in collaboration with the SCC extension software development (AHG8)

--

JCTVC-X0008 JCT-VC AHG report: SCC extensions software development (AHG8) [K. Rapaka, B.Li (AHG co-chairs), R. Cohen, T.-D. Chuang, X. Xiu (AHG vice-chairs)]

This report summarizes the activities of Ad Hoc Group 8 on screen content extension software (SCM) developments that had taken place between the JCT-VC 23rd meeting in San Diego, USA, and the 24th meeting in Geneva, Switzerland.

One software revision (HM16.8-SCM7.0) was produced and announced on the JCT-VC email reflector. The integration details and performance summary is provided in the next subsections. The performance results of the software revision was observed to be consistent with the adopted techniques.

HM-16.8_SCM-7.0 was announced on the email reflector on March 21st 2015. The software was tagged as https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.8+SCM-7.0/ .

HM-16.8_SCM-7.0 incorporates following adoptions/bug fixes:

- JCTVC-W0075:  Encoder improvements of PLT in lossless mode.

- JCTVC-W0077:  DPB process update for CPR.

- JCTVC-W0078:  Bottom up hash calculation

- Software Bug fixes for previous integrations of JCTVC-U0181/JCTVC-V0057

- Ticket #1422:  Parsing order mismatch between spec and software.

- Merge to HM 16.8

- General code cleanup and removal of macro’s related to SCM 6.0.

The performance HM-16.8_SCM-7.0 compared to HM-16.7_SCM-6.0 was described according to the common test conditions in JCTVC-U1015. No noticeable performance change was observed for the lossy 4:4:4 configuration, lossy 420 configuration and lossless 444 configuration.

For the lossless 4:2:0 configuration, it is observed that this version provides BD-rate reduction of 1.1%, 0.5% and 0.5% for YUV 1080p & 720p text and graphics category in AI/RA/LB configurations respectively. This was reported to be due to encoder-only improvement for lossless operation.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ had been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-8.0, version SCC-7.0 (HM16.8).

Ticket #1422 was closed during the meeting cycle. There were no remaining open tickets.

AHG recommendations:

- Continue to develop reference software based on HM16.8_SCM7.0 and improve its quality.

- Remove macros introduced in previous versions before starting integration towards SCM-6.x/SCM-7.0 such as to make the software more readable.

- Continue merging with later HM versions.

It would be desirable to have software prepared that is suitable for ITU-T Consent at the current meeting.

It was commented that it would be desirable to replace the HM with the SCM ASAP, rather than maintaining them separately. This would pose some difficulties for the SHM, HTM, and JEM, but would reduce the number of codebases to maintain and provide a single codebase for single-layer HEVC.

It was commented that some code cleanup is needed regarding duplication of code in the SCM (which is needed anyway, but becomes more important if this becomes the core codebase).

Revisit to confirm.

JCTVC-X0009 JCT-VC AHG report: SHVC software development (AHG9) [V. Seregin, Y. He, G. Barroux]

This report summarizes activities of the AHG9 on SHVC software development between 23th and 24h JCT-VC meetings.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

The latest version was SHM-12.0 and it was released along with JCTVC-W1013 for the ISO/IEC DAM ballot.

SHM-12.0 is based on HM-16.9 with the following software improvements:

- Fixes for tickets ## 101, 102, 104, 108, 110, 114

- Integrated multi-view support from JCTVC-W0134

- Updated RA configuration files to reflect temporal layers setting adopted to HM

Anchor data and templates have been generated based on common test conditions JCTVC-Q1009 and are attached to this report.

A loss in RA configuration arose from the updated temporal layers setting.

AHG proposed development plan and recommendations

- Continue to develop reference software based on SHM-12.0 and improve its quality.

- Fix open tickets.

JCTVC-X0010 JCT-VC AHG report: Test sequence material (AHG10) [T. Suzuki, V. Baroncini, R. Cohen, E. Francois, T. K. Tan, P. Topiwala, S. Wenger, H. Yu] [miss]

Deferred TBP.

JCTVC-X0011 JCT-VC AHG report: HDR/WCG visual testing (AHG11) [V. Baroncini, P. Topiwala, E. Alshina]

Deferred TBP.

-

JCTVC-X0012 JCT-VC AHG report: HDR/WCG verification test preparation and testing (AHG12) [A.K. Ramasubramonian, R. Sjoberg]

This document provides a report of the AhG12 activity on verification test preparation and testing of HDR/WCG technology conducted between 23rd and 24th JCT-VC meetings. This document report contained the mandates, summary of AhG activities and email discussions, list of AhG-related input contributions to 24th JCT-VC meeting, and recommendations.

During 23rd JCTVC meeting, JCTVC initiated a work on producing a technical report on conversion and coding practices for HDR/WCG video coding using HEVC main10 profile. A plan for HDR/WCG video verification test for single-layer coding was drafted to test the coding performance of High Dynamic Range (HDR) content using HEVC Main 10 coding as specified in HEVC version 2.

AhG12 was formed to finalize the verification test plan for HDR/WCG video, conduct the verification tests and begin drafting a preliminary report of the test results.

A kick-off message was sent on March 8, 2016, listing the mandates of ad hoc group 12 and suggesting an emails discussion on these issues to be submitted to JCT-VC reflector, jct-vc@lists.rwth-aachen.de.

There were around five emails mostly relating to frequency of the playback system and monitor.

It is reported that the testing activity has been completed on schedule, and the results of the subjective testing conducted have been submitted. A summary of the activities related to the mandates is presented below:

The finalized verification test plan JCTVC-W1018 was submitted on 11 March, 2016.

The bitstreams for HDR/WCG verification testing as described in JCTVC-W1018 were prepared on 15 March, 2016, and was cross-checked by Qualcomm and Apple.

Subjective testing was performed at two locations:

- Ericsson AB, Sweden – tests were conducted between April 11 and April 15, 2016

- Waitek Laboratory, Italy between May 11 and May 22, 2016.

The subjective testing results have been submitted to the 24th JCT-VC meeting in the following contribution: V. Baroncini, K. Andersson, V. Kulyk, “Results of the HDR/WCG Verification test using HEVC Main 10 Profile in Stockholm and Rome,” JCTVC-X0033, Geneva, CH, May 26 – June 1, 2016.

The ad hoc group recommended to review the verification test results reported in JCTVC-X0033.

The goals of the test were discussed. The primary goal is to show that HEVC can be used to effectively deliver good quality HDR with some reasonable bit rate. A secondary and more internal goal is to confirm that the guidelines we are developing provide a visual benefit.

It was suggested to refer to the outcome of the test as a "validation" rather than "verification". Revisit to further discuss that and other aspects of the test and its results.

-

JCTVC-X0013 JCT-VC AHG report: HDR/WCG coding practices guideline development (AHG13) [J. Samuelsson (chair), C. Fogg, A. Norkin, J. Sole, J. Strom, A. Tourapis, P. Yin (vice-chairs)]

This document reports the activity of the ad hoc group on HDR/WCG coding practices guideline development (AHG13). The first version of this ad hoc group report contains the ad hoc group mandates and the notes from the first teleconference meeting. In the second version of this ad hoc group report, the notes from the second teleconference meeting have been added. In the third version of this ad hoc group report sections have been added related to email discussions, input contributions and recommendations.

[add more notes]

Guidelines scheduling target: October PDTR; January TR & ITU-T approval. [Check process]

- JCTVC-X0014 /

- JCTVC-X0081 AhG report: HDR/WCG technology for backward compatibility and display adaptivity (AHG14) [Dmytro Rusanovskyy, Edouard Francois, Walt Husak]

[Note: Check that X0014 is correct and mark X0081 as withdrawn]

This document provides a report of the AhG14 activity on HDR/WCG technology for backward compatibility and display adaptivity conducted between 24 and 24th JCT-VC meetings. This document reports contained the mandates, summary of AhG activities and email discussions, list of AhG-related input contributions to 24th JCT-VC meeting and recommendations.

During 23rd JCTVC meeting there were multiple input contributions on HDR video coding. The contributions targeted several aspects of HDR video coding, including new tools for improving compression efficiency of HDR/WCG video signals, providing backward compatibility as well advanced encoding configuration of HEVC main10 video codec specific for HDR/WCG signals.

As output of the study conducted in MPEG and JCTVC, it was concluded that no improving compression efficiency technology has been identified that would justify the creation of a new HDR specific HEVC profile. JCTVC initiated a work on producing a technical report on conversion and coding practices for HDR/WCG video coding using HEVC main10 profile.

JCTVC also identified several standard technologies that can be utilized for backward compatible HDR services, among which are HLG in combination with alternative transfer characteristics SEI message, the colour remapping information SEI message, and the SHVC.

AhG14 was formed to continue study on HDR/WCG technologies for backward compatibility and display adaptivity, including but not limiting to study of single-layer solutions with existing and potentially new SEI messages, dual-layer coding with SHVC and study on relevant test conditions and experiments.

A kick-off message was sent on March 9, 2016, listing the mandates of ad hoc group 14 and suggesting an emails discussion on these issues to be submitted to JCT-VC reflector, jct-vc@lists.rwth-aachen.de.

There were around 20 emails submitted to the reflector. The topic related to the mandate 2 of the AhG14 received the most of expert’s attention.

There are 12 input contributions to this meeting which address problems of all four mandates of the AhG14.

Mandate 1: Study the technical characteristics of single-layer coding with HEVC using existing SEI and VUI indicators for backward compatibility and display adaptivity.

There was no discussion over the reflector on issues related to the mandate 1. There are 2 contributions to this meeting which may be categorized as related to the mandate 1 of the AhG14.

Contribution [1] address’ the JCTVC conclusion from the 23rd meeting about possibility to utilize the Colour Remapping Information SEI message for providing a single-layer HDR coding with backward compatibility. Proposal [1] presents simulation results of utilizing CRI signaling and CRI post-processing to enable 2 types of backward compatibility: SDR display compatibility and SDR bitstream compatibility.  Contribution also proposes a draft text to document “Conversion and Coding Practices for HDR/WCG Video” on utilizing the CRI for enabling backward compatible HDR video.  Contribution [2] provides a cross-check of these results.

Mandate 2: Study the technical characteristics of two-layer coding with SHVC for backward compatibility and display adaptivity.

During the discussion on mandate 2 this was pointed out, that SHVC verification test conducted for the previous meeting included a test on color gamut scalability. In this configuration, SDR/BT.709 video signal was coded at the base layer and HDR/BT.2020 video was coded as enhanced layer. Although testing a valid use case of CGS, the original verification test plan omitted another useful scenario of SDR to HDR scalability, namely when SDR/BT.2020 signal is coded at the base layer and HDR/BT.2020 is coded at the enhanced layer. It was suggested to supplement the SHVC verification test with additional results addressing this use case.

Between 23rd and 24th meetings, experts collected additional test material, updated the encoder configuration and drafted a supplemental SHVC verification test plan. The following is the timeline of the conducted SHVC-related activity:

- Test material in SDR/BT.2020 collected by 12th of April 2016

- Updated HDRTools configurations and md5 sums distributed on 15th of April 2016

- Updated SHVC software and suggested tested rate points distributed on 25th of April 2016

- Test set was updated to address recommendations of experts on 9th of May, 2016 (by adding "P3" content that includes colours not representable in BT.709)

- Encoded bitstreams made available to the test labs by May 11, 2016

- Subjective evaluation preparation and test conducted on: May 11-22, 2016

A detailed plan of the proposed supplemental SHVC verification test is available in [3] and discussion on the proposed test plan is suggested in [4]. Details on utilized SHVC configuration is available in [5] and subjective evaluation results conducted by GBTech Laboratory and Waitek Laboratory available in [6]. Cross-check of the conducted test is provided in [7].

Mandate 3: Identify and study the technical characteristics of proposed approaches to backward compatibility and display adaptivity with potential additional new SEI messages.

There was no discussion over the reflector on issues related to the mandate 3. There are 3 contributions to this meeting which are addressing upcoming standardization of SMPTE ST2094 group of specifications and may be categorized as related to the mandate 3 of the AhG14.

Contributions [8] and [9], proposes to introduce to the HEVC new SEI messages to support metadata of SMPTE 2094-10, 40 specifications. Individual SEI for each specification is proposed. Contribution [10] proposes regional nesting SEI to address the carriage means for region descriptors specified in SMPTE 2094-1.

Mandate 4: Study and propose test conditions for associated experiments.

There was no discussion over the reflector on issues related to this mandate. There are 2 input contributions to this meeting which may be categorized as related to the mandate 4 of the

Contributions [11] and [12] propose two approaches for enabling the backward compatibility HDR video.

Input contributions

There 12 input contributions related to AhG14 were submitted to the 24th JCTVC meeting.

- D. Rusanovskyy, D. Sansli, A. Ramasubramonian, J. Sole, M. Karczewicz, Usage of CRI for guided mapping (dynamic range adaptation) , JCTVC document X0060

- F. Hiron, E. Francois (Technicolor), AHG13: cross-check of JCTVC-X0060 on usage of CRI for DRA, JCTVC document X0076.

- A.K. Ramasubramonian, J. Sole, D. Rusanovskyy, D.B. Sansli, “SHM encoder improvements for HDR verification testing”, JCTVC-X0035, Geneva, CH, May-June, 2016

- A.K. Ramasubramonian, J. Sole, Y.-K. Wang, V. Baroncini, “Draft supplemental SHVC Verification test plan,” JCTVC-X0046, Geneva, CH, May–June 2016.

- W. Husak, F. Pu, T. Lu, P. Yin, T. Chen, AHG14: On Test Plan of SHVC Main 10 for HDR/WCG Backward Compatibility, JCTVC document X0055

- V. Baroncini (MPEG Test Chair), “Draft supplement to the SHVC verification test”, Geneva, CH, 26 May– 1 June 2016

- Y. Ye, Y. He, Cross check of JCTVC-X0046: Draft supplemental SHVC verification test plan, JCTVC document X0058,

- R. Yeung, S. Qu, P. Yin, T. Lu, T. Chen, W. Husak (Dolby), "Indication of SMPTE 2094-10 metadata in HEVC", …

- X0075

- L. Tao, Y-T Kim, S. Park, B. Min, S. Rhyu, H. Najaf-Zadeh, M. Budagav (Samsung), Indication of SMPTE 2094-40 metadata in HEVC, JCTVC document X0061

- K. Ramasubramonian, J. Sole, Y.-K. Wang, D. Rusanovskyy, D. Bugdayci Sansli, M. Kacrzewicz (Qualcomm), P. Andrivon, E. Francois (Technicolor), W. de Haan, R. Brondijk  (Phillips), Regional nesting SEI message, JCTVC document X0062.

- P. Topiwala, W. Dai, M. Krishnan, Improvements to HDR10 with Backward Compatibility Options, JCTVC document X0044-r1

- C. Jung, Q. Lin, S. Yu, Backward Compatible Opto-Electrical Transfer Function for HDR Video Coding Based on Rational Quantization, JCTV document X0064

The ad hoc group recommended to create a break out group to continue the work and the discussion related to mandate 2, namely SHVC testing.  The ad hoc group further recommended to review the input contributions falling under the AhG14 mandates.

-

Project development, status, and guidance (6)

## Corrigenda items (0)

See the AHG 2 report.

## Profile/level definitions (1)

JCTVC-X0034 On profiles and per-feature Flags [?? (??)]

## Conformance test set development (0)

See the AHG report JCTVC-X0004 and outputs JCTVC-X10XX for … .

## Specification text development (1)

JCTVC-X0079 Suggestion for new draft version of Conversion and Coding Practices for HDR/WCG Video [J. Samuelsson, A. Tourapis, J. Strom, C. Fogg] [late]

## HEVC coding performance, implementation demonstrations and design analysis (1)

JCTVC-X0033 Results of the HDR/WCG Verification test using HEVC Main 10 Profile in Stockholm and Rome [V. Baroncini, K. Andersson, V. Kulyk (Ericsson AB)] [late]

This document reports the testing procedure and results from the HDR verification testing with naïve viewers held in Stockholm during April 11 to April 15 and in Rome during May 11 to May 22. The results reportedly show that HEVC Main 10 Profile can be used to compress 1080p HDR content in BT.2020 format with close to visually lossless quality compared to an HDR source signal at bit rates below 6500 kbps for HDR sequences, including two sport sequences. It was also shown significant improvements of using state-of-the-art technology when pre-processing and encoding HDR content using HEVC Main 10 Profile compared to pre-processing and encoding without considering that the content was HDR. BD-rate(MOS) numbers are presented based on the subjective scores in the test, showing an average bit rate reduction of 27% over all sequences of both test sites.

This document reports results for the HDR (High Dynamic Range) verification test for HEVC Main 10 Profile using both state-of-the-art technology for HDR pre-processing and coding, and without such considerations according to test plan in JCTVC-W1018. In this document the term “using state-of-the-art technology” is used to represent what is sometimes called “Anchor 3.2” and which represent technology described in more detail in JCTVC-W1017. The term “without using state-of-the-art” is used to represent what is sometimes called “Anchor 1.0” and which represents conventional processing generally described in [5]. Ericsson has generated the processed test sequences and they have been cross-checked by Qualcomm and Apple.

Comments:

- Results from each test lab

  - Adjectives versus no adjectives for rating scores

  - Two versus one viewing per sequence pair

  - Guidance about what type of artefacts to look for, with sequences in the test set

  - Different SIM2 models (prototype, brightness range)

- 4:2:2 input to SIM2

- Bit rates

JCTVC-X0074 Draft verification test plan for SCC extensions [H. Yu, R. Cohen, K. Rapaka, J. Xu] [late]

JCTVC-X0046 Draft supplemental SHVC verification test plan [A.K. Ramasubramonian, J. Sole, Y.-K. Wang (Qualcomm), V. Baroncini]

JCTVC-X0058 Cross check of JCTVC-X0046: Draft supplemental SHVC verification test plan [Y. Ye, Y. He (InterDigital)] [late]

JCTVC-X0080 Supplement to SHVC Verification Test [V. Baroncini] [late]

## Software development (1)

JCTVC-X0053 HDRTools: Software status [A.M. Tourapis, D. Singer (Apple)] [late]

## Source video test material (1)

JCTVC-X0068 New 4K HDR Proposed Test Material [P. Topiwala (FastVDO)] [late]

## New application domains (2)

JCTVC-X0039 On MCTS extraction [R. Skupin, Y. Sánchez, K. Grüneberg, C. Hellge, T. Schierl (HHI)]

JCTVC-X0071 Machine learning in multimedia applications [C. Fogg (MovieLabs)] [miss] [late]

# Core experiments (0)

No CEs were run during the recent meeting cycle.

# Technical contributions (34)

## HDR coding (27)

### Best practices of HDR coding using Main 10 (5)

JCTVC-X0036 Modified Linearization of Luma Adjustment [J. Ström, J. Samuelsson, K. Andersson, P. Hermansson (Ericsson)]

JCTVC-X0038 AHG 3 Recommended settings for HM [K. Andersson, P. Wennersten, J. Samuelsson, J. Ström, P. Hermansson, M. Pettersson (Ericsson)]

JCTVC-X0082 Cross-check of AHG 3 recommended settings for HM (JCTVC-X0038) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-X0065 Comments on Conversion and Coding Practices for HDR/WCG Video [C. Fogg (MovieLabs)] [miss] [late]

JCTVC-X0066 Proposed text for usage of Colour Remapping Information (CRI) SEI message for Conversion and Coding Practices for HDR/WCG Video [E. Francois, F. Hiron, P. Andrivon (Technicolor)]

### Usage of CRI for HDR coding (4)

JCTVC-X0041 Usage of CRI for HDR video compression with dynamic range adaptation [E. Francois, F. Hiron, P. Andrivon (Technicolor)]

JCTVC-X0059 Cross-check of JCTVC-X0041: Usage of CRI for HDR video compression with dynamic range adaptation [D. Rusanovskyy (Qualcomm)] [late]

JCTVC-X0060 Usage of CRI for guided mapping (dynamic range adaptation) [D.Rusanovskyy, A.K.Ramasubramonian, D.Sansli, J.Sole, M.Karczewicz (Qualcomm]

JCTVC-X0076 AHG13: cross-check of JCTVC-X0060 on usage of CRI for DRA [F. Hiron, E. Francois (Technicolor)] [late]

### Luma sample adjustment (3)

JCTVC-X0043 AHG13: on Luma Adjustment [F. Pu, T. Lu, P. Yin, T. Chen, W. Husak (Dolby)]

JCTVC-X0056 AHG13: Crosscheck report of luma adjustment (JCTVC-X0043) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-X0054 AHG13: Further results for LUT-based luma sample adjustment [C. Rosewarne, V. Kolesnikov (Canon)]

### ICtCp colour representation (4)

JCTVC-X0047 Further ICtCp testing [C. Fogg (Movielabs)] [miss] [late]

JCTVC-X0050 AHG13: ICtCp Compression Using HEVC Main 10 [F. Pu, T. Lu, P. Yin, T. Chen, W. Husak (Dolby)]

JCTVC-X0057 AHG13: Crosscheck report of ICtCp Compression Using HEVC Main 10 (JCTVC-X0050) [Y. He, Y. Ye (InterDigital)] [late]

JCTVC-X0051 ICtCp colour representation: Observations and Findings [A. M. Tourapis, Y. Su, D. Singer (Apple)] [late]

### SDR backward compatible HDR coding (6)

JCTVC-X0035 SHM encoder improvements for HDR verification testing [A. K. Ramasubramonian, J. Sole, D. Rusanovskyy, D.B. Sansli (Qualcomm)]

JCTVC-X0044 Improvements to HDR10 with Backward Compatibility Options [P. Topiwala, W. Dai, M. Krishnan (FastVDO)]

JCTVC-X0055 AHG14: On Test Plan of SHVC Main 10 for HDR/WCG Backward Compatibility [W. Husak, F. Pu, T. Lu, P. Yin, T. Chen (Dolby)]

JCTVC-X0064 Backward Compatible Opto-Electrical Transfer Function for HDR Video Coding Based on Rational Quantization [C. Jung, Q. Lin, S. Yu (Xidian Univ.), M. Li, P. Wu (ZTE)]

### Sampling/subsampling (5)

JCTVC-X0037 Evaluation of Subsampling Methods [J. Ström, J. Samuelsson, K. Andersson, P. Hermansson (Ericsson)]

JCTVC-X0045 A Study of Chroma Resampling for HDR10 [P. Topiwala (FastVDO)] [late]

JCTVC-X0048 Non-linear resampling between 4:4:4 and 4:2:0 chroma [C. Fogg (Movielabs)] [late]

JCTVC-X0072 On HDR 4:2:0 chroma subsampling (AHG13 related) [A. Norkin (Netflix)] [late]

JCTVC-X0078 Crosscheck of JCTVC-X0072: On closed form HDR 4:2:0 chroma subsampling (AHG13 related) [Taoran Lu, Fangjun Pu, Peng Yin (Dolby)] [late]

### Other (2)

JCTVC-X0049 Conversion between PQ and Hybrid Log Gamma (HLG) [C. Fogg (Movielabs)] [late]

JCTVC-X0063 Adaptive Perceptual Quantizer for HDR Video Coding [C. Jung, S. Yu, R. Yu (Xidian Univ.), M. Li, P. Wu (ZTE)]

## HL syntax (1)

JCTVC-X0077 Tile based VR video encoding and decoding schemes [Y.-K. Wang, Hendry, M. Karczewicz (Qualcomm)] [late]

## SEI messages and VUI (7)

JCTVC-X0040 Content colour gamut SEI message [H. M. Oh, J. W. Choi, J.-Y. Suh (??)]

JCTVC-X0042 Indication of SMPTE 2094-10 metadata in HEVC [R. Yeung, S. Qu, P. Yin, T. Lu, T. Chen, W. Husak (Dolby)]

JCTVC-X0052 Improvements to the Effective Colour Volume SEI [A.M. Tourapis, Y. Su, D. Singer (Apple)] [late]

JCTVC-X0061 Indication of SMPTE 2094-40 metadata in HEVC [L. Tao, Y-T Kim, S. Park, B. Min, S. Rhyu, H. Najaf-Zadeh, M. Budagavi (??)]

JCTVC-X0062 Regional nesting SEI message [A.K. Ramasubramonian, J. Sole, Y.-K. Wang, D. Rusanovskyy, D.B. Sansli, M. Karczewicz (Qualcomm), P. Andrivon, E. Francois (Technicolor), W. Haan, R. Brondijk (Philips)]

JCTVC-X0069 Content colour volume SEI message [A.K. Ramasubramonian, D.B. Sansli, J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm)] [late]

JCTVC-X0075 Indication of SMPTE ST 2094-20 metadata in HEVC [W. de Haan, R. Brondijk, L. van de Kerkhof, R. Goris, R. Nijland] [late]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (1)

JCTVC-X0070 HEVC Encoder Optimization Based on Perceptual Block Merging [C. Jung, Q. Lin, S. Yu (Xidian Univ.)] [late]

# Withdrawn, unclear allocation (4)

JCTVC-X0031 Withdrawn

JCTVC-X0032 Withdrawn

JCTVC-X0067 Withdrawn

JCTVC-X0073 Withdrawn

# Plenary discussions, joint meetings, BoG reports, and summary of actions taken

## General

Topics for general discussion at the plenary level:

- …

## Project development

Joint meetings are discussed in this section of this report. Additional notes on the same topics may appear elsewhere in this report. Joint discussions were held on XXXX., as recorded below.

## BoGs

There were no formal break-out groups established at this meeting, and hence no BoG reports were submitted.

## List of actions taken affecting the draft HEVC specification

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the draft text of the HEVC specification. Both technical and editorial issues are included (although some relatively minor editorial / bug-fix matters are not listed). This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that what it proposes was adopted (in whole or in part).

- …

# Project planning

## Text drafting and software quality

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text. Similarly, software coordinators have the discretion to evaluate contributed software for suitability in regard to proper code style, bugginess, etc., and to not integrate code that is determined inadequate in software quality.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the XXday of the week preceding the meeting (xx Oct 2016).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments have been planned in previous work, although none were established at the current meeting. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-L1100.

The general timeline agreed for CEs was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

A deadline of four weeks after the meeting would be established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on CE work on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document would be reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE would be described in an associated output document numbered as, for example, JCTVC-W11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans would be recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities have been established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common test conditions for HEVC Coding Experiments

No particular changes were noted w.r.t. the prior CTC for work within the current scope of JCT-VC, and particularly for the SCC extensions development, as that work is at such a late stage of development that such changes would seem necessary to consider. update esp. for HDR?

## Software development planning (update)

Software coordinators were asked to work out the detailed schedule with the proponents of adopted changes.

Any adopted proposals where necessary software is not delivered by the scheduled date in a timely manner may be rejected.

The planned timeline for software releases was established as follows:

- HM 16.6 available prior to the meeting.

- SCM 5.0 (based on HM 16.6 or newer) should be available within 3 weeks after the meeting.

- SHM 10.x U1013 (DAM, based on HM 16.2 or newer) should be available within 5 weeks after the meeting.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable. Further study was encouraged to determine the appropriate approach to future software maintenance, especially in regard to alignment of 3D video software with the SHM software.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-W1002 HEVC Test Model 16 (HM 16) Update 5 of Encoder Description - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 16.x software versions and the reference configuration encodings according to JCTVC-L1100 and JCTVC-P1006 common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), K. Sharman (vice-chair) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Prepare and deliver the JCTVC-W1008 SHVC conformance draft 5 specification. - Develop proposed improvements to the SCC conformance testing draft W1016. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt, SHVC, and SCC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, R. Joshi, K. Kazui, A. K. Ramasubramonian, W. Wan, Y. Ye (vice-chairs) | N |
| SHVC verification test reporting (AHG5) (jct-vc@lists.rwth-aachen.de) - Edit and produce the final SHVC verification test report. | V. Baroncini, Y.-K. Wang, Y. Ye (co-chairs) | N |
| SCC extensions verification testing (AHG6) (jct-vc@lists.rwth-aachen.de) - Study test conditions and coding performance analysis methods for verification of SCC coding performance. - Prepare a proposed draft verification test plan for SCC | H. Yu (chair), R. Cohen, A. Duenas, K. Rapaka, X. Xu, J. Xu (vice-chairs) | N |
| SCC extensions text editing (AHG7) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions draft 6 and test model 7 text. - Gather and address comments for refinement of the test model text. - Coordinate with AHG8 to address issues relating to mismatches between software and text. | R. Joshi, J. Xu (co-chairs), R. Cohen, S. Liu, G. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SCM software and its distribution. - Prepare and deliver HM 16.x-SCM-7.0 software version and the reference configuration encodings according to JCTVC-U1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), P. Chuang, R. Cohen, X. Xiu (vice-chairs) | N |
| SHVC software development (AHG9) (jct-vc@lists.rwth-aachen.de) - Prepare and deliver the SHM 11.x software (based on HM 16.x) for scalable HEVC extensions draft 4 JCTVC-W1013. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | G. Barroux, Y. He, V. Seregin (co-chairs) | N |
| Test sequence material (AHG10) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material, especially focusing on new needs for HDR test material and corresponding SDR test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG6 regarding screen content coding testing and …. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), E. Francois, T. K. Tan, P. Topiwala, S. Wenger, H. Yu (vice-chairs) | N |
| HDR/WCG visual testing (AHG11) (jct-vc@lists.rwth-aachen.de) - Study content characteristics and identify appropriate test sequences for visual testing. - Identify and develop test methodologies incl. consideration and characterization of test equipment | V. Baroncini, P. Topiwala, E. Alshina (co-chairs) | N |
| HDR/WCG verification test planning (AHG12) (jct-vc@lists.rwth-aachen.de) - Finalize the verification test plan JCTVC-W1018 - Generate and collect the encoded bitstreams for the HDR/WCG verification test. - Identify and coordinate arrangements toward the preparation of test sites for subjective testing. - Perform the subjective testing as described in JCTVC-W1018. - Analyze the test results and prepare a draft report of the subjective testing. | A. K. Ramasubramonian, R. Sjöberg (co-chairs) | N |
| HDR/WCG coding practices guideline development (AHG13) (jct-vc@lists.rwth-aachen.de) - Identify and study tech approaches to single-layer HDR coding using the existing HEVC standard (up to SCC-extended edition) with ST 2084 transfer characteristics, including potential use of SEI messages - Study and consider potential guidelines for use of the ICTCP colour representation with the HEVC standard - Study and propose improvements of draft guidelines W10xx for HEVC single-layer coding using ST 2084 with YCbCr NCL narrow range - Conduct one or more teleconferences to discuss these matters, with appropriate advance notice. | J. Samuelsson (chair), C. Fogg, A. Norkin, J. Strom, J. Sole, A. Tourapis, P. Yin (vice-chairs) | Tel. |
| HDR/WCG technology for backward compatibility and display adaptivity (AHG14) (jct-vc@lists.rwth-aachen.de) - Study the technical characteristics of single-layer coding with HEVC using existing SEI and VUI indicators for backward compatibility and display adaptivity - Study the technical characteristics of two-layer coding with SHVC for backward compatibility and display adaptivity - Identify and study the technical characteristics of proposed approaches to backward compatibility and display adaptivity with potential additional new SEI messages - Study and propose test conditions for associated experiments | E. Francois, W. Husak, D. Rusanovskyy (co-chairs) | N |

# Meeting resolution to the hosting parent body

In consultation with the parent bodies, the JCT-VC reports the conclusion reached at the 114th meeting of WG 11 that the creation of a new HEVC profile or other new normative specification is not necessary to properly enable HDR/WCG video compression using the current edition of HEVC (3rd edition for ISO/IEC). No technology has been identified that would justify the creation of such a new specification. Future work on HDR/WCG in JCT-VC will focus primarily on formal verification testing and guidelines for encoding practices.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JCTVC-W1000 Meeting Report of the 23rd JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2016-05-13] (near next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

JCTVC-W1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 5 of Encoder Description [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 16048) [2016-05-13] (near next meeting)

JCTVC-W1003 Draft text for ICTCP support in HEVC (Draft 1) [P. Yin, C. Fogg, G. J. Sullivan, A. Tourapis (editors)] (WG 11 N 16047) [2016-03-11] (2 weeks)

JCTVC-W1004 SHVC Verification Test Report [Y. Ye, V. Baroncini, Y.-K. Wang (editors)] (WG 11 N 16051) [2016-03-18] (3 weeks)

JCTVC-W1005 HEVC Screen Content Coding Draft Text 6 [R. Joshi, S. Liu, G. J. Sullivan, Y.-K. Wang, J. Xu, Y. Ye (editors)] (WG 11 N 16046 Text of ISO/IEC FDIS 23008-2:201X 3rd Edition) [2016-04-22] (8 weeks)

Basic elements (no changes of features at the current meeting):

- IBC

- Adaptive colour transform

- Palette mode

- Adaptive MV resolution

- Intra boundary filtering disabling

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)]

Remains valid – not reissued: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778)

JCTVC-W1008 Conformance Testing for SHVC Draft 5 [J. Boyce, A. K. Ramasubramonian (editors)] (Included in WG 11 N 16061 Text of ISO/IEC FDIS 23008-8:201x 2nd Edition) [2016-04-22] (8 weeks)

(Merged into a new edition)

Remains valid – not updated JCTVC-Q1009 Common SHM Test Conditions and Software Reference Configurations [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-W1011 Reference Software for Screen Content Coding Draft 1 (WG 11 N 16057 Text of ISO/IEC 23008-5:201x/PDAM1) [K. Rapaka, B. Li, X. Xiu (editors)] [2016-03-11] (2.5 months prior to next meeting)

JCTVC-W1012 Conformance Testing for Improved HEVC Version 1 Testing and Format Range Extensions Profiles Draft 6 (Included in WG 11 N 16061 Text of ISO/IEC FDIS 23008-8:201x 2nd Edition) [T. Suzuki, K. Kazui (editors)] [2016-04-22] (8 weeks)

(Merged into a new edition)

JCTVC-W1013 Reference software for Scalable HEVC (SHVC) Extensions Draft 4 (Merged into WG 11 N 16055 Text of ISO/IEC FDIS 23008-5:201x Reference Software for High Efficiency Video Coding [2nd ed.]) [Y. He, V. Seregin (editors)] [2016-04-22] (8 weeks)

(Merged into a new edition)

JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 16049) [2016-04-30] (near next meeting)

Remains valid – not updated: JCTVC-U1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)]

JCTVC-W1016 Conformance Testing for HEVC Screen Content Coding (SCC) Extensions Draft 1 (MPEG WD) [R. Joshi, J. Xu] (6 weeks)

JCTVC-W1017 Conversion and Coding Practices for HDR/WCG Video [J. Samuelsson] (now)

(Using local activity for QP setting as well as luma)

JCTVC-W1018 Verification Test Plan for HDR/WCG Video Coding Using HEVC Main 10 Profile [R. Sjöberg, V. Baroncini, A. K. Ramasubramonian] [2016-03-09] (2 weeks)

(with the existing HEVC standard)

JCTVC-W1020 CTC for HDR/WCG video coding experiments [E. François, J. Sole, J. Strom, P. Yin] (now)

Remains valid – not updated: JCTVC-L1100 Common Test Conditions and Software Reference Configurations for HM [F. Bossen (editor)]

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Fri. 14 – Fri. 21 Oct. 2016, 25th meeting under WG 11 auspices in Chengdu, CN.

- Thu. 12 – Wed. 18 Jan. 2017, 26th meeting under ITU-T auspices in Geneva, CH.

- Fri. 31 Mar. – Fri. 7 Apr. 2017, 27th meeting under WG 11 auspices in Hobart, AU.

- …

The agreed document deadline for the 25th JCT-VC meeting is XXday XX Oct 2016. Plans for scheduling of agenda items within that meeting remained TBA.

ITU was thanked for the excellent hosting of the 24th meeting of the JCT-VC.

Companies that provided equipment [Technicolor, EBU] that was used for subjective viewing of video – Technicolor and EBU, were also thanked.

The JCT-VC meeting was closed at approximately 1XXX hours on Wednesday, 01 June 2016.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the twenty-fourth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately 159 people in total), were as follows:

- …

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.