| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the 25th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Chengdu, CN, 14–21 October 2016 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-fifth meeting during 14–21 October 2016 at the California Garden Hotel, Chengdu, CN. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 14 October 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 21 October 2016. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ISO/IEC JTC 1/SC 29/WG 11 (MPEG) – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information.

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-fourth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 Update 6 encoder description, and an update of HM common test conditions (CTC);

- Draft text 1 of picture repetition signaling in HEVC;

- For the scalable extensions (SHVC), an update of common test conditions, and a supplemental verification test report;

- For the HEVC screen content coding (SCC) extensions, a description of common test conditions (CTC), reference software draft 2, conformance testing draft 2, and a draft verification test plan.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 2, a technical report text of conversion and coding practices for HDR/WCG video draft 2, a verification test report for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

The other most important goals were to review the work on High Dynamic Range (HDR) and wide colour gamut (WCG) video coding, and review other technical input documents. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC and Screen Content Coding) was also a significant goal. Preparation of SCC verification tests was continued, and possible needs for corrections to the prior HEVC specification text were also considered.

The JCT-VC produced XX particularly important output documents from the meeting (update):

- The HEVC test model (HM) 16 Update 6 encoder description, and an update of HM common test conditions (CTC);

- Draft text 1 of picture repetition signalling in HEVC;

- For the scalable extensions (SHVC), an update of common test conditions, and a supplemental verification test report;

- For the HEVC screen content coding (SCC) extensions, a description of common test conditions (CTC), reference software draft 2, conformance testing draft 2, and a draft verification test plan.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 2, a technical report text of conversion and coding practices for HDR/WCG video draft 2, a verification test report for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for for Thu. 12 – Wed. 18 Jan. 2017 under ITU-T auspices in Geneva, CH, during Fri. 31 Mar. – Fri. 7 Apr. 2017 under WG 11 auspices in Hobart, AU, and during ….

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twenty-fifth meeting during 14–21 October 2016 at the California Garden Hotel, Chengdu, CN. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 14 October 2016. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 21 October 2016. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of ISO/IEC JTC 1/SC 29/WG 11 (MPEG) – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

- 22nd "V" meeting (Geneva, 2015-10)		155 people, 75 input documents

- 23rd "W" meeting (San Diego, 2016-02)		159 people, 125 input documents

- 24th "X" meeting (Geneva, 2016-05/06)		162 people, 60 input documents

- 25th "Y" meeting (Chengdu, 2016-10)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2016_10_Y_Chengdu/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-fourth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 Update 6 encoder description, and an update of HM common test conditions (CTC);

- Draft text 1 of picture repetition signalling in HEVC;

- For the format range extensions (RExt), conformance testing draft 6 (including improved HEVC version 1 testing);

- For the scalable extensions (SHVC), an update of common test conditions, and a supplemental verification test report;

- For the HEVC screen content coding (SCC) extensions, a description of common test conditions (CTC), reference software draft 2, conformance testing draft 2, and a draft verification test plan.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 2, a technical report text of conversion and coding practices for HDR/WCG video draft 2, a verification test report for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

The other most important goals were to review the work on High Dynamic Range (HDR) and wide colour gamut (WCG) video coding, and review other technical input documents. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC and Screen Content Coding) was also a significant goal. Preparation of SCC verification tests was started, and possible needs for corrections to the prior HEVC specification text were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 5 October 2016.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Thursday 6 October 2016 were considered "officially late".

All contribution documents with registration numbers JCTVC-Y0040 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). An exception is JCTVC-Y0043 which had been previously registered and uploaded as JCTVC-Y0029 but was lost due to an error in the document management system. No official break-out activity reports were generated during this meeting (update).

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-Y00XX (a proposal of …) [uploaded 10-XX]

- …

The following technical design proposal contributions were both registered late and uploaded late:

- JCTVC-Y00XX (a proposal of …) [uploaded 10-XX]

- …

The following other documents were registered on time but were uploaded late:

- JCTVC-Y00XX (an information document on …) [uploaded 10-XX]

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-Y00XX [uploaded 10-XX], … .

(Documents that were both registered late and uploaded late, other than technical proposal documents, are not listed in this section, in the interest of brevity.)

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-Y00XX, …

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting. The initial uploads of the such contribution documents are rejected as "placeholders" if they are uploaded without any significant content and are not corrected until after the upload deadline: JCTVC-X00XX [improved on 05-XX], … .

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, uploading of corrupted unreadable files, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload, along with a record of uploading times.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-X1000, the HEVC Test Model 16 (HM16) Update 6 JCTVC-X1002, Common Test Conditions for HM JCTVC-X1100, the draft text 2 for ICTCP support in HEVC JCTVC-X1003, the draft text 1 for Picture Repetition Indication in HEVC JCTVC-X1005, the draft Cerification Test Plan for SCC extensions JCTVC-X1006, the SHVC supplemental Verification Test Report JCTVC-X1004, the SHVC Common Test Conditions JCTVC-X1009, the SCC Reference Software Draft 2 JCTVC-X1011, the SCC Conformance Testing Draft 1 JCTVC-X1016, SCC Common Test Conditions JCTVC-X1015, draft 2 of Conversion and Coding Practices for HDR/WCG Video JCTVC-X1017, Common Test Conditions for HDR/WCG JCTVC-X1020, and the Verification Test Report for HDR/WCG Video Coding Using HEVC Main 10 Profile JCTVC-X1018, were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group was initially asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable basic description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities (none for this meeting)

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of video coding technology and supplemental enhancement information proposal contributions

- Consideration of contributions on the development of conformance test sets, reference software, verification testing, and non-normative guidance information

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages, and subscribers must respond adequately to basic inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the JCT-VC email reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the headers and registrations of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- 3D-HEVC: A set of extensions of HEVC that includes the combined coding of depth and texture information for 3D video coding.

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: May refer either to block copy (see CPR or IBC) or backward compatibility. In the case of backward compatibility, this often refers to what is more formally called forward compatibility.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (MV used for intra BC prediction, not a term used in the standard).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment for which there is a draft design and associated test model software that have been established, e.g., as in experiments conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE, and TE).

- CGS: Colour gamut scalability (historically, also coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector sometimes called a block vector, in a manner basically the same as motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions – a set of agreed conditions for coding experiments.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element of AVC or HEVC).

- EL: Enhancement layer.

- ET: Encoding time.

- ETM: Experimental test model (design and software used for prior HDR/WCG coding experiments in MPEG).

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HDR: High dynamic range – referring to video content having a brightness range that includes values greater than approximately 100 nits (often implicitly including WCG as well, since HDR video is typically also WCG video).

- HDR10: A term that refers to the single-layer coding of HDR/WCG video content using the HEVC Main 10 profile with a Y′CbCr 4:2:0 10 bit per sample colour representation with ITU-R BT.2020 colour primaries and the PQ transfer characteristics EOTF.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized in ITU-T as Rec. ITU-T H.265 and in ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – the draft reference software and its (non-normative) encoder algorithms used for HEVC experiments.

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (esp. 8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (esp. 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (as in AVC and HEVC).

- JM: Joint model – the primary software codebase and associated (non-normative) encoding algorithms that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network element.

- MC: Motion compensation.

- MOS: Mean opinion score – a measurement of subjective video quality as reported by human test subjects.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector; alternatively, multiview.

- MV-HEVC: A set of extensions of HEVC using layered coding to enable the coding of video with multiple views or depth maps.

- NAL: Network abstraction layer (as in AVC and HEVC, contrast with VCL).

- NCL: Non-constant luminance, a type of colour difference representation.

- Nits: Candelas per square metre (cd/m2).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g., light input to a camera) to output light (e.g., light emitted by a display).

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- PDTR: Proposed draft technical report – the draft of a TR that is sent for a ballot in the ISO/IEC approval process.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- PQ: Perceptual quantization – the name given to an HDR EOTF curve specified in SMPTE ST 2084 and Rec. ITU-R BT.2100.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RExt: Format range extensions – a set of extensions of HEVC addressing high bit rate operation, high bit depths, and alternative chroma formats such as monochrome, 4:2:2, 4:4:4, high bit depths, and high throughput.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment (for SHVC).

- SCCE: Screen content core experiment (for SCC).

- SCM: Screen coding model (for SCC).

- SD: Slice data; alternatively, standard-definition.

- SDR: Standard dynamic range – referring to video content having a brightness range that would produce a maximum brightness of approximately 100 nits on a reference display under reference viewing conditions.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM (for SHVC).

- SHVC: Scalable high efficiency video coding – a set of extensions of HEVC that uses layered coding to enable the coding of supplemental pictures, quality enhancement layers, spatial resolution enhancement layers, and colour gamut enhancement layers.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- Supplement: In ITU-T terminology, a document that assists its readers by providing non-normative information and suggestions (sometimes considered a TR in ISO/IEC terminology).

- SVC: Scalable video coding, especially when referring to the associated extensions of AVC.

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design at a more preliminary stage of work than those of CEs, e.g., as between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TR: Technical report – e.g., a collection of non-normative suggestion guidance on appropriate technical practices (sometimes considered a “supplement” in ITU-T terminology).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VCL: Video coding layer (as in AVC and HEVC, contrast with NAL).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut – referring to video content having a colour gamut that includes colours substantially outside of the range of values that is representable using Rec. ITU-R BT.709.

- WD: Working draft – a term for a draft standard, especially one prior to its first ballot in the ISO/IEC approval process, although the term is sometimes used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

## Liaison activity

The JCT-VC did not directly send or receive formal liaison communications at this meeting. However, there was relevant liaison communication at the parent-body level; see section 7.2.

## Opening remarks (update)

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

- Take up some remaining work from JCT-3V (corrigendum and proposed SEI message)

It was noted that for the ICIP “Grand Challenge on Still-Image Coding”, HEVC SCC (submitted for consideration by Nokia) was judged the best submission (add link).

Primary topic areas were noted as follows:

- Screen content coding

  - Software [Code cleanup complete to become HM 17?]

  - Conformance

  - Verification testing

- HDR

  - Development of TR on HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics

  - ICTCP support

  - Possible other SEI & VUI

  - Possible work on additional TR on HDR/WCG

    - backward-compatibility (single-layer / multi-layer)

    - dynamic metadata (e.g., “reshaping”, display adaptation, quality enhancement post-processing)

    - ICTCP usage

    - HLG

  - Reference software

- Corrigenda items for version 4

- Test model texts and software manuals

Key deliverables initially planned from this meeting (update):

- SCC Reference software [Code cleanup complete to become HM 17?]

- SCC Conformance

- SCC Verification test plan

- Frame repetition signalling?

- HDR outputs

  - Suggested practices TR

  - CEs?

  - ICTCP support

- New HM, SHM, SCM document versions? HM17 with SCM integrated?

A single meeting track was followed for most meeting discussions.

## Scheduling of discussions

Scheduling: Generally, meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on Screen Content Coding CE work and related contributions during the first few days. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Fri. 14 Oct, 1st day

  - 0900–1300, Opening remarks, status review, AHG report review (GJS & JRO)

  - …

There were no requests in the closing plenary to present any remaining "TBP" contributions.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized as follows. Some plenary sessions were chaired by both co-chairmen, and others by only one. Chairing of other discussions is noted for particular topics.

- AHG reports (14) (section 2)

- Project development status (6) (section 3)

- Core experiments (0) (section 4)

- HDR coding (12) (section 5.1)

- High-level syntax (2) (section 5.2)

- VUI and SEI messages (6) (section 5.3)

- Non-normative, encoder optimization (1) (section 5.4)

- Withdrawn (1) (section 6)

- Plenary discussions (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

## Topics discussed in final wrap-up at the end of the meeting (update)

Notes on potential remainders near the end of the meeting:

- …

- Output preparations (see section 10 for full list)

  - …

- Plans

  - AHGs

  - CEs

  - Reflectors (jct-vc) & sites (test sequence location to be listed in CTC doc) to be used in future work

  - Meeting dates (Thu - Wed)

  - Doc deadline (Mon 10 days prior)

There were no requests to present any "TBP" contributions in the closing plenary.

# AHG reports (14)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Friday 14th, 09:00–13:00 except as noted.)

JCTVC-Y0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm] [miss]

Discussed Fri. 10-14 0930 (GJS & JRO)

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

At the 24th meeting of the ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC), an ad hoc group on Project Management was established with the following mandates:

- Coordinate overall JCT-VC interim efforts

- Report on project status to JCT-VC reflector

- Provide report to next meeting on project coordination status

The reflector used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

In the interim period since the 24th JCT-VC meeting, the following (13) documents had been produced:

- The HEVC test model (HM) 16 Update 6 encoder description, and an update of HM common test conditions (CTC);

NOTE: This was given a different version number than anticipated, since only a minor update was needed rather than the major update that had been discussed initially at the meeting.

- Draft text 1 of picture repetition signalling in HEVC;

- For the scalable extensions (SHVC), an update of common test conditions, and a supplemental verification test report;

- For the HEVC screen content coding (SCC) extensions, a description of common test conditions (CTC), reference software draft 2, conformance testing draft 2, and a draft verification test plan.

- For high dynamic range (HDR) and wide colour gamut (WCG) video coding extensions, a text amendment for ICTCP colour representation support in HEVC draft 2, a technical report text of conversion and coding practices for HDR/WCG video draft 2, a verification test report for HDR/WCG video coding using the HEVC Main 10 profile, and a description of common test conditions (CTC) for HDR/WCG video coding experiments.

The work of the JCT-VC overall had proceeded well and actively in the interim period with a considerable number of input documents to the current meeting. Active discussion had been carried out on the group email reflector (which had 1642 subscribers as of 2016-10-12), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2016_05_X_Geneva/), particularly including the following:

- The meeting report (JCTVC-X1000) [Posted 2016-10-13]

- The HM 16 Update 6 encoder description (JCTVC-X1002) [Posted 2016-10-11]

- Draft text 2 for ICTCP support in HEVC (JCTVC-X1003) [Posted 2016-06-11]

- Supplemental verification test report for scalable HEVC (JCTVC-X1004) [Posted 2016-07-01]

- Draft text 1 for picture repetition indication in HEVC (JCTVC-X1005) [Posted 2016-06-11]

- Draft verification test plan for HEVC SCC extensions (JCTVC-X1006) [Posted 2016-10-13]

- Common test conditions for SHVC (JCTVC-X1009) [First posted 2016-06-30, last updated 2016-07-01]

- HEVC Reference Software for Screen Content Coding Draft 2 (JCTVC-X1011) [First posted 2016-07-30, last updated 2016-08-01]

- Common test conditions for SCC (JCTVC-X1015) [First posted 2016-07-18, last updated 2016-08-14]

- Conformance testing for HEVC Screen Content Coding (SCC) Extensions Draft 2 (JCTVC-X1016) [Posted 2016-10-13]

- Conversion and coding practices for HDR/WCG video, Draft 2 (JCTVC-X1017) [First posted 2016-06-03, last updated 2016-10-11]

- Verification test report for HDR/WCG video coding Using HEVC Main 10 Profile (JCTVC-X1018) [Posted 2016-07-11]

- Common Test Conditions for HDR/WCG Video Coding Experiments (JCTVC-X1020) [Posted 2016-07-11]

- Common test conditions for HM (JCTVC-X1100) [Posted 2016-06-16]

The fourteen ad hoc groups had made progress, and reports from those activities had been submitted.

The different software modules (HM16.9+SCM8.0, SHM12.1) had been prepared and released with appropriate updates approximately as scheduled.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-16.9.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-16.9-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 27 input contributions to the current meeting had been registered. Approximately half of these relates to coding of HDR video using HEVC. Some late-registered and late-uploaded contributions were noted.

A preliminary basis for the document subject allocation and meeting notes for the 25th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

On the ballot for JCTVC-X1011 / w16270, Text of ISO/IEC 23008-5:201x/DAM1 Reference Software for Screen Content Coding Profiles, Late registration of the ballot (despite on-time delivery by the editors) has delayed the closing date beyond the January meeting. (SC 29 N 16072; Ballot closing 2017-02-01).

There was a LC comment submitted to ITU-T SG 16: Microsoft Submitted a comment 08-09 requesting

1) bug fix for chroma delta QP,

2) encoder support for SPS-level palette predictor syntax,

3) any other improvements.

There was an LC comment submitted to ITU-T SG 16: Comments submitted 10-12 by Microsoft:

- ICTCP

- The precise scaling factor associated with video_full_range_flag for transfer_characteristics equal to 16 and 18 (See also JCTVC-Y0039)

- Check “inverse transform”

- Any other editorial issues noticed

Generally fewer contributions to this meeting than in the past.

JCTVC-Y0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. Sullivan, Y.-K. Wang (vice-chairs)]

----Discussed Fri. 10-14 1000 (GJS & JRO)

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 24th meeting in Geneva, CH (May/June 2016) and the 25th meeting in Chengdu, CN (October 2016).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The ‘High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 6 of Encoder Description’ was published as JCTVC-X1002. This document represented a refinement of the previous HM16 Update 5 of the Encoder Description document (JCTVC-W1002). The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

During this cycle, effort was focused on updating the description of the rate control functionality in HM to reflect the current software status.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Review the list of bug fixes collected for HEVC Edition 3, and include all confirmed bug fixes, including the outcome of the above items, if any, into a JCT-VC output document for the purpose of HEVC Edition 3 defect reporting.

See also notes for Y0001 on two errata issues.

It was also remarked that a problem had been identified in the 3D-HEVC text, which should be studied for correction.

JCTVC-Y0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring (chair), K. Sharman, A. Tourapis (vice-chairs)]

-Discussed Fri. 10-14 1010 (GJS & JRO)

This report summarises the activities of the AhG on HEVC HM and HDRTools software development and software technical evaluation that have taken place between the 24th and 25th JCT-VC meetings. Activities focused on integration of software adoptions and software maintenance, i.e. code tidying and fixing bugs.

A brief summary of activities related to each mandate for the HM is given below.

- HM16.10 was released during the previous meeting.

- Changed configuration files to match the new common test conditions document.

- Back-ported the SIMD vector coding from JEM; enabled by default in HM16.11.

- Integrated “Anchor 3.2” processing capability in the encoder, including multiple PPS support; released in HM16.12.

- Replaced JCTVC-W0062 with integrated JCTVC-X0038 in the HM-development branch; the GOP configuration syntax has now changed.

- In addition, some minor bug fixes and cleanups were addressed. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector, and http://hevc.info has been updated.

- There are a number of reported software bugs that should be fixed.

For HDRTools, the following activities were performed:

- V0.12 was released during the previous meeting.

- The configuration conversion files were modified to match the current recommended practices document  and common test conditions documents.

- Created V0.13-dev branch with the following changes compared to V0.12.

  - Added a Look-Up Table (LUT) based method for the derivative TF computation. Method is used for fast luma adjustment computations.

  - Fixed SDR conversion issues.

  - Fixed some scaling issues.

  - Added 64 bit compilation support under Visual Studio.

  - Added xPSNR computation for YCbCr content.

- Scaling support in the software is still limited and there are issues identified mostly for the scaling of YCbCr content. These should be addressed as well as support for generalized scaling should be provided at some point.

HM16.10 was tagged on 28 May during the previous meeting. The changes can be found in JCTVC-X0003.

HM16.11 was tagged on 13 June and announced on the reflector. It included the following modifications:

- Changed random-access configuration files to use GOP16.

- Back-port of the SIMD vector coding from JEM, enabled by default on many systems that support SSE2.

- Some minor bug-fixes for interlace.

HM16.11 was produced quickly as it was tied with the release of the latest common test conditions document, JCTVC-X1100.

Results of the two changes were released on the reflector. The table below summarizes the changes due to the increase of the random-access GOP structure from 8 to 16:

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -4.4% | -4.2% | -4.0% | -4.4% | -4.2% | -4.8% |
| Class B | -6.0% | -8.4% | -10.5% | -5.9% | -9.1% | -11.4% |
| Class C | -5.4% | -5.9% | -5.7% | -5.3% | -6.7% | -6.6% |
| Class D | -5.1% | -7.7% | -7.7% | -5.0% | -8.2% | -8.1% |
| Class E |  |  |  |  |  |  |
| Class F | -5.7% | -6.2% | -5.7% | -5.7% | -6.6% | -6.5% |
| Overall | -5.3% | -6.6% | -6.9% | -5.3% | -7.0% | -7.7% |
|  | -5.1% | -6.5% | -6.9% | -5.1% | -7.0% | -7.6% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

The changes in run-time (there are no changes in bit-rate / PSNR) caused by the back-port of the SIMD vector coding (on X86) are:

|  | Main (8-bit) | HE10 (10-bit) |
| --- | --- | --- |
|  | Encoding time | Encoding time |
| All Intra | 94% | 94% |
| Random access | 73% | 74% |
| Low delay B | 70% | 70% |
| Low delay P | 77% | 77% |

It was recommended by the AHG that JEM is compared against at least HM16.11 when gauging complexity increase (or JEM’s vector coding is disabled when comparing against earlier revisions of HM).

HM16.12 was tagged on 27 June and later announced on the reflector; the emphasis being the integration of “Anchor 3.2” HDR capabilities to aid HDR studies. The changes were

- Addition of the ability to define multiple PPS within the encoder.

- Added luma-based modulation of QP within the encoder.

- Added chroma-QP-offset modulation using multiple PPS within the encoder.

- Minor fix of some Visual Studio project configurations.

The “Anchor 3.2” patch was modified as it had been designed for specific test conditions, which would have prevented certain features being usable in HM; the handling of multiple PPS was also significantly altered to allow more generic control.

There were no changes to the CTCs in HM16.12 relative to HM16.11.

At the previous JCTVC meeting it was agreed that JCTVC-X0038 should be integrated (but be disabled via command line options), replacing JCTVC-W0062.

A suitable patch was finally agreed and added to the HM-dev development branch. Although, as agreed, it was disabled by default, there is a tiny coding improvement in the CTC for HM16.12 due to a simplified method of deciding the flag collocated_from_l0_flag.

Integration of JCTVC-X0038 still requires review by the JCT-VC, and it should be decided what happens to the other lambda controls if X0038 is to be always enabled. I.e. should “RecalculateQPAccordingToLambda” and the individual or gop-entry lambda modifiers be removed?

To recap, the new derivation process in X0038 is:

If sliceType == I_SLICE

QPslice = QPbase + QPintraOffset

else

QP’ = QPbase + QPgopEntryOffset[gopIndex]

QPslice = QP’ + floor(Clip3(0, 3, QP’*scale[gopIndex]+offset[gopIndex]+0.5) )

Lambda is the calculated for QPslice (with optional lambda modifiers, although these are not required in the supplied configurations).

In HM, QPslice is only a function of QPbase and QPgopEntryOffset. The lambda is calculated from the QPslice, but adjusted on a per-gop entry basis. Proposals have been made to then recalculate the QP from this lambda (i.e. JCTVC-J0242 [“RecalculateQPAccordingToLambda”] and JCTVC-W0062), but these were commented as being potentially confusing to the user as it was not clear what the user was controlling.

When X0038 is enabled, the following results are observed (relative to X0038 being disabled via command line options in HM-dev):

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) | -3.0% | -4.3% | -5.8% | -3.0% | -4.7% | -5.8% |
| Class B | -2.1% | -5.8% | -7.6% | -2.4% | -6.2% | -7.8% |
| Class C | -3.2% | -7.2% | -7.7% | -3.2% | -7.1% | -7.4% |
| Class D | -2.5% | -7.3% | -7.6% | -2.6% | -7.5% | -7.8% |
| Class E |  |  |  |  |  |  |
| Overall | -2.7% | -5.9% | -6.8% | -2.8% | -6.2% | -6.7% |
|  | -2.7% | -5.7% | -6.5% | -2.8% | -5.9% | -6.5% |
| Class F | -3.0% | -4.2% | -4.4% | -3.1% | -4.6% | -4.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | -1.7% | -5.5% | -5.7% | -1.7% | -5.2% | -5.6% |
| Class C | -2.0% | -6.0% | -6.6% | -2.0% | -6.1% | -6.4% |
| Class D | -1.6% | -5.4% | -5.9% | -1.5% | -5.5% | -5.7% |
| Class E | -2.8% | -5.3% | -3.8% | -2.7% | -6.3% | -5.5% |
| Overall | -2.0% | -5.6% | -5.5% | -1.9% | -5.5% | -5.9% |
|  | -2.0% | -5.3% | -5.2% | -1.9% | -5.1% | -5.5% |
| Class F | -2.1% | -5.6% | -4.8% | -2.0% | -4.5% | -6.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8 bit) |  |  |  |  |  |  |
| Class B | -1.8% | -5.4% | -5.8% | -1.8% | -5.5% | -5.8% |
| Class C | -2.0% | -6.1% | -6.5% | -1.9% | -5.9% | -6.5% |
| Class D | -1.6% | -5.5% | -6.1% | -1.6% | -5.5% | -5.4% |
| Class E | -3.2% | -6.2% | -4.3% | -3.2% | -6.8% | -5.7% |
| Overall | -2.0% | -5.6% | -5.5% | -2.0% | -5.7% | -5.8% |
|  | -2.0% | -5.3% | -5.1% | -2.1% | -5.3% | -5.4% |
| Class F | -1.9% | -4.9% | -4.5% | -2.0% | -5.0% | -5.6% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Similar results are observed for the RExt general test conditions and the RExt SCC lossy test conditions. Little change is observed for the RExt high bit depth and SCC lossless, but some degradation is visible for the high bit depth 12-bit low-delay HDR test sequence for the high bit depth lossy test configuration.

Should X0038 be enabled by default? Decision (SW): Yes.

Should “RecalculateQPAccordingToLambda” be removed? Decision (SW): Yes.

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA:

Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices:

Tickets #1314, #1338, #1339.

- Field-coding:

Tickets #1145, #1153.

- Decoded picture buffer:

Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing:

Tickets #1335, #1336, #1393.

- Additional decoder checks:

Tickets #1367, #1383.

However, a patch has been generated that adds some conformance checks. It is being considered for potential inclusion in a future release.

As described to the community at the last four JCT-VC meetings, alterations to remove the unused software hierarchy in the entropy coding sections of the code, and to remove terms such as CAVLC is being considered. However, this will now need to also consider the impact on the JEM branch.

Version 0.12 of HDRTools was tagged on June 29th, 2016 (during the previous meeting).

A new branch v0.13-dev was created for the continuing development of the HDRTools software. Currently this new branch contains the following modifications versus v0.12:

- Added a Look-Up Table (LUT) based method for the derivative TF computation (see JCTVC-X0072). This method is used for fast luma adjustment computations.

- Fixed SDR conversion bugs.

- Fixed some down/upsampling scaling issues.

- Added 64 bit compilation support under Visual Studio.

- Added xPSNR (cross-component PSNR) computation for YCbCr content (see also JCTVC-Y0037).

It has been identified that the software still only supports very limited down/upsampling support, especially when using YCbCr content. This might be fixed in a future version, but likely not in v0.13.

Support also of full scaling in HDRTools has been requested and is currently under study.

Recommendations of the AHG:

- Continue to develop reference software based on HM version 16.12 and HDRTools v0.12 and improve its quality.

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bitstreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

- Continue to investigate the merging of branches with the other software coordinators.

See AHG8 report regarding SCC software status.

It was remarked that support for the new High Throughput profile is needed in the software, and that work has taken place to develop that.

JCTVC-Y0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki (chair), J. Boyce, R. Joshi, K. Kazui, A. Ramasubramonian, Y. Ye (vice-chairs)]

-Discussed Fri. 10-14 1040 (GJS & JRO)

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

There was no updates from the last JCTVC meeting. All known problems were resolved. The latest  bitstreams are available at the following site.

http://wftp3.itu.int/av-arch/jctvc-site/draft_conformance/

The report included tables describing the available bitstreams.

The status of SCC conformance was updated, relative to prior reports.

A significant amout of further work for SCC conformance is needed.

One relevant input contribution was noted, on the High Throughput profile, in contribution JCTVC-Y0047.

JCTVC-Y0005 JCT-VC AHG report: SHVC verification test reporting (AHG5) [V. Baroncini, W.J. Husak, A.K. Ramasubramonian, Y.-K. Wang, Y. Ye (co-chairs)]

-Discussed Fri. 10-14 11500 (GJS & JRO)

Discussed verbally before upload. The verification test report was completed.

JCTVC-Y0006 JCT-VC AHG report: SCC extensions verification testing (AHG6) [H. Yu, V. Baroncini (co-chairs), R. Cohen, A. Duenas, J. Xu, X. Xu (vice-chairs)]

--Discussed Fri. 10-14 1130 (GJS & JRO)

This report summarizes the activities of the JCT-VC ad hoc group on SCC extensions verification testing (AHG6) between the JCT-VC 24th meeting in Geneva, Switzerland, and the 25th meeting in Chengdu, China.

There were a few emails on the reflector discussing the coding results of the new SCC sequences submitted in JVET-C0044. Basically these sequences are typical screen content that were captured from computer display. They exhibit text and graphics in motion that we commonly see in cloud/cloud-mobile computing, remote desktop, PC-over-IP, and interactive TV applications. More information about these sequences as well as the coding test results is available in JVET-C0044 and JCTVC-Y0036.

A draft test plan JCTVC-X1006 is submitted. It provides a list of the candidate sequences for the verification tests. These are all the non-CTC sequences available today for screen content. The final selection of test material is expected to be made in Chengdu. The draft test plan proposes the following test condition:

- Software: SCM-8.1, HM-16.12, JM-19.0

- 8 test cases: 4:4:4 lossless, 4:2:0 lossless, 4:4:4 AI/RA/LB, and 4:2:0 AI/RA/LB

- Test points: 4 QPs for SCM, i.e. QP=22, 27, 32, and 37.

It also provides a DCR-based procedure for subjection evaluation.

Open issues:

- Timeline

- Test material selection

- Decision on color space, YUV only or RGB only or both

- Decision on color sampling format, 4:4:4 only or both 4:4:4 and 4:2:0

- Selection and decision on subjective evaluation test points. It may not be practical to run subjective test for all “Lossy” test points.

- Matching bit-rates or QP values for HM and JM?

It is recommended to discuss the open issues above, select test material, provide recommendations for bitstream generation, subjective test points, and timeline.

JCTVC-Y0007 JCT-VC AHG report: Content colour volume representation (AHG7) [H. M. Oh, A. K. Ramasubramonian, A. Tourapis (co-chairs)]

-Discussed Fri. 10-14 1145 (GJS & JRO)

This document provides a report for the AhG7 activity on content colour volume representation. This report contains the mandates, summary of the AhG activities, and a list of AhG-related input contributions to the 25th JCT-VC meeting, and provides recommendations.

During the 24rd JCTVC meeting, three proposals, JCTVC-X0040, JCTVC-X0052 and JCTVC-X0069, were presented requesting the creation of new SEI messages that would provide a description of the content colour volume representation. It was agreed, in principle, that such an SEI message was highly desirable and potentially useful, and AhG7 was formed to provide a combined syntax using the three proposals.

It was reported that different approaches for colour volume representation were studied and that a combined syntax was proposed in JCTVC-Y0040 by some participants. Contribution JCTVC-Y0032 proposes a simpler scheme than JCTVC-Y0040.

JCTVC-Y0008 JCT-VC AHG report: Screen content extensions software development (AHG8) [K. Rapaka, B. Li (co-chairs), R. Cohen, X. Xiu, T.-D. Chuang (vice-chairs)]

-Deferred.

This report summarizes the activities of Ad Hoc Group 8 on screen content extension software (SCM) developments that have taken place between the JCT-VC 24th meeting in Geneva, Switzerland, and the 25th meeting in Chengdu, China.

Seven software revisions (HM-16.8+SCM-8.0, HM-16.9+SCM-8.0, HM-16.10+SCM-8.0, HM-16.10+SCM-8.1, HM-16.11+SCM-8.1, HM-16.12+SCM-8.1, and HM-16.12+SCM-8.2) were produced. The integration details and performance summary is provided below. The performance results of the software revision was observed to be consistent with the adopted techniques.

[add further notes]

JCTVC-Y0009 JCT-VC AHG report: SHVC software development (AHG9) [V. Seregin, Y. He, G. Barroux]

Discussed Fri. 10-14 1155 (GJS & JRO)

This report summarizes activities of the AHG9 on SHVC software development between 24th and 25h JCT-VC meetings.

The latest software version is SHM-12.1.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

The latest version is SHM-12.1, which was updated to HM-16.10 software base, and was used to generate new anchor data for the increased GOP size. The common test condition was updated as well to reflect this change and can be found in JCTVC-X1009.

A comparison of the increased GOP size from 8 to 16 for RA configuration is summarized in the below table and can be found in more detail in the accompanying Excel tables.

SHM-12.1 against SHM-12.0 under common test conditions (HEVC base layer)

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall (Test vs single layer) | 12.8% | 14.8% | 14.6% | 10.5% | 9.7% | 9.3% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.8% | 14.6% | 10.5% | 9.7% | 9.3% |  |  |  |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 107.0% |  |  | 105.8% |  |  |  |  |  |
| Dec Time[%] | 105.1% |  |  | 107.2% |  |  |  |  |  |
| Enc Mem[%] | 99.5% |  |  | 99.0% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -4.8% | -2.7% | -3.7% |  |  |  | -3.7% | -1.5% | -2.7% |
| Class B | -5.0% | -5.6% | -7.7% | -5.0% | -5.5% | -7.7% | -3.7% | -3.5% | -5.6% |
| Overall (Test vs Ref) | -5.0% | -4.8% | -6.5% | -5.0% | -5.5% | -7.7% | -3.7% | -2.9% | -4.7% |
| Overall (Test vs single layer) | 13.4% | 25.3% | 23.7% | 10.4% | 19.3% | 18.8% | 10.2% | 26.3% | 28.1% |
| Overall (Ref vs single layer) | 19.2% | 31.0% | 31.9% | 16.1% | 26.2% | 28.8% | 14.5% | 29.9% | 34.6% |
| EL only (Test vs Ref) | -4.6% | -4.5% | -6.4% | -4.0% | -4.8% | -7.3% | -2.0% | -1.2% | -3.1% |
| Enc Time[%] | 110.0% |  |  | 115.6% |  |  | 98.1% |  |  |
| Dec Time[%] | 103.7% |  |  | 107.3% |  |  | 96.7% |  |  |
| Enc Mem[%] | 158.0% |  |  | 157.8% |  |  | 158.6% |  |  |
| BL Match | Not matched |  |  | Not matched |  |  | Not matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 28.3% | 38.7% | 39.9% | 24.7% | 33.0% | 35.5% | 24.3% | 34.8% | 39.5% |
| Overall (Ref vs single layer) | 28.3% | 38.7% | 39.9% | 24.7% | 33.0% | 35.5% | 24.3% | 34.8% | 39.5% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103.1% |  |  | 99.2% |  |  | 99.1% |  |  |
| Dec Time[%] | 100.4% |  |  | 100.4% |  |  | 98.3% |  |  |
| Enc Mem[%] | 99.7% |  |  | 99.4% |  |  | 99.9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 26.5% | 37.8% | 39.4% | 22.8% | 32.6% | 35.4% | 23.4% | 34.7% | 39.1% |
| Overall (Ref vs single layer) | 26.5% | 37.8% | 39.4% | 22.8% | 32.6% | 35.4% | 23.4% | 34.7% | 39.1% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104.3% |  |  | 90.8% |  |  | 89.1% |  |  |
| Dec Time[%] | 102.1% |  |  | 96.8% |  |  | 93.0% |  |  |
| Enc Mem[%] | 99.8% |  |  | 99.7% |  |  | 99.7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Development plan and recommendations

- Continue to develop reference software based on SHM-12.1 and improve its quality.

- Fix open tickets.

JCTVC-Y0010 JCT-VC AHG report: Test sequence material (AHG10) [T. Suzuki, V. Baroncini, R. Cohen (co-chairs), E. Francois, T. K. Tan, P. Topiwala, S. Wenger, H. Yu (vice-chairs)]

--

Discussed Fri. 10-14 1205 (GJS & JRO)

It was reported that some of the test sequences in CTC use video full range and exceed gamut Those exceeds the limitation of ITU-R BT.709 or BT.2020. If further testing is necessary for JCTVC, it should be clarified.

The report said:

“From these, we cam say that Cactus, ChinaSpeed, and SlideShow are definitely full range content. For BasketballDrillText, unfortunately whoever created the content ignored the fact that the content was in limited range and added the text in full range. I guess though that this was only text, one could assume that the text is in limited range and just ignore the original intent of the one who created it.

There is a considerable number of clips that contain out of gamut values, but this is likely due to some processing done on the content (e.g. scaling or poor conversion). Out of the remaining clips, Kimono and ParkScene have some indications that they might be in full range but we have not carefully looked at these sequences. Given the Y minimum there is an indication that they might not be, but the number of pixels that were out of gamut for Kimono seemed a bit too high to me from what I recall.”

Sharp reported that Cactus and the Class F sequences definitely have areas that are substantially outside of “video range”.

The AHG report lists CTC test sequences and some V1 verification test sequences. Some of the V1 verification test sequences are not on our ftp site.

Notes about violations of video range should be added in a future revision.

A contribution JVET-D0083 to JVET provides three HDR test sequences and one SDR test sequence. There were also some other test sequences previously submitted to JVET, and some more arriving there. To avoid duplication of efforts, JCT-VC will just track CTC (and perhaps some VT) test sequences.

HDR/WCG visual testing (AHG11) V. Baroncini, P. Topiwala, E. Alshina (co-chairs)

Deferred.

HDR/WCG verification test reporting (AHG12) V. Baroncini (chair), K. Andersson, W. Husak, A. K. Ramasubramonian, G. Sullivan (co-chairs)

Discussed Fri. 10-14 1400 (GJS & JRO)

This document provides a report for the AHG12 activity on HDR/WCG verification test reporting. It is reported that the test report for HDR/WCG video coding using HEVC Main 10 Profile has been finalized and uploaded.

The AhG produced the final verification test report for HDR/WCG video coding using HEVC Main 10 Profile in document JCTVC-X1018. The report was uploaded on July 11, 2016.

JCTVC-Y0013 JCT-VC AHG report: HDR/WCG conversion and coding practices report development for PQ Y′CbCr 4:2:0 (AHG13) [J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Strom, G. J. Sullivan, P. Topiwala, A. Tourapis, P. Yin]

Discussed Fri. 10-14 1410 (GJS & JRO)

This document reports the activity of the ad hoc group on HDR/WCG coding practices guideline development (AHG13). The report presents the mandates of the AHG and a list of  input contributions that are relevant to the scope of the AHG.

The editors of “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics” have continued the editorial work on improving the text, which corresponds to the second mandate of the AHG. The latest version of the text is available both as JCTVC-X1017 and JCTVC-Y0046. Several input contributions provide results related to mandate 3.

Plan to issue PDTR in ISO from the current meeting, and reach ITU consent in January 2017.

Four relevant input contributions were noted: JCTVC-Y0030, JCTVC-Y0033, JCTVC-Y0034, JCTVC-Y0046.

The ad hoc group recommended

- To review all input contributions related to AHG13;

- To continue the editorial work of the technical report: “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics” during the JCT-VC meeting with the target of sending the report, at the end of this meeting or potentially after an additional editing period, to the parent bodies for approval and publication.

JCTVC-Y0014 JCT-VC AHG report: HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing (AHG14) [E. François, W. Husak, D. Rusanovskyy, P. Topiwala, P. Wu]

Discussed Fri. 10-14 1420 (GJS & JRO)

This document provides a report of the AhG14 activity on HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing (AHG14), conducted between the 24th and 25th JCT-VC meetings. This document reports the mandates, summary of AHG activities and email discussions, list of AHG-related input contributions to 25th JCTV meeting and recommendations.

There were 12 email messages sent to the reflector. Most of the discussion was related to the production of a tentative initial draft technical report (TR), covering mandates 1 to 4. Following email discussions, it was suggested to draft an initial text, listing technologies studied by JCT-VC and identified as properly addressing the functionalities of backward compatibility, and/or display adaptation, and/or quality enhancement post-processing.

In particular, the following documents were considered as a starting point:

- Documents related to usage of the CRI SEI message for dynamic range adaptation, also known as reshaping

  - JCTVC-X0041 (cross-check X0059)

  - JCTVC-X0060 (cross-check X0076)

  - JCTVC-X0066

  - JCTVC-X0084

- Documents related to usage of two-layer SHVC coding for backward compatibility

  - JCTVC-W0005

  - JCTVC-X0005

  - JCTVC-X0080

- Documents related to usage of ICTCP colour representation with HEVC

  - JCTVC-W0047

  - JCTVC-W0050

  - JCTVC-W1003

  - JCTVC-X0050 (cross-check X0057)

  - JCTVC-X1003

As a result, a first version of a draft TR was distributed on the JCT-VC email reflector on July 25, 2016. This version mainly addressed the first two categories above.

A second version was distributed on September 8, 2016, with added text to address ICTCP usage.

HLG transfer function usage has not yet been addressed. The latest draft TR shared on JCT-VC reflector mentions its signaling in VUI, but there is not yet any description on how to use the HLG transfer function with HEVC.

There was no specific discussion on test conditions. Common test conditions, as described in JCTVC-X1020, have been used for experiments related to AHG14.

The following new input contributions were reported to be related to AHG14, and were listed in the report.

- JCTVC-Y0029 proposed TR draft text

- JCTVC-Y0039 proposed ICTCP draft text

- JCTVC-Y0041 Chroma resampling (cross-check JCTVC-Y0044)

- JCTVC-Y0042 Tone mapping and related SEI messages (cross-check JCTVC-Y0045).

# Project development, status, and guidance (6)

## Corrigenda items (1)

See also the section discussing the AHG1 and AHG2 reports. It was also noted during discussions that variable names ER, EG, and EB in D.3.35 for CLL SEI message may need subscipting, and the intended domain of interpretation of the SEI message may need study.

JCTVC-Y0050 AHG2: Errata report for illumination compensation in 3D-HEVC [M. W. Park, C. Kim] [late]

## Profile/level definitions (1)

JCTVC-Y0043 HEVC Profile for Main 10 Still Picture [T. Toma, M. Kozuka (Panasonic), J. Boyce (Intel), A. Minezawa (Mitsubishi Electric), W. de Haan (Philips), H. Nishiwaki (Pioneer), T. Ikai (Sharp)]

## Conformance test set development (1)

See also the AHG report JCTVC-Y0004 and outputs … .

JCTVC-Y0047 High Throughput profile bitstreams for Conformance Testing [K. Rapaka, A. Tourapis] [late] [miss]

## HEVC coding performance, implementation demonstrations and design analysis (1)

JCTVC-Y0036 AHG6: Compression Performance Comparison of Thirteen TGM Sequences [S. Wang, J. Guo, L. Zhao, T. Lin (Tongji Univ.)]

## Software development (3)

JCTVC-Y0028 Support for 10-bit video in SHM DownConvert tool [Y. He, Y. He, Y. Ye (InterDigital)]

JCTVC-Y0037 Cross-Component Peak Signal to Noise Ratio (xPSNR) support in the HM and JEM software [Y.T. Peng, A.M. Tourapis, D. Singer, Y. Su (??)] [late] [miss]

JCTVC-Y0038 Closed GOP Support for the Random Access Common Conditions [Y.T. Peng, A.M. Tourapis, Y. Su, D. Singer (??)] [late]

## Source video test material (0)

## New application domains (0)

# Core experiments (0)

No CEs were run during the preceding meeting cycle.

# Technical contributions (21)

## HDR coding (12)

### Conversion and coding practices for HDR coding (1)

JCTVC-Y0046 Suggested new draft text of Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics [J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Strom, G. J. Sullivan, A. Tourapis] [late]

Discussed Fri. 10-14 1420 (GJS & JRO)

This document contains a draft of the conversion and coding practices technical report on HDR/WCG video.

This contribution has the same content as the X1017 output document.

Agreed changes (aside from minor editorial refinements):

- instead of referring to “Model A” and “Model B”, we should refer to the “naïve reference method” and “enhanced method” (or something similar).

- Move Annex B configuration description to HDR CTC document and revised VT report and remove it from the TR.

- Remove explicit JM and HM variable settings

- Add mention of “consumer distribution” target in Summary section.

- Don’t use “shall”.

Other SEI messages? Ambient light? Others?

Editors’ notes in the text – offline review/revisit.

### Resampling 4:2:0 issues and luma sample adjustment (6)

JCTVC-Y0033 On HDR 4:2:0 chroma subsampling (AHG13 related) [A. Norkin (Netflix)]

Discussed Fri. 10-14 1650 (GJS & JRO)

At the San Diego meeting, two algorithms were proposed for removal of color artefacts in saturated colorus of HDR video that appear in non-constant luminance Y'CbCr 4:2:0 color subsampling. The document reports the numbers for the algorithms performance then the LUT are used to speed-up the transfer functions and TF derivatives computation. It is reported that when implemented in the HDRTools software package, the proposed algorithms show the total running time of color conversion on the order of 150%–200% compared to the direct downsampling of chroma, whereas the iterative micro-grading algorithm shows running times of 430%–500%. In particular, Algorithm 2 is 2.6–2.8 times faster than the iterative micro-grading algorithm on average. The objective metrics performance of Algorithm 2 is reported to be similar to the iterative algorithm.

Slide deck to be uploaded in a revision.

This is an information contribution, not a proposal for action to include in the technical report.

No action was needed on this.

JCTVC-Y0030 AHG13 related: Multi-LUT Luma Adjustment Implementation [J. Ström (Ericsson)]

Discussed Fri. 10-14 1625 (GJS & JRO)

This contribution presents a way to implement luma adjustment using multiple look-up tables (LUTs). Trilinear interpolation is used, and in an attempt to reduce errors due to interpolation over discontinuities, different LUTs are employed for different cases, depending upon whether the output colour will clip any of its colour components. Seven 3D LUTs are used, indexed based on Cb, Cr and tfiYo, where the latter is the luminance of the colour fed through the inverse transfer function. The different tables have different dimensions, such as 32×32×4 values or 16×16×16 values. In total 99200 values are used for the LUTs and other tables, which the contribution states makes it feasible for video-rate hardware implementations. In an estimate of the accuracy, on one frame of the Market sequence it is claimed that the RMSE error in luma is 0.53 code levels and a maximum error in luma of 1.61 code levels.

Slide deck to be uploaded in a revision.

This is more of a hardware-oriented approach than something that would be used in software.

It was commented that more accuracy than this is not really needed, and also commented that the tables could be made even smaller without an important effect. Compression effects will outweigh small differences in this type of processing.

This is an information contribution, not a proposal for action to include in the technical report.

No action was needed on this.

JCTVC-Y0034 AHG13: Further results for luma sample adjustment [C. Rosewarne, V. Kolesnikov (Canon)]

TBP

The ‘Draft 2 of Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics’ (JCTVC-X1017) describes an iterative method known as ‘luma sample adjustment’ to address chroma leakage that my occur in the non-constant luminance (NCL) representation. Luma sample adjustment compensates for a shift in luminance that occurs e.g. when downsampling from 4:4:4 to 4:2:0 using NCL and YCbCr. The described method performs an iterative search to determine each luma sample value, which is asserted to be overly complex for real time implementation. This contribution shows a method for performing luma sample adjustment that replaces an iterative search with a fixed-complexity linearization method which is applied to predict the adjusted luma sample. This method is asserted to be an alternative to the ‘linearisation’ approach to luma sample adjustment presented in JCTVC-X0072 which does not require the values of EOTF derivative f ' (X).

The proposed method achieves the following tPSNR results of 53.80dB, 69.70dB, and 43.08dB in X, Y, and Z components, respectively. Note that in the Y component, this result is 0.07dB below that achieved by the iterative method of luma sample adjustment.

JCTVC-Y0049 A Crosscheck of JCTVC-Y0033 [T. Lu, F. Pu, P. Yin (Dolby)] [late] [miss]

JCTVC-Y0041 AHG14: Chroma Resampling Filters for HDR/WCG Coding [P. Topiwala, W. Dai, M. Krishnan (FastVDO)] [late]

Discussed Sunday 1100 (GJS & JRO)

A number of chroma resampling techniques used in the 4:4:4 to 4:2:0 and 4:2:0 to 4:4:4 conversions are studied in this paper. The filters studied are all part of the current HDRTools package. Since the combination of downsampling and upsampling is a highly lossy operation even without compression processing, this study looks at just the impact of chroma resampling in the HDR video coding processing chain, without compression.

Six filters were compared in terms of several objective metrics:

- One contributed by the submitter: 9 tap down with [-1 9 9 -1]/16 upsampling

- “Default Filter 1” [1 6 1]/8 with [-1 9 9 -1]/16 upsampling (in HDRTools and mentioned in draft TR)

- “Default Filter 2” [1 2 1]/4 with [-1 9 9 -1]/16 upsampling (in HDRTools and mentioned in draft TR)

- TM_Filter 11 tap downsampling, 6 tap upsampling that came from MPEG-2 work

- GS_Filter 15 tap downsampling, [-1 9 9 -1]/16 upsampling proposed by G. Sullivan JVT-I018

- Adaptive downsampling filters with [-1 9 9 -1]/16 upsampling proposed by A. Tourapis in JCTVC-W0051

It was remarked that visual effects are more important than known objective metrics, and that in some cases there are filters that may be more visually pleasing that are worse in terms of objective metrics.

Test clips could be made available for viewing.

All are odd length filters for downsampling.

It was commented by a participant that the submitter-proposed 9-tap filter has somewhat more aliasing (i.e., is somewhat more high-pass) than a conventional half-band filter.

The contributor proposed proving information about these filters in a technical report under preparation. It was agreed that to justify such action, some visual evidence would be needed that the results from specific filters are visually superior to the results from the simple filters that are described in the current draft TR. Side activity or further study was recommended toward collecting such evidence.

Review of specific language about filters from Y0046:

Discussed Sunday 1200 (GJS & JRO)

“It has been observed that, especially due to the step transfer slope characteristics and quantization behaviour of ST.2084, special caution needs to be made when selecting the filter coefficients of such a resampling filter, in order to mitigate chroma leakage. Conventional filters, such as linear filters, that are commonly used for down-conversion of SDR chroma signals may potentially result in visual artefacts when applied to HDR/WCG signals. This document however only describes linear FIR filters. In particular, the following FIR filters may result in less objectionable artefacts.”

Decision:  “This document however only considers two short-tap-length linear FIR filters which have been used in experiments conducted for its preparation.”

“The characteristics (magnitude and phase) of these filters are shown in Figure 7-4. Filter f1 has a stronger attenuation that is equal to -6 dB at 0.5 rad/s, whereas filter f0 could potentially cause some aliasing artifacts due to a significant amount of energy left in its stop-band. However, filter f0 may help in reducing chroma leakage since edges passed through this filter could output less severe transients.”

Decision:  Just remove the “However, …”

…

“It is quite likely that FIR linear filters would be used by many implementations for this process. Similar to the down-conversion case, it has been reported that non-linear and/or adaptive filters could result in improved performance, but this work is still under investigation. Regardless, it has been observed that the 2-phase resampling filter shown in Table 10-1 could provide relatively reasonable performance results on highly compressed HDR/WCG material. The same filter is applied both vertically and horizontally.”

Decision:  “The simple 2-phase resampling filter shown in Table 10-1 was used for the experiments conducted for preparation of this report.”

“NOTE 15 – This is essentially a Lanczos 2 filter. Higher precision and order filters could potentially be used when up sampling content of very high quality/no compression.”

JCTVC-Y0044 AHG14: Cross-checking results of JCTVC-Y0041 on chroma resampling filters for HDR/WCG Coding [F. Hiron, E. François (Technicolor)] [late]

### ICTCP colour representation (1)

JCTVC-Y0039 Draft suggested Text for ICTCP Support in HEVC (with traditional full range code mapping of BT.2100) [P. Yeng, C. Fogg, G. Sullivan, A. Tourapis]

### SDR backward compatible HDR coding (3)

JCTVC-Y0029 AHG14: Suggested draft text for HDR/WCG technology for SDR backward compatibility, display adaptation, and quality enhancement processing [E. François (Technicolor), D. Rusanovskyy (Qualcomm), P. Yin (Dolby)]

Discussed Fri. 10-14 1715 (GJS & JRO)

This document relates to high dynamic range (HDR) and wide color gamut (WCG) video distribution, based on HEVC single-layer or dual-layer coding. Its provides a suggested draft text for HDR and WCG video processing guidelines, using HEVC signalling, for SDR backward compatibility, display adaptation capabilities, and quality enhancement processing. It includes a description of HEVC signalling mechanisms, and of pre-encoding, coding, and post-decoding steps, enabling to address these three features. For SDR backward compatibility, the usage of HEVC VUI and SEI messages with single-layer coding, and of dual-layer SHVC coding, is discussed. For display adaptation, the document describes how HEVC SEI messages can be used. For quality enhancement processing, and in particular for improved coding efficiency (typically compared to coding of ST 2084, Non Constant Luminance, Y′CbCr, 4:2:0 video signals), solutions based on specific usage of VUI and SEI messages in single-layer coding solutions are presented. The document also provides conversion and coding practices for the ICTCP colour representation.

Agreed (in addition to minor editorial aspects):

- Generally remove opinionated commentary that could be challenged – e.g., asserting superiority of one colour representation over another; just describe how to work within each context.

- Add mention (and short “neutral tone” description) of SMPTE and ETSI mapping specs

- Strive for editorial consistency with the other TR

- On DRA discussion for “reshaping”

  - if included, be careful about opinionated statements regarding usefulness;

  - it would need to be stressed that the image quality may be unwatchable unless the metadata is used (and there is associated complexity).

  - this is something not being standardized for use elsewhere - do we need it in the TR? - revisit after some drafting side activity to produce “straw man” text.

  - should we describe an example of how an encoder would use it?

Further discussion Sunday 1300 (GJS & JRO):

It was commented that we should consider the scope of the TR, especially regarding whether the TR is a survey of syntax approaches for handling HDR video with HEVC and AVC or a recommendation of one or more of such particular approaches. As currently drafted, it is more of a survey of syntax approaches.

JCTVC-Y0042 AHG14: Tone Mapping Info. SEI for HDR Coding [P. Topiwala, W. Dai, M. Krishnan (FastVDO)] [late]

Discussed Sunday 1230 (GJS & JRO)

This is proposed to affect the second TR.

This proposal presents a backward compatible approach to coding HDR video, modifying the pre- and post-processing components of the anchor HDR video coding processing chain (essentially an HDR10 system). The usage of several SEI messages: the tone mapping information (TMI), colour remapping information (CRI), and mastering display color volume (MDCV), are also described.

Objective results based on quality metrics, as compared in a manner selected by the contributor: RGB-PSNR, DE100, MD100, PSNRL100) for the system (compared with HDR10 anchor v3.2) reportedly include: (a) -42.16%, -15.3%, -35.0% and 7.8% respectively using TMI SEI (b) -39.17%, -21.6%, -30.0% and 6.9%, respectively, using CRI SEI. It is asserted that the proposed coding scheme provides superior visual quality to the HDR10 anchor.

The contribution refers to the scenario known as “bitstream backward compatibility”, rather than what is known as “display adaptivity”. It uses the TMI SEI message rather than the CRI SEI message, for a similar purpose.

Decision: Add mention of the TMI SEI message approach to the 2nd draft TR (with no expression of opinion or claim of superiority).

JCTVC-Y0045 AHG14: Cross-checking results of JCTVC-Y0042 on usage of TMI, CRI and MDCV SEIs for HDR Coding [F. Hiron, E. François (Technicolor)] [late]

### Other (2)

JCTVC-Y0022 Transcoding between PQ and Hybrid Log-Gamma [C. Fogg (MovieLabs)]

JCTVC-Y0027 Professional HDR/WCG workflows [C. Fogg (Movielabs)] [late] [miss]

## HL syntax (3)

JCTVC-Y0023 Essential metadata to support virtual reality in AVC and HEVC elementary video streams [C. Fogg (MovieLabs)]

JCTVC-Y0031 On MCTS extraction [R. Skupin, Y. Sánchez, K. Grüneberg, C. Hellge, T. Schierl (HHI)]

JCTVC-Y0048 Video and File Format unspecified nal_unit_type [W. Husak, M. Dolan (Dolby), D. Singer (Apple)] [late]

## SEI messages and VUI (6)

### Content colour volume SEI message (3)

JCTVC-Y0032 AHG7: On Content Colour Volume SEI message [T. Chen, P. Yin, T. Lu, F. Pu, W. Husak (Dolby)]

Discussed Fri. 10-14 1830 (GJS & JRO)

In the May/June 2016 JCT-VC meeting, several proposals (JCTVC-X0040, JCTVC-X0052, and JCTVC-X0069) were submitted on how to represent content colour volume. Among all those proposals, some concerns arise: 1) which information is most important in the display adaptation for the optimal picture quality; 2) the minimum set of metadata that the bitstream should carry; 3) the complexity of generating and using those metadata. This contribution proposes a source content colour volume SEI which aims at providing a compact set of metadata.

The contribution noted that effects such as resampling and bit depth reduction can cause colours to go outside of the original gamut and tend to fill the “container”.

It was remarked that the concept of a “source” is problematic with respect to a decoder perspective.

Includes a representation of maximum, minimum and average luminance values (which are not in other proposals).

There was a comment about potential use of negative colour primaries.

Presentation deck to be uploaded.

E. François was requested to coordinate the further discussion of Y0032 and Y0040 and report back. That’s all until Sunday.

JCTVC-Y0040 Content colour volume SEI message [A. M. Tourapis, Y. Su, D. Singer (Apple), H. M. Oh, J. W. Choi, J.-Y. Suh (LG Electronics), A. K. Ramasubramonian, D. B. Sansli, J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

Discussed Fri. 10-14 1900 (GJS & JRO)

This document proposes an SEI message to describe the colour volume of the video content. The colour volume is specified using several different methods that are characterized by different degrees of complexity and precision. It is suggested that this information may be utilized by decoder-side entities and displays when processing and/or displaying the content.

In revision 1 of this document, a few clarifications to the semantics of the SEI message are provided along with aligning the representation of certain syntax elements.

There are three “modes” defined in the proposal. (There are 8 mode presence flags, three of which are used and five are reserved.) One of the modes includes spatial partitions (of equal size).

It was commented that it is not clear how a decoder (or decoding system) would use this information.

It was commented that this could be described as a “valid colour boundary” establishment.

The granularity (spatial, luma, number of colour primaries) could be rather fine.

It was commented that we should have more of an idea of how the data would be used.

- One purpose would be to identify whether any tone mapping is needed to adapt the content to the local display

- Revisit use case discussion

The second mode was suggested to be sufficiently capable. The first mode is strictly a special case of the second one, except perhaps for the min/max luma aspect. It was further asserted that something basically similar to what the third mode can do would also be possible with the second mode (by using the second mode with multiple colour volumes within it, with the final colour volume being the intersection). There was not general agreement about these assertions.

Revisit.

JCTVC-Y0051 BoG report on Content Colour Volume SEI message

Discussed Sunday 0900 (GJS & JRO)

This document provides a report of the BoG on Content Colour Volume SEI message.

A Break-out group (BoG) meeting on “Content Colour Volume SEI message” has taken place on Oct. 16 2016 from 10.00 to 12.05.

The mandate of the BoG was to further analyze the proposed technologies (Y0032 and Y0040), to clarify the use cases, and to recommend possible next steps.

A short review of the different solutions was made.

Y0032

- 3 primaries in xyY + min, max and average Luminance + optional 3 cross-sections, with 3 primaries per cross-section (1 section for min lum, 1 for max lum, 1 for avg lum)

  - Worst case max size 	 8 + 3*32 + 96 + 3*(3*32)  500 bits (30kbps at 60fps)

Y0040

- 3 non-exclusive modes

- Mode 1 (CcvModeTypeFlag[ 0 ]) – 3 to 18 primaries in xyY (polygon) + min and max Luminance

  - Worst case max size 	 18*32 + 64  640-650 bits (40kbps at 60fps)

- Mode 2 (CcvModeTypeFlag[ 1 ]) – picture divided in partitions – for each partition, up to 7 colour volume representations can be used – for each color volume, 0 to 6 xyY primaries signaled + min and max Luminance, depending on representation space (RGB, RYGCB, RYGCBM, YCbCr, XYZ, HSV, CIECAM02 LMS)

  - Worst case max size 	 ( nbPartitions * 7 * 6 * (32+48) ) + 16 + X bits
			 16+X+ nbPartitions*3360 bits

  - nbPartitions coded on X bits (2 ue(v)), no max value specified

  - if 16 partitions  53770 bits (3000kbps at 60fps)

  - can send 7 versions of the 6 primary case, using 6 different primary values; allows to create different volumes and define the final volume as the cross-section (even with that same representation).

- Mode 3 (CcvModeTypeFlag[ 2 ]) – up to 2 colour volumes – for each colour volume, 1 representation (xyY, Lab, YCbCr) – up to 256 ranges (cross-section of colour volume) per colour volume – up to 16 primaries (polygon) per cross-section

  - Worst case max size 	 8 + 2 * (8 + 256*(32+16*48)) bits
			 409624 bits (25000kbps at 60fps)

  - Proponent does not believe that worst case complexity (metadata bit rate) may be exercised by a reasonable encoder, and indicates that the ranges were put large due to what proponent thought was common in JCT-VC (8 cross-sections, 8 primaries per section was judged as a reasonable maximum).

  - Expected max size	 8 + 2 * (8 + 8*(32+8*48)) bits
			 6680 bits (400kbps at 60fps)

It was asked why 3 exclusive modes were needed

- Proponents explained that Mode1 is needed, as the simplest signalling. Mode3 is an extension of mode1, and is required for finer representation. Mode2 is an alternative to mode3.

Usage:

- An accurate color gamut description is relevant for processing applications such as colour volume mapping (incl. gamut mapping, display adaptation), adaptive clipping or similar operations for fidelity improvement, adaptive precision processing (improving processing fidelity for colours within a known range)

- Regarding spatial partitioning feature

  - The proponent indicates that the benefits are to allow parallel processing, precise process per region; this could allow control of localized power management on the display (e.g. LED regions)

  - It was indicated that this feature is actually not tested, and the gain is hypothetical

  - It was noted that the spatial partitioning concept could apply to the 3 modes of Y0040, or to Y0032

  - It was also noted that a spatial partitioning concept is also used in ST 2094 (a spec for mapping a “full-dynamic range” HDR signal to a lower dynamic range display, but this is for colour grading/ production side)

- Gain from cross-section based description?

  - Example of adaptive clipping mentioned by Alexis shows potential interest of such a description

  - It was indicated that the typical application of this is gamut mapping

- 3 main questions to answer

  - Do we need spatial partitioning

  - Do we need accurate color volume description

    - If yes, what type of accuracy do we need (e.g. do we need cross-section based representation)

  - What impact on complexity in pre-/post-processing

- Tests would be needed to answer these questions  it was suggested to set-up a verification experiment to address these questions

- Start with a light SEI message without spatial partition signalling, 3 component colour primaries coordinates corresponding to a triangle (CIE 1931 x, y, Y coordinates), negative x and y coordinates allowed, min/max luminance as anchor

  - Not including: multiple colour volume intersection, segmentation of luma levels, more than three colour primaries, min/max for each colour primary, average luminance, spatial rectangular partitioning

- Goal: show any benefit of additional information in the SEI message – if benefit is established, then propose an enhanced SEI message

- No consensus on this point, 3 proponents agreed to have a verification experiment, 1 proponent objected that such tests would oblige companies to disclose their proprietary processing

- One discussed option was: test could be done with only reporting results of processing, without describing the processing

- Details of potential verification experiment to be discussed (test processing chain, test content, usage of compression or not)

It was commented that it needs to be clear how to transform the content into a domain in which the colour values can be compared to the expressed gamut boundaries. This domain may or may not represent light in a source or display domain – it is merely a gamut representation domain rather than necessarily being a representation of actual light in either the scene or display domain. In Y0040, the BT.709 transfer characteristics are treated differently from others.

- When transfer_characteristics indicates BT.709, the content values would be interpreted in terms of displayed light resulting from BT.1886 (after inversion of the scaling and matrix coefficients).

- Otherwise, the inverse of the transfer_characteristics function would be applied (regardless of whether that function inversion produces Lc or Lo).

The domain of the representation is not necessarily true light from a source or light on a display. In the case of BT.2100 PQ, the above interpretation does correspond to displayed light on a reference display under reference viewing conditions.

We would need to make sure the semantics are clear about this.

One participant disagreed with treating BT.709 differently, saying the inversion of the transfer_characteristics function (producing Lc) should be used for BT.709 rather than using BT.1886.

Looking for “showcase” demonstrations of what can be achieved with this and with some more elaborate representation and evidence of which more elaborate scheme is needed.

The usefulness of the basic form should also be shown - e.g., visual benefit from the process described in equations in U0042.

X0040 was reported to show an example benefit. Someone please check that.

Need draft and further discussion of how to resolve the question of multiple approaches to more elaborate representations.

### Other SEI message proposals (3)

See also JCTVC-Y0042 and JCTVC-Y0045.

JCTVC-Y0024 Centralized Texture Depth Packing SEI Message for HEVC [J.-F. Yang, G.-C. Chen, W.-J. Yang (NCKU)]

JCTVC-Y0025 Centralized Texture Depth Packing SEI Message for AVC [J.-F. Yang, G.-C. Chen (NCKU)]

Not in scope of JCT-VC; should be considered by parent bodies (related to JCTVC-Y0024, but for AVC rather than HEVC).

JCTVC-Y0035 Regional nesting SEI message [A.K. Ramasubramonian, J. Sole, Y.-K. Wang, D. Rusanovskyy, D. Sansli, M. Karczewicz (Qualcomm), P. Andrivon, E. François (Technicolor), W. de Haan, R. Brondijk (Philips)]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (1)

JCTVC-Y0026 Tile-Level Rate Control for Multi-core Platform on HM [I. Marzuki, Y.-J. Ahn, W. Wiratama, D. Sim (KWU)]

# Withdrawn, unclear allocation (0)

[Ed. note: Check: "color", "neighbor", "artifact", "signaled", "signaling", "modeling"]

# Plenary discussions, joint meetings, BoG reports, and summary of actions taken

## General

Topics for general discussion at the plenary level: (Update)

- …

## Project development

Joint meetings are discussed in this section of this report. Additional notes on the same topics may appear elsewhere in this report. Joint discussions were held on XXXX., as recorded below.

## BoGs

JCTVC-Y0051 BoG report on Content Colour Volume SEI message [E. François]

See section 5.3.1.

## List of actions taken affecting the HEVC specification and draft technical report for HDR coding practices

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the draft text of the HEVC specification or the planned report on conversion and coding practices for HDR/WCG video content. Both technical and editorial issues are included (although some relatively minor editorial / bug-fix matters may not be listed). This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that what it proposes was adopted (in whole or in part).

- …

# Project planning

## Text drafting and software quality

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text. Similarly, software coordinators have the discretion to evaluate contributed software for suitability in regard to proper code style, bugginess, etc., and to not integrate code that is determined inadequate in software quality.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the XXday of the week preceding the meeting (xx Oct 2016).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments have been planned in previous work, although none were established at the current meeting. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments remained as described in the prior output document JCTVC-X1100.

The general timeline agreed for CEs was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

A deadline of four weeks after the meeting would be established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on CE work on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document would be reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE would be described in an associated output document numbered as, for example, JCTVC-X11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans would be recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities have been established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common test conditions for HEVC Coding Experiments

Update No particular changes were noted w.r.t. the prior CTC for work within the current scope of JCT-VC. See the prior output documents JCTVC-X1100 for HEVC test conditions, JCTVC-X1009 for SHVC test conditions, JCTVC-X1015 for SCC test conditions., and JCTVC-X1020 for HDR/WCG test conditions.

## Software development planning (update)

Software coordinators were asked to work out the detailed schedule for software updates with the proponents of adopted changes as applicable.

Any adopted proposals where necessary software is not delivered by the scheduled date in a timely manner may be rejected.

The planned timeline for software releases was established as follows:

- HM 16.6 available prior to the meeting.

- SCM 5.0 (based on HM 16.6 or newer) should be available within 3 weeks after the meeting.

- SHM 10.x U1013 (DAM, based on HM 16.2 or newer) should be available within 5 weeks after the meeting.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable. Further study was encouraged to determine the appropriate approach to future software maintenance, especially in regard to alignment of 3D video software with the SHM software.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-X1002 HEVC Test Model 17 (HM 17) of Encoder Description - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM and HDRTools software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM and HDRTools software and its distribution. - Integrate "Anchor 3.2" processing capability for HDR/WCG video coding into the HM - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 17 and additional HM 17.x "point" software versions according to JCTVC-X1100 and X1020 common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), K. Sharman, A. Tourapis, (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Develop proposed improvements to the SCC conformance testing draft X1016. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt, SHVC, and SCC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), R. Joshi, Y. Ye, J. Xu (vice-chairs) | N |
| SHVC verification test reporting (AHG5) (jct-vc@lists.rwth-aachen.de) - Edit and produce the final SHVC supplemental verification test report. | V. Baroncini, W. Husak, A. K. Ramasubramonian, Y.-K. Wang, Y. Ye (co-chairs) | N |
| SCC extensions verification testing (AHG6) (jct-vc@lists.rwth-aachen.de) - Study test conditions and coding performance analysis methods for verification of SCC coding performance. - Prepare the draft verification test plan for SCC X10XX - Develop and propose further improvements of the test plan | V. Baroncini, H. Yu (co-chairs), R. Cohen, A. Duenas, K. Rapaka, X. Xu, J. Xu (vice-chairs) | N |
| Content colour volume representation (AHG7) (jct-vc@lists.rwth-aachen.de) - Study approaches to the representation of the range of colours in video content - Develop proposed syntax and semantics for SEI message representation of content colour volume - Consider the proposals X0040, X0052, and X0069 in this work | H. M. Oh, A. K. Ramasubramonian, A. Tourapis (co-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SCM software and its distribution. - Prepare and deliver the reference configuration encodings according to JCTVC-X1015. - Prepare and deliver improved HM 16.x-SCM-8.x "dot" version software releases as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), P. Chuang, R. Cohen, X. Xiu (vice-chairs) | N |
| SHVC SHM software development and software technical evaluation (AHG9) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SHM software and its distribution. - Prepare and deliver SHM 12.x software versions according to JCTVC-X1009. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | G. Barroux, Y. He, V. Seregin (co-chairs) | N |
| Test sequence material (AHG10) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material, especially focusing on new needs for HDR test material and corresponding SDR test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG6 regarding screen content coding testing and AHG11 and AHG12 regarding HDR/WCG testing. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), E. François, T. K. Tan, P. Topiwala, S. Wenger, H. Yu (vice-chairs) | N |
| HDR/WCG visual testing (AHG11) (jct-vc@lists.rwth-aachen.de) - Study content characteristics and identify appropriate test sequences for visual testing. - Identify and develop test methodologies including consideration and characterization of test equipment | V. Baroncini, P. Topiwala, E. Alshina (co-chairs) | N |
| HDR/WCG verification test reporting (AHG12) (jct-vc@lists.rwth-aachen.de) - Finalize the verification test report JCTVC-X1018 | V. Baroncini (chair), K. Andersson, W. Husak, A. K. Ramasubramonian, G. Sullivan (co-chairs) | N |
| HDR/WCG conversion and coding practices report development for PQ Y′CbCr 4:2:0 (AHG13) (jct-vc@lists.rwth-aachen.de) - Identify and study technical approaches to single-layer HDR PQ Y′CbCr 4:2:0 10 bit coding using the existing HEVC standard, including potential use of SEI messages - Study and propose improvements of the draft guidelines X10xx for HEVC single-layer coding HDR/WCG PQ Y′CbCr 4:2:0 10 bit coding - Particularly study and proposed improvements of chroma resampling filtering in this work | J. Samuelsson (chair), C. Fogg, A. Norkin, A. Segall, J. Ström, G. Sullivan, P. Topiwala, A. Tourapis, P. Yin (vice-chairs) | N |
| HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing (AHG14) (jct-vc@lists.rwth-aachen.de) - Study technology and develop potential guidelines for single-layer coding with HEVC using SEI messages and VUI indicators for backward compatibility, display adaptation, quality enhancement post-processing - Study the technical characteristics and develop potential guidelines for two-layer coding with SHVC for backward compatibility and display adaptation - Study and develop potential guidelines for use of the ICTCP colour representation with the HEVC standard - Study and develop potential guidelines for use of the HLG transfer function with the HEVC standard - Study and propose test conditions for associated experiments | E. François (chair), W. Husak, D. Rusanovskyy, P. Topiwala, P. Wu (vice-chairs) | N |

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JCTVC-X1000 Meeting Report of the 24th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2016-10-30] (near next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

JCTVC-X1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 6 [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 16267) [2016-10-23] (near next meeting)

JCTVC-X1003 Draft text for ICTCP support in HEVC (Draft 2) [P. Yin, C. Fogg, G. J. Sullivan, A. Tourapis (editors)] (WG 11 PDAM N 16265) [2016-06-01] (now)

JCTVC-X1004 SHVC Supplemental Verification Test Report [V. Baroncini, W. Husak, A. K. Ramasubramonian, J. Sole, G. J. Sullivan, Y.-K. Wang, Y. Ye (editors)] (WG 11 N 16268) [2016-06-24] (3 weeks)

JCTVC-X1005 Draft Text for Picture Repetition Indication in HEVC (Draft 1) [C. Fogg, G. J. Sullivan (editors)] (WG 11 WD N 16266) [2016-06-10] (1 week)

JCTVC-X1006 Draft verification test plan for HEVC screen content coding extensions [V. Baroncini, H. Yu, R. Cohen, A. Duenas, K. Rapaka, X. Xu, J. Xu (editors)] (WG 11 N 16254) [2016-07-15] (6 weeks)

Remains valid – not reissued: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778)

JCTVC-X1008 [doc number unused]

JCTVC-X1009 Common Test Conditions for SHVC [V. Seregin, Y. He (editors)] [2016-06-17] (2 weeks)

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-X1011 Reference Software for Screen Content Coding Draft 2 (WG 11 DAM N 16270) [K. Rapaka, B. Li, X. Xiu (editors)] [2016-07-15] (in time for DAM ballot)

JCTVC-X1012 [doc number unused]

JCTVC-X1013 [doc number unused]

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 16049) [2016-04-30]

JCTVC-X1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2016-06-24] (3 weeks)

JCTVC-X1016 Conformance Testing for HEVC Screen Content Coding (SCC) Extensions Draft 2 (WG 11 WD 2 N 16271) [R. Joshi, J. Xu (editors)] [2016-08-19] (2.5 months)

JCTVC-X1017 Draft 2 of Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics [J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. Sullivan, A. Tourapis (editors)] (WG 11 WD 2 N 16272) [2016-09-16] (3.5 months)

JCTVC-X1018 Revised Verification Test Report for HDR/WCG Video Coding Using HEVC Main 10 Profile [V. Baroncini, K. Andersson, A. K. Ramasubramonian, G. Sullivan (editors)] (WG 11 N 16273) [2016-06-24] (3 weeks)

(Adding further detail on configurations used).

JCTVC-X1020 Common Test Conditions for HDR/WCG video coding experiments [E. François, J. Sole, J. Ström, P. Yin (editors)] [2016-07-01] (4 weeks)

(including HDR config information formerly in the TR draft)

JCTVC-X1100 Common Test Conditions for HM [K. Sharman, K. Sühring (editors)] [2016-06-17] (2 weeks)

# Future meeting plans, expressions of thanks, and closing of the meeting (update)

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Thursday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Thu. 12 – Wed. 18 Jan. 2017, 26th meeting under ITU-T auspices in Geneva, CH.

- Fri. 31 Mar. – Fri. 7 Apr. 2017, 27th meeting under WG 11 auspices in Hobart, AU.

- …

The agreed document deadline for the 26th JCT-VC meeting is XXday XX Jan 2017. Plans for scheduling of agenda items within that meeting remained TBA.

XXX was thanked for the excellent hosting of the 25th meeting of the JCT-VC.

Companies that provided equipment that was used for subjective viewing of video – XXX, were also thanked.

The JCT-VC meeting was closed at approximately XXXX hours on Friday, 21 October 2016.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the twenty-fifth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows:

- …

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.