| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 25th Meeting: Chengdu, CN, 14–21 October 2016 | Document: JCTVC-Y0042r2 |
| --- | --- |

| Title: | AHG14: Tone Mapping Information and Related SEIs for HDR Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pankaj Topiwala Wei Dai Madhu Krishnan 3097 Cortona Dr., Melbourne, FL 32940 | Tel: Email: | 321-355-7376 {pankajtva, daisy, madhupk83 @ gmail.com} |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

This proposal presents a backward compatible solution to coding HDR video, by effectively modifying the pre and post processing components of the anchor HDR video coding processing chain (essentially an HDR10 compliant system). The usage of several SEI’s: Tone mapping information (TMI), Color remapping information (CRI), and Mastering display color volume (MDCV), is also described. Representative objective results based on quality metrics: RGB-PSNR, DE100, MD100, PSNRL100) for the system (compared with HDR10 anchor v3.2) include: (a) -42.16%, -15.3%, -35.0% and 7.8% respectively using TMI SEI (b) -39.17%, -21.6%, -30.0% and 6.9% respectively using CRI SEI. It is asserted that the proposed coding scheme provides superior visual quality to the HDR10 anchor.

# Introduction

As part of the investigations done to leverage existing video standards for efficiently encoding HDR/WCG videos, FastVDO developed several effective approaches for HDR video coding [1]. All our methods can be implemented entirely as pre/post processing steps, and do not necessitate any normative changes to the core Main10 encode/decode processes. Moreover, the output of a Main10 decoder is an optimized SDR rendering.

The HEVC standard [3] describes two different SEI messages, Tone mapping (TMI) SEI message and the Color remapping information (CRI) SEI message, for content mapping. In this proposal modification to the pre and post processing components of the anchor HDR video coding processing chain and details of metadata signalling using MDCV, TMI and CRI SEI are described. Our results have previously been presented at the SPIE [6]. Technicolor has kindly cross-checked our results, in document JCTVC-Y0045.

# System Overview

Pre-processing: The pre-processing step in the proposed HDR coding chain is presented in Figure 1.

Grade HDR to SDR in 4:4:4

RGBHDR		RGBSDR

Generate Data Dependent HDR to SDR Grading Function

Metadata

Figure 1. Pre-Processing, which regrades HDR to SDR.

The pre-processing step involves the computation of a data dependent HDR-to-SDR grading function/process. A base transfer function, which can be either static or adaptive, is modified depending on the statistical analysis of the linear light HDR RGB input signal. Example base transfer functions that can be used include ST-2084 (Dolby PQ), Philips TF [2], and HLG (ARIB STD-B67). The computed grading function/process is represented as a 1D LUT. The generated metadata consists of the color volume information and the 1D LUT represented using a piecewise linear function with 10 pivot points. The grading function/process is then used to grade the HDR RGB input signal to an SDR RGB signal. The SDR RGB signal is then color transformed to Y’CbCr, downsampled from 4:4:4 to 4:2:0 and encoded using HEVC reference codec (Main10 profile). The metadata is signaled using MDCV and either the TMI or CRI SEI.

- Data dependent HDR-to-SDR grading function/process: Figure 2 describes the technique involved in generating the grading function/process.

Piecewise

Linear Tone

Mapping

Generate

Grading

Factor

Apply adaptive OETF

Apply adaptive OETF

Coding TF

Generate

Luma

RGBHDR

Figure 2. Data dependent grading

- Generate Luma: A luma component (Y) is generated using either the common conversion to the Y’CbCr color space. This part is linear. All other components are nonlinear, and they can be all combined into a single function for purposes of implementation.

- Apply data adaptive OETF: The linear luma (Y) input signal is converted to a perceptually uniform signal, but using a FastVDO-modified version of ST-2084 [4], herein “FVPQ”. The FVPQ function is described as:

Y_OETF=powf(((c2 *powf(Y,m1) + c1)/(1.0 + c3 *powf(Y,m1))),m2)  where

m1=(2610.0        ) / (4096.0 * 4.0) ,

m2=(((2523.0 * 128.0))/4096)* (1 + 0.25 * log(10000/InputPeakBrightness)),

c1 = (3424.0        ) / 4096.0,

c2= (2413.0 *  32.0) / 4096.0,

c3= (2392.0 *  32.0) / 4096.0

Here, powfa,b=ab, and InputPeakBrightness  is new parameter, indicative of the peak brightness of the data. When the parameter equal 10,000, this reduces to the usual PQ function.

This represents a first level of adaptivity, and is an important step in creating SDR intermediates that better capture the artistic intent of the HDR content. Since this is a tone mapping, it can be signaled using the Tone Mapping SEI, among other ways.

Figure 3. FVPQ OETF, for values of InputPeakBrightness = 10000, 4000, and 1000 nits (at 10K nits it coincides with the ST-2084 PQ function), as pictured from top to bottom. Note that when the input data is below 10000 nits, our TF affords more codewords at the brighter end than the standard PQ transfer function.

- Piecewise Linear Tone Mapping:   A great variety of tone mapping schemes can be applied for HDR coding, and implemented in a variety of ways as well. In this paper, for convenience, we will only mention a piecewise linear mapping scheme. To add a modest amount of flexibility, we consider an L-piece linear model, where L >=1. For each piece the model can be represented as:

y=a(k)*x+b(k), k=0, 1, …, 8,

where, k represents the kth piece or bin. These equations are applied to the  YOETF  to generate a new signal Y'OETF. Let MAX_INPUT_LUMA be the maximum luminance of the input signal and MAX_DISPLAY_LUMA be the maximum luminance of the target SDR display. The coefficients a(k), b(k) are sequence dependent and are derived based on the distribution characteristics of the HDR input signal. As a specific example, they can be based on a histogram analysis, as follows. The input luma signal is segmented to L bins, and a count is kept for the number of pixels in each bin. Under an optimization process, the number of bins and their respective sizes are chosen such that the lower and mid tone levels have an improved representation compared to the unadjusted transfer function representation. Thus the parameters a, b are chosen for each piece such that:

ak= binsizekNumTotPixels*MAX_DISPLAY_LUMA/MAX_LUMA_BINk

b0=0

bk=ak-1* MAX_LUMA_BINk-1+bk-1-ak*MAX_LUMA_BINk-1

where MAX_LUMA_BINk is the maximum luma value in bin k, and binsizek is the number of pixels in bin k.

Figure 4. Example histograms of first frame of (a) BalloonFestival, and (b) Market sequence, as example statistical basis for tone maps.

Figure 5. Example piecewise linear mappings corresponding to the statistics (histogram here) for (a) BalloonFestival, and (b) Market.

- Apply data adaptive EOTF: The perceptual domain signal Y'OETF is converted back to linear light (Y’) using the matching inverse FastVDO-modified ST-2084 OETF (FVInvPQ). The function is described as:

Y'=powfdMax0.0,powfY’OETF,1.0m2- c1c2 - c3 * powfY’OETF,1.0m2,1.0m1;

Here dMax(a,b) returns a if a > b else b.

- Generate Grading Factor: The grading factor (actually, a function of x, y, and t) is calculated as:

λ=Y'/Y

This is then used to generate an intermediate signal (RGBSDR)’ = RGBHDR * λ.

- Coding TF: Finally, a backward compatible SDR signal, RGBSDR, is generated by further applying a function like the ITU-R Rec. BT.2020 power-law gamma curve [5] or the ST-2084 (PQ TF).

The entire processing chain in Figure 2 can be visualized as a single tone mapping scheme.

Post-processing: The post-processing step in the proposed HDR coding chain is presented in Figure 2.

Regrade SDR to HDR in 4:4:4

RGBSDR	RGBHDR

Generate Data Dependent SDR to HDR Regrading Function

Metadata

Figure 2. Post-Processing, which recovers HDR from SDR.

The HEVC reference codec is used to decode the bitstream and generate the reconstructed Y’CbCr 4:2:0 signal and metadata. The Y’CbCr 4:2:0 signal is then upsampled from 4:2:0 to 4:4:4 and color transformed to SDR RGB. The regrading function/process is computed using the metadata. The process is very similar to that described in Figure 2, except that the blocks are in reverse order. Finally, regrading is performed on the SDR RGB input to obtain the linear light reconstructed HDR RGB signal.

Tone mapping (TMI) SEI

The TMI SEI syntax describes five methods for signaling a custom tone mapping curve represented as a 1D LUT: a) Linear mapping with clipping; b) Sigmoidal mapping; c) User defined table mapping; d) Piecewise Linear mapping and e) Luminance dynamic range information. In this proposal the piecewise linear content mapping scheme is used.

An example of settings used in the SEI:

SEIToneMappingInfo: 1

SEIToneMapId: 1

SEIToneMapCancelFlag:0

SEIToneMapPersistenceFlag: 1

SEIToneMapModelId: 3

SEIToneMapNumPivots: 10

SEIToneMapCodedDataBitDepth: coded data bit depth

SEIToneMapTargetBitDepth: target data bit depth

Color remapping information (CRI) SEI

The CRI SEI syntax consists of three sequential but optional parts: a) Piecewise linear Pre-LUT function; b) 3x3 transformation matrix and c) Piecewise linear Post-LUT function. In this proposal the Post-LUT content mapping scheme is used.

An example of settings used in the SEI:

colour_remap_id:1

colour_remap_cancel_flag:0

colour_remap_persistence_flag:1

colour_remap_video_signal_info_present_flag:1

colour_remap_full_range_flag:0

colour_remap_primaries:9

colour_remap_transfer_function:16

colour_remap_matrix_coefficients:9

colour_remap_input_bit_depth: coded data bit depth

colour_remap_output_bit_depth: target data bit depth

Mastering display color volume (MDCV) SEI

The MDCV SEI syntax describes the color volume of the mastering display used for viewing while authoring the video content.

An example of the settings used in the SEI:

SEIMasteringDisplayColourVolume: 1

SEIMasteringDisplayMinLuminance: 47

SEIMasteringDisplayMaxLuminance: 40000000

SEIMasteringDisplayPrimaries: 13250,34500,7500,3000,34000,16000

SEIMasteringDisplayWhitePoint: 15635,16450

VUI Settings

The following settings are used in the VUI:

VuiParametersPresent: 1

VideoSignalTypePresent: 1

VideoFullRange: 0

ColourDescriptionPresent: 1

ColourPrimaries: 9

TransferCharacteristics: 14, 15 or 16

MatrixCoefficients: 9

Results

The objective results obtained by the proposed system using a) TMI SEI and b) CRI SEI are shown in Table 1 and Table 2 respectively. The reference for these tests is the HDR10 anchor v3.2. Spreadsheets with the complete metrics are provided as evidence. The proposed coding scheme provides superior visual quality to the HDR10 anchor v3.2. The generated SDR YUV 4:2:0 sequences generated after pre-processing looks more natural, and more similar to the original HDR video than the anchor (HDR10) generated YUV 4:2:0 videos.

Table 1: Objective Results (using TMI SEI)

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR R | PSNR G | PSNR B | PSNR-RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 69.4% | 56.7% | 186.6% | 93.1% | 124.1% | 93.3% | -31.7% | 25.2% | -71.6% | -47.3% | -71.0% | -53.33% |
|  | Market3Clip4000r2 | 51.6% | 52.4% | 43.0% | 48.6% | 24.3% | -41.0% | 0.0% | -0.5% | -46.4% | -42.2% | -55.9% | -44.41% |
|  | SunRise | 64.7% | 72.4% | 72.4% | 70.0% | 34.6% | -67.8% | -45.4% | 9.5% | -57.4% | -43.8% | -67.7% | -48.51% |
| class B | BikeSparklers cut 1 | 32.6% | 36.9% | 37.0% | 35.5% | 38.7% | 1.1% | -22.9% | 23.8% | -44.0% | -26.6% | -42.3% | -30.74% |
|  | BikeSparklers cut 2 | 27.6% | 33.4% | 31.6% | 30.6% | 34.0% | -5.6% | -30.0% | 23.7% | -36.3% | -19.0% | -30.0% | -22.53% |
|  | GarageExit | 14.4% | 19.0% | 5.4% | 12.3% | 10.1% | -21.1% | -82.4% | 10.8% | -41.2% | -30.2% | -50.0% | -34.05% |
| class C | ShowGirl2Teaser | 26.8% | 29.2% | 42.9% | 33.7% | 19.5% | -19.1% | -83.6% | 2.7% | -51.4% | -39.7% | -62.2% | -43.96% |
| class D | StEM_MagicHour cut 1 | 6.3% | 9.7% | 13.4% | 10.5% | 9.9% | -8.1% | -80.5% | 4.3% | -63.5% | -42.9% | -42.9% | -45.50% |
|  | StEM_MagicHour cut 2 | 7.4% | 8.8% | 10.6% | 9.2% | 8.6% | -5.7% | -40.4% | 2.3% | -72.0% | -52.1% | -24.7% | -51.13% |
|  | StEM_MagicHour cut 3 | 8.3% | 9.6% | 12.4% | 10.7% | 11.2% | -3.4% | -25.3% | 1.6% | -64.1% | -46.8% | -30.5% | -46.94% |
|  | StEM_WarmNight cut 1 | 9.5% | 12.1% | 10.9% | 10.8% | 10.3% | -6.4% | -35.1% | 2.2% | -69.4% | -45.8% | -65.0% | -51.11% |
|  | StEM_WarmNight cut 2 | 7.8% | 9.0% | 17.8% | 12.4% | 15.3% | -8.7% | 0.0% | 0.7% | -78.7% | -52.8% | -65.0% | -57.58% |
| class G | BalloonFestival | 11.7% | 19.3% | 21.0% | 17.7% | 9.0% | -32.1% | 0.0% | -6.1% | -48.2% | -37.8% | -66.2% | -42.68% |
| class H | EBU_04_Hurdles | 13.3% | 18.0% | 2.2% | 9.9% | -4.0% | -60.1% | -47.5% | 1.0% | -55.2% | -27.2% | -52.4% | -33.88% |
|  | EBU_06_Start | 44.3% | 50.1% | 36.5% | 43.2% | 25.9% | -45.2% | 0.0% | 15.9% | -46.3% | -19.6% | -44.2% | -26.01% |
|  | Overall | 26.4% | 29.1% | 36.2% | 29.9% | 24.8% | -15.3% | -35.0% | 7.8% | -56.4% | -38.3% | -51.3% | -42.16% |

Table 2: Objective Results (using CRI SEI)

|  |  | X | Y | Z | XYZ | tOSNR-XYZ | DE100 | MD100 | PSNRL100 | PSNR R | PSNR G | PSNR B | PSNR-RGB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| class A | FireEaterClip4000r1 | 51.1% | 43.3% | 107.6% | 61.8% | 75.8% | 63.0% | -32.9% | 20.3% | -70.3% | -44.8% | -69.6% | -51.05% |
|  | Market3Clip4000r2 | 42.9% | 44.1% | 31.7% | 39.0% | 18.2% | -45.0% | 0.0% | -1.6% | -43.9% | -39.3% | -53.4% | -41.67% |
|  | SunRise | 52.4% | 59.8% | 50.9% | 54.4% | 24.1% | -70.5% | -47.2% | 6.7% | -55.0% | -41.4% | -67.4% | -46.37% |
| class B | BikeSparklers cut 1 | 28.7% | 33.0% | 26.3% | 29.3% | 31.7% | -5.9% | -28.1% | 21.4% | -41.7% | -23.9% | -40.5% | -28.19% |
|  | BikeSparklers cut 2 | 24.8% | 30.6% | 22.4% | 25.8% | 28.5% | -12.0% | -28.2% | 22.1% | -34.3% | -16.6% | -28.9% | -20.36% |
|  | GarageExit | 12.5% | 17.3% | 1.4% | 9.3% | 7.1% | -25.6% | -81.9% | 10.2% | -38.2% | -27.0% | -47.1% | -30.89% |
| class C | ShowGirl2Teaser | 19.4% | 22.6% | 26.4% | 23.4% | 9.0% | -25.9% | -82.9% | 1.2% | -48.8% | -36.6% | -60.3% | -41.11% |
| class D | StEM_MagicHour cut 1 | 3.8% | 7.8% | 6.2% | 5.9% | 5.3% | -12.8% | 0.0% | 4.0% | -60.8% | -38.9% | -40.6% | -41.87% |
|  | StEM_MagicHour cut 2 | 5.0% | 6.9% | 4.4% | 5.0% | 4.2% | -10.2% | -40.9% | 2.4% | -68.9% | -47.4% | -23.3% | -47.11% |
|  | StEM_MagicHour cut 3 | 5.7% | 7.5% | 5.1% | 5.7% | 5.5% | -8.2% | -19.4% | 2.0% | -60.6% | -42.6% | -29.0% | -43.13% |
|  | StEM_WarmNight cut 1 | 6.0% | 9.0% | 3.8% | 5.8% | 5.1% | -11.8% | -34.0% | 2.1% | -66.6% | -41.4% | -62.0% | -47.16% |
|  | StEM_WarmNight cut 2 | 3.9% | 6.8% | 6.6% | 5.7% | 7.1% | -14.0% | 0.0% | 1.0% | -76.1% | -47.8% | -61.7% | -53.05% |
| class G | BalloonFestival | 8.5% | 16.3% | 9.4% | 11.0% | 3.0% | -36.5% | 0.0% | -5.2% | -45.6% | -34.7% | -64.1% | -39.74% |
| class H | EBU_04_Hurdles | 11.8% | 16.6% | -0.3% | 7.9% | -4.9% | -61.0% | -54.6% | 1.3% | -54.2% | -26.0% | -51.4% | -32.68% |
|  | EBU_06_Start | 39.5% | 45.9% | 29.0% | 37.3% | 22.5% | -47.4% | 0.0% | 16.0% | -43.6% | -16.7% | -41.7% | -23.17% |
|  | Overall | 21.1% | 24.5% | 22.1% | 21.8% | 16.2% | -21.6% | -30.0% | 6.9% | -53.9% | -35.0% | -49.4% | -39.17% |

Conclusion

A framework is presented for effective coding of HDR/WCG video, which can be HDR10 compliant, and provide a measure of backward compatibility.

- References

[1] 	Pankaj Topiwala, Wei Dai and Madhu Krishnan, “HDR CE5: Report of Experiment 5.3.2”, JCTVC-W0055, San Diego, Feb., 2016.

[2] R. Goris, R. Brondijk, and R. van der Vleuten, “Philips Response to CFE on HDR and WCG,” ISO/IEC MPEG doc. m36266, Warsaw, PL, July, 2015.

[3] JCTVC-W1005; High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 6

[4] Society of Motion Picture and Television Engineers ST 2084 (2014), High Luminance EOTF.

[5]	ITU-R BT.2020 (2012), “Parameter Values for ultra-high definition television systems for production and internal programme exchange.”

[6] P. Topiwala, W. Dai, and M. Krishnan, “FV10: An efficient single-layer approach to HDR coding, with backward compatibility options,” Proc. International SPIE Symp., San Diego, CA, Aug., 2016.

# Patent rights declaration(s)

FastVDO LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).