| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 29th Meeting: Macao, CN, 19–24 Oct. 2017 | Document: JCTVC-AC_Notes_d4 |
| --- | --- |

| Title: | Meeting report of the 29th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Macao, CN, 19–24 Oct. 2017 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its twenty-ninth meeting during 19–24 Oct. 2017 at the Holiday Inn Macao Cotai Central (Sands Cotai Central Cotai Strip, Macau, CN). The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 19 Oct. 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1000 hours on Tuesday 24 Oct. 2017. Approximately 107 people attended the JCT-VC meeting, and approximately XX input documents and 8 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG 16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions, and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information.

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-eighth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 Update 9 encoder description;

- For the HEVC text specification, Draft 1 of a defect report listing potential issues that may require corrective action;

- For supplemental enhancement information (SEI) and video usability information (VUI), Draft 3 of additional SEI message for HEVC and Draft 5 of ICtCp support in HEVC;

- For the HEVC format range extensions (RExt), Draft 1 of an HEVC Monochrome 10 profile;

- For high dynamic range (HDR) considerations, Draft 3 of a technical report on Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video;

- For the HEVC screen content coding (SCC) extensions, draft 6 of conformance testing specification.

The other most important goals were to review the work on High Dynamic Range (HDR) and wide colour gamut (WCG) video coding, and on new SEI messages, and to review other technical input documents. Advancing the work on development of conformance and reference software for the recently finalized HEVC extensions on Screen Content Coding was also a significant goal. Possible needs for corrections to the prior HEVC specification text were also considered.

The JCT-VC produced XX particularly important output documents from the meeting (update):

- The HEVC test model (HM) 16 Update 9 encoder description;

- For the HEVC text specification, Draft 1 of a defect report listing potential issues that may require corrective action;

- For supplemental enhancement information (SEI) and video usability information (VUI), Draft 3 of additional SEI message for HEVC and Draft 5 of ICtCp support in HEVC;

- For the HEVC format range extensions (RExt), Draft 1 of an HEVC Monochrome 10 profile;

- For high dynamic range (HDR) considerations, Draft 3 of a technical report on Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video;

- For the HEVC screen content coding (SCC) extensions, draft 6 of conformance testing specification.

For the organization and planning of its future work, the JCT-VC established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings were planned for being held during Sat. 20 – Fri. 26 Jan. 2018 under WG 11 auspices in Gwangju, KR, during Sat. 14 – Fri. 20 Apr. 2018 under WG 11 auspices in San Diego, US, during Thu. 12 – Wed. 18 July 2018 under ITU-T auspices in Ljubljana, SI, and during XX – XX Oct. 2018 under WG11 auspices in XX, XX.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

Administrative topics

Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its twenty-ninth meeting during 19–24 Oct. 2017 at the Politecnico di Torino, Torino, IT. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 19 Oct. 2017. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Tuesday 24 Oct. 2017. Approximately XX people attended the JCT-VC meeting, and approximately XX input documents and 8 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of ITU-T SG 16 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions, and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

- 22nd "V" meeting (Geneva, 2015-10)		155 people, 75 input documents

- 23rd "W" meeting (San Diego, 2016-02)		159 people, 125 input documents

- 24th "X" meeting (Geneva, 2016-05/06)		162 people, 60 input documents

- 25th "Y" meeting (Chengdu, 2016-10)		93 people, 40 input documents

- 26th "Z" meeting (Geneva, 2017-01)		95 people, 30 input documents

- 27th "AA" meeting (Hobart, 2017-03/04)	76 people, 25 input documents

- 28th "AB" meeting (Turin, 2017-07)		71 people, 25 input documents

- 29th "AC" meeting (Macao, 2017-10)		107 people, XX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2017_10_AC_Macao/.

Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the twenty-seventh JCT-VC meeting in producing:

- The HEVC test model (HM) 16 Update 9 encoder description;

- For the HEVC text specification, Draft 1 of a defect report listing potential issues that may require corrective action;

- For supplemental enhancement information (SEI) and video usability information (VUI), Draft 3 of additional SEI message for HEVC and Draft 5 of ICtCp support in HEVC;

- For the HEVC format range extensions (RExt), Draft 1 of an HEVC Monochrome 10 profile;

- For high dynamic range (HDR) considerations, Draft 3 of a technical report on Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video;

- For the HEVC screen content coding (SCC) extensions, draft 6 of conformance testing specification.

The other most important goals were to review the work on High Dynamic Range (HDR) and wide colour gamut (WCG) video coding, and on new SEI messages, and to review other technical input documents. Advancing the work on development of conformance and reference software for the recently finalized HEVC extensions on Screen Content Coding was also a significant goal. Possible needs for corrections to the prior HEVC specification text were also considered.

Documents and document handling considerations

General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is done using the keyword “Decision”, e.g., as follows:

- Decisions made by the group that affect the normative content of the draft standard are identified by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. Considering the high workload of this meeting and the related meetings held in a collocated fashion, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 10 Oct. 2017.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 11 Oct. 2017 were considered "officially late".

All contribution documents with registration numbers JCTVC-AC0036 and higher were registered after the "officially late" deadline.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following non-normative proposals and non-proposal documents were both registered late and uploaded late:

- JCTVC-AC00XX (a proposal about …) [uploaded 10-XX].

The following other non-normative proposals and non-proposal documents were registered on time but were uploaded late:

- JCTVC-AC00XX (a proposal about …) [uploaded 10-XX].

The following other high-numbered input documents were administrative reports or follow-up on other contributions and discussions of the meeting, and thus may not be considered late contributions:

- JCTVC-AC00XX (a proposal about …) [uploaded 10-XX].

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting. The initial uploads of such contribution documents are rejected as "placeholders" if they are uploaded without any significant content and are not corrected until after the upload deadline. Such “placeholder” cases did not occur at this meeting.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, uploading of corrupted unreadable files, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload, along with a record of uploading times.

Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-AB1000, the High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9 JCTVC-AB1002, the draft text 5 for ICTCP support in HEVC JCTVC-AB1003, the draft text 1 of HEVC Monochrome 10 profile JCTVC-AB1004, the draft text 3 of additional SEI Messages in HEVC JCTVC-AB1005, the draft text 1 of defect report for HEVC text JCTVC-AB1011, the draft text 3 of Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video JCTVC-AB1012, and the SCC Conformance Testing Draft 6 JCTVC-AB1016, were also approved.

The group was initially asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, regarding software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work. There should be time to study combinations of different adopted tools with more detail prior to adoption.

Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

Agenda

The agenda for the JCT-VC meeting the meeting, for development of the High Efficiency Video Coding (HEVC) standard and its format range (RExt), scalability (SHVC), screen content coding (SCC), and high-dynamic-range (HDR) extensions, and associated conformance test sets, reference software, verification testing, non-normative guidance information, and coding-independent code point specifications was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities (none for this meeting)

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of errata reports and needs for maintenance and enhancements of the HEVC standard and its associated conformance test specification and reference software

- Consideration of errata reports and needs for maintenance and enhancements of the specification of coding-independent code points for video signal type identification

- Consideration of proposals and preparations toward finalization of the specification of additional supplemental enhancement information and video usability information metadata for the HEVC standard

- Consideration of proposed content for a technical report on common combinations of video signal type code point identifiers

- Consideration of additional video coding technology and supplemental enhancement information proposal contributions

- Consideration of contributions on the development of conformance test sets, reference software, verification testing, and non-normative guidance information

- Consideration of information contributions

- Coordination activities relating to the work of the JCT-VC

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, such as patent rights, may exist that are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

The JCT-VC email list is managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages, and subscribers must respond adequately to basic inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the JCT-VC email reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the headers and registrations of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

Terminology

Some terminology used in this report is explained below:

- 3D-HEVC: A set of extensions of HEVC that includes the combined coding of depth and texture information for 3D video coding.

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: May refer either to block copy (see CPR or IBC) or backward compatibility. In the case of backward compatibility, this often refers to what is more formally called forward compatibility.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (MV used for intra BC prediction, not a term used in the standard).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment for which there is a draft design and associated test model software that have been established, e.g., as in experiments conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE, and TE).

- CGS: Colour gamut scalability (historically, also coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector sometimes called a block vector, in a manner basically the same as motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions – a set of agreed conditions for coding experiments.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element of AVC or HEVC).

- EL: Enhancement layer.

- ET: Encoding time.

- ETM: Experimental test model (design and software used for prior HDR/WCG coding experiments in MPEG).

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HDR: High dynamic range – referring to video content having a brightness range that includes values greater than approximately 100 nits (often implicitly including WCG as well, since HDR video is typically also WCG video).

- HDR10: A term that refers to the single-layer coding of HDR/WCG video content using the HEVC Main 10 profile with a Y′CbCr 4:2:0 10 bit per sample colour representation with ITU-R BT.2020 colour primaries and the PQ transfer characteristics EOTF.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized in ITU-T as Rec. ITU-T H.265 and in ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – the draft reference software and its (non-normative) encoder algorithms used for HEVC experiments.

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (esp. 8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (esp. 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (as in AVC and HEVC).

- JM: Joint model – the primary software codebase and associated (non-normative) encoding algorithms that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network element.

- MC: Motion compensation.

- MCTS: Motion-constrained tile set.

- MOS: Mean opinion score – a measurement of subjective video quality as reported by human test subjects.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector; alternatively, multiview.

- MV-HEVC: A set of extensions of HEVC using layered coding to enable the coding of video with multiple views or depth maps.

- NAL: Network abstraction layer (as in AVC and HEVC, contrast with VCL).

- NCL: Non-constant luminance, a type of colour difference representation.

- Nits: Candelas per square metre (cd/m2).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g., light input to a camera) to output light (e.g., light emitted by a display).

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- PDTR: Proposed draft technical report – the draft of a TR that is sent for a ballot in the ISO/IEC approval process.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- PQ: Perceptual quantization – the name given to an HDR EOTF curve specified in SMPTE ST 2084 and Rec. ITU-R BT.2100.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RExt: Format range extensions – a set of extensions of HEVC addressing high bit rate operation, high bit depths, and alternative chroma formats such as monochrome, 4:2:2, 4:4:4, high bit depths, and high throughput.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment (for SHVC).

- SCCE: Screen content core experiment (for SCC).

- SCM: Screen coding model (for SCC).

- SD: Slice data; alternatively, standard-definition.

- SDR: Standard dynamic range – referring to video content having a brightness range that would produce a maximum brightness of approximately 100 nits on a reference display under reference viewing conditions.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM (for SHVC).

- SHVC: Scalable high efficiency video coding – a set of extensions of HEVC that uses layered coding to enable the coding of supplemental pictures, quality enhancement layers, spatial resolution enhancement layers, and colour gamut enhancement layers.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- Supplement: In ITU-T terminology, a document that assists its readers by providing non-normative information and suggestions (sometimes considered a TR in ISO/IEC terminology).

- SVC: Scalable video coding, especially when referring to the associated extensions of AVC.

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design at a more preliminary stage of work than those of CEs, e.g., as between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TR: Technical report – e.g., a collection of non-normative suggestion guidance on appropriate technical practices (sometimes considered a “supplement” in ITU-T terminology).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VCL: Video coding layer (as in AVC and HEVC, contrast with NAL).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut – referring to video content having a colour gamut that includes colours substantially outside of the range of values that is representable using Rec. ITU-R BT.709.

- WD: Working draft – a term for a draft standard, especially one prior to its first ballot in the ISO/IEC approval process, although the term is sometimes used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

Liaison activity

The JCT-VC did not directly send or receive formal liaison communications at this meeting. However, there was relevant liaison communication at the parent-body level; see also section 6.2.

Opening remarks

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

- It was again noted that number of contributions to this meeting is less than for the previous meeting, and has tremendously declined compared to other past meetings.

Primary topic areas were noted as follows:

- HEVC text publication:

  - 5th ed. for ITU – Consent at this meeting – incl. colour aspects, Main 10 Still, additional SEI messages, Monochrome 10, corrections

  - 4th for ISO/IEC – consider issuing a CD for that and a DoCR for SEI messages DAM ballot (closed 2017-10-16) – check/clarify

- Primetime Emmy Engineering award announced by ATAS

- Screen content coding (Consents?)

  - Software (code cleanup remains needed for this to become a completely adequate replacement for the HM) – FDAM1 ballot closed 2017-10-06

    - New bug report from Intel – Document JCTVC-AC0040.

  - Conformance – This is one of the top needs for work (currently at the WD stage, could not be progressed to PDAM1 of new edition yet; basis is pending FDIS ballot to be issued)

- HDR

  - ICTCP support – finalized by last meeting

  - Other SEI & VUI (see below)

  - Development of TR on HDR/WCG signalling, backward compatibility & display adaptation, finalized last meeting

  - Reference software to be developed – software relating to HDR was currently in the HM separate from SCM, plus a separate HDRTools library

- Potential new TR on signalling combinations in practical use

- Corrigenda items for version 4 (defect report of last meeting)

- Main 10 Still Picture profile – Not yet Consented in ITU-T, had completed its DAM2 ballot in ISO/IEC, resolution was issued to publish without changes

- Other SEI & VUI – Not yet Consented in ITU-T, DAM3 ballot completion in ISO/IEC (expected ballot closing date 2017-10-17 for the text issued as WG 11 N 16881), study text had been issued in last meeting – containing SEI messages for [update per previous meeting report]

  - Content colour volume: A. Tourapis, H. M. Oh, A. K. Ramasubramonian, P. Yin

  - Motion-constrained tile sets extraction information (2 messages): R. Skupin

  - Equirectangular 360° projection: J. Boyce, A. Tourapis, C. Fogg, G. J. Sullivan

  - Omnidirectional viewport [new]: J. Boyce

  - Region nesting: A. K. Ramasubramonian, E. François

  - updates from Study text …

- Test model texts and software manuals

Key deliverables initially planned from this meeting:

- SCC Reference software? (code cleanup remains needed for this to become a completely adequate replacement for the HM)

- SCC Conformance working draft update (request previously issued)

- Output on SEI messages (FDAM3 in ISO/IEC, ITU Consent)

- Updated Defect Report (incorporate into new edition)

- Monochrome 10 (was WD, incorporate into new edition)

- Potential new TR3 draft on colour combinations

- New HM, SHM, SCM document versions? HM17 with SCM integrated? (code cleanup remains needed for this to become a completely adequate replacement for the HM)

A single meeting track was followed for most meeting discussions.

Scheduling of discussions

Scheduling: Generally, meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and then proceed with review of contributions during the first few days. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Thu. 19 Oct., 1st day

  - 0900 Opening remarks, status review, AHG report review (GJS & JRO)

  - 1115 360° omnidirectional video SEI messages

- Fri. 20 Oct., 2nd day

  - 0900 Non-360° SEI messages

  - 1500 360° SEI messages

- Sat. 21 Oct., 3rd day

  - 0900 Non-360° topics

    - JCTVC-AC0040 errata

    - JCTVC-AC0036 technical report on video type identifiers

    - JCTVC-AC0039 figures for chroma type indication

    - JCTVC-AC0025 SEI manifest and prefix indication SEI messages

  - 1400 360° video topics

- Mon. 23 Oct., 5th day

  - 1400-1630 General review and finalization

- Tue. 24 Oct., 6th day

  - 0930-1000 Item 13 of JCTVC-AC0029

There were no requests in the closing plenary to present any remaining "TBP" contributions.

Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized as follows. Some plenary sessions were chaired by both co-chairmen, and others by only one. Chairing of other discussions is noted for particular topics.

- AHG reports (8) (section 2)

- Project development status (6) (section 3)

- Core experiments (0) (section 4)

- VUI and SEI messages (14) (section 5.1)

- Non-normative, encoder optimization (0) (section 5.2)

- Plenary discussions (section 6)

- Outputs & planning: AHG plans, Conformance, Reference software, Verification testing, CTC (sections 7, 8, and 9)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Topics discussed in final wrap-up at the end of the meeting (update)

Notes on potential remainders near the end of the meeting:

- Output preparations (see section 9 for full list)

- Plans

  - AHGs

  - CEs – None.

  - OLSs to be produced by the parent bodies (routine between each other only)

  - Reflectors (jct-vc) & sites (phenix and ftp3) to be used in future work

  - Meeting dates (Start Saturday, 20 January)

  - Doc deadline (Thursday, 11 January)

On issue was raised at the end of the meeting

(also Rapaka)

There were no requests to present any "TBP" contributions in the closing plenary.

AHG reports (8)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Thursday 19th a.m., except as noted.)

JCTVC-AC0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 28th JCT-VC meeting, the following (7) documents had been produced:

- The HEVC test model (HM) 16 Update 9 encoder description;

- For the HEVC text specification, Draft 1 of a defect report listing potential issues that may require corrective action;

- For supplemental enhancement information (SEI) and video usability information (VUI), Draft 3 of additional SEI message for HEVC and Draft 5 of ICtCp support in HEVC;

- For the HEVC format range extensions (RExt), Draft 1 of an HEVC Monochrome 10 profile;

- For high dynamic range (HDR) considerations, Draft 3 of a technical report on Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video;

- For the HEVC screen content coding (SCC) extensions, draft 6 of conformance testing specification.

The work of the JCT-VC overall had proceeded well in the interim period with a number of input documents submitted to the current meeting. Some discussion had been carried out on the group email reflector (which had 1288 subscribers as of 2017-10-17), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2017_07_AB_Torino/), particularly including the following:

- The meeting report (JCTVC-AB1000) [Posted 2017-10-18]

- High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9 (JCTVC-AB1002) [Posted 2017-10-15]

- Draft Text for ICtCp Support in HEVC (Draft 5) (JCTVC-AB1003) [Posted 2017-09-16, last updated 2017-10-11]

- HEVC Monochrome 10 Profile (Draft 1) (JCTVC-AB1004) [Posted 2017-10-09, last updated 2017-10-11]

- HEVC Additional Supplemental Enhancement Information (Draft 3) (JCTVC-AB1005) [Posted 2017-09-11]

- Defect Report for HEVC Text Specification (Draft 1) (JCTVC-AB1011) [Posted 2017-07-18]

- Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video (Draft 3) (JCTVC-AB1012) [Posted 2017-10-18]

- Conformance Testing for HEVC Screen Content Coding (SCC) Extensions and Non-Intra High Throughput Profiles (Draft 6) (JCTVC-AB1016) [Posted 2017-10-18]

The eight ad hoc groups had made progress, and reports from those activities had been submitted.

The software version HM16.16 had been prepared and released with appropriate updates and bug fixes approximately as scheduled.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-16.16.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-16.16-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

At the time of the opening of the meeting, 19 input contributions to the current meeting (not counting the AHG reports) had been registered for consideration at the meeting. The majority of these related to SEI messages and VUI usage. The official deadline for submissions had been established as Tuesday 10 October 2017. Eight of the 19 contributions had been registered or uploaded more than 24 hours past that deadline (in Paris/Geneva time).

A preliminary basis for the document subject allocation and meeting notes for the 29th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-AC0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne, M. Naccari, J.-R. Ohm, K. Sharman, G. Sullivan, Y.-K. Wang]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 28th meeting in Torino, IT (July 2017) and the 29th meeting in Macao, CN (October 2017).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The following work is noted in the context of AHG2:

- The output document “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9” (JCTVC-AB1002) was produced and uploaded, replacing the previous release of the HM Encoder Description.

- The output document “Defect report for HEVC text specification (Draft 1)” (JCTVC-AB1011) includes a list of problems reported previously

- A new bug report in JCTVC-AC0040 was submitted for consideration at the current meeting.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description.

- Review contributions with errata and decide on their inclusion in future releases of the HEVC specification.

- Decide whether to issue a further update to the ‘Encoder Description’ document incorporating any suggested changes. In particular, decide a plan for potential merging of the HM and SCC Encoder Description documents.

JCTVC-AC0003 JCT-VC AHG report: HEVC HM, SCM, SHM and HDRTools software development and software technical evaluation (AHG3) [K. Suehring  (chair), B. Li, K. Sharman, V. Seregin, A. Tourapis (vice-chairs)]

This report summarizes the activities of the AhG on HEVC HM, SCM, SHM and HDRTools software development and software technical evaluation that have taken place between the 28th and 29th JCT-VC meetings. Activities focused on integration of software adoptions and software maintenance, i.e. code tidying and fixing bugs.

A brief summary of activities related to each mandate is given below. In particular, for the HM, the following activities were performed:

- Include software contributions from the last meeting

- Fix errors

- Update configuration files for search range and add HDR configurations

- Release HM 16.16 and HM 16.17.

For SCM, the following activities were performed:

- Release HM16.16+SCM8.5

For HDRTools, no activities are reported.

Two new HM revisions were released since the last meeting.

HM 16.16 contains the following changes:

- Change random access search range to 96/384 (JCTVC-AB0035)

- calculation of QP adjustment for second part of sequence (JVET-E0059/JVET-G0101/JCTVC-AB0043)

- multi-scale structural similarity (MSSSIM) (JVET-F0064)

- apply bug fix for mapping from luma level to delta QP (as reported during JEM integration)

- add HDR configuration files

- add capability to use “none” as profile for experimental coding without encoder conformance checks

Due to the search range change, for 4:2:0 random access test conditions, performance changes as follows:

|  | Random Access Main |  |  | Random Access Main 10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | −0.1% |
| Class A2 | −0.4% | −0.4% | −0.4% | −0.4% | −0.4% | −0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | −0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | −0.1% | 0.0% | −0.1% | −0.1% | −0.1% | 0.0% |
|  | −0.1% | 0.0% | −0.1% | −0.1% | −0.1% | −0.1% |
| Class F | −0.1% | −0.1% | −0.1% | −0.1% | −0.2% | −0.2% |
| Enc. Time[%] | 107% |  |  | 106% |  |  |
| Dec. Time[%] | 101% |  |  | 100% |  |  |

The full performance data are reported in attached Excel sheets.

HM 16.17 contains the following changes:

- Add High-throughput RExt profile definitions.

- Compile fixes (clang)

There was no change in coding performance related to this change. Thus, no performance data is reported.

Towards HM 16.18, there were a number of agreed modifications still to be included:

- The adopted changes in JCTVC-Y0038 that include changes in the GOP settings, which require coordination with JVET for JEM development.

- The cross-component peak signal to noise ratio calculation, as discussed in JCTVC-Y0037.

HM16.16+SCM8.5 includes the following modifications:

- Merge with HM16.16.

Compared with HM-16.15+SCM-8.5, there is no coding efficiency change.

There has not been a release of a new SHM revision.

There has not been a release of a new HDRTools revision.

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA:

Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices:
		Tickets #1314, #1338, #1339.

- Field-coding:
		Tickets #1145, #1153.

- Decoder picture buffer:

Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing:

Tickets #1335, #1336, #1393.

- Additional decoder checks:

Tickets #1367, #1383.

Further testing and possibly extensions of the scaling support in HDRTools, as well integration of other display mapping mechanisms, is currently in progress.

JVET revised several class A test sequences during the last meeting in Torino. These changes are not yet reflected in JCT-VC CTC.

- Continue to develop reference software based on HM version 16.17, HM16.16+SCM8.5, SHM 12.3 and HDRTools v0.16 and improve their quality.

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Encourage people to submit configuration files that trigger bugs in HDRTools.

- Continue to investigate the merging of branches.

- Keep common test conditions aligned with JVET

JCTVC-AC0004 JCTVC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, R. Joshi, Y. Ye, J. Xu]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

For SCC:

HEVC Screen Content Coding extensions conformance testing will test the following profiles:

Screen-Extended Main, Screen-Extended Main 10, Screen-Extended Main 4:4:4, Screen-Extended Main 4:4:4 10, Screen-Extended High Throughput 4:4:4, Screen-Extended High Throughput 4:4:4 10, and Screen-Extended High Throughput 4:4:4 14.

The following bitstreams were planned to generate. All bitstreams except High Throughput profiles were generated and were available at the ftp site.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/draft_SCC_conformance/

- Palette size 0/1:

  - Zero_and_One_Palette_Size_A_Canon (Screen-Extended Main)

- Slice ACT QP offsets:

  - Slice_ACT_QP_Offsets_A_Qualcomm (Screen-Extended Main 4:4:4)

- delta QP and chroma QP offsets signalled in the palette block:

  - Delta_QP_Chroma_QP_Offsets_A_Qualcomm (Screen-Extended Main 4:4:4)

- Motion vector resolution set to full pel or quarter pel:

  - MVRESIDC_A_MS (Screen-Extended Main 4:4:4)

  - MVRESIDC_B_MS (Screen-Extended Main 4:4:4)

  - MVRESIDC_C_MS (Screen-Extended Main 4:4:4)

- High Throughput profiles:

  - HT_A_SCC_Apple (Screen-Extended High Throughput 4:4:4 10)

  - HT_A_SCC_Apple (Screen-Extended High Throughput 4:4:4 14)

  - HT_A_SCC_Apple (Screen-Extended High Throughput 4:4:4 14)

The table below lists the bitstream features to be tested. Some bitstream features may be represented using multiple bitstreams.

| Chroma format | Bit depth | Category | Sub category | Bitstream feature | Volunteers | Candidates |
| --- | --- | --- | --- | --- | --- | --- |
| 4:4:4 4:2:0 | 8/10 | Palette | Predictor palette initialization (PPS/SPS/initialized to zero) |  | InterDigital |  |
|  |  |  | Palette size 0/1 |  | Canon |  |
| 4:4:4 4:2:0 | 8/10 | Current picture reference (CPR) | bi-prediction restriction (conversion from bi to uni) |  | Qualcomm |  |
|  |  |  | DPB | in-loop filtering enabled/disabled | MediaTek |  |
| 4:4:4 4:2:0 | 8/10 | adaptive residual transform | slice ACT QP offsets |  | Qualcomm | InterDigital |
| 4:4:4 4:2:0 | 8/10 | adaptive motion vector resolution | motion_vector_resolution_ control_idc = 0/1/2 |  | Microsoft |  |
| 4:4:4 4:2:0 | 8/10 | Intra coding | Intra boundary filtering disable |  | MediaTek |  |
| 4:4:4 4:2:0 | 8/10 | delta QP / chroma QP offset signalling | delta QP and chroma QP offsets are signalled in the palette block |  |  |  |
| 4:2:2 4:4:4 | 8/10/14 | Screen-extended high throughput profiles | Enable tiles and wavefronts in the same bitstream |  | Apple |  |

It was remarked that one bitstream is missing from there that is described in the document, and that there are a couple of bitstreams on the ftp site that are not listed in the document.

It was also remarked that some new bitstreams appear to have recently been made available, so the AHG report was not up to date.

JCTVC-AC0005 JCT-VC AHG report: Test sequence material (AHG5) [T. Suzuki, V. Baroncini, E. Francois, P. Topiwala, S. Wenger, H. Yu]

No new activity was reported. The report includes the information about what test sequences have previously been collected and where that data is found.

JCTVC-AC0006 JCT-VC AHG report: Report development for usage of video signal type code points (AHG6) [Y. Syed, C. Fogg (co-chairs)] [miss]

This report was discussed together with JCTVC-AC0036 on Saturday 21 October 0900 (chaired by GJS).

No reflector discussions were reported prior to the Macao meeting.

Input document JCTVC-AC0036 contained proposed draft text and was recommended to be used as a starting point for discussions.

JCTVC-AC0007 JCT-VC AHG Report: Supplemental enhancement information (AHG7) [J. Boyce, A. K. Ramasubramonian, G. J. Sullivan, R. Skupin, A. Tourapis]

This document summarizes the activity of AHG7: Supplemental enhancement information between the 28th meeting at Torino, IT July 2017 and the 29th JCT-VC meeting at Macau China, October 2017.

The main activity of the AHG was to prepare the output document JCTVC-AB1005 HEVC Additional Supplemental Enhancement Information (Draft 3), to incorporate adopted changes from the Torino meeting. JCTVC-AB1005 contains draft text for an amendment to HEVC containing SEI messages for content colour volume, omnidirectional 360° projection, omnidirectional viewport, regional nesting, and motion-constrained tile sets extraction information.

There was no significant email reflector discussion.

There are 16 SEI related contributions. 14 contributions are related to the SEI messages included in JCTVC-AB1005, of which 12 are related to omnidirectional video, and 2 are related to MCTS. Two contributions propose new SEI messages. The contributions are categorized below.

The AHG recommends the following:

- Review input contributions

- Coordinate with MPEG OMAF activity to harmonize SEI message contents with OMAF information (suggested to meet on Saturday morning)

- Finalize SEI message decisions for the HEVC amendment

JCTVC-AC0008 JCT-VC AHG report: Report development for HDR/WCG signalling, backward compatibility, and display adaptation (AHG8) [E. François (chair), W. Husak, D. Rusanovskyy, P. Topiwala, P. Wu (vice-chairs)]

This document reports the activity of the ad hoc group on HDR/WCG technology for backward compatibility, display adaptation, and quality enhancement post-processing, conducted between the 28th and 29th JCT-VC meetings. The report presents the mandates of the AHG, a summary of the AhG activity, and recommendations.

A kick-off message was sent to the e-mail reflector on August 4, 2017, listing the AHG mandates and suggesting emails discussion on these issues to take place on the JCT-VC reflector.

The editors of “Signalling, Backward Compatibility and Display Adaptation for HDR/WCG Video Coding” conducted the editorial work on improving the text. The editing of the technical report was completed and the revised text was delivered as JCT-VC contribution JCTVC-AB1012 “Signalling, Backward Compatibility, and Display Adaptation for HDR/WCG Video (Draft 3)”.

No input contribution appears to be related to AHG8.

Project development, status, and guidance (5)

Text improvements and corrigenda items

JCTVC-AC0040 SCC software and text discrepancy for rounding with bipred restriction [J. Boyce, I. Moccagatta, L. Xu (Intel)]

This contribution was discussed Saturday 21 October 0900 (chaired by GJS).

A discrepancy is reported between the SCC draft text and the SCM software, related to rounding and bipred restriction. It is suggested that the draft text is correct, and that the software should be changed to correct the discrepancy. A software ticket has been submitted, #1479.

This requires correction of the conformance specification as well as the software.

The number of affected bitstreams has not been thoroughly determined. The problem was noticed when checking the MVRESIDC_C_MS_1 conformance bitstream.

This highlights both the importance of the conformance test set and its lack of readiness at this stage.

Evaluation metrics

JCTVC-AC0026 BD-Rate/BD-PSNR Excel extensions [A. M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc)]

See notes in the JVET report. It was agreed that this did not need separate presentation in JCT-VC.

NB ballot comments on the DAM ballot for additional SEI messages

This topic was discussed Monday 23 October 1515, chaired by GJS.

The ballot results for the DAM ballot ISO/IEC 23008-2:2017/DAM3 (HEVC additional supplemental enhancement information) were reported to WG11 in its input document m41418 and were reviewed. These comments were generally supportive of the current state of work in the JCT-VC on additional SEI message development. A desire for alignment with the design of the OMAF specification under development in WG11 was expressed in the comments, and outcomes on such topics were coordinated with that work to ensure appropriate alignment. Some of the NB comments overlapped substantially with other input considered by JCT-VC and discussed elsewhere in this report.

A Disposition of Comments Report N 17201 was issued by WG 11 to document the outcome of the comment review. Non-editorial aspects on which action was taken that may not be fully covered in other notes in this report are recorded in the table below. Clearly redundant aspects are not repeated here.

The numbering used in the table below is merely for identification purposes herein and does not correspond with the numbering of the comments used for MPEG ballot response or disposition recording purposes.

| Index | Topic | Suggestion and response |
| --- | --- | --- |
| 1 | The replacement semantics of general_non_packed_constraint_flag do not include the region-wise packing SEI message. The semantics of the region-wise packing SEI message make no mention of the general_non_packed_constraint_flag. | It was suggested to 1) add the region-wise packing SEI message to the replacement semantics of general_non_packed_constraint_flag, or 2) add a constraint to the semantics of the region-wise packing SEI message forbidding presence of the region-wise packing SEI message when general_non_packed_constraint_flag is equal to 1 in the active SPS for the current layer JCT-VC response: Accepted in spirit. The presence of the region-wise packing SEI message has been restricted to when the equirectangular projection SEI message, or cubemap projection SEI message is present, and these require general_non_packed_constraint_flag equal to 0. |
| 2 | The two SEI messages related to omnidirectional projections are grouped together using payloadType equal to 150 and 151. PayloadType equal to 152 is not available for potential future SEIs for further omnidirectional projections. | It was suggested to ensure that existing and future SEIs omnidirectional projections may lie within a common range of payloadTypes. JCT-VC response: Accepted. A gap in numbers has been reserved after the two projection types. |
| 3 | There are differences between ISO/IEC 23008-2 DAM 3 and the OMAF specification with respect to the 360 video: e.g., region-wise packing structure with guard band, fisheye video structure, and cube map projection format. As those structures and format are critical for some applications designed in OMAF, it is necessary to define them in the current specification to avoid confusion in the market. | Consider defining structures and a format that is not included in the specification but defined in OMAF, that is, region-wise packing with guard band structure, fisheye video structure, and cube map projection formation. JCT-VC response: Partially accepted. Guard band signalling was added to the region-wise packing SEI message. A cubemap projection SEI message was added. |
| 4 | It seems “cleaner” from a design perspective for different projection types to be specified in different SEI messages, rather than having different types supported within the same SEI message. This would make the structure of the specification text simpler, and improve the clarity of what it means for a product to support an SEI message. | It was suggested to remove the reserved projection type syntax element (omni_projection_type) from the omnidirectional projection indication SEI message and rename the name of the SEI message to reflect a scope limitation to equirectangular projection. JCT-VC response: Accepted |
| 5 | Rotation signalling for omnidirectional video content is duplicated in both projection format SEIs while in OMAF, rotation is signalled separately from projection format. | In the spirit of aligning the rotation signalling aspect with OMAF, it was suggested to use a separate, not projection format specific SEI message for signalling of rotation parameters instead of the current design. JCT-VC response: Accepted. |
| 6 | First, padding in cube map projected video seems not fully specified, e.g. wrt the position of padding samples. Second, similar functionality for equirectangular projected video uses a different signalling mechanism (explicit sample range signalling vs. a coverage indication with greater than 360 degree yaw angle). Third, position of padding samples is not flexible in both projection formats, e.g. for address tilling scenarios. | In the spirit of unification of the padding signaling aspect and alignment with OMAF, it was suggested to add guard band functionality to region-wise packing as defined in OMAF and instead of the current design. JCT-VC response: Accepted. Guard band signalling was added to the region-wise packing SEI message, and the existing padding signalling in the cubemap projection SEI message was removed. |
| 7 | There are errors in the design of the rotation aspects of the omnidirectional projection indication SEI message. The drafted equations are incorrect. To enable any location on the sphere to be placed at the center of the coverage region with any rotation, seven parameters are needed to describe the region covered by the raster – e.g., three angular rotation parameters to establish a position and rotation of the sphere for the center of a conceptual complete projection map, and a min and max latitude and min and max longitude on the rotated sphere to identify the area covered by the conformance cropping window. | Replace the current design with the described seven-parameter design. Consider, however, that it may be appropriate for some of these parameters to be sent in a different SEI message than others rather than carrying all the related syntax elements in the same SEI message, as discussed in other US comments. JCT-VC response: Partially accepted. The sample remapping equations for the equirectangular projection were aligned between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 8 | In some use cases, regions outside of an omnidirectional projection mapped area may be handled using a “padding” or “guard band” scheme. There are several potential variations of what a decoder might want to do with the decoded video if it is made aware of the type of handling used in these regions. | Provide a support for a signalling of “padding” / “guard band” regions and indication of the type of guard band that is present. Coordinate the approach with the OMAF activity. JCT-VC response: Accepted. Guard band with an indicated type for the equirectangular projection can be realized by using either the guard band signalling added into the equirectangular projection SEI message or the guard band signalling added into the region-wise packing SEI message. |
| 9 | The omni_viewport_id syntax element has no currently defined usage, and may be useful for indicating a “director’s cut” viewport or a “most popular” viewport indication. | Define ID value 0 to mean “director’s cut” (and clarify that the loop counter can be used for priority order, with first being highest priority), and ID value 1 means “most viewed by statistical measurements” (and clarify that the loop counter is in descending popularity order), keeping the other ID values defined as currently. JCT-VC response: Accepted. |
| 10 | In the extraction of a sub-picture from a decoded picture, regions in the separated boundaries of ERP could be considered as one output picture when they are in the wrap-around position. As using those regions is a practical use case in 360 video applications, this should be considered in MCTS extraction information set SEI message. | Find solutions to support the rearrangement of MCTSs in an output picture where the MCTSs are allocated in the wrap-around position of the decoded picture. One possibility of the solution could be a syntax efficient approach which derives the new slice segment addresses using the existing syntax element. Given a precise technical description of the slice address derivation process, consider to support the syntax efficient solution for the benefit of avoiding burdensome lengthy syntax elements. JCT-VC response: Partially accepted. A method of enabling the described functionality is included, although somewhat different than suggested. It was not clear that the amount of bit rate savings that could be provided by the implicit derivation method would be sufficient to justify its inclusion. |
| 11 | In some use cases, the syntax and semantics of the MCTS extraction information sets SEI message is insufficient to enable the decoder/rewriter to determine replacement slice addresses for the slice addresses that are found in the bitstream. Such a scheme is described in the study text. | Add the ability for the encoder to signal replacement slice addresses for use in the rewriting process or define a way to derive replacement slice addresses from the ordering of the associated syntax elements, e.g., as described in the study text. However, please note that the study text may have a lack of clarity in the semantics, since without specifying the index values when referring to a syntax element like “output_slice_segment_address[ ][ ]” could be technically unclear and cause interoperability issues. Clearly specify the index values and their usage when referring to such syntax elements. JCT-VC response: Accepted. Also, referring to a syntax element like “output_slice_segment_address[ ][ ]” without clearly indicating the array index values was avoided. |
| 12 | The additions to the informative persistence table miss entries for the region-wise packing SEI message and the cubemap projection SEI message. The additions to the table table contain an outdated entry for the former omnidirectional projection information SEI message. | It is suggested to update table D.1 with entries for each SEI message in HEVC DAM3. JCT-VC response: Accepted. |
| 13 | The semantics of the projection format related SEI messages define coordinate systems with, amongst others, defining one axis as “Lateral (side-to-side)”. | It is suggested to clarify the positive direction of the lateral axis, i.e. left or right. JCT-VC response: Accepted. |
| 14 | The region-wise packing SEI message provides information to enable remapping of the colour samples of the cropped output pictures onto projected pictures. The projected picture uses projection format. However, presence of a region-wise packing SEI message does not require presence of either an equirectangular projection SEI message or a cubemap projection SEI message. | It is suggested to add a requirement that the region-wise packing SEI message shall not be present for a picture unless a projected picture is constituted through presence of a SEI message defining a projection format, e.g. identified through a range of SEI payloadTypes. JCT-VC response: Accepted. |

360° video specification issues (2)

JCTVC-AC0021 Topics for coordination between OMAF and JCT-VC in Macau [Y.-K. Wang (Qualcomm)]

This contribution was discussed Thursday 19 October 2017 1115 (chaired by GJS).

This contribution, submitted as JCTVC-AC0021 and MPEG m41441, provides a list of topics that the author thinks should be coordinated between OMAF and JCT-VC, targeting at aligned designs between OMAF version 1 and the omnidirectional video related SEI messages that covering the same functionalities. For each topic, a suggestion was made.

| Index | Topic | Suggestion and response |
| --- | --- | --- |
| 1 | OMAF clause 5.1 includes a detailed description of the global and local coordinate axes as well as the rotations for conversion between the two, using two figures (Figures 5-1 and Figure 5-2) with some discrepancy between them. In JCTVC-AB1005-v1, a simpler description without using a figure is included (in the semantics of both the equirectangular projection SEI message and the cubemap projection SEI message). | Due to that 1) there was some complaint about using the drafted smiley face figure in standard specs being a bit strange, and 2) there is some redundancy and discrepancy between the two figures in OMAF clause 5.1, it is suggested to remove the two figures from OMAF clause 5.1 and align the description therein with the description in JCTVC-AB1005-v1. No specification text is provided for this suggestion. JCT-VC response: Relatively low priority, purely editorial; a figure could be included if an appropriate figure is provided. No action was really necessary, and this was delegated to the editors. |
| 2 | The equirectangular projection SEI message includes the signalling of rotation information as part of the SEI message. The contribution asserts, however, that the cubemap projection SEI message does not include the signalling of rotation information. In OMAF, the rotation information is signalled in the file format level using a separate structure than that for the projection, and that rotation information signalling applies to any projection type. | It is suggested to use a separate SEI message for signalling of rotation parameters such that the same syntax could be used for any type of projection. JCTVC-AC0022 contains a proposal that includes detailed specification text changes relative to JCTVC-AB1005-v1 for this suggestion. No text changes to the OMAF draft specification is considered needed for this suggestion. JCT-VC response: See notes for JCTVC-AC0023, JCTVC-AC0030, and JCTVC-0041. (Actually, the drafted cubemap SEI message had also included rotation parameters, but rotation was agreed to be put into a separate message that can apply to either projection type.) |
| 3 | The equirectangular projection SEI message includes signalling of the sphere coverage information as part of the SEI message. However, the cubemap projection SEI message does not include signalling of the sphere coverage information. In OMAF, the sphere coverage information is signalled in file format level using a separate structure than that for the projection, and that sphere coverage signalling applies to any projection type. | It is suggested to use a separate SEI message for signalling of sphere coverage information such that the same syntax could be used for any type of projection. See another aspect in item 4 below. JCTVC-AC0024 contains a proposal with detailed specification text changes relative to JCTVC-AB1005-v1 for this suggestion. No text changes to the OMAF draft specification is considered needed for this suggestion. JCT-VC response: See notes for JCTVC-AC0024 and JCTVC-AC0032. The sample remapping equations for the equirectangular projection were aligned between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 4 | The equirectangular projection SEI message uses four parameters for signalling of the sphere coverage information: erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max. To allow signalling of coverage sphere region that spans across the left and right boundaries of the projected picture, the values of erp_azimuth_min and erp_aizmuth_max can be outside the range of −180 to 180 degrees, inclusive. Allowing the azimuth range to be greater than 360 degrees enables support of ERP padding. In multiple places in OMAF and in the omnidirectional viewport SEI message, a sphere region (including a sphere coverage region) is signalled by indicating its center position and the azimuth and elevation ranges. In addition, a sphere region may be a tilted sphere region indicated by the tilt angle, which is asserted to be missing in the current sphere coverage signalling in JCTVC-AB1005-v1. | It is suggested to signal the sphere coverage information by specifying the center point of the coverage sphere region, the azimuth and elevation ranges, and the tilt angle of the sphere region. This would make the signalling aligned with sphere region signalling in the omnidirectional viewport SEI message as well as sphere coverage signalling in OMAF. JCTVC-AC0024 contains a proposal with detailed specification text changes relative to JCTVC-AB1005-v1 for this suggestion. No text changes to the OMAF draft specification is considered needed for this suggestion. JCT-VC response: Min & max versus center & range is basically a trivial matter; it was agreed to do that under the assumption that it helps with alignment. The additional tilt angle signalling does not seem necessary, so no action was needed on that. (We suggest that OMAF should remove the extra tilt aspect from their coverage signalling.) See also the notes on JCTVC-AC0024. |
| 5 | The semantics of the equirectangular projection SEI message and the cubemap projection SEI message allow the use of omnidirectional projection with frame packing types 3, 4, and 5, while in OMAF omnidirectional projection can be used only with frame packing types 3 and 4. Packing type 5 is temporal interleaving. | The contributor said that only frame packing types 3 and 4 are widely used and supported for omnidirectional video, and suggested to disallow the use of omnidirectional projection with frame packing type 5, for both the equirectangular projection and the cubemap projection. No specification text is provided for this suggestion. JCT-VC response: That mode seems useful and not difficult to include. (We suggest that OMAF should support it rather than removing our drafted support for it.) |
| 6 | On guard band signalling, OMAF relies on the signalling that is part of the region-wise packing syntax, while in JCTVC-AB1005-v1, the syntax of the region-wise packing SEI message does not include guard band signalling, and the cubemap projection SEI message includes a guard band syntax (although it is called padding) that is different than that in OMAF. Also, it should be noted that there some are issues in JCTVC-AB1005-v1 regarding padding or guard band signalling for the cubemap projection, as described below. The pictureWidth and pictureHeight (of the monoscopic projected luma picture) in the cubemap projection equations should not count any padded samples. Therefore, when the region-wise packing signalling is not present, in which case the size of the projected picture is not signalled, the size of the projected picture needs to be derived based on the cubemap projection SEI message syntax (the padding part). The width should be set equal to the width of the cropped output picture minus the total number of columns of padded samples, and the height should be set equal to the height of the cropped output picture minus the total number of row of padded samples, and there needs to be a constraint to require that all the remaining samples of the cropped output picture (i.e., excluding all the padded samples) shall exactly form a rectangle, which is the projected picture. When the region-wise packing signalling is present for CMP, there needs to be a constraint that no packed region shall contain any padded sample. Specifications for the above derivation of the size of the projected picture and the constraints are currently missing. In addition, the semantics of the CMP padding parameters are not clear, e.g., when cmp_padding_type is equal to 2 or 3, and the entire semantics of cmp_padding_chroma_sample_range_minus1 (e.g., regarding the position of the padded samples). Also the naming of the syntax element cmp_padding_chroma_sample_range_minus1 is a bit strange. Why chroma? So the padding here has nothing to do with luma? The above issues can be resolved by adding guard band padding signalling into region-wise packing syntax, same as in OMAF, and relying on that for providing support of guard band padding, i.e., remove the padding signalling from the cubemap projection SEI message syntax. However, on the other hand, it is believed that it'd be beneficial to allow support of simple padding for CMP without the need of supporting the RWP signalling. | The contributor proposed to - Add guard band padding signalling into the syntax of the region-wise packing SEI message, as in OMAF. - Replace the current CMP padding with an optional CMP padding along the four picture boundaries and in the middle of the picture between the boundaries of the upper three cubemap faces and the lower three cubemap faces. When the optional CMP padding exists, the RWP signalling shall not be present. - Make text changes to the sample location remapping process for addressing the cases where the optional CMP padding exists. For the above suggestion, JCTVC-AC0023 contains a proposal with detailed specification text changes relative to JCTVC-AB1005-v1. The MPEG input document m41459 contains text changes to the latest OMAF draft specification in the MPEG output document N16950 for the suggestion. JCT-VC response: See notes for JCTVC-AC0023, JCTVC-AC0030, and JCTVC-AC0041. |
| 7 | In the semantics of the region-wise packing SEI message, currently (same as in the OMAF draft text before the Torino MPEG meeting in July 2017), the unit of the size of the projected picture and the size of projected and packed regions are either unspecified or specified as luma samples. In the latest OMAF draft text, these sizes are specified in relative units, to allow the use of the same region-wise packing syntax for multiple bitstreams representing the same source video content. For example, multiple bitstreams representing the same source video content may be generated for adaptive streaming purpose. | It is suggested to align the syntax and semantics of the region-wise packing SEI message with the region-wise packing syntax and semantics in the latest OMAF draft text, to signal packed picture sizes and the sizes of the projected and packed regions in relative units. For the above suggestion, JCTVC-AC0023 contains a proposal with detailed specification text changes relative to JCTVC-AB1005-v1. No text changes to the OMAF draft specification is considered needed for this suggestion. JCT-VC response: See notes for JCTVC-AC0023. |
| 8 | In JCTVC-AB1005-v1, recommended viewports, generated per director's cut or viewing frequency, may be signalled using the viewport omnidirectional viewport SEI message. In OMAF, the director's cut type of recommended viewport is supported, but not the type per viewing frequency. | It is suggested to align OMAF with the HEVC omnidirectional viewport SEI message by adding the recommended viewport type of most-viewed viewports by statistical measurements. For the above suggestion, the MPEG input document m41462 contains a proposal with detailed specification text changes relative to the latest OMAF draft specification in the MPEG output document N16950. No text changes to the OMAF-related SEI messages in JCTVC-AB1005-v1 were considered needed for this suggestion. JCT-VC response: Not an action item for JCT-VC. (We suggest that OMAF should align with the JCT-VC approach.) |
| 9 | The overall sample location mapping processes are aligned between JCTVC-AB1005-v1 and OMAF. However, there is one significant discrepancy in the mapping equations for the equirectangular projection. In JCTVC-AB1005-v1, the mapping equations for the equirectangular projection involve the sphere coverage parameters, while this is not the case in OMAF. A basic assumption in OMAF of the overall sample location mapping processes, which involve RWP, is that the projected picture conceptually covers exactly the entire sphere. However, the current ERP equations in JCTVC-AB1005-v1 violates this assumption, but rather assumes that the projected picture covers exactly the indicated sphere coverage, which can be a subset of, the same as, or a subset of the entire sphere. When both the ERP SEI message the RWP SEI message are present, both of the two conflicting assumptions are in use in the sample location mapping processes, and the result would not be correct in this case. On the other hand, in OMAF, the RWP signalling needs to be present even when the true RWP functionality like region resizing, repositioning, rotation, mirroring as well as advanced guard band are not needed, e.g., for support of sub-sphere coverage and simple ERP padding. | It is suggested to align the ERP mapping equations between the two specifications as follows: - For ERP, if an applicable region-wise packing (RWP) SEI message is not present, the projection equations that involve the sphere coverage parameters apply; otherwise, the projection equations that do not involve the sphere coverage parameters apply. - For both cases, the azimuth equation is changed a little bit such that the left side of a full coverage ERP picture corresponds to −180 degree instead of 180 degrees. For the above suggestion, JCTVC-AC0024 contains a proposal with detailed specification text changes relative to JCTVC-AB1005-v1. The MPEG input document m41459 contains a proposal with detailed specification text changes relative to the latest OMAF draft specification in the MPEG output document N16950 for the above suggestion. JCT-VC response: See notes for JCTVC-AC0024. JCT-VC response: Ordering, persistence scope, and allowed presence combinations should be checked/clarified. Interpretation and constraints on the syntax may be dependent on whether RWP is present in combination with the ERP. RWP and rotation message shall only be present when ERP or CMP is active, and decoders shall ignore them if they are present in such a case. |
| 10 | Regarding the equations for the cubemap projection (in clause D.3.4.51.2 of JCTVC-AB1005-v1 and clause 5.2.2 of the OMAF SoDIS), there are the following discrepancies: - In JCTVC-AB1005-v1 there is a constraint that requires that pictureWidth shall be a multiple of 3 and pictureHeight shall be a multiple of 2, but not in the OMAF SoDIS. - There were a few switches between '/' and '÷' at the beginning of the equations (rows 1, 2, 5, 6). | It is suggested to discuss these discrepancies and align them. No specification text is provided for this suggestion in this contribution. Experts who are familiar with the CMP equations are encouraged to study the discrepancies and provide alignment suggestions. JCT-VC response: Not a clear action item for JCT-VC; it is suggested that some OMAF aspects should be aligned with JCT-VC text. |
| 11 | In OMAF clauses 10.1.2.2, there is the following constraint: When the video does not provide full 360 coverage, for each picture, there shall be a region-wise packing SEI messages present in the bitstream that applies to the picture. However, this constraint wouldn't be needed when the sphere coverage information is present, since for the equirectangular projection the coverage in sphere domain is identical to the coverage in 2D picture domain, which the region-wise packing box provides in this scenario. In the context of the OMAF-related SEI messages, there were also requests, e.g., from Minhua Zhou of Broadcom, to make the design of the OMAF-related SEI messages work for sub-360 coverage scenarios without mandating the presence of the region-wise packing SEI message when not necessary (i.e., when the projection type is the equirectangular projection and the sphere coverage information is present). | It is suggested to make aligned designs for OMAF and the OMAF-related SEI messages to address this issue. Based on the suggestion for item#9 above, RWP is no longer needed for sub-360 coverage support for the viewport independent profiles. Therefore, it is suggested to require the absence of the RWP signalling altogether for the viewport independent profile. The MPEG input document m41459 contains a proposal with detailed specification text changes relative to the latest OMAF draft specification in the MPEG output document N16950 for the above suggestion. JCT-VC response: Not an action item for JCT-VC. |
| 12 | OMAF supports fisheye omnidirectional video. The fisheye video metadata is signalled in a file format box. However, there is no fisheye SEI message for support of fisheye omnidirectional video on elementary stream level. The 3GPP SA4 standardization work on FLUS (Framework for Live Uplink Streaming) reportedly needs a fisheye SEI message. For the background of FLUS, the proponent suggested to look at the latest description of FLUS available in S4-AHM363 (http://www.3gpp.org/FTP/tsg_sa/WG4_CODEC/Ad-hoc_MTSI/Docs/S4-AHM363zip) plus recently agreed additions documented in S4-170843 (ftp://ftp.3gpp.org/tsg_sa/WG4_CODEC/TSGS4_95/Docs/S4-170843.zip). | It is suggested to specify a fisheye SEI message, and align the fisheye video metadata signalling between the SEI message and the OMAF file format level signalling. JCT-VC response: A related proposal has been submitted as JCTVC-AC0034. See notes for that contribution. It was noted that fish-eye projection is not in the “baseline” of OMAF and that its specification did not seem mature when reviewed at the previous meeting. |
| 13 | There will most likely be OMAF and JCT-VC inputs to Macau that contain other topics than listed above for which coordination between OMAF and JCT-VC is necessary or desirable. | It is suggested to discuss as early as possible in OMAF meeting sessions (including Saturday and Sunday OMAF AHG meeting sessions) all OMAF inputs containing topics for which coordination between OMAF and JCT-VC is necessary or desirable, to allow joint discussions between OMAF and JCT-VC, knowing that JCT-VC will end by Tuesday. These topics should be identified early and considered in preparing the meeting agendas of both OMAF and JCT-VC. JCT-VC response: Not a specific action item for JCT-VC. |

JCTVC-AC0029 Comments on JCTVC-AB1005-v1 [M. M. Hannuksela (Nokia)]

This contribution was discussed Saturday 21 October 1535 (chaired by GJS).

This contribution included a set of comments on JCTVC-AB1005-v1 (HEVC Study of DAM3) and proposes modifications of the specification text or other actions to resolve the comments.

| 1 |  | The comments are authored relative to HEVC SoDAM3 (JCTVC-AB1005-v1). | No action requested. |
| --- | --- | --- | --- |
| 2 | All | There are significant similarities in clause HEVC SoDAM3 (JCTVC-AB1005-v1) compared to Omnidirectional MediaA Format (OMAF) SoDIS (MPEG N16950): - The semantics of sample locations in a decoded picture are specified very similarly in D.3.41.5 of HEVC SoDAM3 when compared to clauses 7.2.1 and 5 of OMAF SoDIS. - Additional definitions for clause 3 in HEVC SoDAM3 are identical or similar to the respective definitions in OMAF SoDIS. The amount of similar text is significant. For example, clause D.3.41.5 of HEVC SoDAM3 is about 6 pages in length. There is a burden of keeping the HEVC and OMAF standardization processes aligned in time, features, and detailed design (e.g., syntax structures and elements). Any possible corrigendum needs to be carried out in synchronization for both SEI and box definitions. | Instead of including specification text for, refer to MPEG-I metadata part when it comes to: - equations for omnidirectional projection formats, - region-wise packing equations and conversions, - conversion from one set of coordinate axes to another, - process to specify the semantics of sample locations in a decoded picture in terms of sphere coordinates. Refer to MPEG-I metadata part for definitions, or use identical definitions to those of OMAF. JCT-VC response: No action taken on the cross-referencing dependency, to avoid publication delay. However, we will editorially structure the text to be generalized and avoid aspects that would be different in one standard from the other, so that this part of the text can be identical (or as close to identical as possible), and perhaps referencing could be used in a future edition. |
| 3 | All | Some functionality that can be expressed by OMAF boxes cannot be expressed by omnidirectional video SEI messages (e.g., guard band indications in region-wise packing). Hence, a key design goal of the HEVC profiles of OMAF, i.e., replicating the metadata of boxes as SEI messages, is not reached. This functionality would be needed for the following purposes: - For encrypted video, the displaying process that interprets the omnidirectional video metadata may reside in the trusted domain and thus may only have access to the SEI messages and not the equivalent file boxes. - By having equivalent metadata present in both boxes and SEI messages, different player implementation architectures are enabled. In some players, the player logic has access to the file boxes and controls the displaying process, while in other players the displaying process runs independently on the basis of the decoder output only. - SEI messages form a convenient standardized interface between the video encoder and the file encapsulator. In other words, the content of the file boxes can be generated based on the SEI messages, and no implementation-specific interface to the file encapsulator is needed. | Provide the same functionality in the omnidirectional video SEI messages as available in the OMAF boxes. Design the syntax and semantics of the SEI messages so that a straightforward conversion from the SEI messages to the OMAF boxes is enabled. JCT-VC response: Resolved by the outcome recorded for JCTVC-AC0041; see notes for that contribution. |
| 4 | D.2.41, D.3.41 | The equirectangular projection SEI message and the cubemap projection SEI message include the rotation information. The rotation information is independent of which projection format is used and would also apply to any projection formats that might be specified in the future. | Exclude rotation syntax elements from the equirectangular projection SEI message and the cubemap projection SEI message. Specify a rotation SEI message that includes the rotation syntax elements. Align the syntax elements with OMAF's RotationBox and with OMAF's clause 5.3 for conversion from the local coordinate axes to the global coordinate axes. JCT-VC response: Resolved by outcome recorded for JCTVC-AC0041; see notes for that contribution. |
| 5 | D.2.41, D.3.41 | Frame packing arrangement SEI can be misinterpreted by decoders that omit the decoding of the region-wise packing SEI message. When region-wise packing SEI message is present in the bitstream, the arrangement indicated by the frame packing arrangement SEI message is intended to indicate the frame packing of the projected picture, i.e., the picture that results when the regions of the decoded picture have been "unpacked" according to the region-wise packing SEI message. However, the semantics of the frame packing arrangement SEI message, as specified in clause D.3.16 of HEVC, apply to the decoded picture. Decoders are not required to decode all SEI messages for conforming output. Decoders not parsing the region-wise packing SEI message would interpret the frame packing arrangement SEI message incorrectly, when both SEI messages are present in the bitstream. | Include syntax elements for indicating the frame packing arrangement scheme of the projected pictures and the association of the left or right view to the specific constituent pictures within the region-wise packing SEI message. Require the region-wise packing SEI message and the frame packing arrangement SEI messages be mutually exclusive: when region-wise packing SEI message is present, frame packing arrangement SEI message shall not be present, and vice versa. Consider input contributions on this subject. JCT-VC response: Resolved by outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was rejected. Instead, the semantics of the frame packing arrangement SEI message was updated such that if an applicable region-wise packing SEI message exists, the packing arrangement scheme applies to the projected picture, otherwise, the packing arrangement scheme applies to the cropped decoded picture. |
| 6 | D.2.41.1, D.3.41.1, D.3.41.5 | erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max supposedly indicate azimuth and elevation angles of the center point of the top-left, top-right, bottom-left, and bottom-right samples of the cropped output picture. This conclusion can be drawn from the equations of the equirectangular projection in D.3.41.5.2. Consequently, in order to set erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max to correctly, the encoder needs to be careful. For example, if picture consists exactly the left half of an original ERP picture of size 4096x2048, the intuitive choices erp_azimuth_min equal to −180 (in degrees) and erp_azimuth_max equal to 0 are not correct values. erp_azimuth_min should be set equal to −180 + 360÷4096÷2 in degrees (i.e., the azimuth angle of the and center point of the left-most sample column) and erp_azimuth_max should be set equal to −360÷4096÷2 in degrees. The values in degrees should be further converted to the units of 2−16 degrees. It is believed that indicating the coverage as angular ranges is vulnerable to implementation mistakes. This could lead to interoperability problems between encoders and decoders/renderers. The correct choices of erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max depend on the picture width and height of the ERP picture from which the content represented by the bitstream is extracted, or equivalently the width and height of the samples in degrees. For example, if the pictures in bitstream A contain exactly the left half of original ERP pictures of size 4096x2048 and if pictures in bitstream B contain exactly the left half of original ERP pictures of size 3072x1536, erp_azimuth_min and erp_azimuth_max values for these two bitstreams would respectively differ. This is unintuitive and should be avoided. | It is proposed to use relative integer sample coordinates for indicating the coverage. In other words, rather than using erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max, it is proposed to indicate proj_picture_width, proj_picture_height, proj_reg_left, proj_reg_top, proj_reg_width, and proj_reg_height to indicate the coverage. proj_picture_width and proj_picture_height specify the width and height of a conceptual projected picture. proj_reg_left and proj_reg_top specify the sample location of the top-left sample of the cropped output picture on the conceptual projected picture. proj_reg_width and proj_reg_height specify the width and height of the cropped output picture in units of samples of the conceptual projected picture. For example, if the pictures contain exactly the left half of original ERP pictures, the proposed syntax elements can be set to the following values as an example: proj_picture_width = 4096, proj_picture_height = 2048, proj_reg_left = 0, proj_reg_top = 0, proj_reg_width = 2048, proj_reg_height = 2048. It is noted that proj_reg_left + proj_reg_width is allowed to be greater than proj_picture_width (and is handled in wraparound manner). It is also remarked that this design is aligned with OMAF's way of specifying coverage with RegionWisePackingBox, which when used with one region (for monoscopic video) specifies coverage on projected picture domain (i.e., on local coordinates) in manner that affects the mapping of locations on the decoded picture to sphere coordinates. JCT-VC response: Resolved by outcome recorded for JCTVC-AC0041; see notes for that contribution. The sample remapping equations for the equirectangular projection were aligned between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 7 | D.2.41.1, D.3.41.1, D.3.41.5 | Multiple syntax structures that achieve the same functionality should be avoided. As discussed in the previous comments erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max achieve the functionality offered by region-wise packing of one region (for monoscopic video) but are more vulnerable to implementation errors and less intuitive. | Exclude erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max from the equirectangular projection SEI message and their impact from the equations in D.3.41.5.2. When region-wise packing SEI message is present, require it to be present in the same SEI NAL unit as the equirectangular projection SEI message or the cubemap projection SEI message. Consequently, it is reasonably simple to discover that region-wise packing is applied. Provide in the region-wise packing SEI message a mechanism to indicate early in the message that the values of the syntax elements comply with certain pre-defined constraints. Specify a code point corresponding to the constraints of 'erpv' scheme type of OMAF. This suggestion aims at: - providing pre-defined constrained sets of region-wise packing so that decoders/renderers need not support its full flexibility, and - simplifying encapsulation of bitstreams into OMAF-compliant files. For example, the file encapsulator does not need to check the bitstream whether all constraints of the 'erpv' scheme type are fulfilled, since the SEI message already asserts that. JCT-VC response: Resolved by outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was accepted. The sample remapping equations for the equirectangular projection were aligned with the use of region-wise packing signalling between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 8 | D.2.41.1, D.3.41.1, D.3.41.5 | In the equirectangular projection SEI message, erp_azimuth_min and erp_azimuth_max are required to be in the range of −360°, inclusive, to 360°, exclusive, erp_azimuth_max is required to be greater than erp_azimuth_min. This provides the flexibility of content with greater than 360° azimuth range, supposedly intended to provide a guard band for the back seam of ERP and thus making it unperceivable. However, it is unclear which operation should be used in displaying decoded pictures that have content overlapping on sphere domain. For example, in stereoscopic ERP packed side-by-side, the guard band could simply represent a boundary pixel extension in order to improve inter prediction and should not be used for displaying. The syntax is not flexible enough to indicate stereoscopic ERP packed side-by-side and a guard in between the constituent pictures. The syntax is not flexible enough to indicate stereoscopic ERP packed in top-bottom manner with a guard band between the constituent pictures. | The proposal for item 7 applies. JCT-VC response: Resolved by outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was accepted. The sample remapping equations for the equirectangular projection were aligned, with the use of region-wise packing signalling between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 9 | D.2.41.1, D.3.41.1, D.3.41.5 | It is not specified in D.3.41.1 whether erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max are relative to the global or local coordinate axes. Based on the equations of the equirectangular projection in D.3.41.5.2, erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max are relative to the local coordinate axes. This design choice differs from CoverageInformationBox of OMAF, which indicates the coverage relative to the global coordinate axes. Moreover, CoverageInformationBox does not affect the projection equations mapping a location of 2D picture to sphere coordinates. In OMAF it is intended to use the coverage information for player's content selection. The player has access to the current viewing orientation relative to the global coordinate axes. In order to avoid conversion between coordinate axes, the coverage is indicated relative to the global coordinate axes. | Exclude erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max from the equirectangular projection SEI message and their impact from the equations in D.3.41.5.2. It is noted that it is not considered useful to specify a coverage information SEI message that would correspond to OMAF's CoverageInformationBox, i.e., would not affect the projection equations and would be relative to global coordinate axes. The coverage information SEI message is not envisioned to be used for content selection and is not used for rendering (since it does not affect the projection equations). JCT-VC response: Resolved by the outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was accepted. The sample remapping equations for the equirectangular projection were aligned, with the use of region-wise packing signalling, between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 10 | D.2.41.2, D.2.41.3, D.3.41.2 D.3.41.3 | The cubemap projection SEI message includes some padding functionality, which is similar to but not aligned with OMAF's guard bands and is more constrained. The differences include the following: - OMAF's guard bands are generic to any omnidirectional projection format, which is asserted to be a useful feature. - cmp_padding_type values of the cubemap projection SEI message do not have the same semantics as gb_type[i][j] values of RegionWisePackingStruct. Alignment of the semantics would be desirable. - cmp_padding_type is the same for each padding region of each cube face in the cubemap projection SEI message. gb_type[i][j] values can be freely selected per each side of each packed region. - The cubemap projection SEI message specifies padding that have equal thickness in each side of each cube face, whereas the thickness of guard bands on each side of a packed region can be freely selected in RegionWisePackingStruct. - The padding specified in the cubemap projection SEI message is not reflected in the equations in D.3.41.5.2 and hence specified incorrectly. | Include guard band signalling into region-wise packing SEI message with syntax structures and elements aligned with OMAF. Remove the padding related syntax elements from the cubemap projection SEI message. Provide in the region-wise packing SEI message a mechanism to indicate early in the message that the values of the syntax elements comply with certain pre-defined constraints. Specify a code point corresponding to a cube map that is otherwise identical to "unpacked" cube map but has a guard band between the rows of cube faces. This suggestion aims at providing pre-defined constrained sets of region-wise packing so that decoders/renderers need not support its full flexibility. JCT-VC response: Resolved by the outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was partially accepted. Signalling of guard band was added into the region-wise packing SEI message, and the existing padding signalling in the cubemap projection SEI message was removed. |
| 11 | D.2.41.3, D.3.41.3 | The region-wise packing SEI message lacks guard band indications, which are included in OMAF's RegionWisePackingBox. Guard bands are asserted to be useful when region-wise packing is in use, e.g. in assisting the rendering process, regardless which omnidirectional projection format is in use. | Include guard band signalling into region-wise packing SEI message with syntax structures and elements aligned with OMAF. JCT-VC response: Resolved by the outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was accepted. |
| 12 | D.3.41.1 | The semantics of erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max are unclear in the following aspects: - It is not specified whether these syntax elements are relative to the global or local coordinate axes. Based on the equations of the equirectangular projection in D.3.41.5.2, erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max are relative to the local coordinate axes, but this should be explicitly stated also in the semantics. - erp_azimuth_min, erp_azimuth_max, erp_elevation_min, and erp_elevation_max supposedly indicate azimuth and elevation angles of the center point of the top-left, top-right, bottom-left, and bottom-right samples of the cropped output picture. This conclusion can be drawn from the equations of the equirectangular projection in D.3.41.5.2. | If these syntax elements remain in the equirectangular projection SEI message, clarify their semantics as commented. JCT-VC response: Resolved by the outcome recorded for JCTVC-AC0041; see notes for that contribution. The suggestion was accepted. The sample remapping equations for the equirectangular projection were aligned, with the use of region-wise packing signalling, between the HEVC amendment 3 and OMAF version 1 specifications. With that change, signalling of a coverage region in an SEI message is no longer necessary and hence not in the projection type SEI messages. The region-wise packing SEI message can be used to indicate partial coverage. |
| 13 | D.3.41.4 | The omnidirectional viewport SEI message lacks an indication whether the viewport is taken from the left view or the right view of stereoscopic content, or if the metadata is meant for displaying on a stereoscopic display. | Include this functionality in the omnidirectional viewport SEI message. Initial planned action: The viewport applies to both views. Add a “left view preferred flag” to indicate which view is preferred for monoscopic viewing of a stereoscopic viewport. This was discussed further Tuesday 24 October 0930 (chaired by GJS) The question of exactly how to draft the planned action that was requested in response to this item was discussed. We had said we planned to add a flag, but it did not seem very clear how to fit in such a flag. The flag could hypothetically either be placed inside a loop or outside of it, and would also undesirably affect byte alignment in an area of syntax that was clearly intended to achieve that purpose. Proposed text from the proponent was shown, which was to add a byte inside the view loop (a byte instead of a bit in order to preserve byte alignment). It was said that this had not yet been discussed in OMAF. For ordinary FPA, in some modes the VUI can be used to provide a default display window for 2D viewing. It may also be possible that a preference might be expressable by other means (e.g. the system level). It was noted that as proposed, a preference must be indicated. There would be no way to not indicate a preference. There is also some interaction with RWP if that also applies. The flag would only be useful in the frame packing case with a viewport message when a 2D adaptation preference would be desired. It was commented that this may be a rather obscure use case, and since the viewport message is not especially for the stereo case, putting special FPA-related syntax into the viewport message seemed undesirable. Hypothetically, another SEI message could be added later to express a left/right preference for FPA, perhaps not even coupled with viewport usage. JCT-VC response: Considering the above, it was agreed not to add the indicator. The viewport SEI message was agreed to be clarified to apply to both views when a frame packing arrangement SEI message indicates that stereoscopic views are present. Additional functionality to identify a preferred adaptation method for the viewing of stereoscopic content on a monoscopic display was considered outside the preferred scope of the omnidirectional viewport SEI message (although it could hypothetically be added later as an additional SEI message if that functionality is eventually later deemed necessary). |
| 14 | D.3.41.4 | The definition of OMAF's recommended viewport timed metadata differs from that of the omnidirectional viewport SEI message. In OMAF the intent is clear: The recommended viewport timed metadata track indicates the viewport that should be displayed when the user does not have control of the viewing orientation or has released control of the viewing orientation. The following is specified in JCTVC-AB1005-v1: The omnidirectional viewport SEI message specifies the coordinates of one or more regions of spherical-coordinate geometry, bounded by four great circles, corresponding to viewports recommended for display. It is not specified in which situation or use case the regions are recommended for display. | Align the definition of omnidirectional viewport SEI message with that of OMAF's recommended viewport timed metadata. In discussion, it was remarked that OMAF’s definition considers only the single-viewport case. The SEI message supports more than one viewport. What if there is more than one of these recommended viewport timed metadata track? JCT-VC response: Decision: Add a statement saying that the lower-numbered viewport is suggested to be displayed when the user does not have control of the viewing orientation or has released control of the viewing orientation. Also change “omni_viewport_id equal to 1 indicates that the recommended viewports are most-viewed viewports by statistical measurements” to something like “omni_viewport_id equal to 1 indicates that the recommended viewports are selected based on measurements of viewing statistics”. |
| 15 | D.3.41.4 | Collection of user statistics on viewing orientations for omnidirectional video typically results into a heat map sequence without clear distinct viewport boundaries. It is unclear - how a heat map sequence or such statistics are to be converted to the omnidirectional viewport SEI message; - whether the most-viewed viewport sequence would be the most suited form of representing such statistical data for target use cases. For example, would it be beneficial to allow excluding the width (in azimuth range) and height (in elevation range) of the viewport from the SEI message? If statistics have been collected that a viewport at orientation (0,0) has the highest popularity and a viewport at orientation (0,1) (in azimuth, elevation in units of units of 2−16 degrees) has the second highest popularity, which options does the content author have for generating the omnidirectional viewport SEI message? If statistics have been collected that a viewport at orientation (0,0) with 95° field of view has the highest popularity and a viewport at orientation (0,0) with 100° field of view has the second highest popularity, which options does the content author have for generating the omnidirectional viewport SEI message? | Please clarify. JCT-VC response: Decision: Say that this indicates the most popular among some selection of candidate viewports, where there is authoring discretion for the selection of candidate viewports. |
| 16 | D.3.41.4 | Are the viewports continuous in time, i.e., does the N-th viewport within an omnidirectional viewport SEI message in access unit A correspond to the N-th viewport within an omnidirectional viewport SEI message applicable to the next access unit in output order? In other words, when a player chooses to follow the N-th viewport in displaying, should it display the N-th viewport consistently? If yes, is there consistency across CVS boundaries too? It is specified that omni_viewport_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional viewport SEI message in output order. It could be desirable to have a cancelling mechanism that is specific to a particular viewport rather than all of them. | Please clarify. JCT-VC response: No action. Leave this to the discretion of the receiver; we don’t need to control the whole user experience, and this could be addressed in an application-specific way if further guidance is needed. |
| 17 | D.3.41.4 | How is the omnidirectional viewport orientation SEI message used if two or more viewports have an equal or unspecified priority relative to each other? | Please clarify. JCT-VC response: Decision: Clarify that the priority described is just for a suggestion that would not apply if no information is available about what to do, using some means not specified in the standard. |
| 18 | D.3.41.4 | The phrase director's cut is used in the normative semantics but not defined. Current language is “omni_viewport_id equal to 0 indicates that the recommended viewports are per director’s cut” | If the phrase "director's cut" is kept as part of normative text, it is proposed to clarify its meaning or avoid the use of the term. JCT-VC response: Decision: Use phrasing similar to “… are per ‘director’s cut’, i.e., a viewport suggested according to the creative intent of the content author or content provider”. |
| 19 | D.3.41.4 | It is not specified whether omni_viewport_azimuth_centre[ i ] and omni_viewport_elevation_centre[ i ] are relative to the local or global coordinate axes. | Specify omni_viewport_azimuth_centre[ i ] and omni_viewport_elevation_centre[ i ] to be relative to the global coordinate axes. JCT-VC response: Decision: Agreed. |

Other (1)

JCTVC-AC0036 Technical Report on Industry In-Use Combinations of Video Property Description Points [Y. Syed, C. Seeger, C. Fogg (??)] [late]

This contribution was discussed Saturday 21 October 0900 (chaired by GJS).

This document contains a proposed draft of a technical report on prominent video property description point combinations applied in industry. This input document is proposed to be used for a discussion on a planned Technical Report (ITU-T H.SuppX | ISO/IEC 23091-4) which would be a non-normative document of industry best practices on describing existing combinations of video properties, some of which are coding points, and their representations in different carriage systems. The combinations of properties and the permutations of all possible values being considered can reach hundreds of choices to describe a stream and some of these combinations are not expected to be used in practice (such as using the ITU-R BT.2100 perceptual quantization (PQ) transfer characteristics with ITU-R BT.601 colour primaries). Only a small subset of these combinations (such as BT.2100 colour primaries with BT.2100 PQ and hybrid log-gamma transfer characteristics) is used in practice. This document aims to avoid mistaken assumptions on video property combinations made by vendors of various content processing tools that may be part of a production equipment chain. It is suggested that with the increased usage of high-dynamic range and the increased use of look-up tables in television systems, these content processing mistakes could increasingly become magnified. Lastly, this proposed text aims to help its readers, especially toolset developers, avoiding unnecessary, complicated tailoring of their content processing tools to specific areas of the workflow, and thus to improve the repurposing of tools to different parts of the workflow.

References

- SMPTE 2067-21:2016 Interoperable Master Format - Application #2E (a mastering file container and metadata tagging format)

- Apple QuickTime Video Color Management in AV Foundation

- Apple ProRes SMPTE RDD Draft for MXF

- SMPTE ST 2086:2014: Mastering Display Color Volume Metadata Supporting High Luminance and Wide Color Gamut Images

- SMPTE 377-1:2011 Am1:2012 Am2:2012: Material Exchange Format (MXF)- File Format Specification

- Various vague references and undefined abbreviations RAW, TiFF-16, DPX, OpenEXR, MOV

Remarks:

- The text contains a number of vague undefined terms that would be better to just replace with concept descriptions, since that would be simpler than formally defining the jargon terms

- References to things that are not ITU/ISO/IEC standards should be reviewed to try to avoid such references as much as possible

- Avoid random capitalization – generally use lowercase

- Tables should be formatted as tables and editable, not pasted in as raster graphics, and colouring should be avoided

- It may be best to indicate code values rather than names (e.g., there are several different code values for the BT.709 transfer function)

- Include chroma sampling phase

- Oxford spelling fixes

- Regarding “domain carriage formats”, is it really necessary? How much value is there?

- Consider adding “production” to the title

- “nits” term is not defined and is informal – just use cd/m2

- One of the coauthors questioned the combination “709-Gamma_2.4, D65, 1000nit”

- Reference H.273 | ISO/IEC 23091-2 (ex ISO/IEC 23001-08)

- For MDCV, discuss/clarify the relationship to the SEI message

- Table 5 contains full-range RGB, which seems questionable – change to narrow-range

- Clarify or remove mentions of CL/NCL (BT.2020 has CL, so may need to mention NCL to )

- Remove ICtCp (for now)

Further work in an AHG was requested to produce an improved draft by the next meeting.

JCTVC-AC0039 Improved figures for chroma 4:2:0 location type [C. Fogg (MovieLabs)] [late]

Discussed Saturday 21 October 1000 (chaired by GJS).

As first proposed in the previous meeting input JCTVC-AB0027, additional drawings to complement Figure E-1 (location of chroma samples) in the HEVC specification (and the analogous VUI section in AVC) should provide cleaner, easier to understand illustrations for some readers.  In several products, chroma 4:2:0 subsample location associated with BT.709 (chroma_sample_loc_type_top_field and chroma_sample_loc_type_bottom_field = 0) has been used instead of the (chroma_sample_loc_type’ = 2) sub-sample location specified in BT.2020 and BT.2100, resulting in visible artifacts for some HDR/WCG content patterns. It is proposed that figures, tables, and formula provided in this input be added to the current draft amendment of HEVC in order to help readers better understand the difference between the various chroma sub-sample locations.

It is proposed to add this information to the text (not remove current content).

(The wrong division symbol is used – fix that.)

Decision: Adopted (just informative).

Core experiments (0)

No CEs were run during the preceding meeting cycle.

Technical contributions (14)

SEI messages and VUI (14)

Motion constrained tile sets extraction SEI messages (2)

JCTVC-AC0033 MCTS extraction with implicit slice reordering [H.-M. Oh, S. Hwang, S. Oh (LGE)]

This contribution was discussed Friday 20 October 1100 (chaired by GJS).

With regard to the motion-constrained tile set (MCTS) extraction information set SEI message described in JCTVC-AB1005, this contribution proposes to replace the explicit slice reordering syntax with an implicit slice reordering approach. Instead of signalling the slice segment address of all slice segments included in the output picture, the ordering of MCTS indexes signalled in the SEI message and given parameters are used to induce the slice segment address of each MCTS in the output bitstream.

At the previous meeting, a similar proposal AB0037 was submitted for implicit signalling and a different proposal AB0028 suggested explicit signalling. AB0037 was revised and clarified late in the meeting. (The original version of the AB0037 proposal did not provide specification text for the inference rule.) But a couple of problems were found in that approach. It seemed clear that the explicit signalling approach was easier to specify clearly and that the proposed alternative was not sufficiently mature in accomplishing the specification of its inference rule, and therefore AB0028 was selected.

Part of the motivation of the proposal is to save the need for signalling in a use case with extraction of areas that are adjacent in a 360° projection mapping case that are not adjacent in the coded rectangular picture (esp. due to right-left wrap-around effects).

In this implicit approach, horizontal and vertical offset values are calculated from the position of the first MCTS in the set and used in the derivation of the slice addresses for the extracted set. Wrap-around is computed from relative spatial positions. It was remarked that this assumes a particular geometry that is trying to be supported – basically modelling that the left edge of the image is wrapped around from the right edge of the image – similar to complete ERP and cylinder projections. It also models the top of the image as being wrapped from below the bottom of the image. It was remarked that this is not a completely general model, as the other approach can do arbitrary reorderings rather than assuming these spatial relationships.

In the explicit approach, the order of the SEI message syntax data is in the input bitstream order.

This proposal does not actually suggest removing the scheme that is there now; it proposes another mode of operation in which it is not necessary to send the explicit syntax.

The proposal suggests to change how the mapping process works when the explicit data is not provided. Some participants remarked that the way that case currently works is intended as a simple case in which no reordering of slices is needed that would no longer be as easily detectable if this is done. If there is a desire to preserve detection of the simple case, there would need to be another mode indicated. The primary usage is expected to be the simple case.

The case was discussed where a single MCTS contains, for example, a tile on left side of the picture and a tile on the right side of the picture. In the explicit scheme, reordering these tiles could be supported, but not with this implicit scheme.

The main purpose of this seemed to be to save some bits used for signalling slice addresses in the more complex rewriting case (which is not expected to be the primary usage). The amount of potential bit savings from this was not estimated. It would depend on the number of slices used in the pictures. It was not clear that this bit savings is really needed for the less prevalent use case, especially as we trying to finalize the specification at the current meeting, so no action was taken.

JCTVC-AC0038 Temporal MCTS Coding Constraints Implementation [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI), R. Eun-Seok, J. Son (Gachon University)] [late]

This contribution was discussed Friday 20 October 1230 (chaired by GJS).

This contribution presents a software implementation of encoding constraints required for temporal motion-constrained tile sets SEI messages with mc_all_tiles_exact_sample_value_match_flag equal to 1, as well as decoder-side checks to verify that a decoded bitstream follows these constraints. The modifications are implemented based on the HM 16.16 reference software.

This is offered to become part of the HM codebase. This provides MCTS constrained encoding, which is a functionality we have been wanting for some time. Decision: Adopt into HM (pending review by Karl Sharman for maturity check).

Projection mapping, including coverage, padding and region-wise packing (7)

JCTVC-AC0023 On CMP padding and region-wise packing [Y.-K. Wang (Qualcomm), M. Zhou (Broadcom)]

This contribution was discussed Thursday 19 October 2017 1245 (chaired by GJS).

This is closely related to AC0030 for padding.

This contribution proposes to align the semantics of the region-wise packing (RWP) SEI message with the RWP signalling in the Omnidirectional MediA Format (OMAF) that is being developed by the MPEG Systems subgroup, and to fix asserted issues associated with the padding signalling in the cubemap projection (CMP) SEI message.

The proposal includes the following five parts:

- Support of an optional CMP padding along the four picture boundaries and in the middle of the picture between the boundaries of the upper three cubemap faces and the lower three cubemap faces (with the padding area being twice the height of that at the top/bottom/left/right sides of the picture). When the optional CMP padding exists, the RWP signalling shall not be present.

  - Padding was already in the text; the proposal limits the padding size to be 2N+2 in luma samples. No action on this aspect (pending review of AC0030).

  - The proposal removes the padding type indication. In OMAF, guard band 0 means unspecified, 1 means it is just for sub-sample position interpolation, 2 is actual content in spherically adjacent area, 3 is similar to 2. In our CMP draft, 0 is unspecified, 1 is edge extrapolation, 2 is samples from the adjacent face, 3 is extending the face. These definitions seem reasonably aligned (but might benefit from clarification). No action on this aspect (other than editorial improvement, pending review of AC0030).

  - It is proposed to prohibit RWP presence when CMP padding is used. This is related to contribution JCTVC-AC0030. This aspect was discussed further Monday 23 October 1540, and it was noted that the padding indication within the CMP SEI message was removed.

  - Editor action item: Editorial clarification of the sizes and positions of the padding areas and the padding type interpretation is needed (as per above). This aspect was discussed further Monday 23 October 1540, and it was noted that the issue may be obsolete.

- Text changes to the sample location remapping process for addressing the cases where the optional CMP padding exists.

  - Decision: Agreed, pending review of AC0030, with editorial improvement (e.g. potentially improve variable names). Also add a recommendation that the aspect ratio VUI should indicate 1:1 or unspecified. Note that we need to fix the non-unicode Greek symbols in the text (and minus signs and primes).

- Added a constraint that each CMP face shall be a square.

  - Decision: Agreed.

- Alignment of the syntax and semantics of the RWP SEI message with the RWP signalling in OMAF, including addition of guard band signalling, specifying the sizes of projected/packed pictures and regions in relative units (signalling of the width and height of the packed picture is part of this), and refinement of constraints of syntax element values (allowing overlapping packed regions is part of this).

  - Decision: This includes additional syntax elements:

    - constituent_picture_matching_flag, for use with frame packing

    - packed_picture_width and packed_picture_height

    - “guard band” type signalling

- Some minor bug fixes related to the presence of the RWP SEI message. Decision: Agreed.

Marked text changes relative to JCTVC-AB1005-v1 are provided in this document (JCTVC-AC0023). The corresponding text changes to the draft OMAF specification for alignment are provided in MPEG input document m41459.

It is claimed that the proposal resolves existing issues of padding for CMP, and makes the designs of RWP and guard band padding aligned between OMAF-related SEI messages and OMAF.

It is further claimed that the proposal resolves issues #6 and #7 in JCTVC-AC0021, and fixes some minor bug fixes related to the presence of the RWP SEI message.

JCTVC-AC0030 On padding signalling in omnidirectional video [R. Skupin, Y. Sanchez (HHI)]

This contribution was discussed Thursday 19 October 2017 1545 (chaired by GJS).

This is closely related to AC0023 for padding. This contribution is about both ERP and CMP.

This contribution reports asserted issues of the current padding (aka guard band) signalling for omnidirectional video in JCTVC-AB1005 and proposes the following:

- Remove padding signalling from the equirectangular projection SEI message and the cubemap projection SEI message. Using a “guard band” would require the presence of a RWP SEI message.

- Add a constrained padding flag to the equirectangular projection SEI message and the cubemap Projection SEI message that indicates that either none or a restricted from of padding is used.

- Introduce padding (aka guard band) signalling to the region-wise packing SEI message following document JCTVC-AC0023-v4.

An error was identified in the constraint expressions for the CMP case, in regard to accounting for the padding regions in the expression of constraints.

The apparent advantage of this approach is to have only one way to signal a “guard band”, rather than having this supported (in some form) within the projection-specific messages and also in the RWP message. The disadvantages would be 1) somewhat more syntax overhead, 2) it becomes more difficult to explain what is supported in a product, since the simple case becomes a constrained general combination rather than a binary decision of whether to support a particular SEI message type or not.

It was remarked that if we choose this approach we should also remove the partial coverage case of ERP without RWP, since that can alternatively be expressed as a combination of ERP with the same constrained form of RWP.

It was remarked that with this proposal, if partial coverage is removed from the ERP message, there wouldn’t be hardly anything left in the ERP message and the CMP message (except rotation, which is common between them).

It was remarked that the proposed flag may not really be necessary, as it is just an indication of a constraint case that can be readily detected without the flag (and the constraint could be imposed at the application level rather than in our syntax and semantics).

Decision:

- ERP SEI message (only persistence info in it)

- CMP SEI message (only persistence info in it)

- Omnidirectional projection rotation message

- RWP message (with guard band signalling, includes 2D coverage subset capability)

- Maybe/maybe-not: Sphere coverage message (pure metadata, no tilt) – tentatively not (see confirmation of this below).

YKW was asked to prepare text for review to reflect the first four items above. See AC0041.

This was further discussed Saturday 21 October 1400 (chaired by GJS and YKW)

For the last item above, it was confirmed not to create a sphere coverage message.

Decision: For the region-wise packing message, since the projected picture width and height and region width and height and horizontal and vertical offsets were 16 bits, which provides lower precision than the 32-bit angular representations, increase those to be 32 bits.

JCTVC-AC0041 Text changes for decisions on omnidirectional video SEI messages [Y.-K. Wang (Qualcomm)]

This topic was discussed Friday 20 October 1500 (chaired by GJS).

This contribution provides specification text changes, marked relative to JCTVC-AB1005-v1, for decisions recorded during the meeting on omnidirectional video SEI messages.

It was commented that it would probably be best to have the rotation message before the RWP message.

The contributor put in a sign change for azimuth. This was agreed to be an error. ϕ is the azimuth (longitude, increasing eastward). The perspective is from the center, so New York (which has a higher value of ϕ) is to the left of California on an ERP projection. Therefore, as we move from left to right (increasing index order) in the picture, ϕ needs to decrease.

Minor issue: Syntax element names using “picture” are ordinarily abbreviated to “pic” in our syntax.

It was commented that the simple degenerate cases of ERP (with partial coverage and/or padding) and CMP (with padding), when using this syntax, involve complex syntax and difficult-to-understand semantics in the RWP SEI message, with roughly 300 bits of such syntax.

A participant indicated that the padding case for ERP is more well understood than the padding case for CMP, and suggested only having a “short cut” mode for ERP.

It was further commented that the full coverage case for ERP (with guard band) is the most important one to support with a simple “shortcut”.

It was suggested for the shortcut case to be in the ERP SEI message (but aligned with the scheme used in ERP).

This was further discussed Saturday 21 October 1430 (chaired by GJS and YKW).

Syntax and semantics for a shortcut expressed in the ERP SEI message syntax was reviewed. The padding width and height in the shortcut are required to be even when the chroma is at half resolution in the corresponding dimension.

The region-wise packing type indicator was removed, since it is not used.

The drafted scheme was refined in the discussion.

JCTVC-AC0024 On ERP equations for sample location remapping and sphere coverage signalling [Y.-K. Wang (Qualcomm)]

This is closely related to AC0032 for coverage. The contributor said this did not need separate presentation.

This contribution proposes to align the sample location remapping process for the equirectangular projection (ERP) and signalling of sphere coverage between the Omnidirectional MediA Format (OMAF) being developed by the MPEG Systems subgroup and the OMAF-related SEI messages being developed by JCT-VC.

The proposal includes the following two parts to the OMAF-related SEI messages:

- Use a separate SEI message for signalling of sphere coverage information such that the same syntax could be used for any type of projection, and signal the sphere coverage information by specifying the center point of the coverage sphere region, the azimuth and elevation ranges, and the tilt angle of the sphere region. Consequently, remove the sphere coverage parameters from the ERP SEI message.

- For ERP, if an applicable region-wise packing (RWP) SEI message is not present, the projection equations that involve the sphere coverage parameters apply; otherwise, the projection equations that do not involve the sphere coverage parameters apply. For both cases, the azimuth equation is changed a little bit such that the left side of a full coverage ERP picture corresponds to −180 degree instead of 180 degrees.

Text changes relative to JCTVC-AB1005-v1 are provided in this document (JCTVC-AC0024). The corresponding text changes to the draft OMAF specification for alignment are provided in the MPEG input document m41459.

It was claimed that the proposal and the corresponding OMAF text changes in the MPEG input document m41459 resolve issues #3, #4, #9, and #11 listed in JCTVC-AC0021.

JCTVC-AC0032 On coverage signalling for omnidirectional video [R. Skupin, Y. Sanchez (HHI)]

This is closely related to AC0024 for coverage. The contributor said this did not need separate presentation.

This document reports asserted issues in the coverage signalling of JCTVC-AB1005 and proposed an addition to a projection format independent coverage signalling in fashion similar to OMAF as proposed in JCTVC-AC0024.

It also proposed to add a sphere_exact_match_flag to indicate the presents of samples outside the indicated sphere coverage within the decoded picture.

JCTVC-AC0028 On the relation of frame packing arrangement and region-wise packing SEI messages [M. M. Hannuksela (Nokia)]

This topic was discussed Friday 20 October 1700 (chaired by GJS).

It is proposed to include the frame packing arrangement scheme signalling within the region-wise packing SEI message rather than having it in an accompanying FPA SEI message. The FPA and RWP SEI messages are proposed to be mutually exclusive: when an FPA SEI message applies to a picture, no RWP SEI message would be allowed to apply to the same picture.

It is asserted that in the proposed design the interpretation of the FPA SEI message would not depend on the presence of the RWP SEI message or vice versa.

A similar proposal has been made for OMAF boxes in MPEG M41460.

It was commented that service providers that use FPA currently are likely to be able to control their system environment sufficiently that such decoders would be unlikely to unexpectedly encounter 360° projection mapped content.

It was commented that it is more reasonable to try to make a new SEI message compatible with the prior FPA than to add new FPA indicators. The fact that this is the last meeting at which the projection format SEI messages are finalized also makes it appropriate to be conservative about design changes. No support was expressed.

We do need to check for errors in the current specification of how 360° video works with FPA and check/correct as necessary. One such issue is identified in the contribution. We also need to ensure that appropriate constraints are expressed. We also need to check on cropped vs. decoded picture for FPA semantics. Decision: If an applicable region-wise packing SEI message exists, the packing arrangement scheme applies to the projected picture, otherwise, the packing arrangement scheme applies to the cropped decoded picture. Clarify that as necessary (which may involve clarifying the frame packing arrangement SEI message semantics.).

JCTVC-AC0031 On static regions for region-wise packing [R. Skupin, Y. Sanchez (HHI)]

This topic was discussed Friday 20 October 1725 (chaired by GJS).

This contribution reports asserted issues for the current region-wise packing SEI message design in JCTVC-AB1005 which affect viewport-dependent streaming schemes. It is asserted that the current design prohibits usage of prediction structures when streams in a stream-switching scenario share part of the packed picture.

Switching without an I frame is envisioned, either with part of the picture using an I slice or with switching drift.

Changes are proposed to the existing region-wise packing SEI message to define two regions types within the region-wise packing SEI message: namely static (remaining static for whole CLVS) and dynamic (that can change at any picture).

A participant suggested that this sort of issue should be handled at the system level rather than having this signalling inside the bitstream. Whether included in the SEI message or not, the envisioned use case would need support at the systems level. It was suggested that this should be considered first in the OMAF group.

This was further discussed Saturday 21 October 1515 (chaired by GJS & YKW).

No action on this unless requested to be supported by systems group.

JCTVC-AC0037 On Region-Wise Packing [S. Deshpande (Sharp)] [late]

This contribution was discussed Friday 20 October 1800 (chaired by GJS).

In this document modifications are proposed to region-wise packing (RWP) SEI Message and sample location mapping. The proposed modifications provide alignment between the region-wise packing SEI messages and the corresponding region-wise packing signalling in OMAF SoDIS.

The following is proposed:

- Signalling of a constituent pictures matching regions flag in RWP SEI

- Updates to constraints related to RWP SEI syntax elements

- Signalling of packed picture width and height in the RWP SEI

- Changes to the text for mapping of luma sample locations within a decoded picture to sphere coordinates relative to the global coordinate axes

Some aspects were covered already in review of other contributions (the first three points in the abstract).

The final point is text corrections to correct the expression of the existing design intent – basically editorial input.

Decision (BF): Adopted. (Packing type 5 should also be accounted for properly.)

Other 360° video SEI (2)

JCTVC-AC0035 Preferred rendering operation for a recommended viewport [Y.-K. Wang (Qualcomm)]

This was briefly discussed Saturday 21 October 1520 (chaired by GJS).

Detailed presentation was not requested. No action was planned on this unless requested to be supported by systems group.

This contribution proposes to signal a preferred rendering operation for a recommended viewport in the omnidirectional viewport SEI message. The signalling indicates the rendering operation that is considered as preferred by the content creator to keep the aspect ratio of the viewport region and minimize the rendered parts that are not covered by the viewport after a possible spatial resolution scaling operation.

The MPEG input document m41462 proposes the same signalling to the recommended viewport signalling in the Omnidirectional MediA Format (OMAF) that is being developed by the MPEG Systems subgroup.

It is suggested that this aspect should be aligned between the omnidirectional viewport SEI message and the recommended viewport signalling in OMAF.

JCTVC-AC0022 Sphere rotation SEI message [Y.-K. Wang (Qualcomm)]

The contributor said this did not need separate presentation.

This contribution proposes a new SEI message, named the sphere rotation SEI message, for signalling of rotation parameters in a separate SEI message from the equirectangular projection (ERP) SEI message. It is claimed that, this way, the rotation signalling can be applied to any projection type, and the design is more aligned with the Omnidirectional MediA Format (OMAF) that is being developed by the MPEG Systems subgroup. It is further claimed that the proposal resolves issue#2 in JCTVC-AC0021.

JCTVC-AC0034 Omnidirectional fisheye video SEI message [H.-M. Oh, S. Oh (LGE)]

This was briefly discussed Saturday 21 October 1525 (chaired by GJS & YKW).

Detailed presentation was not requested. No action was planned on this unless requested to be supported by the MPEG systems group, which did not occur.

This contribution proposes to define an omnidirectional fisheye video SEI message that would carry parameters used by the receiver to reconstruct an omnidirectional spherical video from circular fisheye camera outputs contained in a decoded picture. With the proposed SEI message, the active image areas captured by the fisheye lens are indicated by the intersection of rectangular and circular regions. In aid of the field of view of the lens and the spherical coordinate matching information, the active areas in each circular fisheye image could be mapped onto the 3D sphere by a post-decoder rendering process.

Other SEI messages and high-level syntax (3)

JCTVC-AC0025 SEI manifest and prefix indication SEI messages [Y.-K. Wang, T. Stockhammer (Qualcomm), D. Singer, A. M. Tourapis (Apple)]

This was briefly discussed Saturday 21 October 1540 (chaired by GJS).

SEI messages for carrying manifest information and prefix indications for SEI messages are proposed. The manifest information includes an indication of whether certain types of SEI messages may be assumed to be present or absent, and an assertion of their essentiality. A prefix indication for a particular type of SEI messages would provide a bit string that some or all SEI messages of the particular type may be assumed to start with.

An SEI message is proposed to be considered as essential by the encoder (i.e., the content producer) when the carried information is considered as essential from the decoder’s side to properly process the content and enable a desirable user experience. It is at the discretion of the encoder (i.e., the content producer) to determine which SEI messages are considered as essential in a particular bitstream. However, some SEI messages, such as the frame packing arrangement, segmented rectangular frame packing arrangement, display orientation, and omnidirectional projection indication SEI messages, would be expected to typically be considered essential.

In addition to proposing this for HEVC, the authors advocate that the two proposed SEI messages should also be considered for inclusion into AVC and future video coding standards.

This contribution is a follow up of JCTVC-AB0025 titled "SEI manifest and prefix indication SEI messages". In this contribution (JCTVC-AC0025), the proposed syntax and semantics are the same as in JCTVC-AB0025, with the difference being that in JCTVC-AC0025 the authors tried to address comments received when reviewing JCTVC-AB0025 at the previous JCT-VC meeting in Torino. How the comments have been addressed is discussed in Section 4 of JVTVC-AC0025 (the current document).

The same content as in this document (JVTVC-AC0025) was also submitted, in MPEG document m41457, to the MPEG Systems subgroup for review, trying to get an opinion from the Systems subgroup regarding the usefulness of the proposed SEI messages from systems point of view, as well as on systems-level exposure of the information carried in these SEI messages. This is discussed in Section 6 of the JVTVC-AC0025 contribution.

No action was planned on this unless requested to be supported by systems group.

On Monday 23 October, it was reported that the MPEG Systems group had considered this useful and recommended its adoption. Further study was encouraged in view of this new information.

JCTVC-AC0027 Centralized Texture Depth Packing SEI Message for HEVC and AVC [J.-F. Yang, G.-C. Chen]

This contribution was discussed Friday 20 October 0900 (chaired by GJS).

This document is a revision of output document JCTVC-AA1008. The purpose of the proposed “Centralized Texture Depth Packing SEI message for HEVC and AVC” is to deliver one texture view plus one depth map packed into the samples of a monoscopic (texture only) stream for HEVC and AVC. In this document, “large_depth_type” flag is removed for simplicity and “coulor_packing_type” flag is added to support some existed broadcasting systems that YCbCr components are not accessible. Accordingly, a new adjusted RGB method for colour depth packing is proposed and evaluated. Figures and texts are revised for better presentations.

It proposes to support two “packing types”, as previously in AA1008: top/bottom and left/right.

Relative to AA1008 (an April 2017 Hobart meeting output as a “conceptual study”), this proposal removes the “large_depth_type” flag, as a simplification to reduce the number of proposed modes of operation.

However, it adds a proposal of a “colour_packing_type”.

Some depth information is put into colour planes and some is put into the luma planes. The texture area is coded normally, except for a resampling process described further below.

For the depth, there is also a DPCM prediction/residual formatting and a companding of residual values.

Some quincunx sampling structure is used.

A new technique involving deleting some lines of depth and replacing them with texture, called DDTR (deletion of depth and texture reserved), is proposed. This was in AA1008 but was not it AA0025 (perhaps erroneously at the time). It concerned the case when the picture height H is not divisible by M (36 in the new proposal for top/bottom and 48 for left-right). In this case, the depacked texture is further divided in three portions: top reserved texture (TRT), center texture (TC), and bottom reserved texture (TRB) from top to bottom. The center texture portion is downsampled and then the picture is reassembled so that the texture is a little less tall. The idea of this is to avoid needing an unusual resampling ratio for the texture. Some depth information is removed in this process (which is handled using extrapolation after decoding). This was reported to have been motivated by technology testing experiments. It was commented that the rationale for this seemed like it might be somewhat motivated by specific picture sizes.

A new technique involving RGB and YUV transformation and “RGB adjustment” is also used. This has something to do with the proposed “colour packing type”.

Artefacts around depth map edges had been a problem in the previous version and was said to be reduced by the modifications.

Part of the new aspects of the proposal had to do with a lack of direct access to decoded output in the YUV domain – only having access to data in the RGB domain.

Some of the described signal processing was not fully understood by the group.

Testing with cable broadcasting in the “HYA” cable network has started for AR/VR game video (not yet using STB), and there is a plan for terrestrial broadcast testing. Having real services deployed in 3 years is the stated goal.

The proponent offered to provide reference software to clarify the described signal processing pre-processing and post-processing, and extra explanatory information.

The group agreed to accept such software and additional explanatory material for further study and cross-verification testing of the technology – issuing an output “Software and explanatory material for study of CDTP technology”. The software will be offered with our usual copyright license header. Cross-verification by other organizations is requested.

Non-normative: encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (0)

Plenary discussions, joint meetings, BoG reports, and summary of actions taken

General

Topics for general discussion at the plenary level: (Update)

- …

Project development (update)

Joint meetings are discussed in this section. Additional notes on the same topics may appear elsewhere in this report. Joint discussions were held on XXXX, as recorded below.

Topics needing joint discussion:

- …

BoGs

None.

Project planning

Text drafting and software quality

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text. Similarly, software coordinators have the discretion to evaluate contributed software for suitability in regard to proper code style, bugginess, etc., and to not integrate code that is determined inadequate in software quality.

Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the XXday of the week preceding the meeting (xx Jan 2018).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

General issues for CEs and TEs

Group coordinated experiments have been planned in previous work, although none were established at the current meeting. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology. Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable basic description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

The general agreed common conditions for single-layer coding efficiency experiments remained as described in the prior output document JCTVC-X1100.

The general timeline agreed for CEs was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

When a CE is planned, a deadline of four weeks after the meeting would be established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on CE work on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document would be reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE would be described in an associated output document numbered as, for example, JCTVC-X11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans would be recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities have been established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

Common test conditions for HEVC Coding Experiments

Update No particular changes were noted w.r.t. the prior CTC for work within the current scope of JCT-VC. See the prior output documents JCTVC-X1100 for HEVC test conditions, JCTVC-X1009 for SHVC test conditions, JCTVC-X1015 for SCC test conditions., and JCTVC-X1020 for HDR/WCG test conditions.

Software development planning (update)

Software coordinators were asked to work out the detailed schedule for software updates with the proponents of adopted changes as applicable.

Any adopted proposals where necessary software is not delivered by the scheduled date in a timely manner may be rejected.

The planned timeline for software releases was established as follows:

- HM 16.6 available prior to the meeting.

- SCM 5.0 (based on HM 16.6 or newer) should be available within 3 weeks after the meeting.

- SHM 10.x U1013 (DAM, based on HM 16.2 or newer) should be available within 5 weeks after the meeting.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable. Further study was encouraged to determine the appropriate approach to future software maintenance, especially in regard to alignment of 3D video software with the SHM software.

Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and develop proposed improvements to the JCTVC-AB1002 HEVC Test Model 16 (HM 16) Update 9 of Encoder Description [Remove - No such output] - Collect reports of errata for the HEVC specification and the HDR technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM, SCM, SHM and HDRTools software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM, SCM, SHM, and HDRTools software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver results reporting templates and anchor test results according to JCT-VC common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), B. Li, K. Sharman, V. Seregin, A. Tourapis, (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Produce and develop proposed improvements to the conformance testing draft JCTVC-AC1016 for SCC and non-intra HT profiles. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt, SHVC, and SCC conformance testing. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), R. Joshi, Y. Ye, J. Xu (vice-chairs) | N |
| Test sequence material (AHG5) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material, especially focusing on new needs for HDR/WCG test material and corresponding SDR test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG6 regarding HDR/WCG testing. | T. Suzuki, V. Baroncini (co-chairs), E. François, P. Topiwala, S. Wenger, H. Yu (vice-chairs) | N |
| Report development for usage of video signal type code points (AHG6) (jct-vc@lists.rwth-aachen.de) - Study the industry usage of video signal type code points and identify the most common and important combinations of such code points (including study of the draft text ACxxxx). - Produce proposed improvements of the draft text for the planned new technical report on the subject. | Y. Syed and C. Fogg (co-chairs) | N |
| Supplemental enhancement information (AHG7) (jct-vc@lists.rwth-aachen.de) - Produce the draft text JCTVC-AC1005. - Consider proposals for additional SEI message data and associated syntax and semantics specification. - Study JCTVC-AC0025. [refine wording] - Develop usage scenario descriptions and showcase demonstrations. | J. Boyce (chair), A. K. Ramasubramonian, R. Skupin, G. J. Sullivan, A. Tourapis (vice-chairs) | N |
| No AHG8 (AHG8) (jct-vc@lists.rwth-aachen.de) - X - Y - Z | XX (chair), XXX (vice-chairs) | N |

Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. (Update links)

New item in work programme for Q6/16.

JCTVC-AC1000 Meeting Report of the 29th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2018-01-12] (near next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

Remains valid – not updated:

JCTVC-AB1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9 [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 17047) [2017-10-04] (near next meeting)

JCTVC-AC1003 Usage of video signal type code points (Draft 1) [L. Borg, C. Fogg, C. Seeger, G. J. Sullivan, Y. Syed, A. Tourapis (editors)] (WG 11 FDAM1 N 17044) [2017-12-08] (6 weeks)

WG 11 should request to add registered editors Sullivan and Tourapis (Registered editors: Yasser Syed, Chris Seeger, Chad Fogg, Lars Borg).

Later consider changing title – e.g., to “Usage of colour-related code points and description data for video content production” (Registered title: “Usage of video signal type code points”)

Remains valid – not updated:

JCTVC-AB1004 HEVC Monochrome 10 Profile (Draft 1) [A. Tourapis (editor)] (WG 11 working draft N 17052) [2017-08-15] (4 weeks)

Consent in ITU; may include in CD or issue PDAM in WG 11.

JCTVC-AC1005 HEVC Additional Supplemental Enhancement Information (Draft 4) [J. Boyce, A. K. Ramasubramonian, R. Skupin, G. J. Sullivan, A. Tourapis, Y.-K. Wang] (WG 11 xxxx N 17045) [2017-12-08] (6 weeks)

Clarify whether FDAM or CD in WG 11.

A corresponding Disposition of Comments Report N 17201 was also issued by WG 11 (and was discussed Monday 23 October 1515, chaired by GJS).

Contacts identified for specific sub-topics were as follows:

- Content colour volume: A. Tourapis, H. M. Oh, A. K. Ramasubramonian, P. Yin (editors)

- Motion-constrained tile sets extraction information (2 messages): R. Skupin

- Equirectangular and cube map projection [CMP is new AB0044]: J. Boyce, A. Tourapis, C. Fogg, R. Skupin G. J. Sullivan, Y.-K. Wang

- Omnidirectional recommended viewport: J. Boyce, Y.-K. Wang

- Region nesting: A. K. Ramasubramonian, E. François

- Region-wise packing [new]: Y-K. Wang (AB0026)

- Figures for chroma relative positioning: C. Fogg (AC0039)

Note padding should be constrained as an integer multiple of chroma samples (can move this note elsewhere).

Software work is needed, esp. for the new packing and projection (e.g. in 360Lib and HM).

Remains valid – prior meeting output (with additional editing): JCTVC-AA1006 Verification test report for HEVC screen content coding extensions [V. Baroncini, H. Yu, R. Joshi, S. Liu, X. Xiu, J. Xu (editors)] (WG 11 N 16882) [2016-05-19]

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778)

Remains valid – not updated: JCTVC-AC1008 Software and Explanatory Material for Study of Centralized Texture Depth Packing Technology [J.-F. Yang, G. J. Sullivan (editors)] [2018-01-12] (near next meeting)

Remains valid – not updated: JCTVC-X1009 Common Test Conditions for SHVC [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

Remains valid – not updated:

JCTVC-AB1011 Defect report for HEVC text specification (Draft 1) [G. J. Sullivan (editor)] (WG 11 N 17046) [2017-08-25] (5 weeks)

No output: JCTVC-Z1012

No output: JCTVC-Z1013

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 16049)

Remains valid – not updated: JCTVC-Z1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2017-02-17]

JCTVC-AC1016 Conformance Testing for HEVC Screen Content Coding (SCC) Extensions and Non-Intra High Throughput Profiles (Draft 7) [R. Joshi, T. Suzuki, A. Tourapis, J. Xu (editors)] (WG 11 WD 7 N 1xxxx) [2018-01-12] (near next meeting)

WG 11 should add T. Suzuki to list of editor if not already listed. K. Rapaka requested removal.

Issue a request for amd? Issue a PDAM? The bitstreams have been collected (although problems have been identified in a few of them). The basis is pending FDIS ballot closure. Check with secretary.

Remains valid – not updated: JCTVC-Z1017 Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics (Draft 4) [J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. J. Sullivan, P. Topiwala, A. Tourapis (editors)] (WG 11 TR N 16692) [2017-01-25]

No output: JCTVC-Z1018

No output: JCTVC-Z1019

Remains valid – not updated: JCTVC-Z1020 Common Test Conditions for HDR/WCG video coding experiments [E. François, J. Sole, J. Ström, P. Yin (editors)] [2017-02-17] (1 month)

JCTVC-AC1100 Common Test Conditions for HM video coding experiments [K. Sharman, K. Sühring (editors)] [2017-11-30] (1 month)

Test sequence changes to align with JVET CTC (see AHG3 report).

Also update the spreadsheet template (using the modified BD computation method, 2 digits past the decimal point, with ordinary Excel rounding, since that is what is used in JVET).

Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (usually starting meetings on the Friday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 5–5.5 meeting days, although different next time due to unusual WG 11 meeting date alignment), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 6.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Sat. 20 – Fri. 26 Jan. 2018, 30th meeting under WG 11 auspices in Gwangju, KR.

- Sat. 14 – Fri. 20 Apr. 2018, 31st meeting under WG 11 auspices in San Diego, US.

- Fri. 13 – Wed. 18 July 2018, 32nd meeting under ITU-T auspices in Ljubljana, SI.

The agreed document deadline for the 30th JCT-VC meeting is Thursday 11 January 2018. Plans for scheduling of agenda items within that meeting remained TBA.

The WG 11 parent body, the local host XXX, and the meeting arranger YYYY were thanked for the excellent hosting of the 29th meeting of the JCT-VC.

The JCT-VC meeting was closed at approximately 1000 hours on Tuesday, 24 October 2017.

- Annex A to JCT-VC report:
List of documents

| JCT-VC number | MPEG number | Created | First Upload | Last upload | Title | Source |
| --- | --- | --- | --- | --- | --- | --- |
| JCTVC-AC0001 | m41788 | 2017-10-17 11:43:44 | 2017-10-19 00:18:13 | 2017-10-19 00:18:13 | JCT-VC AHG report: Project management (AHG1) | G. J. Sullivan, J.-R. Ohm |
| JCTVC-AC0002 | m41774 | 2017-10-17 00:29:41 | 2017-10-18 03:51:11 | 2017-10-19 04:24:20 | JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) | B. Bross, C. Rosewarne, M. Naccari, J.-R. Ohm, K. Sharman, G. Sullivan, Y.-K. Wang |
| JCTVC-AC0003 | m41831 | 2017-10-18 19:14:06 | 2017-10-18 19:14:39 | 2017-10-18 19:14:39 | JCT-VC AHG report: HEVC HM, SCM, SHM and HDRTools software development and software technical evaluation (AHG3) | K. Suehring, B. Li, K. Sharman, V. Seregin, A. Tourapis |
| JCTVC-AC0004 | m41680 | 2017-10-16 11:02:53 | 2017-10-17 11:14:08 | 2017-10-17 11:14:08 | JCTVC AHG report: HEVC conformance test development (AHG4) | T. Suzuki, R. Joshi, Y. Ye, J. Xu |
| JCTVC-AC0005 | m41684 | 2017-10-16 11:27:37 | 2017-10-17 11:14:27 | 2017-10-17 11:14:27 | JCT-VC AHG report: Test sequence material (AHG5) | T. Suzuki, V. Baroncini, E. Francois, P. Topiwala, S. Wenger, H. Yu |
| JCTVC-AC0006 | m41870 | 2017-10-20 14:10:57 | 2017-10-20 14:11:36 | 2017-10-20 14:11:36 | JCT-VC AHG Report: Development for usage of video signal types code points (AHG6) | Yasser Syed, Chad Fogg |
| JCTVC-AC0007 | m41775 | 2017-10-17 01:00:45 | 2017-10-18 11:59:51 | 2017-10-18 11:59:51 | JCT-VC AHG Report: Supplemental enhancement information (AHG7) | J. Boyce, A. K. Ramasubramonian, G. J. Sullivan, R. Skupin, A. Tourapis |
| JCTVC-AC0008 | m41832 | 2017-10-19 01:41:57 | 2017-10-19 01:42:58 | 2017-10-19 01:42:58 | JCT-VC AHG report: Report development for HDR/WCG signalling, backward compatibility, and display adaptation (AHG8) | E. François, W. Husak, D. Rusanovskyy, P. Topiwala, P. Wu |
| JCTVC-AC0021 | m41442 | 2017-09-28 18:29:54 | 2017-09-28 19:00:14 | 2017-10-09 06:22:57 | Topics for coordination between OMAF and JCT-VC in Macau | Y.-K. Wang (Qualcomm) |
| JCTVC-AC0022 | m41444 | 2017-09-29 22:23:17 | 2017-09-29 23:46:12 | 2017-09-29 23:46:12 | Sphere rotation SEI message | Y.-K. Wang (Qualcomm) |
| JCTVC-AC0023 | m41445 | 2017-09-30 01:22:20 | 2017-10-03 00:31:46 | 2017-10-09 06:23:20 | On CMP padding and region-wise packing | Y.-K. Wang (Qualcomm), M. Zhou (Broadcom) |
| JCTVC-AC0024 | m41455 | 2017-10-05 19:52:30 | 2017-10-08 07:43:10 | 2017-10-10 00:12:04 | On ERP equations for sample location remapping and sphere coverage signalling | Y.-K. Wang (Qualcomm) |
| JCTVC-AC0025 | m41456 | 2017-10-05 19:55:31 | 2017-10-10 19:42:40 | 2017-10-10 19:42:40 | SEI manifest and prefix indication SEI messages | Y.-K. Wang, T. Stockhammer (Qualcomm), D. Singer, A. M. Tourapis (Apple) |
| JCTVC-AC0026 | m41483 | 2017-10-09 21:46:53 | 2017-10-11 23:50:57 | 2017-10-12 22:08:29 | BD-Rate/BD-PSNR Excel extensions | A.M. Tourapis, D. Singer, Y. Su, K. Mammou (Apple Inc) |
| JCTVC-AC0027 | m41491 | 2017-10-10 08:02:24 | 2017-10-10 08:08:43 | 2017-10-19 08:16:05 | Centralized Texture Depth Packing SEI Message for HEVC and AVC | J.-F. Yang, G.-C. Chen (NCKU) |
| JCTVC-AC0028 | m41502 | 2017-10-10 17:40:18 | 2017-10-10 18:32:02 | 2017-10-10 18:32:02 | On the relation of frame packing arrangement and region-wise packing SEI messages | M. M. Hannuksela (Nokia) |
| JCTVC-AC0029 | m41508 | 2017-10-10 18:22:24 | 2017-10-10 18:32:44 | 2017-10-10 18:32:44 | Comments on JCTVC-AB1005-v1 | M. M. Hannuksela (Nokia) |
| JCTVC-AC0030 | m41523 | 2017-10-10 23:46:44 | 2017-10-10 23:52:07 | 2017-10-19 05:44:47 | On padding signalling in omnidirectional video | R. Skupin, Y. Sanchez (HHI) |
| JCTVC-AC0031 | m41524 | 2017-10-10 23:47:48 | 2017-10-10 23:52:33 | 2017-10-20 09:09:19 | On static regions for region-wise packing | R. Skupin, Y. Sanchez (HHI) |
| JCTVC-AC0032 | m41526 | 2017-10-10 23:50:35 | 2017-10-10 23:52:45 | 2017-10-19 05:45:35 | On coverage signalling for omnidirectional video | R. Skupin, Y. Sanchez (HHI) |
| JCTVC-AC0033 | m41540 | 2017-10-11 08:45:17 | 2017-10-11 08:47:33 | 2017-10-11 08:47:33 | MCTS extraction with implicit slice reordering | H.-M. Oh, S. Hwang, S. Oh (LGE) |
| JCTVC-AC0034 | m41541 | 2017-10-11 08:49:41 | 2017-10-11 08:56:25 | 2017-10-11 08:56:25 | Omnidirectional fisheye video SEI message | H.-M. Oh, S. Oh (LGE) |
| JCTVC-AC0035 | m41551 | 2017-10-11 20:02:08 | 2017-10-11 20:36:39 | 2017-10-11 20:36:39 | Preferred rendering operation for a recommended viewport | Y.-K. Wang (Qualcomm) |
| JCTVC-AC0036 | m41562 | 2017-10-12 09:06:16 | 2017-10-12 09:18:41 | 2017-10-17 06:42:37 | AHG 6: Input Doc Industry In-Use Combinations of Video Property Description Points | Y. Syed (Comcast), C. Seeger (NBCU), C. Fogg (Movielabs), L. Borg (Adobe) |
| JCTVC-AC0037 | m41587 | 2017-10-13 09:05:55 | 2017-10-13 09:08:52 | 2017-10-13 09:08:52 | On Region-Wise Packing | S Deshpande |
| JCTVC-AC0038 | m41626 | 2017-10-13 20:44:21 | 2017-10-19 04:35:40 | 2017-10-19 04:35:40 | Temporal MCTS Coding Constraints Implementation | R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI), Ryu Eun-Seok, Jangwoo Son (Gachon University) |
| JCTVC-AC0039 | m41796 | 2017-10-17 16:23:42 | 2017-10-21 03:46:02 | 2017-10-21 03:46:02 | Improved figures for chroma 4:2:0 location type | C. Fogg (MovieLabs) |
| JCTVC-AC0040 | m41834 | 2017-10-19 03:36:50 | 2017-10-19 04:50:56 | 2017-10-19 04:50:56 | SCC software and text discrepancy for rounding with bipred restriction | J. Boyce, I. Moccagatta, L. Xu (Intel) |
| JCTVC-AC0041 | m41867 | 2017-10-19 23:07:53 | 2017-10-19 23:27:59 | 2017-10-21 09:40:18 | Text changes for decisions on omnidirectional video SEI messages | Y.-K. Wang (Qualcomm) |
| JCTVC-AC1000 | m41928 | 2017-11-09 03:48:41 |  |  | Meeting Report of the 29th JCT-VC Meeting (19–24 Oct. 2017, Macao, CN) | G. J. Sullivan, J.-R. Ohm |
| JCTVC-AC1003 | m41929 | 2017-11-09 03:52:23 |  |  | Usage of video signal type code points (Draft 1) | L. Borg, C. Fogg, C. Seeger, G. J. Sullivan, Y. Syed |
| JCTVC-AC1005 | m41897 | 2017-10-24 23:44:00 | 2017-10-24 23:44:45 | 2017-10-24 23:44:45 | HEVC Additional Supplemental Enhancement Information (Draft 4) | J. Boyce, A. Ramasubramonian, R. Skupin, G. J. Sullivan, A. Tourapis, Y.-K. Wang |
| JCTVC-AC1008 | m41930 | 2017-11-09 03:53:26 |  |  | Software and Explanatory Material for Study of Centralized Texture Depth Packing Technology | J.-F. Yang, G. J. Sullivan |
| JCTVC-AC1016 | m41931 | 2017-11-09 03:54:44 |  |  | Conformance Testing for HEVC Screen Content Coding (SCC) Extensions and Non-Intra High Throughput Profiles (Draft 7) | R. Joshi, T. Suzuki, A. Tourapis, J. Xu |
| JCTVC-AC1100 | m41932 | 2017-11-09 03:55:46 | 2018-01-10 23:46:41 | 2018-01-10 23:46:41 | Common Test Conditions for HM video coding experiments | K. Sharman, K. Sühring |

- Annex B to JCT-VC report:
List of meeting participants

The participants of the twenty-ninth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately 110 people in total), were as follows:

- Adeel Abbas (GoPro)

- Peter Amon (Siemens AG)

- Jicheng An (Huawei)

- Kenneth Andersson (LM Ericsson)

- Stefano Andriani (Arnold & Richter Cine Technik GmbH)

- Max Blaeser (RWTH Aachen Univ.)

- Frank Bossen ()

- Jill Boyce (Intel)

- Benjamin Bross (Fraunhofer HHI)

- Eric (Chi W.) Chai (Real Commun.)

- Jianle Chen (Qualcomm)

- Lulin Chen (MediaTek USA)

- Xu Chen (Huawei Tech.)

- Dugyoung Choi (Chips & Media)

- Kiho Choi (Samsung Electronics)

- Takeshi Chujoh (Sharp)

- Zhipin Deng (Intel)

- Sachin Deshpande (Sharp)

- Xavier Ducloux (Harmonic)

- Alberto Duenas (ARM)

- Chad Fogg (MovieLabs)

- Edouard François (Technicolor)

- Alexandre Gabriel (TNO)

- Christophe Gisquet (Canon Research France)

- Soo-Chul Han (Vidyo)

- Miska Hannuksela (Nokia)

- Ryoji Hashimoto (Renesas)

- Tomonori Hashimoto (Sharp)

- Cordula Heithausen (RWTH Aachen Univ.)

- Christopher Hollman (Ericsson)

- Michael Horowitz (Sharp)

- Chih-Wei Hsu (MediaTek)

- Yu-Wen Huang (MediaTek)

- Hyunsu Hwang (Yonsei Univ.)

- Atsuro Ichigaya (NHK (Japan Broadcasting Corp.))

- Ando Ichiro (Nikon)

- Tomohiro Ikai (Sharp)

- Masaru Ikeda (Sony)

- Shunsuke Iwamura (NHK (Japan Broadcasting Corp.))

- Dong Jiang (Bytedance)

- Ryuichi Kanoh (Panasonic)

- Kei Kawamura (KDDI)

- Michel Kerdranvat (Technicolor)

- Yoshitaka Kidani (KDDI)

- Hyo-Song Kim (Pixtree)

- Jae-Gon Kim (Korea Aerosp. Univ.)

- Jani Lainema (Nokia)

- Shawmin Lei (MediaTek)

- Xiang Li (Qualcomm Tech.)

- Chongsoon Lim (Panasonic)

- Jeongyeon Lim (SK Telecom)

- Sung-Chang Lim (Electronics and Telecom Research Institute (ETRI))

- Hung Chih Lin (MediaTek)

- Jian-Liang Lin (MediaTek)

- Ajay Luthra (Arris)

- Gaëlle Martin-Cocher (Blackberry)

- Akira Minezawa (Mitsubishi Electric)

- Kiran Misra (Sharp)

- Taeyoung Na (SK Telecom)

- Shimpei Nemoto (NHK (Japan Broadcasting Corp.))

- Hyun Mook Oh (LG Electronics)

- Sejin Oh (LG Electronics)

- Jens-Rainer Ohm (RWTH Aachen Univ.)

- Jee-Yoon Park (Sungkyunkwan Univ. (SKKU))

- Seanae Park (KWU)

- Magali Philippe (Sony)

- Chirag Pujara (Samsung)

- Fabien Racape (Technicolor)

- Justin Ridge (Nokia)

- Dmytro Rusanovskyy (Qualcomm)

- Hochan Ryu (Digital Insights)

- Jonatan Samuelsson (Divideon)

- Yago Sanchez De La Fuente (Fraunhofer HHI)

- Andrew Segall (Sharp Labs)

- Vadim Seregin (Qualcomm)

- Karl Sharman (Sony Europe Broad. & Prof. Research Labs)

- Masato Shima (Canon)

- Jae Seop Shin (Pixtree)

- Rickard Sjöberg (Ericsson)

- Robert Skupin (Fraunhofer HHI)

- Sehoon Son (Pixtree)

- Jong-Yeul Suh (LG Electronics)

- Karsten Sühring (Fraunhofer HHI

- )

- Gary Sullivan (Microsoft)

- Yule Sun (Zhejiang Univ.)

- Teruhiko Suzuki (Sony)

- Yasser Syed (Comcast Cable)

- Herbert Thoma (Fraunhofer IIS)

- Emmanuel Thomas (TNO)

- Tadamasa Toma (Panasonic)

- Alexandros Tourapis (Apple)

- Yi-Shin Tung (ITRI USA / MStar Semi.)

- Geert Van der Auwera (Qualcomm Tech.)

- Li Wang (Hikvision)

- Ye-Kui Wang (Qualcomm)

- Ping Wu (ZTE UK)

- Yongjun Wu (Amazon)

- Yong Yan (Polycom)

- Haitao Yang (Huawei Tech.)

- Jar-Ferr (Kevin) Yang (National Cheng Kung Univ.)

- Jongsu Yoon (Yonsei Univ.)

- Vladyslav Zakharchenko (Huawei)

- Xianguo Zhang (Kingsoft Cloud)

- Liang Zhao (Hulu)

- Jianhua Zheng (Huawei Tech.)

- Xiaozhen Zheng (DJI)

- Minhua Zhou (Broadcom Limited)

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.