| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 38th Meeting: Brussels, BE, 10–17 January 2020 | Document: JCTVC-AL0023-v1 |
| --- | --- |

| Title: | Additional proposed modifications of the draft shutter interval information SEI message syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | {Sean.McCarthy,pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution discusses the draft shutter interval information SEI message as found in JCTVC-AK1005, which was adopted at the 37th JCT-VC meeting of October 2019; and proposed modifications to the draft as found in JCTVC-AL0021.  JCTVC-AL0021 asserts that the syntax, as currently drafted, has undesirable characteristics and proposes six variant syntaxes to address these issues.  This contribution proposes two additional variants to address those syntax characteristics that we agree are undesirable and provides clarification for an assertion with which we disagree.

# Problem Statement

JCTVC-AL0021 asserts the following are undesirable characteristics of the current syntax (enumerated here for clarity):

- That it seems pointless and confusing to send sii_num_units_in_shutter_interval when fixed_shutter_interval_within_clvs_flag is equal to 0, since the syntax element has no apparent purpose or meaning in that case.

- That it seems pointless and confusing to send sii_max_sub_layers_minus1 when fixed_shutter_interval_within_clvs_flag is equal to 1, since the syntax element has no apparent purpose in that case.

- That when the value of sps_max_sub_layers_minus1 in the SPS is equal to 0, it is strange to allow fixed_shutter_interval_within_clvs_flag to be equal to 0, since this case becomes just an alternative way to express the same case as with fixed_shutter_interval_within_clvs_flag to be equal to 1.

- That the syntax seems excessive in terms of bit usage, using 68 + n * 32 bits for a CLVS with n temporal sub-layers.

We agree with #1 and #3. We also agree that #4 is desirable but assert that it would not be achieved by any of the variants proposed in JCTVC-AL0021. We disagree with #2: The value of sii_max_sub_layers_minus1 needs to be signalled even if shutter interval of each sublayer is the same.

# Proposal

Syntax and semantics for two proposed variants are presented below. Differences from JCTVC-AK1005 are highlighted in green below.  Corresponding modifications of JCTVC-AK1005 are provided separately as JCTVC-AL0023-spec_text-var_A-v1.docx and JCTVC-A0023-spec_text-var_B-v1.docx

Note that proposed variant B (below) is similar to syntax discussed previously as JCTVC-AK0026 variant #1, which was not adopted.  Instead, JCTVC-AK0026 variant #2 was adopted and formed the basis of the current syntax and semantics specified in JCTVC-AK1005.

## Proposed variation A (addressing issues #1 and #3)

### Shutter interval information SEI message syntax

| shutter_interval_info( payloadSize ) { | Descriptor |
| --- | --- |
| sii_time_scale | u(32) |
| sii_max_sub_layers_minus1 | u(3) |
| fixed_shutter_interval_within_clvs_flag | u(1) |
| if( fixed_shutter_interval_within_clvs_flag ) |  |
| sii_num_units_in_shutter_interval | u(32) |
| else { |  |
| for( i = 0; i <= sii_max_sub_layers_minus1; i++ ) |  |
| sub_layer_num_units_in_shutter_interval[ i ] | u(32) |
| } |  |
| } |  |

### Shutter interval information SEI message syntax

The shutter interval information SEI message indicates the shutter interval for the associated video source pictures prior to encoding and display, e.g., for camera-captured content, the shutter interval is amount of time that an image sensor is exposed to produce each source picture.

sii_time_scale specifies the number of time units that pass in one second. The value of sii_time_scale shall be greater than 0. For example, a time coordinate system that measures time using a 27 MHz clock has an sii_time_scale of 27 000 000.

sii_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CLVS referring to the SPS. The value of sii_max_sub_layers_minus1 shall be equal to the value of sps_max_sub_layers_minus1 in the SPS.

fixed_shutter_interval_within_clvs_flag equal to 1 specifies that the indicated shutter interval is the same for all temporal sub-layers in the CLVS. fixed_shutter_interval_within_clvs_flag equal to 0 specifies that the indicated shutter interval may not be the same for all temporal sub-layers in the CLVS. When the value of sii_max_sub_layers_minus1 is equal to 0, the value of fixed_shutter_interval_within_clvs_flag shall be equal to 1.

sii_num_units_in_shutter_interval, when fixed_shutter_interval_within_clvs_flag is equal to 1, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the indicated shutter interval of each picture in the CLVS. The value 0 may be used to indicate that the associated video content contains screen capture content, computer generated content, or other non-camera-captured content.

The indicated shutter interval, denoted by the variable shutterInterval, in units of seconds, is equal to the quotient of sii_num_units_in_shutter_interval divided by sii_time_scale. For example, to represent a shutter interval equal to 0.04 seconds, sii_time_scale may be equal to 27 000 000 and sii_num_units_in_shutter_interval may be equal to 1 080 000.

sub_layer_num_units_in_shutter_interval[ i ], when present, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the shutter interval of each picture in the sub-layer representation with TemporalId equal to i in the CLVS. The sub-layer shutter interval for the sub-layer representation with TemporalId equal to i, denoted by the variable subLayerShutterInterval[ i ], in units of seconds, is equal to the quotient of sub_layer_num_units_in_shutter_interval[ i ] divided by sii_time_scale.

The variable subLayerShutterInterval[ i ], corresponding to the indicated shutter interval of each picture in the sub-layer representation with TemporalId equal to i in the CLVS, is thus derived as follows:

if( fixed_shutter_interval_within_clvs_flag )
	subLayerShutterInterval[ i ] = sii_num_units_in_shutter_interval ÷ sii_time_scale	(D.X)
else
	subLayerShutterInterval[ i ] = sub_layer_num_units_in_shutter_interval[ i ] ÷ sii_time_scale

### Bit usage

Bit usage of proposed variant A is the same as in the syntax as currently drafted: 68 + n * 32 bits for a CLVS with n temporal sub-layers.

## Proposed variation B (addressing issues #1, #3, and #4)

### Shutter interval information SEI message syntax

| shutter_interval_info( payloadSize ) { | Descriptor |
| --- | --- |
| sii_num_units_in_shutter_interval | u(32) |
| sii_time_scale | u(32) |
| sii_max_sub_layers_minus1 | u(3) |
| fixed_shutter_interval_within_clvs_flag | u(1) |
| if( !fixed_shutter_interval_within_clvs_flag ) |  |
| for( i = 0; i <= sii_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_shutter_interval_numer[ i ] | ue(v) |
| sub_layer_shutter_interval_denom[ i ] | ue(v) |
| } |  |
| } |  |

### Shutter interval information SEI message syntax

The shutter interval information SEI message indicates the shutter interval for the associated video source pictures prior to encoding and display, e.g., for camera-captured content, the shutter interval is amount of time that an image sensor is exposed to produce each source picture.

sii_num_units_in_shutter_interval, when fixed_shutter_interval_within_clvs_flag is equal to 1, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the indicated shutter interval of each picture in the CLVS. The value 0 may be used to indicate that the associated video content contains screen capture content, computer generated content, or other non-camera-captured content.

The indicated shutter interval, denoted by the variable shutterInterval, in units of seconds, is equal to the quotient of sii_num_units_in_shutter_interval divided by sii_time_scale. For example, to represent a shutter interval equal to 0.04 seconds, sii_time_scale may be equal to 27 000 000 and sii_num_units_in_shutter_interval may be equal to 1 080 000.

sii_time_scale specifies the number of time units that pass in one second. The value of sii_time_scale shall be greater than 0. For example, a time coordinate system that measures time using a 27 MHz clock has an sii_time_scale of 27 000 000.

sii_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CLVS referring to the SPS. The value of sii_max_sub_layers_minus1 shall be equal to the value of sps_max_sub_layers_minus1 in the SPS.

fixed_shutter_interval_within_clvs_flag equal to 1 specifies that the indicated shutter interval is the same for all temporal sub-layers in the CLVS. fixed_shutter_interval_within_clvs_flag equal to 0 specifies that the indicated shutter interval may not be the same for all temporal sub-layers in the CLVS. When the value of sii_num_units_in_shutter_interval is equal to 0, or when the value of sii_max_sub_layers_minus1 is equal to 0, the value of fixed_shutter_interval_within_clvs_flag shall be equal to 1.

sub_layer_shutter_interval_numer[ i ] specifies the numerator used to derive sub layer shutter interval, defined by variable subLayerShutterInterval[ i ], in units of seconds, when HighestTid is equal to i. The value 0 may be used to indicate that the associated video content contains screen capture content, computer generated content, or other non-camera-captured content.

sub_layer_shutter_interval_denom[ i ] specifies the denominator used to derive sub layer shutter interval, defined by variable subLayerShutterInterval[ i ], in units of seconds, when HighestTid is equal to i. The value of sub_layer_shutter_interval_denom[ i ] shall be greater than 0.

The variable subLayerShutterInterval[ i ], corresponding to the indicated shutter interval of each picture in the sub-layer representation with TemporalId equal to i in the CLVS, is thus derived as follows:

if( fixed_shutter_interval_within_clvs_flag )
	subLayerShutterInterval[ i ] = sii_num_units_in_shutter_interval ÷ sii_time_scale	(D.X)
else
	subLayerShutterInterval[ i ] = sub_layer_num_units_in_shutter_interval ÷ sii_time_scale * sub_layer_shutter_interval_numer[ i ] ÷  sub_layer_shutter_interval_denom[ i ]

### Bit usage

The number of bits used for 0-th order Exp-Golomb code, ue(v), for various ranges of codeNum are tabulated in Table 1.

Table 1. Number of bits for various ranges of codeNum for 0-th order Exp-Golomb codewords

| Range of codeNum |  | Number of bits |
| --- | --- | --- |
| min | max |  |
| 1 | 2 | 3 |
| 3 | 6 | 5 |
| 7 | 14 | 7 |
| 15 | 30 | 9 |
| 31 | 62 | 11 |
| 63 | 126 | 13 |
| 127 | 254 | 15 |
| 255 | 510 | 17 |
| 511 | 1022 | 19 |
| 1023 | 2046 | 21 |
| 2047 | 4094 | 23 |
| 4095 | 8190 | 25 |
| 8191 | 16382 | 27 |
| 16383 | 32766 | 29 |
| 32767 | 65534 | 31 |

Bit usage of proposed variant B may be less than that for the syntax as currently drafted when the values of sub_layer_shutter_interval_numer[ i ] and sub_layer_shutter_interval_denom[ i ] have certain relationships. As examples, the total number of bits coded for the combination of sub_layer_shutter_interval_numer[ i ] and sub_layer_shutter_interval_denom[ i ] may be less than or equal to 32 for the following:

- when the value of sub_layer_shutter_interval_numer[ i ] is 1 or 2 and the value of sub_layer_shutter_interval_denom[ i ] is less than or equal to 32766;

- when the value of sub_layer_shutter_interval_numer[ i ] is equal to 254 and the value of sub_layer_shutter_interval_denom[ i ] is equal to 255.

In the second example, the value of the ratio of sub_layer_shutter_interval_numer[ i ] and sub_layer_shutter_interval_denom[ i ] would be respresent the finest precision possible less than or equal to 1.  In this example, the precision of the ratio would be 1/255.

Thus, values of sub_layer_shutter_interval_numer[ I ] and sub_layer_shutter_interval_denom[ I ] may be selected so that the bit usage of proposed variant B would be less than 68 + n * 32 bits for a CLVS with n temporal sub-layers; but may consequently results in lower precision than would be possible with the syntax as currently drafted.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).