| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0004 |
| --- | --- |

| Title: | VCEG AHG report on test sequences selection (AHG4 of VCEG) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki Jill Boyce Andrey Norkin | Email: | teruhikos@jp.sony.com jill.boyce@intel.com anorkin@netflix.com |
| Source: | VCEG AHG |  |  |

_____________________________

- Abstract

The following document summarizes activities on test sequences selection made from the last meeting in Geneva. Sequences chosen for evaluation at the last meeting are available at the Aachen university ftp. There has been a number of input contributions studying the selecting sequences. Four new SCC test sequences have been proposed.

Introduction

The mandates of this AHG were:

- Identify test sequences available for video coding technology testing

- Particularly seek useful source material for non-4:2:0 colour formats, high

bit depth, high frame rate, high dynamic range and wide colour gamut

- Select suggested subsets for conducting experiments

Test sequences status

Parts of Chimera and El Fuente test sequences chosen at the last meeting have been made available by Netflix under the creative commons-attribution-no commercial use-no derivative 4.0 license.

The following sequences have been uploaded and are available from the Aachen university ftp ftp.ient.rwth-aachen.de.

- Bcom

- Huawei

- Netflix (chosen parts of Chimera and ElFuente)

- University of Bristol.

Full ElFuente and Chimera sequences are available at www.cdvl.org under CDVL license.

Four new screen content coding sequences have been proposed by Tongii university. There are four sequences, 1920x1080, 30 fps, 300 frames, both RGB and YCbCr versions are available.

Format conversions

The conversion software (4:4:4 to 4:2:0) and scripts were provided by the contents provides, except SJTU test sequences). In the last meeting, it was agreed to use ffmpeg. But there was a suggestion to use HDRtools, because filtering and color conversion process is not clear in ffmpeg. Appropriate settings of HDRtools must be clarified if it is used for the conversion.

Contributions to this Meeting

New SCC test sequences:

- JVET-B0049 "Four new SCC test sequences for ultra high quality and ultra high efficiency SCC", Jing Guo, Liping Zhao, Tao Lin (Tongji)

- JVET-B0050 "Performance comparison of HEVC SCC CTC sequences between HM16.6 and JEM1.0", Shuhui Wang, Tao Lin (Tongji)

Contributions related to studying of proposed test sequences.

- JVET-B0024 "Evaluation report of SJTU Test Sequences", Thibaud Biatek, Xavier Ducloux.

- JVET-B0025 "Evaluation Report of Chimera Test Sequence for Future Video Coding", H. Ko, S.-C. Lim, J. Kang, D. Jun, J. Lee, H. Y. Kim (ETRI).

- JVET-B0026 "JEM1.0 Encoding Results of Chimera Test Sequence", S.-C. Lim, H. Ko, J. Kang, H. Y. Kim (ETRI).

- JVET-B0027 "SJTU 4K test sequences evaluation report from Sejong University", NamUk Kim, JunWoo Choi, Ga-Ram Kim.

- JVET-B0029 "Evaluation report of B-Com test sequence (JCTVC-V0086)", O. Nakagami, T. Suzuki (Sony).

- JVET-B0030 "Comment on test sequence selection", O. Nakagami, T. Suzuki (Sony).

- JVET-B0031 "Evaluation report of Huawei test sequencev", Kiho Choi, E. Alshina, A. Alshin, M. Park.

- JVET-B0035 "Evaluation Report of Chimera and Huawei Test Sequences for Future Video Coding", Pierrick Philippe (Orange).

- JVET-B0040 "Evaluation Report of Huawei and B-Com Test Sequences for Future Video Coding", Fabien Racapé, Fabrice Le Léannec, Tangi Poirier.

- JVET-B0042 "Evaluation Report of B-COM Test Sequence for Future Video Coding (JCTVC-V0086)", Han Boon Teo, Meng Dong.

- JVET-B0046 "Evaluation report of Netflix Chimera and SJTU test sequences", F. Zou, J. Chen, X. Li, M. Karczewicz (Qualcomm).

- JVET-B0052 "Report of evaluating Huawei surveillance test sequences", Ching-Chieh Lin, Jih-Sheng Tu, Yao-Jen Chang, Chun-Lung Lin (ITRI).

- JVET-B0053 "Report of evaluating Huawei UGC test sequences", Jih-Sheng Tu, Ching-Chieh Lin, Yao-Jen Chang, Chun-Lung Lin (ITRI).

- JVET-B0055 "Netflix Chimera test sequences evaluation report", Maxim Sychev, Huanbang Chen(Huawei)   - Not uploaded

- JVET-B0056 "Evaluation report of SJTU Test Sequences from Sharp", T. Ikai (Sharp).

- JVET-B0061 "Evaluation report of SJTU test sequences for future video coding standardization", Sang-hyo Park, Haiyan Xu, Euee S. Jang.

- JVET-B0065 "Coding results of 4K surveillance and 720p portrait sequences for future video coding", K. Kawamura, S. Naito (KDDI).

Recommendations

The AHG recommends:

- To review all related contributions.

- To view new sequences from Tongii University.

- To perform viewing of the test material.

- Make selection of test material for the JVET activity.

- Annex: Work plan for performing encodings of new sequences

  - Summary

This contribution summarizes the BoG discussion the test sequences contributed by the following contributions. BoG discussed the work plan to evaluate new test sequences for future video coding standard development. The new test sequences are proposed in the following contributions.

- JCTVC-V0083, JCTVC-V0086, JCTVC-V0092, JCTVC-V0093

- Netflix Chimera (m36685, VCEG-AZ09)

  - Participants

The following volunteers are identified for the encoding and the evaluation of the sequences.

- B-COM (Xavier Ducloux)

- KDDI (Kei Kawamura)

- Hanyang University (Sang-hyo Park)

- Panasonic (Chong Soon Lim)

- Huawei (Maxim Sychev)

- Sony (Ohji Nakagami)

- Qualcomm (Xiang Li)

- Samsung (Kiho Choi)

- Orange (Pierrick Philippe)

- ETRI (Sung Chang Lim)

- Sejong University (Yung-Lyul Lee)

- Sharp (Ikai Tomohiro)

- Technicolor (Fabien Racapé)

  - Test procedures

At Geneva, viewing session is planned to understand what kind of test sequences are proposed. All test sequences proposed at this meeting will be available at the ftp sites. Volunteers will encode the assigned test sequences and evaluate quality. Then volunteers will report the conclusions by the next San Diego meeting.

The preselected test sequences in section 2 will be reviewed again in the next San Diego meeting. The volunteers assigned to those sequences should bring the encoded bitstreams and origian to the next meeting.

    - Encoding conditions

The following encoding conditions will be used.

- Encoder: HM16.6 (Mandatory), HM16.6KTA2.0 (Optional)

- Encode only 4:2:0 yuv 10 bit

- Whole sequence must be encoded except high frame rate sequence. For high frame rate sequence only the first 5 sec will be encoded

- Conversion software: ffmpeg will be used for the conversion from 4:4:4 to 4:2:0 (and from 4:2:2 to 4:2:0). The script will be provided later

- Contents provide will provide md5 for each sequence to check if conversion is correct

- QP: 5 QP will be used. (22, 27, 32, 37 and 42)

- RA & LDB

- I period: 32 (30fps), 48 (50 fps), 64 (60 fps), 96 (100 fps) and 120 (120 fps)

    - Report of the evaluation

The template (the example) for the report will be distributed. (like m37074)

The followings must be included in the report.

- Rate distortion curve (PSNR)

- Still picture of encoded sequence (the point to see should be highlighted)

- Summary table (sequence name, description of scene, thumbnail, comments), like below.

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| Bund Nightscape | The nightscape of the gallery of International Architecture in Bund |  |  |
| Campfire Party | The flames before the crew of the NERC-DTV in a campfire party |  |  |

- Conclusion of the evaluation (recommendation of the sequence, etc.)

    - Preparation for the next meeting

Volunteers will bring the followings to the next meeting for subjective viewing.

- Bitstreams (and YUV of QP=37 and/or 32 if possible)

- Original file (Contents provides should bring and volunteers may bring if possible)

    - Availability of test sequences

- Netflix: Within two weeks (Nov. 6)

- SJTU: Publicly available at the web site

- B-COM: Provide by HDD to assigned volunteer at this meeting, and upload to the ftp site later

- Huawei: Provide by HDD to assigned volunteer at this meeting, and upload to the ftp site later

The files uploaded to the test sequence ftp site will be Huawei’s and B-COM’s sequences.

For Huawei sequences, 100 GB is necessary. For B-COM sequences, xxx GB is necessary.

    - Display

Displays with all post processing disabled to be available in San Diego

    - Assignment

The following is the mandatory assignment. Two organizations are assigned to each sequences and we can collect the conclusions from at least two independent organizations.

| Volunteer # | Sequences |
| --- | --- |
| 1 | SJTU sequences: Bund Nightscape, Campfire Party, Construction Field, Fountains and Library |
| 2 | Huawei sequences: both surveillance & UGC |
| 3 | SJTU: Marathon, Runners, Residential Building, Rush Hour, Tall Buildings |
| 4 | B-COM sequences |
| 5 | Chimera sequences |
| 6 | B-COM sequences |
| 7 | Chimera, SJTU sequences |
| 8 | Huawei sequences: both surveillance & UGC |
| 9 | Huawei and Chimera sequences |
| 10 | Chimera sequences |
| 11 | SJTU sequences: Traffic and Building, Traffic Flow and Tree Shade |
| 12 |  |
| 13 | B-Com sequences and Huawei sequences: both surveillance & UGC |

  - Annex: Summary of new test sequences for future video coding standard development

| Sequence | Owner | Resolution | Bit depth | Frame rate | Color Sampling | Color space | Duration | Capturing | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bund Nightscape | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Campfire Party | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Construction Field | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Fountains | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Library | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Marathon | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Runners | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Residential Building | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Rush Hour | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Tall Buildings | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Teaffic and Building | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Traffic Flow | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Tree Shade | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Scarf | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| Wood | SJTU (V0083) | 3840x2160 | 10 8 | 30p | 4:4:4 4:2:0 (8) | ?? | 10s | Sony F65 RAW |  |
| CatRobot | B-COM (V0086) | 3840x2160 | 10 | 60p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Sony F65 |  |
| Drums | B-COM (V0086) | 3840x2160 | 10 | 50p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Dragon Epic |  |
|  | B-COM (V0086) | 3840x2160 | 10 | 100p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Mysterium |  |
| FreeSardines | B-COM (V0086) | 3840x2160 | 10 | 60p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Dragon Epic |  |
|  | B-COM (V0086) | 3840x2160 | 10 | 120p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Mysterium |  |
| Katana | B-COM (V0086) | 3840x2160 | 10 | 50p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Dragon Epic |  |
|  | B-COM (V0086) | 3840x2160 | 10 | 100p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Mysterium |  |
| Rowing | B-COM (V0086) | 3840x2160 | 10 | 60p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Dragon Epic |  |
|  | B-COM (V0086) | 3840x2160 | 10 | 120p | 4:4:4 (RGB&YUV) | BT.709 | 10s | Red Mysterium |  |
| Metro | Huawei (V0092) | 3840x2160 | 10 | 60p | RGB 4:4:4 |  | 3600 frames | Sony F65 |  |
| BridgeViewTraffic | Huawei (V0092) | 3840x2160 | 10 | 60p | RGB 4:4:4 |  | 3600 frames | Sony F65 |  |
| Square | Huawei (V0092) | 3840x2160 | 10 | 60p | RGB 4:4:4 |  | 3600 frames | Sony F65 |  |
| DaylightRoad | Huawei (V0092) | 3840x2160 | 10 | 60p | RGB 4:4:4 |  | 3600 frames | Sony F65 |  |
| NightRoad | Huawei (V0092) | 3840x2160 | 10 | 60p | RGB 4:4:4 |  | 3600 frames | Sony F65 |  |
| Aerial Crowd | Huawei (V0092) | 3840x2160 | 8 | 60p | 4:2:0 |  | 3600 frames | Sony F65 |  |
| CStoreWalking | Huawei (V0093) | 720x960 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| DrivingRecorder1 | Huawei (V0093) | 720x960 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| DrivingRecorder2 | Huawei (V0093) | 720x960 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| LakeWalking | Huawei (V0093) | 720x960 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| CStoreGoods | Huawei (V0093) | 720x1280 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| ParkSunny | Huawei (V0093) | 720x1280 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |
| ParkWalking | Huawei (V0093) | 720x1280 | 8 | 30p | 4:2:0 |  | 10s | Huawei Honor 6 mobile-phone |  |

          - JCTVC-V0083 SJTU Test Sequences for video coding development [L. Song (SJTU)] [late]

4:4:4 and 4:2:0 4K@30fps

Sony F65 camera

300 frames or 10s

The raw data of the images quantified with 10 bit were exported at resolution of 3840x2160 and frame rate of 30fps with DPX format. The DPX image files were combined and converted into uncompressed YUV files using FFMPEG software. Both YUV 4:4:4 color sampling, 10 bits per sample and YUV 4:2:0 color sampling, 8 bits per sample formats are offered in our dataset.

All video sequences presented in the dataset are of 300 frames or 10s.

Table 1: Summary of the SJTU 4K video sequences dataset

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| Bund Nightscape | The nightscape of the gallery of International Architecture in Bund |  |  |
| Campfire Party | The flames before the crew of the NERC-DTV in a campfire party |  |  |
| Construction Field | The excavator arms in a construction area |  |  |
| Fountains | The artificial fountains jetting water into the air in front of a tall building |  |  |
| Library | The sparsely moving students in the front of the new library |  |  |
| Marathon | The early stages of 2012 Shanghai International Marathon Race |  |  |
| Runners | The runners in midway of 2012 Shanghai International Marathon Race |  |  |
| Residential Building | A residential building with dense texture of the outside wall and intricate lines |  |  |
| Rush Hour | The students on the ways to canteen or dormitory after classes |  |  |
| Tall Buildings | Skyscraper in Lujiazui, Pudong New District in Shanghai |  |  |
| Traffic and Building | Cars on the road and distant buildings |  |  |
| Traffic Flow | The roadways with the automobiles moving in a different direction |  |  |
| Tree Shade | The shade under a large tree |  |  |
| scarf | The vivid exhibition of an accessory shop |  |  |
| Wood | A wood in the campus of SJTU with sunlight piercing through |  |  |

    - Copyright information

The test sequences(SJTU-4K) and all intellectual property rights therein remain the property of Institute of Image Communication & Network Engineering, Shanghai Jiao Tong University.

The following uses are allowed for the SJTU-4K sequences:

- Can be published in technical papers.

- Can be played at technology research and development events.

- Can be used by Standards committees. (e.g., ITU, MPEG, VQEG,).

The following uses are not allowed for the SJTU-4K sequences:

- Do not publish snapshots in product brochures.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

          - JCTVC-V0086 B-Com Test Sequences for video coding development [Felix Henry, Xavier Ducloux, Thibaud Biatek, Jean-Yves Aubie (bcom)] [late]

The test set is composed of five UltraHD (3840x2160) sequences in RGB 4:4:4 10 bits linear DPX format or YUV 4:4:4 10 bits format.

Table 2 – Test set

| Sequence | Camera | Frame rate | Aperture | Color primaries | Length |
| --- | --- | --- | --- | --- | --- |
| CatRobot | Sony F65 | 60 fps | 180° | 709 | 10 s |
| Drums | Red Dragon Epic | 50 fps | 180° | 709 | 10 s |
|  | Red Mysterium | 100 fps | 180° | 709 | 10 s |
| FreeSardines | Red Dragon Epic | 60 fps | 180° | 709 | 10 s |
|  | Red Dragon Epic | 120 fps | 180° | 709 | 10 s |
| Katana | Red Dragon Epic | 50 fps | 180° | 709 | 10 s |
|  | Red Mysterium | 100 fps | 90° | 709 | 10 s |
| Rowing | Red Dragon Epic | 60 fps | 180° | 709 | 10 s |
|  | Red Dragon Epic | 120 fps | 180° | 709 | 10 s |

They have been shot for the purpose of compression research. They cover a large variety of content with studio or external shooting and contain challenging video characteristics for codecs, like:

- camera pan,

- non-rigid object motion, irregular motion trajectories,

- challenging texture structures,

- saturated colour,

- large contrast/dynamic range.

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| CatRobot |  |  |  |
| Drums |  |  |  |
| FreeSardines |  |  |  |
| Katana |  |  |  |
| Rowing |  |  |  |

    - Allowance of usage

Please note that this test material remains the property of the Institute of Research & Technology b<>com and is solely provided “as is" to ITU-T SG16 Q6/16 and ISO/IEC JTC1/SC29/WG11, on a non-transferable and non-exclusive basis, and solely for the purpose of development, testing and promulgation of video coding standards. Any disclosure to third parties, exhibition, broadcast, distribution, public display or other exploitation of the test material is strictly prohibited.

          - JCTVC-V0092 Surveillance sequences for video coding development [H. Zhang, W. Qiu, Q. Xie, M. Sychev, H. Yang, J. Zhou (Huawei), X. Tang (NERC)] [late] [miss]

Table 3 – Specification of Surveillance test sequences

| Sequences | Resolution | Frame count | Frame rate(fps) |  |
| --- | --- | --- | --- | --- |
| Metro | 3840x2160 | 3600 | 60 |  |
| BridgeViewTraffic | 3840x2160 | 3600 | 60 |  |
| Square | 3840x2160 | 3600 | 60 |  |
| DaylightRoad | 3840x2160 | 3600 | 60 |  |
| NightRoad | 3840x2160 | 3600 | 60 |  |
| Aerial Crowd | 3840x2160 | 3600 | 30 |  |

Raw data in DPX format, RGB444, 10bit.

For Aerial Crowd sequence, only YUV 420, 8 bit is available.

Table 4 – Snapshots and brief content descriptions of proposed Surveillance test sequences

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| Metro | Sunshine, Cloudy Walking people |  | stational Complex motion |
| BridgeViewTraffic | Sunshine, Cloudy Travelling vehicle |  | stational Complex motion |
| DaylightRoad | Sunshine, Cloudy Vehicle monitoring |  |  |
| Square | Sunny square monitoring |  | Building structure Grass detail panning |
| NightRoad | low light, road lamp high noise vehicle monitoring |  |  |
| AerialCrowd | Clear Day with Sunshine Walking crowd Aerial monitoring |  |  |

    - Copyright information

The proposed test sequences in this document and all intellectual property rights therein remain the property of Huawei Technologies Co., Ltd., and National Engineering Research Center.

The following uses are allowed for the proposed sequences:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., ITU, MPEG, VQEG,).

The following uses are not allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

          - JCTVC-V0093 Huawei test sequences of UGC feature for video coding development [X. Ma, H. Zhang, Y. Zhao, M. Sun, M. Sychev, H. Yang, J. Zhou (Huawei)] [late] [miss]

Table 5 – Specification of UGC test sequences

| Sequences | Resolution | Frame count | Frame rate(fps) | Chrome format | Bit depth |  |
| --- | --- | --- | --- | --- | --- | --- |
| CStoreWalking | 720x960 | 300 | 30 | 4:2:0 | 8 |  |
| DrivingRecorder1 | 720x960 | 300 | 30 | 4:2:0 | 8 |  |
| DrivingRecorder2 | 720x960 | 300 | 30 | 4:2:0 | 8 |  |
| LakeWalking | 720x960 | 300 | 30 | 4:2:0 | 8 |  |
| CStoreGoods | 720x1280 | 300 | 30 | 4:2:0 | 8 |  |
| ParkSunny | 720x1280 | 300 | 30 | 4:2:0 | 8 |  |
| ParkWalking | 720x1280 | 300 | 30 | 4:2:0 | 8 |  |

Table 6 – Snapshots and brief content descriptions of proposed UGC test sequences

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| CStoreWalking | Walking in convenience store, watching goods, camera moves horizontally with shakiness. |  |  |
| DrivingRecorder1 | Driving on road and turn around, camera is shaking |  |  |
| DrivingRecorder2 | Driving on road, camera is shaking |  |  |
| LakeWalking | Walking by lake, camera moves horizontally with rotation and shakiness. |  |  |
| CStoreGoods | Watching goods in convenience store, camera moves horizontally and vertically, |  |  |
| ParkSunny | Watching scene in park, camera move horizontally, |  |  |
| ParkWalking | Walking along road, camera is shaking. |  |  |

    - Copyright information

The proposed test sequences in this document and all intellectual property rights therein remain the property of Huawei Technologies.

The following uses are allowed for the proposed sequences:

- Can be published in technical papers, played at technology research and development events.

- Can be used by Standards committees. (e.g., ITU, MPEG, VQEG,).

The following uses are not allowed for the proposed sequences:

- Do not publish snapshots in product brochures.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

Table 7 – Specification of Chimera test sequences

| Sequence | Owner | Resolution | Bit depth | Frame rate | Color Sampling | Color space | Duration | Capturing | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chimera | Netflix (m36685 VCEG-AZ09) | 4096x2160 | 10 | 60p | 4:4:4 | BT.709 |  | Red Dragon |  |

- Netflix: Chimera

- Chimera, RGB BT.709, 10-bit, 60fps.

Sequence name

Bar scene

Dancers

Rollercoaster

Indoor table scene

Drone

Fountain

Rider inside car

Grass

Seaside and pier

Netflix: ElFuente

ElFuente, RGB BT.709, 10-bit, 60fps.

Sequence name

Boat

BoxingPractice

Crosswalk

FoodMarket

FoodMarket2

Narrator

RitualDance

SquareAndTimeLapse

Tango

TunnelFlag

    - Copyright information

The proposed video sequences will be hosted and distributed by the Consumer Digital Video Library (CDVL), run by the Institute of Telecommunication Services (ITS). For this content, the CDVL copyright license is adopted. The users downloading the content would have to sign the user license. The plain language description of the user license can be found below.

Most of the CDVL audiovisual content is available for research and development purposes (CDVL R&D content).

The following uses are allowed for CDVL R&D content:

- To the extent that an authorized commercial entity is conducting internal research and development, use of the CDVL R&D content is appropriate.

- Sample frames from CDVL R&D content can be published in technical papers. CDVL R&D content can be played at technology research and development events.

- Standards committees can use CDVL R&D content within subjective tests to validate objective video quality models (e.g., ATIS, VQEG, ITU).

- A commercial entity can use CDVL R&D content to develop a new product or to improve an existing product.

- A commercial entity can use CDVL R&D content for a free educational demonstration at a trade show, during a customer visit, or on their website. Care should be taken that no product marketing takes place during this free educational demonstration.

The following uses are not allowed for CDVL R&D content:

- Do not market a product

- Do not publish snapshots in product brochures

- Do not redistribute video with a commercial product

- Do not perform a subjective test for profit

- Do not use in television shows, commercials, or movies