| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0035 |
| --- | --- |

| Title: | Evaluation Report of Chimera and Huawei Test Sequences for Future Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Tel: Email: | +33299124061 pierrick.philippe@orange.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

This contribution reports the potential of some video sequences for future video coding activities.

# Introduction

This contribution reports the evaluation and encoding of video material according to the procedure defined in N15791 (Work plan for investigation of test sequence encoding).

Three sets of sequences are investigated in this contribution:

- Huawei Surveillance sequences (as described in JCTVC-V0092)

- Huawei User Generated Content (as described in JCTVC-V0093)

- Netflix Chimera sequences

# Methodology

All the items were coded according to the conditions depicted in N15791 after downloading the clips and MD5 checking.

An Excel spreadsheet, containing the bitrates and PSNR is attached to this contribution.

A viewing session was set up to watch the coded sequences, as it was difficult to notice coding impairments for the lower QP range, the viewing session was performed at QP37 essentially.

This session was performed on an 85 inches display for the 4K sequences (4096x2160 and 3840x2160) and on a 32 inches PC monitor for the UGC content. For the Huawei Surveillance sequences, the LDB coded sequences were used, the RA setup was used for the other kinds.

# JCTVC-V0092 Huawei Surveillance sequences

## Description

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| Metro 3840x2160 60fps | Sunshine, Cloudy Walking people |  | Camera is fixed |
| BridgeViewTraffic 3840x2160 60fps | Sunshine, Cloudy Travelling vehicle |  | Camera is static |
| DaylightRoad 3840x2160 60fps | Sunshine, Cloudy Vehicle monitoring |  | Shaking camera |
| Square 3840x2160 60 fps | Sunny square monitoring |  | Fast panning blurs the texture |
| NightRoad 3840x2160 60fps | low light, road lamp high noise vehicle monitoring |  | Few details, no rich textures |
| AerialCrowd 3840x2160 30fps | Clear Day with Sunshine Walking crowd Aerial monitoring |  | Camera panning |

## Coding results

### Random Access

### Low Delay B

### Evaluation

In terms of PSNR/rate characteristic, Nightroad has a flat characteristic (would probably make a BDRate comparison failure).

All the other sequences exhibit a regular increase of PSNR with the bitrate, Square tends to saturate with the rate.

AerialCrowd, BridgeViewTraffic DaylightRoad and Metro have a regular PSNR characteristic and a wide PSNR range (>5dB).

AerialCrowd includes a panning, BridgeViewTraffic and Metro have a fixed point of view, DaylightRoad includes some camera movements.

From the subjective point of view all those 4 sequences are challenging with respect to coding, AerialCrowd being the more complex (lots of moving people with de-occlusions).

The sequences that we propose to retain, looking appropriate for both objective and subjective evaluations:

- AerialCrowd

- BridgeViewTraffic

- DaylightRoad

# JCTVC-V0093 Huawei UGC

## Sequence description

Table 2 Snapshots and brief content descriptions of proposed UGC test sequences

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| CStoreWalking 720x960 30fps | Walking in convenience store, watching goods, camera moves horizontally with shakiness. |  | Fast camera movement |
| DrivingRecorder1 720x960 30fps | Driving on road and turn around, camera is shaking |  | Shaky capture |
| DrivingRecorder2 720x960 30fps | Driving on road, camera is shaking |  | Shaky capture |
| LakeWalking 720x960 30fps | Walking by lake, camera moves horizontally with rotation and shakiness. |  | Fast camera rotation |
| CStoreGoods 720x1280 30fps | Watching goods in convenience store, camera moves horizontally and vertically, |  | Fast camera movements |
| ParkSunny 720x1280 30fps | Watching scene in park, camera move horizontally, |  | Slow camera motion |
| ParkWalking 720x1280 30fps | Walking along road, camera is shaking. |  | Same as previous with more complex (rolling) camera movement. |

## Coding results

### Random Access

### Low Delay B

### Evaluation

CStoreWalking: Camera movement a little bit to fast to distinguish artefacts

Driving recorder1:shaky capture, artefacts clearly visible (road texture and tree leaves)

DrivingRecorder2: shaky capture, artefacts visible, artefacts noticeable in the building textures.

LakeWalking: fast camera rotation critical at higher QP but disturbing for subjective evaluation

CStoreGoods: fast camera motion making subjective assessment difficult

ParkSunny:Slow camera pan: visible artefacts for each kind of texture (building, grass, ground)

ParkWalking: camera movement including slow rotations, tree branches are critical with coding (especially with sky background)

The sequence we retain for subjective and objective evaluation purposes:

- DrivingRecorder2

- ParkSunny

# Netflix: Chimera

## Sequence description

| Scenario | Description | Thumbnail | Comments |
| --- | --- | --- | --- |
| Aerial |  |  | Fast movements at the beginning. |
| Barscene |  |  | 2 heterogenous scenes, focus change in the first 5 seconds, then 2 cuts. |
| Dancers |  |  | Dark and limited area of movements. |
| DinnerScene |  |  | A long panning then two cuts. Few movements and dark scene. |
| DrivingPOV |  |  | Camera is shaking in the first cut. A second scene lasts 1s. |
| PierSeaside |  |  | Continuous filming. |
| Rollercoaster |  |  | The camera is shaking, large diversity of movements. |
| TaddleFountain |  |  | Water jets, semi transparent occlusion, 2 cuts, the scene in the middle lasts around 8s |
| WindAndNature |  |  | Static camera. Two scenes with a cross fade (>1s long) |

## Coding results

### Random Access

### Low Delay B

## Evaluation

The content looks like a typical movie content (depth of field is narrow, some amount of “film grain”).

The content exhibits a variety of camera variation (zooming, motion, focus change) and includes editing effects (cross-fade, scene cuts).

The scenes are often short: a cut appears after a few seconds, making difficult a subjective evaluation as far as the texture need to be evaluated.

DinnerScene, Dancers and Barscene, which are dark an present a limited amount (or surface) of movement, have a rather flat characteristic (PSNR range within 2dB) which might prevent a robust BDRate evaluation. DinnerScene has two cuts, BarScene includes a focus change and two scene cuts. Dancers is continuous, the woman’s robe includes critical movements for the HM16 coder.

Toddler fountain is the most critical material, as far as the objective measurement is concerned. This was also confirmed during the viewing session: especially due to the rapid movements of the boy behind the water jets.

Aerial has rich movements and does not present any scene cuts. It has lots of high frequency textures that present coding artifacts from QP32. Its PSNR characteristics has a wide range (10dB).

Driving POV has shaking camera movements, the viewing is uncomfortable especially on a big display.

PearSeaSide includes 2 type of contents: a pond with a background sky (6 first seconds) then, after a camera pan, the focus is put on the sea. As such the coding artifacts are different for the two scenes which exhibit a different degree of criticity.

Roller has rich movements and a rich level of details. The camera has some erratic shaking movements which are not constant over time which sometimes stabilize.

WindAndNature include two different scenes: windmill then a cross fade with some grass content with narrow depth of field. The windmill is critical and exhibits lots of blocking artifacts at higher QPs.

The sequences that are thought relevant for objective and subjective evaluations are:

- Aerial

- RollerCoaster

- WindAndNature

# Patent rights declaration(s)

Orange does not have any current or pending patent rights relating to the technology described in this contribution.