| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 2nd Meeting: San Diego, USA, 20–26 February 2016 | Document: JVET-B0041 |
| --- | --- |

| Title: | Adaptive reference sample smoothing simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy Huawei Research Center, 1/7, Altufievskoe shosse, Moscow, Russia | Tel: Email: | +7-495-660-44-59 alexey.filippov@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution presents a simplification of the RDO procedure used by the adaptive reference sample smoothing (ARSS) filter, i.e. a non-normative modification of ARSS is considered. The results of different tests are presented and analyzed. The proposed solution provides a better ratio of the coding gain to encoder-side computational complexity.

# Introduction

This contribution addresses the question of the ARSS computational complexity at the encoder side and is aimed at simplifying its RDO-cost estimation, i.e. the proposed modification of ARSS is non-normative. As noted in [1], ARSS requires a joint optimization of the prediction generation decision together with the residual signal information. The reason for this is that instead of entropy coding, hiding an ARSS flag value within quantized transform coefficients is used for signaling the reference sample smoothing filter selection decision at the TU level. Since reference sample smoothing impacts the intra prediction mode selection procedure, the decision on what of the candidate intra prediction modes will be selected depends on the availability of the residues for hiding.

In this case, limiting the number of residual quad-tree (RQT) levels defined by HHI_RQT_INTRA_SPEEDUP can be resulted in such a situation, when, on a higher RQT level, the number of available quantized transform coefficients is enough to hide a ARSS flag in a TU but, on a lower level, hiding cannot be performed (see Figure 1). Hence, the estimation based on a RQT with a limited number of decomposition levels can be significantly more inaccurate than for coding tools that do not use the joint optimization of the prediction generation decision and the residual signal information. In other words, ARSS requires more detailed information on TU residues (to check whether the smoothing flag can be hidden or not) like ATMVP needs more accurate motion field [2].

# Technical description

Current ARSS solution has a straightforward implementation of encoder-side decision making on ARSS flag value for a TU. In fact, each TU is compressed twice: when ARSS flag is set to zero, and when it is one (see Figure 2).

Figure 1. Hiding procedure in ARSS and RQT decomposition

Figure 2. Encoder-side decision making of ARSS flag value before simplification

In this contribution, several mechanisms are used to simplify the ARSS computational complexity at the encoder side:

- If CBF equals 0 for default reference sample smoothing (i.e. ARSS is disabled), the RD-cost estimation is skipped for enabled ARSS as its decision cannot be signaled anyway.

- If ARSS is enabled and ARSS flag could not be hidden for a given RQT level, the RD-cost is not estimated for the lower RQT levels.

Figures 3 and 4 illustrate the first and the second simplification mechanisms.

All the results given further are presented within a short length simulation using first 30 frames of each CTC test sequence. Current JEM1.0 ARSS “tool-ON” test results are given in Table 1. The simulation results for the 1st simplification mechanism are presented in Table 2. For both simplification mechanisms, the results shown in Table 3 were obtained. Tables 2-4 uses CBF-based variant of the first simplification mechanism (“Simplification #1a” as shown in Figure 3). “Simplification #1b” mechanism is used only in Table 5 in combination with the second simplification mechanism.

Figure 3. The first simplification mechanism

Figure 4. The second simplification mechanism

Table 1. Performance of the adaptive reference samples smoothing filter. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.97% | -1.09% | -1.01% | 164% | 95% |
| Class B | -0.92% | -1.17% | -1.18% | 159% | 96% |
| Class C | -0.91% | -1.22% | -1.10% | 161% | 102% |
| Class D | -0.75% | -1.12% | -0.91% | 166% | 100% |
| Class E | -1.22% | -2.05% | -1.99% | 155% | 100% |
| Class F | -0.73% | -0.44% | -0.19% | 156% | 98% |
| Overall (Ref) | -0.90% | -1.14% | -1.03% | 160% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.32% | -0.83% | -0.82% | 106% | 106% |
| Class B | -0.40% | -0.86% | -0.97% | 105% | 106% |
| Class C | -0.49% | -0.77% | -1.03% | 105% | 100% |
| Class D | -0.21% | -0.02% | -2.26% | 106% | 101% |
| Class E |  |  |  |  |  |
| Class F | -0.52% | 0.37% | -0.73% | 104% | 100% |
| Overall (Ref) | -0.39% | -0.44% | -1.15% | 105% | 103% |

Table 2. Simplification #1a of the adaptive reference samples smoothing filter. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.96% | -1.08% | -0.99% | 158% | 89% |
| Class B | -0.92% | -1.16% | -1.20% | 154% | 93% |
| Class C | -0.94% | -1.15% | -1.18% | 158% | 100% |
| Class D | -0.75% | -1.11% | -0.90% | 160% | 101% |
| Class E | -1.22% | -1.93% | -2.02% | 147% | 99% |
| Class F | -0.73% | -0.34% | -0.29% | 151% | 97% |
| Overall (Ref) | -0.91% | -1.10% | -1.06% | 155% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.32% | -1.04% | -1.16% | 109% | 96% |
| Class B | -0.48% | -0.41% | -0.96% | 102% | 100% |
| Class C | -0.50% | -0.16% | -0.83% | 105% | 100% |
| Class D | -0.13% | -0.25% | -2.06% | 108% | 113% |
| Class E |  |  |  |  |  |
| Class F | -0.73% | -0.48% | -0.33% | 103% | 98% |
| Overall (Ref) | -0.43% | -0.47% | -1.06% | 105% | 101% |

Table 3. Simplifications #1a, #2 of the adaptive reference samples smoothing filter. “Tool-on” test, anchor is HM-16.6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.84% | -1.18% | -1.17% | 153% | 98% |
| Class B | -0.75% | -1.23% | -1.37% | 140% | 99% |
| Class C | -0.72% | -1.14% | -1.24% | 149% | 100% |
| Class D | -0.62% | -1.11% | -0.91% | 150% | 100% |
| Class E | -0.81% | -1.70% | -1.66% | 129% | 99% |
| Class F | -0.51% | -0.86% | -0.47% | 136% | 97% |
| Overall (Ref) | -0.71% | -1.18% | -1.13% | 143% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.25% | -1.14% | -1.17% | 106% | 102% |
| Class B | -0.41% | -1.17% | -1.02% | 104% | 101% |
| Class C | -0.53% | -0.40% | -0.67% | 105% | 101% |
| Class D | -0.14% | 0.43% | -1.51% | 103% | 100% |
| Class E |  |  |  |  |  |
| Class F | -0.41% | 0.50% | -0.03% | 103% | 101% |
| Overall (Ref) | -0.35% | -0.40% | -0.89% | 104% | 101% |

The experimental results for both simplifications of JEM1 is presented in Table 4. Similarly to Table 3, “Simplification #1a” and “Simplification #2” were used in this JEM1-based simulation. Reference-2 is HM16.6.

Table 4. Simplifications #1a, #2 of the adaptive reference samples smoothing filter. Anchor is JEM1.0, all JEM1.0 tools are enabled

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | 0.06% | 0.09% | -0.10% | 96% | 95% |
| Class B | 0.09% | -0.17% | -0.05% | 93% | 96% |
| Class C | 0.21% | -0.15% | -0.05% | 96% | 100% |
| Class D | 0.06% | 0.42% | 0.03% | 96% | 99% |
| Class E | 0.21% | -0.22% | -0.32% | 92% | 100% |
| Class F | -0.49% | -0.32% | -0.32% | 94% | 99% |
| Overall (Ref) | 0.02% | -0.06% | -0.12% | 95% | 98% |
| Overall (Ref-2) | -12.12% | -21.30% | -22.01% | 2012% | 139% |

As shown in Figure 3, the 1st simplification mechanism could be improved by using the hiding condition (“Simplification #1b”) instead of checking a CBF value (“Simplification #1a”) as a criterion to skip or not the next ARSS pass for a TU. The results of the “on-test” simulation for this variant are given in Table 5 for both simplification mechanisms but with replacing the criterion based on checking a CBF value (“Simplification #1a”)  by the criterion based on checking whether the hiding condition is fulfilled for a given TU or not (“Simplification #1b”).

Table 5. Simplifications #1b, #2 of the adaptive reference samples smoothing filter. “Tool-on” test, anchor is HM-16.6.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A | -0.91% | -1.11% | -1.05% | 147% | 83% |
| Class B | -0.84% | -1.21% | -1.30% | 139% | 91% |
| Class C | -0.83% | -1.12% | -1.09% | 149% | 97% |
| Class D | -0.68% | -1.01% | -0.90% | 151% | 100% |
| Class E | -1.02% | -1.91% | -1.70% | 128% | 92% |
| Class F | -0.63% | -0.58% | -0.43% | 135% | 92% |
| Overall (Ref) | -0.81% | -1.13% | -1.06% | 142% | 92% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A | -0.42% | -1.18% | -1.17% | 100% | 88% |
| Class B | -0.40% | -1.26% | -1.08% | 100% | 95% |
| Class C | -0.48% | -0.70% | -1.07% | 103% | 98% |
| Class D | -0.15% | -0.48% | -1.84% | 102% | 99% |
| Class E |  |  |  |  |  |
| Class F | -0.48% | 0.58% | -0.76% | 98% | 96% |
| Overall (Ref) | -0.39% | -0.64% | -1.18% | 101% | 95% |

To estimate the simplification effect, the following procedure is performed for AI results of tool-on tests:

- Calculate average BD-rate BD=(6×Y+U+V)/8

- Complexity of the encoder C. Decoder complexity is not considered, because it is close to 100% for all the tests.

- Calculate a gain/complexity ratio R of coding gain BD to complexity increase (C-100).

This procedure estimates the BD-rate reduction per one percent of complexity increase. The estimation results are presented in Table 6.

Table 6. Estimation of simplification effect of Adaptive reference samples smoothing filter using “tool-ON” tests

| Criterion | Non-modified ARSS | Simplified ARSS (“Simplification #1a”) | Simplified ARSS (“Simplification #1a”, “Simplification #2”) | Simplified ARSS (“Simplification #1b”, “Simplification #2”) |
| --- | --- | --- | --- | --- |
| Average BD-rate | -0.95% | -0.95% | -0.82% | -0.88% |
| Complexity | 160% | 155% | 143% | 142% |
| Gain / complexity ratio | -0.016 | -0.017 | -0.019 | -0.021 |

# Conclusions

The proposed simplifications provide the reduction of more than a quarter of the ARSS computational complexity for 0.1% coding loss. Based on the simulation results, we propose studying them in an exploration experiment (EE).

# References

[1] G.J Sullivan, and J. Boyce. Report of Question 6/16 "Visual coding," TD 215/WP3, Geneva, Switzerland, 12-23 October 2015.

[2] J. Chen, Y. Chen, M. Karczewicz, X. Li, H. Liu, L. Zhang, and X. Zhao. "Coding tools investigation for next generation video coding," COM 16 – C 806 – E, January 2015.

# Patent rights declaration

Huawei Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).